ETSI SMG

Issued by:		ETSI PT12

Date:			June 1992

Report:			GSM 02.08

Title:			QUALITY OF SERVICE

List of contents:

0.	SCOPE

1.	GENERAL

2.	DESCRIPTION

3.	DEFINITION OF TERMS

3.1.	ALL SERVICES

3.2.	TELEPHONY SERVICES

3.3.	DATA SERVICES

4.	TABLES OF VALUES OF PARAMETERS

4.1.	ALL SERVICES

4.2.	TELEPHONY SERVICES

4.3.	DATA SERVICES

Original Language:	English

No. of Pages:		8

0.	SCOPE

This report describes the Quality of Service of the GSM Connection Part that should be offered to users of the GSM service.  It indicates the performance of the Mobile Network expected to be perceived by the user.  This should be distinguished from GSM 03.05 - Technical Performance Objectives - which defines infrastructure performance at a more detailed level.

This report gives design objectives,  but should not form the basis of any legally binding contract.

1.	GENERAL

This Quality of Service report defines the performance that may be expected from major classes of tele- and bearer service, or from all services.  Figures particular to a given bearer service may be shown on the data sheet for that service, although the definition will be found within this report.

Except in the case of Mobile to Mobile calls within a PLMN, the performance seen by the user will be a function of the performance of both the PLMN and the other network(s) which together form the path between the two end users.  This report defines the performance of the PLMN,  between the MS and the interconnection between the PLMN and the connected network (PSTN,  ISDN,  PSPDN etc).  It cannot define the performance of this connected network since this is beyond the scope of GSM.

2.	DESCRIPTION

Unless otherwise stated, this report defines the performance within the GSM PLMN Area, see GSM 02.11, in 90% of cases, since with a cellular network,  it is not possible to specify the performance achievable 100%, on account of propagation characteristics.

It covers use of MS both static and mobile, at speeds up to 250 km/h, in areas where the multipath delay, as defined in GSM 05.05, does not exceed approx 16us, whether or not the network employs Frequency Hopping. Coverage performance is based on vehicle-mounted mobile stations.  The service area of Portable and Hand Held Units may prove inferior.

It defines the performance of the mobile network, one leg only in the case of mobile to mobile calls within a PLMN.  End to end performance will generally be the summation of that offered by each network (mobile, fixed or transit) and any service connected to it.

This report defines the performance expected in a correctly functioning network.  It does not describe parameters relating to the fault frequency or repair time of infrastructure or mobile equipment, nor to the performance of Service Providers in their commercial transactions with their customers.  It is assumed that only Type Approved MS, in good working order, with a valid IMSI (card, module etc.) are used. Clearly a faulty MS may have an adverse effect on the performance.

Some parameters will be a function of the volume of traffic (radio and/or infrastructure).  It is the target to meet the performance as stated in this report but the implementation of the network is the responsibility of the PLMN operators.

Some parameters, such as call set up time, are difficult to measure directly for the PLMN connection part only.  What actually concerns the user is the total call set up time which may be seen as the total of the call set up times for the PLMN and the connected network(s).  PLMN call set up time may thus be deduced by subtracting the typical call set up time experienced within the connected network from the total call set up time.

3.	DEFINITION OF TERMS

3.1.	ALL SERVICES

3.1.1.	Time to Provide Service

The maximum time from switch-on to when the user is able to send or receive calls.  It is necessary to distinguish between the case when the BCCHs in use on this PLMN are known to the MS and when they are not. This is likely to be known for the Home PLMN,  but with Visited PLMNs, this will depend on whether this MS has previously been operated on that PLMN, and on the storage capability of that MS.

These times do not include the time for any entry or validation of a PIN (where this is required),  but do include signalling exchanges such as authentication, registration/IMSI attach where required.

3.1.2.	Call Success Rate (Mobile Originated)

The probability that a call attempt, made from a mobile within the coverage area, in 90% of cases,  will be successfully signalled to the called network, within the time specified in 3.1.3.  It does not refer to the performance of the called network, and does not include the effects of any congestion on the air interface or within the infrastructure.

3.1.3.	Time to Connect Call (Mobile Originated - PLMN Connection Part)

The maximum time from initiating the CALL SET-UP command to when this command is passed to the called network.  This shall include the time for authentication, where required.

3.1.4	Time to Confirm Instruction to Connect

The maximum time from initiating the CALL SET-UP command to when this (acceptance or rejection) is acknowledged to the user.  This is concurrent with, not additional to, the Time to Connect Call (3.1.3) and indicates to the user that the network has received a request.  It does not include the authentication of the caller or any attempt to connect a call.

3.1.5.	Time to Release Call (Mobile Originated - PLMN Connection Part)

The maximum time from initiating the DISCONNECT command to when this command is passed to the called network.

3.1.6.	Call Reception Success Rate (Mobile Terminated)

The probability of success in calling a mobile station, located in the coverage area, in 90% of cases, within the time specified in 3.1.7.  This is on the basis that the MS is switched on and ready to receive a call, and that there is no congestion.

3.1.7	Time to Alert MS

The maximum time from when the PLMN receives a call for an MS (assumed to be within coverage) to when the Alert is energised. This time includes any delay caused by the use of Discontinuous Reception and authentication (if required).

The time specified is for the case when the first page is successful.  Where this not successful, further paging will be required. The manner of this is a GSM operator choice, but the maximum time should not exceed the "final" figure stated,  after which other actions may occur (eg call divert, if activated).

3.1.8.	Time to Invoke or Change a Supplementary Service

The maximum time from initiating a Supplementary Service Request until when this service has become available or changed (as requested by the user),  including authentication,  if required.

3.1.9.	Duration of Interruption of Call due to Handover

The maximum total interruption to a traffic channel (in either or both directions) due to a single handover.  If there are several short breaks during this process, the sum total of these applies. Different figures may apply for:

a)	MSC to  MSC handover

b)	BS to BS handover within an MSC area

c)	Change of frequency channel on a given BS

d)	Change of timeslot within a channel on a given BS

3.1.10.	Duration of Interruption of User Traffic - inband signalling

The maximum duration of any single interruption of user traffic due to inband signalling, except for the handover break itself. The frequency of this interruption shall be such as to avoid exceeding the limits of performance specified elsewhere in this report.

It is assumed that a DTMF signal, sent during a speech call, is intended to be in place of speech transmission, so is not regarded as an interruption of user traffic.  Its duration is defined in GSM 03.14.

3.1.11.	Handover Success Rate

The probability of handover being successful, within the GSM PLMN coverage area, in 90% of cases,  under low traffic conditions, without resorting to the use of call re-establishment.

3.2.	TELEPHONY SERVICES

3.2.1.	Maximum One Way Delay Time

The maximum delay time in one direction, measured between the microphone/earphone of the MS and the boundary of the PLMN. This figure is likely to be doubled on mobile to mobile calls within a PLMN.

3.2.2.	Probability of Intelligibility

The probability that a speech call within the coverage area is intelligible despite interference.

3.3.	DATA SERVICES

The values of the QOS parameters defined below will vary considerably with the Bearer Service employed, and are therefore listed according to service.  They are TARGET worst case for the conditions given, unless otherwise stated.  The actual figures achieved will also depend on the network realisation by the PLMNs.  Since the networks will be configured primarily for telephony,  it will not be possible to state definitively the performance to be achieved on all networks.  The performance stated here is for the PLMN connection part only.  End to end performance will also be a function of any connected network.

The performance of transparent bearer services will suffer due to lack of inherent ARQ, due to the absence of flow control. However, it is intended that these services will primarily be employed as bearers to carry user protocols,  which themselves will perform these functions.  Where there is no such protocol, the error rate may become excessive at 9.6kb/s on Full Rate Channels (Bm) and 4.8kb/s on Half Rate Channels (Lm), but the performance at lower data rates should be acceptable over most of the cell area.

3.3.1.	Bit Error Ratio (BER)

The proportion of bits in error, as perceived by the user, after FEC (and ARQ if employed by the particular Bearer Service).  The figure quoted is the target average worst-case experienced at the cell boundary, representing 90% of cases.

(It should be noted that where ARQ is employed (non-transparent services) the BER will be a function of the falsing rate of the CRC check.  Errors detected by the CRC check will cause a block to be repeated, hence potentially affecting the delay. The ARQ employed on the non-transparent services is applied between the MS and the IWF.  Any errors on a connected network will be additional.)

3.3.2.	Delay

The single path delay from when a character is sent from the DTE to when it leaves the PLMN (or reverse direction).  This figure may be doubled for mobile to mobile calls within the PLMN.  With the Transparent Services, this is a fixed value, generally irrespective of signal quality or error rate.

With Non-transparent Services,  the figure shown is the value which will apply under ideal conditions, when no repeats occur, so is the lowest value that can be achieved.  ARQ repeats will increase this delay, which may reduce the maximum throughput rate to be expected near the cell boundary.  The maximum delay under worst-case conditions depends on the statistical nature of the propagation and interference. The maximum delay under a model of defined conditions can be the basis of further study.

3.3.3.	Throughput - Non-transparent Services

Throughput is the average user data rate that can be achieved over the stated coverage area.  It is intended that this should be the nominal maximum data rate for asynchronous services.  The throughput may be expected to fall when outside the stated coverage area.

With non-transparent,  synchronous services, the throughput may be significantly reduced, particularly in the case of less efficient user protocols.  The figure shown is the best which is likely to be able to be achieved under the stated conditions;  it could well be considerably worse.

3.3.4.	Coverage Area - Non-transparent Services

On Non-transparent Services, the error rate is determined by the CRC check in the ARQ protocol, but the throughput may fall if too many repeats occur.  Coverage area is defined as the area of the cell over which there is useful service, with the throughput typically achieved stated in kb/s.  Where appropriate,  this includes an allowance for some repeats without degradation of throughput.

3.3.5.	Transparent Services

On Transparent Services, ARQ will not be employed, so there will be no repeats.  Hence the throughput will be the same as the input rate, so is not a useful concept.  The typical error rate over 90% of cases is shown.

4.	TABLES OF VALUES OF PARAMETERS DEFINED IN 3.

4.1.	ALL SERVICES

4.1.1.	Time to Provide Service

- BCCHs known                                       4 sec

- BCCHs not known                                  10 sec

4.1.2.	Call Success Rate                               >99%

4.1.3.	Time to Connect Call (to boundary of PLMN):    4 sec

4.1.4.	Time to Confirm Instruction to Connect:        1 sec

4.1.5.	Time to Release Call (to boundary of PLMN):    2 sec

4.1.6.	Call Reception Success Rate:                    >99%

4.1.7.	Time to Alert MS:     a) First page            4 sec

b) Final attempt        15 sec

4.1.8.	Time to Invoke or Change a Suppl. Service:    10 sec

4.1.9.	Duration of Interruption of Call due to Handover:

a) MSC to  MSC handover                                150 ms

b) BS to BS handover within an MSC area                150 ms

c) Change of frequency channel on a given BS           100 ms

d) Change of timeslot within a channel on a given BS   100 ms

4.1.10.	Duration of Interruption of User Traffic -

Inband Signalling (not due to handover)             40 ms

4.1.11.	Handover Success Rate                           >99%

4.2.	TELEPHONY SERVICES

4.2.1.	Maximum Delay Time:

Full Rate Channel                      90 ms

Half Rate Channel                      90 ms

4.2.2.	Probability of Intelligibility:                 >90%

4.3.	DATA SERVICES

Performance - PLMN Connection Part only:

Service            BER       Delay      Throughput    Coverage

Max        Max         Rate          Area

Non-transparent   10E-8      190ms        9.6kb/s        90%

Async Bm

Non-transparent   10E-8      325ms        4.8kb/s        90%

Async Lm

Non-transparent   10E-8      190ms          8kb/s        90%

Sync Bm

Non-transparent   10E-8      325ms          4kb/s        90%

Sync Lm

Trans Bm          10E-3      100ms           NA          90%

9.6kb/s

Trans Bm          10E-5      100ms           NA          90%

4.8kb/s

Trans Lm          10E-3      200ms           NA          90%

4.8kb/s

Trans Bm          10E-6      200ms           NA          90%

2.4kb/s and below

Trans Lm          10E-5      200ms           NA          90%

2.4kb/s and below