ETSI/GSM

Recommendation GSM 08.58-DCS                        version 3.0.0

Title :Base Station Controller (BSC) to Base Transceiver Station (BTS) Interface (based on GSM 08.58 v. 3.5.0).

Layer 3 Specification

Date :    January 1991

List of Contents :

1. Scope

2. Protocol Model

3. Radio Link Layer Management Procedures

4. Dedicated Channel Management Procedures

5. Common Channel Management Procedures

6. TRX Management Procedures

7. Error Handling

8. Message Formats and Contents

9. Information Element Codings

Original Language : English

Number of Pages :   3

GSM 08.58-DCS - v. 3.0.0 - page 2

1.   SCOPE

The use and general aspects of the BSC to BTS interface (the A-bis interface) are given in Recommendation GSM 08.51.

This recommendation specifies the general structure of layer 3 and traffic management procedures and messages used on the A-bis interface to support signalling procedures as defined in Recommendation GSM 04.08.

Network management procedures and messages for the A-bis interface are defined in Recommendation GSM 08.59.

The functional split between BSC and BTS is defined in recommendation 08.52. The procedures and messages required to support this split are defined in detail in this recommendation.

This delta recommendation only includes modified part concerning specifically the DCS 1800 short term solution (phase 1).

2.   PROTOCOL MODEL

A model for L3 can be found in figure 2.1.

L2 addressing is made to TRX (or BCF) using the TEI of LAPD. Different L2 links are used for traffic management messages (RSL, Radio Signalling Link), network management messages (OML, Operation & Maintenance Link) and L2 management messages (L2ML, Layer 2 Management Link).

For traffic management, two types of signalling messages have been defined:

Transparent Messages:Messages which are forwarded by BTS without interpretation or changes.

Non-Transparent Messages:Messages which are sent only between BSC and BTS and which BTS is acting upon or which are the results of BTS actions.

In addition, the messages have been grouped into four main groups: Radio Link Layer Management, Dedicated Channel Management, Common Channel Management and TRX Management messages.

Discrimination between these types and groups is based on the Message Discriminator which is sent as the first octet in all messages. Transparent and non-transparent messages are discriminated by a transparancy flag (T-bit) in the Message Discriminator. Transparent messages are merely forwarded to L2 on the radio interface.

In order to address the relevant radio channel, a Channel Number element is included to support the distribution of messages to relevant physical channels on the TRX. A Link Identifier element supports the distribution on logical links/channels on the radio interface (compare the DLCI element of the A interface, Rec 08.06).

All messages in this recommendation are to be transmitted on the A-bis interface using the I format of LAPD, except for "MEASUREMENT RESULT" which is sent in UI format.

GSM 08.58-DCS - v. 3.0.0 - page 77

9.3.28  Message Identifier

This element is used to indicate a message type within a message.

8     7     6     5     4     3     2     1

┌───────────────────────────────────────────┐

│            Element identifier             │ 1

├───────────────────────────────────────────┤

│            Message Type                   │ 2

└───────────────────────────────────────────┘

Octet 2 is coded as the Message Type information element, section 9.2.

9.3.29  Message Indicator

This element is used to indicate that a complete message as defined in section 8 follows, starting in the next octet. The element consists of only the information element identifier.

8     7     6     5     4     3     2     1

┌───────────────────────────────────────────┐

│            Element identifier             │ 1

└───────────────────────────────────────────┘

9.3.30  System Info Type

This element is used to indicate the type of SYSTEM INFORMATION message as defined in Recommendation GSM 04.08 (SYSTEM INFORMATION 1 through 8).

8     7     6     5     4     3     2     1

┌───────────────────────────────────────────┐

│            Element identifier             │ 1

├─────────────────────┬─────────────────────┤

│     Reserved        │   Sys Info Type     │ 2

└─────────────────────┴─────────────────────┘

The Sys Info Field (bits 1-4 of octet 2) indicates the SYSTEM INFORMATION message. It is coded as follows:

Value          System information message

0 0 0 1     SYSTEM INFORMATION 1

0 0 1 0     SYSTEM INFORMATION 2

0 0 1 1     SYSTEM INFORMATION 3

0 1 0 0     SYSTEM INFORMATION 4

0 1 0 1     SYSTEM INFORMATION 5

0 1 1 0     SYSTEM INFORMATION 6

1 0 1 0     SYSTEM INFORMATION 2bis

1 1 0 1     SYSTEM INFORMATION 5bis

All other values are reserved.