3GPP TS 23.203 V1.1.0 (2006-08)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Policy and charging control architecture

(Release 7)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Charging, QoS, policy control

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2006, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

# Contents

Foreword	7

Introduction	7

1	Scope	8

2	References	8

3	Definitions, symbols and abbreviations	9

3.1	Definitions	9

3.3	Abbreviations	10

4	High level requirements	10

4.2	Charging related requirements	11

4.2.1	General	11

4.2.2	 Charging models	11

4.2.3	 Examples of Service Data Flow Charging	12

4.3	Policy control requirements	12

4.3.1	General	12

4.3.2	Gating control	12

4.3.3	QoS control	12

4.3.3.1	QoS control at service data flow level	12

4.3.3.2	QoS control at IP-CAN bearer level	13

5	Architecture model and reference points	13

5.1	 Reference architecture	13

5.2	Reference points	14

5.2.2	 Gx reference point	14

5.2.3	 Sp reference point	15

5.2.4	 Gy reference point	15

5.2.5	Gz reference point	15

6	Functional description	15

6.1	Overall description	15

6.1.0	General	15

6.1.1	Binding mechanism	15

6.1.2	Reporting	16

6.1.3	Credit management	17

6.1.4	Event Triggers	17

6.1.5	Policy Control	18

6.2	Functional entities	19

6.2.1	Policy Control and Charging Rules Function (PCRF)	19

6.2.1.1	Input for PCC decisions	19

6.2.2	Policy and Charging Enforcement Function (PCEF)	20

6.2.2.1	General	20

6.2.2.2	Service data flow detection	22

6.2.2.3	Measurement	24

6.2.3	Application Function (AF)	25

6.2.4	Subscription Profile Repository (SPR)	25

6.2.5	Service Data Flow Based Credit Control Function	25

6.2.6	Offline Charging System (OFCS)	25

6.3	Policy control and charging rule	26

6.3.1	General	26

6.3.2	Policy and charging control rule operations	29

6.4	IP-CAN bearer and IP-CAN session related policy information	29

7	PCC Procedures and flows	30

7.1	Introduction	30

7.2	IP-CAN Session Establishment	30

7.3	IP-CAN Session Termination	32

7.3.1	UE initiated IP-CAN Session termination	32

7.3.2	GW initiated IP-CAN Session termination	33

7.4	IP-CAN Session Modification	34

7.4.1	IP-CAN Session Modification; GW(PCEF) initiated	34

7.4.2	IP-CAN Session Modification; PCRF initiated	35

Annex A (normative):	Access specific aspects	37

A.1	GPRS	37

A.1.0	General	37

A.1.1	High level requirements	37

A.1.1.1	General	37

A.1.1.2	Charging related requirements	37

A.1.1.3	Policy control requirements	37

A.1.2	Architecture model and reference points	38

A.1.2.1	Reference points	38

A.1.2.1.1	Gx reference point	38

A.1.3	Functional description	38

A.1.3.1	Overall description	38

A.1.3.1.1	Binding mechanism	38

A.1.3.1.2	Reporting	38

A.1.3.1.3	Credit management	38

A.1.3.1.4	Event Triggers	38

A.1.3.2	Functional entities	39

A.1.3.2.1	 Policy Control and Charging Rules Function (PCRF)	39

A.1.3.2.1.1	Input for PCC decisions	39

A.1.3.2.2	 Policy and Charging Enforcement Function (PCEF)	39

A.1.3.2.2.1	General	39

A.1.3.2.2.2	Service data flow detection	40

A.1.3.2.2.3	Packet Routeing and Transfer Function	40

A.1.3.2.2.4	Measurement	40

A.1.3.2.3	 Application Function (AF)	40

A.1.3.3	Policy and charging control rule	41

A.1.3.3.1	General	41

A.1.3.3.2	 Policy and charging control rule operations	41

A.1.3.4	IP-CAN bearer and IP-CAN session related policy information	41

A.1.4	PCC Procedures and flows	41

A.1.4.1	Introduction	41

A.1.4.2	IP-CAN Session Establishment	41

A.1.4.3	IP-CAN Session Termination	41

A.1.4.3.1	UE initiated IP-CAN Session termination	41

A.1.4.3.2	GW initiated IP-CAN Session termination	41

A.1.4.4	IP-CAN Session Modification	42

A.1.4.4.1	IP-CAN Session Modification; GW (PCEF) initiated	42

A.1.4.4.2	IP-CAN Session Modification; PCRF initiated	42

A.2	I-WLAN	42

A.2.1	High level requirements	43

A.2.1.1	General	43

A.2.1.2	Charging related requirements	43

A.2.1.3	Policy control requirements	43

A.2.2	Architecture model and reference points	43

A.2.2.1	Reference points	43

A.2.2.1.1	Rx reference point	43

A.2.2.1.2	Gx reference point	43

A.2.2.1.3	Sp reference point	43

A.2.3	Functional description	44

A.2.3.1	Overall description	44

A.2.3.1.1	Binding mechanism	44

A.2.3.2	Functional entities	44

A.2.3.2.1	 Policy Control and Charging Rules Function (PCRF)	44

A.2.3.2.1.1	Input for PCC decisions	44

A.2.3.2.2	 Policy and Charging Enforcement Function (PCEF)	44

A.2.3.2.3	 Application Function (AF)	44

A.2.3.3	Policy and charging control rule	44

A.2.3.3.1	General	44

A.2.3.3.2	 Policy and charging control rule operations	44

A.2.4	PCC Procedures and flows	44

A.2.4.1	Introduction	44

A.1.4.2	IP-CAN Session Establishment	44

A.1.4.3	IP-CAN Session Termination	44

A.1.4.3.1	UE initiated IP-CAN Session termination	44

A.1.4.3.2	GW initiated IP-CAN Session termination	45

A.1.4.4	IP-CAN Session Modification	45

A.1.4.4.1	IP-CAN Session Modification; GW (PCEF) initiated	45

A.1.4.4.2	IP-CAN Session Modification; PCRF initiated	45

A.3	DOCSIS IP-CAN	45

A.3.1	General	45

A.3.1	High level requirements	46

A.3.1.1	General	46

A.3.1.2	Charging related requirements	47

A.3.1.3	Policy control requirements	47

A.3.2	Architecture model and reference points	47

A.3.2.1	Reference points	47

A.3.2.1.1	Rx reference point	47

A.3.2.1.2	Gx reference point	47

A.3.2.1.3	Sp reference point	47

A.3.3	Functional description	47

A.3.3.1	Overall description	47

A.3.3.1.1	Binding mechanism	47

A.3.3.2	Functional entities	48

A.3.3.2.1	 Policy Control and Charging Rules Function (PCRF)	48

A.3.3.2.1.1	Input for PCC decisions	48

A.3.3.2.2	 Policy and Charging Enforcement Function (PCEF)	48

A.3.3.2.3	 Application Function (AF)	48

A.3.3.3	Policy and charging control rule	49

A.3.3.3.1	General	49

A.3.3.3.2	Policy and charging control rule operations	49

A.x	<IP-CAN>	49

A.x.1	High level requirements	49

A.x.1.1	General	49

A.x.1.2	Charging related requirements	49

A.x.1.3	Policy control requirements	49

A.x.2	Architecture model and reference points	49

A.x.2.1	Reference points	49

A.x.2.1.1	Rx reference point	49

A.x.2.1.2	Gx reference point	49

A.x.2.1.3	Sp reference point	49

A.x.3	Functional description	50

A.x.3.1	Overall description	50

A.x.3.1.1	Binding mechanism	50

A.x.3.2	Functional entities	50

A.x.3.2.1	 Policy Control and Charging Rules Function (PCRF)	50

A.x.3.2.1.1	Input for PCC decisions	50

A.x.3.2.2	 Policy and Charging Enforcement Function (PCEF)	50

A.x.3.2.3	 Application Function (AF)	50

A.x.3.3	Policy and charging control rule	50

A.x.3.3.1	General	50

A.x.3.3.2	 Policy and charging control rule operations	50

Annex B (informative):	PCC and usage of visited network services	51

B.1	Introduction	51

B.1.1	General aspects	51

B.1.2	Charging related aspects	51

B.1.2.1	Reporting	51

B.1.2.2	Credit Control	51

B.1.3	Policy control related aspects	51

B.1.4	Subscription related aspects	51

B.1.5	Architectural aspects	52

B.1.5.1	Logical architecture of PCEF in the visited network	52

Annex C  (informative):	PCC (GGSN) initiated QoS upgrade in GPRS	53

C.1	 Introduction	53

C.1.1	General aspects	53

C.1.2	QoS upgrade via PDP context modification	53

C.1.3	QoS upgrade via modification at PDP context establishment or UE initiated context modification	53

C.1.4	QoS upgrade via establishment of a new PDP context	53

C.1.5	Potential Issues	53

C.1.6	Architectural Issues	54

Annex D (informative):	Change history	55

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

Previous Releases have separately specified the following features:

-	Enhanced policy control to allow the operator to perform service based QoS policy control for GPRS access for their session-based PS applications;

-	IP Flow-based charging to allow for more granularity for end-user charging, accounting and online credit control.

While some level of convergence between these functions has already been achieved in previous Releases, a full harmonization and merger of these functions is specified hereby to allow for optimizing real-time interactions in the PS network.

NOTE:	The present document replaces TS 23.125 [7] from this Release onwards.

# 1	Scope

The present document specifies the overall stage 2 level functionality for Policy and Charging Control that encompasses the following high level functions for IP-CANs (e.g. GPRS, I-WLAN, Fixed Broadband, etc.):

-	Flow Based Charging, including charging control and online credit control;

-	 Policy control (e.g. gating control, QoS control, etc.).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 41.001: "GSM Release specifications".

[2]	Void.

[3]	3GPP TS 32.240: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; Charging management; Charging architecture and principles".

[4]	IETF RFC 4006: "Diameter Credit-Control Application".

[5]	3GPP TS 23.207: "End-to-end Quality of Service (QoS) concept and architecture"

[6]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and Functional Description".

[7]	3GPP TS 23.125: "Overall architecture aspects for IP flow based bearer level charging – stage 2".

[8]	3GPP TS 21.905: "Vocabulary for 3GPP Specifications".

[9]	3GPP TS 32.251: "Packet Switched (PS) domain charging".

[10]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[11]	3GPP TR 33.919: "Generic Authentication Architecture (GAA)".

[12]	3GPP TS 23.060: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; General Packet Radio Service (GPRS); Service description; Stage 2".

[13]	3GPP TS 23.234: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3GPP system to Wireless Local Area Network (WLAN) interworking; System description".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [8] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [8].

authorised QoS: The maximum QoS that is authorised for a service data flow. In case of an aggregation of multiple service data flows within one IP-CAN bearer (e.g. for GPRS a PDP context), the combination of the "Authorised QoS" information of the individual service data flows is the "Authorised QoS" for the IP-CAN bearer. It contains the QoS class and the data rate.

binding: The association between a service data flow and the IP-CAN bearer (for GPRS the PDP context) transporting that service data flow.

binding mechanism: The method for creating, modifying and deleting bindings.

charging control: The process of associating packets, belonging to a service data flow, to a charging key and applying online charging and/or offline charging, as appropriate.

charging key: information used by the online and offline charging system for rating purposes.

gating control: The process of blocking or allowing packets, belonging to a service data flow, to pass through to the desired endpoint.

GPRS IP-CAN: This IP-CAN incorporates GPRS over GERAN and UTRAN, see TS 23.060 [12].

IP-CAN bearer: An IP transmission path of defined capacity, delay and bit error rate, etc. See TS 21.905 [8] for the definition of bearer.

IP-CAN session: The association between a UE and a PDN identifier (for GPRS, APN). The association is identified by a UE IP address together with a UE identity information, if available. An IP-CAN session incorporates one or more IP-CAN bearers. Support for multiple IP-CAN bearers per IP-CAN session is IP-CAN specific. An IP-CAN session exists as long as the UE IP address is established and announced to the IP network.

I-WLAN IP-CAN: This IP-CAN incorporates 3GPP IP access of I-WLAN, see TS 23.234 [13].

packet flow: A specific user data flow carried through the PCEF. A packet flow can be an IP flow.

PCC rule: A set of information enabling the detection of a service data flow and providing parameters for policy control and/or charging control.

service data flow: An aggregate set of packet flows.

service data flow filter: A set of IP header parameter values/ranges used to identify one or more of the packet flows constituting a service data flow. A service data flow filter of a PCC rule that is predefined in the PCEF may use parameters that extend the packet inspection beyond the IP 5 tuple.

service data flow template: The set of service data flow filters in a PCC rule, required for defining a service data flow.

service identifier: An identifier for a service. The service identifier provides the most detailed identification, specified for flow based charging, of a service data flow. A concrete instance of a service may be identified if additional AF information is available (further details to be found in clause 6.3.1).

session based service: An end user service requiring application level signalling, which is separated from service rendering.

PCC decision: A decision consists of PCC rules and IP-CAN bearer attributes, which is provided by the PCRF to the PCEF for policy and charging control.

policy control: The process whereby the PCRF indicates to the PCEF how to control the IP-CAN bearer. Policy control includes QoS control and/or gating control.

subscriber category: is a means to group the subscribers into different classes, e.g. gold user, the silver user and the bronze user.

Pre-defined PCC Rule: a PCC rule that has been provisioned directly into the PCEF by the operator.

Dynamic PCC Rule: a PCC rule for which the definition is provided into the PCEF via the Gx reference point.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [8] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [8].

AF	Application Function

CRF	Charging Rules Function

FBC	Flow Based Charging

H-PCEF	A PCEF in the HPLMN

IP-CAN	IP Connectivity Access Network

OFCS	Offline Charging System

OCS	Online Charging System

PCC	Policy and Charging Control

PCEF	Policy and Charging Enforcement Function

PCRF	Policy and Charging Rules Function

PDF	Policy Decision Function

PEP	Policy Enforcement Point

SBLP	Service Based Local Policy

SPR	Subscription Profile Repository

TPF	Traffic Plane Function

V-PCEF	A PCEF in the VPLMN

# 4	High level requirements

## 4.1			General requirements

Editors' note:	The requirements currently included in TS 23.125 need to be checked to ensure all appropriate text is included in this TS.

Editors' note:	Check TR 23.803 for suitable text to cover backwards compatibility requirements.

It shall be possible for the PCC architecture to base decisions upon subscription information.

It shall be possible to apply policy and charging control to any kind of 3GPP IP-CAN. Applicability of PCC to other IP-CANs is not restricted. However, it shall be possible for the PCC architecture to base decisions upon the type of IP-CAN used.

The PCC architecture shall discard packets that don't match any service data flow filter of the active PCC rules. It shall also be possible for the operator to define PCC rules, with wild-carded service data flow filters, to allow for the passage and charging for packets that do not match any service data flow filter of any other active PCC rules.

The PCC architecture shall allow the charging control to be applied on a per service data flow basis, independent of the policy control.

The PCC architecture shall have a binding method that allows the unique association between service data flows and their IP-CAN bearer.

A single service data flow template shall suffice, to detect a service data flow, for the purpose of both policy control and flow based charging.

A PCC rule may be predefined or dynamically provisioned at establishment and during the lifetime of an IP-CAN session. The latter is referred to as a dynamic PCC rule.

The number of real-time PCC interactions shall be minimized. This requires a single optimized interface between the PCC nodes.

PCC shall be enabled on a per PDN basis (represented by an access point and the configured range of IP addresses) at the PCEF. It shall be possible for the operator to configure the PCC architecture to perform charging control, policy control or both for a PDN access.

PCC shall support roaming users.

Editor's Note:	Detailed aspects of PCC usage in roaming scenarios are being developed in Annex B.

## 4.2	Charging related requirements

### 4.2.1	General

In order to allow for charging control, the information in the PCC rule identifies the service data flow and specifies the parameters for charging control. The PCC rule information may depend on subscription data.

For the purpose of charging correlation between application level (e.g. IMS) and service data flow level, applicable charging identifiers shall be passed along within the PCC architecture, if such identifiers are available.

### 4.2.2		Charging models

When developing the charging solutions, the following charging models should be considered, even though the full solution to support the models may not be within the scope of this TS.

Shared revenue services shall be supported. In this case settlement for all parties shall be supported, including the third parties that may have been involved providing the services.

The charging solution shall allow various charging models such as:

-	Volume based charging;

-	Time based charging;

-	Volume and time based charging;

-	Event based charging;

-	No charging.

Note: The charging model - "No charging" implies that that charging control is not applicable.

It shall be possible to apply different rates and charging models when a user is identified to be roaming from when the user is in the home network. Furthermore, it shall be possible to apply different rates and charging models based on the location of a user, beyond the granularity of roaming.

It shall be possible to restrict special rates to a specific service, e.g. allow the user to download a certain volume of data from one service for free, but this allowed volume is not transferable to other services. It shall be possible also to apply special rates based on the time of day.

It shall be possible to enforce per-service usage limits for a service data flow using online charging on a per user basis (may apply to prepaid and post-paid users).

It shall be possible for online charging systems to check the amount of data used over some time period. The online charging systems can provide both volume credit and time indication. In case the PCEF detects the counted volume reaches the volume credit or the counted time reaches the indicated period of time, the PCEF shall send a request for credit to the OCS with the remaining time value and/or remaining credit volume.

In the case of online charging, it shall be possible to perform rating and allocate credit depending on the characteristics of the IP-CAN bearer resources allocated initially.

The flow based bearer level charging can support dynamic selection of charging to apply. A number of different inputs can be used in the decision to identify the specific charging to apply. For example, a service data flow may be charged with different rates depending on what QoS is applicable. The charging rate may thus be modified when an IP-CAN bearer is created or removed, to change the QoS provided for a service data flow.

The charging rate or charging model applicable to a service data flow may also be changed as a result of events in the service (e.g. insertion of a paid advertisement within a user requested media stream). The charging model applicable to a service data flow may also change as a result of events identified by the OCS (e.g. after having spent a certain amount of time and/or volume, the user gets to use some services for free). The charging rate or charging model applicable to a service data flow may also be changed as a result of having used the service data flow for a certain amount of time and/or volume.

In the case of online charging, it shall be possible to apply an online charging action upon PCEF events (e.g. re-authorization upon QoS change).

It shall be possible to indicate to the PCEF that interactions with the charging systems are not required for a PCC rule, i.e. to perform neither accounting nor credit control for this service data flow, and then no offline charging information is generated.

### 4.2.3		Examples of Service Data Flow Charging

There are many different services that may be used within a network, including both user-user and user-network services. Service data flows from these services may be identified and charged in many different ways. A number of examples of configuring PCC rules for different service data flows are described below.

A network server provides an FTP service. The FTP server supports both the active (separate ports for control and data) and passive modes of operation. A PCC rule is configured for the service data flows associated with the FTP server for the user. The PCC rule uses a filter specification for the uplink that identifies packets sent to port 20 or 21 of the IP address of the server, and the origination information is wildcarded. In the downlink direction, the filter specification identifies packets sent from port 20 or 21 of the IP address of the server.

A network server provides a "web" service. A PCC rule is configured for the service data flows associated with the HTTP server for the user. The PCC rule uses a filter specification for the uplink that identifies packets sent to port 80 of the IP address of the server, and the origination information is wildcarded. In the downlink direction, the filter specification identifies packets sent from port 80 of the IP address of the server.

The same server also provides a WAP service. The server has multiple IP addresses, and the IP address of the WAP server is different from the IP address of the web server. The PCC rule uses the same filter specification as for the web server, except the IP address is different.

An operator offers a zero rating for network provided DNS service. A PCC rule is established setting all DNS traffic to/from the operators DNS servers as offline charged. The data flow filter identifies the DNS port number, and the source/destination address within the subnet range allocated to the operators network nodes.

An operator has a specific charging rate for user-user VoIP traffic over the IMS. A PCC rule is established for this service data flow. The filter information to identify the specific service data flow for the user-user traffic is provided by the P CSCF.

An operator is implementing UICC based authentication mechanisms for HTTP based services utilizing the GAA Framework as defined in TR 33.919 [11] by e.g. using the Authentication Proxy. The Authentication Proxy may appear as an AF and provide information to the PCRF for the purpose of selecting an appropriate PCC Rule.

## 4.3	Policy control requirements

### 4.3.1	General

The policy control features comprise gating control and  QoS control.

### 4.3.2	Gating control

Gating control shall be applied on a per service data flow basis.

To enable the PCRF gating control decisions, the AF shall report session events (e.g. session termination, modification) to the PCRF. For example, session termination, in gating control, may trigger the blocking of packets or "closing the gate".

### 4.3.3	QoS control

#### 4.3.3.1	QoS control at service data flow level

It shall be possible to apply QoS control on a per service data flow basis.

Editor's note:	Minimum QoS authorization is FFS.

QoS control per service data flow allows the PCC architecture to provide the PCEF with the authorized QoS to be enforced for each specific service data flow.  Criteria such as the QoS subscription information may be used together with policy rules such as, service-based, subscription-based, or pre-defined PCRF internal policies to derive the authorized QoS to be enforced for a service data flow.

It shall be possible to apply multiple PCC rules, without application provided information, using different authorised QoS within a single IP-CAN session and within the limits of the Subscribed QoS profile.

Editors note: the applicability of the subscribed QoS profile for PCC rules that are pre-defined in the PCEF are FFS.

Editor's note:	IP-CAN specifics of QoS control per service data flow need to be detailed in the corresponding annexes.

Editor's note:	The relation between policy control and accounting when QoS control per service data flow is applied is for further study. e.g. accounting of a discarded packet.

Editor's note:	Dependencies between IP-CAN bearer level QoS control and service data flow QoS control are FFS.

#### 4.3.3.2	QoS control at IP-CAN bearer level

It shall be possible for the PCC architecture to support control of QoS reservation procedures (UE-initiated or network-initiated) for IP-CANs that support such procedures for its IP-CAN bearers. It shall be possible to determine the QoS to be applied in QoS reservation procedures (QoS control) based on the authorised QoS of the service data flows that are applicable to the IP-CAN bearer and on criteria such as the QoS subscription information, service based policies, and/or pre-defined PCRF internal policies. Details of QoS reservation procedures are IP-CAN specific and therefore, the control of these procedures is described in Annex A.

It shall be possible for the PCC architecture to support control of QoS for the packet traffic of IP-CANs.

The PCC architecture shall be able to provide policy control in the presence of NAT devices. This may be accomplished by providing appropriate address and port information to the PCRF.

The enforcement of the control for QoS reservation procedures for an IP-CAN bearer shall allow for a downgrading of the requested QoS as part of a UE-initiated IP-CAN bearer establishment and modification. The PCC architecture shall be able to provide a mechanism to initiate IP-CAN bearer establishment and modification (for IP-CANs that support such procedures for its bearers) as part of the QoS control.

Editor's note:	The specific details of network controlled IP-CAN bearer establishment and modifications may be detailed in the IP-CAN specific annexes and are FFS.

Editor's note:	the ability to upgrade the requested IP-CAN bearer QoS as part of IP-CAN bearer establishment and modification is FFS.

Editor's note:	QoS enforcement shall be supported in line with PEP capabilities defined for SBLP in TS 23.207 [5].

# 5	Architecture model and reference points

## 5.1		Reference architecture

The PCC functionality is comprised by the functions of the Policy and Charging Enforcement Function, the Policy and Charging Rules Function, the Application Function, the Online Charging System, the Offline Charging System and the Subscription Profile Repository. The PCC architecture extends the architecture of an IP-CAN, where the Policy and Charging Enforcement Function is a functional entity in the Gateway node implementing the IP access to the PDN (i.e. for GPRS, the GGSN, and for WLAN, the PDG).

- Figure 5.1: Overall PCC logical architecture

## 5.2	Reference points

The Rx reference point resides between the AF and the PCRF.

Editors' note:	The Rx reference point shall combine Rel-6 Rx reference point together with Rel-6 Gq reference point within a single protocol. This single protocol shall allow Rel-7 Rx to be backwards compatible with all Rel-6 capabilities of Gq and Rx reference points (i.e. Rel-7 PCRF can support interacting with Rel-6 AFs and Rel-7 AFs can support interacting with a Rel-6 PDF and/or Rel-6 CRF).

This reference point enables transport of application level session information from AF to PCRF. Such information includes, but is not limited to:

-	IP filter information to identify the service data flow for policy control and/or differentiated charging;

-	Media/application bandwidth requirements for QoS control.

### 5.2.2		Gx reference point

The Gx reference point resides between the PCEF and the PCRF.

The Gx reference point enables a PCRF to have dynamic control over the PCC behaviour at a PCEF.

Editors' note-i:	The evolved Gx reference point shall allow for all Rel-6 Gx capabilities enabling the use of service data flow based charging rules.

Editors' note-ii:	In the PCC architecture the existing functionality from previous releases of the Go reference point is realized together with Gx reference point with a single protocol, using single message sequence to communicate both SBLP decisions and FBC decisions. Thus, the existing rel-6 Gx protocol shall be enhanced with the necessary information elements to fulfil also SBLP requirements as described in clause 4.3 of the present specification.

Editors' note-iii:	Rel-7 Gx shall evolve the charging rules defined in TS 23.125 [7] to support policy functionality (uplink and downlink).

The Gx reference point enables the signalling of PCC decision, which governs the PCC behaviour, and it supports the following functions:

-	Initialisation and maintenance of connection (IP-CAN session);

-	Request for PCC decision from PCEF to PCRF;

-	Provision of PCC decision from PCRF to PCEF;

-	Negotiation of IP-CAN bearer establishment mode (UE only, NW only or NW/UE);

-	Termination of connection (IP-CAN session).

A PCC decision consists of one or more PCC rule(s) and IP-CAN attributes. The information contained in a PCC rule is defined in clause 6.3.

### 5.2.3		Sp reference point

The Sp reference point lies between the SPR and the PCRF.

The Sp reference point allows the PCRF to request subscription information related to the IP-CAN transport level policies from the SPR based on a subscriber ID, a PDN identifier and possible further IP-CAN session attributes, see Annex A. For example, the subscriber ID can be IMSI. The reference point allows the SPR to notify the PCRF when the subscription information has been changed if the PCRF has requested such notifications.

Editor's Note:	Authorization of service usage based on subscription information may also be undertaken on the application level. Precedence and resolution of possible conflicts between transport level and application level service authorization is FFS.

### 5.2.4		Gy reference point

The Gy reference point resides between the OCS and the PCEF.

The Gy reference point allows online credit control for service data flow based charging. The functionalities required across the Gy reference point use existing functionalities and mechanisms, based on RFC 4006 [4].

### 5.2.5	Gz reference point

The Gz reference point resides between the PCEF and the OFCS.

The Gz reference point enables transport of service data flow based offline charging information.

The Gz interface is specified in TS 32.240 [3].

# 6	Functional description

## 6.1	Overall description

### 6.1.0	General

The PCC architecture works on a service data flow level. The PCC architecture provides the functions for policy and charging control as well as event reporting for service data flows.

### 6.1.1	Binding mechanism

The binding mechanism is the procedure that associates a service data flow (defined in a PCC rule by means of the SDF template), to the IP-CAN bearer deemed to transport the service data flow. Thus, the binding mechanism shall associate the AF session information with the IP-CAN bearer that is intended to carry the service data flow.

Note: 	The relation between AF sessions and PCC rules depends only on the operator configuration. An AF session can be covered by one or more PCC rules (e.g. one PCC rule per media component of an IMS session). Alternatively, a PCC rule could comprise multiple AF sessions.

The binding mechanism creates bindings. The algorithm, employed by the binding mechanism, may contain elements specific for the kind of IP-CAN.

The binding mechanism includes three steps:

1.	Session binding, i.e. the association of the AF session information and applicable PCC rules to an IP-CAN session. 

The PCRF shall perform the session binding, which shall take the following IP-CAN parameters into account:

a)	The UE IP address;

b)	The UE identity (of the same kind), if present.

NOTE 1:	In case the UE identity in the IP-CAN and the application level identity for the user are of different kinds, the PCRF needs to maintain, or have access to, the mapping between the identities. Such mapping is not subject to specification within this TS.

c)	The information about the packet data network (PDN) the user is accessing.

- PCC Rule authorization, i.e. the selection of an QoS class identifier for the PCC rule. 

The PCRF shall perform the PCC rule authorization for the dynamic PCC rules that have been selected in step 1, taking into account the IP-CAN specific restrictions and other information available to the PCRF. Each PCC rule receives a QoS class that can be supported by the IP-CAN.

- Bearer binding, i.e. the association of the PCC rule to an IP-CAN bearer within that IP-CAN session.

The PCEF performs the bearer binding, unless specified differently in Annex A (e.g. for GPRS running UE only IP-CAN bearer establishment mode).

NOTE 2:	For an IP-CAN, limited to a single IP-CAN bearer per IP-CAN session, the bearer is implicit, so finding the IP-CAN session is sufficient for successful binding.

For an IP-CAN which allows for multiple IP-CAN bearers for each IP-CAN session, the binding mechanism shall use  the following parameters to create the bearer binding for a service data flow:

a)	The session binding result;

b)	The QoS class of the IP-CAN bearer, if available;

c)	The traffic mapping information, if available.

The bearer binding mechanism works in the following way:

-	If the PCEF performs the bearer binding, then the QoS class assigned in step 2 above to the service data flow is the main input for this mapping. The PCEF shall evaluate whether it is possible to use one of the existing bearers or not. If none of the existing bearers are possible to use, the PCEF should initiate the establishment of a suitable bearer. The binding is created between service data flow(s) and the IP-CAN bearer which have the same QoS class.

-

If the PCRF performs the bearer binding, then the binding mechanism shall associate the PCC rule with the IP-CAN bearer that is intended to carry the service data flow, as indicated by the traffic mapping information synchronized between the PCEF and UE. The PCRF shall compare the available traffic mapping information of all IP-CAN bearers, for the same IP-CAN session, with the existing service data flow filter information. Each part of the traffic mapping information shall be evaluated separately in the order of their related precedence. Any matching service data flow filter creates the binding of its corresponding service data flow with the IP-CAN bearer to which the traffic mapping information belongs.

Since a PCC rule can contain multiple service data flow filters it shall be ensured by the PCRF that a service data flow is only bound to a single IP-CAN bearer, i.e. the same PCC rule may not be established on multiple IP-CAN bearers.

NOTE 3: 	For example, a PCC rule containing multiple service data flow filters that match traffic mapping information of more than one IP-CAN bearer could be segmented by the PCRF according to the different matching traffic mapping information. Afterwards, the PCRF can bind the generated PCC rules individually.

Requirements, specific for each type of IP-CAN, are defined in Annex A.

For an IP-CAN, where the PCEF gains no information on what IP-CAN bearer the UE selects to send an uplink IP flow, the binding mechanism shall assume that, for bi-directional service data flows, both downlink and uplink packets travel on the same IP-CAN bearer.

PCC shall re-evaluate existing bindings, i.e. perform the binding mechanism, whenever the service data flow template, the QoS authorization or the negotiated traffic mapping information changes. The re-evaluation may, for a service data flow, require a new binding with another IP-CAN bearer.

### 6.1.2	Reporting

Reporting refers to the differentiated IP-CAN bearer usage information (measured at the PCEF) being reported to the online or offline charging functions.

NOTE 1:	Reporting usage information to the online charging function is distinct from credit management. Hence multiple PCC rules may share the same charging key for which one credit is assigned whereas reporting may be at higher granularity if serviced identifier level reporting is used.

The PCEF shall report usage information for online and offline charging.

The PCEF shall report usage information for each charging key value.

The PCEF shall report usage information for each charging key/service identifier combination if service identifier level reporting is requested in the PCC rule.

NOTE 2:	For reporting purposes a) the charging key value identifies a service data flow if the charging key value is unique for that particular service data flow and b) if the service identifier level reporting is present then the service identifier value of the PCC rule together with the charging key identify the service data flow.

Charging information shall be reported based on the result from the service data flow detection and measurement on a per IP-CAN bearer basis.

A report may contain multiple containers, each container associated with a charging key or charging key/service identifier.

### 6.1.3	Credit management

The credit management applies for online charging only and shall operate on a per charging key basis. The PCEF shall support credit management on a per IP-CAN bearer basis.

NOTE 1:	Independent credit control for an individual service data flow may be achieved by assigning a unique charging key value for the service data flow in the PCC rule.

The PCEF shall request a credit for each charging key occurring in a PCC rule that is active for a IP-CAN bearer. The OCS may either grant or deny the request for credit. The OCS shall strictly control the rating decisions.

NOTE 2:	The term 'credit' as used here does not imply actual monetary credit, but an abstract measure of resources available to the user. The relationship between this abstract measure, actual money, and actual network resources or data transfer, is controlled by the OCS.

It shall be possible for the OCS to form a credit pool for multiple (one or more) charging keys, applied at the PCEF, e.g. with the objective of avoiding credit fragmentation. Multiple pools of credit shall be allowed per IP-CAN bearer. The OCS shall control the credit pooling decisions. The OCS shall, when credit authorization is sought, either grant a new pool of credit, together with a new credit limit, or give a reference to a pool of credit that is already granted for that IP-CAN bearer. The grouping of charging keys into pools shall not restrict the ability of the OCS to do credit authorisation and provide termination action individually for each charging key of the pool. It shall be possible for the OCS to group service data flows charged at different rates or in different units (e.g. time/volume/event) into the same pool.

For each charging key, the PCEF may receive credit re-authorisation trigger information from the OCS, which shall cause the PCEF to perform a credit re-authorisation when the event occurs. The credit re-authorisation trigger detection shall cause the PCEF to request re-authorisation of the credit in the OCS. It shall be possible for the OCS to instruct the PCEF to seek re-authorisation of credit in case of the events listed in table 6.1.

- Table 6.1: Credit re-authorization triggers

| Credit re-authorization trigger | Description |
| --- | --- |
| Credit authorisation lifetime expiry | The OCS has limited the validity of the credit to expire at a certain time. |
| Idle timeout | The service data flow has been empty for a certain time. |
| PLMN change | The UE has moved to another operators' domain. |
| QoS changes | The QoS of the IP-CAN bearer has changed. |
| NOTE: This list is not exhaustive. Events specific for each IP-CAN are specified in clause A, and the protocol description may support additional events. |  |

Some of the re-authorization triggers are related to IP-CAN bearer modifications. IP-CAN bearer modifications, which do not match any credit re-authorization trigger (received from the OCS for the bearer) shall not cause any credit re-authorization interaction with the OCS.

### 6.1.4	Event Triggers

The PCEF shall receive information from the PCRF that define the conditions when the PCEF shall interact again with PCRF after an IP-CAN bearer establishment.

The event triggers are provided by the PCRF to the PCEF using the Provision of PCC Rules procedure. Event triggers are associated with all PCC rules of an IP-CAN session. Event triggers determine when the PCEF shall signal to the PCRF that a IP-CAN bearer has been modified. It shall be possible for the PCRF to instruct the PCEF to react on the event triggers listed in table 6.2.

- Table 6.2: Event triggers

| Event trigger | Description |
| --- | --- |
| PLMN change | The UE has moved to another operators' domain. |
| QoS change (all or exceeding authorization only) | The QoS of the IP-CAN bearer has changed. Two settings shall be possible: all changes of the QoS or only those that exceed the authorized QoS. |
| Traffic mapping information change | The traffic mapping information of the IP-CAN bearer has changed. |
| Change in type of IP-CAN (see note 1) | The access type of the IP-CAN bearer has changed. |
| NOTE 1: This list is not exhaustive. Events specific for each IP-CAN are specified in clause A. NOTE 2: A change in the type of IP-CAN may also result in a change in the PLMN. |  |

IP-CAN bearer modifications, which do not match any event trigger shall cause no interaction with the PCRF.

The QoS change event trigger shall allow two different settings to trigger the PCRF interaction for all changes of the IP-CAN bearer QoS or only for those changes that exceed the QoS of the IP-CAN bearer that has been authorized by the PCRF previously. The QoS parameters of the IP-CAN bearer that have to be checked by the PCEF against a change shall only comprise the bandwidth and the QoS class.

### 6.1.5	Policy Control

Policy control comprises functionalities for:

-	Gating control, i.e. the blocking or allowing of packets, belonging to a service data flow, to pass through to the desired endpoint;

- 	Event reporting, i.e. the notification of and reaction to application events to trigger new behaviour in the user plane as well as the reporting of events related to the resources in the GW(PCEF);

-	QoS control, i.e. the authorisation and enforcement of the maximum QoS that is authorised for a service data flow or an IP-CAN bearer.

-	IP-CAN bearer establishment for IP-CANs that support network initiated procedures for IP-CAN bearer establishment.

In case of an aggregation of multiple service data flows (e.g. for GPRS a PDP context), the combination of the authorised QoS information of the individual service data flows is provided as the authorised QoS for this aggregate.

The enforcement of the authorized QoS of the IP-CAN bearer may lead to a downgrading of the requested bearer QoS by the GW(PCEF) as part of a UE-initiated IP-CAN bearer establishment or modification. Alternatively, the enforcement of the authorised QoS may, depending on operator policy and network capabilities, lead to network initiated IP-CAN bearer establishment or modification. If the PCRF provides authorized QoS for both, the IP-CAN bearer and PCC rule(s), the enforcement of authorized QoS of the individual PCC rules shall take place first.

QoS authorization information may be dynamically provisioned by the PCRF or it can be a pre-defined PCC rule in the PCEF. In case the PCRF provides PCC rules dynamically, authorised QoS information for the IP-CAN bearer (combined QoS) may be provided. For a predefined PCC rules within the PCEF the authorized QoS information shall take affect when the PCC rule is activated. The PCEF shall combine the different sets of authorized QoS information, i.e. the information received from the PCRF and the information corresponding to the predefined PCC rules.

For policy control, the AF interacts with the PCRF and the PCRF interacts with the PCEF as instructed by the AF. For certain events related to policy control, the AF shall be able to give instructions to the PCRF to act on its own, i.e. based on the service information currently available. The following events are subject to instructions from the AF:

-	The authorization of the IP-CAN session modification;

-	The gate control (i.e. whether there is a common gate handling per AF session or an individual gate handling per AF session component required);

-	The forwarding of IP-CAN bearer level events.

Editor's note:	It is FFS how to control whether a service may start on any bearer that could transfer the traffic or whether a bearer dedicated for this traffic is required.

## 6.2	Functional entities

### 6.2.1	Policy Control and Charging Rules Function (PCRF)

The PCRF encompasses policy control decision and flow based charging control functionalities.

Editor's note:	This functional entity encompass the harmonization of the PDF and CRF release 6 logical entities.

The PCRF provides network control regarding the service data flow detection, gating, QoS and flow based charging (except credit management) towards the PCEF.

The PCRF shall apply the security procedures, as required by the operator, before accepting service information from the AF.

The PCRF shall decide how a certain service data flow shall be treated in the PCEF, and ensure that the PCEF user plane traffic mapping and treatment is in accordance with the user's subscription profile.

Editor's note:	For GPRS, it shall be possible to support policy control on a per PDP context basis.

The PCRF may check that the service information provided by the AF is consistent with both the operator defined policy rules and the related subscription information as received from the SPR during IP-CAN session establishment before storing the service information. The service information shall be used to derive the QoS for the service. The PCRF may reject the request received from the AF when the service information is not consistent with either the related subscription information or the operator defined policy rules and as a result the PCRF shall indicate that this service information is not covered by the subscription information or may indicate, in the response to the AF, the service information that can be accepted by the PCRF. In the absence of other policy control mechanisms outside the scope of PCC, it is recommended that the PCRF include this information in the response.

Editor's Note:	For Go it was defined that the controller provides the authorized QoS to the PEP. In PCC it is FFS what the gain and benefits would be to change this concept such that the PCRF receives the requested QoS. Then the PCRF checks it against the authorized QoS and hence may downgrade the requested QoS from the PCEF when it exceeds the authorized QoS.

The PCRF authorizes QoS resources. The PCRF uses the service information received from the AF (e.g. SDP information or other available application information) and/or the subscription information received from the SPR to calculate the proper QoS authorization (QoS class identifier, bitrate). The PCRF may also take account the requested QoS received from the PCEF via Gx interface.

The PCRF may use the subscription information as basis for the policy and charging control decisions. The subscription information may apply for both session based and non-session based services.

Editors' note:	The subscription specific information for each service may contain e.g. max QoS class and max bit rate for each APN the subscriber has access permission to and a corresponding charging key.

#### 6.2.1.1	Input for PCC decisions

The PCRF shall accept input for PCC decision-making from the PCEF, SPR and if the AF is involved, from the AF, as well as the PCRF may use its own pre-defined information. These different nodes should provide as much information as possible to the PCRF. At the same time, the information below describes examples of the information provided. Depending on the particular scenario all the information may not be available or is already provided to the PCRF.

The PCEF may provide the following information:

-	Subscriber Identifier;

-	IP address of the UE;

-	IP-CAN bearer attributes;

-	Request type (initial, modification, etc.);

-	Type of IP-CAN;

-	Location of the subscriber;

-	A PDN identifier;

-	IP-CAN bearer establishment mode.

NOTE:	Depending on the type of IP-CAN, the limited update rate for the location information at the PCEF may lead to a UE moving outside the area indicated in the detailed location information without notifying the PCEF.

The SPR may provide the following information for a subscriber, connecting to a specific PDN:

-	Subscriber's allowed services, i.e. list of Service IDs;

-	Information on subscriber's allowed QoS;

-	Subscriber's charging related information;

-	Subscriber category.

The AF, if involved, may provide the following application session related information, e.g. based on SIP and SDP:

-	Subscriber Identifier;

-	IP address of the UE;

-	Media Type;

-	Media Format, e.g. media format sub-field of the media announcement and all other parameter information (a= lines) associated with the media format;

-	Bandwidth;

-	Flow description, e.g. source and destination IP address and port numbers and the protocol;

-	AF Application Identifier;

-	AF Communication Service Identifier (e.g. IMS Communication Service Identifier), UE provided via AF;

-	AF Application Event Identifier;

-	AF Record Information;

-	Flow status (for gating decision);

-	Priority indicator, which may be used by the PCRF to guarantee service for an application session of a higher relative priority.

Editor's note:	Use of a priority indicator may result in conflicts that the PCRF or other PCC function may need to resolve.

In addition, the pre-defined information in the PCRF may contain additional rules based on charging policies in the network, whether the subscriber is in its home network or roaming, depending on the IP-CAN bearer attributes.

The QoS Class Identifier (see section 6.3.1) in the PCC rule is derived by the PCRF from AF or SPR interaction if available. The input can be SDP information or other available application information, in line with operator policy.

#### 6.2.1.2	Subscription information management in the PCRF

The PCRF may request subscription information from the SPR for an IP-CAN session at establishment. The PCRF should specify the subscriber ID and the PDN identifier in the request. The PCRF should retain the subscription information that is relevant for PCC decisions until the IP-CAN session termination.

The PCRF may request notifications from the SPR on changes in the subscription information. Upon reception of a notification, the PCRF shall make the PCC decisions necessary to accommodate the change in the subscription and updates the PCEF by providing the new PCC decisions if needed.

### 6.2.2	Policy and Charging Enforcement Function (PCEF)

#### 6.2.2.1	General

The PCEF encompasses service data flow detection, policy enforcement and flow based charging functionalities.

Editor's note:	This functional entity encompass the harmonization of the PEP and TPF release 6 logical entities.

This functional entity is located at the Gateway (e.g. GGSN in the GPRS case, and PDG in the WLAN case). It provides service data flow detection, user plane traffic handling, triggering control plane session management (where the IP-CAN permits), QoS handling, and service data flow measurement as well as online and offline charging interactions.

A PCEF shall ensure that an IP packet, which is discarded at the PCEF as a result from policy enforcement or flow based charging, is neither reported for offline charging nor cause credit consumption for online charging.

NOTE:	For certain cases e.g. suspected fraud an operator shall be able to block the service data flow but still be able to account for any packets associated with any blocked service data flow.

The PCEF is enforcing the Policy Control as indicated by the PCRF in two different ways:

-	Gate enforcement. The PCEF shall allow a service data flow, which is subject to policy control, to pass through the PCEF if and only if the corresponding gate is open;

-	QoS enforcement:

-	PCC rule QoS enforcement. The PCEF shall enforce the authorized QoS of a service data flow according to the active PCC rule (e.g. to enforce uplink DSCP marking).

-	IP-CAN bearer QoS enforcement. The PCEF controls the QoS that is provided to a combined set of service data flows. The policy enforcement function ensures that the resources which can be used by an authorized set of service data flows are within the "authorized resources" specified via the Gx interface by "authorized QoS". The authorized QoS provides an upper bound on the resources that can be reserved (GBR) or allocated (MBR) for the IP-CAN bearer. The authorized QoS information is mapped by the PCEF to IP-CAN specific QoS attributes.

The PCEF is enforcing the charging control in the following way:

-	For a service data flow (defined by an active PCC rule) that is subject to charging control, the PCEF shall allow the service data flow to pass through the PCEF if and only if there is a corresponding active PCC rule with and, for online charging, the OCS has authorized credit for the charging key. The PCEF may let a service data flow pass through the PCEF during the course of the credit re-authorization procedure.

For a service data flow (defined by an active PCC rule) that is subject to both Policy Control and Charging Control, the PCEF shall allow the service data flow to pass through the PCEF if and only if the right conditions from both policy control and charging control happen. I.e. the corresponding gate is open and in case of online charging the OCS has authorized credit for its charging key.

For a service data flow (defined by an active PCC rule) that is subject to policy control only and not charging control, the PCEF shall allow the service data flow to pass through the PCEF if and only if the conditions for policy control are met.

A PCEF may be served by one or more PCRF nodes. The PCEF shall contact the appropriate PCRF based on the packet data network (PDN) connected to, together with, a UE identity information (if available, and which may be IP-CAN specific). It shall be possible to ensure that the same PCRF is contacted for a specific UE irrespective of the IP-CAN used.

The PCEF shall, on request from the PCRF, modify a PCC rule, using the equivalent PCEF behaviour as the removal of the old and the installation of the new (modified) PCC rule. The PCEF shall modify a PCC rule as an atomic operation. The PCEF shall not modify a predefined PCC rule on request from the PCRF.

Editor´s note:	It is FFS whether PCC rules need to be installed and activated as separate procedures.

The PCEF should support predefined PCC rules.

For online charging, the PCEF shall manage credit as defined in clause 6.1.3.

The operator may apply different PCC rules depending on different PLMN. The PCEF shall be able to provide identifier of serving network to the PCRF, which may be used by the PCRF in order to select the PCC rule to be applied.

The operator may configure whether Policy and Charging Control is to be applied based on different access point.

The PCEF shall gather and report IP-CAN bearer usage information according to clause 6.1.2.

At IP-CAN session establishment the PCEF shall initiate the IP-CAN Session Establishment procedure, as defined in clause 7.2. If no PCC rule was activated for the IP-CAN session the PCEF shall reject the IP-CAN session establishment.

If there is no PCC rule active for a successfully established IP-CAN session at any later point in time, e.g., through a PCRF initiated IP-CAN session modification, the PCEF shall initiate an IP-CAN session termination procedure, as defined in clause 7.3.2.

If there is no PCC rule active for a successfully established IP-CAN bearer at any later point in time, e.g., through a PCRF initiated IP-CAN session modification, the PCEF shall initiate an IP-CAN bearer termination procedure, as defined in clause 7.4.1.

If the IP-CAN session is modified, e.g. by changing the characteristics for an IP-CAN bearer, the PCEF shall first use the event trigger to determine whether to request the PCC rules for the modified IP-CAN session from the PCRF; afterwards, the PCEF shall use the re-authorisation triggers, if available, in order to determine whether to require re-authorisation for the PCC rules that were either unaffected or modified. If the PCEF receives an unsolicited update of the PCC rules from the PCRF (IP-CAN session modification, clause 7.4.2), the PCC rules shall be installed, modified or removed as indicated by the PCRF.

If network initiated procedures for IP-CAN bearer establishment apply this also includes provisioning the UE with uplink traffic mapping information. See Annex A for details.

If another IP-CAN session is established by the same user, this is treated independently from the existing IP-CAN session.

#### 6.2.2.2	Service data flow detection

This section refers to the detection process that identifies the packets belonging to a service data flow:

-	Each PCC rule contains a service data flow template, which defines the data for the service data flow detection;

-	Each service data flow template may contain any number of service data flow filters;

-	Service data flow filters are unidirectional, so that the detection is applied independently for the downlink and uplink directions.

NOTE 1:	A service data flow template may include service data flow filters for one direction, or for both directions.

Figure 6.3: Relationship of service data flow, packet flow, service data flow template and service data flow filter

Service data flow filters identifying the service data flow may:

-	be a pattern for matching the IP 5 tuple (source IP address, destination IP address, source port number, destination port number, protocol ID of the protocol above IP). In the pattern:

-	a value left unspecified in a filter matches any value of the corresponding information in a packet;

-	an IP address may be combined with a prefix mask;

-	port numbers may be specified as port ranges.

-	extend the packet inspection beyond the IP 5 tuple and look further into the packet and/or define other operations (e.g. maintaining state). Such service data flow filters must be predefined in the PCEF.

NOTE 2:	Such filters may be used to support filtering with respect to a service data flow based on the transport and application protocols used above IP. This shall be possible for HTTP and WAP. This includes the ability to differentiate between TCP, Wireless-TCP according to WAP 2.0, WDP, etc, in addition to differentiation at the application level. Filtering for further application protocols and services may also be supported.

For downlink traffic, the downlink parts of all the service data flow templates associated with the IP-CAN session for the destination address are candidates for matching in the detection process.

Figure 6.4: The service data flow template role in detecting the downlink part of a service data flow and mapping to IP-CAN bearers

For uplink traffic, the uplink parts of all the service data flow templates associated with the IP-CAN bearer (details according to clause A), are candidates for matching in the detection process.

- Figure 6.5: The service data flow template role in detecting the uplink part of a service data flow

The PCEF shall discard a packet in case there is no service data flow filter of the same direction (i.e. of the IP-CAN session for the downlink or of the IP-CAN bearer for the uplink) detecting the packet.

NOTE 3:	To avoid the PCEF discarding packets due to no matching service data flow template, the operator may apply open PCC rules (with wild-carded service data flow filters) to allow for the passage of packets that do not match any other candidate service data flow template.

Service data flow filters shall be applied in the order of their precedence.

#### 6.2.2.3	Measurement

The PCEF shall support data volume, duration, combined volume/duration and event based measurement. The Measurement method indicates what measurement type is applicable for the PCC rule.

Note: 	Event based charging is only applicable to pre-defined PCC rules

The PCC measurement measures all the user plane traffic, except traffic that PCC causes to be discarded.

The PCEF shall maintain a measurement per IP-CAN bearer (IP-CAN specific details according to Annex A), and Charging Key combination.

If Service identifier level reporting is mandated in a PCC rule, the PCEF shall maintain a measurement for that Charging Key and Service Identifier combination, for the IP-CAN bearer (IP-CAN specific details according to Annex A).

Editor's note:	It is for further study if the PCEF shall maintain measurements on the IP-CAN session level.

### 6.2.3	Application Function (AF)

The Application Function (AF) is an element offering applications that require dynamic policy and/or charging control over the IP-CAN user plane behaviour. The AF shall communicate with the PCRF to transfer dynamic session information, required for PCRF decisions as well as to receive IP-CAN specific information and notifications about IP-CAN bearer level events. One example of an AF is the P-CSCF of the IM CN subsystem.

The AF may receive an indication that the service information is not accepted by the PCRF together with service information that the PCRF would accept. In that case, the AF rejects the service establishment towards the UE. If possible the AF forwards the service information to the UE that the PCRF would accept.

An AF may communicate with multiple PCRFs. The AF shall contact the appropriate PCRF based on either:

-	the end user IP Address; and/or

-	a UE identity that the AF is aware of.

NOTE:	By using the end user IP address, an AF is not required to acquire any UE identity in order to provide information, for a specific user, to the PCRF.

For certain events related to policy control, the AF shall be able to give instructions to the PCRF to act on its own, i.e. based on the service information currently available as described in clause 6.1.5.

### 6.2.4	Subscription Profile Repository (SPR)

Editor's note:	The SPRs relation to existing subscriber databases need to be considered, specifically HSS, AAA and AF need to be considered.

The SPR logical entity contains all subscriber/subscription related information needed for subscription-based policies and IP-CAN bearer level PCC rules by the PCRF. The SPR may be combined with or distributed across other databases in the operator's network, but those functional elements and their requirements for the SPR are out of scope of this document.

The SPR may provide the following subscription profile information (per PDN, which is identified by the PDN identifier):

-	Subscriber's allowed services;

-	Information on subscriber's allowed QoS;

-	Subscriber's charging related information (e.g. location information relevant for charging);

-	Subscriber category.

### 6.2.5	Service Data Flow Based Credit Control Function

The Service Data Flow Based Credit Control Function performs online credit control functions. It is a functional entity within the Online Charging System.

The Online Charging System is specified in TS 32.240 [3].

The OCS may trigger the PCEF to initiate a IP-CAN bearer service termination at any point in time.

NOTE:	As the OCS performs the credit control per charging key basis (and thus has not necessarily the knowledge about the existence of any specific service data flow), it is recommended to use different charging keys for any service data flows that shall not be unintentionally interrupted.

There may be several OCSs in a PLMN. The default OCS addresses (i.e. the primary address and secondary address) shall be locally pre-configured within the PCEF. OCS addresses may also be passed once per IP-CAN session from the PCRF to the PCEF. The OCS addresses provided by the PCRF shall have a higher priority than the pre-configured ones.

### 6.2.6	Offline Charging System (OFCS)

The Offline Charging System is specified in TS 32.240 [3].

There may be several OFCSs in a PLMN. The default OFCS addresses (i.e. the primary address and secondary address) shall be locally pre-configured within the PCEF. OFCS addresses may also be passed once per IP-CAN session from the PCRF to the PCEF. The addresses provided by the PCRF shall have a higher priority than the pre-configured ones.

## 6.3	Policy and charging control rule

### 6.3.1	General

Editor's note-i:	To be filled with relevant information from section 5.2 of TS 23.125.

The Policy and charging control rule (PCC rule) comprises the information that is required to enable the user plane detection of, the policy control and proper charging for a service data flow. The packets detected by applying the service data flow template of a PCC rule are designated a service data flow.

Two different types of PCC rules exist: Dynamic rules and predefined rules. The dynamic PCC rules are provisioned by the PCRF via the Gx reference point, while the predefined PCC rules are directly provisioned into the PCEF and only referenced by the PCRF.

NOTE:	The procedure for provisioning predefined PCC rules is out of scope for this TS.

Note:	There may be another type of predefined rules that are not explicitly known in the PCRF and not under the control of the PCRF. The operator may define such predefined PCC rules, to be activated by the PCEF on one IP-CAN bearer within the IP-CAN session.

There are defined procedures for activation, modification and deactivation of PCC rules (as described in clause 6.3.2). The PCRF may activate, modify and deactivate a PCC rule at any time, over the Gx reference point. However, the modification procedure is applicable to dynamic PCC rules only.

The operator defines the PCC rules.

Table 6.3 lists the information contained in a PCC rule, including the information name, the description and whether the PCRF may modify this information in a dynamic PCC rule which is active in the PCEF. The Category field indicates if a certain piece of information is mandatory or not for the construction of a PCC rule, i.e. if it is possible to construct a PCC rule without it.

- Table 6.3 The PCC rule information

| Information name | Description | Category | PCRF permitted to modify for a dynamic PCC rule in the PCEF |
| --- | --- | --- | --- |
| Rule identifier | Uniquely identifies the PCC rule, within an IP-CAN session. It is used between PCRF and PCEF for referencing PCC rules. | Mandatory | no |
| Service data flow detection | This section defines the method for detecting packets belonging to a service data flow. |  |  |
| Precedence | Determines the order, in which the service data flow templates are applied at service data flow detection. | Mandatory | yes |
| Service data flow template | A list of service data flow filters for the detection of the service data flow. | Mandatory | yes |
| Charging | This section defines identities and instructions for charging and accounting that is required for an access point where flow based charging is configured |  |  |
| Charging key | The charging system (OCS or OFCS) uses the charging key to determine the tariff to apply for the service data flow. |  | yes |
| Service identifier | The identity of the service or service component the service data flow in a rule relates to. |  | yes |
| Charging method | Indicates the required charging method for the PCC rule. Values: online, offline or neither. | Mandatory | no |
| Measurement method | Indicates whether the service data flow data volume, duration, combined volume/duration or event shall be measured. This is applicable for reporting, if the charging method is online or offline. Note: Event based charging is only applicable to pre-defined PCC rules. |  | yes |
| Application Function Record Information | An identifier, provided from the AF, correlating the measurement for the Charging key/Service identifier values in this PCC rule with application level reports. |  | no |
| Service identifier level reporting | Indicates that separate usage reports shall be generated for this Service identifier. Values: mandated or not required |  | Yes |
| Policy control | This section defines how the PCEF shall appy policy control for the service data flow. |  |  |
| Gate status | The gate status indicates whether the service data flow, detected by the service data flow template, may pass (Gate is open) or shall be discarded (Gate is closed) at the PCEF. |  | Yes |
| QoS class identifier | The authorized QoS class for the service data flow |  | Yes |
| UL-bitrate | The uplink bit-rate authorized for the service data flow |  | Yes |
| DL-bitrate | The downlink bit-rate authorized for the service data flow |  | Yes |

The PCC Rule identifier shall be unique for a PCC rule within an IP-CAN session. A dynamically provided PCC rule that has the same Rule identifier value as a predefined PCC rule shall replace the predefined rule within the same IP-CAN session.

The PCC Service data flow template may comprise any number of Service data flow filters. A Service data flow filter contains information for matching user plane packets. A Service data flow filter, provided from the PCRF, contains information elements for matching against the IP 5-tuple. The Service data flow template filtering information within an activated PCC rule is applied at the PCEF to identify the packets belonging to a particular service data flow.

NOTE 2:	Predefined PCC rules may include service data flow filters, which support extended capabilities, including enhanced capabilities to identify events associated with application protocols.

The PCC Precedence defines in what order the activated PCC rules within the same IP-CAN session shall be applied at the PCEF for service data flow detection. When a dynamic PCC rule and a predefined PCC rule have the same precedence, the dynamic PCC rule takes precedence.

NOTE 3:	The operator shall ensure that overlap between the predefined PCC rules can be resolved based on precedence of each predefined PCC rule in the PCEF. The PCRF shall ensure that overlap between the dynamically allocated PCC rules can be resolved based on precedence of each dynamically allocated PCC rule.

For downlink packets all the service data flow templates, activated for the IP-CAN session shall be applied for service data flow detection and for the mapping to the correct IP-CAN bearer. For uplink packets the service data flow templates activated on their IP-CAN bearer shall be applied for service data flow detection.

The PCC Charging key is the reference to the tariff for the service data flow. Any number of PCC Rules may share the same charging key value. The charging key values for each service shall be operator configurable.

Note:	Assigning the same Charging key for several service data flows implies that the charging does not require the credit management to be handled separately.

The PCC Service identifier identifies the  service. PCC Rules may share the same service identifier value. The service identifier provides the most detailed identification, specified for flow based charging, of a service data flow.

Editor's note:	The mapping/relation of PCC service identifier and service identifiers used on the AF level (e.g. IMS communication service identifier) is FFS.

The PCC Charging method indicates whether online charging, offline charging, or both are required or the service data flow is not subject to any end user charging. If the PCC charging method identifies that the service data flow is not subject to any end user charging, a PCC Charging key shall not be included in the PCC rule for that service data flow, along with other charging related parameters.

The PCC Measurement method indicates what measurements apply for charging for PCC rule.

The PCC Service Identifier Level Reporting indicates whether the PCEF shall generate reports per Service Identifier. The PCEF shall accumulate the measurements from all PCC rules with the same combination of Charging key/Service identifier values in a single report.

The PCC Application function record information identifies an instance of service usage. A subsequently generated usage report, generated as a result of the PCC rule, may include the Application function record information, if available. The Application Function Record Information may contain the AF Charging Identifier and/or the Flow identifiers. The report is however not restricted to include only usage related to the Application function record information reported, as the report accumulates the usage for all PCC rules with the same combination of Charging key/Service identifier values.  If exclusive charging information related to the Application function record information is required, the PCRF shall provide a service identifier, not used by any other PCC rule of the IP-CAN session at this point in time, for the AF session.

NOTE 4:	For example, the PCRF may be configured to maintain a range of service identifier values for each service which require exclusive per instance charging information. Whenever a separate counting or credit management for an AF session is required, the PCRF shall select a value, which is not used at this point in time, within that range. The uniqueness of the service identifier in the PCEF ensures a separate accounting/credit management while the AF record information identifies the instance of the service.

The PCC Gate indicates whether the PCEF shall let a packet matching the PCC Service data flow template, pass through (gate is open) the PCEF or the PCEF shall discard (gate is closed) the packet.

NOTE 5:	A packet, matching a PCC Rule with an open gate, may be discarded due to credit management reasons.

The QoS Class Identifier indicates the authorized QoS class for the service data flow.

The UL-bitrate indicates the authorized bitrate for the uplink component of the service data flow. The interpretation of the bitrate depends on the QoS class and the IP-CAN.

The DL-bitrate indicates the authorized bitrate for the downlink component of the service data flow. The interpretation of the bitrate depends on the QoS class and the IP-CAN.

### 6.3.2	Policy and charging control rule operations

Policy and charging control rule operations consist of activation, modification and de-activation of PCC rules.

Activation of a dynamic PCC rule provides the PCC rule information to the PCEF via the Gx reference point.

Activation of a predefined PCC rule provides an identifier of the relevant PCC rule to the PCEF via the Gx reference point.

Activation of a predefined PCC rule, not known in the PCRF, may be done by the PCEF based on operator policy.

An active PCC rule means that:

-	the service data flow template shall be used for service data flow detection;

-	the service data flow template shall be used for mapping of downlink packets to an IP-CAN bearer;

-	usage data for the service data flow shall be recorded(further details can be found in clause 6.1.2 Reporting and clause 6.1.3 Credit Management);

-	policies associated with the PCC rule, if any, shall be invoked.

A predefined PCC rule is known at least, within the scope of one access point.

NOTE:	The same predefined PCC rule can be activated for multiple IP-CAN bearers in multiple IP-CAN sessions.

The PCRF may, at any time, modify an active, dynamic PCC rule.

The PCRF may, at any time, deactivate an active PCC rule in the PCEF via the Gx reference point.  At IP-CAN bearer termination all active PCC rules on that bearer are deactivated without explicit instructions from the PCRF to do so.

## 6.4	IP-CAN bearer and IP-CAN session related policy information

The purpose of the IP-CAN bearer  and IP-CAN session related policy information is to provide policy and charging control related information that is applicable to a single IP-CAN bearer or the whole IP-CAN session respectively. The PCRF provides the IP-CAN bearer and IP-CAN session related policy information to the PCEF using the PCC rule provision procedure. The IP-CAN bearer related policy information may be provided together with PCC rules or separately.

Table 6.4 lists the PCC related IP-CAN bearer and IP-CAN session related policy information.

- Table 6.4: PCC related IP-CAN bearer and IP-CAN session related policy information

| Attribute | Description | PCRF permitted to modify the attribute | Scope |
| --- | --- | --- | --- |
| Charging information | Defines the containing OFCS and/or OCS addresses. | No | IP-CAN session |
| Event trigger | Defines the event(s) that shall cause a re-request of PCC rules for the IP-CAN bearer. | Yes | IP-CAN session |
| Authorized QoS | Defines the maximum authorised QoS for the IP-CAN bearer. | Yes | IP-CAN bearer |

Upon the initial interaction with the PCEF, the PCRF may provide Charging information containing OFCS and/or OCS addresses to the PCEF defining the offline and online charging system addresses respectively. These shall override any possible predefined addresses at the PCEF.

Upon every interaction with the PCEF, the PCRF may provide event triggers for the IP-CAN session. Event triggers are used to determine which IP-CAN bearer modification causes the PCEF to re-request PCC rules. The triggers are listed in section 6.1.4.

The authorized QoS provides an upper bound on the resources that can be reserved or allocated for the IP-CAN bearer. The authorized QoS applies for an IP-CAN bearer and may be included to the initial or to subsequent PCC rule provisioning procedures. The authorized QoS is specific for each IP-CAN and is described in Annex A.

# 7	PCC Procedures and flows

## 7.1	Introduction

The specification of the PCC procedures and flows is valid for the general scenario. Access specific information is included in Annex A.

The description includes procedures for IP-CAN Session Establishment, Modification and Termination. The IP-CAN Session modification comprises IP-CAN bearer establishment, modification, termination, as well as unsolicited PCC decisions.

## 7.2	IP-CAN Session Establishment

This clause describes the signalling flow for IP-CAN Session establishment and IP address assignment to the UE. The AF is not involved.

- Figure 7.1: IP-CAN Session Establishment

1.	The GW(PCEF) receives a request for IP-CAN Bearer establishment. The GW(PCEF) accepts the request and assigns an IP address for the user.

2.	The PCEF determines that the PCC authorization is required, requests the authorization of allowed service(s) and PCC Rules information. The PCEF includes the following information; IP-CAN type and, if available, the IP-CAN bearer establishment modes supported.

3.	If the PCRF does not have the subscriber's subscription related information, it sends a request to the SPR in order to receive the information related to the IP-CAN session. The PCRF provides the subscriber ID and, if applicable, the PDN identifier to the SPR.

4.	The PCRF stores the subscription related information containing the information about the allowed service(s) and PCC Rules information.

5.	The PCRF makes the authorization and policy decision.

6.	The PCRF sends the decision(s) , including the chosen IP-CAN bearer establishment mode, to the PCEF. The GW(PCEF) enforces the decision.

7.	If online charging is applicable, and at least one PCC rule was activated, the PCEF shall request credit from the OCS for any charging key of the activated PCC rules, and provide relevant input information for the OCS decision.

8.	If online charging is applicable the OCS provides the credit information to the PCEF and may provide re-authorisation triggers for each of the credits.

9.	If network control applies the GW may initiate the establishment of additional IP-CAN bearers. See Annex A for details.

10.		If at least one PCC rule was successfully activated and when online charging is applicable credit for it was not denied, the GW(PCEF) acknowledges the IP-CAN Bearer Establishment Request.

## 7.3	IP-CAN Session Termination

### 7.3.1	UE initiated IP-CAN Session termination

- Figure 7.2: IP-CAN Session Termination

1.	The GW(PCEF) receives a request to remove the last IP-CAN bearer associated to this IP-CAN session.

2.	The PCEF indicates that the IP-CAN Session is being removed and provides relevant information to the PCRF.

3.	The PCRF finds the PCC Rules that require an AF to be notified.

4.	The PCEF removes all PCC Rules associated with the IP-CAN session.

5.	The PCRF notifies the AF that there are no transmission resources for the service if this is requested by the AF.

6.	The AF acknowledges the notification of the loss of transmission resources.

7.	The PCRF removes the information related to the terminated IP-CAN Session (subscription information etc.), and acknowledges to the PCEF that the PCRF handling of the IP-CAN session has terminated. This message is flagged as the response to the PCEF request.

8.	If online charging is applicable, the PCEF issues final reports and returns the remaining credit to the OCS.

9.	If online charging is applicable the OCS acknowledges that credit report.

10.	The GW(PCEF) continues the IP-CAN Bearer removal procedure.

NOTE:	The IP-CAN Session removal procedure may proceed in parallel with the indication of IP-CAN Session termination.

### 7.3.2	GW(PCEF) initiated IP-CAN Session termination

- Figure 7.3: GW(PCEF) Initiated IP-CAN Session Termination

1.	The GW(PCEF) detects that IP-CAN Session termination is required.

2.	The GW(PCEF) sends a request to remove the IP-CAN bearer. For IP-CAN with multiple IP-CAN bearers this applies for each IP-CAN bearer associated to this IP-CAN session.

3.	The GW(PCEF) receives the response for the IP-CAN bearer removal.

4.	The PCEF indicates the IP-CAN Session termination and provides the relevant information to the PCRF.

5.	The PCRF finds the PCC Rules that require an AF to be notified.

6.	The PCRF notifies the AF that there are no transmission resources for the service if this is requested by the AF.

7.	The AF acknowledges the notification on the loss of transmission resources.

8.	The PCEF removes all the PCC Rules associated with the IP-CAN session.

9.	The PCRF removes the information related to the terminated IP-CAN Session (subscription information etc.), and acknowledges the IP-CAN Session termination.

10.	If online charging is applicable, the GW issues final reports and returns the remaining credit to the OCS.

11.	If online charging is applicable the OCS acknowledges the credit report.

## 7.4	IP-CAN Session Modification

### 7.4.1	IP-CAN Session Modification; GW(PCEF) initiated

This sub-clause describes the signalling flow for the IP-CAN Session modification initiated by the GW(PCEF). These modifications include IP-CAN bearer establishment and termination as well as modification if the triggering conditions given to the PCEF are fulfilled. The AF may be involved. An example of the scenario is authorization of a session-based service for which an IP-CAN Session is also modified.

- Figure 7.4: IP-CAN Session Modification; GW(PCEF) initiated

1.	Optionally, the AF provides service information to the PCRF due to AF session signalling..

2.	The PCRF stores the service information and responds with the Acknowledgement to the AF.

3.	The GW(PCEF) makes an internal decision or receives a request for IP-CAN Bearer establishment, modification or termination.

4.	The PCEF determines that the PCC interaction is required and sends the PCC Rules request to the PCRF. If there is a limitation or termination of the transmission resources for a PCC Rule, the PCEF reports this to the PCRF.

5.	The PCRF correlates the request for PCC Rules with the IP-CAN session and service information available at the PCEF.

6.	The PCRF may need to report to the AF an event related to the transmission resources and/or if the AF requested it at initial authorisation or if the PCRF requires more information from the AF before authorising the network resources modification..

7. 	The AF acknowledges the event report and/or responds with the requested information.

8.	The PCRF makes the authorization and policy decision.

9.	The PCRF sends the decision(s) to the PCEF. The GW(PCEF) enforces the decision.

10.	If online charging is applicable, the PCEF shall request credit from and/or issue final reports and return remaining credit to the OCS.

11. If online charging is applicable, the OCS provides the credit information to the PCEF, and/or acknowledges the credit report.

Editor’s Note: The sequence of resource authorization and OCS interaction is FFS.

12	The GW(PCEF) acknowledges or rejects any IP-CAN bearer signalling received in step 3. The IP-CAN bearer establishment or modification is accepted if at least one PCC rule is active for the IP-CAN bearer and in case of online charging if credit is available for at least one charging key. Otherwise, the IP-CAN bearer establishment or modification is rejected. An IP-CAN bearer termination is always acknowledged by the GW(PCEF).

13.	In case of a GW(PCEF) internal decision the GW(PCEF) initiates any IP-CAN bearer signalling required for completion of the IP-CAN Sesssion modification.

### 7.4.2	IP-CAN Session Modification; PCRF initiated

This clause describes the signalling flow for the IP-CAN Session modification initiated by the PCRF. The AF may be involved. An example of the scenario is initiation and authorization of a session-based service for which an IP-CAN Session is modified. IP-CAN Session handling and handling of PCC rules for non-session based services, and also general handling of PCC rules that are not subject to AF-interaction is also applicable here.

- Figure 7.5: IP-CAN Session Modification; PCRF initiated

1.	Optionally, the AF provides service information to the PCRF due to AF session signalling.

Editor’s Note: Optionally, without AF interaction, a trigger event in the PCRF may cause the PCRF to determine that the PCC rules require updating at the PCEF, e.g. change to configured policy.

2.	The PCRF makes the authorization and policy decision.

3.	The PCRF sends the decision(s) to the PCEF.

4.	The PCEF enforces the decision.

5.	If online charging is applicable, the PCEF shall request credit from and/or returns the remaining credit to the OCS.

6.	If online charging is applicable, the OCS provides the credit information to the PCEF, and/or acknowledges the credit report

Editor’s Note: The sequence of resource authorization and OCS interaction is FFS.

7.	The GW(PCEF) may send an IP-CAN Bearer establishment, modification or termination request. 
An IP-CAN bearer modification is sent by the GW(PCEF) if the QoS of the IP-CAN bearer exceeds the authorized QoS provided by the PCRF in step 3. 
An IP-CAN bearer termination request is sent by the GW(PCEF) if all PCC rules for an IP-CAN bearer have been removed.

8.	The GW(PCEF) receives the response for the IP-CAN Bearer modification or termination request.

9.	The PCEF sends ACK (accept or reject of the PCC decision(s)) to the PCRF.

10.	The PCRF stores the service information and responds with an Acknowledgement to the AF.

## 7.5	Update of the subscription information in the PCRF

- Figure-7.6: Procedure for update of the subscription information in the PCRF

1. The SPR detects that the related subscription profile of an IP-CAN session has been changed.

2. If requested by the PCRF, the SPR notifies the PCRF on the changed profile.

3. The PCRF responds to the SPR.

4. The PCRF stores the updated profile and makes resulting PCC decisions.

5. The PCRF provides all new PCC decisions to the PCEF, using the PCRF initiated IP-CAN session modification procedure in section 7.4.2.

Annex A (normative):
Access specific aspects

# A.1	GPRS

## A.1.0	General

The GPRS IP-CAN employs, for an IP-CAN session, the concept of PDP contexts in order to provide an information transmission path of defined capacity (QoS). For GPRS, the IP-CAN bearer is the PDP context.

- Figure A.1: The GPRS IP-CAN

## A.1.1	High level requirements

### A.1.1.1	General

### A.1.1.2	Charging related requirements

### A.1.1.3	Policy control requirements

IP-CAN Bearer QoS control allows the PCC architecture to control the "Authorised QoS" of a PDP context. Criteria such as the QoS subscription information may be used together with service-based, subscription-based, or a pre-defined PCRF internal policies to derive the “Authorized QoS” of a PDP context.

The concept of QoS class identifier and the associated bitrates specify the "Authorized QoS" on the Gx reference point.

Editor's Note:	Dependencies between PDP context QoS control and service data flow QoS control are FFS.

## A.1.2	Architecture model and reference points

### A.1.2.1	Reference points

#### A.1.2.1.1	Gx reference point

The Gx reference point enables the signalling of PCC rules, which govern the PCC behaviour, and it supports the following GPRS-specific functions:

-	Indication of PDP context activation, modification and deactivation.

## A.1.3	Functional description

### A.1.3.1	Overall description

#### A.1.3.1.1	Binding mechanism

As explained in clause 6.1.1, the binding mechanism is performed in three different steps: session binding, PCC rule authorization and bearer binding.  Session binding has no GPRS specifics. For the GPRS case bearer binding is performed by:

-	PCRF, when the selected operation mode is MS-only, see [12], either due to PCRF decision or network/UE capability.

Editor’s Note: Bearer binding is located in the PCEF in other operation modes. Details are FFS.

Editor’s note: Handling of bearer binding has to be revisited to cater for UE/NW mode (mixed mode).

In order to identify the candidate PDP context the bearer binding shall compare

-	the PCC rule service data flow template with the TFT filters and

-	the PCC rule QoS parameters with the PDP context QoS parameters

The binding mechanism shall comply with the established traffic flow template (TFT) packet filters (for the whole IP-CAN session).

The bearer binding shall bind a PCC rule:

-	to a candidate PDP context with a matching QoS class;

-	to a candidate PDP context with a matching QoS class that, after modification of the bitrate, fulfils the PCC rule QoS demands;

-	to a new PDP context with a matching QoS class, if there is no suitable candidate PDP context present.

The bearer binding mechanism associates the PCC rule with the PDP context to carry the service data flow. The association shall

-	cause the downlink part of the service data flow to be directed to the PDP context in the association, and

-	assume that the UE directs the uplink part of the service data flow to the PDP context in the association.

Thus, the detection of the uplink part of a service data flow shall be active on the PDP context, which the downlink packets of the same service data flow is directed to. The detection of the uplink part of the service data flow may be active, in parallel, on any number of additional PDP contexts.

##### A.1.3.1.1.1	Bearer binding mechanism allocated to the PCEF

Editor’s note:	Section to be detailed upon addition of network controlled QoS. Details are FFS.

##### A.1.3.1.1.2	Bearer binding mechanism allocated to the PCRF

If a new PDP context is required in order to successfully perform the bearer binding the PCRF will set the PCC rule as binding-pending status until the PCEF reports the establishment of a PDP context that fulfils the PCC rule demands or the PCC rule is removed.

The following particularities apply when the bearer binding mechanism is allocated to the PCRF:

-	The PCEF

-	shall include a bearer reference in all requests for PCC decisions;

-	shall report bearer QoS class identifier and the associated bitrates for new/modified PDP contexts;

-	shall report the TFT filter status for new PDP contexts and for modified TFT:s;

-	shall report the deactivation of a PDP context

-	The PCRF

-	shall provide the bearer reference for the binding result when activating a PCC rule;

-	shall arm the GPRS-specific IP-CAN event trigger "PDP context activity".

NOTE 1:	For the above case, the allocation of the bearer binding mechanism to the PCRF facilitates the migration from Rel-6 products to Rel-7 products. The allocation of the binding mechanism may be re-evaluated in future releases.

#### A.1.3.1.2	Reporting

A container may be closed and a new container opened by the triggering of event triggers.

#### A.1.3.1.3	Credit management

For GPRS the credit re-authorisation triggers in table A.1 shall apply in addition to the ones in table 6.1.

- Table A.1: GPRS specific credit re-authorization triggers

| Credit re-authorization trigger | Description |
| --- | --- |
| SGSN change | The UE has moved to a new SGSN. |
| RAT type change. | The characteristics of the air interface, communicated as the radio access type, has changed. |
| Location change (routeing area) | The routeing area of the UE has changed. |
| Location change (CGI/SAI) | The CGI/SAI of the UE has changed. |

If the Location change trigger for CGI / SAI or RAI is armed, the GGSN should request the SGSN to report any changes in location to the level indicated by the trigger according to the procedures described in 3GPP TS 23.060 [12]. If credit-authorization triggers and event triggers require different levels of reporting of location change for different PDP contexts for a single UE, the SGSN reports location changes to the highest level of detail required. However, the GGSN should not trigger a credit re-authorization if the report received is more detailed than requested by the OCS.

#### A.1.3.1.4	Event Triggers

For GPRS the event triggers in table A.2 shall apply in addition to the ones in table 6.2.

- Table A.2: GPRS specific event triggers

| Event trigger | Description |
| --- | --- |
| SGSN change | The UE has moved to a new SGSN. |
| RAT type change. | The characteristics of the air interface, communicated as the radio access type, has changed. |
| PDP Context Activity | The GGSN has received a request for a PDP context activation, modification or deactivation. Note 1. |
| Location change (routeing area) | The routeing area of the UE has changed. |
| Location change (CGI/SAI) | The CGI/SAI of the UE has changed. |
| Note 1: Available only when the bearer binding mechanism is allocated to the PCRF. |  |

If the Location change trigger is armed, the GGSN should request the SGSN to report any changes in location to the level indicated by the trigger according to the procedures described in 3GPP TS 23.060 [12]. If credit-authorization triggers and event triggers require different levels of reporting of location change for different PDP contexts for a single UE, the SGSN reports location changes to the highest level of detail required. However, the GGSN should not trigger a request for PCC rules if the report received is more detailed than requested by the PCRF.

For GPRS the QoS parameters of the IP-CAN bearer that have to be checked by the PCEF against a change shall only comprise the maximum/guaranteed bitrate and the UMTS traffic class.

For GPRS the traffic mapping information is represented by the TFT information.

### A.1.3.2	Functional entities

#### A.1.3.2.1		Policy Control and Charging Rules Function (PCRF)

##### A.1.3.2.1.1	Input for PCC decisions

The PCRF shall accept any of the following input, specific for GPRS, as a basis for decisions on PCC rule operations:

-	Per IP-CAN session (e.g.: UE IP address):

-	APN;

-	Requested QoS, for a PDP context;

-	RAT-type;

-	Location (CGI/SAI or RAI).

The PCEF may provide the following information:

-	Subscriber Identifier in the form of IMSI, MSISDN;

-	APN;

-	RAT type;

-	Requested QoS, for PDP Context;

-	TFT, to enable the identification of the corresponding PDP Context,

-	Location (CGI/SAI or RAI).

The SPR may provide the following information:

-	Subscribers maximum allowed QoS class for the GPRS access (this shall be mapped to UMTS traffic class in the GGSN);

-	Subscriber's maximum allowed bit rate for uplink and downlink per UMTS QoS class (these shall be mapped in the GGSN to the 'Maximum bitrate' UMTS QoS parameters for the PDP context established with the corresponding UMTS traffic class);

Editor's note:	This needs to be revisited once the IP bearer debate has been resolved.

#### A.1.3.2.2		Policy and Charging Enforcement Function (PCEF)

##### A.1.3.2.2.1	General

This functional entity is located in the GGSN. The GGSN provides the GPRS-specific bearer QoS handling.

If the GGSN receives an Authorization token and Flow Id(s) from an UE, the PCEF shall report them to the PCRF over Gx.

The PCEF shall contact the PCRF based on PCRF address information that shall be configured for the access point name (APN) together with the IMSI or MSISDN (if needed).

The QoS information transferred over Gx is in terms of QoS Class Identifier and bitrates.

The PCEF shall maintain a mapping from the QoS Class Identifier to a UMTS QoS profile and vice versa.

Editor's note:	Details of QoS Class Identifier mapping to UMTS QoS profile is FFS.

For each PDP context, the PCEF shall accept information during bearer establishment and modification relating to:

-	The user and terminal (e.g. MSISDN, IMEISV);

-	Bearer characteristics (e.g. QoS negotiated, APN, IM CN Subsystem signalling flag);

-	Network related information (e.g. MCC and MNC).

The PCEF shall use this information in the OCS request/reporting or request for PCC rules.

A GGSN may provide more than one APN for access to the same PDN. It should be possible to enable or disable PCC functionality for each APN, independent from the other APNs for access to the same PDN. Once the PCC functionality is disabled, regular GPRS charging and policy methods would be applied, i.e. no PCRF interaction would occur.

For each PDP context, there shall be a separate OCS request/OFCS reporting, so this allows the OCS and offline charging system to apply different rating depending on the PDP context.

The GGSN shall report the service data flow based charging data on a per PDP context basis.

The GGSN shall be able to request the SGSN to provide reports of changes in CGI/SAI/RAI of a UE as directed by the credit re-authorization triggers and/or event triggers.

##### A.1.3.2.2.2	Service data flow detection

For uplink traffic, in the case of GPRS, all the uplink parts of service data flows templates, which are associated with the PDP context are candidates for matching in the detection process.

NOTE:	Service data flow templates, which are not associated with the PDP context the packet was received, are not candidates for matching (dashed in the figure).

##### A.1.3.2.2.3	Packet Routeing and Transfer Function

The PCEF performs the packet routeing and transfer functions as specified in TS 23.060 [12], with the differences specified in this clause.

For the PDP address of an UE, the PCEF routes downlink packets to the different PDP contexts based on the downlink parts of the service data flow templates, in the active PCC rules and their routeing associations to the PDP contexts. The association between an active PCC rule and a PDP context shall correspond to the downlink TFT received from the UE. Each active PCC rule shall have a single routeing association to a PDP context. Upon reception of a packet, the PCEF evaluates the downlink part of the service data flow templates of the PCC rules activated for the PDP address in order of precedence to find a match. When the first match is found, the packet is tunnelled to the SGSN via the PDP context, for which the PCC rule has the routeing association. If no match is found, the PCEF shall silently discard the packet.

The UE shall define TFTs that enable successful binding at the PCRF for service data flows requiring a binding to occur.

For each uplink packet, the UE should choose the PDP context that is used for the downlink direction of the same service data flow, as declared in the TFT information. The PCEF shall only apply the uplink parts of the service data flow templates of the PCC rules, which are associated with the same PDP context as the uplink packet arrived on.

Editor's note:	The treatment of uplink only service data flows is FFS.

The packet filters, to be applied on dedicated signalling PDP contexts, shall form PCC rules, which shall be granted higher precedence than any other PCC rule and be active on the dedicated signalling context.

##### A.1.3.2.2.4	Measurement

The details of measurement are specified in TS 32.251 [9].

#### A.1.3.2.3		Application Function (AF)

Editor's note:	An AF providing an accurate service data flow description reduces the risk for creating multiple bindings to occur due to the UE providing traffic flow template filters, which do not cover the complete authorization, for a PDP context.

### A.1.3.3	Policy and charging control rule

#### A.1.3.3.1	General

Editor's note:	For GPRS an operator may optionally define predefined PCC rules that operate on MBMS bearer contexts, see TS 23.246 [6]. Such PCC rules are not applicable to any PDP contexts. Pre-defined charging rules for MBMS Bearer contexts are not available for reference from a PCRF and hence they cannot be dynamically activated over the Gx reference point. For MBMS a GGSN may collect charging data records on a per MBMS bearer context basis. The report may, depending on the configuration of the charging rule, include volume- and/or time-usage for a certain MBMS service. The purpose of the reporting may be to form a basis for settlement with a 3rd party content provider. Since multiple users share an MBMS bearer context it is not possible to derive any per user information for such bearers.

#### A.1.3.3.2		Policy and charging control rule operations

The PCRF associates, at activation, a PCC rule with a PDP context at the PCEF.

### A.1.3.4	IP-CAN bearer and IP-CAN session related policy information

For GPRS, the authorized QoS consists of the QoS class and the bitrate for uplink and downlink. The authorized QoS parameters shall be mapped by the PCEF to the QoS parameters of the PDP context: the QoS class to the UMTS traffic class and the bitrate to the maximum/guaranteed bitrate depending on the QoS class.

## A.1.4	PCC Procedures and flows

### A.1.4.1	Introduction

For GPRS, the GW(PCEF) is the GGSN. The IP-CAN bearer is the PDP context and the IP-CAN Session is established by the Create PDP Context message. The IP-CAN Session is terminated when the last PDP Context of the specific IP address is deleted and the IP Address is released.

### A.1.4.2	IP-CAN Session Establishment

The IP-CAN session establishment procedure (described in clause 7.2) is triggered at the GGSN by receiving a Create PDP Context Request message for the first PDP Context that is created for a new IP Address. The successful procedure results in an establishment of a UE IP Address and a PDP Context for the UE. The Create PDP Context Response message, indicating that a new PDP context is created, is sent to the SGSN.

During the PDP context activation procedure, it shall be possible to forward the network capability of reporting of changes in CGI/SAI/RAI to the PCRF.

### A.1.4.3	IP-CAN Session Termination

#### A.1.4.3.1	UE initiated IP-CAN Session termination

The UE initiated IP-CAN Session termination procedure (described in clause 7.3.1) is triggered at the GGSN by receiving a Delete PDP Context request message if this is the deletion of the last PDP Context for the IP Address or the Teardown Indicator in the Delete PDP Context Request indicates that all PDP contexts that share the same IP address should be deactivated. All PDP Contexts in the IP-CAN Session are deleted in the GGSN. The IP Address of the UE is released. The Delete PDP Context Response message, indicating that the PDP context(s) is deleted, is sent to the SGSN.

#### A.1.4.3.2	GW initiated IP-CAN Session termination

The GW initiated IP-CAN Session termination procedure (described in clause 7.3.2) is triggered if the GGSN detects that the IP-CAN Session shall be terminated. The Delete PDP Context request message is sent by the GGSN to the SGSN.

This may be the deletion of the last PDP Context for the IP Address. If not, the GGSN shall set the Teardown Indicator in the Delete PDP Context Request message to indicate that all PDP contexts that share that same IP address shall also be deactivated. All PDP Contexts in the IP-CAN Session are deleted. The IP Address of the UE is released. The Delete PDP Context Response, indicating that the PDP context(s) is deleted, is received from the SGSN.

### A.1.4.4	IP-CAN Session Modification

#### A.1.4.4.1	IP-CAN Session Modification; GW (PCEF) initiated

The GW(PCEF) initiated IP-CAN Session modification procedure (described in clause 7.4.1) is triggered at the GGSN by receiving one of the following messages:

-	Create PDP Context Request message;

-	Update PDP Context Request message;

-	Delete PDP Context Request message;

-	a Change Notification message (indicating the new CGI, SAI or RAI) – see 3GPP TS 23.060 [12].

In case of a Create PDP Context Request message, the modification of the IP-CAN Session is the addition of a new PDP Context to the IP-CAN Session. The new PDP Context is added with specific QoS requirements and traffic mapping information (TFT). A Create PDP Context Response message, indicating that a new PDP context is created, is sent to the SGSN.

In case of an Update PDP Context Request, a PDP Context in the IP-CAN Session is modified. The modification may include modifying the QoS and/or the traffic mapping information. The Update PDP Context Response message, indicating that a PDP context is modified, is sent to the SGSN.

In case of a Delete PDP Context Request message, a PDP Context in the IP-CAN Session is deleted. The Delete PDP Context Response message, indicating that a PDP context is deleted, is sent to the SGSN.

A Change Notification message indicating a change in CGI / SAI or RAI information is received when there are only changes regarding the current location of the UE. A change in CGI / SAI or RAI may also be notified within other session management messages.

#### A.1.4.4.2	IP-CAN Session Modification; PCRF initiated

The PCRF initiated IP-CAN Session modification procedure (described in clause 7.4.2) may result in a GGSN initiated PDP Context Modification or Deactivation.

In case a PDP Context in the IP-CAN Session needs to be modified, the GGSN sends an Update PDP Context Request message. The modification may include modifying the QoS negotiated or the required CGI/SAI/RAI change reporting. The Update PDP Context Response message, indicating that a PDP context is modified, will be received from the SGSN.

In case a PDP Context in the IP-CAN Session needs to be deleted, the GGSN sends a Delete PDP Context Request message. The Delete PDP Context Response message, will be received from the SGSN.

# A.2	I-WLAN

The I-WLAN IP-CAN employs, for an IP-CAN session, the concept of an I-WLAN IPSec tunnel in order to provide an information transmission path between the WLAN UE and the PDG. For I-WLAN QoS differentiation is realized by marking IP packets with an appropriate DSCP.

Editor's note: The IP-CAN bearer for I-WLAN is FFS.

- Figure A.2.1: The I-WLAN IP-CAN

## A.2.1	High level requirements

### A.2.1.1	General

### A.2.1.2	Charging related requirements

### A.2.1.3	Policy control requirements

## A.2.2	Architecture model and reference points

### A.2.2.1	Reference points

#### A.2.2.1.1	Rx reference point

#### A.2.2.1.2	Gx reference point

#### A.2.2.1.3	Sp reference point

## A.2.3	Functional description

### A.2.3.1	Overall description

#### A.2.3.1.1	Binding mechanism

### A.2.3.2	Functional entities

#### A.2.3.2.1		Policy Control and Charging Rules Function (PCRF)

##### A.2.3.2.1.1	Input for PCC decisions

#### A.2.3.2.2		Policy and Charging Enforcement Function (PCEF)

This functional entity is located in the PDG. The PDG provides the I-WLAN-specific bearer QoS handling.

#### A.2.3.2.3		Application Function (AF)

### A.2.3.3	Policy and charging control rule

#### A.2.3.3.1	General

#### A.2.3.3.2		Policy and charging control rule operations

The PCEF associates, at activation, a PCC rule with a DSCP at the PCEF.

Editor's note:	It is FFS how the PDG maps the QoS Class Identifier in the PCC rule to DSCP for the WLAN IPSec tunnel.

## A.2.4	PCC Procedures and flows

### A.2.4.1	Introduction

For I-WLAN, the GW(PCEF) is the PDG. The IP-CAN Session is established by the WLAN UE-Initiated Tunnel Establishment procedure. The IP-CAN Session is terminated when the WLAN IPSec Tunnel is disconnected.

Editor's note:	The IP-CAN bearer for I-WLAN is FFS.

### A.1.4.2	IP-CAN Session Establishment

### A.1.4.3	IP-CAN Session Termination

#### A.1.4.3.1	UE initiated IP-CAN Session termination

#### A.1.4.3.2	GW(PCEF) initiated IP-CAN Session termination

### A.1.4.4	IP-CAN Session Modification

#### A.1.4.4.1	IP-CAN Session Modification; GW (PCEF) initiated

Editor’s note: It is FFS how GW(PCEF) initiated IP-CAN session modification is applicable to I-WLAN.

#### A.1.4.4.2	IP-CAN Session Modification; PCRF initiated

The PCRF initiated IP-CAN Session modification procedure (described in clause 7.4.2) is triggered by AF interaction or separate operator action.

# A.3	DOCSIS IP-CAN

## A.3.1	General

In the DOCSIS IP-CAN, each UE is connected to the network via a Cable Modem (CM) which is connected through a Hybrid Fibre Coax (HFC) access network to a Cable Modem Termination System (CMTS). Though the UE and CM may or may not be embedded within the same physical package, they remain separate logical devices. One or more UEs may subtend a single CM. Because the CMTS provides the IP connectivity and traffic scheduling and manages quality of service for the CM and the UEs which subtend it, the CMTS fulfils the role of PCEF for the DOCSIS IP-CAN. In the DOCSIS IP-CAN, the Application Manager (AM) and the Policy Server (PS) fulfil the role of the PCRF.

When accessing resources via a DOCSIS IP-CAN, the Rx interface can be used to request resources. The communication between the AM and PS and the PS and CMTS uses the PKT-MM-2 interface which is based on COPS and defined in J.179. The remainder of this section (and its sub-sections) documents the mapping of PCC terminology to the DOCSIS IP-CAN and how the DOCSIS IP-CAN realizes the defined PCC functionality. This section also establishes the requirements of the Rx interface as it is used for the DOCSIS IP-CAN.

The PKT-MM-2 interface is shown here for information to illustrate the organization of the DOCSIS IP-CAN. References that specify the PKT-MM-2 interface do not constitute normative requirements for the 3GPP architecture. The DOCSIS IP-CAN does not intend to pose any new normative requirements for the Gx interface.

- Figure A.3.1: DOCSIS IP-CAN

## A.3.1	High level requirements

### A.3.1.1	General

The DOCSIS IP-CAN employs for an IP-CAN session, the concept of a DOCSIS registration.

The DOCSIS IP-CAN employs for an IP-CAN bearer, the concept of a DOCSIS service flow in order to provide an information path between the UE and the CMTS. Note that DOCSIS service flows are unidirectional, either upstream (toward the CMTS) or downstream (toward the CM). When a CM is registered in the DOCSIS IP-CAN, it is assigned a unique IP Address and separate primary service flows are created for both the upstream and downstream direction These primary service flows are typically given best effort scheduling and are used to carry all IP traffic through the CM for which a more specific service flow has not been created. When a UE is registered in the DOCSIS IP-CAN, it is assigned its own IP Address and is identified by its MAC address. A UE does not have a service flow assigned to it as a result of registration; rather it is associated with the primary service flows of the CM through which it is attached to the network. Additional bearers for the UE are created dynamically as required to provide appropriate QoS for service flows.

Bearer creation is triggered when media descriptors (Media Type and Format) for the SIP session are sent from the AF to the AM over the Rx interface. The AM translates the media descriptors into a QoS request for a DOCSIS service flow.  The AM then forwards the QoS request towards the bearer enforcement point using the PKT-MM-2 interface. The PKT-MM-2 interface is not a 3GPP reference point, Specifications that detail the PKT-MM-2 interface do not impose normative requirements on the 3GPP architecture.

The following figure provides a graphical representation of the DOCSIS IP-CAN and how it maps into the generic PCC terminology.

- Figure A.3.2: PCC to DOCSIS terminology mapping

The DOCSIS IP-CAN defines an IP-Flow to be a unidirectional sequence of packets identified by OSI Layer 3 and Layer 4 header information. This information includes source/destination IP addresses, source/destination port numbers, protocol ID. Multiple Multimedia streams may be carried in a single IP Flow.

In a DOCSIS IP-CAN, there is no equivalent concept as a service data flow. Further a DOCSIS service flow is uni-directional and each service flow is an aggregation of the QoS needs for all the IP-Flows which make up the service flow. As such, the QoS enforcement is done at the service flow level not at the IP-Flow level.

### A.3.1.2	Charging related requirements

### A.3.1.3	Policy control requirements

## A.3.2	Architecture model and reference points

### A.3.2.1	Reference points

#### A.3.2.1.1	Rx reference point

#### A.3.2.1.2	Gx reference point

#### A.3.2.1.3	Sp reference point

Editor's note:	Requirements placed on the Sp interface are FFS.

## A.3.3	Functional description

### A.3.3.1	Overall description

The DOSCIS IP-CAN employs for an IP-CAN bearer, the concept of a DOCSIS service flow in order to provide an information path between the UE and the CMTS. When a Cable Modem is registered in the DOCSIS IP-CAN, primary upstream and downstream service flows are created.

When a UE is registered in the DOCSIS IP-CAN it is associated with the primary service flows of the cable modem through which it is attached to the network. Based on session information provided by the AF using the Rx reference point, the Application Manager will determine QoS requirements for each IP flow. IP flows which do not require special quality of service treatment may be carried over the primary service flows. For other IP flows which require specific QoS treatment, the Policy Server requests the CMTS to admit the flows using the pkt-mm-2 interface providing detailed information of the QoS requirements. Provided that resources are available, the CMTS will create additional bearers dynamically and push the appropriate traffic filters to the cable modem.

#### A.3.3.1.1	Binding mechanism

In the DOCSIS IP-CAN, the binding mechanism is achieved through the use of traffic Classifiers. These Classifiers filter traffic destined to a UE behind a Cable Modem or sourced from a UE behind a Cable Modem, to a particular DOCSIS service flow. DOCSIS Classifiers contain the following attributes which can be used to filter IP traffic:

-	IP Type of Service – Range and Mask;

-	IP Protocol;

-	IP Source Address;

-	IP Source Mask;

-	IP Destination Address;

-	IP Destination Mask;

-	TCP/UDP Source Port Start;

-	TCP/UDP Source Port End;

-	TCP/UDP Destination Port Start;

-	TCP/UDP Destination Port End.

The Classifier(s) which are used for a particular DOCSIS service flow are communicated to the CMTS by the Policy Server (on behalf of the Application Manager) via the pkt-mm-2 interface.  The Application Manager will specify the QoS requirements for the IP flow, the direction of the IP flow, and the Classifier(s) which are to be used for the DOCSIS service flow serving the IP flow.

When a session is no longer in use, the Application Manager communicates to the CMTS to tear down the resources associated with the session. Based on this communication, the CMTS will remove the DOCSIS service flow(s) and any Classifier(s) associated with the service flow(s), and inform the Cable Modem of the removal. Traffic which previously matched the removed Classifier(s) will now be placed on either the upstream or downstream primary DOCSIS service flow, depending on the direction of the traffic.

### A.3.3.2	Functional entities

#### A.3.3.2.1		Policy Control and Charging Rules Function (PCRF)

In the DOCSIS IP-CAN, the Application Manager (AM) and the Policy Server (PS) fulfil the role of the PCRF.

The AM receives media descriptors (Media Type and Format) from the AF for SIP sessions and maps the  QoS needs of the session to a FlowSpec, The FlowSpec is a layer 2 independent representation of the bandwidth and QoS requirements for the flow derived from the media descriptors using a well defined algorithm. The AM and PS provide network resource control in the DOCSIS IP-CAN by managing the CMTS using the PacketCable Multimedia interface pkt-mm-2.

The AM and PS map IP flows to DOCSIS service flows in accordance with the operator's policies and based on the media format information provided by the AF.

##### A.3.3.2.1.1	Input for PCC decisions

The AM accepts any of the following input as a basis for decisions on PCC rule operations:

-	Per IP-CAN session (e.g.: UE IP address);

-	Requested QoS, media format, priority indicator.

The SPR may provide the following information:

-	Subscribers maximum allowed QoS resources.

Subscriber's maximum allowed bit rate for upstream and downstream.

#### A.3.3.2.2		Policy and Charging Enforcement Function (PCEF)

The CMTS provides PCEF equivalent functionality within the DOCSIS IP-CAN. The CMTS creates, modifies, and deletes DOCSIS service flows upon request of the Policy Server. The CMTS receives requests from the Policy Server over the pkt-mm-2 interface.

#### A.3.3.2.3		Application Function (AF)

### A.3.3.3	Policy and charging control rule

#### A.3.3.3.1	General

#### A.3.3.3.2	Policy and charging control rule operations

# A.x	<IP-CAN>

<overall description>

## A.x.1	High level requirements

### A.x.1.1	General

### A.x.1.2	Charging related requirements

### A.x.1.3	Policy control requirements

## A.x.2	Architecture model and reference points

### A.x.2.1	Reference points

#### A.x.2.1.1	Rx reference point

#### A.x.2.1.2	Gx reference point

#### A.x.2.1.3	Sp reference point

## A.x.3	Functional description

### A.x.3.1	Overall description

#### A.x.3.1.1	Binding mechanism

### A.x.3.2	Functional entities

#### A.x.3.2.1		Policy Control and Charging Rules Function (PCRF)

##### A.x.3.2.1.1	Input for PCC decisions

#### A.x.3.2.2		Policy and Charging Enforcement Function (PCEF)

#### A.x.3.2.3		Application Function (AF)

### A.x.3.3	Policy and charging control rule

#### A.x.3.3.1	General

#### A.x.3.3.2		Policy and charging control rule operations

Annex B (informative):
PCC and usage of visited network services

# B.1	Introduction

This section relates to the use, by a roaming user, of services accessed via a GW in the visited network (i.e. a generic network; GPRS or IP-CAN).

## B.1.1	General aspects

It cannot be assumed that service identifiers, PCC rule names and PCC rule base names may be commonly understood between the home network and the visited network.

## B.1.2	Charging related aspects

It cannot be assumed that charging keys may be commonly understood between the home network and the visited network.

### B.1.2.1	Reporting

Editor's note: it is FFS what reporting level is applicable between the visited network and the home network in regard to the reporting of service usage within the visited network.

### B.1.2.2	Credit Control

The account balance management function is located in the home network.

Editor's note:	It is FFS whether charging key is applicable for credit control between the visited network and the home network in regard to the reporting of service usage within the visited network.

Editor's note:	It is FFS whether Rating occurs in the home network or the visited network.

Editor's note:	It is FFS how to perform credit control where the home and visiting network are subject to different monetary currencies.

## B.1.3	Policy control related aspects

## B.1.4	Subscription related aspects

The subscription profile for the visiting user is located in the home network.

Typically, the home network shall provide sufficient subscription information so that the appropriate PCC rules can be provisioned to the visited GW. This includes providing information to identify PCC rules at bearer or access point level, but may be unable to provide detailed enough subscription information for the visited network to construct service flow based rules. Charging parameters (e.g. charging method and OCS/OFCS addresses) may be provided only at bearer or access point level.

## B.1.5	Architectural aspects

Editor's note:	It is FFS which credit control related interfaces (e.g. Gy, Re, Rc) must support inter-operator communication.

Editor's note:	It is FFS which subscription related interfaces (e.g. Sp, Wm) must support inter-operator communication.

### B.1.5.1	Logical architecture of PCEF in the visited network

When the PCEF is in the visited network for routing optimization, a reference point between the home PCRF and the visited PCRF is needed for information exchange. A logical roaming architecture is shown below.

Figure B.1: Logical architecture of PCEF in the visited network

Editor's note-i:	Whether to introduce a new reference point between the home PCRF and the visited PCRF or just to extend the existing reference point to support the PCC roaming is FFS.

Editor's note-ii: 	The need for a proxy OCS and whether to introduce a new reference point between the home OCS and the proxy OCS or just to extend the existing reference points to support the PCC roaming is FFS.

Editor’s note-iii	the functional split between the H-PCRF and V-PCRF is FFS

NOTE:	the interface between the Billing systems is out of scope of this TS.

The following aspects have been identified for the architecture in figure B.1:

-	Predefined PCC rules that are not part of the roaming agreement can not be dynamically activated by the H-PCRF. I.e. if a service data flow in the home network GW would use a dynamically activated pre-defined PCC rule that is not contained in the roaming agreement, then this service data flow will have to use a different PCC rule in the visited GW.

-   The solution of the PCC roaming case should consider the topology hiding issue among operators.

#### B.1.5.1.1	Functional Requirements for supporting PCC Rules in the visited network

The support of V-PCEF requires the following functionality to be supported by the PCC Architecture:

-	The PCC Architecture should enable the H-PCRF to determine whether a PCC rule needs to be implemented on a V-PCEF or a H-PCEF.

-	The PCC Architecture should enable the initialization and maintenance of the connection between the V-PCRF and H-PCRF.

#### B.1.5.1.2	Functional Requirements for Supporting On-line Charging in the visited network

The support of on-line charging in the VPLMN requires the following functionality to be supported by the PCC Architecture:

-	The PCC Architecture should enable the HPLMN to indicate to the VPLMN that on-line charging is required for a particular IP-CAN session.

-	The PCC Architecture should enable the VPLMN to discover the identity of the home OCS to which Credit Control messages will be sent.

Annex C  (informative):
PCC (GGSN) initiated QoS upgrade in GPRS

# C.1		Introduction

This section investigates mechanisms by which GPRS can realise a PCC (GGSN) initiated QoS upgrade. This requires the GGSN to attempt to establish a higher QoS than was initially requested by the UE.

## C.1.1	General aspects

A number of possible mechanisms exist for the GGSN to modify the QoS associated with a PDP context. It can be modified by the GGSN after or during PDP context establishment. In addition new mechanisms could be introduced to allow the network to establish completely new secondary PDP contexts. This section elaborates on these three possibilities.

## C.1.2	QoS upgrade via PDP context modification

The GGSN initiated PDP context modification procedure (see TS 23.060 [12]) and allows the GGSN to change the QoS for any established PDP context. Upon indication from the PCRF the PCC capable GGSN may use this procedure to change the QoS for a context to match that indicated from the PCRF. The GGSN initiated PDP context modification procedure can also be used immediately after activation to change the QoS for a PDP context which was activated with a different QoS from the one desired by the PCRF. This can be used as a way of working around the restriction that currently exists in TS 23.060 [12] which says that the network can only "restrict" the QoS requested by the mobile at PDP context activation. By using the GGSN Initiated PDP context modification procedure the GGSN can restrict or enhance the QoS originally requested from the mobile.

## C.1.3	QoS upgrade via modification at PDP context establishment or UE initiated context modification

Currently TS 23.060 [12] describe the GPRS network nodes as having the ability to restrict the QoS requested by the UE at PDP context establishment or UE initiated context modification. In order to bring in the capability to remove this restriction and allow network nodes to facilitate GPRS QoS upgrade, enhancements would be required to be made to GPRS in Release 7.

Editor's note:	The enhancements required are FFS.

## C.1.4	QoS upgrade via establishment of a new PDP context

At present no mechanism is specified in GPRS which enables the GGSN to establish a completely new secondary PDP context, however this could be a way of establishing a PDP context with the desired QoS due to AF interaction.

The PCC support required in this case is a PCC rule push mechanism over Gx, allowing the GGSN to derive a TFT, uplink traffic mapping information and a PDP context QoS profile to send to the UE.

The decision to trigger a new PDP context versus modifying an existing PDP context could be based on the QoS Class Identifier assigned to the service data flow in the PCC rule. If this decision is taken in the PCRF or the GGSN is FFS.

## C.1.5	Potential Issues

Recurring QoS upgrades may result due the reception of Update PDP Context Request messages by the GGSN since the GGSN does not know the origin of the update (UE-initiated, SGSN-initiated, or RAN-initiated modifications). QoS upgrades should be attempted once especially when there has been a RAN downgrade. The GGSN should provide a mechanism to avoid attempts to upgrade QoS in short succession. This can be achieved by three possible mechanisms:

1.	Timer based approach whereby the GGSN does not attempt a QoS upgrade before timer expiry. The timer is triggered by the Create PDP Context Response message and should be long enough to cover any updates caused by the RAN downgrading the request.

2.	For PDP context activation, the GGSN shall not attempt to upgrade QoS until the first uplink packet is received.

3.	Explicit indication of the cause of an Update PDP context request – e.g. if RAN or SGSN initiated then the GGSN should not attempt an upgrade. This will require changes to both GTP and SGSN implementation and as a result cannot be relied upon in all situations, especially roaming.

It may be necessary that a combination of the 3 is required to ensure that all roaming/implementation scenarios are catered for.

## C.1.6	Architectural Issues

Editor's note:At MS initiated PDP context modification there is a lack of confirmation message if the RAN does not accept the increased QoS. This may be an issue for charging.

This section may also cover a brief description of any potential general architecture issues related to both the UE and the Network attempting set the desired QoS.

On completion of this analysis, if the GPRS QoS upgrade mechanism has proved to be viable it is proposed that the general requirement for QoS upgrade be included in the main body of 23.203 and corresponding CRs to 23.060 submitted for approval. Finally appropriate text should be moved from this informative annex to the normative GPRS annex before removing this annex to avoid duplication on information.

Annex D (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2005-10-13 | - | - | - | - | S2-052246 approved at SA2#48 | 0.0.1 | 0.1.0 |
| 2005-10-13 | - | - | - | - | S2-052387 approved at SA2#48 | 0.0.1 | 0.1.0 |
| 2005-10-13 | - | - | - | - | S2-052247 approved at SA2#48 | 0.0.1 | 0.1.0 |
| 2005-10-13 | - | - | - | - | S2-052249 approved at SA2#48 | 0.0.1 | 0.1.0 |
| 2005-10-13 | - | - | - | - | S2-052389 approved at SA2#48 | 0.0.1 | 0.1.0 |
| 2005-10-14 | - | - | - | - | S2-052390 approved at SA2#48 | 0.1.0 | 0.1.1 |
| 2005-12-28 | - | - | - | - | S2-052997 approved at SA2#49 | 0.1.1 | 0.2.0 |
| 2005-12-28 | - | - | - | - | S2-052731 approved at SA2#49 | 0.1.1 | 0.2.0 |
| 2005-12-28 | - | - | - | - | S2-052830rev2 approved at SA2#49 | 0.1.1 | 0.2.0 |
| 2005-12-28 | - | - | - | - | S2-052831rev2 approved at SA2#49 | 0.1.1 | 0.2.0 |
| 2005-12-28 | - | - | - | - | S2-052832 approved at SA2#49 | 0.1.1 | 0.2.0 |
| 2005-12-28 | - | - | - | - | S2-052833rev2 approved at SA2#49 | 0.1.1 | 0.2.0 |
| 2005-12-28 | - | - | - | - | S2-052836rev1 approved at SA2#49 | 0.1.1 | 0.2.0 |
| 2005-12-28 | - | - | - | - | S2-052837rev2 approved at SA2#49 | 0.1.1 | 0.2.0 |
| 2006-01-20 | - | - | - | - | S2-060257 approved at SA2#50 | 0.2.1 | 0.3.0 |
| 2006-01-20 | - | - | - | - | S2-060330 approved at SA2#50 | 0.2.1 | 0.3.0 |
| 2006-01-20 | - | - | - | - | S2-060332 approved at SA2#50 | 0.2.1 | 0.3.0 |
| 2006-01-20 | - | - | - | - | S2-060336 approved at SA2#50 | 0.2.1 | 0.3.0 |
| 2006-01-20 | - | - | - | - | S2-060337 approved at SA2#50 | 0.2.1 | 0.3.0 |
| 2006-01-20 | - | - | - | - | S2-060339 approved at SA2#50 | 0.2.1 | 0.3.0 |
| 2006-01-20 | - | - | - | - | S2-060342 approved at SA2#50 | 0.2.1 | 0.3.0 |
| 2006-01-20 | - | - | - | - | S2-060343 approved at SA2#50 | 0.2.1 | 0.3.0 |
| 2006-01-20 | - | - | - | - | S2-060344 approved at SA2#50 | 0.2.1 | 0.3.0 |
| 2006-01-20 | - | - | - | - | S2-060345 approved at SA2#50 | 0.2.1 | 0.3.0 |
| 2006-01-20 | - | - | - | - | S2-060346 approved at SA2#50 | 0.2.1 | 0.3.0 |
| 2006-01-20 | - | - | - | - | S2-060347 approved at SA2#50 | 0.2.1 | 0.3.0 |
| 2006-01-20 | - | - | - | - | S2-060348 approved at SA2#50 | 0.2.1 | 0.3.0 |
| 2006-01-20 | - | - | - | - | S2-060352 approved at SA2#50 | 0.2.1 | 0.3.0 |
| 2006-01-20 | - | - | - | - | S2-060353 approved at SA2#50 | 0.2.1 | 0.3.0 |
| 2006-01-20 | - | - | - | - | S2-060354 approved at SA2#50 | 0.2.1 | 0.3.0 |
| 2006-01-20 | - | - | - | - | S2-060356 approved at SA2#50 | 0.2.1 | 0.3.0 |
| 2006-01-20 | - | - | - | - | S2-060357 approved at SA2#50 | 0.2.1 | 0.3.0 |
| 2006-02-28 | - | - | - | - | S2-060869 approved at SA2#51 | 0.3.1 | 0.4.0 |
| 2006-02-28 | - | - | - | - | S2-061076 approved at SA2#51 | 0.3.1 | 0.4.0 |
| 2006-02-28 | - | - | - | - | S2-061079 approved at SA2#51 | 0.3.1 | 0.4.0 |
| 2006-02-28 | - | - | - | - | S2-061080 approved at SA2#51 | 0.3.1 | 0.4.0 |
| 2006-02-28 | - | - | - | - | S2-061082 approved at SA2#51 | 0.3.1 | 0.4.0 |
| 2006-02-28 | - | - | - | - | S2-061084 approved at SA2#51 | 0.3.1 | 0.4.0 |
| 2006-02-28 | - | - | - | - | S2-061086 approved at SA2#51 | 0.3.1 | 0.4.0 |
| 2006-02-28 | - | - | - | - | S2-061087 approved at SA2#51 | 0.3.1 | 0.4.0 |
| 2006-02-28 | - | - | - | - | S2-061090 approved at SA2#51 | 0.3.1 | 0.4.0 |
| 2006-02-28 | - | - | - | - | S2-061092 approved at SA2#51 | 0.3.1 | 0.4.0 |
| 2006-02-28 | - | - | - | - | S2-061096 approved at SA2#51 | 0.3.1 | 0.4.0 |
| 2006-02-28 | - | - | - | - | S2-061097 approved at SA2#51 | 0.3.1 | 0.4.0 |
| 2006-02-28 | - | - | - | - | S2-061209 approved at SA2#51 | 0.3.1 | 0.4.0 |
| 2006-05-22 | - | - | - | - | S2-061548 approved at SA2#52 | 0.4.0 | 0.5.0 |
| 2006-05-22 | - | - | - | - | S2-061706 approved at SA2#52 | 0.4.0 | 0.5.0 |
| 2006-05-22 | - | - | - | - | S2-061709 approved at SA2#52 | 0.4.0 | 0.5.0 |
| 2006-05-22 | - | - | - | - | S2-061710 approved at SA2#52 | 0.4.0 | 0.5.0 |
| 2006-05-22 | - | - | - | - | S2-061711 approved at SA2#52 | 0.4.0 | 0.5.0 |
| 2006-05-22 | - | - | - | - | S2-061714 approved at SA2#52 | 0.4.0 | 0.5.0 |
| 2006-05-22 | - | - | - | - | S2-061718 approved at SA2#52 | 0.4.0 | 0.5.0 |
| 2006-05-22 | - | - | - | - | S2-061719 approved at SA2#52 | 0.4.0 | 0.5.0 |
| 2006-05-22 | - | - | - | - | S2-061951 approved at SA2#52 | 0.4.0 | 0.5.0 |
| 2006-05-31 | SP-32 | SP-060296 | - | - | MCC Editorial update for presentation to TSG SA for information | 0.5.0 | 1.0.0 |
| 2006-08-08 | SA2#53 | S2-062454 |  |  | SA2#53 | 1.0.0 | 1.1.0 |
| 2006-08-08 | SA2#53 | S2-062455 |  |  | SA2#53 | 1.0.0 | 1.1.0 |
| 2006-08-08 | SA2#53 | S2-062456 |  |  | SA2#53 | 1.0.0 | 1.1.0 |
| 2006-08-08 | SA2#53 | S2-062459 |  |  | SA2#53 | 1.0.0 | 1.1.0 |
| 2006-08-08 | SA2#53 | S2-062462 |  |  | SA2#53 | 1.0.0 | 1.1.0 |
| 2006-08-08 | SA2#53 | S2-062466 |  |  | SA2#53 | 1.0.0 | 1.1.0 |
| 2006-08-08 | SA2#53 | S2-062470 |  |  | SA2#53 | 1.0.0 | 1.1.0 |
| 2006-08-08 | SA2#53 | S2-062471 |  |  | SA2#53 | 1.0.0 | 1.1.0 |
| 2006-08-08 | SA2#53 | S2-062472 |  |  | SA2#53 | 1.0.0 | 1.1.0 |
| 2006-08-08 | SA2#53 | S2-062463 |  |  | SA2#53, email | 1.0.0 | 1.1.0 |
| 2006-08-08 | SA2#53 | S2-062464 |  |  | SA2#53, email | 1.0.0 | 1.1.0 |
| 2006-08-08 | SA2#53 | S2-062465 |  |  | SA2#53, email, r6 | 1.0.0 | 1.1.0 |
| 2006-08-08 | SA2#53 | S2-062469 |  |  | SA2#53, email | 1.0.0 | 1.1.0 |
| 2006-08-08 | SA2#53 | S2-062473 |  |  | SA2#53, email, r1 | 1.0.0 | 1.1.0 |