3GPP TS 23.204 V1.2.0 (2006-05)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Support of SMS and MMS over generic 3GPP IP access;

Stage 2

(Release 7)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

3GPP, SMS, MMS, IP access

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2005, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions, symbols and abbreviations	7

3.1	Definitions	7

3.3	Abbreviations	7

4	Overall Requirements	7

5	Architecture model and reference points	7

5.1	Reference architecture	7

5.2	Reference points	8

5.2.1	General	8

5.2.2	C’ interface	8

5.2.3	Sh interface	8

5.2.4	E/Gd interface	8

5.3	Functional entities	9

5.3.1	IP-Short-Message-Gateway (IP-SM-GW)	9

5.3.2	HSS	9

6	Procedures	11

6.1	Registration procedure	11

6.2	De-registration procedure	12

6.2.1	UE initiated	12

6.3	Successful SMS MO delivery procedure	13

6.4	Successful SMS MT deliver procedure	14

6.5	Successful Delivery Report procedure	16

6.6 	SMS Alerting procedure when IP Based UE has memory available to receive one or more short messages	17

Annex A: Change history	18

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The  Short Message Service (SMS) and Multimedia Message Service (MMS) have enjoyed great success in cellular networks. At the same time, specifications have been and are being developed for extending 3GPP services to non-cellular IP Connectivity Access Networks (IP-CANs). In this same spirit, this specification describes the capabilities needed to support SMS for generic IP-CANs.

This specification also describes the means to deliver the content of IMS Messages (i.e. a SIP MESSAGE) to non-IMS users as SMS messages, as well as means to deliver the content of SMS messages to IMS users as IMS Messages (i.e. SIP Messages).

For MMS over generic IP access, existing mechanisms are used (HTTP and SMTP) for transfer of the MMS itself, these mechanisms are not described in the present document. For MMS notification, SMS over generic IP access can be used, the mechanisms for this are described in the present document.

# 1	Scope

The present document specifies the new capabilities and enhancements needed to support SMS over a generic IP Connectivity Access Network (IP-CAN) using IMS capabilities. These include (but are not limited) to:

1	Enhancements to the HSS;

2	 Communication between the  SMS-GMSC/SMS-IWMSC and the HSS;

3	Authentication of service usage and registration;

4	Transfer of Mobile Terminated SMS, Mobile Originated SMS, Delivery reports;

5	Mechanisms to handle SMS when there is more than one IP connection active with the mobile, etc.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Vocabulary for 3GPP Specifications".

[2]	TS 23.040: "3rd Generation Partnership Project; Technical Specification Group Terminals; Technical realization of the Short Message Service (SMS)".

[3]	3GPP TS 22.101: "Service principles".

[4]	3GPP TS 23.002: "Network architecture".

[5]	3GPP TS 23.003: "Numbering, addressing and identification".

[6]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[7]	3GPP TS 23.140: "Multimedia Messaging Service (MMS)".

[8]	3GPP TS 24.011:" Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

[9]	3GPP TS 23.228: " IP Multimedia (IM) Subsystem – Stage 2".

[10]	3GPP TS 26.141: “IP Multimedia System (IMS) Messaging and Presence; Media formats and codecs”.

[11]	IETF Internet-Draft: “Instant Message Disposition Notification”, draft-ietf-simple-imdn-00.txt, work in progress

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the definitions given in TR 21.905 [1] and the following definitions apply:

IMS core: Refers to the core session control elements of the IM CN Subsystem, i.e. the CSCFs, and the IBCF.

## 3.3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

Editor's note:	Abbreviations to be added.

# 4	Overall Requirements

It shall be possible to support the delivery of SMS over WLAN and other IP Connectivity Access Networks (IP-CANs).

When a user accesses SMS services over a generic IP-CAN:

-	appropriate security mechanisms shall be supported;

-	a registration and de-registration mechanism shall be supported;

-	existing services that use SMS service functionality shall not be degraded.

It shall be possible to deliver the content of IMS Messages (i.e. SIP MESSAGEs) initiated by IMS users to non-IMS users as SMS messages.

It shall be possible to deliver the content of SMS messages to IMS users as IMS Messages (i.e. SIP MESSAGEs).

- It shall be possible for a user to send an IMS message or SMS message to another user without having to know beforehand which messaging client the receiving user has.

- The user experience shall be consistent with the SMS and IMS service expectations.

When more than one IP connections are activated (e.g. UE with combined WLAN/GPRS/UMTS card may be GPRS attached and/or CS attached while the WLAN connection is activated), selecting the connection to use for both mobile originated and terminated SMS shall be possible to be based on operator policy and user preference.

Provisioning of UICC configuration data (i.e OTA mechanisms) over generic IP access shall be supported.

# 5	Architecture model and reference points

## 5.1	Reference architecture

Figure 5.1 below shows the overall architecture for providing SMS over a generic IP CAN.

Figure 5.1: Architecture for providing SMS over a generic IP-CAN

## 5.2	Reference points

### 5.2.1	General

The sub-sections below describe the needed enhancements and specific considerations to existing interfaces in order to support SMS over a generic IP-CAN.

Editor’s note:	At this point there is no new interface defined for SMS over IP.

### 5.2.2	C’ interface

C’ denotes that the following enhancements are needed for the C interface:

-	it shall be possible for the HLR to return the IP-Message-GW address in addition to the MSC and/or SGSN address and/or IP-Message-GW address as routing information to the SMS-GMSC.

### 5.2.3	Sh interface

From SMS over IP point of view, the deployment of the Sh interface is only necessary in case the operator offers multiple bearers besides IP (e.g. CS, GPRS) for SMS. The Sh interface allows the HSS to be informed that the user is capable of receiving SMS over IMS.

### 5.2.4	E/Gd interface

The E/Gd interface allows the IP-SM-GW to connect to the GMSC using MAP, appearing to the GMSC as an MSC or SGSN.

## 5.3	Functional entities

### 5.3.1	IP-Short-Message-Gateway (IP-SM-GW)

The IP-SM-GW shall provide the protocol interworking for delivery of the short message between the IP-based UE and the SMSC. The message is routed to the  SMSC for delivery to the SMS-based user or the message is received from the SMSC of an SMS-based UE for delivery to an IP-based UE. There are two possibilities for delivery of the SMS message: it can either be carried encapsulated in an IMS message, or it can be carried as a text body of an IMS Immediate Message.

Editor’s note: The criteria and means for the IP-SM-GW to make a decision on the means to deliver an SMS via IMS: encapsulate SMS service or IMS Immediate Messaging service is FFS.

The general functions of the IP-SM-GW are:

-	to connect to the GMSC using established MAP protocols over SS7, appearing to the GMSC as an MSC or SGSN using the E or Gd interfaces;

-	to connect to the SMS-IWMSC using established MAP protocols over SS7, appearing to the SMS-IWMSC as an MSC or SGSN using the E or Gd interfaces;

-	to acquire knowledge of the association between the MSISDN and the IP address of the terminal;

-	to check that it has a valid address in SMS for the sender as well as the recipient when receiving an IMS message for an SMS user. The IP-SM-GW shall obtain a valid address for both from the SIP headers of the IMS message (e.g. the sender would be identified in the asserted id in form of TEL URI).

- 	to map the recipient’s address from an SMS MSISDN/IMSI to TEL URI format when receiving an SMS for an IP-based UE, and then it is the responsibility of the IMS core to perform any further mapping towards a SIP URI as required.

-	to act as an Application Server towards the IMS core.

The additional functions of the IP-SM-GW when interworking is done by carrying encapsulated SMS messages in IMS messages are:

-	to communicate with the UE using IMS messaging as transport while maintaining the format and functionality of the SMS message;

-	to carry the SMS status messages as encapsulated bodies of IMS messages;

The additional functions of the IP-SM-GW when interworking is done by translating SMS messages into IMS messages with text bodies are:

-	to translate the SMS message to or from the IMS message using a text encoded format, and to ensure that the IMS message body is one of the allowed text-based content types defined in 3GPP TS 26.141 [26.141 ref];

-	to concatenate the content of several sequence SMS messages and assemble them into a IMS Immediate message as a text body, or to split the content of a IMS Immediate message text body and assemble it into several SMS messages with the sequence numbers;

-	to translate the SMS status messages to or from the IMS Messaging report messages (requires support of SIP Instant Message Disposition Notification I-D draft-ietf-simple-imdn-00.txt [new ref]);

### 5.3.2	HSS

In order to support SMS over generic IP access, the HSS shall support the following functions:

-	storing the address of the IP-SM-GW and providing it as part of the "send routing information for short message" procedure;

-	an indication that the terminal is registered with an IP-SM-GW for delivery of SMS.

NOTE:	The mechanism for prioritizing whether the short message is delivered via a GSM/UMTS or other IP-CAN connection when the terminal is simultaneously connected to both access networks is outside the scope of this specification.

In order to be able to return the address of the IP-SM-GW in response to a "SendRoutingInfoForShortMsg" request from the GMSC, the HLR/HSS needs to have an indication that the terminal is registered with an IP Short Message Gateway and that this is the preferred method for delivery of short messages.

The IP-SM-GW is informed of the UE's registration status in the IMS. On registration, the IP-SM-GW shall send a message to the HSS indicating that the UE has successfully registered.

# 6	Procedures

## 6.1	Registration procedure

Figure 6.1: Registration procedure

1)	The UE establishes IP connection.

2)	At any time after the establishment of the IP connection, the UE registers at the  S-CSCF according to the IMS registration procedures. Note, that for simplicity not all messages between UE and S-CSCF and between S-CSCF and HSS are shown in detail. In addition, I-CSCF and P-CSCF are also not shown.

3)	S-CSCF checks the initial filter criteria retrieved from the HSS during the IMS registration procedure.

4)	After successful IMS registration and based on the retrieved initial filter criteria the S-CSCF informs the IP-SM-GW (AS) about the registration of the user.

5)	The IP-SM-GW (AS) returns OK to the S-CSCF.

6)	The IP-SM-GW (AS) sends IP-SM-GW Register Req to the HSS.

7)	The HSS stores the received information and responses to the IP-SM-GW (AS) with IP-IWF Register Res , including the subscriber data needed for providing service.

Editor’s note: the detailed content of the subscriber data is FFS, e.g., whether the data is same with the current CS/PS SMS subscriber data, or additional enhancement and process need to be defined.

## 6.2	De-registration procedure

### 6.2.1	UE initiated

Figure 6.2: UE initiated de-registration procedure

1)	At any time after the registration procedure, the UE may initiate a de-registration procedure. The UE sends a De-Register request (Register request with Expires header having value 0) to the S-CSCF. Note, that I-CSCF and P-CSCF are not shown in this figure.

2)	S-CSCF responds to the UE with a 200 OK.

3)	S-CSCF checks the initial filter criteria retrieved from the HSS during the IMS registration procedure.

4)	Based on initial filter criteria the S-CSCF informs the IP-SM-GW (AS) about the de-registration of the user.

5)	The IP-SM-GW (AS) returns OK to the S-CSCF.

6)	The IP-SM-GW (AS) de-registers the UE at the HSS sending a De-register Req. It is FFS, whether steps 6 and 7 are needed. If the IP-SM-GW (AS) address is pre-configured in the HSS, steps 6 and 7 can be omitted.

7)	The HSS de-registers the UE, i.e. removes the IP-SM-GW address, and responds to the IP-SM-GW (AS) with De-register Res.

## 6.3	Successful SMS MO delivery procedure

Figure 6.3: Successful SMS MO submit procedure

1)	The UE registers to S-CSCF according the IMS registration procedure. Note, that I-CSCF and P-CSCF are not shown in this figure.

2)	UE submits the SMS message to the S-CSCF using the appropriate SIP method.

3)	S-CSCF forwards the Message to IP-SM-GW (AS).An appropriate initial filter criterion may be required in this step

Editor’s note: The details of this iFC are FFS..

4)	IP-SM-GW (AS) acknowledges the message.

5)	Message sent information is forwarded by S-CSCF to UE.

6)	Upon receipt the delivered SMS message, the IP-SM-GW performs service authorization based on the subscriber data retrieved from the HLR/HSS in the registration/re-registration procedure as described in clause 8.1. If the result of service authorization is negative, the IP-SM-GW shall return the appropriate error information to the UE in a failure report. Otherwise, the IP-SM-GW (AS) extracts the SMS message and forwards it towards SMSC via the SMS-IWMSC using standard MAP signalling (as described in TS 23.040 [2]). The address of SMSC is extracted either from R-URI of the Message or in the SM content. It is FFS whether the SIP Message contains an SMS indication provided by the UE or whether the IP-SM-GW (AS) uses e.g. the SMS service to transport text messages.

Editor’s note: the detailed content of the subscriber data is FFS, e.g., whether the data is same with the current CS/PS SMS subscriber data, or additional enhancement and process need to be defined.

7)	The SMS-IWMSC forwards the SMS message to the SMSC (see TS 23.040 [2]).

8)	SMSC sends submit report to SMS-IWMSC (see TS 23.040 [2]).

9)	SMS-IWMSC sends submit report to IP-SM-GW (AS) (see TS 23.040 [2]).

10)	IP-SM-GW (AS) sends Submit report to S-CSCF.

11)	S-CSCF sends the submit report to the UE.

12)	UE acknowledges the submit report.

13)	Acknowledgement of the submit report is forwarded by S-CSCF to IP-SM-GW (AS).

## 6.4	Successful SMS MT deliver procedure

Figure 6.4: Successful SMS MT delivery procedure

1)	The UE registers to the S-CSCF according to the IMS registration procedure. Note, that I-CSCF and P-CSCF are not shown in this figure.

2)	The SMSC forwards the SMS message to the SMS-GMSC.

3)	The GMSC interrogates the HSS to retrieve routeing information. When a user is IMS registered, the HSS returns the address of IP-SM-GW (AS) along with the address of the MSC.  The logic for selecting the preferred route for message delivery is FFS.

4)	SMS-GMSC delivers the short message to IP-SM-GW (AS), in the same manner that it delivers the short message to an MSC or SGSN, carrying the MSISDN of the target UE.

5)	The IP-SM-GW (AS) uses the TEL-URI of the target UE to populate the SIP Request URI, and then sends the short message using the appropriate SIP method towards the S-CSCF.

6)	S-CSCF forwards the Message to the UE.

7)	The UE responds with an  OK to the S-CSCF

NOTE:	This is not yet the deliver report.

8)	S-CSCF responds with an OK to IP-SM-GW (AS).

## 6.5	Successful Delivery Report procedure

Figure 6.5: Successful Delivery report procedure

1)	The UE has successfully received the short message.

2)	The UE sends a Delivery Report to the S-CSCF.

3)	Based on initial filter criteria S-CSCF forwards the Delivery Report to the IP-SM-GW (AS).

4)	IP-SM-GW (AS) acknowledges the Delivery report to S-CSCF.

5)	S-CSCF acknowledges the Delivery report to the UE.

6)	The IP-SM-GW (AS) sends a Delivery report to the SMS-GMSC.

7)	The SMS-GMSC sends a SM Delivery report status to the HSS.

8)	The SMS-GMSC sends a Delivery report to the SMSC.

### 6.6 	SMS Alerting procedure when IP Based UE has memory available to receive one or more short messages

SMSC

SMS-GMSC

HLR/HSS

IP-

SM-GW

UE

1. SMS-MT delivery attempts (via IP-MESSAGE-GW) failed and the message was queued in SM-SC as described in section 6.x

4. MAP-Alert-SC Req.

5. SC-Alert Req.

2. Memory capacity available

3. MAP_READY_FOR_SM (Memory capacity available)

S-CSCF

Figure 6.6: SMS Alerting procedure over IP connection when UE’s memory available

1)   If SMS MT delivery attempts (via IP-SM-GW) failed because the UE’s Memory Capacity Exceeded, the message to be transferred to IP Based UE is queued in the SMSC.

2)   UE sends a message to IP-SM-GW indicating that the UE has memory available to receive one or more short messages.

3)   IP-SM-GW notifies the HLR/HSS of memory available in the UE.

4)   If the HLR/HSS receives the indication that the UE has memory available to receive one or more short messages, it transfer alert information with the SC addresses and the MSIsdn-Alert to GMSC as described in TS 23.040.

5)   The GMSC sends SC Alert request messages to SM-SCs whose addresses are informed by the HLR/HSS in step 4).

Editor’s note: the feasibility of stage 3 realization is FFS.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2005-09 |  |  |  |  | Skeleton of the new TS | - | 0.0.0 |
| 2005-11 | SA2#49 |  |  |  | Approval of the ad-hoc meeting TDs: S2H050360, S2H050361, S2H050362, S2H050396 | 0.0.0 | 0.1.0 |
| 2005-11 | SA2#49 | S2-053011 | - | - | Update after SA2#49 by editor | 0.1.0 | 0.1.1 |
| 2005-11 | SA#30 | SP-050678 | - | - | Edited by MCC for presentation to TSG SA for information | 0.1.1 | 1.0.0 |
| 2005-02 | SA2#51 | S2-061143 S2-061145 S2-061146 S2-061147 |  |  | Approval of pseudo-CRs IN SA2#51 | 1.0.0 | 1.1.0 |
| 2005-05 | SA2#52 | S2-061868 S2-061990 S2-061871 |  |  | Approval of pseudo-CRs in SA2#52 and the email approval | 1.1.0 | 1.2.0 |