3GPP TS 23.279 V7.3.0 (2006-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Combining Circuit Switched (CS) and 
IP Multimedia Subsystem (IMS) services;

Stage 2

(Release 7)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

UMTS, GSM, circuit mode, packet mode, IMS

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2006, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Abbreviations	6

4	Overall requirements	6

4.1	General description	6

4.2	Service requirements	6

5	Architectural requirements	6

5.1	Architectural requirements	6

5.2	Session scenarios	7

5.3	UE logic	7

6	Architecture	7

7	Capability exchange	9

7.1	General	9

7.2	Capability information	9

7.2.1	Information about the current radio environment	9

7.2.2	UE capability information	9

7.2.3	IM Status	10

7.3	Registering UE capability information	10

7.4	Multiple MEs per user	10

8	Information flows	11

8.1	Exchange of capability information at CS call setup	11

8.2	Exchange of UE capability information	12

8.3	User adds an IMS service to an ongoing CS call	14

8.3.1	IMS session set up without media requiring resource reservation	14

8.3.2	IMS session set- up with media requiring resource reservation	16

8.4	User adds a CS call to an ongoing IMS session	18

8.5	Release of CSI	19

9	Interaction with supplementary services	19

9.1	General	19

9.2	Line Identification	19

9.2.1	Calling Line Identity Presentation (CLIP)	19

9.2.2	Calling Line Identification Restriction (CLIR)	19

9.2.3	Connected Line Identification Presentation (COLP)	20

9.2.4	Connected Line Identification Restriction (COLR)	20

9.3	Call Forwarding	20

9.4	Call Offering	20

9.4.1	Explicit Call Transfer (ECT)	20

9.5	Call Completion	20

9.5.1	Call Waiting (CW) and Call Hold (CH)	20

9.6	Call Barring	21

10	Other considerations	21

10.1	Handover	21

10.2	Relation to SMS	21

Annex A (informative):	Change history	22

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The combination of CS and IMS services (CSI) is the parallel operation of a CS service and an IMS session between the same two users. It does not require a specific subscription and a specific charging correlation.

# 1	Scope

The present document provides architectural details to combine CS services and IMS services for using them in parallel between the same two users in a peer-to-peer context. The document provides a detailed description of how capabilities and identities are exchanged to enable the combination of CS and IMS services between the same two UEs.

The present document includes the following capabilities that enable the combination of CS and IMS services:

-	Radio capability exchange.

-	SIP based UE terminal capability exchange.

-	MSISDN number exchange in SIP.

-	Establishing an IMS session in parallel to an ongoing CS call between the same two UEs.

-	Establishing a CS call in parallel to an ongoing IMS session between the same two users UEs.

The individual CS call or IMS service that are combined are described in their respective specifications.

# 2	References

The following documents contain provisions, which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[3]	3GPP TS 23.081: "Line Identification supplementary services; Stage 2".

[4]	3GPP TS 23.221: "Architectural Requirements".

[5]	3GPP TS 23.002: "Network Architecture".

[6]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[7]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[8]	3GPP TS 23.082: "Call Forwarding (CF) supplementary services; Stage 2".

[9]	3GPP TS 23.083: "Call Waiting (CW) and Call Hold (HOLD) supplementary services; Stage 2".

[10]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[11]	3GPP TS 23.088: "Call Barring (CB) Supplementary Service; Stage 2".

[12]	3GPP TS 23.091: "Explicit Call Transfer (ECT) Supplementary Service; Stage 2".

[13]	3GPP TS 22.279: "Combined CS Calls and IMS Sessions; Stage 1".

[14]	3GPP TS 22.115: "Service Aspects; Charging and Billing".

[15]	3GPP TS 23.087: "User-to-User Signalling (UUS) Supplementary Service; Stage 2".

# 3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

AS	Application Server

CSI	Combination of CS and IMS services

DTM	Dual Transfer Mode

IAM	Initial Address Message

CON	Connect Message

MRFC	Media Resource Function; Control part

MRFP	Media Resource Function; Physical Part

MSRP	Message Session Relay Protocol

RAT	Radio Access Technology

RTP	Real-time Transfer Protocol

# 4	Overall requirements

## 4.1	General description

The "combination of CS and IMS services" (CSI) is essentially a combination of existing CS and IMS services, i.e. mechanisms and procedures for the IMS part of the CSI session apply according to TS 23.228 [2].

The UE presents the CS call and IMS session within one context to the user. To facilitate this, the following capabilities shall be provided:

1.	Exchange of information related to the current radio environment;

2.	Exchange of terminal capability information;

3.	Addition of an IMS session to an ongoing CS call;

4.	Addition of a CS call to an ongoing IMS session.

## 4.2	Service requirements

The service requirements of combining IMS and CS services are described in TS 22.279 [13].

# 5	Architectural requirements

## 5.1	Architectural requirements

The following general requirements are applicable to CSI:

-	The solution is applicable to GERAN and UTRAN;

-	A CSI capable UE requires DTM capability (in case of GERAN access) and MultiRAB capability (in case of UTRAN access);

-	IMS networks and IMS UEs without CSI support should not be impacted;

-	CS core, PS core, xRAN are not to be impacted. Conclusively, changes should be restricted to the IMS elements and the UEs that support CSI for IMS;

-	Procedures connecting the IMS to the CS domain, to the PSTN and to other SIP networks, including other IMS networks should remain unchanged;

-	CS only UEs and PS only UEs are not to be impacted;

-	CSI capable UE provides capabilities to associate the corresponding peer-to-peer CS and IMS communication  to present it within one context for the user. The IMS communication may be peer-to-peer session or session unrelated communication, e.g. IMS immediate messaging;

-	The quality of the CS call (e.g. voice quality, setup delay, handover, etc.) shall not be impacted from a user perception point of view regardless of whether the CS call is combined with an IMS session or not;

-	The use of CSI requires that the UE is CS attached, PS attached and IMS registered;

-	The solution shall be transparent for the end-user;

-	Existing security mechanisms for CS and IMS shall be re-used;

-	For network efficiency, the UE capability exchange functionality requires the terminal to store information about the other terminals' capabilities;

-	Functionality is required to handle remote parties who use more than one device (e.g. with the same MSISDN or the same public user ID).

-	The same MSISDN should be used for the users IMS subscription and their CS subscription. The system behaviour is not specified for the case where the MSISDN for the IMS subscription and the CS subscription are different.

-	It should be possible to provision the UE IMS service mode of operation (see TS 22.279 [13]), i.e. when the UE should perform the IMS registration.

-	If the UE is not IMS registered and gets engaged in a CS call, then the UE should make an IMS registration using a Public User Identity causing the MSISDN used in the CS call to be implicitly registered.

## 5.2	Session scenarios

The generic architectural requirements, as described in TS 23.221 [4], are applicable, and specifically

-	The architectural solution shall support handover scenarios, including inter-system handover;

-	The architectural solution shall support roaming scenarios with home GGSN ("IMS with GPRS roaming");

-	The architectural solution shall support roaming scenarios with visited GGSN ("IMS roaming");

-	The architectural solution shall be compatible with the IMS home control paradigm;

-	The architectural solution shall consider future evolution to support interworking with conversational IMS services, which use PS bearers;

-	The architectural solution shall consider future evolution to support migration towards conversational IMS services, which use PS bearers.

## 5.3	UE logic

A CSI-capable UE shall have logic to trigger the capability and identity exchange required for simultaneous communication on the CS and IMS domains.  Further, the logic shall be able to co-ordinate current activities in the UE , the user preferences, whether support for simultaneous CS and PS access is available and available IMS enablers in such a way, that only those services/enablers are offered to a user, which can be used simultaneously. This logic shall function in such a way that it makes the simultaneous usage of the CS and IMS domains for the media flows as transparent as possible for the user.

For the scenario of a CS call and an IMS session being established at the same time from an end user perspective, an IMS session can be setup first followed by adding a CS call to the IMS session using the call-flow of Section 8.4, or a CS call can be setup first followed by adding the IMS session to the CS call using the call-flow of Section 8.3.

# 6	Architecture

The figure below shows a high level E2E architecture of a simultaneous IMS session and CS call between two end-users belonging to the same operator.

Figure 6-1: High level architecture

NOTE 1:	No specific IMS user plane handling capabilities that are required to support CSI have been identified, i.e. regular IMS user plane handling applies.

-	UE
The UE needs to support simultaneous CS and PS domain access i.e. GERAN DTM and/or UTRAN multiRAB capabilities. Additionally, the UE should support the capability exchange mechanism outlined in Section 7, and the capability to present the CS call and IMS session within the same context to the user.

-	xRAN
 The Radio Access Network is not impacted by Combinational Services. However, for CSI to function, for GERAN access DTM is required, for UTRAN multiRAB is required

-	PS Core
The Packet Switched Core network remains unchanged.

NOTE 2:	For CSI to function, the PS core needs to support IMS.

-	CS Core
The CS Core Network remains unchanged. The CS core network contains MSC/VLR, HLR, and possibly other logical elements according to the 3GPP specifications TS 23.002 [5], TS 24.008 [6] and TS 29.002 [7]. However for the Current Radio Environment information exchange to work, support for User-User Signalling Service 1 is required (TS 22.087 [x]).

-	IMS Core
The IMS routes the SIP signalling between the UE (A) and UE (B). In addition, the IMS provides the session control and supports UE capability exchange mechanism for the support of CSI. The IMS core includes the HSS, the CSCFs, and other logical elements like MRFC, MRFP, MGCF, or Messaging AS, according to 3GPP specifications TS 23.228 [2].

-	AS
The means of using an AS is identical to its usage in IMS. The AS may be utilised to handle the control of the IMS specific aspects of a CSI session, for example service-based charging, as described in TS 22.115 [14]. If service-based charging mechanisms like charging based on the content of a multimedia message, the message type or the number of sent and/or received messages are required, then the AS should be involved. The AS may also provide support for time- and/or volume based charging, see TS 23.228 [2] for a more detailed description.

# 7	Capability exchange

## 7.1	General

It is highly advantageous if the set of services that can be supported between two endpoints is known to the endpoints when (or shortly after) communication is established. This information can be used to provide an indication to the user of the services that are available for a particular user-to-user communication session. This can encourage use of available services and avoid invocation of unavailable services, thereby avoiding customer dissatisfaction and unnecessary resource and bearer establishment attempts.

Two types of capability information are described: information about the current radio environment, and UE capability information.

During the radio capability exchange procedure, if the UE find that the remote UE and its current radio environment supports simultaneous CS and PS services and the IM Status indicates that an IMS communication is likely to be successful, then the UE should attempt an IMS registration (in case IMS registration had not previously been performed) based on preconfigured user's preference.

## 7.2	Capability information

### 7.2.1	Information about the current radio environment

The purpose of the information about the current radio environment is to use it as input to the UE's and/or the user's decision whether to initiate further procedures (e.g. whether to start UE capability exchange, or an IMS session, or "in call MMS", etc.).

This radio environment information exchange occurs over the CS domain during CS call set-up.

This radio environment information is only valid during the lifetime of the CS call. At the end of the CS call, the UE should not store the radio environment information. This information can be used while the CS call is on going to help decide how to present service options to the user and/or whether to initiate a UE capability information exchange.

The following information is exchanged:

a)	The terminal is capable of simultaneous CS and PS services and initiated/received the CS call in a radio environment that currently supports simultaneous CS and PS services.

b)	The IM Status.

c)	UE capability version, which is used for identifying current capabilities of a terminal (to notify capability update).

d)	Personal ME Identifier (as defined in subclause 7.4).

The information flows for exchanging this information are shown in subclause 8.1.

### 7.2.2	UE capability information

The UE capability information provides input to determine the set of services that can be successfully invoked between two users.

NOTE 1:	This UE capability information is exchanged only over the IMS domain. The exchange of such capabilities may occur during peer-to-peer session or session unrelated communication.

It shall be possible to exchange the UE capabilities described below in this subclause. Note that the exchange of these capabilities is subject to the availability of the information and privacy control.

-	IMS Media types which can be supported as IMS media streams (i.e. media component definitions of IMS sessions).

-	Media format parameters for supported IMS media types (Codecs, media file formats etc.).

-	MSISDN and preferred SIP URI for the UE sending the UE capability information.

-	Personal ME Identifier to identify which of the user's MEs the UE capability information is related to.

-	UE capability version.

Additionally, it shall be possible for the UE to use IMS to exchange capability information about:

-	CS video telephony capability;

-	CS voice capability;

-	MMS version supported;

-	Support for other IMS based capabilities or services e.g. PoC.

The UE capability information is exchanged between the calling party and the called party.

NOTE 2:	due to varying radio environments (e.g. DTM/non-DTM, etc.) a UE capability exchange has the best success rate when performed outside of any other service, i.e. when no other CS/PS/IMS service is currently invoked.

The information flows for exchanging UE capabilities are shown in subclause 8.2.

### 7.2.3	IM Status

The IM Status provided by the sending UE as part of the current radio environment information can be used by the receiving UE as an input for IMS registration, session initiation and any subsequent attempt to perform UE capability exchange via OPTIONS.

NOTE:	The IM Status includes the aspect that a UE may always register if the remote UE is in a state where IMS communication is possible.

The IM Status provided by a UE during CS all setup is valid for the duration of that CS call.

## 7.3	Registering UE capability information

During IMS registration, a UE may register its capability information using SIP User Agent capability registration mechanism specified in RFC 3840 and endorsed by TS 23.228 [2]. To facilitate the operation of CSI, it shall be possible for the UE to register at least the following UE capabilities:

-	CS video telephony capability;

-	CS voice capability.

Registration of these UE capabilities could help the core IMS network in routing SIP messages to appropriate UE when the caller indicates preference for these capabilities in the a SIP message using mechanism specified in RFC 3841 and endorsed by TS 23.228 [2].

The UE may update registered capabilities as specified in TS 23.228 [2].

## 7.4	Multiple MEs per user

For network efficiency, the capability detection functionality requires the terminal to store information about the other terminals' capabilities.

In order to cater for remote parties who use more than one ME (e.g. with the same MSISDN or the same public user ID), CSI needs a mechanism that allows for identifying a particular user's ME. This mechanism shall be capable of identifying a ME upon UE capability exchange, CS call setup and IMS session initiation. This enables the remote party to retrieve the correct stored ME capabilities.

In order to limit network signalling (e.g. use of SIP OPTIONS to trigger other party's request) and avoid an inconsistency between the actual and stored capabilities, it is necessary to exchange the UE capability version.

Procedures are needed to avoid MEs of one user having the same Personal ME Identifier.

The Personal ME Identifier and UE capability version shall fulfil the following requirements:

-	Minimal impact on SIP signalling and no impact on the IM CN subsystem;

-	The identifier and the capability version should fit into UUS-1 signalling, also allowing for other services to be run over UUS-1;

-	The Personal ME Identifier uniquely identifies an ME of the user.

-	The capability version is updated when UE changes its capability (e.g. UE performing online upgrade or configuration change). The capability version is unique for a given set of capabilities of a UE.

# 8	Information flows

## 8.1	Exchange of capability information at CS call setup

It shall be possible for the UE to perform end-to-end information exchange about the current radio environment during CS call setup. The current radio environment information exchange procedure shall include the information as outlined in subclause 7.2.1.

NOTE:	There will exist UEs, which do not support the radio environment exchange procedure, but do support parallel CS calls and IMS sessions, e.g. Rel-5 IMS-capable UMTS UEs. Thus lack of an answer in the radio capability exchange procedure does not mean that the remote UE cannot handle a parallel IMS session or the SIP based capability exchange.

The sequence diagram in figure 8-1 outlines the exchange of information about the current radio environment, at CS call setup. The diagram the messages that should be used to transport the information of the current radio environment: the full message sequences for UUS-1 are specified in TS 23.087 [15]. For this procedure to be successful, the network must handle the radio capability information transparently.

Figure 8-1: Exchange of current radio environment information "at" CS call setup

1)	The UE-A initiates a CS call by sending a SETUP message towards UE-B, including the current radio environment information encoded in the User-User Signalling IE.

2)	The CS domain of the originating network sends an IAM message including the current radio environment information of UE-A to the CS domain of the terminating network. Whether the MSC performs the procedures for UUS-1 (refer to TS 23.087 [15]), or, whether the MSC merely copies the User-User Signalling IE from the SETUP message into the IAM is implementation dependent. Additional MSC based policing of the UUS information content is also implementation dependent.

3)	The CS domain of the terminating network sends a SETUP message IAM including the current radio environment information of UE-A to the UE-B. Whether the GMSC and/or the VMSC performs the procedures for UUS-1, or, whether they merely copy the information into the User-User Signalling IE of the SETUP message from the IAM is implementation dependent.

- The UE-B stores the current radio environment information of UE-A and sends the current radio environment information of UE-B in the final response to the SETUP message, i.e the CONNECT message. UE-B takes the current radio environment information of UE-A into account when deciding what service options to present to the user and/or whether to initiate a UE capability information exchange, see subclause 8.2.

If  UE-B find that UE-A and UE-A's current radio environment supports simultaneous CS and PS services and the IM Status indicates that an IMS communication is likely to be successful, then UE-B should attempt an IMS registration (in case IMS registration had not previously been performed) based on preconfigured user's preference.

NOTE:	The radio environment information is only sent in the CONNECT message to avoid sending non-relevant information to the originating side, e.g. in case Call Forwarding on No Reply is active.

5)	The CS domain of the terminating network sends an ANM or CON message including the current radio environment information of UE-B to the CS domain of the originating network.

6)	The CS domain of the originating network sends a CONNECT message including the current radio environment information of UE-B to the UE-A.

7)	The UE-A takes the current radio environment information of UE-B received into account when deciding what service options to present to the user and/or whether to initiate a UE capability information exchange, see sub-clause 8.2.

If  UE-A find that UE-B and UE-B's current radio environment supports simultaneous CS and PS services and the IM Status indicates that an IMS communication is likely to be successful, then UE-A should attempt an IMS registration (in case IMS registration had not previously been performed) based on preconfigured user's preference.

## 8.2	Exchange of UE capability information

This Section outlines the exchange of UE related capability information using the SIP OPTIONS procedure to minimize the amount of network signalling and resource usage as well as the number of failed SIP INVITE requests. It also allows an up-to-date indication to the user which capabilities he could add to the ongoing call. Note that UE capability information exchange at IMS session initiation is specified in subclause 8.4.

It shall be possible for a UE to request the SIP OPTIONS request to be sent to any other registered UE. In case of existing IMS session between UE-A and UE-B, to guarantee that SIP OPTIONS request is routed to UE-B, the SIP OPTIONS request should be sent as part of the existing IMS session. In case there is an ongoing CS call between UE-A and UE-B, it should be possible to provide a higher probability that the UE capability exchange is routed to the UE-B.

As the SIP OPTIONS request include both the IMS Public User Identity in the form of an SIP URI and the MSISDN the procedure enables both UE-A and UE-B to correlate the IMS session with the CS call and within one context inform the user what capabilities the user is able to use.

NOTE:	If the UICC is not provisioned with the MSISDN the UE may get it during the IMS registration as an associated identity.

The execution of this SIP OPTIONS request procedure is recommended when UE-A has not stored capability information for UE-B, or when UE-A has become aware that UE-B has changed its capabilities by comparing the stored and received UE capability version.

A SIP OPTIONS may also be sent by UE-A to UE-B in case UE-A's capabilities have been updated. This request triggers UE-B to initiate SIP OPTIONS request towards UE-A to retrieve the updated capabilities.

Figure 8-2: Exchange of UE capability information

1)	UE-A sends an SIP OPTIONS request towards UE-B preferably using a SIP URI of UE-B, or a TEL URI, if no valid SIP URI is available. In case of an existing IMS session, the OPTIONS request should be sent as part of the existing session.  Subject to privacy controls, in UE-A the SIP OPTIONS request shall contain MSISDN of UE-A, if available.

2)	The IMS Core (A) performs the normal security procedures and forwards the SIP OPTIONS request towards IMS Core (B). If the destination address is in the format of a TEL URI, IMS Core (A) performs MSISDN to SIP URI translation as per subclause 4.3.5 in TS 23.228 [2], before forwarding the SIP OPTIONS request to IMS Core (B).

The IMS Core (A) should add the MSISDN of UE-A to the SIP OPTIONS request, if not included by UE-A.

3)	If the SIP OPTIONS request is not sent as part of an existing dialog, the IMS Core (B) makes a routing decision based on information in the caller preferences, as defined in RFC 3841, in the SIP OPTIONS request and any registered caller capabilities, as defined in RFC 3840, (e.g. CS-Voice or CS-Video).

4)	The IMS Core (B) then forwards the SIP OPTIONS request to UE-B. If privacy is requested, IMS Core (B) shall remove the MSISDN of UE-A.

5)	The UE-B stores the address information of UE-A.

6)	The UE-B sends a 200 OK that, subject to UE-B's privacy settings contain the information outlined in subclause 7.2.2.

7)	The IMS Core (B) forwards the 200 OK to IMS Core (A).

The IMS Core (B) should add the MSISDN of UE-B to the 200 OK, if not included by UE-B.

8)	The IMS Core (A) forwards the 200 OK to UEA-A. If privacy is requested, IMS Core (A) shall remove the MSISDN of UE-B.

9)	The UE-A stores or updates the UE capability information received and if not already available stores the address information of UE-B.

For the capability exchange procedure to work properly UE-B should send an SIP OPTIONS request towards UE-A, in the following situations, provided that the associated conditions are met:

1.	An SIP INVITE request is received from UE-A, and

-	The SIP INVITE request received from UE-A did not include any UE's capabilities capability information, and

-	UE-B has not stored capability information for UE-A's capabilities, or UE-B's capabilities have been updated e.g. UE-B has been upgraded with video capability or supports a new service, or;

-	The UE capability version included in the SIP INVITE request received from UE-A is different from the previously stored UE-A's capability version.

2.	UE-B is in a CS call with UE-A, and

-	UE-B has not stored capability information for UE-A or has received a UE capability version different from previously stored UE-A's capability version from UE-A during CS call setup, and

-	If received, the current radio environment information indicates that UE-A is capable of supporting CS and PS simultaneously.

3.	A SIP OPTIONS request is received from UE-A, and

-	There is no ongoing (or recently finished) UE-B initiated capability exchange with UE-A.

NOTE:	The received SIP OPTIONS is not a result of a recent UE-B capability version sent from UE-B.

In the situations 1 and 2 above, a SIP OPTIONS request may also be sent by UE-B to UE-A in case UE-B's capabilities have been updated. This request triggers UE-A to initiate SIP OPTIONS request towards UE-B to retrieve the updated capabilities.

## 8.3	User adds an IMS service to an ongoing CS call

### 8.3.1	IMS session set up without media requiring resource reservation

The following sequence diagram shows an IMS service being added to an ongoing CS call when the CSI capabilities of UE-B have not previously been stored by UE-A and are therefore exchanged after CS call setup.

NOTE 1:	The SIP session may setup any service based on IMS and normal requirements as per TS 23.228 [2] apply.

Figure 8.3-1: User adds an IMS session to an ongoing CS call

1)	A CS call is setup as per subclause 8.1.

2)	The UE-A should initiate an IMS capability exchange as described in section 8.2.

NOTE 2:	This step is only needed when UE-A does not have the UE-B IMS capabilities stored and vice versa.

NOTE 3:	The IMS Capability exchange will also include the correlation between the MSISDN and the SIP URI.

3)	The UE-A shall send the SIP INVITE request to the IMS Core along the signalling path established during registration.

4)	The IMS Core (A) forwards the INVITE request to IMS Core (B).

5)	The IMS Core (B) forwards the INVITE request to UE-B.

6)	The UE-B shall associate the INVITE request with the ongoing CS call by using the MSISDN and SIP URI, obtained through the IMS Capability exchange procedure and/or included in the INVITE request

7)	The UE-B invokes the correct application, which associates the SIP session with the ongoing call by matching the identities used in the CS call and the SIP session.  The UE-B then sends a 200 OK.

8)	The IMS Core (B) forwards the 200 OK to IMS Core (A).

9)	The IMS Core (A) forwards the 200 OK to UE-A.

10)	The UE-A acknowledges the 200 OK.

11)	The IMS Core (A) forwards the acknowledgement to IMS Core (B).

12)	The IMS Core (B) forwards the acknowledgement to UE-B.

13)	Media as per the session setup is sent between the two UEs.

### 8.3.2	IMS session set- up with media requiring resource reservation

For an IMS session setup in the context of CSI it shall be possible to require media resource reservation as per procedures in TS 23.228 [2], illustrated in the use case below.

The following sequence diagram shows an IMS service being added to an ongoing CS call when the CSI capabilities of UE-B have not previously been stored by UE-A and are therefore exchanged after CS call setup. Only media resource reservation based on the "inactive" mechanism is shown.

Figure 8.3-2: User adds an IMS session to an ongoing CS call

1)	A CS call is setup as per subclause 8.1.

2)	The UE-A should initiate an IMS capability exchange as described in section 8.2. If UE-B does not receive any IMS capability exchange from UE-A within a certain time limit the UE-B should initiate the IMS capability exchange, if required.

NOTE 1:	This step is only needed when UE-A does not have the UE-B IMS capabilities stored and vice versa.

NOTE 2:	The IMS Capability exchange will also include the correlation between the MSISDN and the SIP URI.

3)	The UE-A shall send the SIP INVITE request with the media components marked "inactive" to the IMS Core along the signalling path established during registration.

4)	The IMS Core (A) forwards the INVITE request to IMS Core (B).

5)	The IMS Core (B) forwards the INVITE request to UE-B.

6)	The UE-B shall associate the INVITE request with the ongoing CS call by using the MSISDN and SIP URI, obtained through the IMS Capability exchange procedure and/or included in the INVITE request. If required, UE-B immediately initiates IP-CAN bearer setup. No alerting of user B needs to be carried out.

7)	The UE-B directly sends a 200 OK with the media components marked 'inactive'.

8)	The IMS Core (B) forwards the 200 OK to IMS Core (A).

9)	The IMS Core (A) forwards the 200 OK to UE-A.

10) The UE-A initiates IP-CAN bearer setup for the media and acknowledges the 200 OK.

11)	The IMS Core (A) forwards the acknowledgement to IMS Core (B).

12)	The IMS Core (B) forwards the acknowledgement to UE-B.

13) 	The UE-A shall send the SIP INVITE request with the media components marked "active" to the IMS core when the IP-CAN bearer is established on UE-A access.

14)	The IMS Core (A) forwards the INVITE request to IMS Core (B).

15)	The IMS Core (B) forwards the INVITE request to UE-B.

16)	 The UE-B shall perform necessary service action to receive/send user plane media.

17)		The UE-B shall send 200 OK with the media components marked 'active' when the IP-CAN bearer is setup and the UE is ready to receive media.

18)	The IMS Core (B) forwards the 200 OK to IMS Core (A).

19)	The IMS Core (A) forwards the 200 OK to UE-A.

20)	The UE-A acknowledges the 200 OK.

21)	The IMS Core (A) forwards the acknowledgement to IMS Core (B).

22)	The IMS Core (B) forwards the acknowledgement to UE-B.

23)	User plane connection is established.

## 8.4	User adds a CS call to an ongoing IMS session

Figure 8.4-1: User adds a CS call to an ongoing IMS Session

1)	The UE-A sends the SIP INVITE request to the IMS Core (A) using the address obtained during registration.

The SIP INVITE may contain CSI specific information including MSISDN and current CSI capabilities in addition to the standard information for the desired IMS service.

2)	The IMS-Core (A) forwards the SIP INVITE request to the IMS Core (B)

3)	The IMS-Core (B) forwards the SIP INVITE request to UE-B.

4)	The UE-B should send a provisional response i.e. 18x (or a final response) and include the MSISDN of UE-B. If the session includes media requiring resource reservation then same principles apply as described in subclause 8.3.2, except that the UE-B should reply with a provisional response to allow the user to answer from other UEs.

5)	The IMS Core (B) forwards the provisional or final response to IMS Core (A).

6)	The IMS Core (A) forwards the provisional or final response to UE-A

7)	The IMS flow continues as standard.

8)	The UE-A initiates a CS call by sending a SETUP message towards UE-B.

9)	The CS domain of the originating network sends an IAM message to the CS domain of the terminating network.

10)	The CS domain of the terminating network sends a SETUP message IAM of UE-A to the UE-B.

UE-B recognises the calling party number as negotiated in SIP session setup.

NOTE:	Without exchanging radio capabilities in IMS, the PS connection could be suspended. From the user experience perspective this is considered as acceptable.

11)	The UE-B sends ALERTING message to UE-A.

12)	The CS domain of the terminating network sends an ACM message to the CS domain of the originating network.

13)	The CS domain of the originating network sends an ALERTING message to the UE-A.

14)	The UE-B sends CONNECT message to UE-A.

15)	The CS domain of the terminating network sends an ANM message to the CS domain of the originating network.

16)	The CS domain of the originating network sends an CONNECT message to the UE-A.

## 8.5	Release of CSI

The UE shall release the CS call and the IMS session independently of each other.

# 9	Interaction with supplementary services

## 9.1	General

CS supplementary services apply to the CS component of the CSI call only. The present clause describes how best to configure and utilize CS Supplementary Services in the context of CSI.

NOTE:	The CS supplementary services are defined in TS 23.081 [3] (Line Identification), TS 23.082 [8] (Call Forwarding), TS 23.083 [9] (Call Waiting and Hold), TS 23.088 [11] (Barring) and TS 23.091 [12] (Explicit Call Transfer).

This TS covers only the Supplementary Services that are identified as having an impact on CSI within the current release as stated in TS 22.279 [13].

## 9.2	Line Identification

### 9.2.1	Calling Line Identity Presentation (CLIP)

It is beneficial to utilize CLIP in the context of CSI.

1)	The called party uses the CLI of the calling party to correlate an incoming SIP INVITE with the CS call.

2)	When the called party wishes to establish an IMS session with the calling party in the context of the CS call, the called party uses the CLI of the calling party to derive the destination URI of the IMS session. The UE may use the CLI as TEL URL or may use the CLI to derive a SIP URI.

### 9.2.2	Calling Line Identification Restriction (CLIR)

If the calling party is subscribed to the automatic suppression of the presentation of her CLI, then it must be anticipated that the network must also automatically suppress her "IMS CLI", and, that her UE shall not reveal her CLI to other parties without her explicit permission. This can be achieved by either:

a)	The network operator refuses to give an IMS subscription to her.

b)	Appropriate mechanism for the HSS to control the removal of "CLI" based on subscription information.

NOTE:	Point B is related to an IETF privacy mechanism and is identified as a generic IMS issue, not one specifically related to CSI. As this causes subscriber information to be sent around more than usual, it will be worked on as a generic IMS issue within 3GPP.

The calling party may also wish to use CLIR on a "per call" basis. In this case, the UE shall not include any CLI information in any OPTIONS data exchange linked to the CS call.

There are several mechanisms that can be imagined for the UEs to swap static terminal information as a background task, e.g., outside of CS calls and 'user initiated' IMS sessions. Because the E.164/identity information may need to be restricted from transmission to certain destinations, the UE shall ensure that the user's permission is obtained before such sensitive information is transmitted.

Given that CLIP is highly desirable and useful for CSI, it is accepted that the use of CLIR causes significant degradation to the overall user experience in case of CSI.

### 9.2.3	Connected Line Identification Presentation (COLP)

It is beneficial to utilize COLP in the context of CSI:

1)	The calling party uses the COL of the connected party to correlate an incoming SIP INVITE with the CS call.

2)	When the calling party wishes to establish an IMS session with the connected party, the calling party uses the COL of the called party as the destination URI of the IMS session. The UE may use the COL as TEL URL or may use the COL to derive a SIP URI.

NOTE:	The availability of the COL may be affected by Call Forwarding GSM supplementary service, regulations and network services such as IN.

### 9.2.4	Connected Line Identification Restriction (COLR)

If the presentation of her COL is suppressed by means of a subscription or on a per call basis, then automatic combination of the IMS session and the CS call is unavailable. Note that user can still manually combine the CS call and the IMS session.

## 9.3	Call Forwarding

When a call is subject to CS call forwarding, the calling party is notified that the call has been forwarded. In CS first scenario, when the user would like to establish an IMS session that is to be automatically combined with this call then the user initiates the IMS session to the forwarded-to user. In IMS first scenario, if the UE can not associate the Public User Identity of the remote UE (of the IMS session) with the COL of the CS connected party, the UE realizes that the CS call can't be established in context of the existing IMS session and appropriately notifies the user.

Call forwarding may result in the restriction of the presentation of the COL, depending on subscriber option settings.  Refer to the section on Line Identification for the usage of the CLI and COL for establishing an IMS session associated with the CS call and for correlating an incoming IMS session with the CS call.

## 9.4	Call Offering

### 9.4.1	Explicit Call Transfer (ECT)

If a UE has an ongoing IMS session with one of two parties and invokes ECT, the end user may keep or terminate this IMS session when ECT is invoked.

The two parties that have established a CS session after ECT will not have each others line identification and are therefore incapable of establishing a CSI call / session.

If the IMS session is established prior to the establishment of the CS call/session, then the two parties will not have each others line identification and are therefore incapable of establishing a CSI call/session.

## 9.5	Call Completion

### 9.5.1	Call Waiting (CW) and Call Hold (CH)

When a subscriber (calling or called) is engaged in a CS call and a second call is offered to her (Call Waiting), an IMS session may be ongoing between that subscriber and her speech partner of the ongoing call. The offering of the second call (i.e. the alerting) does not affect the ongoing IMS session.

When a subscriber (calling or called) receives a CS call when already engaged in another CS call, then she may act as follows.

a)	Reject the incoming call. This action does not affect the IMS session of the active call.

b)	Release the first CS call and answer the second CS call. The user may decide whether to keep the IMS session that was established in the context of the first CS call. The user may also decide to establish a new IMS session to be combined with the second CS call.

c)	Invoke Call Hold. The first call is placed on hold and the second call is answered. The following options apply to the IMS session for the first call:

Option I:	The IMS session is retained, but the sending and receiving of streaming data is suspended.

Option II:	The IMS session is retained and the sending and receiving of non real-time data continues.

Similar principles apply to the case where A-party places an ongoing call on hold and establishes a second CS call.

## 9.6	Call Barring

If a CS call is barred, then IMS sessions in the context of the CS call are not applicable.

If an IMS session is active and the user intends to establish a CS call, then Call Barring categories apply.

# 10	Other considerations

## 10.1	Handover

-	Handover from DTM GERAN or UTRAN to non-DTM GERAN
If, during a simultaneous IMS session and CS call between two end-users, one of the end-users makes an intersystem handover into a non-DTM GERAN access, in this case the data traffic on the PDP contexts are handled as per procedures described in TS 23.060 [10].

-	Handover from non-DTM GERAN to DTM GERAN or UTRAN
When a UE is participating in a CS call and not able to operate in Class A mode of operation, the UE cannot perform IMS capability exchange procedures.  When the UE is again able to operate in Class A mode of operation, the UE can perform the IMS capability exchange procedures during the CS call, if required according to procedures outlined in sections 7 and 8.

## 10.2	Relation to SMS

A user should be able to send or receive SMS during an ongoing CSI session. SMS is in general treated independently from CSI.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2005-09 | SA#29 | SP-050483 | - | - | Editorially updated by MCC for presentation to TSG SA#29 for approval | 2.3.0 | 2.3.1 |
| 2005-09 | - | - | - | - | Updated by MCC for Publication as version 7.0.0 | 2.3.1 | 7.0.0 |
| 2005-12 | SA#30 | SP-050669 | 0001 | 1 | Deletion of Radio Capability Information when adding a CS Call to an ongoing IMS session | 7.0.0 | 7.1.0 |
| 2005-12 | SA#30 | SP-050669 | 0002 | 1 | Cleanup of CS flow | 7.0.0 | 7.1.0 |
| 2005-12 | SA#30 | SP-050669 | 0003 | 2 | IMS registration trigger | 7.0.0 | 7.1.0 |
| 2005-12 | SA#30 | SP-050669 | 0004 | 1 | Information about the current radio environment | 7.0.0 | 7.1.0 |
| 2005-12 | SA#30 | SP-050669 | 0005 | 1 | Clarification to missing up-to-date information | 7.0.0 | 7.1.0 |
| 2005-12 | SA#30 | SP-050669 | 0006 | 1 | Send UE capability update notification | 7.0.0 | 7.1.0 |
| 2005-12 | SA#30 | SP-050669 | 0009 | 1 | Clarify the interaction between CSI and Call Forwarding | 7.0.0 | 7.1.0 |
| 2006-03 | SA#31 | SP-060128 | 0010 | 1 | Registration of Personal ME Identifier | 7.1.0 | 7.2.0 |
| 2006-03 | SA#31 | SP-060228 | 0011 | 2 | Extend UE capability update Indication to cover IMS first case | 7.1.0 | 7.2.0 |
| 2006-03 | SA#31 | SP-060128 | 0012 | 1 | Clarification to meaningless sentences | 7.1.0 | 7.2.0 |
| 2006-06 | SA#32 | SP-060280 | 0016 | 2 | Introduction and exchange of IM Status | 7.2.0 | 7.3.0 |
| 2006-06 | SA#32 | SP-060399 | 0017 | - | Removing Registration of Personal ME Identifier | 7.2.0 | 7.3.0 |