3GPP TS 24.167 V6.4.0 (2006-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

3GPP IMS Management Object (MO);

Stage 3

(Release 6)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

UMTS, IMS, SIP, Multimedia, Management, GSM

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2006, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	3GPP IMS Management Object	6

5	Management Object parameters	7

5.1	General	7

5.2	Node: /<X>	7

5.3	/<X>/AppID	8

5.4	/<X>/Name	8

5.5	/<X>/ConRefs/	8

5.6	/<X>/ConRefs/<X>	8

5.7	/<X>/ConRefs/<X>/ConRef	8

5.8	/<X>/PDP_ContextOperPref	8

5.9	/<X>/P-CSCF_Address	9

5.10	/<X>/Timer_T1	9

5.11	/<X>/Timer_T2	9

5.12	/<X>/Timer_T4	9

5.13	/<X>/Private_user_identity	10

5.14	/<X>/Public_user_identity_List/	10

5.15	/<X>/Public_user_identity_List/<X>	10

5.16	/<X>/Public_user_identity_List/<X>/Public_user_identity	10

5.17	/<X>/Home_network_domain_name	11

5.18	/<X>/Ext/	11

Annex A (informative):	Management Object DDF	12

Annex B (informative):	3GPP IMS Client Provisioning Application Characteristics (CP AC)	19

B.1	General	19

B.2	Definition of the 3GPP IMS Client Provisioning Application Characteristics	19

Annex C (informative):	Change history	22

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

This document defines a mobile device 3GPP IMS Management Object. The management object is compatible with OMA Device Management protocol specifications, version 1.1.2 and upwards, and is defined using the OMA DM Device Description Framework as described in the Enabler Release Definition OMA-ERELD-SyncML_DM-V1_1_2-20031209-A [9].

The 3GPP IMS Management Object consists of relevant parameters that can be managed for the IM CN Subsystem. This includes the basic framework defined in 3GPP TS 23.228 [4] and 3GPP TS 24.229 [5], and early IMS as defined in 3GPP TS 23.221 [3].

The IMS Management Object defines a repository of data into the ME including parameters that are provisioned from the ISIM application (i.e. 3GPP TS 31.103 [11]) or, after derivation, from the USIM application (i.e. 3GPP TS 31.102 [10]).

If the procedures for OMA Device Management is not supported by an implementation, the OMA Client Provisioning can be used as a fallback. Annex B of this document defines the 3GPP IMS Client Provisioning Application Characteristics as described in the Enabler Release Definition OMA-WAP-ProvCont-V1_1-20050428-C [13].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the 3GPP IMS Management Object document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.003: "Numbering, addressing and identification".

[3]	3GPP TS 23.221: "Architectural requirements".

[4]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[5]	3GPP TS 24.229: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[6]	Void.

[7]	Void.

[8]	RFC 1123: "Requirements for Internet Hosts -- Application and Support".

[9]	OMA-ERELD-SyncML_DM-V1_1_2-20031209-A: "Enabler Release Definition for OMA Device Management (based on SyncML DM), Version 1.1.2".

[10]	3GPP TS 31.102: "Characteristics of the USIM application".

[11]	3GPP TS 31.103: "Characteristics of the IP Multimedia Services Identity Module; (ISIM) Application".

[12]	OMA-ERELD-DM-V1_2-20050607-C: "Enabler Release Definition for OMA Device Management, Candidate Version 1.2".

[13]	OMA-WAP-ProvCont-V1_1-20050428-C: "Enabler Release Definition for Client Privisioning, Version 1.1".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] apply.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

AC	Application Characteristics

CN	Core Network

CP	Client Provisioning

CSCF	Call Session Control Function

DDF	Device Description Framework

DM	Device Management

IMS	IP Multimedia core network Subsystem

IP	Internet Protocol

ISIM	IM Services Identity Module

MO	Management Object

OMA	Open Mobile Alliance

P-CSCF	Proxy – CSCF

PDP	Packet Data Protocol

SIP	Session Initiation Protocol

UE	User Equipment

USIM	Universal Subscriber Identity Module

# 4	3GPP IMS Management Object

The 3GPP IMS Management Object is used to manage settings of the UE for IM CN Subsystem protocols. The Management Object covers generic parameters for the IM CN subsystem. The Management Object enables the management of the settings on behalf of the end user.

The Management Object Identifier is: org.3gpp/1.0/SIPCore.

Protocol compatibility: This MO is compatible with OMA DM 1.1.2.

The following nodes and leaf objects are possible under the 3GPP_IMS node:

Figure 1: The 3GPP IMS Management Object

# 5	Management Object parameters

## 5.1	General

This clause describes the parameters for the 3GPP IMS Management Object.

## 5.2	Node: /<X>

This interior node acts as a placeholder for one or more accounts for a fixed node.

- Occurrence: OneOrMore

- Format: node

- Access Types: Get

- Values: N/A

The interior node is mandatory if the UE supports the IM CN Subsystem. Support for a UE is defined by the user agent role as defined in 3GPP TS 24.229 [5].

NOTE:	One node is normally used. More nodes are only used in case the terminal supports multiple UICCs.

## 5.3	/<X>/AppID

The AppID identifies the type of the application service available at the described application service access point. The value is globally unique.

- Occurrence: One

- Format: chr

- Access Types: Get

- Value: <ap2001>

NOTE:	The value of the 3GPP_IMS/AppID is determined by OMA.

## 5.4	/<X>/Name

The Name leaf is a name for the 3GPP_IMS settings.

- Occurrence: ZeroOrOne

- Format: chr

- Access Types: Get

- Values: <User displayable name>

## 5.5	/<X>/ConRefs/

The ConRefs interior node is used to allow a reference to a list of network access point objects.

- Occurrence: One

- Format: node

- Access Types: Get

- Values: N/A

## 5.6	/<X>/ConRefs/<X>

This run-time node acts as a placeholder for one or more network access point objects.

- Occurrence: OneOrMore

- Format: node

- Access Types: Get

- Values: N/A

## 5.7	/<X>/ConRefs/<X>/ConRef

The ConRef leaf represents one or more network access point objects.

- Occurrence: One

- Format: chr

- Access Types: Get, Replace

- Values: <A network access point object>

## 5.8	/<X>/PDP_ContextOperPref

The PDP_ContextOperPref leaf indicates an operator’s preference to have a dedicated PDP context for SIP signalling.

- Occurrence: One

- Format: bool

- Access Types: Get, Replace

- Values: 0, 1

0 – Indicates that the operator has no preference for a dedicated PDP context for SIP signalling.

1 – Indicates that the operator has preference for a dedicated PDP context for SIP signalling.

The PDP_ContextOperPref leaf indicates a preference only. 3GPP TS 24.229 [5] describes the normative options and the procedures for establishment of a dedicated PDP context for SIP signalling.

## 5.9	/<X>/P-CSCF_Address

The P-CSCF_Address leaf defines an FQDN or an IPv4 address to an IPv4 P-CSCF.

- Occurrence: ZeroOrOne

- Format: chr

- Access Types: Get, Replace

- Values: <A fully qualified domain name> or IPv4 address>

The P-CSCF_Address leaf shall only be used in early IMS implementations as described in 3GPP TS 23.221 [3].

The FQDN, or host name as defined by RFC 1123 [8], is represented as character-labels with dots as delimiters.

EXAMPLE:	pcscf.operator.com

## 5.10	/<X>/Timer_T1

The Timer_T1 leaf defines the SIP timer T1 – the RTT estimate.

- Occurrence: One

- Format: int

- Access Types: Get, Replace

- Values: <The round trip time>

The Timer_T1 leaf is an estimate for the round trip time in the system (UE – P-CSCF). The timer value shall be given in milliseconds. The default value is recommended in 3GPP TS 24.229 [5]. The Timer_T1 is a 32 bits unsigned integer.

EXAMPLE:	2000 (milliseconds)

## 5.11	/<X>/Timer_T2

The Timer_T2 leaf defines the SIP timer T2 – the maximum retransmit interval for non-INVITE requests and INVITE responses.

- Occurrence: One

- Format: int

- Access Types: Get, Replace

- Values: < The maximum retransmit interval for non-INVITE requests and INVITE responses>

The Timer_T2 leaf is an estimate for the maximum retransmit interval for non-INVITE requests and INVITE responses. The timer value shall be given in milliseconds. The default value is recommended in 3GPP TS 24.229 [5]. The Timer_T2 is a 32 bits unsigned integer.

EXAMPLE:	16000 (milliseconds)

## 5.12	/<X>/Timer_T4

The Timer_T4 leaf defines the SIP timer T4 – the maximum duration a message will remain in the network.

- Occurrence: One

- Format: chr

- Access Types: Get, Replace

- Values: <The maximum duration a message will remain in the network>

The Timer_T4 leaf is an estimate for the maximum duration a message will remain in the network. The timer value shall be given in milliseconds. The default value is recommended in 3GPP TS 24.229 [5]. The Timer_T4 is a 32 bits unsigned integer.

EXAMPLE:	17000 (milliseconds)

## 5.13	/<X>/Private_user_identity

The Private_user_identity leaf represents the private user identity.

- Occurrence: One

- Format: chr

- Access Types: Get

- Values: <A private user identity>

NOTE:	The Private_user_identity leaf value is populated by the UE using the procedures to obtain the private user identity specified in 3GPP TS 24.229 [5].

The format of the private user identity is defined by 3GPP TS 23.003 [2].

EXAMPLE:	234150999999999@ims.mnc015.mcc234.3gppnetwork.org

## 5.14	/<X>/Public_user_identity_List/

The Public_user_identity_List interior node is used to allow a reference to a list of public user identities.

- Occurrence: One

- Format: node

- Access Types: Get

- Values: N/A

## 5.15	/<X>/Public_user_identity_List/<X>

This run-time node acts as a placeholder for one or more public user identities.

- Occurrence: OneOrMore

- Format: node

- Access Types: Get

- Values: N/A

## 5.16	/<X>/Public_user_identity_List/<X>/Public_user_identity

The Public_user_identity leaf represents one or more public user identities.

- Occurrence: One

- Format: chr

- Access Types: Get

- Values: <A public user identity>

NOTE:	The Public_user_identity leaf value is populated by the UE using the procedures to obtain the public user identity specified in 3GPP TS 24.229 [5].

The temporary public user identity if derived is populated and stored as the topmost element in the Public_user_identity_List as specified in 3GPP TS 24.229 [5].

The format of the public user identity is defined by 3GPP TS 23.003 [2].

EXAMPLE:	sip: 234150999999999@ims.mnc015.mcc234.3gppnetwork.org

## 5.17	/<X>/Home_network_domain_name

The Home_network_domain_name leaf indicates the operator’s home network domain.

- Occurrence: One

- Format: chr

- Access Types: Get

- Values: <The home network domain name>

NOTE:	The Home_network_domain_name leaf value is populated by the UE using the procedures to obtain the home network domain name specified in 3GPP TS 24.229 [5].

The format of the home network domain name is defined by 3GPP TS 23.003 [2].

EXAMPLE:	ims.mnc015.mcc234.3gppnetwork.org

## 5.18	/<X>/Ext/

The Ext is an interior node for where the vendor specific information about the 3GPP-IMS MO is being placed (vendor meaning application vendor, device vendor etc.). Usually the vendor extension is identified by vendor specific name under the ext node. The tree structure under the vendor identified is not defined and can therefore include one or more un-standardized sub-trees.

- Occurrence: ZeroOrOne

- Format: node

- Access Types: Get

- Values: N/A

Annex A (informative):
Management Object DDF

This DDF is the standardized minimal set. A vendor can define it’s own DDF for the complete device. This DDF can include more features than this minimal standardized version.

<?xml version="1.0" encoding="UTF-8"?>

<!DOCTYPE MgmtTree PUBLIC "-//OMA//DTD SYNCML-DMDDF 1.2//EN"

http://www.openmobilealliance.org/tech/DTD/OMA-SyncML-DMDDF-1_2.dtd>

<MgmtTree>

<VerDTD>1.2</VerDTD>

<Man>--The device manufacturer--</Man>

<Mod>--The device model--</Mod>

<Node>

<NodeName>3GPP_IMS</NodeName>

<DFProperties>

<AccessType>

<Get/>

</AccessType>

<Description>3GPP IMS settings</Description>

<DFFormat>

<node/>

</DFFormat>

<Occurrence>

<OneOrMore/>

</Occurrence>

<Scope>

<Permanent/>

</Scope>

<DFTitle>The 3GPP IMS Management Object.</DFTitle>

<DFType>

<DDFName/>

</DFType>

</DFProperties>

<Node>

<NodeName>AppID</NodeName>

<DFProperties>

<AccessType>

<Get/>

</AccessType>

<DFFormat>

<chr/>

</DFFormat>

<Occurrence>

<One/>

</Occurrence>

<Scope>

<Permanent/>

</Scope>

<DFTitle>The Application ID.</DFTitle>

<DFType>

<MIME>text/plain</MIME>

</DFType>

</DFProperties>

</Node>

<Node>

<NodeName>Name</NodeName>

<DFProperties>

<AccessType>

<Get/>

</AccessType>

<DFFormat>

<chr/>

</DFFormat>

<Occurrence>

<ZeroOrOne/>

</Occurrence>

<Scope>

<Dynamic/>

</Scope>

<DFTitle>User displayable name for the node.</DFTitle>

<DFType>

<MIME>text/plain</MIME>

</DFType>

</DFProperties>

</Node>

<Node>

<NodeName>ConRefs</NodeName>

<!-- The ConRefs node starts here. -->

<DFProperties>

<AccessType>

<Get/>

</AccessType>

<DFFormat>

<node/>

</DFFormat>

<Occurrence>

<One/>

</Occurrence>

<Scope>

<Permanent/>

</Scope>

<DFTitle>A collection of network access point objects.</DFTitle>

<DFType>

<DDFName/>

</DFType>

</DFProperties>

<Node>

<NodeName/>

<DFProperties>

<AccessType>

<Get/>

</AccessType>

<DFFormat>

<node/>

</DFFormat>

<Occurrence>

<OneOrMore/>

</Occurrence>

<Scope>

<Dynamic/>

</Scope>

<DFTitle>The "name" node for a network access point object.</DFTitle>

<DFType>

<DDFName/>

</DFType>

</DFProperties>

<Node>

<NodeName>ConRef</NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<chr/>

</DFFormat>

<Occurrence>

<One/>

</Occurrence>

<Scope>

<Permanent/>

</Scope>

<DFTitle>The ConRef (network access point object).</DFTitle>

<DFType>

<MIME>text/plain</MIME>

</DFType>

</DFProperties>

</Node>

</Node>

</Node>

<Node>

<NodeName>PDP_ContextOperPref</NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<bool/>

</DFFormat>

<Occurrence>

<One/>

</Occurrence>

<Scope>

<Permanent/>

</Scope>

<DFTitle>Indication of the operator’s preference for a dedicated PDP context for IMS signalling.</DFTitle>

<DFType>

<MIME>text/plain</MIME>

</DFType>

</DFProperties>

</Node>

<Node>

<NodeName>P-CSCF_Address</NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<chr/>

</DFFormat>

<Occurrence>

<ZeroOrOne/>

</Occurrence>

<Scope>

<Dynamic/>

</Scope>

<DFTitle>The address of the P-CSCF.</DFTitle>

<DFType>

<MIME>text/plain</MIME>

</DFType>

</DFProperties>

</Node>

<Node>

<NodeName>Timer_T1</NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<int/>

</DFFormat>

<Occurrence>

<One/>

</Occurrence>

<Scope>

<Permanent/>

</Scope>

<DFTitle>RFC 3261, timer T1.</DFTitle>

<DFType>

<MIME>text/plain</MIME>

</DFType>

</DFProperties>

</Node>

<Node>

<NodeName>Timer_T2</NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<int/>

</DFFormat>

<Occurrence>

<One/>

</Occurrence>

<Scope>

<Permanent/>

</Scope>

<DFTitle>RFC 3261, timer T2.</DFTitle>

<DFType>

<MIME>text/plain</MIME>

</DFType>

</DFProperties>

</Node>

<Node>

<NodeName>Timer_T4</NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<int/>

</DFFormat>

<Occurrence>

<One/>

</Occurrence>

<Scope>

<Permanent/>

</Scope>

<DFTitle>RFC 3261, timer T4.</DFTitle>

<DFType>

<MIME>text/plain</MIME>

</DFType>

</DFProperties>

</Node>

<Node>

<NodeName>Private_user_identity</NodeName>

<DFProperties>

<AccessType>

<Get/>

</AccessType>

<DFFormat>

<chr/>

</DFFormat>

<Occurrence>

<One/>

</Occurrence>

<Scope>

<Permanent/>

</Scope>

<DFTitle>The private user identity.</DFTitle>

<DFType>

<MIME>text/plain</MIME>

</DFType>

</DFProperties>

</Node>

<Node>

<NodeName>Public_user_identity_List</NodeName>

<!-- The Public_user_identity_List node starts here. -->

<DFProperties>

<AccessType>

<Get/>

</AccessType>

<DFFormat>

<node/>

</DFFormat>

<Occurrence>

<One/>

</Occurrence>

<Scope>

<Permanent/>

</Scope>

<DFTitle>A collection of public user identity objects.</DFTitle>

<DFType>

<DDFName/>

</DFType>

</DFProperties>

<Node>

<NodeName/>

<DFProperties>

<AccessType>

<Get/>

</AccessType>

<DFFormat>

<node/>

</DFFormat>

<Occurrence>

<OneOrMore/>

</Occurrence>

<Scope>

<Dynamic/>

</Scope>

<DFTitle>The "name" node for a public user identity object.</DFTitle>

<DFType>

<DDFName/>

</DFType>

</DFProperties>

<Node>

<NodeName>Public_user_identity</NodeName>

<DFProperties>

<AccessType>

<Get/>

</AccessType>

<DFFormat>

<chr/>

</DFFormat>

<Occurrence>

<One/>

</Occurrence>

<Scope>

<Permanent/>

</Scope>

<DFTitle>The public user identity.</DFTitle>

<DFType>

<MIME>text/plain</MIME>

</DFType>

</DFProperties>

</Node>

</Node>

</Node>

<Node>

<NodeName>Home_network_domain_name</NodeName>

<DFProperties>

<AccessType>

<Get/>

</AccessType>

<DFFormat>

<chr/>

</DFFormat>

<Occurrence>

<One/>

</Occurrence>

<Scope>

<Permanent/>

</Scope>

<DFTitle>The home network domain name.</DFTitle>

<DFType>

<MIME>text/plain</MIME>

</DFType>

</DFProperties>

</Node>

<Node>

<NodeName>Ext</NodeName>

<!-- The Extension node starts here. -->

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<node/>

</DFFormat>

<Occurrence>

<ZeroOrOne/>

</Occurrence>

<Scope>

<Dynamic/>

</Scope>

<DFTitle>A collection of all Extension objects.</DFTitle>

<DFType>

<DDFName/>

</DFType>

</DFProperties>

</Node>

</Node>

</MgmtTree>

Annex B (informative):
3GPP IMS Client Provisioning Application Characteristics (CP AC)

## B.1	General

This annex defines the 3GPP IMS Client Provisioning Application Characteristics (CP AC). The definition is based on the Enabler Release Definition OMA-WAP-ProvCont-V1_1-20050428-C [13].

If the initial IMS parameters cannot be provisioned as specified in OMA-ERELD-DM-V1_2-20050607-C [12], initial IMS parameters can be provisioned using OMA CP.

If a client is provisioned with 3GPP IMS related parameters by both OMA DM and OMA CP, the client will use the 3GPP IMS related parameters provisioned by OMA DM.

## B.2	Definition of the 3GPP IMS Client Provisioning Application Characteristics

IDENTIFYING INFORMATION

#######################

APPID: 3GPP_IMS.

APPID type: OMNA.

Owner: 3GPP CT1 Working Group.

Contact: 3GPP TSG CT WG1.

Registration version: 1.0.

Registration timestamp: 2005-10-01.

Application description: IP Multimedia Subsystem.

Application reference:

IMS specifications, 3GPP TS 24.229, 3GPP TS  23.221 and 3GPP TS 23.228.

URL:http://ftp.3gpp.org/.

WELL-KNOWN PARAMETERS

#####################

Characteristic/name: APPLICATION/APPID.

Status: Required.

Occurs: 1/1.

Default value: None.

Used values: N/A.

Interpretation: The Application ID - this is the same as for the MO DDF.

-------

Characteristic/name: APPLICATION/NAME.

Status: Required.

Occurs: 0/1.

Default value: ap2001.

Used values: ap2001.

Interpretation: User displayable name for the application.

-------

Characteristic/name: APPLICATION/PROVIDER-ID.

Status: Required.

Occurs: 0/1.

Default value: None.

Used values: N/A.

Interpretation: An identifier for the IMS service provider that provides the client provisioning. This is used to distinguish between settings for different IMS service providers within a client.

-------

Characteristic/name: APPLICATION/APPREF.

Status: Required.

Occurs: 0/1.

Default value: None.

Used values: N/A.

Interpretation:

The APPREF parameter defines the reference identity of the IMS APPLICATION characteristic. The APPREF parameter value is unique in the scope of the provisioning document. The TO-APPREF parameter included in other

APPLICATION characteristic can be used for referring to the IMS APPLICATION characteristic.

-------

Characteristic/name: APPLICATION/TO-NAPID.

Status: Required.

Occurs: 1/1.

Default value: None.

Used values: N/A.

Interpretation: The reference to the connectivity characteristics used for IMS.

-------

APPLICATION-SPECIFIC PARAMETERS

###############################

Characteristic/name: APPLICATION/PDP_CONTEXTOPERPREF.

Status: Required.

Occurs: 1/1.

Default value: None.

Used values: 0 and 1.

Interpretation: Indication of the operator’s preference for a dedicated PDP context for IMS signalling.

-------

Characteristic/name: APPLICATION/P-CSCF_ADDRESS.

Status: Optional.

Occurs: 0/1.

Default value: 0.

Used values: N/A.

Interpretation: The address of the P-CSCF in FQDN format or an IPv4 address.

-------

Characteristic/name: APPLICATION/TIMER_T1.

Status: Required.

Occurs: 0/1.

Default value: N/A.

Used values: Integer.

Interpretation: RFC 3261, timer T1.

-------

Characteristic/name: APPLICATION/TIMER_T2.

Status: Required.

Occurs: 0/1.

Default value: N/A.

Used values: Integer.

Interpretation: RFC 3261, timer T2.

-------

Characteristic/parameter: APPLICATION/TIMER_T4.

Status: Required.

Occurs: 0/1.

Default value: N/A.

Used values: Integer.

Interpretation: RFC 3261, timer T4.

-------

PARAMETER VALUES

################

Characteristic/name/parameter: APPLICATION/PDP_CONTEXTOPERPREF/0.

Status: Optional.

Interpretation: Indicates that the operator has no preference for a dedicated PDP context for SIP signalling.

-------

Characteristic/name/parameter: APPLICATION/PDP_CONTEXTOPERPREF/1.

Status: Optional.

Interpretation: Indicates that the operator has preference for a dedicated PDP context for SIP signalling.

-------

###END###

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New | WG doc |
| 2004-10 |  |  |  |  | Version 0.0.1: Preliminary proposal |  | 0.0.1 |  |
| 2004-11 |  |  |  |  | Version 0.0.2: Version after CN1 #36 | 0.0.1 | 0.0.2 |  |
| 2004-12 |  |  |  |  | Version 1.0.0: Version after CN1#36 and editorial corrections | 0.0.2 | 1.0.0 |  |
| 2005-02 |  |  |  |  | Version 1.1.0: Version after CN1#37 and editorial corrections | 1.0.0 | 1.1.0 | N1-050330 N1-050393 |
| 2005-02 | TSG-27 | NP-050066 |  |  | Version 2.0.0 created by MCC | 1.1.0 | 2.0.0 |  |
| 2005-03 |  |  |  |  | Version 2.0.0. approved in TSG-27. V6.0.0 created. | 2.0.0 | 6.0.0 |  |
| 2005-06 | TSG-28 | CP-050060 | 1 | 1 | Corrections to TS 24.167 due to comments from OMA DM | 6.0.0 | 6.1.0 | C1-050704 |
| 2005-06 | TSG-28 | CP-050060 | 2 | 1 | Miscellaneous corrections | 6.0.0 | 6.1.0 | C1-050705 |
| 2005-06 | TSG-28 | CP-050060 | 3 | 1 | Removal of APN from the IMS MO | 6.0.0 | 6.1.0 | C1-050706 |
| 2005-09 | TSG-29 | CP-050359 | 6 | - | Corrections for the 3GPP IMS Management Object | 6.1.0 | 6.2.0 | C1-050978 |
| 2005-09 | TSG-29 | CP-050359 | 7 | 3 | IMS MO and ISIM usage clarification | 6.1.0 | 6.2.0 | C1-051197 |
| 2005-09 | TSG-29 | CP-050359 | 9 | 1 | P-CSCF address used in early IMS implementations | 6.1.0 | 6.2.0 | C1-051090 |
| 2005-09 | TSG-29 | CP-050360 | 10 | 2 | Adding Client Provisioning Application Characteristics to IMS MO Rel-6 | 6.1.0 | 6.2.0 | C1-051091 |
| 2005-12 | TSG-30 | CP-050544 | 11 |  | AppID for the 3GPP_IMS MO determined by OMA | 6.2.0 | 6.3.0 | C1-050544 |
| 2006-06 | TSG-32 | CP-060266 | 15 |  | Corrections to client provisioning for the 3GPP IMS MO | 6.3.0 | 6.4.0 | C1-060627 |
| 2006-06 | TSG-32 | CP-060266 | 16 |  | Value range for int-parameters | 6.3.0 | 6.4.0 | C1-060628 |