3GPP TR 24.930 V0.3.0 (2006-09)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Core Network;

Signalling flows for the session setup in the IM CN subsystem based on Session Initiation Protocol (SIP)

and Session Description Protocol (SDP);

Stage 3

(Release 7)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Session Setup, SIP, SDP

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2004, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

Contents

Foreword	6

Introduction	6

1	Scope	7

2	References	7

3	Definitions, symbols and abbreviations	7

3.1	Definitions	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Methodology	8

4.1	General	8

4.2	Key required to interpret signalling flows	8

5	Signalling flows for session initiation	9

5.1	Establishing a session when UE#1 and UE#2 need to reserve resources	9

5.1.1	Introduction	9

5.1.2	Signalling Flow	9

5.2	Establishing a session when UE#1 needs to reserve resources while UE#2 has resources already available	23

5.2.1	Introduction	23

5.2.2	Signalling Flow	23

5.3	Establishing a session when UE#1 has resources availabe while UE#2 needs to perform resource reservation	37

5.3.1	Introduction	37

5.3.2	Signalling Flow (with SDP answer in 200 OK for INVITE)	37

5.3.3	Signalling Flow (with SDP answer in reliable 180 Ringing)	43

5.4	Establishing a session when UE#1 need to reserve resources and UE#2 is non-IMS	52

5.4.1	Introduction	52

5.4.2	Signalling Flow	52

5.5	Establishing a session when UE#1 is non-IMS and UE#2 needs to reserve resources	60

5.5.1	Introduction	60

5.5.2	Signalling Flow	60

5.6	Establishing a session when UE#1 and UE#2 do not need to reserve resources and do not use preconditions.	64

5.6.1	Introduction	64

5.6.2	Signalling Flow	64

Annex A (normative):	69

Annex <X>: Change history	70

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

This clause is optional. If it exists, it is always the second unnumbered clause.

# 1	Scope

The present document gives examples of the session setup in the IM CN subsystem based on SIP and SDP.

These signalling flows provide detailed signalling flows, which expand on the overview information flows provided in 3GPP TS 23.228 [2]. The flows focus on a basic session setup, i.e. no flows will be provided for topology hiding, for sessions with IBCF involved or for sessions having certain features.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 24.229: "IP Multimedia Call Control Protocol based on SIP and SDP; Stage 3"..

[2]	3GPP TS 24.228 Release 5: "Signalling flows for the IP multimedia call control based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP)  - Stage 3".

[3]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[4]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[5]	IETF RFC 3312: "Integration of Resource Management and Session Initiation Protocol (SIP)".

[6]	IETF RFC 3262: "Reliability of Provisional Responses in Session Initiation Protocol (SIP)".

[7]	IETF RFC 3311: "The Session Initiation Protocol (SIP) UPDATE Method".

[8]	IETF RFC 3264: "An Offer/Answer Model with Session Description Protocol (SDP)".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the [following] terms and definitions [given in ... and the following] apply.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

## 3.3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

AMR	Adaptive Multi-Rate

AS	Application Server

CN	Core Network

CSCF	Call Session Control Function

DSL	Digital Subscriber Line

FQDN	Fully Qualified Domain Name

HSS	Home Subscriber Server

HTTP	Hyper Text Transfer Protocol

I-CSCF	Interrogating CSCF

IM	IP Multimedia

IMS	IP Multimedia CN subsystem

IP	Internet Protocol

IP-CAN	IP-Connectivity Access Network

MGCF	Media Gateway Control Function

MRFC	Multimedia Resource Function Controller

MRFP	Multimedia Resource Function Processor

NGN	Next Generation Network

SDP	Session Description Protocol

SBLP	Service Based Local Policy

P-CSCF	Proxy CSCF

PSI	Public Service Identity

S-CSCF	Serving CSCF

SDP	Session Description Protocol

SIP	Session Initiation Protocol

UE	User Equipment

# 4	Methodology

## 4.1	General

The signalling flows provided in this annex follow the methodology developed in 3GPP TS 24.228 [2]. The following additional considerations apply:

a)	3GPP TS 24.228 [2] shows separate signalling flows with no configuration hiding between networks, and with configuration hiding between networks. Separate signalling flows are not show in the present document;

b)	3GPP TS 24.228 [2] breaks down the functionality of the various CSCFs. The functionality of the S-CSCF and I-CSCF is not relevant for the session setup procedure. Therefore S-CSCFs and I-CSCFs are collapsed into a single entity labelled "Intermediate IM CN subsystem entities".

## 4.2	Key required to interpret signalling flows

The key to interpret signalling flows specified in 3GPP TS 24.228 [2] subclauses 4.1 and 4.2 applies with the additions specified below.

-

Editor's note: Need to include here any specify concepts associated with the flows in this document.

Each signalling flow table contains descriptions for headers where the content of the header is new to that signalling flow, as is already performed in 3GPP TS 24.228 [2].

However, 3GPP TS 24.228 [3] includes extensive descriptions for the contents of various headers following each of the tables representing the contents of the signalling flows. Where the operation of the header is identical to that shown in 3GPP TS 24.228 [2], then such text is not reproduced in the present document.

Additional text may also be found on the contents of headers within 3GPP TS 24.228 [2] in addition to the material shown in the present document.

In order to differentiate between messages for SIP and media, the notation in figure 4.1-1 is used.

Figure 4.1-1: Signalling flow notation

# 5	Signalling flows for session initiation

# 5.1	Establishing a session when UE#1 and UE#2 need to reserve resources

### 5.1.1	Introduction

The following flow shows the establishment of a session where UE#1 and UE#2 need to reserve local resources.

It is assumed that both the originating UE and terminating UE are using a dedicated IP-CAN bearer for SIP signalling and dedicated  IP-CAN bearer for media.

The box intermediate IM CN subsystem nodes stands for the combination of I-CSCF/S-CSCF on the originating and on the terminating side. Routing of messages between those nodes is not described in the flow below.

### 5.1.2	Signalling Flow

Figure 5.1-1: IMS session setup, resource reservation on both sides

The details of the signalling flows are as follows:

1.	INVITE request (UE#1 to P-CSCF#1) see example in table 5.1-1

For this example, it is assumed that UE#1 is willing to establish a multimedia session comprising a video stream and an audio stream. The video stream supports two codecs, either H.263 or MPEG-4 Visual. The audio stream supports the AMR codec.

UE#1indicates that it supports precondition and it indicates that it supports reliable provisional responses. However, it does not use the "Require” header for these capabilities.

UE#1 does not have available the resources that are necessary to transport the media.

For this example it is assumed, that signalling encryption was negotiated between UE and P-CSCF in the security mode set-up procedure during the last successful authentication. This option will only be shown in this example.

Table 5.1-1: INVITE (UE#1 to P-CSCF)

INVITE tel:+1-212-555-2222 SIP/2.0

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 70

Route: <sip:pcscf1.visited1.net:7531;lr;comp=sigcomp>, <sip:scscf1.home1.net;lr>

P-Preferred-Identity: "John Doe" <sip:user1_public1@home1.net>

P-Access-Network-Info: 3GPP-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11

Privacy: none

From: <sip:user1_public1@home1.net>;tag=171828

To: <tel:+1-212-555-2222>

Call-ID: cb03a0s09a2sdfglkj490333

Cseq: 127 INVITE

Require: sec-agree

Supported: precondition, 100rel

Proxy-Require: sec-agree

Security-Verify: ipsec-3gpp; q=0.1; alg=hmac-sha-1-96; ealg=aes-cbc; spi-c=98765432; spi-s=87654321; port-c=8642; port-s=7531

Contact: <sip:[5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp>

Allow: INVITE, ACK, CANCEL, BYE, PRACK, UPDATE, REFER, MESSAGE

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 2987933615 2987933615 IN IP6 5555::aaa:bbb:ccc:ddd

s=-

c=IN IP6 5555::aaa:bbb:ccc:ddd

t=0 0

m=video 3400 RTP/AVP 98 99

b=AS:75

a=curr:qos local none

a=curr:qos remote none

a=des:qos mandatory local sendrecv

a=des:qos none remote sendrecv

a=inactive

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

a=rtpmap:99 MP4V-ES

m=audio 3456 RTP/AVP 97 96

b=AS:25.4

a=curr:qos local none

a=curr:qos remote none

a=des:qos mandatory local sendrecv

a=des:qos none remote sendrecv

a=inactive

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; maxframes=2

a=rtpmap:96 telephone-event

Supported:	The UE indicates support for the “precondition” mechanism and the support for reliable provisional responses

SDP	The SDP contains a set of codecs supported by UE#1 and desired by the user at UE#1 for this session. As the local preconditions are not fulfilled, both media are set to "inactive".

Security-Verify:		The Security-Verify contains the content of the Security-Server header as received during last successful authentication. It indicates that integrity protection and encryption are in use for this session.

2.	100 (Trying) response (P-CSCF#1 to UE#1)

The P-CSCF responds to the INVITE request with a 100 (Trying) provisional.

- INVITE request (P-CSCF#1 to S-CSCF#1) -  see example in table 5.1-2

Table 5.1-2: INVITE (P-CSCF#1 to S-CSCF#1)

INVITE tel:+1-212-555-2222 SIP/2.0

Via: SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 69

Route: <sip:scscf1.home1.net;lr>

Record-Route: <sip:pcscf1.visited1.net;lr>

P-Asserted-Identity: "John Doe" <sip:user1_public1@home1.net>

P-Access-Network-Info:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024"

Privacy:

From:

To:

Call-ID:

Cseq:

Supported:

Contact:

Allow:

Content-Type:

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

4.	100 (Trying) response (S-CSCF#1 to P-CSCF#1)

The S-CSCF responds to the INVITE request with a 100 (Trying) provisional.

5.	INVITE request (S-CSCF#2 to P-CSCF#2) see example in table 5.1-3

Table 5.1-3: INVITE (S-CSCF#2 to P-CSCF#2)

INVITE sip:[5555::eee:fff:aaa:bbb]:8805;comp=sigcomp SIP/2.0

Via: SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP icscf2_s.home2.net;branch=z9hG4bK871y12.1, SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 66

Route: <sip:pcscf2.visited2.net;lr>

Record-Route: <sip:scscf2.home2.net;lr>, <sip:scscf1.home1.net;lr>, <sip:pcscf1.visited1.net;lr>

P-Asserted-Identity:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024"

Privacy:

From:

To:

Call-ID:

Cseq:

Require:

Supported:

Contact:

Allow:

P-Called-Party-ID: <sip:user2_public1@home2.net>

Content-Type:

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

a=

6.	100 (Trying) response (P-CSCF#2 to S-CSCF#2)

The P-CSCF#2 responds to the INVITE request with a 100 (Trying) provisional.

7.	 INVITE request (P-CSCF#2 to UE #2) - see example in table 5.1-4

P-CSCF#2 forwards the INVITE request to UE#2.

Table 5.1-4: INVITE (P-CSCF#2 to UE#2

INVITE sip:user2_public1@home2.net SIP/2.0

Via: SIP/2.0/UDP pcscf2.visited2.net:5088;comp=sigcomp;branch=z9hG4bK361k21.1, SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP icscf2_s.home2.net;branch=z9hG4bK871y12.1, SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 67

Route: <sip:scscf2.home2.net;lr>

Record-Route: <sip:pcscf2.visited2.net:5088;lr;comp=sigcomp>, <sip:scscf2.home2.net;lr>, <sip:scscf1.home1.net;lr>, <sip:pcscf1.visited1.net;lr>

P-Asserted-Identity:

Privacy:

P-Media-Authorization: 0020000100100101706466312e686f6d65312e6e6574000c02013331533134363231

From:

To:

Call-ID:

Cseq:

Require:

Supported:

Contact:

Allow:

Content-Type:

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

8.	100 (Trying) response (UE#2 to P-CSCF)

The UE responds to the INVITE request with a 100 (Trying) provisional.

9.	183 (Session Progress) response (UE#2 to P-CSCF) - - see example in table 5.1-5

UE#2 determines the complete set of codecs that it is capable of supporting for this session. It determines the intersection with those appearing in the SDP in the INVITE request. UE#2 makes the final codec selection and chooses H.263 and AMR.

UE#2 responds with a 183 Session Progress response containing SDP back to the originator. This response is sent to P-CSCF. UE#2 uses a conf line in the SDP to request a confirmation from UE#1 when the local resources are available at UE#1.

Table 5.1-5: 183 Session Progress (UE#2 to P-CSCF#2)

SIP/2.0 183 Session Progress

Via: SIP/2.0/UDP pcscf2.visited2.net:5088;comp=sigcomp;branch=z9hG4bK361k21.1, SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP icscf2_s.home2.net;branch=z9hG4bK871y12.1, SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Record-Route: <sip:pcscf2.visited2.net:5088;lr;comp=sigcomp>, <sip:scscf2.home2.net;lr>, <sip:scscf1.home1.net;lr>, <sip:pcscf1.visited1.net;lr>

P-Access-Network-Info: 3GPP-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11

Privacy: none

From:

To: <tel:+1-212-555-2222>;tag=314159

Call-ID:

CSeq:

Require: 100rel, precondition

Contact: <sip:[5555::eee:fff:aaa:bbb]:8805;comp=sigcomp>

Allow: INVITE, ACK, CANCEL, BYE, PRACK, UPDATE, REFER, MESSAGE

RSeq: 9021

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 2987933623 2987933623 IN IP6 5555::eee:fff:aaa:bbb

s=-

c=IN IP6 5555::eee:fff:aaa:bbb

t=0 0

m=video 10001 RTP/AVP 98

b=AS:75

a=curr:qos local none

a=curr:qos remote none

a=des:qos mandatory local sendrecv

a=des:qos mandatory remote sendrecv

a=inactive

a=conf:qos remote sendrecv

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

m=audio 6544 RTP/AVP 97 96

b=AS:25.4

a=curr:qos local none

a=curr:qos remote none

a=des:qos mandatory local sendrecv

a=des:qos mandatory remote sendrecv

a=inactive

a=conf:qos remote sendrecv

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; maxframes=2

a=rtpmap:96 telephone-event

10.	Reserve IP-CAN bearer for media

The terminating UA sets up the bearer in accordance with the media description received SDP.

11.	Authorize QoS

P-CSCF authorizes the resources necessary for this session.

- 183 (session progress) response (P-CSCF#” to S-CSCF#2) – see example in table 5.1-6

Table 5.1-6: 183 Session Progress (P-CSCF#2 to S-CSCF#2)

SIP/2.0 183 Session Progress

Via: SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP icscf2_s.home2.net;branch=z9hG4bK871y12.1, SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7)

Record-Route:

P-Asserted-Identity: "John Smith" <sip:user2_public1@home2.net>

P-Access-Network-Info:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024"

Privacy:

From:

To:

Call-ID:

CSeq:

Require:

Contact:

Allow:

RSeq:

Content-Type:

Content-Length:

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

a=

a=

m=

13.	183 (session progress) response (S-CSCF#1 to P-CSCF#1) – see example in table 5.1-7

Table 5.1-7: 183 Session Progress (P-CSCF#2 to S-CSCF#2)

SIP/2.0 183 Session Progress

Via: SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7)

Record-Route:

P-Asserted-Identity:

P-Access-Network-Info:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024"

Privacy:

From:

To:

Call-ID:

CSeq:

Require:

Contact:

Allow:

RSeq:

Content-Type:

Content-Length:

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

a=

a=

m=

14.	Authorize QoS

P-CSCF authorizes the resources necessary for this session.

- 183 Session Progress (P-CSCF to UE) – see example in table 5.1-8

Table 5.1-8: 183 Session Progress (P-CSCF#1 to UE#1)

SIP/2.0 183 Session Progress

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Record-Route:

P-Asserted-Identity:

Privacy:

P-Media-Authorization: 0020000100100101706466322e76697369746564322e6e6574000c020139425633303732

From:

To:

Call-ID:

CSeq:

Require:

Contact:

Allow:

RSeq:

Content-Type:

Content-Length:

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

a=

16.	Reserve IP-CAN bearer for media

The originating UE sets up the bearer in accordance with the media description received SDP.

17 -24.	PRACK request / 200OK exchange

The PRACK request does not carry SDP as the final codec decision is already made as part of the initial offer/answer exchange.

25.	UPDATE  request (UE#1 to P-CSCF#1) - see example in table 5.1-9

UE#1 indicates that it can send and receive media as the necessary resources are available.

Table 5.1-9: UDPATE (UE#1 to P-CSCF#1)

UPDATE <sip:[5555::eee:fff:aaa:bbb]:8805;comp=sigcomp

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 70

Route: <sip:pcscf1.visited1.net:7531;lr;comp=sigcomp>, <sip:orig@scscf1.home1.net;lr>

From: <sip:user1_public1@home1.net>; tag=171828

To: <tel:+12125552222> tag=314159

Call-ID: cb03a0s09a2sdfglkj490333

Cseq: 129 UPDATE

Require: sec-agree

Proxy-Require: sec-agree

Security-Verify: ipsec-3gpp; q=0.1; alg=hmac-sha-1-96; ealg=aes-cbc; spi-c=98765432; spi-s=87654321; port-c=8642; port-s=7531

Contact: <sip:[5555::aaa:bbb:ccc:ddd]:1357;comp=sigcompContent-Type: application/sdp

Content-Length: (…)

v=0

o=- 2987933615 2987933615 IN IP6 5555::aaa:bbb:ccc:ddd

s=-

c=IN IP6 5555::aaa:bbb:ccc:ddd

t=0 0

m=video 3400 RTP/AVP 98

b=AS:75

a=curr:qos local sendrececv

a=curr:qos remote none

a=des:qos mandatory local sendrecv

a=des:qos mandatory remote sendrecv

a=sendrecv

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

m=audio 3456 RTP/AVP 97 96

b=AS:25.4

a=curr:qos local sendrecv

a=curr:qos remote none

a=des:qos mandatory local sendrecv

a=des:qos mandatory remote sendrecv

a=sendrecv

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; maxframes

- UPDATE  request (P-CSCF#1 to S-CSCF#1 ) - see example in table 5.1-10

Table 5.1-10: UDPATE (P-CSCF#1 to S-CSCF#1)

UPDATE sip:[5555::eee:fff:aaa:bbb]:8805;comp=sigcomp SIP/2.0

Via: SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 69

P-Access-Network-Info:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024"; ggsn=[5555::4b4:3c3:2d2:1e1]; pdp-sig=no; gcid=723084371; auth-token=43876559; flow-id=3

Route: <sip:scscf1.home1.net;lr>, <sip:scscf2.home2.net;lr>, <sip:pcscf2.visited2.net;lr>

From:

To:

Call-ID:

Cseq:

Content-Type:

Content-Length:

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

27.	UPDATE  request (S-CSCF#2 to P-CSCF#2 ) - see example in table 5.1-11

Table 5.1-11: UPDATE (S-CSCF#2 to P-CSCF#2)

UPDATE sip:[5555::eee:fff:aaa:bbb]:8805;comp=sigcomp SIP/2.0

Via: SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 67

Route: <sip:pcscf2.visited2.net;lr>

From:

To:

Call-ID:

Cseq:

Contact:

Content-Type:

Content-Length:

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

28.	UPDATE  request (P-CSCF#2 to UE#2 ) - see example in table 5.1-12

Table 5.1-12: UPDATE (S-CSCF#2 to P-CSCF#2)

UPDATE sip:[5555::eee:fff:aaa:bbb]:8805;comp=sigcomp SIP/2.0

Via: SIP/2.0/UDP pcscf2.visited2.net:5088;comp=sigcomp;branch=z9hG4bK361k21.1, SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 66

From:

To:

Call-ID:

Cseq:

Content-Type:

Content-Length:

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

29.	2000 (OK) response (UE#2 to P-CSCF#1)  - see example in table 5.1-13

UE acknowledges the UPDATE request  with a 200 OK response.

UE indicates that the local resources are available

Table 5.1-13: 200OK (UE to P-CSCF)

SIP/2.0 200 OK

Via: SIP/2.0/UDP pcscf2.visited2.net:5088;comp=sigcomp;branch=z9hG4bK361k21.1, SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP icscf2_s.home2.net;branch=z9hG4bK871y12.1, SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

P-Access-Network-Info: 3GPP-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11

From: <sip:user1_public1@home1.net>;tag=171828

To: <tel:+12125552222>;tag=314159

Call-ID: cb03a0s09a2sdfglkj490333

Cseq: 129 UPDATE

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 2987933615 2987933615 IN IP6 5555::eee:fff:aaa:bbb

s=-

c=IN IP6 5555::eee:fff:aaa:bbb

t=0 0

m=video 3400 RTP/AVP 98

b=AS:75

a=curr:qos local sendrececv

a=curr:qos remote sendrecv

a=des:qos mandatory local sendrecv

a=des:qos mandatory remote sendrecv

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

m=audio 3456 RTP/AVP 97 96

b=AS:25.4

a=curr:qos local sendrecv

a=curr:qos remote sendrecv

a=des:qos mandatory local sendrecv

a=des:qos mandatory remote sendrecv

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; maxframes

30.	200 (OK) response (P-CSCF#2 to S-CSCF#2 )  - see example in table 5.1-14

Table 5.1-14: 200OK (P-CSCF#2 to S-CSCF#2)

SIP/2.0 200 OK

Via: SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

P-Access-Network-Info:

From:

To:

Call-ID:

CSeq:

Content-Type:

Content-Length:

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

m=

b=

a=

a=

a=

a=

a=

a=

a=

31.	200 (OK) response (S-CSCF#1 to P-CSCF#21)  - see example in table 5.1-15

Table 5.1-15: 200OK (S-CSCF#1 to P-CSCF#1)

SIP/2.0 200 OK

Via: SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

From:

To:

Call-ID:

CSeq:

Content-Type:

Content-Length:

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

m=

b=

a=

a=

a=

a=

a=

a=

a=

32.	200 (OK) response (P-CSCF#1 to UE#1)  - see example in table 5.1-16

Table 5.1-16: 200OK (P-CSCF#1 to UE#1)

SIP/2.0 200 OK

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

From:

To:

Call-ID:

CSeq:

Content-Type:

Content-Length:

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

m=

b=

a=

a=

a=

a=

a=

a=

a=

33 -36 . 180 (Ringing) response

UE#2 indicates that it is ringing. The UE#2 does not use Require “100rel” as the 180 (Ringing) does not have a SDP and therefore need not to be sent reliable.

37 –40 .200 (OK) response

When the called party answers the UE sends a 200 OK final response to the INVITE request (6) to P-CSCF, and starts the media flow(s) for this session.

40-44 ACK request

The calling party  responds to the 200 (OK) response with an ACK request.

# 5.2	Establishing a session when UE#1 needs to reserve resources while UE#2 has resources already available

### 5.2.1	Introduction

The following flow shows the establishment of a session where UE#1 needs to reserve local resources (e.g. using a GRPS IP-CAN) while UE#2 does not need to perform resource reservation (e.g. connected via IWLAN IP-CAN).

The box intermediate IM CN subsystem nodes stands for the combination of I-CSCF/S-CSCF on the originating and on the terminating side. Routing of messages between those nodes is not described in the flow below.

### 5.2.2	Signalling Flow

Figure 5.1-1: IMS session setup, resource reservation on originating side only

The details of the signalling flows are as follows:

1.	INVITE request (UE#1 to P-CSCF#1) see example in table 5.2-1

For this example, it is assumed that UE#1 is willing to establish a multimedia session comprising a video stream and an audio stream. The video stream supports H.263 codec. The audio stream supports the AMR codec.

UE#1indicates that it supports precondition and it indicates that it supports reliable provisional responses. However, it does not use the "Require” header for these capabilities.

UE#1 does not have available the resources that are necessary to transport the media.

Table 5.2-1: INVITE (UE#1 to P-CSCF)

INVITE tel:+1-212-555-2222 SIP/2.0

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 70

Route: <sip:pcscf1.visited1.net:7531;lr;comp=sigcomp>, <sip:scscf1.home1.net;lr>

P-Preferred-Identity: "John Doe" <sip:user1_public1@home1.net>

P-Access-Network-Info: 3GPP-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11

Privacy: none

From: <sip:user1_public1@home1.net>;tag=171828

To: <tel:+1-212-555-2222>

Call-ID: cb03a0s09a2sdfglkj490333

Cseq: 127 INVITE

Require: sec-agree

Supported: precondition, 100rel

Proxy-Require: sec-agree

Supported: 100rel

Security-Verify: ipsec-3gpp; q=0.1; alg=hmac-sha-1-96; spi-c=98765432; spi-s=87654321; port-c=8642; port-s=7531

Contact: <sip:[5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp>

Allow: INVITE, ACK, CANCEL, BYE, PRACK, UPDATE, REFER, MESSAGE

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 2987933615 2987933615 IN IP6 5555::aaa:bbb:ccc:ddd

s=-

c=IN IP6 5555::aaa:bbb:ccc:ddd

t=0 0

m=video 3400 RTP/AVP 98

b=AS:75

a=curr:qos local none

a=curr:qos remote none

a=des:qos mandatory local sendrecv

a=des:qos none remote sendrecv

a=inactive

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

m=audio 3456 RTP/AVP 97 96

b=AS:25.4

a=curr:qos local none

a=curr:qos remote none

a=des:qos mandatory local sendrecv

a=des:qos none remote sendrecv

a=inactive

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; maxframes=2

a=rtpmap:96 telephone-event

Supported:	The UE indicates support for the “precondition” mechanism and the support for reliable provisional responses

SDP	The SDP contains a set of codecs supported by UE#1 and desired by the user at UE#1 for this session. As the local preconditions are not fulfilled, both media are set to "inactive".

2.	100 (Trying) response (P-CSCF#1 to UE#1)

The P-CSCF responds to the INVITE request with a 100 (Trying) provisional.

3.	INVITE request (P-CSCF#1 to S-CSCF#1) -  see example in table 5.2-2

Table 5.2-2: INVITE (P-CSCF#1 to S-CSCF#1)

INVITE tel:+1-212-555-2222 SIP/2.0

Via: SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 69

Route: <sip:scscf1.home1.net;lr>

Record-Route: <sip:pcscf1.visited1.net;lr>

P-Asserted-Identity: "John Doe" <sip:user1_public1@home1.net>

P-Access-Network-Info:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024"

Privacy:

From:

To:

Call-ID:

Cseq:

Require: precondition

Supported:

Contact:

Allow:

Content-Type:

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

4.	100 (Trying) response (S-CSCF#1 to P-CSCF#1)

The S-CSCF responds to the INVITE request with a 100 (Trying) provisional.

5.	INVITE request (S-CSCF#2 to P-CSCF#2) see example in table 5.2-3

Table 5.2-3: INVITE (S-CSCF#2 to P-CSCF#2)

INVITE sip:[5555::eee:fff:aaa:bbb]:8805;comp=sigcomp SIP/2.0

Via: SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP icscf2_s.home2.net;branch=z9hG4bK871y12.1, SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 66

Route: <sip:pcscf2.visited2.net;lr>

Record-Route: <sip:scscf2.home2.net;lr>, <sip:scscf1.home1.net;lr>, <sip:pcscf1.visited1.net;lr>

P-Asserted-Identity:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024"

Privacy:

From:

To:

Call-ID:

Cseq:

Require:

Supported:

Contact:

Allow:

P-Called-Party-ID: <sip:user2_public1@home2.net>

Content-Type:

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

a=

6.	100 (Trying) response (P-CSCF#2 to S-CSCF#2)

The P-CSCF#2 responds to the INVITE request with a 100 (Trying) provisional.

7.	 INVITE request (P-CSCF#2 to UE #2) - see example in table 5.2-4

P-CSCF#2 forwards the INVITE request to UE#2.

Table 5.2-4: INVITE (P-CSCF#2 to UE#2

INVITE sip:user2_public1@home2.net SIP/2.0

Via: SIP/2.0/UDP pcscf2.visited2.net:5088;comp=sigcomp;branch=z9hG4bK361k21.1, SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP icscf2_s.home2.net;branch=z9hG4bK871y12.1, SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 67

Route: <sip:scscf2.home2.net;lr>

Record-Route: <sip:pcscf2.visited2.net:5088;lr;comp=sigcomp>, <sip:scscf2.home2.net;lr>, <sip:scscf1.home1.net;lr>, <sip:pcscf1.visited1.net;lr>

P-Asserted-Identity:

Privacy:

From:

To:

Call-ID:

Cseq:

Require:

Supported:

Contact:

Allow:

Content-Type:

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

8.	100 (Trying) response (UE#2 to P-CSCF)

The UE responds to the INVITE request with a 100 (Trying) provisional.

9.	183 (Session Progress) response (UE#2 to P-CSCF) - - see example in table 5.2-5

UE#2 determines the complete set of codecs that it is capable of supporting for this session. It determines the intersection with those appearing in the SDP in the INVITE request. UE#2 supports both offered media streams

UE#2 responds with a 183 Session Progress response containing SDP back to the originator. This response is sent to P-CSCF. UE#2 uses a conf line in the SDP to request a confirmation from UE#1 when the local resources are available at UE#1.

UE#2 has all necessary resources available and indicates that in the SDP

Table 5.2-5: 183 Session Progress (UE#2 to P-CSCF#2)

SIP/2.0 183 Session Progress

Via: SIP/2.0/UDP pcscf2.visited2.net:5088;comp=sigcomp;branch=z9hG4bK361k21.1, SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP icscf2_s.home2.net;branch=z9hG4bK871y12.1, SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Record-Route: <sip:pcscf2.visited2.net:5088;lr;comp=sigcomp>, <sip:scscf2.home2.net;lr>, <sip:scscf1.home1.net;lr>, <sip:pcscf1.visited1.net;lr>

P-Access-Network-Info: IEEE-802.11a

Privacy: none

From:

To: <tel:+1-212-555-2222>;tag=314159

Call-ID:

CSeq:

Require: 100rel, precondition

Contact: <sip:[5555::eee:fff:aaa:bbb]:8805;comp=sigcomp>

Allow: INVITE, ACK, CANCEL, BYE, PRACK, UPDATE, REFER, MESSAGE

RSeq: 9021

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 2987933623 2987933623 IN IP6 5555::eee:fff:aaa:bbb

s=-

c=IN IP6 5555::eee:fff:aaa:bbb

t=0 0

m=video 10001 RTP/AVP 98

b=AS:75

a=curr:qos local sendrecv

a=curr:qos remote none

a=des:qos mandatory local sendrecv

a=des:qos mandatory remote sendrecv

a=inactive

a=conf:qos remote sendrecv

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

m=audio 6544 RTP/AVP 97 96

b=AS:25.4

a=curr:qos local sendrecv

a=curr:qos remote none

a=des:qos mandatory local sendrecv

a=des:qos mandatory remote sendrecv

a=inactive

a=conf:qos remote sendrecv

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; maxframes=2

a=rtpmap:96 telephone-event

10.	183 (session progress) response (P-CSCF#2 to S-CSCF#2) – see example in table 5.2-6

Table 5.2-6: 183 Session Progress (P-CSCF#2 to S-CSCF#2)

SIP/2.0 183 Session Progress

Via: SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP icscf2_s.home2.net;branch=z9hG4bK871y12.1, SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7)

Record-Route:

P-Asserted-Identity: "John Smith" <sip:user2_public1@home2.net>

P-Access-Network-Info:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024"

Privacy:

From:

To:

Call-ID:

CSeq:

Require:

Contact:

Allow:

RSeq:

Content-Type:

Content-Length:

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

a=

a=

11.	183 (session progress) response (S-CSCF#1 to P-CSCF#1) – see example in table 5.2-7

Table 5.2-7: 183 Session Progress (P-CSCF#2 to S-CSCF#2)

SIP/2.0 183 Session Progress

Via: SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7)

Record-Route:

P-Asserted-Identity:

P-Access-Network-Info:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024"

Privacy:

From:

To:

Call-ID:

CSeq:

Require:

Contact:

Allow:

RSeq:

Content-Type:

Content-Length:

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

a=

a=

12.	Authorize QoS

P-CSCF authorizes the resources necessary for this session.

13.	183 Session Progress (P-CSCF to UE) – see example in table 5.2-8

Table 5.2-8: 183 Session Progress (P-CSCF#1 to UE#1)

SIP/2.0 183 Session Progress

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Record-Route:

P-Asserted-Identity:

Privacy:

P-Media-Authorization: 0020000100100101706466322e76697369746564322e6e6574000c020139425633303732

From:

To:

Call-ID:

CSeq:

Require:

Contact:

Allow:

RSeq:

Content-Type:

Content-Length:

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

a=

14.	Reserve IP-CAN bearer for media

The originating UE sets up the bearer in accordance with the media description received SDP.

15 -22.	PRACK request / 200OK exchange

The PRACK request does not carry SDP as the final codec decision is already made as part of the initial offer/answer exchange.

23.	UPDATE  request (UE#1 to P-CSCF#1 ) - see example in table 5.2-9

UE#1 indicates that it can send and receive media as the necessary resources are available.

Table 5.2-9: UDPATE (UE#1 to P-CSCF#1)

UPDATE <sip:[5555::eee:fff:aaa:bbb]:8805;comp=sigcomp

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 70

Route: <sip:pcscf1.visited1.net:7531;lr;comp=sigcomp>, <sip:orig@scscf1.home1.net;lr>

From: <sip:user1_public1@home1.net>; tag=171828

To: <tel:+12125552222> tag=314159

Call-ID: cb03a0s09a2sdfglkj490333

Cseq: 129 UPDATE

Require: sec-agree

Proxy-Require: sec-agree

Security-Verify: ipsec-3gpp; q=0.1; alg=hmac-sha-1-96; spi=87654321; port1=7531

Contact: <sip:[5555::aaa:bbb:ccc:ddd]:1357;comp=sigcompContent-Type: application/sdp

Content-Length: (…)

v=0

o=- 2987933615 2987933615 IN IP6 5555::aaa:bbb:ccc:ddd

s=-

c=IN IP6 5555::aaa:bbb:ccc:ddd

t=0 0

m=video 3400 RTP/AVP 98

b=AS:75

a=curr:qos local sendrececv

a=curr:qos remote sendrecv

a=des:qos mandatory local sendrecv

a=des:qos mandatory remote sendrecv

a=sendrecv

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

m=audio 3456 RTP/AVP 97 96

b=AS:25.4

a=curr:qos local sendrecv

a=curr:qos remote none

a=des:qos mandatory local sendrecv

a=des:qos mandatory remote sendrecv

a=sendrecv

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; maxframes

24.	UPDATE  request (P-CSCF#1 to S-CSCF#1 ) - see example in table 5.2-10

Table 5.2-10: UDPATE (P-CSCF#1 to S-CSCF#1)

UPDATE sip:[5555::eee:fff:aaa:bbb]:8805;comp=sigcomp SIP/2.0

Via: SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 69

P-Access-Network-Info:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024"; ggsn=[5555::4b4:3c3:2d2:1e1]; pdp-sig=no; gcid=723084371; auth-token=43876559; flow-id=3

Route: <sip:scscf1.home1.net;lr>, <sip:scscf2.home2.net;lr>, <sip:pcscf2.visited2.net;lr>

From:

To:

Call-ID:

Cseq:

Content-Type:

Content-Length:

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

25.	UPDATE  request (S-CSCF#2 to P-CSCF#2 ) - see example in table 5.2-11

Table 5.2-11: UPDATE (S-CSCF#2 to P-CSCF#2)

UPDATE sip:[5555::eee:fff:aaa:bbb]:8805;comp=sigcomp SIP/2.0

Via: SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 67

Route: <sip:pcscf2.visited2.net;lr>

From:

To:

Call-ID:

Cseq:

Contact:

Content-Type:

Content-Length:

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

26.	UPDATE  request (P-CSCF#2 to UE#2 ) - see example in table 5.2-12

Table 5.2-12: UPDATE (S-CSCF#2 to P-CSCF#2)

UPDATE sip:[5555::eee:fff:aaa:bbb]:8805;comp=sigcomp SIP/2.0

Via: SIP/2.0/UDP pcscf2.visited2.net:5088;comp=sigcomp;branch=z9hG4bK361k21.1, SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 66

From:

To:

Call-ID:

Cseq:

Content-Type:

Content-Length:

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

27.	2000 (OK) response (UE#1 to P-CSCF#1)  - see example in table 5.2-13

UE acknowledges the UPDATE request  with a 200 OK response.

UE indicates that the local resources are available

Table 5.2-13: 200OK (UE to P-CSCF)

SIP/2.0 200 OK

Via: SIP/2.0/UDP pcscf2.visited2.net:5088;comp=sigcomp;branch=z9hG4bK361k21.1, SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP icscf2_s.home2.net;branch=z9hG4bK871y12.1, SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

P-Access-Network-Info: 3GPP-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11

From: <sip:user1_public1@home1.net>; tag=171828

To: <tel:+12125552222>;tag=314159

Call-ID: cb03a0s09a2sdfglkj490333

Cseq: 129 UPDATE

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 2987933615 2987933615 IN IP6 5555::eee:fff:aaa:bbb

s=-

c=IN IP6 5555:: eee:fff:aaa:bbb

t=0 0

m=video 3400 RTP/AVP 98

b=AS:75

a=curr:qos local sendrececv

a=curr:qos remote sendrecv

a=des:qos mandatory local sendrecv

a=des:qos mandatory remote sendrecv

a=sendrecv

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

m=audio 3456 RTP/AVP 97 96

b=AS:25.4

a=curr:qos local sendrecv

a=curr:qos remote sendrecv

a=des:qos mandatory local sendrecv

a=des:qos mandatory remote sendrecv

a=sendrecv

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; maxframes

28.	200 (OK) response (P-CSCF#2 to S-CSCF#2 )  - see example in table 5.2-14

Table 5.2-14: 200OK (P-CSCF#2 to S-CSCF#2)

SIP/2.0 200 OK

Via: SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

P-Access-Network-Info:

From:

To:

Call-ID:

CSeq:

Content-Type:

Content-Length:

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

29.	200 (OK) response (S-CSCF#1 to P-CSCF#21)  - see example in table 5.2-15

Table 5.2-15: 200OK (S-CSCF#1 to P-CSCF#1)

SIP/2.0 200 OK

Via: SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

From:

To:

Call-ID:

CSeq:

Content-Type:

Content-Length:

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

30.	200 (OK) response (P-CSCF#1 to UE#1)  - see example in table 5.2-16

Table 5.2-16: 200OK (P-CSCF#1 to UE#1)

SIP/2.0 200 OK

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

From:

To:

Call-ID:

CSeq:

Content-Type:

Content-Length:

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

31 -34 . 180 (Ringing) response

UE#2 indicates that it is ringing. The UE#2 does not use Require “100rel” as the 180 (Ringing) does not have a SDP and therefore need not to be sent reliable.

35 –38 .200 (OK) response

When the called party answers the UE sends a 200 OK final response to the INVITE request (6) to P-CSCF, and starts the media flow(s) for this session.

39-42 ACK request

The calling party  responds to the 200 (OK) response with an ACK request.

# 5.3	Establishing a session when UE#1 has resources availabe while UE#2 needs to perform resource reservation

### 5.3.1	Introduction

The following flows show the establishment of a session where UE# has all necessary local resources available (e.g. having an appropriate PDP context for the desired media available) while UE#2 has to perform resource reservation.

The box intermediate IM CN subsystem nodes stands for the combination of I-CSCF/S-CSCF on the originating and on the terminating side. Routing of messages between those nodes is not described in the flow below.

### 5.3.2	Signalling Flow (with SDP answer in 200 OK for INVITE)

Figure 5.3-1: IMS session setup, resource reservation only on terminating side

The details of the signalling flows are as follows:

1.	INVITE request (UE#1 to P-CSCF#1) see example in table 5.3-1

For this example, it is assumed that UE#1 is willing to establish a multimedia session comprising a video stream and an audio stream. The video stream supports H.263 codec. The audio stream supports the AMR codec.

UE#1indicates that it supports precondition and it indicates that it supports reliable provisional responses. However, it does not use the "Require” header for these capabilities.

UE#1 does have available the resources that are necessary to transport the media.

Table 5.3-1: INVITE (UE#1 to P-CSCF)

INVITE tel:+1-212-555-2222 SIP/2.0

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 70

Route: <sip:pcscf1.visited1.net:7531;lr;comp=sigcomp>, <sip:scscf1.home1.net;lr>

P-Preferred-Identity: "John Doe" <sip:user1_public1@home1.net>

P-Access-Network-Info: 3GPP-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11

Privacy: none

From: <sip:user1_public1@home1.net>;tag=171828

To: <tel:+1-212-555-2222>

Call-ID: cb03a0s09a2sdfglkj490333

Cseq: 127 INVITE

Require: sec-agree

Supported: precondition, 100rel

Proxy-Require: sec-agree

Security-Verify: ipsec-3gpp; q=0.1; alg=hmac-sha-1-96; spi-c=98765432; spi-s=87654321; port-c=8642; port-s=7531

Contact: <sip:[5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp>

Allow: INVITE, ACK, CANCEL, BYE, PRACK, UPDATE, REFER, MESSAGE

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 2987933615 2987933615 IN IP6 5555::aaa:bbb:ccc:ddd

s=-

c=IN IP6 5555::aaa:bbb:ccc:ddd

t=0 0

m=video 3400 RTP/AVP 98

b=AS:75

a=curr:qos local sendrecv

a=curr:qos remote none

a=des:qos mandatory local sendrecv

a=des:qos none remote sendrecv

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

m=audio 3456 RTP/AVP 97 96

b=AS:25.4

a=curr:qos local sendrecv

a=curr:qos remote none

a=des:qos mandatory local sendrecv

a=des:qos none remote sendrecv

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; maxframes=2

a=rtpmap:96 telephone-event

Supported:	The UE indicates support for the “precondition” mechanism and the support for reliable provisional responses

SDP	The SDP contains a set of codecs supported by UE#1 and desired by the user at UE#1 for this session.

2.	100 (Trying) response (P-CSCF#1 to UE#1)

The P-CSCF responds to the INVITE request with a 100 (Trying) provisional.

3.	INVITE request (P-CSCF#1 to S-CSCF#1) -  see example in table 5.3-2

Table 5.3-2: INVITE (P-CSCF#1 to S-CSCF#1)

INVITE tel:+1-212-555-2222 SIP/2.0

Via: SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 69

Route: <sip:scscf1.home1.net;lr>

Record-Route: <sip:pcscf1.visited1.net;lr>

P-Asserted-Identity: "John Doe" <sip:user1_public1@home1.net>

P-Access-Network-Info:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024"

Privacy:

From:

To:

Call-ID:

Cseq:

Supported:

Contact:

Allow:

Content-Type:

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

4.	100 (Trying) response (S-CSCF#1 to P-CSCF#1)

The S-CSCF responds to the INVITE request with a 100 (Trying) provisional.

5.	INVITE request (S-CSCF#2 to P-CSCF#2) see example in table 5.3-3

Table 5.3-3: INVITE (S-CSCF#2 to P-CSCF#2)

INVITE sip:[5555::eee:fff:aaa:bbb]:8805;comp=sigcomp SIP/2.0

Via: SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP icscf2_s.home2.net;branch=z9hG4bK871y12.1, SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 66

Route: <sip:pcscf2.visited2.net;lr>

Record-Route: <sip:scscf2.home2.net;lr>, <sip:scscf1.home1.net;lr>, <sip:pcscf1.visited1.net;lr>

P-Asserted-Identity:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024"

Privacy:

From:

To:

Call-ID:

Cseq:

Require:

Supported:

Contact:

Allow:

P-Called-Party-ID: <sip:user2_public1@home2.net>

Content-Type:

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

a=

6.	100 (Trying) response (P-CSCF#2 to S-CSCF#2)

The P-CSCF#2 responds to the INVITE request with a 100 (Trying) provisional.

7.	 INVITE request (P-CSCF#2 to UE #2) - see example in table 5.3-4

P-CSCF#2 forwards the INVITE request to UE#2.

Table 5.3-4: INVITE (P-CSCF#2 to UE#2

INVITE sip:user2_public1@home2.net SIP/2.0

Via: SIP/2.0/UDP pcscf2.visited2.net:5088;comp=sigcomp;branch=z9hG4bK361k21.1, SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP icscf2_s.home2.net;branch=z9hG4bK871y12.1, SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 67

Route: <sip:scscf2.home2.net;lr>

Record-Route: <sip:pcscf2.visited2.net:5088;lr;comp=sigcomp>, <sip:scscf2.home2.net;lr>, <sip:scscf1.home1.net;lr>, <sip:pcscf1.visited1.net;lr>

P-Asserted-Identity:

Privacy:

From:

To:

Call-ID:

Cseq:

Require:

Supported:

Contact:

Allow:

Content-Type:

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

8.	100 (Trying) response (UE#2 to P-CSCF)

The UE responds to the INVITE request with a 100 (Trying) provisional.

9.	Reserve IP-CAN bearer for media

The terminating UE sets up the bearer in accordance with the media description.

10. – 13. 180 (Ringing) response

UE#2 indicates that it is ringing. The UE#2 does not use Require “100rel” as the 180 (Ringing) does not have a SDP and therefore need not to be sent reliable.

- 200 (OK) response (UE#2 to P-CSCF#2)  - see example in table 5.3-5

UE indicates that the local resources are available

Table 5.3-5: 200OK (UE to P-CSCF)

SIP/2.0 200 OK

Via: SIP/2.0/UDP pcscf2.visited2.net:5088;comp=sigcomp;branch=z9hG4bK361k21.1, SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP icscf2_s.home2.net;branch=z9hG4bK871y12.1, SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

P-Access-Network-Info: 3GPP-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11

From:

To:

Call-ID:

Cseq:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 2987933615 2987933615 IN IP6 5555::eee:fff:aaa:bbb

s=-

c=IN IP6 5555:: eee:fff:aaa:bbb

t=0 0

m=video 3400 RTP/AVP 98

b=AS:75

a=curr:qos local sendrececv

a=curr:qos remote sendrecv

a=des:qos mandatory local sendrecv

a=des:qos mandatory remote sendrecv

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

m=audio 3456 RTP/AVP 97 96

b=AS:25.4

a=curr:qos local sendrecv

a=curr:qos remote sendrecv

a=des:qos mandatory local sendrecv

a=des:qos mandatory remote sendrecv

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; maxframes

- 200 (OK) response (P-CSCF#2 to S-CSCF#2)  - see example in table 5.3-6

Table 5.3-6: 200OK (P-CSCF#2 to S-CSCF#2)

SIP/2.0 200 OK

Via: SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP icscf2_s.home2.net;branch=z9hG4bK871y12.1, SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

P-Access-Network-Info: 3GPP-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11

From:

To:

Call-ID:

Cseq:

Content-Type: application/sdp

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

a=

- 200 (OK) response (S-CSCF#1 to P-CSCF#1)  - see example in table 5.3-7

Table 5.3-7: 200OK (S-CSCF#1 to P-CSCF#1)

SIP/2.0 200 OK

Via: pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

P-Access-Network-Info: 3GPP-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11

From:

To:

Call-ID:

Cseq:

Content-Type: application/sdp

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

a=

- 200 (OK) response (P-CSCF#1 to UE#1)  - see example in table 5.3-8

Table 5.3-8: 200OK (P-CSCF#1 to UE#1)

SIP/2.0 200 OK

Via: [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

P-Access-Network-Info: 3GPP-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11

From:

To:

Call-ID:

Cseq:

Content-Type: application/sdp

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

a=

18. –21. .	ACK request

The calling party  responds to the 200 (OK) response with an ACK request.

### 5.3.3	Signalling Flow (with SDP answer in reliable 180 Ringing)

In the call flow shown in subclause 5.3.2, the SDP answer is returned to UE#1 in the final 200 OK response. An alternative call flow is shown in this section where SDP answer is returned to UE#1 in a reliable 180 Ringing message.

Figure 5.3-2: IMS session setup, resource reservation only on terminating side

The details of the signalling flows are as follows:

1.	INVITE request (UE#1 to P-CSCF#1) see example in table 5.3-9

For this example, it is assumed that UE#1 is willing to establish a multimedia session comprising a video stream and an audio stream. The video stream supports H.263 codec. The audio stream supports the AMR codec.

UE#1indicates that it supports precondition and it indicates that it supports reliable provisional responses. However, it does not use the "Require” header for these capabilities.

UE#1 does have available the resources that are necessary to transport the media.

Table 5.3-9: INVITE (UE#1 to P-CSCF)

INVITE tel:+1-212-555-2222 SIP/2.0

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 70

Route: <sip:pcscf1.visited1.net:7531;lr;comp=sigcomp>, <sip:scscf1.home1.net;lr>

P-Preferred-Identity: "John Doe" <sip:user1_public1@home1.net>

P-Access-Network-Info: 3GPP-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11

Privacy: none

From: <sip:user1_public1@home1.net>;tag=171828

To: <tel:+1-212-555-2222>

Call-ID: cb03a0s09a2sdfglkj490333

Cseq: 127 INVITE

Require: sec-agree

Supported: precondition, 100rel

Proxy-Require: sec-agree

Security-Verify: ipsec-3gpp; q=0.1; alg=hmac-sha-1-96; spi-c=98765432; spi-s=87654321; port-c=8642; port-s=7531

Contact: <sip:[5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp>

Allow: INVITE, ACK, CANCEL, BYE, PRACK, UPDATE, REFER, MESSAGE

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 2987933615 2987933615 IN IP6 5555::aaa:bbb:ccc:ddd

s=-

c=IN IP6 5555::aaa:bbb:ccc:ddd

t=0 0

m=video 3400 RTP/AVP 98

b=AS:75

a=curr:qos local sendrecv

a=curr:qos remote none

a=des:qos mandatory local sendrecv

a=des:qos none remote sendrecv

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

m=audio 3456 RTP/AVP 97 96

b=AS:25.4

a=curr:qos local sendrecv

a=curr:qos remote none

a=des:qos mandatory local sendrecv

a=des:qos none remote sendrecv

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; maxframes=2

a=rtpmap:96 telephone-event

Supported:	The UE indicates support for the “precondition” mechanism and the support for reliable provisional responses

SDP	The SDP contains a set of codecs supported by UE#1 and desired by the user at UE#1 for this session.

2.	100 (Trying) response (P-CSCF#1 to UE#1)

The P-CSCF responds to the INVITE request with a 100 (Trying) provisional.

3.	INVITE request (P-CSCF#1 to S-CSCF#1) -  see example in table 5.3-10

Table 5.3-10: INVITE (P-CSCF#1 to S-CSCF#1)

INVITE tel:+1-212-555-2222 SIP/2.0

Via: SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 69

Route: <sip:scscf1.home1.net;lr>

Record-Route: <sip:pcscf1.visited1.net;lr>

P-Asserted-Identity: "John Doe" <sip:user1_public1@home1.net>

P-Access-Network-Info:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024"

Privacy:

From:

To:

Call-ID:

Cseq:

Supported:

Contact:

Allow:

Content-Type:

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

4.	100 (Trying) response (S-CSCF#1 to P-CSCF#1)

The S-CSCF responds to the INVITE request with a 100 (Trying) provisional.

5.	INVITE request (S-CSCF#2 to P-CSCF#2) see example in table 5.3-11

Table 5.3-11: INVITE (S-CSCF#2 to P-CSCF#2)

INVITE sip:[5555::eee:fff:aaa:bbb]:8805;comp=sigcomp SIP/2.0

Via: SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP icscf2_s.home2.net;branch=z9hG4bK871y12.1, SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 66

Route: <sip:pcscf2.visited2.net;lr>

Record-Route: <sip:scscf2.home2.net;lr>, <sip:scscf1.home1.net;lr>, <sip:pcscf1.visited1.net;lr>

P-Asserted-Identity:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024"

Privacy:

From:

To:

Call-ID:

Cseq:

Supported:

Contact:

Allow:

P-Called-Party-ID: <sip:user2_public1@home2.net>

Content-Type:

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

a=

6.	100 (Trying) response (P-CSCF#2 to S-CSCF#2)

The P-CSCF#2 responds to the INVITE request with a 100 (Trying) provisional.

7.	 INVITE request (P-CSCF#2 to UE #2) - see example in table 5.3-12

P-CSCF#2 forwards the INVITE request to UE#2.

Table 5.3-12: INVITE (P-CSCF#2 to UE#2)

INVITE sip:[5555::eee:fff:aaa:bbb]:8805;comp=sigcomp SIP/2.0

Via: SIP/2.0/UDP pcscf2.visited2.net:5088;comp=sigcomp;branch=z9hG4bK361k21.1, SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP icscf2_s.home2.net;branch=z9hG4bK871y12.1, SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 65

Record-Route: <sip:pcscf2.visited2.net:5088;lr;comp=sigcomp>, <sip:scscf2.home2.net;lr>, <sip:scscf1.home1.net;lr>, <sip:pcscf1.visited1.net;lr>

P-Asserted-Identity:

Privacy:

From:

To:

Call-ID:

Cseq:

Supported:

Contact:

Allow:

Content-Type:

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

8.	100 (Trying) response (UE#2 to P-CSCF)

The UE responds to the INVITE request with a 100 (Trying) provisional.

9.	Reserve IP-CAN bearer for media

The terminating UE sets up the bearer in accordance with the media description.

10. – 13. 180 (Ringing) response  - see example in table 5.3-13

After the resources have been reserved, UE#2 indicates that it is ringing. Since the message includes the SDP answer, this message is sent reliably.

Table 5.3-13: 180 (Ringing) (UE2 to P-CSCF2)

SIP/2.0 180 Ringing

Via: SIP/2.0/UDP pcscf2.visited2.net:5088;comp=sigcomp;branch=z9hG4bK361k21.1, SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP icscf2_s.home2.net;branch=z9hG4bK871y12.1, SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP

Record-Route: <sip:pcscf2.visited2.net:5088;lr;comp=sigcomp>, <sip:scscf2.home2.net;lr>, <sip:scscf1.home1.net;lr>, <sip:pcscf1.visited1.net;lr>

From:

To: <tel:+1-212-555-2222>;tag=2236

Call-ID:

Cseq:

Require: 100rel

Allow: INVITE, ACK, CANCEL, BYE, PRACK, UPDATE, REFER, MESSAGE

RSeq: 9022

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 2987933615 2987933615 IN IP6 5555::eee:fff:aaa:bbb

s=-

c=IN IP6 5555:: eee:fff:aaa:bbb

t=0 0

m=video 3400 RTP/AVP 98

b=AS:75

a=curr:qos local sendrecv

a=curr:qos remote sendrecv

a=des:qos mandatory local sendrecv

a=des:qos mandatory remote sendrecv

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

m=audio 3456 RTP/AVP 97 96

b=AS:25.4

a=curr:qos local sendrecv

a=curr:qos remote sendrecv

a=des:qos mandatory local sendrecv

a=des:qos mandatory remote sendrecv

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; maxframes

14. - 17.	PRACK - see example in table 5.3-14

UE#1 acknowledges the receipt of the 180 (Ringing). It does not contain SDP as the final codec decision is already made as part of the initial offer/answer exchange

Table 5.3-14: PRACK (UE1 to P-CSCF1)

PRACK sip:[5555::eee:fff:aaa:bbb]:8805;comp=sigcomp SIP/2.0

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 70

P-Access-Network-Info: 3GPP-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11

Route: <sip:pcscf2.visited2.net:5088;lr;comp=sigcomp>, <sip:scscf2.home2.net;lr>, <sip:scscf1.home1.net;lr>, <sip:pcscf1.visited1.net;lr>

From: <sip:user1_public1@home1.net>;tag=171828

To: <tel:+1-212-555-2222>;tag=2236

Call-ID: cb03a0s09a2sdfglkj490333

Cseq: 128 PRACK

Require: precondition, sec-agree

Proxy-Require: sec-agree

Security-Verify: ipsec-3gpp; q=0.1; alg=hmac-sha-1-96; spi-c=98765432; spi-s=87654321; port-c=8642; port-s=7531

RAck: 9021 127 INVITE

Content-Length: 0

18 - 21.	200 (OK) (PRACK)

UE#2 acknowledges the receipt of the PRACK with the 200 (OK)

22.		200 (OK) response (UE#2 to P-CSCF#2)  - see example in table 5.3-15

User #2 answers the phone and this triggers UE #2 to send the 200 (OK).

Table 5.3-15: 200OK (UE to P-CSCF)

SIP/2.0 200 OK

Via: SIP/2.0/UDP pcscf2.visited2.net:5088;comp=sigcomp;branch=z9hG4bK361k21.1, SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP icscf2_s.home2.net;branch=z9hG4bK871y12.1, SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

P-Access-Network-Info: 3GPP-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11

From:

To:

Call-ID:

Cseq: 127 INVITE

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 2987933615 2987933615 IN IP6 5555::eee:fff:aaa:bbb

s=-

c=IN IP6 5555:: eee:fff:aaa:bbb

t=0 0

m=video 3400 RTP/AVP 98

b=AS:75

a=curr:qos local sendrecv

a=curr:qos remote sendrecv

a=des:qos mandatory local sendrecv

a=des:qos mandatory remote sendrecv

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

m=audio 3456 RTP/AVP 97 96

b=AS:25.4

a=curr:qos local sendrecv

a=curr:qos remote sendrecv

a=des:qos mandatory local sendrecv

a=des:qos mandatory remote sendrecv

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; maxframes

23.	200 (OK) response (P-CSCF#2 to S-CSCF#2)  - see example in table 5.3-16

Table 5.3-16: 200OK (P-CSCF#2 to S-CSCF#2)

SIP/2.0 200 OK

Via: SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP icscf2_s.home2.net;branch=z9hG4bK871y12.1, SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

P-Access-Network-Info: 3GPP-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11

From:

To:

Call-ID:

Cseq:

Content-Type: application/sdp

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

a=

24.	200 (OK) response (S-CSCF#1 to P-CSCF#1)  - see example in table 5.3-17

Table 5.3-17: 200OK (S-CSCF#1 to P-CSCF#1)

SIP/2.0 200 OK

Via: pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

P-Access-Network-Info: 3GPP-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11

From:

To:

Call-ID:

Cseq:

Content-Type: application/sdp

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

a=

25.	200 (OK) response (P-CSCF#1 to UE#1)  - see example in table 5.3-18

Table 5.3-18: 200OK (P-CSCF#1 to UE#1)

SIP/2.0 200 OK

Via: [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

P-Access-Network-Info: 3GPP-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11

From:

To:

Call-ID:

Cseq:

Content-Type: application/sdp

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

a=

26 - 29.	ACK request

The calling party  responds to the 200 (OK) response with an ACK request.

# 5.4	Establishing a session when UE#1 need to reserve resources and UE#2 is non-IMS

### 5.4.1	Introduction

The following flow shows the establishment of a session where UE#1 is connected to the IM CN subsystem and needs to reserve local resources while UE#2 is plain SIP, i.e. does not support the preconditions framework

It is assumed that the originating UE uses a dedicated IP-CAN bearer for SIP signalling and dedicated  IP-CAN bearer for media.

The box intermediate IM CN subsystem nodes stands for the combination of I-CSCF/S-CSCF on the originating Routing of messages between those nodes is not described in the flow below.

As the topology on the non-IMS, terminating sideis not know, only a UE is shown on the terminating side. However, this does not rule out, that there are proxies in the terminating signalling path.

### 5.4.2	Signalling Flow

Figure 5.4-1: IMS session setup, resource reservation on both sides

The details of the signalling flows are as follows:

1.	INVITE request (UE#1 to P-CSCF#1) see example in table 5.4-1

For this example, it is assumed that UE#1 is willing to establish a multimedia session comprising a video stream and an audio stream. The video stream supports the  H.263 coded. The audio stream supports the AMR codec.

UE#1indicates that it supports precondition and it indicates that it supports reliable provisional responses. However, it does not use the "Require” header for these capabilities.

UE#1 does not have available the resources that are necessary to transport the media.

Table 5.4-1: INVITE (UE#1 to P-CSCF)

INVITE tel:+1-212-555-2222 SIP/2.0

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 70

Route: <sip:pcscf1.visited1.net:7531;lr;comp=sigcomp>, <sip:scscf1.home1.net;lr>

P-Preferred-Identity: "John Doe" <sip:user1_public1@home1.net>

P-Access-Network-Info: 3GPP-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11

Privacy: none

From: <sip:user1_public1@home1.net>;tag=171828

To: <tel:+1-212-555-2222>

Call-ID: cb03a0s09a2sdfglkj490333

Cseq: 127 INVITE

Require: sec-agree

Supported: precondition, 100rel

Proxy-Require: sec-agree

Security-Verify: ipsec-3gpp; q=0.1; alg=hmac-sha-1-96; spi-c=98765432; spi-s=87654321; port-c=8642; port-s=7531

Contact: <sip:[5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp>

Allow: INVITE, ACK, CANCEL, BYE, PRACK, UPDATE, REFER, MESSAGE

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 2987933615 2987933615 IN IP6 5555::aaa:bbb:ccc:ddd

s=-

c=IN IP6 5555::aaa:bbb:ccc:ddd

t=0 0

m=video 3400 RTP/AVP 98

b=AS:75

a=curr:qos local none

a=curr:qos remote none

a=des:qos mandatory local sendrecv

a=des:qos none remote sendrecv

a=inactive

a=rtpmap:98 H263

m=audio 3456 RTP/AVP 97 96

b=AS:25.4

a=curr:qos local none

a=curr:qos remote none

a=des:qos mandatory local sendrecv

a=des:qos none remote sendrecv

a=inactive

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; maxframes=2

a=rtpmap:96 telephone-event

Supported:	The UE indicates support for the "precondition" mechanism and the support for reliable provisional responses

SDP	The SDP contains a set of codecs supported by UE#1 and desired by the user at UE#1 for this session. As the local preconditions are not fulfilled, both media are set to "inactive".

2.	100 (Trying) response (P-CSCF#1 to UE#1)

The P-CSCF responds to the INVITE request with a 100 (Trying) provisional.

3.	INVITE request (P-CSCF#1 to S-CSCF#1) -  see example in table 5.4-2

Table 5.4-2: INVITE (P-CSCF#1 to S-CSCF#1)

INVITE tel:+1-212-555-2222 SIP/2.0

Via: SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 69

Route: <sip:scscf1.home1.net;lr>

Record-Route: <sip:pcscf1.visited1.net;lr>

P-Asserted-Identity: "John Doe" <sip:user1_public1@home1.net>

P-Access-Network-Info:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024"

Privacy:

From:

To:

Call-ID:

Cseq:

Supported:

Contact:

Allow:

Content-Type:

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

4.	100 (Trying) response (S-CSCF#1 to P-CSCF#1)

The S-CSCF responds to the INVITE request with a 100 (Trying) provisional.

5.	INVITE request (S-CSCF#1 to UE#2) see example in table 5.4-3

Table 5.4-3: INVITE (S-CSCF#1 to UE#2)

INVITE sip:[5555::eee:fff:aaa:bbb]:8805;comp=sigcomp SIP/2.0

Via: SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 66

Route: <sip:pcscf2.visited2.net;lr>

Record-Route: <sip:scscf1.home1.net;lr>, <sip:pcscf1.visited1.net;lr>

P-Asserted-Identity:

Privacy:

From:

To:

Call-ID:

Cseq:

Require:

Supported:

Contact:

Allow:

P-Called-Party-ID: <sip:user2_public1@home2.net>

Content-Type:

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

a=

6.	UE#2 assumes itself on hold

UE#2 assumes itself on hold as media are set to inactive, i.e. UE#2 does not await or send media

Resources are available

7 -9 . 180 (Ringing) response

UE#2 indicates that it is ringing. It is assumed that UE#2 does not support the support the “100rel” extension and therefore the 180 (Ringing) is not sent reliable, i.e. no SDP is sent in the 180 Ringing.

10.	200 (OK) response (UE#2 to S-CSCF)  - see example in table 5.4-5

User on the terminating side goes off hook. It is assumed that UE#2 plays some announcement to indicate to the user that the call is on hold.

UE#2 ignores the precondition that it received in the INVITE as it does not support them. No preconditions are included in the SDP answer. Both media streams are set to "inactive".

Table 5.4-5: 200OK (UE#2 to S-CSCF)

SIP/2.0 200 OK

Via: SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

From:

To: <tel:+1-212-555-2222>;tag=314159

Call-ID:

Cseq:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 2987933615 2987933615 IN IP6 5555::eee:fff:ggg:hhh

s=-

c=IN IP6 5555::eee:fff:ggg:hhh

t=0 0

m=video 3400 RTP/AVP 98

b=AS:75

a=inactive

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

m=audio 3456 RTP/AVP 97 96

b=AS:25.4

a=inactive

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; maxframes

11.	200 (OK) response (S-CSCF#1 to P-CSCF#1)  - see example in table 5.4-6

Table 5.4-6: 200OK (S-CSCF#1 to P-CSCF#1)

SIP/2.0 200 OK

Via: SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

From:

To:

Call-ID:

Cseq:

Content-Type: application/sdp

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

m=

b=

a=

a=

a=

12.	200 (OK) response (P-CSCF#1 to UE#1)  - see example in table 5.4-7

Table 5.4-7: 200OK (P-CSCF to UE#1)

SIP/2.0 200 OK

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

From:

To:

Call-ID:

Cseq:

Content-Type: application/sdp

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

m=

b=

a=

a=

a=

13 . Reserve IPCAN bearer for media

The originating UE sets up the bearer in accordance with the media description received SDP.

14.-16. .ACK request

The calling party  responds to the 200 (OK) response with an ACK request.

17.	INVITE request (UE#1 to P-CSCF#1) see example in table 5.4-8

UE#1 sets the media streams to active using a re-INVITE request

As UE#2 does not support preconditions, UE#1 does not include preconditions in the SDP.

Table 5.4-8: INVITE (UE#1 to P-CSCF)

INVITE <sip:[5555::eee:fff:aaa:bbb]:8805;comp=sigcomp

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 70

Route: <sip:pcscf1.visited1.net:7531;lr;comp=sigcomp>, <sip:scscf1.home1.net;lr>

From: <sip:user1_public1@home1.net>;tag=171828

To: <tel:+1-212-555-2222>; tag=314159

Call-ID: cb03a0s09a2sdfglkj490333

Cseq: 128 INVITE

Require: sec-agree

Supported: precondition, 100rel

Proxy-Require: sec-agree

Security-Verify: ipsec-3gpp; q=0.1; alg=hmac-sha-1-96; spi-c=98765432; spi-s=87654321; port-c=8642; port-s=7531

Contact: <sip:[5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp>

Allow: INVITE, ACK, CANCEL, BYE, PRACK, UPDATE, REFER, MESSAGE

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 2987933615 2987933615 IN IP6 5555::aaa:bbb:ccc:ddd

s=-

c=IN IP6 5555::aaa:bbb:ccc:ddd

t=0 0

m=video 3400 RTP/AVP 98

b=AS:75

a=rtpmap:98 H263

m=audio 3456 RTP/AVP 97 96

b=AS:25.4

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; maxframes=2

a=rtpmap:96 telephone-event

18.	INVITE request (P-CSCF#1 to S-CSCF#1) see example in table 5.4-9

Table 5.4-9: INVITE (P-CSCF#1 to S-CSCF)

INVITE <sip:[5555::eee:fff:aaa:bbb]:8805;comp=sigcomp

Via: SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 69

Route: <sip:scscf1.home1.net;lr>

Record-Route: <sip:pcscf1.visited1.net;lr>

From:

To:

Call-ID:

Cseq:

Supported:

Contact:

Allow:

Content-Type:

Content-Length:

v=

o=

s=

c=

t=

m=

b=

a=

m=

b=

a=

a=

a=

19.	INVITE request (S-CSCF#1 to UE#2) see example in table 5.4-10

Table 5.4-10: INVITE (S-CSCF#1 to UE#2)

INVITE <sip:[5555::eee:fff:aaa:bbb]:8805;comp=sigcomp

Via: SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards:

Record-Route: < sip:scscf1.home1.net;lr>, <sip:pcscf1.visited1.net;lr>

From:

To:

Call-ID:

Cseq:

Supported:

Contact:

Allow:

Content-Type:

Content-Length:

v=

o=

s=

c=

t=

m=

b=

a=

m=

b=

a=

a=

a=

20.	200 (OK) response (UE#2 to S-CSCF)  - see example in table 5.4-11

UE#2 confirms the new SDP offer. Media is set to active

Table 5.4-11: 200OK (UE#2 to S-CSCF)

SIP/2.0 200 OK

Via: SIP/2.0/UDP scscf1.home1.net;branch=z9hG4bK332b23.1, SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

From:

To: <

Call-ID:

Cseq:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 2987933615 2987933615 IN IP6 5555::eee:fff:ggg:hhh

s=-

c=IN IP6 5555::eee:fff:ggg:hhh

t=0 0

m=video 3400 RTP/AVP 98

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

m=audio 3456 RTP/AVP 97 96

b=AS:25.4

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; maxframes

21.	200 (OK) response (S-CSCF#1 to P-CSCF#1)  - see example in table 5.4-12

Table 5.4-12: 200OK (S-CSCF#1 to P-CSCF#1)

SIP/2.0 200 OK

Via: SIP/2.0/UDP pcscf1.visited1.net;branch=z9hG4bK240f34.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

From:

To:

Call-ID:

Cseq:

Content-Type:

Content-Length:(…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

m=

b=

a=

a=

a=

22.	200 (OK) response (P-CSCF#1 to UE#1)  - see example in table 5.4-12

Table 5.4-7: 200OK (P-CSCF to UE#1)

SIP/2.0 200 OK

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

From:

To:

Call-ID:

Cseq:

Content-Type: application/sdp

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

m=

b=

a=

a=

a=

23.-25. .ACK request

The calling party  responds to the 200 (OK) response with an ACK request.

# 5.5	Establishing a session when UE#1 is non-IMS and UE#2 needs to reserve resources

### 5.5.1	Introduction

The following flow shows the establishment of a session where UE#1 is a non-IMS UE. i.e. is plain SIP and UE#2 is an IMS and needs to perform resource reservation.

It is assumed that the terminating UE uses a dedicated IP-CAN bearer for SIP signalling and dedicated  IP-CAN bearer for media.

The box intermediate IM CN subsystem nodes stands for the combination of I-CSCF/S-CSCF on the terminating side Routing of messages between those nodes is not described in the flow below.

As the topology on the non-IMS, originating side is not know, only a UE is shown on the terminating side. However, this does not rule out, that there are proxies in the originating signalling path.

### 5.5.2	Signalling Flow

Figure 5.5-1: IMS session setup, resource reservation on terminating side

The details of the signalling flows are as follows:

1.	INVITE request (UE#1 to P-CSCF#1) see example in table 5.5-1

For this example, it is assumed that UE#1 is willing to establish a multimedia session comprising a video stream and an audio stream. The video stream supports the  H.263 coded. The audio stream supports the AMR codec.

UE# does not indicate that it supports precondition and does not indicate support for the 100rel extension.

Table 5.5-1: INVITE (UE#1 to IM CN Subsytem entities)

INVITE tel:+1-212-555-2222 SIP/2.0

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 70

Route: <sip:pcscf1.visited1.net:7531;lr;comp=sigcomp>, <sip:scscf1.home1.net;lr>

P-Preferred-Identity: "John Doe" <sip:user1_public1@home1.net>

P-Access-Network-Info: 3GPP-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11

Privacy: none

From: <sip:user1_public1@home1.net>;tag=171828

To: <tel:+1-212-555-2222>

Call-ID: cb03a0s09a2sdfglkj490333

Cseq: 127 INVITE

Contact: <sip:[5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp>

Allow: INVITE, ACK, CANCEL, BYE, REFER, MESSAGE

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 2987933615 2987933615 IN IP6 5555::aaa:bbb:ccc:ddd

s=-

c=IN IP6 5555::aaa:bbb:ccc:ddd

t=0 0

m=video 3400 RTP/AVP 98

b=AS:75

a=rtpmap:98 H263

m=audio 3456 RTP/AVP 97 96

b=AS:25.4

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; maxframes=2

a=rtpmap:96 telephone-event

SDP	The SDP contains a set of codecs supported by UE#1 and desired by the user at UE#1 for this session.

2.	100 (Trying) response

The IM CN subsystem respond to the INVITE request with a 100 (Trying) provisional.

3.	INVITE request (S-CSCF to P-CSCF) -  see example in table 5.5-2

Table 5.5-2: INVITE (S-CSCF to P-CSCF)

INVITE sip:[5555::eee:fff:aaa:bbb]:8805;comp=sigcomp SIP/2.0

Via: SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 66

Route: <sip:pcscf2.visited2.net;lr>

Record-Route: <sip:scscf2.home2.net;lr>,

From:

To:

Call-ID:

Cseq:

Contact:

Allow:

Content-Type:

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

4.	100 (Trying) response (S-CSCF#1 to P-CSCF#1)

The S-CSCF responds to the INVITE request with a 100 (Trying) provisional.

5.	INVITE request (P-CSCF to UE) see example in table 5.5-3

Table 5.5-3: INVITE (P-CSCF to UE)

INVITE sip:[5555::eee:fff:aaa:bbb]:8805;comp=sigcomp SIP/2.0

Via: SIP/2.0/UDP pcscf2.visited2.net:5088;comp=sigcomp;branch=z9hG4bK361k21.1, SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 66

Record-Route: <sip:pcscf2.visited2.net:5088;lr;comp=sigcomp>, <sip:scscf2.home2.net;lr>, <sip:scscf1.home1.net;lr>,,

From:

To:

Call-ID:

Cseq:

Require:

Contact:

Allow:

P-Called-Party-ID: <sip:user2_public1@home2.net>

Content-Type:

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

a=

a=

a=

a=

6.	. Reserve IPCAN bearer for media

The IMS UE sets up the bearer in accordance with the media description received SDP and if necessary its codec decission.

7 -9 . 180 (Ringing) response

UE#2 indicates that it is ringing..

10.	200 (OK) response (UE#2 to P-CSCF)  - see example in table 5.5-4

UE#2 does not use preconditions as they are not supported by the originating side.

Table 5.5-4: 200OK (UE#2 to P-CSCF)

SIP/2.0 200 OK

Via: SIP/2.0/UDP pcscf2.visited2.net:5088;comp=sigcomp;branch=z9hG4bK361k21.1, SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Record-Route: <sip:pcscf2.visited2.net:5088;lr;comp=sigcomp>, <sip:scscf2.home2.net;lr>

P-Access-Network-Info: 3GPP-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11

Privacy: none

From:

To: <tel:+1-212-555-2222>;tag=314159

Call-ID:

Cseq:

Contact: <sip:[5555::eee:fff:aaa:bbb]:8805;comp=sigcomp>

Allow: INVITE, ACK, CANCEL, BYE, PRACK, UPDATE, REFER, MESSAGE

RSeq: 9021

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 2987933615 2987933615 IN IP6 5555::eee:fff:ggg:hhh

s=-

c=IN IP6 5555::eee:fff:ggg:hhh

t=0 0

m=video 3400 RTP/AVP 98

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

m=audio 3456 RTP/AVP 97 96

b=AS:25.4

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; maxframes

11.	200 (OK) response (P-CSCF to S-CSCF)  - see example in table 5.5-5

Table 5.5-5: 200OK (P-CSCF to S-CSCF)

SIP/2.0 200 OK

Via: SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Record-Route:

P-Access-Network-Info:

Privacy:

From:

To: <tel:+1-212-555-2222>;tag=314159

Call-ID:

Cseq:

Contact:

Allow:

RSeq:

Content-Type: application/sdp

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

m=

b=

a=

a=

a=

12.	200 (OK) response (P-CSCF#1 to UE#1)  - see example in table 5.5-6

Table 5.5-6: 200OK (S-CSCF to UE#1)

SIP/2.0 200 OK

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Record-Route:

Privacy:

From:

To: <tel:+1-212-555-2222>;tag=314159

Call-ID:

Cseq:

Contact:

Allow:

RSeq:

Content-Type: application/sdp

Content-Length: (…)

v=

o=

s=

c=

t=

m=

b=

a=

a=

a=

m=

b=

a=

a=

a=

13.-15. .ACK request

The calling party responds to the 200 (OK) response with an ACK request.

# 5.6	Establishing a session when UE#1 and UE#2 do not need to reserve resources and do not use preconditions.

### 5.6.1	Introduction

The following flow shows the establishment of a session where both UE#1 and UE#2 is an IMS and do not need to perform resource reservation and do not use preconditions. The example is based on the Push to Talk over Cellular (PoC) on demand session establishment  automatic answer  scenario from OMA PoC 1.0 enabler but with a confirmed indication (no media buffering performed by the PoC Server).

It is assumed that the both UEs uses a dedicated IP-CAN bearer for SIP signalling and dedicated  IP-CAN bearer for media.

The box intermediate IM CN subsystem nodes stands for the combination of P-CSCF/I-CSCF/S-CSCF nodes in the network. Routing of messages between those nodes is not described in the flow below.

### 5.6.2	Signalling Flow

Figure 5.6-1: IMS session setup, no resource reservation, no preconditions

The details of the signalling flows are as follows:

1.	INVITE request (UE#1 to P-CSCF#1) see example in table 5.6-1

For this example, it is assumed that UE#1 is willing to establish a multimedia session comprising a video stream and an audio stream. The video stream supports the  H.263 coded. The audio stream supports the AMR codec.

UE# does not indicate that it supports precondition and does not indicate support for the 100rel extension.

Within the Intermediate IM CN subsystem entities are two PoC Servers that acts as B2BUAs

Table 5.6-1: INVITE (UE#1 to IM CN Subsytem entities)

INVITE sip:PoCConferenceFactoryURI.home1.net SIP/2.0

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Max-Forwards: 70

Route: <sip:pcscf1.visited1.net:7531;lr;comp=sigcomp>, <sip:scscf1.home1.net;lr>

P-Preferred-Identity: "John Doe" <sip:user1_public1@home1.net>

P-Access-Network-Info: 3GPP-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11

Privacy: none

From: <sip:user1_public1@home1.net>;tag=171828

To: <tel:+1-212-555-2222>

Call-ID: cb03a0s09a2sdfglkj490333

Cseq: 127 INVITE

Require: sec-agree,recipient-list-invite

Supported: timer

Proxy-Require: sec-agree

Security-Verify: ipsec-3gpp; q=0.1; alg=hmac-sha-1-96; spi-c=98765432; spi-s=87654321; port-c=8642; port-s=7531

Contact: <sip:[5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp>;+g.poc.talkburst

Allow: INVITE, ACK, CANCEL, BYE, PRACK, UPDATE, REFER, MESSAGE, SUBSCRIBE, NOTIFY, PUBLISH

Accept-Contact: *;+g.poc.talkburst;require;explicit

User-Agent: PoC-client/OMA1.0 Acme-Talk5000/v1.01

Session-Expires: 1800;refresher=uac

Content-Type: multipart/mixed

Content-Length: (…)

--boundary1

Content-Type: application/sdp

v=0

o=- 2987933615 2987933615 IN IP6 5555::aaa:bbb:ccc:ddd

s=-

c=IN IP6 5555::aaa:bbb:ccc:ddd

t=0 0

m=audio 3456 RTP/AVP 97

b=AS:25.4

a=rtpmap:97 AMR

a=rtcp:5560

a=fmtp:97 mode-set=0,2,5,7; maxframes=2

m=application 2000 udp TBCP

a=fmtp:TBCP queuing=1; tb_priority=2; timestamp=1

--boundary1

Content-Type: application/resource-lists+xml

Content-Disposition: recipient-list

<?xml version="1.0" encoding="UTF-8"?>

<resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists">

<list>

<entry uri="tel:+1-212-555-2222"/>

</list>

</resource-lists>

--boundary1--

SDP	The SDP contains a set of codecs supported by UE#1 and desired by the user at UE#1 for this session.

Resource List	The Resource List the URI of UE#2 being invited by the user at UE#1 for this session.

2.	100 (Trying) response

The IM CN subsystem respond to the INVITE request with a 100 (Trying) provisional.

3.	INVITE request (P-CSCF to UE) see example in table 5.6-2

Table 5.6-2: INVITE (IM CN Subsytem entities to UE)

INVITE sip:[5555::eee:fff:aaa:bbb]:8805;comp=sigcomp SIP/2.0

Via: SIP/2.0/UDP pcscf2.visited2.net:5088;comp=sigcomp;branch=z9hG4bK361k21.1, SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP PoC-SessionABCDEF@pocserver2.home2.net;session=1-1

Max-Forwards: 68

Record-Route: <sip:pcscf2.visited2.net:5088;lr;comp=sigcomp>, <sip:scscf2.home2.net;lr>

From: <sip:user1_public1@home1.net>;tag=487651

To:

Call-ID: 03a0sdcglkj433s09a290bf3

Cseq: 10227 INVITE

Require: sec-agree

Supported: norefersub,timer

Contact: <PoC-SessionABCDEF@pocserver2.home2.net;session=1-1>;+g.poc.talkburst

Allow:

P-Called-Party-ID: <sip:user2_public1@home2.net>

Accept-Contact: *;+g.poc.talkburst; require;explicit

User-Agent: PoC-serv/OMA1.0

Session-Expires: 1800;refresher=uas

Answer-Mode: Auto

Content-Type: application/sdp

Content-Length: (…)

v=

o=- 3361529879 3361529879 IN IP6 60333::ddd:ccc:aaa:bbb

s=

c=IN IP6 5555::ddd:ccc:aaa:bbb

t=

m=audio audio 63776 RTP/AVP 97

b=AS:25.4

a=rtpmap:97 AMR

a=rtcp:6390

a=fmtp:97 mode-set=0,2,5,7; maxframes=2

m=application 60000 udp TBCP

a=fmtp:TBCP queuing=1; tb_priority=2; timestamp=1

4.	200 (OK) response (UE#2 to IM CN Subsytem entities)  - see example in table 5.6-3

UE#2 does not use preconditions as they are not supported by the originating side.

Table 5.6-3: 200OK (UE#2 to IM CN Subsytem entities)

SIP/2.0 200 OK

Via: SIP/2.0/UDP pcscf2.visited2.net:5088;comp=sigcomp;branch=z9hG4bK361k21.1, SIP/2.0/UDP scscf2.home2.net;branch=z9hG4bK764z87.1, SIP/2.0/UDP SessionABCDEF@pocserver2.home2.net;session=1-1

Record-Route: <sip:pcscf2.visited2.net:5088;lr;comp=sigcomp>, <sip:scscf2.home2.net;lr>

P-Access-Network-Info: 3GPP-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11

Privacy: none

From:

To: <tel:+1-212-555-2222>;tag=314159

Call-ID:

Cseq:

Require:timer

Contact: <sip:[5555::eee:fff:aaa:bbb]:8805;comp=sigcomp>;+g.poc.talkburst

Allow: INVITE, ACK, CANCEL, BYE, PRACK, UPDATE, REFER, MESSAGE

Server: PoC-client/OMA1.0 Acme-Talk5000/v1.01

Session-Expires: 1800;refresher=uas

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 2987933615 2987933615 IN IP6 5555::eee:fff:aaa:bbb

s=-

c=IN IP6 5555::eee:fff:aaa:bbb

t=0 0

m=audio 3456 RTP/AVP 97

b=AS:25.4

a=rtpmap:97 AMR

a=rtcp:75000

a=fmtp:97 mode-set=0,2,5,7; maxframes=2

m=application 75590 udp TBCP

a=fmtp:TBCP queuing=1; tb_priority=2; timestamp=1

5.	200 (OK) response (IM CN Subsytem entities to UE#1)  - see example in table 5.6-4

Table 5.6-4: 200OK (IM CN Subsytem entities to UE#1)

SIP/2.0 200 OK

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd]:1357;comp=sigcomp;branch=z9hG4bKnashds7

Record-Route: <sip:scscf1.home1.net;lr>,<sip:pcscf1.visited1.net:7531;lr;comp=sigcomp>

Privacy:

From: <sip:user1_public1@home1.net>;tag=171828

To: <tel:+1-212-555-2222>;tag=257645

Call-ID: cb03a0s09a2sdfglkj490333

Cseq: 127 INVITE

Contact: <sip:PoC-SessionABCDEF@PoC-ServerA.home1.net;session=1-1>;+g.poc.talkburst

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 3362987915 3362987915 IN IP6 57777::eee:fff:aaa:bbb

s=-

c=IN IP6 57777::eee:fff:aaa:bbb

t=0 0

m=audio 57787 RTP/AVP 97

b=AS:25.4

a=rtpmap:97 AMR

a=rtcp:57000

a=fmtp:97 mode-set=0,2,5,7; maxframes=2

m=application 57790 udp TBCP

a=fmtp:TBCP queuing=1; tb_priority=2; timestamp=1

6.-7. .ACK request

The calling party responds to the 200 (OK) response with an ACK request.

Annex A (normative):

Annex <X>:
Change history

It is usual to include an annex (usually the final annex of the document) for reports under TSG change control which details the change history of the report using a table as follows:

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2006-02 |  |  |  |  | skeleton of the TR | 0.0.0 | 0.0.0 |
| 2006-02 |  |  |  |  | Version 0.1.0 created as a result of CT1#41 The following CR's were incorporated and the editor adopted their content / structure to the revised TR structure: C1-060227 - only UE#1 needs to perform resource reservation C1-060537 - UE#1 and UE#2 need to perform resource reservation C1-060538 - only UE#2 needs to perform resource reservation | 0.0.0 | 0.1.0 |
| 2006-05 |  |  |  |  | The following CR's were incorporated and the editor adopted their content / structure to the revised TR structure: C1-060701 - Establishing a session when UE#1 need to reserve resources and UE#2 is non-IMS C1-060703 - Miscellaneous Corrections against 24.930 C1-061066 - Establishing a session when UE#1 is non-IMS and UE#2 needs to reserve resources | 0.1.0 | 0.2.0 |
| 2006-09 |  |  |  |  | The following CR's were incorporated and the editor adopted their content / structure to the revised TR structure: C1-061640 - PoC Session Establishment Flow C1-061757 - show encryption in Security-Verify C1-061878 - Call flow when originator has resources reserved and the called party needs to reserve resources | 0.2.0 | 0.3.0 |