3GPP TS 25.202 V7.0.0 (2006-03)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

7.68Mcps Time Division Duplex (TDD) option;

Overall description: Stage 2

(Release 7)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

UMTS, radio, stage 2

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2006, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Symbols	7

3.3	Abbreviations	7

4	Background and introduction	7

5	Requirements	7

6	Physical layer structure	8

6.0	Services offered to higher layers	8

6.1	Frame structure	8

6.2	Burst structure	8

6.3	Midambles	9

6.4	Coding and Modulation	12

6.5	Scrambling Codes	12

6.6	Synchronisation Codes	13

6.7	Transmit diversity	13

6.9	Indicator Channels	13

6.9.1	Paging Indicator Channel (PICH)	13

6.9.2	MBMS Indicator Channel (MICH)	13

6.10	Mapping of transport channels to physical channels	13

7	Physical layer procedures	14

7.1	Power Control	14

7.2	Timing Advance	14

7.3	HSDPA procedures	15

7.4	Synchronisation procedures	15

7.5	RACH procedures	15

8	UE capabilities	15

9	Layer 2/3 protocol aspects	16

9.1	Protocol architecture	16

9.2	Signalling	16

9.2.1	General	16

9.2.2	L2/MAC differences	16

9.2.3	L2/RRC differences	16

9.3	HSDPA related issues	16

9.4	Mobility	17

9.5	Idle Mode Procedures	17

10	Iub/Iur aspects	17

10.1  Impacts on Iub/Iur interfaces – general aspects	17

10.1.1	Timing advance and Rx Timing Deviation	17

10.1.2	Paging	17

10.1.3	DSCH Power Control from the RNC	17

10.2  Impacts on Iub/Iur control plane protocols	18

10.3  Impacts on Iub/Iur user plane protocols	18

11	Radio aspects	18

11.1	UE radio transmission and reception	18

11.1.1	 Transmitter characteristics	18

11.1.1.1	Transmit power	18

11.1.1.2	Output RF spectrum emissions	18

11.1.1.2.1	Occupied bandwidth	18

11.1.1.2.2	Out of band emission	18

11.1.1.2.2.1	Spectrum emission mask	18

11.1.1.2.2.2	Adjacent Channel Leakage power Ratio (ACLR)	19

11.1.1.2.2.3	Spurious emissions	19

11.1.2	 Receiver characteristics	20

11.1.2.1	Reference sensitivity level	20

11.1.2.1.1	Minimum Requirement	20

11.1.2.2	Adjacent Channel Selectivity (ACS)	20

11.1.2.2.1	Minimum Requirement	20

11.1.2.3	Blocking characteristics	20

11.1.2.3.1	Minimum Requirement	21

11.1.2.4	Spurious response	21

11.1.2.4.1	Minimum Requirement	21

11.1.2.5	Spurious emissions	22

11.1.2.5.1	Minimum Requirement	22

11.2	Base station radio transmission and reception	22

11.2.1	 Transmitter characteristics	22

11.2.1.1	Base station output power	22

11.2.1.2	Output RF spectrum emissions	22

11.2.1.2.1	Occupied bandwidth	22

11.2.1.2.2	Out of band emission	22

11.2.1.2.2.1	Spectrum emission mask	23

11.2.1.2.2.2	Adjacent Channel Leakage power Ratio (ACLR)	24

11.2.1.2.2.2.1	Minimum requirement	25

11.2.1.2.2.3	Spurious emissions	25

11.2.2	 Receiver characteristics	25

11.2.2.1	Reference sensitivity level	25

11.2.2.1.1	Minimum requirement	25

11.2.2.2	Adjacent Channel Selectivity (ACS)	26

11.2.2.2.1	Minimum requirement	26

11.2.2.3	Blocking characteristics	26

11.2.2.3.1	Minimum requirement	26

11.2.2.3.2	Collocation with GSM900 and/or DCS 1800	28

11.2.2.4	Spurious emissions	28

11.2.2.4.1	Minimum requirement	28

Annex A (informative):	Change history	29

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is the overall technical specification for the support of the 7.68Mcps TDD option in UTRA.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 25.895 (V6.0.0): "Analysis of higher chip rates for UTRA TDD evolution".

[2]	3GPP TS 25.221: "Physical channels and mapping of transport channels onto physical channels (TDD)".

[3]	3GPP TS 25.222: "Multiplexing and channel coding (TDD)".

[4]	3GPP TS 25.223: "Spreading and modulation (TDD)".

[5]	3GPP TS 25.224: "Physical layer procedures (TDD)".

[6]	3GPP TS 25.225: "Physical layer; Measurements (TDD)".

[7]	3GPP TS 25.301: "Radio Interface Protocol Architecture".

[8]	3GPP TS 25.306: "UE Radio Access capabilities".

[9]	3GPP TS 25.321: "Medium Access Control (MAC) protocol specification".

[10]	3GPP TS 25.102: "User Equipment (UE) radio transmission and reception (TDD)".

[11]	3GPP TS 25.105 "UTRAN (BS) TDD; Radio transmission and reception".

[12]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [12] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [12].

(void)

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

(void)

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [12] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [12].

BCH	Broadcast Channel

CCPCH	Common Control Physical Channel

DCH	Dedicated Channel

DPCH	Dedicated Physical Channel

DSCH	Downlink Shared Channel

FACH	Forward Access Channel

HS-DSCH	High Speed Downlink Shared Channel

HS-PDSCH	High Speed Physical Downlink Shared Channel

HS-SCCH	Shared Control Channel for HS-DSCH

HS-SICH	Shared Information Channel for HS-DSCH

P-CCPCH	Primary CCPCH

PCH	Paging Channel

PDSCH	Physical Downlink Shared Channel

PI	Paging Indicator (value calculated by higher layers)

PICH	Page Indicator Channel

PRACH	Physical Random Access Channel

PUSCH	Physical Uplink Shared Channel

RACH	Random Access Channel

S-CCPCH	Secondary CCPCH

SCH	Synchronisation Channel

TrCH	Transport Channel

USCH	Uplink Shared Channel

# 4	Background and introduction

The 7.68Mcps TDD option is an evolution of the 3.84Mcps TDD option to a higher chip rate. There exists a great degree of commonality between the 3.84Mcps TDD option and the 7.68Mcps TDD option. Nevertheless, there are many aspects of the 7.68Mcps TDD option that require separate specification to the 3.84Mcps TDD option. The following aspects are specified at a high level in this document:

-	Physical layer structure;

-	Physical layer procedures;

-	UE capabilities;

-	Layer 2/3 protocol aspects;

-	Iub / Iur aspects;

-	Radio aspects;

# 5	Requirements

-	The 7.68Mcps TDD option shall provide significant enhancements in terms of user experience (throughput and delay) and/or capacity (at least to the extent shown in [1]).

-	Full mobility shall be supported, i.e., mobility should be supported for high-speed UE cases also, but optimisation should be for low-speed to medium-speed scenarios.

-	It is highly desirable for the 7.68Mcps TDD option to maintain commonality with the 3.84Mcps TDD option. New features shall therefore provide significant incremental gain for an acceptable complexity.

-	The UE and network complexity shall be minimised for a given level of system performance.

-	The impact on current releases in terms of both protocol and hardware perspectives shall be taken into account.

# 6	Physical layer structure

## 6.0	Services offered to higher layers

The 7.68Mcps TDD option supports an identical set of transport channels and indicators to the 3.84Mcps TDD option.

## 6.1	Frame structure

The 7.68Mcps TDD option frame is of length 10ms and consists of 15 timeslots of duration 5120 * Tc, where Tc is the chip duration (Tc = 1 / 7.68 * 106 = 130.2ns). Any timeslot in the frame can be either uplink or downlink. At least one timeslot in the frame is assigned to the uplink and at least one timeslot in the frame is assigned to the downlink. The frame structure is shown in Figure 6.1.1.

time

frequency

10ms

5120 * Tc

7.68Mcps

Figure 6.1.1: The 7.68Mcps TDD option frame structure

## 6.2	Burst structure

The 7.68Mcps burst consists of two data field portions, a midamble portion containing a training sequence and a guard period as shown in Figure 6.2.1. Several bursts can be transmitted at the same time where each burst uses a different OVSF channelisation code, but the same scrambling code.

Data field

Data field

Midamble

guard period

5120 * Tc

Figure 6.2.1: 7.68Mcps TDD option burst structure

Three burst types are specified: burst types 1, 2 and 3. The maximum number of training sequences supported in burst types 1 and 3 is either 4, 8 or 16 depending on cell configuration and either 4 or 8 for burst type 2 depending on cell configuration. The lengths of the fields within each burst are defined in Table 6.2.1.

Table 6.2.1: Number of chips within fields of the 7.68Mcps burst

| Field | Burst Type 1 | Burst Type 2 | Burst Type 3 |
| --- | --- | --- | --- |
| Data field 1 | 1952 | 2208 | 1952 |
| Midamble | 1024 | 512 | 1024 |
| Data field 2 | 1952 | 2208 | 1760 |
| Guard Period | 192 | 192 | 384 |

On the downlink, a spreading factor of 32 is supported. Additionally for DPCH, PDSCH and HS-PDSCH, a spreading factor of 1 is supported on the downlink.

On the uplink, spreading factors of 1, 2, 4, 8, 16 and 32 are supported for DPCH and PUSCH. PRACH only supports spreading factors 16 and 32 and HS-SICH only supports spreading factor 32.

The spreading factors and burst types supported for different physical channels are defined in Table 6.2.2.

Table 6.2.2: Spreading factors and burst types supported by physical channels

| Physical channel | Supported spreading factors | Supported burst types |
| --- | --- | --- |
| UL DPCH | 1, 2, 4, 8, 16, 32 | 1, 2, 3 |
| DL DPCH | 1, 32 | 1, 2 |
| P-CCPCH | 32 | 1 |
| S-CCPCH | 32 | 1, 2 |
| PRACH | 16, 32 | 3 |
| PUSCH | 1, 2, 4, 8, 16, 32 | 1, 2, 3 |
| PDSCH | 1, 32 | 1, 2 |
| HS-PDSCH | 1, 32 | 1, 2 |
| HS-SCCH | 32 | 1, 2 |
| HS-SICH | 32 | 1, 2 |

Transmission of TPC and TFCI are performed in accordance with the general procedures used for the existing 3.84 Mcps TDD option.  Due to the maximum spreading factor being increased from 16 (3.84Mcps) to 32 (7.68Mcps), usage of SF16 for TPC/TFCI is replaced with SF32 where appropriate.

## 6.3	Midambles

Midambles for burst types 1, 2 and 3 are created using the method applied for 3.84Mcps TDD. The basic midamble code for burst types 1 and 3 is of length 912; for burst type 2 the basic midamble code is of length 456.

Default, common and UE specific midamble modes are supported in the 7.68Mcps TDD option. The characteristics of these midamble allocations at 7.68Mcps are identical to their characteristics at 3.84Mcps. The number of active channelisation codes is signaled via midamble through an extension of the scheme applied at 3.84Mcps TDD (the extension accounts for the higher spreading factor supported at 7.68Mcps).

Midamble transmit powers are allocated as for 3.84Mcps TDD.

The association between midambles and channelisation codes for burst types 1, 2 and 3 are as shown in figure 6.3.1 for Kcell = 16, figure 6.3.2 for Kcell = 8 and figure 6.3.3 for Kcell = 4. Secondary channelisation codes are marked with a *. These associations apply both for UL and DL.

m (1) - c32(2)*

m (1) - c8(1)

m (1) - c16(1)

m (1) - c32(1)

m (1) - c4(1)

m (1) - c2(1)

m (1) - c1(1)

m (5) - c2(2)

m (3) - c4(2)

m (5) - c4(3)

m (7) - c4(4)

m (2) - c8(2)

m (3) - c8(3)

m (6) - c8(4)

m (5) - c8(5)

m (4) - c8(6)

m (7) - c8(7)

m (8) - c8(8)

m (9) - c16(2)

m (2) - c16(3)

m (10) - c16(4)

m (3) - c16(5)

m (11) - c16(6)

m (6) - c16(7)

m (14) - c16(8)

m (5) - c16(9)

m (13) - c16(10)

m (4) - c16(11)

m (12) - c16(12)

m (7) - c16(13)

m (15) - c16(14)

m (8) - c16(15)

m (16) - c16(16)

m (9) - c32(3)

m (9) - c32(4)*

m (2) - c32(5)

m (2) - c32(6)*

m (10) - c32(7)

m (10) - c32(8)*

m (3) - c32(9)

m (3) - c32(10)*

m (11) - c32(11)

m (11) - c32(12)*

m (6) - c32(13)

m (6) - c32(14)*

m (14) - c32(15)

m (14) - c32(16)*

m (5) - c32(17)

m (5) - c32(18)*

m (13) - c32(19)

m (13) - c32(20)*

m (4) - c32(21)

m (4) - c32(22)*

m (12) - c32(23)

m (12) - c32(24)*

m (7) - c32(25)

m (7) - c32(26)*

m (15) - c32(27)

m (15) - c32(28)*

m (8) - c32(29)

m (8) - c32(30)*

m (16) - c32(31)

m (16) - c32(32)*

Figure 6.3.1: Association of Midambles to Spreading Codes for KCell = 16

m (1) - c32(2)*

m (1) - c8(1)

m (1) - c16(1)

m (1) - c32(1)

m (1) - c4(1)

m (1) - c2(1)

m (1) - c1(1)

m (5) - c2(2)

m (3) - c4(2)

m (5) - c4(3)

m (7) - c4(4)

m (2) - c8(2)

m (3) - c8(3)

m (6) - c8(4)

m (5) - c8(5)

m (4) - c8(6)

m (7) - c8(7)

m (8) - c8(8)

m (1) - c16(2)*

m (2) - c16(3)

m (2) - c16(4)*

m (3) - c16(5)

m (3) - c16(6)*

m (6) - c16(7)

m (6) - c16(8)*

m (5) - c16(9)

m (5) - c16(10)*

m (4) - c16(11)

m (4) - c16(12)*

m (7) - c16(13)

m (7) - c16(14)*

m (8) - c16(15)

m (8) - c16(16)*

m (1) - c32(3)*

m (1) - c32(4)*

m (2) - c32(5)

m (2) - c32(6)*

m (2) - c32(7)*

m (2) - c32(8)*

m (3) - c32(9)

m (3) - c32(10)*

m (3) - c32(11)*

m (3) - c32(12)*

m (6) - c32(13)

m (6) - c32(14)*

m (6) - c32(15)*

m (6) - c32(16)*

m (5) - c32(17)

m (5) - c32(18)*

m (5) - c32(19)*

m (5) - c32(20)*

m (4) - c32(21)

m (4) - c32(22)*

m (4) - c32(23)*

m (4) - c32(24)*

m (7) - c32(25)

m (7) - c32(26)*

m (7) - c32(27)*

m (7) - c32(28)*

m (8) - c32(29)

m (8) - c32(30)*

m (8) - c32(31)*

m (8) - c32(32)*

Figure 6.3.2: Association of Midambles to Spreading Codes for KCell = 8

m (1) - c32(2)*

m (1) - c8(1)

m (1) - c16(1)

m (1) - c32(1)

m (1) - c4(1)

m (1) - c2(1)

m (1) - c1(1)

m (5) - c2(2)

m (3) - c4(2)

m (5) - c4(3)

m (7) - c4(4)

m (1) - c8(2)*

m (3) - c8(3)

m (3) - c8(4)*

m (5) - c8(5)

m (5) - c8(6)*

m (7) - c8(7)

m (7) - c8(8)*

m (1) - c16(2)*

m (1) - c16(3)*

m (1) - c16(4)*

m (3) - c16(5)

m (3) - c16(6)*

m (3) - c16(7)*

m (3) - c16(8)*

m (5) - c16(9)

m (5) - c16(10)*

m (5) - c16(11)*

m (5) - c16(12)*

m (7) - c16(13)

m (7) - c16(14)*

m (7) - c16(15)*

m (7) - c16(16)*

m (1) - c32(3)*

m (1) - c32(4)*

m (1) - c32(5)*

m (1) - c32(6)*

m (1) - c32(7)*

m (1) - c32(8)*

m (3) - c32(9)

m (3) - c32(10)*

m (3) - c32(11)*

m (3) - c32(12)*

m (3) - c32(13)*

m (3) - c32(14)*

m (3) - c32(15)*

m (3) - c32(16)*

m (5) - c32(17)

m (5) - c32(18)*

m (5) - c32(19)*

m (5) - c32(20)*

m (5) - c32(21)*

m (5) - c32(22)*

m (5) - c32(23)*

m (5) - c32(24)*

m (7) - c32(25)

m (7) - c32(26)*

m (7) - c32(27)*

m (7) - c32(28)*

m (7) - c32(29)*

m (7) - c32(30)*

m (7) - c32(31)*

m (7) - c32(32)*

Figure 6.3.3: Association of Midambles to Spreading Codes for KCell = 4

For RACH, up to 16 midambles and channelisation codes may be supported. The training sequences, i.e. midambles, of different users active in the same time slot are time shifted versions of a basic midamble code, m1, or a second basic midamble code, m2, which is a time inverted version of the basic midamble code m1. A fixed association exists between PRACH midambles and channelisation codes.

## 6.4	Coding and Modulation

Multiplexing and channel coding is aligned with 3.84Mcps TDD with the exception that physical channel sequence numbering and the coding of the channelisation code set information on HS-SCCH shall account for the support of SF32 at 7.68Mcps.

## 6.5	Scrambling Codes

The binary scrambling code, c7.68n, for cell parameter n in the 7.68Mcps TDD option is formed from the concatenation of the binary scrambling codes c3.84nand c3.84n+2mod128shown in Annex A of [4].

## 6.6	Synchronisation Codes

The synchronisation codes for the 7.68Mcps TDD option are formed by repetition coding of the 3.84Mcps TDD synchronisation code words. Unique modulation sequences are applied to these code words that enable the UE to determine the code group, frame alignment and chip rate of the cell.

The synchronization channel (SCH) is constructed in an identical manner to the construction at 3.84Mcps. The relationship between code group, n, and toffset,n at 7.68Mcps is:

toffset,n=

## 6.7	Transmit diversity

Support for beamforming and transmit diversity are aligned with the 3.84Mcps TDD option.

6.8	Measurements

## 6.9	Indicator Channels

### 6.9.1	Paging Indicator Channel (PICH)

The paging indicator channel is spread at SF32, but in other respects is identical to the 3.84Mcps TDD PICH [2].

The PICH block may comprise up to NPICH = 8 frames. The PCH block may comprise up to 2 × NPCH = 2 × 16 frames.

### 6.9.2	MBMS Indicator Channel (MICH)

The MBMS indicator channel is spread at SF32, but in other respects is identical to the 3.84Mcps TDD MICH [2].

## 6.10	Mapping of transport channels to physical channels

In the 7.68Mcps TDD option, transport channels are mapped onto physical channels according to figure 6.10.1.

| Transport Channels | Physical Channels |
| --- | --- |
| DCH | Dedicated Physical Channel (DPCH) |
| BCH | Primary Common Control Physical Channel (P-CCPCH) |
| FACH | Secondary Common Control Physical Channel (S-CCPCH) |
| PCH |  |
| RACH | Physical Random Access Channel (PRACH) |
| USCH | Physical Uplink Shared Channel (PUSCH) |
| DSCH | Physical Downlink Shared Channel (PDSCH) |
|  | Paging Indicator Channel (PICH) |
|  | MBMS Indication Channel (MICH) |
|  | Synchronisation Channel (SCH) |
| HS-DSCH | High Speed Physical Downlink Shared Channel (HS-PDSCH) |
|  | Shared Control Channel for HS-DSCH (HS-SCCH) |
|  | Shared Information Channel for HS-DSCH (HS-SICH) |

Figure 6.10.1: Transport channel to physical channel mapping

The mapping between DCH, BCH, FACH, USCH and DSCH transport channels to physical channels is identical to the mapping at 3.84Mcps TDD.

The mapping between the RACH transport channel and the PRACH physical channel is identical to the mapping at 3.84Mcps TDD.

The mapping between the HS-DSCH transport channel and HS-PDSCH physical channels is identical to the mapping at 3.84Mcps TDD. The association and timing between HS-SCCH, HS-DSCH and HS-SICH is identical to the association and timing at 3.84Mcps TDD with the exception that the UE must monitor up to a maximum of eight HS-SCCH (M=8).

# 7	Physical layer procedures

## 7.1	Power Control

Transmitter power control, both on the uplink and downlink, is aligned with that of 3.84Mcps TDD.

## 7.2	Timing Advance

The timing advance architecture is the same as for 3.84Mcps TDD. The required timing advance, 'UL Timing Advance' TAul will be represented as a 7 bit number (0-127) and shall be the multiplier of 4 chips which is nearest to the required timing advance.

PUSCH, UL DPCH and HS-SICH are timing advanced. PRACH is not timing advanced.

## 7.3	HSDPA procedures

The HS-DSCH procedure is aligned with 3.84Mcps TDD. When SCTD antenna diversity is applied to HS-PDSCH on the beacon channel, the presence of channelisation code c32(k=1)shall implicitly indicate presence of channelisation code c32(k=2).

## 7.4	Synchronisation procedures

The synchronization procedures are aligned with 3.84Mcps TDD.

## 7.5	RACH procedures

The RACH procedure is aligned with 3.84Mcps TDD. However, the use of higher layer signaling to indicate that in some frames a timeslot shall be blocked for RACH uplink transmission is not supported.

7.6	Discontinuous transmission (DTX) procedure

The DTX procedure is aligned with that of 3.84Mcps TDD.

7.7	Downlink transmit diversity procedure

The downlink transmit diversity procedure is aligned with that of 3.84Mcps TDD. In Space Code Transmit Diversity mode the data sequence is spread with the channelisation codes c32(k=1)and c32(k=2), the spread sequence on code c32(k=2) is then transmitted on the diversity antenna.

7.8	DSCH procedure

Higher layer signaling is used to indicate to the UE the need for PDSCH detection. Physical layer signaling is not used to indicate to the UE the need for PDSCH detection.

7.9	Macrodiversity procedure

The macrodiversity procedure is aligned with that of 3.84Mcps TDD.

  - IPDL procedure

The IPDL procedure is aligned with that of 3.84Mcps TDD.

# 8	UE capabilities

UE capabilities for the 7.68 Mcps TDD mode are based on those for 3.84 Mcps TDD. The capabilities for 7.68Mcps TDD account for the higher number of physical channels supported and additionally support higher peak bit rates. The minimum MBMS capability at 7.68Mcps is twice the minimum capability at 3.84Mcps. The detailed UE capabilities for 7.68Mcps TDD are described in [8].

# 9	Layer 2/3 protocol aspects

## 9.1	Protocol architecture

The protocol architecture for 7.68 Mcps TDD is the same as the protocol architecture for 3.84 Mcps TDD. Section 5.1 of [7] provides an overview of the radio interface protocol architecture.

## 9.2	Signalling

### 9.2.1	General

There are signalling differences between 7.68 Mcps TDD and 3.84 Mcps TDD. These differences concern L2/MAC and L3/RRC (see Section 5.1 of [7]) only. L2/RLC, L2/BMC, L2/PDCP and L3 U-plane information are not impacted.

### 9.2.2	L2/MAC differences

The L2/MAC differences between 7.68Mcps TDD and 3.84Mcps TDD are due to the support of a higher capability HSDPA UE at 7.68Mcps (20.4Mbps). The L2/MAC differences concern:

-	the maximum number of PDUs transmitted in a single TTI (636 at 7.68Mcps compared to 318 for3.84 Mcps TDD).

-	HSPDA transport block size signalling. The maximum transport block size that can be signalled at 7.68Mcps is twice that at 3.84Mcps. A new table and formula for transport block size signalling for 7.68 Mcps TDD is included in [9].

### 9.2.3	L2/RRC differences

The L2/RRC differences concern:

Use of SF 32: The signalling is extended to include support for SF32. The 7.68 Mcps cell will be configured to use SF 16 or 32 for PRACH rather than SF 8 and 16 as 3.84 Mcps

Open Loop Power Control: Configuration of a the cell for use of SF 16 or 32 with respect to the PRACH impacts calculation of the uplink transmit power for PRACH and requires the UE to add 3dB to the RACH Constant Value in the equation:

PPRACH = LPCCPCH + IBTS + PRACH Constant value

for the case where RACH Spreading Factor = 16.

Capability Update Requirement: A new IE "UE radio access 7.68 Mcps TDD capability update requirement" is used.

Uplink Timing Advance: A different Uplink Timing Advance IE is required at 7.68Mcps to account for the number of bits used to signal timing advance at 7.68Mcps. A number of RRC messages are impacted due to the use of a different Uplink Timing Advance IE for 7.68 Mcps TDD to 3.84 Mcps TDD.

DL Physical Channel Capability: The physical channel capability at 7.68Mcps is extended in order to account for the greater number of physical channels supported at 7.68Mcps.

Burst Types and Midambles: Signalling related to burst types is modified since burst type 2 at 7.68Mcps supports Kcell of 4 or 8.

## 9.3	HSDPA related issues

The highest UE capability at 7.68Mcps is double that at 3.84Mcps, hence the maximum transport block size and the maximum number of PDUs that can be transmitted in a single TTI are double that of 3.84 Mcps. The range of UE capabilities is extended and the maximum UE capablitity for 7.68 Mcps is 20.4 Mbits/s.

## 9.4	Mobility

Inter RAT and intra RAT handover for 7.68 Mcps TDD is as for 3.84 Mcps TDD with handover between 3.84 Mcps TDD and 7.68 Mcps TDD cells also supported. Bands a), b), c), a + b), a + c), b + c) and a + b + c) can be configured for 7.68 Mcps TDD or 3.84 Mcps TDD or 1.28 Mcps TDD.

## 9.5	Idle Mode Procedures

Idle mode procedures are as for 3.84 Mcps TDD.

# 10	Iub/Iur aspects

## 10.1  Impacts on Iub/Iur interfaces – general aspects

### 10.1.1	Timing advance and Rx Timing Deviation

The timing advance algorithm (in RRM, at the RNC) uses Rx Timing Deviation measurements made by the Node B and passed to the RNC in frame protocols.  At 3.84 Mcps the resolution is 4 chips.  The timing advance determined by RRM is signalled to the UE (RRC).

In addition, the Node B can be configured to take more accurate Rx Timing Deviation measurements of a UE, which are sent to the RNC as dedicated measurements.  At 3.84 Mcps the resolution of these is 0.0625 chips.  These accurate measurements can be used in location (they are passed to the location system using the PCAP protocol).

Strategy for 7.68 Mcps :

Timing advance & Rx Timing Deviation over FP

>	4 chip resolution

>	same dynamic range as 3.84 Mcps (in secs)

Rx Timing Deviation, dedicated measurement

>	0.0625 chip resolution giving greater measurement accuracy

>	same dynamic range as 3.84 Mcps (in secs)

### 10.1.2	Paging

For the 7.68Mcps option, the maximum number of paging indicators per paging block should be doubled to accommodate the greater number of users that may be supported by the 10 MHz carrier.  To achieve this:

•	the number of PICH blocks per paging block (NPICH) is extended from {2,4} to {2,4,8}

•	the number of PCH blocks per paging block (NPCH) is extended from {1..8} to {1..16}.

Consequently, a unique value range for the PI-bitmap needs to be defined for 7.68 Mcps.

### 10.1.3	DSCH Power Control from the RNC

In 3.84 Mcps TDD, the PDSCH may be power controlled from the RNC by sending a transmit power level value in the DSCH DATA FRAME that carries DSCH transport blocks to the Node B.  For 7.68 Mcps, the same method can be used and this has been agreed by RAN1.  Since the transmit power level is expressed relative to the maximum transmit power, no changes are needed to accommodate 7.68Mcps.

## 10.2  Impacts on Iub/Iur control plane protocols

There are a number of changes to RNSAP, PCAP & NBAP protocols to incorporate:

- Use of SF 32: The signalling is extended to include support for SF32. The 7.68 Mcps cell will be configured to use SF 16 or 32 for PRACH rather than SF 8 and 16 as 3.84 Mcps

- Burst Types and Midambles: Signalling related to burst types is modified since burst type 2 at 7.68Mcps supports Kcell of 4 or 8.

- Number of physical channels: the SF32 change implies an increase in the number of physical channels that may be supported.

- Measurements: changes are introduced for Rx Timing Deviation and SFN-SFN measurements.

- Cell Synchronisation: this procedure is not supported.

## 10.3  Impacts on Iub/Iur user plane protocols

Specifications 25.425, 25.427 and 25.435 are modified to include 7.68Mcps operation in a similar fashion to 3.84 Mcps.  Changes are also needed to accommodate the different rx timing deviation and timing advance signalling for 7.68Mcps compared to 3.84Mcps (see Section 10.1 above).  The paging indicator bit-map is also revised (see Section 10.1 above).

# 11	Radio aspects

## 11.1	UE radio transmission and reception

### 11.1.1		Transmitter characteristics

#### 11.1.1.1	Transmit power

Common with 3.84Mcps TDD option.

#### 11.1.1.2	Output RF spectrum emissions

##### 11.1.1.2.1	Occupied bandwidth

Occupied bandwidth is a measure of the bandwidth containing 99% of the total integrated power of the transmitted spectrum, centred on the assigned channel frequency. The occupied channel bandwidth shall be less than 10 MHz based on a chip rate of 7.68 Mcps.

##### 11.1.1.2.2	Out of band emission

Out of band emissions are unwanted emissions immediately outside the nominal channel resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. This out of band emission limit is specified in terms of a spectrum emission mask and adjacent channel leakage power ratio (ACLR).

###### 11.1.1.2.2.1	Spectrum emission mask

The spectrum emission mask of the UE applies to frequencies, which are between 5 MHz and 25MHz from the UE centre carrier frequency. The out of channel emission is specified relative to the RRC filtered mean power of the UE carrier. The power of any UE emission shall not exceed the levels specified in Table 11.1.1.

Table 11.1.1: Spectrum Emission Mask of higher chip rate reference configuration

| Δf* in MHz | Minimum requirement | Measurement bandwidth |
| --- | --- | --- |
| 5.0 – 7.0 | −38−7.5⋅ΔfMHz−5.0dBc | 30 kHz ** |
| 7.0 - 15 | −38−0.5⋅ΔfMHz−7.0dBc | 1 MHz *** |
| 15.0 – 17.0 | −42−5.0⋅ΔfMHz−15.0dBc | 1 MHz *** |
| 17.0 – 25.0 | -53 dBc | 1 MHz *** |
| * f is the separation between the carrier frequency and the centre of the measuring filter. |  |  |
| ** The first and last measurement position with a 30 kHz filter is at f equals to 5.015 MHz and 6.985 MHz |  |  |
| *** The first and last measurement position with a 1 MHz filter is at f equals to 7.5 MHz and 24.5 MHz. As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be different from the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. |  |  |
| The lower limit shall be –47dBm/7.68 MHz or the minimum requirement presented in this table which ever is the higher. |  |  |

###### 11.1.1.2.2.2	Adjacent Channel Leakage power Ratio (ACLR)

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the RRC filtered mean power centered on the assigned channel frequency to the RRC filtered mean power centered on an adjacent channel frequency.

If the adjacent channel RRC filtered mean power is greater than –50dBm measured with a 3.84 Mcps RRC filter then the ACLR shall be higher than the value specified in Table 11.1.2.

Table 11.1.2: UE ACLR of higher chip rate reference configuration

| Power Class | adjacent channel | Chip Rate for RRC Measurement Filter | ACLR limit |
| --- | --- | --- | --- |
| 2, 3 | UE channel ± 7.5 MHz | 3.84 MHz | 33 dB |
| 2, 3 | UE channel ± 12.5 MHz | 3.84 MHz | 43 dB |
| 2 ,3 | UE channel ± 20.0 MHz | 7.68 MHz | 43 dB |

NOTE:

1)	The requirement shall still be met in the presence of switching transients.

2)	The ACLR requirements reflect what can be achieved with present state of the art technology.

###### 11.1.1.2.2.3	Spurious emissions

The spurious emissions limits shall be common with 3.84 Mcps TDD option and shall be applicable for offsets greater than 25 MHz from the UE centre frequency.

### 11.1.2		Receiver characteristics

#### 11.1.2.1	Reference sensitivity level

The reference sensitivity level is the minimum mean power received at the UE antenna port at which the BIT Error Ratio BER shall not exceed a specific value.

##### 11.1.2.1.1	Minimum Requirement

The BER shall not exceed 0.001 for the parameters specified in Table 11.1.3.

Table 11.1.3: Test parameters for reference sensitivity (7.68 Mcps TDD Option)

| Parameter | Level | Unit |
| --- | --- | --- |
|  | 0 | dB |
|  | -105 | dBm/7.68 MHz |

#### 11.1.2.2	Adjacent Channel Selectivity (ACS)

Adjacent Channel Selectivity is a measure of a receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receiver filter attenuation on the adjacent channel(s).

##### 11.1.2.2.1	Minimum Requirement

The ACS shall be better than the value indicated in Table 11.1.4 for the test parameters specified in Table 11.1.5 where the BER shall not exceed 0.001

Table 11.1.4: Adjacent Channel Selectivity (7.68 Mcps TDD Option)

| Power Class | Unit | ACS |
| --- | --- | --- |
| 2 | dB | 33 |
| 3 | dB | 33 |

Table 11.1.5: Test parameters for Adjacent Channel Selectivity (7.68 Mcps TDD Option)

| Parameter | Unit | Level |
| --- | --- | --- |
|  | dB | 0 |
| Îor | dBm/7.68 MHz | -91 |
| Ioac mean power (modulated) | dBm | -52 |
| Fuw offset (3.84 Mcps Modulated) | MHz | +7.5 or –7.5 |
| Fuw offset (7.68 Mcps Modulated) | MHz | +10 or -10 |

#### 11.1.2.3	Blocking characteristics

The blocking characteristics is a measure of the receiver ability to receive a wanted signal at is assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occur.

##### 11.1.2.3.1	Minimum Requirement

The BER shall not exceed 0.001 for the parameters specified in table 11.1.6 and table 11.1.7. For table 11.1.7 up to 24 exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size.

Table 11.1.6: In-band blocking

| Parameter | Level |  | Unit |
| --- | --- | --- | --- |
| ΣDPCHEcIor | 0 |  | dB |
| Ior | -102 |  | dBm/7.68 MHz |
| Iouwmean power (modulated) | -53 (for Fuw offset ±20 MHz) | -41 (for Fuw offset ±30 MHz) | dBm |

Table 11.1.7: Out of band blocking

| Parameter | Band 1 | Band 2 | Band 3 | Unit |
| --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | dB |
|  | –102 | –102 | –102 | dBm/7.68 MHz |
| (CW) | -44 | -30 | -15 | dBm |
| Fuw For operation in frequency bands as definded in subclause 5.2(a) of TS25.102 [10] | 1840 <f <1870 1950 <f <1980 2055 <f <2085 | 1815 <f <1840 2085 <f <2110 | 1< f <1815 2110< f <12750 | MHz |
| Fuw For operation in frequency bands as definded in subclause 5.2(b) of TS25.102 [10] | 1790 < f < 1820 2020 < f < 2050 | 1765 < f < 1790 2050 < f < 2075 | 1 < f < 1765 2075 < f < 12750 | MHz |
| Fuw For operation in frequency bands as definded in subclause 5.2(c) of TS25.102 [10] | 1850 < f < 1880 1960 < f < 1990 | 1825 < f < 1850 1990 < f < 2015 | 1 < f < 1825 2015 < f < 12750 | MHz |
| 1. For operation referenced in 5.2(a) of TS25.102 [10], from 1870 <f< 1900 MHz, 1920 <f< 1950 MHz, 1980 <f< 2010 MHz and 2025<f< 2055 MHz , the appropriate in-band blocking in table 11.1.6 or adjacent channel selectivity in section 11.1.4 shall be applied. |  |  |  |  |
| 2. For operation referenced in 5.2(b) of TS25.102 [10], from 1820 < f < 1850 MHz and 1990< f < 2020 MHz, the appropriate in-band blocking in table 11.1.6 or adjacent channel selectivity in section 11.1.4 shall be applied. |  |  |  |  |
| 3. For operation referenced in 5.2(c) of TS25.102 [10], from 1880 < f < 1910 MHz and 1930< f < 1960 MHz, the appropriate in-band blocking in table 11.1.6 or adjacent channel selectivity in section 11.1.4 shall be applied. |  |  |  |  |

#### 11.1.2.4	Spurious response

Spurious response is a measure of the receiver's ability to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency at which a response is obtained i.e. for which the blocking limit is not met.

##### 11.1.2.4.1	Minimum Requirement

The BER shall not exceed 0.001 for the parameters specified in Table 11.1.8.

Table 11.1.8: Spurious Response

| Parameter | Level | Unit |
| --- | --- | --- |
|  | 0 | dB |
|  | –102 | dBm/7.68 MHz |
| (CW) | -44 | dBm |
| Fuw | Spurious response frequencies | MHz |

#### 11.1.2.5	Spurious emissions

The Spurious Emissions Power is the power of emissions generated or amplified in a receiver that appear at the UE antenna connector.

##### 11.1.2.5.1	Minimum Requirement

The power of any spurious emission shall not exceed:

Table 11.1.9: Receiver spurious emission requirements

| Band | Maximum level | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -57 dBm | 100 kHz |  |
| 1 GHz – 1.9 GHz and 1.92 GHz – 2.01 GHz and 2.025 GHz – 2.11 GHz | -47 dBm | 1 MHz | With the exception of frequencies between 25MHz below the first carrier frequency and 25MHz above the last carrier frequency used by the UE. |
| 1.9 GHz – 1.92 GHz and 2.01 GHz – 2.025 GHz and 2.11 GHz – 2.170 GHz | -57 dBm | 7.68 MHz | With the exception of frequencies between 25MHz below the first carrier frequency and 25MHz above the last carrier frequency used by the UE. |
| 2.170 GHz – 12.75 GHz | -47 dBm | 1 MHz |  |

## 11.2	Base station radio transmission and reception

### 11.2.1		Transmitter characteristics

#### 11.2.1.1	Base station output power

Common with 3.84Mcps TDD option.

#### 11.2.1.2	Output RF spectrum emissions

##### 11.2.1.2.1	Occupied bandwidth

Occupied bandwidth is a measure of the bandwidth containing 99% of the total integrated power for transmitted spectrum and is centered on the assigned channel frequency. The occupied channel bandwidth is less than 10 MHz based on a chip rate of 7.68 Mcps.

##### 11.2.1.2.2	Out of band emission

Out of band emissions are unwanted emissions immediately outside the channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. This out of band emission requirement is specified both in terms of a spectrum emission mask and adjacent channel power ratio for the transmitter.

###### 11.2.1.2.2.1	Spectrum emission mask

The mask defined in Table 11.2.1 to 11.2.4 below may be mandatory in certain regions. In other regions this mask may not be applied.

For regions where this clause applies, the requirement shall be met by a base station transmitting on a single RF carrier configured in accordance with the manufacturer's specification. Emissions shall not exceed the maximum level specified in tables 11.2.1 to 11.2.4 for the appropriate BS maximum output power, in the frequency range from f  = 5 MHz to f max from the carrier frequency, where:

-	f is the separation between the carrier frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the carrier frequency and the center frequency of the measuring filter.-	f_offsetmax is either 25 MHz or the offset to the UMTS Tx band edge as defined in TS25.105 [11], whichever is the greater.

-	f max  is equal to f_offsetmax   minus half of the bandwidth of the measurement filter.

Figure 11.2.1: Spectrum emission mask

Table 11.2.1: Spectrum emission mask values, BS maximum output power P  43 dBm

| Frequency offset of measurement filter –3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Maximum level | Measurement bandwidth |
| --- | --- | --- | --- |
| 5 MHz  f < 5.2 MHz | 5.015MHz  f_offset < 5.215MHz | -17 dBm | 30 kHz |
| 5.2 MHz  f < 6 MHz | 5.215MHz  f_offset < 6.015MHz | −17dBm−15⋅foffsetMHz−5.215dB | 30 kHz |
| (see note) | 6.015MHz  f_offset < 6.5MHz | -29 dBm | 30 kHz |
| 6 MHz  f  fmax | 6.5MHz  f_offset < f_offsetmax | -16 dBm | 1 MHz |

Table 11.2.2: Spectrum emission mask values, BS maximum output power 39  P < 43 dBm

| Frequency offset of measurement filter –3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Maximum level | Measurement bandwidth |
| --- | --- | --- | --- |
| 5 MHz  f < 5.2 MHz | 5.015MHz  f_offset < 5.215MHz | -17 dBm | 30 kHz |
| 5.2 MHz  f < 6 MHz | 5.215MHz  f_offset < 6.015MHz | −17dBm−15⋅foffsetMHz−5.215dB | 30 kHz |
| (see note) | 6.015MHz  f_offset < 6.5MHz | -29 dBm | 30 kHz |
| 6 MHz  f < 15 MHz | 6.5MHz  f_offset < 15.5MHz | -16 dBm | 1 MHz |
| 15 MHz  f  fmax | 15.5MHz  f_offset < f_offsetmax | P - 59 dB | 1 MHz |

Table 11.2.3: Spectrum emission mask values, BS maximum output power 31  P < 39 dBm

| Frequency offset of measurement filter –3dB point,f | Frequency offset of measurement filter centre frequency, f_offset | Maximum level | Measurement bandwidth |
| --- | --- | --- | --- |
| 5 MHz  f < 5.2 MHz | 5.015MHz  f_offset < 5.215MHz | P - 56 dB | 30 kHz |
| 5.2 MHz  f < 6 MHz | 5.215MHz  f_offset < 6.015MHz | P−56dB−15⋅foffsetMHz−5.215dB | 30 kHz |
| (see note) | 6.015MHz  f_offset < 6.5MHz | P – 68 dB | 30 kHz |
| 6 MHz  f < 15 MHz | 6.5MHz  f_offset < 15.5MHz | P – 55 dB | 1 MHz |
| 15 MHz  f  fmax | 15.5MHz  f_offset < f_offsetmax | P - 59 dB | 1 MHz |

Table 11.2.4: Spectrum emission mask values, BS maximum output power  P < 31 dBm

| Frequency offset of measurement filter –3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Maximum level | Measurement bandwidth |
| --- | --- | --- | --- |
| 5 MHz  f < 5.2 MHz | 5.015MHz  f_offset < 5.215MHz | -25 dBm | 30 kHz |
| 5.2 MHz  f < 6 MHz | 5.215MHz  f_offset < 6.015MHz | −25dBm−15⋅foffsetMHz−5.215dB | 30 kHz |
| (see note) | 6.015MHz  f_offset < 6.5MHz | -37 dBm | 30 kHz |
| 6 MHz  f < 15 MHz | 6.5MHz  f_offset < 15.5MHz | -24 dBm | 1 MHz |
| 15 MHz  f  fmax | 15.5MHz  f_offset < f_offsetmax | -28 dBm | 1 MHz |

NOTE:	This frequency range ensures that the range of values of f_offset is continuous.

###### 11.2.1.2.2.2	Adjacent Channel Leakage power Ratio (ACLR)

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the RRC filtered mean power centered on the assigned channel frequency to the RRC filtered mean power centered on an adjacent channel frequency. The requirements shall apply for all configurations of BS (single carrier or multi-carrier), and for all operating modes foreseen by the manufacturer's specification.

In some cases the requirement is expressed as adjacent channel leakage power, which is the RRC filtered mean power for the given bandwidth of the victim system at the defined adjacent channel offset.

The requirement depends on the deployment scenario. Different deployment scenarios have been defined as given below.

11.2.1.2.2.2.1	Minimum requirement

The ACLR of a single carrier BS or a multi-carrier BS with contiguous carrier frequencies shall be higher than the value specified in Table 11.2.5.

Table 11.2.5: BS ACLR

| BS adjacent channel offset below the first or above the last carrier frequency used | Chip Rate for RRC Measurement Filter | ACLR limit |
| --- | --- | --- |
| 7.5 MHz | 3.84 Mcps | 45 dB |
| 12.5 MHz | 3.84 Mcps | 55 dB |
| 10.0 MHz | 7.68 Mcps | 45 dB |
| 20.0 MHz | 7.68 Mcps | 55 dB |

If a BS provides multiple non-contiguous single carriers or multiple non-contiguous groups of contiguous single carriers, the above requirements shall be applied individually to the single carriers or group of single carriers.

###### 11.2.1.2.2.3	Spurious emissions

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emission, intermodulation products and frequency conversion products, but exclude out of band emissions. This is measured at the base station RF output port.

The requirements shall apply whatever the type of transmitter considered (single carrier or multi carrier). It applies for all transmission modes foreseen by the manufacturer.

The requirement applies at frequencies within the specified frequency ranges which are more than 25 MHz under the first carrier frequency used or more than 25 MHz above the last carrier frequency used.

The mandatory requirements for Category A and Cateogry B shall be common with 3.84 Mcps TDD option.

### 11.2.2		Receiver characteristics

#### 11.2.2.1	Reference sensitivity level

The reference sensitivity level is the minimum mean power received at the antenna connector at which the BER shall not exceed the specific value indicated in section 11.2.2.1.1.

##### 11.2.2.1.1	Minimum requirement

The UL reference measurement channel used in the simulations of TR25.895 is the 12.2 kbps channel specified in Annex A.2.1 of TS25.105 [11] with twice the spreading factor (SF=16) and mid-amble (1024 chips).  The reference sensitivity level and performance of the BS shall be as specified in Table 11.2.6.

Table 11.2.6: BS reference sensitivity level

| BS Class | Reference measurement channel data rate | BS reference sensitivity level | BER |
| --- | --- | --- | --- |
| Wide Area BS | 12.2 kbps | -109 dBm | BER shall not exceed 0.001 |
| Local Area BS | 12.2 kbps | -95 dBm | BER shall not exceed 0.001 |

#### 11.2.2.2	Adjacent Channel Selectivity (ACS)

Adjacent channel selectivity (ACS) is a measure of the receiver ability to receive a wanted signal at its assigned channel frequency in the presence of a single code CDMA modulated adjacent channel signal at a given frequency offset from the center frequency of the assigned channel. ACS is the ratio of the receiver filter attenuation on the assigned channel frequency to the receiver filter attenuation on the adjacent channel(s).

##### 11.2.2.2.1	Minimum requirement

The BER shall not exceed 0.001 for the parameters specified in table 11.2.7.

Table 11.2.7: Adjacent channel selectivity

| Parameter |  | Level | Unit |
| --- | --- | --- | --- |
| Reference measurement channel data rate |  | 12.2 | kbps |
| Wanted signal mean power | Wide Area BS | -103 | dBm |
|  | Local Area BS | -89 | dBm |
| Interfering signal mean power | Wide Area BS | –49 | dBm |
|  | Local Area BS | –35 | dBm |
| Fuw offset (Modulated) |  | 10 | MHz |

#### 11.2.2.3	Blocking characteristics

The blocking characteristics is a measure of the receiver ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of  the adjacent channels. The blocking performance requirement applies to interfering signals with center frequency within the ranges specified in the tables below, using a 1MHz step size.

##### 11.2.2.3.1	Minimum requirement

The static reference performance as specified in clause 11.2.2.1.1 shall be met with a wanted and an interfering signal coupled to BS antenna input using the parameters as specified in Table 11.2.8 to 11.2.10 for the Wide Area BS and as specified in Table 11.2.11 to 11.2.13 for the Local Area BS.

Table 11.2.8: Blocking requirements for Wide Area BS for operating bands defined in 5.2(a) of TS 25.105 [11]

| Centre Frequency of Interfering Signal | Interfering Signal Mean Power | Wanted Signal Mean Power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| 1900 – 1920 MHz, 2010 – 2025 MHz | -40 dBm | -103 dBm | 20 MHz | WCDMA signal with one code |
| 1880 – 1900 MHz, 1990 – 2010 MHz, 2025 – 2045 MHz | -40 dBm | -103 dBm | 20 MHz | WCDMA signal with one code |
| 1920 – 1980 MHz | -40 dBm | -103 dBm | 20 MHz | WCDMA signal with one code |
| 1 – 1880 MHz, 1980 – 1990 MHz, 2045 – 12750 MHz | -15 dBm | -103 dBm |  | CW carrier |

Table 11.2.9: Blocking requirements for Wide Area BS for operating bands defined in 5.2(b) of TS 25.105 [11]

| Centre Frequency of Interfering Signal | Interfering Signal Mean Power | Wanted Signal Mean Power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| 1850 – 1990 MHz | -40 dBm | -103 dBm | 20 MHz | WCDMA signal with one code |
| 1830 – 1850 MHz, 1990 – 2010 MHz | -40 dBm | -103 dBm | 20 MHz | WCDMA signal with one code |
| 1 – 1830 MHz, 2010 – 12750 MHz | -15 dBm | -103 dBm |  | CW carrier |

Table 11.2.10: Blocking requirements for Wide Area BS for operating bands defined in 5.2(c) of TS 25.105 [11]

| Centre Frequency of Interfering Signal | Interfering Signal Mean Power | Wanted Signal Mean Power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| 1910 – 1930 MHz | -40 dBm | -103 dBm | 20 MHz | WCDMA signal with one code |
| 1890 – 1910 MHz, 1930 – 1950 MHz | -40 dBm | -103 dBm | 20 MHz | WCDMA signal with one code |
| 1 – 1890 MHz, 1950 – 12750 MHz | -15 dBm | -103 dBm |  | CW carrier |

Table 11.2.11: Blocking requirements for Local Area BS for operating bands defined in 5.2(a) of TS25.105 [11]

| Centre Frequency of Interfering Signal | Interfering Signal mean power | Wanted Signal mean power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| 1900 – 1920 MHz, 2010 – 2025 MHz | -30 dBm | –89 dBm | 20 MHz | WCDMA signal with one code |
| 1880 – 1900 MHz, 1990 – 2010 MHz, 2025 – 2045 MHz | -30 dBm | –89 dBm | 20 MHz | WCDMA signal with one code |
| 1920 – 1980 MHz | -30 dBm | –89 dBm | 20 MHz | WCDMA signal with one code |
| 1 – 1880 MHz, 1980 – 1990 MHz, 2045 – 12750 MHz | -15 dBm | –89 dBm |  | CW carrier |

Table 11.2.12: Blocking requirements for Local Area BS for operating bands defined in 5.2(b) of TS 25.105 [11]

| Centre Frequency of Interfering Signal | Interfering Signal mean power | Wanted Signal mean power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| 1850 – 1990 MHz | -30 dBm | –89 dBm | 20 MHz | WCDMA signal with one code |
| 1830 – 1850 MHz, 1990 – 2010 MHz | -30 dBm | –89 dBm | 20 MHz | WCDMA signal with one code |
| 1 – 1830 MHz, 2010 – 12750 MHz | -15 dBm | –89 dBm |  | CW carrier |

Table 11.2.13: Blocking requirements for Local BS for operating bands defined in 5.2(c) of TS25.105 [11]

| Centre Frequency of Interfering Signal | Interfering Signal mean power | Wanted Signal mean power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| 1910 – 1930 MHz | -30 dBm | –89 dBm | 20 MHz | WCDMA signal with one code |
| 1890 – 1910 MHz, 1930 – 1950 MHz | -30 dBm | –89 dBm | 20 MHz | WCDMA signal with one code |
| 1 – 1890 MHz, 1950 – 12750 MHz | -15 dBm | –89 dBm |  | CW carrier |

##### 11.2.2.3.2	Collocation with GSM900 and/or DCS 1800

Common with 3.84 Mcps TDD option.

#### 11.2.2.4	Spurious emissions

The spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the BS antenna connector. The requirements apply to all BS with separate RX and TX antenna port. The test shall be performed when both TX and RX are on with the TX port terminated.

##### 11.2.2.4.1	Minimum requirement

The power of any spurious emission shall not exceed:

Table 11.2.14: Receiver spurious emission requirements

| Band | Maximum level | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -57 dBm | 100 kHz |  |
| 1 GHz – 1.9 GHz and 1.98 GHz – 2.01 GHz | -47 dBm | 1 MHz | With the exception of frequencies between 25MHz below the first carrier frequency and 25MHz above the last carrier frequency used by the BS. |
| 1.9 GHz – 1.98 GHz and 2.01 GHz – 2.025 GHz | -75 dBm | 7.68 MHz | With the exception of frequencies between 25MHz below the first carrier frequency and 25MHz above the last carrier frequency used by the BS. |
| 2.025 GHz – 12.75 GHz | -47 dBm | 1 MHz | With the exception of frequencies between 25MHz below the first carrier frequency and 25MHz above the last carrier frequency used by the BS. |

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2005-04 | RAN1#40bis | R1-050343 |  |  | Frame Structure for 7.68Mcps TDD Option | - | 0.0.1 |
| 2005-04 | RAN1#40bis | R1-050344 |  |  | Timing Advance for 7.68Mcps TDD Option | - | 0.0.1 |
| 2005-05 | RAN1#41 |  |  |  | V0.0.1 approved by RAN1. | 0.0.1 | 0.1.0 |
| 2005-08 | RAN1#41 | R1-050455 |  |  | Services offered to higher layers by 7.68Mcps TDD option | 0.1.0 | 0.1.1 |
| 2005-08 | RAN1#41 | R1-050556 |  |  | Spreading factors and burst types for 7.68Mcps TDD option | 0.1.0 | 0.1.1 |
| 2005-09 | RAN1#42 | R1-050974 |  |  | V0.1.1 approved by RAN1 | 0.1.1 | 0.2.0 |
| 2005-09 | RAN1#42 | R1-050858 |  |  | 7.68Mcps TDD: Mapping of transport channels to physical channels | 0.2.0 | 0.2.1 |
| 2005-09 | RAN1#42 | R1-050859 |  |  | 7.68Mcps TDD: Paging aspects | 0.2.0 | 0.2.1 |
| 2005-09 | RAN1#42 | R1-050860 |  |  | 7.68Mcps TDD: PRACH Aspects | 0.2.0 | 0.2.1 |
| 2005-09 | RAN1#42 | R1-050861 |  |  | 7.68Mcps TDD: midamble aspects | 0.2.0 | 0.2.1 |
| 2005-09 | RAN1#42 | R1-050862 |  |  | 7.68Mcps TDD: Transmission of TPC and TFCI | 0.2.0 | 0.2.1 |
| 2005-09 | RAN1#42 | R1-050930 |  |  | Synchronisation aspects for 7.68Mcps TDD option | 0.2.0 | 0.2.1 |
| 2005-09 | RAN1#42 | R1-050931 |  |  | 7.68Mcps TDD: Transmitter power control | 0.2.0 | 0.2.1 |
| 2005-10 | RAN1#42bis | R1-051251 |  |  | V0.2.1 approved by RAN1 | 0.2.1 | 0.3.0 |
| 2005-10 | RAN1#42bis | R1-051223 |  |  | 7.68Mcps TDD option: HSDPA aspects of TS25.221 | 0.3.0 | 0.3.1 |
| 2005-10 | RAN1#42bis | R1-051224 |  |  | SCH channel definition for the 7.68Mcps TDD option | 0.3.0 | 0.3.1 |
| 2005-10 | RAN1#42bis | R1-051226 |  |  | Physical layer procedures for the 7.68Mcps TDD option | 0.3.0 | 0.3.1 |
| 2005-10 | RAN1#42bis | R1-051228 |  |  | Tx diversity for the 7.68Mcps TDD option | 0.3.0 | 0.3.1 |
| 2005-10 | RAN1#42bis | R1-051230 |  |  | 7.68Mcps TDD option: beacon channel aspects | 0.3.0 | 0.3.1 |
| 2005-11 | RAN1#43 | R1-051564 |  |  | V0.3.1 approved by RAN1 | 0.3.1 | 0.4.0 |
| 2005-11 | RAN1#43 | R1-051520 |  |  | Transport channel processing for the 7.68Mcps TDD option | 0.4.0 | 0.4.1 |
| 2005-11 | RAN1#43 | R1-051522 |  |  | Spreading and modulation for the 7.68Mcps TDD option | 0.4.0 | 0.4.1 |
| 2005-11 | RAN1#43 | R1-051620 |  |  | Updated to v0.4.2 | 0.4.1 | 0.4.2 |
| 2005-11 | TSG-RAN#30 | RP-050829 |  |  | v1.0.0 created for presentation to RAN plenary for information | 0.4.2 | 1.0.0 |
| 2006-02 | RAN1#44 | R1-060628 |  |  | MICH Aspects for the 7.68Mcps TDD option | 1.0.0 | 1.1.0 |
| 2006-02 | RAN1#44 | R1-060629 |  |  | IPDL Aspects for the 7.68Mcps TDD option | 1.0.0 | 1.1.0 |
| 2006-02 | RAN1#44 | R1-060728 |  |  | Response LS from RAN3 on input to TS25.202 | 1.1.0 | 1.1.1 |
| 2006-02 | RAN1#44 | R1-060744 |  |  | Response LS from RAN2 on input to TS25.202 | 1.1.0 | 1.1.1 |
| 2006-02 | RAN1#44 | R1-060736 |  |  | Response LS from RAN4 on input to TS25.202 | 1.1.0 | 1.1.1 |
| 2006-03 | RAN_31 | RP-060117 |  |  | v2.0.0 created for presentation to RAN plenary for approval | 1.1.1 | 2.0.0 |
| 20/03/06 | RAN_31 | RP-060117 | - | - | Approved as v7.0.0 to put under change control | 2.0.0 | 7.0.0 |