3GPP TS 25.424 V7.0.0 (2006-03)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

UTRAN Iur Interface Data Transport & Transport Signalling

for Common Transport Channel Data Streams

(Release 7)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Keywords

UMTS, radio

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2006, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

# Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	Data Link Layer	7

4.1	ATM Transport Option	7

4.2	IP Transport Option	7

5	Iur Data Transport for Common Transport Channel Data Streams	7

5.1	Introduction	7

5.2	ATM Transport Option	8

5.3	IP Option	8

6	I ur Transport Signalling Application for Common Transport Channel Data Streams	8

6.1	Introduction	8

6.2	Transport Signalling in case of ATM option	8

6.3	Transport Signalling in case of IP Transport Option	9

7	Signalling Bearer for ALCAP on Iur Interface	9

7.1	ATM Transport Option	9

7.2	IP Transport Option	9

8	Interworking between ATM and IP Transport Options	9

Annex A (informative):	Change history	10

# Foreword

This Technical Specification (TS) has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document shall provide a specification of the UTRAN RNC-RNC (Iur) interface Data Transport and Transport Signalling for Common Transport Channel data streams.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- 	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- 	For a specific reference, subsequent revisions do not apply.

- 	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	ITU-T Recommendation I.361 (11/95): "B-ISDN ATM Layer Specification".

[2]	ITU-T Recommendation I.363.2 (11/2000): "B-ISDN ATM Adaptation Layer type 2".

[3]	ITU-T Recommendation I.366.1 (6/98): "Segmentation and Re-assembly Service Specific Convergence Sublayer for the AAL type 2".

[4]	New ITU-T Recommendation Q.2630.1 (12/99): "AAL Type 2 signalling protocol (Capability Set 1)".

5	ITU-T Recommendation E.191 (03/00): "B-ISDN addressing".

[6]	3GPP TS 25.426: "UTRAN Iur and Iub Interface Data Transport & Transport Signalling for DCH Data Streams".

[7]	3GPP TS 25.434: "UTRAN Iub Interface Data Transport & Transport Signalling for Common Transport Channel Data Streams".

[8]	ITU-T Recommendation Q.2630.2 (12/2000): "AAL Type 2 signalling protocol (Capability Set 2)".

[9]	ITU-T Recommendation X.213 (11/95): "Information Technology - Open Systems Interconnection - Network Service Definition".

[10]	IETF STD 51, RFC 1661 (July 1994): "The Point-To-Point Protocol (PPP)".

[11]	 IETF STD 51, RFC 1662 July 1994: "PPP in HDLC-like Framing".

[12]	IETF RFC 2507 (February 1999): "IP header compression".

[13]	IETF RFC 1990 "The PPP Multilink Protocol (MP)".

[14]	IETF RFC 2686 "The Multi-Class Extension to Multi-Link PPP".

[15]	IETF RFC 2509 (February 1999): "IP Header Compression over PPP".

[16]	IETF RFC 2460 "Internet Protocol, Version 6 (Ipv6) Specification".

[17]	IETF RFC 791 (1981): "Internet Protocol".

[18]	IETF RFC 2474 (December 1998): "Definition of the Differentiated Services Field (DS Field) in the IPv4 and IPv6 Headers".

[19]	IETF RFC 768 (8/1980): "User Datagram Protocol".

[20]	IETF RFC 3153 (1/2001): "PPP Multiplexing".

[21]	IETF RFC 2364 (1/2001): "PPP over AAL5".

[22]	IETF RFC 3031 (1/2001):"Multiprotocol Label Switching Architecture".

[23]	ITU-T Recommendation E.164 (5/97): " The international public telecommunication numbering plan ".

# 3	Definitions and abbreviations

## 3.1	Definitions

Common Transport Channels are defined as transport channels that are shared by several users i.e. RACH, FACH, DSCH [TDD], USCH [TDD] and HS-DSCH.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

AAL2	ATM Adaptation Layer type 2

AAL5	ATM Adaptation Layer type 5

AESA	ATM End System Address

ALCAP 	Access Link Control Application Part

ATM	Asynchronous Transfer Mode

CPS	Common Part Sublayer

DiffServ	Differentiated Services

DSCH	Downlink Shared Channel

FACH	Forward Access Channel

HDLC	High level Data Link Control

HS-DSCH	High Speed Downlink Shared Channel

IP	Internet Protocol

IPv4	Internet Protocol, version 4

IPv6	Internet Protocol, version 6

IWF	Interworking Function

IWU	Interworking Unit

LC	Link Characteristics

ML/MC PPP	Multilink-Multiclass PPP

MPLS	Multiprotocol Label Switching

MTP	Message Transfer Part

NNI	Network-Node Interface

NSAP	Network Service Access Point

PPP	Point-to-Point Protocol

PPPMux	PPP Multiplexing

PT	Path Type

QoS	Quality of Service

RACH	Random Access Channel

SAAL	Signalling ATM Adaptation Layer

SDU	Service Data Unit

SSCOP	Service Specific Connection Oriented Protocol

SSCF	Service Specific Co-ordination Function

SSCS	Service Specific Convergence Sublayer

SSSAR	Service Specific Segmentation and Re-assembly sublayer

STC	Signalling Transport Converter

TNL	Transport Network Layer

UDP	User Datagram Protocol

UNI	User-Network Interface

USCH	Uplink Shared Channel

# 4	Data Link Layer

## 4.1	ATM Transport Option

ATM shall be used in the transport network user plane and the transport network control plane according to ITU-T Recommendation I.361 [1]. The structure of the cell header used in the UTRAN Iur interface is the cell header format and encoding at NNI (see Figure 3/I.361 [1]).

## 4.2	IP Transport Option

A UTRAN Node supporting IP transport option shall support PPP protocol with HDLC framing [10], [11].

Note: This does not preclude the single implementation and use of any other data link layer protocols (e.g. PPPMux/AAL5/ATM [20, 21], PPP/AAL2/ATM, Ethernet, MPLS/ATM [22], etc.) fulfilling the UTRAN requirements toward the upper layers.

An RNC using IP transport option having interfaces connected via slow bandwidth PPP links like E1/T1/J1 shall also support IP Header Compression [12] and the PPP extensions ML/MC-PPP [13], [14]. In this case, negotiation of header compression [12] over PPP shall be performed via [15].

# 5	Iur Data Transport for Common Transport Channel Data Streams

## 5.1	Introduction

This clause specifies the transport layers that support Common Channels (FACH, RACH, DSCH [TDD], USCH [TDD], HS-DSCH) Iur data streams.

There are two options for the transport layer of the Common Channels data streams in Iur and Iub:

1)	ATM based Transport (ATM transport option)

2)	IP based Transport (IP transport option)

The following figure shows the protocol stacks of the two options.

Protocol stack for ATM transport option				                Protocol stack for IP transport option

Figure 1: Transport network layer for DCH data streams over Iur and Iub interfaces

## 5.2	ATM Transport Option

ATM [1], AAL type 2 (ITU-T Recommendations I.363.2 [2] and I.366.1 [3]) is used as the standard transport layer for RACH, FACH, USCH [TDD], DSCH [TDD] and HS-DSCH Iur data streams.

These AAL2 connections are established via the transport signalling protocol described in clause 5.

Figure 1 shows the protocol stack for the transport of RACH, FACH, USCH [TDD], DSCH [TDD] and HS-DSCH Iur data streams using the ATM Transport Option. Service Specific Segmentation and Re-assembly (SSSAR) is used for the segmentation and re-assembly of AAL2 SDUs (i.e. SSSAR is only considered from ITU-T Recommendation I.366.1 [3]).

## 5.3	IP Option

UDP [18] over IP shall be used as the transport for RACH, FACH, USCH [TDD], DSCH [TDD] and HS-DSCH data streams on Iur. The data link layer is as specified in subclause 4.2.

An IP UTRAN Node shall support IPv6 [16]. The support of IPv4 [17] is optional.

Note: This does not preclude single implementation of IPv4.

IP dual stack support is recommended for the potential transition period from IPv4 to IPv6 in the transport network.

The transport bearer is identified by the UDP port number and the IP address (source UDP port number, destination UDP port number, source IP address, destination IP address).

IP Differentiated Services code point marking [18] shall be supported. The mapping between traffic categories and Diffserv code points shall be configurable by O&M. Traffic categories are implementation-specific and may be determined from the application parameters.

# 6	I ur Transport Signalling Application for Common Transport Channel Data Streams

## 6.1	Introduction

This clause specifies the transport signalling protocol(s) used to establish the user plane transport bearers. The protocol stack is shown in [6].

## 6.2	Transport Signalling in case of ATM option

AAL2 signalling protocol Capability Set 2, ITU-T Recommendation Q.2630.2 [8], is the signalling protocol to control the AAL2 connections on Iur interfaces. Q.2630.2 [8] adds new optional capabilities to Q.2630.1 [4].

AAL2 transport layer addressing is based on embedded E.164 or other AESA variants of the NSAP addressing format [5,9]. Native E.164 [23] addressing shall not be used.

Binding ID provided by the radio network layer shall be copied in SUGR parameter of ESTABLISH.request primitive of [8]. The binding identifier shall already be assigned and tied to a radio application procedure when the Establish Request message is received over the Iur interface in the Drift RNC.

User Plane Transport bearers are established by the ALCAP in Serving RNC, and in all normal cases released by the ALCAP in the RNC which established the AAL2 connection.

The Link Characteristics parameter (LC) shall be included in the Establish Request message and in the Modification Request message of AAL2 signalling protocol.

If there is an AAL2 switching function in the transport network layer of the interface, the Path Type parameter (PT) may be included in the Establish Request message of AAL2 signalling protocol for prioritisation at ATM level.

If the value in either the Maximum CPS-SDU Bit Rate or the Average CPS-SDU Bit Rate of the Link Characteristics(LC) in AAL 2 signalling messages as specified in reference [8] is 2048 Kbit/s, it shall be interpreted as bit rate 2048 Kbit/s or higher.

NOTE:	Separation of traffic (e.g. HS-DSCH) that is using this modified interpretation of Link Characteristics in ref. [8] from other traffic is highly recommended. Otherwise the potential bursty nature of this specific traffic in combination with its unknown bit rate may decrease the QoS of all traffic within the same AAL type 2 path.

## 6.3	Transport Signalling in case of IP Transport Option

An ALCAP protocol is not required in case both RNCs are using the IP transport option.

# 7	Signalling Bearer for ALCAP on Iur Interface

## 7.1	ATM Transport Option

The signalling bearer for the ALCAP on the Iur interface for common transport channels data streams is the same as the signalling bearer for the ALCAP on the Iur interface for DCH data streams, defined in [6].

## 7.2	IP Transport Option

An ALCAP protocol is not required in case both RNCs are using the IP transport option.

# 8	Interworking between ATM and IP Transport Options

An RNC supporting IP transport option shall provide interworking to an RNC supporting only ATM transport option. The interworking alternatives are defined in [6].

Annex A (informative):
Change history

| Change history |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| TSG RAN# | Version | CR | Tdoc RAN | New Version | Subject/Comment |
| RAN_04 | - | - | - | 3.0.0 | Approved by TSG-RAN by correspondence |
| RAN_05 | 3.0.0 | - | - | 3.1.0 | Approved by TSG-RAN #5 |
| RAN_07 | 3.1.0 | - | - | 3.2.0 | Approved at TSG RAN #7 |
| RAN_08 | 3.2.0 | - | RP-000245 | 3.3.0 | Approved at TSG RAN #8 |
| RAN_09 | 3.3.0 | 005 | RP-000382 | 3.4.0 | Approved at TSG RAN #9 |
| RAN_10 | 3.4.0 | 006 | RP-000622 | 3.5.0 | Approved at TSG RAN #10 |
| RAN_11 | 3.5.0 | 007 008 | RP-010119 | 3.6.0 | Approved at TSG RAN #11 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| March 01 | 11 | RP-010162 | 009 |  | Approved at TSG RAN #11 and placed under Change Control | - | 4.0.0 |
| 12/2001 | 14 | RP-010857 | 013 |  | Reference corrections | 4.0.0 | 4.1.0 |
| 03/2002 | 15 | RP-020171 | 019 |  | Alignment of 25.424 to 25.426 and Correction to transport bearers release initiation | 4.1.0 | 4.2.0 |
| 03/2002 | 15 | RP-020189 | 020 | 1 | Introduction of IP transport in UTRAN. | 4.2.0 | 5.0.0 |
| 06/2002 | 16 | RP-020408 | 023 |  | Correction of Aesa formats | 5.0.0 | 5.1.0 |
| 09/2003 | 21 | RP-030538 | 025 | 3 | Handling of maximum bit rate exceeding 2048kbit/s | 5.1.0 | 5.2.0 |
| 12/2003 | 22 | RP-030685 | 026 | 1 | Diffserv marking is configurable | 5.2.0 | 5.3.0 |
| 12/2003 | 22 | - | - | - | Introduction of Release 6 specification | 5.3.0 | 6.0.0 |
| 03/2004 | 23 | RP-040061 | 028 | - | Inclusion of HSDPA | 6.0.0 | 6.1.0 |
| 06/2005 | 28 | RP-050234 | 029 | 1 | Correction to the initiation of the release of an AAL2 connection | 6.1.0 | 6.2.0 |
| 06/2005 | 28 | RP-050225 | 031 | - | Feature Cleanup: Removal of CPCH | 6.1.0 | 6.2.0 |
| 06/2005 | 28 | RP-050222 | 033 | - | Feature clean-up: Removal of DSCH (FDD mode) | 6.1.0 | 6.2.0 |
| 09/2005 | 29 | RP-050431 | 035 |  | IP Transport Option for Common TrCH Data Streams | 6.2.0 | 6.3.0 |
| 03/2006 | 31 | - | - | - | Introduction of Release 7 specification | 6.3.0 | 7.0.0 |