3GPP TS 25.461 V7.2.0 (2005-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

UTRAN Iuant interface: Layer 1

(Release 7)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

UMTS, radio, EMC, power supply, antenna

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2005, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions and abbreviations	5

3.1	Definitions	5

3.2	Abbreviations	5

4	Iuant layer 1	6

4.1	General	6

4.2	RS485 option	7

4.3	Modem option	7

4.3.1	Interference with existing systems	8

4.3.1.1	Carrier frequency and frequency stability	8

4.3.1.2	Modem isolation and modem emissions	8

4.3.1.3	Modem intermodulation attenuation	9

4.3.2	Recovery time	9

4.3.3	Impedance	9

4.3.4	Modulator characteristics	10

4.3.4.1	Levels	10

4.3.4.2	Spectrum emission mask	10

4.3.5	Demodulator characteristics	10

4.3.6	Duty cycle variation	10

4.3.7	Operating bands	11

4.3.8	 Time delay and accuracy	12

4.3.9	 Insertion Loss	12

4.3.10	DC port isolation	12

4.3.11	RET control unit spurious emission	12

4.4	DC power supply	13

4.4.1	Power consumption	13

4.4.2	Conducted emission	13

4.4.3	Power-up characteristics	13

Annex A (normative):	Test procedures	14

Annex B (informative):	Change history	15

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the standards allowed to implement layer 1 on the Iuant interface.

The specification of transmission delay requirements and O&M requirements are not in the scope of the present document.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 25.462: "UTRAN Iuant interface: Signalling transport".

[2]	ISO/IEC 8482 (1993): "Information technology - Telecommunications and information exchange between systems - Twisted pair multipoint interconnections".

[3]	TIA/EIA TSB89: “Application guidelines for TIA/EIA-485-A”.

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply:

On-Off-Keying: A modulation system in which a carrier is switched between two states, ON and OFF.

Common feeder cable: Feeder cable where some antenna line devices (e.g. RET, TMA) are connected via the same feeder cable.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

BS	Base Station

DC	Direct Current

DL	Downlink

FDD	Frequency Division Duplex

ISB	Idle-State Biasing

OOK	On-Off-Keying

RET	Remote Electrical Tilting

RF	Radio Frequency

TMA	Tower Mounted Amplifier

UE	User Equipment

UL	Uplink

UMTS	Universal Mobile Telecommunications System

UTRA	UMTS Terrestrial Radio Access

# 4	Iuant layer 1

## 4.1	General

There are two layer 1 options:

-	RS485 option: A screened multicore cable, which supports a conventional RS485 serial multi-drop bus.

-	Modem option: A connection to a RET control unit by way of a coaxial cable which is shared with DC supply and RF signals.

Both layer 1 options support the connection of two-way serial data and DC power to the RET antenna device.

At least one of these two layer 1 options shall be supported.

The default data rate for both layer 1 options shall be 9.6 kbps. Higher data rates of 38.4 kbps for both layer 1 options and 115.2 kbps only for the RS485 layer 1 option may optionally be supported. Each unit communicates on one of the three data rates, but different units on the same interface may use different data rates.

After a reset, a secondary station shall alternate between supported data rates. When alternating between data rates, the data rate shall be held constant for 300 ms. After every correctly received device scan command (see [1]) independent of whether it matches or not, at one of the supported data rates, that data rate shall be held constant for 1.5 seconds. After successful reception of an address assignment frame, the secondary station shall use that data rate until it is reset.

Data rates:

-	9.6 kbps  3 %

-	38.4 kbps  3 %

-	115.2 kbps  3 %

The format of the data octet shall be as shown in figure 4.1.1:

Figure 4.1.1: Format and order of transmitted data

## 4.2	RS485 option

This option is constituted by a two wire bi-directional multi-drop configuration conforming to [2]. The mapping of mark/space to logical one and zero as referred to in [2] shall be according to figure 4.1.1.

The use of ISB, also called idle-line failsafe in [3], is mandatory. The bias voltages shall be applied only by the primary station to any separate RS485 bus. The polarity of the idle-state bias is defined as a transmitted 1.

The RS485 transmitter shall be set to drive the bus before the first start bit is sent and held active until the last stop bit is sent. The RS485 transmitter shall stop driving the bus within 20 bit-times after the last stop bit is sent.

If a RET modem is used ISB shall be implemented by the RET modem.

## 4.3	Modem option

The connection to a RET control unit by way of a coaxial cable which is shared with DC supply and RF signals is provided by two modems, a BS modem and a RET modem. The BS modem shall be either connected to the antenna connector of the BS or integrated in the BS. It provides signal transmission to the RET modem and signal reception from the RET modem over the antenna feeder cable. The RET modem is located between the antenna feeder cable and the antenna. Modem configurations and reference points for modem characteristics are specified in figure 4.3.1 and figure 4.3.2. Unless otherwise stated, requirements in this section apply to both BS modem and RET modem.

Figure 4.3.1: Modem configuration and modem reference points for a BS without BS modem

Figure 4.3.2: Modem configuration and modem reference points for
a BS with integrated BS modem

### 4.3.1	Interference with existing systems

The modem circuit shall be capable of managing its transmitting characteristic according to subclause 4.3.5.

#### 4.3.1.1	Carrier frequency and frequency stability

The following carrier frequency shall be used for this application:

2.176 MHz  100 ppm

#### 4.3.1.2	Modem isolation and modem emissions

The external BS modem shall provide minimum attenuation according to figure 4.3.1.2.1 between reference point 2 and reference point 1 to protect the BS from emissions of the RET modem.

External BS modem emissions at reference point 1 shall be attenuated at least according to the modem attenuation in figure 4.3.1.2.1 below the levels specified for the modem spectrum emission mask in subclause 4.3.4.2 to protect the BS from emissions of the BS modem.

The RET modem shall provide minimum attenuation according to figure 4.3.1.2.1 between reference point 3 and reference point 4 to protect other radio systems from emission of the BS modem.

RET modem emissions at reference point 4 shall be attenuated at least according to the modem attenuation in figure 4.3.1.2.1 below the levels specified for the modem spectrum emission mask in subclause 4.3.4.2 to protect other radio systems from emission of the RET modem.

Figure 4.3.1.2.1: Modem attenuation

#### 4.3.1.3	Modem intermodulation attenuation

The modem intermodulation attenuation is specified in terms of the power in intermodulation products of WCDMA modulated carriers present at reference point 1 or reference point 3.

For 2 downlink carriers of 43 dBm the power of third order intermodulation products in the UL operating bands for the external BS modem and RET modem shall not exceed:

-	– 130 dBm/100 kHz for frequencies < 1 GHz

-	– 120 dBm/1 MHz for frequencies ≥ 1 GHz

NOTE: 	Using the modem with higher power than 43 dBm and or with more carriers than 2 carriers at 43dBm/carrier may increase intermodulation products and may degrade the receiver sensitivity of the BS if these intermodulation products fall at BS receive frequencies.

For the worst input configuration of power and number of carriers declared by the modem manufacturer the power of any intermodulation product for the external BS modem and RET modem shall not exceed:

-	– 98dBm/100kHz

In addition, for the worst input configuration of power and number of carriers declared by the modem manufacturer the power of fifth or higher order intermodulation products in the UL operating bands for the external BS modem and RET modem shall not exceed:

-	– 135 dBm/100 kHz for frequencies < 1 GHz

-	– 125 dBm/1 MHz for frequencies  ≥ 1 GHz

### 4.3.2	Recovery time

A minimum recovery time shall be allowed between receiving and transmitting messages on the bus. For this reason a minimum permitted response time is specified in subclause 4.5 in [1].

### 4.3.3	Impedance

The modem transceiver shall provide constant impedance in both transmitting and receiving modes:

-	Nominal impedance Z0: 50 ;

-	Return loss at modem carrier frequency  0.1 MHz > 10 dB;

-	Return loss in external BS and RET modem operating bands > 20 dB.

### 4.3.4	Modulator characteristics

#### 4.3.4.1	Levels

ON-Level:	+3 dBm  2 dB

OFF-Level:	 -40 dBm

#### 4.3.4.2	Spectrum emission mask

The modem spectrum emission mask is specified in figure 4.3.4.2.1. Intermediate values may be obtained by linear interpolation between the points shown. The corresponding measurement bandwidths are specified in table 4.3.4.2.1. For modem configurations according to figure 4.3.1 the BS modem emissions shall not exceed the limits of the spectrum emission mask at reference point 2. For modem configurations according to figure 4.3.2 the BS with integrated BS modem emissions shall not exceed the limits of the spectrum emission mask at reference point 2 only for frequencies below 20 MHz. RET modem emissions shall not exceed the limits of the spectrum emission mask at reference point 3.

Figure 4.3.4.2.1: Modem spectrum emission mask.

Note 1:	For frequencies <1GHz the general emission limit is -108dBm, except modem operating band UL frequencies where the emission limit is -135 dBm.
For frequencies ≥1GHz the general emission limit is -98dBm, except modem operating band UL frequencies where the emission limit is -125 dBm.

Table 4.3.4.2.1: Modem spectrum emission mask measurement bandwidth

| Band | Measurement Bandwidth |
| --- | --- |
| 9 kHz - 150 kHz | 1 kHz |
| 150 kHz - 30 MHz | 10 kHz |
| 30 MHz - 1 GHz | 100 kHz |
| 1 GHz - 12.75 GHz | 1 MHz |

### 4.3.5	Demodulator characteristics

The demodulator shall fulfil the requirement in subclause 4.3.6 for a carrier ON-Level within +5 dBm to -12 dBm and a carrier OFF-Level less than -18 dBm. The levels within –12 dBm to –18 dBm are undefined.

### 4.3.6	Duty cycle variation

In order to guarantee proper transmission of data bits through the processes of modulation and demodulation, the following limit shall be met for the duty cycle variation:

DCSYSTEM = |DCRX – DCTX|  10 %

Where:	DCSYSTEM is the difference between the duty cycles of the transmitted and received bit streams,

DCTX = Duty cycle for the input bit stream, and

DCRX = Duty cycle for the output bit stream.

Figure 4.3.6.1: Duty cycles of the bit stream and OOK modulated subcarrier

For transmission through a coaxial cable, two converters are required, one from a bit stream to OOK (modulator) and one from OOK back to a bit stream (demodulator). Therefore half of the total duty cycle tolerance is available for each converter.

For an input bit stream with a duty ratio of 50 %, the cascaded modulator and demodulator shall provide an output bit stream with a duty ratio within the limits 40 % - 60 %, measured in each case at 0.5 times peak amplitude (see figure 4.3.6.1).

### 4.3.7	Operating bands

A UTRA/FDD BS or RET modem is designed to operate in one or several of the following paired frequency bands:

Table 4.3.7.1: Frequency bands

| Operating Band | UL Frequencies UE transmit, Node B receive | DL frequencies UE receive, Node B transmit |
| --- | --- | --- |
| I | 1920 – 1980 MHz | 2110 – 2170 MHz |
| II | 1850 –1910 MHz | 1930 – 1990 MHz |
| III | 1710 – 1785 MHz | 1805 – 1880 MHz |
| IV | 1710 – 1755 MHz | 2110 – 2155 MHz |
| V | 824 – 849 MHz | 869 – 894 MHz |
| VI | 830 – 840 MHz | 875 – 885 MHz |
| VII | 2500 – 2570 MHz | 2620 – 2690 MHz |
| VIII | 880 – 915 MHz | 925 – 960 MHz |
| IX | 1749.9 – 1784.9 MHz | 1844.9 – 1879.9 MHz |

The operating bands of the BS or RET modem shall be declared by the manufacturer.

### 4.3.8		Time delay and accuracy

The time delay in the operating bands shall be declared by the manufacturer with ± 1 ns accuracy. The time delay shall not exceed 30 ns. This requirement is only applicable to external BS and RET modems.

### 4.3.9		Insertion Loss

The insertion loss in the external BS or RET modem operating band shall be ≤ 0.3 dB.

The actual insertion loss shall be declared by the manufacturer.

### 4.3.10	DC port isolation

The isolation between DC port and RF ports shall meet the minimum values in figure 4.3.10.1 and 4.3.10.2. Figure 4.3.10.1 is valid for RET modems between reference point 5 and 4 as well as 5 and 3 and for BS modems without integrated power supply between reference point 6 and 2 as well as 6 and 1. Figure 4.3.10.2 is valid as additional requirement for RET modems between reference point 5 and 3 and for BS modems without integrated power supply between reference point 6 and 2.

Figure 4.3.10.1: DC port isolation

Figure 4.3.10.2: DC port isolation

### 4.3.11	RET control unit spurious emission

The RET control unit, or a combination of RET control units, shall not generate spurious emission, at reference point 5, above a level that will violate the spectrum emission mask requirement according to chapter 4.3.4.2. The DC port isolation according to chapter 4.3.10 shall be taken into account.

## 4.4	DC power supply

### 4.4.1	Power consumption

The DC supply requirements refers to reference points 3 and 5 in subclause 4.3.

The RET control unit and a RET modem shall be able to operate with a DC supply voltage range of 10 V – 30 V.

The RET control unit has two power consumption modes:

Table 4.4.1.1: Power consumption modes for RET control unit

| Power mode | Maximum power consumption |
| --- | --- |
| High | < 13 W |
| Low | < 2 W |

A RET modem maximum power consumption shall be < 2 W.

A RET modem shall impose a voltage drop less than 2 V between reference point 3 and 5.

### 4.4.2	Conducted emission

The levels of generated conducted noise and ripple on DC Power supply shall be within the limits given in table 4.4.2.1.

Table 4.4.2.1: Noise and ripple

| Item | Limit | Frequency | Remarks |
| --- | --- | --- | --- |
| RET power mode High | 70 mVpp | 0.15 - 30 MHz | Only one operating unit a time |
| RET power mode Low | 20 mVpp | 0.15 - 30 MHz |  |

All RET units connected to a DC supply bus shall exhibit full performance up to the limit of 112 mVpp total noise and ripple within 0.15 - 30 MHz.

### 4.4.3	Power-up characteristics

A BS modem, RET modem or RET control unit shall have a power-up period of 3 s.

During the power-up period a BS modem, RET modem or a RET control unit shall exhibit the circuit equivalent of a DC power consumer with a current consumption of maximum 400 mA in parallel with a capacitor of maximum 0.5 µF.

After the power-up period, the RET unit shall be fully functional and the power consumption requirement as described in subclause 4.4.1 applies.

Annex A (normative):
Test procedures

Test pattern

Spectrum mask and emission requirement shall be tested both with a consecutive series of “0” and an alternating sequence of “0” and “1”.

Emission requirement below noise floor

As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy and sensitivity when measuring close to or below the noise floor, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

Conversion between modulated and CW for IM measurement

The requirement for IM3 below 1 GHz shall be relaxed 15 dB and tested with CW interferers at the specified levels.
The requirement for IM3 above 1 GHz shall be relaxed 5 dB and tested with CW interferers at the specified levels.

The requirement for IM5 or higher below 1 GHz shall be relaxed 10 dB and tested with CW interferers at the specified levels.

The requirement for IM5 or higher above 1 GHz shall be relaxed 0 dB and tested with CW interferers at the specified levels.

Example: A –130 dBm/100 kHz requirement below 1 GHz with two WCDMA-modulated carriers at 43 dBm is converted to a –115 dBm requirement with two CW carriers at 43 dBm.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| September 2004 | TSG-RAN#25 | RP-040344 | _ | _ | presentation to TSG-RAN for information | _ | 1.0.0 |
| September 2004 | TSG-RAN#25 | RP-040344 | _ | _ | approved at TSG-RAN#25 and placed under change control | 1.0.0 | 6.0.0 |
| 12/2004 | 26 | RP-040444 | 1 |  | DC power supply distribution | 6.0.0 | 6.1.0 |
| 12/2004 | 26 | RP-040444 | 2 | 1 | Improved demodulator characteristics specification | 6.0.0 | 6.1.0 |
| 12/2004 | 26 | RP-040444 | 3 | - | Requirements missing for when the RS485 bus shall not be driven by the secondary device | 6.0.0 | 6.1.0 |
| 12/2004 | 26 | RP-040444 | 4 | 1 | RET DC power consumption modes | 6.0.0 | 6.1.0 |
| 12/2004 | 26 | RP-040444 | 5 | - | Minor Corrections and editorial changes to 25.461 | 6.0.0 | 6.1.0 |
| 03/2005 | 27 | RP-050061 | 7 | 1 | Minor Corrections and editorial changes to 25.461 | 6.1.0 | 6.2.0 |
| 03/2005 | 27 | RP-050061 | 8 | 1 | Power consumption clarification of RET | 6.1.0 | 6.2.0 |
| 03/2005 | 27 | RP-050061 | 9 |  | Modem Operating Bands | 6.1.0 | 6.2.0 |
| 03/2005 | 27 | RP-050061 | 10 |  | Modem Return loss | 6.1.0 | 6.2.0 |
| 03/2005 | 27 | RP-050061 | 11 |  | Modem Time Delay and Accuracy | 6.1.0 | 6.2.0 |
| 03/2005 | 27 | RP-050061 | 12 |  | Modem Insertion Loss | 6.1.0 | 6.2.0 |
| 06/2005 | 28 | RP-050237 | 13 | 1 | DC power on sequence | 6.2.0 | 6.3.0 |
| 06/2005 | 28 | RP-050237 | 15 | - | BS Modem and RET Modem Filtering | 6.2.0 | 6.3.0 |
| 06/2005 | 28 | RP-050237 | 16 | - | BS Modem and RET modem spectrum emission mode | 6.2.0 | 6.3.0 |
| 06/2005 | 28 | RP-050237 | 17 | - | BS modem and RET modem return loss at modem frequency | 6.2.0 | 6.3.0 |
| 06/2005 | 28 | RP-050237 | 18 | - | Time delay clarification | 6.2.0 | 6.3.0 |
| 06/2005 | 28 | RP-050226 | 19 | - | Introduction of UMTS2600 requirements | 6.3.0 | 7.0.0 |
| 09/2005 | 29 | RP-050438 | 21 | 1 | Power-Up period clarifications | 7.0.0 | 7.1.0 |
| 09/2005 | 29 | RP-050438 | 23 |  | Insertion loss for RET | 7.0.0 | 7.1.0 |
| 09/2005 | 29 | RP-050438 | 25 |  | Testing for RET | 7.0.0 | 7.1.0 |
| 09/2005 | 29 | RP-050438 | 27 |  | Time delay for RET | 7.0.0 | 7.1.0 |
| 09/2005 | 29 | RP-050438 | 29 |  | Intermodulation attenuation for RET | 7.0.0 | 7.1.0 |
| 09/2005 | 29 | RP-050438 | 31 |  | BS and RET modem isolation | 7.0.0 | 7.1.0 |
| 12/2005 | 30 | RP-050704 | 32 | 1 | Introduction of UMTS 1700 requirements | 7.1.0 | 7.2.0 |
| 12/2005 | 30 | RP-050703 | 33 |  | Introduction of UMTS 900 | 7.1.0 | 7.2.0 |
| 12/2005 | 30 | RP-050689 | 35 |  | Test Procedures | 7.1.0 | 7.2.0 |
| 12/2005 | 30 | RP-050689 | 37 |  | Correction to modem isolation and emission | 7.1.0 | 7.2.0 |
| 12/2005 | 30 | RP-050689 | 39 |  | correction of referencing | 7.1.0 | 7.2.0 |
| 12/2005 | 30 | RP-050689 | 41 | 1 | RET Control Unit spurious emission simplified version | 7.1.0 | 7.2.0 |