3GPP TR 25.839 V0.3.0(2001-07)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Uplink Synchronous Transmission Scheme (USTS)

(Iur/Iub aspects)

(Release5)

- The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Reference

<Workitem> (<Shortfilename>.PDF)

Keywords

<keyword[, keyword]>

3GPP

Postal address

Office address

Internet

secretariat@3gpp.org

Individual copies of this  deliverable
can be downloaded from

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

©

All rights reserved.

- Contents

- 1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1		Definitions	5

3.2 		Symbols	5

3.3		Abbreviations	5

4	Introduction	6

4.1		Background	6

5	Requirements	6

6	Study Area	7

6.1		General	7

6.2		Discussions in Leading Group	7

6.3		Discussions in WG3	7

6.4		Impacts on other WGs	17

7	Agreements and associated agreed contributions	17

8	Specification Impact and associated Change Requests	18

9	Backward Compatibility	18

10	Project Plan	18

10.1	Schedule	18

10.2	Study Item Status	18

11	History	18

# Intellectual Property Rights

# Foreword

- This Technical Report(TR) has been produced by the 3rd Generation Partnership Project (3GPP), Technical Specification Group RAN.

- The contents of this TR are subject to continuing work within the 3GPP TSG and may change following formal TSG approval. Should the TSG modify the contents of this TR, it will be re-released with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Scope

- The purpose of the present document is to help the TSG RAN WG3 group to specify the changes to existing specifications, needed for the introduction of the “Uplink Synchronous Transmission Scheme(USTS)” option for Release5.

- WG1 is the leading working group of this Study Item and has its own TR. The purpose of this TR is not to replace the TR or any decisions made in the leading WG. Rather, it is intended to gather all the information in order to trace the history and the status of the Work Task in other RAN WGs, and discuss issues that WG3 has impact on.

- It is not intended to replace contributions and Change Requests, but only to list conclusions and make references to agreed contributions and CRs. When solutions are sufficiently stable, the CRs can be issued.

- It describes agreed requirements related to the Study Item.

It identifies the affected specifications with related Change Requests.

It also describes the schedule of the Study Item.

This document is a ‘living’ document, i.e. it is permanently updated and presented to all TSG-RAN meetings

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.

- TS 25.211 : Physical channels and mapping of transport channels onto physical channels (FDD)

- TS 25.213 : Spreading and modulation (FDD)

- TS 25.214 : FDD : Physical layer procedures

- TS 25.302 : Services provided by the Physical Layer

- TS 25.331 : Radio Resource Control (RRC) Protocol Specification

- TS 25.423 : UTRAN Iur Interface RNSAP Signalling

- TS 25.433 : UTRAN Iub Interface NBAP Signalling

- TS 25.435 : UTRAN interface User Plane Protocol for Common Transport channel Data Streams

- TR 25.926 : UE Radio access capabilities definition

- TR 25.854 : Study Report for Uplink Synchronous Transmission Scheme (USTS)

# Definitions, symbols and abbreviations

## Definitions

For the purposes of the present document, the following terms and definitions apply.

## Symbols

## Abbreviations

For the purposes of the present document, the following abbreviations apply:

CFN	Connection Frame Number

DPCCH	Dedicated Physical Control CHannel

DPDCH	Dedicated Physical Data CHannel

RTT	Round Trip Time

RTPD	Round Trip Propagation Delay

TAB	Time Alignment Bit

Tref 	Reference time

UE	User Equipment

USTS	Uplink Synchronous Transmission Scheme

UTRAN	Universal Terrestrial Radio Access Network

# Introduction

## Background

- Uplink Synchronous Transmission Scheme feature is one of the study items for the WCDMA Release5. In this technical report, the requirements and detail solutions are described

## Overview

- Uplink Synchronous Transmission Scheme(USTS) is an alternative technology applicable for low mobility terminals.  USTS can reduce  uplink intra-cell interference by means of making a cell receive orthogonalized signals from UEs. To orthogonalize receiving signals from UEs,

- the network may allocate the same scrambling code to more than one UE,

- the different channelization codes are allocated to all dedicated physical channels across all UEs in a cell and the spreading factor and node number of channelization code are delivered from network to each UE, and the signal transmission time of each UE is adjusted by UTRAN.

- The spreading and modulation scheme for USTS is same as section 4 of TS 25.213. In case of USTS, the long scrambling code described in section 4.3.2.2. of TS 25.213 is used. However, this long scrambling code is not UE specific, but cell specific.  In order to generate the cell specific long scrambling code, the initial loading value of PN generator is determined by the network

- The channelisation codes for DPDCH and DPCCH in a UE are chosen to follow OVSF code allocation rule. More information about the method for the channelization codes allocation for USTS is described in this document later.

- In a USTS mode, time alignment is required to preserve orthogonality between channelisation codes from different UEs and also to properly despread the cell-specific long scrambling code. The transmission time control is carried out by two steps. The first step is Initial synchronization and the second is tracking.

- Initial synchronization: Adjust transmission time through the initial timing control message given by higher layer

- Tracking process (Closed Loop Timing control): Adjust the transmission time through the Time Alignment Bit (TAB) over DPCCH.

# Requirements

This section describes the requirements to the solutions of USTS regarding Iur/Iub Interfaces.

## General requirements

The solution should be described in the TR 25.854[10] and provided by leading WG.

## Requirements in case of USTS

- Required changes in RNC, Node B and UE are given in the TR 25.854[10], For the support of USTS, the following functionality’s should be provided.

- Parameter configuration for USTS in a cell

- Initiation and termination of USTS

- Soft Handover for USTS

# Study Area

- This section gives a summary of areas that have been identified where study needs to be performed to complete the study item.

## General

- In this section, the general aspect of the uplink syncronous transmission scheme will be discussed.

## Discussions in Leading Group

- TR 25.854

## Discussions in WG3

## New parameters

| - New parameters for USTS | - Functions | ###### Messages |
| --- | --- | --- |
| - USTS Indicator | - To let Node B setup the radio link in USTS mode | - CRNC  Node B - RADIO LINK SETUP REQUEST - RADIO LINK RECONFIGURATION PREPARE |
|  | - To let DRNC setup the radio link in USTS mode | - SRNC  DRNC - RADIO LINK SETUP REQUEST - RADIO LINK RECONFIGURATION PREPARE |
| - USTS Support Indicator | - To notify SRNC whether DRNC’s Node B supports USTS or not | - DRNC  SRNC - RADIO LINK SETUP RESPONSE |
| - TINITSYNC | - To be calculated from Tref and Propagation Delay in SRNC | - N/A |
| - Tref | - To let Node B execute the tracking process with this | - CRNC  Node B - CELL SETUP REQUEST - CELL RECONFIGURATION REQUEST |
|  | - To be given to DRNC as initial loading Data - To calculate TINITSYNC in SRNC | - DRNC  SRNC - RADIO LINK SETUP RESPONSE - RADIO LINK RECONFIGURATION READY |
| - USTS Scrambling Code | - To let Node B know the scrambling code of UE | - CRNC  Node B - RADIO LINK SETUP REQUEST |
|  | - To let DRNC know the scrambling code of UE (In case of handover) | - SRNC  DRNC - RADIO LINK SETUP REQUEST - RADIO LINK ADDITION REQUEST |
|  | - To let SRNC know the scrambling code of UE assigned by DRNC | - DRNC  SRNC - RADIO LINK SETUP RESPONSE - RADIO LINK RECONFIGURATION READY |
| - USTS Scrambling Code Offset | - To let Node B know the scrambling code Offset of UE (In case of handover) | - CRNC  Node B - RADIO LINK SETUP REQUEST |
|  | - To let DRNC know the scrambling code offset of UE (In case of handover) | - SRNC  DRNC - RADIO LINK SETUP REQUEST - RADIO LINK ADDITION REQUEST |
| - USTS Channelisation Code Number | - To notify Node B of Channelisation Code information | - CRNC  Node B - RADIO LINK SETUP REQUEST |
|  | - To let DRNC know the channelisation code information of UE (In case of handover) | - SRNC  DRNC - RADIO LINK SETUP REQUEST |
|  | - To retrieve Channelisation Code for USTS | - DRNC  SRNC - RADIO LINK SETUP RESPONSE - RADIO LINK RECONFIGURATION READY |

Table 6-1 New paramters for USTS

## Impacts on Interfaces

      - Iub Interface

To support USTS in Iub Interface, the followings will be done.

- To transmit the USTS Indicator from CRNC to Node B, USTS Indicator should be added in NBAP messages, RADIO LINK SETUP REQUEST and RADIO LINK RECONFIGURATION PREPARE.

- Through CELL SETUP REQUEST and CELL RECONFIGURATION REQUEST, Tref will be sent to Node B from CRNC and will be used to execute the tracking process in Node B.

- USTS Scrambling Code & USTS Scrambling Code Offset will be sent to Node B from CRNC through RADIO LINK SETUP REQUEST.

- In NBAP messages, RADIO LINK SETUP REQUEST, USTS Channelisation Code Number should be added to retrieve USTS channelization code.

      - Iur Interface

To support USTS in Iur Interface, the followings will be done. And these are very similar to Iub’s.

- SRNC should indicate to DRNC whether or not the UE is in USTS mode. Therefore, USTS indicator should be added in RNSAP messages, RADIO LINK SETUP REQUEST and RADIO LINK RECONFIGURATION PREPARE.

- To let SRNC know whether or not DRNC’s Node B supports USTS, USTS Support Indicator should be added in RNSAP messages, RADIO LINK SETUP RESPONSE (in neighbouring cell informaiton).

- To transmit the Tref of DRNC’s node B to SRNC, Tref should be added in RNSAP messages, RADIO LINK SETUP RESPONSE in the case of set up with USTS and RADIO LINK RECONFIGURATION READY in the case of mode change from non-USTS to USTS when USTS indicator has been received by DRNC.

- USTS Scrambling Code and USTS Scrambling Code Offset will be sent to DRNC from SRNC through RADIO LINK SETUP REQUEST and RADIO LINK ADDITION REQUEST in the case of handover

- To send the USTS Scrambling Code assigned by DRNC to SRNC, USTS Scrambling Code should be added in RNSAP messages, RADIO LINK SETUP RESPONSE and RADIO LINK RECONFIGURATION READY.

- USTS Channelisation Code Number will be sent to DRNC from SRNC through RADIO LINK SETUP REQUEST in the case of handover

- To send the USTS Channelisation Code Number assigned by DRNC to SRNC, this IE should be added in RNSAP messages, RADIO LINK SETUP RESPONSE and RADIO LINK RECONFIGURATION READY.

      - Iu Interface

- No impacts on Iu Interface.

## Procedures

      - Mode Change procedure

- The radio link can be in one of the following three modes:

- Normal mode : No timing control, UE discrimination by scrambling code

- Non-USTS mode : No timing control, UE discriminated by both scrambling and channelisation codes

- USTS mode : Timing control, UE discriminated by both scrambling and channelisation codes

- The difference between Normal mode and Non-USTS mode is as follows. If one of the radio links to the cell sites in Active set is in USTS mode, it is discriminated by both scrambling code and channelisation codes assigned for USTS mode in all cells in Active set. Therefore, the other links should be in non-USTS mode. This is because the UE has only a single transmitter and there can be more than one UEs who enter the SHO region from the same original cell and accordingly, they use the common scrambling code and the discrimination can be done only by channelisation codes. The UE in USTS mode has an USTS-mode RL and non-USTS mode RL(s). In normal mode, the UEs in SHO region use their own unique scrambling codes.

- The Mode Change from USTS mode to normal mode will be executed before normal handover when the target DRNC’s node B cannot support USTS. In this mode change, Both new scrambling and channelisation codes are allocated to perform normal handover. But if the target DRNC’s node B can support USTS, Non-USTS mode to USTS mode change will be executed after normal handover procedure shown in Figure 6.1. In this mode change, New parameter values such as scrambling code and channelisation code should be assigned to perform USTS. And the reverse mode change, from normal mode to USTS mode will be executed in case of RRC state change shown in Figure 6.3.

- In the last call flow (Inter RNS Handover procedure – Figure 6.4), it will be shown how Mode Change can be applied to support soft handover with USTS technology.

- The following is the procedure of Mode Change from Non-USTS to USTS mode. Mode Change will be executed after soft handover procedure. The scrambling and channelisation codes for USTS will be assigned to UE and UE will synchronize the timing with TINITSYNC which was calculated by SRNC. From Mode Change UE can get the timing control and the codes for USTS.

- RADIO LINK RECONFIGURATION PREPARE

- This message transmits USTS Indicator to DRNC from SRNC. DRNC will be requested by SRNC whether or not the UE would be in  USTS mode with USTS Indicator.

- RADIO LINK RECONFIGURATION PREPARE

- This message sends USTS Indicator , USTS scrambling code and USTS Channelisation Code Number to Node B from DRNC.

- RADIO LINK RECONFIGURATION READY

- NBAP Message RADIO LINK RECONFIGURATION READY is sent from DRNC’s Node B to DRNC.

- RADIO LINK RECONFIGURATION READY

- This message transmits Tref, USTS scrambling code and USTS Channelisation Code Number to SRNC from DRNC.

- RADIO LINK RECONFIGURATION COMMIT

- RNSAP Message RADIO LINK RECONFIGURATION COMMIT is sent from SRNC to DRNC.

- RADIO LINK RECONFIGURATION COMMIT

- NBAP Message RADIO LINK RECONFIGURATION COMMIT is sent from DRNC to Node B.

- PHYSICAL CHANNEL RECONFIGURATION

- SRNC sends RRC message PHYSICAL CHANNEL RECONFIGURATION to UE.

- Actualizing modification

Both UE and Node B actualise modification of the physical channel.

- PHYSICAL CHANNEL RECONFIGURATION COMPLETE

- UE sends RRC message PHYSICAL CHANNEL RECONFIGURATION COMPLETE to SRNC.

## Call flows of USTS

- The following are call flows for USTS. The first one is for basic call that Iur Interface is  not necessary, and the second is for the call which needs Iur Interface because of RRC State Change. And the last one is for the call which have to use Iur Interface because of Inter RNS handover.

      - Call Setup

- This is the basic call procedure for USTS. In this scenario, Iur Interface is not necessary. So SRNC and CRNC are same.

- RRC CONNECTION REQUEST

- UE sends RRC message RRC CONNECTION REQUEST to SRNC.

- RADIO LINK SETUP REQUEST

- This message transmits USTS Indicator, USTS Scrambling Code, and USTS Channelisation Code Number to the Node B.

- RADIO LINK SETUP RESPONSE

- Node B allocates resources, starts PHY reception, and responses with NBAP message RADIO LINK SETUP RESPONSE.

- ALCAP Iub Data Transport Bearer Setup

- SRNC initiates set-up of Iub Data Transport bearer using ALCAP protocol. This request contains the AAL2 Binding Identity to bind the Iub Data Transport Bearer to the DCH. The request for set-up of Iub Data Transport bearer is acknowledged by Node B.

- User Plane Synchronisation

- The Node B and SRNC establish synchronism for the Iub and Iur Data Transport Bearer by means of exchange of the appropriate DCH Frame Protocol frames.

- RRC CONNECTION SETUP

- SRNC sends RRC message RRC CONNECTION SETUP to UE.

- RRC CONNECTION COMPLETE

- UE sends RRC message RRC CONNECTION COMPLETE to SRNC.

      - Channel and Mobile State Change

- Regarding to RRC state changes which gives some impacts on procedures for USTS, Two cases could be considered. One is the transition from CELL_DCH to CELL_FACH state, which might occur when all dedicated channels have been released via explicit signalling (e.g. PHYSICAL CHANNEL RECONFIGURATION, Radio Bearer Reconfiguration, Radio Bearer Release, Radio Bearer Setup, Transport Channel Reconfiguration, etc.). The other case is the transition from CELL_FACH to CELL_DCH state, which might occur, when a dedicated physical channel is established via explicit signalling (e.g. PHYSICAL CHANNEL RECONFIGURATION, Radio Bearer Reconfiguration, Radio Bearer Release, Radio Bearer Setup, RRC CONNECTION REESTABLISHMENT, Transport Channel Reconfiguration, etc.).

- Transition from CELL_DCH to CELL_FACH state

- Since there exists no more dedicated channel in this case, No additional procedures are required

- Transition from CELL_FACH to CELL_DCH state

- In this case, dedicated channel is established by explicit signalling. New parameters for USTS should be added to the messages in the process of state transition, which follows Mode Change procedure described above. When this transition is associated with handover, call flow for USTS is illustrated in Figure 6-3.

- This is the call procedure for USTS which needs Iur Interface because of RRC state change. In this scenario, Iur Interface should be considered. So the impact of RNC will be described in the point of  SRNC’s and DRNC’s.

- SRNC decides to switch to CELL_DCH state, setting up a new radio link via a new cell controlled by DRNC.

- RADIO LINK SETUP REQUEST

- This message transmits USTS Indicator to DRNC from SRNC. DRNC will be known whether or not the UE will be in USTS mode with USTS Indicator.

- RADIO LINK SETUP REQUEST

- This message transmits USTS Indicator, Tref, USTS scrambling code, and USTS Channelisation Code Number to Node-B from DRNC.

- RADIO LINK SETUP RESPONSE

- Node B allocates resources, starts PHY reception, and responses with NBAP message RADIO LINK SETUP RESPONSE.

- RADIO LINK SETUP RESPONSE

- This message sends USTS Support Indicator, Tref, USTS scrambling code and USTS Channelisation Code Number to SRNC from DRNC.

- ALCAP Iub Bearer Setup & ALCAP Iur Bearer Setup

- SRNC initiates setup of Iur, while DRNC is in charge to setup Iub, Data Transport Bearer using ALCAP protocol. This request contains the AAL2 Binding Identity to bind the Iub Data Transport Bearer to DCH. 
Note: there is not a time relation between set up of Iur and Iub. Both must be carried out before next step.

- User Plane Synchronisation

- Node B and SRNC establish synchronism for the Data Transport Bearer by means of exchange of the appropriate DCH Frame Protocol frames.

- RADIO BEARER SETUP

- SRNC sends RRC message  RADIO BEARER SETUP to UE.

- Actualizing modification

Both UE and Node B actualise modification of the physical channel.

- RADIO BEARER SETUP COMPLETE

- UE sends RRC message RADIO BEARER SETUP COMPLETE to SRNC.

- Common Transport  Channel Resources Release

The SRNC releases the UE context for CELL_FACH state in the source DRNC by sending a Common Transport Channel Resources Release message.

      - Soft Handover

- Detailed description on handover in USTS mode is included in WG1’s TR[10]. Making a summary of the TR, the UE maintains an USTS mode RL while it is within USTS supporting Node B area. When it moves from the area to other USTS supporting Node B area, non-USTS mode RLs are added or deleted according to the movement of UE. After the USTS mode RL is deleted, the UE continues to communicate with the remaining non-USTS mode RLs. When only a single RL remains, the mode change procedure is performed to switch the mode of the RL to USTS mode.  This candidate for handover procedure imposes a minimal impact on the network side by eliminating the necessity of performing mode change for one of the RLs in Active set and proper RL reconfigurations for the other RLs at the same time.

This subclause presents the following soft handover procedures

- Radio Link Addition (Branch Addition)

- Radio Link Deletion (Branch Deletion)

- Radio Link Addition (Branch Addition)

- Figure 6-4 shows the procedure of Radio Link Addition (Branch Addition)

- RADIO LINK SETUP REQUEST

- This message transmits USTS Indicator to DRNC from SRNC. DRNC will be known whether or not the UE will be in USTS mode with USTS Indicator.

- RADIO LINK SETUP REQUEST

- This message transmits USTS Indicator, Tref, USTS scrambling code, and USTS Channelisation Code Number to Node-B from DRNC.

- RADIO LINK SETUP RESPONSE

- Node B allocates resources, starts PHY reception, and responses with NBAP message RADIO LINK SETUP RESPONSE.

- RADIO LINK SETUP RESPONSE

- This message sends USTS Support Indicator, Tref, USTS scrambling code and USTS Channelisation Code Number to SRNC from DRNC.

- ALCAP Iub/Iur Data Bearer Setup

- SRNC initiates set-up of Iub/Iur Data Transport bearer using ALCAP protocol. This request contains the AAL2 Binding Identity to bind the Iub Data Transport Bearer to the DCH.

- User Plane Synchronisation

- Node B and SRNC establish synchronism for the Data Transport Bearer by means of exchange of the appropriate DCH Frame Protocol frames.

- ACTIVE SET UPDATE

- SRNC sends RRC Message ACTIVE SET UPDATE to UE on DCCH

- ACTIVE SET UPDATE COMPLETE

- UE acknowledges with RRC message ACTIVE SET UPDATE COMPLETE

- Radio Link Deletion (Branch Deletion)

- This procedure is the same as normal RL deletion procedure.

## Impacts on other WGs

- In this section, the impact to other WGs of the discussions in WG3 will be described.

# Agreements and associated agreed contributions

- This section documents agreements that have been reached and makes reference to contributions agreed in RAN-WG3 with respect to this Study item.

# Specification Impact and associated Change Requests

- This section is intended to list the affected specifications and the related agreed Change Requests. It also lists the possible new specifications that may be needed for the completion of the Work Task.

# Backward Compatibility

- In this section, the backward compatibility will be discussed.

# Project Plan

## Schedule

| - Date | - Meeting | - Scope | - [expected] Input | [expected]Output |
| --- | --- | --- | --- | --- |

## Study Item Status

|  | Planned Date | Milestone | Status |
| --- | --- | --- | --- |

# History

| - Document history |  |  |
| --- | --- | --- |
| - V0.0.1 | - 2000-08 | - First proposal |
| - V0.1.1 | - 2001-05 | - Chapter2(References) and chapter3 (Abbreviations ) were modified. - Chapter4(Introduction) and chapter5(Requirements) were included |
| - V0.2.0 | - 2001-05 | - V0.1.1 was approved as V0.2.0 |
| - Rapporteur for 3GPP RAN TR 25.839 is: |  |  |
| - JinHyo Park, SK Telecom Tel: +82 31 710 5063 Fax: +82 31 710 5199 jhpark90@sktelecom.com |  |  |
| - This document is written in Microsoft Word version 97 SR-1. |  |  |