3GPP TR 25.843 V4.1.0 (2001-03)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

1,28 Mcps TDD UE Radio Access Capabilities

(Release 4)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

UMTS, radio

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2001, 3GPP Organizational Partners (ARIB, CWTS, ETSI, T1, TTA, TTC).

All rights reserved.

Contents

Foreword	4

1	Scope	5

2	References	5

3	Abbreviations	5

4	Background and Introduction	5

5	UE radio access capability parameters	5

5.1	PDCP parameters	5

5.2	BMC parameters	6

5.3	RLC parameters	6

5.4	MAC parameters	6

5.5	PHY parameters	6

5.5.1	Transport channel parameters in downlink	6

5.5.2	Transport channel parameters in uplink	6

5.5.3	Physical channel parameters in downlink	6

5.5.4	Physical channel parameters in uplink	6

5.5.5	RF parameters	7

5.6	Multi-mode related parameters	7

5.7	Multi-RAT related parameters	7

5.8	LCS related parameters	7

5.9	Measurement related capabilities	7

6	Possible UE radio access capability parameter settings	7

6.1	Value ranges	7

6.2	Reference UE radio access capability combinations	11

6.2.1	Combinations of common UE Radio Access Parameters for UL and DL	11

6.2.2	Combinations of UE Radio Access Parameters for DL	13

6.2.3	Combinations of UE Radio Access Parameters for UL	14

Annex A: Change history	16

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document describes the UE radio access capabilities for 1,28 Mcps TDD, identifies the commonalties and explains the differences to 3,84 Mcps TDD.

# 2	References

The following documents contain provisions that, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 25.306: "UE Radio Access Capabilities".

[2]	3GPP TR 25.928: "1,28 Mcps functionality for UTRA TDD Physical Layer".

[3]	3GPP TS 25.101: "UE Radio Transmission and Reception (FDD)".

[4]	3GPP TS 25.102: "UE Radio Transmission and Reception (TDD)".

# 3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

UE	User Equipment

UMTS	Universal Mobile Telecommunications System

# 4	Background and Introduction

Low chip rate TDD UE radio access capabilities is a release 2000 work item that was agreed in RAN#8 plenary meeting. This work item involves the definition of UE radio access capabilities 1,28 Mcps TDD. It is assumed that the physical parameter set of the 1,28 Mcps TDD is basically the same as the 3,84 Mcps TDD.

Therefore many UE radio access capability parameters do not need to be changed. However, due to the specific physical layer structure and key features of 1,28 Mcps TDD specific parameters have to be added or modified.

The aim of this report is to describe the UE radio access capabilities for 1,28 Mcps TDD as well as to identify the commonalties and to explain the differences to 3,84 Mcps TDD.

# 5	UE radio access capability parameters

NOTE:	This clause needs to be reviewed by TSG RAN WG1.

## 5.1	PDCP parameters

No modifications for 1,28 Mcps TDD are required compared to 3,84 Mcps TDD.

## 5.2	BMC parameters

No UE radio access capability parameters identified (as in [1]).

## 5.3	RLC parameters

No modifications for 1,28 Mcps TDD are required compared to 3,84 Mcps TDD.

## 5.4	MAC parameters

No UE radio access capability parameters identified (as in [1]).

## 5.5	PHY parameters

### 5.5.1	Transport channel parameters in downlink

No modifications for 1,28 Mcps TDD are required compared to 3,84 Mcps TDD.

### 5.5.2	Transport channel parameters in uplink

No modifications for 1,28 Mcps TDD are required compared to 3,84 Mcps TDD.

### 5.5.3	Physical channel parameters in downlink

An additional parameter Support of 8PSK is needed:

Support of 8PSK

Defines whether 8PSK modulation is supported or not.

[Explanation of difference:]

The modulation scheme for FDD and 3,84 Mcps TDD is QPSK. For 1,28 Mcps TDD 8PSK modulation is additionally needed to provide high data rate services up to 2 Mbps.

The parameter Maximum number of timeslots per frame for 3,84 Mcps TDD is replaced by Maximum number of timeslots per subframe for 1,28 Mcps TDD. The parameter Maximum number of physical channels per frame for 3,84 Mcps TDD is replaced by Maximum number of physical channels per subframe for 1,28 Mcps TDD.

[Explanation of difference:]

1,28 Mcps TDD has a different frame structure than 3,84 Mcps TDD. Each 10ms radio frame consists of two 5ms subframes.

No further modifications for 1,28 Mcps TDD are required compared to 3,84 Mcps TDD.

### 5.5.4	Physical channel parameters in uplink

An additional parameter Support of 8PSK is needed:

Support of 8PSK

Defines whether 8PSK modulation is supported or not.

[Explanation of difference:]

The modulation scheme for FDD and 3,84 Mcps TDD is QPSK. For 1,28 Mcps TDD 8PSK modulation is additionally needed to provide high data rate services.

The parameter Maximum number of timeslots per frame for 3,84 Mcps TDD is replaced by Maximum number of timeslots per subframe for 1,28 Mcps TDD.

[Explanation of difference:]

1,28 Mcps TDD has a different frame structure than 3,84 Mcps TDD. Each 10ms radio frame consists of two 5ms subframes.

No further modifications for 1,28 Mcps TDD are required compared to 3,84 Mcps TDD.

### 5.5.5	RF parameters

It is proposed to remove the Chip rate capability parameter and to indicate the support of 1,28 Mcps TDD and / or 3,84 Mcps TDD by the Multi-Mode related parameters.

## 5.6	Multi-mode related parameters

For simplification, it is proposed to replace the parameter Support of UTRA FDD/TDD by three separate parameters Support of UTRA FDD, Support of UTRA TDD 3,84 Mcps and Support of UTRA TDD 1,28 Mcps:

Support of UTRA FDD

Defines whether UTRA FDD is supported.

Support of UTRA TDD 3,84 Mcps

Defines whether UTRA TDD 3,84 Mcps is supported.

Support of UTRA TDD 1,28 Mcps

Defines whether UTRA TDD 1,28 Mcps is supported.

## 5.7	Multi-RAT related parameters

No modifications for 1,28 Mcps TDD are required compared to 3,84 Mcps TDD.

## 5.8	LCS related parameters

No modifications for 1,28 Mcps TDD are required compared to 3,84 Mcps TDD.

## 5.9	Measurement related capabilities

No modifications for 1,28 Mcps TDD are required compared to 3,84 Mcps TDD.

# 6	Possible UE radio access capability parameter settings

NOTE: This clause needs to be reviewed by WG1 and WG2.

## 6.1	Value ranges

Compared to [1], table 6.1 contains additional rows for TDD 1,28 Mcps physical channel parameters in uplink, TDD physical channel parameters in downlink and TDD 1,28 Mcps RF parameters. Furthermore, some restructuring of the multi-mode related parameters was required. The parameter Support of UTRA FDD/TDD was replaced by three separate parameters Support of UTRA FDD, Support of UTRA TDD 3,84 Mcps and Support of UTRA TDD 1,28 Mcps. Therefore, the parameter Chip rate capability of the RF parameters is no longer needed.

Table 6.1: UE radio access capability parameter value ranges

|  |  | UE radio access capability parameter | Value range |
| --- | --- | --- | --- |
| PDCP parameters |  | Header compression algorithm supported | Yes/No |
| RLC parameters |  | Total RLC AM buffer size | 2,10,50,100,150,500,1000 kBytes |
|  |  | Maximum number of AM entities | 3,4,5,6,8,16,32 |
| PHY parameters | Transport channel parameters in downlink | Maximum sum of number of bits of all transport blocks being received at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840 |
|  |  | Maximum sum of number of bits of all convolutionally coded transport blocks being received at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840 |
|  |  | Maximum sum of number of bits of all turbo coded transport blocks being received at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840 |
|  |  | Maximum number of simultaneous transport channels | 4, 8, 16, 32 |
|  |  | Maximum number of simultaneous CCTrCH | 1, 2, 3, 4, 5, 6, 7, 8 |
|  |  | Maximum total number of transport blocks received within TTIs that end within the same 10 ms interval | 4, 8, 16, 32, 48, 64, 96, 128, 256, 512 |
|  |  | Maximum number of TFC in the TFCS | 16, 32, 48, 64, 96, 128, 256, 512, 1024 |
|  |  | Maximum number of TF | 32, 64, 128, 256, 512, 1024 |
|  |  | Support for turbo decoding | Yes/No |
|  | Transport channel parameters in uplink | Maximum sum of number of bits of all transport blocks being transmitted at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840 |
|  |  | Maximum sum of number of bits of all convolutionally coded transport blocks being transmitted at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840 |
|  |  | Maximum sum of number of bits of all turbo coded transport blocks being transmitted at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840 |
|  |  | Maximum number of simultaneous transport channels | 2, 4, 8, 16, 32 |
|  |  | Maximum number of simultaneous CCTrCH of DCH type (TDD only) | 1, 2, 3, 4, 5, 6, 7, 8 |
|  |  | Maximum total number of transport blocks transmitted within TTIs that start at the same time | 2, 4, 8, 16, 32, 48, 64, 96, 128, 256, 512 |
|  |  | Maximum number of TFC in the TFCS | 4, 8, 16, 32, 48, 64, 96, 128, 256, 512, 1024 |
|  |  | Maximum number of TF | 32, 64, 128, 256, 512, 1024 |
|  |  | Support for turbo encoding | Yes/No |
|  | FDD Physical channel parameters in downlink | Maximum number of DPCH/PDSCH codes to be simultaneously received | 1, 2, 3, 4, 5, 6, 7, 8 |
|  |  | Maximum number of physical channel bits received in any 10 ms interval (DPCH, PDSCH, S-CCPCH) | 600, 1200, 2400, 3600, 4800, 7200, 9600, 14400, 19200, 28800, 38400, 48000, 57600, 67200, 76800 |
|  |  | Support for SF 512 | Yes/No |
|  |  | Support of PDSCH | Yes/No |
|  |  | Simultaneous reception of SCCPCH and DPCH | Yes/No |
|  |  | Simultaneous reception of SCCPCH, DPCH and PDSCH | Yes/No |
|  |  | Maximum number of simultaneous S-CCPCH radio links | 1 NOTE: Only the value 1 is part of R99 |
|  | FDD Physical channel parameters in uplink | Maximum number of DPDCH bits transmitted per 10 ms | 600, 1200, 2400, 4800, 960, 19200, 28800, 38400, 48000, 57600 |
|  |  | Support of PCPCH | Yes/No |
|  | TDD 3,84 Mcps physical channel parameters in downlink | Maximum number of timeslots per frame | 1..14 |
|  |  | Maximum number of physical channels per frame | 1,2,3..,224 |
|  |  | Minimum SF | 16, 1 |
|  |  | Support of PDSCH | Yes/No |
|  |  | Maximum number of physical channels per timeslot | 1..16 |
|  | TDD 3,84 Mcps physical channel parameters in uplink | Maximum number of timeslots per frame | 1..14 |
|  |  | Maximum number of physical channels per timeslot | 1, 2 |
|  |  | Minimum SF | 16,8,4,2,1 |
|  |  | Support of PUSCH | Yes/No |
|  | TDD 1,28 Mcps physical channel parameters in downlink | Maximum number of timeslots per subframe | 1..6 |
|  |  | Maximum number of physical channels per subframe | 1,2,3..,96 |
|  |  | Minimum SF | 16,1 |
|  |  | Support of PDSCH | Yes/No |
|  |  | Maximum number of physical channels per timeslot | 1..16 |
|  |  | Support of 8PSK | Yes/No |
|  | TDD 1,28 Mcps physical channel parameters in uplink | Maximum number of timeslots per subframe | 1..6 |
|  |  | Maximum number of physical channels per timeslot | 1,2 |
|  |  | Minimum SF | 16,8,4,2,1 |
|  |  | Support of PUSCH | Yes/No |
|  |  | Support of 8PSK | Yes/No |
| RF parameters | FDD RF parameters | UE power class ([3]) | 3, 4 NOTE: Only power classes 3 and 4 are part of R99 |
|  |  | Tx/Rx frequency separation ([3]) . NOTE: Not applicable if UE is not operating in frequency band a | 190 MHz 174.8-205.2 MHz 134.8-245.2 MHz |
|  | TDD 3,84 Mcps RF parameters | UE power class ([4]) | 2,3 NOTE: Only power classes 2 and 3 are part of R99 |
|  |  | Radio frequency bands ([4]) | a), b), c), a+b), a+c), b+c), a+b+c) |
|  | TDD 1,28 Mcps RF parameters | UE power class ([4]) | 2,3 |
|  |  | Radio frequency bands ([4]) | a), b), c), a+b), a+c), b+c), a+b+c) |
| Multi-mode related parameters |  | Support of UTRA FDD | Yes/No |
|  |  | Support of UTRA TDD 3,84 Mcps | Yes/No |
|  |  | Support of UTRA TDD 1,28 Mcps | Yes/No |
| Multi-RAT related parameters |  | Support of GSM | Yes/No (per GSM frequency band) |
|  |  | Support of multi-carrier | Yes/No |
| LCS related parameters |  | Standalone location method(s) supported | Yes/No |
|  |  | Network assisted GPS support | Network based / UE based / Both/ None |
|  |  | GPS reference time capable | Yes/No |
|  |  | Support for IPDL | Yes/No |
|  |  | Support for OTDOA UE based method | Yes/No |
| Measurement related capabilities |  | Need for downlink compressed mode | Yes/No (per frequency band, UTRA mode and RAT) |
|  |  | Need for uplink compressed mode | Yes/No (per frequency band, UTRA mode and RAT) |

## 6.2	Reference UE radio access capability combinations

### 6.2.1	Combinations of common UE Radio Access Parameters for UL and DL

NOTE:	It is FFS whether measurement-related capabilities need to be included in the combinations. These capabilities are independent from the supported RABs.

Table 6.2.1.1: UE radio access capability parameter combinations, parameters common for UL and DL

| Reference combination of UE Radio Access capability parameters common for UL and DL | 32kbps class | 64kbps class | 128kbps class | 384kbps class | 768kbps class | 2048kbps class |
| --- | --- | --- | --- | --- | --- | --- |
| PDCP parameters |  |  |  |  |  |  |
| Header compression algorithm supported | No | No/Yes NOTE 1 | No/Yes NOTE 1 | No/Yes NOTE 1 | No/Yes NOTE 1 | No/Yes NOTE 1 |
| RLC parameters |  |  |  |  |  |  |
| Total RLC AM buffer size (kbytes) | 10 | 10 | 50 | 50 | 100 | 500 |
| Maximum number of AM entities | 4 | 4 | 5 | 6 | 8 | 8 |
| Multi-mode related parameters |  |  |  |  |  |  |
| Support of UTRA FDD | Yes/No NOTE 1 |  |  |  |  |  |
| Support of UTRA TDD 3,84 Mcps | Yes/No NOTE 1 |  |  |  |  |  |
| Support of UTRA TDD 1,28 Mcps | Yes/No NOTE 1 |  |  |  |  |  |
| Multi-RAT related parameters |  |  |  |  |  |  |
| Support of GSM | Yes/No NOTE 1 |  |  |  |  |  |
| Support of multi-carrier | Yes/No NOTE 1 |  |  |  |  |  |
| LCS related parameters |  |  |  |  |  |  |
| Standalone location method(s) supported | Yes/No NOTE 1 |  |  |  |  |  |
| Network assisted GPS support | Network based / UE based / Both/ None NOTE 1 |  |  |  |  |  |
| GPS reference time capable | Yes/No NOTE 1 |  |  |  |  |  |
| Support for IPDL | Yes/No NOTE 1 |  |  |  |  |  |
| Support for OTDOA UE based method | Yes/No NOTE 1 |  |  |  |  |  |
| RF parameters for FDD |  |  |  |  |  |  |
| UE power class | 3 / 4 NOTE 1 |  |  |  |  |  |
| Tx/Rx frequency separation | 190 MHz |  |  |  |  |  |
| RF parameters for TDD 3,84 Mcps |  |  |  |  |  |  |
| UE power class | 2 / 3 NOTE 1 |  |  |  |  |  |
| Radio frequency bands | A / b / c / a+b / a+c / b+c / a+b+c NOTE 1 |  |  |  |  |  |
| RF parameters for TDD 1,28 Mcps |  |  |  |  |  |  |
| UE power class | 2 / 3 NOTE 1 |  |  |  |  |  |
| Radio frequency bands | A / b / c / a+b / a+c / b+c / a+b+c NOTE 1 |  |  |  |  |  |

NOTE 1:	Options represent different combinations that should be supported with Conformance Tests.

### 6.2.2	Combinations of UE Radio Access Parameters for DL

Table 6.2.2.1: UE radio access capability parameter combinations, DL parameters

| Reference combination of UE Radio Access capability parameters in DL | 32kbps class | 64kbps class | 128kbps class | 384kbps class | 768kbps class | 2048kbps class |
| --- | --- | --- | --- | --- | --- | --- |
| Transport channel parameters |  |  |  |  |  |  |
| Maximum sum of number of bits of all transport blocks being received at an arbitrary time instant | 640 | 3840 | 3840 | 6400 | 10240 | 20480 |
| Maximum sum of number of bits of all convolutionally coded transport blocks being received at an arbitrary time instant | 640 | 640 | 640 | 640 | 640 | 640 |
| Maximum sum of number of bits of all turbo coded transport blocks being received at an arbitrary time instant | NA | 3840 | 3840 | 6400 | 10240 | 20480(1) 10240(2) NOTE 3 |
| Maximum number of simultaneous transport channels | 8 | 8 | 8 | 8 | 8 | 16 |
| Maximum number of simultaneous CCTrCH (FDD) | 1 | 2/1 NOTE 2 | 2/1 NOTE 2 | 2/1 NOTE 2 | 2 | 2 |
| Maximum number of simultaneous CCTrCH (TDD) | 2 | 3 | 3 | 3 | 4 | 4 |
| Maximum total number of transport blocks received within TTIs that end at the same time | 8 | 8 | 16 | 32 | 64 | 96 |
| Maximum number of TFC in the TFCS | 32 | 48 | 96 | 128 | 256 | 1024 |
| Maximum number of TF | 32 | 64 | 64 | 64 | 128 | 256 |
| Support for turbo decoding | No | Yes | Yes | Yes | Yes | Yes |
| Physical channel parameters (FDD) |  |  |  |  |  |  |
| Maximum number of DPCH/PDSCH codes to be simultaneously received | 1 | 2/1 NOTE 2 | 2/1 NOTE 2 | 3 | 3 | 3 |
| Maximum number of physical channel bits received in any 10 ms interval (DPCH, PDSCH, S-CCPCH). | 1200 | 3600/2400 NOTE2 | 7200/4800 NOTE2 | 19200 | 28800 | 57600 |
| Support for SF 512 | No | No | No | No | No | No |
| Support of PDSCH | No | Yes/No NOTE 1 | Yes/No NOTE 1 | No/Yes NOTE 1 | Yes | Yes |
| Maximum number of simultaneous S-CCPCH radio links | 1 | 1 | 1 | 1 | 1 | 1 |
| Physical channel parameters (TDD 3,84 Mcps) |  |  |  |  |  |  |
| Maximum number of timeslots per frame | 1 | 2 | 4 | 5 | 10 | 12 |
| Maximum number of physical channels per frame | 8 | 9 | 14 | 28 | 64 | 136 |
| Minimum SF | 16 | 16 | 16 | 1/16 NOTE 1 | 1/16 NOTE 1 | 1/16 NOTE 1 |
| Support of PDSCH | Yes/No NOTE 1 | Yes | Yes | Yes | Yes | Yes |
| Maximum number of physical channels per timeslot | 8 | 9 | 9 | 9 | 9 | 13 |
| Physical channel parameters (TDD 1,28 Mcps) |  |  |  |  |  |  |
| Maximum number of timeslots per subframe | 1 | 2 | 3 | 4 | 6 | 6 |
| Maximum number of physical channels per subframe | 8 | 12 | 18 | 43 | 77 | 77 |
| Minimum SF | 16 | 16 | 16 | 1/16 NOTE 1 | 1/16 NOTE 1 | 1 |
| Support of PDSCH | Yes/No Note 1 | Yes | Yes | Yes | Yes | Yes |
| Maximum number of physical channels per timeslot | 8 | 11 | 14 | 14 | 14 | 14 |
| Support of 8PSK | No | No | No | No | No | Yes |

NOTE 1:	Options represent different combinations that should be supported with conformance tests.

NOTE 2:	Options depend on the support of PDSCH. The highest value is required if PDSCH is supported.

NOTE3:	(1) For FDD and 3,84 Mcps TDD (2) For 1,28 Mcps TDD

NOTE4:	The parameters in table 6.2.2.1 need to be verified by RAN1 in more detail.

### 6.2.3	Combinations of UE Radio Access Parameters for UL

Table 6.2.3.1: UE radio access capability parameter combinations, UL parameters

| Reference combination of UE Radio Access capability parameters in UL | 32kbps class | 64kbps class | 128kbps class | 384kbps class | 768kbps class |
| --- | --- | --- | --- | --- | --- |
| Transport channel parameters |  |  |  |  |  |
| Maximum sum of number of bits of all transport blocks being transmitted at an arbitrary time instant | 640 | 3840 | 3840 | 6400 | 10240 |
| Maximum sum of number of bits of all convolutionally coded transport blocks being transmitted at an arbitrary time instant | 640 | 640 | 640 | 640 | 640 |
| Maximum sum of number of bits of all turbo coded transport blocks being transmitted at an arbitrary time instant | NA | 3840 | 3840 | 6400 | 10240 |
| Maximum number of simultaneous transport channels | 4 | 8 | 8 | 8 | 8 |
| Maximum number of simultaneous CCTrCH(TDD only) | 1 | 2 | 2 | 2 | 2 |
| Maximum total number of transport blocks transmitted within TTIs that start at the same time | 4 | 8 | 8 | 16 | 32 |
| Maximum number of TFC in the TFCS | 16 | 32 | 48 | 64 | 128 |
| Maximum number of TF | 32 | 32 | 32 | 32 | 64 |
| Support for turbo encoding | No | Yes | Yes | Yes | Yes |
| Physical channel parameters (FDD) |  |  |  |  |  |
| Maximum number of DPDCH bits transmitted per 10 ms | 1200 | 2400 | 4800 | 9600 | 19200 |
| Simultaneous reception of SCCPCH and DPCH NOTE 2 | No | No | Yes/No NOTE 1 | Yes/No NOTE 1 | Yes/No NOTE 1 |
| Simultaneous reception of SCCPCH, DPCH and PDSCH NOTE 2 | No | No | No | No | No |
| Support of PCPCH | No | No | No | No | No |
| Physical channel parameters (TDD 3,84 Mcps) |  |  |  |  |  |
| Maximum number of timeslots per frame | 1 | 2 | 3 | 7 | 9 |
| Maximum number of physical channels per timeslot | 1 | 1 | 1 | 1 | 2 |
| Minimum SF | 8 | 2 | 2 | 2 | 2 |
| Support of PUSCH | Yes/No NOTE 1 | Yes | Yes | Yes | Yes |
| Physical channel parameters (TDD 1,28 Mcps) |  |  |  |  |  |
| Maximum number of timeslots per subframe | 1 | 2 | 3 | 5 | 5 |
| Maximum number of physical channels per timeslot | 1 | 1 | 1 | 1 | 2 |
| Minimum SF | 4 | 2 | 2 | 2 | 2 |
| Support of PUSCH | Yes/No Note 1 | Yes | Yes | Yes | Yes |
| Support of 8PSK | No | No | No | No | No |

NOTE 1:	Options represent different combinations that should be supported with conformance tests.

NOTE 2:	The downlink parameters 'Simultaneous reception of SCCPCH and DPCH' and 'Simultaneous reception of SCCPCH, DPCH and PDSCH' are included in the combinations for uplink as their requirements relate to the uplink data rate. Simultaneous reception of SCCPCH and DPCH is required for the DRAC procedure that is intended for controlling uplink transmissions. In release 99, this is limited to 1 SCCPCH.

NOTE3:	The parameters in table 6.2.3.1 need to be verified by RAN1 in more detail.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 12/2000 | RP-10 | RP-000558 | - |  | Approved at TSG-RAN #10 and placed under Change Control | - | 4.0.0 |
| 03/2001 | RP-11 | RP-010038 | 001 |  | Update of TR 25.843 | 4.0.0 | 4.1.0 |