3GPP TR 25.926 V3.2.0 (2000-09)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

UE Radio Access Capabilities

(Release 1999)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Digital cellular telecommunications system, Universal Mobile Telecommunication System (UMTS), UTRA, IMT-2000

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2000, 3GPP Organizational Partners (ARIB, CWTS, ETSI, T1, TTA,TTC).

All rights reserved.

Contents

Foreword	4

1	Scope	5

2	Void	5

3	Abbreviations	5

4	UE radio access capability parameters	5

4.1	PDCP parameters	5

4.2	BMC parameters	5

4.3	RLC parameters	5

4.4	MAC parameters	6

4.5	PHY parameters	6

4.5.1	Transport channel parameters in downlink	6

4.5.2	Transport channel parameters in uplink	7

4.5.3	FDD Physical channel parameters in downlink	9

4.5.4	FDD physical channel parameters in uplink	10

4.5.5	TDD physical channel parameters in downlink	10

4.5.6	TDD physical channel parameters in uplink	11

4.5.7	RF parameters	11

4.6	Multi-mode related parameters	11

4.7	Multi-RAT related parameters	11

4.8	LCS related parameters	12

4.9	Measurement related capabilities	12

5	Possible UE radio access capability parameter settings	13

5.1	Value ranges	13

5.2	Reference UE radio access capability combinations	14

5.2.1	Combinations of common UE Radio Access Parameters for UL and DL	16

5.2.2	Combinations of UE Radio Access Parameters for DL	17

5.2.3	Combinations of UE Radio Access Parameters for UL	18

6	Usage of UE radio access capabilities	19

6.1	Examples of reference radio access bearers	19

6.2	Example mappings between reference RABs and capability combinations	19

7	Mandatory UE radio access capabilities	20

Annex A (informative): Change history	21

# Foreword

This Technical Report (TR) has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document identifies the parameters of the access stratum part of the UE radio access capabilities. Furthermore, some reference configurations of these values are defined. The intention is that these configurations will be used for test specifications.

# 2	References

[1]	3GPP TS 25.323: "Packet Data Convergence Protocol (PDCP) protocol".

# 3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

UE	User Equipment

UMTS	Universal Mobile Telecommunication System

UTRAN	UMTS Terrestrial Radio Access Network

# 4	UE radio access capability parameters

In the following the UE radio capability parameters are defined. In addition the relevant RRC configuration parameters are shown when applicable. When using the RRC configuration parameters, UTRAN needs to respect the UE capabilities. Only parameters for which there is a need to set different values for different UEs are considered as UE capability parameters. Therefore, the capabilities that are the same for all UEs, including baseline capabilities, are not listed here.

UTRAN is responsible for the respect of the UE capabilities when configuring the RBs. Actions in the UE when capabilities are in conflict with a UTRAN request are specified in RRC.

## 4.1	PDCP parameters

Header compression algorithm supported

Defines whether header compression algorithms will be supported by the UE. If it will be supported it will be the RFC 2507 as specified in 3GPP TS 25.323.

## 4.2	BMC parameters

No UE radio access capability parameters identified.

## 4.3	RLC parameters

NOTE:	It is FFS whether some of the RLC functions should be considered as UE capabilities.

Total RLC AM buffer size

The total buffer size across all RLC AM entities puts requirements on memory.

UTRAN controls that the UE capability can be fulfilled through the following parameters:

1.	The number of RLC AM entities configured (no explicit RRC parameter);

2.	UL PU size;

3.	Transmission window size (#PUs);

4.	Receiving window size (FFS whether this is configurable).

The following criterion must be fulfilled in the configuration:

i=1#RLCs Transmission_window  size ⋅UL_PU_size +i=1#RLC_AMentities Receiving  window  size ⋅ DL_PU_size  ≤ TotalRLCbuffersize

where i is the RLC "entity number"

Maximum number of AM entities

The number of AM entities affect the main part of the total processing and memory capacity to be shared between different RLC machines.

## 4.4	MAC parameters

No capability parameters identified.

## 4.5	PHY parameters

### 4.5.1	Transport channel parameters in downlink

Maximum sum of number of bits of all transport blocks being received at an arbitrary time instant

NOTE:	"Being received" refers to all bits in the active TFC within the TFCS over all simultaneous transport channels received by the UE. "Arbitrary time instant" means that the time instant corresponding to the highest sum of number of bits is relevant. This note also applies to similar parameter definitions below

This parameter is defined as:

i(Ni)

where Ni is defined as the number of bits in transport block #i, and the sum is over all transport blocks being received at an arbitrary time instant. All transport blocks that are to be simultaneously received by the UE on DCH, FACH, PCH and DSCH transport channels are included in the parameter.

A UE does not need to support a TFC within the TFCS for which the sum of Number of Transport Blocks * Transport Block size over all simultaneous transport channels is larger than what the UE capability indicates.

Maximum sum of number of bits of all convolutionally coded transport blocks being received at an arbitrary time instant.

This parameter is defined similar to the parameter above, but the sum includes only convolutionally coded transport blocks.

Maximum sum of number of bits of all turbo coded transport blocks being received at an arbitrary time instant.

This parameter is defined similar to the parameter above, but the sum includes only turbo coded transport blocks.

Maximum number of simultaneous transport channels

This is defined as the maximum number of Transport Channels that should be possible to process simultaneously, not taking into account the rate of each Transport Channel.

The number of simultaneous transport channels affects how the total memory space and processing capacity can be shared among the transport channels.

A UE does not need to support more simultaneous transport channels than the UE capability allows for.

Maximum number of simultaneous CCTrCH

CCTrCH should be interpreted as CCTrCH of any type, i.e. consisting of DCH, FACH or DSCH.

Maximum total number of transport blocks received within TTIs that end within the same 10 ms interval

All transport blocks that are to be simultaneously received by the UE on DCH, FACH, PCH and DSCH transport channels are included in the parameter.

Relates to processing requirements for CRC in downlink.

A UE does not need to support a TFC within the TFCS for which the sum of Number of Transport Blocks is larger than what the UE capability indicates.

Maximum number of TFC in the TFCS

The maximum number of TFC in a TFCS sets the size of the TFCI to TFCS mapping table to be handled by the UE.

Maximum number of TF

The maximum total number of downlink transport formats the UE can store.

Support for turbo decoding

Defines whether turbo decoding is supported or not.

The UTRAN configuration parameter is Type of channel coding which is part of the Transport format set (TFS) of each transport channel.

### 4.5.2	Transport channel parameters in uplink

Maximum sum of number of bits of all transport blocks being transmitted at an arbitrary time instant

NOTE:	"Being transmitted" refers to all bits in the active TFC within the TFCS over all simultaneous transport channels transmitted by the UE. "Arbitrary time instant" means that the time instant corresponding to the highest sum of number of bits is relevant. This note also applies to similar parameter definitions below.

This parameter is defined as:

i(Ni)

where Ni is defined as the number of bits in transport block #i, and the sum is over all transport blocks being transmitted at an arbitrary time instant.

This parameter is related to memory requirements for uplink data received from MAC before it can be transmitted over the radio interface. As shown in Figure 4.1 the worst case occurs for the maximum TTI.

A UE does not need to support a TFC within the TFCS for which the sum of Number of Transport Blocks * Transport Block size over all simultaneous transport channels is larger than what the UE capability indicates.

Maximum sum of number of bits of all convolutionally coded transport blocks being transmitted at an arbitrary time instantThis parameter is defined similar to the parameter above, but the sum includes only convolutionally coded transport blocks.

Maximum sum of number of bits of all turbo coded transport blocks being transmitted at an arbitrary time instant

This parameter is defined similar to the parameter above, but the sum includes only turbo coded transport blocks.

Maximum number of simultaneous transport channels

The number of simultaneous transport channels affects how the total memory space and processing capacity can be shared among the transport channels.

UTRAN shall not set up more simultaneous transport channels than the UE capability allows for.

Maximum number of simultaneous CCTrCH

TDD only. For FDD there is always only one CCTrCH at a time.

Maximum total number of transport blocks transmitted within TTIs that start at the same time

Relates to processing requirements for CRC in uplink.

A UE does not need to support the TFC within the TFCS for which the sum of Number of Transport Blocks is larger than what the UE capability allows for.

Maximum number of TFC in the TFCS

The maximum number of TFC in a TFCS sets the size of the TFCI to TFCS mapping table to be handled by the UE.

Maximum number of TF

The maximum total number of uplink transport formats the UE can store.

Support for turbo encoding

Defines whether turbo encoding is supported or not.

The UTRAN configuration parameter is Type of channel coding which is part of the Transport format set (TFS) of each transport channel.

Figure 4.1: UE transport channel processing limitations in uplink

NOTE:	When CPCH is supported, then simultaneous DPCCH & SCCPCH reception is needed.

### 4.5.3	FDD Physical channel parameters in downlink

Maximum number of DPCH/PDSCH codes to be simultaneously received

Defines the number of codes the UE is capable of receiving in parallel. For DPCH in soft/softer handover, each DPCH is only calculated once in this capability. The capability does not include codes used for S-CCPCH.

Maximum number of physical channel bits received in any 10 ms interval (DPCH, PDSCH, S-CCPCH)

Defines the number of physical channel bits the UE is capable of receiving. For DPCH in soft/softer handover, each DPCH is only calculated once in this capability.

The number of DPCH channel bits indicates the capability for normal, un-compressed mode.

The parameter also indicates the capability of the UE to support compressed mode by spreading factor reduction. For parameter values up to and including 9600 bits, the UE shall also be able to support compressed mode by SF reduction when operating in normal mode, at any value up to the reported capability. For parameter values greater than 9600 bits, the UE shall be able to support compressed mode by spreading factor reduction when operating, in normal mode, at any value up to half the reported capability or 9600bits, whichever is greater.

Support for SF 512

Spreading factor 512 should not be mandatory for all UEs.

The corresponding configuration parameter is Spreading factor which is part of Downlink DPCH info.

Support of PDSCH

Support of PDSCH is only required for some RAB realizations, and is therefore a UE capability.

The corresponding configuration parameter is Downlink transport channel type, which is part of RB mapping info.

Simultaneous reception of SCCPCH and DPCH

Simultaneous reception of SCCPCH and DPCH, i.e. simultaneous reception of FACH and DCH is required for e.g. DRAC procedure, but it should not be mandatory for all UEs (e.g. speech only UEs).

There is no specific configuration parameter.

Simultaneous reception of SCCPCH, DPCH and PDSCH

Simultaneous reception of SCCPCH, DPCH and PDSCH, i.e. simultaneous reception of FACH, DCH and DSCH is required for e.g. simultaneous use of DSCH and the DRAC procedure, but it should not be mandatory for all UEs (e.g. speech only UEs). The PDSCH part of this capability is only relevant if the UE supports PDSCH, as covered by the capability "Support of PDSCH".

There is no specific configuration parameter.

Maximum number of simultaneous S-CCPCH radio links

Defines the maximum number of radio links on which the UE is capable of receiving S-CCPCH simultaneously.

### 4.5.4	FDD physical channel parameters in uplink

Maximum number of DPDCH bits per 10 ms

This capability combines the 'Max number of DPDCH' and 'Minimum SF' capabilities into one capability. Note that no flexibility is lost due to this, as multiple DPDCH is only used for SF=4, i.e. when the number of DPDCH bits exceed a certain value.

The number of DPDCH channel bits indicates the capability for normal, un-compressed mode. The UE shall also be able to support compressed mode by SF reduction when operating at this value.

Support of PCPCH

Support of PCPCH is only required for some RAB realizations, and is therefore a UE capability.

There is no specific configuration parameter.

### 4.5.5	TDD physical channel parameters in downlink

Maximum number of timeslots per frame

Defines the maximum number of timeslots per frame that the UE can receive.

Maximum number of physical channels per frame

This parameter defines how many physical channels can be received during one frame. The distribution of the received physical channels on the received timeslots can be arbitrary.

Minimum SF

Defines the minimum SF supported by the UE.

Support of PDSCH

Defines whether PDSCH is supported or not.

Maximum number of physical channels per timeslot

This parameter defines how many physical channels can be received within one timeslot.

### 4.5.6	TDD physical channel parameters in uplink

Maximum Number of timeslots per frame

Defines the maximum number of timeslots per frame that the UE can transmit.

Maximum number of physical channels per timeslot

Defines the maximum number physical channels transmitted in parallel during one timeslot.

Minimum SF

Defines the minimum SF supported by the UE.

Support of PUSCH

Defines whether PUSCH is supported or not.

### 4.5.7	RF parameters

UE power class

The value is fixed per UE and is not related to any configuration parameter.

Radio frequency bands

Defines the uplink and downlink frequency bands supported by the UE.

Configuration parameters are UTRA RF Channel numbers for uplink and downlink, which are part of Frequency info.

Tx/Rx frequency separation

Defines the uplink/downlink frequency separations supported by the UE.

Configuration parameters are UTRA RF Channel numbers for uplink and downlink, which are part of Frequency info.

Chip rate capability

Chip rates supported by the UE.

Corresponding configuration parameter is chip rate, which is part of Frequency info.

## 4.6	Multi-mode related parameters

Support of UTRA FDD/TDD

Defines whether UTRA FDD and/or TDD are supported.

There is no explicit configuration parameter.

## 4.7	Multi-RAT related parameters

Support of GSM

Defines whether GSM is supported or not.

There is no explicit configuration parameter.

Support of multi-carrier

Defines whether multi-carrier is supported or not.

There is no explicit configuration parameter.

## 4.8	LCS related parameters

Standalone location method(s) supported

Defines if a UE can measure its location by some means unrelated to UTRAN (e.g. if the UE has access to a standalone GPS receiver).

OTDOA UE based method supported

Defines if a UE supports the OTDOA UE based schemes.

Network Assisted GPS support

Defines if a UE supports either of the two types of assisted GPS schemes, namely "Network based", "UE based", "Both", or "none".

GPS reference time capable

Defines if a UE has the capability to measure GPS reference time as defined in 25.215.

Support for IPDL

Defines if a UE has the capability to use IPDL to enhance its "SFN-SFN observed time difference –type 2" measurement.

## 4.9	Measurement related capabilities

Need for downlink compressed mode

Defines whether the UE needs compressed mode in the downlink in order to perform inter-frequency or inter-RAT measurements. There are separate parameters for measurements on each UTRA mode, on each RAT, and in each frequency band.

Need for uplink compressed mode

Defines whether the UE needs compressed mode in the uplink in order to perform inter-frequency or inter-RAT measurements. There are separate parameters for measurements on each UTRA mode, on each RAT, and in each frequency band.

# 5	Possible UE radio access capability parameter settings

## 5.1	Value ranges

- Table 5.1: UE radio access capability parameter value ranges

|  |  | UE radio access capability parameter | Value range |
| --- | --- | --- | --- |
| PDCP parameters |  | Header compression algorithm supported | Yes/No |
| RLC parameters |  | Total RLC AM buffer size | 2,10,50,100,150,500,1000 kBytes |
|  |  | Maximum number of AM entities | 3,4,5,6,8,16,32 |
| PHY parameters | Transport channel parameters in downlink | Maximum sum of number of bits of all transport blocks being received at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840 |
|  |  | Maximum sum of number of bits of all convolutionally coded transport blocks being received at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840 |
|  |  | Maximum sum of number of bits of all turbo coded transport blocks being received at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840 |
|  |  | Maximum number of simultaneous transport channels | 4, 8, 16, 32 |
|  |  | Maximum number of simultaneous CCTrCH | 1, 2, 3, 4, 5, 6, 7, 8 |
|  |  | Maximum total number of transport blocks received within TTIs that end within the same 10 ms interval | 4, 8, 16, 32, 48, 64, 96, 128, 256, 512 |
|  |  | Maximum number of TFC in the TFCS | 16, 32, 48, 64, 96, 128, 256, 512, 1024 |
|  |  | Maximum number of TF | 32, 64, 128, 256, 512, 1024 |
|  |  | Support for turbo decoding | Yes/No |
|  | Transport channel parameters in uplink | Maximum sum of number of bits of all transport blocks being transmitted at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840 |
|  |  | Maximum sum of number of bits of all convolutionally coded transport blocks being transmitted at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840 |
|  |  | Maximum sum of number of bits of all turbo coded transport blocks being transmitted at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840 |
|  |  | Maximum number of simultaneous transport channels | 2, 4, 8, 16, 32 |
|  |  | Maximum number of simultaneous CCTrCH of DCH type (TDD only) | 1, 2, 3, 4, 5, 6, 7, 8 |
|  |  | Maximum total number of transport blocks transmitted within TTIs that start at the same time | 2, 4, 8, 16, 32, 48, 64, 96, 128, 256, 512 |
|  |  | Maximum number of TFC in the TFCS | 4, 8, 16, 32, 48, 64, 96, 128, 256, 512, 1024 |
|  |  | Maximum number of TF | 32, 64, 128, 256, 512, 1024 |
|  |  | Support for turbo encoding | Yes/No |
|  | FDD Physical channel parameters in downlink | Maximum number of DPCH/PDSCH codes to be simultaneously received | 1, 2, 3, 4, 5, 6, 7, 8 |
|  |  | Maximum number of physical channel bits received in any 10 ms interval (DPCH, PDSCH, S-CCPCH) | 600, 1200, 2400, 3600, 4800, 7200, 9600, 14400, 19200, 28800, 38400, 48000, 57600, 67200, 76800 |
|  |  | Support for SF 512 | Yes/No |
|  |  | Support of PDSCH | Yes/No |
|  |  | Simultaneous reception of SCCPCH and DPCH | Yes/No |
|  |  | Simultaneous reception of SCCPCH, DPCH and PDSCH | Yes/No |
|  |  | Maximum number of simultaneous S-CCPCH radio links | 1 NOTE: Only the value 1 is part of R99 |
|  | FDD Physical channel parameters in uplink | Maximum number of DPDCH bits transmitted per 10 ms | 600, 1200, 2400, 4800, 960, 19200, 28800, 38400, 48000, 57600 |
|  |  | Support of PCPCH | Yes/No |
|  | TDD physical channel parameters in downlink | Maximum number of timeslots per frame | 1..14 |
|  |  | Maximum number of physical channels per frame | 1,2,3..,224 |
|  |  | Minimum SF | 16, 1 |
|  |  | Support of PDSCH | Yes/No |
|  |  | Maximum number of physical channels per timeslot | 1..16 |
|  | TDD physical channel parameters in uplink | Maximum Number of timeslots per frame | 1..14 |
|  |  | Maximum number of physical channels per timeslot | 1, 2 |
|  |  | Minimum SF | 16,8,4,2,1 |
|  |  | Support of PUSCH | Yes/No |
| RF parameters | FDD RF parameters | UE power class (25.101 subclause 6.2.1) | 3, 4 NOTE: Only power classes 3 and 4 are part of R99 |
|  |  | Tx/Rx frequency separation (25.101 subclause 5.3) . NOTE: Not applicable if UE is not operating in frequency band a | 190 MHz 174.8-205.2 MHz 134.8-245.2 MHz |
| RF parameters | TDD RF parameters | UE power class (25.102) | 2,3 NOTE: Only power classes 2 and 3 are part of R99 |
|  |  | Radio frequency bands (25.102) | a), b), c), a+b), a+c), a+b+c) |
|  |  | Chip rate capability (25.102) | 3.84,1.28 |
| Multi-mode related parameters |  | Support of UTRA FDD/TDD | FDD, TDD, FDD+TDD |
| Multi-RAT related parameters |  | Support of GSM | Yes/No |
|  |  | Support of multi-carrier | Yes/No |
| LCS related parameters |  | Standalone location method(s) supported | Yes/No |
|  |  | Network assisted GPS support | Network based / UE based / Both/ None |
|  |  | GPS reference time capable | Yes/No |
|  |  | Support for IPDL | Yes/No |
|  |  | Support for OTDOA UE based method | Yes/No |
| Measurement related capabilities |  | Need for downlink compressed mode | Yes/No (per frequency band, UTRA mode and RAT) |
|  |  | Need for uplink compressed mode | Yes/No (per frequency band, UTRA mode and RAT) |

## 5.2	Reference UE radio access capability combinations

Based on required UE radio access capabilities to support reference RABs as defined in clause 6, this clause lists reference UE Radio Access capability combinations. Subclause 5.2.1 defines reference combinations of UE radio access capability parameters common for UL and DL. Subclause 5.2.2 and 5.2.3 define reference combinations of UE radio access capability parameters that are separate for DL and UL respectively. A reference combination for common UL and DL parameters, one combination for UL parameters and one combination for DL parameters together relate to a UE with a certain implementation complexity, that allows support for one or several combined reference RABs. Combinations for UL and DL can be chosen independently. The bit rate supported by the selected combination of common UL and DL parameters needs to be at least as high as the maximum out of the supported bit rates of the selected combination of DL parameters and the selected combination of UL parameters. Different combinations have different levels of implementation complexity.

For defined reference RABs, it is possible to require a UE to meet a certain reference UE radio access capability combination. Each UE needs to have capabilities complying with a given reference radio access capability combination. Each individual radio access capability parameter as defined in Subclause 5.1 shall be signalled.

The reference combination numbers shall not be used in the signalling of UE radio access capabilities between the UE and UTRAN. Reference UE radio access capability combinations provide default configurations that should be used as a basis for conformance testing against reference RABs.

Allowed values of UE capability parameters are limited by the defined range and granularity of values in Subclause 5.1. Values might change depending on further definition of reference RABs for testing.

### Combinations of common UE Radio Access Parameters
for UL and DL

NOTE:	It is FFS whether measurement-related capabilities need to be included in the combinations. These capabilities are independent from the supported RABs.

Table 5.2.1.1: UE radio access capability parameter combinations, parameters common for UL and DL

| Reference combination of UE Radio Access capability parameters common for UL and DL | 32kbps class | 64kbps class | 128kbps class | 384kbps class | 768kbps class | 2048kbps class |
| --- | --- | --- | --- | --- | --- | --- |
| PDCP parameters |  |  |  |  |  |  |
| Header compression algorithm supported | No | No/Yes NOTE 1 | No/Yes NOTE 1 | No/Yes NOTE 1 | No/Yes NOTE 1 | No/Yes NOTE 1 |
| RLC parameters |  |  |  |  |  |  |
| Total RLC AM buffer size (kbytes) | 10 | 10 | 50 | 50 | 100 | 500 |
| Maximum number of AM entities | 4 | 4 | 5 | 6 | 8 | 8 |
| Multi-mode related parameters |  |  |  |  |  |  |
| Support of UTRA FDD/TDD | FDD / FDD+TDD / TDD NOTE 1 |  |  |  |  |  |
| Multi-RAT related parameters |  |  |  |  |  |  |
| Support of GSM | Yes/No NOTE 1 |  |  |  |  |  |
| Support of multi-carrier | Yes/No NOTE 1 |  |  |  |  |  |
| LCS related parameters |  |  |  |  |  |  |
| Standalone location method(s) supported | Yes/No NOTE 1 |  |  |  |  |  |
| Network assisted GPS support | Network based / UE based / Both/ None NOTE 1 |  |  |  |  |  |
| GPS reference time capable | Yes/No NOTE 1 |  |  |  |  |  |
| Support for IPDL | Yes/No NOTE 1 |  |  |  |  |  |
| Support for OTDOA UE based method | Yes/No NOTE 1 |  |  |  |  |  |
| RF parameters for FDD |  |  |  |  |  |  |
| UE power class | 3 / 4 NOTE 1 |  |  |  |  |  |
| Tx/Rx frequency separation | 190 MHz |  |  |  |  |  |
| RF parameters for TDD |  |  |  |  |  |  |
| Radio frequency bands | A / b / c / a+b / a+c / b+c / a+b+c NOTE 1 |  |  |  |  |  |
| Chip rate capability | 1.28 / 3.84 Mchip/sec NOTE 1 |  |  |  |  |  |
| UE power class | 2 / 3 NOTE 1 |  |  |  |  |  |

NOTE 1:	Options represent different combinations that should be supported with Conformance Tests.

### 5.2.2	Combinations of UE Radio Access Parameters for DL

Table 5.2.2.1: UE radio access capability parameter combinations, DL parameters

| Reference combination of UE Radio Access capability parameters in DL | 32kbps class | 64kbps class | 128kbps class | 384kbps class | 768kbps class | 2048kbps class |
| --- | --- | --- | --- | --- | --- | --- |
| Transport channel parameters |  |  |  |  |  |  |
| Maximum sum of number of bits of all transport blocks being received at an arbitrary time instant | 640 | 3840 | 3840 | 6400 | 10240 | 20480 |
| Maximum sum of number of bits of all convolutionally coded transport blocks being received at an arbitrary time instant | 640 | 640 | 640 | 640 | 640 | 640 |
| Maximum sum of number of bits of all turbo coded transport blocks being received at an arbitrary time instant | NA | 3840 | 3840 | 6400 | 10240 | 20480 |
| Maximum number of simultaneous transport channels | 8 | 8 | 8 | 8 | 8 | 16 |
| Maximum number of simultaneous CCTrCH (FDD) | 1 | 2/1 NOTE 2 | 2/1 NOTE 2 | 2/1 NOTE 2 | 2 | 2 |
| Maximum number of simultaneous CCTrCH (TDD) | 2 | 3 | 3 | 3 | 4 | 4 |
| Maximum total number of transport blocks received within TTIs that end at the same time | 8 | 8 | 16 | 32 | 64 | 96 |
| Maximum number of TFC in the TFCS | 32 | 48 | 96 | 128 | 256 | 1024 |
| Maximum number of TF | 32 | 64 | 64 | 64 | 128 | 256 |
| Support for turbo decoding | No | Yes | Yes | Yes | Yes | Yes |
| Physical channel parameters (FDD) |  |  |  |  |  |  |
| Maximum number of DPCH/PDSCH codes to be simultaneously received | 1 | 2/1 NOTE 2 | 2/1 NOTE 2 | 3 | 3 | 3 |
| Maximum number of physical channel bits received in any 10 ms interval (DPCH, PDSCH, S-CCPCH). | 1200 | 3600/2400 NOTE2 | 7200/4800 NOTE2 | 19200 | 28800 | 57600 |
| Support for SF 512 | No | No | No | No | No | No |
| Support of PDSCH | No | Yes/No NOTE 1 | Yes/No NOTE 1 | No/Yes NOTE 1 | Yes | Yes |
| Maximum number of simultaneous S-CCPCH radio links | 1 | 1 | 1 | 1 | 1 | 1 |
| Physical channel parameters (TDD) |  |  |  |  |  |  |
| Maximum number of timeslots per frame | 1 | 2 | 4 | 5 | 10 | 12 |
| Maximum number of physical channels per frame | 8 | 9 | 14 | 28 | 64 | 136 |
| Minimum SF | 16 | 16 | 16 | 1/16 NOTE 1 | 1/16 NOTE 1 | 1/16 NOTE 1 |
| Support of PDSCH | Yes/No NOTE 1 | Yes | Yes | Yes | Yes | Yes |
| Maximum number of physical channels per timeslot | 8 | 9 | 9 | 9 | 9 | 13 |

NOTE 1:	Options represent different combinations that should be supported with conformance tests.

NOTE 2:	Options depend on the support of PDSCH. The highest value is required if PDSCH is supported.

### 5.2.3	Combinations of UE Radio Access Parameters for UL

Table 5.2.3.1: UE radio access capability parameter combinations, UL parameters

| Reference combination of UE Radio Access capability parameters in UL | 32kbps class | 64kbps class | 128kbps class | 384kbps class | 768kbps class |
| --- | --- | --- | --- | --- | --- |
| Transport channel parameters |  |  |  |  |  |
| Maximum sum of number of bits of all transport blocks being transmitted at an arbitrary time instant | 640 | 3840 | 3840 | 6400 | 10240 |
| Maximum sum of number of bits of all convolutionally coded transport blocks being transmitted at an arbitrary time instant | 640 | 640 | 640 | 640 | 640 |
| Maximum sum of number of bits of all turbo coded transport blocks being transmitted at an arbitrary time instant | NA | 3840 | 3840 | 6400 | 10240 |
| Maximum number of simultaneous transport channels | 4 | 8 | 8 | 8 | 8 |
| Maximum number of simultaneous CCTrCH(TDD only) | 1 | 2 | 2 | 2 | 2 |
| Maximum total number of transport blocks transmitted within TTIs that start at the same time | 4 | 8 | 8 | 16 | 32 |
| Maximum number of TFC in the TFCS | 16 | 32 | 48 | 64 | 128 |
| Maximum number of TF | 32 | 32 | 32 | 32 | 64 |
| Support for turbo encoding | No | Yes | Yes | Yes | Yes |
| Physical channel parameters (FDD) |  |  |  |  |  |
| Maximum number of DPDCH bits transmitted per 10 ms | 1200 | 2400 | 4800 | 9600 | 19200 |
| Simultaneous reception of SCCPCH and DPCH NOTE 2 | No | No | Yes/No NOTE 1 | Yes/No NOTE 1 | Yes/No NOTE 1 |
| Simultaneous reception of SCCPCH, DPCH and PDSCH NOTE 2 | No | No | No | No | No |
| Support of PCPCH | No | No | No | No | No |
| Physical channel parameters (TDD) |  |  |  |  |  |
| Maximum Number of timeslots per frame | 1 | 2 | 3 | 7 | 9 |
| Maximum number of physical channels per timeslot | 1 | 1 | 1 | 1 | 2 |
| Minimum SF | 8 | 2 | 2 | 2 | 2 |
| Support of PUSCH | Yes/No NOTE 1 | Yes | Yes | Yes | Yes |

NOTE 1:	Options represent different combinations that should be supported with conformance tests.

NOTE 2:	The downlink parameters 'Simultaneous reception of SCCPCH and DPCH' and 'Simultaneous reception of SCCPCH, DPCH and PDSCH' are included in the combinations for uplink as their requirements relate to the uplink data rate. Simultaneous reception of SCCPCH and DPCH is required for the DRAC procedure that is intended for controlling uplink transmissions. In release 99, this is limited to 1 SCCPCH.

# 6	Usage of UE radio access capabilities

NOTE:	The rationale for the parameter combination settings will be explained here.

## 6.1	Examples of reference radio access bearers

In Table 6.1 reference RAB A-G are defined with some characteristics that impact the required UE Radio Access capabilities. These reference RABs shall be seen as example RABs covered by the reference UE radio access capability combinations defined in Subclause 5.2. Reference RABs for conformance testing are specified in TS 34.108.

- Table 6.1: Reference RABs

| Reference RAB | A | B | C | D | E | F | G |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RAB characteristics and mapping to DCH Coding (CC/TC) | Conversational speech 4.75-12.2 kbps (20 ms TTI) CC, Only one rate per RAB | Conversational 64 kbps (40 ms TTI) TC | Streaming max. 57.6 kbps (40 ms TTI) TC | Interactive/ Background max. 32 kbps (10 ms TTI) CC | Interactive/ Background max. 64 kbps (20 ms TTI) TC | Interactive/ Background max. 384 kbps (10/20 ms TTI) TC | Interactive/ Background max. 2048 kbps (10 ms TTI) TC |
| DCH carrying DCCH (rate, TTI) | 3.4kpbs, 40ms | 3.4kbps, 40ms/ 6.4kbps, 20ms | 3.4kbps, 40ms/ 6.4kbps, 20ms | 3.4kbps, 40ms/ 12.8kbps, 10ms | 3.4kbps, 40ms/ 12.8kbps, 10ms | 3.4kbps, 40ms/ 12.8kpbs, 10ms | 3.4kbps, 40ms/ 12.8kpbs, 10ms |

## 6.2	Example mappings between reference RABs and capability combinations

The following examples show how the reference RABs of Table 6.1 can be mapped to the reference UE radio access capability combinations that are listed in Clause 5.

- Table 6.2: Example mappings between capability combinations and RAB combinations

| Reference UE radio access capability combinations | Examples of supported reference RAB combination |
| --- | --- |
| 32kbps class | One at the time of the following: - A - D |
| 64kbps class | One at the time of the following: - B - C - E - A and D simultaneously - A and E simultaneously - A and B simultaneously - A and C simultaneously - The RAB combination supported by 32kbps class |
| 128kbps class | One at the time of the following: - 2 times E - The RAB combination supported by 64kbps class |
| 384kbps class | One at the time of the following: - E + B - 2 times B - F (TTI 10 ms) - A and F (TTI 10 ms) simultaneously - The RAB combination supported by 128kbps class |
| 768kbps class | One at the time of the following: - F (TTI 20 ms) - A and F (TTI 20 ms) simultaneously - 2 times F (TTI 10 ms) in DL. - The RAB combination supported by 384kbps class |
| 2048kbps class | One at the time of the following: - G in DL only - A and G simultaneously - The RAB combination supported by 768kbps class |

# 7	Mandatory UE radio access capabilities

NOTE:	In this section features and requirements that are mandatory for UEs (capabilities that do not need to be signalled) will be listed for information. The normative descriptions are part of the respective specifications.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| TSG-RAN# | Version | CR | Tdoc RAN | New Version | Subject/Comment |
| RAN_07 | - | - | RP-000052 | 3.0.0 | (03/00) Approved at TSG-RAN #7 and placed under Change Control |
| RAN_08 | 3.0.0 | 003 | RP-000229 | 3.1.0 | (06/00) Updated Ad Hoc changes |
| RAN_08 | 3.0.0 | 008 | RP-000229 | 3.1.0 | CPCH note to the the parameter definitions |
| RAN_09 | 3.1.0 | 010 | RP-000368 | 3.2.0 | (09/00) TDD DL Physical Channel Capability per Timeslot |
| RAN_09 | 3.1.0 | 012 | RP-000368 | 3.2.0 | Change to UE Capability definition |
| RAN_09 | 3.1.0 | 013 | RP-000368 | 3.2.0 | Physical parameter changes |