3GPP TS 29.199-7 V7.0.0 (2006-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

Open Service Access (OSA);

Parlay X Web Services;

Part 7: Account management

(Release 7)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

UMTS, network, API, OSA

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2006, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	7

4	Detailed service description	7

5	Namespaces	7

6	Sequence diagrams	8

6.1	Prepaid account recharge using a voucher	8

6.2	Prepaid account recharge using direct payment	9

7	XML Schema data type definition	10

7.1	DatedTransaction structure	10

7.2	Balance structure	10

7.3	BalanceExpireDetails structure	10

7.4	AccountChangedEvent enumeration	10

8	Web Service interface definition	10

8.1	Interface: AccountManagement	10

8.1.1	Operation: GetBalance	10

8.1.1.1	Input message: GetBalanceRequest	11

8.1.1.2	Output message: GetBalanceResponse	11

8.1.1.3	Referenced faults	11

8.1.2	Operation: GetCreditExpiryDate	11

8.1.2.1	Input message: GetCreditExpiryDateRequest	11

8.1.2.2	Output message: GetCreditExpiryDateResponse	11

8.1.2.3	Referenced faults	11

8.1.3	Operation: BalanceUpdate	12

8.1.3.1	Input message: BalanceUpdateRequest	12

8.1.3.2	Output message: BalanceUpdateResponse	12

8.1.3.3	Referenced faults	12

8.1.4	Operation: VoucherUpdate	12

8.1.4.1	Input message: VoucherUpdateRequest	13

8.1.4.2	Output message: VoucherUpdateResponse	13

8.1.4.3	Referenced Faults	13

8.1.5	Operation: GetHistory	13

8.1.5.1	Input message: GetHistoryRequest	13

8.1.5.2	Output message: GetHistoryResponse	14

8.1.5.3	Referenced faults	14

8.1.6	Operation: GetBalanceTypes	14

8.1.6.1	Input message: GetBalanceTypesRequest	14

8.1.6.2	Output message: GetBalanceTypesResponse	14

8.1.6.3	Referenced faults	14

8.2	Interface: AccountManagementNotificationManager	14

8.2.1	Operation: startNotification	15

8.2.1.1	Input message: startNotificationRequest	15

8.2.1.2	Output message: startNotificationResponse	15

8.2.1.3	Referenced faults	15

8.2.2	Operation: endNotification	15

8.2.2.1	Input message: endNotificationRequest	15

8.2.2.2	Output message: endNotificationResponse	15

8.2.2.3	Referenced faults	15

8.3	Interface: AccountManagementNotification	16

8.3.1	Operation: accountCharged	16

8.3.1.1	Input message: accountChargedRequest	16

8.3.1.2	Output message: accountChargedResponse	16

8.3.1.3	Referenced faults	16

8.3.2	Operation: accountRecharged	16

8.3.2.1	Input message: accountRechargedRequest	16

8.3.2.2	Output message: accountRechargedResponse	16

8.3.2.3	Referenced faults	16

8.3.3	Operation: accountLow	16

8.3.3.1	Input message: accountLowRequest	17

8.3.3.2	Output message: accountLowResponse	17

8.3.3.3	Referenced faults	17

9	Fault definitions	17

9.1	Fault: ServiceException	17

9.1.1	End user authentication failed	17

9.1.2	Unknown Voucher	17

9.2	Fault: PolicyException	17

9.2.1	Vouchers not accepted	17

10	Service policies	18

Annex A (normative):	WSDL for Account Management	19

Annex B (informative):	Change history	20

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

3GPP acknowledges the contribution of the Parlay X Web Services specifications from The Parlay Group. The Parlay Group is pleased to see 3GPP acknowledge and publish the present document, and the Parlay Group looks forward to working with the 3GPP community to improve future versions of the present document.

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part 7 of a multi-part deliverable covering the 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Open Service Access (OSA); Parlay X Web Services, as identified below:

Part 1:	"Common ";

Part 2:	"Third party call";

Part 3:	"Call Notification";

Part 4:	"Short Messaging";

Part 5:	"Multimedia Messaging";

Part 6:	"Payment";

Part 7:	"Account management";

Part 8:	"Terminal Status";

Part 9:	"Terminal location";

Part 10:	"Call handling";

Part 11:	"Audio call";

Part 12:	"Multimedia conference";

Part 13:	"Address list management";

Part 14:	"Presence".

Part 15:	"Message Broadcast";

Part 16: 	"Geocoding";

Part 17:	"Application-driven Quality of Service (QoS)".

# 1	Scope

The present document is Part 7 of the Stage 3 Parlay X Web Service  specification for Open Service Access (OSA).

The OSA specifications define an architecture that enables application developers to make use of network functionality through an open standardized interface, i.e. the OSA APIs. The concepts and the functional architecture for the OSA are contained in 3GPP TS 23.198 [3]. The requirements for OSA are contained in 3GPP TS 22.127 [2].

The present document specifies the Account Management Web Service aspects of the interface. All aspects of the Account Management Web Service are defined here, these being:

- Name spaces.

- Sequence diagrams.

- Data definitions.

- Interface specification plus detailed method descriptions.

- Fault definitions.

- Service policies.

- WSDL Description of the interfaces.

The present document has been defined jointly between 3GPP TSG CT WG5, ETSI TISPAN and The Parlay Group.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.127: "Service Requirement for the Open Services Access (OSA); Stage 1".

[3]	3GPP TS 23.198: "Open Service Access (OSA); Stage 2".

4]	3GPP TS 22.101: "Service aspects; Service principles".

[5]	W3C Recommendation (2 May 2001): "XML Schema Part 2: Datatypes".

NOTE:	Available at http://www.w3.org/TR/2001/REC-xmlschema-2-20010502/.

[6]	3GPP TS 29.199-1: "Open Service Access (OSA); Parlay X Web Services; Part 1: Common".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TS 29.199-1 [6] apply.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TS 29.199-1 [6] apply.

# 4	Detailed service description

Pre-paid subscribers, whether they have subscribed to pre-paid telephony, SMS, or data service, have credits with their service providers; the consumption of services will lead to reduction of their credit, or the credit may expire. Therefore, from time to time, subscribers may have to recharge their accounts.  This occurs through an application that interfaces with the subscriber either directly or indirectly. Examples of direct interaction are voice prompts and WAP/web pages, or even SMS. Typically, such multi-modal applications either request a currency amount and, e.g. credit card information, or a voucher number plus credentials. The voucher number and credentials are then validated and causes a pre-determined currency amount to be transferred.

The Parlay X Account Management API described in the present document supports account querying, direct recharging and recharging through vouchers. As a side effect, it may prevent subscribers from having their account balance credits expire.

# 5	Namespaces

The AccountManagement interface uses the namespace:

http://www.csapi.org/wsdl/parlayx/account_management/v3_0

The AccountManagementNotification interface uses the namespace:

http://www.csapi.org/wsdl/parlayx/ account_management /notification/v3_0

The AccountManagementNotificationManager interface uses the namespace:

http://www.csapi.org/wsdl/parlayx/ account_management /notification_manager/v3_0

The data types are defined in the namespace:

http://www.csapi.org/schema/parlayx/account_management/v3_0

The 'xsd' namespace is used in the present document to refer to the XML Schema data types defined in XML Schema [5]. The use of the name 'xsd' is not semantically significant.

# 6	Sequence diagrams

This subclause discusses three scenarios; one where a subscriber uses a voucher, one where the subscriber directly recharges after the payment is cleared, and one where the subscriber checks the recent transactions.

NOTE:	Associated Account Management API messages are shown in 'bold' format: e.g. (getBalance).

## 6.1	Prepaid account recharge using a voucher

The prepaid subscriber wishes to recharge their account with a voucher and query their account balance. The subscriber uses their mobile phone or other wireline phone to interact with an IVR system. In order to recharge their account, the subscriber must enter the voucher number, the MSISDN to be recharged , and PIN(s). The IVR system accesses an external voucher database to validate the voucher number. The subscriber's account balance is then increased with the value of the voucher (voucherUpdate). The subscriber queries their account balance (getBalance), before and/or after the recharge.

Figure 1

## 6.2	Prepaid account recharge using direct payment

Directly recharging (i.e. without a voucher) works much along the same way.  In this case, we assume the prepaid subscriber interacts with a web page. After providing the MSISDN, along with the PIN, the user can query the account balance (getBalance). For recharging, the subscriber must enter payment details, for example credit card information, from which the payment will be made. After clearing the payment details, the currency amount will be transferred and the subscriber's prepaid account balance expiration date will be reset (balanceUpdate). The subscriber also queries their account balance expiration date (getCreditExpiryDate), after the recharge.

Figure 2

# 7	XML Schema data type definition

## 7.1	DatedTransaction structure

This data structure represents a transaction record.

| Element Name | Element Type | Optional | Description |
| --- | --- | --- | --- |
| TransactionDate | xsd:dateTime | No | The date the transaction occurred. |
| TransactionDetails | xsd:string | No | The transaction details. |

## 7.2	Balance structure

This data structure represents a balance record.

| Element Name | Element Type | Optional | Description |
| --- | --- | --- | --- |
| BalanceType | xsd:string | No | Identifies the type of balance. End user accounts may have one or more balances for different types of usage (e.g Voice, SMS, gaming etc) |
| Amount | xsd:decimal | No | Amount of balance |

## 7.3	BalanceExpireDetails structure

This data structure represents balance expiry details.

| Element Name | Element Type | Optional | Description |
| --- | --- | --- | --- |
| BalanceType | xsd:string | No | Identifies the type of balance. End user accounts may have one or more balances for different types of usage (e.g Voice, SMS, gaming etc) |
| Date | xsd:dateTime | Yes | It is the date the identified balance will expire. Do not specify if the balance does not expire |

## 7.4	AccountChangedEvent enumeration

List of possible account changed event values.

| Enumeration value | Description |
| --- | --- |
| Charge | Account is charged |
| Recharge | Account is rechared |
| AccountLow | Account balance is below the balance threshold |

# 8	Web Service interface definition

## 8.1	Interface: AccountManagement

The Account Management interface provides access to account information for update and query operations.

### 8.1.1	Operation: GetBalance

This message results in getting account balances indicated by the end user identifier and associated end user PIN. The returned amount for each balance is specified as a currency amount.

End users accounts may have a single balance for all usage, or may have multiple balances for different uses. For example, an end user may have a separate balance for voice calls, SMS messages, and GPRS usage.

#### 8.1.1.1	Input message: GetBalanceRequest

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| EndUserIdentifier | xsd:anyURI | No | This parameter identifies the end user's account. |
| EndUserPin | xsd:string | Yes | Contains the end user's credentials for authorizing access to the account |

#### 8.1.1.2	Output message: GetBalanceResponse

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| result | Balance [1…unbounded] | No | It is a Balance array that consists of types with a string and a decimal field i.e. the balance type and balance amount respectively. |

#### 8.1.1.3	Referenced faults

ServiceException from 3GPP TS 29.199-1 [6]:

- SVC0001 - Service error.

- SVC0002 - Invalid input value.

- SVC0250 - End user authentication failed.

PolicyException from 3GPP TS 29.199-1 [6]:

- POL0001 - Policy error.

### 8.1.2	Operation: GetCreditExpiryDate

This message results in getting the expiration date of the credit indicated by the end user identifier and associated end user PIN.  The returned date is the date the current balance will expire.

#### 8.1.2.1	Input message: GetCreditExpiryDateRequest

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| EndUserIdentifier | xsd:anyURI | No | This parameter identifies the end user's account. |
| EndUserPin | xsd:string | Yes | Contains the end user's credentials for authorizing access to the account. |

#### 8.1.2.2	Output message: GetCreditExpiryDateResponse

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| result | BalanceExpireDetails [1…unbounded] | No | It is a BalanceExpireDetails array that consists of types with a string and a date field. i.e Balance type and date balance will expire respectively |

#### 8.1.2.3	Referenced faults

ServiceException from 3GPP TS 29.199-1 [6]:

- SVC0001 - Service error.

- SVC0002 - Invalid input value.

- SVC0250 - End user authentication failed.

PolicyException from 3GPP TS 29.199-1 [6]:

- POL0001 - Policy error.

### 8.1.3	Operation: BalanceUpdate

This message results in directly recharging the account indicated by the end user identifier and optional associated end user PIN. The reference code is used to uniquely identify the request; it is the application's responsibility to provide a unique reference code within the scope of the application. The charge is specified as a currency amount. The balance is requested to expire in the number of days indicated by the period parameter. The operator's policies may overrule this parameter.  If the optional period parameter is not present, the operator's policy on balance expiration is always in effect.

#### 8.1.3.1	Input message: BalanceUpdateRequest

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| EndUserIdentifier | xsd:anyURI | No | This parameter identifies the end user's account. |
| EndUserPin | xsd:string | Yes | Contains the end user's credentials for authorizing access to the account. |
| ReferenceCode | xsd:string | No | Textual information to uniquely identify the request, e.g. in case of disputes |
| BalanceType | xsd:string | No | Identifies the type of balance. End user accounts may have one or more balances for different types of usage (e.g. Voice, SMS, gaming etc) |
| Amount | xsd:decimal | No | Currency amount that should be added to the end user's account. |
| Period | xsd:int | Yes | The balance is requested to expire in the number of days indicated by this parameter. The operator's policies may overrule this parameter. If this optional parameter is not present, the operator's policy on balance expiration is always in effect. |

#### 8.1.3.2	Output message: BalanceUpdateResponse

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| None |  |  |  |

#### 8.1.3.3	Referenced faults

ServiceException from 3GPP TS 29.199-1 [6]:

- SVC0001 - Service error.

- SVC0002 - Invalid input value.

- SVC0250 - End user authentication failed.

PolicyException from 3GPP TS 29.199-1 [6]:

- POL0001 - Policy error

### 8.1.4	Operation: VoucherUpdate

This message results in directly recharging the account indicated by the end user identifier and optional associated end user PIN. The reference code is used to uniquely identify the request; it is the application's responsibility to provide a unique reference code within the scope of the application. A voucher identifier indirectly specifies the charge. The optional voucher PIN code can be used to verify the voucher.

#### 8.1.4.1	Input message: VoucherUpdateRequest

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| EndUserIdentifier | xsd:anyURI | No | This parameter identifies the end user's account. |
| EndUserPin | xsd:string | Yes | Contains the end user's credentials for authorizing access to the account. |
| ReferenceCode | xsd:string | No | Textual information to uniquely identify the request, e.g. in case of disputes |
| VoucherIdentifier | xsd:string | No | This parameter identifies the voucher. |
| VoucherPin | xsd:string | Yes | Contains the voucher's credentials for authentication. |

#### 8.1.4.2	Output message: VoucherUpdateResponse

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| None |  |  |  |

#### 8.1.4.3	Referenced Faults

ServiceException from 3GPP TS 29.199-1 [6]:

- SVC0001 - Service error.

- SVC0002 - Invalid input value.

- SVC0250 - End user authentication failed.

- SVC0251 - Unknown voucher.

PolicyException from 3GPP TS 29.199-1 [6]:

- POL0001 - Policy error.

- POL0220 - Vouchers not accepted.

### 8.1.5	Operation: GetHistory

This message results in returning the transaction history of the account indicated by the end user identifier and associated optional end user PIN. The maximum number of entries to return and the start date define the range of transactions that are of interest to the requester.

If the total number of entries in the transaction history, starting at the specified date, is larger than the specified maximum number of entries, only the most recent events are returned. Note that the operator might limit the maximum amount of entries to be returned or the period for which the entries are to be returned.

#### 8.1.5.1	Input message: GetHistoryRequest

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| EndUserIdentifier | xsd:anyURI | No | This parameter identifies the end user's account. |
| EndUserPin | xsd:string | Yes | Contains the end user's credentials for authorizing access to the account. |
| Date | xsd:dateTime | Yes | This parameter indicates the desired starting date for the entries to be returned. If this parameter is not present, it is up to the discretion of the service to decide this date. |
| MaxEntries | xsd:int | Yes | This parameter indicates the maximum number of entries that shall be returned. If this parameter is not present, it is up to the discretion of the service to decide how many entries to return. |

#### 8.1.5.2	Output message: GetHistoryResponse

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| result | DatedTransaction [0 .. unbounded] | Yes | It is a DatedTransaction array that consists of types with a date field and a string field: i.e. the date of the occurrence and the transaction details, respectively. |

#### 8.1.5.3	Referenced faults

ServiceException from 3GPP TS 29.199-1 [6]:

- SVC0001 - Service error.

- SVC0002 - Invalid input value.

PolicyException from 3GPP TS 29.199-1 [6]:

- POL0001 - Policy error.

### 8.1.6	Operation: GetBalanceTypes

This message results in getting a set of valid account balance types per account.

#### 8.1.6.1	Input message: GetBalanceTypesRequest

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| EndUserIdentifier | xsd:anyURI | No | This parameter identifies the end user's account. |
| EndUserPin | xsd:string | Yes | Contains the end user's credentials for authorizing access to the account |

#### 8.1.6.2	Output message: GetBalanceTypesResponse

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| result | xsd:string [1…unbounded] | No | Identifies the types of balance. End user accounts may have one or more balances for different types of usage (e.g Voice, SMS, gaming etc) |

#### 8.1.6.3	Referenced faults

ServiceException from 3GPP TS 29.199-1 [6]:

- SVC0001 - Service error.

- SVC0002 - Invalid input value.

- SVC0250 - End user authentication failed.

PolicyException from 3GPP TS 29.199-1 [6]:

- POL0001 - Policy error.

## 8.2	Interface: AccountManagementNotificationManager

Set up notifications for account changes.

### 8.2.1	Operation: startNotification

Notifications of account changes are made available to applications. The correlator provided in the reference must be unique for this Web Service at the time the notification is initiated, otherwise a ServiceException (SVC0005) will be returned to the application. The endUserIdentifier is the end user's account to monitor. The criteria provides the filtering condition to make notifications to the application.

#### 8.2.1.1	Input message: startNotificationRequest

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| reference | common:SimpleReference | No | Notification endpoint definition |
| endUserIdentifier | xsd:anyURI | No | Account to monitor |
| criteria | AccountChangedEvent [0…unbounded] | Yes | List of account changed event values to generate notifications for (these apply to all accounts specified) |

#### 8.2.1.2	Output message: startNotificationResponse

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| None |  |  |  |

#### 8.2.1.3	Referenced faults

ServiceException from 3GPP TS 29.199-1 [6]:

- SVC0001 - Service error.

- SVC0002 - Invalid input value.

- SVC0005- Duplicate correlator.

- SVC0250 - End user authentication failed.

PolicyException from 3GPP TS 29.199-1 [6]:

- POL0001 - Policy error.

### 8.2.2	Operation: endNotification

The application may end a notification using this operation. Until this operation returns, notifications may continue to be received by the application.

#### 8.2.2.1	Input message: endNotificationRequest

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| correlator | xsd:string | No | Correlator of request to end |

#### 8.2.2.2	Output message: endNotificationResponse

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| None |  |  |  |

#### 8.2.2.3	Referenced faults

ServiceException from 3GPP TS 29.199-1 [6]:

- SVC0001 - Service error.

- SVC0002 - Invalid input value.

PolicyException from 3GPP TS 29.199-1 [6]:

- POL0001 - Policy error.

## 8.3	Interface: AccountManagementNotification

Notification interface to which notifications are delivered.

### 8.3.1	Operation: accountCharged

When a monitored account is charged, a notification is delivered to the application with the new account information.

#### 8.3.1.1	Input message: accountChargedRequest

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| Correlator | xsd:string | No | Correlator provided in request to set up this notification |
| Amount | xsd: decimal | No | Charging currency amount |

#### 8.3.1.2	Output message: accountChargedResponse

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| None |  |  |  |

#### 8.3.1.3	Referenced faults

None

### 8.3.2	Operation: accountRecharged

When a monitored account is recharged, a notification is delivered to the application with the new account information.

#### 8.3.2.1	Input message: accountRechargedRequest

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| Correlator | xsd:string | No | Correlator provided in request to set up this notification |
| Amount | xsd: decimal | No | Recharging currency amount |

#### 8.3.2.2	Output message: accountRechargedResponse

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| None |  |  |  |

#### 8.3.2.3	Referenced faults

None

### 8.3.3	Operation: accountLow

When a monitored account is below the balance threshold, a notification is delivered to the application with the new account information.

#### 8.3.3.1	Input message: accountLowRequest

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| Correlator | xsd:string | No | Correlator provided in request to set up this notification |
| Balance | Balance [1…unbounded] | No | It is a set of Balance records, where each record specifies a balance type and the associated amount |

#### 8.3.3.2	Output message: accountLowResponse

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| None |  |  |  |

#### 8.3.3.3	Referenced faults

None

# 9	Fault definitions

## 9.1	Fault: ServiceException

### 9.1.1	End user authentication failed

| Name | Description |
| --- | --- |
| Message Id | <SVC0250> |
| Text | End user authentication failed. |
| Variables | None. |

### 9.1.2	Unknown Voucher

| Name | Description |
| --- | --- |
| Message Id | <SVC0251> |
| Text | Voucher %1 is not valid. |
| Variables | %1 Voucher identifier. |

## 9.2	Fault: PolicyException

### 9.2.1	Vouchers not accepted

| Name | Description |
| --- | --- |
| Message Id | <POL0220> |
| Text | Vouchers not accepted. |
| Variables | None. |

# 10	Service policies

The following service policies are defined for this service.

| Name | Type | Description |
| --- | --- | --- |
| VouchersAccepted | xsd:Boolean | Are vouchers accepted |

Annex A (normative):
WSDL for Account Management

The document/literal WSDL representation of this interface specification is compliant to 3GPP TS 29.199-1 [6] and is contained in text files (contained in archive 29199-07-700-doclit.zip) which accompanies the present document.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Dec 2003 | CN_21 | NP-030552 | -- | -- | Submitted to CN#22 for Information | -- | 1.0.0 | -- |
| Jan 2004 | -- | -- | -- | -- | Added The W3C WSDL representation of the APIs specified in the present document is contained in a set of files which accompany the present document: px0326rpcenc.zip px0326rpclit.zip | -- | 1.0.1 | -- |
| Jun 2004 | CN_24 | NP-040274 | -- | -- | Split into multi-part specification. 29.199-0n, for n=1,2…9. Submitted to CN#24 for Information | -- | 1.0.3 | -- |
| Sep 2004 | CN_25 | NP-040360 | -- | -- | Draft v200 submitted to TSG CN#25 for Approval. | -- | 2.0.0 | 6.0.0 |
| Jun 2005 | CT_28 | CP-050161 | 0001 | -- | Change Return single- to multiple-balances to GetBalance operation | C | 6.0.0 | 6.1.0 |
| Jun 2005 | CT_28 | CP-050221 | 0002 | -- | Optionals for Part 7 | F | 6.0.0 | 6.1.0 |
| Dec 2005 | CT_30 | CP-050572 | 0003 | -- | Make balance expiry date an optional parameter | F | 6.1.0 | 6.2.0 |
| Dec 2005 | CT_30 | CP-050572 | 0004 | -- | Inconsistent part naming in PX response messages | F | 6.1.0 | 6.2.0 |
| Jan 2006 | -- | -- | -- | -- | Changed WSDL attachment: there is an extraneous letter 's' in the BalanceExpireDetails name (it is in the file as BalanceExpiresDetail). | -- | 6.2.0 | 6.2.1 |
| Jun 2006 | CT_32 | CP-060195 | 0005 | -- | Change reference to OSA Stage 2 from 23.127 to 23.198 | F | 6.2.1 | 6.3.0 |
| Jun 2006 | CT_32 | CP-060210 | 0006 | -- | Add Notification Interface in Account Management | B | 6.3.0 | 7.0.0 |