3GPP TR 29.801 V0.2.0 (2006-07)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

Feasibility study of using M2PA in 3GPP networks

(Release 7)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Select keywords from list provided in specs database.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2006, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Analysis of M3UA	6

4.1	Introduction	6

4.2	Status of M3UA	7

4.3	Protocol architecture and function of M3UA	8

4.4	M3UA Adoption in 3GPP	9

4.5	Limitation of M3UA in IP-based SS7 signalling transport network	10

5	Analysis of M2PA	10

5.1	Introduction	10

5.2	Status of M2PA	10

5.3	Protocol architecture and function of M2PA	10

5.4	Limitation of M2PA in IP-based SS7 signalling transport network	12

6	Scenarios of SS7 signalling transport network	12

6.1	Introduction	12

6.2	M3UA+M3UA	12

6.3	M3UA+M2PA	12

6.4	M2PA+M2PA	12

6.5	Difference among these scenarios	12

7	Compare of M2PA and M3UA	13

7.1	Advantage and disadvantage of M2PA	13

7.2	Advantage and disadvantage of M3UA	13

8	Proposed solution	13

8.1	Introduction	13

8.2	Solution 1: introducing M2PA in 3GPP signalling network	13

8.2	Solution 2: improving M3UA	13

9	Conclusion and recommendations	13

Annex <A>: Test result of M2PA and M3UA in labs	14

A.1	Heading levels in an annex	14

Annex <X>: Change history	16

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

In the current 3GPP specifications, M3UA is the only choice when signalling over IP in 3GPP core network, in other words MAP, CAP and BICC can only base on M3UA/SCTP/IP for the IP based signalling network.

Now, IETF has changed M2PA draft into RFC 4165, which means the accomplishment of M2PA protocol definition. There may be possible improvements over the existing signalling protocol by using M2PA in 3GPP core networks.

This clause is optional. If it exists, it is always the second unnumbered clause.

Editor's Note:	this section to be completed as and when content is added. It may be removed later, if there is no content proposed for it.

# Scope

The present document provides a study into the advantage and disadvantage of M2PA comparing with M3UA and provides a study into the possibility of introducing M2PA in 3GPP.

The present document also specifies the functionalities required on the protocol level to meet the requirements of BICC, MAP and CAP.

If necessary, the interactive between M2PA and M3UA should be studied.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 29.202: "SS7 Signalling Transport in Core Network, Stage 3".

[2]	RFC 3332:Signaling System 7 (SS7) Message Transfer Part 3 (MTP3) - User Adaptation Layer (M3UA)"

[3]	RFC 4165:Signaling System 7 (SS7) Message Transfer Part 2 (MTP2) - User Peer-to-Peer Adaptation Layer (M2PA), September 2005

[4]	3GPP TR 21.905: "3G Vocabulary"

[5]	ITU-T Recommendation Q.701: "Functional description of the message transfer part (MTP) of signalling system No. 7"

[6]	ITU-T Recommendation Q.702: "Signalling data link"

[7]	ITU-T Recommendation Q.703: "Signalling link"

[8]	ITU-T Recommendation Q.704: "Signalling network functions and messages"

[10]	ITU-T Recommendation Q.705: "Signalling network structure"

[11]	3GPP TR 29.903: "Feasibility Study on SS7 signalling transport in the core network with SCCP-User Adaptation (SUA) layerS7".

[12]	3GPP TS 29.205: "Application of Q.1900 series to bearer-independent Circuit Switched (CS) core network architecture; Stage 3"

[13]	3GPP TS 29.232: "Media Gateway Controller (MGC) - Media Gateway (MGW) interface; Stage 3"

[14]	3GPP TS 25.412: "UTRAN Iu interface signalling transport"

[15]	3GPP TS 25.422: "UTRAN Iur interface signalling transport"

It is preferred that the reference to 21.905 be the first in the list.

# 3	Definitions, symbols and abbreviations

Delete from the above heading those words which are not applicable.

Subclause numbering depends on applicability and should be renumbered accordingly.

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [4] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [4].

Definition format

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

Editor's Note:	this section to be completed as and when content is added. Clause numbers are subject to change until such time as the document is approved at version 7.0.0

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format

<symbol>	<Explanation>

Editor's Note:	this section to be completed as and when content is added. Clause numbers are subject to change until such time as the document is approved at version 7.0.0

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [4] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [4].

IP	Internet Protocol

MTP	Message Transfer Part

MTP1	Message Transfer Part layer 1

MTP2	Message Transfer Part layer 2

MTP3	Message Transfer Part layer 3

M3UA	MTP3-User Adaptation

M2PA	MTP2 - User Peer-to-Peer Adaptation

SCTP	Stream Control Transmission Protocol

SG	Signalling Gateway

SP	Signalling Point

STP	Signalling Transfer Point

# 4	Analysis of M3UA

## 4.1	Introduction

This clause provides a study into the current use of M3UA. It discusses the limitation of M3UA.

## 4.2	Status of M3UA

MTP3-User Adaptation (M3UA) is a protocol, currently developed by IETF, for the transport of any SS7 MTP3-User signalling (e.g. ISUP, SCCP and TUP) over IP using the Stream Control Transport Protocol (SCTP). M3UA can also work in diverse architectures, such as a Signalling Gateway to IP Signalling Endpoint architecture as well as a peer-to-peer IP Signalling Endpoint architecture [2], [11].

M3UA provides the service of MTP3 between SG and application server.

M3UA supports MTP3 users ISUP and SCCP.

M3UA replaces ss7 signalling link, signalling link set, combined link set and signalling routes.

M3UA is designed to allow legacy SS7 TDM nodes to communicate with SIGTRAN IP-capable nodes.

- Consists of Signalling Gateway Process (SGP) and Application Process (ASP) side;

- Supports a higher bandwidth like M2PA;

- Recreates limited MTP3 capabilities for network management and congestion;

- Carries enough MTP3 information to permit routing of received message to other nodes.

3GPP TS 29.202 mandates  in 3GPP networks to use M3UA as IP-based SS7 signalling transport link at the A and the C interface.  For the B interface, it mandates the use of Q.701-Q.705 [5],[6],[7],[8],[9] or Q.2210.  Figure 2 shows the use of  M3UA in 3GPP signalling network. The signalling gateways in this picture  are pure MTP3/3B-M3UA signalling gateways. They do not include any M3UA users.  Still there could be a node including an M3UA user (eg SCCP functions) and a M3UA signalling gateway functions.  In that case the node will support all the interfaces A, B and C.

Figure 4-1: Use of M3UA in 3GPP core network

## 4.3	Protocol architecture and function of M3UA

The M3UA delivery mechanism provides the following functionality:

- Support for transfer of SS7 MTP3-User Part messages;

- Support for the management of SCTP transport protocol between a Signalling Gateway and one or more IP-based signalling nodes to ensure transport availability to MTP3 user signalling applications;

- Support for the seamless operation of MTP3-User protocol peers;

- Support for distributed IP-based signalling nodes; and

- Support for the asynchronous reporting of status changes to management.

M3UA provides function like Signalling Point Code Representation, Routing Contexts and Routing Keys, SS7 and M3UA Interworking , Application Server Redundancy, Flow Control, Congestion Management, SCTP Stream Mapping, Client/Server Model to initial the SCTP association.

Signalling Point Code Representation: Within an SS7 network, a Signalling Gateway might be charged with representing a set of nodes in the IP domain into the SS7 network for routing purposes. The SG itself acts as a signalling point in the SS7 network.

Routing Contexts and Routing Keys: A Routing Key describes a set of SS7 parameters and parameter values that uniquely define the range of signalling traffic to be handled by a particular Application Server. Routing Context is a value that uniquely identifies a Routing Key.

SS7 and M3UA Interworking: For SS7 and M3UA interworking, the M3UA adaptation layer is designed to provide an extension of the MTP3 defined user primitives. The SGP provides a functional interworking of transport functions between the SS7 network and the IP network by also supporting the M3UA adaptation layer.

Application Server Redundancy: The Application Server is the set of all ASPs associated with a specific Routing Key. Each ASP in this set may be active, inactive or unavailable.  Active ASPs handle traffic; inactive ASPs might be used   when active ASPs become unavailable. The failover model supports an "n+k" redundancy model, where "n" ASPs is the minimum number of redundant ASPs required to handle traffic and "k" ASPs are available to take over for a failed or unavailable ASP.

Flow Control: Local Management at an ASP may wish to stop traffic across an SCTP association to temporarily remove the association from service or to perform testing and maintenance activity.  The function could optionally be used to control the start of traffic on to a newly available SCTP association.

Congestion Management: The M3UA layer is informed of local and IP network congestion by means of an implementation-dependent function. M3UA may resolve the congestion (by means of  indicating congestion to local MTP3-Users or triggering SS7 MTP3 Transfer Controlled management messages to originating SS7 nodes or indicate local congestion to an   M3UA peer with an SCON message etc. ).

SCTP Stream Mapping:   The M3UA layer at both the SGP and ASP also supports the assignment of signalling traffic into streams within an SCTP association. Traffic that requires sequencing SHOULD be assigned to the same stream.

Client/Server Model:  SCTP association need to be initiated, client should always peer which take on the role of initiating the SCTP association to the server.

The usage of M3UA to transport MAP and TCAP messages is illustrated in Figure 4-2.

Figure 4-2: Transportation of MAP and CAP via M3UA

An example of SCCP transport between IPSPs is illustrated in Figure 4-3. SCCP messages are exchanged directly between two IP resident IPSPs with SCCP user protocol like TCAP, RANAP and RNSAP.

Figure 4-3: Transportation of SCCP-user messages via M3UA in all IP network

## 4.4	M3UA Adoption in 3GPP

SCCP/M3UA is the only specified protocol for IP based transportation of 3GPP SS7 like core network signalling. The transport of  MAP & CAP messages in a 3GPP core network using M3UA is specified  in 3GPP  TS 29.202[1]. The usage of  M3UA in Mc interface between MSC and MGW  is defined in 3GPP TS 29.205 [12] and 3GPP TS 29.232 [13].  Meanwhile, M3UA is specified as an option for RANAP in 3GPP TS 25.412 [14] and RNSAP in 3GPP TS 25.422 [15] to transport SCCP messages in a packet switched domain.

## 4.5	Limitation of M3UA in IP-based SS7 signalling transport network

Editor's Note:	this section is to hold discussion on the drawbacks of the current use of M3UA in 3GPP networks. The limitation possibly lies in link management functions and stabilization of signalling networks. The limitations are controversial at this point so this section is intended to be deleted and covered in the sections on advantages and disadvantages.

# 5	Analysis of M2PA

## 5.1	Introduction

This clause provides an introduction and study into the M2PA.

M2PA provides MTP3 with an interface and services similar to MTP2; in fact MTP2 and lower layers of the legacy SS7 protocol stack are replaced by an IP equivalent.

## 5.2	Status of M2PA

In November 2000 the first draft of M2PA was available where  September 2005, M2PA became an RFC (RFC 4165) [3]. M2PA is going to be used in many signalling networks.

M2PA provides the service of MTP2 between two SG (peer to peer).

M2PA supports MTP3.

M2PA replaces ss7 signalling link and the core of the SS7 network by a SS7-over-IP-network.

The objective was here to take advantage of the high bandwidth an IP network can provide.

M2PA can be used as an adaptation protocol with the following characteristics:

- M2PA is a symmetric Peer-to-Peer protocol;

- M2PA recreates the qualities of MTP2 (e.g. Error Correction, Congestion Procedures);

- M2PA signalling point codes contain a MTP3 layer;

- All MTP3 network management procedures are available;

- Possible for connections to end points.

## 5.3	Protocol architecture and function of M2PA

- The protocol architecture applicable in the case of IP-based SS7 signalling transport network is shown in Figure 5-1

MTP3

M2PA

SCTP

IP

Figure 5-1: Protocol architecture in the case of IP-based SS7 signalling transport network

MTP2 Functionality

M2PA provides MTP2 functionality that is not provided by SCTP, thus, together M2PA and SCTP provide functionality similar to that of MTP2.

M2PA functionality includes:

- Data retrieval to support the MTP3 changeover procedure;

- Reporting of link status changes to MTP3;

- Processor outage procedure;

- Link alignment procedure.

Procedures to Support MTP2 Features

- SCTP provides reliable, in-sequence delivery of user messages. Therefore the related functionality of MTP2 is not needed.  SCTP does not provide functions related to Link State Control in MTP2.  These functions must be provided by M2PA.

- Since SCTP provides delivery of messages, there is no need for M2PA to delimit its messages with a flag, as is done in MTP2. Furthermore, M2PA does not need to perform zero bit insertion and deletion on its messages.

- Since SCTP uses a checksum to detect transmission errors, there is no need for an M2PA checksum, as is needed in MTP2.  This also eliminates the need for the error rate monitors of MTP2.

- Since SCTP provides reliable delivery and ordered delivery, M2PA does not perform retransmissions.  This eliminates the need for the forward and backward indicator bits in MTP2 signal units.

- Acceptance of a message is indicated by a successful receipt of the message from SCTP.

Mapping of SS7 and IP Entities

- Each MTP link corresponds to an SCTP association. To prevent duplicate associations from being established, it is RECOMMENDED that each endpoint know the IP address (or IP addresses, if multi-homing is used) and port number of both endpoints.  SCTP prevents two associations with the same IP addresses and port numbers from being established.

- It is necessary for at least one of the endpoints to be listening on the port on which the other endpoint is trying to establish the association. Therefore, at least one of the port numbers SHOULD be the M2PA registered port.

- If only one association is to be established between these two IP addresses, then the association SHOULD be established using the M2PA registered port at each endpoint.

- If it is desirable to create multiple associations (for multiple links) between the two IP addresses, different port numbers can be used for each association. Nevertheless, the M2PA registered port number SHOULD be used at one end of each association.

- Each combination of IP address/port for the two endpoints (i.e., each association) MUST be mapped to the same Signalling Link Code (SLC) at each endpoint, so that each endpoint knows which link is being created at the time the SCTP association is established.  However, M2PA does not do any processing based on the SLC.

Note:	A link is an SCTP association identified by two endpoints. Each endpoint is identified by an IP address and port number. Each association is mapped to an SLC.

SCTP Association Management

SCTP allows a user-specified number of streams to be opened during the initialization.  It is the responsibility of the M2PA layer to ensure proper management of the streams allowed within each association.

M2PA uses two streams in each direction for each association.  Stream 0 in each direction is designated for Link Status messages.  Stream 1 is designated for User Data messages, as well as Link Status messages that must remain in sequence with the User Data messages. Separating the Link Status and User Data messages into separate streams allows M2PA to prioritize the messages in a manner similar to MTP2.

Retention of MTP3 in the SS7 Network

M2PA allows MTP3 to perform all of its Message Handling and Network Management functions with IPSPs as it does with other SS7 nodes.

SCTP Association Problems

The SCTP association for a link may become unusable, such as when one of the following occurs:

- SCTP sends a Send Failure notification to M2PA.

- SCTP sends a Communication Lost notification to M2PA.

- SCTP sends a Communication Error notification to M2PA.

- The SCTP association is lost. If the SCTP association for a link becomes unable to transmit or receive messages, M2PA SHALL report to MTP3 that the link is out of service and enter the OUT OF SERVICE state.

## 5.4	Limitation of M2PA in IP-based SS7 signalling transport network

Editor's Note:	this section is to hold discussion on the drawbacks of using M2PA in signalling networks. As the limitations are controversial at this point they are dealt with under the advantages and disadvantages section.

# 6	Scenarios of SS7 signalling transport network

## 6.1	Introduction

This clause discusses possible protocol scenarios of signalling networks with STP. Then there will be a compare of there scenarios and a conclusion. There will be three possible scenarios.

## 6.2	M3UA+M3UA

Editor's Note:	this section to discuss the scenario of using M3UA between SP and STP, STP and STP.

## 6.3	M3UA+M2PA

Editor's Note:	this section to discuss the scenario of using M3UA between SP and STP, using M2PA between STP and STP.

## 6.4	M2PA+M2PA

Editor's Note:	this section to discuss the scenario of using M2PA between SP and STP, STP and STP.

## 6.5	Difference among these scenarios

Editor's Note:	this section to compare the scenarios listed above. Possible difference lie in success rate when changing over, lost rate at high load, performance of STP, etc.

# 7	Compare of M2PA and M3UA

## 7.1	Advantage and disadvantage of M2PA

Editor's Note:	this section is to compare M2PA against M3UA. Possible aspects may be protocol architecture and function itself, network maintain, support for change over when link failure, interactive with signalling based on TDM, etc. this compare is to see if M2PA can fulfil the 3GPP requirement.

## 7.2	Advantage and disadvantage of M3UA

Editor's Note:	this section is to compare M3UA against M2PA. Possible aspects may be protocol architecture and function itself, network maintain, support for change over when link failure, interactive with signalling based on TDM, etc. this compare is to see if M2PA can fulfil the 3GPP requirement.

# 8	Proposed solution

## 8.1	Introduction

This clause highlights the requirement that bring us to the feasibility study, discusses the solution proposed. There are two possible solutions, one is to introduce M2PA in 3GPP signalling network and the other one is to improve the functionality of M3UA. For each solution, signalling architecture, detailed technical solution, and any enhancement it brings shall be discussed.

## 8.2	Solution 1: introducing M2PA in 3GPP signalling network

Editor's Note:	this section is to hold proposed solution when introducing M2PA. It will discuss signalling network architecture with M2PA, interworking between M2PA and M3PA, interworking between IP based signalling network with M2PA and TDM signalling network, impact the solution brings. Subclause is subject to be added.

## 8.2	Solution 2: improving M3UA

Editor's Note:	this section is to hold proposed solution when improving M3UA. It will discuss how to improve the M3UA based on the current functionality and the requirement, and the impact it brings. Subclause is subject to be added.

# 9	Conclusion and recommendations

Editor's Note:	this section is to hold the conclusion of the study and provide recommendations on the way forward.

Annexes are only to be used where appropriate:

Annex <A>:
Test result of M2PA and M3UA in labs

Annexes are labeled A, B, C, etc. and are "informative"(3G TRs are informative documents by nature).

Editor's Note:	this section is to state some result and conclusion from the test in labs. The test result will help in compare between M2PA and M3UA.

# A.1	Heading levels in an annex

Heading levels within an annex are used as in the main document, but for Heading level selection, the "A.", "B.", etc. are ignored. e.g. A.1.2 is formatted using Heading 2 style.

Bibliography

The Bibliography is optional. If it exists, it shall follow the last annex in the document.

The following material, though not specifically referenced in the body of the present document (or not publicly available), gives supporting information.

Bibliography format

-	<Publication>: "<Title>".

OR

<Publication>: "<Title>".

Annex <X>:
Change history

It is usual to include an annex (usually the final annex of the document) for reports under TSG change control which details the change history of the report using a table as follows:

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2006-05 | CT4#31 | C4-060849 |  |  | The first draft publish for future work | 0.0.0 | 0.1.0 |
| 2006-07 | CT4#31bis | C4-060970 |  |  | The second draft publish for future work | 0.1.0 | 0.2.0 |

Editor's Note:	this section to be completed as and when content is added. Annex letter is subject to change!