3GPP TS 31.103 V7.0.0 (2006-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

Characteristics of the IP Multimedia Services Identity Module (ISIM) application

(Release 7)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

UMTS, SIM, Card, IP, Multimedia

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2006, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions, symbols, abbreviations and coding conventions	7

3.1	Definitions	7

3.2	Symbols	7

3.3	Abbreviations	8

3.4	Coding Conventions	9

4	Files	9

4.1	Contents of the EFs at the MF level	9

4.2	Contents of files at the ISIM ADF (Application DF) level	9

4.2.1	Void	9

4.2.2	EFIMPI (IMS private user identity)	9

4.2.3	EFDOMAIN (Home Network Domain Name)	10

4.2.4	EFIMPU (IMS public user identity)	10

4.2.5	EFAD (Administrative Data)	11

4.2.6	EFARR (Access Rule Reference)	11

4.2.7	EFIST (ISIM Service Table)	12

4.2.8	EFP-CSCF (P-CSCF Address)	13

4.2.9	EFGBABP (GBA Bootstrapping parameters)	14

4.2.10	EFGBANL (GBA NAF List)	15

4.3	ISIM file structure	16

5	Application protocol	16

5.1	ISIM management procedures	17

5.1.1	Initialisation	17

5.1.1.1	ISIM application selection	17

5.1.1.2	ISIM initialisation	17

5.1.2	ISIM Session termination	17

5.1.3	ISIM application closure	18

5.1.4	UICC presence detection	18

5.1.5	Administrative information request	18

5.2	ISIM security related procedures	18

5.2.1	Authentication procedure	18

5.2.2	IMPI request	18

5.2.3	IMPU request	18

5.2.4	SIP Domain request	18

5.2.5	Void	18

5.2.6	ISIM Service Table request	18

5.2.7	P-CSCF address request	18

5.2.8	Generic Bootstrapping architecture (Bootstrap)	19

5.2.9	Generic Bootstrapping architecture (NAF Derivation)	19

6	Security features	19

6.1	User verification and file access conditions	19

7	ISIM Commands	19

7.1	AUTHENTICATE	19

7.1.1	Command description	19

7.1.1.1	IMS AKA security context	20

7.1.1.2	GBA security context (Bootstrapping Mode)	20

7.1.1.3	GBA security context (NAF Derivation Mode)	21

7.1.2	Command parameters and data	22

7.1.2.1	IMS AKA security context	22

7.1.2.2	HTTP Digest security context	23

7.1.2.3	 GBA security context (Bootstrapping Mode)	23

7.1.2.4	 GBA security context (NAF Derivation Mode)	24

7.1.3	Status Conditions Returned by the ISIM	24

7.1.3.1	Security management	24

7.1.3.2	Status Words of the Commands	25

7.2	GET CHALLENGE	25

8	void	25

Annex A (informative):	EF changes via Data Download or CAT applications	26

Annex B (informative):	Tags defined in 31.103	27

Annex C (informative):	Suggested contents of the EFs at pre-personalization	28

Annex D (informative):	List of SFI Values	29

D.1	List of SFI Values at the ISIM ADF Level	29

Annex E (informative):	ISIM Application Session Activation / Termination	30

Annex F (informative):	Change History	31

# Foreword

This Technical Specification (TS) has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document defines the IM Services Identity Module (ISIM) application. This application resides on the UICC, an IC card specified in TS 31.101 [3]. In particular, TS 31.101 [3] specifies the application independent properties of the UICC/terminal interface such as the physical characteristics and the logical structure.

TS 31.101 [3] is one of the core documents for this specification and is therefore referenced in many places in the present document.

# 1	Scope

The present document defines the ISIM application for access to IMS services.

The present document specifies:

-	specific command parameters;

-	file structures;

-	contents of EFs (Elementary Files);

-	security functions;

-	application protocol to be used on the interface between UICC (ISIM) and Terminal.

This is to ensure interoperability between an ISIM and Terminal independently of the respective manufacturer, card issuer or operator.

The present document does not define any aspects related to the administrative management phase of the ISIM. Any internal technical realisation of either the ISIM or the Terminal is only specified where these are reflected over the interface. The present document does not specify any of the security algorithms that may be used.

# 2	References

The following documents contain provisions that, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication and/or edition number or version number) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 21.111: "USIM and IC Card Requirements".

[2]	3GPP TS 31.102: "Characteristics of the USIM Application".

[3]	3GPP TS 31.101: "UICC-Terminal Interface, Physical and Logical Characteristics".

[4]	3GPP TS 33.102: "3G Security; Security Architecture".

[5]	3GPP TS 33.103: "3G Security; Integration Guidelines".

[6]	ISO/IEC 7816-4: "Identification cards - Integrated circuit cards,Part 4: Organization, security and  commands for interchange".

[7]	Void

[8]	void

[9]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[10]	Void

[11]	Void

[12]	3GPP TS 25.101: "UE Radio Transmission and Reception (FDD)".

[13]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[14]	3GPP TS 33.203: "3G security; Access security for IP-based services".

[15]	3GPP TS 24.228: "Signalling flows for the IP multimedia call control based on SIP and SDP; Stage 3".

[16]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[17]	3GPP TS 23.038: "Alphabets and language-specific information".

[18]	ISO 639 (1988): "Code for the representation of names of languages".

[19]	3GPP TS 51.011 Release 4: "Specification of the Subscriber Identity Module - Mobile Equipment (SIM-ME) interface".

[20]	ISO/IEC 8825(1990): "Information technology - Open Systems Interconnection - Specification of Basic Encoding Rules for Abstract Syntax Notation One (ASN.1)" Second Edition.

[21]	3GPP TS 22.101: "Service aspects; Service principles".

[22]	ETSI TS 102 223 Release 6: "Smart cards; Card Application Toolkit (CAT)".

[23]	ETSI TS 101 220: "Smart cards; ETSI numbering system for telecommunication application providers".

[24]	IETF RFC 2486: "The Network Access Identifier"

[25]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic bootstrapping architecture"

[26]	IETF RFC 2617: "HTTP Authentication: Basic and Digest Access Authentication". (http://www.ietf.org/rfc/rfc2617.txt)

[27]	IETF RFC 3629 (2003): "UTF-8, a transformation format of ISO 10646".

# 3	Definitions, symbols, abbreviations and coding conventions

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply:

ISIM: application residing on the UICC, an IC card specified in 3GPP TS 31.101 [3]
In particular, 3GPP TS 31.101 [3] specifies the application independent properties of the UICC/terminal interface such as the physical characteristics and the logical structure

The AID of ISIM is defined in ETSI TS 101 220 [23] and is stored in EFDIR.

ADM: access condition to an EF which is under the control of the authority which creates this file

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

||	Concatenation

Å	Exclusive or

f1	Message authentication function used to compute MAC

f1*	A message authentication code (MAC) function with the property that no valuable information can be inferred from the function values of f1* about those of f1, ... , f5 and vice versa

f2	Message authentication function used to compute RES and XRES

f3	Key generating function used to compute CK

f4	Key generating function used to compute IK

f5	Key generating function used to compute AK

## 3.3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

3GPP	3rd Generation Partnership Project

AC	Access Condition

ADF	Application Dedicated File

AID	Application IDentifier

AK	Anonymity Key

AKA	Authentication and Key Agreement

ALW	ALWays

AMF	Authentication Management Field

ASN.1	Abstract Syntax Notation One

AuC	Authentication Centre

AUTN	AUthentication TokeN

BER-TLV	Basic Encoding Rule - TLV

B-TID	Bootstrapping Transaction IDentifier

CK	Cipher Key

DF	Dedicated File

EF	Elementary File

FFS	For Further Study

HE	Home Environment

HN	Home Network

ICC	Integrated Circuit Card

ID	IDentifier

IK	Integrity Key

IM	IP Multimedia

IMPI	IM Private Identity

IMPU	IM PUblic identity

IMS	IP Multimedia Subsystem

ISIM	IM Services Identity Module

K	long-term secret Key shared between the ISIM and the AuC

KSI	Key Set Identifier

LI	Language Indication

LSB	Least Significant Bit

MAC	Message Authentication Code

MF	Master File

MSB	Most Significant Bit

NAI	Network Access Identifier

NEV	NEVer

PIN	Personal Identification Number

PL	Preferred Languages

PS_DO	PIN Status Data Object

RAND	RANDom challenge

RES	user RESponse

RFU	Reserved for Future Use

RST	ReSeT

SDP	Session Description Protocol

SFI	Short EF Identifier

SIP	Session Initiation Protocol

SQN	SeQuence Number

SW	Status Word

TLV	Tag Length Value

UE	User Equipment

XRES	eXpected user RESponse

## 3.4	Coding Conventions

- The following coding conventions apply to the present document.

- All lengths are presented in bytes, unless otherwise stated. Each byte is represented by bits b8 to b1, where b8 is the most significant bit (MSB) and b1 is the least significant bit (LSB). In each representation, the leftmost bit is the MSB.

- The coding of Data Objects in the present document is according to TS 31.101 [3].

'XX':	Single quotes indicate hexadecimal values. Valid elements for hexadecimal values are the numbers '0' to '9' and 'A' to 'F'.

# 4	Files

This clause specifies the EFs for the IMS session defining access conditions, data items and coding. A data item is a part of an EF which represents a complete logical entity.

For an overview containing all files see figure 4.1.

## 4.1	Contents of the EFs at the MF level

There are four EFs at the Master File (MF) level. These EFs are specified in 3GPP TS 31.101 [3].

## 4.2	Contents of files at the ISIM ADF (Application DF) level

The EFs in the ISIM ADF contain service and network related information and are required for UE to operate in an IP Multimedia Subsystem.

The File IDs '6F1X' (for EFs), '5F1X' and '5F2X' (for DFs) with X ranging from '0' to 'F' are reserved under the ISIM ADF for administrative use by the card issuer.

### 4.2.1	Void

### 4.2.2	EFIMPI (IMS private user identity)

This EF contains the private user identity of the user.

| Identifier: '6F02' |  | Structure: transparent |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: ‘02’ |  |  |  |  |  |  |
| File size: X bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to X | NAI TLV data object |  |  | M |  | X bytes |

-	NAI

Contents:

-	Private user identity of the user.

Coding:

-	For contents and syntax of NAI TLV data object values see IETF RFC 2486 [24]. The NAI shall be encoded to an octet string according to UTF-8 encoding rules as specified in IETF RFC 3629 [27]. The tag value of the NAI TLV data object shall be '80'.

### 4.2.3	EFDOMAIN (Home Network Domain Name)

This EF contains the home operator’s network domain name SIP URI.

| Identifier: '6F03' |  | Structure: transparent |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: ‘05’ |  |  |  |  |  |  |
| File size: X bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to X | URI TLV data object |  |  | M |  | X bytes |

-	URI

Contents:

-	Home Network Domain Name SIP URI.

Coding:

-	For contents and syntax of URI TLV data object values see IETF RFC 3261 [16]. The URI shall be encoded to an octet string according to UTF-8 encoding rules as specified in IETF RFC 3629 [27]. The tag value of the URI TLV data object shall be '80'.

### 4.2.4	EFIMPU (IMS public user identity)

This EF contains one or more records, with each record able to hold a public SIP Identity (SIP URI) of the user. The first (or only) record in the EF shall be used as the default SIP Identity Identity in case that no record is explicitly selected either in the current session or as a carryover from a prior session.

| Identifier: '6F04' |  | Structure: linear fixed |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: ‘04’ |  |  |  |  |  |  |
| Record length: X bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to X | URI TLV data object |  |  | M |  | X bytes |

-	URI

Contents:

-	SIP URI by which other parties know the subscriber.

Coding:

-	For contents and syntax of URI TLV data object values see IETF RFC 3261 [16]. The URI shall be encoded to an octet string according to UTF-8 encoding rules as specified in IETF RFC 3629 [27]. The tag value of the URI TLV data object shall be '80'.

### 4.2.5	EFAD (Administrative Data)

This EF contains information concerning the mode of operation according to the type of ISIM, such as normal (to be used by IMS subscribers for IMS operations), type approval (to allow specific use of the Terminal during type approval procedures of e.g. the network equipment), manufacturer specific (to allow the Terminal manufacturer to perform specific proprietary auto-test in its Terminal during e.g. maintenance phases).

It also provides an indication of whether some Terminal features should be activated during normal operation.

| Identifier: '6FAD' |  | Structure: transparent |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: '03' |  |  |  |  |  |  |
| File size: 3+X bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALW UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | UE operation mode |  |  | M |  | 1 byte |
| 2 to 3 | Additional information |  |  | M |  | 2 bytes |
| 4 to 3+X | RFU |  |  | O |  | X bytes |

-	UE operation mode:

Contents:

-	mode of operation for the UE

Coding:

-	Initial value

-	'00' normal operation.

-	'80' type approval operations.

-	'01' normal operation + specific facilities.

-	'81' type approval operations + specific facilities.

-	'02' maintenance (off line).

-	Additional information:

Coding:

-	specific facilities (if b1=1 in byte 1);

Bytes 2 and 3 (first byte of additional information):

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101) |

### 4.2.6	EFARR (Access Rule Reference)

This EF contains the access rules for files located under the ISIM ADF in the UICC. If the security attribute tag '8B' is indicated in the FCP it contains a reference to a record in this file.

- Structure of EFARR at ADF-level

| Identifier: '6F06' |  | Structure: Linear fixed |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: '06' |  |  |  |  |  |  |
| Record Length: X bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALW UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to X | Access Rule TLV data objects |  |  | M |  | X bytes |

This EF contains one or more records containing access rule information according to the reference to expanded format as defined in ISO/IEC 7816-4 [6]. Each record represents an access rule. Unused bytes in the record are set to 'FF'.

If the card cannot access EFARR , any attempt to access a file with access rules indicated in this EFARR shall not be granted.

### 4.2.7	EFIST (ISIM Service Table)

This EF indicates which optional services are available. If a service is not indicated as available in the ISIM, the ME shall not select this service. The presence of this file is mandatory if optional services are provided in the ISIM.

| Identifier: '6F07' |  | Structure: transparent |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: '07' |  |  |  |  |  |  |
| File size: X bytes, X >= 1 |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | Services n°1 to n°8 |  |  | M |  | 1 byte |
| 2 | Services n°9 to n°16 |  |  | O |  | 1 byte |
| 3 | Services n°17 to n°24 |  |  | O |  | 1 byte |
| 4 | Services n°25 to n°32 |  |  | O |  | 1 byte |
| etc. |  |  |  |  |  |  |
| X | Services n°(8X-7) to n°(8X) |  |  | O |  | 1 byte |

| -Services |  |  |
| --- | --- | --- |
| Contents: | Service n°1: | P-CSCF address |
|  | Service n°2 | Generic Bootstrapping Architecture (GBA) |

The EF shall contain at least one byte. Further bytes may be included, but if the EF includes an optional byte, then it is mandatory for the EF to also contain all bytes before that byte. Other services are possible in the future and will be coded on further bytes in the EF. The coding falls under the responsibility of the 3GPP.

Coding:

1 bit is used to code each service:
bit = 1: service available;
bit = 0: service not available.

-	Service available means that the ISIM has the capability to support the service and that the service is available for the user of the USIM.
Service not available means that the service shall not be used by the ISIM user, even if the ISIM has the capability to support the service.

First byte:

|  |  | b8 |  | b7 |  | b6 |  | B5 |  | b4 |  | b3 |  | b2 |  | b1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°1 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°2 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°3 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°4 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°5 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°6 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°7 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°8 |

Second byte:

|  |  | b8 |  | b7 |  | b6 |  | B5 |  | b4 |  | b3 |  | b2 |  | b1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°9 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°10 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°11 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°12 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°13 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°14 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°15 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°16 |

etc.

### 4.2.8	EFP-CSCF (P-CSCF Address)

If service n°1 is "available", this file shall be present.

This EF does not apply for 3GPP and shall not be used by a terminal using a 3GPP access network or a 3GPP Interworking WLAN.

NOTE:	The current 3GPP procedures for P-CSCF discovery provide a flexible way for the UE to discover the P-CSCF address(es). Procedures include both GPRS PDP context based solution and a generic DHCP based approach that can be used for other access technologies.

This EF contains one or more Proxy Call Session Control Function addresses. The first record in the EF shall be considered to be of the highest priority.  The last record in the EF shall be considered to be the lowest priority.

| Identifier: '6F09' |  | Structure: linear fixed |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| Record length: X bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to X | P-CSCF Address TLV data object |  |  | M |  | X bytes |

P-CSCF

Contents:

- 	Address of Proxy Call Session Control Function, in the format of a FQDN, an IPv4 address, or an IPv6 address.

Coding:

- 	The tag value of this P-CSCF address TLV data object shall be '80'. The format of the data object is as follows:

| Field | Length (bytes) |
| --- | --- |
| Tag | 1 |
| Length | 1 |
| Address Type | 1 |
| P-CSCF Address | Address Length |

Address Type: 	Type of the P-CSCF address.

This field shall be set to the type of the P-CSCF address according to the following:

| Value | Name |
| --- | --- |
| 0x00 | FQDN |
| 0x01 | IPv4 |
| 0x02 | IPv6 |
| All other values are reserved |  |

P-CSCF Address: 	Address of the Proxy Call Session Control Function

This field shall be set to the address of the Proxy Call Session Control Function. When the P-SCSF type is set to 0x00, the corresponding P-CSCF Address shall be encoded to an octet string according to UTF-8 encoding rules as specified in IETF RFC 3629 [27].

Unused bytes shall be set to 'FF'.

### 4.2.9	EFGBABP (GBA Bootstrapping parameters)

If service n°2 is "available", this file shall be present.

This EF contains the AKA Random challenge (RAND) and Bootstrapping Transaction Identifier (B-TID) associated with a GBA bootstrapping procedure.

| Identifier: '6FD5' |  | Structure: transparent |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| File length: L+X+N+3 bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE PIN DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | Length of RAND (16) |  |  | M |  | 1 byte |
| 2 to (X+1) | RAND |  |  | M |  | X bytes |
| X+2 | Length of B-TID (L) |  |  | M |  | 1 byte |
| (X+3) to (X+2+L) | B-TID |  |  | M |  | L bytes |
| X+L+3 | Length of key lifetime |  |  | M |  | 1 byte |
| (X+L+4) to (X+L+N+3) | Key lifetime |  |  | M |  | N bytes |

-	Length of RAND

Contents: number of bytes, not including this length byte, of RAND field

-	RAND

Contents: Random challenge used in the GBA_U bootstrapping procedure.

Coding: as defined in 33.103 [13]

-	Length of B-TID

Contents: number of bytes, not including this length byte, of B-TID field

- B-TID
Content: Bootstrapping Transaction Identifier the GBA_U bootstrapped keys

Coding: As defined in TS 33.220 [25]

-	Length of key lifetime

Contents: number of bytes, not including this length byte, of key lifetime field

- Key lifetime
Content: Lifetime of the GBA_U bootstrapped keys

Coding: As defined in TS 33.220 [25]

### 4.2.10	EFGBANL (GBA NAF List)

If service n°2 is "available", this file shall be present.

This EF contains the list of NAF_ID and B-TID associated to a GBA NAF derivation procedure.

| Identifier: '6FD7' |  | Structure: Linear fixed |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| Record length: Z bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to Z | NAF Key Identifier TLV objects |  |  | M |  | Z bytes |

NAF Key Identifier tags

| Description | Tag Value |
| --- | --- |
| NAF_ID Tag | '80' |
| B-TID Tag | '81' |

NAF Key Identifier information

| Description | Value | M/O | Length (bytes) |
| --- | --- | --- | --- |
| NAF_ID Tag | '80' | M | 1 |
| Length | X | M | Note |
| NAF_ID value | -- | M | X |
| B-TID Tag | '81' | M | 1 |
| Length | Y | M | Note |
| B-TID value | -- | M | Y |
| NOTE: The length is coded according to ISO/IEC 8825 [20] |  |  |  |

-	NAF_ID Tag '80'

Contents:

-	Identifier of Network Application Function used in the GBA_U NAF Derivation procedure.

Coding:

-	As defined in 33.220 [25]

-	B-TID Tag '81'

Content:

-	Bootstrapping Transaction Identifier of the GBA_U bootstrapped key

Coding:

-	As defined in TS 33.220 [25]

Unused bytes shall be set to 'FF'

## 4.3	ISIM file structure

This subclause contains a figure depicting the file structure of the ADFISIM. ADFISIM shall be selected using the AID and information in EFDIR.

|  | ADFISIM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | EFIST |  |  | EFIMPI |  |  | EFDOMAIN |  |  | EFIMPU |  |  |
|  |  |  |  | '6F07' |  |  | '6F02' |  |  | '6F03' |  |  | '6F04' |  |  |
|  |  |  |  | EFAD |  |  | EFARR |  |  | EFP-CSCF |  |  | EFGBAP |  |  |
|  |  |  |  | '6FAD' |  |  | '6F06' |  |  | '6F09' |  |  | '6FD5' |  |  |
|  |  |  |  | EFGBANL |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | '6FD7' |  |  |  |  |  |  |  |  |  |  |  |

- Figure 1: File identifiers and directory structures of ISIM

# 5	Application protocol

The requirements stated in the corresponding section of 3GPP TS 31.101 [3] apply to the ISIM application.

The procedures listed in subclause "ISIM management procedures" are required for execution of the procedures in the subsequent subclause "ISIM security related procedures". The procedures authentication procedure, IMPI request, IMPU request and SIPdomain request, which are listed in subclause "ISIM security related procedures" are mandatory. If an ISIM Service table is available, the additional procedures are only executable if these services are indicated as "service available" in the ISIM Service table. However, if the procedures are implemented, it shall be in accordance with subclause " ISIM security related procedures".

## 5.1	ISIM management procedures

### 5.1.1	Initialisation

#### 5.1.1.1	ISIM application selection

If the Terminal wants to engage in IMS operation, then after UICC activation (see 3GPP TS 31.101 [3]), the Terminal shall select an ISIM application, if an ISIM application is listed in the EFDIR file, using the SELECT by DF name as defined in 3GPP TS 31.101.

After a successful ISIM application selection, the selected ISIM (AID) is stored on the UICC. This application is referred to as the last selected ISIM application. The last selected ISIM application shall be available on the UICC after a deactivation followed by an activation of the UICC.

If a ISIM application is selected using partial DF name, the partial DF name supplied in the command shall uniquely identify a ISIM application. Furthermore if a ISIM application is selected using a partial DF name as specified in TS 31.101 [3] indicating in the SELECT command the last occurrence the UICC shall select the ISIM application stored as the last ISIM application. If, in the SELECT command, the options first, next/previous are indicated, they have no meaning if an application has not been previously selected in the same session and shall return an appropriate error code.

#### 5.1.1.2	ISIM initialisation

The ISIM shall not indicate any language preference. It shall use the language indicated by any other application currently active on the UICC or by default, choose a language from EFPL at the MF level according the procedure defined in 3GPP TS 31.101[3].

If the terminal does not support the languages of EFPL, then the terminal shall use its own internal default selection.

The Terminal then runs the user verification procedure. If the procedure is not performed successfully, the ISIM initialisation stops.

Then the Terminal performs the administrative information request.

If all these procedures have been performed successfully then the ISIM session shall start. In all other cases the  ISIM session shall not start.

After the previous procedures have been completed successfully, the Terminal runs the following procedures:

-	IMPI request.

-	IMPU request.

-	SIP Domain request.

-	ISIM Service Table request. If the ISIM Service Table is not present, the terminal shall assume that no optional services are available.

- 	P-CSCF address request

After the ISIM initialisation has been completed successfully, the Terminal is ready for an ISIM session and shall indicate this to the ISIM by sending a particular STATUS command.

### 5.1.2	ISIM Session termination

NOTE 1:	This procedure is not to be confused with the deactivation procedure in 3GPP TS 31.101 [3].

The ISIM session is terminated by the Terminal as follows.

The Terminal shall indicate to the ISIM by sending a particular STATUS command that the termination procedure is starting.

Finally, the ME deletes all these subscriber related information elements from its memory.

NOTE 2:	If the Terminal has already updated any of the subscriber related information during the ISIM session, and the value has not changed until ISIM session termination, the Terminal may omit the respective update procedure.

To actually terminate the session, the Terminal shall then use one of the mechanisms described in 3GPP TS 31.101 [3].

### 5.1.3	ISIM application closure

After termination of the ISIM session as defined in subclause 5.1.2, the ISIM application may be closed by closing the logical channels that are used to communicate with this particular ISIM application.

### 5.1.4	UICC presence detection

The Terminal checks for the presence of the UICC according to 3GPP TS 31.101 [3] within all 30 s periods of inactivity on the UICC-Terminal interface during a IMS session. If the presence detection according to 3GPP TS 31.101 [3] fails the session shall be terminated as soon as possible but at least within 5s after the presence detection has failed.

### 5.1.5	Administrative information request

The Terminal performs the reading procedure with EFAD.

## 5.2	ISIM security related procedures

### 5.2.1	Authentication procedure

The Terminal selects an ISIM application and uses the AUTHENTICATE command (see subclause 7.1). The response is sent to the Terminal(in case of the T=0 protocol when requested by a subsequent GET RESPONSE command).

### 5.2.2	IMPI request

The Terminal performs the reading procedure with EFIMPI.

### 5.2.3	IMPU request

The Terminal performs the reading procedure with EFIMPU.

### 5.2.4	SIP Domain request

The Terminal performs the reading procedure with EFDOMAIN.

### 5.2.5	Void

### 5.2.6	ISIM Service Table request

Requirement:	ISIM Service Table available in the ISIM

Request:		The ME performs the reading procedure with EFIST.

### 5.2.7	P-CSCF address request

Requirement:	ISIM Service n°1 "available".

Request:		The ME performs the reading procedure with EFP-CSCF.

### 5.2.8	Generic Bootstrapping architecture (Bootstrap)

Requirement:	ISIM Service n°2 "available".

The Terminal uses the AUTHENTICATE command in GBA security context (Bootstrapping Mode) (see 7.1.1). The response is sent to the Terminal.

After a successful GBA_U Procedure, the Terminal shall update the B-TID field and the Key Life Time field in EFGBABP.

### 5.2.9	Generic Bootstrapping architecture (NAF Derivation)

Requirement:	ISIM Service n°2 "available".

The Terminal shall first read EFGBABP. The Terminal then uses the AUTHENTICATE command in GBA security context (NAF Derviation Mode) (see 7.1.1). The response is sent to the Terminal.

# 6	Security features

The security aspects of IMS are specified in 3GPP TS 33.203 [14]. This clause gives information related to security features supported by the ISIM with respect to user verification and file access conditions.

## 6.1	User verification and file access conditions

The security architecture as defined in 3GPP TS 31.101 [3] applies to the ISIM and UICC with the following definitions and additions:

-	The ISIM application shall use a global key referenceas PIN1 as specified in 3GPP TS 31.101 [3].

-	For access to DFTELECOM the PIN shall be verified.

-	The only valid usage qualifier is '08' which means user authentication knowledge based (PIN) as defined in ISO/IEC 7816-4 [6].

# 7	ISIM Commands

The commands specified in 3GPP TS 31.101 are supported by ISIM, with the restrictions identified in this clause.

## 7.1	AUTHENTICATE

### 7.1.1	Command description

The function can be used in several different contexts:

-	an IMS AKA security context during the procedure for authenticating the ISIM to its HN and vice versa when IMS AKA authentication data are available. The function shall be used whenever an IMS context shall be established, i.e. when the terminal receives a challenge from the IMS. A cipher key and an integrity key are calculated. For the execution of the command the ISIM uses the subscriber authentication key K, which is stored in the ISIM.

-	a HTTP Digest security context, when HTTP Digest authentication data are available. Digest authentication operations are described in IETF RFC 2617 [26].

-	a GBA_U security context, when a GBA bootstrapping procedure is requested. In this context the function is used in two different modes:

a)	Bootstrapping Mode: during the procedure for muthual authenticating of the ISIM and the Bootstrapping Server Function (BSF) and for deriving Bootstrapped key material from the AKA run.

b)	NAF Derivation Mode: during the procedure for deriving Network Application Function (NAF) specific keys from previous bootstrapped key material.

The function is related to a particular ISIM and shall not be executable unless the ISIM application has been selected and activated, and the current directory is the ISIM ADF or any subdirectory under this ADF and a successful PIN verification procedure has been performed (see clause 5).

#### 7.1.1.1	IMS AKA security context

The ISIM first computes the anonymity key AK = f5K (RAND) and retrieves the sequence number SQN = (SQN Å AK) Å AK.

Then the ISIM computes XMAC = f1K (SQN || RAND || AMF) and compares this with the MAC which is included in AUTN. If they are different, the ISIM abandons the function.

Next the ISIM verifies that the received sequence number SQN is previously unused. If it is unused and its value is lower than SQNMS, it shall still be accepted if it is among the last 32 sequence numbers generated. A possible verification method is described in 3GPP TS 33.102 [4].

NOTE:	This implies that the ISIM has to keep a list of the last used sequence numbers and the length of the list is at least 32 entries.

If  the ISIM detects the sequence numbers to be invalid, this is considered as a synchronisation failure and the ISIM abandons the function. In this case the command response is AUTS, where:

-	AUTS = Conc(SQNMS ) || MACS;

-	Conc(SQNMS) = SQNMS  f5*K(RAND) is the concealed value of the counter SQNMS in the ISIM; and

-	MACS = f1*K(SQNMS || RAND || AMF) where:

-	RAND is the random value received in the current user authentication request;

the AMF assumes a dummy value of all zeroes so that it does not need to be transmitted in clear in the resynchronisation message.

If the sequence number is considered in the correct range, the ISIM computes RES = f2K (RAND), the cipher key CK = f3K (RAND) and the integrity key IK = f4K (RAND) and includes these in the command response. Note that if this is more efficient, RES, CK and IK could also be computed earlier at any time after receiving RAND.

The use of AMF is HN specific and while processing the command, the content of the AMF has to be interpreted in the appropriate manner. The AMF may e.g. be used for support of multiple algorithms or keys or for changing the size of lists, see 3GPP TS 33.102 [4].

#### 7.1.1.2	GBA security context (Bootstrapping Mode)

ISIM operations in GBA security context are supported if service n°2 is "available".

The ISIM receives the RAND and AUTN. The ISIM first computes the anonymity key AK = f5K (RAND) and retrieves the sequence number SQN = (SQN Å AK) Å AK.

The ISIM calculates IK = f4K (RAND) and MAC (by performing the MAC modification function described in TS 33.220 [25]). Then the ISIM computes XMAC = f1K (SQN || RAND || AMF) and compares this with the MAC previously produced. If they are different, the ISIM abandons the function.

Then the ISIM proceeds as in IMS security context by checking AUTN. If the ISIM detects the sequence numbers to be invalid, this is considered as a synchronisation failure and the ISIM abandons the function. In this case the command response is AUTS, which is computed as in ISIM security context.

If the sequence number is considered in the correct range, the ISIM computes RES = f2K (RAND) and the cipher key CK = f3K (RAND).

The ISIM then derives and stores GBA_U bootstrapped key material from CK, IK values. The ISIM also stores RAND in the RAND field of EFGBABP

The ISIM stores GBA_U bootstrapped key material from only one bootstrapping procedure. The previous bootstrapped key material, if present, shall be replaced by the new one. This key material is linked with the data contained in EFGBABP : RAND, which is updated by the ISIM and B-TID, which shall be further updated by the ME.

NOTE:	According to TS 33.220 [25], NAF-specific keys that may be stored on the ISIM are not affected by this bootstrapping operation.

RES is included in the command response after flipping the least significant bit.

Input:

-	RAND, AUTN

Output:

-	RES

or

-	AUTS

#### 7.1.1.3	GBA security context (NAF Derivation Mode)

ISIM operations in GBA security context are supported if service n°2 is "available".

The ISIM receives the NAF_ID.

The ISIM performs Ks_ext_NAF and Ks_int_NAF derivation as defined in TS 33.220 [25] using the key material from the previous GBA_U bootstrapping procedure and the IMPI value from EFIMPI

If no key material is available this is considered as a GBA Bootstrapping failure and the ISIM abandons the function. The status word ‘6985’ (Conditions of use not satisfied) is returned.

Otherwise, the ISIM stores Ks_int_NAF and associated B-TID together with NAF_ID in its memory. The Ks_int_NAF keys related to other NAF_IDs, which are already stored in the ISIM, shall not be affected. The ISIM updates EFGBANL as follows:

-	If a record with the given NAF_ID already exists, the ISIM updates the B-TID field of this record with the B-TID value associated to the GBA_U bootstrapped key involved in this GBA_U NAF derivation procedure.

-	If a record with the given NAF_ID does not exist, the ISIM uses an empty record to store the NAF_ID and the B-TID value associated to the GBA_U bootstrapped key involved in this GBA_U NAF Derivation procedure.

NOTE: According to TS 33.220 [25], the ISIM can contain several Ks_int_NAF together with the associated B-TID and NAF_ID, but there is at most one pair of Ks_int_NAF and associated B-TID stored per NAF_ID.

Then, the ISIM returns Ks_ext_NAF.

Input:

-	NAF_ID

Output:

-	Ks_ext_NAF

### 7.1.2	Command parameters and data

| Code | Value |
| --- | --- |
| CLA | As specified in 3GPP TS 31.101 |
| INS | '88' |
| P1 | '00' |
| P2 | See table below |
| Lc | See below |
| Data | See below |
| Le | '00', or maximum length of data expected in response |

Parameter P2 specifies the authentication context as follows:

Coding of the reference control P2:

| Coding b8-b1 | Meaning |
| --- | --- |
| '1-------' | Specific reference data (e.g. DF specific/application dependant key) |
| '-XXXXXX-' | '000000' |
| '-----XXX' | Authentication context: 000 Reserved 001 IMS AKA 010 HTTP Digest 100 GBA context |

All other codings are RFU.

Command parameters/data:

#### 7.1.2.1	IMS AKA security context

| Byte(s) | Description | Length |
| --- | --- | --- |
| 1 | Length of RAND (L1) | 1 |
| 2 to (L1+1) | RAND | L1 |
| (L1+2) | Length of AUTN (L2) | 1 |
| (L1+3) to (L1+L2+2) | AUTN | L2 |

The coding of AUTN is described in 3GPP TS 33.102 [4]. The most significant bit of RAND is coded on bit 8 of byte 2. The most significant bit of AUTN is coded on bit 8 of byte (L1+3).

Response parameters/data, case 1, command successful:

| Byte(s) | Description | Length |
| --- | --- | --- |
| 1 | "Successful 3G authentication" tag = 'DB' | 1 |
| 2 | Length of RES (L3) | 1 |
| 3 to (L3+2) | RES | L3 |
| (L3+3) | Length of CK (L4) | 1 |
| (L3+4) to (L3+L4+3) | CK | L4 |
| (L3+L4+4) | Length of IK (L5) | 1 |
| (L3+L4+5) to (L3+L4+L5+4) | IK | L5 |

The most significant bit of RES is coded on bit 8 of byte 3. The most significant bit of CK is coded on bit 8 of byte (L3+4). The most significant bit of IK is coded on bit 8 of byte (L3+L4+5).

Response parameters/data, case 2, synchronization failure:

| Byte(s) | Description | Length |
| --- | --- | --- |
| 1 | "Synchronisation failure" tag = 'DC' | 1 |
| 2 | Length of AUTS (L1) | 1 |
| 3 to (L1+2) | AUTS | L1 |

The coding of AUTS is described in 3GPP TS 33.102 [4]. The most significant bit of AUTS is coded on bit 8 of byte 3.

#### 7.1.2.2	HTTP Digest security context

| Byte(s) | Description | Length |
| --- | --- | --- |
| 1 | Length of realm (L1) | 1 |
| 2 to (L1+1) | Realm | L1 |
| (L1+2) | Length of nonce (L2) | 1 |
| (L1+3) to (L1+L2+2) | Nonce | L2 |
| (L1+L2+3) | Length of cnonce (L3) | 1 |
| (L1+L2+4) to (L1+L2+L3+3) | Cnonce | L3 |

The codings of realm, nonce and cnonce are described in IETF RFC 2617 [26].

Response parameters/data command successful:

| Byte(s) | Description | Length |
| --- | --- | --- |
| 1 | "HTTP Digest context reponse" tag = 'DB' | 1 |
| 2 | Length of Response(L4) | 1 |
| 3 to (L4+2) | Response | L4 |
| (L4+3) | Length of Session Key (L5) | 1 |
| (L4+4) to (L4+L5+3) | Session Key | L5 |

#### 7.1.2.3		GBA security context (Bootstrapping Mode)

| Byte(s) | Description | Length |
| --- | --- | --- |
| 1 | “GBA Security Context Bootstrapping Mode” tag = ‘DD’ | 1 |
| 2 | Length of RAND (L1) | 1 |
| 3 to (L1+2) | RAND | L1 |
| (L1+3) | Length of AUTN (L2) | 1 |
| (L1+4) to (L1+L2+3) | AUTN | L2 |

Response parameters/data, GBA security context (Bootstrapping  Mode), synchronisation failure:

| Byte(s) | Description | Length |
| --- | --- | --- |
| 1 | "Synchronisation failure" tag = 'DC' | 1 |
| 2 | Length of AUTS (L1) | 1 |
| 3 to (L1+2) | AUTS | L1 |

AUTS coded as for IMS Security context.

Response parameters/data, GBA security context (Bootstrapping  Mode), command successful:

| Byte(s) | Description | Length |
| --- | --- | --- |
| 1 | "Successful GBA operation" tag = 'DB' | 1 |
| 2 | Length of RES (L) | 1 |
| 3 to (L+2) | RES | L |

RES coded as for IMS Security context.

#### 7.1.2.4		GBA security context (NAF Derivation Mode)

| Byte(s) | Description | Length |
| --- | --- | --- |
| 1 | “GBA Security Context NAF Derivation Mode” tag = ‘DE’ | 1 |
| 2 | Length of NAF_ID (L1) | 1 |
| 3 to (L1+2) | NAF_ID | L1 |

Response parameters/data, GBA security context (NAF Derivation Mode), command successful:

| Byte(s) | Description | Length |
| --- | --- | --- |
| 1 | "Successful GBA operation" tag = 'DB' | 1 |
| 2 | Length of Ks_ext_NAF (L) | 1 |
| 3 to (L+2) | Ks_ext_NAF | L |

Coding of Ks_ext_NAF as described in TS 33.220 [25].

### 7.1.3	Status Conditions Returned by the ISIM

Status of the card after processing of the command is coded in the status bytes SW1 and SW2. This subclause specifies coding of the status bytes in the following tables.

#### 7.1.3.1	Security management

| SW1 | SW2 | Error description |
| --- | --- | --- |
| '98' | '62' | - Authentication error, incorrect MAC |

#### 7.1.3.2	Status Words of the Commands

The following table shows for each command the possible status conditions returned (marked by an asterisk *).

- Commands and status words

| Status Words | AUTHENTICATE |
| --- | --- |
| 90 00 | * |
| 91 XX | * |
| 93 00 |  |
| 98 50 |  |
| 98 62 | * |
| 62 00 | * |
| 62 81 |  |
| 62 82 |  |
| 62 83 |  |
| 63 CX |  |
| 64 00 | * |
| 65 00 | * |
| 65 81 | * |
| 67 00 | * |
| 67 XX – (see note) | * |
| 68 00 | * |
| 68 81 | * |
| 68 82 | * |
| 69 81 |  |
| 69 82 | * |
| 69 83 |  |
| 69 84 | * |
| 69 85 | * |
| 69 86 |  |
| 6A 80 |  |
| 6A 81 | * |
| 6A 82 |  |
| 6A 83 |  |
| 6A 86 | * |
| 6A 87 |  |
| 6A 88 | * |
| 6B 00 | * |
| 6E 00 | * |
| 6F 00 | * |
| 6F XX – (see note) | * |
| NOTE: Except SW2 = '00'. |  |

## 7.2	GET CHALLENGE

The GET CHALLENGE command is optional for the ISIM application.

# 8	void

Annex A (informative):
EF changes via Data Download or CAT applications

This annex defines if changing the content of an EF by the network (e.g. by sending an SMS), or by a CAT Application [22], is advisable. Updating of certain EFs "over the air" could result in unpredictable behavior of the UE; these are marked "Caution" in the table below. Certain EFs are marked "No"; under no circumstances should "over the air" changes of these EFs be considered.

| File identification | Description | Change advised |
| --- | --- | --- |
| '6F02' | IMS private user identity | Caution (note) |
| '6F03' | Home Network Domain Name | Caution (note) |
| '6F04' | IMS public user identity | Caution (note) |
| '6FAD' | Administrative Data | Caution |
| '6F06' | Access Rule Reference | Caution |
| ‘6F07’ | ISIM Service Table | Caution |
| ‘6F09’ | P-CSCF address | Caution (note) |
| '6FD5’ | GBA Bootstrapping parameters | Caution |
| '6FD7’ | GBA NAF List | Caution |
| NOTE: If EFIMPI, EFIMPU, EFDOMAIN or P-CSCF are changed, the UICC should issue a CAT REFRESH command [22]. |  |  |

Annex B (informative):
Tags defined in 31.103

| Tag | Name of Data Element | Usage |
| --- | --- | --- |
| '80' | NAF_ID | EFGBANL |
| '80' | NAI TLV data object | EFIMPI |
| '80' | P-CSCF TLV data object | EFP-CSCF |
| '80' | URI TLV data object | EFIMPU, EFDOMAIN |
| '81' | B-TID | EFGBANL |
| 'DB' | Successful IMS authentication | Response to AUTHENTICATE“IMS AKA security context” |
| 'DB' | HTTP Digest Context response | Response to AUTHENTICATE “HTTP Digest security context” |
| 'DB' | Successful GBA operation | Response to AUTHENTICATE “GBA security context” |
| 'DC' | Synchronisation failure | Response to AUTHENTICATE “IMS AKA security context” or “GBA security context (Bootstrapping Mode)” |
| 'DD' | GBA Security Context Bootstrapping Mode | AUTHENTICATE “GBA security context” |
| 'DE' | GBA Security Context NAF Derivation Mode | AUTHENTICATE “GBA security context” |

NOTE:	the value 'FF' is an invalid tag value. For ASN.1 tag assignment rules see ISO/IEC 8825 [20]

Annex C (informative):
Suggested contents of the EFs at pre-personalization

If EFs have an unassigned value, it may not be clear from the main text what this value should be. This annex suggests values in these cases.

| File Identification | Description | Value |
| --- | --- | --- |
| '6F02' | IMS private user identity | ‘8000FF…FF’ |
| '6F03' | Home Network Domain Name | ‘8000FF…FF’ |
| '6F04' | IMS public user identity | ‘8000FF…FF’ |
| '6FAD' | Administrative Data | Operator dependant |
| '6F06' | Access Rule Reference | Card issuer/operator dependant |
| '6FD5’ | GBA Bootstrapping parameters | ‘FF…FF’ |
| ‘6F07’ | ISIM Service Table | Operator dependant |
| ‘6F09’ | P-CSCF address | Operator dependant |
| ‘6FD7’ | GBA NAF List | ‘FF…FF’ |

Annex D (informative):
List of SFI Values

This annex lists SFI values assigned in the present document.

# D.1	List of SFI Values at the ISIM ADF Level

| File Identification | SFI | Description |
| --- | --- | --- |
| '6F02' | '02' | IMS private user identity |
| '6F03' | '05' | Home Network Domain Name |
| '6F04' | '04' | IMS public user identity |
| '6FAD' | '03' | Administrative Data |
| '6F06' | '06' | Access Rule Reference |
| ‘6F07’ | ‘07’ | ISIM Service Table |

All other SFI values are reserved for future use.

Annex E (informative):
ISIM Application Session Activation / Termination

The purpose of this annex is to illustrate the different Application Session procedures.

Figure E.1: ISIM Application Session Activation procedure

Figure E.2: ISIM Application Session Termination procedure

Annex F (informative):
Change History

The table below indicates all CRs that have been incorporated into the present document since it was initially approved.

| TSG # / Date | TSG Doc. |  | CR | Rev | Cat | Subject/Comment | New |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TP-16 | TP-020124 |  |  |  |  | Initial version for information and approval in one step | 1.0.0 |
|  |  |  |  |  |  | Comment: T#16 approved the specification to be part of Rel-5. The only changes to v1.0.0 are in the references clause for the reference in [16] | 5.0.0 |
| TP-17 | TP-020211 |  | 001 |  | F | Corrections | 5.1.0 |
| TP-18 | TP-020281 |  | 002 |  | F | Replace reference to TS 31.110 by reference to ETSI TS 101 220 | 6.0.0 |
|  |  |  | 003 |  | F | Management of last selected ISIM |  |
|  |  |  | 004 |  | D | Gather all 3GPP-specific card platform requirements into TS 31.101, and remove them from TS 31.103. NOTE: This CR created Rel-6 of TS 31.103. |  |
| TP-19 | TP-030019 |  | 006 |  | F | Alignment with the Stage 2 terminology | 6.1.0 |
| TP-20 | TP-030122 |  | 008 |  | A | Clarification that the home operator's network domain name is a SIP URI. | 6.2.0 |
| TP-23 | TP-040025 |  | 012 |  |  | Essential corrections to remove Session Keys | 6.3.0 |
|  |  |  | 014 |  |  | Creation of an ISIM Service Table |  |
|  | TP-040067 |  | 013 |  |  | New EF for P-CSCF Addresses in ISIM |  |
| TP-24 | TP-040102 |  | 015 |  | F | Clarification that the P-CSCF address shall not be used by a 3GPP terminal accessing a Interworking WLAN | 6.4.0 |
| TP-25 | TP-040182 |  | 017 |  | B | GBAU ME-ISIM interface | 6.5.0 |
|  | TP-040182 |  | 016 |  | B | New 3GPP2 IMS authentication context in ISIM | 6.5.0 |
| TP-26 | TP-040257 |  | 019 |  | B | Storage of the lifetime of the GBA_U bootstrapped keys | 6.6.0 |
|  | TP-040257 |  | 021 |  | F | Correction of non specific references | 6.6.0 |
| TP-27 | TP-050019 | T3-050126 | 022 |  | A | Reservation of file IDs under ADF ISIM | 6.7.0 |
| TP-27 | TP-050019 | T3-050182 | 024 |  | F | Completion of GBA_U-related procedures | 6.7.0 |
| TP-27 | TP-050019 | T3-050185 | 025 |  | F | Storage of NAF-keys identifiers in GBA_U | 6.7.0 |
| CT-28 | TP-050136 | C6-050416 | 027 |  | A | ISO/IEC 7816-series revision | 6.8.0 |
| CT-29 | CP-050330 | C6-050731 | 029 | 1 | A | Default Record for EFIMPU | 6.9.0 |
|  | CP-050335 | C6-050693 | 030 |  | F | Corrections of EFIST Service No. referencing |  |
|  |  | C6-050694 | 031 |  | F | Changes in Application Protocol due to the use of ISIM Service Table |  |
| CT-30 | CP-050492 | C6-050885 | 035 |  | F | Subscription related procedures | 6.10.0 |
|  |  | C6-050904 | 033 |  | F | Encoding of IMPI, IMPU and Domain |  |
| CT-32 | CP-060243 | C6-060282 | 0036 |  | F | Add missing EF in ISIM file structure | 7.0.0 |
|  |  | C6-060319 | 0037 |  | F | Update of the table summarizing the tags defined in 31.103 |  |