3GPP TS 32.106-7 V3.3.0 (2001-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Part 7: Basic Configuration Management IRP:

CMIP Solution Set 1:1

(Release 1999)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Keywords

UMTS, management

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2001, 3GPP Organizational Partners (ARIB, CWTS, ETSI, T1, TTA, TTC).

All rights reserved.

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	Basic aspects	6

4.1	CMIP specific aspects	6

4.1.1	About Associations	6

4.1.2	About getContainment	6

4.1.3	About getMoAttributes	7

4.1.4	Allowed Alarms of MOCs	7

4.2	Mapping	7

4.2.1	Mapping of Operations	7

4.2.2	Mapping of operation parameters	8

4.2.2.1	Mapping of Parameters of ‘getMoAttributes’	8

4.2.2.2	Mapping of Parameters of ‘getContainment’	9

4.2.2.3	Mapping of parameters of ‘getBasicCmIRPVersion’	9

4.2.3	Mapping of notifications	10

4.2.4	Mapping of notification parameters	10

4.2.4.1	Mapping of parameters of the notification ‘notifyObjectCreation’	10

4.2.4.2	Mapping of parameters of the notification ‘notifyObjectDeletion’	10

4.2.4.3	Mapping of parameters of the notification ‘notifyAttributeValueChange’	11

4.2.5	Mapping of MOCs	11

4.2.6	Mapping of Attributes	12

5	GDMO Definitions	13

5.1	Managed Object Classes (MOCs)	13

5.2	Packages	19

5.3	Actions	23

5.4	Attributes	23

5.5	Name Bindings	28

5.6	Behaviours	34

6.	ASN.1 Definitions	43

Annex A (informative):	Change history	46

# Foreword

This Technical Specification (TS) has been produced by the 3rd Generation Partnership Project (3GPP).

The present document is part 7 of a multi-part TS covering the 3rd Generation Partnership Project: Technical Specification Group Services and System Aspects; Telecommunication Management; Configuration Management, as identified below:

Part 1:	“3G Configuration Management: Concept and Requirements”;

Part 2:	“Notification Integration Reference Point: Information Service Version 1”;

Part 3:	“Notification Integration Reference Point: CORBA Solution Set Version 1:1”;

Part 4:	“Notification Integration Reference Point: CMIP Solution Set Version 1:1”;

Part 5:	“Basic Configuration Management IRP: Information Model Version 1”;

Part 6:	“Basic Configuration Management IRP CORBA Solution Set Version 1:1”;

Part 7:	“Basic Configuration Management IRP CMIP Solution Set Version 1:1”;

Part 8:	“Name Convention for Managed Objects”.

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

Configuration Management (CM), in general, provides the operator with the ability to assure correct and effective operation of the 3G-network as it evolves. CM actions have the objective to control and monitor the actual configuration on the NEs and NRs, and they may be initiated by the operator or functions in the OSs or NEs.

CM actions may be requested as part of an implementation programme (e.g. additions and deletions), as part of an optimisation programme (e.g. modifications), and to maintain the overall Quality of Service. The CM actions are initiated either as a single action on a Network Element of the 3G-network or as part of a complex procedure involving actions on many Network Elements.

The Itf-N interface for Configuration Management is built up by a number of Integration Reference Points (IRPs) and a related Name Convention, which realise the functional capabilities over this interface. The basic structure of the IRPs is defined in 3GPP TS 32.101 [1] and 3GPP TS 32.102 [2]. For CM, a number of IRPs (and the Name Convention) are defined, used by this as well as other technical specifications for telecom management produced by 3GPP. All these documents are included in Parts 2 and onwards of the 3GPP TS 32.106.

This document constitutes 32.106 Part 7 - Basic Configuration Management IRP CMIP Solution Set Version 1:1.

# 1	Scope

The present document defines a CMIP Solution Set for the Basic CM IRP introduced in 3GPP TS 32.106-5 [15]. 
The version of this CMIP Solution Set is 1:1, where the first “1” means that it corresponds to the Information Model version 1, and the second “1” means that it is the first CMIP Solution Set corresponding to Information Model version 1.

Clause 4 maps the protocol- and technology-independent operations, parameters, notifications and the Network Resource Model specified in the Basic CM IRP Information Model (Chapter 6 of 3GPP 3GPP TS 32.106-5 [15]) onto the corresponding CMIP/CMISE equivalences. The important technical aspects specific to this CMIP Solution Set are also described there. The GDMO definitions are introduced in Clause5. Clause 6 contains the ASN.1 definitions related to the GDMO definitions provided in clause 5.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "3G Telecom Management principles and high level requirements".

[2]	3GPP TS  32.102: "3G Telecom Management architecture".

[3]	3GPP TS  32.106 Part 2: "Notification IRP: Information Service".

[4]	ITU-T Recommendation M.3100 (07/95) - Generic Network Information Model

[5]	ITU-T Recommendation M.3100 Corrigendum 1 (07/98)"

[6]	ITU-T Recommendation M.3100 Amendment 1 (03/99)"

[7]	ITU-T Recommendation X.710 (1991) - Common Management Information Service  Definition for CCITT Applications.

[8]	ITU-T Recommendation X.721 (02/92) - Information Technology - Open Systems Interconnection – Structure of Management Information: Definition of Management Information.

[9]	ITU-T Recommendation X.730 (01/92) - Information Technology - Open Systems Interconnection – Systems Management:  Object Management Function.

[10]	ITU-T Recommendation X.731 (02/92) - Information Technology - Open Systems Interconnection - Systems Management: State Management Function.

[11]	ITU-T Recommendation X.732 (01/92) - Information technology - Open Systems Interconnection - Systems Management: Attributes for Representing Relationships.

[12]	ETS 300 622 (GSM 12.20): “Digital cellular telecommunications system (Phase 2); Base Station System (BSS) Management Information, June 1996”.

[13]	3GPP TS  32.106 Part 8:  “Name Convention for Managed Objects”.

[14]	3GPP TS  32.106 Part 1: “3G Configuration Management”.

[15]	3GPP TS  32.106 Part 5: “3G Configuration Management IRP: Information Model Version 1”.

[16]	3GPP TS  32.106 Part 4: “Notification Integration Reference Point: CMIP Solution Set Version 1:1”.

# 3	Definitions and abbreviations

## 3.1	Definitions

The terms and definitions introduced in 3GPP TS 32.106-1, 3GPP TS 32.106-2 and 3GPP TS 32.106-5 apply in this document.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

CMIP	Common Management Information Protocol

DN	Distinguished Name

GDMO	Guidelines for the Definition of Managed Objects

IDL	Interface Definition Language

IEC	International Electro-technical Commission

ISO	International Standards Organization

ITU-T	International Telecommunication Union, Telecommunication Sector

MIB	Management Information Base

MIM	Management Information Model

MIT	Management Information Tree (or Naming Tree)

MOC	Managed Object Class

MOI	Managed Object Instance

NE	Network Element

NR	Network Resource

NRM	Network Resource Model

TMN	Telecommunications Management Network

# Basic aspects

## CMIP specific aspects

This clause describes some technical details specific to CMIP technology, which are not easy to be handled in the related GDMO definitions.

### About Associations

In the GDMO definitions, except the containment relations, all associations among different object classes and object instances are modelled with dedicated pointers of the concerned objects, i.e. various relation role attributes. These pointers are normal object attributes and don’t require any special treatment. The service operation getMoAttributes defined in 3GPP TS 32.106-5 [15] and mapped on M-GET in this CMIP solution set is applied for managers to retrieve the values of these association pointers and the notification attributeValueChange is applied for agents to report any change of the values of these association pointers.

### About getContainment

In the GDMO definition the containment relations of the Managed Object Classes and those of the managed object instances are described by the name bindings. The service operation getContainment is defined in 3GPP TS 32.106-5 [15] to enable managers to retrieve the management information about the containment tree of the local MIB of an agent. This service operation is mapped to CMISE M-GET in this CMIP solution set. The information about the containment relation of a local MIB is consists of all MOIs abstracted from the output parameter AttributeList of a M-GET operation.

### About getMoAttributes

The service operation getMoAttributes defined in the Basic CM IRP IM (3GPP TS 32.106-5 [15]) provides the basic functionality required to retrieve managed objects and their attributes, which is a subset of the functionality provided by the corresponding CMISE service operation M-GET. getMoAttributes is mapped to M-GET  in this standard. This doesn’t mean any limitation for using M-GET. Users of this standard are encouraged to use the whole functionality provided by M-Get, especially the input parameter “Attribute Identifier List” (see ITU-T X.710 [7]).

### Allowed Alarms of MOCs

Neither the Basic CM IRP IM (3GPP TS 32.106-5 [15]) nor Alarm IRP IS (3GPP TS 32.111-2 [3]) specifies the allowed alarms of each MOC in the sense of EventType or/and ExtendedEventType. Table 1 defines the allowed alarms of each MOCs for this CMIP Solution Set. Further study of table 1 and its relationship to 3GPP TS 32.106-5 [15] is planed for Releases 4/5.

Table 1: Allowed alarms of MOCs

| MOCs | Legal Alarms |
| --- | --- |
| G3SubNetwork | EnvironmentalAlarm |
| G3ManagedElement | environmentalAlarm equipmentAlarm communicationsAlarm processingErrorAlarm |
| ManagementNode | environmentalAlarm equipmentAlarm communicationsAlarm processingErrorAlarm |
| ManagedFunction | communicationsAlarm processingErrorAlarm QualityofServiceAlarm |
| IRPAgent | communicationsAlarm processingErrorAlarm |
| AlarmIRP | alarmListRebuiltAlarm |

The MOCs, which do not appear in table 1, may not issue any alarm except the alarms that are defined allowed for its parent MOC(s).

## 4.2	Mapping

The semantics of the Basic CM IRP IM are defined in 3GPP TS 32.106-5 [15]. The definitions of the management services and management information defined there are independent of any implementation technology and protocol. 
This subclause maps these technology and protocol independent definitions onto the equivalencies of the CMIP Solution Set of the Basic CM IRP.

### 4.2.1	Mapping of Operations

Table 2 maps the operations defined in the Basic CM IRP Information Service onto the equivalent Actions/Services of the CMIP Solution Set. The CMIP Actions/Services are qualified as Mandatory (M) or Optional (O).

Table 2: Mapping of operations

| Operations of Information Services of the Basic CM IRP defined in 3GPP TS 32.106-5 [15] | Equivalent operation of the CMIP solution set of the Basic CM IRP | Qualifier |
| --- | --- | --- |
| getMoAttributes | M-GET (CMISE Service) | M |
| getContainment | M-GET (CMISE Service) | O |
| getBasicCmIRPVersion | M-ACTION getBasicCmIRPVersion (Action of MOC bcmControl) | M |

### 4.2.2	Mapping of operation parameters

Tables 3, 4 and 5 in the following subclauses show the parameters of each operation defined in the Information Service described in 3GPP TS 32.106-5 [15] and their equivalences in the CMIP Solution Set.

#### 4.2.2.1	Mapping of Parameters of ‘getMoAttributes’

Table 3: Mapping of parameters of ‘getMoAttributes’

| Parameters of the operation ‘getMoAttributes’ defined in 3GPP TS 32.106-5 [15] | CMISE M-GET parameters | Qualifier |
| --- | --- | --- |
| baseObjectInstance | Base object instance | M |
| scope | Scope | M |
| filter | Filter | M |
| no equivalence | Invoker identifier This is a CMISE specific parameter. There is no equivalent parameter defined in the Information Service for ‘getMoAttributes’. | O |
| no equivalence | Basic object class This is a CMISE specific parameter. There is no equivalent parameter defined in the Information Service for ‘getMoAttributes’. | M |
| no equivalence | Access Control This is a CMISE specific parameter. There is no equivalent parameter defined in the Information Service for ‘getMoAttributes’. | O |
| no equivalence | Synchronisation This is a CMISE specific parameter. There is no equivalent parameter defined in the Information Service for ‘getMoAttributes’. | O |
| attributeListIn | Attribute identifier list | M |
| managedObjectClass | Managed object class | M |
| managedObjectInstance | Managed object instance | M |
| attributeListOut | Attribute list | M |
| status | Errors | M |
| no equivalence | Current time This is a CMISE specific parameter. There is no equivalence parameter defined in the Information Service for ‘getMoAttributes’. | O |

#### Mapping of Parameters of ‘getContainment’

Table 4: Mapping of parameters of ‘getContainment’

| Parameters of the operation ‘getContainment’ defined in 3GPP TS 32.106-5 [15] | CMISE M-GET parameter | Qualifier |
| --- | --- | --- |
| baseObjectInstance | Base object instance | M |
| scope | Scope | O |
| no equivalence | filter This is a CMISE specific parameter. There is no equivalent parameter defined in the Information Service for ‘getContainment’. The value of this parameter shall be ‘empty’. | O |
| no equivalence | Invoker identifier This is a CMISE specific parameter. There is no equivalent parameter defined in the Information Service for ‘getContainment’. | O |
| no equivalence | Basic object class This is a CMISE specific parameter. There is no equivalent parameter defined in the Information Service for ‘getContainment’. | M |
| no equivalence | Access Control This is a CMISE specific parameter. There is no equivalent parameter defined in the Information Service for ‘getContainment’. | O |
| no equivalence | Synchronisation This is a CMISE specific parameter. There is no equivalent parameter defined in the Information Service for ‘getContainment’. | O |
| no equivalence | Attribute identifier list This is a CMISE specific parameter. There is no equivalence parameter defined in the Information Service for ‘getContainment’. It is recommended to use ‘objectClass’ or/and ‘nameBinding’ defined in X.721 for the MOC top as the value of this input parameter. | O |
| containment | Managed object class | M |
|  | Managed object instance | M |
|  | Attribute list | M |
| status | Errors | M |
| no equivalence | Current time This is a CMISE specific parameter. There is no equivalence parameter defined in the Information Service for ‘getMoAttributes’. | O |

#### 4.2.2.3	Mapping of parameters of ‘getBasicCmIRPVersion’

Table  5: Mapping of parameters of “getBasicCmIRPVersion”

| Operation parameters of the Basic CM IRP Information Services | CMISE M-ACTION Parameters | Qualifier |
| --- | --- | --- |
| no equivalence | Invoke identifier | M |
| no equivalence | Linked identifier | O |
| no equivalence | Mode | M |
| no equivalence | Base object class (input) | M |
| no equivalence | Base object instance (input) | M |
| no equivalence | Scope | O |
| no equivalence | Filter | O |
| no equivalence | Managed object class (output) | O |
| no equivalence | Managed object instance (output) | O |
| no equivalence | Access control | O |
| no equivalence | Synchronization | O |
| no equivalence | Action type | M |
| no equivalence | Action information | O |
| no equivalence | Current time | O |
| versionNumberList, status | Action reply | O |
| no equivalence | Errors | O |

### 4.2.3	Mapping of notifications

Table 6 maps the notifications defined in the Basic CM IRP Information Service onto the equivalent notification of the CMIP Solution Set. The CMIP notifications are qualified as Mandatory (M) or Optional (O).

Table  6: Mapping of notifications

| Notifications of Basic CM IRP Information Service | Notifications of the Basic CM IRP CMIP solution set | Qualifier |
| --- | --- | --- |
| notifyObjectCreation | objectCreation ITU-T X.721 {smi2Notification 6} | O |
| notifyObjectDeletion | objectDeletion ITU-T X.721 {smi2Notification 7} | O |
| notifyAttributeValueChange | attributeValueChange ITU-T X.721 {smi2Notification 1} | O |

### 4.2.4	Mapping of notification parameters

Tables 7, 8 and 9 in the following subclauses show the parameters of each notification defined in the Information Service described in 3GPP TS 32.106-5 [15] and their equivalence in the CMIP Solution Set.

The mapping of common parameters of all kinds of notifications defined in 3GPP TS 32.106-2 [3] is described in 3GPP TS 32.106-4 [16] and will not be repeated in the present document. These common parameters are managedObjectClass, managedObjectInstance, NotificationId, eventType, extendedEventType, eventTime and systemDN.

#### 4.2.4.1	Mapping of parameters of the notification ‘notifyObjectCreation’

Table  7: Mapping of parameters of the notification ‘notifyObjectCreation’

| Parameters of the Basic CM IRP IS notification ‘notifyObjectCreation’ | Parameters of the CMIP SS notification ‘objectCreation’ | Qualifier |
| --- | --- | --- |
| correlatedNotifications | correlatedNotifications | O |
| sourceIndicator | sourceIndicator | O |
| attributeList | attributeList | O |
| no equivalence | additionalText | O |
| no equivalence | additionalInformation | O |

#### Mapping of parameters of the notification ‘notifyObjectDeletion’

Table  8: Mapping of parameters of the notification ‘notifyObjectDeletion’

| Parameter of the Basic CM IRP IS notification ‘notifyObjectDeletion’ | parameter of the CMIP SS notification ‘objectDeletion’ | Qualifier |
| --- | --- | --- |
| correlatedNotifications | correlatedNotifications | O |
| sourceIndicator | sourceIndicator | O |
| attributeList | attributeList | O |
| no equivalence | additionalText | O |
| no equivalence | additionalInformation | O |

#### Mapping of parameters of the notification ‘notifyAttributeValueChange’

Table  9: Mapping of parameters of the notification ‘notifyAttributeValueChange’

| Parameter of the Basic CM IRP IS notification ‘notifyAttributeValueChange’ | parameter of the CMIP SS notification ‘attributeValueChange’ | Qualifier |
| --- | --- | --- |
| correlatedNotifications | correlatedNotifications | O |
| sourceIndicator | sourceIndicator | O |
| attributeValueChangeDefinition | attributeValueChangeDefinition | M |
| no equivalence | attributeIdentifierList | O |
| no equivalence | additionalText | O |
| no equivalence | additionalInformation | O |

### Mapping of MOCs

Table 10 maps the MOCs defined in the CM IRP Network Resource Model onto the equivalent MOCs of the CMIP Solution Set.

Table 10: Mapping of MOCs

| MOCs of the Basic CM IRP NRM | MOCs of the CMIP SS |
| --- | --- |
| AlarmIRP | alarmControl (defined in 3GPP TS 32.111-4 [16]) |
| AucFunction | aucFunction |
| BasicCmIRP | bcmControl |
| G3ManagedElement | g3ManagedElement |
| G3SubNetwork | g3SubNetwork |
| GgsnFunction | ggsnFunction |
| GmscFunction | gmscFunction |
| HlrFunction | hlrFunction |
| IRPAgent | irpAgent |
| IubLink | iubLink |
| ManagedFunction | managedFunction |
| ManagementNode | managementNode |
| MeContext | meContext |
| MscFunction | mscFunction |
| NodeBFunction | nodeBFunction |
| NotificationIRP | notificationControl (defined in 3GPP TS 32.106-4 [16]) |
| RncFunction | rncFunction |
| SgsnFunction | sgsnFunction |
| SmsGmscFunction | smsGmscFunction |
| SmsIwmscFunction | smsIwmscFunction |
| UtranCell | utranCell |
| VlrFunction | vlrFunction |

### Mapping of Attributes

Table 11: Mapping of Attributes

| Attribute defined in 3GPP TS 32.106-5 [15] | Attribute defined in this CMIP SS |
| --- | --- |
| alarmIRPId | alarmControlId |
| aucFunctionId | aucFunctionId |
| basicCmIRPId | basicCmControlId |
| bgFunctionId | bgFunctionId |
| dnPrefix | systemTitle |
| eirFunctionId | eirFunctionId |
| g3ManagedElementId | g3ManagedElementId |
| g3SubNetworkId | g3SubNetworkId |
| ggsnFunctionId | ggsnFunctionId |
| gmscFunctionId | gmscFunctionId |
| hlrFunctionId | hlrFunctionId |
| irpAgentId | irpAgentId |
| irpVersion | supportedBcmIRPVersions supportedAlarmIRPVersion (3GPP TS 32.111-4 [16]) supportedNotificationIRPVersion (3GPP TS 32.106-4 [16]) |
| iubLinkId | iubLinkId |
| iubLink-NodeBFunction | iubLinkNodeBFunctionLink |
| iubLink-UtranCell | iubLinkUtranCellLink |
| locationName | locationName |
| managedBy | meManagedBy |
| managedElementType | managedElementType |
| managementNodeId | managementNodeId |
| manages | mnManagesList |
| meContextId | meContextId |
| mscFunctionId | mscFunctionId |
| vlrFunctionId | vlrFunctionId |
| nodeBFunctionId | nodeBFunctionId |
| nodeBFunction-IubLink | nodeBiubLinkLink |
| notificationIRPId | notificationControlId |
| rncFunctionId | rncFunctionId |
| sgsnFunctionId | sgsnFunctionId |
| smsGmscFunctionId | smsGmscFunctionId |
| smsIwmscFunctionId | smsIwmscFunctionId |
| systemDN | no equivalence |
| userDefinedState | userDefinedState |
| userLabel | userLabel |
| utranCellId | utranCellId |
| utranCell-IubLink | utranCelliubLinkLink |
| vendorName | vendorName |

# 5	GDMO Definitions

## 5.1	Managed Object Classes (MOCs)

aucFunction MANAGED OBJECT CLASS

DERIVED FROM       managedFunction;

CHARACTERIZED BY

aucFunctionBasicPackage PACKAGE

BEHAVIOUR

aucFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"An instance of MOC represents the logical function of an AUC";;

ATTRIBUTES

aucFunctionId GET;;;

REGISTERED AS {ts32-106-7BCMObjectClass 1};

bcmControl MANAGED OBJECT CLASS

DERIVED FROM       "Recommendation X.721: 1992":top;

CHARACTERIZED BY

bcmControlBasicPackage,

bcmIRPVersionPackage;

REGISTERED AS {ts32-106-7BCMObjectClass 2};

bgFunction MANAGED OBJECT CLASS

DERIVED FROM       managedFunction;

CHARACTERIZED BY

bgFunctionBasicPackage PACKAGE

BEHAVIOUR

bgFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"An instance of MOC represents the logical function of an BG";;

ATTRIBUTES

bgFunctionId GET;;;

REGISTERED AS {ts32-106-7BCMObjectClass 3};

eirFunction MANAGED OBJECT CLASS

DERIVED FROM       managedFunction;

CHARACTERIZED BY

eirFunctionBasicPackage PACKAGE

BEHAVIOUR

eirFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"An instance of MOC represents the logical function of an EIR";;

ATTRIBUTES

eirFunctionId GET;;;

REGISTERED AS {ts32-106-7BCMObjectClass 4};

ggsnFunction MANAGED OBJECT CLASS

DERIVED FROM       managedFunction;

CHARACTERIZED BY

ggsnFunctionBasicPackage PACKAGE

BEHAVIOUR

ggsnFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"An instance of MOC represents the logical function of an GGSN";;

ATTRIBUTES

ggsnFunctionId GET;;;

REGISTERED AS {ts32-106-7BCMObjectClass 5};

g3ManagedElement MANAGED OBJECT CLASS

DERIVED FROM       "Recommendation X.721: 1992":top;

CHARACTERIZED BY

g3ManagedElementBasicPackage,

g3ManagedElementAssociationPackage;

CONDITIONAL PACKAGES

"Recommendation M.3100: 1995":createDeleteNotificationsPackage  PRESENT IF

"the objectCreation and the objectDeletion defined in Recommendation

X.721 are supported by an instance of this class.",

"Recommendation M.3100: 1995":attributeValueChangeNotificationPackage PRESENT IF

"the attributeValueChange notifications defined in Recommendation X.721

are supported by an instance of this class.",

"Recommendation M.3100: 1995":processingErrorAlarmPackage  PRESENT IF

"the processingErrorAlarm notifications defined in Recommendation X.721

are supported by an instance of this class.",

"Recommendation M.3100: 1995":environmentalAlarmPackage  PRESENT IF

"the environmentalAlarm notifications defined in Recommendation X.721

are supported by an instance of this class.",

communicationsAlarmPackage  PRESENT IF

"the communicationsAlarm notifications defined in Recommendation X.721

are supported by an instance of this class.",

equipmentAlarmPackage  PRESENT IF

"the equipmentAlarm notifications defined in Recommendation X.721

are supported by an instance of this class.";

REGISTERED AS {ts32-106-7BCMObjectClass 6};

g3SubNetwork MANAGED OBJECT CLASS

DERIVED FROM       "Recommendation X.721: 1992":top;

CHARACTERIZED BY

g3SubNetworkBasicPackage;

CONDITIONAL PACKAGES

"Recommendation M.3100: 1995":attributeValueChangeNotificationPackage  PRESENT IF

"the attributeValueChange notifications defined in Recommendation X.721

are supported by an instance of this class.",

"Recommendation M.3100: 1995":environmentalAlarmPackage  PRESENT IF

"the environmentalAlarm notifications defined in Recommendation X.721

are supported by an instance of this class.";

REGISTERED AS {ts32-106-7BCMObjectClass 7};

hlrFunction MANAGED OBJECT CLASS

DERIVED FROM       managedFunction;

CHARACTERIZED BY

hlrFunctionBasicPackage PACKAGE

BEHAVIOUR

hlrFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"An instance of MOC represents the logical function of a HLR";;

ATTRIBUTES

hlrFunctionId GET;;;

REGISTERED AS {ts32-106-7BCMObjectClass 8};

gmscFunction MANAGED OBJECT CLASS

DERIVED FROM       managedFunction;

CHARACTERIZED BY

gmscFunctionBasicPackage PACKAGE

BEHAVIOUR

gmscFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"An instance of MOC represents the logical function of a GMSC";;

ATTRIBUTES

gmscFunctionId GET;;;

REGISTERED AS {ts32-106-7BCMObjectClass 9};

irpAgent MANAGED OBJECT CLASS

DERIVED FROM       "Recommendation X.721: 1992":top;

CHARACTERIZED BY

irpAgentBasicPackage;

CONDITIONAL PACKAGES

"Recommendation M.3100: 1995":processingErrorAlarmPackage  PRESENT IF

"the processingErrorAlarm notifications defined in Recommendation X.721

are supported by an instance of this class.",

communicationsAlarmPackage  PRESENT IF

"the communicationsAlarm notifications defined in Recommendation X.721

are supported by an instance of this class.";

REGISTERED AS {ts32-106-7BCMObjectClass 10};

iubLink MANAGED OBJECT CLASS

DERIVED FROM       managedFunction;

CHARACTERIZED BY

iubLinkBasicPackage,

iubLinkAssociationPackage;

REGISTERED AS {ts32-106-7BCMObjectClass 11};

managedFunction MANAGED OBJECT CLASS

DERIVED FROM       "Recommendation X.721: 1992":top;

CHARACTERIZED BY

managedFunctionBasicPackage;

CONDITIONAL PACKAGES

"Recommendation M.3100: 1995":createDeleteNotificationsPackage  PRESENT IF

"the objectCreation and the objectDeletion defined in Recommendation

X.721 are supported by an instance of this class.",

"Recommendation M.3100: 1995":attributeValueChangeNotificationPackage  PRESENT IF

"the attributeValueChange notifications defined in Recommendation X.721

are supported by an instance of this class.",

"Recommendation M.3100: 1995":processingErrorAlarmPackage  PRESENT IF

"the processingErrorAlarm notifications defined in Recommendation X.721

are supported by an instance of this class.",

communicationsAlarmPackage  PRESENT IF

"the communicationsAlarm notifications defined in Recommendation X.721

are supported by an instance of this class.",

qualityOfServiceAlarmPackage  PRESENT IF

"the qualityOfServiceAlarm notifications defined in Recommendation X.721

are supported by an instance of this class.";

REGISTERED AS {ts32-106-7BCMObjectClass 12};

managementNode MANAGED OBJECT CLASS

DERIVED FROM       "Recommendation X.721: 1992":top;

CHARACTERIZED BY

managementNodeBasicPackage,

managementNodeAssociationPackage;

CONDITIONAL PACKAGES

"Recommendation M.3100: 1995":createDeleteNotificationsPackage  PRESENT IF

"the objectCreation and the objectDeletion defined in Recommendation

X.721 are supported by an instance of this class.",

"Recommendation M.3100: 1995":attributeValueChangeNotificationPackage  PRESENT IF

"the attributeValueChange notifications defined in Recommendation X.721

are supported by an instance of this class.",

"Recommendation M.3100: 1995":processingErrorAlarmPackage  PRESENT IF

"the processingErrorAlarm notifications defined in Recommendation X.721

are supported by an instance of this class.",

"Recommendation M.3100: 1995":environmentalAlarmPackage  PRESENT IF

"the environmentalAlarm notifications defined in Recommendation X.721

are supported by an instance of this class.",

communicationsAlarmPackage  PRESENT IF

"the communicationsAlarm notifications defined in Recommendation X.721

are supported by an instance of this class.",

equipmentAlarmPackage  PRESENT IF

"the equipmentAlarm notifications defined in Recommendation X.721

are supported by an instance of this class.";

REGISTERED AS {ts32-106-7BCMObjectClass 13};

meContext MANAGED OBJECT CLASS

DERIVED FROM       "Recommendation X.721: 1992":top;

CHARACTERIZED BY

meContextBasicPackage;

CONDITIONAL PACKAGES

"Recommendation M.3100: 1995":createDeleteNotificationsPackage  PRESENT IF

"the objectCreation and the objectDeletion defined in Recommendation

X.721 are supported by an instance of this class.";

REGISTERED AS {ts32-106-7BCMObjectClass 14};

mscFunction MANAGED OBJECT CLASS

DERIVED FROM       managedFunction;

CHARACTERIZED BY

mscFunctionBasicPackage PACKAGE

BEHAVIOUR

mscFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"An instance of MOC represents the logical function of a MSC";;

ATTRIBUTES

mscFunctionId GET;;;

REGISTERED AS {ts32-106-7BCMObjectClass 15};

nodeBFunction MANAGED OBJECT CLASS

DERIVED FROM       managedFunction;

CHARACTERIZED BY

nodeBFunctionBasicPackage,

nodeBFunctionAssociationPackage;

REGISTERED AS {ts32-106-7BCMObjectClass 16};

rncFunction MANAGED OBJECT CLASS

DERIVED FROM       managedFunction;

CHARACTERIZED BY

rncFunctionBasicPackage;

REGISTERED AS {ts32-106-7BCMObjectClass 17};

sgsnFunction MANAGED OBJECT CLASS

DERIVED FROM       managedFunction;

CHARACTERIZED BY

sgsnFunctionBasicPackage PACKAGE

BEHAVIOUR

sgsnFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"An instance of MOC represents the logical function of an SGSN";;

ATTRIBUTES

sgsnFunctionId GET;;;

REGISTERED AS {ts32-106-7BCMObjectClass 18};

smsGmscFunction MANAGED OBJECT CLASS

DERIVED FROM       managedFunction;

CHARACTERIZED BY

smsGmscFunctionBasicPackage PACKAGE

BEHAVIOUR

smsGmscFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"An instance of MOC represents the logical function of an smsGMSC";;

ATTRIBUTES

smsGmscFunctionId GET;;;

REGISTERED AS {ts32-106-7BCMObjectClass 19};

smsIwmscFunction MANAGED OBJECT CLASS

DERIVED FROM       managedFunction;

CHARACTERIZED BY

smsIwmscFunctionBasicPackage PACKAGE

BEHAVIOUR

smsIwmscFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"An instance of MOC represents the logical function of an smsIWMSC";;

ATTRIBUTES

smsIwmscFunctionId GET;;;

REGISTERED AS {ts32-106-7BCMObjectClass 20};

utranCell MANAGED OBJECT CLASS

DERIVED FROM       managedFunction;

CHARACTERIZED BY

utranCellBasicPackage,

utranCellIubLinkAssociationPackage;

REGISTERED AS {ts32-106-7BCMObjectClass 21};

vlrFunction MANAGED OBJECT CLASS

DERIVED FROM       managedFunction;

CHARACTERIZED BY

vlrFunctionBasicPackage PACKAGE

BEHAVIOUR

vlrFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"An instance of MOC represents the logical function of a VLR";;

ATTRIBUTES

vlrFunctionId GET;;;

REGISTERED AS {ts32-106-7BCMObjectClass 22};

## Packages

bcmControlBasicPackage PACKAGE

BEHAVIOUR

bcmControlBasicPackageBehaviour;

ATTRIBUTES

bcmControlId GET;

REGISTERED AS {ts32-106-7BCMPackage 1};

bcmIRPVersionPackage PACKAGE

BEHAVIOUR

bcmIRPVersionPackageBehaviour;

ATTRIBUTES

supportedBcmIRPVersions GET;

ACTIONS

getBCmIRPVersion;

REGISTERED AS {ts32-106-7BCMPackage 2};

communicationsAlarmPackage PACKAGE

NOTIFICATIONS

"Recommendation X.721:1992": communicationsAlarm;

REGISTERED AS {ts32-106-7BCMPackage 3};

equipmentAlarmPackage PACKAGE

NOTIFICATIONS

"Recommendation X.721:1992": equipmentAlarm;

REGISTERED AS {ts32-106-7BCMPackage 4};

g3ManagedElementAssociationPackage PACKAGE

BEHAVIOUR

g3ManagedElementAssociationPackageBehaviour;

ATTRIBUTES

meManagedBy GET;

REGISTERED AS {ts32-106-7BCMPackage 5};

g3ManagedElementBasicPackage PACKAGE

BEHAVIOUR

g3ManagedElementBasicPackageBehaviour;

ATTRIBUTES

managedElementId GET,

managedElementType GET,

userDefinedState GET,

"Recommendation X.721: 1992" : systemTitle GET,

"Recommendation M.3100: 1995" : userLabel GET,

"Recommendation M.3100: 1995" : vendorName GET,

"Recommendation M.3100: 1995" : locationName GET;

REGISTERED AS {ts32-106-7BCMPackage 6};

g3SubNetworkBasicPackage PACKAGE

BEHAVIOUR

g3SubNetworkBasicPackageBehaviour;

ATTRIBUTES

g3SubNetworkId GET,

"Recommendation X.721: 1992": systemTitle GET,

"Recommendation M.3100: 1995" : userLabel GET;

REGISTERED AS {ts32-106-7BCMPackage 7};

irpAgentBasicPackage PACKAGE

BEHAVIOUR

irpAgentBasicPackageBehaviour;

ATTRIBUTES

irpAgentId GET,

"Recommendation M.3100: 1995" : userLabel GET,

supportedIRPs GET;

REGISTERED AS {ts32-106-7BCMPackage 8};

iubLinkAssociationPackage PACKAGE

BEHAVIOUR

iubLinkAssociationPackageBehaviour;

ATTRIBUTES

iubLinkNodeBFunctionLink GET,

iubLinkUtranCellLink GET;

REGISTERED AS {ts32-106-7BCMPackage 9};

iubLinkBasicPackage PACKAGE

BEHAVIOUR

iubLinkBasicPackageBehaviour;

ATTRIBUTES

iubLinkId GET;

REGISTERED AS {ts32-106-7BCMPackage 11};

managedFunctionBasicPackage PACKAGE

BEHAVIOUR

managementFunctionBasicPackageBehaviour;

ATTRIBUTES

"Recommendation M.3100: 1995" : userLabel GET;

REGISTERED AS {ts32-106-7BCMPackage 12};

managementNodeAssociationPackage PACKAGE

BEHAVIOUR

managementNodeAssociationPackageBehaviour;

ATTRIBUTES

mnManagesList GET;

REGISTERED AS {ts32-106-7BCMPackage 13};

managementNodeBasicPackage PACKAGE

ATTRIBUTES

managementNodeId GET,

userDefinedState GET,

"Recommendation M.3100: 1995" : userLabel GET,

"Recommendation M.3100: 1995" : vendorName GET,

"Recommendation M.3100: 1995" : locationName GET;

REGISTERED AS {ts32-106-7BCMPackage 14};

meContextBasicPackage PACKAGE

BEHAVIOUR

meContextBasicPackageBehaviour;

ATTRIBUTES

meContextId GET,

"Recommendation X.721: 1992" : systemTitle GET;

REGISTERED AS {ts32-106-7BCMPackage 15};

nodeBFunctionAssociationPackage PACKAGE

BEHAVIOUR

nodeBFunctionAssociationPackageBehaviour;

ATTRIBUTES

nodeBiubLinkLink GET;

REGISTERED AS {ts32-106-7BCMPackage 17};

nodeBFunctionBasicPackage PACKAGE

BEHAVIOUR

nodeBFunctionBasicPackageBehaviour;

ATTRIBUTES

nodeBFunctionId GET;

REGISTERED AS {ts32-106-7BCMPackage 18};

qualityOfServiceAlarmPackage PACKAGE

NOTIFICATIONS

"Recommendation X.721:1992": qualityofServiceAlarm;

REGISTERED AS {ts32-106-7BCMPackage 19};

rncFunctionBasicPackage PACKAGE

BEHAVIOUR

rncFunctionBasicPackageBehaviour;

ATTRIBUTES

rncFunctionId GET;

REGISTERED AS {ts32-106-7BCMPackage 20};

utranCellIubLinkAssociationPackage PACKAGE

BEHAVIOUR

utranCellIubLinkAssociationPackageBehaviour;

ATTRIBUTES

utranCelliubLinkLink GET;

REGISTERED AS {ts32-106-7BCMPackage 21};

utranCellBasicPackage PACKAGE

BEHAVIOUR

utranCellBasicPackageBehaviour;

ATTRIBUTES

utranCellId GET;

REGISTERED AS {ts32-106-7BCMPackage 23};

## 5.3	Actions

getBCmIRPVersion ACTION

BEHAVIOUR

getBCmIRPVersionBehaviour;

MODE CONFIRMED;

WITH REPLY SYNTAX TS32-106-7TypeModule.GetBCmIRPVersionReply;

REGISTERED AS {ts32-106-7BCMAction 1};

## 5.4	Attributes

aucFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

aucFunctionIdBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 1};

bcmControlId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

bcmControlIdBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 2};

bgFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

bgFunctionIdBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 3};

eirFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

eirFunctionIdBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 4};

ggsnFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

ggsnFunctionIdBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 5};

gmscFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

gmscFunctionIdBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 6};

g3SubNetworkId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

g3SubNetworkIdBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 7};

hlrFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

hlrFunctionIdBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 8};

irpAgentId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

irpAgentIdBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 9};

iubLinkNodeBFunctionLink ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectPointer;

MATCHES FOR EQUALITY;

BEHAVIOUR

iubLinkNodeBFunctionLinkBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 10};

iubLinkUtranCellLink  ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectPointerList;

MATCHES FOR EQUALITY;

BEHAVIOUR

iubLinkUtranCellLinkBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 11};

iubLinkId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

iubLinkIdBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 12};

managedElementId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

managedElementIdBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 13};

managedElementType ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .ManagedElementType;

MATCHES FOR EQUALITY;

BEHAVIOUR

managedElementBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 14};

managementNodeId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

managmentNodeIdBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 15};

meContextId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

meContextIdBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 16};

meManagedBy ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectPointer;

MATCHES FOR EQUALITY;

BEHAVIOUR

meManagedByBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 17};

mnManagesList ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectPointerList;

MATCHES FOR EQUALITY;

BEHAVIOUR

mnManagesListBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 18};

mscFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

mscFunctionIdBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 19};

nodeBiubLinkLink ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectPointer;

MATCHES FOR EQUALITY;

BEHAVIOUR

nodeBiubLinkLinkBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 20};

nodeBFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

nodeBFunctionIdBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 22};

rncFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

rncFunctionIdBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 23};

sgsnFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

sgsnFunctionIdBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 24};

smsIwmscFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

smsIwmscFunctionIdBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 25};

smsGmscFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

smsGmscFunctionIdBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 26};

supportedBcmIRPVersions ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .SupportedBCmIRPVersions;

MATCHES FOR EQUALITY;

BEHAVIOUR

supportedBCmIRPVersionsBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 27};

supportedIRPs ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .SupportedIRPs;

MATCHES FOR EQUALITY;

BEHAVIOUR

supportedIRPsBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 28};

vlrFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

vlrFunctionIdBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 29};

utranCelliubLinkLink ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectPointer;

MATCHES FOR EQUALITY;

BEHAVIOUR

utranCelliubLinkLinkBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 30};

userDefinedState ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .UserDefinedState;

MATCHES FOR EQUALITY;

BEHAVIOUR

userDefinedStateBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 32};

utranCellId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule .GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

utranCellIdBehaviour;

REGISTERED AS {ts32-106-7BCMAttribute 33};

## 5.5	Name Bindings

alarmControl-irpAgent NAME BINDING

SUBORDINATE OBJECT CLASS "3GPP TS 32.111-4":alarmControl;

NAMED BY SUPERIOR OBJECT CLASS irpAgent;

WITH ATTRIBUTE "3GPP TS 32.111-4":alarmControlId;

BEHAVIOUR

alarmControl-irpAgentBehavior;

CREATE WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-106-7BCMNameBinding 1};

aucFunction-g3ManagedElement NAME BINDING

SUBORDINATE OBJECT CLASS aucFunction;

NAMED BY SUPERIOR OBJECT CLASS g3ManagedElement;

WITH ATTRIBUTE aucFunctionId;

BEHAVIOUR

aucFunction-g3ManagedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-106-7BCMNameBinding 2};

bcmControl-irpAgent NAME BINDING

SUBORDINATE OBJECT CLASS bcmControl;

NAMED BY SUPERIOR OBJECT CLASS irpAgent;

WITH ATTRIBUTE bcmControlId;

BEHAVIOUR

bcmControl-irpAgentBehavior;

CREATE WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-106-7BCMNameBinding 3};

bsFunction-g3ManagedElement NAME BINDING

SUBORDINATE OBJECT CLASS bgFunction;

NAMED BY SUPERIOR OBJECT CLASS g3ManagedElement;

WITH ATTRIBUTE bgFunctionId;

BEHAVIOUR

bgFunction-g3ManagedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-106-7BCMNameBinding 4};

eirFunction-g3ManagedElement NAME BINDING

SUBORDINATE OBJECT CLASS eirFunction;

NAMED BY SUPERIOR OBJECT CLASS g3ManagedElement;

WITH ATTRIBUTE eirFunctionId;

BEHAVIOUR

eirFunction-g3ManagedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-106-7BCMNameBinding 5};

ggsnFunction-g3ManagedElement NAME BINDING

SUBORDINATE OBJECT CLASS ggsnFunction;

NAMED BY SUPERIOR OBJECT CLASS g3ManagedElement;

WITH ATTRIBUTE ggsnFunctionId;

BEHAVIOUR

ggsnFunction-g3ManagedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-106-7BCMNameBinding 6};

gmscFunction-g3ManagedElement NAME BINDING

SUBORDINATE OBJECT CLASS mscFunction;

NAMED BY SUPERIOR OBJECT CLASS g3ManagedElement;

WITH ATTRIBUTE gmscFunctionId;

BEHAVIOUR

gmscFunction-g3ManagedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-106-7BCMNameBinding 7};

g3ManagedElement-meContext NAME BINDING

SUBORDINATE OBJECT CLASS g3ManagedElement;

NAMED BY SUPERIOR OBJECT CLASS meContext;

WITH ATTRIBUTE managedElementId;

BEHAVIOUR

g3ManagedElement-meContextBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-106-7BCMNameBinding 8};

g3ManagedElement-g3SubNetwork NAME BINDING

SUBORDINATE OBJECT CLASS g3ManagedElement;

NAMED BY SUPERIOR OBJECT CLASS g3SubNetwork;

WITH ATTRIBUTE managedElementId;

BEHAVIOUR

g3ManagedElement-g3SubNetworkBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-106-7BCMNameBinding 9};

hlrFunction-g3ManagedElement NAME BINDING

SUBORDINATE OBJECT CLASS hlrFunction;

NAMED BY SUPERIOR OBJECT CLASS g3ManagedElement;

WITH ATTRIBUTE hlrFunctionId;

BEHAVIOUR

hlrFunction-g3ManagedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-106-7BCMNameBinding 10};

irpAgent-g3ManagedElement NAME BINDING

SUBORDINATE OBJECT CLASS irpAgent;

NAMED BY SUPERIOR OBJECT CLASS g3ManagedElement;

WITH ATTRIBUTE irpAgentId;

BEHAVIOUR

irpAgent-g3ManagedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-106-7BCMNameBinding 11};

irpAgent-g3SubNetwork NAME BINDING

SUBORDINATE OBJECT CLASS irpAgent;

NAMED BY SUPERIOR OBJECT CLASS g3SubNetwork;

WITH ATTRIBUTE irpAgentId;

BEHAVIOUR

irpAgent-g3SubNetworkBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-106-7BCMNameBinding 12};

irpAgent-managementNode NAME BINDING

SUBORDINATE OBJECT CLASS irpAgent;

NAMED BY SUPERIOR OBJECT CLASS managementNode;

WITH ATTRIBUTE irpAgentId;

BEHAVIOUR

irpAgent-managementNodeBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-106-7BCMNameBinding 13};

iubLink-rncFunction NAME BINDING

SUBORDINATE OBJECT CLASS iubLink;

NAMED BY SUPERIOR OBJECT CLASS rncFunction;

WITH ATTRIBUTE iubLinkId;

BEHAVIOUR

iubLink-rncFunctionBahaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-106-7BCMNameBinding 14};

managementNode-g3SubNetwork NAME BINDING

SUBORDINATE OBJECT CLASS managementNode;

NAMED BY SUPERIOR OBJECT CLASS g3SubNetwork;

WITH ATTRIBUTE managementNodeId;

BEHAVIOUR

managementNode-g3SubNetworkBahaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-106-7BCMNameBinding 15};

meContext-g3SubNetwork NAME BINDING

SUBORDINATE OBJECT CLASS meContext;

NAMED BY SUPERIOR OBJECT CLASS g3SubNetwork;

WITH ATTRIBUTE meContextId;

BEHAVIOUR

meContext-g3SubNetworkBahaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-106-7BCMNameBinding 16};

mscFunction-g3ManagedElement NAME BINDING

SUBORDINATE OBJECT CLASS mscFunction;

NAMED BY SUPERIOR OBJECT CLASS g3ManagedElement;

WITH ATTRIBUTE mscFunctionId;

BEHAVIOUR

mscFunction-g3ManagedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-106-7BCMNameBinding 17};

nodeBFunction-g3ManagedElement NAME BINDING

SUBORDINATE OBJECT CLASS nodeBFunction;

NAMED BY SUPERIOR OBJECT CLASS g3ManagedElement;

WITH ATTRIBUTE nodeBFunctionId;

BEHAVIOUR

nodeBFunction-g3ManagedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-106-7BCMNameBinding 18};

notificationControl-irpAgent NAME BINDING

SUBORDINATE OBJECT CLASS "3GPP TS 32.106-4":notificationControl;

NAMED BY SUPERIOR OBJECT CLASS irpAgent;

WITH ATTRIBUTE "3GPP TS 32.106-4":notificationControlId;

BEHAVIOUR

notificationControl-irpAgentBehavior;

CREATE WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-106-7BCMNameBinding 19};

rncFunction-g3ManagedElement NAME BINDING

SUBORDINATE OBJECT CLASS rncFunction;

NAMED BY SUPERIOR OBJECT CLASS g3ManagedElement;

WITH ATTRIBUTE rncFunctionId;

BEHAVIOUR

rncFunction-g3ManagedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-106-7BCMNameBinding 20};

sgsnFunction-g3ManagedElement NAME BINDING

SUBORDINATE OBJECT CLASS sgsnFunction;

NAMED BY SUPERIOR OBJECT CLASS g3ManagedElement;

WITH ATTRIBUTE sgsnFunctionId;

BEHAVIOUR

sgsnFunction-g3ManagedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-106-7BCMNameBinding 21};

smsGmscFunction-g3ManagedElement NAME BINDING

SUBORDINATE OBJECT CLASS smsGmscFunction;

NAMED BY SUPERIOR OBJECT CLASS g3ManagedElement;

WITH ATTRIBUTE smsGmscFunctionId;

BEHAVIOUR

smsGmscFunction-g3ManagedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-106-7BCMNameBinding 22};

smsIwmscFunction-g3ManagedElement NAME BINDING

SUBORDINATE OBJECT CLASS smsIwmscFunction;

NAMED BY SUPERIOR OBJECT CLASS g3ManagedElement;

WITH ATTRIBUTE smsIwmscFunctionId;

BEHAVIOUR

smsIwmscFunction-g3ManagedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-106-7BCMNameBinding 23};

utranCell-rncFunction NAME BINDING

SUBORDINATE OBJECT CLASS utranCell;

NAMED BY SUPERIOR OBJECT CLASS rncFunction;

WITH ATTRIBUTE utranCellId;

BEHAVIOUR

utranCell-rncFunctionBahaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-106-7BCMNameBinding 24};

vlrFunction-g3ManagedElement NAME BINDING

SUBORDINATE OBJECT CLASS vlrFunction;

NAMED BY SUPERIOR OBJECT CLASS g3ManagedElement;

WITH ATTRIBUTE vlrFunctionId;

BEHAVIOUR

vlrFunction-g3ManagedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-106-7BCMNameBinding 25};

## Behaviours

alarmControl-irpAgentBehavior BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a irpAgent contains and

controls an alarmControl. When automatic instance naming is used, the choice

of name bindings left as a local matter.";

aucFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a aucFunction instance.";

aucFunction-g3ManagedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a 3gManagedElement contains

and controls a aucFunction. When automatic instance naming is used, the choice

of name bindings left as a local matter.";

bcmControlBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The object class bcmControl offers the functions defined in the CM IRP IS

enabling to control the behaviour and to retrieve the management information

related a Basic CM IRP agent.

An instance of the 'BCmControl' MOC is identified by the value of the attribute

'bcmControlId'.";

bcmControlIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute names an instance of the 'bcmControl' object class.";

bcmIRPVersionPackageBehaviour BEHAVIOUR

DEFINED AS

"This package has been defined to allow the Manager to get information about the

Basic CM IRP versions supported by the Agent.

The attribute 'supportedBCmIRPVersions' indicates all versions of the Basic IRP

currently supported by the Agent. .

With the action 'getBasicCmIRPVersion' a manager can find out the versions of

the Basic CM IRP CMIP solution sets the Agent supports.";

bcmControl-irpAgentBehavior BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a irpAgent contains and

controls an bcmControl. When automatic instance naming is used, the choice

of name bindings left as a local matter.";

bgFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a bgFunction instance.";

bgFunction-g3ManagedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a 3gManagedElement contains

and controls a bgFunction. When automatic instance naming is used, the choice

of name bindings left as a local matter.";

eirFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a eirFunction instance.";

eirFunction-g3ManagedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a 3gManagedElement contains

and controls a eirFunction. When automatic instance naming is used, the choice

of name bindings left as a local matter.";

getBCmIRPVersionBehaviour BEHAVIOUR

DEFINED AS

"A Manager invokes this action to enquiry about the versions of the Basic CM IRP

CMIP solution set which the concerned Agent supports.

The 'Action information' field contains no data:

The 'Action response' is composed of the following data:

* versionNumbersList It contains a list of versions supported by the concerned

agent which are backwards compatible. A list containing no element, i.e. a NULL

list means that the concerned agent doesn't support any version of the

Notification IRP.

* status It contains the results of this action. Possible values: noError (0),

error (the value indicates the reason of the error).";

ggsnFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a ggsnFunction instance.";

ggsnFunction-g3ManagedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a 3gManagedElement contains

and controls a ggsnFunction. When automatic instance naming is used, the choice

of name bindings left as a local matter.";

g3ManagedElement-meContextBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a meContext contains and

controls a 3gManagedElement. When automatic instance naming is used, the choice

of name bindings left as a local matter.";

g3ManagedElement-g3SubNetworkBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a 3gMnetwork contains and

controls a 3gManagedElement. When automatic instance naming is used, the choice

of name bindings left as a local matter.";

g3ManagedElementAssociationPackageBehaviour BEHAVIOUR

DEFINED AS

"The attribute 'meManagedBy' points to the g3ManagmentNode instance which

manages this g3ManagedElement instance. It implements the attribute managedBy

of MOC G3ManagedElement defined in TS32.106-5.";

g3ManagedElementBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"This managed object class represents telecommunications equipment within the

telecommunications network that performs managed element functions, i.e.

provides support and/or service to the subscriber. A managed element

communicates with a manager (directly or indirectly) over one or more standard

interfaces for the purpose of being monitored and/or controlled. A managed

element contains equipment that may or may not be geographically distributed. A

Managed Element is often referred to as a 'node' or a 'network element'.";

g3SubNetworkBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"This managed object class represents collections of interconnected

telecommunications and management objects (logical or physical) capable of

exchanging information. A network may be nested within another (larger) network,

thereby forming a containment relationship.";

g3SubNetworkIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute names an instance of the '3gSubNetwork' object class.";

gmscFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a gmscFunction instance.";

gmscFunction-g3ManagedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a 3gManagedElement contains

and controls a gmscFunction. When automatic instance naming is used, the choice

of name bindings left as a local matter.";

hlrFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a hlrFunction instance.";

hlrFunction-g3ManagedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a 3gManagedElement contains

and controls a hlrFunction. When automatic instance naming is used, the choice

of name bindings left as a local matter.";

irpAgentBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"This MOC may have only one instance in R99. The instance of this MOC represents

the behavior of an IRP Agent which implements one or more IRPs, e.g. AlarmIRP

defined in 3GPP TS 32.111-2, NotificationIRP defined in TS32.106-2 and Basic CM IRP

defined in 3GPP TS 32.106-5";

irpAgentIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies an irpAgent instance.";

irpAgent-g3SubNetworkBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a g3SubNetwork contains and

controls a irpAgent. When automatic instance naming is used, the choice of name

bindings left as a local matter.";

irpAgent-g3ManagedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a g3ManagedElement contains and

controls a irpAgent. When automatic instance naming is used, the choice of name

bindings left as a local matter.";

irpAgent-managementNodeBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managementNode contains and

controls a irpAgent. When automatic instance naming is used, the choice of name

bindings left as a local matter.";

iubLinkNodeBFunctionLinkBehaviour BEHAVIOUR

DEFINED AS

"This attribute points to the nodeBFunction instance which this iubLink instance

connects directly to.";

iubLink-rncFunctionBahaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a rncFunction contains and

controls a iubLink. When automatic instance naming is used, the choice of name

bindings left as a local matter.";

iubLinkAssociationPackageBehaviour BEHAVIOUR

DEFINED AS

"The attribute 'iubLinkNodeBFunctionLink' points to the nodeBFunction instance which this

iubLink instance connects to. The attribute ‘iubLinkUtranCellLink’ points from an

iubLink to a list of utranCell.";

iubLinkBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"This managed object class models the Iub Link between a Node-B and a RNC.";

iubLinkIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute names an instance of the 'iubLink' object class.";

iubLinkUtranCellLinkBehaviour BEHAVIOUR

DEFINED AS

"This attribute points from an iubLink instance to a list of utranCell instance";

managedElementBehaviour BEHAVIOUR

DEFINED AS

"This is a multi-value attribute specifying the sub-classes of managedFunction

this managedElement instance is containing, for instances, RNC, NodeB or

RNC+NodeB.";

managedElementIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute names an instance of the '3gManagedElement' object class.";

managementFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"This Managed Object class corresponds to the class gsmManagedFunction defined

in GSM 12.20 0 and is provided for sub-classing only. It provides the attributes

that are common to functional MO classes. Note that a managed element may

contain several managed functions. The ManagedFunction may be extended in the

future if more common characteristics to functional objects are identified.";

managementNode-g3SubNetworkBahaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a 3gSubNetwork contains and

controls a managementNode. When automatic instance naming is used, the choice of

name bindings left as a local matter.";

managementNodeAssociationPackageBehaviour BEHAVIOUR

DEFINED AS

"The attribute 'mnManagesList' points to all g3ManagedElement instances which

this managementNode instance manages. It implements the attribute manages of

MOC ManagementNode defined in TS32.106-5.";

managementNodeBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"This managed object class represents a telecommunications management system (EM

or NM) within the TMN, that manages a number of Managed Elements. The management

system communicates with the MEs directly or indirectly over one or more

standard interfaces for the purpose of monitoring and/or controlling these MEs.";

managmentNodeIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute names an instance of the 'managmentNode' object class.";

meContext-g3SubNetworkBahaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a 3gSubnetwork contains and

controls a meContext. When automatic instance naming is used, the choice of name

bindings left as a local matter.";

meContextBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"This managed object class represents the Managed Element from the network

perspective. It can be used to hold surveillance status information, and also

planning status information for the case when the managed element is part of a

planned configuration in a management system, before it has been taken into

service. It can also support unambiguous naming in all cases, also for scenarios

when the Managed Elements have been pre-configured where some of them may have

equal names (to avoid necessary administration to make all of them globally

unique at creation/installation time). Thus, by means of globally unique names

for the MEContext instances, and by using these in the DN, the DNs for all MEs

(and MOIs contained in them) can be assured to be globally unique, even in such

a scenario as described above.";

meContextIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute names an instance of the 'MEContext' object class.";

meManagedByBehaviour BEHAVIOUR

DEFINED AS

"This attribute points to the managementNode instance which manages the

related 3gManagedElement instance.";

mnManagesListBehaviour BEHAVIOUR

DEFINED AS

"This attribute points to all 3gManagedElement instances which this

3gManagmentNode instance manages.";

mscFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a mscFunction instance.";

mscFunction-g3ManagedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a 3gManagedElement contains

and controls a mscFunction. When automatic instance naming is used, the choice

of name bindings left as a local matter.";

nodeBiubLinkLinkBehaviour BEHAVIOUR

DEFINED AS

"This attribute points to the IubLink instance which connectes to the related

nodeBFunction instance directly.";

nodeBFunction-g3ManagedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a 3gManagedElement contains

and controls a nodeBFunction. When automatic instance naming is used, the choice

of name bindings left as a local matter.";

nodeBFunctionAssociationPackageBehaviour BEHAVIOUR

DEFINED AS

"The attribute 'nodeBiubLinkLink' points to the iubLink instance which connectes

to this nodeBFunction instance directly. It implements the attribute nodeB-IubLink

of MOC NodeBFunction defined in TS32.106-5.";

nodeBFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"This managed object class represents the NodeB functionality.";

nodeBFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute names an instance of the 'nodeBFunction' object class.";

notificationControl-irpAgentBehavior BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a irpAgent contains and

controls an otificationControl. When automatic instance naming is used, the choice

of name bindings left as a local matter.";

rncFunction-g3ManagedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a 3gManagedElement contains

and controls a rncFunction. When automatic instance naming is used, the choice

of name bindings left as a local matter.";

rncFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"This MOC represents RNC function.";

rncFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute names an instance of the 'rncFunction' object class.";

sgsnFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a sgsnFunction instance.";

sgsnFunction-g3ManagedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a 3gManagedElement contains

and controls a sgsnFunction. When automatic instance naming is used, the choice

of name bindings left as a local matter.";

smsGmscFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a smsGmscFunction instance.";

smsGmscFunction-g3ManagedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a 3gManagedElement contains

and controls a smsGmscFunction. When automatic instance naming is used, the choice

of name bindings left as a local matter.";

smsIwmscFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a iwmscFunction instance.";

smsIwmscFunction-g3ManagedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a 3gManagedElement contains

and controls a smsIwmscFunction. When automatic instance naming is used, the choice

of name bindings left as a local matter.";

supportedBCmIRPVersionsBehaviour BEHAVIOUR

DEFINED AS

"This attribute provides the information concerning the Basic CM IRP versions

currently supported by the Agent.";

supportedIRPsBehaviour BEHAVIOUR

DEFINED AS

"This attribute provides the information about IRPs an IRPAgent supports.";

utranCelliubLinkLinkBehaviour BEHAVIOUR

DEFINED AS

"This attribute implements the attribute UtranCell-IubLink of MOC UtranCell

Defined in TS32.106-5.";

userDefinedStateBehaviour BEHAVIOUR

DEFINED AS

"This attribute specifies an operator defined state for operator specific usage.";

utranCell-rncFunctionBahaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a rncFunction contains and

controls a utranCell. When automatic instance naming is used, the choice of name

bindings left as a local matter.";

utranCellIubLinkAssociationPackageBehaviour BEHAVIOUR

DEFINED AS

"The attribute 'utranCelliubLinkLink' points to the iubLink instance which

connectes to this utranCell instance.";

utranCellNodeBAssociationPackageBehaviour BEHAVIOUR

DEFINED AS

"The attribute 'utranCellNodeBLink' points to the nodeBFunction instance which

connectes to this utranCell instance.";

utranCellBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"This managed object class represents the radio cell controlled by the RNC.";

utranCellIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute names an instance of the 'utranCell' object class.";

vlrFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a vlrFunction instance.";

vlrFunction-g3ManagedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a 3gManagedElement contains

and controls a vlrFunction. When automatic instance naming is used, the choice

of name bindings left as a local matter.";

# 6.	ASN.1 Definitions

TS32-106-7TypeModule {ccitt (0) identified-organization (4) etsi (0)

mobileDomain (0) umts-Operation-Maintenance (3) ts-32-106 (106) part7 (7)

informationModel (0) asn1Module (2) version1 (1)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

--EXPORTS everything

IMPORTS

ObjectInstance FROM CMIP-1 {joint-iso-ccitt ms(9) cmip(1) modules(0) protocol(3)};

-- 3GPP TS 32.106-7 related Object Identifiers

baseNodeUMTS  OBJECT IDENTIFIER ::= {itu-t(0) identified-organization(4) etsi(0) mobileDomain(0) 				      				umts-Operation-Maintenance(3)}

ts32-106Prefix		OBJECT IDENTIFIER ::= { baseNodeUMTS ts-32-106(106)}

ts32-106Part7		OBJECT IDENTIFIER ::= { ts32-106Prefix	part7(7)}

ts32-106-7InfoModel	OBJECT IDENTIFIER ::= { ts32-106Part7 informationModel(0)}

ts32-106-7BCMObjectClass  	OBJECT IDENTIFIER ::= { ts32-106-7InfoModel managedObjectClass(3)}

ts32-106-7BCMPackage		OBJECT IDENTIFIER ::= { ts32-106-7InfoModel package(4)}

ts32-106-7BCMParameter		OBJECT IDENTIFIER ::= { ts32-106-7InfoModel parameter(5)}

ts32-106-7BCMNameBinding  	OBJECT IDENTIFIER ::= { ts32-106-7InfoModel nameBinding(6)}

ts32-106-7BCMAttribute		OBJECT IDENTIFIER ::= { ts32-106-7InfoModel attribute(7)}

ts32-106-7BCMAction		OBJECT IDENTIFIER ::= { ts32-106-7InfoModel action(9)}

ts32-106-7BCMNotification	OBJECT IDENTIFIER ::= { ts32-106-7InfoModel notification(10)}

-- Start of 3gPP SA5 own definitions

ErrorCauses ::= ENUMERATED

{

noError (0),

wrongInput (1),

unspecifiedErrorReason (255)

}

GeneralObjectId ::= INTEGER

GeneralObjectPointer ::= ObjectInstance

GeneralObjectPointerList ::= SEQUENCE OF ObjectInstance

GeneralUserLable ::= GraphicString

GetBCmIRPVersionReply ::= SEQUENCE

{

versionNumbersList		SupportedBCmIRPVersions,

status				ErrorCauses

}

IRPNames::= SET OF ENUMERATED

{

notificationIRP	(1),

alarmIRP		(2),

basicCMIRP	(3)

}

IRPVersionNumber ::= GraphicString

ManagedElementType::= SET OF ENUMERATED

{

rnc		(1),

nodeB		(2),

msc		(3),

hLR		(4),

vLR		(5),

aUC		(6),

eIR		(7),

sms-IWNSC	(8),

sms-GMSC	(9),

sGSN		(10),

gGSN		(11),

bG		(12),

gmsc		(13)

}

SupportedBCmIRPVersions ::= SET OF IRPVersionNumber

SupportedIRPs ::= SET OF IRPNames

UserDefinedState ::= INTEGER

END	-- of TS32-106-7TypeModule

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| Dec 2000 | S_10 | SP-000515 | -- | -- | Approved at TSG SA #10 and placed under Change Control. | 1.0.0 | 3.0.0 |
| Mar 2001 | S_11 | SP-010031 | 001 | -- | Making 32106-7 (CMIP SS) compliant to 32106-5 (IS/IM) | 3.0.0 | 3.1.0 |
| Mar 2001 | S_11 | SP-010031 | 002 | -- | Update 32.106-7 based on CR 001 of 32.106-5 (S5-010133) | 3.0.0 | 3.1.0 |
| Jun 2001 | S_12 | SP-010284 | 003 | -- | Correction of UTRAN attributes | 3.1.0 | 3.2.0 |
| Dec 2001 | S_14 | SP-010636 | 004 | -- | Correction of improper module name in GDMO definition | 3.2.0 | 3.3.0 |