3GPP TS 32.324 V6.0.0 (2004-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Test management Integration Reference Point (IRP);

Common Management Information Protocol (CMIP)

Solution Set (SS)

(Release 6)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

UMTS, management

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2004, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	Basic aspects	6

4.1	Architectural aspects	6

4.2	Mapping	6

4.2.1	Mapping of Information Object Classes (IOCs)	6

4.2.2	Mapping of IOC attributes	7

4.2.3	Mapping of operations	7

4.2.4	Mapping of Operation Parameters	7

4.2.4.1	Parameter Mapping of the Operation initiateTest	7

4.2.4.2	Parameter Mapping of the Operation terminateTest	8

4.2.4.3	Parameter Mapping of the Operation monitorTest	9

4.2.5	Mapping of Notifications	9

4.2.6	Mapping of Notification Parameters	10

4.2.6.1	Parameter Mapping of the Notification notifyTestResults	10

5	GDMO Definitions	11

5.1	Managed Object Classes	11

5.1.1	testManagementIRP	11

5.2	Packages	11

5.2.1	testManagementIRPIdPackage	11

5.2.2	testManagementIRPVersionPackage	11

5.2.3	testManagementIRPProfilePackage	12

5.3	Actions	12

5.3.1	getTestManagementIRPVersion (M)	12

5.3.2	getTestManagementIRPOperationProfile (O)	12

5.3.3	getTestManagementIRPNotificationProfile (O)	13

5.4	Attributes	13

5.4.1	testManagementIRPId	13

5.4.2	supportedTestManagementIRPVersions	13

5.5	Parameters	14

5.5.1	fileReference	14

5.5.2	fileExpiryDate	14

6	ASN.1 Definitions	14

Annex A (informative):	Change history	16

# Foreword

This Technical Specification (TS) has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; as identified below:

32.321:	"Test management Integration Reference Point (IRP); Requirements";

32.322:	"Test management Integration Reference Point (IRP): Information Service (IS)";

32.323:	"Test management Integration Reference Point (IRP): Common Object Request Broker Architecture (CORBA) Solution Set (SS)";

32.324:	"Test management Integration Reference Point (IRP): Common Management Information Protocol (CMIP) Solution Set (SS)".

A 3G telecommunication network is composed of a multitude of different Network Elements (NE). For a successful operation of the network the operator must be provided with mechanisms allowing him to manage the network. These management activities can be grouped into several areas: configuration management, fault management, performance management, accounting management and security management.

A management function assisting in different high level management areas such as fault management and performance management is test management. The purpose of testing is to get information about the functionality and performance of the 3G managed network subject to the test.

The present document is part of a TS-family defining defining the Telecommunication Management (TM) of 3G systems. The TM principles are described in 3GPP TS 32.101 [5]. The TM architecture is described in 3GPP TS 32.102 [6]. The other specifications define the interface (Itf-N) between the managing system (manager), which is in general the Network Manager (NM) and the managed system (agent), which is either an Element Manager (EM) or the managed NE itself. The Itf-N is composed of a number of Integration Reference Points (IRPs) defining the information in the agent that is visible for the manager, the operations that the manager may perform on this information and the notifications that are sent from the agent to the manager. One of these IRPs is the Test IRP.

Each IRP is specified by the requirements part, the Information Service part, the CORBA SS and the CMIP SS.

# 1	Scope

The present document specifies the CMIP SS for the Test Management IRP IS defined in 3GPP TS 32.322 [8]. 
In detail:

- Clause 4 provides the basic architectural concept of the CMIP SS and the mapping between the IOCs, operations and notifications defined in 3GPP TS 32.322 [8] to the corresponding CMIP SS equivalents.

- Clause 5 contains the GDMO definitions for the Test Management IRP over the CMIP interfaces.

- Clause 6 contains the ASN.1 definitions supporting the GDMO definitions provided in clause 5.

This Solution Set specification is related to 3GPP TS 32.322 (V6.0.X).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[4]	3GPP TS 32.301: "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP): Requirements".

[5]	3GPP TS 32.304: "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP): Common Management Information Protocol (CMIP) Solution Set (SS)".

[6]	3GPP TS 32.312: "Telecommunication management; Generic Integration Reference Point (IRP) management; Information Service (IS)".

[7]	3GPP TS 32.321: "Telecommunication management; Test management Integration Reference Point (IRP): Requirements".

[8]	3GPP TS 32.322: "Telecommunication management; Test management Integration Reference Point (IRP): Information Service (IS)".

[9]	ITU-T Recommendation X.710: "Information technology - Open Systems Interconnection - Common Management Information Service".

[10]	ITU-T Recommendation X.745:"Information technology - Open Systems Interconnection - Systems Management: Test management function".

[11]	ITU-T Recommendation X.737: "Information technology; Open Systems Interconnection; Systems management: Confidence and diagnostic test categories".

[12]	ITU-T Recommendation X.721: "Information technology - Open Systems Interconnection - Structure of management information: Definition of management information".

[13]	ISO/IEC 10165-2: "Information technology - Open Systems Interconnection - Structure of management information: Definition of management information".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TS 32.101 [1], 3GPP TS 32.102 [2] and 3GPP TS 32.321 [7] apply.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

ASN.1	Abstract Syntax Notation One

CMISE	Common Management Information Service

CMIP	Common Management Information Protocol

CORBA	Common Object Request Broker Architecture

EM	Element Manager

GDMO	Guidelines for the Definition of Managed Objects

GSM	Global System for Mobile communications

IOC	Information Object Class

IS	Information Service

MOC	Managed Object Class

NE	Network Element

NM	Network Manager

SS	Solution Set

TO	Test Object

# 4	Basic aspects

## 4.1	Architectural aspects

The architecture of the Test Management IRP CMIP Solution Set is based on the test management function defined in ITU-T Recommendation X.745 [10].

## 4.2	Mapping

The semantics of the Test Management IRP are defined in 3GPP TS 32.322 [8]. The definitions of the management information defined there are independent of any implementation technology and protocol. This clause maps these protocol independent definitions onto their equivalents of the CMIP SS of the Test Management IRP.

### 4.2.1	Mapping of Information Object Classes (IOCs)

Table 1 maps the IOCs defined in 3GPP TS 32.322 [8] to the corresponding Managed Object Classes (MOCs) defined in this CMIP SS. The MOCs are qualified either as Mandatory (M) or Optional (O).

Table 1: Mapping of IOCs

| IS IOC | MOC of the CMIP SS | Qualifier |
| --- | --- | --- |
| TestManagementIRP | testManagementIRP | M |
| TestActionPerformer | testActionPerformer (ITU-T Recommendation X.745 [10]) | M |
| TesterObject | testObject (ITU-T Recommendation X.745 [10]) | M |
| TestInvocation | testObject (ITU-T Recommendation X.745 [10]) | M |
| ResourceSelfTestTesterObject | resourceSelfTestObject (ITU-T Recommendation X.737 [11]) | M |

### 4.2.2	Mapping of IOC attributes

Table 2 maps the attributes defined in 3GPP TS 32.322 [8] to the corresponding attributes of the CMIP SS. 
Only attributes, that are qualified as public in the IS, require a corresponding attribute in the CMIP SS.

Table 2: Mapping of attributes

| IS Attribute | Attribute of the CMIP SS | Qualifier |
| --- | --- | --- |
| supportedTOClasses | supportedTOClasses (ITU-T Recommendation X.745 [10]) | M |
| testActionPerformerId | testActionPerformerId (ITU-T Recommendation X.745 [10]) | M |
| testOutcome | testOutcome (ITU-T Recommendation X.745 [10]) | M |
| testState | testState (ITU-T Recommendation X.745 [10]) | M |
| testerObjectId | testObjectId (ITU-T Recommendation X.745 [10]) | M |
| actualStartTime | actualStartTime (ITU-T Recommendation X.745 [10]) | O |
| actualStopTime | actualStopTime (ITU-T Recommendation X.745 [10]) | O |
| testInvocationId | testInvocationId (ITU-T Recommendation X.745 [10]) | M |

### 4.2.3	Mapping of operations

Table 3 and table 4 map the operations defined in 3GPP TS 32.322 [8] and 3GPP TS 32.312 [6] to corresponding GDMO actions and CMISE services. The operations are qualified either as Mandatory (M) or Optional (O).

The CMISE services are defined in ITU-T Recommendation X.710 [9].

Table 3: Mapping of operations of the Test Management IRP: IS

| Interface | Operation | GDMO Action or CMISE of CMIP SS | Qualifier |
| --- | --- | --- | --- |
| TestManagementIRPControlOperations | initiateTest | testRequestControlledAction (ITU-T Recommendation X.745 [10]) | M |
|  | terminateTest | testTerminateAction (ITU-T Recommendation X.745 [10]) | M |
| TestIManagementRPMonitorOperations | monitorTest | M-GET (CMISE) | M |

Table 4: Mapping of operations inherited from the Generic IRP Management: IS

| Interface | Operation | GDMO Action or CMISE of CMIP SS | Qualifier |
| --- | --- | --- | --- |
| GenericIRPVersionsOperations | getIRPVersion | getTestManagementIRPVersion | M |
| GenericIRPProfileOperations | getOperationProfile | getTestManagementIRPOperationProfile | O |
|  | getNotificationProfile | getTestManagementIRPNotificationProfile | O |

### 4.2.4	Mapping of Operation Parameters

The tables in the following clauses list the parameters of each operation defined in 3GPP TS 32.322 [8] and their equivalents in the CMIP SS.

#### 4.2.4.1	Parameter Mapping of the Operation initiateTest

The operation initiateTest is mapped to the GDMO action testRequestControlledAction defined in ITU-T Recommendation X.745 [10]. This action shall be implemented using the CMISE M-ACTION service.

All input parameters are mapped to the M-ACTION request parameter 'Action information'. The syntax and semantics of this parameter is specified in ITU-T Recommendation X.745 [10] for the testRequestControlledAction by the ASN.1 definition TestRequestControlledInfo.

If all tests specified by the IS parameter toBeInitiatedTests were successfully instantiated, the output parameter response is mapped to the M-ACTION response parameter 'Action reply', which is specified in ITU-T Recommendation X.745 [10] for the testRequestControlledAction by the ASN.1 definition TestRequestControlledResponse.

If at least one test failed to be instantiated, the output parameter response is mapped the M-ACTION parameter 'Errors'. The errors defined in ITU-T Recommendation X.745 [10] for testRequestControlledAction are noSuchMORT, mORTNotAvailable, mistypedTestCategoryInformation, noSuchAssociatedObject, associatedObjectNotAvailable, independentTestInvocationError.

Table 5: Parameter mapping of the operation initiateTest

| IS Parameter | IN/OUT | CMIP SS Equivalent | Qualifier |
| --- | --- | --- | --- |
| testInvocationInitiator | IN | This parameter is conditional and not used in the CMIP SS. | -- |
| maxTestingStateDuration | IN | M-ACTION parameter 'Action information' (TestRequestControlledInfo): timeoutPeriod | O |
| toBeInitiatedTests: toBeTestedMORT | IN | M-ACTION parameter 'Action information' (TestRequestControlledInfo): toBeTestedMORTs | O |
| toBeInitiatedTests: testerObjectClass | IN | M-ACTION parameter 'Action information' (TestRequestControlledInfo): testObjectList: tOClass | M |
| toBeInitiatedTests: testerObjectName | IN | M-ACTION parameter 'Action information' (TestRequestControlledInfo): testObjectList: tOInstance | O |
| toBeInitiatedTests: testerObjectInitialAttributeList | IN | M-ACTION parameter 'Action information' (TestRequestControlledInfo): testObjectList: initialAttributeList | O |
| response | OUT | All tests were successfully initiated: TestRequestControlledResponse: CHOICE independentTestResponseList At least one test failed to be initiated: M-ACTION response parameter 'Errors' | M |

#### 4.2.4.2	Parameter Mapping of the Operation terminateTest

The operation terminateTest is mapped to the GDMO action testTerminateAction defined in ITU-T Recommendation X.745 [10]. This action shall be implemented using the CMISE M-ACTION service.

All input parameters are mapped to the M-ACTION request parameter 'Action information'. This parameter is specified for the testTerminateAction in ITU-T Recommendation X.745 [10] by the ASN.1 definition TestTerminateInfo, which is the CMIP SS equivalent of the IS parameter toBeTerminatedTests.

If all tests specified by the IS parameter toBeTerminatedTests are successfully terminated, the output parameter response is mapped to the M-ACTION response parameter 'Action reply', which is specified in ITU-T Recommendation X.745 [10] for the testTerminateAction by the ASN.1 definition TestTerminateResult.

If at least one test failed to be terminated, the output parameter response is mapped the M-ACTION parameter 'Errors'. The errors defined in ITU-T Recommendation X.745 [10] for testTerminateAction are invalidTestOperation, noSuchTestInvocationId, noSuchTestSessionId, testTerminateError.

Table 6: Parameter mapping of the operation terminateTest

| IS Parameter | IN/OUT | CMIP SS Equivalent | Qualifier |
| --- | --- | --- | --- |
| toBeTerminatedTests | IN | M-ACTION parameter 'Action information' (TestTerminateInfo): indicatedTests with CHOICE SET OF testInvocationId | M |
| response | OUT | All tests are successfully terminated: M-ACTION parameter 'Action reply' (TestTerminateResult) At least one test failed to be terminated: M-ACTION response parameter 'Errors' | M |

#### 4.2.4.3	Parameter Mapping of the Operation monitorTest

The TO attributes reflecting the status of the test can be retrieved by the manager using the CMISE M-GET service. The TO to be monitored is specified by the M-GET request parameter 'Base object instance' and the attributes to be retrieved by the M-GET request parameter 'Attribute identifier list'.

The attribute values are returned in the M-GET response parameter 'Attribute list'.

Table 7: Parameter mapping of the operation monitorTest

| IS Parameter | IN/OUT | CMIP SS Equivalent | Qualifier |
| --- | --- | --- | --- |
| to BeMonitoredTO | IN | M-GET request parameter 'Base object instance' | M |
| toBeMonitoredAttributes | IN | M-GET request parameter 'Attribute identifier list': attribute identifier of the TO attributes testState, testOutcome and the other attributes to be monitored | M |
| monitoredAttributeValues | OUT | M-GET response parameter 'Attribute list': attribute identifier and value of the TO attributes operationalState, proceduralStatus, testState and testOutcome | M |
| error | OUT | M-GET response parameter 'Errors' | M |

### 4.2.5	Mapping of Notifications

The notification notifyTestResults is mapped to the GDMO notification testResultNotification defined in ITU-T Recommendation X.745 [10]. This notification shall be implemented using the CMISE M-EVENT-REPORT service.

Table 8: Mapping of notifications of the Test Management IRP IS

| Interface | Operation | GDMO Notification or CMISE of CMIP SS | Qualifier |
| --- | --- | --- | --- |
| TestManagementIRPNotifications | notifyTestResults | testResultNotification | M |

### 4.2.6	Mapping of Notification Parameters

The tables in the following subclauses show the parameters of each notification defined in 3GPP TS 32.322 [8] and their equivalents in the CMIP Solution Set.

#### 4.2.6.1	Parameter Mapping of the Notification notifyTestResults

Except for objectClass, objectInstance, eventTime and notificationType all parameters defined in the IS are mapped to the M-EVENT-REPORT parameter 'Event information'. The syntax and semantics of this structured parameter are defined for the notification testResultNotification in ITU-T Recommendation X.745 [10] by the ASN.1 definition TestResultInfo.

Table 9: Parameter mapping of the notification notifyTestResults

| IS Parameter | CMIP SS Equivalent | Qualifier |
| --- | --- | --- |
| objectClass | M-EVENT REPORT parameter 'Managed object class' | M |
| objectInstance | M-EVENT REPORT parameter 'Managed object instance' | M |
| notificationId | M-EVENT REPORT parameter 'Event information' (TestResultInfo): notificationIdentifier | O |
| eventTime | M-EVENT REPORT parameter 'Event time' | M |
| systemDN | This IS parameter is conditional and not used in the CMIP SS. | -- |
| notificationType | M-EVENT REPORT parameter 'Event type' | M |
| testInvocationId | M-EVENT REPORT parameter 'Event information' (TestResultInfo): testInvocationId | O |
| testInvocationInitiator | This IS parameter is conditional and not used in the CMIP SS. | -- |
| testOutcome | M-EVENT REPORT parameter 'Event information' (TestResultInfo): testOutcome | O |
| mORT | M-EVENT REPORT parameter 'Event information' (TestResultInfo): mORTs | O |
| proposedRepairActions | M-EVENT REPORT parameter 'Event information' (TestResultInfo): proposedRepairActions | O |
| additionalInformation | M-EVENT REPORT parameter 'Event information' (TestResultInfo): additionalInformation | O |
| fileReference | M-EVENT REPORT parameter 'Event information' (TestResultInfo): additionalInformation: fileReference | M see note |
| fileExpiryDate | M-EVENT REPORT parameter 'Event information' (TestResultInfo): additionalInformation: fileExpiryDate | M see note |
| NOTE: This parameter contains only information, if the test result data are captured in a file. Otherwise it shall contain no information or be absent. |  |  |

# 5	GDMO Definitions

## 5.1	Managed Object Classes

### 5.1.1	testManagementIRP

testManagementIRP MANAGED OBJECT CLASS

DERIVED FROM

"Rec. X.721 | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

testManagementIRPIdPackage,

testManagementIRPVersionPackage;

CONDITIONAL PACKAGES

testManagementIRPProfilePackage PRESENT IF "an instance supports it";

REGISTERED AS {ts32-324ObjectClass 1};

## 5.2	Packages

### 5.2.1	testManagementIRPIdPackage

testManagementIRPIdPackage PACKAGE

BEHAVIOUR

testManagementIRPIdPackageBehaviour;

ATTRIBUTES

testManagementIRPId;

REGISTERED AS {ts32-324Package 1};

testManagementIRPIdPackageBehaviour BEHAVIOUR

DEFINED AS

"An instance of the MOC testManagementIRP is identified by the value of the attribute testManagementIRPId.";

### 5.2.2	testManagementIRPVersionPackage

testManagementIRPVersionPackage PACKAGE

BEHAVIOUR

testManagementIRPVersionPackageBehaviour;

ATTRIBUTES

supportedTestManagementIRPVersions		GET;

ACTIONS

getTestManagementIRPVersion;

REGISTERED AS {ts32-324Package 2};

testManagementIRPVersionPackageBehaviour BEHAVIOUR

DEFINED AS

"This package has been defined to allow the IRPManager to get information about the Test Management IRP versions supported by the IRPAgent.

The attribute supportedTestManagementIRPVersions indicates all versions of the Test Management IRP currently supported by the IRPAgent.

The action getTestManagementIRPVersion is invoked by the IRPManager to get information about the Test Management IRP versions supported by the IRPAgent.";

### 5.2.3	testManagementIRPProfilePackage

testManagementIRPProfilePackage PACKAGE

BEHAVIOUR

testManagementIRPProfilePackageBehaviour;

ACTIONS

getTestManagementIRPOperationProfile,

getTestManagementIRPNotificationProfile;

REGISTERED AS {ts32-324Package 3};

testManagementIRPProfilePackageBehaviour BEHAVIOUR

DEFINED AS

"This package has been defined to allow the Manager to get detailed information about the profile of the Test Management IRP. The action getOperationProfile is invoked by the Manager to get detailed information about the operations supported by the Test Management IRP. The action getNotificationProfile is invoked by the Manager to get detailed information about the notifications supported by the Test Management IRP.";

## 5.3	Actions

### 5.3.1	getTestManagementIRPVersion (M)

getTestManagementIRPVersion ACTION

BEHAVIOUR

getTestManagementIRPVersionBehaviour;

MODE

CONFIRMED;

WITH REPLY SYNTAX

TS32-324TypeModule.GetTestManagementIRPVersionReply;

REGISTERED AS {ts32-324Action 1};

getTestManagementIRPVersionBehaviour BEHAVIOUR

DEFINED AS

"The behaviour of this functionality is defined within 32.322 – below provides an overview and CMIP specific semantics.

The Manager invokes this action to get information about the Test Management IRP versions supported by the Agent. The M-ACTION request parameter 'Action information' contains no data. The M-ACTION response parameter 'Action reply' is composed of the following data:

- versionNumbersList

- status

The parameter versionNumbersList defines a list of Test Management IRP versions supported by the Agent. A list containing no element, i.e. a NULL list, means that the concerned Agent doesn't support any version of the Test Management IRP. The parameter status contains the results of the Manager action. Possible values: noError (0), error (the value indicates the reason of the error).";

### 5.3.2	getTestManagementIRPOperationProfile (O)

getTestManagementIRPOperationProfile ACTION

BEHAVIOUR

getTestManagementIRPOperationProfileBehaviour;

MODE

CONFIRMED;

WITH INFORMATION SYNTAX

TS32-324TypeModule.IRPVersionNumber;

WITH REPLY SYNTAX

TS32-324TypeModule.GetOperationProfileReply;

REGISTERED AS {ts32-324Action 3};

getTestManagementIRPOperationProfileBehaviour BEHAVIOUR

DEFINED AS

"The behaviour of this functionality is defined within 32.322 – below provides an overview and CMIP specific semantics.

A Manager invokes this action to enquiry about the operation profile (supported operations and supported parameters) for this specific Test Management IRP version.

The M-ACTION request parameter 'Action information' contains the following data:

- irpVersionNumber

This mandatory parameter identifies the Test Management IRP version.

The M-ACTION response 'Action reply' is composed of the following data:

- operationNameProfile

- operationParameterProfile

- status

The parameter operationNameProfile contains a list of operation names. The parameter operationParameterProfile contains a set of elements, each element corresponds to an operation name and is composed by a set of parameter names. The parameter status contains the results of this action. Possible values: noError (0), error (the value indicates the reason of the error).";

### 5.3.3	getTestManagementIRPNotificationProfile (O)

getTestManagementIRPNotificationProfile ACTION

BEHAVIOUR

getTestManagementIRPNotificationProfileBehaviour;

MODE

CONFIRMED;

WITH INFORMATION SYNTAX

TS32-324TypeModule.IRPVersionNumber;

WITH REPLY SYNTAX

TS32-324TypeModule.GetNotificationProfileReply;

REGISTERED AS {ts32-324Action 2};

getTestManagementIRPNotificationProfileBehaviour BEHAVIOUR

DEFINED AS

"The behaviour of this functionality is defined within 32.322 – below provides an overview and CMIP specific semantics.

A Manager invokes this action to enquiry about the notification profile (supported notifications and supported parameters) for this specific Test Management IRP version.

The M-ACTION request parameter 'Action information' contains the following data:

- irpVersionNumber

This mandatory parameter identifies the Test Managemnt IRP version.

The M-ACTION response parameter 'Action reply' is composed of the following data:

- notificationNameProfile

- notificationParameterProfile

- status

The parameter notificationNameProfile contains a list of notification names, i.e. a NULL list means that the Test Management IRP doesn't support any notification. The parameter notificationParameterProfile contains a set of elements, each element corresponds to a notification name and is composed by a set of parameter names. The parameter status contains the results of this action. Possible values: noError (0), error (the value indicates the reason of the error).";

## 5.4	Attributes

### 5.4.1	testManagementIRPId

testManagementIRPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-324TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

testManagementIRPIdBehaviour;

REGISTERED AS {ts32-324Attribute 1};

testManagementIRPIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute names an instance of the MOC testManagementIRP.";

### 5.4.2	supportedTestManagementIRPVersions

supportedTestManagementIRPVersions ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-324TypeModule.SupportedTestManagementIRPVersions;

MATCHES FOR

EQUALITY;

BEHAVIOUR

supportedTestManagementIRPVersionsBehaviour;

REGISTERED AS {ts32-324Attribute 2};

supportedTestManagementIRPVersionsBehaviour BEHAVIOUR

DEFINED AS

"This attribute provides the information concerning the Test Management IRP versions currently supported by the Agent.";

## 5.5	Parameters

### 5.5.1	fileReference

fileReference PARAMETER

CONTEXT

TS32-324TypeModule.TestResultInfo.additionalInformation;

WITH SYNTAX

TS32-324TypeModule.FileReference;

REGISTERED AS {ts32-324Parameter 1};

### 5.5.2	fileExpiryDate

fileExpiryDate PARAMETER

CONTEXT

TS32-324TypeModule.TestResultInfo.additionalInformation;

WITH SYNTAX

TS32-324TypeModule.FileExpiryDate;

REGISTERED AS {ts32-324Parameter 2};

# 6	ASN.1 Definitions

TS32-324TypeModule {itu-t(0) identified-organization(4) etsi(0) mobileDomain(0) umts-Operation-Maintenance(3) ts32-324(324) informationModel(0) asn1Module(2) version1(1)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

--EXPORTS everything

IMPORTS

TestResultInfo

FROM Test-ASN1Module { joint-iso-ccitt ms(9) function(2) part12(12) asn1Module(2) 0 };

baseNodeUMTS				OBJECT IDENTIFIER ::= {	itu-t (0) identified-organization (4) etsi (0) 													mobileDomain (0) umts-Operation-Maintenance (3)}

ts32-324Prefix				OBJECT IDENTIFIER ::= {baseNodeUMTS ts32-324                (324)}

ts32-324InfoModel			OBJECT IDENTIFIER ::= {ts32-324Prefix informationModel      (  0)}

ts32-324ObjectClass			OBJECT IDENTIFIER ::= {ts32-324InfoModel managedObjectClass (  3)}

ts32-324Package			O	BJECT IDENTIFIER ::= {ts32-324InfoModel package             (  4)}

ts32-324Parameter			OBJECT IDENTIFIER ::= {ts32-324InfoModel parameter          (  5)}

ts32-324Attribute			OBJECT IDENTIFIER ::= {ts32-324InfoModel attribute          (  7)}

ts32-324Action				OBJECT IDENTIFIER ::= {ts32-324InfoModel action             (  9)}

ts32-324Notification		OBJECT IDENTIFIER ::= {ts32-324InfoModel notification       ( 10)}

ErrorCauses ::= ENUMERATED

{

noError (0),						-- operation / notification successfully performed

unspecifiedErrorReason (255)		-- operation failed, specific error unknown

}

FileReference ::= GraphicString

FileExpiryDate ::= GeneralizedTime

GetNotificationProfileReply ::= SEQUENCE

{

notificationNameProfile			NotificationList,

notificationParameterProfile	ParameterListOfList,

status							ErrorCauses

}

GetOperationProfileReply ::= SEQUENCE

{

operationNameProfile			OperationList,

operationParameterProfile		ParameterListOfList,

status							ErrorCauses

}

GetTestManagementIRPVersionReply ::= SEQUENCE

{

versionNumberList					SupportedTestManagementIRPVersions,

status								ErrorCauses

}

GeneralObjectId ::= INTEGER

IRPVersionNumber ::= GraphicString

NotificationList ::= SET OF NotificationName

NotificationName ::= GraphicString

OperationList ::= SET OF OperationName

OperationName ::= GraphicString

ParameterList ::= SET OF ParameterName

ParameterListOfList ::= SET OF ParameterList

ParameterName ::= GraphicString

SupportedTestManagementIRPVersions ::= SET OF IRPVersionNumber

END – of module TS32-324TypeModule

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| Jun 2002 | S_16 | SP-020328 | -- | -- | Submitted to TSG SA #16 for Information | 1.0.0 |  |
| Sep 2002 | S_17 | SP-020459 | -- | -- | Submitted to TSG SA #17 for Approval | 2.0.0 | 5.0.0 |
| Dec 2002 | -- | -- | -- | -- | Cosmetics | 5.0.0 | 5.0.1 |
| Jun 2004 | S_24 | SP-040243 | 001 | -- | Add missing parameter to the operation initiateTests | 5.0.1 | 5.1.0 |
| Sep 2004 | S_25 | SP-040541 | -- | -- | Automatic upgrade to Rel- 6 (no CR) as per request in SP-040541 SA5_presentation_SA_25.ppt (slide 17) | 5.1.0 | 6.0.0 |