3GPP TS 32.601-4 V2.0.0 (2001-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

3G Configuration Management:

Basic Configuration Management IRP: CMIP Solution Set;

(Release 4)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Configuration Management

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2001, 3GPP Organizational Partners (ARIB, CWTS, ETSI, T1, TTA, TTC).

All rights reserved.

Contents

Foreword	4

Introduction	4

1	Scope	6

2	References	6

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	7

4	Basic aspects	7

4.1	CMIP specific aspects	7

4.1.1	About Associations	7

4.1.2	About getContainment	7

4.1.3	About getMoAttributes	7

4.1.4	About cancelOperation	8

4.2	Mapping	9

4.2.1	Mapping of Operations	9

4.2.2	Mapping of operation parameters	9

4.2.2.1	Mapping of Parameters of ‘getMoAttributes’	10

4.2.2.2	Mapping of Parameters of ‘getContainment’	11

4.2.2.4	Mapping of parameters of ‘getBasicCmIRPVersion’	11

4.2.2.3	Mapping of Parameters of ‘cancelOperation’	12

4.2.3	Mapping of notifications	12

4.2.4	Mapping of notification parameters	12

4.2.4.1	Mapping of parameters of the notification ‘notifyObjectCreation’	12

4.2.4.2	Mapping of parameters of the notification ‘notifyObjectDeletion’	13

4.2.4.3	Mapping of parameters of the notification ‘notifyAttributeValueChange’	13

5	GDMO Definitions	14

6	ASN.1 Definitions	15

Annex A (informative):	Change history	16

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

Configuration Management (CM), in general, provides the operator with the ability to assure correct and effective operation of the 3G network as it evolves. CM actions have the objective to control and monitor the actual configuration on the Network Elements (NEs) and Network Resources (NRs), and they may be initiated by the operator or by functions in the Operations Systems (OSs) or NEs.

Due to the growing number of specifications to model new services and Resource Models for Configuration Management (CM), as well as the expected growth in size of each of them from 3GPP Release 4 onwards, a new structure of the specifications is already needed in Release 4. This structure is needed for several reasons, but mainly to enable more independent development and release for each part, as well as a simpler document identification and version handling. Another benefit would be that it becomes easier for bodies outside 3GPP, such as the ITU-T, to refer to telecom management specifications from 3GPP. The new structure of the specifications does not lose any information or functionality supported by the Release 1999. The restructuring also includes defining new IRPs for the Network Resource Models (Generic, Core Network and UTRAN NRM).

Finally, the Name convention for Managed Objects (in Release 1999: 32.106-8) has been moved to a separate number series used for specifications common between several management areas (e.g. CM, FM, PM).

The following table shows an overview of the mapping between the old Release 1999 and new Release 4 CM specification structure.

Table: Mapping between Release '99 and the new Rel-4 specifications

| R99 Old no. | Old (R99) specification title | Rel-4 New no. | New (Rel-4) specification title |
| --- | --- | --- | --- |
| 32.106-1 | 3G Configuration Management: Concept and Requirements | 32.600 | 3G Configuration Management: Concept and High-level Requirements |
| 32.106-1 | <Notification IRP requirements from 32.106-1 and 32.106-2> | 32.301-1 | Notification IRP: Requirements |
| 32.106-2 | Notification IRP: IS | 32.301-2 | Notification IRP: Information Service |
| 32.106-3 | Notification IRP: CORBA SS | 32.301-3 | Notification IRP: CORBA SS |
| 32.106-4 | Notification IRP: CMIP SS | 32.301-4 | Notification IRP: CMIP SS |
| 32.106-8 | Name convention for Managed Objects | 32.300 | Name Convention for Managed Objects |
| 32.106-1 | <Basic CM IRP IS requirements from 32.106-1 and 32.106-5> | 32.601-1 | Basic CM IRP: Requirements |
| 32.106-5 | Basic CM IRP IM (Intro & IS part) | 32.601-2 | Basic CM IRP: Information Service |
| 32.106-6 | Basic CM IRP CORBA SS (IS related part) | 32.601-3 | Basic CM IRP: CORBA SS |
| 32.106-7 | Basic CM IRP CMIP SS (IS related part) | 32.601-4 | Basic CM IRP: CMIP SS |
| 32.106-1 | <Basic CM IRP Generic NRM requirements from 32.106-1 and 32.106-5> | 32.620-1 | Generic Network Resources IRP: Requirements |
| 32.106-5 | Basic CM IRP IM (Generic NRM part) | 32.620-2 | Generic Network Resources IRP: NRM |
| 32.106-6 | Basic CM IRP CORBA SS (Generic NRM related part) | 32.620-3 | Generic Network Resources IRP: CORBA SS |
| 32.106-7 | Basic CM IRP CMIP SS (Generic NRM related part) | 32.620-4 | Generic Network Resources IRP: CMIP SS |
| 32.106-1 | <Basic CM IRP CN NRM requirements from 32.106-1 and 32.106-5> | 32.621-1 | Core Network Resources IRP: Requirements |
| 32.106-5 | Basic CM IRP IM (CN NRM part) | 32.621-2 | Core Network Resources IRP: NRM |
| 32.106-6 | Basic CM IRP CORBA SS (CN NRM related part) | 32.621-3 | Core Network Resources IRP: CORBA SS |
| 32.106-7 | Basic CM IRP CMIP SS (CN NRM related part) | 32.621-4 | Core Network Resources IRP: CMIP SS |
| 32.106-1 | <Basic CM IRP UTRAN NRM requirements from 32.106-1 and 32.106-5> | 32.622-1 | UTRAN Network Resources IRP: Requirements |
| 32.106-5 | Basic CM IRP IM (UTRAN NRM part) | 32.622-2 | UTRAN Network Resources IRP: NRM |
| 32.106-6 | Basic CM IRP CORBA SS (UTRAN NRM related part) | 32.622-3 | UTRAN Network Resources IRP: CORBA SS |
| 32.106-7 | Basic CM IRP CMIP SS (UTRAN NRM related part) | 32.622-4 | UTRAN Network Resources IRP: CMIP SS |

The present document is 3GPP TS 32.601-4: Basic Configuration Management IRP: CMIP Solution Set.

# 1	Scope

The present document specifies the Common Management Information Protocol (CMIP) Solution Set (SS) for the Basic CM Integration Reference Point (IRP): Information Service defined in 3GPP TS 32.601-2. In detail:

- Clause 4 contains an introduction to some concepts that are the base for some specific aspects of the CMIP interfaces.

- Clause 5 contains the GDMO definitions for the Alarm Management over the CMIP interfaces

- Clause 6 contains the ASN.1 definitions supporting the GDMO definitions provided in clause 5.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "3G Telecom Management principles and high level requirements".

[2]	3GPP TS 32.102: "3G Telecom Management architecture".

[3]	3GPP TS 32.301-4: "Telecommunication Management; Notification Management; 
Part 4: Notification Integration Reference Point; CMIP Solution Set".

[4]	3GPP TS 32.601-2: "Telecommunication Management; Configuration Management: Basic CM Integration Reference Point; Information Services".

[5]	ITU-T Recommendation X.710 (1991): "Common Management Information Service Definition for CCITT Applications".

[6]	ITU-T Recommendation X.721 (02/92): "Information Technology - Open Systems Interconnection – Structure of Management Information: Definition of Management Information".

[7]	ITU-T Recommendation X.730 (01/92): "Information Technology - Open Systems Interconnection – Systems Management: Object Management Function".

[8]	ITU-T Recommendation X.733 (02/92): "Information Technology - Open Systems Interconnection - Alarm Reporting Function".

[9]	ITU-T Recommendation M.3100 (07/95): "Maintenance Telecommunications Management Network – Generic Network Information Model".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TS 32.600 and 3GPP TS 32.601-2 apply.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

CMIP	Common Management Information Protocol

DN	Distinguished Name

GDMO	Guidelines for the Definition of Managed Objects

IDL	Interface Definition Language

IEC	International Electro-technical Commission

ISO	International Standards Organization

ITU-T	International Telecommunication Union, Telecommunication Sector

MIB	Management Information Base

MIM	Management Information Model

MIT	Management Information Tree (or Naming Tree)

MOC	Managed Object Class

MOI	Managed Object Instance

NE	Network Element

NR	Network Resource

NRM	Network Resource Model

TMN	Telecommunications Management Network

# Basic aspects

## CMIP specific aspects

This clause describes some technical details specific to CMIP technology, which are not easy to be handled in the related GDMO definitions.

### About Associations

In the GDMO definitions, except the containment relations, all associations among different object classes and object instances are modelled with dedicated pointers of the concerned objects, i.e. various relation role attributes. These pointers are normal object attributes and don’t require any special treatment. The service operation getMoAttributes defined in 3GPP TS 32.602-2 and mapped on M-GET in this CMIP solution set is applied for managers to retrieve the values of these association pointers and the notification attributeValueChange is applied for agents to report any change of the values of these association pointers.

### About getContainment

In the GDMO definition the containment relations of the Managed Object Classes and those of the managed object instances are described by the name bindings. The service operation getContainment is defined in 3GPP TS 32.602-2 to enable managers to retrieve the management information about the containment tree of the local MIB of an agent. This service operation is mapped to CMISE M-GET in this CMIP solution set. The information about the containment relation of a local MIB is consists of all MOIs abstracted from the output parameter AttributeList of a M-GET operation.

### About getMoAttributes

The service operation getMoAttributes defined in the Basic CM IRP IS (3GPP TS 32.602-2) provides the basic functionality required to retrieve managed objects and their attributes, which is a subset of the functionality provided by the corresponding CMISE service operation M-GET. getMoAttributes is mapped to M-GET  in this standard. This doesn’t mean any limitation for using M-GET. Users of this standard are encouraged to use the whole functionality provided by M-Get, especially the input parameter “Attribute Identifier List” (see ITU-T X.710 [7]).

### 4.1.4	About cancelOperation

The service operation cancelOperation defined in the Basic CM IRP IS (3GPP TS 32.601-2) provides the basic functionality required to cancel an on-going getContainment or getMoAttributes operation, which is a subset of the functionality provided by the corresponding CMISE service operation M-CANCEL-GET. cancelOperation is mapped to M-CANCEL-GET  in this standard. This doesn’t mean any limitation for using M-Cancel. Users of this standard are encouraged to use the whole functionality provided by M-CANCEL-GET.

## 4.2	Mapping

The semantics of the Basic CM IRP ISis defined in 3GPP TS 32.602-2. The definitions of the management services and management information defined there are independent of any implementation technology and protocol. 
This subclause maps these technology and protocol independent definitions onto the equivalencies of the CMIP Solution Set of the Basic CM IRP.

### 4.2.1	Mapping of Operations

Table 2 maps the operations defined in the Basic CM IRP Information Service onto the equivalent Actions/Services of the CMIP Solution Set. The CMIP Actions/Services are qualified as Mandatory (M) or Optional (O).

Table 1: Mapping of operations

| Operations of Information Services of the Basic CM IRP defined in 3GPP TS 32.601-2 | Equivalent operation of the CMIP solution set of the Basic CM IRP | Qualifier |
| --- | --- | --- |
| GetMoAttributes | M-GET (CMISE Service) | M |
| GetContainment | M-GET (CMISE Service) | O |
| CancelOperation | M-CANCEL-GET (CMISE Service) | O |
| GetBasicCmIRPVersion | M-ACTION getBCmIRPVersion (3GPP TS 32.106-7: 6.2001) | M |

### 4.2.2	Mapping of operation parameters

Tables 3, 4 and 5 in the following subclauses show the parameters of each operation defined in the Information Service described in 3GPP TS 32.602-2 and their equivalences in the CMIP Solution Set.

#### 4.2.2.1	Mapping of Parameters of ‘getMoAttributes’

Table 2: Mapping of parameters of ‘getMoAttributes’

| Parameters of the operation ‘getMoAttributes’ defined in 3GPP TS 32.601-2 | CMISE M-GET parameters | Qualifier |
| --- | --- | --- |
| invokeIdentifier | Invoke identifier | M |
| baseObjectInstance | Base object instance | M |
| scope | Scope | M |
| filter | Filter | M |
| no equivalence | Invoker identifier This is a CMISE specific parameter. There is no equivalent parameter defined in the Information Service for ‘getMoAttributes’. | O |
| no equivalence | Basic object class This is a CMISE specific parameter. There is no equivalent parameter defined in the Information Service for ‘getMoAttributes’. | M |
| no equivalence | Access Control This is a CMISE specific parameter. There is no equivalent parameter defined in the Information Service for ‘getMoAttributes’. | O |
| no equivalence | Synchronisation This is a CMISE specific parameter. There is no equivalent parameter defined in the Information Service for ‘getMoAttributes’. | O |
| attributeListIn | Attribute identifier list | M |
| managedObjectClass | Managed object class | M |
| managedObjectInstance | Managed object instance | M |
| attributeListOut | Attribute list | M |
| status | Errors | M |
| no equivalence | Current time This is a CMISE specific parameter. There is no equivalence parameter defined in the Information Service for ‘getMoAttributes’. | O |

#### Mapping of Parameters of ‘getContainment’

Table 3: Mapping of parameters of ‘getContainment’

| Parameters of the operation ‘getContainment’ defined in 3GPP TS 32.601-2 | CMISE M-GET parameter | Qualifier |
| --- | --- | --- |
| invokeIdentifier | Invoke identifier | M |
| baseObjectInstance | Base object instance | M |
| scope | Scope | O |
| no equivalence | filter This is a CMISE specific parameter. There is no equivalent parameter defined in the Information Service for ‘getContainment’. The value of this parameter shall be ‘empty’. | O |
| no equivalence | Invoker identifier This is a CMISE specific parameter. There is no equivalent parameter defined in the Information Service for ‘getContainment’. | O |
| no equivalence | Basic object class This is a CMISE specific parameter. There is no equivalent parameter defined in the Information Service for ‘getContainment’. | M |
| no equivalence | Access Control This is a CMISE specific parameter. There is no equivalent parameter defined in the Information Service for ‘getContainment’. | O |
| no equivalence | Synchronisation This is a CMISE specific parameter. There is no equivalent parameter defined in the Information Service for ‘getContainment’. | O |
| no equivalence | Attribute identifier list This is a CMISE specific parameter. There is no equivalence parameter defined in the Information Service for ‘getContainment’. It is recommended to use ‘objectClass’ or/and ‘nameBinding’ defined in X.721 for the MOC top as the value of this input parameter. | O |
| containment | Managed object class | M |
|  | Managed object instance | M |
|  | Attribute list | M |
| status | Errors | M |
| no equivalence | Current time This is a CMISE specific parameter. There is no equivalence parameter defined in the Information Service for ‘getMoAttributes’. | O |

#### 4.2.2.4	Mapping of parameters of ‘getBasicCmIRPVersion’

Table  4: Mapping of parameters of “getBasicCmIRPVersion”

| Operation parameters of the Basic CM IRP Information Services | CMISE M-ACTION Parameters | Qualifier |
| --- | --- | --- |
| no equivalence | Invoke identifier | M |
| no equivalence | Linked identifier | O |
| no equivalence | Mode | M |
| no equivalence | Base object class (input) | M |
| no equivalence | Base object instance (input) | M |
| no equivalence | Scope | O |
| no equivalence | Filter | O |
| no equivalence | Managed object class (output) | O |
| no equivalence | Managed object instance (output) | O |
| no equivalence | Access control | O |
| no equivalence | Synchronization | O |
| no equivalence | Action type | M |
| no equivalence | Action information | O |
| no equivalence | Current time | O |
| versionNumberList, status | Action reply | O |
| no equivalence | Errors | O |

#### Mapping of Parameters of ‘cancelOperation’

Table 5: Mapping of parameters of ‘cancelOperation’

| Parameters of the operation ‘getContainment’ defined in 3GPP TS 32.601-2 | CMISE M-CANCEL-GET parameter | Qualifier |
| --- | --- | --- |
| No equivalence | Invoke identifier | M |
| invokeIdentifier | Get invoke identifier | M |
| status | Errors | M |

### 4.2.3	Mapping of notifications

Table 6 maps the notifications defined in the Basic CM IRP Information Service onto the equivalent notification of the CMIP Solution Set. The CMIP notifications are qualified as Mandatory (M) or Optional (O).

Table  6: Mapping of notifications

| Notifications of Basic CM IRP Information Service | Notifications of the Basic CM IRP CMIP solution set | Qualifier |
| --- | --- | --- |
| notifyObjectCreation | objectCreation ITU-T X.721 {smi2Notification 6} | O |
| notifyObjectDeletion | objectDeletion ITU-T X.721 {smi2Notification 7} | O |
| notifyAttributeValueChange | AttributeValueChange ITU-T X.721 {smi2Notification 1} | O |

### 4.2.4	Mapping of notification parameters

Tables 7, 8 and 9 in the following subclauses show the parameters of each notification defined in the Information Service described in 3GPP TS 32.602-2 and their equivalence in the CMIP Solution Set.

The mapping of common parameters of all kinds of notifications defined in 3GPP TS 32.602-2 is described in 3GPP TS 32.300-4 and will not be repeated in the present document. These common parameters are managedObjectClass, managedObjectInstance, NotificationId, eventType, extendedEventType, eventTime and systemDN.

#### 4.2.4.1	Mapping of parameters of the notification ‘notifyObjectCreation’

Table  7: Mapping of parameters of the notification ‘notifyObjectCreation’

| Parameters of the Basic CM IRP IS notification ‘notifyObjectCreation’ | Parameters of the CMIP SS notification ‘objectCreation’ | Qualifier |
| --- | --- | --- |
| correlatedNotifications | correlatedNotifications (ITU-T X.721) | O |
| sourceIndicator | sourceIndicator (ITU-T X.721) | O |
| attributeList | attributeList (ITU-T X.721) | O |
| no equivalence | additionalText (ITU-T X.721) | O |
| no equivalence | additionalInformation (ITU-T X.721) | O |

#### Mapping of parameters of the notification ‘notifyObjectDeletion’

Table  8: Mapping of parameters of the notification ‘notifyObjectDeletion’

| Parameter of the Basic CM IRP IS notification ‘notifyObjectDeletion’ | parameter of the CMIP SS notification ‘objectDeletion’ | Qualifier |
| --- | --- | --- |
| correlatedNotifications | correlatedNotifications (ITU-T X.721) | O |
| sourceIndicator | sourceIndicator (ITU-T X.721) | O |
| attributeList | attributeList (ITU-T X.721) | O |
| no equivalence | additionalText (ITU-T X.721) | O |
| no equivalence | additionalInformation (ITU-T X.721) | O |

#### Mapping of parameters of the notification ‘notifyAttributeValueChange’

Table  9: Mapping of parameters of the notification ‘notifyAttributeValueChange’

| Parameter of the Basic CM IRP IS notification ‘notifyAttributeValueChange’ | parameter of the CMIP SS notification ‘attributeValueChange’ | Qualifier |
| --- | --- | --- |
| correlatedNotifications | correlatedNotifications (ITU-T X.721) | O |
| sourceIndicator | sourceIndicator (ITU-T X.721) | O |
| attributeValueChangeDefinition | attributeValueChangeDefinition (ITU-T X.721) | M |
| no equivalence | attributeIdentifierList (ITU-T X.721) | O |
| no equivalence | additionalText (ITU-T X.721) | O |
| no equivalence | additionalInformation (ITU-T X.721) | O |

# GDMO Definitions

No GDMO specification is currently required for this document.

# 6	ASN.1 Definitions

No ASN.1 specification is currently required for this document.

Annex A (informative):
Change history

This annex lists all change requests approved for the present document since the specification was first approved by 3GPP TSG-SA.

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| Jun 2001 | S_12 | SP-010283 | -- | -- | Approved at TSG SA #12 and placed under Change Control | 2.0.0 | 4.0.0 |