3GPP TS 32.621-4 V2.0.0 (2001-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

3G Configuration Management:

CN Network Resources IRP: CMIP Solution Set;

(Release 4)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Configuration Management

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2001, 3GPP Organizational Partners (ARIB, CWTS, ETSI, T1, TTA, TTC).

All rights reserved.

Contents

Foreword	5

Introduction	5

1	Scope	7

2	References	7

3	Definitions, symbols and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	8

4	Basic aspects	8

4.1	Explanation	8

4.2	Mapping	8

4.2.1	Mapping of MOCs	8

4.2.2	Mapping of Attributes	9

5	GDMO Definitions	10

5.1	Managed Object Classes	10

5.1.1	smlcFunction	10

5.1.2	gmlcFunction	10

5.1.3	scfFunction	10

5.1.4	srfFunction	11

5.1.5	cbcFunction	11

5.1.6	cgfFunction	11

5.1.7	mgwFunction	12

5.1.8	gmscFunction	12

5.1.9	iwfFunction	12

5.1.10	mnpSrfFunction	13

5.1.11	npdbFunction	13

5.1.12	rSgwFunction	14

5.1.13	ssfFunction	14

5.1.14	bsFunction	14

5.2	Attributes	15

5.2.1	smlcFunctionId	15

5.2.2	gmlcFunctionId	15

5.2.3	sfcFunctionId	15

5.2.4	srfFunctionId	16

5.2.5	CbcFunctionId	16

5.2.6	CgfFunctionId	16

5.2.7	mgwFunctionId	16

5.2.8	gmscFunctionId	17

5.2.9	iwfFunctionId	17

5.2.10	mnpSrfFunctionId	17

5.2.11	npdbFunctionId	17

5.2.12	rSgwFunctionId	18

5.2.13	ssfFunctionId	18

5.2.14	bsFunctionId	18

5.3	Name Binding	19

5.3.1	smlcFunction - managedElement	19

5.3.2	gmlcFunction - managedElement	19

5.3.3	sfcFunction - managedElement	19

5.3.4	srfFunction - managedElement	20

5.3.5	cbcFunction - managedElement	20

5.3.6	cgfFunction - managedElement	21

5.3.7	mgwFunction - managedElement	21

5.3.8	gmscFunction - managedElement	21

5.3.9	iwfFunction - managedElement	22

5.3.10	mnpSrfFunction - managedElement	22

5.3.11	npdbFunction - managedElement	22

5.3.12	rSgwFunction - managedElement	23

5.3.13	ssfFunction - managedElement	23

5.3.14	bsFunction - managedElement	24

6	ASN.1 Definitions	25

Annex A (informative):	Change history	26

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

Configuration Management (CM), in general, provides the operator with the ability to assure correct and effective operation of the 3G network as it evolves. CM actions have the objective to control and monitor the actual configuration on the Network Elements (NEs) and Network Resources (NRs), and they may be initiated by the operator or by functions in the Operations Systems (OSs) or NEs.

Due to the growing number of specifications to model new services and Resource Models for Configuration Management (CM), as well as the expected growth in size of each of them from 3GPP Release 4 onwards, a new structure of the specifications is already needed in Release 4. This structure is needed for several reasons, but mainly to enable more independent development and release for each part, as well as a simpler document identification and version handling. Another benefit would be that it becomes easier for bodies outside 3GPP, such as the ITU-T, to refer to telecom management specifications from 3GPP. The new structure of the specifications does not lose any information or functionality supported by the Release 1999. The restructuring also includes defining new IRPs for the Network Resource Models (Generic, Core Network and UTRAN NRM).

Finally, the Name convention for Managed Objects (in Release 1999: 32.106-8) has been moved to a separate number series used for specifications common between several management areas (e.g. CM, FM, PM).

The following table shows an overview of the mapping between the old Release 1999 and new Release 4 CM specification structure.

Table: Mapping between Release '99 and the new Rel-4 specifications

| R99 Old no. | Old (R99) specification title | Rel-4 New no. | New (Rel-4) specification title |
| --- | --- | --- | --- |
| 32.106-1 | 3G Configuration Management: Concept and Requirements | 32.600 | 3G Configuration Management: Concept and High-level Requirements |
| 32.106-1 | <Notification IRP requirements from 32.106-1 and 32.106-2> | 32.301-1 | Notification IRP: Requirements |
| 32.106-2 | Notification IRP: IS | 32.301-2 | Notification IRP: Information Service |
| 32.106-3 | Notification IRP: CORBA SS | 32.301-3 | Notification IRP: CORBA SS |
| 32.106-4 | Notification IRP: CMIP SS | 32.301-4 | Notification IRP: CMIP SS |
| 32.106-8 | Name convention for Managed Objects | 32.300 | Name Convention for Managed Objects |
| 32.106-1 | <Basic CM IRP IS requirements from 32.106-1 and 32.106-5> | 32.601-1 | Basic CM IRP: Requirements |
| 32.106-5 | Basic CM IRP IM (Intro & IS part) | 32.601-2 | Basic CM IRP: Information Service |
| 32.106-6 | Basic CM IRP CORBA SS (IS related part) | 32.601-3 | Basic CM IRP: CORBA SS |
| 32.106-7 | Basic CM IRP CMIP SS (IS related part) | 32.601-4 | Basic CM IRP: CMIP SS |
| 32.106-1 | <Basic CM IRP Generic NRM requirements from 32.106-1 and 32.106-5> | 32.620-1 | Generic Network Resources IRP: Requirements |
| 32.106-5 | Basic CM IRP IM (Generic NRM part) | 32.620-2 | Generic Network Resources IRP: NRM |
| 32.106-6 | Basic CM IRP CORBA SS (Generic NRM related part) | 32.620-3 | Generic Network Resources IRP: CORBA SS |
| 32.106-7 | Basic CM IRP CMIP SS (Generic NRM related part) | 32.620-4 | Generic Network Resources IRP: CMIP SS |
| 32.106-1 | <Basic CM IRP CN NRM requirements from 32.106-1 and 32.106-5> | 32.621-1 | Core Network Resources IRP: Requirements |
| 32.106-5 | Basic CM IRP IM (CN NRM part) | 32.621-2 | Core Network Resources IRP: NRM |
| 32.106-6 | Basic CM IRP CORBA SS (CN NRM related part) | 32.621-3 | Core Network Resources IRP: CORBA SS |
| 32.106-7 | Basic CM IRP CMIP SS (CN NRM related part) | 32.621-4 | Core Network Resources IRP: CMIP SS |
| 32.106-1 | <Basic CM IRP UTRAN NRM requirements from 32.106-1 and 32.106-5> | 32.622-1 | UTRAN Network Resources IRP: Requirements |
| 32.106-5 | Basic CM IRP IM (UTRAN NRM part) | 32.622-2 | UTRAN Network Resources IRP: NRM |
| 32.106-6 | Basic CM IRP CORBA SS (UTRAN NRM related part) | 32.622-3 | UTRAN Network Resources IRP: CORBA SS |
| 32.106-7 | Basic CM IRP CMIP SS (UTRAN NRM related part) | 32.622-4 | UTRAN Network Resources IRP: CMIP SS |

The present document is 3GPP TS 32.621-4: CN Network Resource IRP: CMIP Solution Set.

# 1	Scope

The present document specifies the Common Management Information Protocol (CMIP) Solution Set (SS) for the CN Network Resource Integration Reference Point (IRP): Network Resource Model defined in 3GPP TS 32.621-2. In detail:

- Clause 4 contains an introduction to some concepts that are the base for some specific aspects of the CMIP interfaces.

- Clause 5 contains the GDMO definitions for the Alarm Management over the CMIP interfaces

- Clause 6 contains the ASN.1 definitions supporting the GDMO definitions provided in clause 5.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "3G Telecom Management principles and high level requirements".

[2]	3GPP TS 32.102: "3G Telecom Management architecture".

[3]	3GPP TS 32.301-4: "Telecommunication Management; Notification Management; 
Part 4: Notification Integration Reference Point; CMIP Solution Set".

[4]	3GPP TS 32.621-2: "Telecommunication Management; Configuration Management: CN Network Resource Integration Reference Point: Network Resource Model".

[5]	ITU-T Recommendation X.710 (1991): "Common Management Information Service Definition for CCITT Applications".

[6]	ITU-T Recommendation X.721 (02/92): "Information Technology - Open Systems Interconnection – Structure of Management Information: Definition of Management Information".

[7]	ITU-T Recommendation X.730 (01/92): "Information Technology - Open Systems Interconnection – Systems Management: Object Management Function".

[8]	ITU-T Recommendation X.733 (02/92): "Information Technology - Open Systems Interconnection - Alarm Reporting Function".

[9]	ITU-T Recommendation M.3100 (07/95): "Maintenance Telecommunications Management Network – Generic Network Information Model".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TS 32.600 and 3GPP TS 32.621-2 apply.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

CMIP	Common Management Information Protocol

DN	Distinguished Name

GDMO	Guidelines for the Definition of Managed Objects

IDL	Interface Definition Language

IEC	International Electro-technical Commission

ISO	International Standards Organization

ITU-T	International Telecommunication Union, Telecommunication Sector

MIB	Management Information Base

MIM	Management Information Model

MIT	Management Information Tree (or Naming Tree)

MOC	Managed Object Class

MOI	Managed Object Instance

NE	Network Element

NR	Network Resource

NRM	Network Resource Model

TMN	Telecommunications Management Network

UTRAN	UMTS Terrestrial Radio Access Network

# Basic aspects

## Explanation

A technology independent CN network resource model is defined in 3GPP TS 32.621-2 for 3G networks. This document provides an implementation of this CN network resource model by using CMIP technology.

## 4.2	Mapping

The semantic of the CN Network Resource Model is defined in 3GPP TS 32.621-2. The specification of the information object classes defined there is independent of any implementation technology and protocol. 
This subclause maps these technology and protocol independent definitions onto the equivalencies of the CMIP Solution Set of the UTRAN Network Resource IRP.

### 4.2.1	Mapping of MOCs

Table 2 maps the information object classes defined in the CN Network Resource Model onto the equivalent MOCs of the CMIP Solution Set.

Table 1: Mapping of MOCs

| Managed Objects of the CN NR IRP NRM | MOCs of this CMIP SS |
| --- | --- |
| AucFunction | AucFunction (3GPP TS 32.106-7: 6.2001) |
| BgFunction | bgFunction (3GPP TS 32.106-7: 6.2001) |
| EirFunction | eirFunction (3GPP TS 32.106-7: 6.2001) |
| GgsnFunction | ggsnFunction (3GPP TS 32.106-7: 6.2001) |
| GmscFunction | gmscFunction (3GPP TS 32.106-7: 6.2001) |
| HlrFunction | hlrFunction (3GPP TS 32.106-7: 6.2001) |
| MscFunction | mscFunction (3GPP TS 32.106-7: 6.2001) |
| SgsnFunction | sgsnFunction (3GPP TS 32.106-7: 6.2001) |
| SmsGmscFunction | smsGmscFunction (3GPP TS 32.106-7: 6.2001) |
| SmsIwmscFunction | smsIwmscFunction (3GPP TS 32.106-7: 6.2001) |
| VlrFunction | vlrFunction (3GPP TS 32.106-7: 6.2001) |
| SmlcFunction | smlcFunction |
| GmlcFunction | gmlcFunction |
| SfcFunction | sfcFunction |
| SrfFunction | srfFunction |
| CbcFunction | cbcFunction |
| CqfFunction | cqfFunction |
| MgwFunction | mgwFunction |
| GmscFunction | gmscFunction |
| IwfFunction | iwfFunction |
| MnpSrfFunction | mnpSrfFunction |
| NpdbFunction | npdbFunction |
| RSgwFunction | rSgwFunction |
| SsfFunction | ssfFunction |
| BsFunction | bsFunction |

### Mapping of Attributes

Table 2: Mapping of Attributes

| Attribute defined in 3GPP TS 32.621-2 | Attribute defined in this CMIP SS |
| --- | --- |
| UserLabel | userLabel (3GPP TS 32.106-7: 6.2001) |
| AucFunctionId | AucFunctionId (3GPP TS 32.106-7: 6.2001) |
| BgFunctionId | bgFunctionId (3GPP TS 32.106-7: 6.2001) |
| eirFunctionId | eirFunctionId (3GPP TS 32.106-7: 6.2001) |
| ggsnFunctionId | ggsnFunctionId (3GPP TS 32.106-7: 6.2001) |
| gmscFunctionId | gmscFunctionId (3GPP TS 32.106-7: 6.2001) |
| hlrFunctionId | hlrFunctionId (3GPP TS 32.106-7: 6.2001) |
| mscFunctionId | mscFunctionId (3GPP TS 32.106-7: 6.2001) |
| vlrFunctionId | vlrFunctionId (3GPP TS 32.106-7: 6.2001) |
| sgsnFunctionId | sgsnFunctionId (3GPP TS 32.106-7: 6.2001) |
| smsGmscFunctionId | smsGmscFunctionId (3GPP TS 32.106-7: 6.2001) |
| smsIwmscFunctionId | smsIwmscFunctionId (3GPP TS 32.106-7: 6.2001) |
| smlcFunctionId | smlcFunctionId |
| gmlcFunctionId | gmlcFunctionId |
| sfcFunctionId | sfcFunctionId |
| srfFunctionId | srfFunctionId |
| cbcFunctionId | cbcFunctionId |
| cqfFunctionId | cqfFunctionId |
| mgwFunctionId | mgwFunctionId |
| gmscFunctionId | gmscFunctionId |
| iwfFunctionId | iwfFunctionId |
| mnpSrfFunctionId | mnpSrfFunctionId |
| npdbFunctionId | npdbFunctionId |
| rSgwFunctionId | rSgwFunctionId |
| ssfFunctionId | ssfFunctionId |
| bsFunctionId | bsFunctionId |

# 5	GDMO Definitions

## Managed Object Classes

### 5.1.1	smlcFunction

smlcFunction MANAGED OBJECT CLASS

DERIVED FROM       “3GPP TS 32.106-7:  6.2001”: managedFunction;

CHARACTERIZED BY

smlcFunctionBasicPackage PACKAGE

BEHAVIOUR smlcFunctionBasicPackageBehaviour;

ATTRIBUTES

smlcFunctionId GET;;;

REGISTERED AS {ts32-621ObjectClass 1};

smlcFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

" This Managed Object Class represents SMLC functionality. For more information about the SMLC, see 3GPP TS 23.002";

### 5.1.2	gmlcFunction

gmlcFunction MANAGED OBJECT CLASS

DERIVED FROM       “3GPP TS 32.106-7:  6.2001”: managedFunction;

CHARACTERIZED BY

gmlcFunctionBasicPackage PACKAGE

BEHAVIOUR gmlcFunctionBasicPackageBehaviour;

ATTRIBUTES

gmlcFunctionId GET;;;

REGISTERED AS {ts32-621ObjectClass 2};

gmlcFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

" This Managed Object Class represents GMLC functionality. For more information about the GMLC, see 3GPP TS 23.002";

### 5.1.3	scfFunction

scfFunction MANAGED OBJECT CLASS

DERIVED FROM       “3GPP TS 32.106-7:  6.2001”: managedFunction;

CHARACTERIZED BY

scfFunctionBasicPackage PACKAGE

BEHAVIOUR scfFunctionBasicPackageBehaviour;

ATTRIBUTES

scfFunctionId GET;;;

REGISTERED AS {ts32-621ObjectClass 3};

scfFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

" This Managed Object Class represents SCF functionality. For more information about the SCF, see 3GPP TS 23.002";

### 5.1.4	srfFunction

srfFunction MANAGED OBJECT CLASS

DERIVED FROM       “3GPP TS 32.106-7:  6.2001”: managedFunction;

CHARACTERIZED BY

scfFunctionBasicPackage PACKAGE

BEHAVIOUR srfFunctionBasicPackageBehaviour;

ATTRIBUTES

srfFunctionId GET;;;

REGISTERED AS {ts32-621ObjectClass 4};

srfFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

" This Managed Object Class represents SRF functionality. For more information about the SRF, see 3GPP TS 23.002";

### 5.1.5	cbcFunction

cbcFunction MANAGED OBJECT CLASS

DERIVED FROM       “3GPP TS 32.106-7:  6.2001”: managedFunction;

CHARACTERIZED BY

cbcFunctionBasicPackage PACKAGE

BEHAVIOUR cbcFunctionBasicPackageBehaviour;

ATTRIBUTES

cbcFunctionId GET;;;

REGISTERED AS {ts32-621ObjectClass 5};

cbcFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

" This Managed Object Class represents SBC functionality. For more information about the SBC, see 3GPP TS 23.002";

### 5.1.6	cgfFunction

cgfFunction MANAGED OBJECT CLASS

DERIVED FROM       “3GPP TS 32.106-7:  6.2001”: managedFunction;

CHARACTERIZED BY

cgfFunctionBasicPackage PACKAGE

BEHAVIOUR cgfFunctionBasicPackageBehaviour;

ATTRIBUTES

cgfFunctionId GET;;;

REGISTERED AS {ts32-621ObjectClass 6};

cgfFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

" This Managed Object Class represents CGF functionality. For more information about the CGF, see 3GPP TS 23.002";

### 5.1.7	mgwFunction

mgwFunction MANAGED OBJECT CLASS

DERIVED FROM       “3GPP TS 32.106-7:  6.2001”: managedFunction;

CHARACTERIZED BY

mgwFunctionBasicPackage PACKAGE

BEHAVIOUR mgwFunctionBasicPackageBehaviour;

ATTRIBUTES

mgwFunctionId GET;;;

REGISTERED AS {ts32-621ObjectClass 7};

mgwFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

" This Managed Object Class represents MGW functionality. For more information about the MGW, see 3GPP TS 23.002";

### 5.1.8	gmscFunction

gmscFunction MANAGED OBJECT CLASS

DERIVED FROM       “3GPP TS 32.106-7:  6.2001”: managedFunction;

CHARACTERIZED BY

scfFunctionBasicPackage PACKAGE

BEHAVIOUR gmscFunctionBasicPackageBehaviour;

ATTRIBUTES

gmscFunctionId GET;;;

REGISTERED AS {ts32-621ObjectClass 8};

gmscFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

" This Managed Object Class represents gmsc functionality. For more information about the gmsc, see 3GPP TS 23.002";

### 5.1.9	iwfFunction

iwfFunction MANAGED OBJECT CLASS

DERIVED FROM       “3GPP TS 32.106-7:  6.2001”: managedFunction;

CHARACTERIZED BY

scfFunctionBasicPackage PACKAGE

BEHAVIOUR iwfFunctionBasicPackageBehaviour;

ATTRIBUTES

iwfFunctionId GET;;;

REGISTERED AS {ts32-621ObjectClass 9};

iwfFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

" This Managed Object Class represents IWF functionality. For more information about the IWF, see 3GPP TS 23.002";

### 5.1.10	mnpSrfFunction

mnpSrfFunction MANAGED OBJECT CLASS

DERIVED FROM       “3GPP TS 32.106-7:  6.2001”: managedFunction;

CHARACTERIZED BY

scfFunctionBasicPackage PACKAGE

BEHAVIOUR mnpSrfFunctionBasicPackageBehaviour;

ATTRIBUTES

mnpSrfFunctionId GET;;;

REGISTERED AS {ts32-621ObjectClass 10};

mnpSrfFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

" This Managed Object Class represents MNPSRF functionality. For more information about the MNPSRF, see 3GPP TS 23.002";

### 5.1.11	npdbFunction

npdbFunction MANAGED OBJECT CLASS

DERIVED FROM       “3GPP TS 32.106-7:  6.2001”: managedFunction;

CHARACTERIZED BY

scfFunctionBasicPackage PACKAGE

BEHAVIOUR npdbFunctionBasicPackageBehaviour;

ATTRIBUTES

npdbFunctionId GET;;;

REGISTERED AS {ts32-621ObjectClass 11};

npdbFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

" This Managed Object Class represents NPDB functionality. For more information about the NPDB, see 3GPP TS 23.002";

### 5.1.12	rSgwFunction

rSgwFunction MANAGED OBJECT CLASS

DERIVED FROM       “3GPP TS 32.106-7:  6.2001”: managedFunction;

CHARACTERIZED BY

scfFunctionBasicPackage PACKAGE

BEHAVIOUR rSgwFunctionBasicPackageBehaviour;

ATTRIBUTES

rSgwFunctionId GET;;;

REGISTERED AS {ts32-621ObjectClass 12};

rSgwFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

" This Managed Object Class represents R-SGW functionality. For more information about the R-SGW, see 3GPP TS 23.002";

### 5.1.13	ssfFunction

ssfFunction MANAGED OBJECT CLASS

DERIVED FROM       “3GPP TS 32.106-7:  6.2001”: managedFunction;

CHARACTERIZED BY

scfFunctionBasicPackage PACKAGE

BEHAVIOUR ssfFunctionBasicPackageBehaviour;

ATTRIBUTES

ssfFunctionId GET;;;

REGISTERED AS {ts32-621ObjectClass 13};

ssfFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

" This Managed Object Class represents SSF functionality. For more information about the SSF, see 3GPP TS 23.002";

### 5.1.14	bsFunction

bsFunction MANAGED OBJECT CLASS

DERIVED FROM       “3GPP TS 32.106-7:  6.2001”: managedFunction;

CHARACTERIZED BY

scfFunctionBasicPackage PACKAGE

BEHAVIOUR bsFunctionBasicPackageBehaviour;

ATTRIBUTES

bsFunctionId GET;;;

REGISTERED AS {ts32-621ObjectClass 14};

bsFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

" This Managed Object Class represents BS functionality. For more information about the BS, see 3GPP TS 23.002";

## Attributes

### smlcFunctionId

smlcFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule.GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

smlcFunctionIdBehaviour;

REGISTERED AS {ts32-621Attribute 1};

smlcFunctionIdBehaviour BEHAVIOUR

DEFINED AS

" This attribute identifies a smlcFunction instance.”;

### gmlcFunctionId

gmlcFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule.GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

gmlcFunctionIdBehaviour;

REGISTERED AS {ts32-621Attribute 2};

gmlcFunctionIdBehaviour BEHAVIOUR

DEFINED AS

" This attribute identifies a gmlcFunction instance.”;

### sfcFunctionId

sfcFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule.GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

sfcFunctionIdBehaviour;

REGISTERED AS {ts32-621Attribute 3};

sfcFunctionIdBehaviour BEHAVIOUR

DEFINED AS

" This attribute identifies a sfcFunction instance.”;

### srfFunctionId

srfFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule.GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

srfFunctionIdBehaviour;

REGISTERED AS {ts32-621Attribute 4};

srfFunctionIdBehaviour BEHAVIOUR

DEFINED AS

" This attribute identifies a srfFunction instance.”;

### CbcFunctionId

CbcFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule.GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

CbcFunctionIdBehaviour;

REGISTERED AS {ts32-621Attribute 5};

CbcFunctionIdBehaviour BEHAVIOUR

DEFINED AS

" This attribute identifies a CbcFunction instance.”;

### CgfFunctionId

CgfFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule.GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

CgfFunctionIdBehaviour;

REGISTERED AS {ts32-621Attribute 6};

CgfFunctionIdBehaviour BEHAVIOUR

DEFINED AS

" This attribute identifies a CgfFunction instance.”;

### mgwFunctionId

mgwFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule.GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

mgwFunctionIdBehaviour;

REGISTERED AS {ts32-621Attribute 7};

mgwFunctionIdBehaviour BEHAVIOUR

DEFINED AS

" This attribute identifies a mgwFunction instance.”;

### gmscFunctionId

gmscFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule.GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

gmscFunctionIdBehaviour;

REGISTERED AS {ts32-621Attribute 8};

gmscFunctionIdBehaviour BEHAVIOUR

DEFINED AS

" This attribute identifies a gmscFunction instance.”;

### iwfFunctionId

iwfFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule.GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

iwfFunctionIdBehaviour;

REGISTERED AS {ts32-621Attribute 9};

iwfFunctionIdBehaviour BEHAVIOUR

DEFINED AS

" This attribute identifies a iwfFunction instance.”;

### mnpSrfFunctionId

mnpSrfFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule.GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

mnpSrfFunctionIdBehaviour;

REGISTERED AS {ts32-621Attribute 10};

mnpSrfFunctionIdBehaviour BEHAVIOUR

DEFINED AS

" This attribute identifies a mnpSrfFunction instance.”;

### npdbFunctionId

npdbFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule.GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

npdbFunctionIdBehaviour;

REGISTERED AS {ts32-621Attribute 11};

npdbFunctionIdBehaviour BEHAVIOUR

DEFINED AS

" This attribute identifies a npdbFunction instance.”;

### rSgwFunctionId

rSgwFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule.GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

rSgwFunctionIdBehaviour;

REGISTERED AS {ts32-621Attribute 12};

rSgwFunctionIdBehaviour BEHAVIOUR

DEFINED AS

" This attribute identifies a rSgwFunction instance.”;

### ssfFunctionId

ssfFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule.GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

ssfFunctionIdBehaviour;

REGISTERED AS {ts32-621Attribute 13};

ssfFunctionIdBehaviour BEHAVIOUR

DEFINED AS

" This attribute identifies a ssfFunction instance.”;

### bsFunctionId

bsFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX    TS32-106-7TypeModule.GeneralObjectId;

MATCHES FOR EQUALITY;

BEHAVIOUR

bsFunctionIdBehaviour;

REGISTERED AS {ts32-621Attribute 14};

bsFunctionIdBehaviour BEHAVIOUR

DEFINED AS

" This attribute identifies a bsFunction instance.”;

## 5.3	Name Binding

### 5.3.1	smlcFunction - managedElement

smlcFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS rncFunction;

NAMED BY SUPERIOR OBJECT CLASS “3GPP TS 32.620-4: 6.2001”: managedElement;

WITH ATTRIBUTE smlcFunctionId;

BEHAVIOUR

smlcFunction-managedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-621NameBinding 1};

smlcFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and

controls a smlcFunction. When automatic instance naming is used, the choice

of name bindings is left as a local matter.";

### 5.3.2	gmlcFunction - managedElement

gmlcFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS rncFunction;

NAMED BY SUPERIOR OBJECT CLASS “3GPP TS 32.620-4: 6.2001”: managedElement;

WITH ATTRIBUTE gmlcFunctionId;

BEHAVIOUR

gmlcFunction-managedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-621NameBinding 2};

gmlcFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and

controls a gmlcFunction. When automatic instance naming is used, the choice

of name bindings is left as a local matter.";

### 5.3.3	sfcFunction - managedElement

sfcFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS rncFunction;

NAMED BY SUPERIOR OBJECT CLASS “3GPP TS 32.620-4: 6.2001”: managedElement;

WITH ATTRIBUTE sfcFunctionId;

BEHAVIOUR

sfcFunction-managedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-621NameBinding 3};

sfcFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and

controls a sfcFunction. When automatic instance naming is used, the choice

of name bindings is left as a local matter.";

### 5.3.4	srfFunction - managedElement

srfFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS rncFunction;

NAMED BY SUPERIOR OBJECT CLASS “3GPP TS 32.620-4: 6.2001”: managedElement;

WITH ATTRIBUTE srfFunctionId;

BEHAVIOUR

srfFunction-managedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-621NameBinding 4};

srfFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and

controls a srfFunction. When automatic instance naming is used, the choice

of name bindings is left as a local matter.";

### 5.3.5	cbcFunction - managedElement

cbcFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS rncFunction;

NAMED BY SUPERIOR OBJECT CLASS “3GPP TS 32.620-4: 6.2001”: managedElement;

WITH ATTRIBUTE cbcFunctionId;

BEHAVIOUR

cbcFunction-managedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-621NameBinding 5};

cbcFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and

controls a cbcFunction. When automatic instance naming is used, the choice

of name bindings is left as a local matter.";

### 5.3.6	cgfFunction - managedElement

cgfFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS rncFunction;

NAMED BY SUPERIOR OBJECT CLASS “3GPP TS 32.620-4: 6.2001”: managedElement;

WITH ATTRIBUTE cgfFunctionId;

BEHAVIOUR

cgfFunction-managedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-621NameBinding 6};

cgfFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and

controls a cgfFunction. When automatic instance naming is used, the choice

of name bindings is left as a local matter.";

### 5.3.7	mgwFunction - managedElement

mgwFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS rncFunction;

NAMED BY SUPERIOR OBJECT CLASS “3GPP TS 32.620-4: 6.2001”: managedElement;

WITH ATTRIBUTE mgwFunctionId;

BEHAVIOUR

mgwFunction-managedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-621NameBinding 7};

mgwFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and

controls a mgwFunction. When automatic instance naming is used, the choice

of name bindings is left as a local matter.";

### 5.3.8	gmscFunction - managedElement

gmscFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS rncFunction;

NAMED BY SUPERIOR OBJECT CLASS “3GPP TS 32.620-4: 6.2001”: managedElement;

WITH ATTRIBUTE gmscFunctionId;

BEHAVIOUR

gmscFunction-managedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-621NameBinding 8};

gmscFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and

controls a gmscFunction. When automatic instance naming is used, the choice

of name bindings is left as a local matter.";

### 5.3.9	iwfFunction - managedElement

iwfFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS rncFunction;

NAMED BY SUPERIOR OBJECT CLASS “3GPP TS 32.620-4: 6.2001”: managedElement;

WITH ATTRIBUTE iwfFunctionId;

BEHAVIOUR

iwfFunction-managedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-621NameBinding 9};

iwfFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and

controls a iwfFunction. When automatic instance naming is used, the choice

of name bindings is left as a local matter.";

### 5.3.10	mnpSrfFunction - managedElement

mnpSrfFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS rncFunction;

NAMED BY SUPERIOR OBJECT CLASS “3GPP TS 32.620-4: 6.2001”: managedElement;

WITH ATTRIBUTE mnpSrfFunctionId;

BEHAVIOUR

mnpSrfFunction-managedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-621NameBinding 10};

mnpSrfFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and

controls a mnpSrfFunction. When automatic instance naming is used, the choice

of name bindings is left as a local matter.";

### 5.3.11	npdbFunction - managedElement

npdbFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS rncFunction;

NAMED BY SUPERIOR OBJECT CLASS “3GPP TS 32.620-4: 6.2001”: managedElement;

WITH ATTRIBUTE npdbFunctionId;

BEHAVIOUR

npdbFunction-managedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-621NameBinding 11};

npdbFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and

controls a npdbFunction. When automatic instance naming is used, the choice

of name bindings is left as a local matter.";

### 5.3.12	rSgwFunction - managedElement

rSgwFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS rncFunction;

NAMED BY SUPERIOR OBJECT CLASS “3GPP TS 32.620-4: 6.2001”: managedElement;

WITH ATTRIBUTE rSgwFunctionId;

BEHAVIOUR

rSgwFunction-managedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-621NameBinding 12};

rSgwFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and

controls a rSgwFunction. When automatic instance naming is used, the choice

of name bindings is left as a local matter.";

### 5.3.13	ssfFunction - managedElement

ssfFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS rncFunction;

NAMED BY SUPERIOR OBJECT CLASS “3GPP TS 32.620-4: 6.2001”: managedElement;

WITH ATTRIBUTE ssfFunctionId;

BEHAVIOUR

ssfFunction-managedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-621NameBinding 13};

ssfFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and

controls a ssfFunction. When automatic instance naming is used, the choice

of name bindings is left as a local matter.";

### 5.3.14	bsFunction - managedElement

bsFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS rncFunction;

NAMED BY SUPERIOR OBJECT CLASS “3GPP TS 32.620-4: 6.2001”: managedElement;

WITH ATTRIBUTE bsFunctionId;

BEHAVIOUR

bsFunction-managedElementBehaviour;

CREATE WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-621NameBinding 14};

bsFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and

controls a bsFunction. When automatic instance naming is used, the choice

of name bindings is left as a local matter.";

# 6	ASN.1 Definitions

TS32-621TypeModule {ccitt (0) identified-organization (4) etsi (0)

mobileDomain (0) umts-Operation-Maintenance (3) ts-32-621 (621)

informationModel (0) asn1Module (2) version1 (1)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

--EXPORTS everything

--IMPORTS

-- 3GPP TS 32.621-4 related Object Identifiers

baseNodeUMTS  OBJECT IDENTIFIER ::= {itu-t(0) identified-organization(4) etsi(0) mobileDomain(0) 				      				umts-Operation-Maintenance(3)}

ts32-621		OBJECT IDENTIFIER ::= { baseNodeUMTS ts-32-621(621)}

ts32-621InfoModel	OBJECT IDENTIFIER ::= { ts32-621 informationModel(0)}

ts32-621ObjectClass  	OBJECT IDENTIFIER ::= { ts32-621InfoModel managedObjectClass(3)}

ts32-621Package		OBJECT IDENTIFIER ::= { ts32-621InfoModel package(4)}

ts32-621Parameter		OBJECT IDENTIFIER ::= { ts32-621InfoModel parameter(5)}

ts32-621NameBinding  	OBJECT IDENTIFIER ::= { ts32-621InfoModel nameBinding(6)}

ts32-621Attribute		OBJECT IDENTIFIER ::= { ts32-621InfoModel attribute(7)}

ts32-621Action		OBJECT IDENTIFIER ::= { ts32-621InfoModel action(9)}

ts32-621Notification	OBJECT IDENTIFIER ::= { ts32-621InfoModel notification(10)}

-- Start of 3gPP SA5 own definitions

END	-- of TS32-621TypeModule

Annex A (informative):
Change history

This annex lists all change requests approved for the present document since the specification was first approved by 3GPP TSG-SA.

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| Jun 2001 | S_12 | SP-010283 | -- | -- | Approved at TSG SA #12 and placed under Change Control | 2.0.0 | 4.0.0 |