3GPP TS 32.634 V6.2.0 (2005-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Configuration Management (CM);

Core network resources Integration Reference Point (IRP): Common Management Information Protocol (CMIP)

Solution Set (SS)

(Release 6)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

UMTS, management, CMIP

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2005, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

Contents

Foreword	8

Introduction	8

1	Scope	9

2	References	9

3	Definitions, symbols and abbreviations	10

3.1	Definitions	10

3.2	Abbreviations	10

4	Basic aspects	10

4.1	Architectural Aspects	10

4.2	Mapping	10

4.2.1	Mapping of Information Object Classes	11

4.2.2	Mapping of Information Object Class Attributes	12

4.2.2.1	Attribute Mapping of the IOC MscServerFunction	12

4.2.2.2	Attribute Mapping of the IOC HlrFunction	12

4.2.2.3	Attribute Mapping of the IOC VlrFunction	12

4.2.2.4	Attribute Mapping of the IOC AucFunction	12

4.2.2.5	Attribute Mapping of the IOC EirFunction	12

4.2.2.6	Attribute Mapping of the IOC SmsIwmscFunction	13

4.2.2.7	Attribute Mapping of the IOC SmsGmscFunction	13

4.2.2.8	Attribute Mapping of the IOC SgsnFunction	13

4.2.2.9	Attribute Mapping of the IOC GgsnFunction	13

4.2.2.10	Attribute Mapping of the IOC BgFunction	13

4.2.2.11	Attribute Mapping of the IOC GmscFunction	14

4.2.2.12	Attribute Mapping of the IOC SmlcFunction	14

4.2.2.13	Attribute Mapping of the IOC GmlcFunction	14

4.2.2.14	Attribute Mapping of the IOC ScfFunction	14

4.2.2.15	Attribute Mapping of the IOC SrfFunction	14

4.2.2.16	Attribute Mapping of the IOC CbcFunction	14

4.2.2.17	Attribute Mapping of the IOC CgfFunction	15

4.2.2.18	Attribute Mapping of the IOC ImsMgwFunction	15

4.2.2.19	Attribute Mapping of the IOC GmscServerFunction	15

4.2.2.20	Attribute Mapping of the IOC IwfFunction	15

4.2.2.21	Attribute Mapping of the IOC MnpSrfFunction	15

4.2.2.22	Attribute Mapping of the IOC NpdbFunction	15

4.2.2.23	Attribute Mapping of the IOC SgwFunction	16

4.2.2.24	Attribute Mapping of the IOC SsfFunction	16

4.2.2.25	Attribute Mapping of the IOC BsFunction	16

4.2.2.26	Attribute Mapping of the IOC IucsLink	16

4.2.2.27	Attribute Mapping of the IOC IupsLink	16

4.2.2.28	Attribute Mapping of the IOC IubcLink	16

4.2.2.29	Attribute Mapping of the IOC ALink	17

4.2.2.30	Attribute Mapping of the IOC GbLink	17

4.2.2.31	Attribute Mapping of the IOC CsMgwFunction	17

4.2.2.32	Attribute Mapping of the IOC ScscfFunction	17

4.2.2.33	Attribute Mapping of the IOC PcscfFunction	17

4.2.2.34	Attribute Mapping of the IOC IcscfFunction	18

4.2.2.35	Attribute Mapping of the IOC SlfFunction	18

4.2.2.36	Attribute Mapping of the IOC BgcfFunction	18

4.2.2.37	Attribute Mapping of the IOC MrfcFunction	18

4.2.2.38	Attribute Mapping of the IOC AsFunction	18

4.2.2.39	Attribute Mapping of the IOC MgcfFunction	18

4.2.2.40	Attribute Mapping of the IOC MrfpFunction	19

4.2.2.41	Attribute Mapping of the IOC LinkAsAuc	19

4.2.2.42	 Attribute Mapping of the IOC LinkAsHlr	19

4.2.2.43	Attribute Mapping of the IOC LinkAsScscf	20

4.2.2.44	Attribute Mapping of the IOC LinkAsSlf	20

4.2.2.45	Attribute Mapping of the IOC LinkBgcfBgcf	20

4.2.2.46	Attribute Mapping of the IOC LinkBgcfImsMgw	21

4.2.2.47	Attribute Mapping of the IOC LinkBgcfScscf	21

4.2.2.48	Attribute Mapping of the IOC LinkHlrScscf	21

4.2.2.49	Attribute Mapping of the IOC LinkIcscfSlf	22

4.2.2.50	Attribute Mapping of the IOC LinkImsMgwMgcf	22

4.2.2.51	Attribute Mapping of the IOC LinkImsMgwScscf	22

4.2.2.52	Attribute Mapping of the IOC LinkMrfcMrfp	23

4.2.2.53	Attribute Mapping of the IOC LinkMrfcScscf	23

4.2.2.54	Attribute Mapping of the IOC LinkScscfScscf	23

4.2.2.55	Attribute Mapping of the IOC LinkScscfSlf	24

4.2.3	Mapping of Name Containments	25

-- 5	GDMO Definitions	26

-- 5.1	Information Object Classes (IOCs)	26

-- 5.1.1	smlcFunction	26

-- 5.1.2	gmlcFunction	26

-- 5.1.3	scfFunction	26

-- 5.1.4	srfFunction	26

-- 5.1.5	cbcFunction	27

-- 5.1.6	cgfFunction	27

-- 5.1.7	imsMgwFunction	27

-- 5.1.8	gmscFunction	27

-- 5.1.9	iwfFunction	28

-- 5.1.10	mnpSrfFunction	28

-- 5.1.11	npdbFunction	28

-- 5.1.12	rSgwFunction	29

-- 5.1.13	ssfFunction	29

-- 5.1.14	bsFunction	29

-- 5.1.15	aucFunction	29

-- 5.1.16	bgFunction	30

-- 5.1.17	eirFunction	30

-- 5.1.18	ggsnFunction	30

-- 5.1.19	hlrFunction	30

-- 5.1.20	mscServerFunction	31

-- 5.1.21	sgsnFunction	31

-- 5.1.22	smsGmscFunction	31

-- 5.1.23	smsIwmscFunction	31

-- 5.1.24	vlrFunction	32

-- 5.1.25	gbLink	32

-- 5.1.26	aLink	32

-- 5.1.27	iucsLink	32

-- 5.1.28	iupsLink	33

-- 5.1.29	iubcLink	33

-- 5.1.30	csMgwFunction	33

-- 5.1.31	gmscServerFunction	34

-- 5.1.32	scscfFunction	34

-- 5.1.33	pcscfFunction	34

-- 5.1.34	icscfFunction	34

-- 5.1.35	slfFunction	35

-- 5.1.36	bgcfFunction	35

-- 5.1.37	mrfcFunction	35

-- 5.1.38	asFunction	35

-- 5.1.39	mgcfFunction	36

-- 5.1.40	mrfpFunction	36

-- 5.1.41	linkAsAuc	36

-- 5.1.42	linkAsHlr	36

-- 5.1.43	linkAsScscf	36

-- 5.1.44	linkAsSlf	36

-- 5.1.45	linkBgcfBgcf	37

-- 5.1.46	linkBgcfImsMgw	37

-- 5.1.47	linkBgcfScscf	37

-- 5.1.48 linkHlrScscf	37

-- 5.1.49	linkIcscfSlf	37

-- 5.1.50	linkImsMgwMgcf	37

-- 5.1.51	linkImsMgwScscf	37

-- 5.1.52	linkMrfcMrfp	37

-- 5.1.53	linkMrfcScscf	37

-- 5.1.54	linkScscfScscf	38

-- 5.1.55	linkScscfSlf	38

-- 5.2	Packages	38

-- 5.2.1	smlcFunctionBasicPackage	38

-- 5.2.2	gmlcFunctionBasicPackage	38

-- 5.2.3	scfFunctionBasicPackage	38

-- 5.2.4	srfFunctionBasicPackage	38

-- 5.2.5	cbcFunctionBasicPackage	39

-- 5.2.6	cgfFunctionBasicPackage	39

-- 5.2.7	imsMgwFunctionBasicPackage	39

-- 5.2.8	gmscFunctionBasicPackage	39

-- 5.2.9	iwfFunctionBasicPackage	39

-- 5.2.10	mnpSrfFunctionBasicPackage	40

-- 5.2.11	npdbFunctionBasicPackage	40

-- 5.2.12	rSgwFunctionBasicPackage	40

-- 5.2.13	ssfFunctionBasicPackage	40

-- 5.2.14	bsFunctionBasicPackage	40

-- 5.2.15	aucFunctionBasicPackage	41

-- 5.2.16	bgFunctionBasicPackage	41

-- 5.2.17	eirFunctionBasicPackage	41

-- 5.2.18	ggsnFunctionBasicPackage	41

-- 5.2.19	hlrFunctionBasicPackage	41

-- 5.2.20	mscServerFunctionBasicPackage	42

-- 5.2.21	mscServerFunctionAssociationPackage	42

-- 5.2.22	sgsnFunctionBasicPackage	42

-- 5.2.23	sgsnFunctionAssociattionPackage	42

-- 5.2.24	smsGmscFunctionBasicPackage	43

-- 5.2.25	smsIwmscFunctionBasicPackage	43

-- 5.2.26	vlrFunctionBasicPackage	43

-- 5.2.27	gbLinkBasicPackage	43

-- 5.2.28	gbLinkAssociationPackage	43

-- 5.2.29	aLinkBasicPackage	44

-- 5.2.30	aLinkAssociationPackage	44

-- 5.2.31	iucsLinkBasicPackage	44

-- 5.2.32	iucsLinkAssociationPackage	44

-- 5.2.33	iupsLinkBasicPackage	44

-- 5.2.34	iupsLinkAssociationPackage	45

-- 5.2.35	iubcLinkBasicPackage	45

-- 5.2.36	iubcLinkAssociationPackage	45

-- 5.2.37	csMgwFunctionBasicPackage	45

-- 5.2.38	csMgwFunctionAssociationPackage	45

-- 5.2.39	gmscServerFunctionBasicPackage	46

-- 5.2.40	scscfFunctionBasicPackage	46

-- 5.2.41	pcscfFunctionBasicPackage	46

-- 5.2.42	icscfFunctionBasicPackage	46

-- 5.2.43	slfFunctionBasicPackage	46

-- 5.2.44	bgcfFunctionBasicPackage	47

-- 5.2.45	mrfcFunctionBasicPackage	47

-- 5.2.46	asFunctionBasicPackage	47

-- 5.2.47	mgcfFunctionBasicPackage	47

-- 5.2.48	mrfpFunctionBasicPackage	47

-- 5.3	Attributes	48

-- 5.3.1	smlcFunctionId	48

-- 5.3.2	gmlcFunctionId	48

-- 5.3.3	scfFunctionId	48

-- 5.3.4	srfFunctionId	48

-- 5.3.5	cbcFunctionId	48

-- 5.3.6	cgfFunctionId	49

-- 5.3.7	imsMgwFunctionId	49

-- 5.3.8	gmscFunctionId	49

-- 5.3.9	iwfFunctionId	49

-- 5.3.10	mnpSrfFunctionId	49

-- 5.3.11	npdbFunctionId	50

-- 5.3.12	rSgwFunctionId	50

-- 5.3.13	ssfFunctionId	50

-- 5.3.14	bsFunctionId	50

-- 5.3.15	aucFunctionId	51

-- 5.3.16	bgFunctionId	51

-- 5.3.17	eirFunctionId	51

-- 5.3.18	ggsnFunctionId	51

-- 5.3.19	hlrFunctionId	51

-- 5.3.20	mscServerFunctionId	52

-- 5.3.21	vlrFunctionId	52

-- 5.3.22	sgsnFunctionId	52

-- 5.3.23	smsGmscFunctionId	52

-- 5.3.24	smsIwmscFunctionId	52

-- 5.3.25	gbLinkId	53

-- 5.3.26	aLinkId	53

-- 5.3.27	iucsLinkId	53

-- 5.3.28	iupsLinkId	53

-- 5.3.29	iubcLinkId	53

-- 5.3.30	csMgwFunctionId	54

-- 5.3.31	gmscServerFunctionId	54

-- 5.3.32	mccList	54

-- 5.3.33	mncList	54

-- 5.3.34	lacList	55

-- 5.3.35	sacList	55

-- 5.3.36	uraList	55

-- 5.3.37	gcaList	55

-- 5.3.38	mscId	55

-- 5.3.39	mscServerFunction-GSMcell	55

-- 5.3.40	mscServerFunction-ExternalGSMcell	56

-- 5.3.41	mscServerFunction-CsMgwFunction	56

-- 5.3.42	racList	56

-- 5.3.43	sgsnId	56

-- 5.3.44	sgsnFunction-GSMcell	57

-- 5.3.45	sgsnFunction-ExternalGSMcell	57

-- 5.3.46	connectedBss	57

-- 5.3.47	connectedRnc	57

-- 5.3.48	csMgwFunction-MscServerFunction	58

-- 5.3.49	csMgwFunction-IucsLink	58

-- 5.3.50	csMgwFunction-Alink	58

-- 5.3.51	scscfFunctionId	58

-- 5.3.52	pcscfFunctionId	58

-- 5.3.53	icscfFunctionId	59

-- 5.3.54	slfFunctionId	59

-- 5.3.55	bgcfFunctionId	59

-- 5.3.56	mrfcFunctionId	59

-- 5.3.57	asFunctionId	60

-- 5.3.58	mgcfFunctionId	60

-- 5.3.59	mrfpFunctionId	60

-- 5.4	Name Binding	60

-- 5.4.1	smlcFunction - managedElement	60

-- 5.4.2	gmlcFunction - managedElement	61

-- 5.4.3	scfFunction - managedElement	61

-- 5.4.4	srfFunction - managedElement	61

-- 5.4.5	cbcFunction - managedElement	61

-- 5.4.6	cgfFunction - managedElement	62

-- 5.4.7	imsMgwFunction - managedElement	62

-- 5.4.8	gmscFunction - managedElement	62

-- 5.4.9	iwfFunction - managedElement	63

-- 5.4.10	mnpSrfFunction - managedElement	63

-- 5.4.11	npdbFunction - managedElement	63

-- 5.4.12	rSgwFunction - managedElement	64

-- 5.4.13	ssfFunction - managedElement	64

-- 5.4.14	bsFunction - managedElement	64

-- 5.4.15	aucFunction - managedElement	65

-- 5.4.16	bgFunction - managedElement	65

-- 5.4.17	eirFunction - managedElement	65

-- 5.4.18	ggsnFunction - managedElement	66

-- 5.4.19	hlrFunction - managedElement	66

-- 5.4.20	mscServerFunction - managedElement	66

-- 5.4.21	vlrFunction - managedElement	66

-- 5.4.22	sgsnFunction - managedElement	67

-- 5.4.23	smsGmscFunction - managedElement	67

-- 5.4.24	smsIwmscFunction - managedElement	67

-- 5.4.25	gbLink - managedElement	68

-- 5.4.26	aLink - managedElement	68

-- 5.4.27	iucsLink - managedElement	68

-- 5.4.28	iupsLink - managedElement	69

-- 5.4.29	iubcLink - managedElement	69

-- 5.4.30	gmscServerFunction - managedElement	69

-- 5.4.31	csMgwFunction - managedElement	70

-- 5.4.32	scscfFunction - managedElement	70

-- 5.4.33	pcscfFunction - managedElement	70

-- 5.4.34	icscfFunction - managedElement	71

-- 5.4.35	slfFunction - managedElement	71

-- 5.4.36	bgcfFunction - managedElement	71

-- 5.4.37	mrfcFunction - managedElement	71

-- 5.4.38	asFunction - managedElement	72

-- 5.4.39	mgcfFunction - managedElement	72

-- 5.4.40	mrfpFunction - managedElement	72

-- 5.4.41	linkAsAuc - subNetworkR60	73

-- 5.4.42	linkAsHlr - subNetworkR60	73

-- 5.4.43	linkAsScscf - subNetworkR60	73

-- 5.4.44	linkAsSlf - subNetworkR60	74

-- 5.4.45	linkBgcfBgcf - subNetworkR60	74

-- 5.4.46	linkBgcfImsMgw - subNetworkR60	74

-- 5.4.47	linkBgcfScscf-subNetworkR60	75

-- 5.4.48	linkHlrScscf-subNetworkR60	75

-- 5.4.49	linkIcscfSlf-subNetworkR60	75

-- 5.4.50	linkImsMgwMgcf-subNetworkR60	76

-- 5.4.51	linkImsMgwScscf-subNetworkR60	76

-- 5.4.52	linkMrfcMrfp-subNetworkR60	76

-- 5.4.53	linkMrfcScscf-subNetworkR60	76

-- 5.4.54	linkScscfScscf-subNetworkR60	77

-- 5.4.55	linkScscfSlf-subNetworkR60	77

-- 6	ASN.1 Definitions	78

Annex A (informative):	List of assigned Object Identifiers	79

Annex B (informative):	Change history	87

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project Technical Specification Group Services and System Aspects, Telecommunication management; Configuration Management (CM); as identified below:

32.631:	"Core network resources Integration Reference Point (IRP): Requirements";

32.632:	"Core network resources Integration Reference Point (IRP): Network Resource Model (NRM)";

32.633:	"Core network resources Integration Reference Point (IRP): Common Object Request Broker Architecture (CORBA) Solution Set (SS)";

32.634:	"Core network resources Integration Reference Point (IRP): Common Management Information Protocol (CMIP) Solution Set (SS)".

32.635:	"Core network resources Integration Reference Point (IRP): Bulk CM eXtensible Markup Language (XML) file format definition".

A 3G network is composed of a multitude of different network elements (NE). For a successful operation of the network the operator must be provided with mechanisms allowing him to manage the network. These management activities can be grouped into several areas: configuration management, fault management, performance management, and accounting management and security management.

The present document is part of a set of technical specifications defining the telecommunication management (TM) of 3G systems. The TM principles are described in 3GPP TS 32.101 [1]. The TM architecture is described in 3GPP TS 32.102 [2]. The other specifications define the interface (Itf-N) between the managing system (manager), which is in general the network manager (NM) and the managed system (agent), which is either an element manager (EM) or the managed NE itself. The Itf-N is composed of a number of integration reference points (IRPs) defining the information in the agent that is visible for the manager, the operations that the manager may perform on this information and the notifications that are sent from the agent to the manager. One of these IRPs is the Core Network Resources IRP.

Each IRP is specified by five TS: Requirements, IS, CORBA SS, CMIP SS and the XML file format definition.

# 1	Scope

The present document specifies the Common Management Information Protocol (CMIP) Solution Set (SS) for the CN Network Resource Integration Reference Point (IRP): Network Resource Model defined in 3GPP TS 32.632 [4]. In detail:

- Clause 4 contains an introduction to some concepts that are the base for some specific aspects of the CMIP interfaces.

- Clause 5 contains the GDMO definitions for the Alarm Management over the CMIP interfaces

- Clause 6 contains the ASN.1 definitions supporting the GDMO definitions provided in clause 5.

This Solution Set specification is related to 3GPP TS 32.632 V6.2.X [4].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	Void.

[4]	3GPP TS 32.632: "Telecommunication management; Configuration Management (CM); Core Network Resources Integration Reference Point (IRP): Network Resource Model (NRM)".

[5]	ITU-T Recommendation X.710 (1991): "Common Management Information Service Definition for CCITT Applications".

[6]	ITU-T Recommendation X.721 (02/92): "Information Technology - Open Systems Interconnection – Structure of Management Information: Definition of Management Information".

[7]	ITU-T Recommendation X.730 (01/92): "Information Technology - Open Systems Interconnection – Systems Management: Object Management Function".

[8]	ITU-T Recommendation X.733 (02/92): "Information Technology - Open Systems Interconnection - Alarm Reporting Function".

[9]	ITU-T Recommendation M.3100 (07/95): "Maintenance Telecommunications Management Network – Generic Network Information Model".

[10]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TS 32.600 [10] and 3GPP TS 32.632 [4] apply.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

CMIP	Common Management Information Protocol

DN	Distinguished Name

GDMO	Guidelines for the Definition of Managed Objects

IDL	Interface Definition Language

IEC	International Electrotechnical Commission

IOC	Information Object Class

ISO	International Standards Organization

ITU-T	International Telecommunication Union, Telecommunication Sector

MGW	Media GateWay

MIB	Management Information Base

MIM	Management Information Model

MIT	Management Information Tree (or Naming Tree)

MOC	Managed Object Class

MOI	Managed Object Instance

NE	Network Element

NR	Network Resource

NRM	Network Resource Model

TMN	Telecommunications Management Network

UTRAN	UMTS Terrestrial Radio Access Network

# 4	Basic aspects

## 4.1	Architectural Aspects

A technology independent CN network resource model is defined in 3GPP TS 32.632 [4] for 3G networks. This document provides an implementation of this CN network resource model by using CMIP technology.

## 4.2	Mapping

The semantic of the CN Network Resource Model is defined in 3GPP TS 32.632 [4]. The specification of the information object classes defined there is independent of any implementation technology and protocol. 
This clause maps these technology and protocol independent definitions onto the equivalencies of the CMIP Solution Set of the UTRAN Network Resource IRP.

### 4.2.1	Mapping of Information Object Classes

Table 4.1 maps the Information Object Classes (IOCs) defined in the CN Network Resource Model onto the equivalent IOCs of the CMIP Solution Set.

Table 4.1: Mapping of IOCs

| IS IOC | CMIP SS MOC |
| --- | --- |
| AucFunction | aucFunction |
| ALink | aLink |
| IucsLink | iucsLink |
| IupsLink | iupsLink |
| IubcLink | iubcLink |
| BgFunction | bgFunction |
| EirFunction | eirFunction |
| GbLink | gbLink |
| GgsnFunction | ggsnFunction |
| GmscFunction | gmscFunction |
| HlrFunction | hlrFunction |
| MscServerFunction | mscServerFunction |
| GmscServerFunction | gmscServerFunction |
| SgsnFunction | sgsnFunction |
| SmsGmscFunction | smsGmscFunction |
| SmsIwmscFunction | smsIwmscFunction |
| VlrFunction | vlrFunction |
| SmlcFunction | smlcFunction |
| GmlcFunction | gmlcFunction |
| ScfFunction | scfFunction |
| SrfFunction | srfFunction |
| CbcFunction | cbcFunction |
| CgfFunction | cqfFunction |
| CsMgwFunction | csMgwFunction |
| ImsMgwFunction | ImsMgwFunction |
| GmscFunction | gmscFunction |
| IwfFunction | iwfFunction |
| MnpSrfFunction | mnpSrfFunction |
| NpdbFunction | npdbFunction |
| SgwFunction | rSgwFunction |
| SsfFunction | ssfFunction |
| BsFunction | bsFunction |
| ScscfFunction | scscfFunction |
| PcscfFunction | pcscfFunction |
| IcscfFunction | icscfFunction |
| SlfFunction | slfFunction |
| BgcfFunction | bgcfFunction |
| MrfcFunction | mrfcFunction |
| AsFunction | asFunction |
| MgcfFunction | mgcfFunction |
| MrfpFunction | mrfpFunction |
| Link_As_Auc | linkAsAuc |
| Link_As_Hlr | linkAsHlr |
| Link_As_Scscf | linkAsScscf |
| Link_As_Slf | linkAsSlf |
| Link_Bgcf_Bgcf | linkBgcfBgcf |
| Link_Bgcf-ImsMgw | linkBgcfImsMgw |
| Link_Bgcf_Scscf | linkBgcfScscf |
| Link_Hlr_Scscf | linkHlrScscf |
| Link_Icscf_Slf | linkIcscfSlf |
| Link_ImsMgw_Mgcf | linkImsMgwMgcf |
| Link_ImsMgw_Scscf | linkImsMgwScscf |
| Link_Mrfc_Mrfp | linkMrfcMrfp |
| Link_Mrfc_Scscf | linkMrfcScscf |
| Link_Scscf_Scscf | linkScscfScscf |
| Link_Scscf_Slf | linkScscfSlf |

### 4.2.2	Mapping of Information Object Class Attributes

#### 4.2.2.1	Attribute Mapping of the IOC MscServerFunction

Attribute mapping of the IOC MscServerFunction

| IS Attribute | CMIP SS Attributes | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| mscServerFunctionId | mscServerFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |
| mccList | mccList | M | M | M |
| mncList | mncList | M | M | M |
| lacList | lacList | M | M | M |
| sacList | sacList | M | M | M |
| gcaList | gcaList | M | M | M |
| mscId | mscId | M | M | M |
| mscServerFunction-GsmCell | mscServerFunction-GSMcell | M | M | -- |
| mscServerFunction-ExternalGsmCell | mscServerFunction-ExternalGSMcell | M | M | -- |
| mscServerFunction-CsMgwFunction | mscServerFunction-CsMgwFunction | M | M | -- |

#### 4.2.2.2	Attribute Mapping of the IOC HlrFunction

Attribute mapping of the IOC HlrFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| hlrFunctionId | hlrFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.3	Attribute Mapping of the IOC VlrFunction

Attribute mapping of the IOC VlrFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| vlrFunctionId | vlrFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.4	Attribute Mapping of the IOC AucFunction

Attribute mapping of the IOC AucFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| aucFunctionId | aucFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.5	Attribute Mapping of the IOC EirFunction

Attribute mapping of the IOC EirFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| eirFunctionId | eirFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.6	Attribute Mapping of the IOC SmsIwmscFunction

Attribute mapping of the IOC SmsIwmscFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| smsIwmscFunctionId | smsIwmscFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.7	Attribute Mapping of the IOC SmsGmscFunction

Attribute mapping of the IOC SmsGmscFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| smsGmscFunctionId | smsGmscFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.8	Attribute Mapping of the IOC SgsnFunction

Attribute mapping of the IOC SgsnFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| sgsnFunctionId | sgsnFunctionId | M | M | -- |
| UserLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |
| mccList | mccList | M | M | M |
| mncList | mncList | M | M | M |
| lacList | lacList | M | M | M |
| racList | racList | M | M | M |
| sacList | sacList | M | M | M |
| sgsnId | sgsnId | M | M | M |
| sgsnFunction-GSMCell | sgsnFunction-GSMCell | M | M | -- |
| sgsnFunction-ExternalGSMCell | sgsnFunction-ExternalGSMCell | M | M | -- |

#### 4.2.2.9	Attribute Mapping of the IOC GgsnFunction

Attribute mapping of the IOC GgsnFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| ggsnFunctionId | ggsnFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.10	Attribute Mapping of the IOC BgFunction

Attribute mapping of the IOC BgFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| bgFunctionId | bgFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.11	Attribute Mapping of the IOC GmscFunction

Attribute mapping of the IOC GmscFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| gmscFunctionId | gmscFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.12	Attribute Mapping of the IOC SmlcFunction

Attribute mapping of the IOC SmlcFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| smlcFunctionId | smlcFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.13	Attribute Mapping of the IOC GmlcFunction

Attribute mapping of the IOC GmlcFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| gmlcFunctionId | gmlcFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.14	Attribute Mapping of the IOC ScfFunction

Attribute mapping of the IOC ScfFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| scfFunctionId | scfFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.15	Attribute Mapping of the IOC SrfFunction

Attribute mapping of the IOC SrfFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| srfFunctionId | srfFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.16	Attribute Mapping of the IOC CbcFunction

Attribute mapping of the IOC CbcFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| cbcFunctionId | cbcFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.17	Attribute Mapping of the IOC CgfFunction

Attribute mapping of the IOC CgfFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| cgfFunctionId | cgfFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.18	Attribute Mapping of the IOC ImsMgwFunction

Attribute mapping of the IOC ImsMgwFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| imsMgwFunctionId | imsMgwFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.19	Attribute Mapping of the IOC GmscServerFunction

Attribute mapping of the IOC GmscServerFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| gmscServerFunctionId | gmscServerFunctionId | M | M | -- |
| userLabel | userLabel(ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.20	Attribute Mapping of the IOC IwfFunction

Attribute mapping of the IOC IwfFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| iwfFunctionId | iwfFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.21	Attribute Mapping of the IOC MnpSrfFunction

Attribute mapping of the IOC MnpSrfFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| mnpSrfFunctionId | mnpSrfFunctionId | M | M | -- |
| userLabel | userLabel(ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.22	Attribute Mapping of the IOC NpdbFunction

Attribute mapping of the IOC NpdbFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| npdbFunctionId | npdbFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.23	Attribute Mapping of the IOC SgwFunction

Attribute mapping of the IOC SgwFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| sgwFunctionId | sgwFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.24	Attribute Mapping of the IOC SsfFunction

Attribute mapping of the IOC SsfFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| ssfFunctionId | ssfFunctionId | M | M | -- |
| userLabel | UserLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.25	Attribute Mapping of the IOC BsFunction

Attribute mapping of the IOC BsFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| bsFunctionId | bsFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.26	Attribute Mapping of the IOC IucsLink

Attribute mapping of the IOC IucsLink

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| iucsLinkId | iucsLinkId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |
| connectedRnc | connectedRnc | M | M | -- |
| connectedBss | connectedBss | M | M | -- |

#### 4.2.2.27	Attribute Mapping of the IOC IupsLink

Attribute mapping of the IOC IupsLink

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| iupsLinkId | iupsLinkId | M | M | -- |
| userLabel | userLabel(ITU-T Rec. M.3100 [9]) | M | M | M |
| connectedRnc | connectedRnc | O | M | -- |
| connectedBss | connectedBss | O | M | -- |

#### 4.2.2.28	Attribute Mapping of the IOC IubcLink

Attribute mapping of the IOC IubcLink

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| iubcLinkId | iubcLinkId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |
| connectedRnc | connectedRnc | M | M | -- |

#### 4.2.2.29	Attribute Mapping of the IOC ALink

Attribute mapping of the IOC ALink

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| aLinkId | aLinkId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |
| connectedBss | connectedBss | M | M | -- |

#### 4.2.2.30	Attribute Mapping of the IOC GbLink

Attribute mapping of the IOC GbLink

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| gbLinkId | gbLinkId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |
| connectedBss | connectedBss | M | M | -- |

#### 4.2.2.31	Attribute Mapping of the IOC CsMgwFunction

Attribute mapping of the IOC CsMgwFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| csMgwFunctionId | CsmgwFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |
| csMgwFunction-MscServerFunction | csMgwFunction-mscServerFunction | M | M | -- |
| csMgwFunction-IucsLink | csMgwFunction-iucsLink | M | M | -- |
| csMgwFunction-ALink | csMgwFunction-ALink | M | M | -- |

#### 4.2.2.32	Attribute Mapping of the IOC ScscfFunction

Attribute mapping of the IOC ScscfFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| scscfFunctionId | ScscfFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.33	Attribute Mapping of the IOC PcscfFunction

Attribute mapping of the IOC PcscfFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| pcscfFunctionId | pcscfFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.34	Attribute Mapping of the IOC IcscfFunction

Attribute mapping of the IOC IcscfFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| icscfFunctionId | icscfFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.35	Attribute Mapping of the IOC SlfFunction

Attribute mapping of the IOC SlfFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| slfFunctionId | slfFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.36	Attribute Mapping of the IOC BgcfFunction

Attribute mapping of the IOC BgcfFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| bgcfFunctionId | bgcfFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.37	Attribute Mapping of the IOC MrfcFunction

Attribute mapping of the IOC MrfcFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| mrfcFunctionId | mrfcFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.38	Attribute Mapping of the IOC AsFunction

Attribute mapping of the IOC AsFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| asFunctionId | asFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.39	Attribute Mapping of the IOC MgcfFunction

Attribute mapping of the IOC MgcfFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| mgcfFunctionId | mgcfFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.40	Attribute Mapping of the IOC MrfpFunction

Attribute mapping of the IOC MrfpFunction

| IS Attribute | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| mrfpFunctionId | mrfpFunctionId | M | M | -- |
| userLabel | userLabel (ITU-T Rec. M.3100 [9]) | M | M | M |

#### 4.2.2.41	Attribute Mapping of the IOC LinkAsAuc

Attribute mapping of the IOC LinkAsAuc

| Attribute Name | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| linkId (see note) | linkId | M | M | - |
| objectClass (see note) | objectClass | M | M | - |
| objectInstance (see note) | objectInstance | M | M | - |
| userLabel (see note) | userLabel | M | M | M |
| aEnd (see note) | aEnd | M | M | - |
| zEnd (see note) | zEnd | M | M | - |
| linkType (see note) | linkType | O | M | - |
| protocolName (see note) | protocolName | O | M | - |
| protocolVersion (see note) | protocolVersion | O | M | - |
| Note: this attribute is inherited from Link |  |  |  |  |

#### 4.2.2.42		Attribute Mapping of the IOC LinkAsHlr

Attribute mapping of the IOC LinkAsHlr

| Attribute Name | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| linkId (see note) | linkId | M | M | - |
| objectClass (see note) | objectClass | M | M | - |
| objectInstance (see note) | objectInstance | M | M | - |
| userLabel (see note) | userLabel | M | M | M |
| aEnd (see note) | aEnd | M | M | - |
| zEnd (see note) | zEnd | M | M | - |
| linkType (see note) | linkType | O | M | - |
| protocolName (see note) | protocolName | O | M | - |
| protocolVersion (see note) | protocolVersion | O | M | - |
| Note: This attribute is inherited from Link |  |  |  |  |

#### 4.2.2.43	Attribute Mapping of the IOC LinkAsScscf

Attribute mapping of the IOC LinkAsScscf

| Attribute Name | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| linkId (see note) | linkId | M | M | - |
| objectClass (see note) | objectClass | M | M | - |
| objectInstance (see note) | objectInstance | M | M | - |
| userLabel (see note) | userLabel | M | M | M |
| aEnd (see note) | aEnd | M | M | - |
| zEnd (see note) | zEnd | M | M | - |
| linkType (see note) | linkType | O | M | - |
| protocolName (see note) | protocolName | O | M | - |
| protocolVersion (see note) | protocolVersion | O | M | - |
| Note: This attribute is inherited from Link |  |  |  |  |

#### 4.2.2.44	Attribute Mapping of the IOC LinkAsSlf

Attribute mapping of the IOC LinkAsSlf

| Attribute Name | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| linkId (see note) | linkId | M | M | - |
| objectClass (see note) | objectClass | M | M | - |
| objectInstance (see note) | objectInstance | M | M | - |
| userLabel (see note) | userLabel | M | M | M |
| aEnd (see note) | aEnd | M | M | - |
| zEnd (see note) | zEnd | M | M | - |
| linkType (see note) | linkType | O | M | - |
| protocolName (see note) | protocolName | O | M | - |
| protocolVersion (see note) | protocolVersion | O | M | - |
| Note: This attribute is inherited from Link |  |  |  |  |

#### 4.2.2.45	Attribute Mapping of the IOC LinkBgcfBgcf

Attribute mapping of the IOC Link LinkBgcfBgcf

| Attribute Name | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| linkId (see note) | linkId | M | M | - |
| objectClass (see note) | objectClass | M | M | - |
| objectInstance (see note) | objectInstance | M | M | - |
| userLabel (see note) | userLabel | M | M | M |
| aEnd (see note) | aEnd | M | M | - |
| zEnd (see note) | zEnd | M | M | - |
| linkType (see note) | linkType | O | M | - |
| protocolName (see note) | protocolName | O | M | - |
| protocolVersion (see note) | protocolVersion | O | M | - |
| Note: This attribute is inherited from Link |  |  |  |  |

#### 4.2.2.46	Attribute Mapping of the IOC LinkBgcfImsMgw

Attribute mapping of the IOC LinkBgcfImsMgw

| Attribute Name | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| linkId (see note) | linkId | M | M | - |
| objectClass (see note) | objectClass | M | M | - |
| objectInstance (see note) | objectInstance | M | M | - |
| userLabel (see note) | userLabel | M | M | M |
| aEnd (see note) | aEnd | M | M | - |
| zEnd (see note) | zEnd | M | M | - |
| linkType (see note) | linkType | O | M | - |
| protocolName (see note) | protocolName | O | M | - |
| protocolVersion (see note) | protocolVersion | O | M | - |
| Note: This attribute is inherited from Link |  |  |  |  |

#### 4.2.2.47	Attribute Mapping of the IOC LinkBgcfScscf

Attribute mapping of the IOC LinkBgcfScscf

| Attribute Name | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| linkId (see note) | linkId | M | M | - |
| objectClass (see note) | objectClass | M | M | - |
| objectInstance (see note) | objectInstance | M | M | - |
| userLabel (see note) | userLabel | M | M | M |
| aEnd (see note) | aEnd | M | M | - |
| zEnd (see note) | zEnd | M | M | - |
| linkType (see note) | linkType | O | M | - |
| protocolName (see note) | protocolName | O | M | - |
| protocolVersion (see note) | protocolVersion | O | M | - |
| Note: This attribute is inherited from Link |  |  |  |  |

#### 4.2.2.48	Attribute Mapping of the IOC LinkHlrScscf

Attribute mapping of the IOC LinkHlrScscf

| Attribute Name | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| linkId (see note) | linkId | M | M | - |
| objectClass (see note) | objectClass | M | M | - |
| objectInstance (see note) | objectInstance | M | M | - |
| userLabel (see note) | userLabel | M | M | M |
| aEnd (see note) | aEnd | M | M | - |
| zEnd (see note) | zEnd | M | M | - |
| linkType (see note) | linkType | O | M | - |
| protocolName (see note) | protocolName | O | M | - |
| protocolVersion (see note) | protocolVersion | O | M | - |
| Note: This attribute is inherited from Link |  |  |  |  |

#### 4.2.2.49	Attribute Mapping of the IOC LinkIcscfSlf

Attribute mapping of the IOC LinkIcscfSlf

| Attribute Name | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| linkId (see note) | linkId | M | M | - |
| objectClass (see note) | objectClass | M | M | - |
| objectInstance (see note) | objectInstance | M | M | - |
| userLabel (see note) | userLabel | M | M | M |
| aEnd (see note) | aEnd | M | M | - |
| zEnd (see note) | zEnd | M | M | - |
| linkType (see note) | linkType | O | M | - |
| protocolName (see note) | protocolName | O | M | - |
| protocolVersion (see note) | protocolVersion | O | M | - |
| Note: This attribute is inherited from Link |  |  |  |  |

#### 4.2.2.50	Attribute Mapping of the IOC LinkImsMgwMgcf

Attribute mapping of the IOC LinkImsMgwMgcf

| Attribute Name | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| linkId (see note) | linkId | M | M | - |
| objectClass (see note) | objectClass | M | M | - |
| objectInstance (see note) | objectInstance | M | M | - |
| userLabel (see note) | userLabel | M | M | M |
| aEnd (see note) | aEnd | M | M | - |
| zEnd (see note) | zEnd | M | M | - |
| linkType (see note) | linkType | O | M | - |
| protocolName (see note) | protocolName | O | M | - |
| protocolVersion (see note) | protocolVersion | O | M | - |
| Note: This attribute is inherited from Link |  |  |  |  |

#### 4.2.2.51	Attribute Mapping of the IOC LinkImsMgwScscf

Attribute mapping of the IOC LinkImsMgwScscf

| Attribute Name | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| linkId (see note) | linkId | M | M | - |
| objectClass (see note) | objectClass | M | M | - |
| objectInstance (see note) | objectInstance | M | M | - |
| userLabel (see note) | userLabel | M | M | M |
| aEnd (see note) | aEnd | M | M | - |
| zEnd (see note) | zEnd | M | M | - |
| linkType (see note) | linkType | O | M | - |
| protocolName (see note) | protocolName | O | M | - |
| protocolVersion (see note) | protocolVersion | O | M | - |
| Note: This attribute is inherited from Link |  |  |  |  |

#### 4.2.2.52	Attribute Mapping of the IOC LinkMrfcMrfp

Attribute mapping of the IOC LinkMrfcMrfp

| Attribute Name | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| linkId (see note) | linkId | M | M | - |
| objectClass (see note) | objectClass | M | M | - |
| objectInstance (see note) | objectInstance | M | M | - |
| userLabel (see note) | userLabel | M | M | M |
| aEnd (see note) | aEnd | M | M | - |
| zEnd (see note) | zEnd | M | M | - |
| linkType (see note) | linkType | O | M | - |
| protocolName (see note) | protocolName | O | M | - |
| protocolVersion (see note) | protocolVersion | O | M | - |
| Note: This attribute is inherited from Link |  |  |  |  |

#### 4.2.2.53	Attribute Mapping of the IOC LinkMrfcScscf

Attribute mapping of the IOC LinkMrfcScscf

| Attribute Name | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| linkId (see note) | linkId | M | M | - |
| objectClass (see note) | objectClass | M | M | - |
| objectInstance (see note) | objectInstance | M | M | - |
| userLabel (see note) | userLabel | M | M | M |
| aEnd (see note) | aEnd | M | M | - |
| zEnd (see note) | zEnd | M | M | - |
| linkType (see note) | linkType | O | M | - |
| protocolName (see note) | protocolName | O | M | - |
| protocolVersion (see note) | protocolVersion | O | M | - |
| Note: This attribute is inherited from Link |  |  |  |  |

#### 4.2.2.54	Attribute Mapping of the IOC LinkScscfScscf

Attribute mapping of the IOC LinkScscfScscf

| Attribute Name | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| linkId (see note) | linkId | M | M | - |
| objectClass (see note) | objectClass | M | M | - |
| objectInstance (see note) | objectInstance | M | M | - |
| userLabel (see note) | userLabel | M | M | M |
| aEnd (see note) | aEnd | M | M | - |
| zEnd (see note) | zEnd | M | M | - |
| linkType (see note) | linkType | O | M | - |
| protocolName (see note) | protocolName | O | M | - |
| protocolVersion (see note) | protocolVersion | O | M | - |
| Note: This attribute is inherited from Link |  |  |  |  |

#### 4.2.2.55	Attribute Mapping of the IOC LinkScscfSlf

Attribute mapping of the IOC LinkScscfSlf

| Attribute Name | CMIP SS Attribute | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| linkId (see note) | linkId | M | M | - |
| objectClass (see note) | objectClass | M | M | - |
| objectInstance (see note) | objectInstance | M | M | - |
| userLabel (see note) | userLabel | M | M | M |
| aEnd (see note) | aEnd | M | M | - |
| zEnd (see note) | zEnd | M | M | - |
| linkType (see note) | linkType | O | M | - |
| protocolName (see note) | protocolName | O | M | - |
| protocolVersion (see note) | protocolVersion | O | M | - |
| Note: This attribute is inherited from Link |  |  |  |  |

### 4.2.3	Mapping of Name Containments

Mapping of name containments

| IS Name Containment | CMIP SS Name Binding |
| --- | --- |
| smlcFunction - managedElement | smlcFunction-managedElement |
| gmlcFunction - managedElement | gmlcFunction-managedElement |
| scfFunction - managedElement | scfFunction-managedElement |
| srfFunction - managedElement | srfFunction-managedElement |
| cbcFunction - managedElement | cbcFunction-managedElement |
| cbcFunction - managedElement | cbcFunction-managedElement |
| cgfFunction - managedElement | cgfFunction-managedElement |
| imsMgwFunction - managedElement | imsMgwFunction-managedElement |
| gmscFunction - managedElement | gmscFunction-managedElement |
| iwfFunction - managedElement | iwfFunction-managedElement |
| mnpSrfFunction - managedElement | mnpSrfFunction-managedElement |
| npdbFunction - managedElement | npdbFunction-managedElement |
| rSgwFunction - managedElement | rSgwFunction-managedElement |
| ssfFunction - managedElement | ssfFunction-managedElement |
| bsFunction - managedElement | bsFunction-managedElement |
| bsFunction - managedElement | bsFunction-managedElement |
| bsFunction - managedElement | bsFunction-managedElement |
| aucFunction - managedElement | aucFunction-managedElement |
| bgFunction - managedElement | bgFunction-managedElement |
| eirFunction - managedElement | eirFunction-managedElement |
| ggsnFunction - managedElement | ggsnFunction-managedElement |
| hlrFunction - managedElement | hlrFunction-managedElement |
| mscServerFunction - managedElement | mscServerFunction-managedElement |
| vlrFunction - managedElement | vlrFunction-managedElement |
| sgsnFunction - managedElement | sgsnFunction-managedElement |
| smsGmscFunction - managedElement | smsGmscFunction-managedElement |
| smsIwmscFunction - managedElement | smsIwmscFunction-managedElement |
| gbLink - managedElement | gbLink-managedElement |
| aLink - managedElement | aLink-managedElement |
| iucsLink - managedElement | iucsLink-managedElement |
| iupsLink - managedElement | iupsLink-managedElement |
| iubcLink - managedElement | iubcLink-managedElement |
| gmscServerFunction - managedElement | gmscServerFunction-managedElement |
| csMgwFunction - managedElement | csMgwFunction-managedElement |
| scscfFunction - managedElement | scscfFunction-managedElement |
| pcscfFunction - managedElement | pcscfFunction-managedElement |
| icscfFunction - managedElement | icscfFunction-managedElement |
| slfFunction - managedElement | slfFunction-managedElement |
| bgcfFunction - managedElement | bgcfFunction-managedElement |
| mrfcFunction - managedElement | mrfcFunction-managedElement |
| asFunction - managedElement | asFunction-managedElement |
| mgcfFunction - managedElement | mgcfFunction-managedElement |
| mrfpFunction - managedElement | mrfpFunction-managedElement |
| link_As_Auc - subNetworkR60 | linkAsAuc-subNetworkR60 |
| link_As_Hlr - subNetworkR60 | linkAs-Hlr-subNetworkR60 |
| link_As_Scscf - subNetworkR60 | linkAsScscf-subNetworkR60 |
| link_As_Slf - subNetworkR60 | linkAsSlf-subNetworkR60 |
| link_Bgcf_Bgcf - subNetworkR60 | linkBgcfBgcf-subNetworkR60 |
| link_Bgcf-ImsMgw - subNetworkR60 | linkBgcfImsMgw-subNetworkR60 |
| link_Bgcf_Scscf - subNetworkR60 | linkBgcfScscf-subNetworkR60 |
| link_Hlr_Scscf - subNetworkR60 | linkHlrScscf-subNetworkR60 |
| link_Icscf_Slf - subNetworkR60 | linkIcscfSlf-subNetworkR60 |
| link_ImsMgw_Mgcf - subNetworkR60 | linkImsMgwMgcf-subNetworkR60 |
| link_ImsMgw_Scscf - subNetworkR60 | linkImsMgwScscf-subNetworkR60 |
| link_Mrfc_Mrfp - subNetworkR60 | linkMrfcMrfp-subNetworkR60 |
| link_Mrfc_Scscf - subNetworkR60 | linkMrfcScscf-suNnetworkR60 |
| link_Scscf_Scscf - subNetworkR60 | linkScscfScscf-suNnetworkR60 |
| link_Scscf_Slf - subNetworkR60 | linkScscfSlf-subNetworkR60 |

# -- 5	GDMO Definitions

## -- 5.1	Information Object Classes (IOCs)

### -- 5.1.1	smlcFunction

smlcFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

smlcFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 1};

### -- 5.1.2	gmlcFunction

gmlcFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

gmlcFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 2};

### -- 5.1.3	scfFunction

scfFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

scfFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 3};

### -- 5.1.4	srfFunction

srfFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

srfFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 4};

### -- 5.1.5	cbcFunction

cbcFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

cbcFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 5};

### -- 5.1.6	cgfFunction

cgfFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

cgfFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 6};

### -- 5.1.7	imsMgwFunction

imsMgwFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

imsMgwFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 70503};

### -- 5.1.8	gmscFunction

gmscFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

gmscFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 8};

### -- 5.1.9	iwfFunction

iwfFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

iwfFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 9};

### -- 5.1.10	mnpSrfFunction

mnpSrfFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

mnpSrfFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 10};

### -- 5.1.11	npdbFunction

npdbFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

npdbFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 11};

### -- 5.1.12	rSgwFunction

rSgwFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

rSgwFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 12};

### -- 5.1.13	ssfFunction

ssfFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

ssfFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 13};

### -- 5.1.14	bsFunction

bsFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

bsFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 14};

### -- 5.1.15	aucFunction

aucFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

aucFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 15};

### -- 5.1.16	bgFunction

bgFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

bgFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 16};

### -- 5.1.17	eirFunction

eirFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

eirFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 17};

### -- 5.1.18	ggsnFunction

ggsnFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

ggsnFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 18};

### -- 5.1.19	hlrFunction

hlrFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

hlrFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 19};

### -- 5.1.20	mscServerFunction

mscFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

mscServerFunctionBasicPackage,

mscServerFunctionAssociationPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 20};

### -- 5.1.21	sgsnFunction

sgsnFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

sgsnFunctionBasicPackage,

sgsnFunctionAssociationPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 21};

### -- 5.1.22	smsGmscFunction

smsGmscFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

smsGmscFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 22};

### -- 5.1.23	smsIwmscFunction

smsIwmscFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

smsIwmscFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 23};

### -- 5.1.24	vlrFunction

vlrFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

vlrFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 24};

### -- 5.1.25	gbLink

gbLink MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

gbLinkBasicPackage,

gbLinkAssociationPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 25};

### -- 5.1.26	aLink

aLink MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

aLinkBasicPackage,

aLinkAssociationPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 26};

### -- 5.1.27	iucsLink

iucsLink MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

iucsLinkBasicPackage,

iucsLinkAssociationPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 27};

### -- 5.1.28	iupsLink

iupsLink MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

iupsLinkBasicPackage,

iupsLinkAssociationPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 28};

### -- 5.1.29	iubcLink

iubcLink MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

iubcLinkBasicPackage,

iubcLinkAssociationPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 29};

### -- 5.1.30	csMgwFunction

csMgwFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

csMgwFunctionBasicPackage,

csMgwFunctionAssociationPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 30};

### -- 5.1.31	gmscServerFunction

gmscServerFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

gmscServerFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 31};

### -- 5.1.32	scscfFunction

scscfFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

scscfFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 32};

### -- 5.1.33	pcscfFunction

pcscfFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

pcscfFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 33};

### -- 5.1.34	icscfFunction

icscfFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

icscfFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 34};

### -- 5.1.35	slfFunction

slfFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

slfFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 35};

### -- 5.1.36	bgcfFunction

bgcfFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

bgcfFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 36};

### -- 5.1.37	mrfcFunction

mrfcFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

mrfcFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 37};

### -- 5.1.38	asFunction

asFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

asFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 38};

### -- 5.1.39	mgcfFunction

mgcfFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

mgcfFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 39};

### -- 5.1.40	mrfpFunction

mrfpFunction MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624": managedFunction;

CHARACTERIZED BY

mrfpFunctionBasicPackage,

"3GPP TS 32.111-4": x721AlarmNotificationsPackage;

CONDITIONAL PACKAGES

"Rec. M.3100: 1995":createDeleteNotificationsPackage

PRESENT IF

"the objectCreation and the objectDeletion notifications defined in

ITU-T Rec. X.721 are supported by an instance of this class.",

"Rec. M.3100: 1995":attributeValueChangeNotificationPackage

PRESENT IF

"the attributeValueChange notification defined in ITU-T Rec. X.721

is supported by an instance of this class.";

REGISTERED AS {ts32-634ObjectClass 40};

### -- 5.1.41	linkAsAuc

linkAsAuc MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624":link;

REGISTERED AS {ts32-634ObjectClass 410620};

### -- 5.1.42	linkAsHlr

linkAsHlr MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624":link;

REGISTERED AS {ts32-634ObjectClass 420620};

### -- 5.1.43	linkAsScscf

linkAsScscf MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624":link;

REGISTERED AS {ts32-634ObjectClass 430620};

### -- 5.1.44	linkAsSlf

linkAsSlf MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624":link;

REGISTERED AS {ts32-634ObjectClass 440620};

### -- 5.1.45	linkBgcfBgcf

linkBgcfBgcf MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624":link;

REGISTERED AS {ts32-634ObjectClass 450620};

### -- 5.1.46	linkBgcfImsMgw

linkBgcfImsMgw MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624":link;

REGISTERED AS {ts32-634ObjectClass 460620};

### -- 5.1.47	linkBgcfScscf

linkBgcfScscf MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624":link;

REGISTERED AS {ts32-634ObjectClass 470620};

### -- 5.1.48 linkHlrScscf

linkHlrScscf MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624":link;

REGISTERED AS {ts32-634ObjectClass 480620};

### -- 5.1.49	linkIcscfSlf

linkIcscfSlf MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624":link;

REGISTERED AS {ts32-634ObjectClass 490620};

### -- 5.1.50	linkImsMgwMgcf

linkImsMgwMgcf MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624":link;

REGISTERED AS {ts32-634ObjectClass 500620};

### -- 5.1.51	linkImsMgwScscf

linkImsMgwScscf MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624":link;

REGISTERED AS {ts32-634ObjectClass 510620};

### -- 5.1.52	linkMrfcMrfp

linkMrfcMrfp MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624":link;

REGISTERED AS {ts32-634ObjectClass 520620};

### -- 5.1.53	linkMrfcScscf

linkMrfcScscf MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624":link;

REGISTERED AS {ts32-634ObjectClass 530620};

### -- 5.1.54	linkScscfScscf

linkScscfScscf MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624":link;

REGISTERED AS {ts32-634ObjectClass 540620};

### -- 5.1.55	linkScscfSlf

linkScscfSlf MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS 32.624":link;

REGISTERED AS {ts32-634ObjectClass 550620};

## -- 5.2	Packages

### -- 5.2.1	smlcFunctionBasicPackage

smlcFunctionBasicPackage PACKAGE

BEHAVIOUR

smlcFunctionBasicPackageBehaviour;

ATTRIBUTES

smlcFunctionId      GET;

REGISTERED AS {ts32-634Package 1};

smlcFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘SmlcFunction’ Information Object represents the SMLC functionality. For more information

about the SMLC, see 3GPP TS 23.002";

### -- 5.2.2	gmlcFunctionBasicPackage

gmlcFunctionBasicPackage PACKAGE

BEHAVIOUR

gmlcFunctionBasicPackageBehaviour;

ATTRIBUTES

gmlcFunctionId      GET;

REGISTERED AS {ts32-634Package 2};

gmlcFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘GmlcFunction’ Information Object represents the GMLC functionality. For more information

about the GMLC, see 3GPP TS 23.002";

### -- 5.2.3	scfFunctionBasicPackage

scfFunctionBasicPackage PACKAGE

BEHAVIOUR

scfFunctionBasicPackageBehaviour;

ATTRIBUTES

scfFunctionId      GET;

REGISTERED AS {ts32-634Package 3};

scfFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘ScfFunction’ Information Object represents the SCF functionality. For more information

about the SCF, see 3GPP TS 23.002";

### -- 5.2.4	srfFunctionBasicPackage

srfFunctionBasicPackage PACKAGE

BEHAVIOUR

srfFunctionBasicPackageBehaviour;

ATTRIBUTES

srfFunctionId      GET;

REGISTERED AS {ts32-634Package 4};

srfFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘SrfFunction’ Information Object represents the SRF functionality. For more information

about the SRF, see 3GPP TS 23.002";

### -- 5.2.5	cbcFunctionBasicPackage

cbcFunctionBasicPackage PACKAGE

BEHAVIOUR

cbcFunctionBasicPackageBehaviour;

ATTRIBUTES

cbcFunctionId      GET;

REGISTERED AS {ts32-634Package 5};

cbcFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘CbcFunction’ Information Object represents the SBC functionality. For more

information about the SBC, see 3GPP TS 23.002";

### -- 5.2.6	cgfFunctionBasicPackage

cgfFunctionBasicPackage PACKAGE

BEHAVIOUR

cgfFunctionBasicPackageBehaviour;

ATTRIBUTES

cgfFunctionId      GET;

REGISTERED AS {ts32-634Package 6};

cgfFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘CgfFunction’ Information Object represents the CGF functionality. For more information

about the CGF, see 3GPP TS 23.002";

### -- 5.2.7	imsMgwFunctionBasicPackage

imsMgwFunctionBasicPackage PACKAGE

BEHAVIOUR

imsMgwFunctionBasicPackageBehaviour;

ATTRIBUTES

imsMgwFunctionId      GET;

REGISTERED AS {ts32-634Package 70503};

imsMgwFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘imsMgwFunction’ Information Object represents the IMS-MGW functionality. For more

information about the IMS-MGW, see 3GPP TS 23.002";

### -- 5.2.8	gmscFunctionBasicPackage

gmscFunctionBasicPackage PACKAGE

BEHAVIOUR

gmscFunctionBasicPackageBehaviour;

ATTRIBUTES

gmscFunctionId      GET;

REGISTERED AS {ts32-634Package 8};

gmscFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘GmscFunction’ Information Object represents the GMSC functionality. For more information

about the GMSC, see 3GPP TS 23.002";

### -- 5.2.9	iwfFunctionBasicPackage

iwfFunctionBasicPackage PACKAGE

BEHAVIOUR

iwfFunctionBasicPackageBehaviour;

ATTRIBUTES

iwfFunctionId      GET;

REGISTERED AS {ts32-634Package 9};

iwfFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘IwfFunction’ Information Object represents the IWF functionality. For more information

about the IWF, see 3GPP TS 23.002";

### -- 5.2.10	mnpSrfFunctionBasicPackage

mnpSrfFunctionBasicPackage PACKAGE

BEHAVIOUR

mnpSrfFunctionBasicPackageBehaviour;

ATTRIBUTES

mnpSrfFunctionId  GET;

REGISTERED AS {ts32-634Package 10};

mnpSrfFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘MnpSrfFunction’ Information Object represents the MNPSRF functionality. For more

information about the MNPSRF, see 3GPP TS 23.002";

### -- 5.2.11	npdbFunctionBasicPackage

npdbFunctionBasicPackage PACKAGE

BEHAVIOUR

npdbFunctionBasicPackageBehaviour;

ATTRIBUTES

npdbFunctionId  GET;

REGISTERED AS {ts32-634Package 11};

npdbFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘NpdbFunction’ Information Object represents the NPDB functionality. For more information

about the NPDB, see 3GPP TS 23.002";

### -- 5.2.12	rSgwFunctionBasicPackage

rSgwFunctionBasicPackage PACKAGE

BEHAVIOUR

rSgwFunctionBasicPackageBehaviour;

ATTRIBUTES

rSgwFunctionId      GET;

REGISTERED AS {ts32-634Package 12};

rSgwFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘RSgwFunction’ Information Object represents the R-SGW functionality. For more information

about the R-SGW, see 3GPP TS 23.002";

### -- 5.2.13	ssfFunctionBasicPackage

ssfFunctionBasicPackage PACKAGE

BEHAVIOUR

ssfFunctionBasicPackageBehaviour;

ATTRIBUTES

ssfFunctionId      GET;

REGISTERED AS {ts32-634Package 13};

ssfFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘SsfFunction’ Information Object represents the SSF functionality. For more information

about the SSF, see 3GPP TS 23.002";

### -- 5.2.14	bsFunctionBasicPackage

bsFunctionBasicPackage PACKAGE

BEHAVIOUR

bsFunctionBasicPackageBehaviour;

ATTRIBUTES

bsFunctionId      GET;

REGISTERED AS {ts32-634Package 14};

bsFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘BsFunction’ Information Object represents the BS functionality. For more information about

the BS, see 3GPP TS 23.002";

### -- 5.2.15	aucFunctionBasicPackage

aucFunctionBasicPackage PACKAGE

BEHAVIOUR

aucFunctionBasicPackageBehaviour;

ATTRIBUTES

aucFunctionId      GET;

REGISTERED AS {ts32-634Package 15};

aucFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘aucFunction’ Information Object represents the AUC functionality. For more information

about the AUC, see 3GPP TS 23.002";

### -- 5.2.16	bgFunctionBasicPackage

bgFunctionBasicPackage PACKAGE

BEHAVIOUR

bgFunctionBasicPackageBehaviour;

ATTRIBUTES

bgFunctionId      GET;

REGISTERED AS {ts32-634Package 16};

bgFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘bgFunction’ Information Object represents the BG functionality. For more information about

the BG, see 3GPP TS 23.002";

### -- 5.2.17	eirFunctionBasicPackage

eirFunctionBasicPackage PACKAGE

BEHAVIOUR

eirFunctionBasicPackageBehaviour;

ATTRIBUTES

eirFunctionId  GET;

REGISTERED AS {ts32-634Package 17};

eirFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘EirFunction’ Information Object represents the EIR functionality. For more information

about the EIR, see 3GPP TS 23.002";

### -- 5.2.18	ggsnFunctionBasicPackage

ggsnFunctionBasicPackage PACKAGE

BEHAVIOUR

ggsnFunctionBasicPackageBehaviour;

ATTRIBUTES

ggsnFunctionId  GET;

REGISTERED AS {ts32-634Package 18};

ggsnFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘GGSNFunction’ Information Object represents the GGSN functionality. For more information

about the GGSN, see 3GPP TS 23.002";

### -- 5.2.19	hlrFunctionBasicPackage

hlrFunctionBasicPackage PACKAGE

BEHAVIOUR

hlrFunctionBasicPackageBehaviour;

ATTRIBUTES

hlrFunctionId      GET;

REGISTERED AS {ts32-634Package 19};

hlrFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘HLRFunction’ Information Object represents the HLR functionality. For more information

about the HLR, see 3GPP TS 23.002";

### -- 5.2.20	mscServerFunctionBasicPackage

mscServerFunctionBasicPackage PACKAGE

BEHAVIOUR

mscServerFunctionBasicPackageBehaviour;

ATTRIBUTES

mscServerFunctionId      GET,

mccList                  GET-REPLACE,

mncList                  GET-REPLACE,

lacList                  GET-REPLACE,

sacList                  GET-REPLACE,

gcaList                  GET-REPLACE,

mscId                    GET-REPLACE;

REGISTERED AS {ts32-634Package 20};

mscServerFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘MSCServerFunction’ Information Object represents the MSCServer functionality.

For more information about the MSCServer, see 3GPP TS 23.002";

### -- 5.2.21	mscServerFunctionAssociationPackage

mscServerFunctionAssociationPackage PACKAGE

BEHAVIOUR

mscServerFunctionAssociationPackageBehaviour;

ATTRIBUTES

mscServerFunction-GSMcell              GET,

mscServerFunction-ExternalGSMcell      GET,

mscServerFunction-CsMgwFunction        GET;

REGISTERED AS {ts32-634Package 21};

mscServerFunctionAssociationPackageBehaviour BEHAVIOUR

DEFINED AS

"This Package contains the attributes of an ‘MscServerFunction’ information object in

relation with associations to GsmCell, ExternalGsmCell and CsMgwFunction information objects";

### -- 5.2.22	sgsnFunctionBasicPackage

sgsnFunctionBasicPackage PACKAGE

BEHAVIOUR

sgsnFunctionBasicPackageBehaviour;

ATTRIBUTES

sgsnFunctionId      GET,

mccList             GET-REPLACE,

mncList             GET-REPLACE,

lacList             GET-REPLACE,

racList             GET-REPLACE,

sgsnId              GET-REPLACE,

mscId               GET-REPLACE;

REGISTERED AS {ts32-634Package 22};

sgsnFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘sgsnFunction’ Information Object represents the SGSN functionality. For more information

about the SGSN, see 3GPP TS 23.002";

### -- 5.2.23	sgsnFunctionAssociattionPackage

sgsnFunctionAssociationPackage PACKAGE

BEHAVIOUR

sgsnFunctionAssociationPackageBehaviour;

ATTRIBUTES

sgsnFunction-GSMcell              GET,

sgsnFunction-ExternalGSMcell      GET;

REGISTERED AS {ts32-634Package 23};

sgsnFunctionAssociationPackageBehaviour BEHAVIOUR

DEFINED AS

"This Package contains the attributes of an ‘SGSNFunction’ information object in relation

with associations to GsmCell and ExternalGsmCell information objects.";

### -- 5.2.24	smsGmscFunctionBasicPackage

smsGmscFunctionBasicPackage PACKAGE

BEHAVIOUR

smsGmscFunctionBasicPackageBehaviour;

ATTRIBUTES

smsGmscFunctionId      GET;

REGISTERED AS {ts32-634Package 24};

smsGmscFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘smsGmscFunction’ Information Object represents the SMS-GMSC functionality.

For more information about the SMS-GMSC, see 3GPP TS 23.002";

### -- 5.2.25	smsIwmscFunctionBasicPackage

smsIwmscFunctionBasicPackage PACKAGE

BEHAVIOUR

smsIwmscFunctionBasicPackageBehaviour;

ATTRIBUTES

smsIwmscFunctionId      GET;

REGISTERED AS {ts32-634Package 25};

smsIwmscFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘smsIwmscFunction’ Information Object represents the SmsIwMSC functionality.

For more information about the SmsIwMsc, see 3GPP TS 23.002";

### -- 5.2.26	vlrFunctionBasicPackage

vlrFunctionBasicPackage PACKAGE

BEHAVIOUR

vlrFunctionBasicPackageBehaviour;

ATTRIBUTES

vlrFunctionId      GET;

REGISTERED AS {ts32-634Package 26};

vlrFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘vlrFunction’ Information Object represents the VLR functionality. For more information

about the VLR, see 3GPP TS 23.002";

### -- 5.2.27	gbLinkBasicPackage

gbLinkBasicPackage PACKAGE

BEHAVIOUR

gbLinkBasicPackageBehaviour;

ATTRIBUTES

gbLinkId      GET;

REGISTERED AS {ts32-634Package 27};

gbLinkBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘gbLink’ Information Object represents the Gb link functionality. For more information

about the Gb link, see 3GPP TS 23.002";

### -- 5.2.28	gbLinkAssociationPackage

gbLinkAssociationPackage PACKAGE

BEHAVIOUR

gbLinkAssociationPackageBehaviour;

ATTRIBUTES

connectedBss      GET;

REGISTERED AS {ts32-634Package 28};

gbLinkAssociationPackageBehaviour BEHAVIOUR

DEFINED AS

"This Package contains the attributes of an ‘gbLink’ information object in relation with

associations to BssFunction or ExternalBssFunction objects";

### -- 5.2.29	aLinkBasicPackage

aLinkBasicPackage PACKAGE

BEHAVIOUR

aLinkBasicPackageBehaviour;

ATTRIBUTES

aLinkId      GET;

REGISTERED AS {ts32-634Package 29};

aLinkBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘aLink’ Information Object represents the A link functionality. For more information

about the A link, see 3GPP TS 23.002";

### -- 5.2.30	aLinkAssociationPackage

aLinkAssociationPackage PACKAGE

BEHAVIOUR

aLinkAssociationPackageBehaviour;

ATTRIBUTES

connectedBss      GET;

REGISTERED AS {ts32-634Package 30};

aLinkAssociationPackageBehaviour BEHAVIOUR

DEFINED AS

"This Package contains the attributes of an ‘aLink’ information object in relation with

associations to BssFunction or ExternalBssFunction objects";

### -- 5.2.31	iucsLinkBasicPackage

iucsLinkBasicPackage PACKAGE

BEHAVIOUR

iucsLinkBasicPackageBehaviour;

ATTRIBUTES

iucsLinkId      GET;

REGISTERED AS {ts32-634Package 31};

iucsLinkBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘iucsLink’ Information Object represents the Iu-cs link functionality. For more

information about the Iu-cs link, see 3GPP TS 23.002";

### -- 5.2.32	iucsLinkAssociationPackage

iucsLinkAssociationPackage PACKAGE

BEHAVIOUR

iucsLinkAssociationPackageBehaviour;

ATTRIBUTES

connectedRnc      GET,

connectedBss      GET;

REGISTERED AS {ts32-634Package 32};

iucsLinkAssociationPackageBehaviour BEHAVIOUR

DEFINED AS

"This Package contains the attributes of an ‘iucsLink’ information object in relation with

associations to Bss/RncFunction or ExternalBss/RncFunction objects";

### -- 5.2.33	iupsLinkBasicPackage

iupsLinkBasicPackage PACKAGE

BEHAVIOUR

iupsLinkBasicPackageBehaviour;

ATTRIBUTES

iupsLinkId      GET;

REGISTERED AS {ts32-634Package 33};

iupsLinkBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘iupsLink’ Information Object represents the Iu-ps link functionality. For more

information about the Iu-ps link, see 3GPP TS 23.002";

### -- 5.2.34	iupsLinkAssociationPackage

iupsLinkAssociationPackage PACKAGE

BEHAVIOUR

iupsLinkAssociationPackageBehaviour;

ATTRIBUTES

connectedRnc      GET,

connectedBss      GET;

REGISTERED AS {ts32-634Package 34};

iupsLinkAssociationPackageBehaviour BEHAVIOUR

DEFINED AS

"This Package contains the attributes of an ‘iupsLink’ information object in relation with

associations to Bss/RncFunction or ExternalBss/RncFunction objects";

### -- 5.2.35	iubcLinkBasicPackage

iubcLinkBasicPackage PACKAGE

BEHAVIOUR

iubcLinkBasicPackageBehaviour;

ATTRIBUTES

iubcLinkId      GET;

REGISTERED AS {ts32-634Package 35};

iubcLinkBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘iubcLink’ Information Object represents the Iu-bc link functionality. For more

information about the Iu-bc link, see 3GPP TS 23.002";

### -- 5.2.36	iubcLinkAssociationPackage

iubcLinkAssociationPackage PACKAGE

BEHAVIOUR

iubcLinkAssociationPackageBehaviour;

ATTRIBUTES

connectedRnc      GET;

REGISTERED AS {ts32-634Package 36};

iubcLinkAssociationPackageBehaviour BEHAVIOUR

DEFINED AS

"This Package contains the attributes of an ‘iubcLink’ information object in relation with

associations to RncFunction or ExternalRncFunction objects";

### -- 5.2.37	csMgwFunctionBasicPackage

csMgwFunctionBasicPackage PACKAGE

BEHAVIOUR

csMgwFunctionBasicPackageBehaviour;

ATTRIBUTES

csMgwFunctionId      GET;

REGISTERED AS {ts32-634Package 37};

csMgwFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘csMgwFunction’ Information Object represents the CS-MGW functionality. For more

information about the CS-MGW, see 3GPP TS 23.002";

### -- 5.2.38	csMgwFunctionAssociationPackage

csMgwFunctionAssociationPackage PACKAGE

BEHAVIOUR

csMgwFunctionAssociationPackageBehaviour;

ATTRIBUTES

csMgwFunction-MscServerFunction      GET,

csMgwFunction-IucsLink               GET,

csMgwFunction-Alink                  GET;

REGISTERED AS {ts32-634Package 38};

csMgwFunctionAssociationPackageBehaviour BEHAVIOUR

DEFINED AS

"This Package contains the attributes of an ‘csMgwFunction’ information object in relation

with associations to mscServerFunction, iucsLink or aLink objects";

### -- 5.2.39	gmscServerFunctionBasicPackage

gmscServerFunctionBasicPackage PACKAGE

BEHAVIOUR

gmscServerFunctionBasicPackageBehaviour;

ATTRIBUTES

gmscServerFunctionId      GET;

REGISTERED AS {ts32-634Package 39};

gmscServerFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘gmscServerFunction’ Information Object represents the GMSCServer functionality. For more

information about the GMSCServer, see 3GPP TS 23.002";

### -- 5.2.40	scscfFunctionBasicPackage

scscfFunctionBasicPackage PACKAGE

BEHAVIOUR

scscfFunctionBasicPackageBehaviour;

ATTRIBUTES

scscfFunctionId      GET;

REGISTERED AS {ts32-634Package 40};

scscfFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘scscfFunction’ Information Object represents the S-CSCF functionality. For more

information about the S-CSCF, see 3GPP TS 23.002";

### -- 5.2.41	pcscfFunctionBasicPackage

pcscfFunctionBasicPackage PACKAGE

BEHAVIOUR

pcscfFunctionBasicPackageBehaviour;

ATTRIBUTES

pcscfFunctionId      GET;

REGISTERED AS {ts32-634Package 41};

pcscfFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘pcscfFunction’ Information Object represents the P-CSCF functionality. For more

information about the P-CSCF, see 3GPP TS 23.002";

### -- 5.2.42	icscfFunctionBasicPackage

icscfFunctionBasicPackage PACKAGE

BEHAVIOUR

icscfFunctionBasicPackageBehaviour;

ATTRIBUTES

icscfFunctionId      GET;

REGISTERED AS {ts32-634Package 42};

icscfFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘icscfFunction’ Information Object represents the I-CSCF functionality. For more

information about the I-CSCF, see 3GPP TS 23.002";

### -- 5.2.43	slfFunctionBasicPackage

slfFunctionBasicPackage PACKAGE

BEHAVIOUR

slfFunctionBasicPackageBehaviour;

ATTRIBUTES

slfFunctionId      GET;

REGISTERED AS {ts32-634Package 43};

slfFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘slfFunction’ Information Object represents the SLF functionality. For more

information about the SLF, see 3GPP TS 23.002";

### -- 5.2.44	bgcfFunctionBasicPackage

bgcfFunctionBasicPackage PACKAGE

BEHAVIOUR

bgcfFunctionBasicPackageBehaviour;

ATTRIBUTES

bgcfFunctionId      GET;

REGISTERED AS {ts32-634Package 44};

bgcfFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘bgcfFunction’ Information Object represents the BGCF functionality. For more

information about the BGCF, see 3GPP TS 23.002";

### -- 5.2.45	mrfcFunctionBasicPackage

mrfcFunctionBasicPackage PACKAGE

BEHAVIOUR

mrfcFunctionBasicPackageBehaviour;

ATTRIBUTES

mrfcFunctionId      GET;

REGISTERED AS {ts32-634Package 45};

mrfcFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘mrfcFunction’ Information Object represents the MRFC functionality. For more

information about the MRFC, see 3GPP TS 23.002";

### -- 5.2.46	asFunctionBasicPackage

asFunctionBasicPackage PACKAGE

BEHAVIOUR

asFunctionBasicPackageBehaviour;

ATTRIBUTES

asFunctionId      GET;

REGISTERED AS {ts32-634Package 46};

asFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘asFunction’ Information Object represents the AS functionality. For more

information about the AS, see 3GPP TS 23.002";

### -- 5.2.47	mgcfFunctionBasicPackage

mgcfFunctionBasicPackage PACKAGE

BEHAVIOUR

mgcfFunctionBasicPackageBehaviour;

ATTRIBUTES

mgcfFunctionId      GET;

REGISTERED AS {ts32-634Package 47};

mgcfFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘mgcfFunction’ Information Object represents the MGCF functionality. For more

information about the MGCF, see 3GPP TS 23.002";

### -- 5.2.48	mrfpFunctionBasicPackage

mrfpFunctionBasicPackage PACKAGE

BEHAVIOUR

mrfpFunctionBasicPackageBehaviour;

ATTRIBUTES

mrfpFunctionId      GET;

REGISTERED AS {ts32-634Package 48};

mrfpFunctionBasicPackageBehaviour BEHAVIOUR

DEFINED AS

"The ‘mrfpFunction’ Information Object represents the MRFP functionality. For more

information about the MRFP, see 3GPP TS 23.002";

## -- 5.3	Attributes

### -- 5.3.1	smlcFunctionId

smlcFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

smlcFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 1};

smlcFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a smlcFunction instance.";

### -- 5.3.2	gmlcFunctionId

gmlcFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

gmlcFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 2};

gmlcFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a gmlcFunction instance.";

### -- 5.3.3	scfFunctionId

scfFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

scfFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 3};

scfFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a scfFunction instance.";

### -- 5.3.4	srfFunctionId

srfFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

srfFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 4};

srfFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a srfFunction instance.";

### -- 5.3.5	cbcFunctionId

cbcFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

cbcFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 5};

cbcFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a cbcFunction instance.";

### -- 5.3.6	cgfFunctionId

cgfFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

cgfFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 6};

cgfFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a cgfFunction instance.";

### -- 5.3.7	imsMgwFunctionId

imsMgwFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

imsMgwFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 70503};

imsMgwFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies an imsMgwFunction instance.";

### -- 5.3.8	gmscFunctionId

gmscFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

gmscFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 8};

gmscFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a gmscFunction instance.";

### -- 5.3.9	iwfFunctionId

iwfFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

iwfFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 9};

iwfFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a iwfFunction instance.";

### -- 5.3.10	mnpSrfFunctionId

mnpSrfFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

mnpSrfFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 10};

mnpSrfFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a mnpSrfFunction instance.";

### -- 5.3.11	npdbFunctionId

npdbFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

npdbFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 11};

npdbFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a npdbFunction instance.";

### -- 5.3.12	rSgwFunctionId

rSgwFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

rSgwFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 12};

rSgwFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a rSgwFunction instance.";

### -- 5.3.13	ssfFunctionId

ssfFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

ssfFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 13};

ssfFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a ssfFunction instance.";

### -- 5.3.14	bsFunctionId

bsFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

bsFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 14};

bsFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a bsFunction instance.";

### -- 5.3.15	aucFunctionId

aucFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

aucFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 15};

aucFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a aucFunction instance.";

### -- 5.3.16	bgFunctionId

bgFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

bgFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 16};

bgFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a bgFunction instance.";

### -- 5.3.17	eirFunctionId

eirFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

eirFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 17};

eirFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a eirFunction instance.";

### -- 5.3.18	ggsnFunctionId

ggsnFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

ggsnFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 18};

ggsnFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a ggsnFunction instance.";

### -- 5.3.19	hlrFunctionId

hlrFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

hlrFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 19};

hlrFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a hlrFunction instance.";

### -- 5.3.20	mscServerFunctionId

mscServerFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

mscServerFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 20};

mscServerFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a mscServerFunction instance.";

### -- 5.3.21	vlrFunctionId

vlrFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

vlrFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 21};

vlrFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a vlrFunction instance.";

### -- 5.3.22	sgsnFunctionId

sgsnFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

sgsnFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 22};

sgsnFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a sgsnFunction instance.";

### -- 5.3.23	smsGmscFunctionId

smsGmscFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

smsGmscFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 23};

smsGmscFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a smsGmscFunction instance.";

### -- 5.3.24	smsIwmscFunctionId

smsIwmscFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

smsIwmscFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 24};

smsIwmscFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a smsIwmscFunction instance.";

### -- 5.3.25	gbLinkId

gbLinkId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

gbLinkIdBehaviour;

REGISTERED AS {ts32-634Attribute 25};

gbLinkIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a gbLink instance.";

### -- 5.3.26	aLinkId

aLinkId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

aLinkIdBehaviour;

REGISTERED AS {ts32-634Attribute 26};

aLinkIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a aLink instance.";

### -- 5.3.27	iucsLinkId

iucsLinkId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

iucsLinkIdBehaviour;

REGISTERED AS {ts32-634Attribute 27};

iucsLinkIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a iucsLink instance.";

### -- 5.3.28	iupsLinkId

iupsLinkId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

iupsLinkIdBehaviour;

REGISTERED AS {ts32-634Attribute 28};

iupsLinkIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a iupsLink instance.";

### -- 5.3.29	iubcLinkId

iubcLinkId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

iubcLinkIdBehaviour;

REGISTERED AS {ts32-634Attribute 29};

iubcLinkIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a iubcLink instance.";

### -- 5.3.30	csMgwFunctionId

csMgwFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

csMgwFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 30};

csMgwFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a csMgwFunction instance.";

### -- 5.3.31	gmscServerFunctionId

gmscServerFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

gmscServerFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 31};

gmscServerFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a gmscServerFunction instance.";

### -- 5.3.32	mccList

mccList ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.MccList;

MATCHES FOR

EQUALITY;

BEHAVIOUR

mccListBehaviour;

REGISTERED AS {ts32-634Attribute 32};

mccListBehaviour BEHAVIOUR

DEFINED AS

"List of Mobile Country Codes, MCC. The MCC is part of the PLMN Id (Ref. 3 GPP TS 23.003).";

### -- 5.3.33	mncList

mncList ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.MncList;

MATCHES FOR

EQUALITY;

BEHAVIOUR

mncListBehaviour;

REGISTERED AS {ts32-634Attribute 33};

mncListBehaviour BEHAVIOUR

DEFINED AS

"List of Mobile Network Code, MNC. The MNC is part of the PLMN Id (Ref. 3 GPP TS 23.003).";

### -- 5.3.34	lacList

lacList ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.LacList;

MATCHES FOR

EQUALITY;

BEHAVIOUR

lacListBehaviour;

REGISTERED AS {ts32-634Attribute 34};

lacListBehaviour BEHAVIOUR

DEFINED AS

"List of Location Area Codes covered by SGSN (Ref. 3 GPP TS 23.003).";

### -- 5.3.35	sacList

sacList ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.SacList;

MATCHES FOR

EQUALITY;

BEHAVIOUR

sacListBehaviour;

REGISTERED AS {ts32-634Attribute 35};

sacListBehaviour BEHAVIOUR

DEFINED AS

"List of Service Area Codes covered by SGSN (Ref. 3 GPP TS 23.003).";

### -- 5.3.36	uraList

-- Void.

### -- 5.3.37	gcaList

gcaList ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.CgaList;

MATCHES FOR

EQUALITY;

BEHAVIOUR

gcaListBehaviour;

REGISTERED AS {ts32-634Attribute 37};

gcaListBehaviour BEHAVIOUR

DEFINED AS

"List of Group Call Area (Ref. 3 GPP TS 23.003).";

### -- 5.3.38	mscId

mscId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

mscIdBehaviour;

REGISTERED AS {ts32-634Attribute 38};

mscIdBehaviour BEHAVIOUR

DEFINED AS

"Unique MSC ID (Ref. 3 GPP TS 23.002).";

### -- 5.3.39	mscServerFunction-GSMcell

mscServerFunction-GSMcell ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectPointer;

MATCHES FOR

EQUALITY;

BEHAVIOUR

mscServerFunction-GSMcellBehaviour;

REGISTERED AS {ts32-634Attribute 39};

mscServerFunction-GSMcellBehaviour BEHAVIOUR

DEFINED AS

"This value contains the DN of the related GSMcell instance. This is a reference attribute

modelling the role (of the association AssociatedWith) that this MscServerFunction

is associated with to 0-* GSMcell.";

### -- 5.3.40	mscServerFunction-ExternalGSMcell

mscServerFunction-ExternalGSMcell ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectPointer;

MATCHES FOR

EQUALITY;

BEHAVIOUR

mscServerFunction-ExternalGSMcellBehaviour;

REGISTERED AS {ts32-634Attribute 40};

mscServerFunction-ExternalGSMcellBehaviour BEHAVIOUR

DEFINED AS

"This value contains the DN of the related ExternalGSMcell instance. This is a reference

attribute modelling the role (of the association AssociatedWith) that this MscServerFunction

is associated with to 0-* ExternalGSMcell.";

### -- 5.3.41	mscServerFunction-CsMgwFunction

mscServerFunction-CsMgwFunction ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectPointer;

MATCHES FOR

EQUALITY;

BEHAVIOUR

mscServerFunction-CsMgwFunctionBehaviour;

REGISTERED AS {ts32-634Attribute 41};

mscServerFunction-CsMgwFunctionBehaviour BEHAVIOUR

DEFINED AS

"This value contains the DN of the related CsMgwFunction instance. This is a reference

attribute modelling the role (of the association AssociatedWith) that this MscServerFunction

is associated with to 0-* CsMgwFunction.";

### -- 5.3.42	racList

racList ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.RacList;

MATCHES FOR

EQUALITY;

BEHAVIOUR

racListBehaviour;

REGISTERED AS {ts32-634Attribute 42};

racListBehaviour BEHAVIOUR

DEFINED AS

"List of Routing Area Codes covered by SGSN (Ref. 3 GPP TS 23.003).";

### -- 5.3.43	sgsnId

sgsnId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

sgsnIdBehaviour;

REGISTERED AS {ts32-634Attribute 43};

sgsnIdBehaviour BEHAVIOUR

DEFINED AS

"Unique SGSN ID (Ref. 3 GPP TS 23.002).";

### -- 5.3.44	sgsnFunction-GSMcell

sgsnFunction-GSMcell ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectPointer;

MATCHES FOR

EQUALITY;

BEHAVIOUR

sgsnFunction-GSMcellBehaviour;

REGISTERED AS {ts32-634Attribute 44};

sgsnFunction-GSMcellBehaviour BEHAVIOUR

DEFINED AS

"This value contains the DN of the related GSMcell instance. This is a reference attribute

modelling the role (of the association AssociatedWith) that this SgsnFunction is associated

with to 0-* GSMcell.";

### -- 5.3.45	sgsnFunction-ExternalGSMcell

sgsnFunction-ExternalGSMcell ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectPointer;

MATCHES FOR

EQUALITY;

BEHAVIOUR

sgsnFunction-ExternalGSMcellBehaviour;

REGISTERED AS {ts32-634Attribute 45};

sgsnFunction-ExternalGSMcellBehaviour BEHAVIOUR

DEFINED AS

"This value contains the DN of the related ExternalGSMcell instance. This is a reference

attribute modelling the role (of the association AssociatedWith) that this SgsnFunction

is associated with to 0-* ExternalGSMcell.";

### -- 5.3.46	connectedBss

connectedBss ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectPointer;

MATCHES FOR

EQUALITY;

BEHAVIOUR

connectedBssBehaviour;

REGISTERED AS {ts32-634Attribute 46};

connectedBssBehaviour BEHAVIOUR

DEFINED AS

"This value contains the DN of the related BssFunction or ExternalBssFunction instance. This is

a reference attribute modelling the role (of the association AssociatedWith) that link is

connected to 0-1 BssFunction or 0-1 ExternalBssFunction.";

### -- 5.3.47	connectedRnc

connectedRnc ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectPointer;

MATCHES FOR

EQUALITY;

BEHAVIOUR

connectedRncBehaviour;

REGISTERED AS {ts32-634Attribute 47};

connectedRncBehaviour BEHAVIOUR

DEFINED AS

"This value contains the DN of the related RncFunction or ExternalRncFunction instance. This is

a reference attribute modelling the role (of the association AssociatedWith) that link is

connected to 0-1 RncFunction or 0-1 ExternalRncFunction.";

### -- 5.3.48	csMgwFunction-MscServerFunction

csMgwFunction-MscServerFunction ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectPointer;

MATCHES FOR

EQUALITY;

BEHAVIOUR

csMgwFunction-MscServerFunctionBehaviour;

REGISTERED AS {ts32-634Attribute 48};

csMgwFunction-MscServerFunctionBehaviour BEHAVIOUR

DEFINED AS

"This value contains the DN of the related mscServerFunction instance. This is a reference

attribute modelling the role (of the association AssociatedWith) that this csMgwFunction is

associated with to 0-* mscServerFunction.";

### -- 5.3.49	csMgwFunction-IucsLink

csMgwFunction-IucsLink ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectPointer;

MATCHES FOR

EQUALITY;

BEHAVIOUR

csMgwFunction-IucsLinkBehaviour;

REGISTERED AS {ts32-634Attribute 49};

csMgwFunction-IucsLinkBehaviour BEHAVIOUR

DEFINED AS

"This value contains the DN of the related IucsLink instance. This is a reference attribute

modelling the role (of the association AssociatedWith) that this csMgwFunction is

connected to 0-* IucsLink.";

### -- 5.3.50	csMgwFunction-Alink

csMgwFunction-Alink ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectPointer;

MATCHES FOR

EQUALITY;

BEHAVIOUR

csMgwFunction-AlinkBehaviour;

REGISTERED AS {ts32-634Attribute 50};

csMgwFunction-AlinkBehaviour BEHAVIOUR

DEFINED AS

"This value contains the DN of the related ALink instance. This is a reference attribute

modelling the role (of the association AssociatedWith) that this csMgwFunction is

connected to 0-* ALink.";

### -- 5.3.51	scscfFunctionId

scscfFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

scscfFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 51};

scscfFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a scscfFunction instance.";

### -- 5.3.52	pcscfFunctionId

pcscfFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

pcscfFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 52};

pcscfFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a pcscfFunction instance.";

### -- 5.3.53	icscfFunctionId

icscfFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

icscfFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 53};

icscfFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a icscfFunction instance.";

### -- 5.3.54	slfFunctionId

slfFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

slfFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 54};

slfFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a slfFunction instance.";

### -- 5.3.55	bgcfFunctionId

bgcfFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

bgcfFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 55};

bgcfFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a bgcfFunction instance.";

### -- 5.3.56	mrfcFunctionId

mrfcFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

mrfcFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 56};

mrfcFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a mrfcFunction instance.";

### -- 5.3.57	asFunctionId

asFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

asFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 57};

asFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a asFunction instance.";

### -- 5.3.58	mgcfFunctionId

mgcfFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

mgcfFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 58};

mgcfFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a mgcfFunction instance.";

### -- 5.3.59	mrfpFunctionId

mrfpFunctionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-634TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

mrfpFunctionIdBehaviour;

REGISTERED AS {ts32-634Attribute 59};

mrfpFunctionIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute identifies a mrfpFunction instance.";

## -- 5.4	Name Binding

### -- 5.4.1	smlcFunction - managedElement

smlcFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

smlcFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

smlcFunctionId;

BEHAVIOUR

smlcFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 1};

smlcFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

smlcFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.2	gmlcFunction - managedElement

gmlcFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

gmlcFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

gmlcFunctionId;

BEHAVIOUR

gmlcFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 2};

gmlcFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

gmlcFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.3	scfFunction - managedElement

scfFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

scfFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

scfFunctionId;

BEHAVIOUR

scfFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 3};

scfFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

scfFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.4	srfFunction - managedElement

srfFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

srfFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

srfFunctionId;

BEHAVIOUR

srfFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 4};

srfFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

srfFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.5	cbcFunction - managedElement

cbcFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

cbcFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

cbcFunctionId;

BEHAVIOUR

cbcFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 5};

cbcFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

cbcFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.6	cgfFunction - managedElement

cgfFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

cgfFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

cgfFunctionId;

BEHAVIOUR

cgfFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 6};

cgfFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

cgfFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.7	imsMgwFunction - managedElement

imsMgwFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

imsMgwFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

imsMgwFunctionId;

BEHAVIOUR

imsMgwFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 70503};

imsMgwFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls an

mimsMgwFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.8	gmscFunction - managedElement

gmscFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

gmscFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

gmscFunctionId;

BEHAVIOUR

gmscFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 8};

gmscFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

gmscFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.9	iwfFunction - managedElement

iwfFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

iwfFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

iwfFunctionId;

BEHAVIOUR

iwfFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 9};

iwfFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

iwfFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.10	mnpSrfFunction - managedElement

mnpSrfFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

mnpSrfFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

mnpSrfFunctionId;

BEHAVIOUR

mnpSrfFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 10};

mnpSrfFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

mnpSrfFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.11	npdbFunction - managedElement

npdbFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

npdbFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

npdbFunctionId;

BEHAVIOUR

npdbFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 11};

npdbFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

npdbFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.12	rSgwFunction - managedElement

rSgwFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

rSgwFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

rSgwFunctionId;

BEHAVIOUR

rSgwFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 12};

rSgwFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

rSgwFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.13	ssfFunction - managedElement

ssfFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

ssfFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

ssfFunctionId;

BEHAVIOUR

ssfFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 13};

ssfFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

ssfFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.14	bsFunction - managedElement

bsFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

bsFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

bsFunctionId;

BEHAVIOUR

bsFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 14};

bsFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

bsFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.15	aucFunction - managedElement

aucFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

aucFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

aucFunctionId;

BEHAVIOUR

aucFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 15};

aucFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

aucFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.16	bgFunction - managedElement

bgFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

bgFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

bgFunctionId;

BEHAVIOUR

bgFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 16};

bgFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

bgFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.17	eirFunction - managedElement

eirFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

eirFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

eirFunctionId;

BEHAVIOUR

eirFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 17};

eirFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

eirFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.18	ggsnFunction - managedElement

ggsnFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

ggsnFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

ggsnFunctionId;

BEHAVIOUR

ggsnFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 18};

ggsnFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

ggsnFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.19	hlrFunction - managedElement

hlrFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

hlrFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

hlrFunctionId;

BEHAVIOUR

hlrFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 19};

hlrFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

hlrFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.20	mscServerFunction - managedElement

mscServerFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

mscFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

mscServerFunctionId;

BEHAVIOUR

mscServerFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 20};

mscServerFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

mscServerFunction. When automatic instance naming is used, the choice of name bindings is left

as a local matter.";

### -- 5.4.21	vlrFunction - managedElement

vlrFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

vlrFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

vlrFunctionId;

BEHAVIOUR

vlrFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 21};

vlrFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

vlrFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.22	sgsnFunction - managedElement

sgsnFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

sgsnFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

sgsnFunctionId;

BEHAVIOUR

sgsnFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 22};

sgsnFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

sgsnFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.23	smsGmscFunction - managedElement

smsGmscFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

smsGmscFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

smsGmscFunctionId;

BEHAVIOUR

smsGmscFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 23};

smsGmscFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

smsGmscFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.24	smsIwmscFunction - managedElement

smsIwmscFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

smsIwmscFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

smsIwmscFunctionId;

BEHAVIOUR

smsIwmscFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 24};

smsIwmscFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

smsIwmscFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.25	gbLink - managedElement

gbLink-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

gbLink;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

gbLinkId;

BEHAVIOUR

gbLink-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 25};

gbLink-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

gbLink. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.26	aLink - managedElement

aLink-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

aLink;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

aLinkId;

BEHAVIOUR

aLink-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 26};

aLink-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

aLink. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.27	iucsLink - managedElement

iucsLink-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

iucsLink;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

iucsLinkId;

BEHAVIOUR

iucsLink-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 27};

iucsLink-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

iucsLink. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.28	iupsLink - managedElement

iupsLink-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

iupsLink;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

iupsLinkId;

BEHAVIOUR

iupsLink-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 28};

iupsLink-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

iupsLink. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.29	iubcLink - managedElement

iubcLink-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

iubcLink;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

iubcLinkId;

BEHAVIOUR

iubcLink-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 29};

iubcLink-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

iubcLink. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.30	gmscServerFunction - managedElement

gmscServerFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

gmscServerFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

gmscServerFunctionId;

BEHAVIOUR

gmscServerFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 30};

gmscServerFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

gmscServerFunction. When automatic instance naming is used, the choice of name bindings is left

as a local matter.";

### -- 5.4.31	csMgwFunction - managedElement

csMgwFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

csMgwFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

csMgwFunctionId;

BEHAVIOUR

csMgwFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 31};

csMgwFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

csMgwFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.32	scscfFunction - managedElement

scscfFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

scscfFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

scscfFunctionId;

BEHAVIOUR

scscfFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 32};

scscfFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

scscfFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.33	pcscfFunction - managedElement

pcscfFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

pcscfFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

pcscfFunctionId;

BEHAVIOUR

pcscfFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 33};

pcscfFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

pcscfFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.34	icscfFunction - managedElement

icscfFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

icscfFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

icscfFunctionId;

BEHAVIOUR

icscfFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 34};

icscfFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

icscfFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.35	slfFunction - managedElement

slfFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

slfFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

slfFunctionId;

BEHAVIOUR

slfFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 35};

slfFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

slfFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.36	bgcfFunction - managedElement

bgcfFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

bgcfFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

bgcfFunctionId;

BEHAVIOUR

bgcfFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 36};

bgcfFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

bgcfFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.37	mrfcFunction - managedElement

mrfcFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

mrfcFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

mrfcFunctionId;

BEHAVIOUR

mrfcFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 37};

mrfcFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

mrfcFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.38	asFunction - managedElement

asFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

asFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

asFunctionId;

BEHAVIOUR

asFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 38};

asFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

asFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.39	mgcfFunction - managedElement

mgcfFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

mgcfFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

mgcfFunctionId;

BEHAVIOUR

mgcfFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 39};

mgcfFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

mgcfFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.40	mrfpFunction - managedElement

mrfpFunction-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

mrfpFunction;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": managedElement;

WITH ATTRIBUTE

mrfpFunctionId;

BEHAVIOUR

mrfpFunction-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 40};

mrfpFunction-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement contains and controls a

mrfpFunction. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.41	linkAsAuc - subNetworkR60

linkAsAuc–subNetworkR60 NAME BINDING

SUBORDINATE OBJECT CLASS

linkAsAuc;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": subNetworkR60;

WITH ATTRIBUTE

linkId;

BEHAVIOUR

linkAsAuc-subNetworkR60Behaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 410620};

linkAsAuc-subNetworkR60Behaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a subNetworkR60 contains and controls a

linkAsAuc. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.42	linkAsHlr - subNetworkR60

linkAsHlr–subNetworkR60 NAME BINDING

SUBORDINATE OBJECT CLASS

linkAsHlr;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": subNetworkR60;

WITH ATTRIBUTE

linkId;

BEHAVIOUR

linkAsHlr-subNetworkR60Behaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 420620};

linkAsHlr-subNetworkR60Behaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a subNetworkR60 contains and controls a

linkAsHlr. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.43	linkAsScscf - subNetworkR60

linkAsScscf–subNetworkR60 NAME BINDING

SUBORDINATE OBJECT CLASS

linkAsScscf;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": subNetworkR60;

WITH ATTRIBUTE

linkId;

BEHAVIOUR

linkAsScscf-subNetworkR60Behaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 430620};

linkAsScscf-subNetworkR60Behaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a subNetworkR60 contains and controls a

linkAsScscf. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.44	linkAsSlf - subNetworkR60

linkAsSlf–subNetworkR60 NAME BINDING

SUBORDINATE OBJECT CLASS

linkAsSlf;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": subNetworkR60;

WITH ATTRIBUTE

linkId;

BEHAVIOUR

linkAsSlf-subNetworkR60Behaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 440620};

linkAsSlf-subNetworkR60Behaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a subNetworkR60 contains and controls a

linkAsSlf. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.45	linkBgcfBgcf - subNetworkR60

linkBgcfBgcf–subNetwork NAME BINDING

SUBORDINATE OBJECT CLASS

linkBgcfBgcf;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": subNetworkR60;

WITH ATTRIBUTE

linkId;

BEHAVIOUR

linkBgcfBgcf–subNetworkR60Behaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 450620};

linkBgcfBgcf–subNetworkR60Behaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a subNetworkR60 contains and controls a

linkBgcfBgcf. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.46	linkBgcfImsMgw - subNetworkR60

linkBgcfImsMgw-subNetworkR60 NAME BINDING

SUBORDINATE OBJECT CLASS

linkBgcfImsMgw;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": subNetworkR60;

WITH ATTRIBUTE

linkId;

BEHAVIOUR

linkBgcfImsMgw-subNetworkR60Behaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 460620};

linkBgcfImsMgw-subNetworkR60Behaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a subNetworkR60 contains and controls a

linkBgcfIMsMgw. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.47	linkBgcfScscf-subNetworkR60

linkBgcfScscf-subNetworkR60 NAME BINDING

SUBORDINATE OBJECT CLASS

linkBgcfScscf;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": subNetworkR60;

WITH ATTRIBUTE

linkId;

BEHAVIOUR

linkBgcfScscf-subNetworkR60Behaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 470620};

linkBgcfScscf-subNetworkR60Behaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a subNetworkR60 contains and controls a

linkBgcfScscf. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.48	linkHlrScscf-subNetworkR60

linkHlrScscf-subNetworkR60 NAME BINDING

SUBORDINATE OBJECT CLASS

linkHlrScscf;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": subNetworkR60;

WITH ATTRIBUTE

linkId;

BEHAVIOUR

linkHlrScscf-subNetworkR60Behaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 480620};

linkHlrScscf-subNetworkR60Behaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a subNetworkR60 contains and controls a

linkHlrScscf. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.49	linkIcscfSlf-subNetworkR60

linkIcscfSlf-subNetworkR60 NAME BINDING

SUBORDINATE OBJECT CLASS

linkIcscfSlf;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": subNetworkR60;

WITH ATTRIBUTE

linkId;

BEHAVIOUR

linkIcscfSlf-subNetworkR60Behaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 490620};

linkIcscfSlf-subNetworkR60Behaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a subNetworkR60 contains and controls a

linkIcscfSlf. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.50	linkImsMgwMgcf-subNetworkR60

linkImsMgwMgcf-subNetworkR60 NAME BINDING

SUBORDINATE OBJECT CLASS

linkImsMgwMgcf;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": subNetworkR60;

WITH ATTRIBUTE

linkId;

BEHAVIOUR

linkImsMgwMgcf-subNetworkR60Behaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 500620};

linkImsMgwMgcf-subNetworkR60Behaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a subNetworkR60 contains and controls a

linkImsMgwMgcf. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.51	linkImsMgwScscf-subNetworkR60

linkImsMgwScscf-subNetworkR60 NAME BINDING

SUBORDINATE OBJECT CLASS

linkImsMgwScscf;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": subNetworkR60;

WITH ATTRIBUTE

linkId;

BEHAVIOUR

linkImsMgwScscf-subNetworkR60Behaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 510620};

linkImsMgwScscf-subNetworkR60Behaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a subNetworkR60 contains and controls a

linkImsMgwScscf. When automatic instance naming is used, the choice of name bindings is left

as a local matter.";

### -- 5.4.52	linkMrfcMrfp-subNetworkR60

linkMrfcMrfp-subNetworkR60 NAME BINDING

SUBORDINATE OBJECT CLASS

linkMrfcMrfp;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": subNetworkR60;

WITH ATTRIBUTE

linkId;

BEHAVIOUR

linkMrfcMrfp-subNetworkR60Behaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 520620};

linkMrfcMrfp-subNetworkR60Behaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a subNetworkR60 contains and controls a

linkMrfcMrfp. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.53	linkMrfcScscf-subNetworkR60

linkMrfcScscf-subNetworkR60 NAME BINDING

SUBORDINATE OBJECT CLASS

linkMrfcScscf;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": subNetworkR60;

WITH ATTRIBUTE

linkId;

BEHAVIOUR

linkMrfcScscf-subNetworkR60Behaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 530620};

linkMrfcScscf-subNetworkR60Behaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a subNetworkR60 contains and controls a

linkMrfcMrfp. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.54	linkScscfScscf-subNetworkR60

linkScscfScscf-subNetworkR60 NAME BINDING

SUBORDINATE OBJECT CLASS

linkScscfScscf;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": subNetworkR60;

WITH ATTRIBUTE

linkId;

BEHAVIOUR

linkScscfScscf-subNetworkR60Behaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 540620};

linkScscfScscf-subNetworkR60Behaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a subNetworkR60 contains and controls a

linkScscfScscf. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

### -- 5.4.55	linkScscfSlf-subNetworkR60

linkScscfSlf-subNetworkR60   NAME BINDING

SUBORDINATE OBJECT CLASS

linkScscfSlf;

NAMED BY SUPERIOR OBJECT CLASS

"3GPP TS 32.624": subNetworkR60;

WITH ATTRIBUTE

linkId;

BEHAVIOUR

linkScscfSlf-subNetworkR60Behaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-634NameBinding 550620};

linkScscfSlf-subNetworkR60Behaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a subNetworkR60 contains and controls a

linkScscfSlf. When automatic instance naming is used, the choice of name bindings is left as

a local matter.";

# -- 6	ASN.1 Definitions

TS32-634TypeModule {ccitt (0) identified-organization (4) etsi (0) mobileDomain (0) umts-Operation-Maintenance (3) ts32-634 (634) informationModel (0) asn1Module (2) version1 (1)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

--EXPORTS everything

IMPORTS

GeneralObjectId, GeneralObjectPointer

FROM TS32-624TypeModule {ccitt(0) identified-organization(4) etsi(0) mobileDomain(0)

umts-Operation-Maintenance(3) ts32-624(624) informationModel(0) asn1Module(2) version1(1)}

MobileCountryCode, MobileNetworkCode, LocationAreaCode

FROM GSM1220TypeModule {ccitt(0) identified-organization(4) etsi(0) mobileDomain(0)

gsm-Operation-Maintenance(3) gsm-12-20(20) informationModel(0) asn1Module(2)

asn1TypeModule(0)};

-- 3GPP TS 32.634 related Object Identifiers

baseNodeUMTS           OBJECT IDENTIFIER ::= {itu-t(0) identified-organization(4) etsi(0)

mobileDomain(0) umts-Operation-Maintenance(3)}

ts32-634               OBJECT IDENTIFIER ::= {baseNodeUMTS ts32-634(634)}

ts32-634InfoModel      OBJECT IDENTIFIER ::= {ts32-634 informationModel(0)}

ts32-634ObjectClass    OBJECT IDENTIFIER ::= {ts32-634InfoModel managedObjectClass(3)}

ts32-634Package        OBJECT IDENTIFIER ::= {ts32-634InfoModel package(4)}

ts32-634Parameter      OBJECT IDENTIFIER ::= {ts32-634InfoModel parameter(5)}

ts32-634NameBinding    OBJECT IDENTIFIER ::= {ts32-634InfoModel nameBinding(6)}

ts32-634Attribute      OBJECT IDENTIFIER ::= {ts32-634InfoModel attribute(7)}

ts32-634Action         OBJECT IDENTIFIER ::= {ts32-634InfoModel action(9)}

ts32-634Notification   OBJECT IDENTIFIER ::= {ts32-634InfoModel notification(10)}

-- Start of 3GPP SA5 own definitions

MccList ::= SET OF MobileCountryCode

MncList ::= SET OF MobileNetworkCode

LacList ::= SET OF LocationAreaCode

Rac ::= INTEGER

RacList ::= SET OF Rac

Sac ::= INTEGER

SacList ::= SET OF Sac

Cga ::= INTEGER

CgaList ::= SET OF Cga

END -- of TS32-634TypeModule

Annex A (informative):
List of assigned Object Identifiers

This annex provides a list with all Object Identifiers (OIDs) that have been assigned in TS 32.634. 
These OIDs shall not be assigned to new objects.

| Basic Object Name | Name and OID of the current TS Version | Name and OIDs of previous TS Versions |
| --- | --- | --- |
| Managed Object Classes |  |  |
| smlcFunction | Name: smlcFunction OID : ts32-634ObjectClass 1 | -- |
| gmlcFunction | Name: gmlcFunction OID : ts32-634ObjectClass 2 | -- |
| scfFunction | Name: scfFunction OID : ts32-634ObjectClass 3 | -- |
| srfFunction | Name: srfFunction OID : ts32-634ObjectClass 4 | -- |
| cbcFunction | Name: cbcFunction OID : ts32-634ObjectClass 5 | -- |
| cgfFunction | Name: cgfFunction OID : ts32-634ObjectClass 6 | -- |
| mgwFunction | -- | Name: mgwFunction OID : ts32-634ObjectClass 7 |
| imsMgwFunction | Name: imsMgwFunction OID : ts32-634ObjectClass 70503 | -- |
| gmscFunction | Name: gmscFunction OID : ts32-634ObjectClass 8 | -- |
| iwfFunction | Name: iwfFunction OID : ts32-634ObjectClass 9 | -- |
| mnpSrfFunction | Name: mnpSrfFunction OID : ts32-634ObjectClass 10 | -- |
| npdbFunction | Name: npdbFunction OID : ts32-634ObjectClass 11 | -- |
| rSgwFunction | Name: rSgwFunction OID : ts32-634ObjectClass 12 | -- |
| ssfFunction | Name: ssfFunction OID : ts32-634ObjectClass 13 | -- |
| bsFunction | Name: bsFunction OID : ts32-634ObjectClass 14 | -- |
| aucFunction | Name: aucFunction OID : ts32-634ObjectClass 15 | -- |
| bgFunction | Name: bgFunction OID : ts32-634ObjectClass 16 | -- |
| eirFunction | Name: eirFunction OID : ts32-634ObjectClass 17 | -- |
| ggsnFunction | Name: ggsnFunction OID : ts32-634ObjectClass 18 | -- |
| hlrFunction | Name: hlrFunction OID : ts32-634ObjectClass 19 | -- |
| mscFunction | Name: mscFunction OID : ts32-634ObjectClass 20 | -- |
| sgsnFunction | Name: sgsnFunction OID : ts32-634ObjectClass 21 | -- |
| smsGmscFunction | Name: smsGmscFunction OID : ts32-634ObjectClass 22 | -- |
| smsIwmscFunction | Name: smsIwmscFunction OID : ts32-634ObjectClass 23 | -- |
| vlrFunction | Name: vlrFunction OID : ts32-634ObjectClass 24 | -- |
| gbLink | Name: gbLink OID : ts32-634ObjectClass 25 | -- |
| aLink | Name: aLink OID : ts32-634ObjectClass 26 | -- |
| iucsLink | Name: iucsLink OID : ts32-634ObjectClass 27 | -- |
| iupsLink | Name: iupsLink OID : ts32-634ObjectClass 28 | -- |
| iubcLink | Name: iubcLink OID : ts32-634ObjectClass 29 | -- |
| csMgwFunction | Name: csMgwFunction OID : ts32-634ObjectClass 30 | -- |
| gmscServerFunction | Name: gmscServerFunction OID : ts32-634ObjectClass 31 | -- |
| scscfFunction | Name: ScscfFunction OID : ts32-634ObjectClass 32 | -- |
| pcscfFunction | Name: PcscfFunction OID : ts32-634ObjectClass 33 | -- |
| icscfFunction | Name: IcscfFunction OID : ts32-634ObjectClass 34 | -- |
| slfFunction | Name: SlfFunction OID : ts32-634ObjectClass 35 | -- |
| bgcfFunction | Name: BgcfFunction OID : ts32-634ObjectClass 36 | -- |
| mrfcFunction | Name: MrfcFunction OID : ts32-634ObjectClass 37 | -- |
| asFunction | Name: AsFunction OID : ts32-634ObjectClass 38 | -- |
| mgcfFunction | Name: MgcfFunction OID : ts32-634ObjectClass 39 | -- |
| mrfpFunction | Name: MrfpFunction OID : ts32-634ObjectClass 40 | -- |
| linkAsAuc | Name: linkAsAuc OID: ts32634ObjectClass 410620 | -- |
| linkAsHlr | Name: linkAsHlr OID: ts32-634ObjectClass 420620 | -- |
| linkAsScscf | Name: linkAsScscf OID: ts32-634ObjectClass 430620 | -- |
| linkAsSlf | Name: linkAsSlf OID: ts32-634ObjectClass 440620 | -- |
| linkBgcfBgcf | Name: linkBgcfBgcf OID: ts32-634ObjectClass 450620 | -- |
| linkBgcfImsMgw | Name: linkBgcfImsMgw OID: ts32-634ObjectClass 460620 | -- |
| linkBgcfScscf | Name: linkBgcfScscf OID: ts32-634ObjectClass 470620 | -- |
| linkHlrScscf | Name: linkHlrScscf OID: ts32-634ObjectClass 480620 | -- |
| linkIcscfSlf | Name: linkIcscfSlf OID: ts32-634ObjectClass 490620 | -- |
| linkImsMgwMgcf | Name: linkImsMgwMgcf OID: ts32-634ObjectClass 500620 | -- |
| linkImsMgwScscf | Name: linkImsMgwScscf OID: ts32-634ObjectClass 510620 | -- |
| linkMrfcMrfp | Name: linkMrfcMrfp OID: ts32-634ObjectClass 520620 | -- |
| linkMrfcScscf | Name: linkMrfcScscf OID: ts32-634ObjectClass 530620 | -- |
| linkScscfScscf | Name: linkScscfScscf OID: ts32-634ObjectClass 540620 | -- |
| linkScscfSlf | Name: linkScscfSlf OID: ts32-634ObjectClass 550620 | -- |
| Packages |  |  |
| smlcFunctionBasicPackage | Name: smlcFunctionBasicPackage OID : ts32-634Package 1 | -- |
| gmlcFunctionBasicPackage | Name: gmlcFunctionBasicPackage OID : ts32-634Package 2 | -- |
| scfFunctionBasicPackage | Name: scfFunctionBasicPackage OID : ts32-634Package 3 | -- |
| srfFunctionBasicPackage | Name: srfFunctionBasicPackage OID : ts32-634Package 4 | -- |
| cbcFunctionBasicPackage | Name: cbcFunctionBasicPackage OID : ts32-634Package 5 | -- |
| cgfFunctionBasicPackage | Name: cgfFunctionBasicPackage OID : ts32-634Package 6 | -- |
| mgwFunctionBasicPackage | -- | Name: mgwFunctionBasicPackage OID : ts32-634Package 7 |
| imsMgwFunctionBasicPackage | Name: imsMgwFunctionBasicPackage OID : ts32-634Package 70503 | -- |
| gmscFunctionBasicPackage | Name: gmscFunctionBasicPackage OID : ts32-634Package 8 | -- |
| iwfFunctionBasicPackage | Name: iwfFunctionBasicPackage OID : ts32-634Package 9 | -- |
| mnpSrfFunctionBasicPackage | Name: mnpSrfFunctionBasicPackage OID : ts32-634Package 10 | -- |
| npdbFunctionBasicPackage | Name: npdbFunctionBasicPackage OID : ts32-634Package 11 | -- |
| rSgwFunctionBasicPackage | Name: rSgwFunctionBasicPackage OID : ts32-634Package 12 | -- |
| ssfFunctionBasicPackage | Name: ssfFunctionBasicPackage OID : ts32-634Package 13 | -- |
| bsFunctionBasicPackage | Name: bsFunctionBasicPackage OID : ts32-634Package 14 | -- |
| aucFunctionBasicPackage | Name: aucFunctionBasicPackage OID : ts32-634Package 15 | -- |
| bgFunctionBasicPackage | Name: bgFunctionBasicPackage OID : ts32-634Package 16 | -- |
| eirFunctionBasicPackage | Name: eirFunctionBasicPackage OID : ts32-634Package 17 | -- |
| ggsnFunctionBasicPackage | Name: ggsnFunctionBasicPackage OID : ts32-634Package 18 | -- |
| hlrFunctionBasicPackage | Name: hlrFunctionBasicPackage OID : ts32-634Package 19 | -- |
| mscServerFunctionBasicPackage | Name: mscServerFunctionBasicPackage OID : ts32-634Package 20 | -- |
| mscServerFunctionAssociationPackage | Name: mscServerFunctionAssociationPackage OID : ts32-634Package 21 | -- |
| sgsnFunctionBasicPackage | Name: sgsnFunctionBasicPackage OID : ts32-634Package 22 | -- |
| sgsnFunctionAssociationPackage | Name: sgsnFunctionAssociationPackage OID : ts32-634Package 23 | -- |
| smsGmscFunctionBasicPackage | Name: smsGmscFunctionBasicPackage OID : ts32-634Package 24 | -- |
| smsIwmscFunctionBasicPackage | Name: smsIwmscFunctionBasicPackage OID : ts32-634Package 25 | -- |
| vlrFunctionBasicPackage | Name: vlrFunctionBasicPackage OID : ts32-634Package 26 | -- |
| gbLinkBasicPackage | Name: gbLinkBasicPackage OID : ts32-634Package 27 | -- |
| gbLinkAssociationPackage | Name: gbLinkAssociationPackage OID : ts32-634Package 28 | -- |
| aLinkBasicPackage | Name: aLinkBasicPackage OID : ts32-634Package 29 | -- |
| aLinkAssociationPackage | Name: aLinkAssociationPackage OID : ts32-634Package 30 | -- |
| iucsLinkBasicPackage | Name: iucsLinkBasicPackage OID : ts32-634Package 31 | -- |
| iucsLinkAssociationPackage | Name: iucsLinkAssociationPackage OID : ts32-634Package 32 | -- |
| iupsLinkBasicPackage | Name: iupsLinkBasicPackage OID : ts32-634Package 33 | -- |
| iupsLinkAssociationPackage | Name: iupsLinkAssociationPackage OID : ts32-634Package 34 | -- |
| iubcLinkBasicPackage | Name: iubcLinkBasicPackage OID : ts32-634Package 35 | -- |
| iubcLinkAssociationPackage | Name: iubcLinkAssociationPackage OID : ts32-634Package 36 | -- |
| csMgwFunctionBasicPackage | Name: csMgwFunctionBasicPackage OID : ts32-634Package 37 | -- |
| csMgwFunctionAssociationPackage | Name: csMgwFunctionAssociationPackage OID : ts32-634Package 38 | -- |
| gmscServerFunctionBasicPackage | Name: gmscServerFunctionBasicPackage OID : ts32-634Package 39 | -- |
| scscfFunctionBasicPackage | Name: scscfFunctionBasicPackage OID : ts32-634Package 40 | -- |
| pcscfFunctionBasicPackage | Name: pcscfFunctionBasicPackage OID : ts32-634Package 41 | -- |
| icscfFunctionBasicPackage | Name: icscfFunctionBasicPackage OID : ts32-634Package 42 | -- |
| slfFunctionBasicPackage | Name: slfFunctionBasicPackage OID : ts32-634Package 43 | -- |
| bgcfFunctionBasicPackage | Name: bgcfFunctionBasicPackage OID : ts32-634Package 44 | -- |
| mrfcFunctionBasicPackage | Name: mrfcFunctionBasicPackage OID : ts32-634Package 45 | -- |
| asFunctionBasicPackage | Name: asFunctionBasicPackage OID : ts32-634Package 46 | -- |
| mgcfFunctionBasicPackage | Name: mgcfFunctionBasicPackage OID : ts32-634Package 47 | -- |
| mrfpFunctionBasicPackage | Name: mrfpFunctionBasicPackage OID : ts32-634Package 48 | -- |
| Actions |  |  |
| Notifications |  |  |
| Attributes |  |  |
| smlcFunctionId | Name: smlcFunctionId OID : ts32-634Attribute 1 | -- |
| gmlcFunctionId | Name: gmlcFunctionId OID : ts32-634Attribute 2 | -- |
| scfFunctionId | Name: scfFunctionId OID : -634Attribute 3 | -- |
| srfFunctionId | Name: srfFunctionId OID : ts32-634Attribute 4 | -- |
| cbcFunctionId | Name: cbcFunctionId OID : ts32-634Attribute 5 | -- |
| cgfFunctionId | Name: cgfFunctionId OID : ts32-634Attribute 6 | -- |
| mgwFunctionId | -- | Name: mgwFunctionId OID : ts32-634Attribute 7 |
| imsMgwFunctionId | Name: imsMgwFunctionId OID : ts32-634Attribute 70503 | -- |
| gmscFunctionId | Name: gmscFunctionId OID : ts32-634Attribute 8 | -- |
| iwfFunctionId | Name: iwfFunctionId OID : ts32-634Attribute 9 | -- |
| mnpSrfFunctionId | Name: mnpSrfFunctionId OID : ts32-634Attribute 10 | -- |
| npdbFunctionId | Name: npdbFunctionId OID : ts32-634Attribute 11 | -- |
| rSgwFunctionId | Name: rSgwFunctionId OID : ts32-634Attribute 12 | -- |
| ssfFunctionId | Name: ssfFunctionId OID : ts32-634Attribute 13 | -- |
| bsFunctionId | Name: bsFunctionId OID : ts32-634Attribute 14 | -- |
| aucFunctionId | Name: aucFunctionId OID : ts32-634Attribute 15 | -- |
| bgFunctionId | Name: bgFunctionId OID : ts32-634Attribute 16 | -- |
| eirFunctionId | Name: eirFunctionId OID : ts32-634Attribute 17 | -- |
| ggsnFunctionId | Name: ggsnFunctionId OID : ts32-634Attribute 18 | -- |
| hlrFunctionId | Name: hlrFunctionId OID : ts32-634Attribute 19 | -- |
| mscServerFunctionId | Name: mscServerFunctionId OID : ts32-634Attribute 20 | -- |
| vlrFunctionId | Name: vlrFunctionId OID : ts32-634Attribute 21 | -- |
| sgsnFunctionId | Name: sgsnFunctionId OID : ts32-634Attribute 22 | -- |
| smsGmscFunctionId | Name: smsGmscFunctionId OID : ts32-634Attribute 23 | -- |
| smsIwmscFunctionId | Name: smsIwmscFunctionId OID : ts32-634Attribute 24 | -- |
| gbLinkId | Name: gbLinkId OID : ts32-634Attribute 25 | -- |
| aLinkId | Name: aLinkId OID : ts32-634Attribute 26 | -- |
| iucsLinkId | Name: iucsLinkId OID : ts32-634Attribute 27 | -- |
| iupsLinkId | Name: iupsLinkId OID : ts32-634Attribute 28 | -- |
| iubcLinkId | Name: iubcLinkId OID : ts32-634Attribute 29 | -- |
| csMgwFunctionId | Name: csMgwFunctionId OID : ts32-634Attribute 30 | -- |
| gmscServerFunctionId | Name: gmscServerFunctionId OID : ts32-634Attribute 31 | -- |
| mccList | Name: mccList OID : ts32-634Attribute 32 | -- |
| mncList | Name: mncList OID : ts32-634Attribute 33 | -- |
| lacList | Name: lacList OID : ts32-634Attribute 34 | -- |
| sacList | Name: sacList OID : ts32-634Attribute 35 | -- |
| uraList | -- | Name: uraList OID : ts32-634Attribute 36 |
| gcaList | Name: gcaList OID : ts32-634Attribute 37 | -- |
| mscId | Name: mscId OID : ts32-634Attribute 38 | -- |
| mscServerFunction-GSMcell | Name: mscServerFunction-GSMcell OID : ts32-634Attribute 39 | -- |
| mscServerFunction-ExternalGSMcell | Name: mscServerFunction-ExternalGSMcell OID : ts32-634Attribute 40 | -- |
| mscServerFunction-CsMgwFunction | Name: mscServerFunction-CsMgwFunction OID : ts32-634Attribute 41 | -- |
| racList | Name: racList OID : ts32-634Attribute 42 | -- |
| sgsnId | Name: sgsnId OID : -634Attribute 43 | -- |
| sgsnFunction-GSMcell | Name: sgsnFunction-GSMcell OID : ts32-634Attribute 44 | -- |
| sgsnFunction-ExternalGSMcell | Name: sgsnFunction-ExternalGSMcell OID : ts32-634Attribute 45 | -- |
| connectedBss | Name: connectedBss OID : ts32-634Attribute 46 | -- |
| connectedRnc | Name: connectedRnc OID : ts32-634Attribute 47 | -- |
| csMgwFunction-MscServerFunction | Name: csMgwFunction-MscServerFunction OID : ts32-634Attribute 48 | -- |
| csMgwFunction-IucsLink | Name: csMgwFunction-IucsLink OID : ts32-634Attribute 49 | -- |
| csMgwFunction-Alink | Name: csMgwFunction-Alink OID : ts32-634Attribute 50 | -- |
| scscfFunctionId | Name: scscfFunctionId OID : ts32-634Attribute 51 | -- |
| pcscfFunctionId | Name: pcscfFunctionId OID : ts32-634Attribute 52 | -- |
| icscfFunctionId | Name: icscfFunctionId OID : ts32-634Attribute 53 | -- |
| slfFunctionId | Name: slfFunctionId OID : ts32-634Attribute 54 | -- |
| bgcfFunctionId | Name: bgcfFunctionId OID : ts32-634Attribute 55 | -- |
| mrfcFunctionId | Name: mrfcFunctionId OID : ts32-634Attribute 56 | -- |
| asFunctionId | Name: asFunctionId OID : ts32-634Attribute 57 | -- |
| mgcfFunctionId | Name: mgcfFunctionId OID : ts32-634Attribute 58 | -- |
| mrfpFunctionId | Name: mrfpFunctionId OID : ts32-634Attribute 59 | -- |
| Parameters |  |  |
| Name Bindings |  |  |
| smlcFunction-managedElement | Name: smlcFunction-managedElement OID : ts32-634NameBinding 1 | -- |
| gmlcFunction-managedElement | Name: gmlcFunction-managedElement OID : ts32-634NameBinding 2 | -- |
| scfFunction-managedElement | Name: scfFunction-managedElement OID : ts32-634NameBinding 3 | -- |
| srfFunction-managedElement | Name: srfFunction-managedElement OID : ts32-634NameBinding 4 | -- |
| cbcFunction-managedElement | Name: cbcFunction-managedElement OID : ts32-634NameBinding 5 | -- |
| cgfFunction-managedElement | Name: cgfFunction-managedElement OID : ts32-634NameBinding 6 | -- |
| mgwFunction-managedElement | -- | Name: mgwFunction-managedElement OID : ts32-634NameBinding 7 |
| imsMgwFunction-managedElement | Name: imsMgwFunction-managedElement OID : ts32-634NameBinding 70503 | -- |
| gmscFunction-managedElement | Name: gmscFunction-managedElement OID : ts32-634NameBinding 8 | -- |
| iwfFunction-managedElement | Name: iwfFunction-managedElement OID : ts32-634NameBinding 9 | -- |
| mnpSrfFunction-managedElement | Name: mnpSrfFunction-managedElement OID : ts32-634NameBinding 10 | -- |
| npdbFunction-managedElement | Name: npdbFunction-managedElement OID : ts32-634NameBinding 11 | -- |
| rSgwFunction-managedElement | Name: rSgwFunction-managedElement OID : ts32-634NameBinding 12 | -- |
| ssfFunction-managedElement | Name: ssfFunction-managedElement OID : ts32-634NameBinding 13 | -- |
| bsFunction-managedElement | Name: bsFunction-managedElement OID : ts32-634NameBinding 14 | -- |
| aucFunction-managedElement | Name: aucFunction-managedElement OID : ts32-634NameBinding 15 | -- |
| bgFunction-managedElement | Name: bgFunction-managedElement OID : ts32-634NameBinding 16 | -- |
| eirFunction-managedElement | Name: eirFunction-managedElement OID : ts32-634NameBinding 17 | -- |
| ggsnFunction-managedElement | Name: ggsnFunction-managedElement OID : ts32-634NameBinding 18 | -- |
| hlrFunction-managedElement | Name: hlrFunction-managedElement OID : ts32-634NameBinding 19 | -- |
| mscServerFunction-managedElement | Name: mscServerFunction-managedElement OID : ts32-634NameBinding 20 | -- |
| vlrFunction-managedElement | Name: vlrFunction-managedElement OID : ts32-634NameBinding 21 | -- |
| sgsnFunction-managedElement | Name: sgsnFunction-managedElement OID : ts32-634NameBinding 22 | -- |
| smsGmscFunction-managedElement | Name: smsGmscFunction-managedElement OID : ts32-634NameBinding 23 | -- |
| smsIwmscFunction-managedElement | Name: smsIwmscFunction-managedElement OID : ts32-634NameBinding 24 | -- |
| gbLink-managedElement | Name: gbLink-managedElement OID : ts32-634NameBinding 25 | -- |
| aLink-managedElement | Name: aLink-managedElement OID : ts32-634NameBinding 26 | -- |
| iucsLink-managedElement | Name: iucsLink-managedElement OID : ts32-634NameBinding 27 | -- |
| iupsLink-managedElement | Name: iupsLink-managedElement OID : ts32-634NameBinding 28 | -- |
| iubcLink-managedElement | Name: iubcLink-managedElement OID : ts32-634NameBinding 29 | -- |
| gmscServerFunction-managedElement | Name: gmscServerFunction-managedElement OID : ts32-634NameBinding 30 | -- |
| csMgwFunction-managedElement | Name: csMgwFunction-managedElement OID : ts32-634NameBinding 31 | -- |
| scscfFunction-managedElement | Name: scscfFunction-managedElement OID : ts32-634NameBinding 32 | -- |
| pcscfFunction-managedElement | Name: pcscfFunction-managedElement OID : ts32-634NameBinding 33 | -- |
| icscfFunction-managedElement | Name: icscfFunction-managedElement OID : ts32-634NameBinding 34 | -- |
| slfFunction-managedElement | Name: slfFunction-managedElement OID : ts32-634NameBinding 35 | -- |
| bgcfFunction-managedElement | Name: bgcfFunction-managedElement OID : ts32-634NameBinding 36 | -- |
| mrfcFunction-managedElement | Name: mrfcFunction-managedElement OID : ts32-634NameBinding 37 | -- |
| scscfFunction-managedElement | Name: scscfFunction-managedElement OID : ts32-634NameBinding 38 | -- |
| scscfFunction-managedElement | Name: scscfFunction-managedElement OID : ts32-634NameBinding 39 | -- |
| mrfpFunction-managedElement | Name: mrfpFunction-managedElement OID : ts32-634NameBinding 40 | -- |
| linkAsAuc - subNetworkR60 | Name: linkAsAuc – subNetworkR60 OID: ts32-634NameBinding 410620 | -- |
| linkAsHlr - subNetworkR60 | Name: linkAsHlr – subNetworkR60 OID: ts32-634NameBinding 420620 | -- |
| linkAsScscf - subNetworkR60 | Name: linkAsScscf – subNetworkR60 OID: ts32-634NameBinding 430620 | -- |
| linkAsSlf - subNetworkR60 | Name: linkAsSlf – subNetworkR60 OID: ts32-634NameBinding 440620 | -- |
| linkBgcfBgcf - subNetworkR60 | Name: linkBgcfBgcf – subNetworkR60 OID: ts32-634NameBinding 450620 | -- |
| linkBgcfImsMgw - subNetworkR60 | Name: linkBgcfImsMgw – subNetworkR60 OID: ts32-634NameBinding 460620 | -- |
| linkBgcfScscf - subNetworkR60 | Name: linkBgcfScscf – subNetworkR60 OID: ts32-634NameBinding 470620 | -- |
| linkHlrScscf - subNetworkR60 | Name: linkHlrScscf – subNetworkR60 OID: ts32-634NameBinding 480620 | -- |
| linkIcscfSlf - subNetworkR60 | Name: linkIcscfSlf – subNetworkR60 OID: ts32-634NameBinding 490620 | -- |
| linkImsMgwMgcf - subNetworkR60 | Name: linkImsMgwMgcf – subNetworkR60 OID: ts32-634NameBinding 500620 | -- |
| linkImsMgwScscf - subNetworkR60 | Name: linkImsMgwScscf - subNetworkR60 OID: ts32-634NameBinding 510620 | -- |
| linkMrfcMrfp - subNetworkR60 | Name: linkMrfcMrfp – subNetworkR60 OID: ts32-634NameBinding 520620 | -- |
| linkMrfcScscf - subNetworkR60 | Name: linkMrfcScscf – subNetworkR60 OID: ts32-634NameBinding 530620 | -- |
| linkScscfScscf - subNetworkR60 | Name: linkScscfScscf – subNetworkR60 OID: ts32-634NameBinding 540620 | -- |
| linkScscfSlf - subNetworkR60 | Name: linkScscfSlf – subNetworkR60 OID: ts32-634NameBinding 550620 | -- |

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Jun 2001 | SA_12 | SP-010283 | -- | -- | Approved at TSG SA #12 and placed under Change Control | -- | 2.0.0 | 4.0.0 |
| Sep 2001 | SA_13 | SP-010478 | 0001 | -- | Correction due to TS renumbering | F | 4.0.0 | 4.1.0 |
| Sep 2002 | -- | -- | -- | -- | Cosmetics/Styles | -- | 4.1.0 | 4.1.1 |
| Dec 2002 | SA_18 | SP-020749 | 0002 | -- | Alignment of the CMIP SS with the Rel-5 version of the IS in 32.632 | F | 4.1.1 | 5.0.0 |
| Dec 2003 | SA_22 | SP-030642 | 0003 | -- | Add notifications to functional objects - Align with 32.632 (IS) | F | 5.0.0 | 5.1.0 |
| Mar 2004 | SA_23 | SP-040130 | 0004 | -- | Removal of the attribute uraList from the MOC MscServerFunction – Alignment with the IS 32.632 | F | 5.1.0 | 5.2.0 |
| Sep 2004 | SA_25 | SP-040582 | 0005 | -- | Correction of modelling of Media GateWay (MGW) | F | 5.2.0 | 5.3.0 |
| Sep 2004 | SA_25 | SP-040591 | 0006 | -- | Removal of the 3GPP Release# cross references in the GDMO section | F | 5.2.0 | 5.3.0 |
| Sep 2004 | SA_25 | SP-040541 | -- | -- | Automatic upgrade to Rel- 6 (no CR) as per request in SP-040541 SA5_presentation_SA_25.ppt (slide 17) | -- | 5.3.0 | 6.0.0 |
| Mar 2005 | SA_27 | SP-050047 | 0009 | -- | Align with 32.632, regarding the IS template and UML repertoire | A | 6.0.0 | 6.1.0 |
| Mar 2005 | SA_27 | SP-050047 | 0010 | -- | Add the IMS objects - Align with 32.632 Configuration Management (CM); Core Network Resources IRP NRM | F | 6.0.0 | 6.1.0 |
| Dec 2005 | SA_30 | SP-050725 | 0011 | -- | Add link objects to the CN Resource Model IRP CMIP SS - Align with 32.632 | F | 6.1.0 | 6.2.0 |