3GPP TS 32.714 V6.0.0 (2004-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Configuration Management (CM); Transport Network (TN)

Interface Network Resource Model (NRM) Integration

Reference Point (IRP): Common Management Information

Protocol (CMIP) Solution Set (SS)

(Release 6)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Keywords

UMTS, management, CMIP

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2004, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	Architectural features	7

4.1	Notifications	7

5	Mapping	7

5.1	Transport NRM Information Object Class (IOC) mapping	7

5.2	Mapping of IOC Attributes	7

5.2.1	General	7

5.2.2	IOC TransportNetworkInterface	8

5.2.3	IOC ATMChannelTerminationPoint	8

5.2.4	IOC ATMPathTerminationPoint	8

-- 6   GDMO Definitions	9

-- 6.1   Managed Object Classes	9

-- 6.1.1   transportNetworkInterface	9

-- 6.1.2   aTMChannelTerminationPoint	9

-- 6.1.3   aTMPathTerminationPoint	9

-- 6.2    Packages	9

-- 6.2.1   transportNetworkInterfaceMandatoryAttributesPackage	9

-- 6.2.2   aTMChannelTerminationPointMandatoryAttributesPackage	10

-- 6.2.3   aTMChannelTerminationPointOptionalAttributesPackage1	10

-- 6.2.4   aTMChannelTerminationPointOptionalAttributesPackage2	10

-- 6.2.5   aTMChannelTerminationPointOptionalAttributesPackage3	10

-- 6.2.6   aTMChannelTerminationPointOptionalAttributesPackage4	11

-- 6.2.7   aTMChannelTerminationPointOptionalAttributesPackage5	11

-- 6.2.8   aTMChannelTerminationPointOptionalAttributesPackage6	11

-- 6.2.9   aTMPathTerminationPointMandatoryAttributesPackage	11

-- 6.3   Attributes	12

-- 6.3.1   transportNetworkInterfaceId	12

-- 6.3.2   userLabel	12

-- 6.3.3   transportNetworkType	12

-- 6.3.4   aTMChannelTerminationPointId	12

-- 6.3.5   usageChannel	13

-- 6.3.6   virtualPathId	13

-- 6.3.7   virtualChannelId	13

-- 6.3.8   physicalPortId	13

-- 6.3.9   physicalLinkType	13

-- 6.3.10   physicalInterfaceType	14

-- 6.3.11   serviceCategoryIn	14

-- 6.3.12   serviceCategoryEg	14

-- 6.3.13   usedAAL	14

-- 6.3.14   peakCellRateIn	14

-- 6.3.15   peakCellRateEg	15

-- 6.3.16   sustainableCellRateIn	15

-- 6.3.17   sustainableCellRateEg	15

-- 6.3.18   maximumBurstSizeIn	15

-- 6.3.19   maximumBurstSizeEg	16

-- 6.3.20   minimumDesiredCellRateIn	16

-- 6.3.21   minimumDesiredCellRateEg	16

-- 6.3.22   minimumCellRateIn	16

-- 6.3.23   minimumCellRateEg	16

-- 6.3.24   aTMChannelTerminationPoint-aTMPathTerminationPoint	17

-- 6.3.25   aTMChannelTerminationPoint-iubLink	17

-- 6.3.26   aTMPathTerminationPointId	17

-- 6.3.27   physicalPortIdList	17

-- 6.3.28   aTMPathTerminationPoint-aTMChannelTerminationPoint	17

-- 6.4   Name-Binding	18

-- 6.4.1   transportNetworkInterface-managedElement	18

-- 6.4.2   aTMPathTerminationPoint-transportNetworkInterface	18

-- 6.4.3   aTMChannelTerminationPoint-transportNetworkInterface	18

-- 7   ASN.1 definitions for the Transport Network NRM	20

Annex A (informative):	List of assigned Object Identifiers	22

Annex B (informative):	Change history	24

# Foreword

This Technical Specification (TS) has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the

third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part a TS-family covering the 3rd Generation Partnership Project: Technical Specification Group Services and System Aspects; Telecommunication management; Configuration Management (CM); as identified below:

TS 32.711	“Transport Network (TN) Network Resource Model (NRM) Integration Reference Point (IRP): Requirements ";

TS 32.712	“Transport Network (TN) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)";

TS 32.713	“Transport Network (TN) Network Resource Model (NRM) Integration Reference Point (IRP): Common Object Request Broker Architecture (CORBA) Solution Set (SS)";

TS 32.714	“Transport Network (TN) Network Resource Model (NRM) Integration Reference Point (IRP): Common Management Information Protocol (CMIP) Solution Set (SS)".

TS 32.715	“Transport Network (TN) Network Resource Model (NRM) Integration Reference Point (IRP): Bulk CM eXtensible Markup Language (XML) file format definition

The present document is part of a TS-family which describe the requirements and information model necessary for the Telecommunication Management (TM) of 3G systems. The TM principles and TM architecture are specified in 3GPP TS 32.101 [1] and 3GPP TS 32.102 [2].

# 1	Scope

The purpose of this Transport Network (TN) interface Network Resource Model IRP: CMIP Solution Set is to define the mapping of the IRP information model (see 3GPP TS 32.712 [4]) to the protocol specific details necessary for implementation of this IRP in a Q3 environment.

This Solution Set specification is related to 3GPP TS 32.712 V6.0.X

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[4]	3GPP TS 32.712: "Telecommunication management; Configuration Management (CM); Transport Network (TN) interface Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[5]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[6]	3GPP TS 32.304: "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP): Common Management Information Protocol  Solution Set (SS)".

# 3	Definitions and abbreviations

## 3.1	Definitions

For terms and definitions please refer to 3GPP TS 32.101 [1], 3GPP TS 32.102 [2], 3GPP TS 32.600 [3] and 3GPP TS 32.712 [4].

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

ASN.1	Abstarct Syntax Notation 1

CMIP	Common Management Information Protocol

DN	Distinguished Name

GDMO	Guidelines for the Definition of Managed Objects

IS	Information Service

IOC	Information Object Class

IRP	Integration Reference Point

MO	Managed Object

MOC	Managed Object Class

NRM	Network Resource Model

SS	Solution Set

# 4	Architectural features

The overall architectural feature of Transport Network Resources IRP is specified in 3GPP TS 32.712 [4]. 
This clause specifies features that are specific to the CMIP SS.

## 4.1	Notifications

Notifications are sent according to the Notification IRP: CMIP SS (see 3GPP TS 32.304 [6]).

# 5	Mapping

## 5.1	Transport NRM Information Object Class (IOC) mapping

For the Transport Network NRM CMIP Solution Set the Information Object Classes (IOC) and the Interfaces defined in 3GPP TS 32.712 [7] are mapped onto Managed Object Classes (MOC) as given in Table 1. These MOC include all the Attributes, Actions and Notifications necessary to model performance management as described in 3GPP TS 32.712 [7].

Table 5.1: Mapping of Information Object Classes

| IS IOC | CMIP SS MOC |
| --- | --- |
| TransportNetworkInterface | transportNetworkInterface |
| ATMChannelTerminationPoint | aTMChannelTerminationPoint |
| ATMPathTerminationPoint | aTMPathTerminationPoint |

## 5.2	Mapping of IOC Attributes

### 5.2.1	General

The IS parameter name “managedObjectInstance” is mapped into DN.

Attributes modelling associations as defined in the NRM (here also called “reference attributes”) are mapped onto attributes in this SS. The names of the reference attributes in the NRM are mapped to the corresponding attribute names in the MOC. When the cardinality for an association is 0..1 or 1..1 the datatype for the reference attribute is defined as an MOReference. The value of an MOReference contains the distinguished name of the associated MO. When the cardinality for an association allows more than one referred MO, the reference attribute will be of type MOReferenceSet, which contains a sequence of MOReferences.

### 5.2.2	IOC TransportNetworkInterface

Table 5.2: Attribute Mapping of the IOC TransportNetworkInterface

| IS Attribute | CMIP SS Attributes | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| transportNetworkInterfaceId | transportNetworkInterfaceId | M | M | - |
| userLabel | userLabel | M | M | M |
| transportNetworkType | transportNetworkType | M | M | - |

### 5.2.3	IOC ATMChannelTerminationPoint

Table 5.3: Attribute Mapping of IOC ATMChannelTerminationPoint

| IS Attribute | CMIP SS Attributes | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| aTMChannelTerminationPointId | aTMChannelTerminationPointId | M | M | -- |
| usageChannel | usageChannel | M | M | -- |
| virtualPathId | virtualPathId | M | M | O |
| virtualChannelId | virtualChannelId | M | M | O |
| physicalPortId | physicalPortId | M | M | O |
| physicalLinkType | physicalLinkType | M | M | O |
| serviceCategoryIn | serviceCategoryIn | M | M | O |
| serviceCategoryEg | serviceCategoryEg | M | M | O |
| usedAAL | usedAAL | M | M | O |
| peakCellRateIn | peakCellRateIn | M | M | O |
| peakCellRateEg | peakCellRateEg | M | M | O |
| sustainableCellRateIn | sustainableCellRateIn | O | M | O |
| sustainableCellRateEg | sustainableCellRateEg | O | M | O |
| maximumBurstSizeIn | maximumBurstSizeIn | M | M | O |
| maximumBurstSizeEg | maximumBurstSizeEg | M | M | O |
| minimumDesiredCellRateIn | minimumDesiredCellRateIn | O | M | O |
| minimumDesiredCellRateEg | minimumDesiredCellRateEg | O | M | O |
| minimumCellRateIn | minimumCellRateIn | O | M | O |
| minimumCellRateEg | minimumCellRateEg | O | M | O |
| aTMChannelTerminationPoint- ATMPathTerminationPoint | aTMChannelTerminationPoint-aTMPathTerminationPoint | M | M | -- |
| aTMChannelTerminationPoint- IubLink | aTMChannelTerminationPoint-iubLink | M | M | -- |

### 5.2.4	IOC ATMPathTerminationPoint

Table 5.4: Attribute Mapping of NRM IOC ATMPathTerminationPoint

| IS Attribute | CMIP SS Attributes | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| aTMPathTerminationPointId | aTMPathTerminationPointId | M | M | -- |
| virtualPathId | virtualPathId | M | M | O |
| physicalPortIdList | physicalPortIdList | M | M | O |
| peakCellRateIn | peakCellRateIn | M | M | O |
| peakCellRateEg | peakCellRateEg | M | M | O |
| aTMPathTerminationPoint-ATMChannelTerminationPoint | aTMPathTerminationPoint-aTMChannelTerminationPoint | M | M | -- |

# -- 6   GDMO Definitions

--Please do not remove the “—“ in front of the headline numbering, as it is the CMIP code

--for a comment. This way the whole chapter can be put directly into a compiler.

## -- 6.1   Managed Object Classes

### -- 6.1.1   transportNetworkInterface

transportNetworkInterface MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS32.622" : managedFunction;

CHARACTERIZED BY

transportNetworkInterfaceMandatoryAttributesPackage;

REGISTERED AS {ts32-714ObjectClass 10600};

### -- 6.1.2   aTMChannelTerminationPoint

aTMChannelTerminationPoint MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS32.622" : top;

CHARACTERIZED BY

aTMChannelTerminationPointMandatoryAttributesPackage;

CONDITIONAL PACKAGES

aTMChannelTerminationPointOptionalAttributesPackage1

PRESENT IF

"an instance supports it",

aTMChannelTerminationPointOptionalAttributesPackage2

PRESENT IF

"an instance supports it",

aTMChannelTerminationPointOptionalAttributesPackage3

PRESENT IF

"an instance supports it",

aTMChannelTerminationPointOptionalAttributesPackage4

PRESENT IF

"an instance supports it",

aTMChannelTerminationPointOptionalAttributesPackage5

PRESENT IF

"an instance supports it",

aTMChannelTerminationPointOptionalAttributesPackage6

PRESENT IF

"an instance supports it";

REGISTERED AS {ts32-714ObjectClass 20600};

### -- 6.1.3   aTMPathTerminationPoint

aTMPathTerminationPoint MANAGED OBJECT CLASS

DERIVED FROM

"3GPP TS32.622" : top;

CHARACTERIZED BY

aTMPathTerminationPointMandatoryAttributesPackage;

REGISTERED AS {ts32-714ObjectClass 30600};

## -- 6.2    Packages

### -- 6.2.1   transportNetworkInterfaceMandatoryAttributesPackage

transportNetworkInterfaceMandatoryAttributesPackage PACKAGE

BEHAVIOUR

transportNetworkInterfaceMandatoryAttributesPackageBehaviour;

ATTRIBUTES

transportNetworkInterfaceId      GET,

userLabel                        GET-REPLACE,

transportNetworkType             GET;

REGISTERED AS {ts32-714Package 10600};

transportInterfaceMandatoryAttributesPackageBehaviour BEHAVIOUR

DEFINED AS

"The attributes in this package are described in 3GPP TS32.712 [4]. ";

### -- 6.2.2   aTMChannelTerminationPointMandatoryAttributesPackage

aTMChannelTerminationPointMandatoryAttributesPackage PACKAGE

BEHAVIOUR

aTMChannelTerminationPointMandatoryAttributesPackageBehaviour;

ATTRIBUTES

aTMChannelTerminationPointId                             GET,

usageChannel                                             GET,

virtualPathId                                            GET,

virtualChannelId                                         GET,

physicalPortId                                           GET,

physicalInterfaceType                                    GET,

serviceCategoryIn                                        GET,

serviceCategoryEg                                        GET,

usedAAL                                                  GET,

peakCellRateIn                                           GET,

peakCellRateEg                                           GET,

maximumBurstSizeIn                                       GET,

maximumBurstSizeEg                                       GET,

aTMChannelTerminationPoint-aTMPathTerminationPoint       GET,

aTMChannelTerminationPoint-iubLink                       GET;

REGISTERED AS {ts32-714Package 20600};

aTMChannelTerminationPointMandatoryAttributesPackageBehaviour BEHAVIOUR

DEFINED AS

"The attributes in this package are described in 3GPP TS32.712 [4].

aTMChannelTerminationPointId, usageChannel, aTMChannelTerminationPoint-aTMPathTerminationPoint,

aTMChannelTerminationPoint-iubLink are striktly read-only, all the others are read-write if an

instance of the managed object supports read-write.";

### -- 6.2.3   aTMChannelTerminationPointOptionalAttributesPackage1

aTMChannelTerminationPointOptionalAttributesPackage1 PACKAGE

BEHAVIOUR

aTMChannelTerminationPointOptionalAttributesPackage1Behaviour;

ATTRIBUTES

sustainableCellRateIn               GET;

REGISTERED AS {ts32-714Package 30600};

aTMChannelTerminationPointOptionalAttributesPackage1Behaviour BEHAVIOUR

DEFINED AS

"The attributes in this package are described in 3GPP TS32.712 [4].

The attributes are are read-only, unless an instance of the managed object supports

read-write.";

### -- 6.2.4   aTMChannelTerminationPointOptionalAttributesPackage2

aTMChannelTerminationPointOptionalAttributesPackage2 PACKAGE

BEHAVIOUR

aTMChannelTerminationPointOptionalAttributesPackage2Behaviour;

ATTRIBUTES

sustainableCellRateEg               GET;

REGISTERED AS {ts32-714Package 40600};

aTMChannelTerminationPointOptionalAttributesPackage2Behaviour BEHAVIOUR

DEFINED AS

"The attributes in this package are described in 3GPP TS32.712 [4].

The attributes are are read-only, unless an instance of the managed object supports

read-write.";

### -- 6.2.5   aTMChannelTerminationPointOptionalAttributesPackage3

aTMChannelTerminationPointOptionalAttributesPackage3 PACKAGE

BEHAVIOUR

aTMChannelTerminationPointOptionalAttributesPackage3Behaviour;

ATTRIBUTES

minimumDesiredCellRateIn            GET;

REGISTERED AS {ts32-714Package 50600};

aTMChannelTerminationPointOptionalAttributesPackage3Behaviour BEHAVIOUR

DEFINED AS

"The attributes in this package are described in 3GPP TS32.712 [4].

The attributes are are read-only, unless an instance of the managed object supports

read-write.";

### -- 6.2.6   aTMChannelTerminationPointOptionalAttributesPackage4

aTMChannelTerminationPointOptionalAttributesPackage4 PACKAGE

BEHAVIOUR

aTMChannelTerminationPointOptionalAttributesPackage4Behaviour;

ATTRIBUTES

minimumDesiredCellRateEg            GET;

REGISTERED AS {ts32-714Package 60600};

aTMChannelTerminationPointOptionalAttributesPackageBehaviour BEHAVIOUR

DEFINED AS

"The attributes in this package are described in 3GPP TS32.712 [4].

The attributes are are read-only, unless an instance of the managed object supports

read-write.";

### -- 6.2.7   aTMChannelTerminationPointOptionalAttributesPackage5

aTMChannelTerminationPointOptionalAttributesPackage5 PACKAGE

BEHAVIOUR

aTMChannelTerminationPointOptionalAttributesPackage5Behaviour;

ATTRIBUTES

minimumCellRateIn                   GET;

REGISTERED AS {ts32-714Package 70600};

aTMChannelTerminationPointOptionalAttributesPackage5Behaviour BEHAVIOUR

DEFINED AS

"The attributes in this package are described in 3GPP TS32.712 [4].

The attributes are are read-only, unless an instance of the managed object supports

read-write.";

### -- 6.2.8   aTMChannelTerminationPointOptionalAttributesPackage6

aTMChannelTerminationPointOptionalAttributesPackage6 PACKAGE

BEHAVIOUR

aTMChannelTerminationPointOptionalAttributesPackage6Behaviour;

ATTRIBUTES

minimumCellRateEg                   GET;

REGISTERED AS {ts32-714Package 80600};

aTMChannelTerminationPointOptionalAttributesPackage6Behaviour BEHAVIOUR

DEFINED AS

"The attributes in this package are described in 3GPP TS32.712 [4].

The attributes are are read-only, unless an instance of the managed object supports

read-write.";

### -- 6.2.9   aTMPathTerminationPointMandatoryAttributesPackage

aTMPathTerminationPointMandatoryAttributesPackage        PACKAGE

BEHAVIOUR

aTMPathTerminationPointMandatoryAttributesPackageBehaviour;

ATTRIBUTES

aTMPathTerminationPointId         GET,

virtualPathId                     GET,

physicalPortIdList                GET,

peakCellRateIn                    GET,

peakCellRateEg                    GET,

aTMPathTerminationPoint-aTMChannelTerminationPoint      GET;

REGISTERED AS   {ts32-714Package 90600};

aTMPathTerminationPointMandatoryAttributesPackageBehaviour BEHAVIOUR

DEFINED AS

"The attributes in this package are described in 3GPP TS32.712 [4].

aTMPathTerminationPointId, aTMPathTerminationPoint-aTMChannelTerminationPoint are striktly read-

only, all the others are read-write, if an instance of the managed object supports read-write.";

## -- 6.3   Attributes

### -- 6.3.1   transportNetworkInterfaceId

transportNetworkInterfaceId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

transportNetworkInterfaceIdBehaviour;

REGISTERED AS {ts32-714Attribute 10600};

transportNetworkInterfaceIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.2   userLabel

userLabel ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.UserLabel;

MATCHES FOR

EQUALITY;

BEHAVIOUR

userLabelBehaviour;

REGISTERED AS {ts32-714Attribute 20600};

userLabelBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.3   transportNetworkType

transportNetworkType ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.TransportNetworkType;

MATCHES FOR

EQUALITY;

BEHAVIOUR

transportNetworkTypeBehaviour;

REGISTERED AS {ts32-714Attribute 30600};

transportNetworkTypeBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.4   aTMChannelTerminationPointId

aTMChannelTerminationPointId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

aTMChannelTerminationPointIdBehaviour;

REGISTERED AS {ts32-714Attribute 40600};

aTMChannelTerminationPointIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.5   usageChannel

usageChannel ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.UsageChannel;

MATCHES FOR

EQUALITY;

BEHAVIOUR

usageChannelBehaviour;

REGISTERED AS {ts32-714Attribute 50600};

usageChannelBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.6   virtualPathId

virtualPathId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.VirtualPathId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

virtualPathIdBehaviour;

REGISTERED AS {ts32-714Attribute 60600};

virtualPathIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.7   virtualChannelId

virtualChannelId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.VirtualChannelId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

virtualChannelIdBehaviour;

REGISTERED AS {ts32-714Attribute 70600};

virtualChannelIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.8   physicalPortId

physicalPortId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.PhysicalPortId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

physicalPortIdBehaviour;

REGISTERED AS {ts32-714Attribute 80600};

physicalPortIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.9   physicalLinkType

physicalLinkType ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.PhysicalLinkType;

MATCHES FOR

EQUALITY;

BEHAVIOUR

physicalLinkTypeBehaviour;

REGISTERED AS {ts32-714Attribute 90600};

physicalLinkTypeBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.10   physicalInterfaceType

physicalInterfaceType ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.PhysicalInterfaceType;

MATCHES FOR

EQUALITY;

BEHAVIOUR

physicalInterfaceTypeBehaviour;

REGISTERED AS {ts32-714Attribute 100600};

physicalLInterfaceTypeBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.11   serviceCategoryIn

serviceCategoryIn ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.ServiceCategoryIn;

MATCHES FOR

EQUALITY;

BEHAVIOUR

serviceCategoryInBehaviour;

REGISTERED AS {ts32-714Attribute 110600};

serviceCategoryInBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.12   serviceCategoryEg

serviceCategoryEg ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.ServiceCategoryEg;

MATCHES FOR

EQUALITY;

BEHAVIOUR

serviceCategoryEgBehaviour;

REGISTERED AS {ts32-714Attribute 120600};

serviceCategoryEgBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.13   usedAAL

usedAAL ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.UsedAAL;

MATCHES FOR

EQUALITY;

BEHAVIOUR

usedAALBehaviour;

REGISTERED AS {ts32-714Attribute 130600};

usedAALBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.14   peakCellRateIn

peakCellRateIn ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.PeakCellRateIn;

MATCHES FOR

EQUALITY;

BEHAVIOUR

peakCellRateInBehaviour;

REGISTERED AS {ts32-714Attribute 140600};

peakCellRateInBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.15   peakCellRateEg

peakCellRateEg ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.PeakCellRateEg;

MATCHES FOR

EQUALITY;

BEHAVIOUR

peakCellRateEgBehaviour;

REGISTERED AS {ts32-714Attribute 150600};

peakCellRateEgBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.16   sustainableCellRateIn

sustainableCellRateIn ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.SustainableCellRateIn;

MATCHES FOR

EQUALITY;

BEHAVIOUR

sustainableCellRateInBehaviour;

REGISTERED AS {ts32-714Attribute 160600};

sustainableCellRateInBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.17   sustainableCellRateEg

sustainableCellRateEg ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.SustainableCellRateEg;

MATCHES FOR

EQUALITY;

BEHAVIOUR

sustainableCellRateEgBehaviour;

REGISTERED AS {ts32-714Attribute 170600};

sustainableCellRateEgBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.18   maximumBurstSizeIn

maximumBurstSizeIn ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.MaximumBurstSizeIn;

MATCHES FOR

EQUALITY;

BEHAVIOUR

maximumBurstSizeInBehaviour;

REGISTERED AS {ts32-714Attribute 1870600};

maximumBurstSizeInBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.19   maximumBurstSizeEg

maximumBurstSizeEg ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.MaximumBurstSizeEg;

MATCHES FOR

EQUALITY;

BEHAVIOUR

maximumBurstSizeEgBehaviour;

REGISTERED AS {ts32-714Attribute 190600};

maximumBurstSizeEgBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.20   minimumDesiredCellRateIn

minimumDesiredCellRateIn ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.MinimumDesiredCellRateIn;

MATCHES FOR

EQUALITY;

BEHAVIOUR

minimumDesiredCellRateInBehaviour;

REGISTERED AS {ts32-714Attribute 200600};

minimumDesiredCellRateInBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.21   minimumDesiredCellRateEg

minimumDesiredCellRateEg ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.MinimumDesiredCellRateEg;

MATCHES FOR

EQUALITY;

BEHAVIOUR

minimumDesiredCellRateEgBehaviour;

REGISTERED AS {ts32-714Attribute 210600};

minimumDesiredCellRateEgBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.22   minimumCellRateIn

minimumCellRateIn ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.MinimumCellRateIn;

MATCHES FOR

EQUALITY;

BEHAVIOUR

minimumCellRateInBehaviour;

REGISTERED AS {ts32-714Attribute 220600};

minimumCellRateInBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.23   minimumCellRateEg

minimumCellRateEg ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.MinimumCellRateEg;

MATCHES FOR

EQUALITY;

BEHAVIOUR

minimumCellRateEgBehaviour;

REGISTERED AS {ts32-714Attribute 230600};

minimumCellRateEgBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.24   aTMChannelTerminationPoint-aTMPathTerminationPoint

aTMChannelTerminationPoint-aTMPathTerminationPoint ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.ATMChannelTerminationPoint-aTMPathTerminationPoint;

MATCHES FOR

EQUALITY;

BEHAVIOUR

aTMChannelTerminationPoint-aTMPathTerminationPointBehaviour;

REGISTERED AS {ts32-714Attribute 240600};

aTMChannelTerminationPoint-aTMPathTerminationPointBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.25   aTMChannelTerminationPoint-iubLink

aTMChannelTerminationPoint-iubLink ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.ATMChannelTerminationPoint-iubLink;

MATCHES FOR

EQUALITY;

BEHAVIOUR

aTMChannelTerminationPoint-iubLinkBehaviour;

REGISTERED AS {ts32-714Attribute 250600};

aTMChannelTerminationPoint-iubLinkBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.26   aTMPathTerminationPointId

aTMPathTerminationPointId ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.GeneralObjectId;

MATCHES FOR

EQUALITY;

BEHAVIOUR

aTMPathTerminationPointIdBehaviour;

REGISTERED AS {ts32-714Attribute 260600};

aTMPathTerminationPointIdBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.27   physicalPortIdList

physicalPortIdList ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.PhysicalPortIdList;

MATCHES FOR

EQUALITY;

BEHAVIOUR

physicalPortIdListBehaviour;

REGISTERED AS {ts32-714Attribute 270600};

physicalPortIdListBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

### -- 6.3.28   aTMPathTerminationPoint-aTMChannelTerminationPoint

aTMPathTerminationPoint-aTMChannelTerminationPoint ATTRIBUTE

WITH ATTRIBUTE SYNTAX

TS32-714TypeModule.ATMPathTerminationPoint-aTMChannelTerminationPoint;

MATCHES FOR

EQUALITY;

BEHAVIOUR

aTMPathTerminationPoint-aTMChannelTerminationPointBehaviour;

REGISTERED AS {ts32-714Attribute 280600};

aTMPathTerminationPoint-aTMChannelTerminationPointBehaviour BEHAVIOUR

DEFINED AS

"This attribute is described in 3GPP TS32.712 [4].";

## -- 6.4   Name-Binding

### -- 6.4.1   transportNetworkInterface-managedElement

transportNetworkInterface-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS

transportNetworkInterface;

NAMED BY SUPERIOR OBJECT CLASS

managedElement;

WITH ATTRIBUTE

aTMPathTerminationPointId;

BEHAVIOUR

transportNetworkInterface-managedElementBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-714NameBinding 10600};

transportNetworkInterface-managedElementBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a managedElement montains

and controls a Transport Network Interface. When automatic instance naming is used, the choice

of name bindings left as a local matter.";

### -- 6.4.2   aTMPathTerminationPoint-transportNetworkInterface

aTMPathTerminationPoint-transportNetworkInterface NAME BINDING

SUBORDINATE OBJECT CLASS

aTMPathTerminationPoint;

NAMED BY SUPERIOR OBJECT CLASS

transportNetworkInterface;

WITH ATTRIBUTE

aTMPathTerminationPointId;

BEHAVIOUR

aTMPathTerminationPoint-transportNetworkInterfaceBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-714NameBinding 20600};

aTMPathTerminationPoint-transportNetworkInterfaceBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a Transport Network Interface contains

and controls a aTMPathTerminationPoint. When automatic instance naming is used, the choice

of name bindings left as a local matter.";

### -- 6.4.3   aTMChannelTerminationPoint-transportNetworkInterface

aTMChannelTerminationPoint-transportNetworkInterface NAME BINDING

SUBORDINATE OBJECT CLASS

aTMChannelTerminationPoint;

NAMED BY SUPERIOR OBJECT CLASS

transportNetworkInterface;

WITH ATTRIBUTE

aTMChannelTerminationPointId;

BEHAVIOUR

aTMChannelTerminationPoint-transportNetworkInterfaceBehaviour;

CREATE

WITH-REFERENCE-OBJECT, WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {ts32-714NameBinding 30600};

aTMChannelTerminationPoint-transportNetworkInterfaceBehaviour BEHAVIOUR

DEFINED AS

"The name binding represents a relationship in which a Transport Network Interface contains

and controls a aTMChannelTerminationPoint. When automatic instance naming is used, the choice

of name bindings left as a local matter.";

# -- 7   ASN.1 definitions for the Transport Network NRM

TS32-714TypeModule {itu-t(0) identified-organization(4) etsi(0) mobileDomain(0) umts-Operation-Maintenance(3) ts-32-714(714) informationModel(0) asn1Module(2) version10600(10600)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

--EXPORTS everything

--IMPORTS nothing

-- 3GPP TS 32.714 related Object Identifiers

baseNodeUMTS           OBJECT IDENTIFIER ::= {itu-t(0) identified-organization(4)

etsi(0) mobileDomain(0)

umts-Operation-Maintenance(3)}

ts32-714               OBJECT IDENTIFIER ::= {baseNodeUMTS ts32-714(714)}

ts32-714InfoModel      OBJECT IDENTIFIER ::= {ts32-714 informationModel(0)}

ts32-714ObjectClass    OBJECT IDENTIFIER ::= {ts32-714InfoModel managedObjectClass(3)}

ts32-714Package        OBJECT IDENTIFIER ::= {ts32-714InfoModel package(4)}

ts32-714Parameter      OBJECT IDENTIFIER ::= {ts32-714InfoModel parameter(5)}

ts32-714NameBinding    OBJECT IDENTIFIER ::= {ts32-714InfoModel nameBinding(6)}

ts32-714Attribute      OBJECT IDENTIFIER ::= {ts32-714InfoModel attribute(7)}

ts32-714Action         OBJECT IDENTIFIER ::= {ts32-714InfoModel action(9)}

ts32-714Notification   OBJECT IDENTIFIER ::= {ts32-714InfoModel notification(10)}

-- Start of 3GPP SA5 own definitions

ATMChannelTerminationPointId ::= INTEGER

ATMChannelTerminationPoint-aTMPathTerminationPoint ::= GraphicString

ATMChannelTerminationPoint-iubLink ::= GraphicString

ATMPathTerminationPointId ::= INTEGER

ATMPathTerminationPoint-aTMChannelTerminationPoint ::= GraphicString

MaximumBurstSizeIn ::= INTEGER

MaximumBurstSizeEg ::= INTEGER

MinimumCellRateIn ::= INTEGER

MinimumCellRateEg ::= INTEGER

MinimumDesiredCellRateIn ::= INTEGER

MinimumDesiredCellRateEg ::= INTEGER

PeakCellRateIn ::= INTEGER

PeakCellRateEg ::= INTEGER

PhysicalInterfaceType ::= ENUMERATED

{

e1         (0),

e3         (1),

stm1       (2),

stm4       (3),

stm16      (4),

stm64      (5),

other      (255)

}

PhysicalLinkType ::= GraphicString               --same as physicalInterfaceType

PhysicalPortId ::= GraphicalString

PhysicalPortIdList ::= SEQUENCE OF GraphicString

ServiceCategoryIn ::= ENUMERATED

{

cbr          (0),

rt-vbr       (1),

nrt-vbr      (2),

abr          (3),

ubr          (4),

gfr          (5)

}

ServiceCategoryEg ::= ENUMERATED

{

cbr          (0),

rt-vbr       (1),

nrt-vbr      (2),

abr          (3),

ubr          (4),

gfr          (5)

}

SustainableCellRateIn ::= INTEGER

SustainableCellRateEg ::= INTEGER

TransportNetworkInterfaceId ::= INTEGER

TransportNetworkType ::= ENUMERATED

{

atm      (0),

ip       (1)

}

UsageChannel ::= GraphicString     --e.g. NBAP, ALCAP, …

UsedAAL ::= ENUMERATED

{

aal0       (0),

aal1       (1),

aal2       (2),

aal34      (3),

aal5       (4)

}

UserLabel ::= GraphicString

VirtualPathId ::= INTEGER

VirtualChannelId ::= INTEGER

END -- of module TS32-714TypeModule

Annex A (informative):
List of assigned Object Identifiers

This annex provides a list with all object identifiers that have been assigned in TS 32.714. These object identifiers shall not be assigned to new objects (also not in new versions of this document).

| Basic Name | Name and OID of the current TS Version |  | Name and OIDs of previous TS Versions |
| --- | --- | --- | --- |
| Managed Object Classes |  |  |  |
| transportNetworkInterface |  | Name: transportNetworkInterface OID: ts32-714ObjectClass 10600 | -- |
| aTMChannelTerminationPoint |  | Name: aTMCHannelTerminationPoint OID: ts32-714ObjectClass 20600 | -- |
| aTMPathTerminationPoint |  | Name: aTMPathTerminationPoint OID: ts32-714ObjectClass 30600 | -- |
| Packages |  |  |  |
| transportNetworkInterfaceMandatoryAttributesPackage |  | Name: transportNetworkInterfaceMandatoryAttributesPackage OID: ts32-714Package 10600 | -- |
| aTMChannelTerminationPointMandatoryAttributesPackage |  | Name: aTMChannelTerminationPointMandatoryAttributesPackage OID: ts32-714Package 20600 | -- |
| aTMChannelTerminationPointOptionalAttributesPackage1 |  | Name: aTMChannelTerminationPointOptionalAttributesPackage1 OID: : ts32-714Package 30600 | -- |
| aTMChannelTerminationPointOptionalAttributesPackage2 |  | Name: aTMChannelTerminationPointOptionalAttributesPackage1 OID: : ts32-714Package 40600 | -- |
| aTMChannelTerminationPointOptionalAttributesPackage3 |  | Name: aTMChannelTerminationPointOptionalAttributesPackage1 OID: : ts32-714Package 50600 | -- |
| aTMChannelTerminationPointOptionalAttributesPackage4 |  | Name: aTMChannelTerminationPointOptionalAttributesPackage1 OID: : ts32-714Package 60600 | --- |
| aTMChannelTerminationPointOptionalAttributesPackage5 |  | Name: aTMChannelTerminationPointOptionalAttributesPackage1 OID: : ts32-714Package 70600 | -- |
| aTMChannelTerminationPointOptionalAttributesPackage6 |  | Name: aTMChannelTerminationPointOptionalAttributesPackage1 OID: : ts32-714Package 80600 | -- |
| aTMPathTerminationPointMandatoryAttributesPackage |  | Name: aTMPathTerminationPointMandatoryAttributesPackage OID: ts32-714Package 90600 | -- |
| Actions |  |  |  |
| -- |  | -- | -- |
| Notifications |  |  |  |
| -- |  | -- | -- |
| Attributes |  |  |  |
| aTMChannelTerminationPointId |  | Name: aTMChannelTerminationPointId OID: ts32-714Action 40600 | -- |
| aTMChannelTerminationPoint-aTMPathTerminationPoint |  | Name: aTMChannelTerminationPoint-aTMPathTerminationPoint OID: ts32-714Action 240600 | -- |
| aTMChannelTerminationPoint- iubLink |  | Name: aTMChannelTerminationPoint-iubLink OID: ts32-714Action 250600 | -- |
| aTMPathTerminationPointId |  | Name: aTMPathlTerminationPoint OID: ts32-714Action 260600 | -- |
| aTMPathTerminationPoint-aTMChannelTerminationPoint |  | Name: aTMPathTerminationPoint-aTMChannelTerminationPoint OID: ts32-714Action 280600 | -- |
| maximumBurstSizeIn |  | Name maximumBurstSizeIn OID: ts32-714Action 180600 | -- |
| maximumBurstSizeEg |  | Name: maximumBurstSizeEg OID: ts32-714Action 190600 | -- |
| minimumCellRateIn |  | Name: minimumCellRateIn OID: ts32-714Action 220600 | -- |
| minimumCellRateEg |  | Name: minimumCellRateEg OID: ts32-714Action 230600 | -- |
| minimumDesiredCellRateIn |  | Name: minimumDesiredCellRateIn OID: ts32-714Action 200600 | -- |
| minimumDesiredCellRateEg |  | Name: minimumDesiredCellRateEg OID: ts32-714Action 210600 | -- |
| peakCellRateIn |  | Name: peakCellRateIn OID: ts32-714Action 140600 | -- |
| peakCellRateEg |  | Name: peakCellRateEg OID: ts32-714Action 150600 | -- |
| physicalInterfaceType |  | Name: physicalInterfaceType OID: ts32-714Action 100600 | -- |
| physicalLinkType |  | Name: physicalLinkType OID: ts32-714Action 90600 | -- |
| physicalPortId |  | Name: physicalPortId OID: ts32-714Action 80600 | -- |
| physicalPortIdList |  | Name: physicalPortIdList OID: ts32-714Action 270600 | -- |
| serviceCategoryIn |  | Name: serviceCategoryIn OID: ts32-714Action 110600 | -- |
| serviceCategoryEg |  | Name: serviceCategoryEg OID: ts32-714Action 120600 | -- |
| sustainableCellRateIn |  | Name: sustainableCellRateIn OID: ts32-714Action 16600 | -- |
| sustainableCellRateEg |  | Name: sustainableCellRateEg OID: ts32-714Action 17600 | -- |
| transportNetworkInterfaceId |  | Name: transportNetworkInterfaceId OID: ts32-714Action 10600 | -- |
| transportNetworkType |  | Name: transportNetworkType OID: ts32-714Action 30600 | -- |
| usageChannel |  | Name: usageChannel OID: ts32-714Action 50600 | -- |
| usedAAL |  | Name: usedAAL OID: ts32-714Action 120600 | -- |
| userLabel |  | Name: userLabel OID: ts32-714Action 20600 | -- |
| virtualChannelId |  | Name: a virtualChannelId OID: ts32-714Action 70600 | -- |
| virtualPathId |  | Name: virtualPathId OID: ts32-714Action 60600 | -- |
| Parameters |  |  |  |
| -- |  | -- | -- |
| Name Bindings |  |  |  |
| transportNetworkInterface-managedElement |  | Name: transportNetworkInterface-managedElement OID: ts32-714NameBinding 10600 | -- |
| aTMPathTerminationPoint-transportNetworkInterface |  | Name: aTMPathTerminationPoint-transportNetworkInterface OID: ts32-714NameBinding 20600 | -- |
| aTMChannelTerminationPoint-transportNetworkInterface |  | Name: aTMChannelTerminationPoint-transportNetworkInterface OID: ts32-714NameBinding 30600 | -- |

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| Dec 2004 | S_26 | SP-040818 | -- | -- | Submitted to SA#26 for Approval | 1.0.0 | 6.0.0 |