3GPP TS 32.742 V6.2.0 (2006-03)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Configuration Management (CM);

Signalling Transport Network (STN) interface

Network Resource Model (NRM)

Integration Reference Point (IRP):

Information Service (IS)

(Release 6)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners" Publications Offices.

Keywords

UMTS, Network, Management

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2006, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	7

4	Modelling approach	7

5	Information Object Classes	8

5.1	Imported information entities and local labels	8

5.2	Class diagram	8

5.2.1	Attributes and relationships	8

5.2.2	Inheritance	11

5.3	Information object class definitions	12

5.3.1	MtpSignPoint	12

5.3.1.1	Definition	12

5.3.1.2	Attributes	12

5.3.2	SignLinkSetTp	13

5.3.2.1	Definition	13

5.3.2.2	Attributes	13

5.3.3	SignLinkTp	14

5.3.3.1	Definition	14

5.3.3.2	Attributes	14

5.3.4	SignRouteSetNePart	15

5.3.4.1	Definition	15

5.3.4.2	Attributes	15

5.3.5	SignRouteNePart	16

5.3.5.1	Definition	16

5.3.5.2	Attributes	16

5.4	Information relationship definitions	16

5.5	Information attribute definitions	17

5.5.1	Definition and legal values	17

5.5.2	Constraints	18

Annex A (informative):	Change history	19

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project: Technical Specification Group Services and System Aspects; Telecommunication management; Configuration Management (CM); Signalling Transport Network (STN) interface Network Resource Model (NRM) Integration Reference Point (IRP), as identified below:

TS 32.741:	"Signalling Transport Network (STN) interface Network Resource Model (NRM) Integration Reference Point (IRP); Requirements".

TS 32.742:	"Signalling Transport Network (STN) interface Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

TS 32.743:	"Signalling Transport Network (STN) interface Network Resource Model (NRM) Integration Reference Point (IRP); Common Object Request Broker Architecture (CORBA) Solution Set (SS)".

TS 32.744:	"Signalling Transport Network (STN) interface Network Resource Model (NRM) Integration Reference Point (IRP); Common Management Information Protocol (CMIP) Solution Set (SS)".

TS 32.745:	"Signalling Transport Network (STN) interface Network Resource Model (NRM) Integration Reference Point (IRP); eXtensible Markup Language (XML) file format definition".

Configuration Management (CM), in general, provides the operator with the ability to assure correct and effective operation of the 3G network as it evolves. CM actions have the objective to control and monitor the actual configuration on the Network Elements (NEs) and Network Resources (NRs), and they may be initiated by the operator or by functions in the Operations Systems (OSs) or NEs.

CM actions may be requested as part of an implementation programme (e.g. additions and deletions), as part of an optimisation programme (e.g. modifications), and to maintain the overall Quality of Service (QoS). The CM actions are initiated either as single actions on single NEs of the 3G network, or as part of a complex procedure involving actions on many resources/objects in one or several NEs.

# 1	Scope

The present document is part of an Integration Reference Point (IRP) named "Signalling Transport Network (STN) network resources IRP", through which an "IRPAgent" (typically an Element Manager or Network Element) can communicate Configuration Management information to one or several "IRPManagers" (typically Network Managers) concerning Signalling Transport resources. This IRP comprises a set of specifications defining Requirements, a protocol neutral Network Resource Model (NRM) and corresponding Solution Set(s).

The present document specifies the protocol neutral STN resources IRP NRM. It reuses relevant parts of the generic NRM in TS 32.622 [6], either by direct reuse or sub-classing, and in addition to that defines Signalling Transport specific Managed Object Classes.

The Configuration Management (CM) area is very large. The intention is to split the specification of the related interfaces in several IRPs - as described in the Introduction clause above. An important aspect of such a split is that the NRMs defined in different IRPs containing NRMs are consistent, and that NRMs supported by an IRPAgent implementation can be accessed as one coherent model through one IRP Information Service (IS).

To summarize, the present document has the following main purpose: to define the applied Signalling Transport Network specific NRM, based on the generic NRM in 3GPP TS 32.622 [6].

In order to access the information defined by this NRM, an IRP IS is needed, such as the Basic CM IRP: IS (TS 32.602 [7]) or the Bulk CM IRP: IS (TS 32.612 [8]). However, which IS is applicable is outside the scope of this document.

Finally, regarding the support of the State Management IRP: IS (TS 32.672 [3]), all NRMs of one release shall support the same State Management IRP version.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication Management, Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	3GPP TS 32.672: "Telecommunication management; Configuration Management (CM); State Management Integration Reference Point (IRP): Information Service (IS)".

[4]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[5]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[6]	3GPP TS 32.622: " Telecommunication management; Configuration Management (CM); Generic network resources Integration Reference Point (IRP): Network Resource Model (NRM)".

[7]	3GPP TS 32.602: "Telecommunication management; Configuration Management (CM); Basic CM Integration Reference Point (IRP); Information Service (IS)".

[8]	3GPP TS 32.612: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP); Information Service (IS)".

[9]	ITU-T Recommendation Q.700 (03/93): "Introduction to CCITT Signalling System No.7".

[10]	ITU-T Recommendation Q.751.1 (10/95): "Network Element Management Information Model for The Message Transfer Part (MTP)".

[11]	ITU-T Recommendation Q.704 (07/96): "Signalling network functions and messages".

[12]	3GPP TS 32.111-2: "Telecommunication management; Fault Management (FM); Part 2: Alarm Integration Reference Point (IRP); Information Service (IS)".

[13]	ITU-T Recommendation Q.702 (11/88): "Signalling Data Link".

[14] 	3GPP TS 29.202: "Signalling System No. 7 (SS7) signalling transport in core network; Stage 3".

[15]	3GPP TS 25.410: "UTRAN Iu Interface: General Aspects and Principles".

[16]	3GPP TS 25.420: "UTRAN Iur interface general aspects and principles".

[17]	3GPP TS 25.430: "UTRAN Iub interface: general aspects and principles".

[18]	3GPP TS 48.018: "Base Station System (BSS)-Serving GPRS Support Node (SGSN);BSS GPRS Protocol (BSSGP)".

[19]	3GPP TS 48.008: "Mobile Switching Centre-Base Station System (MSC-BSS) interface;Layer 3 specification".

[20]	3GPP TS 32.632: "Telecommunication management; Configuration Management (CM); Core Network Resources Integration Reference Point (IRP);Network Resource Model (NRM)".

[21]	3GPP TS 32.642: "Telecommunication management; Configuration Management (CM); UTRAN network resources Integration Reference Point (IRP); Network Resource Model (NRM)".

[22]	3GPP TS 29.060: "GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[23]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[24]	3GPP TS 29.018: "Serving GPRS Support Node (SGSN)-Visitors Location Register (VLR) Gs interface layer 3 specification".

[25]	3GPP TS 32.741: "Telecommunication management; Configuration Management (CM); Signalling Transport Network (STN) interface Network Resource Model (NRM) Integration Reference Point (IRP); Requirements".

[26]	3GPP TS 32.150: "Telecommunication management; Integration Reference Point (IRP) Concept and definitions".

[27]	ITU-T Recommendation E.600 (03/93): "Terms and Definitions of traffic engineering".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions in 3GPP TS 32.101 [1], 32.102 [2], 32.600 [5], 32.741 [25] and the following apply:

Association: in general it is used to model relationships between Managed Objects. Associations can be implemented in several ways, such as:

1.	name bindings;

2.	reference attributes; and

3.	association objects.

This IRP stipulates that containment associations shall be expressed through name bindings, but it does not stipulate the implementation for other types of associations as a general rule. These are specified as separate entities in the object models (UML diagrams).

Managed Element (ME): an instance of the Managed Object Class ManagedElement defined in 3GPP TS 32.622 [6].

Managed Object (MO): in the context of the present document, a Managed Object (MO) is a software object that encapsulates the manageable characteristics and behaviour of a particular Network Resource. The MO is instance of a MO class defined in a MIM/NRM. This class, called Information Object Class (IOC) has attributes that provide information used to characterize the objects that belong to the class (the term "attribute" is taken from TMN and corresponds to a "property" according to CIM). Furthermore, the IOC can have operations that represent the behaviour relevant for that class (the term "operation" is taken from TMN and corresponds to a "method" according to CIM). The IOC may support the emission of notifications that provide information about an event occurrence within a network resource.

Management Information Model (MIM): also referred to as NRM - see the definition below.

Network Resource Model (NRM): A model representing the actual managed telecommunications network resources that a System is providing through the subject IRP. An NRM identifies and describes the IOCs, their associations, attributes and operations. The NRM is also referred to as "MIM" (see above), which originates from the ITU-T TMN.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

CIM	Common Information Model

CM	Configuration Management

DN	Distinguished Name

IOC	Information Object Class

IRP	Integration Reference Point

ITU-T	International Telecommunication Union, Telecommunication Standardisation Sector

ME	Managed Element

MIM	Management Information Model

MO	Managed Object

MTP	Message Transfer Part

NE	Network Element

NR	Network Resource

NRM	Network Resource Model

OS	Operations System

RDN	Relative Distinguished Name

SLC	Signalling Link Code

SLS	Signalling Link Selection

SP	Signalling Point

STN	Signalling Transport Network

STP	Signalling Transfer Point

TP	Termination Point

UML	Unified Modelling Language

# 4	Modelling approach

The modelling approach used in this IRP is described in 3GPP TS 32.150 [26] and 32.622 [6] Generic Network Resources IRP; NRM.

This model allows for combined managed element functionality, where more than one "function IOC" (inherited from ManagedFunction) modelling more specific managed element functionality may be contained in the ManagedElement IOC.

# 5	Information Object Classes

## 5.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 32.622[6], information object class, Top | Top |

## 5.2	Class diagram

### 5.2.1	Attributes and relationships

This clause depicts the set of IOCs that encapsulate information relevant for this service. It provides the overview of all IOCs in UML. Subsequent clauses provide more detailed specification of various aspects of these IOCs.

Figure 5.2.1.1 shows the name-containment relation and other types of relations of the STN NRM.

NOTE 1:	The name-containment relations between IOCs are indicated by UML "unidirectional aggregation by reference" ("hollow diamonds").

NOTE 2:	The listed cardinality numbers represent transient as well as steady-state numbers, and reflect all managed object creation and deletion scenarios.

Figure 5.2.1.1 : Signalling Transport Network NRM Containment/Naming and Association diagram 1

Each IOC is identified with a Distinguished Name (DN) according to 3GPP TS 32.300 [4] that expresses its containment hierarchy.

NOTE:	The relationship between MtpSignPoint and ManagedFunction shall only be supported for a subset of objects inherited from ManagedFuncion. i.e. the xxxFunction instances that support the MtpSignPoint.

Figure 5.2.1.2 : Signalling Transport Network NRM Containment/Naming and Association diagram 2

NOTE 1:	The listed cardinality numbers represent transient as well as steady-state numbers, and reflect all managed object creation and deletion scenarios.

NOTE 2:	Each instance of the vsDataContainer shall only be contained under one IOC.

Figure 5.2.1.3 : vsDataContainer Containment/Naming and Association in STN NRM diagram

### 5.2.2	Inheritance

This clause depicts the inheritance relationships that exist between IOCs.

The following figure shows the inheritance hierarchy for the STN NRM.

Figure 5.2.2.1 : Signalling Transport Network NRM Inheritance Hierarchy

## 5.3	Information object class definitions

### 5.3.1	MtpSignPoint

#### 5.3.1.1	Definition

This IOC represents the Signalling Point functionality. For more information about the Signalling Point, see ITU-T Q.700 [9] and ITU-T Q751.1 [10].

#### 5.3.1.2	Attributes

Table 5.3.1.2.1 : Attributes of MtpSignPoint

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| mtpSignPointId | + | M | M | - |
| pointCode | + | M | M | - |
| networkIndicator | + | M | M | - |
| pointCodeLength | + | M | M | - |
| spType | + | M | M | - |
| userLabel | + | M | M | M |
| relatedObjects | + | M | M | - |

Table 5.3.1.2.2 : Notifications of MtpSignPoint

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAckStateChanged | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyAttributeValueChange | O |  |
| notifyChangedAlarm | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyClearedAlarm | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyNewAlarm | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyObjectCreation | O |  |
| notifyObjectDeletion | O |  |
| notifyComments | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyAlarmListRebuilt | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyPotentialFaultyAlarmList | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |

### 5.3.2	SignLinkSetTp

#### 5.3.2.1	Definition

This IOC represents a bi-directional Signalling Link Set Termination Point functionality. 
For more information about the Signalling Link Set Termination Point, see ITU-T Q.700 [9] and ITU-T Q751.1 [10].

#### 5.3.2.2	Attributes

Table 5.3.2.2.1 : Attributes of SignLinkSetTp

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| signLinkSetTpId | + | M | M | - |
| adjPc | + | M | M | - |
| userLabel | + | M | M | M |
| maxCapacityLS | + | M | M | - |

Table 5.3.2.2.2 : Notifications of SignLinkSetTp

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAckStateChanged | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyAttributeValueChange | O |  |
| notifyChangedAlarm | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyClearedAlarm | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyNewAlarm | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyObjectCreation | O |  |
| notifyObjectDeletion | O |  |
| notifyComments | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyAlarmListRebuilt | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyPotentialFaultyAlarmList | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |

### 5.3.3	SignLinkTp

#### 5.3.3.1	Definition

This IOC represents a bi-directional Signalling Link Termination Point functionality. 
For more information about the Signalling Link Termination Point, see ITU-T Q.700 [9] and ITU-T Q751.1 [10].

#### 5.3.3.2	Attributes

Table 5.3.3.2.1 : Attributes of SignLinkTp

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| signLinkTpId | + | M | M | - |
| slCode | + | M | M | - |
| slsCodeNormalList | + | O | M | - |
| slsCodeCurrentList | + | M | M | - |
| linkTpStatus | + | M | M | - |
| maxCapacitySL | + | M | M | - |
| userLabel | + | M | M | M |
| signLinkType | + | M | M | - |

Table 5.3.3.2.2 : Notifications of SignLinkTp

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAckStateChanged | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyAttributeValueChange | O |  |
| notifyChangedAlarm | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyClearedAlarm | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyNewAlarm | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyObjectCreation | O |  |
| notifyObjectDeletion | O |  |
| notifyComments | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyAlarmListRebuilt | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyPotentialFaultyAlarmList | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |

### 5.3.4	SignRouteSetNePart

#### 5.3.4.1	Definition

This IOC represents a Signalling Route Set functionality. 
For more information about the Signalling Route Set Network Element Part, see ITU-T Q.700 [9] and ITU-T Q751.1 [10].

#### 5.3.4.2	Attributes

Table 5.3.4.2.1 : Attributes of SignRouteSetNePart

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| signRouteSetNePartId | + | M | M | - |
| destinationPc | + | M | M | - |
| userLabel | + | M | M | M |
| loadsharingInformationRouteSetNePart | + | M | M | - |

Table 5.3.4.2.2 : Notifications of SignRouteSetNePart

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAckStateChanged | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyAttributeValueChange | O |  |
| notifyChangedAlarm | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyClearedAlarm | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyNewAlarm | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyObjectCreation | O |  |
| notifyObjectDeletion | O |  |
| notifyComments | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyAlarmListRebuilt | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyPotentialFaultyAlarmList | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |

### 5.3.5	SignRouteNePart

#### 5.3.5.1	Definition

This IOC represents a Signalling Route functionality. 
For more information about the Signalling Route Network Element Part, see ITU-T Q.700 [9] and ITU-T Q751.1 [10].

#### 5.3.5.2	Attributes

Table 5.3.5.2.1 : Attributes of SignRouteNePart

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| signRouteNePartId | + | M | M | - |
| signLinkSetTpPointer | + | M | M | - |
| fixedPriority | + | M | M | - |
| userLabel | + | M | M | M |

Table 5.3.5.2.2 : Notifications of SignRouteNePart

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAckStateChanged | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyAttributeValueChange | O |  |
| notifyChangedAlarm | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyClearedAlarm | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyNewAlarm | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyObjectCreation | O |  |
| notifyObjectDeletion | O |  |
| notifyComments | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyAlarmListRebuilt | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |
| notifyPotentialFaultyAlarmList | See Alarm IRP (3GPP TS 32.111-2 [12]) |  |

## 5.4	Information relationship definitions

None.

## 5.5	Information attribute definitions

### 5.5.1	Definition and legal values

Table 5.5.1 defines the attributes that are present in several IOCs of the present document.

Table 5.5.1 : Attributes

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| adjPc | The signalling point code information of the signalling point adjacent to the signalling link set. (Ref ITU-T Q.704 [11], Ref ITU-T Q.751.1 [10]) | Type: Integral numeric value |
| destinationPc | The signalling point code information of the destination signalling point of the signalling route set. (Ref ITU-T Q.704 [11], Ref ITU-T Q.751.1 [10]) | Type: Integral numeric value |
| fixedPriority | This attribute determines, if the signallingRoute is used as current route. The signallingRoute instances contained in the same signallingRouteSet are chosen in ascending order as current routes (The lower the value, the higher the priority). The priority is defined by means of assigning priorities to all involved route segments. If from a particular SP two or more route segments are used with the same priority, loadsharing between Signalling Routes may occur (Ref ITU-T Q.751.1 [10]). | Type: Integral numeric value Range: (0…255) -- maximum value is implementation dependent |
| linkTpStatus | This is a set-valued attribute. It contains the functional statuses as described in ITU-T Q.704 [11]. (Ref ITU-T Q.704 [11], Ref ITU-T Q.751.1 [10]) | Type: a set of enumerated types, where the legal statuses can be: - localBlocked - remoteBlocked - localInhibited - remoteInhibited - failed - deactivated The absence of any value indicates a status of available. |
| maxCapacityLS | The maximum capacity of a signalling linkset is the maximum load that should be placed on the linkset, when all links that could be active in the linkset are, and are working in service. Unit: Erlang (Ref ITU-T E.600 [27]) | Type: Numeric value |
| maxCapacitySL | This attribute describes the maximum capacity for the signLinkTp. The maximum capacity of a signalling link is the maximum load that should be placed on the signalling link. Unit: Erlang (Ref ITU-T E.600 [27]) | Type: Numeric value |
| networkIndicator | The network indicator information of the signalling point, (Ref ITU-T Q.704 [11], Ref ITU-T Q.751.1 [10]) | Type: Enumerated, Range: International, Spare, National, NationalSpare |
| pointCode | The signalling point code information of the signalling point. (Ref ITU-T Q.704 [11], Ref ITU-T Q.751.1 [10]) | Type: Integral numeric value |
| pointCodeLength | The signalling point code length information of the signalling point. (Ref ITU-T Q.704 [11]) | Type: Enumerated, Range: 14, 24 |
| signLinkSetTpPointer | It references the signallingLinkSetTp which is intended to be used as first segment of the succession of linksets, which form the signalling route on the network level. |  |
| signLinkSetTpId | An attribute whose "name+value" can be used as an RDN when naming an instance of this object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance |  |
| signLinkTpId | An attribute whose "name+value" can be used as an RDN when naming an instance of this object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance |  |
| mtpSignPointId | An attribute whose "name+value" can be used as an RDN when naming an instance of this object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance |  |
| spType | The type of the signalling point. (Ref ITU-T Q.700[9], Ref ITU-T Q.751.1 [10]) | Type: Enumerated Range: SEP, STP, STEP |
| signRouteNePartId | An attribute whose "name+value" can be used as an RDN when naming an instance of this object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance |  |
| signRouteSetNePartId | An attribute whose "name+value" can be used as an RDN when naming an instance of this object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance |  |
| slCode | The Signalling Link Code (SLC) is used to distinguish signalling link in the signalling link set. It is the same value (between 0 and 15) at each end of the link, and is different from that of any other link between the same two adjacent signalling points. (Ref ITU-T Q.704 [11]), Ref ITU-T Q.751.1 [10]) | Type: Integral numeric value Range: (0…15) |
| slsCodeCurrentList | This attribute represents the SLS-Code which is currently used on the signallingLinkTp. It may be different from the slsNormalList, in case some fault has occurred. (Ref ITU-T Q.751.1 [10]) | SET SIZE (0..16) OF Sls -- Each Sls value can occur at most once in a given SET Sls Type: Integral numeric value Range: (0…15) |
| slsCodeNormalList | This attribute indicates which SLS-Codes are initially administratively assigned to this signallingLinkTp for the normal operation. (Ref ITU-T Q.751.1 [10]) | SET SIZE (0..16) OF Sls -- Each Sls value can occur at most once in a given SET Sls Type: Integral numeric value Range: (0…15) |
| signLinkType | This attribute represents the type of signalling link. | Type: Enumerated Range: 64K, 2M |
| loadsharingInformationRouteSetNePart | This attribute contains specific information for target specific loadsharing via the current routes working on a routeset basis via the current routes. (Ref ITU-T Q.751.1 [10]) |  |
| userLabel | A user-friendly name of this object. |  |
| relatedObjects | This attribute indicates list of related xxxFunction object. |  |

### 5.5.2	Constraints

None.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Jun 2004 | SA_24 | SP-040263 | -- | -- | Submitted to TSG SA#24 for Information | -- | 1.0.0 |  |
| Sep 2004 | SA_25 | SP-040601 | -- | -- | Submitted to TSG SA#25 for Approval | -- | 2.0.0 | 6.0.0 |
| Dec 2005 | SA_30 | SP-050720 | 0001 | -- | Correct definition of linkTpStatus in IS - Align with ITU-T Q.751.1 | F | 6.0.0 | 6.1.0 |
| Mar 2006 | SA_31 | SP-060104 | 0002 | -- | Correct the linkTpStatus definition | F | 6.1.0 | 6.2.0 |