3GPP TS 44.004 V6.1.0 (2005-11)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group GSM EDGE Radio Access Network;

Layer 1;

General requirements

(Release 6)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Keywords

GSM, radio

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2005, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

Contents

Foreword	5

1	Scope	6

1a	References	6

2	Interfaces to the physical layer	7

2.1	Interface to the Data Link Layer	8

2.1a	Interface to the Radio Link Control and Medium Access Control layer	8

2.1b	Flexible Layer One Interface to the Radio Link Control and Medium Access Control layer	8

2.2	Interface to radio resource management	9

2.3	Interface to other functional units	9

3	Service of the physical layer	9

3.1	Service Access Point	9

3.2	Service of the physical layer	11

3.2.1	Specific services of the physical layer in the MS	12

4	Primitives of the physical layer	12

4.1	Generic names of primitives between layers 1 and 2 for the transfer of layer 2 frames and RLC/MAC blocks	13

4.2	Generic names of primitives between layer 1 and the RR-management entity of layer 3	13

4.3	Primitive types	13

4.4	Parameter definition	14

5	Physical layer procedures	15

5.0	General	15

5.1	States of the physical layer	15

5.2	Control procedures	17

5.3	Physical layer interface procedures	17

6	Physical layer protocol header	17

6.0	General	17

6.1	Physical layer protocol fields and procedures	17

7	Block transmission	18

7.0	General	18

7.1	SACCH downlink block format	18

7.1.1	A/Gb mode	18

7.1.2	Iu mode	19

7.2	SACCH uplink block format	20

7.2.1	A/Gb mode	20

7.2.2	Iu mode	20

7.3	FACCH/SDCCH/CCCH/BCCH/CBCH downlink block format	20

7.3.1	CCCH/BCCH/CBCH downlink block format	20

7.3.2	FACCH/SDCCH downlink block format	21

7.3.2.1	A/Gb mode	21

7.3.2.2	Iu mode	21

7.4	FACCH/SDCCH uplink block format	22

7.4.1	A/Gb mode	22

7.4.2	Iu mode	22

7.4a	RACH uplink / Uplink access burst block format	22

7.5	PBCCH/PCCCH downlink/PACCH block format	23

7.6	PDTCH block formats	23

7.6.1	PDTCH block type 1 (CS-1) format	24

7.6.2	PDTCH block type 2 (CS-2) format	24

7.6.3	PDTCH block type 3 (CS-3) format	24

7.6.4	PDTCH block type 4 (CS-4) format	25

7.6.5	PDTCH block type 5 (MCS-1) format	25

7.6.6	PDTCH block type 6 (MCS-2) format	26

7.6.7	PDTCH block type 7 (MCS-3) format	26

7.6.8	PDTCH block type 8 (MCS-4) format	27

7.6.9	PDTCH block type 9 (MCS-5) format	27

7.6.9.1	Uplink	27

7.6.9.2	Downlink	28

7.6.10	PDTCH block type 10 (MCS-6) format	28

7.6.10.1	Uplink	28

7.6.10.2	Downlink	28

7.6.11	PDTCH block type 11 (MCS-7) format	29

7.6.11.1	Uplink	29

7.6.11.2	Downlink	29

7.6.12	PDTCH block type 12 (MCS-8) format	30

7.6.12.1	Uplink	30

7.6.12.2	Downlink	30

7.6.13	PDTCH block type 13 (MCS-9) format	31

7.6.13.1	Uplink	31

7.6.13.2	Downlink	31

7.7	PRACH uplink/PACCH uplink short acknowledgement block formats	31

7.8	PTCCH downlink block format	32

7.9	PTCCH uplink block formats	33

7.9a	Transport block formats (Iu mode)	33

7.9a.1	Generic transport block format	33

7.9a.2	Transport block format for signalling TFC	33

7.10	Order of bit transmission	34

8	Vocabulary	34

Annex A (informative):	Change History	36

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document defines the service offered by the physical layer (3GPP TS 45-series of Technical Specifications) of the MS-BS interface (3GPP TS 45- and 44-series of Technical Specifications). Its main objective is to be a guidance for the interface between the 3GPP Technical Specifications in the 45-series and the 44-series. It also specifies the format of signalling channels and the order of bit transmission.

As far as possible, the present document makes use of the layering principles of the Reference Model for Open System Interconnection (OSI) as contained in ITU-T Recommendations X.200 and X.210.

# 1a	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.011: "Service accessibility".

[3]	3GPP TS 43.013: "Discontinuous Reception (DRX) in the GSM System".

[4]	3GPP TS 43.020: "Security-related network functions".

[5]	Void.

[6]	Void.

[7]	3GPP TS 44.003: "Mobile Station - Base Station System (MS - BSS) interface; Channel structures and access capabilities".

[8]	3GPP TS 44.005: "Data Link (DL) layer; General aspects".

[9]	3GPP TS 44.006: "Mobile Station - Base Station System (MS - BSS) interface; Data Link (DL) layer specification".

[10]	Void.

[11]	3GPP TS 44.018: "Mobile radio interface layer 3 specification; Radio Resource Control Protocol".

[12]	Void.

[13]	Void.

[14]	3GPP TS 44.012: "Short Message Service Cell Broadcast (SMSCB) support on the mobile radio interface".

[15]	Void.

[16]	Void.

[17]	Void.

[17a]	3GPP TS 44.060: "General Packet Radio Service (GPRS); Mobile Station (MS) - Base Station System (BSS) interface; Radio Link Control/Medium Access Control (RLC/MAC) protocol".

[18]	Void.

[19]	Void.

[20]	Void.

[21]	Void.

[22]	Void.

[23]	Void.

[24]	Void.

[25]	Void.

[26]	Void.

[26a]	3GPP TS 44.118: "Mobile radio interface layer 3 specification; Radio Resource Control (RRC) Protocol, Iu Mode".

[26b]	3GPP TS 44.160: "Radio Link Control/Medium Access Control (RLC/MAC) protocol, Iu Mode"

[27]	3GPP TS 45.001: "Physical Layer on the Radio Path (General Description)".

[28]	3GPP TS 45.002: "Multiplexing and multiple access on the radio path".

[29]	3GPP TS 45.003: "Channel coding".

[30]	Void.

[31]	3GPP TS 45.005: "Radio transmission and reception".

[32]	3GPP TS 45.008: "Radio subsystem link control".

[33]	3GPP TS 45.010: "Radio subsystem synchronization".

[34]	Void.

[35]	ITU-T Recommendation X.200: "Information technology; Open Systems Interconnection; Basic Reference Model: The basic model".

[36]	ITU-T Recommendation X.210: "Information technology - Open systems interconnection - Basic Reference Model: Conventions for the definition of OSI services".

[37]	3GPP TS 48.058: “Base Station Controler - Base Transceiver Station  (BCS-BTS) Interface Layer 3 Specification”.

[38]	3GPP TR 45.902: “Flexible Layer One”.

# 2	Interfaces to the physical layer

The physical layer (layer 1) is the lowest layer in the OSI Reference Model and it supports all functions required for the transmission of bit streams on the physical medium. These bit streams are transferred on traffic channels, packet data traffic channels and control channels as defined in 3GPP TS 44.003.

NOTE:	For GSM application the physical layer may also be referred to as the radio subsystem. However, the radio subsystem supports functions additional to those described in the present document.

The physical layer interfaces the Data Link Layer, the Radio Link Control and Medium Access Control layer and the supported functional units of the application (figure 2).

Figure 2: Interfaces with the Physical Layer

## 2.1	Interface to the Data Link Layer

The physical layer interfaces the data link layer. On this interface control channels are supported. The data link layer is specified in 3GPP TS 44.005 and  44.006. Communication between the Physical Layer and the Data Link Layer is in an abstract way performed by means of PH-primitives. They do not constrain implementations.

NOTE:	The terms physical layer and layer 1, and data link layer and layer 2, will be used synonymously in the present document.

The PH-primitives exchanged between the physical layer and the data link layer are used for the transfer of layer 2 frames. They are also used to indicate the establishment of channels to layer 2.

## 2.1a	Interface to the Radio Link Control and Medium Access Control layer

The physical layer interfaces the Radio Link Control and Medium Access Control (RLC/MAC) layer. On this interface packet data control channels and packet data traffic channels are supported when MS is operating in A/Gb mode; when MS is operating in Iu mode, traffic channels, dedicated control channels, packet data control channels and packet data traffic channels are supported. The RLC/MAC layer is specified in 3GPP TS 44.060 (A/Gb mode) and 3GPP TS 44.160 (Iu mode). Communication between the Physical Layer and the RLC/MAC layer is in an abstract way performed by means of PH-primitives. They do not constrain implementations.

The PH-primitives exchanged between the physical layer and the RLC/MAC layer are used for the transfer of RLC/MAC blocks. They are also used to indicate the establishment of packet data physical channels (A/Gb mode) or shared basic physical subchannels (Iu mode) to the RLC/MAC layer.

## 2.1b	Flexible Layer One Interface to the Radio Link Control and Medium Access Control layer

In Iu mode, when the Flexible Layer One is used the physical layer interfaces the Radio Link Control and Medium Access Control (RLC/MAC) layer (see 3GPP TR 45.902). On this interface transport channels are supported. The RLC/MAC layer is specified in 3GPP TS 44.160. Communication between the Physical Layer and the RLC/MAC layer is in an abstract way performed by means of PH-primitives. They do not constrain implementations.

The PH-primitives exchanged between the physical layer and the RLC/MAC layer are used for the transfer of transport blocks.

## 2.2	Interface to radio resource management

The physical layer interfaces the radio resource management (RR or RRC) entity of layer 3 in the MS and in the network. When the MS is operating in A/Gb mode the radio resource management is provided by the RR entity whilst when operating in Iu mode the radio resource management is provided by the RRC entity.

Communication is performed in an abstract way by means of MPH-primitives. They do not constrain implementations.

The primitives exchanged with the RR-management entity are related to the assignment of channels, physical layer system information (including measurement results), etc.

## 2.3	Interface to other functional units

The physical layer interfaces other functional units in the MS and in the network for supporting traffic channels. These interfaces are described in the 26, 27 and 46 series of Technical Specifications.

# 3	Service of the physical layer

The physical layer supports transfer of bit streams on the radio medium according to the Technical Specifications of the 45-series. The scope of the 45-series of Technical Specifications is the definition of a framework for operation on the radio medium. The application of this framework on the radio medium results in a transmission service. General characteristics of the service obtained by applying the framework of the 45-series at the operation on the radio medium are described in this clause.

## 3.1	Service Access Point

In the Reference Model for Open System Interconnection, Service Access Points (SAPs) of a layer are defined as gates through which services are offered to an adjacent higher layer (figure 3.1.a). Through a SAP the physical layer offers a service to the data link layer. The SAP is used both for the control of the service providing entity (in case this is the physical layer; commands related to the establishment and release of channels) and the transfer of data (in case of the physical layer; the transfer of bits). The physical layer service access points defined in the present document differ from the OSI physical layer Service Access Points; the layer 3 RR-management instead of the data link layer controls the SAPs (establishment and release of channels).

Figure 3.1.a: Service Access Point principle

On the physical layer of the GSM system a SAP is defined between the physical layer and the data link layer for each control channel (figure 3.1.b and figure 3.1.b1). The characteristics of SAPs (channels) are listed in 3GPP TS 44.003.

Figure 3.1.b: SAPs between the physical layer and the data link layer when the MS is operating in A/Gb mode

Figure 3.1.b1: SAPs between the physical layer and the data link layer when the MS is capable of operating in Iu mode

Moreover, on the physical layer of the GSM system a SAP is defined between the physical layer and the RLC/MAC layer for the packet data control channels, dedicated control channels (Iu mode), traffic channels (Iu mode) and the packet data traffic channel and the transport channels (FLO in Iu mode) (see figure 3.1.c, figure 3.1.d and figure 3.1.e). Multiplexing of these channels is controlled by the RLC/MAC layer, see 3GPP TS 44.060 (A/Gb mode) and 3GPP TS 44.160 (Iu mode). The characteristics of channels are listed in 3GPP TS 44.003.

Figure 3.1.c: SAP between the physical layer and the RLC/MAC layer when the MS is operating in A/Gb mode

Figure 3.1.d: SAP between the physical layer and the RLC/MAC layer when the MS is operating in Iu mode

Figure 3.1.e: SAP between the physical layer and the RLC/MAC layer 
when the MS is operating in Iu mode with FLO

## 3.2	Service of the physical layer

The physical layer offers a transmission service on a limited set of logical channels. Additionally with FLO in Iu mode, the physical layer offers a transmission service on transport channels. The BS and MS access capabilities and the characteristics of logical channels (SAPs) are defined in 3GPP TS 44.003.

NOTE:	Between 3GPP TS 44.003 and the 3GPP TS 45.0xx series there is a slight difference in terminology. The "channels" mentioned in 3GPP TS 44.003 are "logical channels" according to the 3GPP TS 45.0xx series (especially 3GPP TS 45.002). The "CCCH", a channel name commonly used in the 3GPP TS 44.0xx series, covers the logical channels of the type RACH, PCH and AGCH. Similarly, the “PCCCH” covers the logical channels of the type PPCH, PAGCH and PRACH.

For an MS operating in A/Gb mode, logical channels are multiplexed on physical channels. Physical channels are the units scheduled on the radio medium. Some are reserved by the network for common use (e.g. a combination of CCCH and BCCH), others are assigned to dedicated connections with MSs (dedicated physical channels), or are assigned to a shared usage between MSs for packet switched data traffic (packet data physical channels). In time, the combination of logical channels used on an assigned physical channel may change. Allowed combinations of logical channels on a physical channel are defined in 3GPP TS 44.003. Data on SAPs of control channels is exchanged in discrete blocks with a size of 23 or 21 (SACCH) octets. Data on a SAP of packet data traffic channels is exchanged in discrete blocks with a size dependent on the block type (see clause 7).

For an MS operating in Iu mode, logical channels are multiplexed on basic physical subchannels. Basic physical subchannels are the units scheduled on the radio medium. Some basic physical channels are reserved by the network for common use (e.g. BCCH); dedicated basic physical subchannels are assigned to dedicated connections with MSs, shared basic physical subchannels are assigned to a shared usage between MSs for packet switched data traffic. In time, the combination of logical channels used on an assigned basic physical subchannel may change. Allowed combinations of logical channels on a basic physical subchannel are defined in 3GPP TS 44.003. Data on SAPs of control channels is exchanged in discrete blocks with a size of 23 or 21 (SACCH) octets. Data on a SAP of packet data traffic channels is exchanged in discrete blocks with a size dependent on the block type (see clause 7).

For an MS operating in Iu mode with FLO, transport channels are multiplexed on dedicated basic physical subchannels. The combination of transport channels used on an assigned basic physical subchannel may change in time. Data on SAPs of transport channels is exchanged in discrete transport blocks.

Synchronization between layer 1 and layer 2 (data link layer) is provided for piggy-backing of RR (receive ready) frames, and the starting of timers (T200). See also 3GPP TS 44.006. Synchronization between the physical layer and the RLC/MAC layer is provided for the handling of timers, and the multiplexing of logical channels. See also 3GPP TS 44.060 (A/Gb mode) and 3GPP TS 44.160 (Iu mode).

-	Error detection:

The physical layer offers an error protected transmission service, it includes error detection functions and to a lower level, error correction functions. Erroneous received frames are not offered to the data link layer or the RLC/MAC layer. The probability of one or more errors in a physical block transferred by the physical layer is defined in 3GPP TS 45.005. Due to not specified methods of quality detection, the probability of residual errors in transferred blocks may vary between implementations.

-	Encryption:

Security related functions implemented at the physical layer are described in 3GPP TS 43.020.

An overview of the functions specified in the 45-series which create the service of the physical layer can be found in 3GPP TS 45.001.

### 3.2.1	Specific services of the physical layer in the MS

The access capability service of the physical layer in the MS differs depending on the nature of the channel (traffic, packet data traffic or broadcast/common channels).

-	Establishment of dedicated physical channels (A/Gb Mode):

Establishment of dedicated physical channels on the physical layer is controlled by the radio resources management of layer 3 (3GPP TS 44.018). During operation on a dedicated physical channel, the physical layer measures the signals of neighbouring base stations and the signal quality of the used dedicated physical channel. Measurements are transferred to layer 3, measurement control information is offered by layer 3.

-	Establishment of dedicated basic physical subchannels (Iu mode):

Establishment of dedicated basic physical subchannels on the physical layer is controlled by the radio resources management of layer 3 (3GPP TS 44.018 and 3GPP TS 44.118). During operation on a dedicated basic physical subchannel, the physical layer measures the signals of neighbouring base stations and the signal quality of the used dedicated basic physical subchannel. Measurements are transferred to layer 3, measurement control information is offered by layer 3.

-	Establishment of packet data physical channels (A/Gb mode):

Establishment of packet data physical channels on the physical layer is controlled by the radio resource management of layer 3. Packet access and the reservation of radio resource on packet data physical channels is controlled by the RLC/MAC layer in co-operation with layer 3 (3GPP TS 44.060 and 3GPP TS 44.118). During operation on packet data physical channels, the physical layer measures the signals of neighbouring base stations and the signal quality of the used packet data physical channel. Measurements are transferred to layer 3, measurement control information is offered by layer 3.

-	Establishment of shared basic physical subchannels (Iu mode):

Establishment of shared basic physical subchannels on the physical layer is controlled by the radio resource management of layer 3. Packet access and the reservation of radio resource on shared basic physical subchannels is controlled by the RLC/MAC layer in co-operation with layer 3 (3GPP TS 44.160 and 3GPP TS 44.118). During operation on shared basic physical subchannels, the physical layer measures the signals of neighbouring base stations and the signal quality of the used shared basic physical subchannel. Measurements are transferred to layer 3, measurement control information is offered by layer 3.

-	cell/PLMN selection in idle mode or in packet mode:

In idle mode or in packet mode, the physical layer selects the best cell with its BCCH/CCCH in close co-operation with layer 3, meeting requirements for PLMN selection specified in 3GPP TS 22.011. The idle mode procedures are not modelled within the present document. Examples of procedures for cell selection are described in 3GPP TS 45.008. The physical layer performs automatic crossover.

# 4	Primitives of the physical layer

The Physical layer interacts with other entities as illustrated in figure 2.1. The interactions with the data link layer of Dm channels and the interactions with the RLC/MAC layer of packet data physical channels (A/Gb mode), shared or dedicated basic physical subchannels (Iu mode) are shown in terms of primitives where the primitives represent the logical exchange of information and control between the physical layer and adjacent layers. They do not specify or constrain implementations. The interactions between the physical layer and layer 1 entities for Bm/Lm channels are for further study. For the physical layer two sets of primitives are defined:

-	Primitives between physical layer and data link layer and RLC/MAC layer respectively:

PH - Generic name - Type: Parameters.

-	Primitives between layer 1 and the RR-management layer 3 entity:

MPH - Generic name - Type: Parameters.

## 4.1	Generic names of primitives between layers 1 and 2 for the transfer of layer 2 frames and RLC/MAC blocks

The following primitive generic names are defined on the SAPs between the physical layer and the data link layer:

a)	PH-DATA:

The PH-DATA primitives are used on a SAP to pass message units containing frames used for data link layer and RLC/MAC layer respective peer-to-peer communications to and from the physical layer.

b)	PH-RANDOM ACCESS:

The PH-RANDOM ACCESS (PH-RA) primitives are used on the SAP of the RACH and the PRACH to request and confirm (in the MS) the sending of a random access frame and to indicate (in the network) the arrival of a random access frame. For MS operating in A/Gb mode, the random access protocols are specified in 3GPP TS 44.018 and 3GPP TS 44.060 respectively; for MS operating in Iu mode, they are specified in 3GPP TS 44.160.

c)	PH-CONNECT:

The PH-CONNECT primitive is used on a SAP to indicate that the physical connection on the corresponding control channel or packet data physical channel (A/Gb mode) or shared basic physical subchannel (Iu mode) has been established.

d)	PH-READY-TO-SEND:

The PH-READY-TO-SEND primitive is used by the physical layer to trigger, if applicable, piggy backing, the start of timer for the data link layer or the RLC/MAC layer and the forwarding a data unit to the physical layer. It is passed to the upper layer just before a new physical block is transmitted.

e)	PH-EMPTY-FRAME:

The PH-EMPTY-FRAME primitive can be used by the data link layer and the RLC/MAC layer to indicate that no frame has to be transmitted after receiving the PH-READY-TO-SEND primitive. It enables polling of several upper layer entities by the physical layer and support DTX.

## 4.2	Generic names of primitives between layer 1 and the RR-management entity of layer 3

The following primitive generic name is defined between layer 1 and the RR-management entity of layer 3:

-	MPH-INFORMATION:

MPH-INFORMATION (MPH-INFO) primitives are used for the control of the physical layer by the RR-management of layer 3. This information activates and deactivates, configures and deconfigures, through connects and disconnects physical, logical and transport channels. It is also used for the transfer of measurements and measurement control information from layer 1 to layer 3.

## 4.3	Primitive types

The primitive types defined in the present document are:

a)	REQUEST:

The REQUEST primitive type is used when a higher layer is requesting a service from a lower layer.

b)	INDICATION:

The INDICATION primitive type is used by a layer providing a service to notify the next higher layer of activities in the layer. This activities are directly related to the occurrence of a REQUEST primitive on the peer-protocol side.

c)	RESPONSE:

The RESPONSE primitive type is used by a layer to acknowledge receipt from the INDICATION primitive type.

d)	CONFIRM:

The CONFIRM primitive type is used by the layer providing the requested service to confirm that the activity has been completed.

## 4.4	Parameter definition

Primitives contain a variable amount of parameters. The primitives with included parameters are listed in table 4.1.

Table 4.1: Primitives of the physical layer

|  |  |  |  | message unit |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | channel control parameters |  |  |  |  |
|  |  |  |  |  |  | system information |  |  |  |
|  |  |  |  |  |  |  | absolute frame No. |  |  |
| primitive |  | entity | direction |  |  |  |  | measurement |  |
| MPH-INFO-REQ | MS/BS | RR(L3) | PH(L1) |  | x | x |  |  |  |
| MPH-INFO-CON | MS/BS | PH(L1) | RR(L3) |  | x | x |  |  |  |
| MPH-INFO-IND | MS/BS | PH(L1) | RR(L3) |  | x | x |  | x |  |
| PH-CONNECT-IND | MS/BS | PH(L1) | DL(L2) RLC/MAC |  | x |  |  |  |  |
| PH-READY-TO-SEND | MS/BS | PH(L1) | DL(L2) RLC/MAC |  | x |  | x |  |  |
| PH-EMPTY-FRAME | MS/BS | DL(L2) RLC/MAC | PH(L1) |  | x |  |  |  |  |
| PH-DATA-REQ | MS/BS | DL(L2) RLC/MAC | PH(L1) | x | x |  |  |  |  |
| PH-DATA-IND | MS/BS | PH(L1) | DL(L2) RLC/MAC | x | x |  |  | x |  |
| PH-RA-REQ | MS | DL(L2) RLC/MAC | PH(L1) | x |  |  |  |  |  |
| PH-RA-IND | BS | PH(L1) | DL(L2) RLC/MAC | x |  |  | x |  |  |
| PH-RA-CON | MS | PH(L1) | DL(L2) RLC/MAC | x |  |  | x |  |  |

Parameters involved in the primitive exchange with the physical layer are:

a)	Message unit:

The message unit contains peer-to-peer information of a layer. It is transferred by the physical layer to the peer layer.

b)	Channel control parameters:

These parameters contain information for channel control, specified in 3GPP TS 44.018 and 3GPP TS  44.060 for MS operating in A/Gb mode, 3GPP TS 44.118 and 3GPP TS 44.160 for MS operating in Iu mode.

c)	System information:

This information is exchanged in the cell/PLMN selection procedures. It may also contain control information for DRX (sleep mode, see 3GPP TS 43.013).

d)	Absolute Frame Number:

The absolute frame number is used (in combination with a random access identifier) to uniquely identify a random access.

e)	Measurements:

This parameter is used to report the quality of a dedicated physical channel (MS and network) and to report the quality of surrounding BCCH carriers (MS only).

# 5	Physical layer procedures

## 5.0	General

The main body of physical layer procedures is specified in 3GPP TS 44.018 and 3GPP TS  44.060 for MS operating in A/Gb mode, 3GPP TS 44.118 and 3GPP TS 44.160 for MS operating in Iu mode, and 3GPP TS 45.008.

## 5.1	States of the physical layer

In the physical layer of the MS the following states are defined:

NULL:		the equipment is switched off;

SEARCHING BCH:	the physical layer tracks the best BCCH;

BCH:		the physical layer listens to a BCCH/CCCH or PBCCH/PCCCH and is able to do random access;

TUNING DCH:		For MS operating in A/Gb mode the physical layer seizes on a respective dedicated physical channel or packet data physical channel; for MS operating in Iu mode the physical layer seizes on a basic physical subchannel;

DCH:		For MS operating in A/Gb mode the physical layer has seized a respective dedicated physical channel or packet data physical channel and may establish and through connect logical channels; For MS operating in Iu mode the physical layer has seized a basic physical subchannel and may establish and through connect logical channels.

NOTE:	BCH = Bcch/ccch physical CHannel or pbcch/pccch physical channel;	
DCH = Dedicated physical CHannel or packet data physical channel for MS operating in A/Gb mode; Dedicated basic physical subCHannel or shared basic physical subchannel for MS operating in Iu mode.

Figure 5.1 gives a general state diagram of the physical layer. All state transitions of the physical layer are controlled by MPH-INFORMATION primitives.

Figure 5.1: States of the physical layer in the MS

The states of the physical layer in the network are not specified. The states in the network will reflect the other characteristics of operation on channels.

## 5.2	Control procedures

Requirements and examples of procedures for idle mode and for packet mode operation of the MS are specified in 3GPP TS 22.011 and 45.008. In the idle mode and packet mode procedures the physical layer tracks the best cell and may tune on their BCCH/PBCCH in order to enable layer 3 to read the system information of the BCCH/PBCCH. This system information of the BCCH/PBCCH is used in the selection process.

## 5.3	Physical layer interface procedures

Three types of primitives are defined for the communication between the physical layer and the data link layer both in the MS and the network. When a control channel, packet data physical channel (A/Gb mode), or a shared basic physical subchannel (Iu mode) is being established, a PH-CONNECT-INDICATION is offered to the data link layer or the RLC/MAC layer, the one which is applicable, on the corresponding SAP. On an established full duplex control channel (DCCHs) in both MS and network, on an established packet data physical channel (A/Gb mode), or a shared basic physical subchannel (Iu mode) or on the established BCCH/CCCH in the MS, physical blocks received correctly are offered on the corresponding SAP in PH-DATA-INDICATION primitives. On a full duplex control  channel (DCCHs) or on the BCCH/CCCH in the network, the data link layer will offer physical blocks to be transmitted in PH-DATA-REQUEST primitives. On a packet data physical channel (A/Gb mode) or shared basic physical subchannel (Iu mode), the RLC/MAC layer will offer physical blocks to be transmitted in PH-DATA-REQUEST primitives. In the MS in idle mode or in packetidle mode, random accesses on RACH or on PRACH can be offered in PH-RANDOM ACCESS-REQUEST primitives. The physical layer of the MS will perform a random access as soon as possible. The physical layer of the MS will confirm the data link layer or the RLC/MAC layer, the one which is applicable, the transmission of the random access attempt in a PH-RANDOM ACCESS-CONFIRM. This confirmation contains the absolute frame number in which the random access is transmitted. The physical layer of the BS offers correctly received random accesses to the data link layer or the RLC/MAC layer, the one which is applicable, in a PH-RANDOM ACCESS-INDICATION. This indication contains the absolute frame number in which the random access is received.

# 6	Physical layer protocol header

## 6.0	General

The physical layer implements a peer-to-peer protocol for the control of timing advance, power control and Repeated SACCH at the operation on dedicated physical channels (A/Gb mode) or dedicated basic physical subchannels (Iu mode), and further, for the control of timing advance at the operation on packet data physical channels (A/Gb mode) or shared basic physical subchannels (Iu mode). For this purpose a two octet physical header is defined on all blocks transferred via the SACCH, a logical channel always present on a dedicated physical channel (A/Gb mode) or dedicated basic physical subchannel (Iu mode). Further more, a 16 octet information field is defined on downlink blocks transferred via the PTCCH, a logical channel present on a packet data physical channel (A/Gb mode) or shared basic physical subchannel (Iu mode).

## 6.1	Physical layer protocol fields and procedures

Procedures for handling the ordered and actual power level fields are specified in 3GPP TS 45.005 and 45.008. The ordered MS power level field and the actual MS power level field are coded as the binary representation of the "power control level", see 3GPP TS 45.005.

Procedures for handling the ordered and actual timing advance fields are specified in 3GPP TS 45.010. The numbers corresponding to the timing advance steps in 3GPP TS 45.010 are included binary coded in the 7 bit or in case of GSM 400 8 bit ordered and actual timing advance fields of the physical layer header.

For all the bands except GSM 400, the values 0 to 63 are valid TA values.  The bit pattern "1111111" indicates that the field does not contain a timing advance value. All other bit combinations (64 to 126 decimal) are reserved. For all bands except GSM 400 bit 8 is set to spare.

For GSM 400, the values 0 to 219 decimal are valid TA values. The bit pattern "11111111" indicates that the field does not contain a timing advance value. The remaining values 220 to 254 are reserved.

A MS in packet transfer mode (A/Gb mode) or MAC-shared state (Iu mode) which is assigned a TAI shall, when receiving a PTCCH downlink block on the packet data physical channel (A/Gb mode) or shared basic physical subchannel (Iu mode) containing PACCH, read the corresponding ordered timing advance field in that PTCCH block. The ordered timing advance fields corresponding to other TAIs than the assigned one shall be ignored.

Procedures for handling the FPC_EPC (Fast Power Control/Enhanced Power Control) field (bit 6 of octet 1 in the SACCH downlink block) are specified in 3GPP TS 45.008 and 3GPP TS 48.058. The FPC_EPC field has different interpretation depending on the channel mode of the channel to which the SACCH is associated, and whether that channel is in EPC mode..

If the channel mode is such that FPC may be used, the FPC_EPC field indicates whether Fast Measurement Reporting and Power Control mechanism is used. It is coded as follows:

Value:

0	Fast Power Control not in use,

1	Fast Power Control in use.

If the channel is in EPC mode, the FPC_EPC field indicates whether EPC procedures are used for MS (uplink) power control. It is coded as follows:

Value:

0	EPC not in use for MS power control,

1	EPC in use for MS power control.

If the channel mode is such that FPC may not be used and the channel is not in EPC mode, the MS shall ignore the value of the FPC_EPC field.

Procedures for handling the SRO (SACCH Repetition Order) and the SRR (SACCH Repetition Request) fields are specified in 3GPP TS 44.006. They are coded as follows:

Value:

0	Repeated SACCH not required

1	Repeated SACCH required

# 7	Block transmission

## 7.0	General

The format of the blocks transmitted on the physical interface depends on the mode (A/Gb or Iu mode) in which the MS is connected to the network, as specified in this section.

## 7.1	SACCH downlink block format

### 7.1.1	A/Gb mode

The 23 octets of SACCH blocks are used in the downlink in the following way:

| bit numbering <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| Spare | SRO | FPC EPC | Ordered MS power level |  |  |  |  | octet 1 |  |
|  | Ordered timing advance |  |  |  |  |  |  | octet 2 | octet |
|  |  |  |  |  |  |  |  | octet 3 | numbering |
|  |  |  | 21 |  |  |  |  |  |  |
|  |  |  | octets |  |  |  |  | V |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | layer 2 |  |  |  |  |  |  |
|  |  |  | frame |  |  |  |  | octet 23 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.006 applies.

Figure 7.1.1: SACCH downlink block format for MS operating in A/Gb mode

### 7.1.2	Iu mode

The 23 octets of SACCH blocks are used in the downlink in the following way:

| bit numbering <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| Spare | SRO | FPC EPC | Ordered MS power level |  |  |  |  | octet 1 |  |
|  | Ordered timing advance |  |  |  |  |  |  | octet 2 | octet |
|  |  |  |  |  |  |  |  | octet 3 | numbering |
|  |  |  | 21 |  |  |  |  |  |  |
|  |  |  | Octets |  |  |  |  | V |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | RLC/MAC |  |  |  |  |  |  |
|  |  |  | Block |  |  |  |  | octet 23 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.1.2: SACCH downlink block format for MS operating in Iu mode

## 7.2	SACCH uplink block format

### 7.2.1	A/Gb mode

The 23 octets of SACCH blocks are used in the uplink in the following way:

| bit numbering <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| Spare | SRR | FPC | Actual MS power level |  |  |  |  | octet 1 |  |
|  | Actual timing advance |  |  |  |  |  |  | octet 2 | octet |
|  |  |  |  |  |  |  |  | octet 3 | numbering |
|  |  |  | 21 |  |  |  |  |  |  |
|  |  |  | octets |  |  |  |  | V |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | layer 2 |  |  |  |  |  |  |
|  |  |  | frame |  |  |  |  | octet 23 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.006 applies.

Figure 7.2.1: SACCH uplink block format for MS operating in A/Gb mode

### 7.2.2	Iu mode

The 23 octets of SACCH blocks are used in the uplink in the following way:

| bit numbering <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| Spare | SRR | FPC | Actual MS power level |  |  |  |  | octet 1 |  |
|  | Actual timing advance |  |  |  |  |  |  | octet 2 | octet |
|  |  |  |  |  |  |  |  | octet 3 | numbering |
|  |  |  | 21 |  |  |  |  |  |  |
|  |  |  | octets |  |  |  |  | V |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | RLC/MAC |  |  |  |  |  |  |
|  |  |  | Block |  |  |  |  | octet 23 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.2.2: SACCH uplink block format for MS operating in Iu mode

## 7.3	FACCH/SDCCH/CCCH/BCCH/CBCH downlink block format

### 7.3.1	CCCH/BCCH/CBCH downlink block format

The 23 octets blocks are used in the downlink in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | 23 |  |  |  |  |  |  |
|  |  |  | octets |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | layer 2 |  |  |  |  |  |  |
|  |  |  | frame |  |  |  |  | octet 23 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.006 and 44.012 applies.

Figure 7.3.1: BCCH/CCCH/CBCH downlink block format

### 7.3.2	FACCH/SDCCH downlink block format

#### 7.3.2.1	A/Gb mode

The 23 octets blocks are used in the downlink in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | 23 |  |  |  |  |  |  |
|  |  |  | Octets |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | layer 2 |  |  |  |  |  |  |
|  |  |  | Frame |  |  |  |  | octet 23 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.006 and 3GPP TS 44.012 applies.

Figure 7.3.2.1: FACCH/SDCCH downlink block format for MS operating in A/Gb mode

#### 7.3.2.2	Iu mode

The 23 octets blocks are used in the downlink in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | 23 |  |  |  |  |  |  |
|  |  |  | Octets |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | RLC/MAC |  |  |  |  |  |  |
|  |  |  | block |  |  |  |  | octet 23 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.3.2.2: FACCH/SDCCH downlink block format for MS operating in Iu mode

## 7.4	FACCH/SDCCH uplink block format

### 7.4.1	A/Gb mode

The 23 octets blocks are used in the uplink in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | 23 |  |  |  |  |  |  |
|  |  |  | octets |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | layer 2 |  |  |  |  |  |  |
|  |  |  | frame |  |  |  |  | octet 23 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.006 applies.

Figure 7.4.1: FACCH/SDCCH uplink block format for MS operating in A/Gb mode

### 7.4.2	Iu mode

The 23 octets blocks are used in the uplink in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | 23 |  |  |  |  |  |  |
|  |  |  | octets |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | RLC/MAC |  |  |  |  |  |  |
|  |  |  | block |  |  |  |  | octet 23 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.4.2: FACCH/SDCCH uplink block format for MS operating in Iu mode

## 7.4a	RACH uplink / Uplink access burst block format

Two alternative RACH uplink / Uplink access burst block formats are specified. The 8 bit (1 octet) blocks are used in the uplink in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 8 bit RACH uplink / Uplink access burst block |  |  |  |  |  |  |  | octet 1 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.018 applies.

Figure 7.4a.a: 8 bit RACH uplink / Uplink access burst block format

The 11 bit blocks are used in the uplink in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 11 bit RACH uplink / Uplink access burst block |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  |  |  |  |  |  | octet 2 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.4a.b: 11 bit RACH uplink / Uplink access burst block format

## 7.5	PBCCH/PCCCH downlink/PACCH block format

The 84 bit (23 octets) blocks are used in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | 184 |  |  |  |  |  |  |
|  |  |  | bit |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | RLC/MAC |  |  |  |  |  |  |
|  |  |  | block |  |  |  |  | octet 23 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.5: PBCCH/PCCCH downlink/PACCH blockformat

## 7.6	PDTCH block formats

Unless explicitly stated otherwise, the block formats shown hereafter are valid for both uplink and downlink directions.

### 7.6.1	PDTCH block type 1 (CS-1) format

The 184 bit (23 octets) blocks are used in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | 184 |  |  |  |  |  |  |
|  |  |  | bit |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | RLC/MAC |  |  |  |  |  |  |
|  |  |  | block |  |  |  |  | octet 23 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.6.1: PDTCH block type 1 (CS-1) format

### 7.6.2	PDTCH block type 2 (CS-2) format

The 271 bit blocks are used in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | 271 |  |  |  |  |  |  |
|  |  |  | bit |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | RLC/MAC |  |  |  |  |  |  |
|  |  |  | block |  |  |  |  | octet 34 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.6.2: PDTCH block type 2 (CS-2) format

### 7.6.3	PDTCH block type 3 (CS-3) format

The 315 bit blocks are used in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | 315 |  |  |  |  |  |  |
|  |  |  | bit |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | RLC/MAC |  |  |  |  |  |  |
|  |  |  | block |  |  |  |  | octet 39 |  |
|  |  |  |  |  |  |  |  | octet 40 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.6.3: PDTCH block type 3 (CS-3) format

### 7.6.4	PDTCH block type 4 (CS-4) format

The 431 bit blocks are used in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | 431 |  |  |  |  |  |  |
|  |  |  | bit |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | RLC/MAC |  |  |  |  |  |  |
|  |  |  | block |  |  |  |  | octet 54 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.6.4: PDTCH block type 4 (CS-4) format

### 7.6.5	PDTCH block type 5 (MCS-1) format

The 209 bit blocks are used in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | 209 |  |  |  |  |  |  |
|  |  |  | Bit |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | RLC/MAC |  |  |  |  |  |  |
|  |  |  | Block |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet 27 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.6.5: PDTCH block type 5 (MCS-1) format

### 7.6.6	PDTCH block type 6 (MCS-2) format

The 257 bit blocks are used in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | 257 |  |  |  |  |  |  |
|  |  |  | Bit |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | RLC/MAC |  |  |  |  |  |  |
|  |  |  | Block |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet 33 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.6.6: PDTCH block type 6 (MCS-2) format

### 7.6.7	PDTCH block type 7 (MCS-3) format

The 329 bit blocks are used in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | 329 |  |  |  |  |  |  |
|  |  |  | Bit |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | RLC/MAC |  |  |  |  |  |  |
|  |  |  | Block |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet 42 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.6.7: PDTCH block type 7 (MCS-3) format

### 7.6.8	PDTCH block type 8 (MCS-4) format

The 385 bit blocks are used in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | 385 |  |  |  |  |  |  |
|  |  |  | Bit |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | RLC/MAC |  |  |  |  |  |  |
|  |  |  | Block |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet 49 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.6.8: PDTCH block type 8 (MCS-4) format

### 7.6.9	PDTCH block type 9 (MCS-5) format

#### 7.6.9.1	Uplink

The 487 bit blocks are used in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | 487 |  |  |  |  |  |  |
|  |  |  | Bit |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | RLC/MAC |  |  |  |  |  |  |
|  |  |  | Block |  |  |  |  | octet 61 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.6.9.1: PDTCH block type 9 (MCS-5) uplink format

#### 7.6.9.2	Downlink

The 478 bit blocks are used in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | 478 |  |  |  |  |  |  |
|  |  |  | Bit |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | RLC/MAC |  |  |  |  |  |  |
|  |  |  | Block |  |  |  |  | octet 60 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.6.9.2: PDTCH block type 9 (MCS-5) downlink format

### 7.6.10	PDTCH block type 10 (MCS-6) format

#### 7.6.10.1	Uplink

The 631 bit blocks are used in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | 631 |  |  |  |  |  |  |
|  |  |  | Bit |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | RLC/MAC |  |  |  |  |  |  |
|  |  |  | Block |  |  |  |  | octet 79 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.6.10.1: PDTCH block type 10 (MCS-6) uplink format

#### 7.6.10.2	Downlink

The 622 bit blocks are used in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | 622 |  |  |  |  |  |  |
|  |  |  | Bit |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | RLC/MAC |  |  |  |  |  |  |
|  |  |  | Block |  |  |  |  | octet 78 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.6.10.2: PDTCH block type 10 (MCS-6) downlink format

### 7.6.11	PDTCH block type 11 (MCS-7) format

#### 7.6.11.1	Uplink

The 946 bit blocks are used in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | 946 |  |  |  |  |  |  |
|  |  |  | Bit |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | RLC/MAC |  |  |  |  |  |  |
|  |  |  | Block |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet 119 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.6.11.1: PDTCH block type 11(MCS-7) uplink format

#### 7.6.11.2	Downlink

The 940 bit blocks are used in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | 940 |  |  |  |  |  |  |
|  |  |  | Bit |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | RLC/MAC |  |  |  |  |  |  |
|  |  |  | Block |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet 118 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.6.11.2: PDTCH block type 11 (MCS-7) downlink format

### 7.6.12	PDTCH block type 12 (MCS-8) format

#### 7.6.12.1	Uplink

The 1138 bit blocks are used in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | 1138 |  |  |  |  |  |  |
|  |  |  | Bit |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | RLC/MAC |  |  |  |  |  |  |
|  |  |  | Block |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet 143 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.6.12.1: PDTCH block type 12 (MCS-8) uplink format

#### 7.6.12.2	Downlink

The 1132 bit blocks are used in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | 1132 |  |  |  |  |  |  |
|  |  |  | Bit |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | RLC/MAC |  |  |  |  |  |  |
|  |  |  | Block |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet 142 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.6.12.2: PDTCH block type 12 (MCS-8) downlink format

### 7.6.13	PDTCH block type 13 (MCS-9) format

#### 7.6.13.1	Uplink

The 1234 bit blocks are used in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | 1234 |  |  |  |  |  |  |
|  |  |  | Bit |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | RLC/MAC |  |  |  |  |  |  |
|  |  |  | Block |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet 155 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.6.13.1: PDTCH block type 13 (MCS-9) uplink format

#### 7.6.13.2	Downlink

The 1228 bit blocks are used in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | 1228 |  |  |  |  |  |  |
|  |  |  | Bit |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | RLC/MAC |  |  |  |  |  |  |
|  |  |  | Block |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet 154 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.6.13.2: PDTCH block type 13 (MCS-9) downlink format

## 7.7	PRACH uplink/PACCH uplink short acknowledgement block formats

Two alternative PRACH uplink/PACCH uplink short acknowledgement block formats are specified. The 8 bit (1 octet) blocks are used in the uplink in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 8 bit RLC/MAC block |  |  |  |  |  |  |  | octet 1 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.7.a: PRACH uplink/PACCH uplink short acknowledgement block format

The 11 bit blocks are used in the uplink in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 11 bit RLC/MAC block |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  |  |  |  |  |  | octet 2 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies.

Figure 7.7.b: PRACH uplink/PACCH uplink short acknowledgement block format

## 7.8	PTCCH downlink block format

The 184 bit (23 octets) blocks are used in the downlink in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  | Ordered timing advance for MS assigned TAI = 0 |  |  |  |  |  |  | octet 1 |  |
|  | Ordered timing advance for MS assigned TAI = 1 |  |  |  |  |  |  | octet 2 |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  | Ordered timing advance for MS assigned TAI = 15 |  |  |  |  |  |  | octet 16 |  |
|  |  |  | 7 |  |  |  |  | octet 17 |  |
|  |  |  | octets |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | fill octets: |  |  |  |  |  |  |
|  |  | "00101011" |  |  |  |  |  | octet 23 |  |

Figure 7.8: PTCCH downlink block format

The field mapping convention in this clause is that when a field is contained within a single octet, the highest bit number of the field represents the highest order value and lowest bit number of the field represents the lowest order value.

Spare bits are encoded with the binary value "0". Fill octets are encoded with the binary value "00101011".

## 7.9	PTCCH uplink block formats

Two alternative PTCCH uplink block formats are specified. The 8 bit (1 octet) blocks are used in the uplink in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 8 fixed bits: "01111111" |  |  |  |  |  |  |  | octet 1 |  |

Figure 7.9.a: PTCCH uplink block format

The 11 bit blocks are used in the uplink in the following way:

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 11 fixed bits: "11111111111" |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  |  |  |  |  |  | octet 2 |  |

Figure 7.9.b: PTCCH uplink block format

The field mapping convention of sub-clause 7.10 applies. In the addition, when a field spans more than one octet, the order of bit values within each octet progressively decreases as the octet number increases. In that part of a field contained in a given octet the lowest bit number represents the lowest order value.

## 7.9a	Transport block formats (Iu mode)

### 7.9a.1	Generic transport block format

When FLO is used, a transport block may be of any size between 1 and 450 bits for GMSK channels, and between 1 and 1370 bits for 8PSK channels. Each transport block contains one RLC/MAC block.

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | M |  |  |  |  |  |  |
|  |  |  | Bits |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | RLC/MAC |  |  |  |  |  |  |
|  |  |  | Block |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet N |  |

NOTE 1:	The N-th octet may contain fewer than 8 bits

NOTE 2:	The numbering convention specified in 3GPP TS 44.060 applies

Figure 7.9a.1 Transport block format

### 7.9a.2	Transport block format for signalling TFC

A TFC is defined that shall be used for signalling, as specified in 3GPP TS 44.118. It may also be used for data. For this TFC, the transport block format contains 23 octets.

| <---------------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
|  |  |  |  |  |  |  |  | octet 1 |  |
|  |  |  | M |  |  |  |  |  |  |
|  |  |  | Bits |  |  |  |  |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
| : |  |  |  |  |  |  | : |  |  |
|  |  |  | RLC/MAC |  |  |  |  |  |  |
|  |  |  | Block |  |  |  |  | octet 23 |  |

NOTE:	The numbering convention specified in 3GPP TS 44.060 applies

Figure 7.9a.2 Transport block format for TFC for signalling

## 7.10	Order of bit transmission

On channels for normal burst transmission having a block format with an integer N number of octets, the N octets are mapped onto 8N bits, d(0) to d(8N-1), defined in 3GPP TS 45.003 clause 4 and 5 as follows:

Bit m of octet n shall be transmitted as bit d((n-1)*8+m-1) with m=(1..8) and n=(1..N).

On channels for normal burst transmission having a block format with a non-integer number of octets, or for transmission having a defined block format but which does not follow the normal burst transmission, the octets are mapped onto M bits, d(0) to d(M-1), defined in 3GPP TS 45.003 clause 5 as follows:

Bit m of octet n shall be transmitted as bit d(8(n-1) + m-1) with n = (1 .. ((M-1) DIV 8) + 1)) and m = (1 .. min (8, (M - 8(n-1))), and where 'DIV' is the integer division operator.

# 8	Vocabulary

The terminology used in the present document is as follows:

-	Idle mode:

In this mode the MS is not allocated any dedicated physical channel; when the MS is operating in A/Gb mode it listens to the CCCH and the BCCH, whilst when operating in Iu mode it listens to the PCCCH and the PBCCH.

-	Dedicated mode:

In this mode the MS is allocated a dedicated physical channel, at least containing two logical channels, only one of them being a SACCH.

-	Packet mode:

In this mode the MS is ready to access and operate on a packet data physical channel (A/Gb mode) or shared basic physical subchannel (Iu mode). When the MS is operating in A/Gb mode, packet mode includes the  sub-states packet idle mode and packet transfer mode(see 3GPP TS 44.060). When the MS is operating in Iu mode, packet mode includes the sub-states MAC-Idle and MAC-Shared (see 3GPP TS 44.160).

-	Packet Idle Mode:

In packet idle mode, the mobile station is prepared to transfer LLC PDUs on packet data physical channels. The mobile station is not allocated any radio resource on a packet data physical channel; it listens to the PBCCH and PCCCH or, if those are not provided by the network, to the BCCH and the CCCH.

-	Packet Transfer Mode:

In packet transfer mode, the mobile station is prepared to transfer LLC PDUs on packet data  physical channels. The mobile station is allocated radio resource on one or more packet data physical channels for the transfer of LLC PDUs.

-	Dual Transfer Mode:

In dual transfer mode, the mobile station is allocated radio resources providing an RR connection (3GPP TS 44.018) and a Temporary Block Flow on one or more packet data physical channels. The allocation of radio resource for the RR connection and the Temporary Block Flow is co-ordinated by the network in agreement with the capabilities of the mobile station in dual transfer mode.

-	MAC-dedicated:

A MAC-control-entity state where a DBPSCH is assigned and no SBPSCH is assigned. This state only applies in Iu mode.

-	MAC-DTM:

- A MAC-control-entity state where at least one DBPSCH and one SBPSCH are assigned. This state only applies in Iu mode.

-	MAC-Idle:

A MAC-control-entity state where no basic physical subchannel is assigned.

-	MAC-Shared:

A MAC-control-entity state where at least one shared basic physical subchannel is assigned.

-	Physical block:

The physical block is the minimal unit which can be transferred by the physical layer.

-	Dedicated physical channel:

The physical blocks scheduled on the radio medium assigned to a dedicated connection with a MS.

-	Packet data physical channel:

The physical blocks scheduled on the radio medium assigned to a shared usage between MSs for packet switched data traffic.

- Physical channel:

A physical channel uses a combination of frequency and time division multiplexing and is defined in terms of a radio frequency channel sequence and a timeslot sequence. A physical channel uses the same timeslot number in every TDMA frame. The complete definition of a particular physical channel consists of a description in the frequency domain, and a description in the time domain (see 3GPP TS 45.002).

-	Basic physical channel:

A basic physical channel is defined as a physical channel for which the TDMA frame number sequence is 0,1,.. FN_MAX, where FN_MAX is the maximum TDMA frame number for a hyperframe (i.e. all TDMA frames on a timeslot).

-	Basic physical subchannel:

A basic physical subchannel is defined as a basic physical channel or a part of a basic physical channel and an associated multiframe structure. A basic physical subchannel can either be shared or dedicated.

Annex A (informative):
Change History

| TSG # | TSG Doc. | CR | Rev | Subject/Comment | New |
| --- | --- | --- | --- | --- | --- |
| GP-04 | - | - | - | April 2001. Conversion to 3GPP layout and number. | 44.004 v4.0.0 |
| GP-06 | GP-011675 | 002 |  | Introduction of Enhanced Power Control | 5.0.0 |
| GP-07 | GP-012311 | 004 |  | Editorial corrections | 5.1.0 |
| GP-07 | GP-012357 | 006 |  | Enhanced Power Control Alignment with 48.058 | 5.1.0 |
| GP-09 | GP-021234 | 007 | 4 | Introduction of Rel 5 Functionality | 5.2.0 |
| GP-17 | GP-032469 | 008 |  | Incorrect incorporation of CR 007 | 5.3.0 |
| GP-17 | - | - | - | Rel-6 version created based on version 5.3.0 | 6.0.0 |
| GP-17 | GP-032709 | 009 | 2 | NUM Flexible Layer One | 6.0.0 |
| GP-17 | GP-032800 | 010 | 1 | RACH uplink / Uplink access burst" block format | 6.0.0 |
| GP-27 | GP-052810 | 0012 | 2 | Repeated SACCH signalling | 6.1.0 |