3GPP TS 24.235 V11.1.0 (2012-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

3GPP System to Wireless Local Area Network (WLAN)

Interworking Management Object (MO)

(Release 11)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

I-WLAN, IP, network, LTE

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2012, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, and abbreviations	5

3.1	Definitions	5

3.2	Abbreviations	6

4	I-WLAN Management Object	6

5	Management Object parameters	7

5.1	General	7

5.2	Node: /<X>	7

5.3	<X>/Name	8

5.4	/<X>/HomeOperator/	8

5.5	/<X>/HomeOperator/NetworkID	8

5.6	/<X>/HomeOperator/NetworkID/<X>	8

5.7	/<X>/HomeOperator/NetworkID/<X>/SSID	8

5.8	/<X>/HomeOperator/Ext/	9

5.9	/<X>/Policy/	9

5.10	/<X>/Policy/I-WLAN_HPLMN_Priority_Indication	9

5.11	/<X>/Policy/HPLMN_Direct_Access	9

5.12	/<X>/Policy/I-WLAN_Equivalent_HPLMN_Presentation_Indication	10

5.13	/<X>/Policy/RoamingPartnerPriorityList/	10

5.14	/<X>/Policy/RoamingPartnerPriorityList/ Operator_Controlled_PLMN_Selector_for_WLAN_access_List	10

5.15	/<X>/Policy/RoamingPartnerPriorityList/ Operator_Controlled_PLMN_Selector_for_WLAN_access_List/<X>	10

5.16	/<X>/Policy/RoamingPartnerPriorityList/ Operator_Controlled_PLMN_Selector_for_WLAN_access_List/<X>/PLMN_Realms	11

5.17	<X>/Policy/RoamingPartnerPriorityList/ Operator_Controlled_PLMN_Selector_for_WLAN_access_List/<X>Priority	11

5.18	/<X>/Policy/RoamingPartnerPriorityList/ Operator_Controlled_WLAN_Specific_identifier_List	11

5.19	/<X>/Policy/RoamingPartnerPriorityList/ Operator_Controlled_WLAN_Specific_identifier_List/<X>	12

5.20	/<X>/Policy/RoamingPartnerPriorityList/ Operator_Controlled_WLAN_Specific_identifier_List /<X>/SSID	12

5.21	<X>/Policy/RoamingPartnerPriorityList/ Operator_Controlled_WLAN_Specific_identifier_List /<X>Priority	12

5.22	/<X>/Ext/	12

Annex A (informative):	Management Object DDF	13

Annex B (informative):	Change history	19

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

This document defines the 3GPP System to Wireless Local Area Network interworking Management Object (MO) for I-WLAN PLMN selection as specified in 3GPP TS 24.234 [3]. The management object is compatible with OMA Device Management protocol specifications, version 1.2 and upwards, and is defined using the OMA DM Device Description Framework as described in the Enabler Release Definition OMA-ERELD_DM-V1_2 [2].

The 3GPP System to Wireless Local Area Network interworking Management Object consists of relevant parameters that can be managed for WLAN UE as specified in 3GPP TS 24.234 [3].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	OMA-ERELD-DM-V1_2: "Enabler Release Definition for OMA Device Management".

[3]	3GPP TS 24.234: "3GPP System to Wireless Local Area Network (WLAN) interworking; WLAN User Equipment (WLAN UE) to network protocols".

[4]	Void.

[5]	Void.

[6]	3GPP TS 31.102: "Characteristics of the USIM application".

[7]	"Unicode 5.1.0, Unicode Standard Annex #15; Unicode Normalization Forms", March 2008. http://www.unicode.org

[8]	IETF RFC 4282: "The Network Access Identifier".

[9]	3GPP TS 23.003: "Numbering, addressing and identification".

[10]	3GPP TS 23.002: "Network architecture".

[11]	IETF RFC 3629: "UTF-8, a transformation format of ISO 10646".

[12]	IEEE Std 802.11™-2007: "Information Technology- Telecommunications and information exchange between systems-Local and metropolitan area networks-Specific requirements-Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

# 3	Definitions, and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] apply.

For the purposes of the present document, the following term given in 3GPP TS 23.002 [10] applies:

WLAN UE

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACL	Access Control List

DDF	Device Description Framework

DM	Device Management

HPLMN	Home PLMN

I-WLAN	Interworking Wireless Local Area Network

MCC	Mobile Country Code

MNC	Mobile Network Code

MO	Management Object

OMA	Open Mobile Alliance

PLMN	Public Land Mobile Network

SSID	Service Set ID

UE	User Equipment

# 4	I-WLAN Management Object

The 3GPP System to Wireless Local Area Network interworking Management Object (MO) is used to configure WLAN UE behavior for 3GPP System to Wireless Local Area Network as specified in 3GPP TS 24.234 [3].

The Management Object Identifier is: urn:oma:mo:ext-3gpp-IWLAN:1.0.

Protocol compatibility: This MO is compatible with OMA DM 1.2.

The OMA DM Access Control List (ACL) property mechanism (see OMA-ERELD-DM-V1_2 [2]) may be used to grant or deny access rights to OMA DM servers in order to modify nodes and leaf objects of the I-WLAN MO.

The following nodes and leaf objects are possible under the I-WLAN node as described in figure 4-1:

Figure 4-1: The 3GPP system to WLAN interworking Management Object

# 5	Management Object parameters

## 5.1	General

This clause describes the parameters for the 3GPP system to WLAN interworking MO. The parameters identified in this section correspond to those operator specific files defined in 3GPP TS 24.234 [3] clause 7. This section defines only how the parameters are structured and obtained by means of OMA DM.

## 5.2	Node: /<X>

This interior node acts as a placeholder for a fixed node.

-	Occurrence: OneOrMore

-	Format: node

-	Access Types: Get, Replace

-	Values: N/A

## 5.3	<X>/Name

The Name leaf is a name for the 3GPP system to WLAN interworking MO settings.

-	Occurrence: ZeroOrOne

-	Format: chr

-	Access Types: Get

-	Values: <User displayable name>

The User displayable name shall be represented by Unicode characters encoded as UTF-8 as specified in IETF RFC 3629 [11] and formatted using Normalization Form KC (NFKC) as specified in Unicode Standard Annex #15; Unicode Normalization Forms [7].

## 5.4	/<X>/HomeOperator/

The HomeOperator interior node is used to allow a reference to information required by the WLAN UE to identify and access the home network.

-	Occurrence: One

-	Format: node

-	Access Types: Get, Replace

-	Values: N/A

## 5.5	/<X>/HomeOperator/NetworkID

The NetworkID interior node is used to allow a reference to a list of SSIDs that identify WLANs that have a direct relationship with the Home PLMN (HPLMN).

-	Occurrence: ZeroOrOne

-	Format: node

-	Access Types: Get, Replace

-	Values: N/A

## 5.6	/<X>/HomeOperator/NetworkID/<X>

This run-time node acts as a placeholder for one or more SSIDs.

-	Occurrence: OneOrMore

-	Format: node

-	Access Types: Get, Replace

-	Values: N/A

## 5.7	/<X>/HomeOperator/NetworkID/<X>/SSID

The SSID leaf identifies the SSID for a particular preferred WLAN network as defined in 3GPP TS 24.234 [3] subclause 7.6b.

-	Occurrence: One

-	Format: chr

-	Access Types: Get, Replace

-	Values: <SSID>

The format of the SSID is defined by IEEE Std 802.11™-2007 [12].

## 5.8	/<X>/HomeOperator/Ext/

The Ext is an interior node for where the vendor specific information about the 3GPP System to Wireless Local Area Network MO is being placed (vendor meaning application vendor, device vendor etc.). Usually the vendor extension is identified by vendor specific name under the ext node. The tree structure under the vendor identified is not defined and can therefore include one or more un-standardized sub-trees.

-	Occurrence: ZeroOrOne

-	Format: node

-	Access Types: Get, Replace

-	Values: N/A

## 5.9	/<X>/Policy/

The Policy interior node is used to reference to information required by the WLAN UE to access an I-WLAN and PLMN while roaming.

-	Occurrence: ZeroOrOne

-	Format: node

-	Access Types: Get, Replace

-	Values: N/A

## 5.10	/<X>/Policy/I-WLAN_HPLMN_Priority_Indication

The I-WLAN_HPLMN_Priority_Indication leaf is defined in 3GPP TS 24.234 [3] subclause 7.10.

-	Occurrence: ZeroOrOne

-	Format: bin

-	Access Types: Get, Replace

-	Values:< I-WLAN_HPLMN_Priority_Indication >

The format and value of the I-WLAN_HPLMN_Priority_Indication is defined by 3GPP TS 31.102 [6] subclause 4.4.5.9.

## 5.11	/<X>/Policy/HPLMN_Direct_Access

The HPLMN_Direct_Access leaf corresponds to HPLMN Direct Access Indicator as defined in 3GPP TS 24.234 [3] subclause 7.11.

-	Occurrence: ZeroOrOne

-	Format: bin

-	Access Types: Get, Replace

-	Values: < HPLMN_Direct_Access >

The format and value of the HPLMN_Direct_Access is defined by 3GPP TS 31.102 [6] subclause 4.4.5.11.

## 5.12	/<X>/Policy/I-WLAN_Equivalent_HPLMN_Presentation_Indication

The I-WLAN_Equivalent_HPLMN_Presentation_Indication leaf is defined in 3GPP TS 24.234 [3] subclause 7.12.

-	Occurrence: ZeroOrOne

-	Format: bin

-	Access Types: Get, Replace

-	Values: < I-WLAN_Equivalent_HPLMN_Presentation_Indication >

The format and value of the I-WLAN_Equivalent_HPLMN_Presentation_Indication is defined by 3GPP TS 31.102 [6] subclause 4.4.5.8.

## 5.13	/<X>/Policy/RoamingPartnerPriorityList/

The RoamingPartnerPriorityList interior node is used to reference to information required by the WLAN UE to access an I-WLAN and PLMN while roaming.

-	Occurrence: ZeroOrOne

-	Format: node

-	Access Types: Get, Replace

-	Values: N/A

## 5.14	/<X>/Policy/RoamingPartnerPriorityList/
Operator_Controlled_PLMN_Selector_for_WLAN_access_List

The Operator_Controlled_PLMN_Selector_for_WLAN_access_List interior node is used to identify the roaming partner realms.

-	Occurrence: ZeroOrOne

-	Format: node

-	Access Types: Get, Replace

-	Values: N/A

## 5.15	/<X>/Policy/RoamingPartnerPriorityList/
Operator_Controlled_PLMN_Selector_for_WLAN_access_List/<X>

This run-time node acts as a placeholder for one or more PLMN_Realms.

-	Occurrence: OneOrMore

-	Format: node

-	Access Types: Get, Replace

-	Values: N/A

## 5.16	/<X>/Policy/RoamingPartnerPriorityList/
Operator_Controlled_PLMN_Selector_for_WLAN_access_List/<X>/PLMN_Realms

The PLMN_Realms leaf defines a preferred roaming partner realm and corresponds to an entry of Operator Controlled PLMN Selector for I-WLAN access as defined in 3GPP TS 24.234 [3] subclause 7.5.

-	Occurrence: One

-	Format: chr

-	Access Types: Get, Replace

-	Values: <PLMN_Realms>

The PLMN_Realms contains a PLMN identifier (see 3GPP TS 23.003 [9]) that shall be in the form of an Internet domain name as defined by IETF RFC 4282 [8]. The syntax of the PLMN_Realms shall be as follows:

1.	take the PLMN identifier (see 3GPP TS 23.003 [9]) and separate into MCC and MNC; if the MNC is 2 digits then a zero shall be added at the beginning;

2.	use the MCC and MNC derived in step 1 to create the "mnc<MNC>.mcc<MCC>.3gppnetwork.org" domain name;

3.	add the label "wlan." to the beginning of the domain name.

## 5.17	<X>/Policy/RoamingPartnerPriorityList/
Operator_Controlled_PLMN_Selector_for_WLAN_access_List/<X>Priority

The Priority leaf represents the priority given to one Operator_Controlled_PLMN_Selector_for_WLAN_access_List entry and is represented as a numerical value.

-	Occurrence: One

-	Format: int

-	Access Types: Get, Replace

-	Values: <Priority>

In case more than one valid entry in Operator_Controlled_PLMN_Selector_for_WLAN_access_List exists, the WLAN UE shall treat the Operator_Controlled_PLMN_Selector_for_WLAN_access_List entry with the lowest Priority value as the Operator_Controlled_PLMN_Selector_for_WLAN_access_List entry having the highest priority.

## 5.18	/<X>/Policy/RoamingPartnerPriorityList/
Operator_Controlled_WLAN_Specific_identifier_List

The Operator_Controlled_WLAN_Specific_identifier_List interior node is to identify the preferred SSIDs the WLAN UE should scan for.

-	Occurrence: ZeroOrOne

-	Format: node

-	Access Types: Get, Replace

-	Values: N/A

## 5.19	/<X>/Policy/RoamingPartnerPriorityList/
Operator_Controlled_WLAN_Specific_identifier_List/<X>

This run-time node acts as a placeholder for one or more SSID.

-	Occurrence: OneOrMore

-	Format: node

-	Access Types: Get, Replace

-	Values: N/A

## 5.20	/<X>/Policy/RoamingPartnerPriorityList/
Operator_Controlled_WLAN_Specific_identifier_List /<X>/SSID

The SSID leaf defines the SSID for a particular preferred WLAN network as defined in 3GPP TS 24.234 [3] subclause 7.6a.

-	Occurrence: One

-	Format: chr

-	Access Types: Get, Replace

-	Values: <SSID >

The format of the SSID is defined by IEEE Std 802.11™-2007 [12].

## 5.21	<X>/Policy/RoamingPartnerPriorityList/
Operator_Controlled_WLAN_Specific_identifier_List /<X>Priority

The Priority leaf represents the priority given to one Operator_Controlled_WLAN_Specific_identifer_List entry and is represented as a numerical value.

-	Occurrence: One

-	Format: int

-	Access Types: Get, Replace

-	Values: <Priority>

In case more than one valid entry in Operator_Controlled_WLAN_Specific_identifier_List exists, the WLAN UE shall treat the Operator_Controlled_WLAN_Specific_identifer_List entry with the lowest Priority value as the Operator_Controlled_WLAN_Specific_identifier_List entry having the highest priority.

## 5.22	/<X>/Ext/

The Ext is an interior node for where the vendor specific information about the 3GPP System to Wireless Local Area Network interworking MO is being placed (vendor meaning application vendor, device vendor etc.). Usually the vendor extension is identified by vendor specific name under the ext node. The tree structure under the vendor identified is not defined and can therefore include one or more un-standardized sub-trees.

-	Occurrence: ZeroOrOne

-	Format: node

-	Access Types: Get, Replace

-	Values: N/A

Annex A (informative):
Management Object DDF

This DDF is the standardized minimal set. A vendor can define its own DDF for the complete device. This DDF can include more features than this minimal standardized version.

<?xml version="1.0" encoding="UTF-8"?>

<!DOCTYPE MgmtTree PUBLIC "-//OMA//DTD-DM-DDF 1.2//EN"

"http://www.openmobilealliance.org/tech/DTD/dm_ddf-v1_2.dtd">

<MgmtTree>

<VerDTD>1.2</VerDTD>

<Node>

<NodeName>IWLAN</NodeName>

<DFProperties>

<AccessType>

<Get/>

</AccessType>

<Description> IWLAN settings</Description>

<DFFormat>

<node/>

</DFFormat>

<Occurrence>

<OneOrMore/>

</Occurrence>

<DFTitle>The IWLAN Management Object.</DFTitle>

<DFType>

<DDFName/>

</DFType>

<Node>

<NodeName>Name</NodeName>

<DFProperties>

<AccessType>

<Get/>

</AccessType>

<DFFormat>

<chr/>

</DFFormat>

<Occurrence>

<ZeroOrOne/>

</Occurrence>

<DFTitle>User displayable name for the node</DFTitle>

<DFType>

<MIME>text/plain</MIME>

</DFType>

</DFProperties>

</Node>

<NodeName>HomeOperator</NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<node/>

</DFFormat>

<Occurrence>

<ZeroOrOne/>

</Occurrence>

<DFTitle>HomeOperator Policy.</DFTitle>

<DFType>

<DDFName/>

</DFType>

</DFProperties>

<Node>

<NodeName>Network_ID</NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<node/>

</DFFormat>

<Occurrence>

<ZeroOrOne/>

</Occurrence>

<DFTitle>Home Network IDs.</DFTitle>

<DFType>

<DDFName/>

</DFType>

</DFProperties>

<Node>

<NodeName></NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<node/>

</DFFormat>

<Occurrence>

<OneOrMore/>

</Occurrence>

<DFType>

<DDFName></DDFName>

</DFType>

</DFProperties>

<Node>

<NodeName>SSID</NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<chr/>

</DFFormat>

<Occurrence>

<One/>

</Occurrence>

<DFTitle>SSID Reference</DFTitle>

<DFType>

<MIME>text/plain</MIME>

</DFType>

</DFProperties>

</Node>

<Node>

<Node>

<Node>

<NodeName>Ext</NodeName>

<!-- The Extension node starts here. -->

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<node/>

</DFFormat>

<Occurrence>

<ZeroOrOne/>

</Occurrence>

<DFTitle>A collection of all Extension objects.</DFTitle>

<DFType>

<DDFName/>

</DFType>

</DFProperties>

</Node>

<Node>

<NodeName>Policy</NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<node/>

</DFFormat>

<Occurrence>

<ZeroOrOne/>

</Occurrence>

<DFTitle>Policy Information.</DFTitle>

<DFType>

<DDFName/>

</DFType>

</DFProperties>

<Node>

<NodeName>I-WLAN_HPLMN_Priority_Indication</NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<bin/>

</DFFormat>

<Occurrence>

<ZeroOrOne/>

</Occurrence>

<DFTitle> I-WLAN_HPLMN_Priority_Indication.</DFTitle>

<DFType>

<MIME>text/plain</MIME>

</DFType>

</DFProperties>

</Node>

<Node>

<NodeName>HPLMN_Direct_Access</NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<bin/>

</DFFormat>

<Occurrence>

<ZeroOrOne/>

</Occurrence>

<DFTitle> HPLMN_Direct_Access.</DFTitle>

<DFType>

<MIME>text/plain</MIME>

</DFType>

</DFProperties>

</Node>

<Node>

<NodeName>I-WLAN_Equivalent_HPLMN_Presentation_Indication</NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<bin/>

</DFFormat>

<Occurrence>

<ZeroOrOne/>

</Occurrence>

<DFTitle>Access identity.</DFTitle>

<DFType>

<MIME>text/plain</MIME>

</DFType>

</DFProperties>

</Node>

<Node>

<NodeName>RoamingPartnerPriorityList</NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<node/>

</DFFormat>

<Occurrence>

<ZeroOrOne/>

</Occurrence>

<DFTitle> RoamingPartnerPriorityLists.</DFTitle>

<DFType>

<DDFName/>

</DFType>

</DFProperties>

<Node>

<NodeName>Operator_Controlled_PLMN_Selector</NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<node/>

</DFFormat>

<Occurrence>

<ZeroOrOne/>

</Occurrence>

<DFTitle> Realm Roaming Partners.</DFTitle>

<DFType>

<DDFName/>

</DFType>

</DFProperties>

<Node>

<NodeName></NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<node/>

</DFFormat>

<Occurrence>

<OneOrMore/>

</Occurrence>

<DFType>

<DDFName></DDFName>

</DFType>

</DFProperties>

<Node>

<NodeName>PLMN_Realms</NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<chr/>

</DFFormat>

<Occurrence>

<One/>

</Occurrence>

<DFTitle>List of PLMNs.</DFTitle>

<DFType>

<MIME>text/plain</MIME>

</DFType>

</DFProperties>

</Node>

</Node>

<Node>

<NodeName>Priority</NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<chr/>

</DFFormat>

<Occurrence>

<One/>

</Occurrence>

<DFTitle>Priority of PLMN identity</DFTitle>

<DFType>

<MIME>text/plain</MIME>

</DFType>

</DFProperties>

</Node>

</Node>

<Node>

<NodeName>Operator_Controlled_WLAN_Specific_Identity_list</NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<node/>

</DFFormat>

<Occurrence>

<ZeroOrOne/>

</Occurrence>

<DFTitle> WLAN Roaming Partners.</DFTitle>

<DFType>

<DDFName/>

</DFType>

</DFProperties>

<Node>

<NodeName></NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<node/>

</DFFormat>

<Occurrence>

<OneOrMore/>

</Occurrence>

<DFType>

<DDFName></DDFName>

</DFType>

</DFProperties>

<Node>

<NodeName>SSID</NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<chr/>

</DFFormat>

<Occurrence>

<One/>

</Occurrence>

<DFTitle> SSID Reference.</DFTitle>

<DFType>

<MIME>text/plain</MIME>

</DFType>

</DFProperties>

</Node>

<Node>

<NodeName>Priority</NodeName>

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<chr/>

</DFFormat>

<Occurrence>

<One/>

</Occurrence>

<DFTitle>Priority of SSID identity</DFTitle>

<DFType>

<MIME>text/plain</MIME>

</DFType>

</DFProperties>

</Node>

</Node>

</Node>

</Node>

</Node>

<Node>

<NodeName>Ext</NodeName>

<!-- The Extension node starts here. -->

<DFProperties>

<AccessType>

<Get/>

<Replace/>

</AccessType>

<DFFormat>

<node/>

</DFFormat>

<Occurrence>

<ZeroOrOne/>

</Occurrence>

<DFTitle>A collection of all Extension objects.</DFTitle>

<DFType>

<DDFName/>

</DFType>

</DFProperties>

</Node>

</Node>

</MgmtTree>

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2011-01 |  |  |  |  | Version 0.0.1: Preliminary proposal, contained in C1-110880 |  | 0.0.1 |
| 2011-02 |  |  |  |  | Contained in C1-111199 Updated to include the following: Right clip on the front page Change scope to include PLMN selection Identity that the leafs only cover the operator configurable files in 3GPP TS 24.234 Dereference the OMA WLAN MO Add priority leaf in for SSID and PLMN realm | 0.0.1 | 0.0.2 |
| 2011-02 |  |  |  |  | Contained in C1-111491 | 0.0.2 | 0.0.3 |
| 2011-03 | CT-51 | CP-110150 |  |  | Version 1.0.0 created for presentation to CT-51 for information and approval | 0.0.3 | 1.0.0 |
| 2011-03 | CT-51 |  |  |  | Correction of styles Introduce missing abbreviations | 1.0.0 | 1.0.1 |
| 2011-03 | CT-51 |  |  |  | Introduction of the TS-number | 1.0.1 | 1.0.2 |
| 2011-03 | CT-51 |  |  |  | Version 10.0.0 created after approval at CT-51 | 1.0.2 | 10.0.0 |
| 2011-06 | CT-52 | CP-110473 | 0001 |  | MO identifier registered by OMNA | 10.0.0 | 10.1.0 |
| 2011-09 | CT-53 | CP-110694 | 0002 |  | Correction to terminology and editorials | 10.1.0 | 11.0.0 |
| 2012-06 | CT-56 | CP-120326 | 0003 | 1 | Corrections and editorial cleanup | 11.0.0 | 11.1.0 |