3GPP TR 22.808 V14.1.0 (2015-12)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Study on Flexible Mobile Service Steering (FMSS)

(Release 14)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Mobile, Steering

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - France

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2015, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	Overview	6

5	Use cases	7

5.1	Traffic Classification and Steering based on Application Characteristics	7

5.1.1	Description	7

5.1.2	Pre-conditions	7

5.1.3	Service Flows	7

5.1.4	Post-conditions	8

5.1.5	Potential impacts or interactions with existing Services/Features	8

5.1.6	Potential Requirements	8

5.2	Traffic steering based on MNO's policies or user's subscription	8

5.2.1	Anti-advertisement services	8

5.2.1.1	Description	8

5.2.1.2	Pre-conditions	8

5.2.1.3	Service flows	8

5.2.1.4	Post-conditions	9

5.2.1.5	Potential impacts or interactions with existing Services/Features	9

5.2.1.6 	Potential requirements	9

5.2.2 	Security services	9

5.2.2.1	Description	9

5.2.2.2	Pre-conditions	9

5.2.2.3	Service Flows	9

5.2.2.4	Post-conditions	10

5.2.2.5	Potential Impacts or Interactions with Existing Services/Features	10

5.2.2.6	Potential Requirements	10

5.3	Traffic steering based on Network status	10

5.3.1	Description	10

5.3.2	Pre-conditions	10

5.3.3	Service flows	10

5.3.4	Post-conditions	11

5.3.5	Potential impacts or interactions with existing Services/Features	11

5.3.6 	Potential requirements	11

5.4	Traffic steering in MVNO scenario	11

5.4.1	Description	11

5.4.2	Pre-conditions	11

5.4.3	Service flows	11

5.4.4	Post-conditions	11

5.4.5	Potential impacts or interactions with existing Services/Features	12

5.4.6 	Potential requirements	12

5.5	Traffic Steering based on Time and/or Location	12

5.5.1	Description	12

5.5.2	Pre-conditions	12

5.5.3	Service flows	12

5.5.4	Post-conditions	12

5.5.5	Potential impacts or interactions with existing Services/Features	12

5.5.6 	Potential requirements	12

5.6	Traffic steering based on QoS information	12

5.6.1	Description	12

5.6.2	Pre-conditions	13

5.6.3	Service flows	13

5.6.4	Post-conditions	13

5.6.5	Potential impacts or interactions with existing Services/Features	13

5.6.6 	Potential requirements	13

5.7	Traffic Steering in Roaming Cases	13

5.7.1	Description	13

5.7.2	Pre-conditions	13

5.7.3	Service flows	13

5.7.4	Post-conditions	14

5.7.5	Potential impacts or interactions with existing Services/Features	14

5.7.6 	Potential requirements	14

5.8	Dynamic modification of the traffic steering policy during the connection	14

5.8.1	Description	14

5.8.2	Pre-conditions	14

5.8.3	Service Flows	14

5.8.4	Post-conditions	14

5.8.5	Potential Impacts or Interactions with Existing Services/Features	14

5.8.6	Potential Requirements	14

5.9	 Traffic Steering based on destination network	15

5.9.1	Description	15

5.9.2	Pre-conditions	15

5.9.3	Service flows	15

5.9.4	Post-conditions	15

5.9.5	Potential impacts or interactions with existing Services/Features	15

5.9.6 	Potential requirements	15

5.10	Supporting third party owned (S)Gi-LAN service	15

5.10.1	Description	15

5.10.2	Pre-conditions	15

5.10.3	Service Flows	16

5.10.4	Post-conditions	17

5.10.5	Potential Impacts or Interactions with Existing Services/Features	17

5.10.6	Potential Service Requirements	17

6	Potential requirements	17

6.1	Requirements covered by other existing features	17

6.2	Potential additional requirements	17

6.2.1	Generic potential requirements	17

6.2.2	Additional consolidated requirements	17

7	Conclusions	18

Annex A:	Mapping of potential requirements	19

Annex B:	Change history	21

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

Y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The objective is to study use cases and propose potential requirements for supporting traffic classification and service chain selection capabilities per operator's policy (e.g. based on user's profile, application type, RAN type, RAN status and flow direction) in order to realize efficient and flexible mobile service steering in the (S)Gi-LAN network.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

FMSS	Flexible Mobile Service Steering

# 4	Overview

In order to realize efficient and flexible mobile service steering in the (S)Gi-LAN, the MNO uses some information (e.g. user profile, RAT type, application characteristics, etc.) to define traffic steering policies. These policies are used to steer the MNO's subscriber's traffic data to appropriate enablers (e.g. NAT, FW) in (S)Gi-LAN.

The term (S)Gi-LAN used in the present document represents a system which is out of 3GPP scope.

Figure 1: Overview of Flexible Mobile Service Steering

# 5	Use cases

## 5.1	Traffic Classification and Steering based on Application Characteristics

### 5.1.1	Description

Faced with quick development of data traffic, MNO can deploy many service enablers (e.g. URL filter, video optimization, protocol optimization, etc.) behind Gi/Sgi interface to improve the user's QoE, reduce the bandwidth pressure and provide valued added service.

This use case describes that data traffic can be steered through several service enablers based on its application characteristics. Firstly, mobile network can analyze and identify traffic's application characteristics, such as application's type, application protocol and so on. Secondly, mobile network can steer the traffic through several specific service enablers. For example, if data traffic is generated from Web browsing, the traffic will go through protocol optimization service enabler to improve users' experience according to operators' policy.

### 5.1.2	Pre-conditions

MNO deploys three independent service enablers:

-	video optimization, for example, to adapt video resolution and bitrate according to user's terminal.

-	URL filter, for example, to block some unsafe web URLs that user should not access.

-	protocol optimization, for example, to use multipart or pipeline mechanism to optimize the HTTP protocol.

Bob initiates a Web browsing request to visit a website.

MNO adds a traffic steering policy into mobile network: if data traffic is for a web browsing service, traffic should be steered to protocol optimization enabler.

### 5.1.3	Service Flows

Mobile network analyses all the traffic, and differentiates the Web browsing traffic out. According to operator's policies, the traffic, including upstream and downstream traffic, will be steered through protocol optimization enablers deployed behind Gi/Sgi interface.

Figure 2: Service Flows for Traffic Classification and Steering based on Application Characteristics

### 5.1.4	Post-conditions

The protocol optimization enabler accelerated HTTP web browsing service. Bob got a faster Internet experience.

### 5.1.5	Potential impacts or interactions with existing Services/Features

TBD and maybe none.

### 5.1.6	Potential Requirements

The 3GPP network should be able to recognize the application characteristics, such as application type (video, web browsing, IM, etc.) and application protocol (HTTP, P2P, etc.), target address name (URL) and application provider (e.g. YoutubeTM).

The 3GPP network should be able to steer traffic (upstream, downstream or both) per these application characteristics in order to provide value-added service to third-party application.

## 5.2	Traffic steering based on MNO's policies or user's subscription

### 5.2.1	Anti-advertisement services

#### 5.2.1.1	Description

This use case illustrates the traffic steering policy based on the user's subscription. The MNO distinguishes the user's priority (e.g. gold, silver and bronze priority) and the user's status with respect to optional subscriber services (e.g. web filtering for children, employees and virus detection) based on the user's subscription data and implements traffic steering based on the user's information.

#### 5.2.1.2	Pre-conditions

A high priority user and a normal user can use a popular mobile network application that pushes advertisements to the users. The users have to close the regular pop-up advertisements when using this application.

#### 5.2.1.3	Service flows

When the user's downlink advertisements sent from certain mobile network application arrive at the MNO, the MNO steers the user's traffic through different traffic path:

-	For the high priority (gold) user, traffic is steered through an Anti-Advertisement enabler.

-	For the medium priority (silver) user, traffic is steered through an Anti-Advertisement enabler for a period of time after which pop-up advertisements are allowed again by no longer steered traffic through an Anti-Advertisement enabler.

-	For the normal (bronze) user, traffic is not steered through an Anti-Advertisement enabler.

-	For users with filtering for children enabled, traffic is steered through an enabler that allows or blocks traffic based on algorithms tailored to this.

-	For users with filtering for employees enabled, traffic is steered through an enabler that restricts traffic according to rules set by the subscriber.

-	For users with filtering for viruses enabled, traffic is steered through an enabler that checks for viruses, malware, etc., and allows, blocks or decontaminates traffic as appropriate.

#### 5.2.1.4	Post-conditions

For the high priority user, advertisements pushed by the application would be filtered by the Anti-Advertisements enabler, while the advertisement pushed by the application would not be filtered for normal users.

#### 5.2.1.5	Potential impacts or interactions with existing Services/Features

None.

#### 5.2.1.6 	Potential requirements

Based on end-user's priority and the status of optional subscriber services from the subscription data, the 3GPP network should allow the MNO to define different traffic steering policies. The MNO can then steer all/some parts of the user's traffic through different traffic paths based on the end-user's subscription data in order to offer differentiated services for different end-users, e.g. some high priority end-users could get service with no advertisements.

### 5.2.2 	Security services

#### 5.2.2.1	Description

This use case addresses traffic steering to security elements based on user's subscription or MNO's policies. MNOs deploy various types of security features – include but not limited to DdoS protection, antimalware, anti-virus, and parental control. These security features are enabled on the (S)Gi-LAN for value-added services.

#### 5.2.2.2	Pre-conditions

The following are preconditions for this use case:

-	Users are classified to receive various types of services from MNOs using various criteria.

-	MNOs provide a wide variety of security features, such as DdoS protection, antimalware, and parental control to their subscribers.

-	Service steering of security features is based on user's subscription, type of service, location of subscriber, or other MNO's policies.

#### 5.2.2.3	Service Flows

Users are subscribed to one or more security services. Mobile network classifies each user or user's traffic. As user's traffic reaches (S)Gi interface, its type is identified and it is steered appropriately to one or more security elements, as depicted in Figure 3

User's traffic traversing the core network is aiming to access IP addresses or URLs, which the MNO blacklists, as per its policies.  As a result, the traffic is steered to antimalware services.

Figure 3: Security Service Flows

User A is accessing a URL, which is blacklisted as per MNO's policy. As a result, the MNO steers User A's traffic to Antimalware services. User B is subscribed to DdoS protection service. As such, User B's traffic is steered to DdoS protection service. User C is subscribed to two services – Parental Control and DdoS protection, resulting in User C's traffic to be steered to both, Parental Control and DdoS protection services.

#### 5.2.2.4	Post-conditions

Users receive the appropriate value-added security services from MNOs.

#### 5.2.2.5	Potential Impacts or Interactions with Existing Services/Features

N/A

#### 5.2.2.6	Potential Requirements

The 3GPP core network shall provide different traffic steering rules according to MNO's policies and user's subscription. The MNO can then steer all or some parts of user's traffic through different traffic paths involving security services in order to offer differentiated security services for different end-users or to comply with their policies.

## 5.3	Traffic steering based on Network status

### 5.3.1	Description

This use case illustrates the traffic steering policy based on the user's current RATs and network (RAN and CN) congestion status.

### 5.3.2	Pre-conditions

When a user watches the video in 2G or the network is congested, the video streaming would not be played fluently, as not much bandwidth for transferring the video stream can be assigned to the user. In this case, the user sees a discontinuous video.

While when a user watches the video in 3G, LTE or the network is in a normal load status, the video streaming would be played fluently as enough bandwidth can be assigned to the user.

### 5.3.3	Service flows

When a user watches the video service in 2G or when the network is congested, the MNO steers the user's HTTP request to a HTTP cache proxy to allow UE request for small bit rates video segments.

When a user watches the video service in 3G, LTE or when the network is in a normal load status, the user can watch the video fluently and user's HTTP request is sent to the HTTP server directly without steering to the HTTP cache proxy.

### 5.3.4	Post-conditions

The MNO only steers the HTTP requests to the HTTP cache proxy when the users are in 2G or in a congested network. Other user's HTTP requests are sent directly to the HTTP server. In this case, the video is played fluently, even if the user is watching the video service in 2G or is served by a congested network and at the same time, the burden of HTTP cache proxy can be controlled by the MNO to improve the network reliability.

### 5.3.5	Potential impacts or interactions with existing Services/Features

As a DASH client, the UE requests suitable bits rate video segments based on the radio network conditions.

### 5.3.6 	Potential requirements

Based on the user's current RAT and the load status of the network, the 3GPP network should allow the MNO to define the traffic steering policy. The MNO can then steer the user's traffic through different traffic path.

The operator should be able to change the user's traffic steering policy based on the network (RAN and CN) load change and/or RAT status change.

## 5.4	Traffic steering in MVNO scenario

### 5.4.1	Description

This use case illustrates the traffic steering policy generation in MVNO scenario.

### 5.4.2	Pre-conditions

MVNO1 is a mobile virtual network operator of an Operator MNO-A.

Simon is a user of MNO-A.

Bob is a user of MVNO-1.

Both Simon and Bob access video content.

MVNO-1 has an Service Level Agreement with MNO-A.

MVNO-1 decides to provide a gold service for video to guarantee the experience to its users in certain period. Within this plan, its users can access more advanced feature (e.g. advertisement blocking).

### 5.4.3	Service flows

There is no specific traffic steering policy for MNO-A user Simon.

But MNO-A applies a specific traffic steering policy (e.g. video optimization with advertisement blocking, …) for Bob based on MVNO-1 needs.

### 5.4.4	Post-conditions

Bob experience video service as expected by MVNO-1. Several advertisements are delivered to Simon during the video watching period, and Bob watches the video program without any advertisements.

### 5.4.5	Potential impacts or interactions with existing Services/Features

None identified.

### 5.4.6 	Potential requirements

An Operator shall be able to define specific traffic steering policies to apply to users of an MVNO, based on MVNO's preferences.

The MNO may apply different traffic steering policies for MVNO users than for MNO users.

## 5.5	Traffic Steering based on Time and/or Location

### 5.5.1	Description

There might be high traffic volume during busy hours or at some specific locations where an important event happens (e.g. football stadium, shopping center). It is useful to apply some mechanisms, e.g. traffic compression and transcoding, to reduce traffic volume. This contribution describes a use case where the MNO implements traffic steering based on time and/or location.

### 5.5.2	Pre-conditions

An operator gets the network load from its historical network data. There is high traffic volume at 14:00~17:00 in ABC shopping center.

In addition, there is a football game in a stadium and it causes a high traffic volume.

The operator decides to reduce the traffic volume by traffic compression and transcoding.

Alice is in ABC shopping center at 15:00 and sends her new clothes photos to her Mom from her mobile phone.

Bob goes to the stadium and shares the game video with his friends from his mobile phone.

### 5.5.3	Service flows

Bob's video streaming is steered to traffic transcoding enabler to reduce the bandwidth.

The traffic of Alice's photos is steered to traffic compression enabler.

### 5.5.4	Post-conditions

Mobile network traffic is reduced.

### 5.5.5	Potential impacts or interactions with existing Services/Features

None.

### 5.5.6 	Potential requirements

A 3GPP network should be able to take time and/or location into account when defining traffic steering policies.

## 5.6	Traffic steering based on QoS information

### 5.6.1	Description

This use case illustrates the traffic steering policy to be used for the traffic based on the QoS information of the bearer.

### 5.6.2	Pre-conditions

Tom watches a video program over a non-GBR bearer. Due to frequent radio condition variation, there is no guaranteed bandwidth for video stream delivery to Tom. As a consequence, Tom experiences several discontinuances as the video streaming is not fluent.

Paul watches the same video program over a GBR bearer. The bandwidth for video stream delivery to Paul is guaranteed even if the radio condition varies frequently. As a consequence, the video streaming is fluent.

### 5.6.3	Service flows

According to operator's policy, the 3GPP network steers Tom's video service request to a HTTP cache/proxy with trans-coding support to allow lower resolution content of video program to be delivered to Tom.

As Paul's video streaming QoE is guaranteed, the MNO steers Paul's video service request to the HTTP server directly without HTTP cache/proxy processing.

### 5.6.4	Post-conditions

As Tom is using a non GBR bearer, the HTTP cache/proxy optimizes the video streaming experienced by Tom. The video is played fluently.

Paul's video streaming experience remains the same as it's over a GBR bearer.

### 5.6.5	Potential impacts or interactions with existing Services/Features

None.

### 5.6.6 	Potential requirements

Based on the bearer QoS information, the 3GPP network shall be able to determine the traffic steering policy.

## 5.7	Traffic Steering in Roaming Cases

### 5.7.1	Description

This use case illustrates the traffic steering in roaming cases.

### 5.7.2	Pre-conditions

Operator A and B deploy service chain in (S)Gi-LAN network, which includes video optimization service enabler.

For APN1 in operator A, roaming case is local breakout; For APN2 in operator A, roaming case is home routed.

Operator A provides value added service of video optimization for its gold subscribers in non-roaming cases and roaming cases in some specific areas (e.g. in operator B's network).

John and Tom are gold subscribers of operator A.

John and Tom are roaming in operator B's network.

John accesses "Youku" video through APN1; Tom accesses his company's video through APN2.

### 5.7.3	Service flows

When John accesses "Youku" video through APN1, the video traffic passes through the (S)Gi-LAN network of operator B, operator B takes the home operator's policy into account and decides to apply optimization on the video traffic and the video traffic is steered to the video optimization enabler in operator B.

When Tom accesses his company's video through APN2, the video traffic passes through the (S)Gi-LAN network of operator A, and operator A decides that the video traffic should be optimized and the video traffic is steered to the video optimization enabler in operator A.

### 5.7.4	Post-conditions

John and Tom enjoy the optimized video traffic.

### 5.7.5	Potential impacts or interactions with existing Services/Features

TBD and maybe none.

### 5.7.6 	Potential requirements

The home PLMN should be able to apply traffic steering policies for home routed traffic or, subject to home PLMN operator's policy transfer traffic steering policies to the visited PLMN in the case of local breakout for roaming users.

For the local breakout traffic, the visited PLMN should be able to take the traffic steering policies from the home PLMN into account when passing traffic through (S)Gi-LAN in the visited network.

## 5.8	Dynamic modification of the traffic steering policy during the connection

### 5.8.1	Description

A connection is set up under certain conditions, leading to the use of a traffic steering policy, but these conditions may change during the connection (for example, time of the day and RAT change). If any of those conditions change during an ongoing connection, the 3GPP network shall be able to change the traffic steering policy accordingly.

### 5.8.2	Pre-conditions

The user (a minor) sets up an Internet connection, and freely browses the Internet. Traffic steering policy responsible for NAT is set up for the user.

The user's parents have configured her subscription in such a way that after 18:00 o'clock only whitelisted Internet sites are accessible.

### 5.8.3	Service Flows

Before 18:00 o'clock, the user browses the Internet freely.

At 18:00 o'clock, the 3GPP Network detects that parental controls are needed, and dynamically modifies the traffic steering policy so that also parental control is applied, in addition to NAT, limiting the user's Internet access to whitelisted Internet sites.

### 5.8.4	Post-conditions

After 18:00 o'clock, the user is able to access only whitelisted Internet sites.

### 5.8.5	Potential Impacts or Interactions with Existing Services/Features

None.

### 5.8.6	Potential Requirements

The 3GPP Network shall be able to modify traffic steering policy dynamically during an ongoing connection.

## 5.9	 Traffic Steering based on destination network

### 5.9.1	Description

This contribution describes a use case where the MNO implements traffic steering based on information of the destination of traffic flow (i.e. a PDN or an internal IP network).

### 5.9.2	Pre-conditions

An operator provides video optimization enabler for an enterprise access, and not for internet access.

John accesses the enterprise video through the enterprise access.

Tom accesses Youku video through internet access.

### 5.9.3	Service flows

John's video streaming is steered to video optimization enabler.

Tom's video streaming is not steered to video optimization enabler.

### 5.9.4	Post-conditions

John enjoys a better video experience than Tom.

### 5.9.5	Potential impacts or interactions with existing Services/Features

None.

### 5.9.6 	Potential requirements

An MNO should be able to take information (e.g. APN) of the destination network (i.e. a PDN or an internal IP network) of traffic flow into account when defining traffic steering policies.

## 5.10	Supporting third party owned (S)Gi-LAN service

### 5.10.1	Description

Faced with quick development of Mobile Internet, third party providers are allowed to deploy their own services in (S)Gi-LAN. MNOs can build an open ecosystem to quickly and flexibly introduce many (S)Gi-LAN service functions. Firstly, it can reduce the Time-to-Market of introducing a new service function. Secondly, it is beneficial to introduce more special service functions with better performance which MNOs may be not familiar with. Last but not least, many third party providers may develop richer types of service functions which MNOs have not found.

In this case, the accounting information associated with the IP traffic routed to the third party service functions in (S)Gi-LAN need to be collected by MNOs for the settlement between the operators and the third party.

### 5.10.2	Pre-conditions

Bob wants to watch video. The MNO allows the third party provider A to deploy its own video optimizer service in (S)Gi-LAN. The MNO deploys its own FW/NAT service in (S)Gi-LAN, and all the users’ traffic should go through the FW/NAT service function according to the MNO’s policy.

Scenario A:

Video service provider A subscribes the video optimizer service to the MNO, i.e., all traffic flow to Video service provider A should be optimized by video optimizer in (S)Gi-LAN.

Scenario B:

Bob subscribes the video optimizer service to the MNO, i.e., Bob’s traffic flow to any video website should be optimized by video optimizer in (S)Gi-LAN.

Scenario C:

Bob is a gold user. The MNO has a policy that the gold users’ video traffic should be optimized by video optimizer in (S)Gi-LAN.

Scenario D:

Bob wants to watch video with a VIP account.

Alice wants to watch video with a normal account.

Video service provider A makes a cooperation agreement with the MNO about that video service provider A can ask the MNO to steer their VIP user’s traffic to video optimizer in (S)Gi-LAN to improve users experience.

### 5.10.3	Service Flows

Figure 5.10-1:  supporting third party service in scenario A, B and C

Scenario A:

Bob begins to visit video service provider A’s website and watch videos. The MNO identifies that Bob’s traffic needs to go to video service provider A, and that video service provider A subscribes video optimizer service. According to the traffic steering policy, the traffic should be steered to video optimizer and FW/NAT service functions in (S)Gi-LAN.

Scenario B:

Bob begins to visit video service provider A’s website and watch videos. The MNO identifies that Bob’s traffic is a video traffic, and Bob subscribes the video optimizer service. According to the traffic steering policy, the traffic should be steered to video optimizer and FW/NAT service functions in (S)Gi-LAN.

Scenario C:

Bob begins to visit video service provider A’s website and watch videos. The MNO identifies that Bob is a gold user and Bob’s traffic is a video traffic. According to the traffic steering policy, the traffic should be steered to video optimizer and FW/NAT service functions in (S)Gi-LAN.

Figure 5.10-2:  supporting third party service in scenario D

Scenario D:

Bob begins to visit video service provider A’s website and watch videos with a VIP account. Video service provider A identifies a VIP user logs in and watches videos. Video service provider A asks the MNO to steer this VIP user’s video traffic to video optimizer service. As a result, the traffic should be steered to video optimizer and FW/NAT service functions in (S)Gi-LAN.

Alice begins to visit video service provider A’s website and watch videos with a normal account. Video service provider A identifies a normal user logs in and watches videos. As a result, the traffic should be steered to FW/NAT service functions in (S)Gi-LAN.

### 5.10.4	Post-conditions

Bob’s video traffic is steered to video optimizer and FW/NAT service functions in (S)Gi-LAN.

The MNO can collect the accounting information.

### 5.10.5	Potential Impacts or Interactions with Existing Services/Features

None.

### 5.10.6	Potential Service Requirements

The 3GPP core network shall be able to support the third party owned (S)Gi-LAN service functions.

The 3GPP core network shall be able to update the traffic steering policies based on the third party’s request.

The 3GPP core network shall be able to support accounting for the user traffic routed to the third party (S)Gi-LAN service function (e.g., collecting the accounting information).

# 6	Potential requirements

## 6.1	Requirements covered by other existing features

None identified.

## 6.2	Potential additional requirements

### 6.2.1	Generic potential requirements

[CPR-6.2.1-001]The 3GPP Core Network shall be able to define and modify traffic steering policies that are used to steer traffic in (S)Gi-LAN e.g. to improve e.g. the user's QoE, apply the bandwidth limitation and provide valued added services.

[CPR-6.2.1-002] Traffic steering policy shall be able to distinguish between upstream and downstream traffic.

[CPR-6.2.1-003] the 3GPP Core Network shall be able to define different traffic steering policies for a user's traffic on a per session basis (e.g. for applying parental control, anti-malware, DdoS protection, video optimization).

### 6.2.2	Additional consolidated requirements

[CPR-6.2.2-001] The 3GPP Core Network defines traffic steering policies based on one or more of the following information:

-	network operator's policies

-	user subscription (e.g. user's priority,  the status of optional subscriber services from the subscription data, based on service provider used, subscribed QoS)

-	user's current RAT

-	the network (RAN and CN) load status

-	application characteristics such as: application type (video, web browsing, IM, etc.), application protocol (HTTP, P2P, etc.), target address name (URL) and application provider (e.g. myTubeTM, etc.)

-	time

-	UE location

-	information (e.g. APN) of the destination network (i.e. a PDN or an internal IP network) of traffic flow

[CPR-6.2.2-002] The HPLMN shall be able to apply traffic steering policies for home routed traffic.

[CPR-6.2.2-003] Subject to HPLMN operator's policy, the HPLMN shall be able to transfer traffic steering policies to the VPLMN and the VPLMN shall be able to take these traffic steering policies into account and/or use its own policies in the case of local breakout for roaming users.

[CPR-6.2.2-004] The 3GPP core network shall be able to support the (S)Gi-LAN service functions owned by a third party.

[CPR-6.2.2-005] The 3GPP core network shall be able to apply traffic steering policies for the third party’s owned (S)Gi-LAN service functions.

[CPR-6.2.2-006] The 3GPP core network shall be able to collect accounting information to support accounting between operator and the third party service provider.

# 7	Conclusions

A number of use cases have been identified where the network operator can use 3GPP network information (user subscription, network status, …) to generate and adapt traffic steering policies to enable (S)Gi-LAN routing of user's traffic to the desired service.

The study has resulted in a set of different types of network information and a set of potential requirements as captured in the previous section. Mechanisms to address these requirements need to be specified. Mechanism how traffic flow is routed to different services is out of 3GPP scope.

It is recommended that the requirements identified in this TR are considered for the development of normative requirements.

Annex A:
Mapping of potential requirements

This annex maps potential requirements derived from the use cases to the requirement numbering of consolidated requirements of clause 6.

Table A-1: Mapping of potential requirements

| Potential additional Requirements |  | Consolidated requirement in clause 6 |
| --- | --- | --- |
| Clause 5.1 Traffic Classification and Steering based on Application Characteristics | - The 3GPP network should be able to recognize the application characteristics, such as application type (video, web browsing, IM, etc.), application protocol ( HTTP, P2P, etc.), target address name (URL) and application provider(My tube, etc.). - The 3GPP network should be able to steer traffic (upstream, downstream or both) per these application characteristics in order to provide value-added service to third-party application | [CPR-6.2.2-001] [CPR-6.2.1-002] |
| Clause 5.2 Traffic steering based on MNO's policies or user's subscription | - Based on end-user's priority and the status of optional subscriber services from the subscription data, the 3GPP network should allow the MNO to define different traffic steering policies. The MNO can then steer all/some parts of the user's traffic through different traffic paths based on the end- user's subscription data in order to offer differentiated services for different end-users, e.g. some high priority end-users could get service with no advertisements. - The 3GPP core network shall provide different traffic steering rules according to MNO's policies and user's subscription. The MNO can then steer all or some parts of user's traffic through different traffic paths involving security services in order to offer differentiated security services for different end-users or to comply with their policies. | [CPR-6.2.2-001] [CPR-6.2.1-003] |
| Clause 5.3 Traffic steering based on Network status | - Based on the user's current RAT and/or the load status of the network, the 3GPP network should allow the MNO to define the traffic steering policy. The MNO can then steer the user's traffic through different traffic routing. The operator should be able to change the user's traffic steering policy based on the network (RAN and CN) load change and/or RAT status change. | [CPR-6.2.2-001] |
| Clause 5.4 Traffic steering in MVNO scenario | - An Operator shall be able to define specific traffic steering policies to apply to users of an MVNO, based on MVNO's preferences. The MNO may apply different traffic steering policies for MVNO users than for MNO users. | [CPR-6.2.2-001] |
| Clause 5.5 Traffic Steering based on Time and/or Location | - A 3GPP network should be able to take time and/or location into account when defining traffic steering policies. | [CPR-6.2.2-001] |
| Clause 5.6 Traffic steering based on QoS information | - Based on the bearer QoS information, the 3GPP network shall be able to determine the traffic steering policy. | [CPR-6.2.2-001] |
| Clause 5.7 Traffic Steering in Roaming Cases | - The home PLMN should be able to apply traffic steering policies for home routed traffic or, subject to home PLMN operator's policy transfer traffic steering policies to the visited PLMN in the case of local breakout for roaming users. - For the local breakout traffic, the visited PLMN should be able to take the traffic steering policies from the home PLMN into account when passing traffic through (S)Gi-LAN in the visited network. | [CPR-6.2.2-002] [CPR-6.2.2-003] [CPR-6.2.2-003] |
| Clause 5.8 Dynamic modification of the traffic steering policy during the connection | - The 3GPP Network shall be able to modify traffic steering policy dynamically during an ongoing connection. | [CPR-6.2.1-001] |
| Clause 5.9 Traffic Steering based on destination network | - An MNO should be able to take information (e.g. APN) of the destination network (i.e. a PDN or an internal IP network) of traffic flow into account when defining traffic steering policies. | [CPR-6.2.2-001] |

Annex B:
Change history

| Change history |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TSG SA# | SA Doc. | SA1 Doc | Spec | CR | Rev | Rel | Cat | Subject/Comment | Old | New | WI |
| SP-65 | SP-140518 | - | 22.808 | - | - | Rel-13 | - | Raised to v.2.0.0 by MCC for approval as v.13.0.0 | 2.0.0 | 13.0.0 | FS_eFMSS |
| SP-69 | SP-150548 | S1-152682 | 22.808 | 1 | 2 | Rel-14 | B | eFMSS use case for supporting third party owned (S)Gi-LAN service | 13.0.0 | 14.0.0 | FS_eFMSS |
| SP-70 | SP-150768 | S1-154291 | 22.808 | 0002 | 1 | Rel-14 | B | Potential requirement and conclusion for eFMSS | 14.0.0 | 14.1.0 | FS_eFMSS |