3GPP TR 22.815 V14.0.0 (2014-12)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Study on Multimedia Broadcast Supplement for Public Warning System (MBSP)

(Release 14)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

PWS, Public Warning System, Broadcast

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2014, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	7

4	Use Cases	7

4.1	Geotargetted Warning	7

4.1.1	Description	7

4.1.2	 Pre-conditions	7

4.1.3	Service Flows	7

4.1.4	Post-conditions	7

4.1.5	Potential Requirements	7

4.2	Streaming Media	8

4.2.1	Description	8

4.2.2	 Pre-conditions	8

4.2.3	Service Flows	8

4.2.4	Post-conditions	8

4.2.5	Potential Requirements	8

4.3	Multimedia Download	8

4.3.1	Description	8

4.3.2	Pre-conditions	8

4.3.3	Service flows	9

4.3.4	Post-conditions	9

4.3.5 	Potential requirements	9

4.4	Initialisation of multimedia broadcast	9

4.4.1	Description	9

4.4.2	Pre-conditions	10

4.4.3	Service flows	10

4.4.4	Post-conditions	10

4.4.5 	Potential requirements	10

4.5	MBSP message broadcast area	10

4.5.1	Description	10

4.5.2	Pre-conditions	10

4.5.3	Service Flows	10

4.5.4	Post-conditions	11

4.5.5	Potential Impacts or Interactions with Existing Services/Features	11

4.5.6	Potential Requirements	11

4.6	Linked PWS multi-media messages	11

4.6.1	Description	11

4.6.2	Pre-conditions	11

4.6.3	Service Flows	11

4.6.4	Post-conditions	12

4.6.5	Potential Impacts or Interactions with Existing Services/Features	12

4.6.6	Potential Requirements	12

4.7	UE supporting both PWS Warning Notification and MBSP roams into a network supporting PWS Warning Notification only	12

4.7.1	Description	12

4.7.2	Pre-conditions	12

4.7.3	Service Flows	12

4.7.4	Post-conditions	13

4.7.5	Potential Impacts or Interactions with Existing Services/Features	13

4.7.6	Potential Requirements	13

4.8	Separate PWS Warning Notification and MBSP messages covering the same public warning	13

4.8.1	Description	13

4.8.2	Pre-conditions	13

4.8.3	Service Flows	13

4.8.4	Post-conditions	14

4.8.5	Potential Impacts or Interactions with Existing Services/Features	14

4.8.6	Potential Requirements	14

4.9	Multimedia PWS messages received by a UE in Limited Service state	14

4.9.1	Description	14

4.9.2	Pre-conditions	14

4.9.3	Service Flows	14

4.9.4	Post-conditions	14

4.9.5	Potential Impacts or Interactions with Existing Services/Features	14

4.9.6	Potential Requirements	14

4.10	UE restrictions when the network is overloaded	15

4.10.1	Description	15

4.10.2	Pre-conditions	15

4.10.3	Service Flows	15

4.10.4	Post-conditions	15

4.10.5	Potential Impacts or Interactions with Existing Services/Features	15

4.10.6	Potential Requirements	15

4.11	Multi-media PWS interaction with MBMS	15

4.11.1	Description	15

4.11.2	Pre-conditions	16

4.11.3	Service Flows	16

4.11.4	Post-conditions	16

4.11.5	Potential Impacts or Interactions with Existing Services/Features	17

4.11.6	Potential Requirements	17

4.12	UE and network combinations of PWS support	17

4.12.1	Description	17

4.12.2	Pre-conditions	17

4.12.3	Service Flows	18

4.12.4	Post-conditions	18

4.12.5	Potential Impacts or Interactions with Existing Services/Features	19

4.12.6	Potential Requirements	19

5	Considerations	20

5.1	Considerations on security	20

5.2	Considerations on displaying the message	20

5.3	Considerations on severity levels	20

6	Potential Requirements	20

6.1	General	20

7	Conclusion and Recommendations	21

Annex A:	Change history	22

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The work on Public Warning System (PWS) within the 3GPP as standardised since 3GPP Release 9 has focused on delivering text-based warning notifications of limited content to the public at a large scale. These are known as PWS Warning Notifications. This functionality is essential in providing a basic warning system with the elements as set out in clause 4.3 of TS 22.268 on PWS.

It is noted within PWS that content which might trigger an increase in network load should be avoided as the network likely to be under stress, and capacity diminished depending upon the nature of the emergency. Such a storm of traffic may be hazardous to the health of the network, or may be blocked by the network in order to preserve function or prioritise other traffic.

Experience with the current PWS has led some public safety alerting agencies to have difficulties of trying to include all the essential information needed to inform the public into the current PWS Warning Notification limited size. Some of this essential information includes maps with public safety mark-up, images of missing persons, live news video broadcasts, evacuation information, latest safety briefings, weather warnings, emergency shelter locations and assembly points etc.

This TR studies delivering more extensive multimedia warning notification content than is currently supported in PWS and investigating both the broadcasting of more extensive multimedia content for a PWS and the mechanisms by which users would be able to receive and view this multimedia content.

# 1	Scope

This TR considers enhancements to 3GPP PWS via the addition of multimedia broadcast capability. The use cases for this capability are documented and derived requirements are gathered and consolidated. This TR does not cover use cases or potential requirements for specific PWS based systems (i.e. ETWS, CMAS, EU-ALERT, and Korean Public Alert System).

This TR covers:

-	Scenarios and use cases to provide multi-media information supplemental to PWS;

-	UE awareness of, and ability to receive, multi-media information via PWS;

-	Handling of UEs in limited service mode;

-	Priority of multimedia information

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.268: "Public Warning System (PWS) requirements"

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

MBSP:	Multimedia Broadcast to Supplement PWS

MBSP message:	The message sent from an MNO to a UE containing multimedia component(s)

mPWS message:	The multimedia content sent by a Public Safety agency to a MNO in order for an MNO to create a MBSP message

Public Safety agency:	Any agency that is responsible for the protection of the safety of the general public from significant danger, harm, or damage. Examples include Law enforcement agencies, firefighters, and first responders

PWS message:	The text content sent by a Public Safety agency to a MNO in order for an MNO to create a PWS Warning Notification

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AM	Amplitude Modulation

DAB	Digital Audio Broadcast

FM	Frequency Modulation

MNO	Mobile Network Operator

# 4	Use Cases

## 4.1	Geotargetted Warning

### 4.1.1	Description

This use case describes a user within a PWS-enabled network receiving a PWS Warning Notification, supplemented by a separate transmission of geographical data in the form of a map overlay.

### 4.1.2		Pre-conditions

The MNOs network supports PWS.

The MNOs network supports a mechanism for wide distribution of multimedia content to a large population without overloading the network (e.g. via broadcast).

### 4.1.3	Service Flows

Scenario 1: Flood

A flood occurs and is monitored by the meteorological service. Due to the severity, the decision is made to notify all users within the potential reach of the flood waters. This area is significantly larger than the currently affected area due to the geography of the area.

A PWS message and an mPWS message are created by the Public Safety agency whereby the current reach of the flood waters and the projected reach of the flood waters are plotted onto a map overlay.

A PWS Warning Notification and an MBSP message are sent out to users within a Notification Area defined by the Public Safety agency. As part of the presentation to the user, the map overlay may be presented.

### 4.1.4	Post-conditions

Scenario 1: Flood

All users within the Notification Area receive the PWS Warning Notification warning them about the flooding. Users who chose to view the map overlay are given extra information about the current and future flood area. Users who are notified by PWS that there is a flood but who can see from the map that they are in no immediate or projected danger can be more assured of their safety and gauge their preparations accordingly.

### 4.1.5	Potential Requirements

Current PWS requirements in TS 22.268 [2] already require geo-targeted Warning Notifications. No new requirements have been identified in this use case.

## 4.2	Streaming Media

### 4.2.1	Description

This use case describes a user within a PWS-enabled network receiving a PWS Warning Notification supplemented by an MBSP message in the form of a live continuous audio stream.

### 4.2.2		Pre-conditions

The MNOs network supports PWS.

The MNOs network supports a mechanism for wide distribution of multimedia content to a large population without overloading the network (e.g. via broadcast).

### 4.2.3	Service Flows

Scenario 1: Chemical Fire

A chemical fire is underway in a building near a residential area in the UK.

According to the UK Government Emergency Communication Policy, local radio streaming broadcasts are started and broadcast on local FM, AM and DAB radio stations. In addition, these broadcasts are fed to MNOs as a PWS message and an mPWS message for distribution in the area local to the chemical fire.

A PWS Warning Notification and an MBSP message are sent out to users within a Notification Area defined by the emergency services. As part of the presentation to the user, the user can listen to the live audio transmission for up to date information in the same way as radio listeners can do.

### 4.2.4	Post-conditions

Scenario 1: Chemical Fire

All users within the Notification Area receive the PWS Warning Notification warning them about the fire and giving basic information. Users receive the MBSP message and who chose to listen to the streaming audio are given extra information about evacuations, regrouping areas, information about air-quality safety, and other information.

### 4.2.5	Potential Requirements

[PR-4.2.5-001] Current PWS requirements in TS 22.268 [2] require support of text messages. The PWS may be extended to support multi-media messages as well.

## 4.3	Multimedia Download

### 4.3.1	Description

This use case describes a user within a PWS-enabled network receiving a PWS Warning Notification supplemented by an MBSP message in the form of downloaded multimedia.

### 4.3.2	Pre-conditions

The MNOs network supports PWS.

The MNOs network supports a mechanism for wide distribution of multimedia content to a large population without overloading the network (e.g. via broadcast)

### 4.3.3	Service flows

Scenario 1: Abducted Child

A child has been abducted and the emergency services have been informed. The emergency services have decided that due to information gathered, it may be useful to inform users in an area of this abduction and to enable local residents to be vigilant.

A PWS message and an mPWS message are created by the Public Safety agency whereby details of the abductee (photo, age, height, video, context etc.) are packaged together.

A PWS Warning Notification and MBSP message are sent out to users within a Notification Area defined by the emergency services. As part of the presentation to the user, the multimedia components may be presented.

Scenario 2: Missing persons

An earthquake has occurred some time ago. Communication services are limited and patchy. As part of the response to the disaster, a list of missing persons has been compiled.

A PWS message and an mPWS message are created by the Public Safety agency whereby details of the missing people (photo, age, height, video, context etc.) are packaged together. This MBSP message is then broadcast in an object carousel. As the details of the missing people changes, this package is modified on the server.

A PWS Warning Notification is sent out to users within a Notification Area defined by the emergency services or missing person’s bureau. An MBSP message is also sent out via broadcast.

As part of the presentation to the user, the user views the initial package of missing people’s data.

Later (before the end of the broadcast), the UE is notified that the contents of the carousel have changed and the UE downloads the MBSP message from the broadcast.

As part of the presentation to the user, the user views an updated version of the missing people’s data.

### 4.3.4	Post-conditions

Scenario 1: Abducted Child

All users within the Notification Area receive the PWS Warning Notification and MBSP message warning them about the abduction. Users who chose to view the multimedia content are given extra information about the abductee and context, and are able to be additionally vigilant for the abductee.

Scenario 2: Missing persons

All users within the Notification Area receive the PWS Warning Notification and MBSP message informing them about the missing people. Users who chose to view the multimedia content are given extra information about the missing persons and are able to assist in the recovery effort more effectively.

### 4.3.5 	Potential requirements

[PR-4.3.5-001] Current PWS requirements in TS 22.268 [2] require support of text messages. The PWS may be extended to support multi-media messages as well.

## 4.4	Initialisation of multimedia broadcast

### 4.4.1	Description

This use case describes a user within a PWS-enabled network receiving a PWS Warning Notification, the automatic triggering for a UE to receive an MBSP message broadcast, and the presentation of this information to the user

### 4.4.2	Pre-conditions

The MNOs network supports PWS.

The MNOs network supports a mechanism for wide distribution of multimedia content to a large population without overloading the network (e.g. via broadcast).

### 4.4.3	Service flows

Alice’s UE receives a PWS Warning Notification notifying her of a nearby incident.

Alice’s UE is notified of an MBSP message for this notification and automatically fetches the content from the broadcast.

Alice’s UE alerts Alice that an MBSP message was received and presents the multimedia components when requested by Alice.

### 4.4.4	Post-conditions

Alice receives the multimedia content without having to interact with her device in a complicated manner.

### 4.4.5 	Potential requirements

[PR-4.4.5-001] The MBSP system shall provide a mechanism by which reception of an MBSP message can be initiated without user interaction.

## 4.5	MBSP message broadcast area

### 4.5.1	Description

This use case describes the kind of geographic control required for defining the area where UE users will be notified of and receive MBSP messages.

### 4.5.2	Pre-conditions

The Public Safety agency creating a PWS message and an mPWS message can define a geographic area that they want the message to target.

The MNO has the ability to map any arbitrary geographic shape descriptor into a set of cell sectors that approximates the geographic coverage area by using their network coverage maps

### 4.5.3	Service Flows

1. The Public Safety agency currently dealing with an emergency identifies the need to notify the citizens within their jurisdiction about the emergency. The public warning includes pictures.

2. The Public Safety agency transmits the PWS message and mPWS message to the MNO, including describing the geographic shape of their jurisdiction.

3. The MNO maps the transmitted geographic shape description to a set of cell sectors providing coverage within the Public Safety agency’s jurisdiction.

4. The MNO transmits the PWS Warning Notification and MBSP message in at least the selected set cell sectors.

5. Merely by being located within the coverage area of the mapped cell sectors, UEs which support MBSP messages become aware of this MBSP message, alert the user and make the MBSP message available to the user.

6. Merely by being located outside the coverage area of the mapped cells sectors, UEs which support MBSP messages will not become aware of this MBSP message, will not alert the user and will not make the MBSP message available to the user.

### 4.5.4	Post-conditions

Users within the mapped cell sector coverage area will be notified of the MBSP message and are able to view it.

Note there are other similar MBSP message scenarios besides the geographic jurisdictions this use case can apply to, including:

-	Different MBSP messages with adjacent geographic boundaries

-	The need to distribute a MBSP message only to a geographic area affected by the emergency while ensuring that it is not distributed outside the affected area to avoid potential significant confusion and additional public safety issues.

### 4.5.5	Potential Impacts or Interactions with Existing Services/Features

None identified.

### 4.5.6	Potential Requirements

[PR-4.5.6-001] UEs located within the coverage area of the MBSP message selected set of cell sectors should detect and alert the user to a MBSP message and allow the user to view the MBSP message.

[PR-4.5.6-002] UEs located outside the coverage area of the MBSP message selected set of cell sectors shall not alert the user to any MBSP message and not allow the user to view the MBSP message.

[PR-4.5.6-003] Determination of whether a UE is inside or outside the mapped cell sector coverage area shall be based on information broadcast (or not broadcast) by the PLMN. The UE shall assume that if it receives all the components of a MBSP message (and the MBSP message itself) it is inside the mapped cell sector coverage area.

## 4.6	Linked PWS multi-media messages

### 4.6.1	Description

The current deployed PWS only supports PWS Warning Notifications, during a transition period until MBSP message capability becomes ubiquitous end-to-end, both PWS Warning Notification and MBSP message broadcast will be needed. Many of these messages will need to be linked and their broadcast coordinated.

### 4.6.2	Pre-conditions

The Public Safety agency creating PWS messages will create both PWS Warning Notification content and multi-media content. It is assumed that during the transition period that the same text content is used by UEs to inform the user regardless of their MBSP capability

The MNO supports both the PWS Warning Notification and MBSP message capabilities. The MNO’s subscriber base is in transition from having UEs supporting the PWS Warning Notification only to UEs supporting both PWS Warning Notification and MBSP messages.

### 4.6.3	Service Flows

1. The Public Safety agency creates a PWS message and an mPWS message with both text and other multi-media components and transmits the PWS message and mPWS message to the MNO.

2. The MNO broadcasts the PWS Warning Notification and the MBSP message.

3. UEs which only support PWS Warning Notifications notify the user of the presence of a PWS Warning Notification and display it when requested by the user. The MBSP message is ignored.

4. UEs which support both PWS Warning Notification and MBSP messages will notify the user of the presence of all the supported components (PWS Warning Notification or MBSP message) and display it when requested by the user.

### 4.6.4	Post-conditions

The UEs are able to identify the supported PWS message and mPWS message content and display it when requested. The delivery of all the PWS message and mPWS message components is coordinated so the user is able to view the entire PWS message and mPWS message when notified.

### 4.6.5	Potential Impacts or Interactions with Existing Services/Features

This use case demonstrates some of the backwards compatibility considerations with the existing PWS Warning Notification capabilities.

### 4.6.6	Potential Requirements

[PR-4.6.6-001] The addition of MBSP messaging capability to a PLMN shall not require changes to existing UEs supporting PWS Warning Notification only.

[PR-4.6.6-002] The MNO shall be able to indicate to the UE PWS Warning Notifications and MBSP messages linked together covering the same public warning.

[PR-4.6.6-003] The UE which supports MBSP messages shall be able to ignore the linked PWS Warning Notification .

## 4.7	UE supporting both PWS Warning Notification and MBSP roams into a network supporting PWS Warning Notification only

### 4.7.1	Description

The current deployed PWS only supports PWS Warning Notifications, during a transition period until MBSP message capability becomes ubiquitous end-to-end on a world-wide basis, some networks may only broadcast PWS Warning Notifications and not MBSP messages.

### 4.7.2	Pre-conditions

The MNO supports only the existing PWS Warning Notification, not MBSP message capabilities.

UEs which support both PWS Warning Notifications and MBSP messages are used in this PLMN network either through roaming or a purchase by a subscriber of this MNO.

### 4.7.3	Service Flows

1. The Public Safety agency creates a PWS message.

2. The MNO broadcasts PWS Warning Notification.

3. The UE detects that only a PWS Warning Notification is available.

4. The UE will notify the user of the presence of a PWS Warning Notification and display it when requested by the user.

### 4.7.4	Post-conditions

The UEs are able to identify that only PWS Warning Notifications are broadcast by the serving PLMN, notify the user of the availability of a PWS Warning Notification and display the PWS Warning Notification when requested.

### 4.7.5	Potential Impacts or Interactions with Existing Services/Features

This use case demonstrates some of the backwards compatibility considerations with the existing PWS Warning Notification capabilities.

### 4.7.6	Potential Requirements

[PR-4.7.6-001] UEs supporting both PWS Warning Notifications and MBSP messages shall be able to determine whether a PLMN will broadcast existing PWS Warning Notifications also as MBSP messages.

[PR-4.7.6-002] UEs supporting both PWS Warning Notifications and MBSP messages operating in a PLMN broadcasting only PWS Warning Notifications shall behave in the same fashion as a UE only supporting PWS Warning Notifications.

## 4.8	Separate PWS Warning Notification and MBSP messages covering the same public warning

### 4.8.1	Description

The current deployed PWS only supports PWS Warning Notifications, during a transition period until MBSP message capability becomes ubiquitous end-to-end, both PWS Warning Notification and MBSP message broadcast will be needed. There may be some PWS and mPWS messages which are intended to alert the public regarding the same public warning, but the text content portion of the PWS and mPWS message can’t be shared between PWS Warning Notification and MBSP messages.

### 4.8.2	Pre-conditions

The Public Safety agency originating the PWS messages covering a single public warning will create separate messages for the PWS Warning Notification as well as for MBSP messages. The nature of the MBSP multimedia components is such that the text component can’t be shared between the PWS Warning Notification and MBSP messages.

The MNO supports both the PWS Warning Notification and MBSP message capabilities.

UEs only supporting the PWS Warning Notification are operating in the alerting area.

UEs supporting both PWS Warning Notification and MBSP messages are operating in the alerting area.

### 4.8.3	Service Flows

1. The Public Safety agency creates a PWS message and an mPWS message describing the single public warning. The PWS message and mPWS message include different text.

2. The MNO broadcasts the PWS Warning Notification.

3. The MNO broadcasts the MBSP message.

4. UEs which only support PWS Warning Notifications notify the user of the presence of a PWS text Warning Notification and display it when requested by the user. The MBSP message is ignored by the UE.

5. UEs which support both PWS Warning Notifications and MBSP messages will notify the user of the presence of an MBSP message and display it (including the text component of the MBSP message) when requested. The related PWS Warning Notification is ignored.

### 4.8.4	Post-conditions

The UE supporting only PWS Warning Notification will only handle the PWS Warning Notification.

The UE supporting both PWS Warning Notification and MBSP messages will only handle the MBSP message (including the text component in the MBSP message) and ignore the PWS Warning Notification.

### 4.8.5	Potential Impacts or Interactions with Existing Services/Features

This use case demonstrates some of the backwards compatibility considerations with the existing PWS Warning Notification capabilities.

### 4.8.6	Potential Requirements

[PR-4.8.6-001] UEs supporting both PWS Warning Notification and MBSP messages shall notify the user about the availability of the MBSP message and display it when requested by the user. Any PWS Warning Notifications relating to the same public warning shall be ignored.

## 4.9	Multimedia PWS messages received by a UE in Limited Service state

### 4.9.1	Description

This use case describes a UE which is in Limited Service state.

### 4.9.2	Pre-conditions

The MNO network supports PWS and MBSP.

The UE is in Limited Service state.

### 4.9.3	Service Flows

The UE receives a PWS Warning Notification notifying the user of a nearby incident.

The UE is notified of supplemental multimedia content for this notification and automatically selects the MBSP message from the broadcast.

If the MBSP message was received with errors preventing the content from being displayed, the UE needs to wait for a repetition of the notification broadcast.

The UE alerts the user that a MBSP message was received, displays the text component immediately and presents the multimedia components when requested.

### 4.9.4	Post-conditions

The UE displays the multimedia content if it was received.

### 4.9.5	Potential Impacts or Interactions with Existing Services/Features

None identified.

### 4.9.6	Potential Requirements

[PR-4.9.6-001] Subject to regulatory requirements, the UE shall be able to receive PWS multi-media messages while in a limited service state.

[PR-4.9.6-002] The UE shall detect duplicate multi-media message broadcasts and ignore duplicate multi-media messages.

## 4.10	UE restrictions when the network is overloaded

### 4.10.1	Description

One aspect of PWS is the distribution of emergency messages during large scale emergency scenarios, even when the 3GPP network is overloaded or operating in a stressed condition. The UE’s in obtaining MBSP messages must not contribute additional traffic or network load in response to the messages.

### 4.10.2	Pre-conditions

A wide spread emergency condition arises and the 3GPP network is stressed from the exceptional call traffic relating to the emergency.

UE A and UE B support MBSP and are in the area affected by the emergency.

### 4.10.3	Service Flows

1. A wide spread emergency condition arises and the responsible public safety agency issues an MBSP message.

2. UE A and B are both monitoring for MBSP message broadcast and are alerted that a MBSP message is being broadcast.

3. UE A successfully receives the complete MBSP message and notifies the user.

4. UE A does not interact with the 3GPP network with any response to the MBSP message.

5. UE B receives an incomplete MBSP message and does not alert the user.

6. UE B does not interact with the 3GPP network with any response to the incomplete MBSP message (such as a retransmission or media recovery request).

7. UE B waits for the MBSP message to be re-broadcast.

### 4.10.4	Post-conditions

UE A and B do not create any additional network traffic in response to successfully or unsuccessfully receiving an MBSP message.

UE A and B resume monitoring the 3GPP network for additional or updated MBSP messages.

### 4.10.5	Potential Impacts or Interactions with Existing Services/Features

This use case demonstrates the need to minimize network traffic in response to receiving an MBSP message when the network is operating normally or in an overload condition.

### 4.10.6	Potential Requirements

[PR-4.10.6-001] The UE supporting MBSP shall not initiate automatically any network traffic in response to successful or unsuccessful reception of an MBSP message.

## 4.11	Multi-media PWS interaction with MBMS

### 4.11.1	Description

A UE, while engaged in a MBMS session still needs to handle an MBSP message and then potentially resume the MBMS session.

### 4.11.2	Pre-conditions

UE A supports both MBMS and MBSP.

The 3GPP network supports both MBMS and MBSP.

UE A is engaged in a MBMS session broadcasting a live sports event.

### 4.11.3	Service Flows

1. A wide spread emergency condition arises and the responsible public safety agency issues an MBSP message. The Public Safety agency indicates that this message should interrupt any ongoing multi-media applications running on the UE.

2. UE A is presenting the video and audio media from the MBMS broadcast of the sports event to the user.

3. The 3GPP network broadcasts the MBSP message

4. The UE A is monitoring for MBSP message broadcasts. Based on the detection of the MBSP message alert, the UE A prepares to receive the broadcast MBSP message content.

5. The UE A successfully receives the complete MBSP message content.

6. The UE A determines from the MBSP message that the alerting urgency requires interrupting the current multi-media session presentation.

7. The UE A interrupts the presentation of the live sports event audio and video media and notify the user about the MBSP message.

8. UE A suspends the presentation of the live sports event audio and video media presentation and presents the MBSP message contents.

9. The UE A user views the MBSP message.

10. The UE A resumes presentation of the live sports event audio and video media presentation.

11. The emergency condition mitigation reduces the threat to the citizens in the area and the Public Safety agency decides to inform the affected citizens that the threat level is downgraded and for anybody affected, where to assemble for temporary shelter. The Public Safety agency indicates that this message does not need to interrupt any ongoing multi-media applications running on the UE.

12. The 3GPP network broadcasts the MBSP message

13. The UE A is monitoring for MBSP message broadcasts. Based on the detection of the MBSP message alert, the UE A prepares to receive the broadcast MBSP message content.

14. The UE A successfully receives the complete MBSP message content.

15. The UE A determines from the MBSP message that the alerting urgency does not require interrupting the current multi-media session presentation.

16. The live sports event ends and the user exits from the application which presented the live sports even audio and video media.

17. UE A informs the user about the recent MBSP message contents availability.

18. The UE A user views the MBSP message.

### 4.11.4	Post-conditions

While engaged in MBMS sessions UE A detected and received MBSP messages.

UE A alerted its respective user about the MBSP message

UE A suspended the presentation of the ongoing MBMS sessions based on MBSP message information about interrupting existing UE multi-media applications.

UE A retained the MBSP message which indicated that interrupting active UE multi-media MBMS session is not required and alerted the user when the multi-media presentation was finished.

### 4.11.5	Potential Impacts or Interactions with Existing Services/Features

This use case demonstrates that while engaged in MBMS, a UE still needs to be monitoring the 3GPP network for MBSP message broadcasts.

This use case also demonstrates that presentation of MBMS content to the user must be interruptible to allow presentation of received multi-media PWS message content when indicated in the MBSP message

### 4.11.6	Potential Requirements

[PR-4.11.6-001] The UE supporting MBSP shall continue to monitor for MBSP message broadcasts while engaged in MBMS sessions.

[PR-4.11.6-002] The UE supporting MBSP shall alert the user when an MBSP message was successfully received and is not a duplicate of a previously alerted MBSP message while engaged in MBMS sessions either:

-	Immediately when the MBSP message indicates that current MBMS session presentation must be interrupted.

-	At the completion of current MBMS session presentation when the MBSP message indicates that current MBMS session can continue to completion.

Note: 	Regional regulations may establish additional policy requirements regarding the relative priorities of MBSP message alerting and presentation when different types of non-MBSP multi-media applications and presentations are taking place on the UE.

## 4.12	UE and network combinations of PWS support

### 4.12.1	Description

Based on regional regulatory requirements and extended by operator policy, there are likely to be a wide range of combination of support for current text only PWS and MBSP. This use case describes the possible combinations and the expected results.

### 4.12.2	Pre-conditions

Network A in Xanadu supports only existing text PWS.

Network B in Atlantis supports both exiting text PWS and MBSP.

Network C in Shangri-La supports only MBSP.

The Atlantis Park Police will generate only text PWS Warning Notifications.

The Atlantis Shore Patrol will generate separate text only PWS Warning Notifications and MBSP messages containing text different from the text only PWS Warning Notification.

The Atlantis Crossing Guards will generate a single MBSP message where the text portion applies both standalone (text only PWS Warning Notification) and in conjunction with the MBSP message.

The Atlantis Border Control will generate only a MBSP message where the text content will not be meaningful by itself.

There is a cruise boat with 3 users on board travelling to Xanadu, Atlantis, and Shangri-La.

On board this cruise boat there are the following UEs:

UE X only supports existing text PWS.

UE Y supports both existing text and multi-media PWS.

UE Z supports only MBSP (not the existing text PWS).

### 4.12.3	Service Flows

1. The cruise boat arrives in Xanadu, and a plague of frogs threatens the city.

2. Network A broadcasts a PWS Warning Notification about the frogs in the form of an existing text only PWS.

3. The cruise boat moves on to Atlantis and a plague of wild animals threatens the city.

4. The Atlantis Park Police generate a text only PWS Warning Notification warning to not pet any animals in the park.

5. The Atlantis Shore Patrol generate a text only PWS Warning Notification and a separate MBSP message with a text component warning citizens to not go swimming due to a shark infestation.

6. The Atlantis Crossing Guards generate a single MBSP message with a text component that also applies by itself warning citizens about wild animals jaywalking.

7. The Atlantis Border Control generate a single MBSP message with a text component clarifying an image of which wild animals threatening Atlantis are illegal to export.

8. Network B broadcasts PWS Warning Notifications relating to the wild animals from all four Atlantis Public Safety agencies.

9. The cruise boat moves to Shangri-La and a plague of locusts threatens the city.

10. Network C broadcasts a PWS Warning Notification about the locusts in the form of a MBSP message.

### 4.12.4	Post-conditions

Table 1 describes the network actions and results for the various combinations.

Table 1: Network and UE PWS support combinations

| Operator | Public Safety Agency | PWS Warning Notification initiated by Public Safety Agency | UE X supporting existing text PWS only | UE Y supporting both existing text only PWS and MBSP | UE Z supporting MBSP only |
| --- | --- | --- | --- | --- | --- |
| A |  | A single current text only PWS Warning Notification | Receives the current text only PWS Warning Notification | Receives the current text only PWS Warning Notification | Does not receive PWS Warning Notification |
| B | Park Police | A single current text only PWS Warning Notification | Receives the current text only PWS Warning Notification | Receive a PWS Warning Notification (Note 3) | Receives the MBSP message (Note 4) |
|  | Shore Patrol | A current text only PWS Warning Notification plus a MBSP message with a different text component | Receives the current text only PWS Warning Notification | Receives the MBSP message (Note 5) | Receives the MBSP message |
|  | Crossing Guards | A single combined MBSP message (Note 1) | Receives the current text only PWS Warning Notification (Note 1) | Receives the MBSP message (Note 5) | Receives the MBSP message |
|  | Border Control | A single MBSP message (Note 2) | Does not receive PWS Warning Notification | Receives the MBSP message | Receives the MBSP message |
| C |  | A single MBSP message | Does not receive PWS Warning Notification | Receives the MBSP message | Receives the MBSP message |

Note 1: 	This assumes that the 3GPP network extracts the text component from the MBSP message using it to create an additional current text only PWS Warning Notification.

Note 2: 	This assumes that the 3GPP network can’t extract the text component from the MBSP message since the text component is meaningless by itself.

Note 3: 	The 3GPP network can either send a PWS Warning Notification in the current text only form or can create an additional multi-media only text message using the same text component in the originated message.

Note 4: 	This will only apply if the 3GPP network creates an additional MBSP message using the same text component in the originated message. Otherwise UE Z will not receive the PWS Warning Notification.

Note 5: 	UE Y will need to recognize that both a current text only PWS Warning Notification and a MBSP refer to the same Public Safety warning

### 4.12.5	Potential Impacts or Interactions with Existing Services/Features

The only interaction identified is that there may be a need to identify text components of PWS Warning Notifications for use in a 3GPP network created current text only PWS Warning Notification.

### 4.12.6	Potential Requirements

[PR-4.12.6-001] Dependent on regional regulatory requirements and extended by operator policy the following behaviours shall be supported by 3GPP networks and UEs when deployed in corresponding combinations listed in Table 2.

Table 2: 3GPP network and UE required behaviour combinations based on PWS support.

| 3GPP network PWS support | Originated PWS Warning Notification(s) for a single public warning | UE supporting existing text PWS only | UE supporting both existing text only PWS and MBSP | UE supporting MBSP only |
| --- | --- | --- | --- | --- |
| Current text only PWS supported | A single current text only PWS Warning Notification | Receives the current text only PWS Warning Notification | Receives the current text only PWS Warning Notification | Does not receive PWS Warning Notification |
| Both current text only PWS and MBSP supported | A single current text only PWS Warning Notification | Receives the current text only PWS Warning Notification | Receive a PWS Warning Notification (Note 3) | Receives the MBSP message (Note 4) |
|  | A current text only PWS Warning Notification plus a MBSP message with a different text component | Receives the current text only PWS Warning Notification | Receives the MBSP message (Note 5) | Receives the MBSP message |
|  | A single combined MBSP message (Note 1) | Receives the current text only PWS Warning Notification (Note 1) | Receives the MBSP message (Note 5) | Receives the MBSP message |
|  | A single MBSP message (Note 2) | Does not receive PWS Warning Notification | Receives the MBSP message | Receives the MBSP message |
| Only MBSP supported | A single MBSP message | Does not receive PWS Warning Notification | Receives the MBSP message | Receives the MBSP message |

Note 1: 	This assumes that the 3GPP network extracts the text component from the MBSP message using it to create an additional current text only PWS Warning Notification.

Note 2: 	This assumes that the 3GPP network can’t extract the text component from the MBSP message since the text component is meaningless by itself.

Note 3: 	The 3GPP network can either send a PWS Warning Notification in the current text only form or can create an additional multi-media only text message using the same text component in the originated message.

Note 4:	 This will only apply if the 3GPP network creates an additional MBSP message using the same text component in the originated message.

Note 5:	 The UE will need to recognize that both a current text only PWS Warning Notification and a MBSP refer to the same Public Safety warning.

# 5	Considerations

## 5.1	Considerations on security

No additional security requirements beyond those in clause 4.8 in TS 22.268 [2] are identified.

## 5.2	Considerations on displaying the message

As per TS 22.268 [2], the PWS Warning Notification is presented with a specific ring-tone and a specific vibration cadence. This would apply to multi-media PWS as well.

## 5.3	Considerations on severity levels

As per TS 22.268 [2], the PWS Warning Notification is associated with a severity level. This would apply to MBSP as well and it is assumed that the same severity level will be assigned for linked PWS Warning Notification messages and MBSP messages.

# 6	Potential Requirements

## 6.1	General

[CPR-6.1-001] The requirements for PWS from TS 22.268 [2] also apply to the Multimedia Broadcast Supplement of PWS with the exception of the following:

-	The ETWS primary notification does not apply to MBSP.

[CPR-6.1-002] If an MNO provides text based PWS Warning Notifications as well as MBSP, then it is expected that the granularity will be the same for both types of services.

- 6.2	Consolidated Requirements on the PLMN

[CPR-6.2-001] The PWS may be extended to support MBSP messages as well.

[CPR-6.2-002] The addition of MBSP messaging capability to a PLMN should not require changes to existing UEs supporting PWS Warning Notification only.

[CPR-6.2-003] The MNO should be able to indicate to the UE which PWS Warning Notifications and MBSP messages are linked and are covering the same public warning.

- 6.3	Consolidated Requirements on the UE

[CPR-6.3-001] The PWS-UE may be extended to support MBSP messages as well.

[CPR-6.3-002] The MBSP system shall provide a mechanism by which reception of an MBSP message can be initiated without user interaction.

[CPR-6.3-003] UEs located within the coverage area of the MBSP message selected set of cell sectors should detect and alert the user to a MBSP message and allow the user to view the MBSP message.

[CPR-6.3-004] UEs located outside the coverage area of the MBSP message selected set of cell sectors shall not alert the user to any MBSP message and not allow the user to view the MBSP message.

# 7	Conclusion and Recommendations

The study has led to the conclusion that MBSP, a multimedia broadcast supplement for PWS, has no impact on the current UEs supporting PWS because they are unable to receive the MBPS messages and will ignore them. Since MBSP-capable UEs can ignore the PWS Warning Notification linked to the MBSP message, this PWS Warning Notification will not be displayed.

Since no regulatory requirements for MBSP exist yet, there may be a number of additional issues and use cases introduced by the regulatory requirements which have not been addressed in this study.

It is recommended that once sufficient regulatory input has been received, normative work should be undertaken.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2014-01 | SA1#65 | S1-140113 |  |  | Skeleton TR | - | 0.1.0 |
| 2014-01 | SA1#65 | S1-140279 |  |  | Added Introduction, scope, use cases on Geotargetted Warning, Streaming Media, and Download Media. | 0.1.0 | 0.2.0 |
| 2014-05 | SA1#66 | S1-141519 |  |  | Added use cases on initialisation of multimedia broadcast, PWS multi-media message broadcast area, linked PWS multi-media messages, UE supporting both text and multi-media PWS roams into a network supporting text PWS only, and separate text only and multi-media PWS messages covering the same public warning | 0.2.0 | 0.3.0 |
| 2014-08 | SA1#67 | S1-143422 |  |  | Added use cases on Multimedia PWS in limited service state, UE restrictions when network is overloaded or stressed, multi-media PWS interaction with MBMS, and combinations of current text PWS and multi-media PWS. Considerations were added. Definitions and terminology was aligned, plus editorial corrections. | 0.3.0 | 0.4.0 |
| 2014-09 | SA#65 | SP-140519 | - | - | MCC clean-up for presentation to SA#65 | 0.4.0 | 1.0.0 |
| 2014-11 | SA1#68 | S1-144623 | - | - | Added consolidated requirements and conclusions. Editorial corrections | 1.0.0 | 1.1.0 |
| 2014-12 | SA#66 | - | - | - | Raised by MCC to v.2.0.0 for presentation to SA#66 | 1.1.0 | 2.0.0 |
| 2014-12 | SA#66 | SP-140757 | - | - | Raised by MCC to v.14.0.0 following SA's approval. Requirements now numbered. | 2.0.0 | 14.0.0 |