3GPP TR 28.800 V0.5.0 (2017-02)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Study on management and orchestration architecture of next generation networks and services

(Release 14)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Next generation, Management, Orchestration, Network, Service

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2016, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	Management architecture introduction	6

5	Use cases related to management and orchestration architecture	7

5.1	Management and orchestration of networks containing non-5G NE and 5G NE	7

5.1.1	Introduction	7

5.1.2	Pre-conditions	7

5.1.3	Description	7

5.1.4	Post-conditions	7

5.2	Use case of management and orchestration architecture for network slicing	8

5.2.1	Introduction	8

5.2.2	Pre-conditions	8

5.2.3	Description	8

5.2.4	Post-conditions	8

6	Potential requirements on management and orchestration architecture	8

7	Potential management and orchestration architecture options	8

7.1	Option 1: CCC	8

7.1.1	Description	8

7.1.2	Potential interaction with network architecture (if needed)	8

8	Conclusions	8

9	Recommendations	9

Annex <A>: <Annex title>	10

Annex <X>: Change history	11

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The Study on management and orchestration architecture of next generation networks and services is to align the next generation management architecture with consideration of support the next generation management functions. Studying the management and orchestration architecture will evaluate whether the existing management mechanisms can be applicable or need to be enhanced to support the next generation management and orchestration features.

# 1	Scope

The study investigates and makes recommendations on management and orchestration architecture of next generation networks and services which includes:

- The terminology associated with management and orchestration of next generation networks and services.

- The use cases on the management and orchestration architecture from the output of other next generation management functional studies (e.g. management and orchestration of network slicing, management of next generation features, service management and orchestration, etc.).

- The use cases that may impact the management and orchestration architecture which are not addressed by functional studies.

- The management and orchestration architecture to support network operational features such as real-time, on demand, automation etc. as evolution from LTE management.

- The management and orchestration architecture to support vertical applications (e.g. management architecture support to the V2X, critical communications etc.) in the next generation network based on the other management functional studies.

- The management and orchestration architecture to support the scenario in which the applications are hosted close to the access network based on the other management functional studies.

- The management and orchestration architecture to support management and orchestration aspect of E2E user services in the next generation network based on the other management functional studies.

- Investigate whether the existing 3GPP management architecture can be reused, or needs to be enhanced.

- Take into consideration the outcome on next generation network architecture and study the potential management and orchestration architecture aspects of the end to end architecture.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.799: "Study on Architecture for Next Generation System."

[3]	ETSI GS NFV 003: "Network Functions Virtualisation (NFV); Terminology for Main Concepts in NFV".

[4]	Draft 3GPP TS 28.500: "Management concept, architecture and requirements for mobile networks that include virtualized network functions."

[5]	NGMN Description of Network Slicing Concept

[6]	3GPP TR 38.801 Study on New Radio Access Technology; Radio Access Architecture and Interfaces

[7]	3GPP TS 23.501 V0.2.0 (2017-01) System Architecture for the 5G System; Stage 2.

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

5G NE: refers to a Network Element that is considered part of a 5G system [7].

Infrastructure Resource: network components that build up the environment for 3GPP NFs.

Note: example of Infrastructure Resource can be Physical Resource, Logical Resource, and Network Functions Virtualization Infrastructure.

Physical Resource: A physical asset for computation, storage or transport including radio access. Defined in NGMN Description of Network Slicing Concept [5].

Logical Resource: Partition of a physical resource, or grouping of multiple physical resources dedicated to a Network Function or shared between a set of Network Functions. Defined in NGMN Description of Network Slicing Concept [5].

Network Functions Virtualisation Infrastructure (NFVI): totality of all hardware and software components which build up the environment in which VNFs are deployed. Defined in ETSI GS NFV 003 [3].

Network Function: Network function is a 3GPP adopted or 3GPP defined processing function in a network, which has defined functional behaviour and 3GPP defined interfaces. Defined in 3GPP TR 23.799 [2].

Virtualized Network Function: implementation of an NF that can be deployed on a Network Function

Virtualisation Infrastructure (NFVI). Defined in ETSI GS NFV 003 [3].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ACRONYM>	<Explanation>

# 4	Management architecture introduction

The management system of next generation network will include the management features of the network, the network slicing instances and slice subnet instances etc.

The next generation network may be composed of virtualized network elements and non-virtualized network elements, the management of next generation network may make use of the management of the virtualized network functions. The management of the network element may include the network function management and resource management.

For the management of the virtualized network elements, the virtualized resource of the network element is managed as specified by ETSI NFV. The network functions are managed by network management function as specified by 3GPP. The management of virtualized core network nodes in LTE is supported in Rel-14.

For the management of the non-virtualized network elements, both network function and network resource is managed by network management function as specified by 3GPP.

The next generation network may also include LTE nodes, ref. [6].

If an evolution of the LTE management system is chosen, the following needs to be done to get basic management standards for NR and LTE. Management of new functions will need enhancement of the LTE management systems.

- New NRMs (object models) are needed for the radio access network and core network for the next generation network.

- New measurement specifications are needed for NR features and nodes.

- The existing 3GPP management specifications needs to be checked for which functions will be supported in the first release of the next generation network (e.g. with respect to features like trace and MDT).

New features like Network Slicing and virtualised functions which are part of RAN may need new management features/support.

# 5	Use cases related to management and orchestration architecture

## 5.1	Management and orchestration of networks containing non-5G NE and 5G NE

### 5.1.1	Introduction

This use case highlights potential management options/scenarios for possible network deployments, specifically at the early stages of introducing 5G network elements along with existing non-5G generation 3GPP network(s).

Operator wants to manage a network containing both non-5G NE(s) and 5G NE(s).

### 5.1.2	Pre-conditions

The network contains non-5G NE(s) and 5G NE(s).

The non-5G NE(s) are managed by existing management functions.

### 5.1.3	Description

There are following scenarios for operator to manage the 5G NE functions.

1. Operator enhances the existing management and orchestration function to manage both 5G NE(s) and non-5G NE(s). For example, operator may enhance the fault management function to retrieve the faults information from both non-5G NE and 5G NE, or

2. Operator creates new management functions to manage both 5G NE and non-5G NE. For example, operator may create new management function to manage the next generation network feature massive Internet of Things.

### 5.1.4	Post-conditions

The non-5G network elements and 5G network elements are both managed.

## 5.2	Use case of management and orchestration architecture for network slicing

### 5.2.1	Introduction

The operator wants to manage the next generation network slice instances; thus the management architecture needs to provide the corresponding support.

### 5.2.2	Pre-conditions

The requirements on the network slicing management are available.

### 5.2.3	Description

Refer to the network slicing management use cases specified in TR 28.801[x].

### 5.2.4	Post-conditions

The management architecture can support the network slicing management requirements.

# 6	Potential requirements on management and orchestration architecture

REQ-NWA -CON-01 The 3GPP management system shall support the management requirements of network slice as defined in TR 28.801[x] section 6.

# 7	Potential management and orchestration architecture options

## 7.1	Option 1: CCC

### 7.1.1	Description

### 7.1.2	Potential interaction with network architecture (if needed)

# 8	Conclusions

# 9	Recommendations

Annex <A>:
<Annex title>

Annexes are only to be used where appropriate:

Annexes are labelled A, B, C, etc. and are "informative"(3GPP TRs are informative documents by nature).

Annex <X>:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2016-07 |  |  |  |  |  | First creation of skeleton. | 0.0.0 |
| 2016-07 | SA5#108 | S5-164251 S5-164252 |  |  |  | Update following the agreement in SA5#108. | 0.1.0 |
| 2016-09 | SA5#109 | S5-165355 |  |  |  | Update following the agreement in SA5#109. | 0.2.0 |
| 2016-11 | SA5#110 | S5-165204 S5-166441 |  |  |  | Update S5-165204 following the agreement in SA5#109 Update S5-166441 following the agreement in SA5#110 | 0.3.0 |
| 2017-01 | SA5#111 | S5-171315 |  |  |  | Update S5-171315 following the agreement in SA5#111 | 0.4.0 |
| 2017-02 | SA5#111Bis | S5A-170154 S5A-170156 S5A-170326 |  |  |  | Update S5A-170154 following the agreement in SA5#111Bis Update S5A-170156 following the agreement in SA5#111Bis Update S5A-170326 following the agreement in SA5#111Bis Editorial changes on the section titles and TR title | 0.5.0 |