3GPP TR 32.867 v0.3.0 (2017-05)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Management enhancement of Control and

User Plane Split (CUPS) of Evolved Packet Core (EPC) nodes;

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2016, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions and abbreviations	5

3.1	Definitions	5

3.2	Abbreviations	5

4	Use Cases	5

5	Potential Requirements	5

6	Potential Solution	5

7	Conclusions	6

8	Recommendations	6

Annex <A>(informative): Change history	6

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The split of EPC nodes will cause the user plane and the control plane of S-GW and P-GW to be separated. This will affect management of these nodes and may cause the existing management specifications to be impacted.

# 1	Scope

The present document analyses the impacts for management of EPC nodes caused by the impacts for management of EPC nodes caused by the split of user plane and control plane. The scope of this study includes:

- The affect of the CUPS on the performance measurements of the EPC network [2].

- The impact of the CUPS on the EPC NRM IRP [3].

- The potential impact on E-UTRAN [4].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.426: “Telecommunication management; Performance Management (PM); Performance measurements Evolved Packet Core (EPC) network”.

[3]	3GPP TS 28.708: “Telecommunication management; Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)”.

[4]	3GPP TS 28.658: “Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)”.

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

TBD

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

PGW			PDN Gateway

PGW-C			PDN Gateway Control plane function

PGW-U		PDN Gateway User plane function

SGW			Serving Gateway

SGW-C			Serving Gateway Control plane function

SGW-U		Serving Gateway User plane function

# 4	Use Cases

## 4.1	Measurement related to the PGW in CUPS

### 4.1.1	Pre-conditions

The operator deploys the PGW with separated control plane and user plane.

### 4.1.2	Description

The NM requests EM to initiate one or more of the following measurements at PGW-C and PGW-U. The management system subscribes to the initiated measurements.

- The session management related measurements (counters) in the PGW-C include:

- PDN-GW initiated dedicated bearer creation

- PDN-GW initiated dedicated bearer deletion

- PDN-GW initiated dedicated bearer modification with QoS update

- PDN-GW initiated dedicated bearer modification without QoS update

- Active EPS bearers related measurements

- UE requested bearer resource modification related measurements

- PDN connections related measurements

- Number of EPS bearer

- SGi related measurements fulfilled by counter in the PGW-U.

Editor’s note: Whether there is other PGW related measurement in CUPS is FFS.

The EM notifies to NM when the measurements from PGW-C and PGW-U are available.

### 4.1.3	Post-conditions

The collected measurement data are notified to operator’s the management system.

## 4.2	Measurement related to the SGW in CUPS

### 4.2.1	Pre-conditions

The operator deploys the SGW with separated control plane and user plane.

### 4.2.2	Description

The NM requests EM to initiate one or more of the following measurements at SGW-C and SGW-U. The management system subscribes to the initiated measurements.

1. The GTP related measurements in the SGW-U include:

- GTP S5/S8 data packets related measurements

- GTP S4 data volume related measurements

- GTP S12 data volume related measurements

- Bearer resource usage related measurements

2. The GTP related measurements in the SGW-C include:

- GTP S5/S8 data signalling related measurements

3. The S1-U data volume related measurements fulfilled by counter in SGW-U.

4. The Session management related measurements in the SGW-C includes:

- S4/S11 EPS bearer related measurements

  - EPS bearer creation related measurements

  - EPS bearer modification related measurements

- S5/S8 EPS bearer related measurements

  - EPS bearer creation related measurements

  - EPS bearer modification related measurements

  - EPS bearer deletion related measurements

  - Bearer resource usage related measurements

Editor’s note: Whether there is other SGW related measurement in CUPS is FFS.

The EM notifies to NM when the measurements from SGW-C and SGW-U are available.

### 4.2.3	Post-conditions

The collected measurement data are notified to operator’s the management system.

# 5	Potential Requirements

## 5.1	General Requirements

REQ-GWM-CON-1	The 3GPP management system should have the capability of initiating the measurements related to the control plane and/or the user plane of PGW.

REQ-GWM-CON-2	The 3GPP management system should have the capability of initiating the measurements related to the control plane and/or the user plane of SGW.

REQ-GWM-CON-3	The 3GPP management system should have the capability of subscribing the data of PGW and SGW related measurement.

REQ-GWM-CON-4	The EM should have the capability of notifying the measured data of PGW and/or SGW to NM.

## 5.2	Requirements for PGW-C

REQ-PGW-CON-1		The 3GPP management system should support session management related measurement in PGW-C.

## 5.3	Requirements for PGW-U

REQ-PGW-CON-1	The 3GPP management system should support SGi related measurement in PGW-U.

## 5.4	Requirements for SGW-C

REQ-SGW-CON-1	The 3GPP management system should support session management related measurement in SGW-C.

REQ-SGW-CON-2	The 3GPP management system should support GTP signalling packets related measurement in SGW-C.

## 5.5	Requirements for SGW-U

REQ-SGW-CON-1	The 3GPP management system should support GTP data packets and data volume related measurement in SGW-U.

REQ-SGW-CON-2	The 3GPP management system should support S1-U data volume related measurement in SGW-U.

# 6	Potential Solution

TBD

# 7	Conclusions

TBD

# 8	Recommendations

TBD

Annex <A>(informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-01 |  |  |  |  |  | Skeleton of draft TR 32.867 | 0.0.0 |
| 2017-01 | SA5 #111 |  |  |  |  | Inclusion of SA5 #111 approved pCR S5-171371 and some editorial fixes | 0.1.0 |
| 2017-04 | SA5#112 |  |  |  |  | Inclusion of SA5#112 approved pCR S5-171990 | 0.2.0 |
| 2017-05 | SA5#113 |  |  |  |  | Inclusion of SA5#113 approved pCR S5-173528, S5-173529 and S5-173-530 | 0.3.0 |