3GPP TR 32.869 V0.5.0 (2017-05)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecom Management;

Charging management;

Study on Overload Control for Diameter Charging Applications;

(Release 13)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2015, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	7

3.1	Definitions	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Overview	9

4.1	Introduction	9

4.2	Problems and goals	9

5	Impacts of Diameter overload on 3GPP charging interfaces	9

5.1	Introduction	9

5.1.1	3GPP Diameter Charging Interfaces	9

5.1.2	IETF Diameter Overload Indication Conveyance	10

5.2	Overload of the Offline Charging System (OFCS)	10

5.2.1	Existing overload control mechanism on Diameter charging interface	10

5.2.1.1	Failure handling	10

5.2.1.2	Limitation	11

5.2.2	Solutions for Diameter overload indication handling	11

5.2.2.1	Alternative solution 1- OLR with buffer mechanism	11

5.2.2.2	Alternative solution 2 OFCS with buffer mechanism	12

5.2.3	Evaluation and recommendation	12

5.3	Overload of the Online Charging System (OCS)	13

5.3.1	Existing overload control mechanism on Diameter charging interface	13

5.3.1.1	Failure handling	13

5.3.1.2	Limitation	13

5.3.2	Solutions for Diameter overload indication handling	13

5.3.2.1	Alternative solution 1	13

5.3.2.2	Alternative solution 2	13

5.3.3	Evaluation and recommendation	13

6	Conclusions	13

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The objective is to study overload control mechanisms for Rc, Re, Rf and Ro interfaces based on the conclusion of TR 29.809 [212].

For 3GPP Diameter Charging interfaces, the Diameter overload control mechanism are based on the IETF Draft "Diameter Overload Indication Conveyance" that defines the AVPs for the transport of overload information of Diameter application as well as the basic behaviour of the Diameter endpoints receiving this information.

The IETF RFC 7683 [404] is considered as the reference of this TR and not the IETF Draft draft-ietf-dime-ovli-02 referenced by TR 29.809 [212].

As per TR 29.809 [212], the following conclusions apply and are not studied in this TR:

-	How a reporting node determines it is entering into an overload condition is implementation dependent.

-	How a reporting node determines the contents value of the overload report (i.e. OC-OLR AVP) is implementation dependent.

-	How a reacting node achieves traffic reduction (i.e. selection of impacted sessions) is implementation dependent.

The following issues are covered by the TR:

-	Application and/or modification of current failure handling functionality defined in TS 32.299 [50] for Diameter online and offline charging in the Charging Trigger Function when reacting to overload control indication from the OCS or CDF, respectively.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging Architecture and Principles".

[2] - [10]	Void.

[11]	3GPP TS 32.251: "Telecommunication management; Charging management; Packet Switched (PS) domain charging".

[12] - [19]	Void.

[20]	3GPP TS 32.260: "Telecommunication management; Charging management; IP Multimedia Subsystem (IMS) charging".

[21] - [29]	Void.

[30]	3GPP TS 32.270: "Telecommunication management; Charging management; Multimedia Messaging Service (MMS) charging".

[31] 	3GPP TS 32.271: "Telecommunication management; Charging management; Location Services (LCS) charging".

[32] 	3GPP TS 32.272: "Telecommunication management; Charging management; Push-to-talk over Cellular (PoC) charging".

[33]	3GPP TS 32.273: "Telecommunication management; Charging management; Multimedia Broadcast and Multicast Service (MBMS) charging".

[34]	3GPP TS 32.274: "Telecommunication management; Charging management; Short Message Service (SMS) charging".

[35]	3GPP TS 32.275: "Telecommunication management; Charging management; MultiMedia Telephony (MMTel) charging".

[36] 	3GPP TS 32.276: "Telecommunication management; Charging management; Voice Call Service Charging".

[37] 	3GPP TS 32.277: "Telecommunication management; Charging management; Proximity-based Services (ProSe) Charging".

[38] 	3GPP TS 32.278: "Telecommunication management; Charging management; Monitoring Event charging".

[39]	Void.

[40]	3GPP TS 32.280: "Telecommunication management; Charging management; Advice of Charge (AoC) service".

[41]	3GPP TS 32.281: "Telecommunication management; Charging management; Announcement service".

[42] - [49]	Void.

[50]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging applications".

[51] - [52]	Void.

[53]	3GPP TS 32.296: "Telecommunication management; Charging management; Online Charging System (OCS) applications and interfaces".

[54] - [99]	Void.

[100]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[101]	3GPP TS 22.115: "Service aspects; Charging and billing".

[102] - [199]	Void.

[200] - [211]	Void.

[212]	3GPP TR 29.809: "Study on Diameter overload control mechanisms".

[213] - [299]	Void.

[300] - [399]	Void.

[400]	Void.

[401]	IETF RFC 6733 (2015): "Diameter Base Protocol".

[402]	IETF RFC 4006 (2005): "Diameter Credit-Control Application".

[403]	IETF RFC 7683 (2015): "Diameter Overload Indication Conveyance ".

[404] - [499]	Void.

[500] - [599]	Void.

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [100] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [100].

middle tier TS: term used for the 3GPP charging TSs that specify the domain / subsystem / service specific, online and offline, charging functionality. These are all the TSs in the numbering range from TS 32.250 to TS 32.27x, e.g. TS 32.250 [10] for the CS domain, or TS 32.270 [30] for the MMS service. Currently, there is only one "tier 1" TS in 3GPP, which is the TS 32.240 that specifies the charging architecture and principles. Finally, there are a number of top tier TSs in the 32.29x numbering range ([50] ff) that specify common charging aspects such as parameter definitions, encoding rules, the common billing domain interface or common charging applications.

offline charging: charging mechanism where charging information does not affect, in real-time, the service rendered

online charging: charging mechanism where charging information can affect, in real-time, the service rendered and therefore a direct interaction of the charging mechanism with session/service control is required

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Gy	Online charging reference point between a PCEF and an OCS.

Gz	Offline charging reference point between a PCEF and an OFCS.

Rf	Offline Charging Reference Point between a 3G network element and the CDF.

Ro	Online Charging Reference Point between a 3G network element and the OCS.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [100] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [100].

ACA	ACcounting-Answer

ACR	ACcounting-Request

AS	Application Server

ASA	Abort-Session-Answer

ASR	Abort-Session- Request

AVP	Attribute Value Pair

CCA	Credit-Control-Answer

CCR	Credit-Control-Request

CDF	Charging Data Function

CDR	Charging Data Record

CEA	Capabilities-Exchange-Answer

CER	Capabilities-Exchange-Request

DBPA	Diameter Base Protocol Accounting

DCCA	Diameter Credit-Control Application

DCD	Dynamic Content Delivery

DPA	Disconnect-Peer-Answer

DPR	Disconnect-Peer-Request

DWA	Device-Watchdog-Answer

DWR	Device-Watchdog-Request

ECUR	Event Charging with Unit Reservation

FUI	Final-Unit-Indication

GSU	Granted-Service-Unit

IE	Information Element

IEC	Immediate Event Charging

MSCC	Multiple Services Credit-Control

OCS	Online Charging System

OFCS	OFfline Charging System

RAA	Re-Auth-Answer

RAR	Re-Auth-Request

TDF	Traffic Detection Function

# 4	Overview

## 4.1	Introduction

TR 29.809 [212] has investigated the possible Diameter-based mechanisms to support overload control mechanisms in 3GPP core networks.

Two main activities have been addressed:

- Identification of requirements for an improved overload control mechanism over Diameter based signaling interfaces used in 3GPP core networks;

- Evaluations of the proposed IETF solutions to cover the requirements of Diameter overload control.

The TR 29.809 [212] concludes that the generic solution as currently defined in the draft-ietf-dime-ovli-02, which is IETF RFC 7683 [403] is recommended as the basis to perform overload control over 3GPP Diameter applications (see clause 8.3.2 in TR 29.809 [212]). This solution provides a set of generic Diameter AVPs that can be re-used over any Diameter application to transport overload indication between Diameter endpoints. However, the exact solution to implement will be decided per Diameter application, depending on the specific requirements of each interface. Since the solution is application dependent this study aims to investigate the solution applicable for the Charging applications.

## 4.2	Problems and goals

The focus of the TR 29.809 [212] has been on the existing Diameter applications Protocol Error "DIAMETER_TOO_BUSY" and "DIAMETER_UNABLE_TO_DELIVER". The Protocol Error "DIAMETER_TOO_BUSY" is used by the Diameter node to indicate that the specific server requested is busy and unable to provide the requested service. When a Diameter agent (Relay or Proxy) is in the path between the client and the server, the Diameter client may receive from the agent the Protocol Error "DIAMETER_UNABLE_TO_DELIVER" as answer to the request if there is no server that can handle the request either due an overloaded or no response.

The default behaviour of the client relies only on the availability of alternate peers to offload the requests when the primary server is overloaded. A limitation with all the existing mechanisms is that the downstream Diameter node can only react after overload happens, i.e. after overload is detected. The TR 29.809 [212] points out that a mechanism for overload protection is worth investigated and concludes that the generic solution as defined in the IETF RFC 7683 [403] is recommended as the basis to perform overload control over 3GPP Diameter applications (see clause 8.3.2 in TR 29.809 [212]). However little guidance is provided on how to apply the DOIC (Diameter Overload Indication Conveyance) mechanism for Diameter charging applications in TR 29.809 [212]. This will be investigated in this report.

# 5	Impacts of Diameter overload on 3GPP charging interfaces

## 5.1	Introduction

### 5.1.1	3GPP Diameter Charging Interfaces

The Offline Charging System terminates the Rf reference point as defined in TS 32.240 [1]. Other 3GPP specifications define additional reference points that are functionally equivalent to the Rf reference point. These are the Gz reference point for PCEF embedded in PGW and the Gzn reference point for TDF. The Offline Charging System (OFCS) may be decomposed into Charging Data Function (CDF) and the Charging Gateway Function (CGF) in which case, the CDF provides the Diameter Accounting Application. Between the decomposed function Diameter is not used. As defined in TS 32.240 [1], the network elements connected to the OFCS by the Rf reference point, such as P-GW can be regarded as Diameter client.

The Online Charging System terminates the Ro reference point as defined in TS 32.240 [1]. Other 3GPP specifications define additional reference points that are functionally equivalent to the Ro reference point. These are the Gy for PCEF embedded in PGW and the Gyn reference point for TDF. The Online Charging System (OCS) may be decomposed and between these functions Diameter is used, but for the Ro reference point it is the OCS as such that provides the Diameter Credit-Control Application. As defined in TS 32.240 [1], the network elements connected to the OCS by the Ro reference point, such as P-GWcan be regarded as Diameter client.

The Diameter Accounting Application used on the Rf interface and Diameter Credit-Control Application used on the Ro interface are defined in TS 32.299 [50].

This clause considers application and/or modification of current failure handling functionality defined in TS 32.299 [50] for Diameter online and offline charging in the Charging Trigger Function (CTF) when reacting to overload control indication from the OCS or CDF, respectively.

Today there is a support on the reference points for alternative servers both in online and offline charging in the case one service is not reachable. In most cases charging messages cannot be dropped, even if there is a Diameter overload situation on charging interfaces, since these are the bases for the subscribers’ bill and dropping a charging message will mean that there will be no means to charge for the service.

### 5.1.2	IETF Diameter Overload Indication Conveyance

The interaction between reacting node and reporting node specified in IETF RFC 7683 [403] can be illustrated as following figure.

Figure 5.1.2.1 Interaction between Reacting Node and Reporting Node

One abatement algorithm described in IETF RFC 7683 [403] is to select a random number between 1 and 100. If the random number is less than or equal to the indicated reduction percentage received by the reacting node then the request will be given abatement treatment otherwise the request is given normal routing treatment.

## 5.2	Overload of the Offline Charging System (OFCS)

### 5.2.1	Existing overload control mechanism on Diameter charging interface

#### 5.2.1.1	Failure handling

The current failure handling uses the Charging Characteristics as defined in TS 32.251 [11] Annex A, allows to specify both primary and secondary OFCS addresses as well as the possibility to produce CDR or buffer these for offline charging as a backup for the Diameter offline.

#### 5.2.1.2	Limitation

The current solution will only be used when there is a failure and cannot be used for preventing an overload situation, i.e. it will manage the event that a failure occurs but it cannot decrease the load if the systems are getting close to their limits.

### 5.2.2	Solutions for Diameter overload indication handling

#### 5.2.2.1	Alternative solution 1- OLR with buffer mechanism

Using OLR with buffer mechanism is proposed to be an alternative solution 1. The reacting node determines if overload control treatment should be applied to the request based on the OLR. One approach that could be taken for each request is to select a random number between 1 and 100. If the random number is less than or equal to the indicated reduction percentage received by the reacting node then the request is buffered, otherwise the request is given normal routing treatment. This algorithm enables request messages of the reduction percentage to be buffered. And when the overload ends, according to FIFO (first-in first-out) principle, the buffered request messages will be sent out to CDF in order. The CDF can apply existing duplicate detection and sort ACRs according to sequence number of each ACR to generate CDR. The shortage of the solution is that buffering request messages randomly may lead to additional workload on sorting ACRs in CDF.

OLR_DEFAULT_ALGO (0x0000000000000001) of OC-Feature-Vector AVP specified in IETF RFC 7683 [403] is used.

Supposing a standby CDF case is not considered in this report, the basic idea is illustrated as following,

Figure 5.2.2.1.1 Basic buffer mechanism solution

#### 5.2.2.2	Alternative solution 2 OFCS with buffer mechanism

In this case the OFCS selects which messages to reply with a Protocol Error “DIAMETER_TOO_BUSY” and for these the client will follow the Charging Characteristics on the treatment, e.g. try the secondary address, start buffering or producing CDR. This means that the OFCS would have to decide on which request that should be buffered and this can in this case be based on type of service. This will mean that the OFCS still will receive all Diameter messages and will reject them one by one, that is if the Client itself don’t start using an algorithm to select messages to go to the secondary address would it encounter a Protocol Error “DIAMETER_TOO_BUSY”.

### 5.2.3	Evaluation and recommendation

## 5.3	Overload of the Online Charging System (OCS)

### 5.3.1	Existing overload control mechanism on Diameter charging interface

#### 5.3.1.1	Failure handling

#### 5.3.1.2	Limitation

### 5.3.2	Solutions for Diameter overload indication handling

#### 5.3.2.1	Alternative solution 1

#### 5.3.2.2	Alternative solution 2

### 5.3.3	Evaluation and recommendation

# 6	Conclusions

Editor's note: This subclause describes the conclusions within the present document.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2015-05 |  |  |  |  | Initial skeleton provided by rapporteur (S5-153292) | - | 0.0.0 |
| 2015-06 | SA5#101 |  |  |  | Scope and reference (S5-153293) General introduction (S5-153294) Editorial correction (Title correction for 5.3.2 and Directory update) | 0.0.0 | 0.1.0 |
| 2015-08 | SA5#102 |  |  |  | Problems and goals (S5-154400) Description of Diameter applications in charging framework (S5-154401) | 0.1.0 | 0.2.0 |
| 2016-01 | SA5#105 |  |  |  | Alternative solution 1 – buffer mechanism on offline charging interface (S5-161304) | 0.2.0 | 0.3.0 |
| 2017-04 | SA5#111 |  |  |  | Update of Scope and References (S5-171852) Update of Definitions, symbols and abbreviations (S5-171853) Update of Introduction (S5-171854) | 0.3.0 | 0.4.0 |
| 2017-05 | SA5#112 |  |  |  | Update of Problems and Goals (S5-173402) Update of Introduction (S5-173403) Update of Offline Part (S5-173404) | 0.3.0 | 0.5.0 |