3GPP TR 36.747 V14.0.0 (2017-09)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Enhanced CRS and 4Rx SU-MIMO Interference Mitigation Performance Requirements for LTE;

(Release 14)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<LTE, Radio>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	6

2	References	7

3	Definitions, symbols and abbreviations	8

3.1	Definitions	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Enhanced CRS-IM	9

4.1	Scenarios and simulation assumptions	9

4.1.1	PDSCH	9

4.1.2	PDCCH/PCFICH	12

4.1.3	PHICH	14

4.1.4	EPDCCH	15

4.2	Receiver structures and assumptions	16

4.2.1	Baseline reference receivers	16

4.2.2	Enhanced reference receivers	16

4.2.3	CRS assistance information assumptions	17

4.3	Link-level performance evaluation	18

4.3.1	PDSCH	18

4.3.1.1	PDSCH Test Case #1	18

4.3.1.2	PDSCH Test Case #2	18

4.3.1.3	PDSCH Test Case #3	18

4.3.1.4	PDSCH Test Case #4	19

4.3.1.5	PDSCH Test Case #5	19

4.3.1.6	PDSCH Test Case #6	20

4.3.1.7	PDSCH Test Case #7	20

4.3.2	PDCCH/PCFICH	21

4.3.2.1	PDCCH/PCFICH Test Case #1	21

4.3.2.2	PDCCH/PCFICH Test Case #2	21

4.3.2.3	PDCCH/PCFICH Test Case #3	22

4.3.3	PHICH	22

4.3.3.1	PHICH Test Case #1	22

4.3.3.2	PHICH Test Case #2	23

4.3.3.3	PHICH Test Case #3	23

4.3.4	EPDCCH	24

4.4	Conclusions	24

5	Enhanced SU-MIMO IM based on 4RX	25

5.1	Scenarios	26

5.2	Receiver structures and assumptions	26

5.3	Link-level performance evaluation	27

5.3.1	 Parameters for link-level evaluation	27

5.3.2	Link-level simulation results	27

5.4	Conclusions	31

6	Conclusions	31

Annex A:	Change history	32

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

A work item for Enhanced CRS and 4Rx SU-MIMO Interference Mitigation Performance Requirements for LTE was approved at the 3GPP RAN #73 meeting [2]. This technical report summarizes the work that RAN WG4 has accomplished in this work item to investigate the feasibility of CRS-IM and SU-MIMO IM receivers.

In the LTE Release 11 to 13, multiple UE interference mitigation (IM) receiver enhancements for the interference-limited environments were introduced, including LMMSE-IRC, CRS-IM, SU-MIMO IM, NAICS, and other advanced receivers.

The substantial part of the respective 3GPP work was dedicated to the introduction of the CRS Interference Mitigation (CRS-IM) functionality, which was done in the scope of the Release 11 FeICIC WI and Release 12/13 CRS-IM SI and WI. The purpose of the introduced CRS-IM functionality is to specify the receiver mechanisms to mitigate the dominant CRS interferences from the neighbouring cells, which may become a limiting factor for the DL performance for the synchronous homogeneous deployments. As the result of the 3GPP work in the LTE Release 11-13, multiple CRS-IM UE demodulation performance requirements were defined covering some typical LTE operation scenarios. However, still only a subset of the important operation scenarios was addressed in the UE demodulation performance for other scenarios is not guaranteed:

-	CRS-IM requirements are introduced for the case of the network deployments using 2 CRS APs only, meanwhile, the UE behaviour for the case of 4 CRS APs deployments is undefined.

-	CRS-IM requirements is defined for the case using the same number of CRS APs in the serving and interference cells. Practically, different cells may use different CRS configurations.

-	CRS-IM requirements are specified for the UEs equipped with 2 receive antennas only, while the UEs with 4 receive antennas, which are emerging in the market, may also benefit from using the CRS-IM and should be considered.

Therefore, the CRS-IM functionality is actually not defined for many important use cases and without proper UE demodulation requirements it is not possible to guarantee correct UE implementations. The work item [2] aims to investigate and introduce CRS-IM for the generic LTE deployment scenarios with various combinations of the number of CRS APs and number of UE receive chains.

In the LTE Release 12 the performance requirements for UEs using enhanced interference cancellation and suppression SU-MIMO receivers were introduced. The advanced receivers such as reduced complexity ML (R-ML) were considered for the intra-cell SU-MIMO inter-stream interference handling. The performance requirements were introduced for the UEs equipped with 2 receive antennas. Recently in the LTE Release 13 the basic performance requirements for the 4 receive antennas UEs were introduced, however, the requirements for the SU-MIMO scenarios are assumed to be based on the simple LMMSE receiver. By using SU-MIMO advanced receiver, which can suppress or cancel the inter-stream interference, the significant gain can be achieved in the typical scenarios.

This technical report captures the outcome of the work item technical studies. The content of each specific clause of the report is briefly described as follows.

Clause 1 of this document defines the scope and objectives of this study.

Clause 4 describes studies on the enhanced CRS-IM performance requirements.

Clause 5 describes studies on the enhanced SU-MIMO IM performance requirements.

Clause 6 captures the conclusions.

# 1	Scope

The objective of this report is to investigate feasibility and specify requirements for 1) CRS-IM receivers for the generic scenarios with different number of CRS APs (2, 4) and different number of UE receive antennas and 2) SU-MIMO IM receivers for UEs equipped with 4 receive antennas. The work item [2] has the following detailed objectives:

-	Investigate feasibility and specify requirements for the CRS-IM receivers for the generic scenarios with different number of CRS APs (2, 4) and different number of UE receive antennas for synchronous networks:

-	Stage 1: Investigate performance benefits and feasibility of using CRS-IM receivers:

-	Identify target scenarios including deployment scenarios, interference models, and others

-	Reuse Rel-12 NAICS and Rel-13 CRS-IM WIs assumptions as the starting point

-	Evaluate the CRS-IM performance benefits

-	Identify reference CRS-IM receiver structure assumptions including at least number of cancelled cell(s), and IM algorithm.

-	The tradeoff between complexity and performance should be analyzed in order to define reference receivers.

-	Stage 2: Specify UE demodulation and CSI reporting performance requirements to verify practical CRS-IM operation for the identified scenarios based on the outcome of Stage 1.

-	The work should be done under the following assumptions:

-	The following scenarios are prioritized:

-	2 RX antennas UEs with 4 CRS APs in the serving and interference cell(s)

-	2 RX antennas UEs with a mix of 2 and 4 CRS APs in the serving and interference cell(s)

-	4 RX antennas UEs with 2 and 4 CRS APs in the serving and interference cell(s)

-	CRS-IM can be used for different downlink physical channels:

-	First priority: PDSCH (both CRS- and DMRS-based transmission modes)

-	Second priority: DL control channels including PDCCH, PCFICH, PHICH and EPDCCH

-	Non-colliding CRS scenarios are primarily considered.

-	Colliding CRS scenarios are not precluded.

-	Homogeneous deployment scenarios are primarily considered

-	Heterogeneous deployment scenarios are not precluded.

-	Single carrier case.

-	Reference PDSCH receiver structure: LMMSE-IRC with CRS-IM.

-	Investigate feasibility and specify requirements for the enhanced SU-MIMO inter-stream interference mitigation (SU-MIMO IM) receivers for the scenarios with 4 receive antennas UEs

-	Stage 1: Investigate performance benefits and feasibility of using SU-MIMO IM receivers for the scenarios with 4 receive antennas UEs.

-	Investigate and agree on target scenarios including layer number for spatial multiplexing, MCS, propagation conditions and others for performance evaluation

-	Reuse the assumptions for Rel-12 SU-MIMO IM (Type-C receiver) and Rel-13 4Rx MIMO performance requirements as starting point.

-	SU-MIMO scenarios

-	rank 2/3/4 SU-MIMO is considered

-	The maximum number of layers (e.g. rank3/4) should be determined based on feasibility study (e.g. operating SNR), realistic Tx EVM assumption and the study on performance gains, reference receiver complexity and testability, etc.

-	Strive to reuse agreed Tx EVM for 4Rx in Rel-13 if the related agreement can be reached

-	Consider 2 and 4 CRS APs scenarios

-	Channel correlations

-	Focus on the Medium, Medium A and High antenna correlation models

-	Modulation order: up to 256QAM

-	The considered modulation orders should be decided based on feasibility study (e.g. operating SNR), realistic Tx EVM assumption, performance gains, reference receiver complexity and testability, etc.

-	Strive to reuse agreed Tx EVM for 4Rx in Rel-13 if the related agreement can be reached.

-	Identify the reference receiver structure for SU-MIMO IM evaluation

-	R-ML is considered in high priority

-	Evaluate the performance of enhanced SU-MIMO IM receivers.

-	Stage 2: Specify UE demodulation performance requirements to verify enhanced SU-MIMO receivers for the UEs equipped with 4 RX antennas

-	Reference receiver structure and other test parameters are based on the outcome of Stage 1.

-	Single carrier case.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905, “Vocabulary for 3GPP Specifications”.

[2]	3GPP TSG RAN RP-161877, “New Work Item Proposal: Enhanced CRS and SU-MIMO Interference Mitigation Performance Requirements for LTE”.

[3]	3GPP TSG RAN RP-120860 “Further Enhanced Non CA-based ICIC for LTE”

[4]	3GPP TSG RAN RP-142263 “New Work Item proposal: Perf. Part: CRS Interference Mitigation for LTE Homogenous Deployments”

[5]	3GPP TR 36.829 “Enhanced performance requirement for LTE User Equipment (UE)”

[6]	3GPP TR 36.866 “Study on Network-Assisted Interference Cancellation and Suppression (NAIC) for LTE”

[7]	R4-1702193 “Summary of Enhanced CRS-IM PDSCH simulation results (FDD)”, February 2017

[8]	R4-1702909 “Summary of Enhanced CRS-IM PDSCH simulation results (FDD)”, April 2017

[9]	R4-1702194 “Summary of Enhanced CRS-IM DL Control Channels simulation results (FDD)”, February 2017

[10]	R4-1700937 “Summary of simulation results for enhanced SU-MIMO (FDD)”, February 2017

[11]	R4-1703254 “Summary of simulation results for enhanced SU-MIMO (FDD)”, April 2017

[12]	R4-1704942 “Summary of simulation results for enhanced SU-MIMO”, May 2017

[13]	R4-1707881 “Summary of simulation results for enhanced SU-MIMO”, Aug 2017

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply.

AP	Antenna port

BS	Base station

CRS	Cell-specific Reference signals

CRS-IM	CRS Interference Mitigation

CSI-RS	Channel State Information RS

DL	Downlink

EVM	Error Vector Magnitude

FDD	Frequency Division Duplexing

FeICIC	Further eICIC

FFT	Fast Fourier Transform

FRC	Fixed reference channel

HARQ	Hybrid Automatic Repeat Request

IM	Interference mitigation

INR	Interference to noise ratio

LMMSE-IRC	Linear MMSE interference rejection combining

LTE	Long Term Evolution

MIMO	Multiple input and multiple output

PCFICH	Physical control format indicator channel

PDSCH	Physical downlink shared channel

PDCCH	Physical downlink control channel

PMI	Precoding Matrix Indicator

PRB	Physical resource block

RU	Resource Utilization

SINR	Signal to interference and noise ratio

SNR	Signal to noise ratio

SU-MIMO IM	SU-MIMO inter-stream Interference Mitigation

TDD	Time-division duplex

TM	Transmission mode

UE	User Equipment

WI	Work Item

# 4	Enhanced CRS-IM

The objective of the work item [2] is to investigate performance benefits, complexity, and feasibility of using enhanced CRS-IM receivers in application to the generic scenarios with different number of CRS APs (2, 4) and different number of UE receive antennas:

-	Identify target scenarios including deployment scenarios, interference models.

-	Identify reference CRS-IM receiver structure assumptions.

-	Evaluate the CRS-IM performance benefits.

This section summarizes the results of the respective studies on the performance benefits and feasibility of enhanced CRS-IM receivers. The section includes the following sub-clauses:

-	Clause 4.1 includes the description of the scenarios used to assess the link level performance of CRS-IM receivers.

-	Clause 4.2 describes the reference CRS-IM receiver assumptions considered during the studies.

-	Clause 4.3 provides the results of the link-level performance evaluations and related observations.

-	Clause 4.4 provides summary of conclusions on enhanced CRS-IM investigations.

## 4.1	Scenarios and simulation assumptions

### 4.1.1	PDSCH

In this section scenarios and simulation assumptions for PDSCH enhanced CRS-IM performance evaluations are provided. In Table 4.1.1-1 the list of PDSCH test cases is shown. The detailed simulation assumptions are provided in Table 4.1.1-2 and fixed reference channels (FRCs) are provided in Table 4.1.1-3 and Table 4.1.1-4.

Table 4.1.1-1: PDSCH demodulation test scenarios

| Test | TM | CRS pattern | Number of UE receive chains | Number of CRS APs |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Serving cell | Interference cells |
| 1 | TM4 | Non Colliding | 2 | 4 | 4 |
| 2 | TM4 | Non Colliding | 4 | 2 | 2 |
| 3 | TM9 | Non Colliding | 4 | 2 | 2 |
| 4 | TM4 | Non Colliding | 4 | 4 | 4 |
| 5 | TM4 | Colliding | 2 | 4 | 4 |
| 6 | TM4 | Colliding | 2 | 4 | 2 |
| 7 | TM4 | Colliding | 2 | 2 | 4 |

Table 4.1.1-2: PDSCH simulation assumptions

| Parameter |  |  | Value |  |
| --- | --- | --- | --- | --- |
| Duplexing |  |  | FDD |  |
| System bandwidth |  |  | 10 MHz |  |
| Transmit EVM |  |  | 6% |  |
| Antenna models |  |  | Tests 1, 5, 6: 4x2 low correlation Tests 2, 3: 2x4 low correlation Test 4: 4x4 low correlation Test 7: 2x2 low correlation |  |
| Cell ID pattern |  |  | Non-colliding CRS: - Serving cell: 0 - Interference cell 1: 1 - Interference cell 2: 6 Colliding CRS: - Serving cell: 0 - Interference cell 1: 6 - Interference cell 2: 1 |  |
| Interference power profile |  |  | Interference to noise ratio - Profile 1: I1/Noc = 10.45 dB, I2/No = 4.6 dB - Profile 2: I1/Noc = 15.8 dB, I2/No = 10.5 dB - Profile 3: I1/Noc = 4.7dB, I2/No = 1.4 dB Tests 1, 2, 3, 4, 7: Profile 1 Tests 5, 6: Profile 1, 2, 3 |  |
| Network synchronization in time |  |  | All cells are synchronous |  |
|  |  |  | Time-delay wrt. serving cell |  |
|  |  |  | 1st interfering cell | 2nd interfering cell |
|  |  |  | 3 µs | -1 µs |
| Network synchronization in frequency |  |  | Frequency shift wrt. serving cell |  |
|  |  |  | 1st interfering cell | 2nd interfering cell |
|  |  |  | 300 Hz | -100 Hz |
| Channel model |  |  | EVA-5Hz for all links. Use different channel seed for between cells. |  |
| Number of control region OFDM symbols (CFI) |  |  | CFI = 2. Same value in all cells |  |
| HARQ modelling |  |  | Maximum 4 HARQ retransmissions |  |
| CSI-RS configuration for serving cell (for Test 3) |  |  | Number of CSI-RS ports : 2 CSI-RS periodicity and subframe offset : 5/2 CSI reference signal configuration : 0 Zero-power CSI-RS configuration : 3 /0001000000000000 |  |
| Desired PDSCH transmission parameters | Resource allocation |  | PDSCH is scheduled in subframes 0-4, 6-9 TM4: 50 PRB for all subframes TM9: 50 PRB for subframes 1-4, 6-9; 41 PRB for subframe 0 |  |
|  | PMI |  | TM4: Follow PMI, reporting mode PUSCH 3-1 TM9: Random PMI with 1 PRG / 1 TTI granularity |  |
| Physical channels and signal explicitly transmitted in interfering cells |  |  | CRS PDSCH PSS/SSS/PBCH |  |
| Interference PDSCH transmission parameters |  | Resource allocation | Random full band (50PRB) on/off model, proportional to the average resource utilization in the interfering cells. Average resource utilization (RU) in the interfering cells: - Option 1: 20% (all tests) Option 2: 0% (Tests 4, 5 with Option 2 interference power profile) |  |
|  |  | Rank | Randomly changing rank per allocated sub-band from subframe to su- bframe: 80% rank-1, 20% rank-2 |  |
|  |  | Interference model | Based on TS 36.101 B.5.3 and B.5.4 |  |
|  |  | TM | Same as serving cell TM |  |

Table 4.1.1-3: PDSCH FRC for TM4

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Tests 2,7 |  | Tests 1,4,5,6 |  |
| FRC index |  | FRC.1-1 | FRC.1-2 | FRC.2-1 | FRC.2-2 |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 |
| Allocated resource blocks |  | 50 | 50 | 50 | 50 |
| Allocated number of PDCCH symbols |  | 2 | 2 | 2 | 2 |
| Allocated subframes per Radio Frame |  | 9 | 9 | 9 | 9 |
| Modulation |  | 16QAM | 64QAM | 16QAM | 64QAM |
| Target Coding Rate |  | 0.58 | 0.5 | 0.5 | 0.5 |
| Information Bit Payload |  |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 15264 | 19848 | 12960 | 18336 |
| For Sub-Frame 5 | Bits | n/a | n/a | n/a | n/a |
| For Sub-Frame 0 | Bits | 14112 | 18336 | 11448 | 18336 |
| Number of Code Blocks |  |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 3 | 4 | 3 | 3 |
| For Sub-Frame 5 | Bits | n/a | n/a | n/a | n/a |
| For Sub-Frame 0 | Bits | 3 | 3 | 2 | 3 |
| Binary Channel Bits |  |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 26400 | 39600 | 25600 | 38400 |
| For Sub-Frame 5 | Bits | n/a | n/a | n/a | n/a |
| For Sub-Frame 0 | Bits | 24768 | 37152 | 24064 | 36096 |
| Max. Throughput averaged over 1 frame | Mbps | 13.6224 | 17.712 | 11.513 | 16.502 |
| UE Category |  | ≥ 2 | ≥ 2 | ≥ 1 | ≥ 2 |

Table 4.1.1-4: PDSCH FRC for TM9

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | Test 3 |  |
| FRC index |  | FRC.3-1 | FRC.3-2 |
| Channel bandwidth | MHz | 10 | 10 |
| Allocated resource blocks |  | 50 (Note 3) | 50 (Note 3) |
| Allocated subframes per Radio Frame |  | 9 | 9 |
| Modulation |  | 16QAM | 64QAM |
| Target Coding Rate |  | 0.54 | 0.51 |
| Information Bit Payload |  |  |  |
| For Sub-Frames 1,2,3,4 | Bits | 12960 | 18336 |
| For Sub-Frames 6,7,8,9 | Bits | 12960 | 18336 |
| For Sub-Frame 5 | Bits | n/a | n/a |
| For Sub-Frame 0 | Bits | 10680 | 14688 |
| Number of Code Blocks |  |  |  |
| For Sub-Frames 1,2,3,4 | Bits | 3 | 3 |
| For Sub-Frames 6,7,8,9 | Bits | 3 | 3 |
| For Sub-Frame 5 | Bits | n/a | n/a |
| For Sub-Frame 0 | Bits | 2 | 3 |
| Binary Channel Bits |  |  |  |
| For Sub-Frames 1,4,6,9 | Bits | 24000 | 36000 |
| For Sub-Frames 2,7 | Bits | 23600 | 35400 |
| For Sub-Frames 3,8 | Bits | 23200 | 34800 |
| For Sub-Frame 5 | Bits | n/a | n/a |
| For Sub-Frame 0 | Bits | 19680 | 29520 |
| Max. Throughput averaged over 1 frame | Mbps | 11.436 | 16.138 |
| UE Category |  | ≥ 2 | ≥ 2 |
| Note 1: 2 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: 50 resource blocks are allocated in sub-frames 1, 2, 3, 4, 6, 7, 8, 9 and 41 resource blocks |  |  |  |

### 4.1.2	PDCCH/PCFICH

In this section scenarios and simulation assumptions for PDCCH/PCFICH enhanced CRS-IM performance evaluations are provided. In Table 4.1.2-1 the list of PDCCH/PCFICH test cases is shown. The detailed simulation assumptions are provided in Table 4.1.2-2 and the FRCs are summarized in Table 4.1.2-3.

Table 4.1.2-1: PDCCH/PCFICH demodulation test cases

| Test | CRS pattern | Number of UE receive chains | Number of CRS APs |  |
| --- | --- | --- | --- | --- |
|  |  |  | Serving cell | Interferer cells |
| 1 | Non Colliding | 2 | 4 | 4 |
| 2 | Non Colliding | 4 | 2 | 2 |
| 3 | Non Colliding | 4 | 4 | 4 |

Table 4.1.2-2: PDCCH/PCFICH simulation assumptions

| Parameter | Value |  |
| --- | --- | --- |
| Duplexing | FDD |  |
| System bandwidth | 10 MHz |  |
| Transmit EVM | 6% |  |
| Antenna models | Test 1: 4x2 low correlation Test 2: 2x4 low correlation Test 3: 4x4 low correlation |  |
| Cell ID pattern | Non-colliding CRS: - Serving cell: 0 - Interference cell 1: 1 - Interference cell 2: 6 |  |
| Interference power profile | Profile #1: Interference cell 1: I1/Noc = 13.91 dB Interference cell 2: I2/Noc = 3.34 dB Profile #2: Interference cell 1: I1/Noc = 8.36 dB Interference cell 2: I2/Noc = 1.66 dB |  |
| Network synchronization in time | All cells are synchronous |  |
|  | Time-delay wrt. serving cell |  |
|  | 1st interfering cell | 2nd interfering cell |
|  | 2 µs | 3 µs |
| Network synchronization in frequency | Frequency shift wrt. serving cell |  |
|  | 1st interfering cell | 2nd interfering cell |
|  | 200 Hz | 300 Hz |
| Channel model | EPA-5Hz for all links. Use different channel seed for between cells. |  |
| PHICH parameters | Normal PHICH Ng = 1/6 |  |
| Interference model | TS 36.101 B.7 (Rel-13 CCIM) |  |
| PDCCH/PCFICH parameters | Test #1: PDCCH AL = 2; CFI = 2 Test #2: PDCCH AL = 1,2; CFI = 1 Test #3: PDCCH AL = 1,2; CFI = 2 |  |

Table 4.1.2-3: PDCCH FRCs

| Parameter | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
| Number of transmitter antennas |  | 4 | 2 | 4 |
| Channel bandwidth | MHz | 10 | 10 | 10 |
| Number of OFDM symbols for PDCCH | symbols | 2 | 2 | 2 |
| Aggregation level | CCE | 2 | 1,2 | 1,2 |
| DCI Format |  | 2 | 2 | 2 |
| Cell ID |  | 0 | 0 | 0 |
| Payload (without CRC) | Bits | 46 | 43 | 46 |

### 4.1.3	PHICH

In this section scenarios and simulation assumptions for PHICH enhanced CRS-IM performance evaluations are provided. In Table 4.1.3-1 the list of PHICH test cases is shown. The detailed simulation assumptions are provided in Table 4.1.3-2 and the FRCs are summarized in Table 4.1.3-3.

Table 4.1.3-1: PHICH demodulation test cases

| Test | CRS pattern | Number of UE receive chains | Number of CRS APs |  |
| --- | --- | --- | --- | --- |
|  |  |  | Serving cell | Interferer cells |
| 1 | Non Colliding | 2 | 4 | 4 |
| 2 | Non Colliding | 4 | 2 | 2 |
| 3 | Non Colliding | 4 | 4 | 4 |

Table 4.1.3-2: PHICH simulation assumptions

| Parameter | Value |  |
| --- | --- | --- |
| Duplexing | FDD |  |
| System bandwidth | 10 MHz |  |
| Transmit EVM | 6% |  |
| Antenna models | Test 1: 4x2 low correlation Test 2: 2x4 low correlation Test 3: 4x4 low correlation |  |
| Cell ID pattern | Non-colliding CRS: - Serving cell: 0 - Interference cell 1: 1 - Interference cell 2: 6 |  |
| Interference power profile | Profile #1: Interference cell 1: I1/Noc = 13.91 dB Interference cell 2: I2/Noc = 3.34 dB Profile #2: Interference cell 1: I1/Noc = 8.36 dB Interference cell 2: I2/Noc = 1.66 dB |  |
| Network synchronization in time | All cells are synchronous |  |
|  | Time-delay wrt. serving cell |  |
|  | 1st interfering cell | 2nd interfering cell |
|  | 2 µs | 3 µs |
| Network synchronization in frequency | Frequency shift wrt. serving cell |  |
|  | 1st interfering cell | 2nd interfering cell |
|  | 200 Hz | 300 Hz |
| Channel model | EPA-5Hz for all links. Use different channel seed for between cells. |  |
| PHICH duration | Test #1: Extended Test #2: Normal Test #3: Extended |  |
| Interference model | TS 36.101 B.7 (Rel-13 CCIM) |  |
| PDCCH/PCFICH/PHICH parameters | PHICH Ng = 1 |  |

Table 4.1.3-3: PHICH FRCs

| Parameter | Unit | Test 1, 3 | Test 2 |
| --- | --- | --- | --- |
| Number of transmitter antennas |  | 4 | 2 |
| Channel bandwidth | MHz | 10 | 10 |
| Number of OFDM symbols for PDCCH |  | 3 | 1 |
| User roles |  | W I1 I2 | W I1 I2 |
| Resource allocation |  | (0,0) (0,1) (0,4) | (0,0) (0,1) (0,4) |
| Power offsets | dB | -4 0 -3 | -4 0 -3 |
| Payload |  | A R R | A R R |

### 4.1.4	EPDCCH

In this section scenarios and simulation assumptions for EPDCCH enhanced CRS-IM performance evaluations are provided. In Table 4.1.4-1 the list of EPDCCH test cases is shown. The detailed simulation assumptions are provided in Table 4.1.4-2.

Table 4.1.4-1: EPDCCH demodulation test cases

| Test | CRS pattern | Number of UE receive chains | Number of CRS APs |  |
| --- | --- | --- | --- | --- |
|  |  |  | Serving cell | Interference cells |
| 1 | Non Colliding | 4 | 2 | 2 |

Table 4.1.4-2: EPDCCH simulation assumptions

| Parameter | Value |  |
| --- | --- | --- |
| Duplexing | FDD |  |
| System bandwidth | 10 MHz |  |
| Transmit EVM | 6% |  |
| Antenna models | 2x4 low correlation |  |
| Cell ID pattern | Non-colliding CRS: - Serving cell: 0 - Interference cell 1: 1 - Interference cell 2: 6 |  |
| Network synchronization in time | All cells are synchronous |  |
|  | Time-delay wrt. serving cell |  |
|  | 1st interfering cell | 2nd interfering cell |
|  | 2 µs | 3 µs |
| Network synchronization in frequency | Frequency shift wrt. serving cell |  |
|  | 1st interfering cell | 2nd interfering cell |
|  | 200 Hz | 300 Hz |
| Channel model | EPA-5Hz for all links. Use different channel seed for between cells. |  |
| Serving cell EPDCCH transmission parameters | AL 2, Localized EPDCCH DCI Format 2C (44 bits) EPDCCH starting symbol is 2. CFI = 1. EPDCCH starting symbol is RRC configured. Aligned control regions and EPDCCH starting symbols in the serving and interference cells Number of EPDCCH Sets Configured = 1 Localized EPDCCH set PRBs {0, 7, 14, 21, 28, 35, 42, 49} EPDCCH is transmitted in all subframes EPDCCH precoding model is in accordance to TS 36.101 B.4.4. and B.4.5 |  |
| Interference model | Same as for PDSCH simulations Model #1: - Interference cell 1: I1/Noc = 13.91 dB - Interference cell 2: I2/Noc = 3.34 dB - Average resource utilization in the interfering cells: 0% Model #2: - Interference cell 1: I1/Noc = 10.44 dB - Interference cell 2: I2/Noc = 4.57 dB - Average resource utilization in the interfering cells: 20% Model #3: - Interference cell 1: I1/Noc = 8.36 dB - Interference cell 2: I2/Noc = 1.66 dB - Average resource utilization in the interfering cells: 50% |  |

## 4.2	Receiver structures and assumptions

### 4.2.1	Baseline reference receivers

The performance of the Enhanced CRS-IM receiver structures is analysed against the following Baseline reference receiver structures which are used to define the minimum performance requirements in the existing LTE systems:

-	PDSCH: LMMSE-IRC detector as defined in TR 36.829 [5];

-	DL Control Channels (PDCCH, PCFICH, PHICH, EPDCCH): LMMSE-MRC detector.

-	The baseline LMMSE-IRC and LMMSE-MRC receivers are assumed to not differentiate CRS or data interference when suppressing them.

### 4.2.2	Enhanced reference receivers

The following Enhanced reference receiver structures are considered in the studies:

-	PDSCH: LMMSE-IRC detector as defined in TR 36.829 [5] combined with a CRS-IM receiver;

-	DL Control Channels: LMMSE-IRC detector as defined in TR 36.829 [5] combined with a CRS-IM receiver (Type A DL Control Channel IM receiver).

The CRS IM receiver defined for Release 11 FeICIC [3] and Release 13 CRS-IM [4] is reused. In particular, the following CRS-IM receiver assumptions are used in the studies:

-	Single FFT processing

-	No restriction on whether mitigated CRS interference is intra-/inter-site.

-	Number of cancelled cells for non-colliding CRS-IM:

-	The PDSCH and DL Control Channels minimum performance requirements agreed to be defined under assumption of single dominant interference cell CRS-IM.

-	UEs are not restricted to use more than 1 cell CRS-IM and this is left up to UE implementation.

During the studies different sub-types of CRS-IM receiver implementation were considered for different scenarios

-	CRS-IM receivers for non-colliding CRS and 4 CRS APs ports (PDSCH Tests #1, #4)

-	CRS-IM Receiver #A1: Full complexity four ports CRS-IM processing.

-	CRS-IM Receiver #A2: Reduced complexity CRS-IM processing (e.g. full complexity for CRS APs 0, 1 and low complexity for CRS APs 2,3).

-	CRS-IM receivers for colliding CRS scenarios with same number of CRS APs in the serving and neighbouring cells (PDSCH Test #5)

-	CRS-IM Receiver #B1: LMMSE-IRC with 1 cell CRS-IM to improve channel estimation.

-	CRS-IM Receiver #B2: LMMSE-IRC with 1 cell CRS-IM to improve channel and interference estimation.

-	CRS-IM receivers for colliding CRS with different number of CRS APs in the serving and neighbouring cells (PDSCH Test #6)

-	CRS-IM Receiver #C1: LMMSE-IRC with covariance matrix estimation on serving cell CRS APs 0-3 and 1 cell CRS-IM.

-	CRS-IM Receiver #C2: LMMSE-IRC with covariance matrix estimation on serving cell CRS APs 2-3 and 1 cell CRS-IM.

### 4.2.3	CRS assistance information assumptions

The performance analysis in the technical report is provided under assumption of ideal knowledge of the dominant interferer cell CRS Assistance information including

-	Physical Cell ID;

-	Number of CRS AP;

-	MBSFN subframe configuration.

Different receivers in terms of availability of CRS assistance information were considered for Enhanced CRS-IM

-	Option 1: CRS-IM receivers without capabilities to detect neighboring cell CRS Assistance information.

-	Option 2: CRS-IM receivers capable of blind detection of the number of CRS-APs in the interference cells.

The link-level performance evaluation was conducted under assumption of perfect knowledge of the interference cell CRS assistance information.

## 4.3	Link-level performance evaluation

### 4.3.1	PDSCH

In this section we provide the summary of the link-level performance evaluation results for PDSCH test cases. The detailed link-level performance evaluation results for Enhanced CRS-IM for PDSCH from the companies are provided in [7] and [8].

#### 4.3.1.1	PDSCH Test Case #1

The summary of the PDSCH test case #1 (TM4, 4 CRS APs, Non-colliding CRS pattern and 2 UE receiver chains) performance evaluation results based on [7] is provided in Table 4.3.1.1-1. The following observations were made based on the evaluation results:

-	CRS-IM receivers allow achieving noticeable performance improvement (1.9 – 2.6 dB) over baseline LMMSE-IRC receiver for both 16QAM and 64QAM scenarios.

-	Reduced complexity CRS-IM receiver (CRS-IM #A1) has relatively small performance degradation (less than 0.6 dB) comparing to the full complexity CRS-IM receiver (CRS-IM #A2).

Table 4.3.1.1-1: PDSCH test case #1 performance evaluation summary

| Test | FRC | Interference power profile and RU | Average SINR for 70% of max throughput, dB |  |  | CRS-IM gain over LMMSE, dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | LMMSE-IRC | CRS-IM #A1 | CRS-IM #A2 | CRS-IM #A1 | CRS-IM #A2 |
| Test 1 | FRC.2-1 (16QAM) | Profile #1;RU = 20% | -2.1 | -4.7 | -4.5 | 2.6 | 2.5 |
|  | FRC.2-2 (64QAM) | Profile #1;RU = 20% | 2.5 | 0.5 | 0.7 | 2.0 | 1.9 |

#### 4.3.1.2	PDSCH Test Case #2

The summary of the PDSCH test case #2 (TM4, 2 CRS APs, Non-colliding CRS pattern and 4 UE receiver chains) performance evaluation results based on [7] is provided in Table 4.3.1.2-1. The following observations were made based on the evaluation results:

-	CRS-IM receiver allows achieving noticeable performance improvement (up to 2.5 dB) over baseline LMMSE-IRC receiver for both 16QAM and 64QAM scenarios.

Table 4.3.1.2-1: PDSCH test case #2 performance evaluation summary

| Test | FRC | Interference power profile and RU | Average SINR for 70% of max throughput, dB |  | CRS-IM gain over LMMSE, dB |
| --- | --- | --- | --- | --- | --- |
|  |  |  | LMMSE-IRC | CRS-IM |  |
| Test 2 | FRC.1-1 (16QAM) | Profile #1;RU = 20% | -3.9 | -6.4 | 2.5 |
|  | FRC.1-2 (64QAM) | Profile #1;RU = 20% | -0.5 | -2.8 | 2.3 |

#### 4.3.1.3	PDSCH Test Case #3

The summary of the PDSCH test case #3 (TM9, 2 CRS APs, Non-colliding CRS pattern and 4 UE receiver chains) performance evaluation results based on [7] is provided in Table 4.3.1.3-1. The following observations were made based on the evaluation results:

-	CRS-IM receiver allows achieving noticeable performance improvement (1.8 - 2.3 dB) over baseline LMMSE-IRC receiver for both 16QAM and 64QAM scenarios.

Table 4.3.1.3-1: PDSCH test case #3 performance evaluation summary

| Test | FRC | Interference power profile and RU | Average SINR for 70% of max throughput, dB |  | CRS-IM gain over LMMSE, dB |
| --- | --- | --- | --- | --- | --- |
|  |  |  | LMMSE-IRC | CRS-IM |  |
| Test 3 | FRC.3-1 (16QAM) | Profile #1;RU = 20% | -4.0 | -6.2 | 2.3 |
|  | FRC.3-2 (64QAM) | Profile #1;RU = 20% | -0.2 | -2.0 | 1.8 |

#### 4.3.1.4	PDSCH Test Case #4

The summary of the PDSCH test case #4 (TM4, 4 CRS APs, Non-colliding CRS pattern and 4 UE receiver chains) performance evaluation results based on [7] is provided in Table 4.3.1.4-1. The following observations were made based on the evaluation results:

-	CRS-IM receivers allow achieving noticeable performance improvement (2.3 – 2.9 dB) over baseline LMMSE-IRC receiver for both 16QAM and 64QAM scenarios.

-	Reduced complexity CRS-IM receiver (CRS-IM #A1) has relatively small performance degradation (less than 0.5 dB) comparing to the full complexity CRS-IM receiver (CRS-IM #A2).

Table 4.3.1.4-1: PDSCH test case #4 performance evaluation summary

| Test | FRC | Interference power profile and RU | Average SINR for 70% of max throughput, dB |  |  | CRS-IM gain over LMMSE, dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | LMMSE-IRC | CRS-IM #A1 | CRS-IM #A2 | CRS-IM #A1 | CRS-IM #A2 |
| Test 4 | FRC.2-1 (16QAM) | Profile #1; RU = 20% | -4.8 | -7.6 | -7.4 | 2.9 | 2.7 |
|  | FRC.2-2 (64QAM) | Profile #1; RU = 20% | -0.7 | -3.1 | -2.9 | 2.4 | 2.3 |

#### 4.3.1.5	PDSCH Test Case #5

The summary of the PDSCH test case #5 performance evaluation results based on [8] is provided in Table 4.3.1.5-1. The following observations were made based on the evaluation results:

-	CRS-IM receiver #B1:

-	Provides limited performance improvement for all investigated scenarios.

-	The largest performance improvement (1.2 dB) over baseline LMMSE-IRC receiver is observed for the scenarios no PDSCH transmissions in the neighboring cells (interference Profile 2 and 0% RU).

-	For other scenarios the performance gains are in the range from 0.2 dB to 0.6 dB.

-	CRS-IM receiver #B2:

-	Provides noticeable performance gains (2.4 dB) over LMMSE-IRC receivers for the scenarios with no PDSCH transmissions in the neighboring cells (interference Profile 2 and 0% RU).

-	The gains for the scenarios with interference Profile 1 and 20% RU are reduced to 1.3 dB.

-	In the scenarios with low interference power (Profile 3), the CRS-IM receiver #B2 provides robust performance and does not lead to performance degradation comparing to the LMMSE-IRC receiver.

Table 4.3.1.5-1: PDSCH test case #5 performance evaluation summary

| Test | FRC | Interference power profile and RU | Average SINR for 70% of max throughput, dB |  |  | CRS-IM gain over LMMSE, dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | LMMSE-IRC | CRS-IM #B1 | CRS-IM #B2 | CRS-IM #B1 | CRS-IM #B2 |
| Test 5 | FRC.2-1 (16QAM) | Profile #1; RU = 20% | 1.3 | 0.7 | 0.0 | 0.6 | 1.3 |
|  |  | Profile #2; RU = 0% | -2.4 | -3.5 | -4.7 | 1.2 | 2.4 |
|  |  | Profile #3; RU = 20% | 3.0 | 2.7 | 2.6 | 0.2 | 0.4 |

#### 4.3.1.6	PDSCH Test Case #6

The summary of the PDSCH test case #6 performance evaluation results based on [8] is provided in Table 4.3.1.6-1. The following observations were made based on the evaluation results:

-	CRS-IM receiver #C1

-	Provides limited performance improvement for the investigated scenarios (0.3 to 2.0 dB).

-	The largest performance improvement (2 dB) over baseline LMMSE-IRC receiver is observed for the scenarios no PDSCH transmissions in the neighboring cells (interference Profile 2 and 0% RU).

-	CRS-IM receiver #C2

-	Provides relatively large performance improvement for the investigated scenarios (0.4 to 3 dB).

-	The largest performance improvement (3 dB) over baseline LMMSE-IRC receiver is observed for the scenarios no PDSCH transmissions in the neighboring cells (interference Profile 2 and 0% RU).

-	The gains for the scenarios with interference Profile 1 and 20% RU are reduced to 1.5 dB.

-	In the scenarios with low interference power (Profile 3), the CRS-IM receiver #C2 provides robust performance and does not lead to performance degradation comparing to the LMMSE-IRC receiver.

Table 4.3.1.6-1: PDSCH test case #6 performance evaluation summary

| Test | FRC | Interference power profile and RU | Average SINR for 70% of max throughput, dB |  |  | CRS-IM gain over LMMSE, dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | LMMSE-IRC | CRS-IM #C1 | CRS-IM #C2 | CRS-IM #C1 | CRS-IM #C2 |
| Test 6 | FRC.2-1 (16QAM) | Profile #1;RU = 20% | 0.1 | -0.8 | -1.4 | 0.9 | 1.5 |
|  |  | Profile #2; RU = 0% | -3.7 | -5.7 | -6.7 | 2.0 | 3.0 |
|  |  | Profile #3; RU = 20% | 2.4 | 2.1 | 2.0 | 0.3 | 0.4 |

#### 4.3.1.7	PDSCH Test Case #7

The summary of the PDSCH test case #7 performance evaluation results based on [7] is provided in Table 4.3.1.7-1. The following observations were made based on the evaluation results:

-	CRS-IM receiver provides limited performance improvement (~1dB) over baseline LMMSE-IRC receiver for the investigated scenarios.

Table 4.3.1.7-1: PDSCH test case #7 performance evaluation summary

| Test | FRC | Interference power profile and RU | Average SINR for 70% of max throughput, dB |  | CRS-IM gain over LMMSE, dB |
| --- | --- | --- | --- | --- | --- |
|  |  |  | LMMSE-IRC | CRS-IM |  |
| Test 7 | FRC.1-1 (16QAM) | Profile #1; RU = 20% | 0.4 | -0.7 | 1.1 |
|  | FRC.1-2 (64QAM) | Profile #1; RU = 20% | 3.2 | 2.2 | 1.0 |

### 4.3.2	PDCCH/PCFICH

In this section we provide the summary of the link-level performance evaluation results for Enhanced CRS-IM for PDCCH/PCFICH.

#### 4.3.2.1	PDCCH/PCFICH Test Case #1

The summary of the PDCCH/PCFICH test case #1 performance evaluation results based [9] is provided in Table 4.3.2.1-1. The following observations were made based on the evaluation results:

-	LMMSE-IRC receivers provide 0.4 – 0.5 dB performance gains over LMMSE-MRC receivers for the investigated scenarios.

-	CRS-IM receivers provide 1.5 – 2.7 dB performance gains over LMMSE-MRC receivers for the investigated scenarios.

-	CRS-IM receivers provide 1.0 – 2.3 dB performance gains over LMMSE-IRC receivers for the investigated scenarios.

-	CRS-IM receiver performance improvement in case of CFI=2 is larger in comparison with CFI=3 case.

Table 4.3.2.1-1: PDCCH/PCFICH test case #1 performance evaluation summary

| Test | CFI | Interference Profile | Average SINR @ 1% Pm-dsg, dB |  |  | Gain over LMMSE-MRC, dB |  | Gain over LMMSE-IRC, dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | LMMSE-MRC | LMMSE-IRC | CRS-IM | LMMSE-IRC | CRS-IM | CRS-IM |
| Test 1 | 2 | Profile #1 | 0.2 | -0.3 | -2.7 | 0.5 | 2.9 | 2.4 |
|  |  | Profile #2 | 0.1 | -0.3 | -1.9 | 0.4 | 2.0 | 1.6 |
|  | 3 | Profile #1 | -0.2 | -0.7 | -2.0 | 0.5 | 1.8 | 1.3 |
|  |  | Profile #2 | -0.2 | -0.7 | -1.7 | 0.4 | 1.5 | 1.0 |

#### 4.3.2.2	PDCCH/PCFICH Test Case #2

The summary of the PDCCH/PCFICH test case #2 performance evaluation results based on [9] is provided in Table 4.3.2.2-1. The following observations were made based on the evaluation results:

-	LMMSE-IRC receivers provide 2.3 – 3.9 dB performance gains over LMMSE-MRC receivers for the investigated scenarios.

-	CRS-IM receivers provide 2.9 – 6.0 dB performance gains over LMMSE-MRC receivers for the investigated scenarios.

-	CRS-IM receivers provide 0.2 – 2.1 dB performance gains over LMMSE-IRC receivers for the investigated scenarios.

-	For the investigated scenarios with PDCCH AL 2 and low antenna correlation, the SINR operating point for the CRS-IM receiver SINR operating point is below typical UE SINR operating range.

Table 4.3.2.2-1: PDCCH/PCFICH test case #2 performance evaluation summary

| Test | CFI | Interference Profile | Average SINR @ 1% Pm-dsg, dB |  |  | Gain over LMMSE-MRC, dB |  | Gain over LMMSE-IRC, dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | LMMSE-MRC | LMMSE-IRC | CRS-IM | LMMSE-IRC | CRS-IM | CRS-IM |
| Test 2 | 1 | Profile #1 | -3.1 | -7.0 | -9.1 | 3.9 | 6.0 | 2.1 |
|  |  | Profile #2 | -3.3 | -5.6 | -7.1 | 2.3 | 3.8 | 1.5 |
|  | 3 | Profile #1 | -3.4 | -7.1 | -7.5 | 3.7 | 4.1 | 0.4 |
|  |  | Profile #2 | -3.2 | -5.9 | -6.1 | 2.7 | 2.9 | 0.2 |

#### 4.3.2.3	PDCCH/PCFICH Test Case #3

The summary of the PDCCH/PCFICH test case #3 performance evaluation results based on R4-1702913 is provided in Table 4.3.2.3-1. The following observations were made based on the evaluation results:

-	LMMSE-IRC receivers provide 2.2 – 2.6 dB performance gains over LMMSE-MRC receivers for the investigated scenarios.

-	CRS-IM receivers provide 3.7 – 4.6 dB performance gains over LMMSE-MRC receivers for the investigated scenarios.

-	CRS-IM receivers provide 1.4 – 2.0 dB performance gains over LMMSE-IRC receivers for the investigated scenarios.

-	For PDCCH AL 1 the CRS-IM receiver operating point is -7.8 dB which below typical UE SINR operating range. For the PDCCH AL 1 the CRS-IM receiver operating point is -0.9 dB which in the typical UE SINR operating range.

Table 4.3.2.3-1: PDCCH/PCFICH test case #3 performance evaluation summary

| Test | CFI | PDCCH AL | Interference Profile | Average SINR @1% Pm-dsg, dB |  |  | Gain over LMMSE-MRC, dB |  | Gain over LMMSE-IRC, dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | LMMSE-MRC | LMMSE-IRC | CRS-IM | LMMSE-IRC | CRS-IM | CRS-IM |
| Test 3 | 2 | 2 | Profile #1 | -3.2 | -5.8 | -7.8 | 2.6 | 4.6 | 2.0 |
|  |  | 1 |  | 2.8 | 0.5 | -0.9 | 2.2 | 3.7 | 1.4 |

### 4.3.3	PHICH

In this section we provide the summary of the link-level performance evaluation results for Enhanced CRS-IM for PHICH.

#### 4.3.3.1	PHICH Test Case #1

The summary of the PHICH test case #1 performance evaluation results based on [9] is provided in Table 4.3.3.1-1. The following observations were made based on the evaluation results:

-	LMMSE-IRC receivers provide 0.4 – 0.5 dB performance gains over LMMSE-MRC receivers for the investigated scenarios.

-	CRS-IM receivers provide 1.3 – 1.7 dB performance gains over LMMSE-MRC receivers for the investigated scenarios.

-	CRS-IM receivers provide 0.9 – 1.3 dB performance gains over LMMSE-IRC receivers for the investigated scenarios.

Table 4.3.3.1-1: PHICH test case #1 performance evaluation summary

| Test | Interference Profile | Average SINR @ 0.1% Pm-an, dB |  |  | Gain over LMMSE-MRC, dB |  | Gain over LMMSE-IRC, dB |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LMMSE-MRC | LMMSE-IRC | CRS-IM | LMMSE-IRC | CRS-IM | CRS-IM |
| Test 1 | Profile #1 | -0.3 | -0.8 | -1.9 | 0.5 | 1.6 | 1.1 |
|  | Profile #2 | -0.7 | -1.2 | -2.1 | 0.5 | 1.3 | 0.9 |

#### 4.3.3.2	PHICH Test Case #2

The summary of the PHICH test case #2 performance evaluation results based on summary of results in [9] is provided in Table 4.3.3.2-1. The following observations were made based on the evaluation results:

-	LMMSE-IRC receivers provide 2.3 – 3.7 dB performance gains over LMMSE-MRC receivers for the investigated scenarios.

-	CRS-IM receivers provide 3.8 – 5.5 dB performance gains over LMMSE-MRC receivers for the investigated scenarios.

-	CRS-IM receivers provide 1.4 – 1.8 dB performance gains over LMMSE-IRC receivers for the investigated scenarios.

-	For the investigated scenarios the CRS-IM receiver operating SINR point is below -6dB dB which below typical UE SINR operating range.

Table 4.3.3.2-1: PHICH test case #2 performance evaluation summary

| Test | Interference Profile | Average SINR @ 0.1% Pm-an, dB |  |  | Gain over LMMSE-MRC, dB |  | Gain over LMMSE-IRC, dB |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LMMSE-MRC | LMMSE-IRC | CRS-IM | LMMSE-IRC | CRS-IM | CRS-IM |
| Test 2 | Profile #1 | -2.8 | -6.6 | -8.4 | 3.7 | 5.5 | 1.8 |
|  | Profile #2 | -2.9 | -5.2 | -6.7 | 2.3 | 3.8 | 1.4 |

#### 4.3.3.3	PHICH Test Case #3

The summary of the PHICH test case #3 performance evaluation results based on summary of results in R4-1702913 is provided in Table 4.3.3.3-1. The following observations were made based on the evaluation results:

-	LMMSE-IRC receivers provide 2.6 dB performance gains over LMMSE-MRC receivers for the investigated scenarios.

-	CRS-IM receivers provide 4.0 dB performance gains over LMMSE-MRC receivers for the investigated scenarios.

-	CRS-IM receivers provide 1.4 dB performance gains over LMMSE-IRC receivers for the investigated scenarios.

-	For the investigated scenarios the CRS-IM receiver operating SINR point is below -6dB dB which below typical UE SINR operating range.

Table 4.3.3.3-1: PHICH test case #3 performance evaluation summary

| Test | Interference Profile | Average SINR @ 0.1% Pm-an, dB |  |  | Gain over LMMSE-MRC, dB |  | Gain over LMMSE-IRC, dB |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LMMSE-MRC | LMMSE-IRC | CRS-IM | LMMSE-IRC | CRS-IM | CRS-IM |
| Test 3 | Profile #1 | -2.8 | -5.5 | -6.8 | 2.6 | 4.0 | 1.4 |

### 4.3.4	EPDCCH

In this section we provide the summary of the link-level performance evaluation results for Enhanced CRS-IM for EPDCCH.

The summary of the performance evaluation results based on summary of results in [9] is provided in Table 4.3.4-1. The following observations were made based on the evaluation results:

-	LMMSE-IRC receivers provide up to 4.0 performance gains over LMMSE-MRC receivers for the Model #2 and #3 scenarios. For the Model #1 scenario LMMSE-IRC receiver provides slight performance loss of 0.4 dB comparing to the LMMSE-MRC receiver.

-	CRS-IM receivers provide 5.1 – 6.3 dB performance gains over LMMSE-MRC receivers for the investigated scenarios.

-	CRS-IM receivers provide 1.2 – 5.5 dB performance gains over LMMSE-IRC receivers for the investigated scenarios.

Table 4.3.4-1: EPDCCH test case #1 performance evaluation summary

| Test | Interference model | Average SINR @1% Pm-dsg, dB |  |  | Gain over LMMSE-MRC, dB |  | Gain over LMMSE-IRC, dB |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LMMSE-MRC | LMMSE-IRC | CRS-IM | LMMSE-IRC | CRS-IM | CRS-IM |
| Test 1 | Model #1 | -5.0 | -4.6 | -10.1 | -0.4 | 5.1 | 5.5 |
|  | Model #2 | 0.5 | -3.5 | -5.8 | 4.0 | 6.3 | 2.3 |
|  | Model #3 | 2.2 | -1.8 | -3.0 | 4.0 | 5.2 | 1.2 |

## 4.4	Conclusions

The following conclusions were made on the benefits and feasibility of enhanced CRS-IM receivers:

-	PDSCH

-	PDSCH test cases #1, 2, 3, 4: The performance benefits and feasibility of CRS-IM receivers in application to Tests 1, 2, 3, and 4 are confirmed. The performance requirements to verify CRS-IM performance for such scenarios agreed to be defined in the TS 36.101.

-	PDSCH test case #5: The performance gains of CRS-IM receiver #B2 were confirmed. No consensus reached on the interferer PDSCH presence blind detection feasibility for CRS-IM receiver #B2. It was agreed that the minimum performance requirements will not be defined in the TS 36.101 and the case is left up to UE implementation.

-	PDSCH test case #6: The performance gains and feasibility of CRS-IM receiver #C1 and #C2 were confirmed. It was agreed that the minimum performance requirements will not be defined in the TS 36.101 and the case is left up to UE implementation.

-	PDSCH test case #7: It was agreed that the minimum performance requirements will not be defined in the TS 36.101 due to limited CRS-IM receiver performance gains.

-	PDCCH/PCFICH

-	PDCCH/PCFICH test case #1: The performance benefits and feasibility of CRS-IM receivers were confirmed. The performance requirements to verify CRS-IM performance agreed to be defined in the TS 36.101.

-	PDCCH/PCFICH test case #2: No consensus reached on the performance benefits and feasibility of CRS-IM receivers. It was agreed that the minimum performance requirements will not be defined in the TS 36.101 and the case is left up to UE implementation.

-	PDCCH/PCFICH test case #3: The performance benefits and feasibility of LMMSE-IRC receivers were confirmed. No consensus reached on the performance benefits and feasibility of CRS-IM receivers. It was agreed that the minimum performance requirements will be defined in the TS 36.101 based on the LMMSE-IRC receiver.

-	PHICH

-	PHICH test case #1: The performance benefits and feasibility of CRS-IM receivers were confirmed. It was agreed that the minimum performance requirements will not be defined in the TS 36.101 and the case is left up to UE implementation.

-	PHICH test case #2 and #3: No consensus reached on the performance benefits and feasibility of CRS-IM receivers. It was agreed that the minimum performance requirements will not be defined in the TS 36.101 and the case is left up to UE implementation.

-	EPDCCH

-	The performance gains and feasibility of CRS-IM receiver were confirmed for the investigated scenarios.

-	Work on the CRS-IM performance requirements definition for EPDCCH is deprioritized. The minimum performance requirements will not be defined in the TS 36.101 and the case is left up to UE implementation.

# 5	Enhanced SU-MIMO IM based on 4RX

In Release 12, enhanced SU-MIMO IM receivers were introduced to improve the DL spectral efficiency for UEs equipped with two receive antennas for the scenarios with multiple MIMO stream PDSCH transmissions. The investigated enhanced SU-MIMO IM receivers were proven to be able to handle inter-stream interference more efficiently comparing to the conventional LMMSE receivers. The corresponding enhanced performance requirements were defined in application to the UEs equipped with 2 receive (RX) antennas and for MIMO rank 2 scenarios.

The objective of the work item [2] is to investigate performance benefits, complexity, and feasibility of using SU-MIMO IM receivers in application to the UEs equipped with 4 receive antennas:

-	Investigate and agree on target scenarios including layer number for spatial multiplexing, MCS, propagation conditions, antenna correlation and others for performance evaluation

-	Identify the reference receiver structure for SU-MIMO IM evaluation

-	Evaluate the performance of enhanced SU-MIMO IM receivers

This section provides the summary of the analysis of the enhanced SU-MIMO IM receivers for UEs equipped with 4 receive antennas and includes the following sub-clauses:

-	Clause 5.1 describes the simulation scenarios used for SU-MIMO IM performance analysis;

-	Clause 5.2 describes the receiver structure used for evaluation of enhanced SU-MIMO IM;

-	Clause 5.3 summarizes the results of the link-level performance evaluations.

-	Clause 5.4 provides summary of conclusions on enhanced SU-MIMO IM investigations.

## 5.1	Scenarios

This section describes the evaluation scenarios and simulation assumptions used for the investigations of performance benefits and feasibility for enhanced SU-MIMO IM receiver with 4 receive antennas.

The performance analysis covers a wide set of scenarios:

-	Modulation order: from 16QAM and  up to 256QAM

-	MIMO rank 2, 3 and 4

-	PDSCH transmission modes 3, 4 and 9

In Table 5.1-1, simulation assumptions for scenarios to evaluate enhanced SU-MIMO IM receiver performance are provided. The scenarios with 64QAM with rank 4, 256QAM with rank 3 and rank 4 are deprioritized in terms of complexity and performance. In Table 5.1-2, the evaluated antenna correlation scenarios are provided. The analysis is provided under assumption of noise-limited scenarios without any inter-cell interference.

Table 5.1-1: Scenarios for evaluation

| Test case | MIMO Rank | TM | Modulation and Code Rate | Channel model | Antenna configuration |
| --- | --- | --- | --- | --- | --- |
| 1 | Rank 2 | TM4 | 16QAM CR ½ | ETU70 | 2x4 |
| 2 |  | TM9 | 16QAM CR ½ | EPA5 | 2x4 |
| 3 |  | TM4 | 64QAM CR ½ | EPA5 | 2x4 |
| 4 |  | TM9 | 64QAM CR ½ | EPA5 | 2x4 |
| 5 |  | TM4 | 64QAM CR ½ | EPA5 | 4x4 |
| 6 |  | TM9 | 64QAM CR ½ | EPA5 | 4x4 |
| 7 |  | TM4 | 256QAM CR 0.62 | EPA5 | 4x4 |
| 8 |  | TM9 | 256QAM CR 0.67 | EPA5 | 4x4 |
| 9 | Rank3 | TM3 | 16QAM CR ½ | EVA70 | 4x4 |
| 10 |  | TM3 | 64QAM CR 0.43 | EPA5 | 4x4 |
| 11 | Rank4 | TM4 | 16QAM CR ½ | EPA5 | 4x4 |
| 12 |  | TM9 | 16QAM CR 0.57 | EPA5 | 4x4 |

Table 5.1-2: Antenna correlation scenario for evaluations

|  | Antenna Correlation |
| --- | --- |
| AC #1 | ULA Low correlation |
| AC #2 | ULA Medium correlation (α=0.3, β=0.9) |
| AC #3 | ULA Medium A correlation (α=0.3, β=0.3874) |
| AC #4 | XPOL Medium A correlation (α=0.3, β=0.6, γ=0.2) |

## 5.2	Receiver structures and assumptions

The baseline PDSCH SU-MIMO performance requirements for UEs equipped with both 2 and 4 receive antennas are based on the conventional LMMSE receivers as described in the TR 36.829 [5].

Enhanced non-linear receivers can be used to improve the SU-MIMO performance for the case rank 2 or more PDSCH signal transmissions.

For UEs equipped with 2 receive antennas in Release 12 Enhanced SU-MIMO IM work item two enhanced receiver structures were considered to improve the performance including R-ML (Reduced complexity ML) and CWIC (Code word level – IC) receivers as described in the TR 36.866 [6]. Both receiver structures were shown to provide sufficient and comparable performance improvement over baseline LMMSE receivers and the exact decision on the enhanced SU-MIMO IM receiver structure was left up to UE implementation.

For UEs equipped with 4 receive antennas, the R-ML receiver is considered as the only candidate enhanced SU-MIMO IM receiver structure, while CWIC receivers are considered to be relatively complex for practical implementation.

R-ML receivers are assumed to perform reduced complexity joint detection of multiple MIMO layers modulation symbols in accordance to the maximum likelihood (ML) criterion. R-ML receiver may have different implementations (e.g. sphere decoding, QR-MLD, MLM, etc.). In addition, there may be some adjustable receiver parameters that have impact on the R-ML receiver performance (e.g. the radius of the sphere decoder, the final candidate set size for LLR calculation and so on). The exact selection of R-ML receiver algorithm and parameters depends on the UE implementation.

## 5.3	Link-level performance evaluation

In this section, link-level evaluation parameters and results are provided.

### 5.3.1		Parameters for link-level evaluation

The common test parameters (FDD) are given in Table 5.3.1-1. The evaluation scenarios with specific parameters are listed in Table 5.1-1 including rank 2, 3 and 4. For each scenario, in order to find cases with testable gain, different antenna correlation options are considered in Table 5.1-2, including ULA low, ULA medium, ULA medium A, ULA high and XPOL medium A.

Table 5.3.1-1: Common parameters for evaluations (FDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Bandwidth | MHz | 10 |
| Inter-TTI Distance |  | 1 |
| Number of HARQ processes per component carrier | Processes | 8 |
| Maximum number of HARQ transmission |  | 4 |
| Redundancy version coding sequence |  | {0,1,2,3} for QPSK and 16QAM {0,0,1,2} for 64QAM and 256QAM |
| Number of OFDM symbols for PDCCH | OFDM symbols | 2 for 10 MHz |
| Reporting Mode |  | PUSCH 3-1 |
| Cyclic Prefix |  | Normal |
| Cell ID |  | 0 |
| Tx EVM |  | 6% for 16QAM and 64QAM; 3% for 256QAM |
| Reference receiver |  | LMMSE, R-ML |

### 5.3.2	Link-level simulation results

In this section the summary of the link-level performance evaluation results for enhanced SU-MIMO is provided:

-	MIMO rank 2 scenarios:

-	Table 5.3.2-1: SNR required to reach 70% of max throughput for enhanced R-ML and baseline LMMSE receivers;

-	Table 5.3.2-2: R-ML receiver SNR gain over LMMSE receiver for the 70% of max throughput;

-	MIMO rank 3 scenarios:

-	Table 5.3.2-3: SNR required to reach 70% of max throughput for enhanced R-ML and baseline LMMSE receivers;

-	Table 5.3.2-4: R-ML receiver SNR gain over LMMSE receiver for the 70% of max throughput;

-	MIMO rank 4 scenarios:

-	Table 5.3.2-5: SNR required to reach 70% of max throughput for enhanced R-ML and baseline LMMSE receivers;

-	Table 5.3.2-6: R-ML receiver SNR gain over LMMSE receiver for the 70% of max throughput.

Detailed link-level evaluation results from companies are captured in [10-13].

Note: N/A means that throughput curve does not reach 70 % of maximum throughput for evaluated SNR region. Empty means that results are unavailable.

Table 5.3.2-1: Simulation results summary - SNR for 70% of max throughput (MIMO Rank 2)

| Test case | Antenna correlation | Receiver | SNR for 70% of max throughput, [dB] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Company 1 | Company 2 | Company 3 | Company 4 | Company 5 | Company 6 | Average | Span | STD |
| Test #1 TM4 16QAM 2x4 | AC #2 ULA Med | R-ML | 14.7 | 15.3 | 15.9 | 13.9 | 13.5 | 14.8 | 14.7 | 2.4 | 0.9 |
|  |  | LMMSE | 18.9 | 18.2 | 18.3 | 18.2 | 16.2 | 17.5 | 17.9 | 2.7 | 0.9 |
| Test #2 TM9 16QAM 2x4 | AC #2 ULA Med | R-ML | 14.1 | 13.2 | 14.3 | 13.4 | 13.9 | 14.4 | 13.9 | 1.2 | 0.5 |
|  |  | LMMSE | 17.3 | 16.0 | 17.0 | 17.7 | 16.6 | 17.5 | 17.0 | 1.7 | 0.6 |
| Test #3 TM4 64QAM 4x4 | AC #3 ULA Med A | R-ML | 20.6 | 17.1 | 16.5 | 18.6 |  | 19.1 | 18.4 | 4.2 | 1.7 |
|  |  | LMMSE | 21.9 | 18.4 | 18.8 | 21.0 |  | 20.8 | 20.2 | 3.4 | 1.5 |
|  | AC #2 ULA Med | R-ML | 12.5 | 11.4 | 11.5 | 12.1 |  | 12.4 | 12.0 | 1.1 | 0.5 |
|  |  | LMMSE | 13.6 | 11.7 | 12.0 | 12.9 |  | 12.8 | 12.6 | 1.9 | 0.8 |
| Test #4 TM9 64QAM 4x4 | AC #2 ULA Med | R-ML | 22.0 | 19.3 | 19.3 |  |  | 21.7 | 20.6 | 2.6 | 1.4 |
|  |  | LMMSE | 23.4 | 20.6 | 27.3 |  |  | 23.8 | 23.8 | 6.7 | 2.8 |
|  | AC #3 ULA Med A | R-ML | 14.4 | 13.3 | 17.4 |  |  | 14.2 | 14.8 | 4.1 | 1.8 |
|  |  | LMMSE | 16.0 | 13.6 | 20.4 |  |  | 14.8 | 16.2 | 6.8 | 3.0 |
| Test #5 TM4 256QAM 4x4 | AC #1 ULA Low | R-ML | 18.5 | 14.8 | 15.1 |  |  |  | 16.1 | 3.7 | 2.1 |
|  |  | LMMSE | 19.3 | 14.7 | 15.1 |  |  |  | 16.4 | 4.6 | 2.6 |
|  | AC #2 ULA Med | R-ML | 24.6 | 26.2 | 26.8 |  |  |  | 25.9 | 2.2 | 1.1 |
|  |  | LMMSE | 26.3 | 27.3 | 27.3 |  |  |  | 27.0 | 1.1 | 0.6 |
|  | AC #3 ULA Med A | R-ML | 17.5 | 19.7 | 18.7 |  |  |  | 18.6 | 2.2 | 1.1 |
|  |  | LMMSE | 19.4 | 20.1 | 20.5 |  |  |  | 20.0 | 1.1 | 0.6 |
| Test #6 TM9 256QAM 4x4 | AC #1 ULA Low | R-ML | 18.5 | 16.4 | 14.4 |  |  |  | 16.4 | 4.1 | 2.0 |
|  |  | LMMSE | 20.9 | 16.3 | 14.4 |  |  |  | 17.2 | 6.5 | 3.4 |
|  | AC #2 ULA Med | R-ML | N/A | 28.3 | 22.1 |  |  |  | 25.2 | 6.2 | 4.4 |
|  |  | LMMSE | N/A | 29.5 | 24.6 |  |  |  | 27.0 | 4.9 | 3.5 |
|  | AC #3 ULA Med A | R-ML | 25.1 | 21.6 | 18.1 |  |  |  | 21.6 | 7.0 | 3.5 |
|  |  | LMMSE | 27.5 | 22.0 | 18.6 |  |  |  | 22.7 | 9.0 | 4.5 |

Table 5.3.2-2: Simulation results summary – R-ML receiver gain vs. LMMSE (MIMO Rank 2)

| Test case | Antenna correlation | R-ML receiver SNR gain vs. LMMSE for 70% of max throughput, [dB] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Company 1 | Company 2 | Company 3 | Company 4 | Company 5 | Company 6 | Average | Span | STD |
| Test #1 TM4 16QAM 2x4 | AC #2 ULA Med | 4.3 | 2.9 | 2.4 | 4.3 | 2.7 | 2.7 | 3.2 | 2.0 | 0.8 |
| Test #2 TM9 16QAM 2x4 | AC #2 ULA Med | 3.2 | 2.8 | 2.7 | 4.3 | 2.7 | 3.1 | 3.1 | 1.7 | 0.6 |
| Test #3 TM4 64QAM 4x4 | AC #3 ULA Med A | 1.2 | 1.3 | 2.3 | 2.4 |  | 1.7 | 1.8 | 1.2 | 0.5 |
|  | AC #2 ULA Med | 1.1 | 0.3 | 0.5 | 0.8 |  | 0.4 | 0.6 | 0.8 | 0.3 |
| Test #4 TM9 64QAM 4x4 | AC #3 ULA Med A | 1.4 | 1.2 | 7.9 |  |  | 2.1 | 3.2 | 6.7 | 2.8 |
|  | AC #2 ULA Med | 1.6 | 0.3 | 3.0 |  |  | 0.6 | 1.4 | 2.7 | 1.1 |
| Test #5 TM4 256QAM 4x4 | AC #1 ULA Low | 0.9 | 0.0 | 0.0 |  |  |  | 0.3 | 0.9 | 0.5 |
|  | AC #2 ULA Med | 1.7 | 1.1 | 0.5 |  |  |  | 1.1 | 1.2 | 0.6 |
|  | AC #3 ULA Med A | 1.9 | 0.3 | 1.8 |  |  |  | 1.3 | 1.6 | 0.9 |
| Test #6 TM9 256QAM 4x4 | AC #1 ULA Low | 2.5 | 0.0 | 0.1 |  |  |  | 0.9 | 2.5 | 1.4 |
|  | AC #2 ULA Med |  | 1.1 | 2.5 |  |  |  | 1.8 | 1.4 | 1.0 |
|  | AC #3 ULA Med A | 2.4 | 0.4 | 0.5 |  |  |  | 1.1 | 2.0 | 1.1 |

Table 5.3.2-3: Simulation results summary - SNR for 70% of max throughput (MIMO Rank 3)

| Test case | Antenna correlation | Receiver | SNR for 70% of max throughput, [dB] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Company 1 | Company 2 | Company 3 | Company 4 | Company 5 | Company 6 | Average | Span | STD |
| Test #7 TM3 16QAM 4x4 | AC #1 ULA Low | R-ML | 9.2 | 9.1 | 9.2 |  |  |  | 9.2 | 0.1 | 0.1 |
|  |  | LMMSE | 10.4 | 9.2 | 9.5 |  |  |  | 9.7 | 1.2 | 0.6 |
|  | AC #2 ULA Med | R-ML | N/A | 27.7 | 24.5 |  |  |  | 26.1 | 3.2 | 2.3 |
|  |  | LMMSE | N/A | N/A | N/A |  |  |  |  |  |  |
|  | AC #3 ULA Med A | R-ML | 21.4 | 19.6 | 20.6 | 19.1 | 19.6 | 19.4 | 19.9 | 2.3 | 0.9 |
|  |  | LMMSE | 28.6 | 25.4 | 24.9 | 24.5 | 26.1 | 25.6 | 25.8 | 4.1 | 1.4 |
|  | AC #4 XPL Med A | R-ML | 10.8 | 10.6 |  |  |  |  | 10.7 | 0.2 | 0.1 |
|  |  | LMMSE | 11.7 | 10.8 |  |  |  |  | 11.3 | 0.9 | 0.6 |
| Test #8 TM3 64QAM 4x4 | AC #1 ULA Low | R-ML |  | 11.7 | 12.3 |  |  |  | 12.0 | 0.6 | 0.4 |
|  |  | LMMSE |  | 11.7 | 12.3 |  |  |  | 12.0 | 0.5 | 0.4 |
|  | AC #2 ULA Med | R-ML |  | N/A | N/A |  |  |  |  |  |  |
|  |  | LMMSE |  | N/A | N/A |  |  |  |  |  |  |
|  | AC #3 ULA Med A | R-ML | N/A | 24.8 | 23.3 |  |  |  | 24.1 | 1.5 | 1.1 |
|  |  | LMMSE | 31.6 | 26.7 | 23.9 |  |  |  | 27.4 | 7.8 | 3.9 |
|  | AC #4 XPL Med A | R-ML | 13.6 | 13.1 |  | 13.4 |  |  | 13.3 | 0.5 | 0.3 |
|  |  | LMMSE | 15.2 | 13.2 |  | 13.4 |  |  | 13.9 | 2.0 | 1.1 |

Table 5.3.2-4: Simulation results summary – R-ML receiver gain vs. LMMSE (MIMO Rank 3)

| Test case | Antenna correlation | R-ML receiver SNR gain vs. LMMSE for 70% of max throughput, [dB] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Company 1 | Company 2 | Company 3 | Company 4 | Company 5 | Company 6 | Average | Span | STD |
| Test #7 TM3 16QAM 4x4 | AC #1 ULA Low | 1.1 | 0.1 | 0.3 |  |  |  | 0.5 | 1.0 | 0.5 |
|  | AC #2 ULA Med |  |  |  |  |  |  |  |  |  |
|  | AC #3 ULA Med A | 7.2 | 5.8 | 4.3 | 5.4 | 6.5 | 6.2 | 5.9 | 2.9 | 0.9 |
|  | AC #4 XPL Med A | 0.9 | 0.2 |  |  |  |  | 0.6 | 0.7 | 0.5 |
| Test #8 TM3 64QAM 4x4 | AC #1 ULA Low |  | 0.1 | 0.0 |  |  |  | 0.1 | 0.1 | 0.1 |
|  | AC #2 ULA Med |  |  |  |  |  |  |  |  |  |
|  | AC #3 ULA Med A |  | 1.9 | 0.6 |  |  |  | 1.3 | 1.3 | 0.9 |
|  | AC #4 XPL Med A | 1.6 | 0.1 |  | 0.0 |  |  | 0.6 | 1.6 | 0.9 |

Table 5.3.2-5: Simulation results summary - SNR for 70% of max throughput (MIMO Rank 4)

| Test case | Antenna correlation | Receiver | SNR for 70% of max throughput, [dB] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Company 1 | Company 2 | Company 3 | Company 4 | Company 5 | Company 6 | Average | Span | STD |
| Test #9 TM4 16QAM 4x4 | AC #1 ULA Low | R-ML |  | 9.2 | 10.1 |  |  |  | 9.7 | 0.9 | 0.6 |
|  |  | LMMSE |  | 9.7 | 10.1 |  |  |  | 9.9 | 0.4 | 0.3 |
|  | AC #2 ULA Med | R-ML | N/A | N/A | 28.7 |  |  |  | 28.7 | 0.0 |  |
|  |  | LMMSE | N/A | N/A | N/A |  |  |  |  |  |  |
|  | AC #3 ULA Med A | R-ML | N/A | 21.2 | N/A |  |  |  | 21.2 | 0.0 |  |
|  |  | LMMSE | N/A | 28.9 | N/A |  |  |  | 28.9 | 0.0 |  |
|  | AC #4 XPL Med A | R-ML | 13.8 | 12.6 | 12.1 | 14.2 |  | 13.0 | 13.1 | 2.1 | 0.9 |
|  |  | LMMSE | 15.3 | 13.4 | 12.7 | 15.1 |  | 14.2 | 14.1 | 2.6 | 1.1 |
| Test #10 TM9 16QAM 4x4 | AC #1 ULA Low | R-ML |  | 13.5 | 15.3 |  |  |  | 14.4 | 1.8 | 1.3 |
|  |  | LMMSE |  | 14.8 | 15.4 |  |  |  | 15.1 | 0.6 | 0.4 |
|  | AC #2 ULA Med | R-ML |  | N/A | N/A |  |  |  |  |  |  |
|  |  | LMMSE |  | N/A | N/A |  |  |  |  |  |  |
|  | AC #3 ULA Med A | R-ML | N/A | 27.7 | N/A |  |  |  | 27.7 | 0.0 |  |
|  |  | LMMSE | N/A | N/A | N/A |  |  |  |  |  |  |
|  | AC #4 XPL Med A | R-ML | 17.9 | 15.3 | 15.7 | 16.4 |  | 16.2 | 16.3 | 2.6 | 1.0 |
|  |  | LMMSE | 20.4 | 16.8 | 16.6 | 18.2 |  | 19.0 | 18.2 | 3.9 | 1.6 |

Table 5.3.2-6: Simulation results summary – R-ML receiver gain vs. LMMSE (MIMO Rank 4)

| Test case | Antenna correlation | R-ML receiver SNR gain vs. LMMSE for 70% of max throughput, [dB] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Company 1 | Company 2 | Company 3 | Company 4 | Company 5 | Company 6 | Average | Span | STD |
| Test #9 TM4 16QAM 4x4 | AC #1 ULA Low |  | 0.5 | 0.0 |  |  |  | 0.3 | 0.5 | 0.4 |
|  | AC #2 ULA Med |  |  |  |  |  |  |  |  |  |
|  | AC #3 ULA Med A |  | 7.6 |  |  |  |  | 7.6 | 0.0 |  |
|  | AC #4 XPL Med A | 1.5 | 0.9 | 0.6 | 0.9 |  | 1.2 | 1.0 | 0.9 | 0.3 |
| Test #10 TM9 16QAM 4x4 | AC #1 ULA Low |  | 1.2 | 0.1 |  |  |  | 0.7 | 1.1 | 0.8 |
|  | AC #2 ULA Med |  |  |  |  |  |  |  |  |  |
|  | AC #3 ULA Med A |  |  |  |  |  |  |  |  |  |
|  | AC #4 XPL Med A | 2.6 | 1.5 | 0.9 | 1.8 |  | 2.8 | 1.9 | 1.9 | 0.7 |

## 5.4	Conclusions

The following conclusions were made on the benefits and feasibility of enhanced SU-MIMO IM receivers:

-	For rank 2 scenarios, the performance benefits and feasibility of enhanced SU-MIMO IM receiver for TM4 and TM9 have been confirmed.

-	For rank 3 scenarios, the performance benefits and feasibility of enhanced SU-MIMO IM receiver for TM3 have been confirmed.

-	For rank 4 scenarios, the performance benefits and feasibility of enhanced SU-MIMO IM receiver for TM9 have been confirmed.

# 6	Conclusions

This technical report summarizes the results of the feasibility studies on CRS interference mitigation in application to the generic scenarios with different number of CRS APs (2, 4) and different number of UE receive antennas and on SU-MIMO IM receivers in application to the UEs equipped with 4 receive antennas.

Receiver structures based on LMMSE-IRC detector combined with a CRS-IM receiver were investigated in application to the generic scenarios with different number of CRS APs (2, 4) and different number of UE receive antennas including 2 and 4 receive antennas. The feasibility studies were done for the PDSCH and DL control channels including PDCCH, PCFICH, PHICH and EPDCCH. Based on the results of the studies RAN4 concluded on the feasibility and substantial performance benefits of the CRS-IM receivers.

Receiver structure based on SU-MIMO receiver with 4 receive antennas were investigated in application to the generic scenarios with different modulation order and rank for TM3, TM4 and TM9. The feasibility studies were done for combinations of various antenna correlations including low, medium, medium A and high. Based on the results of the studies RAN4 concluded on the feasibility and substantial performance benefits of the SU-MIMO receiver with 4 receive antennas.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2016-11 | RAN4#81 | R4-1610202 |  |  |  | Draft skeleton TR | 0.1.0 |
| 2017-04 | RAN4#82bis | R4-1704118 |  |  |  | Include agreements in RAN4#82 in clause 4, 5. R4-1702192 “TP for enhanced CRS-IM feasibility study” R4-1702196 “TP for enhanced SU-MIMO feasibility study” | 0.2.0 |
| 2017-05 | RAN4#83 | R4-1705555 |  |  |  | Include agreed TPs in RAN4#82bis: R4-1703961 “TR 36.747 text proposal on Enhanced CRS-IM” R4-1704199 “TP for enhanced SU-MIMO IM receiver” | 0.3.0 |
| 2017-06 | RAN#76 | RP-171144 |  |  |  | Include agreed TPs in RAN4#83: R4-1706030 “TR 36.747 text proposal on Enhanced CRS-IM” R4-1706165 “TP for enhanced SU-MIMO IM receiver” | 1.0.0 |
| 2017-09 | RAN#77 | RP-171611 |  |  |  | Include agreed TPs in RAN4#84: R4-1708642 “TR 36.747 text proposal” R4-1708643 “TP for enhanced SU-MIMO IM receiver” | 2.0.0 |
| 2017-09 | RAN#77 | RP-171611 |  |  |  | TR Approved | 14.0.0 |