3GPP TR 36.753 V0.2.0 (2017-04)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Networks;

TDD operating band in the L-band for LTE

(Release 14)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2016, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	6

4	Background	6

4.1	Work item objective	6

5	Band plan allocation and regulatory background	6

6	List of band specific issues for introduction of TDD operating band in the L-band for LTE	6

7	General issues	6

8	Study of E-UTRA specific issues	6

9	Study of MSR specific issues	6

10	Channel numbering for E-UTRA, MSR	7

11	Required changes to E-UTRA and MSR specifications	7

Annex A: Change history	8

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is a technical report for the work item of introduction of TDD operating band in the L-band for LTE, which was approved at TSG RAN#74. The objective of this work item is to specify technical requirements for deploying LTE TDD operation in the L-band.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 30.007: “Guideline on WI/SI for new Operating Bands”

# 3	Definitions, symbols and abbreviations

Delete from the above heading those words which are not applicable.

Clause numbering depends on applicability and should be renumbered accordingly.

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [x] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [x].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [x] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [x].

Abbreviation format (EW)

<ACRONYM>	<Explanation>

# 4	Background

## 4.1	Work item objective

The objective of this work item is the following:

- Study the feasible channelling arrangement(s) of L-band (1427 – 1518 MHz) considering the above ITU TDD options and relevant protection requirements of EESS below 1 427 MHz and MSS above 1518 MHz, considering the results of relevant ongoing ITU-R studies, including:

  - TDD: 90 MHz (1427 – 1517 MHz), considering 1 MHz GB with MSS at the upper edge,

  - Note: the upper edge refinement is based on results of ongoing compatibility studies with MSS. For example option of 88 MHz (1427 – 1515 MHz) considers 3 MHz GB with MSS at the upper edge.

- Specify RF requirements for the feasible plan, band numbering and core requirements, as necessary.

- Support of 3, 5, 10, 15 and 20 MHz channel bandwidths

# 5	Band plan allocation and regulatory background

## 5.1		Current regulatory background

## According to international regulation, WRC-15 and previous WRCs (ITU-R) have allocated the 1427-1518MHz band to mobile service and the WRC15 has indentified the 1427-1518MHz band for IMT  in a large majority of countries through the world .

## In addition and before the WRC15, CEPT has also allocated a part of the Lband (1452-1492MHz) to MFCN (Mobile/Fixed Communications Networks) the 03-07-2015. MFCN bands are designed for commercial use as the LTE or the 5G-NR but not limited to these standards.

## 5.2	TDD band plan

Based on the work item objective, the possible band plan is:

| 1427 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1517 MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1427 - 1432 | 1432 - 1437 | 1437 - 1442 | 1442 - 1447 | 1447 - 1452 | 1452 - 1457 | 1457 - 1462 | 1462 - 1467 | 1467 - 1472 | 1472 - 1477 | 1477 - 1482 | 1482 - 1487 | 1487 - 1492 | 1492 - 1497 | 1497 - 1502 | 1502 - 1507 | 1507 - 1512 | 1512 - 1517 | 1517 - 1518 |
| TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 90 MHz (18 blocks of 5 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Guard band |

Table 5-1: Possible frequency band channelization in the 1427-1518MHz band

## 5.3	In-band services

Figure 5-1: 1427 - 1518 MHz spectrum allocation

There are several allocations in the LBand (1427-1518MHz) and in adjacent frequency bands:

- EESS (Earth Exploration Satellite Service / adjacent to the new IMT band),

- Radio Astronomy (adjacent to the new IMT band),

- Fixed Service (FS) (in the new IMT band)

- Aeronautical service (in the new IMT band)

- Mobile service (in the new IMT band)

- Terrestrial broadcasting service (in the new IMT band)

- Satellite broadcasting service (in the new IMT band)

- Mobile satellite Service (adjacent to the new IMT band),

For all cases "in IMT band", the best option is to move to new frequency band the incumbent services (e.g. for fixed service, CEPT has study the possibility to move the narrow band fixed service in a new frequency band as describe in the ECC Report 219), or, to find another technical solution (e.g. use fiber). If there is no possible to find a solution to the incumbent users, at national level or between countries, it is necessary to develop some specific agreements between incumbent users and IMT systems.

Thus, for the cases "in IMT band", there is no direct and immediate need to adapt the standard to the regulation.

For the cases "adjacent IMT band", there will be, maybe, the necessity to adapt the new standard to guaranty the compatibility between IMT and the incumbent users in adjacent bands. There are 3 incumbent users in adjacent frequency bands:

- Below 1427MHz => For EESS the maximum OOB has already been defined during the last WRC15. Thus, it is necessary to adapt the standardization to these protections levels,

- Below 1427MHz => For Radio Astronomy  and taking into account the low deployment of this service, the best way is to have some specific agreements. The Agreement can be study at national level or if a Radio Astronomy site is near a borderline, between administrations.

- Above 1518MHz => For Mobile Satellite Service, CEPT has already study the compatibility between MFCN for the base station down link (LTE,…) and  MSS. The standard shall comply to the CEPT requests. For the compatibility between IMT uplink terminal and MSS, this report will propose some compatibility studies.

## Thus, there are two services (EESS & MSS) which shall be studied by 3GPP RAN4 because these two systems will get the protection by the standardization. The other incumbent services will be protected by relevant regulation framework at national level or between administrations.

## 5.4	Earth exploration-satellite service (EESS) (passive) compatibility

At the last WRC15, some protection levels have defined for the IMT base station and for the IMT terminal. The RESOLUTION 750 (REV.WRC-15) provides the protection levels. In the table 5.2-1 below summaries the different protection levels for the EESS in the 1400-1427MHz band:

| EESS (passive) band | Active service band | Active service | Limits of unwanted emission power from active service stations in a specified bandwidth within the EESS (passive) band (1) |
| --- | --- | --- | --- |
| 1 400-1 427 MHz | 1 427-1 452 MHz | Mobile | −72 dBW in the 27 MHz of the EESS (passive) band for IMT base stations −62 dBW in the 27 MHz of the EESS (passive) band for IMT mobile stations (2, 3) |

1 The unwanted emission power level is to be understood here as the level measured at the antenna port.

2 This limit does not apply to mobile stations in the IMT systems for which the notification information has been received by the Radiocommunication Bureau by 28 November 2015. For those systems, −60 dBW/27 MHz applies as the recommended value.

3 The unwanted emission power level is to be understood here as the level measured with the mobile station transmitting at an average output power of 15 dBm.

Table 5.2-1: Limits of unwanted emission power in the 1400-1427MHz band from mobile service in the 1427-1452MHz

Thus, it is proposed to study the RF requirements to comply to the regulatory requests.

## 5.5	Mobile Satellite Service compatibility at 1518MHz

The CEPT has already study the protection levels need between IMT and MSS (see ECC Report 263). The key conclusions about this protection are cited as below:

“Based on the final results of its compatibility studies, it is concluded that:

- the minimum in-band blocking characteristic for land mobile earth stations receivers from a 5 MHz broadband signal interferer (LTE) operating below 1518 MHz shall be −30dBm above 1520 MHz1,

- the base station unwanted emission limits e.i.r.p. for a broadband signal interferer (LTE) operating below 1518 MHz shall be −30dBm/MHz above 1520 MHz. This figure is 10 dB more stringent than ECC Decision (13)03 due to a different service in the adjacent band.

It is noted that the IMT block ends at 1517 MHz”

Based on this request, 3GPP RAN 4 needs to study the feasibility of base station to comply to the CEPT requests.

# 6	List of band specific issues for introduction of TDD operating band in the L-band for LTE

- General issues

-	Frequency band arrangement

- E-UTRA issues

-	UE transmitter requirements

-	UE receiver requirements

-	BS Transmitter requirements

-	BS Receiver requirements

# 7	General issues

<Text will be added>

# 8	Study of E-UTRA specific issues

<Text will be added>

## 8.1	For BS (unwanted regional emission level) in the band(s) 1492 – [1517/1518] MHz

The regulatory requirements for base stations operating within 1492 – [1517/1518] MHz are specified by ECC report (for SDL) in terms of EIRP, which corresponds to the unwanted emission limit of -30dB/MHz in 1MHz MSS (Mobile Satellite Service) band above 1520MHz.  The same requirements may be considered for FDD and TDD BS in other regions. When that is required, manufacturer declaration of unwanted emissions may be used to verify the compliance with the regional requirements.

Table XXXX : Declared emission to 1492-1518 MHz

| Filter centre frequency, Ffilter | Declared emission level [dBm] | Measurement bandwidth |
| --- | --- | --- |
| 1520.5 MHz ≤ Ffilter ≤ 1558.5 MHz | PEM,x | 1 MHz |

NOTE:      The regional requirement is defined in terms of EIRP, which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The requirement defined above provides the characteristics of the base station needed to verify compliance with the regional requirement. The assessment of the EIRP level is described in Annex H of TS36.104.

## 8.2        For BTS in the band(s)  [1427] – 1452 MHz (low edge of the frequency range will be aligned on the low edge of the SDL band plan)

Additional spurious emission requirements for the base station are specified in 36.104 sub-clause 6.6.3.3.  These requirements may apply in certain regions for the protection of other systems operating inside or near each supported E-UTRA BS downlink operating band.  To capture the regulatory requirement for the base station operating in the frequency range [1427] – 1452 MHz (low edge of the frequency range will be aligned on the low edge of the SDL band plan) for the protection of EESS (passive) operating in the range 1400 – 1427 MHz, the following table is proposed to be added to 36.104.

The following requirements may apply in regions. For E-UTRA BS operating in Band XX [and YY] within [1427] – 1452 MHz (low edge of the frequency range will be aligned on the low edge of the SDL band plan), emissions shall not exceed the maximum levels specified in Tables 6.6.3.3-x.

It is observed that for base stations operating on channels located at the lower frequency range of this band, the maximum output power may be reduced since there is no guard band at 1427 MHz to allow for filter transition.

Table XXXXX (Table 6.6.3.3-x in 36.104): Additional operating band unwanted emission limits for E-UTRA band XX [and YY] within [1427] – 1452 MHz

| Operating Band | Filter center frequency, Ffilter | Maximum Level [dBW] | Measurement bandwidth |
| --- | --- | --- | --- |
| XX | Ffilter = 1413.5 MHz | -72 | 27 MHz |

8.3        For UE in the band(s) 1492 – [1517/1518] MHz to protect MSS

[To protect MSS, need further studies and to take a decision about the level to protect MSS at the next RAN4 meeting. Technical solution to be consistent to the MSS protection level should be also proposed at the next RAN4 meeting.]

## 8.4       For UE in the band(s) [1427] – 1452 MHz (low edge of the frequency range will be aligned on the low edge of the SDL band plan) to protect EESS

Additional spurious emission requirements for the user equipment are specified in 36.101.  These requirements may apply in regions for the protection of other systems.  To capture the regulatory requirement for the user equipment operating in the frequency range [1427]– 1452 MHz (low edge of the frequency range will be aligned on the low edge of the SDL band plan) for the protection of EESS (passive) operating in the range 1400 – 1427 MHz, the following table is proposed to be added to 36.101.

For UE operating in Band XX within [1427]– 1452 MHz (low edge of the frequency range will be aligned on the low edge of the SDL band plan), emissions shall not exceed the maximum levels specified in Tables XXX-x.

It is observed that for user equipment operating on channels located at the lower frequency range of this band, the maximum output power may be reduced since there is no guard band at 1427 MHz to allow for filter transition.

Table XXXXX (Table XXX-x in 36.101): Additional operating band unwanted emission limits for E-UTRA band XX within [1427] – 1452 MHz

| Operating Band | Filter center frequency, Ffilter | Maximum Level [dBW] | Measurement bandwidth |
| --- | --- | --- | --- |
| XX | Ffilter = 1413.5 MHz | -62 | 27 MHz |

# 9	Study of MSR specific issues

<Text will be added>

# 10	Channel numbering for E-UTRA, MSR

<Text will be added>

# 11	Required changes to E-UTRA and MSR specifications

The required changes to the 3GPP specifications for the new band are summarised in a Table 11-1.

Table 11-1: Overview of 3GPP specifications with required changes

| 3GPP specification | Clause in TR 30.007 where the required changes are given | Clause in the present document identifying additional changes |
| --- | --- | --- |
| TS 36.101 | 8.2.1.1 |  |
| TS 36.104 | 8.2.1.2 |  |
| TS 36.106 | 8.2.1.3 |  |
| TS 36.113 | 8.2.1.4 |  |
| TS 36.124 | 8.2.1.5 |  |
| TS 36.133 | 8.2.1.6 |  |
| TS 36.141 | 8.2.1.7 |  |
| TS 36.143 | 8.2.1.8 |  |
| TS 36.307 | 8.2.1.9 |  |
| TS 25.101 | 8.2.2.1 |  |
| TS 25.104 | 8.2.2.3 |  |
| TS 25.106 | 8.2.2.5 |  |
| TS 25.123 | 8.2.2.7 |  |
| TS 25.133 | 8.2.2.9 |  |
| TS 25.141 | 8.2.2.10 |  |
| TS 25.143 | 8.2.2.12 |  |
| TS 25.461 | 8.2.2.15 |  |
| TS 25.466 | 8.2.2.16 |  |
| TS 37.104 | 8.2.3.1 |  |
| TS 37.113 | 8.2.2.2 |  |
| TS 37.141 | 8.2.2.3 |  |

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2017-01 | R012-10 12 7 ¦ 6 ¦ 5 ¦ 4 ¦ 19992of f Ericsson and Quaccom000000000000000000000000000000000000000000000000000000000000000000000004#82 | R4-1701272 |  |  | Report skeleton |  | 0.0.1 |
| 2017-04 | RAN4#82bis | R4-1703302 |  |  | TPs approved in RAN#82 were implemented: R4-1701273, “Regulation background for TDD L Band” R4-1701274, “Band specific issues for TDD L Band” |  | 0.1.0 |
| 2017-05 | RAN4#83 |  |  |  | TP approved in RAN#82bis was implemented: R4-1703519, “Work item objective for TDD L band” R4-1704098, “Band plan options 1.5 GHz TDD band” |  | 0.2.0 |