3GPP TS 52.021 V14.0.0 (2017-03)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Network Management (NM) procedures and messages

on the A-bis interface

(Release 14)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

GSM, management

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

Introduction	7

1	Scope	8

2	References	8

3	Definitions and abbreviations	9

3.1	Definitions	9

3.2	Abbreviations	9

4	Functional Split between BSC and BTS	9

5	Information Model	10

5.1	Managed Objects	10

5.2	Addressing of Objects	13

5.3	State Management of Objects	13

5.3.1	Administrative State	13

5.3.2	Operational State and Availability Status	14

6	Elementary Procedures	15

6.1	Definition of the Procedures	16

6.2	SW Download Management Procedures	16

6.2.1	Load Data Initiate	16

6.2.2	Load Data Segment	16

6.2.3	Load Data Abort	17

6.2.4	Load Data End	17

6.2.5	SW Activate Request	17

6.2.6	Activate SW	18

6.2.7	SW Activated Report	18

6.3	A-bis Interface Management Procedures	18

6.3.1	Establish TEI	18

6.3.2	Connect Terrestrial Signalling	18

6.3.3	Disconnect Terrestrial Signalling	19

6.3.4	Connect Terrestrial Traffic	19

6.3.5	Disconnect Terrestrial Traffic	19

6.4	Transmission Management Procedures	19

6.4.1	Connect Multi-drop Link	19

6.4.2	Disconnect Multi-drop Link	20

6.5	Air Interface Management Procedures	20

6.5.1	Set BTS Attributes	20

6.5.2	Set Radio Carrier Attributes	20

6.5.3	Set Channel Attributes	20

6.6	Test Management Procedures	21

6.6.1	Perform Test	21

6.6.2	Test Report	21

6.6.3	Send Test Report	21

6.6.4	Stop Test	22

6.7	State Management and Event Report Procedures	22

6.7.1	State Changed Event Report	22

6.7.2	Failure Event Report	22

6.7.3	Stop Sending Event Reports	23

6.7.4	Restart Sending Event Reports	23

6.7.5	Change Administrative State	23

6.7.6	Change Administrative State Request	23

6.7.7	Report Outstanding Alarms	24

6.8	Equipment Management Procedures	24

6.8.1	Change-over	24

6.8.2	Opstart	24

6.8.3	Reinitialize	24

6.8.4	Set Site Outputs	25

6.9	Measurement Management Procedures	25

6.9.1	Measurement Result Request	25

6.9.2	Measurement Result Response	25

6.9.3	Stop Measurement	25

6.9.4	Start Measurement	26

6.10	Miscellaneous Procedures	26

6.10.1	Get Attributes	26

6.10.2	Set Alarm Threshold	26

6.10.3	Get Attributes Response	26

7	Structured Procedures	26

8	Message Details	27

8.1	Message Categories	27

8.1.1	Formatted O&M Messages	27

8.1.2	MMI Transfer	27

8.1.3	TRAU O&M Messages	28

8.1.4	Manufacturer-Defined O&M messages	28

8.2	Structure of Formatted O&M Messages	29

8.3	SW Download Management Messages	30

8.3.1	Load Data Initiate	30

8.3.2	Load Data Segment	30

8.3.3	Load Data Abort	30

8.3.4	Load Data End	30

8.3.5	SW Activate Request	31

8.3.6	Activate SW	31

8.3.7	SW Activated Report	31

8.4	A-bis Interface Management Messages	31

8.4.1	Establish TEI	31

8.4.2	Connect Terrestrial Signalling	31

8.4.3	Disconnect Terrestrial Signalling	32

8.4.4	Connect Terrestrial Traffic	32

8.4.5	Disconnect Terrestrial Traffic	32

8.5	Transmission Management Messages	32

8.5.1	Connect Multi-drop link	32

8.5.2	Disconnect Multi-drop link	33

8.6	Air Interface Management Messages	33

8.6.1	Set BTS Attributes	33

8.6.2	Set Radio Carrier Attributes	34

8.6.3	Set Channel Attributes	34

8.7	Test Management Messages	34

8.7.1	Perform Test	34

8.7.2	Test Report	35

8.7.3	Send Test Report	35

8.7.4	Stop Test	35

8.8	State Management and Event Report Messages	35

8.8.1	State Changed Event Report	35

8.8.2	Failure Event Report	36

8.8.3	Stop Sending Event Reports	36

8.8.4	Restart Sending Event Reports	37

8.8.5	Change Administrative State	37

8.8.6	Change Administrative State Request	37

8.8.7	Report Outstanding Alarms	37

8.9	Equipment Management Messages	38

8.9.1	Changeover	38

8.9.2	Opstart	38

8.9.3	Reinitialize	38

8.9.4	Set Site Outputs	38

8.9.5	Change HW Configuration	39

8.10	Measurement Management Messages	39

8.10.1	Measurement Result Request	39

8.10.2	Measurement Result Response	39

8.10.3	Start Measurement	39

8.10.4	Stop Measurement	39

8.11	Miscellaneous Messages	40

8.11.1	Get Attributes	40

8.11.2	Set Alarm Threshold	40

8.11.3	Get Attribute Response	40

9	Coding	40

9.1	Message Type	41

9.2	Object Class	44

9.3	Object Instance	44

9.4	Attributes and Parameters	45

9.4.1	Abis Channel	47

9.4.2	Additional Info	48

9.4.3	Additional Text	48

9.4.4	Administrative State	48

9.4.5	ARFCN List	48

9.4.6	Autonomously Report	49

9.4.7	Availability Status	49

9.4.8	BCCH ARFCN	49

9.4.9	BSIC	49

9.4.10	BTS Air Timer	50

9.4.11	CCCH Load Indication Period	50

9.4.12	CCCH Load Threshold	50

9.4.13	Channel Combination	50

9.4.14	Connection Failure Criterion	51

9.4.15	Destination	51

9.4.16	Event Type	51

9.4.17	File Data	52

9.4.18	File Id	52

9.4.19	File Version	52

9.4.20	GSM Time	52

9.4.21	HSN	52

9.4.22	HW Configuration	53

9.4.23	HW Description	53

9.4.24	Intave Parameter	53

9.4.25	Interference level Boundaries	54

9.4.26	List of Required Attributes	54

9.4.27	MAIO	54

9.4.28	Manufacturer Dependent State	54

9.4.29	Manufacturer Dependent Thresholds	55

9.4.30	Manufacturer Id	55

9.4.31	Max Timing Advance	55

9.4.32	Measurement Result	55

9.4.33	Measurement Type	55

9.4.34	Multi-drop BSC Link	56

9.4.35	Multi-drop next BTS Link	56

9.4.36	Nack Causes	56

9.4.37	Ny1	58

9.4.38	Operational State	58

9.4.39	Overload Period	58

9.4.40	Physical Config	58

9.4.41	Power Class	58

9.4.42	Power Output Thresholds	59

9.4.43	Probable Cause	59

9.4.44	RACH Busy Threshold	59

9.4.45	RACH Load Averaging Slots	60

9.4.46	Radio Sub Channel	60

9.4.47	RF Max Power Reduction	60

9.4.48	Site Inputs	60

9.4.49	Site Outputs	61

9.4.50	Source	61

9.4.51	Specific Problems	61

9.4.52	Starting Time	61

9.4.53	T200	62

9.4.54	TEI	62

9.4.55	Test Duration	62

9.4.56	Test No	62

9.4.57	Test Report Info	63

9.4.58	VSWR Thresholds	63

9.4.59	Window Size	63

9.4.60	TSC	63

9.4.61	SW Configuration	64

9.4.62	SW Description	64

9.4.63	Perceived Severity	64

9.4.64	Get Attribute Response Info	64

9.4.65	Outstanding Alarm Sequence	65

9.4.66	HW Conf Change Info	65

Annex A (informative):	Change history	66

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

(Background)

The use and general aspects of the A-bis interface are given in specification 3GPP TS 48.051. The split of telecommunications functions and management procedures between BSC and BTS are defined in specification 3GPP TS 48.052. Specification 3GPP TS 48.056 defines Layer 2 of the signalling messages.

The general aspects of NM are defined in specification GSM 12.00. Qx interface and protocol stack are defined in specification GSM 12.01. GSM 12.06 provides the functional requirements supported by the present document. The NM procedures and messages to support these operations over the A-bis interface are specified here. Specification GSM 12.20 provides the information model as seen on the OMC-BSC interface. Interworking between this model and the NM messages and procedures provided here is specified in GSM 12.22.

# 1	Scope

The present document addresses the network management messages and procedures across the A-bis interface, which is defined as Qx in GSM. The information model included here defines the objects and how they are addressed for purposes of operations and maintenance activities.

There is a requirement for the A-bis interface to be open to allow interoperation between BTSs of different manufacturers working to the same BSC. The present document addresses this requirement from O&M point of view, which allows this interworking to take place. It shows the split of NM functions between BSC and BTS. The procedures and coding of the messages are specified in detail. In practice, in addition to the present document it is necessary that the content of manufacturer-dependent information fields be specified to fulfill the functionality.

It is essential for operation that a BSC can handle the functions used by all its BTSs. Therefore, all items in the present document are considered mandatory unless otherwise indicated in the present document.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 44.006: "Mobile Station - Base Stations System (MS - BSS) Interface Data Link (DL) Layer Specification".

[3]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[4]	3GPP TS 45.002: "Multiplexing and Multiple Access on the Radio Path".

[5]	3GPP TS 45.005: "Radio transmission and reception".

[6]	3GPP TS 45.008: "Radio subsystem link control".

[7]	3GPP TS 48.051: "Base Station Controller - Base Tranceiver Station (BSC-BTS) Interface General Aspects".

[8]	3GPP TS 48.052: "Base Station Controller - Base Tranceiver Station (BSC-BTS) Interface - Interface Principles".

[9]	3GPP TS 48.056: "BSC-BTS Layer 2 Specification".

[10]	3GPP TS 48.058: "Base Station Controler - Base Transceiver Station  (BCS-BTS) Interface Layer 3 Specification".

[11]	GSM 12.00 (GSM Phase 2): "Objectives and structure of Network Management (NM)".

[12]	GSM 12.01 (GSM Phase 2): "Common aspects of GSM Network Management (NM)".

[13]	GSM 12.06 (GSM Phase 2): "GSM Network Configuration Management and Administration".

[14]	GSM 12.20 (GSM Phase 2): "Base Station System (BSS) Management Information".

[15]	GSM 12.22 (GSM Phase 2): "Interworking of GSM Network Management (NM) procedures and messages at the Base Station Controller (BSC)".

[16]	ITU-T Recommendation X.731: "Information technology - Open Systems Interconnection - Systems Management: State management function".

# 3	Definitions and abbreviations

## 3.1	Definitions

Definitions of terms used within the present document may be found mostly in clause5 in text context.

## 3.2	Abbreviations

For the purpose of the present document, the following abbreviations apply:

ASN.1	(CCITT) Abstract Syntax Notation One

BSC	Base Station Controller

BSS	Base Station System

BTS	Base Transceiver Station

cont.	continued

HW	Hardware

LSB	Least Significant Byte

man. dep.	manufacturer dependent (with upper and lower case adjusted as appropriate)

MMI	Man-machine Interface

MSB	Most Significant Byte

MSC	Mobile-services Switching Centre

NE	Network Element

NM	Network Management

O&M	Operations and Maintenance

OMC	Operations and Maintenance Centre

RF	Radio Frequency

SAPI	Service Access Point Indicator

SW	Software

TEI	Terminal End-point Identifier

TMN	Telecommunications Management Network

TSC	Training Sequence Code

Further GSM related abbreviations may be found in 3GPP TS 21.905 [1].

# 4	Functional Split between BSC and BTS

Functional split of management functions between BSC and BTS is shown in table 1.

Table 1/GSM 12.2: Split of management functions between BSC and BTS

|  | BSC | BTS |
| --- | --- | --- |
| Fault Management |  |  |
| BTS |  |  |
| test request | X | - |
| test execution | - | X |
| test analysis | NS | - |
| fault detection | - | X |
| fault localization | X (note) | X |
| fault reporting | X | X |
| Link |  |  |
| testing (req,ex,rpt) | NS | - |
| fault detection | X | X |
| fault localization | X | X |
| fault reporting | X | X |
| Configuration Management |  |  |
| Hardware | control/monitor | control |
| Software | control/monitor | monitor |
| State | control/monitor | control/monitor |
| Parameters | control/monitor | monitor |
| Performance Management |  |  |
| Collection | X | X (radio path only) |
| Reporting | X | X (radio path only) |
| Administration | X | - |
| Security Management (Access Control to BTS) BTS) |  |  |
| Control | - | X |
| Monitoring | - | X |
| NOTE: When fault localisation is not possible at the BTS it must be deduced at the BSC. Legend: Abbreviations: NS = Not Specified; req = request; X = Function exists; ex = execution - = Function non-exists; rpt = report |  |  |

# 5	Information Model

## 5.1	Managed Objects

The BCF mentioned in 3GPP TS 48.052 and 3GPP TS 48.056 is the agent at the BTS end of the A-bis O&M interface. It has four different descriptions depending on the object that is managed: Site Manager, BTS, Radio Carrier and Baseband Transceiver.

This model describes how objects are managed across A-bis interface, but it doesn't specify how information is transferred inside the site. That is, the manner of communication between an object and objects under it is not specified in the present document.

As shown in Figure 1, the Object Classes used on the A-bis interface are a subset of those found under Site Manager on the OMC-BSC interface. The Object Classes are listed below and the functionalities that describe them are found in table 2.

Site Manager: manages common control functions of several BTSs and transceivers on one site. These can include managing external alarms, front-end switch, etc. This model describes logical sites. There can be multiple logical sites in one physical site. Communication between entities within a logical site is manufacturer dependent.

BTS: is associated with one cell. BTSs are typically created at installation phase by connecting transceivers to antennas thus forming cells from the air interface point of view. The BTS can also contain control functions common to various transceivers. The way BTSs are formed from transceivers and how corresponding BTS numbers are determined is configuration dependent information, which is stored during installation.

Radio Carrier: represents manageable properties pertaining to radio transmission and reception of one carrier.

Baseband Transceiver: represents functions common to eight radio time slots.

Channel: is a physical channel in air interface, which can contain several logical channels depending on channel combination. A Channel is described with radio time slot and frequency hopping attributes (see 3GPP TS 45.002).

NOTE:	Site Manager and BTS don't necessarily require separate equipment. For example, the Site Manager and a Baseband Transceiver can be associated with the same physical equipment.

Figure 1/GSM 12.21: Object model seen across A-bis interface

Table 2/GSM 12.21: Objects, attributes and procedures seen across A-bis interface

| Object class | Attributes | Procedures |
| --- | --- | --- |
| Site Manager | Abis Channel Availability Status HW Configuration Manufacturer Dependent State Manufacturer Id Operational State Site Inputs Site Outputs SW Configuration | Equipment Management Establish TEI Get Attributes Measurement Management Set Site Outputs State Management and Event Report SW Download Management Test Management |
| BTS | Administrative State Availability Status BCCH ARFCN BSIC BTS Air Timer CCCH Load Ind. Period CCCH Load Threshold Connection Failure Criterion GSM Time HW Configuration Intave Parameter Interterference Level Boundaries Manufacturer Dependent State Max Timing Advance Ny1 Operational State Overload Period RACH Busy Threshold RACH Load Averaging Slots SW Configuration T200 | Equipment Management Get Attributes Measurement Management Report Procedures Set BTS Attributes State Management and Event Report SW Download Management Test Management |
| Radio Carrier | Administrative State ARFCN List Availability Status HW Configuration Manufacturer Dependent State Manufacturer Id Operational State Power Class RF Max Power Reduction SW Configuration | Equipment Management Get Attributes Measurement Management Set RadioCarrier Attributes State Management and Event Report SW Download Management Test Management |
| Baseband Transceiver | Abis Channel* Administrative State Availability Status HW Configuration Manufacturer Dependent State Manufacturer Id Operational State SW Configuration | Connect Terrestrial Signalling Disconnect Terrestrial Signalling Equipment Management Get Attributes Measurement Management State Management and Event Report SW Download Management Test Management |
| Channel | Abis Channel* Administrative State ARFCN List* Availability Status Channel Combination HW Configuration HSN* MAIO* Operational State SW Configuration TSC | Connect Terrestrial Traffic Disconnect Terrestrial Traffic Equipment Management Get Attributes Measurement Management Set Channel Attributes State Management and Event Report SW Download Management Test Management |

NOTE:	If more than one layer 2 link is defined to handle this information model, the actual implementation may affect on which layer 2 link attributes concerning frequency hopping and front-end switch are sent. Equipment used to implement frequency hopping can be common to several radio carriers or located inside one radio carrier. Front-end switch is handled by Site Manager, therefore all 'Abis Channel' attributes must be sent to it. Those attributes that need a link identifier in BSC data structures are marked with an asterisk (*).

## 5.2	Addressing of Objects

It is a GSM requirement that the BSC is capable of operating with BTSs from different manufacturers. So, it is necessary that the differences between BTSs, as seen by the BSC, are minimised as much as possible. This is achieved by addressing NM messages by the Managed Object Class and Managed Object Instance. There must be in the BSC an object model with a complete layer 2 link description for each object instance in the BTS (e.g., Baseband Transceiver 1, Baseband Transceiver 2, Baseband Transceiver 3,...). When a message has to be sent to an object instance this mapping is used to find the correct link.

The first connection shall be established from the BTS site by using a (semi-) permanently programmed default TEI. Subsequent to the establishment of the default connection to the site, further initial connections to other objects shall be established using TEIs provided by procedures in subclause 6.3.1.

Possible additional TEIs to an object already having an initial TEI are assigned with a GSM 12.21 message found in subclause 6.3.1 through an already assigned signalling link.

Object instances also have a layer 3 address. The instance number is used to address the object instance. Regardless of whether the layer 2 address  uniquely identifies the object instance or not, the layer 3 address must also be provided so that it can be used by management agent to determine which object instance is being addressed. This combination of layer 2 and layer 3 addressing accommodates BTS sites with either single or multiple physical links.

Specific equipment configuration information is manufacturer dependent. However, for interoperability, link configuration, default TEI assignment and instance numbering must be known by both manager and agent. This, as well as supported functions, is considered as Shared Management Knowledge.

## 5.3	State Management of Objects

State management in the present document is generally in line with ITU-T Recommendation X.731. How state values are applied is explained below.

ITU-T Recommendation X.731 states that "the management state of a managed object represents the instantaneous condition of availability and operability of the associated resources from the point of view of management".

In the present document there are two different factors (ITU-T Recommendation X.731 defines usage state in addition to these two) that are considered to affect the management state of a managed object. They are:

-	administration: permission to use or prohibition against using the resource, imposed through the management services;

-	operability: whether or not the resource is physically installed and working.

The present document defines the following three state management attributes to represent the management state of a managed object:

-	administrative state;

-	operational state;

-	availability status (this elaborates the operational state).

### 5.3.1	Administrative State

Administrative states of the managed objects can be controlled only by the BSC. In fact, the control of resource usage comes from the MSC).

Setting the administrative state of a Channel to locked means that it has to discontinue all transmission and reception of information on Air interface and all telecommunication (traffic and signalling) information transmission on A-bis interface.

Setting the administrative state of a Radio Carrier to locked means that it has to discontinue all RF emission on Air interface for the frequencies it is responsible.

Setting the administrative state of a Baseband Transceiver to locked means that it has to discontinue all telecommunication tramissions and the Channels which are affected should indicate that they are Disabled or Degraded.

Setting the administrative state of a BTS to locked means it has to discontinue all RF emission on all Radio Carriers and all telecommunication transmissions on all Channels of all Baseband Transceivers, i.e., Radio Carriers, Channels and Baseband Transceivers are in Disabled, Dependency state.

The administrative state at the BTS site is also used to provide information e.g., for a local MMI user at a BTS site.

The locked state of a 12.21 managed object means that BSC has disconnected all the calls that go through the resource that is represented by the managed object. No new traffic is connected through this resource any more.

The shutting down state means that no new traffic is connected through this resource any more. The on-going calls remain.

The unlocked state means that new traffic is allowed through the resource represented by the managed object.

### 5.3.2	Operational State and Availability Status

ITU-T Recommendation X.731 gives the following definitions for the values of the operational state attribute:

-	disabled: the resource is totally inoperable and unable to provide service to the user(s);

-	enabled: the resource is partially or fully operable and available for use.

In the present document the value Disabled represents the following conditions that the resources could have:

-	hardware or software is not installed;

-	power is turned off;

-	failure has occurred;

-	radio parameters has not yet been set by elementary procedures, therefore, the resource is off line.

The availability status elaborates the operational state attribute. In the present document the following values are used (availability status is a set value):

In test: the resource is undergoing a test procedure.

Failed: the resource has an internal fault that prevents it from operating. The operational state is disabled.

Power off: the resource requires power to be applied and is not powered on. The operational state is disabled.

Off line: the resource requires some manual and/or automatic operation(s) to be performed to make it available for the use.

Dependency: the resource cannot operate because some other resource on which it depends is unavailable. The operational state is disabled.

Degraded: the service available from the resource is degraded in some respect, such as in speed or operating capacity. The operational state is enabled.

Not installed: the hardware or the software associated with the managed object has not been installed at the site. Operational state is disabled.

Figure 2 illustrates the operational state and availability status behaviour of GSM 12.21 objects (i.e., Site Manager, BTS, Radio Carrier, and Baseband Transceiver) during initialization. The initial value of the administrative state is locked.

Figure 2/GSM 12.21: GSM 12.21 Objects' Operational state
and availability status behaviour during initialization

# 6	Elementary Procedures

The operational procedures applicable to the BTS consist of bringing BTS equipment and software into (or taking them out of) service, initiation of tests at the BTS, collection of test results made at the BTS, reporting and clearing of any BTS faults, and reporting of any BTS external alarms. Bringing into service of equipment at the BTS will include manual operations, including turning the switch on, and performing local testing where relevant at the BTS, followed by an indication of the availability of the equipment to the BTS via some man-machine interface (MMI terminal, telephone). It is then a BSC function to ensure that relevant data on the existence of the equipment is resident at the BTS, and to activate (bring into service) the new equipment. Manual procedures may also be employed for software loading, at least initially.

As far as software is concerned, all operational software used at the BTS shall also be kept at the BSC, and can be downloaded from the BSC. At the BTS there may be either single instance of the software, or there may also be back-up versions.

## 6.1	Definition of the Procedures

All the procedures covered in the present document are based on formatted O&M messages. Most formatted O&M messages initiated by the BSC (or by a BTS) will receive a response or acknowledgement at Layer 3. A pair of such messages, or single message if a response is not required, is referred to as an elementary procedure. All messages shall be sent using I frames at Layer 2.

A number of procedures, referred to as structured procedures (see GSM 12.22), are based on a multiplicity of elementary procedures. Some structured procedures may also involve MMI messages, but these are not defined in the present document. For all elementary procedures described in Subclauses 6.2 through 6.10, the protocol scenarios are illustrated with no further explicit reference made from their corresponding subclauses because of their self-explanatory nature.

Descriptions of the messages and the direction of transmission are given in the following subclauses.

No elementary procedure shall be initiated to an object instance which has not yet replied to a previously initiated elementary procedure with a response, an ACK or a NACK within a layer 3 time-out. The layer 3 timeout for ACK, NACK and responses shall have a default value of 10 seconds.

An ACK message is returned to inform the application which initialized the message that the command is performed or will be performed.

The whole message must be rejected if there is something not understood/supported in the original message.

A NACK may not be relevant for some elementary procedures.

The most relevant Nack causes, not covered by the general causes (which are used for understanding of header fields), are given for each elementary procedure with reference to the coding of the Nack causes in subclause 9.4.36.

The general Nack causes are relevant for any Nack message and are also found in subclause 9.4.36.

## 6.2	SW Download Management Procedures

### 6.2.1	Load Data Initiate

This message shall be sent from the BSC to the BTS to initiate the loading of a file. It indicates the number of segments for which a Layer 3 acknowledgement is required (window size). When receiving data the BTS shall send an ACK after this number of segments, except for the last batch.

BSC					BTS

Load Data Initiate

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message : Ready to receive the specified file.

Message specific Nack causes (see subclause 9.4.36): 23, 24.

### 6.2.2	Load Data Segment

These multi-segment messages shall be used to carry the files for the transfer initiated by the Load Data Initiate message. No other file transfer shall be allowed until the current transfer is finished.

BSC					BTS

Load Data Segment				-+

-------------------------------------------->	  ¦

Load Data Segment				  ¦- Window Size

-------------------------------------------->	  ¦

¦

¦				-+

ACK

<--------------------------------------------

Load Data Segment

-------------------------------------------->

etc.

etc.

An ACK shall be sent from the BTS to the BSC every time when Window Size number of segments specified in the Load Data Initiate message are downloaded. A reception of an ACK must not reset the value of the sequence number of the subsequent message segments (see subclause 8.1.1). When all the expected blocks have been received, an ACK must be sent regardless of the window size. If the timer for a time-out for the Layer 3 acknowledgement expires, the BSC shall send a Load Data Abort message and the file transfer shall be aborted.

Meaning of Ack message: A window of Load data segment messages or a complete file has been received.

### 6.2.3	Load Data Abort

This message shall be used by either end if the file transfer can no longer be supported. This message shall also be used by the BTS if the received amount of data exceeds the expected amount.

BSC					BTS

Load Data Abort

<------------------------------------------->

### 6.2.4	Load Data End

This message shall be sent by the BSC to the BTS. The BTS sends an ACK when the file has been received in the BTS.

BSC					BTS

Load Data End

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message : File download is successfully terminated.

Message specific Nack causes (see subclause 9.4.36): 25.

### 6.2.5	SW Activate Request

This message shall be sent by the BTS when the resource represented by the object instance (Site Manager, BTS, Radio Carrier, or Baseband Transceiver) has started up. The initialization of mentioned object instance shall be started with software activation, which may include software download continuing with attribute setting.

BSC					BTS

SW Activate Request

<--------------------------------------------

ACK/NACK

-------------------------------------------->

Meaning of Ack message : The request is granted and software activation will be 	commenced.

Message specific Nack causes (see subclause 9.4.36): None.

### 6.2.6	Activate SW

This message from the BSC to the BTS shall be used to activate the loaded software, indicating which file (or files) is to be activated. The acknowledgement of the Activate SW indicates that the software can be activated. If the software cannot be activated, a NACK must be sent. The activation may include BTS internal software distribution.

BSC					BTS

Activate SW

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message: File will be activated.

Message specific Nack causes (see subclause 9.4.36): 23, 26, 27.

### 6.2.7	SW Activated Report

This message from the BTS to the BSC shall be sent from the addressed object on the BTS at a successful completion of the software distribution to and activation on all indicated destinations in the BTS.

BSC					BTS

SW Activated Report

<--------------------------------------------

## 6.3	A-bis Interface Management Procedures

These procedures (excluding 'Establish TEI') are only relevant with BTSs having flexibility to configure A-bis channels to different Baseband Transceivers. This configuration flexibility is realized with equipment that can be called BTS front-end switch. In other words this switch allows an A-bis channel to be able to be connected in several possibilities.

There must be a default connection to the Site Manager, which manages the front-end switch. The default connection information (AbisLink, TEI) must be (semi-)permanently programmable so that it is available at power up. Through the default connection other A-bis connections are established by configuring the front-end switch.

It is optional for BTS equipment to have front-end switch features.

### 6.3.1	Establish TEI

When an initial/additional TEI is to be established (see 3GPP TS 48.056), this message shall be sent from the BSC on a previously established link giving initial/additional TEI number and the corresponding physical connection. For establishment of additional TEI the procedure specified in specification 3GPP TS 48.056 must be performed and the result shall be indicated in the ACK/NACK response message. If an initial TEI assignment fails, it can mean that TEIs cannot be assigned through the default link (e.g., physical implementation doesn't support transferring TEIs from Site Manager to Baseband Transceivers). In that case there must be subsequent preprogrammed TEIs to establish more links.

BSC					BTS

Establish TEI

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message: The TEI has been established as specified.

Message specific Nack causes (see subclause 9.4.36): 23.

### 6.3.2	Connect Terrestrial Signalling

This message shall be used to connect a particular terrestrial transmission circuit to a Baseband Transceiver. The procedure can be used only for a BTS that has the relevant flexibility.

BSC					BTS

Connect Terrestrial Signalling

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message: The specified signalling link has been connected.

Message specific Nack causes (see subclause 9.4.36): 1E.

### 6.3.3	Disconnect Terrestrial Signalling

This message shall be used to disconnect a Baseband Transceiver from a terrestrial circuit.

BSC					BTS

Disconnect Terrestrial Signalling

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message: The specified signalling link has been disconnected.

Message specific Nack causes (see subclause 9.4.36): 20.

### 6.3.4	Connect Terrestrial Traffic

This message shall be used to relate a terrestrial circuit to a radio time slot, if the BTS has the flexibility. If half rate coding is implemented, two Connect Terrestrial Traffic messages shall be sent for each radio timeslot.

BSC					BTS

Connect Terrestrial Traffic

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message: The specified traffic link has been connected.

Message specific Nack causes (see subclause 9.4.36): 1E.

### 6.3.5	Disconnect Terrestrial Traffic

This message shall be used to disconnect a terrestrial circuit from a radio timeslot.

BSC					BTS

Disconnect Terrestrial Traffic

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message: The specified traffic link has been disconnected.

Message specific Nack causes (see subclause 9.4.36): 20.

## 6.4	Transmission Management Procedures

These prodecures are used to configure on-site transmission equipment. They may not be sufficient and are subject to further standardization.

### 6.4.1	Connect Multi-drop Link

This message shall be used to relate two terrestrial circuits (64 kbps timeslot in a 2 Mbps system to a radio time slot), if the BTS has the flexibility of setting up multi-drop configuration.

BSC					BTS

Connect Multi-drop Link

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message: The specified multi-drop link connection has been connected.

Message specific Nack causes (see subclause 9.4.36): 1E.

### 6.4.2	Disconnect Multi-drop Link

This message shall be used to disconnect a multi-drop configuration in the BTS.

BSC					BTS

Disconnect Multi-drop Link

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message: The specified multi-drop link connection has been disconnected.

Message specific Nack causes (see subclause 9.4.36): 20.

## 6.5	Air Interface Management Procedures

### 6.5.1	Set BTS Attributes

This message shall be sent to provide an object instance on BTS with all the necessary attributes relating to that BTS object. This message does also include common information for all logical channels of one type, e.g., CCCH parameters.

BSC					BTS

Set BTS Attributes

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message: All specified BTS attributes have been set.

Message specific Nack causes (see subclause 9.4.36): 23.

### 6.5.2	Set Radio Carrier Attributes

This message shall be used to set a Radio Carrier's RF-related attributes such as the frequencies and the power.

BSC					BTS

Set Radio Carrier Attributes

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message: All specified Radio Carrier attributes have been set.

Message specific Nack causes (see subclause 9.4.36): 1B, 23.

### 6.5.3	Set Channel Attributes

This message shall be sent for each Channel instance to give the channel combination as defined in 3GPP TS 45.002 for that time slot. It shall be possible to send one message, if all 8 radio time slots need the same attribute value. This shall be done by not addressing one specific Channel instance in the  Instance field in the header, but using a broadcast address. The message may include starting time.

BSC					BTS

Set Channel Attributes

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message: All specified channel attributes have been set.

Message specific Nack causes (see subclause 9.4.36): 23.

## 6.6	Test Management Procedures

### 6.6.1	Perform Test

This message shall be used to tell the BTS to perform a test, if necessary to set a physical configuration for the BSC to carry out a test on the BTS, or to perform a test using a particular configuration. Any measurements may be performed as specific tests. Duration for the test can be given, after which the test report may be autonomously sent if so requested.

Three tests are defined.

1)	A radio loop test via the antenna shall be used to test all equipment needed to provide service of one traffic channel. The loop starts and ends in the transceiver baseband parts and loops one traffic channel back in the air interface i.e., using actual antenna equipment. The baseband parts of the transceiver calculate the bit error rate to describe the quality of service that channel provides.

2)	A radio loop test via the transceiver shall be used to test most of the equipment needed to provide service of one traffic channel. The loop starts and ends in the transceiver baseband parts and loops one traffic channel back inside the transceiver before the antenna combining equipment. The baseband parts of the transceiver calculate the bit error rate to describe the quality of service that channel provides. This test can be used in conjunction with the previous test to discriminate the location of a possible hardware failure.

3)	A functional object self test shall be used to activate an internal self test procedure of a functional object on the BTS made to test equipment that provides the services of the functional object. By its nature this test and its results are proprietary.

BSC					BTS

Perform Test

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message: Test configuration has been set (if necessary) and the specified test has been 	started.

Message specific Nack causes (see subclause 9.4.36): 1C, 1D, 1E.

### 6.6.2	Test Report

This message shall be sent by the BTS giving the result of a test ordered by the BSC and is sent autonomously as soon as the result is available. A Test Report shall also be sent after a specific request from the BSC by a Send Test Report message. The Test Report indicates what was tested, the test type, and the result. No Ack or Nack is returned to the BTS.

BSC					BTS

Test Report

<--------------------------------------------

### 6.6.3	Send Test Report

This message shall be sent from the BSC to ask for the result/report of a test which was not to be sent autonomously now to be started being reported. If the test result was already made to be autonomously reported, this message can also be used to have  the present result of the test be reported immediately. The message must include identification of the test.

BSC					BTS

Send Test Report

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message: The specified test report will be sent.

Message specific Nack causes (see subclause 9.4.36): 1C, 1F.

### 6.6.4	Stop Test

This message shall be used by the BSC to stop a continuously recurring test at the BTS, to reset a physical test configuration to the normal configuration, or to stop the test and to restore to the normal physical configuration. The message must include identification of the test being performed.

BSC					BTS

Stop Test

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message: The specified test has been stopped and test configuration reset to normal (if 	necessary).

Message specific Nack causes (see subclause 9.4.36): 20, 21, 22.

## 6.7	State Management and Event Report Procedures

### 6.7.1	State Changed Event Report

An unsolicited report shall be sent from the BTS to the BSC whenever a change of the operational state or of the optional manufacturer dependent state of a managed object defined in the present document occurs. The message shall be also sent when any site input changes its state.

A failure, causing change of operational state, shall generate two event reports: State Changed Event Report and Failure Event Report.

No Ack or Nack is returned to the BTS.

BSC					BTS

State Changed Event Report

<--------------------------------------------

### 6.7.2	Failure Event Report

An unsolicited report shall be sent from the BTS to the BSC whenever failure events occur in the BTS.

Such failure events are:

-	fault in a resource resulting from passing a threshold but not constituting a failure;

-	failure of a resource.

Pertaining to a failure, there shall be a report for its start and another for its cease.

A failure causing change of operational state shall generate two event reports: State Changed Event Report and Failure Event Report.

No Ack or Nack is returned to the BTS.

BSC					BTS

Failure Event Report

<--------------------------------------------

### 6.7.3	Stop Sending Event Reports

This inhibition of sending of event reports shall be used by the BSC to prevent a flood of event reports which are of no benefit to the BSC. One example of this occurs at a BTS restart following a power failure. The operational capability of the BTS hardware is unlikely to be different from what it was before the failure, and a flood of reports, each stating that a piece of hardware is operating, will delay the software download. Another example concerns the case of a frequently occurring transient fault.

BSC					BTS

Stop Sending Events Reports

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message: Sending of specified Event Report has been stopped.

Message specific Nack causes (see subclause 9.4.36): None.

### 6.7.4	Restart Sending Event Reports

When the BTS is back in normal operation or if it is of interest to check whether the BTS still generates a flood of Event Reports, a Restart Sending Event Reports shall be sent.

BSC					BTS

Restart Sending Events Reports

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message: Sending of specified Event Report has been restarted.

Message specific Nack causes (see subclause 9.4.36): None.

### 6.7.5	Change Administrative State

The Change Administrative State message shall be used by the BSC to change the administrative state (as specified by specification GSM 12.20) of a managed object.

BSC					BTS

Change Administrative State

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message: The specified change of administrative state has been performed.

Message specific Nack causes (see subclause 9.4.36): None.

### 6.7.6	Change Administrative State Request

This request message shall be sent by the BTS when there is a need to change the administrative state of a managed object at the BTS site. This message can only be initiated as a result of a local MMI command.

BSC					BTS

Change Administrative State Request

<--------------------------------------------

ACK/NACK

-------------------------------------------->

Meaning of Ack message: The request is granted and a change administrative state message will be sent.

Message specific Nack causes (see subclause 9.4.36): 28, 29.

### 6.7.7	Report Outstanding Alarms

This message shall be used by the BSC to ask BTS to report all outstanding alarms related to the object instance indicated in the message. The BTS shall report alarms by sending a series of Failure Event Report messages for all outstanding alarms. Only those alarms previously reported and still outstanding shall be re-reported through this procedure. Any new alarms not yet reported but about to be reported shall be excluded and they shall be reported through a separate Failure Event Report procedure spontaneously initiated by the BTS itself. If there is no outstanding alarm, the BTS shall reply with a NACK with that cause indicated.

BSC					BTS

Report Outstanding Alarms

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message: Sending of Failure Event Report will start.

Message specific Nack causes (see subclause 9.4.36): 28, 29, 2A.

## 6.8	Equipment Management Procedures

### 6.8.1	Change-over

This message shall be sent to change over between active and standby units of equipment. The action may be performed on any addressable BTS entity, manufacturer dependent HW units included. Which type of HW unit to address and how to identify certain units of this type of HW are manufacturer dependent.

BSC					BTS

Changeover

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message: The specified change-over operation has been performed.

Message specific Nack causes (see subclause 9.4.36): 19, 1A, 1E, 23.

### 6.8.2	Opstart

This message shall be sent by the BSC to tell the BTS to attempt to operatethe identified object putting it to an initial normal operational state (i.e., "enabled", see subclause 5.3.2). This message does not affect the object's administrative state if there exists a value explicitly assigned by the BSC. If there is yet no administrative state value explicitly set by the BSC (e.g., at an initialization time), the object shall be presumed to be administratively locked by default. No BTS function is required to be responsible for testing the operability of the identified resource as a consequence of this message. Prior to this message being issued, all necessary physical and logical preparations (such as repair of equipment, software downloading, parameter setting, etc., as needed) are expected to have been completed. If the object is in fact not ready to be in an enabled state, the object will be in a fault condition as a consequence of this message, and the condition shall be handled by the object's normal fault handling function as the condition is detected.

BSC					BTS

Opstart

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message: The BTS has reset the operational state of the specified object to "enabled" 	state.

Message specific Nack causes (see subclause 9.4.36): 19, 1A, 23.

### 6.8.3	Reinitialize

This message shall be sent by the BSC to tell the BTS to have specified hardware resource of the indicated object start a re-initialization procedure as sketched in figure 2. The specifics of a re-initialization procedure, which typically takes place at the time of a cold start of the resource, is manufacturer-dependent. For a software reinitialization, Activate SW message shall be used.

BSC					BTS.

Reinitialize

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message: The BTS has received the message and is about to start a 	reinitialization of the specified resource.

Message specific Nack causes (see subclause 9.4.36): 19, 1A, 23.

### 6.8.4	Set Site Outputs

This message shall be sent by the BSC to tell the BTS to set specified site outputs to the specified state.

BSC					BTS

Set Site Outputs

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message: The outputs have been set.

Message specific Nack causes (see subclause 9.4.36): None.

## 6.9	Measurement Management Procedures

### 6.9.1	Measurement Result Request

This message shall be used by the BSC to request the result of a certain measurement  which is running in the BTS to be sent to the BSC. After having responded with the current result, the BTS initializes the measurement result buffer (e.g. setting to zero) and continues the measurement.

BSC					BTS

Measurement Result Request

-------------------------------------------->

Response/NACK

<--------------------------------------------

Message specific Nack causes (see subclause 9.4.36): 2A, 2B.

### 6.9.2	Measurement Result Response

This message shall be used by the BTS to successfully respond to the BSC on an associated Measurement Result Request completing a Measurement Result Request elementary procedure. By this message, the presently registered measurement result requested shall be reported. If it is not possible for any reason, a NACK message must be used instead of this.

### 6.9.3	Stop Measurement

This message shall be used by the BSC to stop a continuously running measurement at the BTS. The message must include identification of the measurement to be stopped.

BSC					BTS

Stop Measurement

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message: The specified measurement has been stopped

Message specific Nack causes (see subclause 9.4.36): 2A

### 6.9.4	Start Measurement

This message shall be used by the BSC to start a measurement continuously at the BTS. The message must include identification of the measurement to be started.

BSC					BTS

Start Measurement

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Meaning of Ack message: The specified measurement has been started

Message specific Nack causes (see subclause 9.4.36): 2A

## 6.10	Miscellaneous Procedures

### 6.10.1	Get Attributes

This message shall be used by the BSC to tell the BTS to send attributes which have previously been set by the BTS. It may be used as a check on accuracy and be incorporated into normal procedures, or may be used by the BSC to recover information which it has lost.

BSC					BTS

Get Attributes

-------------------------------------------->

Response/NACK

<--------------------------------------------

Message specific Nack causes (see subclause 9.4.36): None.

### 6.10.2	Set Alarm Threshold

This message shall be used by the BSC to tell the BTS some threshold parameters related to fault thresholds.

BSC					BTS

Set Alarm Threshold

-------------------------------------------->

ACK/NACK

<--------------------------------------------

Message specific Nack causes (see subclause 9.4.36): None.

### 6.10.3	Get Attributes Response

This message shall be used by the BTS to successfully respond to the BSC on an associated Get Attribute elementary procedure. By this message, the presently set values of all requested attributes shall be reported. Otherwise, a NACK message shall be used instead of this.

# 7	Structured Procedures

Applicable structured A-bis procedures are described in GSM 12.22.

# 8	Message Details

The formats of all messages in subclauses 8.3 through 8.11 are described each by a table format illustration with no formal text description provided because of the self-explanatory nature of the illustration.

## 8.1	Message Categories

This subclause defines the transport format and coding of the four Network. Management message categories sent over the Abis interface. The various message categories may be sent in either direction. In each message, the message discriminator identifies the category and is transmitted first. In a message the octets must be sent in the order shown in the description of the messages. In an octet, bit 1 must be transmitted first.

In the following subclauses, M and O denote whether information elements are mandatory or optional.

### 8.1.1	Formatted O&M Messages

The message format and coding of these messages are as below.

| INFORMATION ELEMENT | M/O | LENGTH | CODING 8 1 |
| --- | --- | --- | --- |
| Message Discriminator | M | 1 | 1 0 0 0 0 0 0 0 |
| Placement Indicator | M | 1 | 1) |
| Sequence Number | M | 1 | 2) |
| Length Indicator | M | 1 | Binary, 3) |
| O&M Data Field | M | V | 4) |

NOTE 1:	The meanings and codings of the Placement Indicator are:

-	only: This message is contained within one segment	1 0 0 0 0 0 0 0;

-	first: The first segment of a multi-segment message	0 1 0 0 0 0 0 0;

-	middle: A middle segment of a multi-segment message	0 0 1 0 0 0 0 0;

-	last: The last segment of a multi-segment message	0 0 0 1 0 0 0 0.

NOTE 2:	This is the sequence number of the segment in the message, modulo 256, starting with 00000000. Thus a single segment message is here coded 00000000. The number can be incremented unlimitedly being wrapped around the modulo to transport very long multi-segment messages.

NOTE 3:	The Length Indicator gives the length of the O&M data field in the message segment being transported which is less than or equal to 255 octets. This length indicator should not be confused with the actual length of the message at the logical level that may go over multiple segments. This length indicator should not be confused also with attribute value length indicator described in subclause 8.2.

NOTE 4:	Coding for O&M Data field is found in subclause 8.2 and the following subclauses.

### 8.1.2	MMI Transfer

The message format and coding of these messages are as below.

| INFORMATION ELEMENT | M/O | LENGTH | CODING 8 1 |
| --- | --- | --- | --- |
| Message Discriminator | M | 1 | 0 1 0 0 0 0 0 0 |
| Placement Indicator | M | 1 | note 1 of subclause 8.1.1 |
| Sequence Number | M | 1 | note 2 of subclause 8.1.1 |
| Length Indicator | M | 1 | Binary, 1) |
| MMI Data Field | M | V | Proprietary |

NOTE:	The Length Indicator gives the length of the MMI data field in the message segment being transported which is less than or equal to 255 octets. See also note 3 of subclause 8.1.1.

The protocol for MMI is not covered in the present document.

### 8.1.3	TRAU O&M Messages

These messages are required for option (ii) of subclause 4.10.1 of specification GSM 08.60. The message format is as below.

| INFORMATION ELEMENT | M/O | LENGTH | CODING 8 1 |
| --- | --- | --- | --- |
| Message Discriminator | M | 1 | 0 0 1 0 0 0 0 0 |
| Channel Number | M | 1 | 1) |
| Repetition Indicator | M | 1 | 2) |
| TRAU O&M Message | O | 33 | 3) |

NOTE 1:	The channel number element shall be coded as defined for the octet of the channel number information element of specification 3GPP TS 48.058.

NOTE 2:	The meanings and codings of the Repetition Indicator are:

-	terminate sending of O&M TRAU frames: 0 0 0 0 0 0 0 1;

-	repeat sending of O&M TRAU frames every 20 ms until a terminate order is received 0 0 0 0 0 0 1 0;

-	send O&M TRAU frame once: 0 0 0 0 0 1 0 0.

NOTE 3:	The TRAU O&M message is not sent when the Repetition Indicator is set to 00000001, but sending of the TRAU O&M message is mandatory otherwise. All 264 bits of the TRAU O&M frame information field are sent unchanged in the 33 octets.

### 8.1.4	Manufacturer-Defined O&M messages

The message format and coding of these messages is as below.

| INFORMATION ELEMENT | M/O | LENGTH | CODING 8 1 |
| --- | --- | --- | --- |
| Message Discriminator | M | 1 | 0 0 0 1 0 0 0 0 |
| Placement Indicator | M | 1 | note 1 of subclause 8.1.1 |
| Sequence Number | M | 1 | note 2 of subclause 8.1.1 |
| Length Indicator | M | 1 | Binary, 1) |
| ManId Length Indicator | M | 1 | Binary, 2) |
| Manuf. Identifier | M | V | 3) |
| Man-Def O&M Data Field | M | V | Proprietary |

NOTE 1:	The Length Indicator gives the length of the Manufacturer-defined O&M data field in the message segment being transported which is less than or equal to 255 octets. See also note 3 of subclause 8.1.1

NOTE 2:	The Length Indicator gives the length of the Manufacturer Identifier field which must be less than or equal to 255 octets.

NOTE 3:	The Manufacturer Identifier is an octet string of maximally 255 octets. This value, to be appropriately determined by an arrangement between the operator and the manufacturer, may or may not be related to the value of the attribute Manufacturer Id (Attribute Id: 1E) listed in subclause 9.4.

Remarks:	Since the Data Field of messages of this category is not subject to a GSM standardization, it should be noted that a compliance to messages of this category does not guarrantee an interoperability between different manufacturers.

## 8.2	Structure of Formatted O&M Messages

This subclause provides details of all the formatted O&M messages.

In every case when particular header octets provide no usable information at the receiver, they shall be coded all 1's.

All fields in the messages are marked with M for Mandatory or O for Optional. This indicates whether the field is mandatory or optional to be contained in a message, and not whether it is mandatory or optional to be used or set for every BTS. This allows to change a single attribute without having to repeat all the attributes not to be changed.

The header fields of formatted O&M messages (see below) are always mandatory. The attributes defined for a certain message supported by the BTS implementation are mandatory to be used if not stated otherwise in an explanatory note.

The first octet of the formatted O&M messages shall identify the message types. Some messages are replied by an ACK or a NACK response. The replies shall be distinguished by different codings of the message type (the first octet of formatted O&M messages). See subclause 9.1.

ACK messages shall return all the attributes in the original message. NACK messages shall add a Nack cause field (two octets) at the end of the message.

None of the messages concerned requires all of the capacity available in a Layer 2 segment, so the NACK message will not need a second Layer 2 frame.

An ACK to a number of Load Data Segments shall  consist of only the header with the 'Load Data Segment Ack' message type.

All attributes shall overwrite those defined in an earlier message since start-up or the last restart. Optional attributes provide new information if they have not been defined in an earlier message.

The message type and managed object identification are given in the message header as is illustrated below:

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |

The Object Class information element shall be filled in with the correct information in accordance with the present document.

The Object Instance information element shall contain three fields:

1)	the BTS Number identifies one BTS in a multi cell site;

2)	the Baseband Transceiver number or the Radio Carrier number identifies which Baseband Transceiver or Radio Carrier is concerned in the message;

3)	the third element identifies a radio timeslot (0-7) of the Baseband Transceiver identified by 2).

For further information see subclause 9.3.

The FORMAT field describes the structure of each information element using T(Tag), L(Length) and V(Value) coding. T is the attribute identifier. V is the actual information presented. L must be indicated if the the information element is of variable length and its prediction is not possible in the context . L shall binary-represent in a two octet space the the number of octets in the remaining part of information element. Note that this Length code differs from the "Length Indicator" described in subclause 8.1.

## 8.3	SW Download Management Messages

### 8.3.1	Load Data Initiate

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| SW Description | 9.4.62 | M | TV | >=2 |
| Window Size | 9.4.59 | M | TV | 2 |

### 8.3.2	Load Data Segment

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| File Data | 9.4.17 | M (note) | TLV | >=2 |

NOTE: File Data is segments of the actual file to be transferred.

### 8.3.3	Load Data Abort

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |

### 8.3.4	Load Data End

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| SW Description | 9.4.62 | M (note) | TV | >=2 |

NOTE: The same SW Description as in the corresponding Load Data Initiate message.

### 8.3.5	SW Activate Request

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| HW Configuration | 9.4.62 | M | TLV | >=2 |
| SW Configuration | 9.4.61 | M | TLV | >=2 |

### 8.3.6	Activate SW

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| SW Description | 9.4.62 | O (note) | TV | >=2 |

NOTE: SW Descriptions may be repeated for multiple software activation. No SW Description entry implies all software for the object instance.

### 8.3.7	SW Activated Report

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |

## 8.4	A-bis Interface Management Messages

### 8.4.1	Establish TEI

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| TEI | 9.4.54 | M | TV | 2 |
| Abis Channel | 9.4.1 | M | TV | 4 |

### 8.4.2	Connect Terrestrial Signalling

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Abis Channel | 9.4.1 | M | TV | 4 |

### 8.4.3	Disconnect Terrestrial Signalling

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Abis Channel | 9.4.1 | M | TV | 4 |

### 8.4.4	Connect Terrestrial Traffic

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Abis Channel | 9.4.1 | M | TV | 4 |
| Radio Sub Channel | 9.4.46 | O | TV | 2 |

### 8.4.5	Disconnect Terrestrial Traffic

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Abis Channel | 9.4.1 | M | TV | 4 |
| Radio Sub Channel | 9.4.46 | O | TV | 2 |

## 8.5	Transmission Management Messages

### 8.5.1	Connect Multi-drop link

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Multi-drop BSC Link | 9.4.34 | M (note 1) | TV | 3 |
| Multi-drop next BTS Link | 9.4.35 | M (note 2) | TV | 3 |

NOTE 1:	This attribute specifies the incoming (in BSC direction) 64 kbps timeslot to be set up to another BTS in a multi-drop configuration.

NOTE 2:	This attribute specifies the outgoing (to the next BTS direction) 64 kbps timeslot to be set up through the BTS in a multi-drop configuration.

### 8.5.2	Disconnect Multi-drop link

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Multi-drop BSC Link | 9.4.34 | M (note 1) | TV | 3 |
| Multi-drop next BTS Link | 9.4.35 | M (note 2) | TV | 3 |

NOTE 1: 	This attribute specifies the incoming (in BSC direction) 64 kbps timeslot to be taken down from another BTS in a multi-drop configuration.

NOTE 2: 	This attribute specifies the outgoing (to the next BTS direction) 64 kbps timeslot to be taken down through the BTS in a multi-drop configuration.

## 8.6	Air Interface Management Messages

### 8.6.1	Set BTS Attributes

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Interference Level Boundaries | 9.4.25 | O (note 1) | TV | 7 |
| Intave Parameter | 9.4.24 | O | TV | 2 |
| Connection Failure Criterion | 9.4.14 | O | TLV | >=4 |
| T200 | 9.4.53 | O (note 2) | TV | 8 |
| Max Timing Advance | 9.4.31 | O (note 3) | TV | 2 |
| Overload Period | 9.4.39 | O (note 5) | TLV | >=2 |
| CCCH Load Threshold | 9.4.12 | O (notes 4 and 5) | TV | 2 |
| CCCH Load Indication Period | 9.4.11 | O (note 5) | TV | 2 |
| RACH Busy Threshold | 9.4.44 | O (note 5) | TV | 2 |
| RACH Load Averaging Slots | 9.4.45 | O (note 5) | TV | 3 |
| BTS Air Timer | 9.4.10 | O | TV | 2 |
| NY1 | 9.4.37 | O | TV | 2 |
| BCCH ARFCN | 9.4.8 | O | TV | 3 |
| BSIC | 9.4.9 | O | TV | 2 |
| Starting Time | 9.4.52 | O (note 6) | TV | 3 |

NOTE 1:	Interference Level Boundaries	Boundary 0

(see 3GPP TS 45.008)			Boundary x1

¦

¦

Boundary x5

NOTE 2:	For FACCH and SACCH.

NOTE 3:	Optional in BTS design and administration use.

NOTE 4:	Presentation of the load threshold in 1% steps.

NOTE 5:	These attributes are special CCCH Parameters

NOTE 6:	For synchronisation of when the specific action defined in the message shall be performed. Needed only to change BCCH ARFCN or BSIC while in operation.

### 8.6.2	Set Radio Carrier Attributes

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| RF Max Power Reduction | 9.4.47 | O | TV | 2 |
| ARFCN list | 9.4.5 | O (note) | TLV | >=2 |

NOTE:	The structure of value will vary depending on the mode of frequency hopping supported.

### 8.6.3	Set Channel Attributes

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Channel Combination | 9.4.13 | O | TV | 2 |
| HSN | 9.4.21 | O (note 1) | TV | 2 |
| MAIO | 9.4.27 | O (note 1) | TV | 2 |
| ARFCN list | 9.4.5 | O (note 2) | TLV | >=2 |
| Starting Time | 9.4.52 | O (note 3) | TV | 3 |
| TSC | 9.4.60 | O (note 4) | TV | 2 |

NOTE 1:	Used only in a hopping channel.

NOTE 2:	If frequency hopping is not in use, the only frequency of the Radio Carrier is set with the message Radio Carrier Attributes.

NOTE 3:	For synchronisation of when the specific action defined in the message shall be performed.

NOTE 4:	A NULL value signifies that the BCC shall be used as the TSC value for the channel.

## 8.7	Test Management Messages

### 8.7.1	Perform Test

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Test No | 9.4.56 | M | TV | 2 |
| Autonomously Report | 9.4.6 | M | TV | 2 |
| Test Duration | 9.4.55 | O | TV | 3 |
| Physical Configuration | 9.4.40 | O (note) | TLV | >=2 |

NOTE:	Use of Physical Configuration depends on the need on extra information in setting up specific test configurations.

### 8.7.2	Test Report

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Test No | 9.4.56 | M | TV | 2 |
| Test Report Info | 9.4.57 | M (note) | TLV | >=2 |

NOTE:	The test report information may give a numerical result or an indication of the range into which the test report falls.

### 8.7.3	Send Test Report

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Test No | 9.4.56 | M | TV | 2 |

### 8.7.4	Stop Test

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Test No | 9.4.56 | M | TV | 2 |

## 8.8	State Management and Event Report Messages

### 8.8.1	State Changed Event Report

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Operational State | 9.4.38 | O | TV | 2 |
| Availability Status | 9.4.7 | O | TLV | >=2 |
| Man.dep. State | 9.4.28 | O | TV | 2 |
| Site Inputs | 9.4.48 | O | TLV | >=2 |

### 8.8.2	Failure Event Report

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Event Type | 9.4.16 | M | TV | 2 |
| Perceived Severity | 9.4.63 | M | TV | 2 |
| Probable Cause | 9.4.43 | M | TV | 4 |
| Specific Problems | 9.4.51 | O (note 1) | TV | 2 |
| HW Description | 9.4.23 | O (notes 1 and 2) | TV | >=2 |
| SW Description | 9.4.62 | O (notes 1 and 2) | TV | >=2 |
| Additional Text | 9.4.3 | O (note 1) | TLV | >=2 |
| Additional Info | 9.4.2 | O (note 1) | TLV | >=2 |
| Outstanding Alarm Sequence | 9.4.65 | O (note 3) | TV | 2 |

NOTE 1:	Depending on the nature of the specific failure and the BTS implementation, only the needed and supported attributes shall be sent.

NOTE 2:	This field shall be included to identify the specific associated equipment or software in case the addressed functional object alone is not sufficient to localize the failure.

NOTE 3:	This field shall be included if and only if this report is a response to a Report Outstanding Alarms message.

### 8.8.3	Stop Sending Event Reports

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Operational State | 9.4.38 | O (note 1) | TV | 2 |
| Availability Status | 9.4.7 | O (note 1) | TLV | >=2 |
| Manufacturer Dependant State | 9.4.28 | O (note 1) | TV | 2 |
| Probable Cause | 9.4.43 | O (note 1) | TV | 4 |
| Specific Problems | 9.4.51 | O (note 1) | TV | 2 |

NOTE 1:	Stop sending event reports concerning events with any of the parameter values in this attribute list. Depending on the type of event report that shall be stopped, one or some of the attributes shall be sent. The effect of multiple optional attributes in one message is that only those events that satisfy all the attributes simultaneously shall stop. The effect of repeated uses of this message with each different optional attribute is accumulative, thus, is different from the effect of putting all the optional attributes listed together at once in one message. If there occurs any inconsistency or confusion between the conditions for stopping and starting (see subclause 8.8.4), the event shall be reported instead of being stopped.

NOTE 2:	This message with no optional attributes means that all event reports shall be stopped from now.

### 8.8.4	Restart Sending Event Reports

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Operational State | 9.4.38 | O (note 1) | TV | 2 |
| Availability Status | 9.4.7 | O (note 1) | TLV | >=2 |
| Manufacturer Dependent State | 9.4.28 | O (note 1) | TV | 2 |
| Probable Cause | 9.4.43 | O (note 1) | TV | 4 |
| Specific Problems | 9.4.51 | O (note 1) | TV | 2 |

NOTE 1:	Restart sending event reports concerning events with any of the parameter values in this attribute list. Depending on the type of event report that needs to be restarted, one or some of the attributes shall be sent. The effect of multiple optional attributes is just same as multiple messages repeated with each attribute one by one and events that satisfy any one of the attribute set shall be reported. Note the difference from the condition stated in note 1 of subclause 8.8.3.

NOTE 2:	This message with no optional attributes means that all event reports shall be started from now.

### 8.8.5	Change Administrative State

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Administrative State | 9.4.4 | M (note) | TV | 2 |

NOTE:	Required new administrative state for the specified managed object.

### 8.8.6	Change Administrative State Request

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Administrative State | 9.4.4 | M (note) | TV | 2 |

NOTE:	The requested administrative state for the specified managed object.

### 8.8.7	Report Outstanding Alarms

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |

## 8.9	Equipment Management Messages

### 8.9.1	Changeover

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Source | 9.4.50 | M (note 1) | TLV | >=2 |
| Destination | 9.4.15 | M (note 2) | TLV | >=2 |

NOTE 1:	Source is the manufacturer dependent identity of piece of equipment that shall be taken out of active servicing (changed-over from) and replaced by the Destination.

NOTE 2:	Destination is the manufacturer dependent identity of piece of equipment that shall be put into active servicing (changed-over to) in place of the Source.

### 8.9.2	Opstart

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |

### 8.9.3	Reinitialize

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| HW Description | 9.4.23 | O (note) | TV | >=2 |

NOTE:	HW Descriptions may be repreated for multiple resources. If no HW Description is provided, all resource for the objectis is implied. For a software reinitialization, Activate SW message shall be used.

### 8.9.4	Set Site Outputs

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Site Outputs | 9.4.49 | M | TLV | >=2 |

### 8.9.5	Change HW Configuration

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Site Outputs | 9.4.49 | M | TLV | >=2 |

## 8.10	Measurement Management Messages

### 8.10.1	Measurement Result Request

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Measurement Identifier | 9.4.33 | M | TV | 2 |

### 8.10.2	Measurement Result Response

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Measurement Identifier | 9.4.33 | M | TV | 2 |
| Measurement Result | 9.4.32 | M | TLV | >= 2 |

### 8.10.3	Start Measurement

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Measurement Identifier | 9.4.33 | M | TV | 2 |

### 8.10.4	Stop Measurement

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Measurement Identifier | 9.4.33 | M | TV | 2 |

## 8.11	Miscellaneous Messages

### 8.11.1	Get Attributes

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| List of Required Attributes | 9.4.26 | M (note) | TLV | >=2 |

NOTE:	Any of the attributes defined in subclause 9.4, including those not marked with an X in the GET column, can be requested by Get Attributes message.

### 8.11.2	Set Alarm Threshold

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Probable Cause | 9.4.43 | M | TV | 4 |
| Power Output Thresholds | 9.4.42 | O | TV | 4 |
| VSWR Thresholds | 9.4.58 | O | TV | 3 |
| Manufacturer Dependent Thresholds | 9.4.29 | O | TLV | >=2 |

### 8.11.3	Get Attribute Response

| INFORMATION ELEMENT | REFERENCE | PRESENCE | FORMAT | LENGTH |
| --- | --- | --- | --- | --- |
| Message Type | 9.1 | M | V | 1 |
| Object Class | 9.2 | M | V | 1 |
| Object Instance | 9.3 | M | V | 3 |
| Get Attribute Response Info | 9.4.64 | M | TLV | >=4 |

# 9	Coding

This clause defines the coding of each field in the messages defined in earlier clauses.

The following conventions are required:

-	the least significant bit shall be transmitted first, followed by bits 2, 3, 4, etc;

-	in an element where octets are identified by an octer number, Octet 1 shall be transmitted first, then octet 2, etc;

-	when a field extends over more than one octet, the order of bit values shall progressively decrease as the octet number increases. The least significant bit of the field shall be represented by the lowest numbered bit of the highest numbered octet of the field;

-	for unpredictable variable length elements, a length indication coding method defined in subclause 8.2 shall be used. Always the length information shall indicate the number of element units (which is octets) following the length indicator excluding the space for the length findicator itself;

-	all defined values are indicated in the present document. Other values are reserved.

## 9.1	Message Type

The Message Type is coded with 1 octet as illustrated below.

| Message Type | 1 |
| --- | --- |

The following message types are used (all other values reserved).

Message Type	hexadecimal code

SW Download Management Messages:

Load Data Initiate	01

Load Data Initiate Ack	02

Load Data Initiate Nack	03

Load Data Segment	04

Load Data Segment Ack	05

Load Data Abort	06

Load Data End	07

Load Data End Ack	08

Load Data End Nack	09

SW Activate Request	0A

SW Activate Request Ack	0B

SW Activate Request Nack	0C

Activate SW	0D

Activate SW Ack	0E

Activate SW Nack	0F

SW Activated Report	10

A-bis Interface Management Messages:

Establish TEI	21

Establish TEI Ack	22

Establish TEI Nack	23

Connect Terrestrial Signalling	24

Connect Terrestrial Signalling Ack	25

Connect Terrestrial Signalling Nack	26

Disconnect Terrestrial Signalling	27

Disconnect Terrestrial Signalling Ack	28

Disconnect Terrestrial Signalling Nack	29

Connect Terrestrial Traffic	2A

Connect Terrestrial Traffic Ack	2B

Connect Terrestrial Traffic Nack	2C

Disconnect Terrestrial Traffic	2D

Disconnect Terrestrial Traffic Ack	2E

Disconnect Terrestrial Traffic Nack	2F

Transmission Management Messages:

Connect Multi-Drop Link	31

Connect Multi-Drop Link Ack	32

Connect Multi-Drop Link Nack	33

Disconnect Multi-Drop Link	34

Disconnect Multi-Drop Link Ack	35

Disconnect Multi-Drop Link Nack	36

Air Interface Management Messages:

Set BTS Attributes	41

Set BTS Attributes Ack	42

Set BTS Attributes Nack	43

Set Radio Carrier Attributes	44

Set Radio Carrier Attributes Ack	45

Set Radio Carrier Attributes Nack	46

Set Channel Attributes	47

Set Channel Attributes Ack	48

Set Channel Attributes Nack	49

Test Management Messages:

Perform Test	51

Perform Test Ack	52

Perform Test Nack	53

Test Report	54

Send Test Report	55

Send Test Report Ack	56

Send Test Report Nack	57

Stop Test  	58

Stop Test Ack	59

Stop Test Nack	5A

State Management and Event Report Messages:

State Changed Event Report	61

Failure Event Report	62

Stop Sending Event Reports	63

Stop Sending Event Reports Ack	64

Stop Sending Event Reports Nack  	65

Restart Sending Event Reports	66

Restart Sending Event Reports Ack	67

Restart Sending Event Reports Nack	68

Change Administrative State	69

Change Administrative State Ack	6A

Change Administrative State Nack	6B

Change Administrative State Request	6C

Change Administrative State Request Ack	6D

Change Administrative State Request Nack	6E

Report Outstanding Alarms	93

Report Outstanding Alarms Ack	94

Report Outstanding Alarms Nack	95

Equipment Management Messages:

Changeover	71

Changeover Ack	72

Changeover Nack	73

Opstart	74

Opstart Ack	75

Opstart Nack	76

Reinitialize	87

Reinitialize Ack	88

Reinitialize Nack	89

Set Site Outputs	77

Set Site Outputs Ack	78

Set Site Outputs Nack	79

Change HW Configuration	90

Change HW Configuration Ack	91

Change HW Configuration Nack	92

Measurement Management Messages:

Measurement Result Request	8A

Measurement Result Response	8B

Stop Measurement	8C

Start Measurement	8D

Other Messages:

Get Attributes	81

Get Attributes Response	82

Get Attributes Nack	83

Set Alarm Threshold	84

Set Alarm Threshold Ack   	85

Set Alarm Threshold Nack	86

## 9.2	Object Class

An Object Class shall be coded with 1 octet. The values of the object class code are as defined below.

| Object Class | 1 |
| --- | --- |

Object Class	hexadecimal code

Site Manager	00

BTS	01

Radio Carrier	02

Baseband Transceiver	04

Channel	03

<reserved for future use>	<05-FE>

NULL	FF

## 9.3	Object Instance

The Object Instance shall be coded with 3 octets, addressing the specific object of the given object class as illustrated below.

| BTS number | 1 |
| --- | --- |
| Baseband Transceiver or Radio Carrier number | 2 |
| Radio Timeslot number | 3 |

All 3 octets are mandatory in the header of every message.

The BTS number distinguishes BTSs which are unique for each cells at a site under the Site Manager.

The Baseband Transceiver or Radio Carrier number distinguishes either Baseband Transceivers or Radio Carriers under the BTS.

The Radio Timeslot number distinguishes channels under the Baseband Transceiver.

When the object class is Site Manager, all three octets shall be NULLs, because there is only one Site Manager, which doesn't need an identifier.

When the object class is BTS, octet 1 shall be a binary presentation of the identifier of the addressed BTS. Octets 2 and 3 shall be coded NULLs. If the BTS number is NULL, it shall be understood as referring to all BTSs under the Site Manager.

When the object class is Baseband Transceiver or Radio Carrier, octet 2 shall be a binary presentation of the identifier of the addressed Baseband Transceiver or Radio Carrier object, and octet 1 is the identiferrof the BTS above it. Octet 3 is coded NULL. If the Baseband Transceiver or Radio Carrier number is NULL, it shall be understood as referring to all instances of the class under the BTS.

When the object class is Channel, octet 3 shall be a binary presentation of the identifier of the addressed radio timeslot, octet 2 shall be the identifier of the Baseband Transceiver, and octet 1 shall be the identifer of the BTS above the addressed Channel. If the radio timeslot number is NULL, it shall be understood as referring to all Channels under the Baseband Transceiver.

To avoid unnecessary complexity of BTS implementation, it shall not be allowed to assign a NULL value to any object above the addressed object class. For example, if the addressed object class is the Baseband Transceiver, it is not allowed to assign a NULL value to both BTS and Baseband Transceiver instances (without this constraint, this could be understood as referring to all the Baseband Transceivers of all the BTSs).

The value for NULL shall be <FF> in all the cases mentioned above in this subclause.

## 9.4	Attributes and Parameters

The Attribute Identifier is coded with 1 octet. The number of parameters within an attribute is at least one. The length of the parameters within an attribute will vary. The attributes used and the coding of their Attribute Identifier fields are listed below. The values are in hexadecimal. The attributes marked with an X in “Get” column are only getable by the BSC, and BSC is not permitted to alter their values.

Attribute Name	Attribute Id.	Get

Abis Channel	01	X

Additional Info	02

Additional Text	03

Administrative State	04	X

ARFCN List	05	X

Autonomously Report	06

Availability Status	07	X

BCCH ARFCN	08	X

BSIC	09	X

BTS Air Timer	0A	X

CCCH Load Indication Period	0B	X

CCCH Load Threshold	0C	X

Channel Combination	0D	X

Connection Failure Criterion	0E	X

Destination	0F

Event Type	11

File Data	47

File Id	12	X

File Version	13	X

GSM Time	14	X

Get Attribute Response Info	44

HSN	15	X

HW Configuration	16	X

HW Conf Change Info	45

HW Description	17	X

Intave Parameter	18	X

Interference Level Boundaries	19	X

List of Required Attributes	1A

MAIO	1B	X

Manufacturer Dependent State	1C	X

Manufacturer Dependent Thresholds	1D	X

Manufacturer Id	1E	X

Max Timing Advance	1F	X

Multi-drop BSC Link	20

Multi-drop next BTS Link	21

Nack Causes	22

Ny1	23	X

Operational State	24	X

Outstanding Alarm Sequence	46

Overload Period	25	X

Physical Config	26

Power Class	27	X

Power Output Thresholds	28	X

Probable Cause	29

RACH Busy Threshold	2A	X

RACH Load Averaging Slots	2B	X

Radio Sub Channel	2C

RF Max Power Reduction	2D	X

Site Inputs	2E	X

Site Outputs	2F	X

Source	30

Specific Problems	31

Starting Time	32

T200	33	X

TEI	34	X

Test Duration	35

Test No	36

Test Report Info	37

VSWR Thresholds	38	X

Window Size	39

TSC	40	X

SW Configuration	41	X

SW Description	42	X

Perceived Severity	43

Measurement Result	48

Measurement Type	49

All other values are reserved for future use.

The data structures of the attributes and parameters are described in the remaining part of this subclause in tablular forms with no formal text description of the individual subclauses provided because of their self-explanatory nature.

Henceforth "Attribute Identifier" in this subclause means the identifier for an attribute or a parameter.

### 9.4.1	Abis Channel

| Attribute Identifier | 1 |
| --- | --- |
| BTS Port Number | 2 |
| Timeslot Number | 3 |
| Subslot Number | 4 |

BTS Port Number	<0-FF>

Timeslot Number

Time slot in 2 Mbps link	<0-1F>

Subslot Number

a (bits 1,2)	00

b (bits 3,4)	01

c (bits 5,6)	02

d (bits 7,8)	03

64 kbps signalling	FF

### 9.4.2	Additional Info

| Attribute Identifier | 1 |
| --- | --- |
| Length | 2-3 |
| Additional Info <man.dep.> | 4 |
| (cont.) |  |
| (cont.) | N |

Additional Info is a manufacturer dependent field.

### 9.4.3	Additional Text

| Attribute Identifier | 1 |
| --- | --- |
| Length | 2-3 |
| Additional Text <man.dep.> | 4 |
| (cont.) |  |
| (cont.) | N |

Additional Text is a manufacturer dependent field and shall be used to include fault localization information.

### 9.4.4	Administrative State

| Attribute Identifier | 1 |
| --- | --- |
| Administrative State | 2 |

Administrative State shall be coded as follows:

Locked	01

Unlocked	02

Shutting Down	03

NULL (Adm. state not supported)	FF

### 9.4.5	ARFCN List

| Attribute Identifier | 1 |
| --- | --- |
| Length | 2-3 |
| ARFCN1 (MSB) | 4 |
| ARFCN1 (LSB) | 5 |
| ARFCNn (MSB) | N-1 |
| ARFCNn (LSB) | N |

ARFCN (see 3GPP TS 45.005)	<0-3FF>

### 9.4.6	Autonomously Report

| Attribute Identifier | 1 |
| --- | --- |
| Autonomously Report | 2 |

The toggle switch for autonomous report shall be coded as follows:

Autonomously Report	01;

Not Autonomously Report	00.

### 9.4.7	Availability Status

| Attribute Identifier | 1 |
| --- | --- |
| Length | 2-3 |
| Availability Status | 4 |
| (cont.) |  |
| (cont.) | N |

Availability Status may contain one or more octets. Each octet shall have a single status value, which shall be coded as follows:

In test	0

Failed	1

Power off	2

Off line	3

<not used>	4

Dependency	5

Degraded	6

Not installed	7

### 9.4.8	BCCH ARFCN

| Attribute Identifier | 1 |
| --- | --- |
| BCCH ARFCN (MSB) | 2 |
| BCCH ARFCN (LSB) | 3 |

BCCH ARFCN (see 3GPP TS 45.005)	<1-3FF>

### 9.4.9	BSIC

| Attribute Identifier | 1 |
| --- | --- |
| BSIC | 2 |

BSIC (see GSM 03.03)	<0-3F>

(BS colour code:	bit 0-2)

(PLMN colour code:	bit 3-5)

### 9.4.10	BTS Air Timer

| Attribute Identifier | 1 |
| --- | --- |
| T3105 | 2 |

T3105 shall be coded as a binary presentation of 10 ms in the range of <0-FF>.

### 9.4.11	CCCH Load Indication Period

| Attribute Identifier | 1 |
| --- | --- |
| CCCH Load Indication Period | 2 |

CCCH Load Indication Period shall be coded as a binary presentation of seconds in the range of <0-FF>.

### 9.4.12	CCCH Load Threshold

| Attribute Identifier | 1 |
| --- | --- |
| CCCH Load Threshold | 2 |

CCCH Load Threshold shall be coded as a binary presentation of maximum (maximum is manufacturer dependent) percentage in the range of <0-64>.

### 9.4.13	Channel Combination

| Attribute Identifier | 1 |
| --- | --- |
| Channel Combination | 2 |

Channel Combination shall be coded as follows:

tCHFull	TCH/F + FACCH/F + SACCH/TF	00

tCHHalf	TCH/H(0,1) + FACCH/H(0,1) + SACCH/TH(0,1)	01

tCHHalf2	TCH/H(0) + FACCH/H(0) + SACCH/TH(0) + TCH/H(1)	02

sDCCH	SDCCH/8 + SACCH/C8	03

mainBCCH	FCCH + SCH + BCCH + CCCH	04

bCCHCombined	FCCH + SCH + BCCH + CCCH + SDCCH/4 + SACCH/C4	05

bCH	BCCH + CCCH	06

bCCHwithCBCH	combination of 05 with CBCH as 3GPP TS 45.002	07 1)

sDCCHwithCBCH	combination of 03 with CBCH as 3GPP TS 45.002	08 1)

NOTE:	These two channel combinations are created by replacing the SDCCH number 2 in channel combination 05 and channel combination 03 by the logical channel.

See 3GPP TS 45.002 for meaning and usage of channel combinations.

### 9.4.14	Connection Failure Criterion

| Attribute Identifier | 1 |
| --- | --- |
| Length | 2-3 |
| Connection Failure Criterion | 4 |
| Criteria Value | 5 |
| (cont.) |  |
| (cont.) | N |

Connection Failure Criterion shall be coded as follows:

Based upon uplink SACCH error rate (RADIO_LINK_TIMEOUT)	01

Based upon RXLEV/RXQUAL measurements	02

If connection failure criterion is based on SACCH error rate, the Criteria Value shall be one octet binary presentation of the number of SACCH multiframes used in the procedure.

If connection failure criterion is based on RXLEV/RXQUAL measurements, coding of the Criteria Value field will be operator/manufacturer dependent.

See 3GPP TS 45.008 subclauses 5.2 and 5.3 and 3GPP TS 24.008 subclause 3.5.2.2.

### 9.4.15	Destination

| Attribute Identifier | 1 |
| --- | --- |
| Length | 2-3 |
| Destination | 4 |
| (cont.) |  |
| (cont.) | N |

Destination identifies a unit of equipment that shall be the destination to be "changed to" on a Change-over operation. How to identify a type of equipment and how to identify a specific unit of this type is manufacturer dependent.

### 9.4.16	Event Type

| Attribute Identifier | 1 |
| --- | --- |
| Event Type | 2 |

Event Type shall be coded as follows:

communication failure	00

quality of service failure	01

processing failure	02

equipment failure	03

environment failure	04

<reserved for future use>	<05-0F>

<man.dep.>	<10-FF>

### 9.4.17	File Data

| Attribute Identifier | 1 |
| --- | --- |
| Length | 2-3 |
| File Data <man.dep.> | 4 |
| (cont.) |  |
| (cont.) | N |

File Data is manufacturer dependent, but must be consistent with the associated GSM 12.20 attribute.

### 9.4.18	File Id

| Attribute Identifier | 1 |
| --- | --- |
| Length | 2-3 |
| File Id <man.dep.> | 4 |
| (cont.) |  |
| (cont.) | N |

File Id is manufacturer dependent, but must be consistent with the associated GSM 12.20 attribute.

### 9.4.19	File Version

| Attribute Identifier | 1 |
| --- | --- |
| Length | 2-3 |
| File Version <man.dep.> | 4 |
| (cont.) |  |
| (cont.) | N |

File Version is manufacturer dependent, but must be consistent with the associated GSM 12.20 attribute.

### 9.4.20	GSM Time

| Attribute Identifier | 1 |
| --- | --- |
| Current Frame No (MSB) | 2 |
| Current Frame No (LSB) | 3 |

Current Frame No shall be a binary presentation of FN in Modulus 42432.

### 9.4.21	HSN

| Attribute Identifier | 1 |
| --- | --- |
| HSN | 2 |

HSN (see 3GPP TS 45.002)	<0-3F>

### 9.4.22	HW Configuration

| Attribute Identifier | 1 |
| --- | --- |
| Length | 2-3 |
| HW Description 1 | 4 |
| ... |  |
| HW Description n | N |

HW Configuration shall consist of a list of HW Descriptions related to a managed object.

### 9.4.23	HW Description

| Attribute Identifier | 1 |
| --- | --- |
| Equipment Id Length | 2-3 |
| Equipment Id |  |
| (cont.) |  |
| Equipment Type Length |  |
| Equipment Type |  |
| (cont.) |  |
| Equipment Version Length |  |
| Equipment Version |  |
| (cont.) |  |
| Location Length |  |
| Location |  |
| (cont.) |  |
| Man. Dep. Info Length |  |
| Man. Dep. Info |  |
| (cont.) | N |

All fields are manufacturer dependent variable length character strings. They must be consistent with associated GSM 12.20 attributes.

Equipment Id distinguishes a piece of equipment out of others of same type.

Equipment Type codes the type of piece of equipment (e.g., Baseband Transceiver Unit).

Equipment Version codes the version of the piece of equipment.

Location codes the place where the piece of equipment is found (e.g., row -rack - shelf - slot).

Man. Dep. Info shall codes additional manufacturer dependent information.

### 9.4.24	Intave Parameter

| Attribute Identifier | 1 |
| --- | --- |
| Intave Parameter | 2 |

Intave Parameter (see 3GPP TS 45.008)	<1-1F>

### 9.4.25	Interference level Boundaries

| Attribute Identifier | 1 |
| --- | --- |
| Interf Boundary 0 | 2 |
| Interf Boundary 1 | 3 |
| Interf Boundary 2 | 4 |
| Interf Boundary 3 | 5 |
| Interf Boundary 4 | 6 |
| Interf Boundary 5 | 7 |

Interf Boundary 0	<0-FF>

Interf Boundary X1	<0-FF>

Interf Boundary X2	<0-FF>

Interf Boundary X3	<0-FF>

Interf Boundary X4	<0-FF>

Interf Boundary X5	<0-FF>

All boundaries shall be coded as a binary presentation of -x dBm (normal values are -115 to -85). For example, an interference boundary of -105 dBm will be represented as integer “105”.

### 9.4.26	List of Required Attributes

| Attribute Identifier | 1 |
| --- | --- |
| Length | 2-3 |
| Attribute Id | 4 |
| (cont.) |  |
| (cont.) | N |

Each Attribute Id is one octet.

### 9.4.27	MAIO

| Attribute Identifier | 1 |
| --- | --- |
| MAIO | 2 |

MAIO (see 3GPP TS 45.002)	<0-3F>

### 9.4.28	Manufacturer Dependent State

| Attribute Identifier | 1 |
| --- | --- |
| Manufacturer Dependent State | 2 |

Manufacturer Dependent State	<0-FF>

This is optional to be used in addition to Operational State in accordance with GSM 12.20.

### 9.4.29	Manufacturer Dependent Thresholds

| Attribute Identifier | 1 |
| --- | --- |
| Length | 2-3 |
| Manufacturer Dependent Thresholds | 4 |
| (cont.) |  |
| (cont.) | N |

The content of Manufacturer Dependent Thresholds is manufacturer dependent.

### 9.4.30	Manufacturer Id

| Attribute Identifier | 1 |
| --- | --- |
| Length | 2-3 |
| Manufacturer Id | 4 |
| (cont.) |  |
| (cont.) | N |

The content of Manufacturer Id is manufacturer dependent.

### 9.4.31	Max Timing Advance

| Attribute Identifier | 1 |
| --- | --- |
| Max Timing Advance | 2 |

Max Timing Advance (see 3GPP TS 48.058)	<0-7F>

### 9.4.32	Measurement Result

| Attribute Identifier | 1 |
| --- | --- |
| Length of measurement result (N) | 2-3 |
| Result (MSB) | 4 |
| (cont.) |  |
| Result (LSB) | N+3 |

### 9.4.33	Measurement Type

| Attribute Identifier | 1 |
| --- | --- |
| Measurement Number | 2 |

Measurement Number shall be coded as follows:

<reserved>	<00-3F>

<man.dep.>	<40-FF>

### 9.4.34	Multi-drop BSC Link

| Attribute Identifier | 1 |
| --- | --- |
| BTS Port Number | 2 |
| Timeslot Number | 3 |

BTS Port Number	<0-FF>

Timeslot Number

Time slot in 2 Mbps link	<0-1F>

### 9.4.35	Multi-drop next BTS Link

| Attribute Identifier | 1 |
| --- | --- |
| BTS Port Number | 2 |
| Timeslot Number | 3 |

BTS Port Number	<0-FF>

Timeslot Number

Time slot in 2 Mbps link	<0-1F>

### 9.4.36	Nack Causes

| Attribute Identifier | 1 |
| --- | --- |
| NACK Cause | 2 |

Nack Causes shall be coded as follows:

General Nack Causes:

Incorrect message structure	01

Invalid message type value	02

<reserved>	03

<reserved>	04

Invalid Object class value	05

Object class not supported	06

BTS no. unknown	07

Baseband Transceiver no. unknown	08

Object Instance unknown	09

<reserved>	<0A-0B>

Invalid attribute identifier value	0C

Attribute identifier not supported	0D

Parameter value outside permitted range	0E

Inconsistency in attribute list	0F	1)

Specified implementation not supported	10

Message cannot be performed	11	2)

<reserved>	<12-18>

Specific Nack Causes:

Resource not implemented	19

Resource not available	1A

Frequency not available	1B

Test not supported	1C

Capacity restrictions	1D

Physical configuration cannot be performed	1E

Test not initiated	1F

Physical configuration cannot be restored	20

No such test	21

Test cannot be stopped	22

Message inconsistent with physical config.	23	3)

Complete file not received	25

File not available at destination	26

File cannot be activated	27

Request not granted	28

Wait	29

Nothing reportable existing	2A

Measurement not supported	2B

Measurement not started	2C

<reserved>	<2D-7F>

<man.dep.>	<80-FE>

NULL	FF

NOTE 1:	This Nack cause shall apply to conflicting or incomplete data in the attribute list which prevents the BTS from performing the message.

NOTE 2:	This Nack cause shall apply when the message is valid and is supported by the BTS, but cannot be performed correctly for reasons not covered by other general or special Nack causes.

NOTE 3:	This Nack cause shall apply to the case where the data in attribute list is valid, but is beyond the capabilities of the particular BTS implementation.

### 9.4.37	Ny1

| Attribute Identifier | 1 |
| --- | --- |
| Ny1 | 2 |

Ny1 shall be a binary presentation of maximum number of repetitions of PHYSICAL INFORMATION (see 3GPP TS 24.008, subclause 3.4.4.2.2).

### 9.4.38	Operational State

| Attribute Identifier | 1 |
| --- | --- |
| Operational State | 2 |

Operational States are in accordance with GSM 12.20 and shall be coded as follows:

Disabled	01

Enabled	02

<reserved for future use>	<03-FE>

NULL(Operat. state not supported)	FF

### 9.4.39	Overload Period

| Attribute Identifier | 1 |
| --- | --- |
| Length | 2-3 |
| Overload Period | 4 |
| (cont.) |  |
| (cont.) | N |

### 9.4.40	Physical Config

| Attribute Identifier | 1 |
| --- | --- |
| Length | 2-3 |
| Required Test Config <man.dep.> | 4 |
| (cont.) |  |
| (cont.) | N |

Required Test Config is manufacturer dependent.

### 9.4.41	Power Class

| Attribute Identifier | 1 |
| --- | --- |
| Power Class | 2 |

Power Class is a binary presentation of transceiver power class as defined in 3GPP TS 45.005.

### 9.4.42	Power Output Thresholds

| Attribute Identifier | 1 |
| --- | --- |
| Output Power Fault Threshold | 2 |
| Reduced Output Power Threshold | 3 |
| Excessive Output Power Threshold | 4 |

When the carrier output power of the transmitter, including power amplifier and transmitter combiner, is below or above a power level, a fault shall be generated. There are two thresholds below the output power to indicate two different levels of fault severity, and one for threshold above. The range of Output Power Fault Threshold and Reduced Output Power Threshold is from -10 to -1 dB . The values shall be coded in 1 dB steps such that -10 dB is expressed with 0 and -1 dB with 9.

Output Power Threshold	<00-09>

Reduced Output Power Threshold	<00-09>

The range of Excessive Output Power Threshold is from 2 dB to 5 dB. The value shall be coded in 1 dB steps such that 2 dB is expressed with binary value 0 and 5 dB with binary value 3.

Excessive Output Power Threshold	<00-03>

### 9.4.43	Probable Cause

| Attribute Identifier | 1 |
| --- | --- |
| Probable Cause Type | 2 |
| Probable Cause Value | 3 |
| Probable Cause Value (cont.) | 4 |

Probable Cause Type shall be coded as follows:

ISO/CCITT values (X.721)	01

GSM specific values 	02

Manufacturer specific values	03

<reserved for future use>	<04-FF>

For Probable Cause Value coding, the last numeric value of the object identifier value specified in ASN.1 syntax coding shall be used if Probable Cause Type is either 01 or 02.

### 9.4.44	RACH Busy Threshold

| Attribute Identifier | 1 |
| --- | --- |
| RACH Busy Threshold | 2 |

RACH Busy Threshold shall be a binary presentation of -x dBm in the range <0-FF>.

### 9.4.45	RACH Load Averaging Slots

| Attribute Identifier | 1 |
| --- | --- |
| RACH Load Averaging Slots (MSB) | 2 |
| RACH Load Averaging Slots (LSB) | 3 |

RACH Load Averaging Slots shall be a binary presentation of number of RACH burst periods in the range <0-FFFF>.

### 9.4.46	Radio Sub Channel

| Attribute Identifier | 1 |
| --- | --- |
| Radio Sub Channel | 2 |

Radio Sub Channel shall be coded as follows:

Half rate channel 0	00

Half rate channel 1	01

### 9.4.47	RF Max Power Reduction

| Attribute Identifier | 1 |
| --- | --- |
| RF Max Power Reduction | 2 |

RF Max Power Reduction shall be a binary presentation of the number of 2 dB power reduction steps as defined in 3GPP TS 45.005.

This represents "Pn" as used in 3GPP TS 48.058.

### 9.4.48	Site Inputs

| Attribute Identifier | 1 |
| --- | --- |
| Length | 2-3 |
| Site Input | 4 |
| (cont.) |  |
| (cont.) | N |

If Site Inputs are requested from Site Manager with message Get Attributes, all inputs shall be listed. Each octet from 4 to N controls one Site input. Each of these octets contain the input number and the state of the input and they shall be coded as follows.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| State | Input number |  |  |  |  |  |  |

State shall be a binary presentation of the input state, 0 or 1. Input number shall be a binary presentation of input number. Note that maximally 128 binary inputs can be supported.

### 9.4.49	Site Outputs

| Attribute Identifier | 1 |
| --- | --- |
| Length | 2-3 |
| Site Output | 4 |
| (cont.) |  |
| (cont.) | N |

If Site Outputs are sent to Site Manager with message Set Site Outputs, the site outputs shall be generated as indicated by the message. Coding of this information element shall be the same as for Site Inputs. Note that maximally 128 binary site outputs can be supported.

### 9.4.50	Source

| Attribute Identifier | 1 |
| --- | --- |
| Length | 2-3 |
| Source | 4 |
| (cont.) |  |
| (cont.) | N |

Source identifies a unit of equipment that shall be "changed from" on a Change-over operation. How to identify a type of equipment and how to identify a specific unit of this type is manufacturer dependent.

### 9.4.51	Specific Problems

| Attribute Identifier | 1 |
| --- | --- |
| Specific Problems | 2 |

Specific Problems

<reserved for future use>	<00-0F>

<man.dep.>	<10-FF>

### 9.4.52	Starting Time

| Attribute Identifier | 1 |
| --- | --- |
| Current Frame No (MSB) | 2 |
| Current Frame No (LSB) | 3 |

Current Frame No shall be a binary presentation FN Modulus 42432.

### 9.4.53	T200

| Attribute Identifier | 1 |
| --- | --- |
| SDCCH ( 5 ms) | 2 |
| FACCH/Full rate ( 5 ms) | 3 |
| FACCH/Half rate ( 5 ms) | 4 |
| SACCH with TCH SAPI0 (10 ms) | 5 |
| SACCH with SDCCH (10 ms) | 6 |
| SDCCH SAPI3 ( 5 ms) | 7 |
| SACCH with TCH SAPI3 (10 ms) | 8 |

Each field shall be a binary presentation of time units mentioned in brackets.

NULL (FF) means the value is not specified (see 3GPP TS 44.006).

### 9.4.54	TEI

| Attribute Identifier | 1 |
| --- | --- |
| TEI | 2 |

TEI (see 3GPP TS 48.056):

Initial TEI	<00-3F>

Additional TEI	<40-7E>

<not used>	<7F-FF>

### 9.4.55	Test Duration

| Attribute Identifier | 1 |
| --- | --- |
| Test Duration | 2-3 |

Test Duration shall be a binary presentation of seconds in range <01-FFFF>indicating the time the test should last.

### 9.4.56	Test No

| Attribute Identifier | 1 |
| --- | --- |
| Test Number | 2 |

Test Number shall be coded as follows:

Radio loop test via antenna	00

Radio loop test via transceiver	01

BTS functional object self test	02

<reserved>	<03-3F>

<man.dep.>	<40-FF>

### 9.4.57	Test Report Info

| Attribute Identifier | 1 |
| --- | --- |
| Length | 2-3 |
| Test Result Info | 4 |
| (cont.) |  |
| (cont.) | N |

If the test was either radio loop test via antenna or radio loop test via transceiver, octets 3 and 4 are defined. Octet 3 shall include a binary presentation of RXLEV value in the range of 0-3F such that value 0 equals to -110 dBm or less and 3F equals to -48 dBm or more. Octet 4 shall express bit error rate (BER) of received bit stream in 0,1% steps in the range 0,0% - 12,8% such that 0 equals to 0,0% BER and 128(dec) equals to 12,8% BER or more.

If the test was BTS functional object self test, octet 3 shall indicate pass or fail for the test of the functional object on the BTS by value 1 or 0 where 0 is the code for fail.

In the defined test cases Test Result Info may also contain manufacturer dependent information in subsequent octets. In other tests, Test Result Info is manufacturer dependent.

### 9.4.58	VSWR Thresholds

| Attribute Identifier | 1 |
| --- | --- |
| Faulty Antenna Threshold | 2 |
| Antenna Not Adjusted Threshold | 3 |

VSWR Thresholds shall be used in association with the "faulty receiver (transmitter) antenna" or "receiver (transmitter) antenna not adjusted" Probable Cause. The VSWR range for both thresholds is 1,2 - 3,0. The value shall be coded in 0,1 steps such that 1,2 is expressed with 0 and 3,0 with 12(hex).

Faulty Antenna Threshold	<00-12>

Antenna Not Adjusted Threshold	<00-12>

### 9.4.59	Window Size

| Attribute Identifier | 1 |
| --- | --- |
| Window Size | 2 |

Window Size shall be a binary presentation of the number of layer 3 Load Data Segment messages to be sent before a layer 3 acknowledgment needs to be issued. Value 0 is not used.

### 9.4.60	TSC

| Attribute Identifier | 1 |
| --- | --- |
| Training Sequence Code | 2 |

For details of Traing Sequence Codes, see 3GPP TS 45.002.

TSC values	<00-07>

<not used>	<08-FE>

### 9.4.61	SW Configuration

| Attribute Identifier | 1 |
| --- | --- |
| Length | 2-3 |
| SW Description 1 | 4 |
| ... |  |
| SW Description n | N |

SW Configuration shall contain a list of SW Descriptions related to the managed object.

### 9.4.62	SW Description

| Attribute Identifier | 1 |
| --- | --- |
| File Id | 2 |
| File Version | N |

### 9.4.63	Perceived Severity

| Attribute Identifier | 1 |
| --- | --- |
| Severity Value | 2 |

Severity Value shall be coded as follows:

failure ceased	00

critical failure	01

major failure	02

minor failure	03

warning level failure	04

indeterminate failure	05

<reserved> 	<06-3F>

<man, dep.>	<40-FF>

### 9.4.64	Get Attribute Response Info

| Attribute Identifier | 1 |
| --- | --- |
| Length | 2-3 |
| Count of not-reported attributes | 4 |
| (Each not-reported atttribute id) |  |
| (cont.) |  |
| (cont.) |  |
| ...... |  |
| (Each reported attribute id and its value) |  |
| (cont.) |  |
| (cont.) | N |

The BTS shall make a best effort to report on all the attributes requested by the associated Get Attributes message. The Length indicates the length of the entire Get Attribute Response Info which may possibly go over a single segment message and should not be confused with the length of the data put in one single message frame (see subclause 8.1.1). The Count of not reported attributes entry shall indicate the binary integer coded value of the count of the attributes requested but the BTS could not report on because the attributes are either not supported or not yet defined. In case of a non-zero value of the above counter, the Each not- reported attribute id entry  shall list the Id of each attrute not reported and shall continue as many as indicated by the above counter. The Each reported attribute id and its value entry shall list each reported attribute id and its value pair coded and formatted for every applicable attribute or parameter as specified in this subclause (subclause 9.4) and shall continue until all reported attributes are included in the response.

### 9.4.65	Outstanding Alarm Sequence

| Attribute Identifier | 1 |
| --- | --- |
| Pending Reports | 2 |

The integer coded Pending Reports field indicates the number of pending Failure Event Report messages to follow the current message as a response to the associated Report Outstanding Alarms message. This value being 0 signals that it is the last message for the outstanding alarms.

### 9.4.66	HW Conf Change Info

| Attribute Identifier | 1 |
| --- | --- |
| Length | 2-3 |
| HW Description Position Index 1 | 4 |
| Operation Code 1 | 5 |
| Modification Data 1 (only if needed) |  |
| ...... |  |
| HW Description Position Index K |  |
| Operation Code K |  |
| Modification Data K (only if needed) | N |

HW Description Position Index, Operation Code, and Modification Data constitute an associated tripplet for each HW Description change operation. The triplet may be repeated as many as needed. All the changes indicated in one Change HW Configuration message must be successfully conducted or the procedure must fail otherwise. The HW Description Position Index, an octet coded integer, indicates the HW Description’s position in the HW Configuration attribute to which the change operation indicated by the associated Operation Code must be applied. In case of multiple entries, all the position indices shall be interpreted against the initial HW Configuration sequence. There are three possible operations as listed below:

deletion	00

insertion	01

modification	02

By a deletion operation, the indexed HW Description is deleted from the HW Configuration attribute. A deletion operation requires no associated Modification Data. An insertion or modification operation requires an associated Modification Data which specifies the new data for the operation. By a modification operation, the indexed HW Description in the HW Configuration attribute is replaced with the provided Modification Data. Deletion and modification operations are mutually exclusive at one position. By an insertion operation, the provided Modification Data is inserted immediately after the indexed HW Description in the HW Configuration attribute. An insertion at a position indexed zero (0) is interpreted as prepending to the very beginning. More than one insertion operation at one identically indexed position shall produce a HW Configuration sequence in which the inserted Descriptions are ordered the way as they appear in the HW Conf Change Info field.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG GERAN# | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2002-04 | 9 | GP-020564 |  |  | Provision of TS 12.21 release 97 version | 5.0.0 | 6.0.0 |
| 2002-04 | 9 | GP-020564 |  |  | Provision of TS 12.21 release 98 version | 6.0.0 | 7.0.0 |
| 2002-04 | 9 | GP-020564 |  |  | Provision of TS 12.21 release 99 version | 7.0.0 | 8.0.0 |
| 2002-04 | 9 | GP-020564 |  |  | Provision of TS 52.021 release 4 version | 8.0.0 | 4.0.0 |
| 2002-06 | 10 |  |  |  | Provision of TS 52.021 release 5 version | 4.0.0 | 5.0.0 |
| 2005-01 | 23 |  |  |  | Provision of TS 52.021 release 6 version | 5.0.0 | 6.0.0 |
| 2007-08 | 35 |  |  |  | Provision of TS 52.021 release 7 version | 6.0.0 | 7.0.0 |
| 2008-12 | 40 |  |  |  | Provision of TS 52.021 release 8 version | 7.0.0 | 8.0.0 |
| 2009-12 | 44 |  |  |  | Provision of TS 52.021 release 9 version | 8.0.0 | 9.0.0 |
| 2011-03 | 49 |  |  |  | Provision of TS 52.021 release 10 version | 9.0.0 | 10.0.0 |
| 2012-09 | 55 |  |  |  | Provision of TS 52.021 release 11 version | 10.0.0 | 11.0.0 |
| 2014-09 | 63 |  |  |  | Version for Release 12 (frozen at SP-65) | 11.0.0 | 12.0.0 |
| 2015-12 | 68 |  |  |  | Version for Release 13 (frozen at SP-70) | 12.0.0 | 13.0.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-03 | 75 |  |  |  |  | Version for Release 14 (frozen at TSG-75) | 14.0.0 |