3GPP TS 22.289 V0.2.0 (2018-05)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Mobile Communication System for Railways;

Stage 1

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword, …]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	6

4	Examples for Styles	6

4.1	Heading Styles	6

4.2	Other common styles	6

"TSG <Name>" on the front page	7

Page setup parameters	7

Proforma copyright release text block	9

Annex <A> (normative): <Normative annex title>	10

Annex <B> (informative): <Informative annex title>	10

B.1	Heading levels in an annex	10

Annex <X> (informative): Change history	12

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

This clause is optional. If it exists, it is always the second unnumbered clause.

# 1	Scope

The present document provides requirements for railway communication. Railway communication is based on the set of Mission Critical specification [1], [2], [3], [4]. Requirements for railway communication not fitting the scope of those specifications are provided in this TS.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe)".

[2]	3GPP TS 22.179: "Mission Critical Push to Talk (MCPTT); Stage 1".

[3]	3GPP TS 22.281: "Mission Critical Video services ".

[4]	3GPP TS 22.282: "Mission Critical Data services ".

[5]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

# 3	Definitions, symbols and abbreviations

Delete from the above heading those words which are not applicable.

Clause numbering depends on applicability and should be renumbered accordingly.

## 3.1	Definitions

CCTV: Closed-circuit television, also known as video surveillance, is the use of video cameras for monitoring and recording people, animals and objects in different places, such as in vehicles and stations.

CCTV offload: Transfer of video surveillance data archives from train to ground through the wireless connection between mobile communication unit in the train and ground communication units located at the depot and at the stations and/or stops.

Inter-carriage link: Wireless connection between the carriages of the train.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

CCTV 	Closed-circuit television

# 4	Railway communication functionality

## 4.1	Bulk Transfer of CCTV archives from Train to Ground

### 4.1.1	Description

The video surveillance system in trains consists of multiple cameras which are recording and encoding the video feeds 24/7 either into the video recorders or into internal memory of the cameras themselves. The amount of video recordings is excessive, which exceeds rather fast the physical storage capacity that is available in video recorders and/or in cameras. The regulations in different countries are ruling retention time for recordings until up to 31 days or even more. However, the recordings need to be stored in the physical storages located in the vehicles for only seven days.

Next, an example estimation is provided to illustrate the offload performance with different uplink speeds, when the train is commuting from Helsinki to Kemijärvi. The parameters for the calculation are provided in Table 1, whereas the offload results for uplink speeds of 400 Mbps, 750 Mbps and 1000 Mbps are provided respectively in Figure 1.1.1-2, Figure 1.1.1-3 and in Figure 1.1.1-4.

Station	Arrives	Departs	Stop time	Time between stations

HELSINKI		18:52

PASILA	18:57	19:00	0:03	0:08

TIKKURILA	19:41	19:44	0:03	0:44

RIIHIMÄKI	20:19	20:22	0:03	0:38

HÄMEENLINNA	20:46	20:48	0:02	0:26

TAMPERE	21:38	22:11	0:33	1:23

PARKANO	23:03	23:06	0:03	0:55

SEINÄJOKI	0:08	0:10	0:02	1:04

KOKKOLA	1:37	1:39	0:02	1:29

YLIVIESKA	3:10	3:12	0:02	1:33

OULU	4:42	5:00	0:18	1:48

KEMI	6:08	6:12	0:04	1:12

ROVANIEMI	7:47	7:55	0:08	1:43

MISI	8:31	8:32	0:01	0:37

KEMIJÄRVI	9:00	9:05	0:05	0:33

Table 4.1.1-1. The parameters for the offload calculation for the train route between Helsinki and Kemijärvi.

Figure 4.1.1-2. The offload results with uplink speed of 450 Mbps.

Figure 4.1.1-3. The offload results with uplink speed of 750 Mbps.

Figure 4.1.1-4. The offload results with uplink speed of 1000 Mbps.

Based on the example estimations presented in the figures 4.1.1-1 – 4.1.1-4, the required throughput speed for the offload should be at least 1 Gbps.

Hence, in order to meet the requirement regarding the retention time and to minimize the required physical storage in the train, FRMCS needs to provide the means to transfer the CCTV archives from train to ground, i.e. enable means for CCTV offload.

In a CCTV offload system, FRMCS provides means for transferring video surveillance data between a mobile communication unit in the train and ground communication units located at the depot and at the stations and/or stops alongside the predetermined route of the train. Whenever the train approaches the stations and/or stops or arrives into the depot, FRMCS facilitates the communication between the ground and mobile communication unit. The mobile communication unit in the train forwards the video surveillance data from the video recorder and/or directly from video surveillance cameras.   The generic offload procedure between the ground and mobile communication units could be e.g as described in the following;

-	The ground communication unit is monitoring whether a transmission signal from the mobile communication unit is available and that the signal quality is sufficient for synchronization.

-	Upon successful synchronization, the ground communication unit requests offload from the mobile communication unit and the connection between the ground communication and mobile communication unit is established as soon as the sufficient signal quality is acknowledged by the FRMCS and mobile communication unit.

-	The transfer of CCTV archives from train to ground is started.

-	The ground communication system forwards the offloaded data further to the ground storage.

-	The transfer of CCTV archives continues as long as sufficient connection between the train and ground system is available and/or aborted by the FRMCS.

### 4.1.2	Requirements

[R4.1.2-1] FRMCS shall facilitate the CCTV offload from train to ground when, during the train stops the stations and/or stops, stations, and whenever the train arrives into the stops and at the depot.

[R4.1.2-2] The FRMCS shall be able to support that CCTV archives can be transferred into the ground system in a time and resource efficient way in dedicated places such as stations, train stops or train depots.

[R4.1.2-3] The CCTV offload shall be initiated by the ground communication unit, once the sufficient signal quality is available between the ground and mobile communication units.

[R4.1.2-4] The transfer of CCTV archives shall not affect mission critical communication.

## 4.2	Bulk transfer of multimedia from ground to train

### 4.2.1	Description

The offering of multimedia services to the passengers is becoming default service during long travels in airplanes and more and more, also in trains. In order to minimize the excessive consumption of network capacity between train and ground, a bulk transfer of multimedia databases from ground to train during stops at the stations and depots is optimal solution. The bulk transfer of multimedia is done when the train stops at the stations, stops and depot, ideally with minimum of one gigabit transfer speed.  Hence, the multimedia content can be more versatile and updated in regular basis. The actual content is also consumed in-train network operated by the FRMCS and hence it does not give burden to the link budget between train and ground. The multimedia may contain movies, TV shows, cached webpages etc.

The bulk transfer of multimedia is facilitated by the FRMCS, which provides means for transferring the multimedia databases data between ground communication units, located at the depot, stations and/or stops alongside the predetermined route of the train. Whenever the train approaches the stations and/or stops or arrives into the depot, FRMCS facilitates the communication between the ground and mobile communication unit, if bulk transfer of multimedia transfer is requested. The ground communication unit uploads the multimedia databases from ground into the mobile communication unit in the train, where the data is stored in the train multimedia data storage.   The generic procedure for the bulk transfer of multimedia between the ground and mobile communication units could be e.g as described in the following;

-	The ground communication unit is monitoring whether a transmission signal from the mobile communication unit is available and that the signal quality is sufficient for synchronization.

-	Upon successful synchronization, the mobile communication unit requests multimedia upload from the mobile communication unit and the connection between the ground communication and mobile communication unit is established as soon as the sufficient signal quality is acknowledged by the FRMCS and mobile communication unit.

-	The transfer of multimedia databases from ground to train is started.

-	The mobile communication system forwards the transferred multimedia databases further into the in-train storage.

-	The transfer of multimedia archives continues as long as sufficient connection between the train and ground system is available and/or aborted by the FRMCS.

### 4.2.2	Requirements

[R4.2.2-1] FRMCS shall facilitate the transfer of multimedia archives from ground to train.

[R4.2.2-2] The FRMCS shall be able to support that multimedia databases can be transferred from ground to train in a time and resource efficient way, when the train stops at the stations, train stops and at the depot.

[R4.2.2-3] FRMCS shall facilitate communication capabilities provided by the train.

[R4.2.2-4] The transfer of multimedia databases shall not impact mission critical communication.

## 4.3	Massive Inter-carriage data transfer

### 4.3.1	Description

The inter-carriage links between train vehicles enable sufficient capacity to enable massive data transfer throughout the train required e.g for transfer of CCTV archives, multimedia databases and live streaming. Mobile communication unit in train provides connection between carriages of the train dedicated for the transfer of CCTV archives to a central node in the train form which the connection to the ground system will take place.

### 4.3.2	Requirements

[R4.3.2-1] The FRMCS shall facilitate the onboard CCTV communication between carriages of a train, to collect CCTV content at one place on the train for transfer to the ground system.

[R4.3.2-2] The Inter-carriage links shall support at least the same throughput speed as the mobile communication unit of FRMCS providing the link between train and ground system.

[R4.3.2-3] The onboard communication between carriages of a train shall not impact mission critical communication.

# Annex <A> (normative):
<Normative annex title>

Annexes are only to be used where appropriate:

Annex <B> (informative):
<Informative annex title>

Annexes are labelled A, B, C, etc. and designated either "normative" or "informative" depending on their content (informative annexes do not comprise requirements for the implementation of the specification).

# B.1	Heading levels in an annex

Heading levels within an annex are used as in the main document, but for Heading level selection, the "A.", "B.", etc. are ignored. e.g. B.1.2 is formatted using Heading 2 style.

Bibliography

The Bibliography is optional. If it exists, it shall follow the last annex in the document.

The following material, though not specifically referenced in the body of the present document (or not publicly available), gives supporting information.

Bibliography format

-	<Publication>: "<Title>".

OR

<Publication>: "<Title>".

Annex <X> (informative):
Change history

This is the last annex for TSs which details the change history using the following table.

This table can be used for recording progress during the WG drafting process till TSG approval of this TR.

For TRs under change control, use one line per approved Change Request

Date: use format YYYY-MM

TSG # : use format RAN#55

CR: four digits, leading zeros as necessary

Rev: blank, or number (max two digits)

Cat: use one of the letters A, B, C, D, F

Subject/Comment: for TSs under change control, include full text of the subject field of the Change Request cover

New vers: use format n[n].n[n].n[n]

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |