3GPP TR 23.715 V0.2.0 (2018-02)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Study on System enhancements for Provision of Access to

Restricted Local Operator Services by Unauthenticated UEs

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

3GPP, access, authentication, enhancements, services

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2018, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	6

4	Architectural Assumptions and Requirements	6

4.1	Architectural Assumptions	6

4.2	Architectural Requirements	6

5	Key Issues for EPC	6

5.1	Key Issue #EPC-1: Network indicating support for Restricted Local Operator Services and related UE behaviour	7

5.2	Key Issue #EPC-2: RLOS request indication	7

5.3	Key Issue #EPC-3: Support of unauthenticated UEs access to RLOS	7

5.4	Key Issue #EPC-4: Support of authenticated UEs access to RLOS	7

5.5	Void	7

5.6	Key Issue #EPC-6: Collection of charging information	7

5.7	Key Issue #EPC-7: Level of security	8

6	Key Issues for IMS	8

6.1	Key issue #IMS-1: Support for unauthenticated and authenticated user	8

6.2	Key issue #IMS-2: Identification of Restricted Local Operator Services at IMS layer	8

6.3	Key issue #IMS-3: Handling IMS session for Restricted Local Operator Service	8

7	Solutions	8

7.X	Solution #X: <Solution Title>	9

7.X.1	Functional Description	9

7.X.2	Procedures	9

7.X.3	Impact on existing entities and interfaces	9

8	Evaluation	9

9	Conclusions	9

Annex <A>: Change history	10

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The objective of this Technical Report is to identify and evaluate potential architecture enhancements of EPS and IMS needed to support Provision of Access to Restricted Local Operator Services by Unauthenticated UEs as identified in TS 22.101 [x], TS 22.115 [x] and TS 22.228 [x], and determine which of the solutions can proceed to normative specifications.

This feature is only applicable to EPS 3GPP access. Access to local operator services does not affect the UICC.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.101: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Service aspects; Service principles".

[3]	3GPP TS 22.115: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Service aspects; Charging and billing".

[4]	3GPP TS 22.228: "3rd Generation Partnership Project; Service requirements for the IP multimedia core network subsystem".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Restricted local operator services: communication services provided by an operator that involve either automated or human assistance (e.g. credit card billing, directory assistance, customer care) for which successful authentication is not necessary.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

RLOS	Restricted Local Operator Services

# 4	Architectural Assumptions and Requirements

## 4.1	Architectural Assumptions

The goal of the study is to enable access to those Restricted Local Operator Services (RLOS), however the definition of such restricted local operator services offered by an operator is out of scope of 3GPP.

Architectural assumptions are the following:

-	Access to RLOS is only possible for UEs when using EPC via E-UTRAN as IPCAN.

-	Both unauthenticated and authenticated UEs can access RLOS via the same architecture.

An authenticated UE allowed to access EPC may access the same services that are provided as RLOS but in this case, it is not within the scope of RLOS.

-	The UE shall indicate to the EPC and the IMS network that the request is a request for RLOS.

-	Allowing access to RLOS is completely under the local operator's control.

-	The solution shall support both non-IMS and IMS RLOS services.

-	When RLOS are accessed via IMS sessions:

1)	they do not require any specific support for location over and above what is defined by IMS already;

2)	they do not require any specific support regarding call back to the user that has initiated the session;

3)	the IMS RLOS are securely isolated to avoid e.g. DOS attacks to IMS entities offering regular IMS services.

-	Only UE-originated RLOS requests are supported.

-	No support of multiple PDN connections.

-	No support of mobile terminated services.

-	This feature is only applicable to EPS 3GPP access.

-	Inter-RAT handovers and handover between 3GPP and non-3GPP accesses are not supported.

## 4.2	Architectural Requirements

Editor's note:	This clause will define the architectural requirements based on the normative stage-1 requirements defined in TS 22.101, TS 22.115 and TS 22.228 with regards to Provision of Access to Restricted Local Operator Services by Unauthenticated UEs.

# 5	Key Issues for EPC

Editor's note:	This clause will describe the key issues for EPC.

## 5.1	Key Issue #EPC-1: Network indicating support for Restricted Local Operator Services and related UE behaviour

TS 22.101 [2] specifies: "When a UE recognizes an origination attempt to a restricted local operator service and has not received an indication from the serving system that restricted local operator services are available, the UE shall block the origination attempt."

This key issue addresses:

-	how the PLMN announces its support of RLOS to all UEs;

-	the behaviour of a supporting UE when it detects the network support of RLOS;

-	the behaviour of a supporting UE when it does not detect the network support of RLOS.

NOTE:	This key issue does not address the mechanisms for rejecting UE requests in case of an unauthorized attempt to access RLOS.

## 5.2	Key Issue #EPC-2: RLOS request indication

The solution shall address how unauthenticated and authenticated UEs indicate to the EPC that a request is for RLOS (e.g. at Attach);

## 5.3	Key Issue #EPC-3: Support of unauthenticated UEs access to RLOS

The solution shall address the EPC mechanisms required to support unauthenticated UEs. In particular, the following aspects are required to be studied:

-	how to allow unauthenticated UEs to access EPC network for RLOS only if the EPC network supports RLOS;

-	how to prevent unauthenticated UEs to access services which are not RLOS;

## 5.4	Key Issue #EPC-4: Support of authenticated UEs access to RLOS

The solution shall address the EPC mechanisms required to support authenticated UEs, in particular:

-	how to allow authenticated UEs to access the same RLOS as unauthenticated UEs. This applies to UEs that the MME has successfully authenticated but the UE is not able to receive normal service (e.g. is in a forbidden area).

NOTE:	Regularly attached UEs able to receive normal service may access the same services that are provided as RLOS but in this case, it is not within the scope of RLOS.

## 5.5	Void

## 5.6	Key Issue #EPC-6: Collection of charging information

The solution shall address how to collect charging information regarding the use of RLOS.

## 5.7	Key Issue #EPC-7: Level of security

The solution shall have a level of security, which should not be less than that which is currently applied to existing equivalent network access methods (e.g., unauthenticated emergency calling).

# 6	Key Issues for IMS

Editor's note:	This clause will describe the key issues for IMS.

## 6.1	Key issue #IMS-1: Support for unauthenticated and authenticated user

The key issue here is that the network enables any UE access to a requested service through RLOS. This applies to all UEs, authenticated and non-authenticated.

The solution should use the same mechanisms for both unauthenticated and authenticated UEs as much as possible.

## 6.2	Key issue #IMS-2: Identification of Restricted Local Operator Services at IMS layer

The key issue here is that it is beneficial if IMS services allow clear service identification. For access to restricted local operator service this means that:

-	the UE need to be able to determine the case and start session setup without requiring IMS registration for RLOS services. Further the UE will need to indicate to the IMS network that its request is for RLOS via appropriate information in the SIP request; and

-	the core network functional entities need to be able to detect the specific information for an RLOS request.

The study will determine what to configure in the IMS network (e.g., phone number, captive portal), in which IMS entity(ies), and what to signal to the UE (e.g. specific CSCF address);

## 6.3	Key issue #IMS-3: Handling IMS session for Restricted Local Operator Service

At IMS layer, RLOS are always provided in the local PLMN without any involvement of the Home PLMN.

The key issue for the UE is to support initiating a session setup for RLOS without requiring IMS registration for RLOS services.

NOTE:	UE only initiates IMS sessions for RLOS if the network has indicated support via signalling in the underlying layers.

The key issue for the IMS core network functional entities is to support session setup for RLOS from users without requiring IMS registration for RLOS services. Sessions for RLOS from authenticated and unauthenticated users are handled in the PLMN that provides the entry point into the IMS. This means that the IMS core network functional entities of the PLMN that provides the P-CSCF to which the UE is attached will handle the IMS session request locally.

# 7	Solutions

Editor's note:	This clause is intended to document the agreed architecture solutions. Each solution should clearly describe which of the key issues it covers and how.

## 7.X	Solution #X: <Solution Title>

### 7.X.1	Functional Description

Editor's note:	This clause will describe the general description, assumption, and principles of the solution. The related key issues it solves will be mentioned.

### 7.X.2	Procedures

Editor's note:	This clause will describe the high-level procedures and information flows for the solution.

### 7.X.3	Impact on existing entities and interfaces

Editor's note:	This clause will describe the impacts to existing nodes or functionality and interfaces.

# 8	Evaluation

Editor's note:	This clause will provide a general evaluation of the solutions.

# 9	Conclusions

Editor's note:	This clause will capture agreed conclusions from the study.

Annex <A>:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-08 | SA2#122BIS | - | - | - | - | TR skeleton (approved in S2-175500) | 0.1.0 |
| 2017-08 | SA2#122BIS | S2-175501 |  |  |  | Scope of TR 23.715 | 0.1.0 |
| 2017-08 | SA2#122BIS | S2-176256 |  |  |  | Assumptions for PARLOS | 0.1.0 |
| 2017-08 | SA2#122BIS | S2-176210 |  |  |  | Key issue on mechanisms to access RLOS | 0.1.0 |
| 2017-08 | SA2#122BIS | S2-176258 |  |  |  | Key issue on RLOS support indication and related UE behaviour | 0.1.0 |
| 2017-08 | SA2#122BIS | S2-176259 |  |  |  | IMS Key Issues for RLOS | 0.1.0 |
| 2017-09 | - | - | - | - | - | MCC Editorial corrections | 0.1.1 |
| 2018-02 | SA2#125 | S2-181001 |  |  |  | Update of RLOS architectural assumptions | 0.2.0 |
| 2018-02 | SA2#125 | S2-181002 |  |  |  | Update to Key issue IMS#1 | 0.2.0 |
| 2018-02 | SA2#125 | S2-181003 |  |  |  | Update to Key issue EPC#3 | 0.2.0 |
| 2018-02 | SA2#125 | S2-180086 |  |  |  | Removal of Key issue EPC#5 | 0.2.0 |
| 2018-02 | SA2#125 | S2-181044 |  |  |  | Update of RLOS key issues | 0.2.0 |