3GPP TSG SA4 meeting #25bis	S4-030248

Berlin, Germany, 24. –28.Feb  2003	 Agenda Item: SES

Title:	Design Constraints for default codec for speech enabled services (SES)

# Source:	SQ-SWG

Contact:	Bernhard Noé , Bernhard.Noe@alcatel.de

Version:	1.4

Summary:

This document describes the design constraints for the codecs for speech enabled services

# Sampling Rates

Sampling rates of 8 & 16kHz will be supported.

# Complexity

The terminal side processing of the codec has to be able to be implemented within the resources of a typical mobile phone terminal.  Accordingly the maximum complexity requirements for terminal side codec have been defined as shown in tables below. Table 1 shows complexity requirements for codec supporting 8kHz sampling rate and table 2 shows numbers for codec supporting 16 kHz sampling rate.

| Measure | Requirement |
| --- | --- |
| WMOPS | Less than 25 |
| ROM size | Less than 20 kwords |
| RAM size | Less than 7 kwords |

Table 1: complexity and memory requirements for codec supporting 8 kHz sampling rate

| Measure | Requirement |
| --- | --- |
| WMOPS | Less than 39 |
| ROM size | Less than 34 kwords |
| RAM size | Less than 8 kwords |

Table 2: complexity and memory requirements for codec supporting 16 kHz sampling rate The definition of the wMOPS measure and recommendations on how to estimate the computation and memory requirements can be found in ETSI Technical document [2]. A word is defined as 16bits. These complexity measures are for the front-end feature extraction and compression and the VAD.

ROM does not include program ROM.

# Latency

The maximum codec latency requirement is 200ms, with the objective of 50 ms. This values contains the algorthmic delay introduced by the codec.

# Data rate for the source codec

#### Voice enabled services need to be able to operate over a variety of channels.
The following channels and datarates will at least be supported

#### a) For conversational class of service [4]:

- The GPRS single slot uplink (Coding scheme CS-1) channel. 
Here the maximum source data rate is 5.6 kbit/sec.

- The EGPRS single slot uplink (Coding scheme MCS -1) channel.
Here the maximum source data rate is 6.4 kbit/sec.

- The Flexible Layer 1 (FLO) channel. Here the maximum data rate is expected to be  between 6.4 and 8.4 kbit/sec.

- For UTRAN packet data channel the maximum source datarate is 24 kbit/sec.

It is assumed one 20ms frame within one RLC/MAC block.

b) For streaming and interactive class of service_

- For GPRS / EGPRS single slot uplink channel the maximum source datarate is 8 kbit/sec (assuming 10 frames per IP packet) or 7.5 kbit/sec (assuming 5 frames per IP packet) .

- For UTRAN packet data channel the maximum source datarate is 24 kbit/sec.

I

- ETSI SMG11 Tdoc SMG11 117/99, “Complexity verification report of the AMR codec, v2.0”, Alcatel,  Philips, ST Microelectronics, Texas Instruments".

- ETSI SMG11 AMR-9 "AMR permanent document (AMR-9) Complexity and delay assessment v1.0", 23rd March 1998

- IETF RFC 3095: "RObust Header Compression (ROHC): Framework and four profiles: RTP, UDP, ESP, and uncompressed".

- S4-030114.doc , TSG SA WG4 , Berlin, Germany, 24. –28.Feb  2003