3GPP TS 28.307 V0.1.0 (2017-10)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Quality of Experience (QoE) measurement collection

Integration Reference Point (IRP);

Requirements

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

MCC selects keywords from stock list.

Keywords

<keyword[, keyword, …]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	6

3.1	Definitions	6

3.3	Abbreviations	6

4	Concepts and background	7

5	Business Level Requirements	7

6	Specification level requirements	7

6.1	Requirements	7

6.2	Actor roles	7

6.3	Telecommunication resources	7

6.4	Use cases	7

6.4.1	Activate collection measurement job for an area in UTRAN	7

6.4.2	Activate collection measurement job for an individual UE in UTRAN	8

6.4.3	De-activate collection measurement job in UTRAN	9

Annex A (informative): Change history	11

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management, as identified below:

TS 28.307: Management of Quality of Experience (QoE) measurement collection Integration Reference Point (IRP); Requirements

TS 28.308: Management of Quality of Experience (QoE) measurement collection Integration Reference Point (IRP); Information Service (IS)

TS 28.309: Management of Quality of Experience (QoE) measurement collection Integration Reference Point (IRP); Solution Set (SS) definitions

One main motivation of mobile network evolution is to improve the user experience why the evaluation at the UE side is vital to network operators, especially when the operators provide some real-time services which require for example high date rate and low latency like streaming services (typically video services).

Quality of Experience (QoE) information collection provides detailed information at session level on one or more specific UE(s).

The operator can initiate logging of QoE information of an end user service. The collected information (specified in Ref. 3GPP TS 26.247 [2]) cannot be deduced from performance measurements in the mobile network.

The QoE information is information collected by the end user application in the UE.

The QoE information is collected by the management system (e.g. an Operations System (OS) in TMN terminology) for analysis and/or KPI calculations.

# 1	Scope

The present document addresses Integration Reference Point (IRP) requirements for the function Quality of Experience (QoE) measurement collection in UMTS and LTE.

The function includes collecting QoE information from UEs frequenting a specified area or an individual UE for a specified end user service/end user service type.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[3]	3GPP TS 28.404: "Quality of Experience (QoE) measurement collection; Concepts, use cases and requirements ".

[4]	3GPP TS 25.331: "RRC Protocol Specification".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

NRS	Network Request Session. See ref. 3GPP TS 28.404 [3]

RECS	RECording Session. See ref. 3GPP TS 28.404 [3]

UERS	UE Request Session. See ref. 3GPP TS 28.404 [3]

# 4	Concepts and background

The collection of QoE information for a specified end user service/end user service type either from UEs in a specified area or a specific UE. The collected informationis transported to a collection centre, where it can be analysed and/or KPIs can be calculated.

A collection can be requested by an operator technician via the management system to the traffic network. As the network do not have any knowledge which UEs have the capability to record the requested data, therefore the UEs will report whether they have this capability or not when a session set up. UEs that has this capability that match the request from the management system will be requested to start recording the requested information when the request constraints are met. The UE will make the recorded data available for management system.

# 5	Business Level Requirements

See business level requirements in Ref. 3GPP TS 28.404 [3].

# 6	Specification level requirements

## 6.1	Requirements

REQ-EUSPC-FUN-1: The IRPManager shall have a capability to request collection of QoE information per end user service/end user service type for a specified area.

REQ-EUSPC-FUN-2: The IRPManager shall have a capability to request collection of QoE information for a specific UE.

REQ-EUSPC-FUN-3: The IRPManager shall have a capability to request the collection of QoE information to be stopped before the time for the NRS has expired.

REQ-EUSPC-FUN-4: The IRPAgent shall have a capability to notify the IRPManager when a collection of QoE information has been stopped before the time for the NRS has expired.

## 6.2	Actor roles

Please see respective use case.

## 6.3	Telecommunication resources

Please see respective use case.

## 6.4	Use cases

### 6.4.1	Activate collection measurement job for an area in UTRAN

| Activate collection measurement job for an area |  |  |
| --- | --- | --- |
| Use case stage | Evolution/Specification | <<Uses>> Related use |
| Goal | To start collecting QoE information for an end user service type in a specified area. |  |
| Actors and roles | IRPManager: Request the start of collection of QoE information. |  |
| Telecom resources | The RNC and the UE |  |
| Assumptions | There are UEs that has the capability to provide the requested information in the specified area. |  |
| Pre-conditions | None. |  |
| Begins when | The IRPManager requests that a NRS shall be started. |  |
| Step 1(M) | An IRPAgent receives the request and forwards it to the appropriate RNC(s). |  |
| Step 2 (M) | The RNC starts a NRS for the specified area and start checking for UEs that supports QoE information collection. |  |
| Step 3 (M) | For UEs that support QoE information collection, the RNC starts a UERS and forwards the request of collection QoE information to the UE. See ref. 3GPP 25.331 [4]. |  |
| Step 4 (M) | When the requested end user service is started in the UE, it starts a RECS and records the requested information. |  |
| Step 5 (M) | The UE reports the recorded information to a collection centre via the RNC as long as the end user service session is active during the time for the NRS. |  |
| Step 6 (M) | When the end user service session ends the UE stops the RECS and reports the collected data to a collection centre via the RNC. |  |
| Ends when | When the time for the NRS elapses or when the IRPManager sends a request for de-activation. |  |
| Exceptions |  |  |
| Post-conditions | None. |  |
| Traceability | REQ-EUSPC-FUN-1. |  |
| NOTE: Steps 4, 5 and 6 are repeated in a NRS for every time a session that starts the requested end user service in the UE. |  |  |

NOTE: A similar use case is valid for LTE and possibly other 3GPP systems.

### 6.4.2	Activate collection measurement job for an individual UE in UTRAN

| Activate collection measurement job for an individual UE |  |  |
| --- | --- | --- |
| Use case stage | Evolution/Specification | <<Uses>> Related use |
| Goal | To start collecting QoE information for an end user service type for an individual UE. |  |
| Actors and roles | IRPManager: Request the start of collection of QoE information. |  |
| Telecom resources | The base station, the HSS, the SGSN and the UE |  |
| Assumptions | None. |  |
| Pre-conditions | None. |  |
| Begins when | The IRPManager requests that a NRS shall be started. |  |
| Step 1(M) | An IRPAgent receives the request and forwards it to the HSS. |  |
| Step 2 (M) | The HSS forwards the request to the SGSN, which is serving the UE. |  |
| Step 3 (M) | When the UE becomes active, the SGSN forwards the request of collection QoE information to the RNC node. |  |
| Step 4 (M) | The RNC node starts the NRS and the UERS and forwards the request of collection QoE information to the UE. See ref. 3GPP 25.331 [4]. |  |
| Step 5 (M) | When the requested end user service is started in the UE, it starts a RECS and starts to record the requested information. |  |
| Step 6 (M) | The UE reports the recorded information to a collection centre via the RNC as long as the end user service session is active during the time for the NRS. See ref. 3GPP 25.331 [4]. |  |
| Step 7 (M) | When the end user service session ends the UE stops the RECS and reports the collected data to a collection centre, via the RNC. |  |
| Ends when | When the time for the NRS elapses or when the IRPManager sends a request for de-activation. |  |
| Exceptions | The UE does not support the collection of the QoE information. |  |
| Post-conditions | None. |  |
| Traceability | REQ-EUSPC-FUN-2. |  |
| NOTE: Steps 5, 6 and 7 are repeated in a NRS for every time a session that starts the requested end user service in the UE. |  |  |

NOTE: A similar use case is valid for LTE and possibly other 3GPP systems.

### 6.4.3	De-activate collection measurement job in UTRAN

| De-activate collection measurement job |  |  |
| --- | --- | --- |
| Use case stage | Evolution/Specification | <<Uses>> Related use |
| Goal | To stop collecting more QoE information for an NRS before the time for the NRS has expired. |  |
| Actors and roles | IRPManager: Request to stop the collection of QoE information. |  |
| Telecom resources | The RAN node, HSS and the UE |  |
| Assumptions | None. |  |
| Pre-conditions | A NRS is started. |  |
| Begins when | The IRPManager requests that a NRS shall be stopped. |  |
| Step 1 (M) | An IRPAgent receives the request and forwards it to the RNC or the HSS. |  |
| Step 2 (O) | The HSS transfers the request to the RNC via the SGSN. |  |
| Step 3 (M) | The RNC node stops the NRS and informs the UE to not start new RECS. |  |
| Step 4 (O) | The RNC node informs HSS, via SGSN, that the NRS is stopped. |  |
| Step 5 (O) | The HSS stops the NRS. |  |
| Ends when | When the IRP Manager receives the notification that the NRS is stopped. |  |
| Exceptions | None. |  |
| Post-conditions | None. |  |
| Traceability | REQ-EUSPC-FUN-3 and REQ-EUSPC-FUN-4. |  |

NOTE: A similar use case is valid for LTE and possibly other 3GPP systems.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-10 | SA5#115 | S5-175398 |  |  |  | R15 pCR 28.307-000 Introduction and Scope for QoE IRP requirements | 0.1.0 |
| 2017-10 | SA5#115 | S5-175399 |  |  |  | R15 pCR 28.307-000 Introduction of specification level use cases and requirements for QoE IRP requirements | 0.1.0 |