3GPP TS 32.273 V15.0.0 (2018-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Charging management;

Multimedia Broadcast and Multicast Service (MBMS) charging

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

GSM, UMTS, LTE, charging, management, MBMS

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2018, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	7

3	Definitions, symbols and abbreviations	8

3.1	Definitions	8

3.2	Symbols	10

3.3	Abbreviations	10

4	Architecture considerations	12

4.1	High level MBMS architecture	12

4.2	MBMS offline charging architecture	12

4.3	MBMS online charging architecture	12

5	MBMS charging principles and scenarios	13

5.1	MBMS charging principles	13

5.1.1	General principles for GPRS	13

5.1.1A	General principles for EPS	14

5.1.2	Triggers for generation of charging information	15

5.2	MBMS offline charging scenarios	16

5.2.1	Basic principles	16

5.2.2	Rf message flows	18

5.2.2.1	Broadcast service	18

5.2.2.1.1	User service charging	18

5.2.2.1.2	Session Start for GPRS	18

5.2.2.1.2A	Session Start for EPS	19

5.2.2.1.2B	Void	19

5.2.2.1.3	Session Stop for GPRS	20

5.2.2.1.3A	Session Stop for E-UTRAN	21

5.2.2.1.3B	Void	21

5.2.2.1.4	BM-SC initiated Registration and Deregistration	21

5.2.2.1.5	Session Update for EPS with E-UTRAN and UTRAN	22

5.2.2.2	Multicast Service	23

5.2.2.2.1	Session Start	23

5.2.2.2.2	Session Stop	24

5.2.2.2.3	BM-SC initiated MBMS Deregistration	25

5.2.2.2.4	UE Activation	26

5.2.2.2.5	UE Deactivation	27

5.2.3	CDR generation	28

5.2.3.1	CDRs related to MBMS subscribers	28

5.2.3.1.1	Triggers for S-BMSC-CDR charging information collection	28

5.2.3.1.2	Triggers for S-BMSC-CDR charging information addition	28

5.2.3.1.3	Triggers for S-BMSC-CDR closure	28

5.2.3.2	CDRs related to content provider	29

5.2.3.2.1	Triggers for BMSC-CDR charging information collection	29

5.2.3.2.2	Triggers for C-BMSC-CDR charging information addition	29

5.2.3.2.3	Triggers for C-BMSC-CDR closure	29

5.2.4	Ga record transfer flows	30

5.2.5	Bmb CDR file transfer	30

5.3	MBMS online charging scenarios	30

5.3.1	Basic principles	30

5.3.2	Ro message flows	30

5.3.2.1	Broadcast Service	30

5.3.2.1.1	User service charging	30

5.3.2.1.2	Session Start	30

5.3.2.1.3	Session Stop	30

5.3.2.1.4	BM-SC initiated Registration and Deregistration	30

5.3.2.2	Multicast Service	31

5.3.2.2.1	Session Start	31

5.3.2.2.2	Session Stop	32

5.3.2.2.3	BM-SC initiated MBMS Deregistration	33

5.3.2.2.4	UE Activation	34

5.3.2.2.5	UE Deactivation	35

5.3.3	Credit-Control related	36

5.3.3.1	Triggers for stopping for an MBMS service Credit-Control session	36

5.3.3.2	Triggers for providing interim information for a MBMS service Credit-Control session	36

6	Definition of charging information	37

6.1	Data description for MBMS offline charging	37

6.1.1	Rf message contents	37

6.1.1.1	Summary of offline charging message formats	37

6.1.1.2	Structure for the Rf message formats	37

6.1.1.2.1	Charging Data Request  message	37

6.1.1.2.2	Charging Data Response message	38

6.1.2	Ga message contents	39

6.1.3	CDR description on the Bmb interface	39

6.1.3.0	Introduction	39

6.1.3.1	CDR description for subscriber charging	39

6.1.3.2	CDR description for content provider charging	40

6.2	Data description for MBMS online charging	41

6.2.1	Ro message contents	41

6.2.1.1	Summary of message formats	41

6.2.1.2	Structure for the  message formats	42

6.2.1.2.1	Debit / Reserve Units Request message	42

6.2.1.2.2	Debit / Reserve Units Response message	43

6.3	MBMS charging specific parameters	44

6.3.1	Definition of the MBMS charging information	44

6.3.1.0	General	44

6.3.1.1	MBMS charging information assignment for Service Information	44

6.3.1.2	Definition of the MBMS Information	44

6.3.2	Formal parameter description	45

6.3.2.1	MBMS charging information for CDRs	45

6.3.2.2	MBMS charging information for charging events	45

6.4	Bindings for MBMS offline charging	45

Annex A (informative):	Bibliography	47

Annex B (informative):	Change history	49

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is part of a series of Technical Specifications (TSs) that specify charging functionality and charging management in GSM/UMTS networks. The GSM/UMTS core network charging architecture and principles are specified in TS 32.240 [1], which provides an umbrella for other charging management TSs that specify:

-	the content of the CDRs per domain / subsystem / service (offline charging);

-	the content of real-time charging messages per domain / subsystem / service (online charging);

-	the functionality of online and offline charging for those domains / subsystems / services;

-	the interfaces that are used in the charging framework to transfer the charging information (i.e. CDRs or charging events).

The complete document structure for these TSs is defined in TS 32.240 [1].

The present document specifies the Offline and Online Charging description for the Multimedia Broadcast and Multicast Service (MBMS), based on the functional stage 2 description in TS 23.246 [200]. 
This charging description includes the offline and online charging architecture and scenarios specific to MBMS, as well as the mapping of the common 3GPP charging architecture specified in TS 32.240 [1] onto MBMS. 
It further specifies the structure and content of the CDRs for offline charging, and the charging events for online charging. The present document is related to other 3GPP charging TSs as follows:

-	The common 3GPP charging architecture is specified in TS 32.240 [1];

-	The parameters, abstract syntax and encoding rules for the CDRs are specified in TS 32.298 [51];

-	A transaction based mechanism for the transfer of CDRs within the network is specified in TS 32.295 [54];

-	The file based mechanism used to transfer the CDRs from the network to the operator’s billing domain (e.g. the billing system or a mediation device) is specified in TS 32.297 [52];

-	The 3GPP Diameter application that is used for MBMS offline and online charging is specified in TS 32.299 [50].

All terms, definitions and abbreviations used in the present document, that are common across 3GPP TSs, are defined in the 3GPP Vocabulary, TR 21.905 [100]. Those that are common across charging management in GSM/UMTS domains or subsystems are provided in the umbrella document TS 32.240 [1] and are copied into clause 3 of the present document for ease of reading. Finally, those items that are specific to the present document are defined exclusively in the present document.

Furthermore, requirements that govern the charging work are specified in TS 22.115 [101].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[2] - [9]	Void.

[10]	3GPP TS 32.250: "Telecommunication management; Charging management; Circuit Switched (CS) domain charging".

[11]	3GPP TS 32.251: "Telecommunication management; Charging management; Packet Switched (PS) domain charging".

[12] - [19]	Void.

[20]	3GPP TS 32.260: "Telecommunication management; Charging management; IP Multimedia Subsystem (IMS) charging".

[21] - [29]	Void.

[30]	3GPP TS 32.270: "Telecommunication management; Charging management; Multimedia Messaging Service (MMS) charging".

[31] - [49]	Void.

[50]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging application".

[51]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

[52]	3GPP TS 32.297: "Telecommunication management; Charging management; Charging Data Record (CDR) file format and transfer".

[53]	Void.

[54]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[55] - [99]	Void.

[100]	3GPP TR 21.905: "Vocabulary for 3GPP specifications".

[101]	3GPP TS 22.115: "Service aspects; Charging and billing".

[102] - [199]	Void.

[200]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[201]	3GPP TS 22.146: "Multimedia Broadcast/Multicast Service (MBMS); Stage 1".

[202]	3GPP TS 22.246: "Multimedia Broadcast/Multicast Service (MBMS) user services; Stage 1".

[203]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[204]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[205]	3GPP TS 23.486: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [100], TS 32.240 [1], TS 23.246 [200] and the following apply:

2G- / 3G-: prefixes 2G- and 3G- refer to functionality that supports only GSM or UMTS, respectively, e.g. 2G-SGSN refers only to the GSM functionality of an SGSN.

accounting: process of apportioning charges between the Home Environment, Serving Network and Subscriber.

Advice of Charge (AoC): real-time display of the network utilization charges incurred by the Mobile Station (MS).
The charges are displayed in the form of charging units. If a unit price is stored by the MS then the display may also include the equivalent charge in the home currency.

AoC service: combination of one or more services, both basic and supplementary, together with a number of other charging relevant parameters to define a customized service for the purpose of Advice of Charge (AoC).

billing: function whereby CDRs generated by the charging function(s) are transformed into bills requiring payment.

Billing Domain (BD): part of the operator network, which is outside the core network, that receives and processes CDR files from the core network charging functions.
It includes functions that can provide billing mediation and billing or other (e.g. statistical) end applications. 
It is only applicable to offline charging (see "Online Charging System" for equivalent functionality in online charging).

chargeable event: activity utilizing telecommunications network infrastructure and related services for:

-	user to user communication (e.g. a single call, a data communication session or a short message); or

-	user to network communication (e.g. service profile administration); or

-	inter-network communication (e.g. transferring calls, signalling, or short messages); or

-	mobility (e.g. roaming or inter-system handover); and

-	that the network operator may want to charge for.

charged party: user involved in a chargeable event that has to pay parts or the whole charges of the chargeable event, or a third party paying the charges caused by one or all users involved in the chargeable event, or a network operator.

charging: function within the telecommunications network and the associated OCS/BD components whereby information related to a chargeable event is collected, formatted, transferred and evaluated in order to make it possible to determine usage for which the charged party may be billed (offline charging) or the subscribers account balance may be debited (online charging).

Charging Data Record (CDR): formatted collection of information about a chargeable event (e.g. time of call set-up, duration of the call, amount of data transferred, etc.) for use in billing and accounting.
For each party to be charged for parts of or all charges of a chargeable event a separate CDR shall be generated, i.e. more than one CDR may be generated for a single chargeable event, e.g. because of its long duration, or because more than one charged party is to be charged.

charging function: entity inside the core network domain, subsystem or service that is involved in charging for that domain, subsystem or service

Fully Qualified Partial CDR (FQPC): partial CDR that contains a complete set of the fields specified in the present document.
This includes all the mandatory and conditional fields as well as those fields that the PLMN operator has provisioned to be included in the CDR. The first Partial CDR is a FQPC.

GPRS: packet switched bearer and radio services for GSM and UMTS systems.

GTP': GPRS protocol used for CDR transport. It is derived from GTP with enhancements to improve transport reliability necessary for CDRs.

NOTE:	This protocol is not used for tunnelling.

inter-system change: change of radio access between different radio access technologies such as GSM and UMTS.

middle tier TS: term used for the 3GPP charging TSs that specify the domain / subsystem / service specific, online and offline, charging functionality.
These are all the TSs in the numbering range from TS 32.250 to TS 32.27x, e.g. TS 32.250 [10] for the CS domain, or TS 32.270 [30] for the MMS service. Currently, there is only one "tier 1" TS in 3GPP, which is TS 32.240 [1] that specifies the charging architecture and principles. Finally, there are a number of top tier TSs in the 32.29x numbering range ([50] ff) that specify common charging aspects such as parameter definitions, encoding rules, the common billing domain interface or common charging applications.

near real time: near real time charging and billing information is to be generated, processed, and transported to a desired conclusion in less than one (1) minute.

offline charging: charging mechanism where charging information does not affect, in real-time, the service rendered.

online charging: charging mechanism where charging information can affect, in real-time, the service rendered and therefore a direct interaction of the charging mechanism with bearer/session/service control is required.

Online Charging System (OCS): the entity that performs real-time Credit-Control.
Its functionality includes transaction handling, rating, online correlation and management of subscriber account balances.

packet switched domain: domain within GSM / UMTS in which data is transferred in packet switched mode.
Corresponds to the term "GPRS".

partial CDR: CDR that provides charging information on part of a subscriber session.
A long session may be covered by several partial CDRs. Two formats are considered for Partial CDRs. 
One that contains all of the necessary fields; the second has a reduced format.

real time: real time charging and billing information is to be generated, processed, and transported to a desired conclusion in less than one (1) second.

Reduced Partial CDR (RPC): partial CDRs that only provide mandatory fields and information regarding changes in the session parameters relative to the previous CDR.

EXAMPLE:	Location information is not repeated in these CDRs if the subscriber did not change its location.

settlement: payment of amounts resulting from the accounting process.

subscriber: entity (associated with one or more users) that is engaged in a subscription with a service provider.
The subscriber is allowed to subscribe and unsubscribe services, to register a user or a list of users authorized to enjoy these services, and also to set the limits relative to the use that associated users make of these services.

successful call: connection that reaches the communication or data transfer phase e.g. the "answered" state for speech connections.
All other connection attempts are regarded as unsuccessful.

tariff period: part of one (calendar) day during which a particular tariff is applied.
Defined by the time at which the period commences (the switch-over time) and the tariff to be applied after switch-over.

tariff: set of parameters defining the network utilization charges for the use of a particular bearer / session / service.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Bmb	Reference point for the CDR file transfer from the MBMS CGF to the BD

Bo	Reference point for the CDR file transfer from the OCF CGF to the BD

Bp	Reference point for the CDR file transfer from the GPRS CGF to the BD

Bx	Reference point between any (generic) 3GPP domain, subsystem or service CGF and the BD

Ga	Reference point for CDR transfer between a CDF and the CGF

Gi	Interface between the Packet-Switched domain and an external packet data network

Gn	Interface between two GSNs within the same PLMN

Gp	Interface between two GSNs in different PLMNs

kbit/s	Kilobits per second. 1 kbit/s = 210 bits per second

Mbit/s	Megabits per second. 1 Mbit/s = 220 bits per second

Rf	Offline charging reference point between a BM-SC and the CDF

Ro	Online charging reference point between a BM-SC and the OCS

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations defined in TR 21.905 [50], TS 32.240 [1], TS 23.246 [200] and the following apply:

ABNF	Augmented Backus-Naur Form

ACA	ACcounting-Answer

ACR	ACcounting-Request

AF	Application Function

AMF	Account balance Management Function

AoC	Advice of Charge

AVP	Attribute Value Pair

BCF	Bearer Charging Function

BCSM	Basic Call State Model

BD	Billing Domain

BMD	Billing Mediation Device

BM-SC	Broadcast Multicast - Service Centre

BS	Billing System

CAI	Charge Advice Information

CCA	Credit-Control-Answer

CCR	Credit-Control-Request

CDF	Charging Data Function

CDR	Charging Data Record

CG	Charging Gateway

CGF	Charging Gateway Function

CSE	CAMEL Service Environment

CTF	Charging Trigger Function

DRP	Data Record Packet

ECF	Event Charging Function

ECUR	Event Charging with Unit Reservation

EDP	Event Detection Point

EPS	Evolved Packet System

FCI	Furnish Charging Information

FQPC	Fully Qualified Partial CDR

FTAM	File Transfer, Access and Management

GCS AS	Group Communication Service Application Server

GTP'	The GPRS protocol used for CDR transport. It is derived from GTP with enhancements to improve transport reliability necessary for CDRs

IEC	Immediate Event Charging

IHOSS:OSP	Internet Hosted Octet Stream Service: Octet Stream Protocol

MBMS GW	Multimedia Broadcast Multicast Service Gateway

M-CDR	Mobility management generated - Charging Data Record

MME	Mobility Management Entity

OAM&P	Operations, Administration, Maintenance and Provisioning

OCS	Online Charging System

PT	Protocol Type (Field in GTP' header)

RF	Rating Function

RPC	Reduced Partial CDR

SCI	Subscriber Controlled Input

SCI	Send Charging Information

SCUR	Session Charging with Unit Reservation

TAP	Transferred Account Procedure

TDP	Trigger Detection Point

TID	Tunnel IDentifier

TLV	Type, Length, Value (GTP header format)

TMGI	Temporary Mobile Group Identifier

TV	Type, Value

VAS	Value Added Service

VASP	Value Added Service Provider

# 4	Architecture considerations

## 4.1	High level MBMS architecture

The high level MBMS architecture is as defined in TS 23.246 [200] with extensions for Group Communication System Enablers for LTE (GCSE_LTE) in TS 23.468 [205].

The following clauses detail only service level charging. MBMS related aspects of bearer level charging are defined in TS 32.251 [11].

## 4.2	MBMS offline charging architecture

Figure 4.2.1 depicts the MBMS offline charging architecture. As defined in TS 32.240 [1], the BM-SC contains an integrated CTF that generates charging events that are passed to the CDF via the Rf reference point.

Figure 4.2.1: Charging architecture for MBMS offline charging

## 4.3	MBMS online charging architecture

Figure 4.3.1 depicts the MBMS online charging architecture.

Figure 4.3.1: Charging architecture for MBMS online charging

For online charging, the BM-SC utilizes the Ro interface and the protocol and application towards the OCS is as specified in TS 32.299 [50] and the present document.

# 5	MBMS charging principles and scenarios

## 5.1	MBMS charging principles

### 5.1.1	General principles for GPRS

A Multimedia Broadcast and Multicast Service consists of an MBMS user service, as defined in TS 22.246 [202] and TS 26.346 [203], that is delivered over one or more MBMS Bearer Services, as defined in TS 22.146 [201] and TS 23.246 [200].

NOTE:	MBMS Bearer Service is referred in TS 22.246 [202] as MBMS transport service.

The BM-SC shall collect charging information for mobile subscribers receiving services through MBMS and/or for content providers delivering content through MBMS. Transactions involving the content provider (or VASP) shall be possible.

The BM-SC collects charging related information, such as:

- -	Identification of the source of content;

- -	Type of user service (streaming, download or carousel);

- -	Type of bearer service used to deliver content (broadcast or multicast);

- -	Identification of subscribers receiving service;

- -	Delivery notification from individual subscribers.

NOTE:	Carousel services are not considered in the present document in the current 3GPP release.

Table 5.1.1.1 shows the parties to be charged for the different MBMS Bearer Services used as identified by TS 22.246 [202] and TS 22.146 [201].

Table 5.1.1.1: Charging requirements for service delivery

| Service Aspects | MBMS Bearer Service used |  |
| --- | --- | --- |
|  | Multicast (one or more) | Broadcast (one or more) |
| User Service (Content) | Receiving subscriber | Receiving subscriber |
| Bearer Service (Transport) | Content provider and/or receiving subscriber | Content provider |

The user service, as shown in table 5.1.1.1, shall be charged either by subscription (out of scope of the present document) or as a one time event charge (e.g. key management). Charging associated with the user service may be treated independently from charging associated with the transport of the user service.

Charging for the bearer service may be based on the session information (e.g. QoS, media type, and service area) and one of the following, as described in TS 22.146 [201]:

- -	Session duration (time from the MBMS Session Start procedure to MBMS Session Stop procedure as defined in TS 23.246 [200]);

- -	Volume of data of a session;

- -	Duration of time whilst a subscriber is registered to receive a user service (or from Join to Leave);

- -	Volume of data transferred whilst a subscriber is registered to receive a user service (from Join to Leave).

Table 5.1.1.2 shows the applicability of the accounting measurements to the different bearer services used.

Table 5.1.1.2: Applicability of accounting measurements

| Accounting measurement | Applicable to (Yes / No) |  |
| --- | --- | --- |
|  | Broadcast Service | Multicast Service |
| Session Duration | Yes | Yes |
| Volume of data of a session | Yes | Yes |
| Duration of time whilst a subscriber is registered to receive a session | No | Yes |
| Volume of data transferred whilst a subscriber is registered to receive a session | No | Yes |

### 5.1.1A	General principles for EPS

The BM-SC shall collect charging information for mobile subscribers receiving services through MBMS and/or for content providers delivering content through MBMS. Transactions involving the content provider (or VASP) shall be possible. When the MBMS Bearer Service is established for Group Communication Service, as specified in TS 23.468 [204], the Group Communication Service Application Server (GCS AS) is considered to be the Content Provider.

The BM-SC collects charging related information, such as:

-	Identification of the source of content;

-	Type of user service (streaming, download or carousel);

-	Type of bearer service used to deliver content (broadcast or enhanced broadcast).

NOTE:	Carousel services are not considered in the present document in the current 3GPP release.

Table 5.1.1A.1 shows the parties to be charged for the different MBMS Bearer Services used as identified by TS 22.246 [202] and TS 22.146 [201].

Table 5.1.1A.1: Charging requirements for service delivery

| Service Aspects | MBMS Bearer Service used |  |
| --- | --- | --- |
|  | Enhanced Broadcast (one or more) | Broadcast (one or more) |
| User Service (Content) | Receiving subscriber | Receiving subscriber |
| Bearer Service (Transport) | Content provider | Content provider |

The user service, as shown in table 5.1.1A.1, shall be charged either by subscription (out of scope of the present document) or as a one time event charge (e.g. key management). Charging associated with the user service may be treated independently from charging associated with the transport of the user service.

Charging for the bearer service may be based on the session information (e.g. QoS, media type, and service area) and one of the following, as described in TS 22.146 [201]:

-	Session duration (time from the MBMS Session Start procedure to MBMS Session Stop procedure as defined in TS 23.246 [200]).

-	Data transfer time (from "MBMS data transfer start" to "MBMS data transfer stop" as defined in TS 23.246 [200]);

-	Volume of data of a session.

Table 5.1.1A.2 shows the applicability of the accounting measurements to the different bearer services used.

Table 5.1.1A.2: Applicability of accounting measurements

| Accounting measurement | Applicable to (Yes / No) |  |
| --- | --- | --- |
|  | Broadcast Service | Enhanced Broadcast Service |
| Session Duration | Yes | Yes |
| Data Transfer Time | Yes | Yes |
| Volume of data of a session | Yes | Yes |
| Duration of time whilst a subscriber is registered to receive a session | No | No |
| Volume of data transferred whilst a subscriber is registered to receive a session | No | No |

### 5.1.2	Triggers for generation of charging information

-	Bearer service initiation/termination.

-	Key management.

## 5.2	MBMS offline charging scenarios

### 5.2.1	Basic principles

As described in clause 5.1, charging may be based on events (such as key management) or based on MBMS sessions. However, as large numbers of users are expected to use services delivered using MBMS, generation of charging information should be performed in a manner that ensures the charging entities and billing domain are not overloaded.

Charging information shall be generated for subscribers and/or for content providers.

This reporting is achieved by sending Charging Data Request [start, interim, stop and event] from the BM-SC to the CDF.

The Diameter client (BM-SC) uses Charging Data Request [start, interim and stop] in procedures related to both subscriber and content provider charging.

In table 5.2.1.1, table 5.2.1.2 and table 5.2.1.3, the terms "configurable" implies that operators may enable or disable the generation of an Charging Data Request message by the MBMS node in response to a particular trigger.

Table 5.2.1.1: Charging Data Request messages for subscriber charging for GPRS

| message | Trigger | Mandatory/ Configurable |
| --- | --- | --- |
| Charging Data Request [Start] | Authorization of UE to MBMS Bearer Service (for multicast only) | Mandatory |
|  | Reception of first Session Start Response from any GGSN (for broadcast only) | Configurable |
| Charging Data Request [Interim] | Authorization of MBMS UE context activation (for multicast only) | Configurable |
|  | Reception of first Session Start Response from any GGSN (for multicast only) | Configurable |
|  | Reception of first Session Stop Response from any GGSN (for multicast only) | Configurable |
|  | Expiration of AVP [Acct-Interim-Interval] | Configurable |
|  | Reception of MBMS UE context modification | Configurable |
| Charging Data Request [Stop] | Reception of Leave Indication from UE (for multicast only) | Mandatory |
|  | Reception of first Session Stop Response from any GGSN (for broadcast only) | Configurable |
|  | Implementation dependent for termination of MBMS User Service | Configurable |
| Charging Data Request [Event] | Implementation dependent for MBMS User Service charging | Configurable |

Table 5.2.1.2: Charging Data Request messages for content provider charging for GPRS

| message | Trigger | Mandatory/ Configurable |
| --- | --- | --- |
| Charging Data Request [Start] | First Session Start Response from any GGSN | Mandatory |
| Charging Data Request [Interim] | Registration or Deregistration Request received from any GGSN | Configurable |
|  | Deregistration Response received from any GGSN | Configurable |
|  | Expiration of AVP [Acct-Interim-Interval] | Configurable |
| Charging Data Request [Stop] | First Session Stop Response from any GGSN | Mandatory |

Table 5.2.1.3: Charging Data Request messages for content provider charging for EPS

| message | Trigger | Mandatory/ Configurable |
| --- | --- | --- |
| Charging Data Request [Start] | First Session Start Response from any MBMS GW. | Mandatory |
| Charging Data Request [Interim] | Deregistration Response received from any MBMS GW. | Configurable |
|  | Expiration of AVP [Acct-Interim-Interval] | Configurable |
| Charging Data Request [Stop] | First Session Stop Response from any MBMS GW. | Mandatory |

### 5.2.2	Rf message flows

#### 5.2.2.1	Broadcast service

##### 5.2.2.1.1	User service charging

An MBMS user service that is delivered using a broadcast bearer may be Event charged or Session charged. 
As there is no 3GPP specified signalling for a UE to activate or deactivate the Broadcast Service, it is MBMS user service dependent (e.g. key management) when the ACR is triggered. 
The Event based and Session based offline charging flows are as defined in TS 32.299 [50].

##### 5.2.2.1.2	Session Start for GPRS

Where charging for the content provider is applied for GPRS, the procedure in figure 5.2.2.1.2.1 applies.

Figure 5.2.2.1.2.1: Rf interaction during Broadcast Session Start Procedure for a broadcast bearer

1)	The BM-SC performs the MBMS Session Start procedure as described in TS 23.246 [200].

2a)	On receiving the first MBMS Session Start Response from any GGSN, the BM-SC sends a Charging Data Request.

2b)	The remainder of the MBMS Session Start procedure may occur in parallel with the Charging Data RequestCharging Data Request procedure in 2a).

The full details of the MBMS Session Start procedure for the broadcast bearer are described in TS 23.246 [200].

##### 5.2.2.1.2A	Session Start for EPS

Figure 5.2.2.1.2A.1 shows the charging interaction for EPS during the MBMS Session Start procedure for a broadcast bearer.

Figure 5.2.2.1.2A.1: Rf interaction during Broadcast Session Start Procedure for a broadcast bearer

1-2)	The BM-SC performs the MBMS Session Start procedure as described in TS 23.246 [200].

3-4)	On receiving the first MBMS Session Start Response from any MBMS GW, the BM-SC sends an Charging Data Request.

5-6)	The remainder of the MBMS Session Start procedure may occur in parallel with the Charging Data Request procedure in 3-4).

The full details of the MBMS Session Start procedure for the broadcast bearer are described in TS 23.246 [200].

##### 5.2.2.1.2B	Void

##### 5.2.2.1.3	Session Stop for GPRS

Figure 5.2.2.1.3.1 shows the charging interaction for GPRS during the MBMS Session Stop procedure for a broadcast bearer.

Figure 5.2.2.1.3.1: Rf interaction during MBMS Session Stop procedure for a broadcast bearer

1) 	The BM-SC performs the MBMS Session Stop procedure as described in TS 23.246 [200].

2a)	On receiving a Session Stop Response from any GGSN, the BM-SC sends a Charging Data Request.

2b)	The remainder of the MBMS Session Stop procedure occurs in parallel with the Charging Data Request procedure in 2a).

The full details of the MBMS Session Stop procedure for the broadcast bearer are described in TS 23.246 [200].

##### 5.2.2.1.3A	Session Stop for E-UTRAN

Figure 5.2.2.1.3A.1 shows the charging interaction for EPS during the MBMS Session Stop procedure for a broadcast bearer.

Figure 5.2.2.1.3A.1: Rf interaction during MBMS Session Stop procedure for a broadcast bearer

1-2)	The BM-SC performs the MBMS Session Stop procedure as described in TS 23.246 [200].

3-4)	On receiving the first MBMS Session Stop Response from any MBMS GW, the BM-SC sends a Charging Data Request.

5-6)	The remainder of the MBMS Session Stop procedure may occur in parallel with the Charging Data Request procedure in 3-4).

The full details of the MBMS Session Stop procedure for the broadcast bearer are described in TS 23.246 [200].

##### 5.2.2.1.3B	Void

##### 5.2.2.1.4	BM-SC initiated Registration and Deregistration

BM-SC initiated Registration and Deregistration are handled through OAM&P towards the GGSNs (and subsequent nodes) and therefore Rf interactions (Charging Data Request [Start] and Charging Data Request [Stop] respectively) may be triggered when the Registration and De-registration is triggered through OAM&P. These Rf interactions should only occur for sessions that have already started.

##### 5.2.2.1.5	Session Update for EPS with E-UTRAN and UTRAN

Figure 5.2.2.1.5.1 shows the charging interaction for EPS during the MBMS Session Update procedure for a broadcast bearer

Figure 5.2.2.1.5.1: Rf interaction during Broadcast Session Update Procedure for a broadcast bearer

1-2) The BM-SC performs the MBMS Session Update procedure as described in TS 23.246 [200].

3-4) On receiving the first MBMS Session Update Response from any MBMS GW, the BM-SC sends a Charging Data Request.

5-6) The remainder of the MBMS Session Update procedure may occur in parallel with the Charging Data Request procedure in 3-4).

The full details of the MBMS Session Update procedure for the broadcast bearer are described in TS 23.246 [200].

#### 5.2.2.2	Multicast Service

##### 5.2.2.2.1	Session Start

Figure 5.2.2.2.1.1 shows the charging interaction during the MBMS Session Start procedure for a multicast bearer.

Figure 5.2.2.2.1.1: Rf interaction during MBMS Session Start procedure for a multicast bearer

1)	The BM-SC performs the MBMS Session Start procedure as described in TS 23.246 [200].

2a)	On receiving the first Session Start Response from any GGSN, the BM-SC sends an Charging Data Request. The accounting request may be for subscriber and/or content provider charging. For subscriber charging, the Charging Data Request shall be "Interim". For content provider charging, the ACR shall be "Start". It shall be possible to send one Charging Data Request message for multiple subscribers of the same Multicast Service, but the procedure in the BM-SC to group subscribers is implementation dependent.

2b)	The remainder of the MBMS Session Start procedure occurs in parallel with the Charging Data Request procedure in 2a.

The full details of the MBMS Session Start procedure for the multicast bearer are described in TS 23.246 [200].

##### 5.2.2.2.2	Session Stop

Figure 5.2.2.2.2.1 shows the charging interaction during the MBMS Session Stop procedure for a multicast bearer.

Figure 5.2.2.2.2.1: Rf interaction during MBMS Session Stop procedure for a multicast bearer

1) 	The BM-SC performs the MBMS Session Stop procedure as described in TS 23.246 [200].

2a)	On receiving the first Session Stop Response from any GGSN, the BM-SC sends a Charging Data Request. 
For subscriber charging, the Charging Data Request shall be Charging Data Request [Interim] and it shall be possible to send one Charging Data Request message for multiple or all subscribers of the same Multicast Service, that are still active, and is implementation and service dependent. 
For content provider charging, the Charging Data Request shall be Charging Data Request [Stop].

2b)	The remainder of the MBMS Session Stop procedure occurs in parallel with the ACR procedure in 2a).

The full details of the Session Stop procedure for the multicast bearer are described in TS 23.246 [200].

##### 5.2.2.2.3	BM-SC initiated MBMS Deregistration

Figure 5.2.2.2.3.1 shows the charging interaction during the BM-SC initiated MBMS Deregistration procedure for a multicast bearer for an already started session.

Figure 5.2.2.2.3.1: Rf interaction during BM-SC initiated MBMS Deregistration procedure
for a multicast bearer

1)	The BM-SC performs the MBMS Deregistration procedure as described in TS 23.246 [200]. 
The BM-SC sends a De-registration Request message to all GGSNs contained in the "list of downstream nodes" parameter of the corresponding MBMS Bearer Context to indicate the session is terminated.

2a)	On receiving an MBMS Deregistration Response from the GGSN, the BM-SC sends an Charging Data Request [Stop].

2b)	The remainder of the MBMS Deregistration procedure occurs in parallel with the Charging Data Request procedure in 2a.

The full details of the MBMS Deregistration procedure for the multicast bearer are described in TS 23.246 [200].

##### 5.2.2.2.4	UE Activation

Figure 5.2.2.2.4.1 should apply to subscriber's that activate the Multicast Service.

Figure 5.2.2.2.4.1: Rf interaction during MBMS Multicast Service Activation procedure
for a multicast bearer

Full details of the activation procedure are described in the MBMS Multicast Service Activation procedure in TS 23.246 [200].

##### 5.2.2.2.5	UE Deactivation

Figure 5.2.2.2.5.1 should only apply to subscriber's that deactivate the Multicast Service before the session has stopped, i.e. before the MBMS Session Stop procedure is invoked.

Figure 5.2.2.2.5.1: Rf interaction during MBMS Multicast Service Deactivation procedure
for a multicast bearer

Full details of the deactivation procedure are described in the MBMS Multicast Service Deactivation procedure in TS 23.246 [200].

### 5.2.3	CDR generation

#### 5.2.3.1	CDRs related to MBMS subscribers

##### 5.2.3.1.1	Triggers for S-BMSC-CDR charging information collection

An S-BMSC-CDR is used to collect charging information related to the MBMS Bearer Service information for a UE/MS in the BM-SC. A CDR is generated for each MBMS Bearer Service used and for each subscriber using the MBMS Bearer Service.

An S-BMSC-CDR shall be opened at UE activation as triggered by an ACR[Start]. The volume for the MBMS bearer context is counted in downlink direction.

The subsequent clauses identify in detail the conditions for adding information to, and closing the BMSC-CDR for generation towards the CGF.

##### 5.2.3.1.2	Triggers for S-BMSC-CDR charging information addition

A new container shall be added to the S-BMSC-CDR on encountering some trigger conditions. 
Table 5.2.3.1.2.1 identifies which conditions are supported to permit addition of a new container to the S-BMSC-CDR. 
The start time of the new container shall indicate the time, whichever is later, at which the first Session Start Response was received, MBMS UE context activation, or the last partial CDR was closed.

Table 5.2.3.1.2.1: Triggers for S-BMSC-CDR addition

| Closure Conditions | Description/Behaviour |
| --- | --- |
| Tariff Time Change | On reaching the Tariff Time Change a set of "List of Traffic Data Volumes" containers, i.e. all active traffic data flow containers, shall be added to the CDR. |
| Session Start | A Traffic Data Volume container may be added when an MBMS Session Start is performed |
| Session Stop | A Traffic Data Volume container may be added when an MBMS Session Stop is performed. |
| MBMS UE context modification | A Traffic Data Volume container may be added when an MBMS UE context modification is received by the BM-SC. See note 1. |
| CDR Closure | All active "List of Traffic Data Volumes" containers shall be added to the eG-CDR. |
| NOTE 1: One trigger for modification of MBMS UE context is as a result of inter-system (RAT) change and there is no reliable mechanism to report the change at the actual time of change. This is due to the UE remaining in IDLE mode from the core network perspective. NOTE 2: MBMS charging is based on the volume of downlink data. Therefore the ""List of Traffic Data Volumes"" shall not count data volumes in uplink direction. |  |

NOTE:	One trigger for modification of MBMS UE context is as a result of inter-system (RAT) change and there is no reliable mechanism to report the change at the actual time of change. This is due to the UE remaining in IDLE mode from the core network perspective.

##### 5.2.3.1.3	Triggers for S-BMSC-CDR closure

- The S-BMSC-CDR shall be closed on encountering some trigger conditions. 
Table 5.2.3.1.3.1 identifies which conditions are supported to permit closure of the S-BMSC-CDR.

Table 5.2.3.1.3.1: Triggers for S-BMSC-CDR closure

| Closure Conditions | Description/Behaviour |
| --- | --- |
| Service Deactivation | Deactivation of the MBMS service in the BM-SC shall result in the CDR being closed. The trigger condition covers: - UE initiated deactivation; - termination of the MBMS User Service - any abnormal release. |
| Charging Data Request[Stop] | On reception of Charging Data Request[Stop], a CDR is closed. |
| Partial Record Reason | OAM&P reasons permit the closure of the CDR for internal reasons. The trigger condition covers: - data volume limit; - time (duration) limit; - maximum number of charging condition changes; - management intervention. |

- The Partial Record generation trigger thresholds are those associated with the Charging Characteristics. 
The Partial Record generation trigger thresholds are configuration parameters defined per charging characteristics profile by the operator through OAM&P means.

#### 5.2.3.2	CDRs related to content provider

##### 5.2.3.2.1	Triggers for BMSC-CDR charging information collection

A C-BMSC-CDR is used to collect charging information related to the MBMS Bearer Service information for a content provider to the BM-SC. A C-BMSC-CDR is generated for each MBMS Bearer Service.

A C-BMSC-CDR shall be opened at MBMS Session Start as triggered by an Charging Data Request[ Start]. The volume for the MBMS bearer context is counted in downlink direction. Not all of the charging information to be collected is static, and may be dependent on dynamic (de-)registration of packet-switched nodes to the MBMS bearer context.

The subsequent clauses identify in detail the conditions for adding information to, and closing the C-BMSC-CDR for passing towards the CGF.

##### 5.2.3.2.2	Triggers for C-BMSC-CDR charging information addition

A new container shall be added to the C-BMSC-CDR on encountering some trigger conditions. 
Table 5.2.3.2.2.1 identifies which conditions are supported to permit addition of a new container to the C-BMSC-CDR.

Table 5.2.3.2.2.1: Triggers for C-BMSC-CDR addition

| Closure Conditions | Description/Behaviour |
| --- | --- |
| Tariff Time Change | On reaching the Tariff Time Change a set of "List of Traffic Data Volumes" containers, i.e. all active service data flow containers, shall be added to the CDR. |
| CDR Closure | All active "List of Traffic Data Volumes" containers shall be added to the eG-CDR. |
| NOTE: MBMS charging is based on the volume of downlink data. Therefore the ""List of Traffic Data Volumes"" shall not count data volumes in uplink direction. |  |

##### 5.2.3.2.3	Triggers for C-BMSC-CDR closure

- The C-BMSC-CDR shall be closed on encountering some trigger conditions. 
Table 5.2.3.2.3 identifies which conditions are supported to permit closure of the C-BMSC-CDR.

Table 5.2.3.2.3: Triggers for C-BMSC-CDR closure

| Closure Conditions | Description/Behaviour |
| --- | --- |
| Service Deactivation | Deactivation of the MBMS service in the BM-SC shall result in the CDR being closed. The trigger condition covers: - MBMS Session Stop; - termination of the MBMS User Service - any abnormal release. |
| Charging Data Request[Stop] | On reception of aCharging Data Request [Stop], the CDR shall be closed. |
| Partial Record Reason | OAM&P reasons permit the closure of the CDR for internal reasons. The trigger condition covers: - data volume limit; - time (duration) limit; - change in list of downstream nodes; - management intervention; - MBMS service area; - access indicator. |

- The Partial Record generation trigger thresholds are configuration parameters defined per charging characteristics profile by the operator through OAM&P means.

### 5.2.4	Ga record transfer flows

For further details on the Ga protocol application refer to TS 32.295 [54].

### 5.2.5	Bmb CDR file transfer

The CGF transfers the CDR files to the BD as described in TS 32.297 [52]. 
For further details on the Bmb protocol application refer to TS 32.297 [52].

## 5.3	MBMS online charging scenarios

### 5.3.1	Basic principles

MBMS online charging uses the Credit-Control application as specified in TS 32.299 [50] and in the present document.

Online charging of content providers is not supported in this release of the present document.

The type of online interaction used is dependent on the user service type, bearer type and whether delivery notification is required. Table 5.3.1.1 shows this dependency.

Table 5.3.1: Online interaction dependency on MBMS service parameters

| User Service Type | Bearer Service Type | Delivery Notification | Online Interaction |
| --- | --- | --- | --- |
| Key Management | N/A | N/A | IEC |
| Streaming | Broadcast | N/A | Operator Configurable |
| Streaming | Multicast | N/A | SCUR |
| Download | Broadcast | Required | Operator Configurable |
| Download | Multicast | Required | Operator Configurable |
| Download | Broadcast | Not required | Operator Configurable |
| Download | Multicast | Not required | Operator Configurable |
| NOTE: Operator configurable options imply that IEC, SCUR and ECUR should be supported |  |  |  |

It is not possible to perform charging transactions in a load efficient manner as in offline charging (see clause 5.2). Therefore, one online charging interaction is necessary for each user.

### 5.3.2	Ro message flows

#### 5.3.2.1	Broadcast Service

##### 5.3.2.1.1	User service charging

An MBMS user service that is delivered using a broadcast bearer may be Event charged or Session charged. 
As there is no 3GPP specified signalling for a UE to activate or deactivate the Broadcast Service, it is MBMS user service dependent (e.g. key management) when the Charging Data Request is triggered. 
The Event based or Session based online charging flows are as defined in TS 32.299 [50].

##### 5.3.2.1.2	Session Start

As online charging does not apply to content provider, this scenario is not applicable.

##### 5.3.2.1.3	Session Stop

As online charging does not apply to content provider, this scenario is not applicable.

##### 5.3.2.1.4	BM-SC initiated Registration and Deregistration

As online charging does not apply to content provider, this scenario is not applicable.

#### 5.3.2.2	Multicast Service

##### 5.3.2.2.1	Session Start

Figure 5.3.2.2.1.1 shows the charging interaction during the MBMS Session Start procedure for a multicast bearer.

Figure 5.3.2.2.1.1: Ro interaction during MBMS Session Start procedure for a multicast bearer

1) 	The BM-SC performs the MBMS Session Start procedure as described in TS 23.246 [200].

2a)	On receiving the first Session Start Response from any GGSN, the BM-SC sends a Debit / Reserve Units Request for each subscriber that has joined the service.

2b)	The remainder of the MBMS Session Start procedure occurs in parallel with the Debit / Reserve Units Request procedure in 2a.

The full details of the MBMS Session Start procedure for the multicast bearer are described in TS 23.246 [200].

##### 5.3.2.2.2	Session Stop

Figure 5.3.2.2.2.1 shows the charging interaction during the MBMS Session Stop procedure for a multicast bearer.

Figure 5.3.2.2.2.1: Ro interaction during MBMS Session Stop procedure for a multicast bearer

1) 	The BM-SC performs the MBMS Session Stop procedure as described in TS 23.246 [200].

2a)	On receiving the first Session Stop Response from any GGSN, the BM-SC sends a Debit / Reserve Units Request for each subscriber that is still joined to the service.

2b)	The remainder of the MBMS Session Stop procedure occurs in parallel with the Debit / Reserve Units Request procedure in 2a.

The full details of the Session Stop procedure for the multicast bearer are described in TS 23.246 [200].

##### 5.3.2.2.3	BM-SC initiated MBMS Deregistration

Figure 5.3.2.2.3.1 shows the charging interaction during the BM-SC initiated MBMS Deregistration procedure for a multicast bearer for an already started session.

Figure 5.3.2.2.3.1: Ro interaction during BM-SC initiated MBMS Deregistration procedure
for a multicast bearer

1)	The BM-SC performs the MBMS Deregistration procedure as described in TS 23.246 [200]. 
The BM-SC sends a Deregistration Request message to all GGSNs contained in the "list of downstream nodes" parameter of the corresponding MBMS Bearer Context to indicate the session is terminated.

2a)	On receiving an MBMS Deregistration Response from the GGSN, the BM-SC sends a Debit / Reserve Units Request [Terminate] for each subscriber that has joined the service.

2b)	The remainder of the MBMS Deregistration procedure occurs in parallel with the Debit / Reserve Units Request procedure in 2a).

The full details of the MBMS Deregistration procedure for the multicast bearer are described in TS 23.246 [200].

##### 5.3.2.2.4	UE Activation

Figure 5.3.2.2.4.1 applies to subscribers that activate the Multicast Service.

Figure 5.3.2.2.4.1: Ro interaction during MBMS Multicast Service Activation procedure
for a multicast bearer

Full details of the activation procedure are described in the MBMS Multicast Service Activation procedure in TS 23.246 [200].

##### 5.3.2.2.5	UE Deactivation

Figure 5.3.2.2.5.1 applies to subscribers that deregister from the Multicast Service before the session has stopped, i.e. before the MBMS Session Stop procedure is invoked.  
This procedure is optionally applied, if the deactivation occurs after the MBMS Session Stop procedure, depending on the charging model applied.

Figure 5.3.2.2.5: Ro interaction during MBMS Multicast Service Deactivation procedure
for a multicast bearer

Full details of the deactivation procedure are described in the MBMS Multicast Service Deactivation procedure in TS 23.246 [200].

### 5.3.3	Credit-Control related

#### 5.3.3.1	Triggers for stopping for an MBMS service Credit-Control session

In addition to message flows in clause 5.3.2, a Debit / Reserve Units Request [Terminate] is sent to OCS when:

a)	Session termination is indicated by the OCS (e.g. Credit Limit Reached);

b)	Abort-Session-Request is received from the OCS, this also results in the deactivation of the MBMS UE Context (from step 4 of clause 5.3.2.2.5), if one exists for the session being terminated.

#### 5.3.3.2	Triggers for providing interim information for a MBMS service Credit-Control session

In addition to the message flows in clause 5.3.2, a Debit / Reserve Units Request [Update] is sent to OCS when:

a)	Granted quota runs out;

b)	Validity time for granted quota expires;

c)	Update is requested by the OCS;

d)	Change of charging conditions occur and according re-authorisation trigger re-authorisation is needed;

e)	Management intervention.

# 6	Definition of charging information

## 6.1	Data description for MBMS offline charging

### 6.1.1	Rf message contents

#### 6.1.1.1	Summary of offline charging message formats

The BM-SC generates accounting information that can be transferred to the CDF. For this purpose, the MBMS Charging Data transfer operation employs the Charging Data Request and Charging Data Response messages. The request can be of type start, stop, interim and event. The Charging Data Request message includes all charging information and the answer is just an acknowledgement of the request message. 
Detailed information about the offline charging application is described in TS 32.299 [50].

The following clauses describe the different information elements used in the Charging Data messages. and the category in the tables are used according to the charging data configuration defined in clause 5.4 of TS 32.240 [1].

Table 6.1.1.1.1 describes the use of these messages for offline charging.

Table 6.1.1.1.1: Offline charging messages reference table

| Message | Source | Destination |
| --- | --- | --- |
| Charging Data Request | BM-SC | CDF |
| Charging Data Response | CDF | BM-SC |

#### 6.1.1.2	Structure for the Rf message formats

##### 6.1.1.2.1	Charging Data Request  message

Table 6.1.1.2.1 illustrates the basic structure of a Diameter ACR message as used for MBMS offline charging.

Table 6.1.1.2.1: ACR message contents for offline charging

| Information Element | Category | Description |
| --- | --- | --- |
| Session Identifier | M | Used as described in TS 32.299 [50]. |
| Originator Host | M | Used as described in TS 32.299 [50]. |
| Originator Domain | M | Used as described in TS 32.299 [50]. |
| Destination Domain | M | Used as described in TS 32.299 [50]. |
| Operation Type | M | Used as described in TS 32.299 [50]. |
| Operation Number | M | Used as described in TS 32.299 [50]. |
| Session Identifier | M | Used as described in TS 32.299 [50]. |
| User Name | OC | Used as described in TS 32.299 [50]. |
| Destination Host | OC | Used as described in TS 32.299 [50]. |
| Operation Interval | OC | Used as described in TS 32.299 [50]. |
| Origination State | OC | Used as described in TS 32.299 [50]. |
| Origination Timestamp | OC | Used as described in TS 32.299 [50]. |
| Proxy Information | - | Not used. |
| Route Information | - | Not used . |
| Operation Token | OM | Used as described in TS 32.299 [50] |
| Service Information | OM | Described in TS 32.299 [50] |
| Subscriber Identifier | OM | Used as described in TS 32.299 [50]. As a minimum the IMSI and the MSISDN have to be included for subscriber charging. |
| PS Information | OC | Described in TS 32.251 [11] . |
| IMS Information | OC | Described in TS 32.260 [20] . |
| MBMS Information | OM | Described in clause 6.3. |
| NOTE: For structured IEs only the "IE" is listed in this table. Detailed description of the IEs is provided according to "Description" column. |  |  |

##### 6.1.1.2.2	Charging Data Response message

Table 6.1.1.2.2 illustrates the basic structure of a Charging Data Response message as used for MBMS charging. 
This message is always used by the CDF as specified below, regardless of the BM-SC it is received from and the Charging Data Request record type that is being replied to.

Table 6.1.1.2.2: Charging Data Response message contents for offline charging

| Information Element | Category | Description |
| --- | --- | --- |
| Session Identifier | M | Used as described in TS 32.299 [50]. |
| Operation Result | M | Used as described in TS 32.299 [50]. |
| Originator Host | M | Used as described in TS 32.299 [50]. |
| Originator Domain | M | Used as described in TS 32.299 [50]. |
| Operation Type | M | Used as described in TS 32.299 [50]. |
| Operation Number | M | Used as described in TS 32.299 [50]. |
| Operation Identifier | OM | Used as described in TS 32.299 [50]. |
| Operation Interval | OC | Used as described in TS 32.299 [50]. |
| Origination State | OC | Used as described in TS 32.299 [50]. |
| Origination Timestamp | OC | Used as described in TS 32.299 [50]. |
| Proxy Information | - | Not used. |

### 6.1.2	Ga message contents

### 6.1.3	CDR description on the Bmb interface

#### 6.1.3.0	Introduction

This clause provides stage 3 specifications of the CDR type and content for the MBMS Bearer Service. For each of the CDR types, a parameter table, which gives a short description of the parameters, is provided. The category in the tables are used according to the charging data configuration defined in clause 5.4 of TS 32.240 [1].

#### 6.1.3.1	CDR description for subscriber charging

Table 6.1.3.1.1: Subscriber BM-SC data (S-BMSC-CDR)

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | S-BM-SC record. |
| Served IMSI | M | IMSI of the served party. This may be obtained from the Subscription Id field, identified by a type of IMSI, of the ACR message. |
| GGSN Address used | C | The control plane IP address of the GGSN used for MBMS UE context activation. Present only for multicast. |
| Access Point Name Network Identifier | OC | The logical name of the connected access point to the external packet data network (network identifier part of APN). Present only for multicast. |
| Served PDP Address | OM | Represents the IP Multicast address associated with the MBMS bearer context. |
| List of Traffic Data Volumes | OM | A list of changes in charging conditions (including MBMS UE context modifications) for this MBMS Bearer Service, each change is time stamped. Charging conditions are used to categorize traffic volumes, such as per tariff period. Initial and subsequently changed QoS and corresponding data volumes are also listed. See note below. |
| Record Opening Time | M | Time stamp when UE activation occurs or record opening time on subsequent partial records. |
| Duration | M | Duration of this record. |
| Cause for Record Closing | M | The reason for the release of record. |
| Diagnostics | OM | A more detailed reason for the release of the connection. |
| Record Sequence Number | C | Partial record sequence number, only present in case of partial records. |
| Node ID | OM | Name of the recording entity. |
| Record Extensions | OC | A set of network operator/manufacturer specific extensions to the record. Conditioned upon the existence of an extension. |
| Local Record Sequence Number | OM | Consecutive record number created by this node. The number is allocated sequentially including all CDR types. |
| Served MSISDN | OM | The primary MSISDN of the subscriber. This may be obtained from the Subscription Id field, identified by a type of E.164, of the ACR message. |
| Bearer Service Description | OC | Holds the Session portion of the SDP data exchanged between the BMSC and UE during the notification phased. |
| MBMS Information | OM | A set of fields hold the MBMS specific parameters. The details are defined in clause 6.3.1.2. |
| Service Context Id | OM | Holds the context information to which the CDR belongs. |
| NOTE: MBMS charging is based on the volume of downlink data. Therefore the "List of Traffic Data Volumes" shall not count data volumes in uplink direction. |  |  |

#### 6.1.3.2	CDR description for content provider charging

Table 6.1.3.2.1: Content provider BM-SC data (C-BMSC-CDR)

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | C-BM-SC record. |
| Content Provider Id | M | Identity of the content provider. This may be obtained from the Subscription ID field of the Accounting Request message. |
| List of Downstream Nodes | M | A list of the control plane IP address of the GGSN/MBMS GWs used by the MBMS Bearer Service. |
| Access Point Name Network Identifier | OC | The logical name of the connected access point to the external packet data network (network identifier part of APN). Present only for multicast. |
| PDP/PDN Type | OM | This field indicates PDN type (i.e. IPv4 or IPv6). |
| Served PDP/PDN Address | OM | Represents the IP Multicast address used to transmit the MBMS user service, i.e. IPv4 or IPv6, if available. |
| List of Traffic Data Volumes | OM | A list of changes in charging conditions (including MBMS UE context modifications) for this MBMS Bearer Service, each change is time stamped. Charging conditions are used to categorize traffic volumes, such as per tariff period. Initial and subsequently changed QoS and corresponding data volumes are also listed. See note below. |
| Record Opening Time | M | Time stamp when MBMS Bearer Context is activated (i.e. MBMS Session Start) or record opening time on subsequent partial records. |
| Duration | M | Duration of this record. |
| Cause for Record Closing | M | The reason for the release of record. |
| Diagnostics | OM | A more detailed reason for the release of the connection. |
| Record Sequence Number | C | Partial record sequence number, only present in case of partial records. |
| Node ID | OM | Name of the recording entity. |
| Record Extensions | OC | A set of network operator/manufacturer specific extensions to the record. Conditioned upon the existence of an extension. |
| Local Record Sequence Number | OM | Consecutive record number created by this node. The number is allocated sequentially including all CDR types. |
| Recipient Address List | OC | The address(es) of the recipients registered to receive the bearer service. This field is applicable to Multicast Service but is not applicable to Broadcast Service. |
| Bearer Service Description | OC | Holds the Session portion of the SDP data exchanged between the BMSC and UE during the notification phased, see IMS-Information in table 6.3.1.1. |
| MBMS Information | OM | A set of fields hold the MBMS specific parameters. The details are defined in clause 6.3.1.2. |
| Service Context Id | OM | Holds the context information to which the CDR belongs. |
| NOTE: MBMS charging is based on the volume of downlink data. Therefore the "List of Traffic Data Volumes" shall not count data volumes in uplink direction. |  |  |

## 6.2	Data description for MBMS online charging

### 6.2.1	Ro message contents

#### 6.2.1.1	Summary of message formats

MBMS Online Charging use Debit / Reserve Units Request and Debit / Reserve Units Request messages defined in TS 32.299 [50]. The Debit / Reserve Units Request triggers the rating of the MBMS service and reserves units on the user's account. 
The Debit / Reserve Units Response is a response including any reserved units or an error code if the user is out of credit. 
Detailed information about the online charging application is described in TS 32.299 [50].

The Debit / Reserve Units Request for the "intermediate interrogation" and "final interrogation" reports the actual number of "units" that were used, from what was previously reserved. This determines the actual amount debited from the subscriber's account.

The following clauses describe the different fields used in the Debit / Reserve Units messages.The category in the tables are used according to the charging data configuration defined in clause 5.4 of TS 32.240 [1].

Table 6.2.1.1 describes the use of these messages for online charging.

Table 6.2.1.1: Online charging messages reference table

| Message | Source | Destination |
| --- | --- | --- |
| Debit / Reserve Units Request | BM-SC | OCS |
| Debit / Reserve Units Response | OCS | BM-SC |

#### 6.2.1.2	Structure for the  message formats

##### 6.2.1.2.1	Debit / Reserve Units Request message

Table 6.2.1.2.1.1 illustrates the basic structure of a Debit / Reserve Units Request message from the BM-SC as used for MBMS online charging.

Table 6.2.1.2.1.1: Debit / Reserve Units Request message contents

| Information Element | Category | Description |
| --- | --- | --- |
| Session Identifier | M | Used as described in TS 32.299 [50]. |
| Originator Host | M | Used as described in TS 32.299 [50]. |
| Originator Domain | M | Used as described in TS 32.299 [50]. |
| Destination Domain | M | Used as described in TS 32.299 [50]. |
| Operation Identifier | M | Used as described in TS 32.299 [50]. |
| Operation Token | M | Used as described in TS 32.299 [50]. |
| Operation Type | M | Used as described in TS 32.299 [50]. |
| Operation Number | M | Used as described in TS 32.299 [50]. |
| Destination Host | OC | Used as described in TS 32.299 [50]. |
| User Name | OC | Used as described in TS 32.299 [50]. |
| Origination State | OC | Used as described in TS 32.299 [50]. |
| Origination Timestamp | OC | Used as described in TS 32.299 [50]. |
| Subscriber Identifier | OC | Used as described in TS 32.299 [50]. As a minimum the IMSI and the MSISDN have to be included. |
| Termination Cause | OC | Used as described in TS 32.299 [50]. |
| Destination Host | OC | Used as described in TS 32.299 [50]. |
| Requested Action | OC | Used as described in TS 32.299 [50]. |
| Multiple Operation | OM | Used as described in TS 32.299 [50]. |
| Multiple Unit Operation | OC | Used as described in TS 32.299 [50]. |
| Subscriber Equipment Number | OC | Used as described in TS 32.299 [50]. |
| Service Information | OM | Defined in TS 32.299 [50] |
| PS Information | OC | Used as described in TS 32.251 [11] . |
| IMS Information | Oc | Used as described in TS 32.260 [20] . |
| MBMS Information | OM | Described in clause 6.3 |

##### 6.2.1.2.2	Debit / Reserve Units Response message

Table 6.2.1.2.2.1 illustrates the basic structure of a Debit / Reserve Units Response message as used for the BM-SC. This message is always used by the OCS as specified below, independent of the receiving BM-SC and the Debit / Reserve Units Request request type that is being replied to.

Table 6.2.1.2.2.1: Debit / Reserve Units Response message

| Information Element | Category | Description |
| --- | --- | --- |
| Session Identifier | M | Used as described in TS 32.299 [50]. |
| Operation Result | M | Used as described in TS 32.299 [50]. |
| Originator Host | M | Used as described in TS 32.299 [50]. |
| Originator Domain | M | Used as described in TS 32.299 [50]. |
| Operation Identifier | M | Used as described in TS 32.299 [50]. |
| Operation Type | M | Used as described in TS 32.299 [50]. |
| Operation Number | M | Used as described in TS 32.299 [50]. |
| Operation Number | OC | Used as described in TS 32.299 [50]. |
| Operation Failover | OC | Used as described in TS 32.299 [50]. |
| Multiple-Services-Credit-Control | OC | Used as described in TS 32.299 [50]. |
| Operation Failure Action | OC | Used as described in TS 32.299 [50]. |
| Operation Event Failure Action | OC | Used as described in TS 32.299 [50]. |
| Redirection Host | OC | Used as described in TS 32.299 [50]. |
| Redirection Host Usage | OC | Used as described in TS 32.299 [50]. |
| Redirection Cache Time | OC | Used as described in TS 32.299 [50]. |
| Proxy Information | OC | Used as described in TS 32.299 [50]. |
| Route Information | OC | Used as described in TS 32.299 [50]. |
| Failed parameter | OC | Used as described in TS 32.299 [50]. |

## 6.3	MBMS charging specific parameters

### 6.3.1	Definition of the MBMS charging information

#### 6.3.1.0	General

The MBMS Information parameter used for MBMS charging is provided in the Service Information parameter.

#### 6.3.1.1	MBMS charging information assignment for Service Information

The components that are used for MBMS charging are provided in the Service Information as described in table 6.3.1.1.1.

Table 6.3.1.1.1: Components of the Service Information used for MBMS charging

| Information Element | Category | Description |
| --- | --- | --- |
| Service Information | OM | A set of fields hold the specific parameter as defined in TS 32.299 [50]. For MBMS Charging the PS Information and IMS Information are used. |
| Subscriber Identifier | OC | This field contains the identification of the charged party (e.g. IMSI, MSISDN, Content Provider Id). |
| PS Information | OC | A set of fields hold the PS specific parameters. The details are defined in TS 32.251 [11]. |
| Node Id | OC | Used as defined in TS 32.251 [11]. |
| PDP/PDN Type | OC | Used as defined in TS 32.251 [11]. See note. |
| Served PDP/PDN Address | OC | Used as defined in TS 32.251 [11]. See note. |
| GGSN Address | OC | Used as defined in TS 32.251 [11]. |
| 3GPP SGSN MCC MNC | OC | Used as defined in TS 32.251 [11]. |
| RAT Type | OC | Used as defined in TS 32.251 [11]. |
| Called Station Id | OC | The logical name of the connected access point to the external packet data network (network identifier part of APN). Present only for multicast. |
| Traffic Data Volumes | OC | Used as defined in TS 32.251 [11]. |
| Change Condition | OC | This field holds the reason for sending ACR from the BM-SC. |
| Diagnostics | OC | This field holds a more detailed reason for the release of the MBMS bearer, and complements the "Change Condition" information. |
| IMS Information | OC | A set of fields hold the MBMS Bearer Service specific parameters within the scope of the present document. The details are defined in TS 32.260 [20]. |
| SDP Session Description | OC | Used as defined in TS 32.260 [20]. |
| SDP Media Components | OC | Used as defined in TS 32.260 [20]. |
| MBMS Information | OM | A set of fields hold the MBMS specific parameters. The details are defined in clause 6.3.1.2. |
| NOTE: The PDP/PDN Type and Served PDP/PDN Address represent the MBMS Bearer Service, i.e. IP multicast address. |  |  |

#### 6.3.1.2	Definition of the MBMS Information

MBMS specific charging information is provided within the MBMS Information. 
The detailed structure of the MBMS Information can be found in table 6.3.1.2.1.

Table 6.3.1.2.1: Structure of the MBMS Information

| Information Element | Category | Description |
| --- | --- | --- |
| TMGI | OM | Used as defined in TS 29.061 [204]. |
| MBMS Service Type | OM | Used as defined in TS 29.061 [204]. |
| MBMS User Service Type | Oc | This IE indicates type of service the MBMS user service that is being delivered. Only available in the BM-SC. |
| File Repair Supported | OC | This IE indicates whether the MBMS user service supports point-to-point file repair. Only available in the BM-SC. |
| Required MBMS Bearer Capabilities | OC | Used as defined in TS 29.061 [204]. |
| MBMS 2G 3G Indicator | OC | Used as defined in TS 29.061 [204]. |
| RAI | OC | Used as defined in TS 29.061 [204]. Only available in the BM-SC. |
| MBMS Service Area | OC | Used as defined in TS 29.061 [204]. |
| MBMS Session Identity | OC | Used as defined in TS 29.061 [204]. |
| MBMS GW Address | OC | This IE holds the IP address of the MBMS GW that generated the Charging Id when MBMS GW is stand-alone. |
| CN IP Multicast Distribution | Oc | Used as defined in TS 29.061 [204]. |
| MBMS Access Indicator | Oc | Used as defined in TS 29.061 [204]. |
| MBMS Charged Party | Oc | This IE indicates whether the content provider or receiving subscriber is being charged. Only available in the BM-SC for offline charging. |
| Recipient Address | Oc | This IE indicates the MSISDN of a recipient registered to receive the bearer service. This field is repeated for each recipient. This field is applicable to Multicast Service but is not applicable to Broadcast Service. |
| MBMS Data Transfer Start | Oc | This IE contains the absolute time stamp of the data delivery start. |
| MBMS Data Transfer Stop | Oc | This IE contains the absolute time stamp of the data delivery stop. |

### 6.3.2	Formal parameter description

#### 6.3.2.1	MBMS charging information for CDRs

The detailed definitions, abstract syntax and encoding of the MBMS CDR parameters are specified in TS 32.298 [51].

#### 6.3.2.2	MBMS charging information for charging events

The detailed charging event parameter definitions are specified in TS 32.299 [50].

## 6.4	Bindings for MBMS offline charging

This clause aims to describe the mapping between the Diameter messages AVP and CDR parameters for MBMS offline charging.

Table 6.4.1: Bindings of Accounting AVP to BMSC-CDR parameter

| S-/C-BMSC-CDR parameter | Information Element | AVP |
| --- | --- | --- |
|  | Service Information | Service-Information |
| Served IMSI | Subscriber Identifier | Subscription-Id |
| Served MSISDN | Subscriber Identifier | Subscription-Id |
| Content Provider Id | Subscriber Identifier | Subscription-Id |
|  | PS Information | PS-Information |
| Node Id | Node Id | Node-Id |
| PDP/PDN Type | PDP/PDN Type | 3GPP-PDP-Type |
| Served PDP/PDN Address | Served PDP/PDN Address | PDP-Address |
| GGSN Address Used | GGSN Address | GGSN-Address |
| List of Downstream Nodes | GGSN Address | GGSN-Address |
| Access Point Name Network Identifier | Called Station Id | Called-Station-Id |
| List of Traffic Data Volumes | Traffic Data Volumes | Traffic-Data-Volumes |
| Cause for Record Closing | Change Condition | Change-Condition |
| Diagnostics | Diagnostics | Diagnostics |
|  | IMS Information | IMS-Information |
| Bearer Service Description | SDP Session Description | SDP-Session-Description |
|  | SDP Media Components | SDP-Media-Component |
| MBMS Information | MBMS Information | MBMS-Information |
| TMGI | TMGI | TMGI |
| MBMS Service Type | MBMS Service Type | MBMS-Service-Type |
| File Repair Supported | File Repair Supported | File-Repair-Supported |
| Required MBMS Bearer Capabilities | Required MBMS Bearer Capabilities | Required-MBMS-Bearer-Capabilities |
| MBMS 2G 3G Indicator | MBMS 2G 3G Indicator | MBMS-2G-3G-Indicator |
| RAI | RAI | RAI |
| MBMS Service Area | MBMS Service Area | MBMS-Service-Area |
| MBMS Session Identity | MBMS Session Identity | MBMS-Session-Identity |
| MBMS GW Address | MBMS GW Address | MBMS-GW-Address |
| CN IP Multicast Distribution | CN IP Multicast Distribution | CN-IP-Multicast-Distribution |
| MBMS Access Indicator | MBMS Access Indicator | MBMS-Access-Indicator |
| Record Type | MBMS Charged Party | MBMS-Charged-Party |
| Recipient Address List | Recipient Address | MSISDN |
| MBMS Data Transfer Start | MBMS Data Transfer Start | MBMS-Data-Transfer-Start |
| MBMS Data Transfer Stop | MBMS Data Transfer Stop | MBMS-Data-Transfer-Stop |

NOTE:	The whole set of S-/C-BMSC-CDR parameters is described in the TS 32.298 [51]. The following fields are generated at the CDF: Record Opening Time, Duration, Record Sequence Number, Local Record Sequence Number.

Annex A (informative):
Bibliography

This Annex is a placeholder for documents which are not explicitly cited in this specification.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Mar 2005 | SA_27 | SP-050034 | -- | -- | Submitted to TSG SA#27 for Information | -- | 1.0.0 |  |
| Jun 2005 | SA_28 | SP-050280 | -- | -- | Submitted to TSG SA#28 for Approval | -- | 2.0.0 | 6.0.0 |
| Sep 2005 | SA_29 | SP-050439 | 0001 | -- | Corrections on MBMS offline charging trigger conditions | F | 6.0.0 | 6.1.0 |
| Dec 2005 | SA_30 | SP-050703 | 0002 | -- | Align with common Diameter handling principles in 32.299 | F | 6.1.0 | 6.2.0 |
| Dec 2005 | SA_30 | SP-050703 | 0003 | -- | Correction of PS information usage in MBMS - Align with 29.061 and 23.060 | F | 6.1.0 | 6.2.0 |
| Mar 2006 | SA_31 | SP-060083 | 0004 | -- | Correction to MBMS behaviour as a result of OCS controlled service termination | F | 6.2.0 | 6.3.0 |
| Jun 2006 | SA_32 | SP-060242 | 0005 | -- | Correction of the BMSC-CDR charging information collection description | F | 6.3.0 | 6.4.0 |
| Jun 2006 | SA_32 | SP-060242 | 0006 | -- | Align BM-SC initiated MBMS De-registration procedure with 23.246 | F | 6.3.0 | 6.4.0 |
| Jun 2006 | SA_32 | SP-060417 | 0007 | 1 | Correct List of Traffic Volume in MBMS charging | F | 6.3.0 | 6.4.0 |
| Sep 2006 | SA_33 | SP-060522 | 0008 | -- | Resolve outstanding "Editor's notes" in MBMS charging | F | 6.4.0 | 6.5.0 |
| Dec 2006 | SA_34 | SP-060704 | 0009 | - | Correction on MBMS Information - Align with 23.246 | F | 6.5.0 | 6.6.0 |
| Dec 2006 | SA_34 | SP-060718 | 0010 | -- | Add an identifier for the service initiating offline Diameter Accounting | B | 6.6.0 | 7.0.0 |
| Dec 2006 | SA_34 | SP-060718 | 0011 | -- | Add an identifier for the served user in offline Diameter Accounting | B | 6.6.0 | 7.0.0 |
| Dec 2008 | SA_42 | SP-080841 | 0012 | - | Correction on Multiple Service Indication category | F | 7.0.0 | 7.1.0 |
| Dec 2008 | SA_42 | -- | -- | -- | Upgrade to Release 8 | -- | 7.1.0 | 8.0.0 |
| Jun 2009 | SA_44 | SP-090296 | 0013 | -- | MBMS charging in EPS alignment in CDR description | B | 8.0.0 | 9.0.0 |
| Jun 2009 | SA_44 | SP-090296 | 0014 | -- | MBMS charging in EPS alignment in Abbreviations and charging principle | B | 8.0.0 | 9.0.0 |
| Jun 2009 | SA_44 | SP-090296 | 0015 | -- | Add message flow for UTRAN access in eMBMS charging | B | 8.0.0 | 9.0.0 |
| Sep 2009 | SA-45 | SP-090536 | 0017 | -- | Addition of multicast delivery related contents in MBMS charging | A | 9.0.0 | 9.1.0 |
| Dec 2009 | SA-46 | SP-090724 | 0018 | -- | Add session update procedure for EPS | B | 9.1.0 | 9.2.0 |
| Dec 2009 | SA-46 | SP-090724 | 0019 | -- | Clean up of session start and session stop procedures for EPS | D | 9.1.0 | 9.2.0 |
| Dec 2009 | SA-46 | SP-090724 | 0020 | -- | Add MBMS access indicator | B | 9.1.0 | 9.2.0 |
| Dec 2009 | SA-46 | SP-090724 | 0021 | -- | Add new triggers for C-MBSC-CDR closure for EPS | B | 9.1.0 | 9.2.0 |
| Mar 2011 | - | - | - | - | Update to Rel-10 version (MCC) | - | 9.2.0 | 10.0.0 |
| 2012-09 | - | - | - | - | Update to Rel-11 version (MCC) | - | 10.0.0 | 11.0.0 |
| 2013-12 | SA-62 | SP-130676 | 0025 | - | Correction for use of Destination-Host AVP in ACR | A | 11.0.0 | 11.1.0 |
| 2014-07 | - | - | - | - | Rapporteur/MCC: General editorial changes and clean-up. | - | 11.1.0 | 11.1.1 |
| 2014-09 | SA-65 | SP-140569 | 0030 | 1 | Corrections for MBMS offline charging using Diameter | A | 11.1.0 | 11.2.0 |
|  |  | SP-140564 | 0027 | 1 | Corrections for alignment between charging specifications | F | 11.2.0 | 12.0.0 |
| 2014-12 | SA-66 | SP-140804 | 0032 | - | Additional corrections for removal of I-WLAN solution | F | 12.0.0 | 12.1.0 |
|  |  | SP-140805 | 0033 | - | Corrections on definition for parameter category | F |  |  |
|  |  | SP-140806 | 0031 | 2 | Clarification for Recipient Address List field in BMSC-CDR | F | 12.1.0 | 13.0.0 |
| 2015-03 | SA-67 | SP-150066 | 0036 | - | Correction for charging based on MBMS Data Transfer Time | A | 13.0.0 | 13.1.0 |
| 2015-09 | SA-69 | SP-150424 | 0038 | 1 | Correction on use of GCS AS as Content Provider | A | 13.1.0 | 13.2.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | Tdoc | CR | Rev | Cat | Subject/Comment | New version |
| 2016-12 |  |  |  |  |  | Correction of LTE logo (MCC) | 13.2.1 |
| 2017-03 | SA#75 | SP-170138 | 0040 | 1 | D | Remove reference to RFC 3588 | 14.0.0 |
| 2018-06 | - | - | - | - | - | Update to Rel-15 version (MCC) | 15.0.0 |