3GPP TS 32.666 V15.1.0 (2019-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Configuration Management (CM);

Kernel CM Integration Reference Point (IRP);

Solution Set (SS) definitions

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

GSM, UMTS, Configuration Management,

CORBA, XML, SOAP

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	7

4	Solution Set definitions	7

Annex A (normative):	 CORBA Solution Set	8

A.1	Architectural Features	8

A.1.1	Syntax for Distinguished Names and Versions	8

A.1.2 	Notifications	8

A.1.3	Filter language	8

A.2	Mapping	8

A.2.1	Operation and Notification mapping	8

A.2.2	Operation parameter mapping	8

A.2.3	Notification attribute mapping	9

A.3	Use of OMG Structured Event	11

A.4	Rules for NRM extensions	15

A.4.1	Extensions not allowed	15

A.5	Solution Set definitions	15

A.5.1	IDL definition structure	15

A.5.2	IDL specification “KernelCmConstDefs.idl”	16

A.5.3	IDL specification “KernelCmIRPSystem.idl”	18

A.5.4	IDL specification “KernelCmIRPNotifications.idl”	19

Annex B (normative):	 XML definitions	22

B.1	Architectural features	22

B.1.1	Syntax for Distinguished Names	22

B.2	Mapping	22

B.3	Solution Set definitions	22

B.3.1	XML definition structure	22

B.3.2	Graphical Representation	22

B.3.3	XML Schema “kernelNtf.xsd”	25

Annex C (normative):	SOAP Solution Set	28

C.1	Architectural Features	28

C.1.1	Syntax for Distinguished Names and versions	28

C.1.2	Notifications	28

C.1.3	IRP document version number string	28

C.2	Mapping	29

C.2.1	General mappings	29

C.2.2	Operation and Notification mapping	29

C.2.3	Operation parameter mapping	30

C.2.3.1	Operation getNRMIRPVersion	30

C.2.3.1.1	Input parameters	30

C.2.3.1.2	Output parameters	30

C.2.2.1.3	Fault definition	30

C.3	Solution Set definitions	31

C.3.1	WSDL definition structure	31

C.3.2	Graphical Representation	31

C.3.3	WSDL specification “KernelCMIRPSystem.wsdl”	32

Annex D (informative):	 Change history	35

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; as identified below:

32.661:	Configuration Management (CM); Kernel CM Requirements.

32.662:	Configuration Management (CM); Kernel CM Information Service (IS).

32.666:	Configuration Management (CM); Kernel CM Integration Reference Point (IRP); Solution Set (SS) definitions.

Configuration Management (CM), in general, provides the operator with the ability to assure correct and effective operation of the 3G network as it evolves. CM actions have the objective to control and monitor the actual configuration on the Network Elements (NEs) and network resources , and they may be initiated by the operator or by functions in the Operations Systems (OSs) or NEs.

CM actions may be requested as part of an implementation programme (e.g. additions and deletions), as part of an optimisation programme (e.g. modifications), and to maintain the overall Quality of Service (QoS). The CM actions are initiated either as single actions on single NEs of the 3G network, or as part of a complex procedure involving actions on many resources/objects in one or several NEs.

# 1	Scope

The present document specifies the Solution Set definitions for the IRP whose semantics is specified in Kernel CM (Configuration Management) IRP: Information Service 3GPP TS 32.662 [7].

This Solution Set definitions specification is related to 3GPP TS 32.662  [7].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[4]	3GPP TS 32.662: "Telecommunication management; Configuration Management (CM); Kernel CM: Information Service (IS)".

[5]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[6]	Object Management Group 98 (November 1998): "Notification Service: Joint Revised Submission OMG TC Document telecom/98-11-01".

[7]	OMG CORBA Services (November 1996): "Common Object Services Specification".

[8]	Void

[9]	3GPP TS 32.306: "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP): Solution Set (SS) definitions".

[10]	Void

[11]	Void

[12]	3GPP TS 32.676: "Telecommunication management; Configuration Management (CM); State Management Integration Reference Point (IRP): Solution Set (SS) definitions".

[13]	3GPP TS 32.311: "Telecommunication management; Generic Integration Reference Point (IRP) management: Requirements".

[14]	3GPP TS 32.336: "Telecommunication management; Notification Log (NL) Integration Reference Point (IRP): Solution Set (SS) definitions".

[15]	3GPP TS 32.331: "Telecommunication management; Notification Log (NL) Integration Reference Point (IRP): Requirements".

[16]	Void

[17]	Void

[18]	W3C SOAP 1.1 specification (http://www.w3.org/TR/2000/NOTE-SOAP-20000508/)

[19]	Void

[20]	Void

[21]	WS-I Basic Profile Version 1.1 (http://www.ws-i.org/Profiles/BasicProfile-1.1-2004-08-24.html)

[22]	W3C SOAP 1.2 specification (http://www.w3.org/TR/soap12-part1/).

[23]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[24]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [24], 3GPP TS 32.101 [1], 3GPP TS 32.102 [2], 3GPP TS 32.600 [3], 3GPP TS 32.662 [4] and 3GPP TS 32.331[15], 3GPP TS 28.622 [23] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [24], 3GPP TS 32.101 [1], 3GPP TS 32.102 [2], 3GPP TS 32.600 [3], 3GPP TS 32.662 [4] and 3GPP TS 32.331[15], 3GPP TS 28.622 [23].

IRP document version number string (or "IRPVersion"): See 3GPP TS 32.311 [13].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [24] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [24]

DN	Distinguished Name

MO	Managed Object

MOC	Managed Object Class

NL	Notification Log

SS	Solution Set

VSE	Vendor Specific Extensions

# 4	Solution Set definitions

This specification defines the following 3GPP Kernel CM IRP Solution Set definitions:

-	3GPP Kernel CM IRP CORBA SS (Annex A);

-	3GPP Kernel CM IRP XML definitions (Annex B);

-	3GPP Kernel CM IRP SOAP Solution Set (Annex C).

Annex A (normative): 
CORBA Solution Set

This annex contains the CORBA Solution Set for the IRP whose semantics is specified in Kernel CM IRP: Information Service (TS 32.662 [4]).

# A.1	Architectural Features

The overall architectural feature of Kernel Configuration Management IRP is specified in 3GPP TS 32.662 [4].

This clause specifies features that are specific to the CORBA SS.

## A.1.1	Syntax for Distinguished Names and Versions

The syntax of a Distinguished Name is defined in 3GPP TS 32.300 [5].

The version of this IRP is represented as a string (see also clause 3.1).

## A.1.2 	Notifications

Notifications are sent according to the Notification IRP: CORBA SS (see 3GPP TS 32.303 [9]).

The contents of the Kernel CM IRP notifications are defined in the present document.

## A.1.3	Filter language

The filter language used in the SS is the Extended Trader Constraint Language (see OMG Notification Service [6]). IRPAgents may throw a FilterComplexityLimit exception when a given filter is too complex.

# A.2	Mapping

## A.2.1	Operation and Notification mapping

The Kernel CM IRP: IS (see 3GPP TS 32.662 [4]) defines semantics of operation and notification visible across the Kernel Configuration Management IRP. The following table in this subclause indicates mapping of these operations and notifications to their equivalents defined in this SS.

Table A.2.1: Mapping from IS Notification/Operation to SS equivalents

| IS Operation/ notification (3GPP TS 32.662 [4]) | SS Method | Qualifier |
| --- | --- | --- |
| getNRMIRPVersion | get_nrm_irp_version | M |
| notifyObjectCreation | See Notification IRP: CORBA SS [9] | O |
| notifyObjectDeletion | See Notification IRP: CORBA SS [9] | O |
| notifyAttributeValueChange | See Notification IRP: CORBA SS [9] | O |
| notifyStateChange | See Notification IRP: CORBA SS [9] | O |
| getIRPVersion | get_kernel_cm_irp_versions | M |
| getOperationProfile | get_kernel_cm_irp operations_profile | O |
| getNotificationProfile | get_kernel_cm_irp_notification_profile | O |
| notifyCMSynchronizationRecommended | See Notification IRP: CORBA SS [9] | O |

## A.2.2	Operation parameter mapping

The Kernel CM IRP: IS (see 3GPP TS 32.662 [4]) defines semantics of parameters carried in operations across the Kernel Configuration Management IRP. The following tables in this subclause indicate the mapping of these parameters, as per operation, to their equivalents defined in this SS.

Table A.2.2.1: Mapping from IS getNRMIRPVersion parameters to SS equivalents

| IS Operation parameter | SS Method parameter | Qualifier |
| --- | --- | --- |
| versionNumberList | ManagedGenericIRPConstDefs::VersionNumberSet version_number_list | M |
| vSEVersionNumberList | ManagedGenericIRPConstDefs::VersionNumberSet vse_version_number_list | M |
| status | Exceptions: GetNRMIRPVersion | M |

Table A.2.2.2: Mapping from IS getIRPVersion parameters to SS equivalents

| IS Operation parameter | SS Method parameter | Qualifier |
| --- | --- | --- |
| versionNumberList | Return value of type ManagedGenericIRPConstDefs::VersionNumberSet | M |
| status | exception GetKernelCmIRPVersionsException | M |

Table A.2.2.3: Mapping from IS getOperationProfile parameters to SS equivalents

| IS Operation parameter | SS Method parameter | Qualifier |
| --- | --- | --- |
| irpVersion | ManagedGenericIRPConstDefs::VersionNumber kernel_cm_irp_version | M |
| operationNameProfile, operationParameterProfile | Return value of type ManagedGenericIRPConstDefs::MethodList | M |
| status | Exceptions: GetKernelCMIRPOperationsProfileException, ManagedGenericIRPSystem::OperationNotSupported, ManagedGenericIRPSystem::InvalidParameter | M |

Table A.2.2.4: Mapping from IS getNotificationProfile parameters to SS equivalents

| IS Operation parameter | SS Method parameter | Qualifier |
| --- | --- | --- |
| irpVersion | ManagedGenericIRPConstDefs::VersionNumber kernel_cm_irp_version | M |
| notificationNameProfile, notificationParameterProfile | Return value of type ManagedGenericIRPConstDefs::MethodList | M |
| status | Exceptions: GetKernelCMIRPNotificationProfileException, ManagedGenericIRPSystem::OperationNotSupported, ManagedGenericIRPSystem::InvalidParameter | M |

## A.2.3	Notification attribute mapping

The Kernel CM IRP: IS (see 3GPP TS 32.662 [4]) identifies and defines the semantics of attributes for notifyObjectCreation, notifyObjectDeletion, notifyAttributeValueChange, notifyStateChange and notifyCMSynchronizationRecommended for use for its IRP. The following table in this subclause shows the mapping of the IS notifications to SS equivalents.

Table A.2.3.1: Mapping from IS notifications to SS equivalents

| IS notifications in 3GPP TS 32.662 [4] | SS notifications | Qualifier |
| --- | --- | --- |
| NotifyObjectCreation | push_structured_event | O |
| NotifyObjectDeletion | push_structured_event | O |
| NotifyAttributeValueChange | push_structured_event | O |
| NotifyStateChange | push_structured_event | O |
| NotifyCMSynchronizationRecommended | push_structured_event | O |

The Kernel CM IRP: IS (see 3GPP TS 32.662 [4]) also qualifies the attributes. The following tables in this subclause show the mapping of these IS attributes to SS equivalents.

Table A.2.3.2: Mapping from IS Notification Header attributes to SS equivalent

| IS Attribute of Notification Header in 3GPP TS 32.662 [4] | SS Attribute | Qualifier |
| --- | --- | --- |
| objectClass,objectInstance | NotificationIRPConstDefs::AttributeNameValue:: MANAGED_OBJECT_INSTANCE | M |
| notificationId | NotificationIRPConstDefs::AttributeNameValue:: NOTIFICATION_ID | M |
| eventTime | NotificationIRPConstDefs::AttributeNameValue:: EVENT_TIME | M |
| systemDN | NotificationIRPConstDefs::AttributeNameValue:: SYSTEM_DN | O |
| notificationType | -- | M |

Table A.2.3.3: Mapping from IS notifyObjectCreation attributes
to SS equivalent OBJECT_CREATION

| IS Attribute of notifyObjectCreation in 3GPP TS 32.662 [4] | SS Attribute | Qualifier |
| --- | --- | --- |
| notificationHeader | See table A.2.3.2 | M |
| correlatedNotifications | KernelCmNotifications::MOCreation::CORRELATED_NOTIFICATIONS | O |
| additionalText | KernelCmNotifications::MOCreation::ADDITIONAL_TEXT | O |
| sourceIndicator | KernelCmNotifications::MOCreation::SOURCE_INDICATOR | O |
| attributeList | KernelCmNotifications::MOCreation::InitialAttributeValues (contained in remainder_of_body) | O |

Table A.2.3.4: Mapping from IS notifyObjectDeletion attributes
to SS equivalent OBJECT_DELETION

| IS Attribute of notifyObjectDeletion in 3GPP TS 32.662 [4] | SS Attribute | Qualifier |
| --- | --- | --- |
| notificationHeader | See table A.2.3.2 | M |
| correlatedNotifications | KernelCmNotifications::MODeletion::CORRELATED_NOTIFICATIONS | O |
| additionalText | KernelCmNotifications::MODeletion::ADDITIONAL_ TEXT | O |
| sourceIndicator | KernelCmNotifications::MODeletion::SOURCE_INDICATOR | O |
| attributeList | KernelCmNotifications::MODeletion::AttributeValues (contained in remainder_of_body) | O |

Table A.2.3.5: Mapping from IS notifyAttributeValueChange attributes
to SS equivalent ATTRIBUTE_VALUE_CHANGE

| IS Attribute of notifyAttributeValue Change in 3GPP TS 32.662 [4] | SS Attribute | Qualifier |
| --- | --- | --- |
| notificationHeader | See table A.2.3.2 | M |
| correlatedNotifications | KernelCmNotifications::AttributeValueChange::CORRELATED_NOTIFICATIONS | O |
| additionalText | KernelCmNotifications::AttributeValueChange::ADDITIONAL_TEXT | O |
| sourceIndicator | KernelCmNotifications::AttributeValueChange::SOURCE_INDICATOR | O |
| attributeValueChangeDefinition | KernelCmNotifications::AttributeValueChange::ModifiedAttributeSet (contained in remainder_of_body) | M |

Table A.2.3.6: Mapping from IS notifyCMSynchronizationRecommended attributes
to SS equivalent REQUEST_CM_SYNCHRONIZATION

| IS Attribute of notifyCMSynchronization Recommended in 3GPP TS 32.662 [4] | SS Attribute | Qualifier |
| --- | --- | --- |
| notificationHeader | See table A.2.3.2 | M |
| baseMOClass | KernelCmNotifications::CMSynchronizationRecommended::BASE_MO_CLASS | M |
| baseMOInstance | KernelCmNotifications::CMSynchronizationRecommended::BASE_MO_INSTANCE | M |
| scope | KernelCmNotifications::CMSynchronizationRecommended::SCOPE | M |
| additionalText | KernelCmNotifications::CMSynchronizationRecommended::ADDITIONAL_TEXT | O |

Table A.2.3.7: Mapping from IS notifyStateChange attributes
to SS equivalent STATE_CHANGE

| IS Attribute of notifyStateChange Change in 3GPP TS 32.662 [4] | SS Attribute | Qualifier |
| --- | --- | --- |
| notificationHeader | See table A.2.3.2 | M |
| stateChange | StateManagementIRPConstDefs::AttributeNameValue (see note) | M |
| correlatedNotifications | KernelCmNotifications::StateChange::CORRELATED_NOTIFICATIONS | O |
| additionalText | KernelCmNotifications::StateChange::ADDITIONAL_TEXT | O |
| sourceIndicator | KernelCmNotifications::StateChange::SOURCE_INDICATOR | O |
| NOTE: The stateChange attribute is mapped into name-value pairs that contain the state identifier in the name and the new and optional old state values in the attribute field (See TS 32.676 [12] StateManagementIRPConstDefs IDL <State Name>OldNewValue structures). |  |  |

# A.3	Use of OMG Structured Event

In CORBA SS, OMG defined StructuredEvent (see OMG Notification Service [6]) is used to carry notifications. This clause identifies the OMG defined StructuredEvent attributes that carry the attributes of notifications defined in 3GPP TS 32.662 [4].

The composition of OMG Structured Event, as defined in OMG Notification Service [6], is:

Header

Fixed Header

domain_name

type_name

event_name

Variable Header

Body

filterable_body_fields

remainder_of_body

The following table in this clause lists all OMG Structured Event attributes in its leftmost column. The second column identifies the SS attributes, if any, that shall be carried there.

Attributes that are denoted as "optional" may be absent from the OMG Structured Event. As an example, if the optional additionalText attribute is not used for a particular notification, then the IRPAgent may exclude additionalText from the filterable body fields for that particular notification. Individual notifications from the same IRPAgent may include or exclude the same optional attribute.

TableA.3.1: Use of OMG Structured Event

| IS Parameters | OMG CORBA Structured Event attribute | Comment |
| --- | --- | --- |
| There is no corresponding IS parameter | domain_name | It contains the supported SS document version. This version is defined by the string constant KernelCmIRPSystem::VERSION defined in this specification. |
| notificationType | type_name | It is an attribute of notificationHeader. It shall indicate one of the following: Object Creation, Object Deletion, Attribute Value Change, State Change and CM Synchronization Recommended. It is a string. Its value is either defined by KernelCmNotifications::MOCreation::EVENT_TYPE, KernelCmNotifications::MODeletion::EVENT_TYPE, KernelCmNotifications::AttributeValueChange::EVENT_TYPE, KernelCmNotifications::StateChange::EVENT_TYPE or KernelCmNotifications::CMSynchronizationRecommended::EVENT_TYPE |
| There is no corresponding IS parameter | event_name | It shall be set to an empty string. |
| There is no corresponding IS parameter | variable Header |  |
| objectClass, objectInstance | One NV pair of filterable_body_fields | NV stands for name-value pair. Order arrangement of NV pairs is not significant. The name of NV-pair is always encoded in string. Name of this NV pair is the MANAGED_OBJECT_INSTANCE of interface AttributeNameValue of module NotificationIRPConstDefs. Value of NV pair is a string. See corresponding table in Notification IRP: CORBA SS (3GPP TS 32.306 [9]). |
| notificationId | One NV pair of remainder_of_body | Name of NV pair is the NOTIFICATION_ID of interface AttributeNameValue of module NotificationIRPConstDefs. Value of NV pair is a long. See corresponding table in Notification IRP: CORBA SS (3GPP TS 32.306 [9]). |
| eventTime | One NV pair of filterable_body_fields | Name of NV pair is the EVENT_TIME of interface AttributeNameValue of module NotificationIRPConstDefs. Value of NV pair is IRPTime. See corresponding table in Notification IRP: CORBA SS (3GPP TS 32.306 [9]). |
| systemDN | One NV pair of filterable_body_fields | Name of NV pair is the SYSTEM_DN of interface AttributeNameValue of module NotificationIRPConstDefs. Value of NV pair is a string. See corresponding table in Notification IRP: CORBA SS [9]. |
| correlatedNotifications | One NV pair of remainder_of_body | It is an attribute of the Object Creation, Object Deletion and Attribute Value Change notifications. Name of NV pair is the KernelCmNotifications::NotificationCommon::CORRELATED_NOTIFICATIONS. Value of NV pair is a NotificationIRPConstDefs::CorrelatedNotificationSet defined in 3GPP TS 32.306 [9]. |
| additionalText | One NV pair of remainder_of_body | It is an attribute of the Object Creation, Object Deletion, Attribute Value Change and CM Synchronization Recommended notifications. Name of NV pair is the KernelCmNotifications::NotificationCommon::ADDITIONAL_TEXT Value of NV pair is a string. |
| sourceIndicator | One NV pair of remainder_of_body | It is an attribute of the Object Creation, Object Deletion and Attribute Value Change notifications. Name of NV pair is the KernelCmConstDefs::AttributeNameValue::SOURCE_INDICATOR. Value of NV pair is the KernelCmNotifications::NotificationCommon::RESOURCE_OPERATION or MANAGEMENT_OPERATION or UNKNOWN_OPERATION. |
| attributeList | remainder_of_non_filterable_body (see 3GPP TS 32.306 [9]) | It is used to transport attribute information. For Object Creation notification, this is defined by KernelCmNotifications::MOCreation::InitialAttributeValues. For Object Deletion notification, this is defined by KernelCmNotifications::MODeletion::AttributeValues. The name component of InitialAttributeValues and AttributeValues will be set to attribute names defined in KernelCmNRMDefs. |
| attributeValueChangeDefinition | remainder_of_non_filterable_body (see 3GPP TS 32.306 [9]) | For Attribute Value Change notification, this is defined by KernelCmNotifications::AttributeValueChange::ModifiedAttributeSet. The name component of ModifiedAttributeSet will be set to attribute name defined in KernelCmNRMDefs. |
| StateManagementIRPConstDefs::AttributeNameValue | A set of up to 9 Name-value pairs See table A.3.2. All these 9 NV pairs are part of the remainder_of_body | For state change notifications a series of up to 9 name-value pairs might be sent corresponding with the new and old values of each state/status attribute which has changed it's value. The new values of each state/status attributes that have changed are sent. The IRP agent may optionally send the old state/status changes. The name of the name-value pairs are defined by StateManagementIRPConstDefs::AttributeNameValue |
| baseMOClass | One NV pair of remainder_of_body | It is an attribute of the CMSynchronizationRecommended notification. Name of NV pair is a string, KernelCmNotifications::CMSynchronizationRecommended::BASE_MO_CLASS. Value of NV pair is a string. This string conveys the semantics of the Managed Object Class. |
| baseMOInstance | One NV pair of remainder_of_body | It is an attribute of the CMSynchronizationRecommended notification. Name of NV pair is the KernelCmNotifications::CMSynchronizationRecommended::BASE_MO_INSTANCE. Value of NV pair is a string. This is the DN string of the Managed Object Instance. |
| scope | One NV pair of remainder_of_body | It is an attribute of the CMSynchronizationRecommended notification. Name of NV pair is the KernelCmNotifications::CMSynchronizationRecommended::SCOPE. Value of NV pair is KernelCmConstDefs::ScopePara. |

Table A.3.2 Name – value pairs for state change notifications

| Name | Value |
| --- | --- |
| OPERATIONAL_STATE | StateManagementIRPConstDefs::OperationalStateOldNewValue |
| USAGE_STATE | StateManagementIRPConstDefs::UsageStateOldNewValue |
| ADMINISTRATIVE_STATE | StateManagementIRPConstDefs::AdministrativeStateOldNewValue |
| ALARM_STATUS | StateManagementIRPConstDefs::AlarmStatusOldNewValue |
| PROCEDURAL_STATUS | StateManagementIRPConstDefs::ProceduralStatusOldNewValue |
| AVAILABILITY_STATUS | StateManagementIRPConstDefs::AvailabilityStatusOldNewValue |
| CONTROL_STATUS | StateManagementIRPConstDefs::ControlStatusOldNewValue |
| STANDBY_STATUS | StateManagementIRPConstDefs::StandbyStatusOldNewValue |
| UNKNOWN_STATUS | StateManagementIRPConstDefs::UnknownStatusOldNewValue |

# A.4	Rules for NRM extensions

This clause discusses how the models and IDL definitions provided in the present document can be extended for a particular implementation and still remain compliant with 3GPP SA5's specifications.

## A.4.1	Extensions not allowed

The IDL specifications in the present document cannot be edited or altered. Any additional IDL specifications shall be specified in separate IDL files.

IDL interfaces (note: not MOCs) specified in the present document may not be subclassed or extended. New interfaces may be defined with vendor-specific methods.

# A.5	Solution Set definitions

## A.5.1	IDL definition structure

Clause A.5.2 defines the constants and types used by the Kernel CM IRP.

Clause A.5.3 defines the operations which are performed by the Kernel CM IRP agent.

Clause A.5.4 defines the notifications which are performed by the Kernel CM IRP agent.

## A.5.2	IDL specification “KernelCmConstDefs.idl”

//File: KernelCmConstDefs.idl

#ifndef _KERNEL_CM_CONST_DEFS_IDL_

#define _KERNEL_CM_CONST_DEFS_IDL_

// This statement must appear after all include statements

#pragma prefix "3gppsa5.org"

module KernelCmConstDefs

{

/**

* Information about one attribute

* - name defines the name of the attribute

* - value defines the value of the attribute

*

*/

struct MOAttribute

{

string name;

any value;

};

/**

* A set of attribute names and values

*/

typedef sequence<MOAttribute> MOAttributeSet;

/**

* ScopeType defines the kind of scope to use in a CM synchronization

* request together with ScopePara.level, in the SCOPE field.

*

* ScopePara.level is always >= 0. If a level is bigger than the

* depth of the tree there will be no exceptions thrown.

* BASE_ONLY: level ignored, just return the base object.

* BASE_NTH_LEVEL: return all subordinate objects that are on "level"

* distance from the base object, where 0 is the base object.

* BASE_SUBTREE: return the base object and all of its subordinates

* down to and including the nth level.

* BASE_ALL: level ignored, return the base object and all of it's

* subordinates.

*/

enum ScopeType

{

BASE_ONLY,

BASE_NTH_LEVEL,

BASE_SUBTREE,

BASE_ALL

};

struct ScopePara

{

ScopeType type;

unsigned long level;

};

/* The format of Distinguished Name (DN) is specified in 3GPP TS 32.300

"Name Conventions for Managed Objects".

*/

typedef string DN;

typedef sequence <long> NotifIdSet;

/*

This holds identifiers of notifications that are correlated.

*/

struct CorrelatedNotification

{

DN source;  // Contains DN of MO that emitted the set of notifications

// DN string format in compliance with Name Convention for

// Managed Object.

// This may be a zero-length string. In this case, the MO

// is identified by the value of the MOI attribute

// of the Structured Event, i.e., the notification.

NotifIdSet notif_id_set;  // Set of related notification ids

};

/*

Correlated Notification sets are sets of Correlated Notification

structures.

*/

typedef sequence <CorrelatedNotification> CorrelatedNotificationSet;

/*

This block identifies attributes which are included as part of the Kernel

CM IRP. These attribute values should not clash with those defined for the

attributes of notification header (see IDL of Notification IRP).

*/

interface AttributeNameValue

{

const string SOURCE_INDICATOR = "SOURCE";

const string ADDITIONAL_TEXT = "ADD_TEXT";

const string CORRELATED_NOTIFICATIONS = "CORREL_NOTIFS";

const string BASE_MO_CLASS = "BASE_MOC";

const string BASE_MO_INSTANCE = "BASE_MOI";

const string SCOPE = "SCOPE";

};

};

#endif // _KERNEL_CM_CONST_DEFS_IDL_

## A.5.3	IDL specification “KernelCmIRPSystem.idl”

//File: KernelCmIRPSystem.idl

#ifndef _KERNEL_CM_IRP_SYSTEM_IDL_

#define _KERNEL_CM_IRP_SYSTEM_IDL_

#include <ManagedGenericIRPConstDefs.idl>

#include <ManagedGenericIRPSystem.idl>

// This statement must appear after all include statements

#pragma prefix "3gppsa5.org"

module KernelCmIRPSystem

{

exception GetKernelCMIRPNotificationProfileException { string reason; };

exception GetKernelCMIRPOperationsProfileException { string reason; };

exception GetNRMIRPVersion { string reason; };

exception GetKernelCMIRPVersionsException { string reason; };

/**

* The KernelCmIrpOperations interface.

* Supports a number of Resource Model versions.

*/

interface KernelCmIrpOperations

{

/*

Return the list of all supported Kernel CM IRP versions.

*/

ManagedGenericIRPConstDefs::VersionNumberSet get_kernel_cm_irp_versions (

)

raises (GetKernelCMIRPVersionsException);

/**

* Get the version(s) of the interface

*

* @raises GetNRMIRPVersion when the system for some reason

*   can not return the supported versions.

* @returns all supported versions.

*/

void get_nrm_irp_version

(

out ManagedGenericIRPConstDefs::VersionNumberSet version_number_list,

out ManagedGenericIRPConstDefs::VersionNumberSet vse_version_number_list

)

raises (GetNRMIRPVersion);

/*

Return the list of all supported operations and their supported

parameters for a specific KernelCM IRP version.

*/

ManagedGenericIRPConstDefs::MethodList get_kernel_cm_irp_operations_profile (

in ManagedGenericIRPConstDefs::VersionNumber kernel_cm_irp_version

)

raises (GetKernelCMIRPOperationsProfileException,

ManagedGenericIRPSystem::OperationNotSupported,

ManagedGenericIRPSystem::InvalidParameter);

/*

Return the list of all supported notifications and their supported

parameters for a specific KernelCM IRP version.

*/

ManagedGenericIRPConstDefs::MethodList

get_kernel_cm_irp_notification_profile

(

in ManagedGenericIRPConstDefs::VersionNumber kernel_cm_irp_version

)

raises (GetKernelCMIRPNotificationProfileException,

ManagedGenericIRPSystem::OperationNotSupported,

ManagedGenericIRPSystem::InvalidParameter);

};

};

#endif // _KERNEL_CM_IRP_SYSTEM_IDL_

## A.5.4	IDL specification “KernelCmIRPNotifications.idl”

//File: KernelCmNotifications.idl

#ifndef _KERNEL_CM_NOTIFICATIONS_IDL_

#define _KERNEL_CM_NOTIFICATIONS_IDL_

#include <KernelCmConstDefs.idl>

#include <NotificationIRPNotifications.idl>

// This statement must appear after all include statements

#pragma prefix "3gppsa5.org"

module KernelCmNotifications

{

/**

*  This interface defines fields that are common for all

*  notification types.

*  All constants in the scope of this interface will be

*  visible in the interfaces that inherits this.

*  For instance constant

*  NotificationCommon::MANAGED_OBJECT_CLASS

*  can be addressed by MODeletion::MANAGED_OBJECT_CLASS

*/

interface NotificationCommon: NotificationIRPNotifications::Notify

{

/**

*  This constant defines the name of the

*  source indicator property.

*/

const string SOURCE_INDICATOR =

KernelCmConstDefs::AttributeNameValue::SOURCE_INDICATOR;

/**

* Valid values for the SOURCE_INDICATOR

* property

*/

const string RESOURCE_OPERATION = "RESOURCE OPERATION";

const string MANAGEMENT_OPERATION = "MANAGEMENT OPERATION";

const string SON_OPERATION = "SON OPERATION";

const string UNKNOWN_OPERATION = "UNKNOWN";

/**

*  This constant defines the name of the

*  additional text property.

*  The data type for the value of this property

*  is a string.

*/

const string ADDITIONAL_TEXT =

KernelCmConstDefs::AttributeNameValue::ADDITIONAL_TEXT;

/**

*  This constant defines the name of the

*  correlated notifications property.

*  The value part of the property is

*    KernelCmConstDefs::CorrelatedNotificationSet

*/

const string CORRELATED_NOTIFICATIONS =

KernelCmConstDefs::AttributeNameValue::CORRELATED_NOTIFICATIONS;

};

/**

*  Constant definitions for the MO deleted notification

*/

interface MODeletion : NotificationCommon

{

const string EVENT_TYPE = "x7";

/**

* This information mapped into the remainder_of_body

* in the StructuredEvent

*/

typedef KernelCmConstDefs::MOAttributeSet AttributeValues;

};

/**

*  Constant definitions for the MO created notification

*/

interface MOCreation : NotificationCommon

{

const string EVENT_TYPE = "x6";

/**

* This information mapped into the remainder_of_body

* in the StructuredEvent

*/

typedef KernelCmConstDefs::MOAttributeSet InitialAttributeValues;

};

/**

*  Constant definitions for the Attribute Value Change

*  notification

*/

interface AttributeValueChange : NotificationCommon

{

const string EVENT_TYPE = "x8";

/**

* Information about modified attributes for

* one MO instance.

* - name defines the name of the attribute

* - new_value defines the new value of the attribute

* - old_value defines the previous value of the attribute

*    The value is optional, which means that it may contain

*    an empty any (null inserted in the any).

*

*/

struct ModifiedAttribute

{

string name;

any new_value;

any old_value;

};

/**

* This information mapped into the remainder_of_body

* in the StructuredEvent.

*/

typedef sequence<ModifiedAttribute> ModifiedAttributeSet;

};

/**

*  Constant definitions for the CM Synchronization Recommended notification

*/

interface CMSynchronizationRecommended: NotificationIRPNotifications::Notify

{

const string EVENT_TYPE = "x9";

/**

*  This constant defines the name of the

*  additional text property.

*  The data type for the value of this property

*  is a string.

*/

const string ADDITIONAL_TEXT =

KernelCmConstDefs::AttributeNameValue::ADDITIONAL_TEXT;

/**

*  This constant defines the name of the

*  base MO class property.

*  The value part of this property will carry

*  the base MO class name as a string.

*/

const string BASE_MO_CLASS =

KernelCmConstDefs::AttributeNameValue::BASE_MO_CLASS;

/**

*  This constant defines the name of the

*  base MO instance property.

*  The value part of this property will carry

*  the base MO distinguished name as a string.

*/

const string BASE_MO_INSTANCE =

KernelCmConstDefs::AttributeNameValue::BASE_MO_INSTANCE;

/**

*  This constant defines the name of the

*  scope property.

*  The data type for the value of this property

*  is KernelCmConstDefs::ScopePara.

*/

const string SCOPE =

KernelCmConstDefs::AttributeNameValue::SCOPE;

};

/**

*  Constant definitions for the State Change notification

*/

interface StateChange : NotificationCommon

{

const string EVENT_TYPE = "xA";

};

};

#endif // _KERNEL_CM_NOTIFICATIONS_IDL_

Annex B (normative): 
XML definitions

The annex specifies the XML Definitions for the Kernel CM Integration Reference Point (IRP) as it applies to Itf-N, in accordance with Kernel CM IRP IS definitions [4] as well as Notification Log IRP XML Definitions [14].

# B.1	Architectural features

The overall architectural feature of Kernel CM IRP is specified in 3GPP TS 32.662 [4].

This clause specifies features that are specific to the XML Schema definitions.

## B.1.1	Syntax for Distinguished Names

The syntax of a Distinguished Name is defined in 3GPP TS 32.300 [6].

# B.2	Mapping

Not present in the current version of this specification.

# B.3	Solution Set definitions

## B.3.1	XML definition structure

Clause B.3.2 provides a graphical representation of the XML elements.

Clause B.3.3 provides XML definitions of Kernel CM IRP notifications as defined in 3GPP TS 32.662 [4].

## B.3.2	Graphical Representation

## B.3.3	XML Schema “kernelNtf.xsd”

<?xml version="1.0" encoding="UTF-8"?>

<schema xmlns="http://www.w3.org/2001/XMLSchema" xmlns:nk="http://www.3gpp.org/ftp/specs/archive/32_series/32.666#kernelNtf" xmlns:xe="http://www.3gpp.org/ftp/specs/archive/32_series/32.306#notification" xmlns:sm="http://www.3gpp.org/ftp/specs/archive/32_series/32.676#stateManagementIRP" xmlns:xn="http://www.3gpp.org/ftp/specs/archive/32_series/32.626#genericNrm" targetNamespace="http://www.3gpp.org/ftp/specs/archive/32_series/32.666#kernelNtf" elementFormDefault="qualified" attributeFormDefault="unqualified">

<import namespace="http://www.3gpp.org/ftp/specs/archive/32_series/32.306#notification"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/32_series/32.676#stateManagementIRP"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/32_series/32.626#genericNrm"/>

<complexType name="correlatedNotification">

<sequence>

<element name="source" type="xn:dn"/>

<element name="notification-id-set">

<complexType>

<sequence>

<element name="notification-id" type="xe:NotificationId" minOccurs="0" maxOccurs="unbounded"/>

</sequence>

</complexType>

</element>

</sequence>

</complexType>

<simpleType name="SourceIndicatorType">

<restriction base="string">

<enumeration value="Resource_operation"/>

<enumeration value="Management_operation"/>

<enumeration value="SON_operation"/>

<enumeration value="Unknown"/>

</restriction>

</simpleType>

<complexType name="attributeNameValueChangedListType">

<sequence>

<element name="attributeNameValue" maxOccurs="unbounded">

<complexType>

<sequence>

<any namespace="##any" processContents="lax" id="newAttribute"/>

<any namespace="##any" processContents="lax" id="oldAttribute" minOccurs="0"/>

</sequence>

</complexType>

</element>

</sequence>

</complexType>

<complexType name="ScopeType">

<sequence>

<element name="category" type="nk:ScopeCategoryType"/>

<element name="level" type="unsignedLong" minOccurs="0"/>

</sequence>

</complexType>

<simpleType name="ScopeCategoryType">

<restriction base="string">

<enumeration value="BASE_OBJECT_ONLY"/>

<enumeration value="NTH_LEVEL_SUBORDINATES"/>

<enumeration value="BASE_NTH_LEVEL"/>

<enumeration value="BASE_ALL"/>

</restriction>

</simpleType>

<complexType name="StateChangeListType">

<sequence>

<element name="StateChange" maxOccurs="unbounded">

<complexType>

<sequence>

<any namespace="http://www.3gpp.org/ftp/specs/archive/32_series/32.676#stateManagementIRP" processContents="strict" id="newState"/>

<any namespace="http://www.3gpp.org/ftp/specs/archive/32_series/32.676#stateManagementIRP" processContents="strict" id="oldState" minOccurs="0"/>

</sequence>

</complexType>

</element>

</sequence>

</complexType>

<complexType name="AnyNrm">

<sequence>

<any namespace="##any" processContents="lax"/>

</sequence>

</complexType>

<!-- notifyObjectCreation notification definition -->

<complexType name="NotifyObjectCreation">

<complexContent>

<extension base="xe:Notification">

<sequence>

<element name="body">

<complexType>

<sequence>

<element name="correlatedNotification" type="nk:correlatedNotification" minOccurs="0"/>

<element name="additionalText" type="string" minOccurs="0"/>

<element name="sourceIndicator" type="nk:SourceIndicatorType" minOccurs="0"/>

<element name="attributeList" type="nk:AnyNrm" minOccurs="0"/>

</sequence>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

<!-- notifyObjectDeletion notification definition -->

<complexType name="NotifyObjectDeletion">

<complexContent>

<extension base="xe:Notification">

<sequence>

<element name="body">

<complexType>

<sequence>

<element name="correlatedNotification" type="nk:correlatedNotification" minOccurs="0"/>

<element name="additionalText" type="string" minOccurs="0"/>

<element name="sourceIndicator" type="nk:SourceIndicatorType" minOccurs="0"/>

<element name="attributeList" type="nk:AnyNrm" minOccurs="0"/>

</sequence>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

<!-- notifyAttributeValueChange notification definition -->

<complexType name="NotifyAttributeValueChange">

<complexContent>

<extension base="xe:Notification">

<sequence>

<element name="body">

<complexType>

<sequence>

<element name="correlatedNotification" type="nk:correlatedNotification" minOccurs="0"/>

<element name="additionalText" type="string" minOccurs="0"/>

<element name="sourceIndicator" type="nk:SourceIndicatorType" minOccurs="0"/>

<element name="attributeList" type="nk:attributeNameValueChangedListType" minOccurs="0"/>

</sequence>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

<!-- notifyCMSynchronizationRecommended notification definition -->

<complexType name="NotifyCMSynchronizationRecommended">

<complexContent>

<extension base="xe:Notification">

<sequence>

<element name="body">

<complexType>

<sequence>

<!--element name="baseMOClass" type="string"/-->

<element name="baseMOInstance" type="string"/>

<element name="scope" type="nk:ScopeType"/>

<element name="additionalText" type="string" minOccurs="0"/>

</sequence>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

<!-- notifyStateChange notification definition -->

<complexType name="NotifyStateChange">

<complexContent>

<extension base="xe:Notification">

<sequence>

<element name="body">

<complexType>

<sequence>

<element name="stateChangeList" type="nk:StateChangeListType"/>

<element name="correlatedNotification" type="nk:correlatedNotification" minOccurs="0"/>

<element name="additionalText" type="string" minOccurs="0"/>

<element name="sourceIndicator" type="nk:SourceIndicatorType" minOccurs="0"/>

</sequence>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

<element name="NotifyObjectCreation" type="nk:NotifyObjectCreation"/>

<element name="NotifyObjectDeletion" type="nk:NotifyObjectDeletion"/>

<element name="NotifyAttributeValueChange" type="nk:NotifyAttributeValueChange"/>

<element name="NotifyCMSynchronizationRecommended" type="nk:NotifyCMSynchronizationRecommended"/>

<element name="NotifyStateChange" type="nk:NotifyStateChange"/>

</schema>

Annex C (normative):
SOAP Solution Set

The overall architectural feature of Kernel Configuration Management IRP is specified in 3GPP TS 32.662 [4].

This clause specifies features that are specific to the SOAP SS.

The SOAP 1.1 specification [18] and WSDL 1.1 specification [21] are supported.

The SOAP 1.2 specification [22] is supported optionally.

This specification uses "document" style in WSDL file.

This specification uses "literal" encoding style in WSDL file.

This specification uses a number of namespace prefixes throughout that are listed in Table C.1.

Table C.1: Prefixes and Namespaces used in this specification

| PREFIX | NAMESPACE |
| --- | --- |
| http | http://schemas.xmlsoap.org/wsdl/http/ |
| soap | http://schemas.xmlsoap.org/wsdl/soap/ |
| SOAP-ENV | http://schemas.xmlsoap.org/soap/envelope/ |
| SOAP-ENC or soapenc | http://schemas.xmlsoap.org/soap/encoding/ |
| xs or xsd | http://www.w3.org/2001/XMLSchema |
| xsi | http://www.w3.org/2001/XMLSchema-instance |
| kernelCMIRPSystem | http://www.3gpp.org/ftp/specs/archive/32_series/32.666#KernelCMIRPSystem |
| kernelCMIRPData | http://www.3gpp.org/ftp/specs/archive/32_series/32.666#KernelCMIRPData |
| ntfIRPSystem | http://www.3gpp.org/ftp/specs/archive/32_series/32.306#NotificationIRPNtfSystem |
| genericIRPSystem | http://www.3gpp.org/ftp/specs/archive/32_series/32.316#GenericIRPSystem |

# C.1	Architectural Features

## C.1.1	Syntax for Distinguished Names and versions

The syntax of a Distinguished Name is defined in 3GPP TS 32.300 [5].

The version of this IRP is represented as a string (see also clause C.1.3).

## C.1.2	Notifications

Notifications are sent according to the Notification IRP: SOAP SS (see 3GPP TS 32.306 [9]).

The contents of the Kernel CM IRP notifications are defined in KernelCM IRP XML definition in Annex B.

## C.1.3	IRP document version number string

The IRP document version number (sometimes called "IRPVersion" or "SS version number") string is used to identify this specification.  The string is derived using a rule described in 3GPP TS 32.311 [13]. The value of this string is defined by a constant in C.3.3.

This string (or sequence of strings, if more than one version is supported) is returned in getIRPVersion method and is carried in the first field of the notification header of all notifications related to this IRP.

# C.2	Mapping

## C.2.1	General mappings

The IS parameter name managedObjectInstance is mapped into DN.

Attributes modelling associations as defined in the NRM (here also called "reference attributes") are in this SS mapped to attributes. The names of the reference attributes in the NRM are mapped to the corresponding attribute names in the MOC. When the cardinality for an association is 0..1 or 1..1 the datatype for the reference attribute is defined as an MOReference. The value of an MO reference contains the distinguished name of the associated MO. When the cardinality for an association allows more than one referred MO, the reference attribute will be of type MOReferenceSet, which contains a sequence of MO references.

If a reference attribute is changed, an AttributeValueChange notification is emitted.

## C.2.2	Operation and Notification mapping

The Kernel CM IRP: IS (see 3GPP TS 32.662 [4]) defines semantics of operation and notification visible across the Kernel Configuration Management IRP. The following table in this subclause indicates mapping of these operations and notifications to their equivalents defined in this SS.

Table C.2.2: Mapping from IS Notification/Operation to SS equivalents

| IS Operation/ notification (3GPP TS 32.662 [4]) | SS Operation | Qualifier |
| --- | --- | --- |
| getNRMIRPVersion | getNRMIRPVersion | M |
| notifyObjectCreation | ntfIRPSystem:notify (note 2) | O |
| notifyObjectDeletion | ntfIRPSystem:notify (note 2) | O |
| notifyAttributeValueChange | ntfIRPSystem:notify (note 2) | O |
| notifyCMSynchronizationRecommended | ntfIRPSystem:notify (note 2) | O |
| notifyStateChange | ntfIRPSystem:notify (note 2) | O |
| getIRPVersion(see note 1) | getIRPVersion | M |
| getOperationProfile(see note 1) | getOperationProfile | O |
| getNotificationProfile(see note 1) | getNotificationProfile | O |
| NOTE 1: This operation is of IOC ManagedGenericIRP specified in [18]. The IOC KernelCmIRP of [4] inherits from it. |  |  |
| NOTE 2: The IS equivalent is mapped into parameters of ntfIRPSystem::notify. |  |  |

## C.2.3	Operation parameter mapping

The Kernel CM IRP: IS (3GPP TS 32.662 [4]) defines semantics of parameters carried in operations across the Kernel Configuration Management IRP. The tables below show the mapping of these parameters, as per operation, to their equivalents defined in this SS.

### C.2.3.1	Operation getNRMIRPVersion

#### C.2.3.1.1	Input parameters

None.

Here is the XML schema fragment of the getNRMIRPVersion request:

<!-- getNRMIRPVersion Request -->

<element name="getNRMIRPVersion">

</element>

#### C.2.3.1.2	Output parameters

Table C.2.3.1.2: Mapping from IS getNRMIRPVersion output parameters to SS equivalents

| IS Operation parameter | SS Operation parameter | Qualifier |
| --- | --- | --- |
| versionNumberList | kernelCMIRPData:VersionNumberSetType versionNumberSet | M |
| vSEVersionNumberList | kernelCMIRPData:VersionNumberSetType vSEVersionNumberList | M |
| status | kernelCMIRPData:getNRMIRPVersionFault | M |

Here is the XML schema fragment of the getNRMIRPVersion response:

<!-- getNRMIRPVersion Response -->

<element name="getNRMIRPVersionResponse">

<complexType>

<sequence>

<element name="versionNumberList" type="kernelCMIRPData:VersionNumberSetType"/>

<element name="vSEVersionNumberList" type=" kernelCMIRPData:VersionNumberSetType"/>

</sequence>

</complexType>

</element>

#### C.2.2.1.3	Fault definition

<!-- getNRMIRPVersion Fault -->

<element name="getNRMIRPVersionFault">

<complexType>

<choice>

<element name="getNRMIRPVersionFault" type="string"/>

</choice>

</complexType>

</element>

# C.3	Solution Set definitions

## C.3.1	WSDL definition structure

Clause C.3.2 provides a graphical representation of the Kernel CM IRP service.

Clause C.3.3 defines the services which are supported the Kernel CM IRP agent.

## C.3.2	Graphical Representation

The WSDL structure is like Figure C.3.2:

Figure C.3.2:  kernelCM IRP SOAP Solution Set WSDL structure

## C.3.3	WSDL specification “KernelCMIRPSystem.wsdl”

<?xml version="1.0" encoding="UTF-8"?>

<definitions xmlns="http://schemas.xmlsoap.org/wsdl/" xmlns:soap="http://schemas.xmlsoap.org/wsdl/soap/" xmlns:http="http://schemas.xmlsoap.org/wsdl/http/" xmlns:xs="http://www.w3.org/2001/XMLSchema" xmlns:soapenc="http://schemas.xmlsoap.org/soap/encoding/"

xmlns:genericIRPSystem="http://www.3gpp.org/ftp/specs/archive/32_series/32.316#GenericIRPSystem" xmlns:kernelCMIRPSystem="http://www.3gpp.org/ftp/specs/archive/32_series/32.666#KernelCMIRPSystem"

xmlns:kernelCMIRPData="http://www.3gpp.org/ftp/specs/archive/32_series/32.666#KernelCMIRPData"

targetNamespace="http://www.3gpp.org/ftp/specs/archive/32_series/32.666#KernelCMIRPSystem">

<import namespace="http://www.3gpp.org/ftp/specs/archive/32_series/32.316#GenericIRPSystem" />

<types>

<schema targetNamespace="http://www.3gpp.org/ftp/specs/archive/32_series/32.666#KernelCMIRPData" xmlns="http://www.w3.org/2001/XMLSchema">

<!-- getNRMIRPVersion Request -->

<element name="getNRMIRPVersion">

</element>

<!-- getNRMIRPVersion Response -->

<element name="getNRMIRPVersionResponse">

<complexType>

<sequence>

<element name="versionNumberList" type="kernelCMIRPData:VersionNumberSetType"/>

<element name="vSEVersionNumberList" type="kernelCMIRPData:VersionNumberSetType"/>

</sequence>

</complexType>

</element>

<!-- getNRMIRPVersion Fault -->

<element name="getNRMIRPVersionFault">

<complexType>

<choice>

<element name="getNRMIRPVersionFault" type="string"/>

</choice>

</complexType>

</element>

<element name="OperationNotSupportedFault" type="string"/>

<element name="InvalidParameterFault" type="string"/>

<simpleType name="VersionNumberType">

<restriction base="string"/>

</simpleType>

<complexType name="VersionNumberSetType">

<sequence>

<element name="versionNumber" type="kernelCMIRPData:VersionNumberType" maxOccurs="unbounded"/>

</sequence>

</complexType>

<complexType name="ParameterSetType">

<sequence>

<element name="parameterName" type="string" maxOccurs="unbounded"/>

</sequence>

</complexType>

<complexType name="OperationType">

<sequence>

<element name="operationName" type="string"/>

<element name="parameterSet" type="kernelCMIRPData:ParameterSetType"/>

</sequence>

</complexType>

<complexType name="OperationSetType">

<sequence>

<element name="operation" type="kernelCMIRPData:OperationType" maxOccurs="unbounded"/>

</sequence>

</complexType>

<complexType name="NotificationType">

<sequence>

<element name="notificationName" type="string"/>

<element name="parameterSet" type="kernelCMIRPData:ParameterSetType"/>

</sequence>

</complexType>

<complexType name="NotificationSetType">

<sequence>

<element name="notification" type="kernelCMIRPData:NotificationType" maxOccurs="unbounded"/>

</sequence>

</complexType>

</schema>

</types>

<message name="getNRMIRPVersionRequest">

<part name="parameter" element="kernelCMIRPData:getNRMIRPVersion"/>

</message>

<message name="getNRMIRPVersionResponse">

<part name="parameter" element="kernelCMIRPData:getNRMIRPVersionResponse"/>

</message>

<message name="getNRMIRPVersionFault">

<part name="parameter" element="kernelCMIRPData:getNRMIRPVersionFault"/>

</message>

<portType name="KernelCMIRP">

<operation name="getIRPVersion">

<input message="genericIRPSystem:getIRPVersionRequest"/>

<output message="genericIRPSystem:getIRPVersionResponse"/>

<fault name="getIRPVersionFault" message="genericIRPSystem:getIRPVersionFault"/>

</operation>

<operation name="getOperationProfile">

<input message="genericIRPSystem:getOperationProfileRequest"/>

<output message="genericIRPSystem:getOperationProfileResponse"/>

<fault name="getOperationProfileFault" message="genericIRPSystem:getOperationProfileFault"/>

</operation>

<operation name="getNotificationProfile">

<input message="genericIRPSystem:getNotificationProfileRequest"/>

<output message="genericIRPSystem:getNotificationProfileResponse"/>

<fault name="getNotificationProfileFault" message="genericIRPSystem:getNotificationProfileFault"/>

</operation>

<operation name="getNRMIRPVersion">

<input message="kernelCMIRPSystem:getNRMIRPVersionRequest"/>

<output message="kernelCMIRPSystem:getNRMIRPVersionResponse"/>

<fault name="getNRMIRPVersionFault" message="kernelCMIRPSystem:getNRMIRPVersionFault"/>

</operation>

</portType>

<binding name="KernelCMIRP" type="kernelCMIRPSystem:KernelCMIRP">

<soap:binding style="document" transport="http://schemas.xmlsoap.org/soap/http"/>

<operation name="getIRPVersion">

<soap:operation soapAction="http://www.3gpp.org/ftp/specs/archive/32_series/32.666#getIRPVersion"/>

<input>

<soap:body use="literal"/>

</input>

<output>

<soap:body use="literal"/>

</output>

<fault name="getIRPVersionFault">

<soap:fault name="getIRPVersionFault" use="literal"/>

</fault>

</operation>

<operation name="getOperationProfile">

<soap:operation soapAction="http://www.3gpp.org/ftp/specs/archive/32_series/32.666#getOperationProfile"/>

<input>

<soap:body use="literal"/>

</input>

<output>

<soap:body use="literal"/>

</output>

<fault name="getOperationProfileFault">

<soap:fault name="getOperationProfileFault" use="literal"/>

</fault>

</operation>

<operation name="getNotificationProfile">

<soap:operation soapAction="http://www.3gpp.org/ftp/specs/archive/32_series/32.666#getNotificationProfile"/>

<input>

<soap:body use="literal"/>

</input>

<output>

<soap:body use="literal"/>

</output>

<fault name="getNotificationProfileFault">

<soap:fault name="getNotificationProfileFault" use="literal"/>

</fault>

</operation>

<operation name="getNRMIRPVersion">

<soap:operation soapAction="http://www.3gpp.org/ftp/specs/archive/32_series/32.666# getNRMIRPVersion"/>

<input>

<soap:body use="literal"/>

</input>

<output>

<soap:body use="literal"/>

</output>

<fault name="getNRMIRPVersionFault">

<soap:fault name="getNRMIRPVersionFault" use="literal"/>

</fault>

</operation>

</binding>

<service name="KernelCMIRP">

<port name="KernelCMIRP" binding="kernelCMIRPSystem:KernelCMIRP">

<soap:address location="To be defined."/>

</port>

</service>

</definitions>

Annex D (informative): 
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2010-09 | SA#49 | SP-100519 | -- | -- | Presentation to SA for Information and Approval | --- | 1.0.0 |
| 2010-10 | -- | -- | -- | -- | Publication | 1.0.0 | 10.0.0 |
| 2012-09 | SA#57 | - | - | - | Automatic upgrade from previous Release version 10.0.0 | 10.0.0 | 11.0.0 |
| 2014-09 | SA#65 | SP-140559 | 001 | - | Update the link from Solution Set to Information Service due to the end of Release 12 | 11.0.0 | 12.0.0 |
| 2016-01 | - | - | - | - | Update to Rel-13 version (MCC) | 12.0.0 | 13.0.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2016-06 | SA#72 | SP-160407 | 0004 | - | F | Update the link from IRP Solution Set to IRP Information Service | 13.1.0 |
| 2017-04 | SA#75 | - | - | - |  | Promotion to Release 14 without technical change | 14.0.0 |
| 2017-06 | SA#76 | SP-170502 | 0005 | - | F | Update the link from IRP Solution Set to IRP Information Service | 14.1.0 |
| 2018-06 | - | - | - | - | - | Update to Rel-15 version (MCC) | 15.0.0 |
| 2019-09 | SA#85 | SP-190752 | 0006 | - | F | Correction of NR definition to avoid misalignment with RAN2 | 15.1.0 |