3GPP TR 32.857 V15.0.0 (2018-03)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Study on management aspects of selected Internet of Things (IoT)-related features (Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

management,IoT

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2018, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	Use Cases	6

4.1	PM Measurements based on NRSRP, NRSRQ	7

4.1.1	Pre-condition	7

4.1.2	Description	7

4.1.3	Post-condition	7

4.2	eMTC PM Measurements	7

4.2.1	Background	7

4.2.2	Pre-condition	7

4.2.3	Description	8

4.2.4	Post-condition	8

5	Potential Requirements	8

5.1	General Requirements	8

5.2	Potential Requirements for RRC connection related measurements	8

5.3	Potential Requirements for E-RAB Setup related measurements	8

5.4	Potential Requirements for E-RAB Release related measurements	8

5.5	Potential Requirements for E-RAB Modification related measurements	8

5.7	Potential Requirements for E-RAB activity related measurements and E-RAB number	8

5.8	Potential requirements for cell level radio bearer QoS related measurements	9

5.9	Potential requirements for radio resource utilization related measurements	9

5.10	Potential requirements for handover related measurements	9

5.11	Potential requirements for paging related measurements	9

5.12	Potential Requirements for TB related measurements	9

5.13	Potential Requirements for UE-associated logical S1-connection related measurements	9

5.14	Potential requirements for CQI and TA distribution related measurements	9

5.15	Potential requirements for measurements related to measurement report	9

6	Potential solutions	10

6.1	General	10

6.2	 Solution for RRC connection related measurements	10

6.3	Solution for E-RAB setup related measurements	10

6.4	Solution for E-RAB release related measurements	10

6.5	Solution for E-RAB modification related measurements	11

6.6	Solution for E-RAB activity related measurements and E-RAB number	11

6.7	Solution for Cell level radio bearer QoS related measurements	11

6.7.1	Cell PDCP SDU bit-rate measurements	11

6.7.2	Active UEs	11

6.8	Solution for radio resource utilization related measurements	12

6.9	Solution for handover related measurements	12

6.9.1	Intra-RAT handovers related measurements	12

6.9.2	Handover measurements on neighbour cell basis	12

6.9.3	Intra- / Inter-frequency handover related measurements	12

6.9.4	Handover related measurements for DRX / non-DRX	13

6.9.5	Handover triggering measurements	13

6.9.6	Inter-RAT handovers	13

6.10	Solution for paging related measurements	13

6.11	Solution for TB related measurements	13

6.12	Solution for UE-associated logical S1-connection related measurements	14

6.13	Solution for CQI and TA distribution related measurements	14

6.14	Solution for measurements related to measurement report	14

7	Conclusions	14

8	Recommendations	15

Annex A:	Change history	16

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

This document investigates management aspects of LTE IoT related functionalities such as eMTC and NB-IoT RAN Features, to identify functionalities that should be added to the existing 3GPP OAM specifications. The document outlines basic use cases, sets potential requirements and suggests potential solutions.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.425: "Performance measurements Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[3]	3GPP TS 32.40: " Performance Management (PM); Concept and requirements".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

eMTC		enhanced Machine Type Communication

NB-IoT	Narrowband Internet of Things

NRSRP	Narrowband Reference Signal Received Power

NRSRQ	Narrowband Reference Signal Received Quality

# 4	Use Cases

## 4.1	PM Measurements based on NRSRP, NRSRQ

### 4.1.1	Pre-condition

The network operator is interested in receiving via network management the information on cell level distribution of NRSRP, NRSRQ values reported by the UEs that are serviced by the NB-IoT cell.

### 4.1.2	Description

The eNodeB collects the measurement results for NRSRP, NRSRQ, performed by the UEs e.g. by triggering the periodical UE measurement reports towards all of the UEs in the measured E-UTRAN cell. The distribution information is derived from the NRSRP, NRSRQ values similarly to existing RSRP and RSRQ based measurements in TS 32.425 [2] clauses 6.1, 6.2; it is delivered to the management system as PM measurement results.

### 4.1.3	Post-condition

The network operator gets the NRSRP, NRSRQ performance measurements.

## 4.2	eMTC PM Measurements

### 4.2.1	Background

There are reasons why cell level performance measurements applied separately to eMTC devices may significantly differ from same measurements applied to non-eMTC devices in same cell:

-	Different L1 capabilities (e.g. HARQ)

-	Different traffic and activity patterns, combined with different number of eMTC and non-eMTC devices in the cell

In such case, non-differentiated total value of such counters as Successful RRC connection establishments or Total Number of DL/UL TBs, will not be representing any of the two groups.

### 4.2.2	Pre-condition

The network operator may be interested in receiving, via PM measurements mechanism, some of performance measurements separately for eMTC devices. The targeted performance measurements belong to one or more of the categories defined in [2]:

-	RRC connection related measurements.

-	E-RAB related measurements.

-	Handover related measurements.

-	Cell level radio bearer QoS related measurements.

-	Radio resource utilization related measurements.

-	UE-associated logical S1-connection related measurements.

-	Paging related measurements.

-	RF Measurements.

-	TB related measurements.

-	Measurements related to Measurement Report.

### 4.2.3	Description

The eNodeBs are configured by the network management system to collect the targeted performance measurements. The collected information is processed and the results are delivered to the management system as PM measurement results, similarly to existing PM measurements in TS 32.425 [2].

### 4.2.4	Post-condition

The network operator gets the targeted performance measurements separately for eMTC devices.

# 5	Potential Requirements

## 5.1	General Requirements

General Performance Management requirements per TS 32.401 apply.

Note:	The "Non-eMTC" category of devices does not include NB-IoT category, as in the NB-IoT cells, operations of eMTC devices are not possible.

## 5.2	Potential Requirements for RRC connection related measurements

The network management system should be capable of collecting RRC connection related performance measurements separately for eMTC devices and non-eMTC devices.

## 5.3	Potential Requirements for E-RAB Setup related measurements

The network management system should be capable of collecting E-RAB Setup related measurements separately for eMTC device and non-eMTC devices.

## 5.4	Potential Requirements for E-RAB Release related measurements

The network management system should be capable of collecting E-RAB Release related measurements separately for eMTC devices and non-eMTC devices.

## 5.5	Potential Requirements for E-RAB Modification related measurements

The network management system should be capable of collecting E-RAB Modification related measurements separately for eMTC devices and non-eMTC devices.

## 5.7	Potential Requirements for E-RAB activity related measurements and E-RAB number

The network management system should be capable of collecting E-RAB Modification related measurements and E-RAB number separately for eMTC devices and non-eMTC devices.

## 5.8	Potential requirements for cell level radio bearer QoS related measurements

The network management system should be capable of collecting cell level radio bearer QoS related measurements separately for eMTC devices and non-eMTC devices.

## 5.9	Potential requirements for radio resource utilization related measurements

The network management system should be capable of collecting Radio resource utilization related measurements separately for eMTC devices and non-eMTC devices.

## 5.10	Potential requirements for handover related measurements

The network management system should be capable of collecting handover related measurements separately for eMTC devices and non-eMTC devices.

## 5.11	Potential requirements for paging related measurements

The network management system should be capable of collecting paging related measurements separately for eMTC devices and non-eMTC devices.

## 5.12	Potential Requirements for TB related measurements

The network management system should be capable of collecting TB related measurements separately for eMTC devices and non-eMTC devices.

## 5.13	Potential Requirements for UE-associated logical S1-connection related measurements

The network management system should be capable of collecting UE-associated logical S1-connection related measurements separately for eMTC devices and non-eMTC devices.

## 5.14	Potential requirements for CQI and TA distribution related measurements

The network management system should be capable of collecting CQI and TA distribution related measurements separately for eMTC devices and non-eMTC devices.

## 5.15	Potential requirements for measurements related to measurement report

The network management system should be capable of collecting measurements related to measurement report separately for eMTC devices and non-eMTC devices.

# 6	Potential solutions

## 6.1	General

No change recommended to the set of valid values of the granularity period defined in the TS 32.401 [3], clause 5.4.1.4.

The solutions outlined below include that the measurements are collected separately for the eMTC devices. In case when the cell contains both eMTC and non-eMTC devices, the information on non-eMTC devices can be derived from

-	measurements based on all UEs/devices in the cell;

-	measurements based on eMTC devices in the cell.

## 6.2		Solution for RRC connection related measurements

The network management system should be capable of collecting at least the following RRC connection related performance measurements defined in the TS 32.425 [2] separately for eMTC devices:

-	RRC connection establishment;

-	RRC connection re-establishment;

-	RRC connection number;

-	RRC connection setup time;

-	UE context release;

-	Inactivity timer.

## 6.3	Solution for E-RAB setup related measurements

The network management system should be capable of collecting at least the following E-RAB Setup related measurements defined in the TS 32.425 [2], separately for eMTC devices :

-	Number of initial E-RABs attempted to setup;

-	Number of initial E-RABs successfully established;

-	Number of initial E-RABs failed to setup;

-	Number of additional E-RABs attempted to setup;

-	Number of additional E-RABs successfully established;

-	Number of additional E-RABs failed to setup;

-	Mean E-RAB Setup time;

-	Maximum E-RAB Setup time;

-	Number of E-RABs attempted to establish for incoming HOs;

-	Number of E-RABs successfully established for incoming HOs.

## 6.4	Solution for E-RAB release related measurements

The network management system should be capable of collecting at least the following E-RAB Release related measurements defined in the TS 32.425 [2], separately for eMTC devices :

-	Number of E-RABs requested to release initiated by eNodeB/RN per QCI;

-	Number of E-RABs attempted to release per QCI;

-	Number of E-RAB successfully released per QCI;

-	Number of E-RAB failed to release per failure cause;

-	Number of released active E-RABs per QCI.

## 6.5	Solution for E-RAB modification related measurements

The network management system should be capable of collecting at least the following E-RAB Modification related measurements defined in the TS 32.425 [2], separately for eMTC devices :

-	Number of E-RABs attempted to modify the QoS parameter, per QCI;

-	Number of E-RABs successfully modified the QoS parameter, per QCI;

-	Number of E-RABs failed to modify the QoS parameter, per failure cause.

## 6.6	Solution for E-RAB activity related measurements and E-RAB number

The network management system should be capable of collecting at least the following E-RAB activity related measurements and E-RAB number defined in the TS 32.425 [2], separately for eMTC devices:

-	In-session activity time for UE;

-	In-session activity time for E-RABs, per QCI;

-	E-RAB number, per QCI.

## 6.7	Solution for Cell level radio bearer QoS related measurements

### 6.7.1	Cell PDCP SDU bit-rate measurements

The network management system should be capable of collecting at least the following Cell PDCP SDU bit-rate measurements defined in the TS 32.425 [2], separately for eMTC devices :

-	Average DL cell PDCP SDU bit-rate;

-	Average UL cell PDCP SDU bit-rate;

-	Maximum DL cell PDCP SDU bit-rate;

-	Maximum UL cell PDCP SDU bit-rate;

-	Average DL cell control plane PDCP SDU bit-rate;

-	Average UL cell control plane PDCP SDU bit-rate.

### 6.7.2	Active UEs

The network management system should be capable of collecting at least the following Active UE measurements defined in the TS 32.425 [2], separately for eMTC devices:

-	Average number of active UEs on the DL;

-	Average number of active UEs on the UL.

## 6.8	Solution for radio resource utilization related measurements

The network management system should be capable of collecting at least the following Radio resource utilization related measurements defined in the TS 32.425 [2], separately for eMTC devices :

-	DL PRB Usage for traffic;

-	UL PRB Usage for traffic;

-	DL Total PRB Usage;

-	UL Total PRB Usage:

-	RACH Usage measurements, including:

-	Mean number of RACH preambles received;

-	Distribution of RACH preambles sent;

-	Distribution of RACH access delay;

-	Percentage of contentious RACH attempts.

## 6.9	Solution for handover related measurements

### 6.9.1	Intra-RAT handovers related measurements

The network management system should be capable of collecting at least the following Intra-RAT handover related measurements defined in the TS 32.425 [2], separately for eMTC devices :

-	Attempted outgoing intra-eNB handovers per handover cause;

-	Successful outgoing intra-eNB handovers per handover cause;

-	Attempted outgoing inter-eNB handover preparations;

-	Attempted outgoing inter-eNB handover executions per handover cause;

-	Successful outgoing inter-eNB handover executions per handover cause;

-	Handover measurements on neighbour cell basis

### 6.9.2	Handover measurements on neighbour cell basis

The network management system should be capable of collecting at least the following handover related measurements on neighbour cell basis defined in the TS 32.425 [2], separately for eMTC devices:

-	Attempted outgoing handovers per handover cause;

-	Successful outgoing handovers per handover cause;

-	Number of handover failures related with MRO.

### 6.9.3	Intra- / Inter-frequency handover related measurements

The network management system should be capable of collecting at least the following Intra- / Inter-frequency Handover related measurements defined in the TS 32.425 [2], separately for eMTC devices :

-	Attempted outgoing intra-frequency handovers;

-	Successful outgoing intra-frequency handovers;

-	Attempted outgoing inter-frequency handovers – gap-assisted measurement;

-	Successful outgoing inter-frequency handovers – gap-assisted measurement;

-	Attempted outgoing inter-frequency handovers – non gap-assisted measurement;

-	Successful outgoing inter-frequency handovers – non gap-assisted measurement.

### 6.9.4	Handover related measurements for DRX / non-DRX

The network management system should be capable of collecting at least the following Handover related measurements for DRX / non-DRX  defined in the TS 32.425 [2], separately for eMTC devices:

-	Attempted outgoing handovers with DRX;

-	Successful outgoing handovers with DRX;

-	Attempted outgoing handovers non-DRX;

-	Successful outgoing handovers non-DRX.

### 6.9.5	Handover triggering measurements

The network management system should be capable of collecting at least the following Handover triggering measurements defined in the TS 32.425 [2], separately for eMTC devices:

-	Average quality of the serving cell when HO is triggered;

-	Average quality of the neighboring cell when HO is triggered.

### 6.9.6	Inter-RAT handovers

The network management system should be capable of collecting at least the following Inter-RAT Handovers related measurements defined in the TS 32.425 [2], separately for eMTC devices:

-	Attempted outgoing inter-RAT handovers per handover cause;

-	Successful outgoing inter-RAT handovers per handover cause;

-	Number of outgoing unnecessary handovers related with inter-RAT MRO.

## 6.10	Solution for paging related measurements

The network management system should be capable of collecting at least the following Paging related measurements defined in the TS 32.425 [2], separately for eMTC devices:

-	Number of paging records discarded at the eNodeB;

-	Number of paging records received by the eNodeB.

NOTE. It is valid only for the case when the eNB and MME implementation and configuration enable reception of UE category from the MME as a part of the Paging IE.

## 6.11	Solution for TB related measurements

The network management system should be capable of collecting at least the following TB related measurements defined in the TS 32.425 [2], separately for eMTC devices:

-	Total Number of DL TBs;

-	Error Number of DL TBs;

-	Total Number of UL TBs;

-	Error Number of UL TBs.

## 6.12	Solution for UE-associated logical S1-connection related measurements

The network management system should be capable of collecting at least the following UE-associated logical S1-connection related measurements defined in the TS 32.425 [2], separately for eMTC devices:

-	Attempted UE-associated logical S1-connection establishment from eNB to MME;

-	Successful UE-associated logical S1-connection establishment from eNB to MME.

## 6.13	Solution for CQI and TA distribution related measurements

The network management system should be capable of collecting at least the following CQI and TA distribution related measurements defined in the TS 32.425 [2], separately for eMTC devices:

-	Wideband CQI distribution;

-	Average sub-band CQI;

-	Timing Advance Distribution.

## 6.14	Solution for measurements related to measurement report

The network management system should be capable of collecting at least the following measurements related to measurement report, defined in the TS 32.425 [2], separately for eMTC devices:

-	RSRP related measurements;

-	RSRQ related measurements;

-	UE power headroom related measurements;

-	UE Rx – Tx time difference related measurements;

-	AOA related measurements.

# 7	Conclusions

The Study has identified two general use cases, 15 requirements and 17 groups of solutions. Every group is related to certain category of measurements which should be done separately for eMTC UEs.

The Study provided recommendations on how the information on non-eMTC UEs can be derived from the measurements based on all UEs in the cell and the measurements based on eMTC UEs in the cell

It is recommended to address the use cases, requirements and the solutions in the normative work.

# 8	Recommendations

It is recommended to extend the LTE PM specifications by adding support to the following:

- NRSRP, NRSRQ measurements;

- the measurements performed separately for eMTC devices and non-eMTC devices, per clause 6.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018-03 | SA#79 | SP-180075 |  |  |  | Presented for information and approval | 1.0.0 |
| 2018-03 | SA#79 |  |  |  |  | Upgrade to change control version | 15.0.0 |