3GPP TR 32.865 V15.0.0 (2017-06)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Study on Operations, Administration and Maintenance (OAM) aspects of Self-Organizing Network (SON) 
for Active Antenna System (AAS) based deployments;

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

OAM, SON, AAS

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	Use Cases	6

4.1	Cell Splitting	6

4.1.1	Overview	6

4.1.2	Issues	6

4.1.2.1	Triggering of Cell Splitting	6

4.1.2.2	Management of SP-Cells	7

4.1.2.3	Coverage and capacity planning for AAS	7

4.2	Cell Merging	8

4.2.1	Overview	8

4.2.2	Issues	8

4.2.2.1	Triggering of Cell Merging	8

4.2.2.2	Management of MT-Cell	8

4.3	Cell Shaping	8

4.3.1	Overview	8

4.3.2	Issues	9

4.3.2.1	Triggering of Cell Shaping	9

4.3.2.2	Scale of change	9

5	Architecture	9

5.1	Potential architecture options	9

5.1.1	General	9

5.1.2	NM centralized SON for AAS	9

5.1.3	EM centralized SON for AAS	10

5.1.4	Distributed SON for AAS	10

5.2	Comparison of the potential architecture options for response to fast conditions change	11

6	Potential Requirements	11

6.1	Requirements for management of Cell Splitting	11

6.2	Requirements for management of Cell Merging	12

6.3	Requirements for management of Cell Shaping	12

7	Potential Solutions	12

7.1	Activation and deactivation of SON for AAS	12

7.2	Cell identities management for AAS	13

7.3	Alternative coverage configurations for AAS	13

7.4	Cell Splitting and Cell Merging relation management	14

8	Conclusions	14

9	Recommendations	15

Annex A (informative):	Change history	16

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The AAS can enable system optimization to be adaptive to traffic demands and address network evolution issues through flexible software re-configuration of the BS and antenna system.

SON mechanism with OAM support can be beneficial to optimize the inter-operability of AAS operations for the following AAS capabilities:

1) 	Cell Shaping: where the main coverage of each cell is maintained unchanged but the cell edge can be adapted to load demand.

2) 	Cell Splitting: it adopts higher order sectorisation (vertical, horizontal or a combination) to an eNB by changing the coefficients of the antenna elements of an active antenna system (AAS) to form more antenna beams, each beam may be presented by one cell.

3)		Cell Merging: the contrary functionality of Cell Splitting.

Note: 	The description about Cell Shaping, Cell Splitting and Cell Merging may need to be realigned with their definitions which are to be specified.

The present document studies how OAM supports the SON for AAS-based deployments.

# 1	Scope

The present document studies the OAM aspects of SON (Self-Organizing Network) for AAS (Active Antenna Systems) based deployments in LTE.

The study includes use cases, potential requirements and possible solutions of OAM aspects of SON for AAS-based deployments in LTE, for both OAM centralized architecture and eNB distributed architecture with OAM supervision.

The study also makes conclusions on the preferred solution(s), and provides recommendations based on the conclusions of the study.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[3]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[4]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 Application Protocol (X2AP)".

[5]	3GPP TR 37.822: "Study on next generation Self-Optimizing Network (SON) for UTRAN and E-UTRAN".

[6]	3GPP TS 32.500: "Telecommunication Management; Self-Organizing Networks (SON); Concepts and requirements".

[7]	3GPP TS 28.627: "Telecommunication management; Self-Organizing Networks (SON) Policy Network Resource Model (NRM) Integration Reference Point (IRP); Requirements".

[8]	3GPP TS 28.628: "Telecommunication management; Self-Organizing Networks (SON) Policy Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[9]	3GPP TS 28.658: "Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[10]	3GPP TR 37.842: "E-UTRA and UTRA; Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AAS	Active Antenna System

SON	Self-Organizing Network

OAM	Operations, Administration and Maintenance

# 4	Use Cases

## 4.1	Cell Splitting

### 4.1.1	Overview

Cell Splitting adopts higher order sectorisation (vertical, horizontal or a combination) to an eNB by changing the coefficients of the antenna elements of an active antenna system (AAS) to form more antenna beams, each beam may be presented by one cell.

The number of cells split by AAS from an original cell is not fixed, i.e. it is flexible based on the antenna capability.

The cells newly split from an original cell may or may not change the coverage of the original cell. In order to simplify the description, the term SO-Cell and SP-Cell are used to signify the original cell and the new cell split from the original cell in Cell Splitting.

### 4.1.2	Issues

#### 4.1.2.1	Triggering of Cell Splitting

The Cell Splitting may be triggered (automatically) by EM (EM centralized SON for AAS based on measurement data (PM data and/or MDT data)), or by eNB (eNB distributed SON for AAS based on Network and/or UE measurements) if the cell coverage change is within the planning.

It may be unfeasible for NM to trigger the Cell Splitting in response to fast changes in the SO-Cell, e.g. within 5 or 15 minutes. EM or eNB may have more recent knowledge about situation in the SO-Cell with detailed location information in the SO-Cell.

As Cell Splitting operation is changing coverage, interference and handover settings for SP-cells and their neighbours, coordination with neighbour eNBs is needed to keep coverage and inter-cell interference under control. For distributed AAS SON function, this is done using X2 signalling. For NM centralized AAS SON it needs to be done by the management system.

Cell Splitting causes UE handovers between the SO-Cell, SP-Cells and their neighbour cells. The handover margins need to be set properly and efficiently at the eNB (regardless of before, during or after the Cell Splitting) and its neighbours for each SP-Cell for both outgoing and incoming handovers with its neighbours to avoid call drops, this is very difficult to be guaranteed if Cell Splitting is performed too often. So Cell Splitting should not perform with a very high frequency (see note). According to [5], Cell Splitting procedure is considered on a long-term time scale (every 1h or more seldom - few times a day).

Note: 	The appropriate frequency is to be discussed and may need to be conducted with RAN WG(s).

NM and EM should also be able to activate (switch on) and deactivate (switch off) the Cell Splitting:

-	based on network performance observed; or

-	for a special time duration for a planned event.

#### 4.1.2.2	Management of SP-Cells

The SP-Cells need to be manageable by NM and EM as soon as possible after they are split from the SO-Cell, for the following reason:

-	the coverage and capacity of each SP-Cell needs to be known by other SON functions (e.g. ANR, MRO and CCO) affecting the parameters about neighbour relations;

-	allowing signalling based MDT, since the id and geographical information of each SP-Cell are needed to locate UEs;

-	allowing Cell traffic trace and/or area based MDT to the SP-Cells;

-	allowing Performance Management (PM) of the SP-Cells;

-	allowing Fault Management (FM) of the SP-Cells;

-	allowing Configuration Management (CM) of the SP-Cells when necessary.

The SP-Cell, once split from the SO-Cell, needs to be made known to NM and EM as soon as possible, at least before AAS operation is applied to the SP-Cell.

The id of the SP-Cell needs to be manageable to avoid conflict and confusion.

The PCI of the SP-Cell may be or may be not the same as the PCI of the SO-Cell, however PCI conflict or confusion with other cells needs to be avoided. So the PCI or PCI range of each SP-Cell may be pre-allocated by NM or EM.

ECGI of each SP-Cell needs to be globally unique, it should be possible to be configured (or pre-configured) by OAM (both NM and EM).

#### 4.1.2.3	Coverage and capacity planning for AAS

The radio network is initially deployed under network planning, and is optimized manually or automatically (by SON functions) based on network performance within the network capability.

AAS Cell Splitting increases network capacity in a certain area, the maximum network capacity of a specific area is normally planned, so the Cell Splitting should be possible to be planned by OAM (NM or EM) for some specific SO-Cell or some specific geographical area though the Cell Splitting action may be triggered at eNB. The Cell Splitting planning may include:

-	The quantity of potential (allowed) SP-Cells;

-	Planned capacity and coverage for each potential (allowed) SP-Cell;

-	PCI value or range of each potential (allowed) SP-Cell; and

-	ECGI of each potential (allowed) SP-Cell.

To support the planning in the multi-vendor networks, NM should be able to pre-configure the information of planned Cell Splitting to EM, and EM then forwards to eNB.

## 4.2	Cell Merging

### 4.2.1	Overview

Cell Merging is the contrary functionality of Cell Splitting (see clause 4.1).

The number of cells can be merged by AAS into one cell is not fixed, i.e. it is based on AAS algorithm.

The cell merged from the original cells (in respect to Cell Merging) may or may not change the coverage of these original cells. In order to simplify the description, the term MO-Cell and MT-Cell are used to signify the original cell and the target cell merged from the original cells in Cell Merging.

### 4.2.2	Issues

#### 4.2.2.1	Triggering of Cell Merging

The Cell Merging may be triggered (automatically) by EM (EM centralized SON for AAS based on measurement data (PM data and/or MDT data)), or by eNB (eNB distributed SON for AAS based on Network and/or UE measurements). NM may not be efficient enough to react the fast change (e.g. within 5 or 15 minutes) within the network.

As Cell Merging operation is changing coverage, interference and handover settings created by the cell under consideration, coordination with neighbour eNBs is needed to keep coverage, interference and handover settings under control. For distributed AAS SON function, this is done using X2 signalling. For NM centralized AAS SON it needs to be done by the management system.

Cell Merging should not happen with a very high frequency (note of clause 4.1 is also applicable to Cell Merging). According to [5], Cell Merging procedure is considered on a long-term time scale (every 1h or more seldom - few times a day).

The entity triggering the Cell Merging should be the same as the entity triggering the Cell Splitting for one active antenna system.

Similarly to Cell Splitting, NM and EM should be also able to activate (switch on) and deactivate (switch off) the Cell Merging.

#### 4.2.2.2	Management of MT-Cell

For Cell Merging, the MT-Cell is normally assigned with a new ECGI, but may inherit the PCI from one of its MO-cells. So, in the view of other (e.g. neighbour/adjacent) cells, the Cell Merging is the case that one MO-Cell (whose id is continuously used by the MT-Cell) enlarges its coverage while the other MO-Cells become unavailable.

In order to avoid PCI conflict or confusion, NM or EM may pre-configure the PCI(s) to be (re)used by MT-Cell for Cell Merging; in case multiple PCIs are pre-configured, eNB needs to choose the appropriate one when a Cell Merging event happens. In multi-vendor networks scenario, NM should be able to do such pre-configuration to EM, and EM then forwards to eNB.

## 4.3	Cell Shaping

### 4.3.1	Overview

Cell Shaping adapts the cell edge to load demand while maintaining the main coverage unchanged.

### 4.3.2	Issues

#### 4.3.2.1	Triggering of Cell Shaping

The Cell Shaping may be triggered (automatically) by EM (EM centralized SON for AAS based on measurement data (PM data and/or MDT data)), or by eNB (eNB distributed SON for AAS based on Network and/or UE measurements). NM may not be efficient enough to react the fast change (e.g. within 5 or 15 minutes) within the network.

As Cell Shaping operation is changing coverage, interference and handover settings created by the cell under consideration, coordination with neighbour eNBs is needed to keep coverage, inter-cell interference and handover settings under control. For distributed AAS SON function, this is done using X2 signalling. For NM centralized AAS SON it needs to be done by the management system.

Cell Shaping may cause UE handovers with its neighbour cells. For each Cell Shaping event, the handover margin needs to be set properly and efficiently for both outgoing and incoming handovers with its neighbours to avoid call drops. So Cell Shaping should not be performed with too high frequency (the note of clause 4.1 is also applicable here).

NM and EM should be also able to activate (switch on) and deactivate (switch off) the Cell Shaping.

#### 4.3.2.2	Scale of change

The cell coverage change made by each Cell Shaping event is undefined. If the eNB executes Call Shaping in a fully unconstrained manner, it may decrease the reliability of handovers with its neighbour cells since in this case an appropriate handover margin cannot be guaranteed.

So, according to [2], the eNB may be pre-configured with alternative coverage configurations and the eNB may autonomously select and switch between these configurations using Active Antenna Systems functions including Cell Shaping. And based on [3] and [4], the number of allowed coverage configurations is limited.

The eNB needs to consider each allowed coverage configuration, to:

1)	make the right selection for the coverage change during Cell Shaping; and

2) 	set the proper handover margin with neighbour cells.

So each pre-configured coverage configuration should include a state identifier and the range of the parameters (e.g. tilt, power) affecting the coverage range.

In multi-vendor networks scenario, NM should be able to pre-configure the allowed alternative coverage configurations to EM, and EM then forwards to eNB.

# 5	Architecture

## 5.1	Potential architecture options

### 5.1.1	General

According to clause 3 of TS 32.500 [6], there may have varied potential architecture options for management of SON for AAS:

-	Centralized SON architecture: includes NM centralized SON and EM centralized SON.

-	Distributed SON architecture.

These potential architecture options are described in the following clauses, followed with comparison among them.

### 5.1.2	NM centralized SON for AAS

NM centralized SON for AAS means the algorithm of SON of AAS is executed at NM level.

Figure 5.1.2.1: Architecture of NM centralized SON for AAS

### 5.1.3	EM centralized SON for AAS

EM centralized SON for AAS means the algorithm of SON of AAS is executed at the EM level. NM provides the management and control functionalities for SON of AAS.

Figure 5.1.3.1: Architecture of EM centralized SON for AAS

### 5.1.4	Distributed SON for AAS

Distributed SON for AAS means the algorithm of SON of AAS is executed at the eNB level. EM and NM provide the management and control functionalities for SON of AAS. The EM provides vender specific management and control functionalities for SON of AAS, while the NM provides management and control functionalities for SON of AAS of multi-vendors.

Figure 5.1.4.1: Architecture of distributed SON for AAS

## 5.2	Comparison of the potential architecture options for response to fast conditions change

A comparison of the potential architecture options is described in the table below. The comparison is based on the analyses and characteristics of each of the architecture options.

|  | NM centralized SON for AAS | EM centralized SON for AAS | Distributed SON for AAS |
| --- | --- | --- | --- |
| Fast conditions change | NM may not be efficient enough to react to the fast change in conditions of the network, if they happen in the short time interval (e.g. 5 minutes). | Up to vendor's implementation, whether EM can react to the fast change in conditions of the network | The eNB is able to react to the fast change in conditions of the network |

# 6	Potential Requirements

## 6.1	Requirements for management of Cell Splitting

REQ-OAM_AAS_CeSp-CON-1 The EM should support a capability allowing NM to switch on/off the Cell Splitting function of an AAS.

REQ-OAM_AAS_CeSp-CON-2 The EM should support a capability allowing NM to set the quantity of allowed SP-Cells to be split from a SO-Cell.

REQ-OAM_AAS_CeSp-CON-3 The EM should support a capability allowing NM to pre-configure the ECGI for the potential SP-Cells.

REQ-OAM_AAS_CeSp-CON-4 The EM should support a capability allowing NM to pre-allocate the PCI range(s) for the potential SP-Cells.

REQ-OAM_AAS_CeSp-CON-5 The EM should support a capability allowing NM to pre-configure the range of parameters affecting the coverage for each potential SP-Cell.

REQ-OAM_AAS_CeSp-CON-6 The EM should be able to notify NM about the creation of SP-Cell as soon as possible after it is split from the SO-Cell.

REQ-OAM_AAS_CeSp-CON-7 The EM should support a capability allowing NM to modify the ECGI and PCI of SP-Cell.

REQ-OAM_AAS_CeSp-CON-8 The EM should enable management operations with the newly created SP-Cell as soon as possible after it is split from the SO-Cell.

REQ-OAM_AAS_CeSp-CON-9 For NM centralized AAS SON, the Cell Splitting operations at the eNB should be configured by OAM in coordination with OAM configuration of the neighbour eNBs to keep coverage, inter-cell interference and handover settings under control.

## 6.2	Requirements for management of Cell Merging

REQ-OAM_AAS_CeMr-CON-1 The EM should support a capability allowing NM to switch on/off the Cell Merging function of an AAS.

REQ-OAM_AAS_CeMr-CON-2 The EM should support a capability allowing NM to pre-configure the ECGI for the potential MT-Cells.

REQ-OAM_AAS_CeMr-CON-3 The EM should support a capability allowing NM to pre-configure the PCI range(s) for the potential MT-Cells.

REQ-OAM_AAS_CeMr-CON-4 The EM should be able to notify NM about the information of MT-Cell as soon as possible after it is merged from the MO-Cells.

REQ-OAM_AAS_CeMr-CON-5 The EM should support a capability allowing NM to modify the ECGI and PCI of MT-Cell.

REQ-OAM_AAS_CeMr-CON-6 For NM centralized AAS SON, the Cell Merging operations should be configured by OAM in coordination with OAM configuration of the neighbour eNBs to keep coverage, inter-cell interference and handover settings under control.

## 6.3	Requirements for management of Cell Shaping

REQ-OAM_AAS_CeSh-CON-1 The EM should support a capability allowing NM to switch on/off the Cell Shaping function of an AAS.

REQ-OAM_AAS_CeSh-CON-2 The EM should support a capability allowing NM to pre-configure the alternative coverage configurations for the SON function for Cell Shaping (SON function for Cell Shaping may reside in eNB or EM depending on the architecture of SON for AAS) to autonomously select and switch between these configurations.

REQ-OAM_AAS_CeSh-CON-3 Each coverage configuration, pre-configured according to REQ-OAM_AAS_CeSh-CON-2, should include a state identifier and the range of the parameters (e.g. tilt, power) affecting the coverage.

REQ-OAM_AAS_CeSh-CON-4 For NM centralized AAS SON, the Cell Shaping operations at the eNB should be configured by OAM in coordination with OAM configuration of the neighbour eNBs to keep coverage, inter-cell interference and handover settings under control.

# 7	Potential Solutions

## 7.1	Activation and deactivation of SON for AAS

To support activation and deactivation of SON for AAS, the following attributes are added to SONControl IOC defined in 3GPP TS 28.628 [8]. The SON for AAS function will be switched on or off according to the value assigned to the attributes.

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| aasSwitch | CM | M | M | - | M |

The constraints of aasSwitch Support Qualifier are AAS function is supported.

The properties of aasSwitch attribute are defined in the following table.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| aasSwitch | This attribute is the list of following entries: - cellSplittingSwitch - cellMergingSwitch - cellShapingSwitch The allowedValues for each entry are: on, off | type: <<data type>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |

## 7.2	Cell identities management for AAS

To support the pre-configuration of PCI of the SP-Cells and MT-Cells, the pciList attribute defined in EUtranGenericCell IOC defined in 3GPP TS 28.658 [9] is reused. The SON for AAS function below Itf-N will allocate the PCI to the SP-Cells and MT-Cells in Cell Splitting and Cell Merging from the value assigned to pciList attribute.

To support the pre-configuration of ECGI of the SP-Cells and MT-Cells, the following attributes are added to EUtranGenericCell IOC defined in 3GPP TS 28.658 [9]. The SON for AAS function below Itf-N will allocate the cell local identifier to the SP-Cells and MT-Cells in Cell Splitting and Cell Merging from the value assigned to cellLocalIdList attribute.

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellLocalIdList | CM | M | M | - | M |

The constraints of cellLocalIdList Support Qualifier are AAS function is supported.

The properties of cellLocalIdList attribute are defined in the following table.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| cellLocalIdList | This holds a list of cell local identities that can be assigned to the cellLocalId attribute of the SP-Cell and MT-Cell by the AAS function. The assignment algorithm is not specified. allowedValues of each entry: 0 : 255 | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |

## 7.3	Alternative coverage configurations for AAS

To support the alternative cell coverage configurations for AAS function, the following attributes are added to EUtranCellSON IOC defined in 3GPP TS 28.628 [8]. The SON for AAS function below Itf-N will select one coverage configuration when executing Cell Splitting, Cell Merging or Cell Shaping from the alternative coverage configurations assigned to AlterCovConfig attribute.

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| AlterCovConfig | CM | M | M | - | M |

The constraints of AlterCovConfig Support Qualifier are AAS function is supported.

The properties of AlterCovConfig attribute are defined in the following table.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| AlterCovConfig | This holds a list of alternative coverage configurations which are used for AAS function to select and switch between these configurations. Each coverage configuration contains the following entries: - StateId, which indicates the identifier of the coverage configuration, see 3GPP TS 36.423 [4]; - horizontalHBW, which indicates the horizontal half-power beamwidth (see note), see 3GPP TR 37.842 [10]; - verticalHBW, which indicates the vertical half-power beamwidth (see note), see 3GPP TR 37.842 [10]; - maximumTransmissionPowerRange, which indicates the range of maximumTransmissionPower, see 3GPP TS 28.658 [9]; - referenceSignalPowerRange, which indicates the range of referenceSignalPower, see 3GPP TS 28.658 [9] allowedValues: See 3GPP TS 36.423 [4] subclause 9.1.2.8 for value range of identifier of the coverage configuration, see 3GPP TR 37.842 [10] subclause 5.3.3.2 for value range of half-power beamwidth, see 3GPP TS 28.658 [9] subclause 4.4.1 for value range of maximumTransmissionPower and referenceSignalPower. | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| Note: The half-power beamwidth is, in a radiation pattern cut containing the direction of the maximum of a lobe, the angle between two directions in which the radiation intensity is one-half the maximum value, see 3GPP TR 37.842 [10]. |  |  |

## 7.4	Cell Splitting and Cell Merging relation management

To support showing the relation between SO-Cell and SP-Cell for Cell Splitting, the relation between MO-Cell and MT-Cell for Cell Merging, the following attributes are added to EUtranCellSON IOC defined in 3GPP TS 28.628 [8]. The SON for AAS function below Itf-N will indicate the SO-Cell that the SP-Cells has replaced in Cell Splitting, and the MO-Cells that the MT-Cell has replaced in Cell Merging, by ReplacedCells attribute.

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ReplacedCells | CM | M | M | - | M |

The constraints of ReplacedCells Support Qualifier are Cell Splitting or Cell Merging function is supported.

The properties of ReplacedCells attribute are defined in the following table.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| ReplacedCells | This holds a list of local identities of the cells which has been replaced by the subject cell within the eNodeB. allowedValues of each entry: 0: 255 | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |

# 8	Conclusions

Based on the studies done so far, the following bullets are the status of agreements on the SON for AAS solutions:

1)	SON for AAS solution may has the following architecture options: centralized SON for AAS or distributed SON for AAS. Different architecture option could meet requirements from different deployment scenarios.

2)	Standardization of these SON for AAS solutions require some enhancements in the SON NRM IRP to support the following SON for AAS related management operations over Itf-N:

-	Activation and deactivation of SON for AAS (see clause 7.1)

-	Alternative coverage configuration for AAS (see clause 7.3)

-	Cell Splitting and Cell Merging relation management (see clause 7.4)

3)	In order to support automatic configuration management of new split / merged cell needed for AAS operations, an enhancement in the E-UTRAN NRM IRP is needed to support transport of pre-assigned cell local identifier over Itf-N (see clause 7.2).

# 9	Recommendations

Based on the potential solutions described in clause 7, the group recommends to start normative work related to SON for AAS on following aspects:

1)	Update 3GPP TS 28.627 [7] with adding SON for AAS requirements.

2)	Update SON NRM IRP in 3GPP TS 28.628 [8] to SON for AAS management operations (see clauses 7.1, 7.3 and 7.4).

3)	Update E-UTRAN NRM IRP in 3GPP TS 28.658 [9] to support configuration management of new split / merged cells raised by SON for AAS operations (see clause 7.2).

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-06 | SA#76 | SP-170478 |  |  |  | Presented for approval | 2.0.0 |
| 2017-06 | SA#76 |  |  |  |  | Upgrade to change control version | 15.0.0 |