3GPP TR 32.869 V15.0.0 (2018-01)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication Management;

Charging management;

Study on overload control for Diameter charging applications;

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

management,Diameter,charging,

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2018, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	7

3.1	Definitions	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Overview	9

4.1	Introduction	9

4.2	Problems and goals	9

5	Impacts of Diameter overload on 3GPP charging interfaces	9

5.1	Introduction	9

5.1.1	3GPP Diameter Charging Interfaces	10

5.1.2	IETF Diameter Overload Indication Conveyance	10

5.2	Overload of the Offline Charging System (OFCS)	11

5.2.1	Existing overload control mechanism on Diameter charging interface	11

5.2.1.1	Failure handling	11

5.2.1.2	Limitation	11

5.2.2	Solutions for Diameter overload indication handling	11

5.2.2.1	Alternative solution #1.1: OLR with buffer mechanism	11

5.2.2.2	Alternative solution #1.2: OFCS without DOIC mechanism	13

5.3	Overload of the Online Charging System (OCS)	13

5.3.1	Existing overload control mechanism on Diameter charging interface	13

5.3.1.1	Failure handling	13

5.3.1.2	Limitation	13

5.3.2	Solutions for Diameter overload indication handling	13

5.3.2.1	Alternative solution #2.1: OLR with buffer mechanism	13

5.3.2.2	Alternative solution #2.2: OCS without DOIC mechanism	15

5.3.2.3	Alternative solution #2.3: OLR with service specific mechanism	15

6	Evaluation and recommendation	17

Annex A:	Change history	18

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document studies the overload control mechanisms for Rc, Re, Rf and Ro interfaces based on the conclusion of TR 29.809 [212].

For 3GPP Diameter Charging interfaces, the Diameter overload control mechanism are based on the IETF Draft "Diameter Overload Indication Conveyance" that defines the AVPs for the transport of overload information of Diameter application as well as the basic behaviour of the Diameter endpoints receiving this information.

The IETF RFC 7683 [404] is considered as the reference of the present document and not the IETF Draft draft-ietf-dime-ovli-02 referenced by TR 29.809 [212].

As per TR 29.809 [212], the following conclusions apply and are not studied in the present document:

-	How a reporting node determines it is entering into an overload condition is implementation dependent.

-	How a reporting node determines the contents value of the overload report (i.e. OC-OLR AVP) is implementation dependent.

-	How a reacting node achieves traffic reduction (i.e. selection of impacted sessions) is implementation dependent.

The following issues are covered by the present document:

-	Application and/or modification of current failure handling functionality defined in TS 32.299 [50] for Diameter online and offline charging in the Charging Trigger Function when reacting to overload control indication from the OCS or CDF, respectively.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging Architecture and Principles".

[2] - [10]	Void.

[11]	3GPP TS 32.251: "Telecommunication management; Charging management; Packet Switched (PS) domain charging".

[12] - [19]	Void.

[20]	3GPP TS 32.260: "Telecommunication management; Charging management; IP Multimedia Subsystem (IMS) charging".

[21] - [29]	Void.

[30]	3GPP TS 32.270: "Telecommunication management; Charging management; Multimedia Messaging Service (MMS) charging".

[31] 	3GPP TS 32.271: "Telecommunication management; Charging management; Location Services (LCS) charging".

[32] 	3GPP TS 32.272: "Telecommunication management; Charging management; Push-to-talk over Cellular (PoC) charging".

[33]	3GPP TS 32.273: "Telecommunication management; Charging management; Multimedia Broadcast and Multicast Service (MBMS) charging".

[34]	3GPP TS 32.274: "Telecommunication management; Charging management; Short Message Service (SMS) charging".

[35]	3GPP TS 32.275: "Telecommunication management; Charging management; MultiMedia Telephony (MMTel) charging".

[36] 	3GPP TS 32.276: "Telecommunication management; Charging management; Voice Call Service Charging".

[37] 	3GPP TS 32.277: "Telecommunication management; Charging management; Proximity-based Services (ProSe) Charging".

[38] 	3GPP TS 32.278: "Telecommunication management; Charging management; Monitoring Event charging".

[39]	Void.

[40]	3GPP TS 32.280: "Telecommunication management; Charging management; Advice of Charge (AoC) service".

[41]	3GPP TS 32.281: "Telecommunication management; Charging management; Announcement service".

[42] - [49]	Void.

[50]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging applications".

[51] - [52]	Void.

[53]	3GPP TS 32.296: "Telecommunication management; Charging management; Online Charging System (OCS) applications and interfaces".

[54] - [99]	Void.

[100]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[101]	3GPP TS 22.115: "Service aspects; Charging and billing".

[102] - [199]	Void.

[200] - [211]	Void.

[212]	3GPP TR 29.809: "Study on Diameter overload control mechanisms".

[213] - [299]	Void.

[300] - [399]	Void.

[400]	Void.

[401]	IETF RFC 6733 (2015): "Diameter Base Protocol".

[402]	IETF RFC 4006 (2005): "Diameter Credit-Control Application".

[403]	IETF RFC 7683 (2015): "Diameter Overload Indication Conveyance ".

[404] - [499]	Void.

[500] - [599]	Void.

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [100] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [100].

middle tier TS: 3GPP charging TSs that specify the domain / subsystem / service specific, online and offline, charging functionality. These are all the TSs in the numbering range from TS 32.250 to TS 32.27x.

NOTE1:	 e.g. TS 32.250 [10] for the CS domain, or TS 32.270 [30] for the MMS service.

NOTE 2:	Currently, there is only one "tier 1" TS in 3GPP, which is the TS 32.240 that specifies the charging architecture and principles. There are a number of top tier TSs in the 32.29x numbering range ([50] ff) that specify common charging aspects such as parameter definitions, encoding rules, the common billing domain interface or common charging applications.

offline charging: charging mechanism where charging information does not affect, in real-time, the service rendered

online charging: charging mechanism where charging information can affect, in real-time, the service rendered and therefore a direct interaction of the charging mechanism with session/service control is required

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Gy	Online charging reference point between a PCEF and an OCS.

Gz	Offline charging reference point between a PCEF and an OFCS.

Rf	Offline Charging Reference Point between a 3G network element and the CDF.

Ro	Online Charging Reference Point between a 3G network element and the OCS.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [100] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [100].

ACA	ACcounting-Answer

ACR	ACcounting-Request

AS	Application Server

ASA	Abort-Session-Answer

ASR	Abort-Session- Request

AVP	Attribute Value Pair

CCA	Credit-Control-Answer

CCR	Credit-Control-Request

CDF	Charging Data Function

CDR	Charging Data Record

CEA	Capabilities-Exchange-Answer

CER	Capabilities-Exchange-Request

DBPA	Diameter Base Protocol Accounting

DCCA	Diameter Credit-Control Application

DCD	Dynamic Content Delivery

DPA	Disconnect-Peer-Answer

DPR	Disconnect-Peer-Request

DWA	Device-Watchdog-Answer

DWR	Device-Watchdog-Request

ECUR	Event Charging with Unit Reservation

FUI	Final-Unit-Indication

GSU	Granted-Service-Unit

IE	Information Element

IEC	Immediate Event Charging

MSCC	Multiple Services Credit-Control

OCS	Online Charging System

OFCS	OFfline Charging System

RAA	Re-Auth-Answer

RAR	Re-Auth-Request

TDF	Traffic Detection Function

# 4	Overview

## 4.1	Introduction

TR 29.809 [212] investigated the possible Diameter-based mechanisms to support overload control mechanisms in 3GPP core networks.

Two main activities were addressed:

-	Identification of requirements for an improved overload control mechanism over Diameter based signaling interfaces used in 3GPP core networks.

-	Evaluations of the proposed IETF solutions to cover the requirements of Diameter overload control.

The TR 29.809 [212] concludes that the generic solution as currently defined in the draft-ietf-dime-ovli-02, which is IETF RFC 7683 [403] is recommended as the basis to perform overload control over 3GPP Diameter applications (see clause 8.3.2 in TR 29.809 [212]). This solution provides a set of generic Diameter AVPs that can be re-used over any Diameter application to transport overload indication between Diameter endpoints. However, the exact solution to implement will be decided per Diameter application, depending on the specific requirements of each interface. Since the solution is application dependent this study aims to investigate the solution applicable for the Charging applications.

## 4.2	Problems and goals

The Diameter Base Protocol, IETF RFC 6733 [401], uses the Protocol Error "DIAMETER_TOO_BUSY", in the Result_Code AVP of the answer to a related request, as a means for overload control. Specifically, the Protocol Error "DIAMETER_TOO_BUSY" is used by the Diameter server nodes to indicate that it is unable to provide the requested service due to overload. The main limitation in the use of the Protocol Error indication "DIAMETER_TOO_BUSY" is that the Diameter client nodes receive overload indication after the request was sent to an overloaded server, which comprise an unnecessary server, and transport network, resource utilisation in overload condition. This limitation in the Diameter Base Protocol have been addressed by the generic DOIC (Diameter Overload Indication Conveyance) mechanisms specified in IETF RFC 7683 [403], which makes possible to convey servers' overload indications to the clients before the sever starts rejecting requests due to overload and that prevents retransmission of the requests due to overload of the servers.

The TR 29.809 [212] recommends the generic DOIC mechanisms to be the base for the overload control over 3GPP Diameter interfaces (see clause 8.3.2 in TR 29.809 [212]). However, the RFC 7683 [403] and TR 29.809 [212] do not provide specific guidance on how to implement the DOIC (Diameter Overload Indication Conveyance) mechanism for Diameter charging applications, it is the scope of the present document to investigate that.

# 5	Impacts of Diameter overload on 3GPP charging interfaces

## 5.1	Introduction

This clause considers application and/or modification of current failure handling functionality defined in TS 32.299 [50] for Diameter online and offline charging in the Charging Trigger Function (CTF) when reacting to overload control indication from the Offline Charging System (OFCS) and Online Charging Systems (OCS), respectively.

Today there is a support on the reference points for alternative servers both in online and offline charging in the case one service is not reachable. In most cases charging messages cannot be dropped, even if there is a Diameter overload situation on charging interfaces, since these are the bases for the subscribers' bill and dropping a charging message will mean that there will be no means to charge for the service.

### 5.1.1	3GPP Diameter Charging Interfaces

The Offline Charging System (OFCS) terminates the Rf reference point as defined in TS 32.240 [1]. Other 3GPP specifications define additional reference points that are functionally equivalent to the Rf reference point. These are the Gz reference point for PCEF embedded in PGW and the Gzn reference point for TDF. The OFCS can be structured into Charging Data Function (CDF) and the Charging Gateway Function (CGF) in which case the CDF provides the Diameter Accounting Application. Diameter protocol is not used for communication between CDF and CGF. As defined in TS 32.240 [1], the network elements connected to the OFCS by the Rf reference point, such as PGW can be regarded as Diameter client.

The Online Charging System (OCS) terminates the Ro reference point as defined in TS 32.240 [1]. Other 3GPP specifications define additional reference points that are functionally equivalent to the Ro reference point. These are the Gy for PCEF embedded in PGW and the Gyn reference point for TDF. The OCS can be structured into Online Charging Function (OCF), Account Balance Management Function (ABMF) and Rating Function (RF) and between these functions Diameter is used, but for the Ro reference point it is the OCS as such that provides the Diameter Credit-Control Application. As defined in TS 32.240 [1], the network elements connected to the OCS by the Ro reference point, such as PGW can be regarded as Diameter clients.

The Diameter Accounting Application used on the Rf interface and Diameter Credit-Control Application used on the Ro interface are defined in TS 32.299 [50].

### 5.1.2	IETF Diameter Overload Indication Conveyance

The interaction between reacting node and reporting node specified in IETF RFC 7683 [403] can be illustrated as following figure.

Figure 5.1.2.1: Interaction between Reacting Node and Reporting Node

The default abatement algorithm described in IETF RFC 7683 [403] is to select a random number between 1 and 100. If the random number is less than or equal to the indicated reduction percentage received by the reacting node then the request will be given abatement treatment otherwise the request is given normal routing treatment.

Editor's note: This needs to be extended with more details about the DIOC concept.

## 5.2	Overload of the Offline Charging System (OFCS)

### 5.2.1	Existing overload control mechanism on Diameter charging interface

#### 5.2.1.1	Failure handling

The current failure handling uses the Charging Characteristics as defined in TS 32.251 [11] Annex A, allows to specify both primary and secondary OFCS addresses as well as the possibility to produce CDR or buffer these for offline charging as a backup for the Diameter offline.

#### 5.2.1.2	Limitation

The current solution will only be used when there is a failure and cannot be used for preventing an overload situation, i.e. it will manage the event that a failure occurs but it cannot decrease the load if the systems are getting close to their limits.

### 5.2.2	Solutions for Diameter overload indication handling

#### 5.2.2.1	Alternative solution #1.1: OLR with buffer mechanism

The default loss algorithm together with buffer mechanism is proposed to be alternative solution 1. The reacting node determines if overload control treatment should be applied to the request based on the OLR. One approach that could be taken for each request is to select a random number between 1 and 100. If the random number is less than or equal to the indicated reduction percentage received by the reacting node then the request is buffered, otherwise the request is given normal routing treatment. This algorithm enables request messages of the reduction percentage to be buffered. And when the overload ends, according to FIFO (first-in first-out) principle, the buffered request messages will be sent out to CDF in order. The CDF can apply existing duplicate detection and sort ACRs according to sequence number of each ACR to generate CDR. The shortage of the solution is that buffering request messages randomly may lead to additional workload on sorting ACRs in CDF.

OLR_DEFAULT_ALGO (0x0000000000000001) of OC-Feature-Vector AVP specified in IETF RFC 7683 [403] is used.

The above described abatement mechanism is illustrated by the diagram in Figure 5.2.2.1.1, which assumes configuration with a single CDF; configuration with redundant standby CDF is not considered in the present document.

Figure 5.2.2.1.1: Basic buffer mechanism solution

1.	Charging session is ongoing.

2.	CTF sends its supported overload control features stating support for the default loss algorithm.

3.	CDF selects OC-Reduction-Percentage based on load situation (i.e. CPU load) and default loss algorithm as expected overload control algorithm and fill value of OLR_DEFAULT_ALGO in OC-Feature-Vector AVP

4.	CDF responds with the selected values.

5.	CTF is set to enforce the buffer mechanism based on the result of the default loss algorithm until next ACR.

6a.	An overload occurs

6b. Any ACA response during overload condition may contain the validity duration and reduction percentage, resulting in buffering of ACRs.

7. When the overload ends, this can either be after the OLR has timed out or a new OLR is sent without restriction, the buffered ACRs are sent to CDF in the order they were buffered.

#### 5.2.2.2	Alternative solution #1.2: OFCS without DOIC mechanism

In this alternative solution 2 the OFCS will not use the DOIC but instead selects which messages to reply with a Protocol Error "DIAMETER_TOO_BUSY" and for these the client will follow the Charging Characteristics on the treatment, e.g. try the secondary address, start buffering or producing CDR. This means that the OFCS would have to decide on which request that should be buffered and this can in this case be based on type of service. This will mean that the OFCS still will receive all Diameter messages and will reject them one by one, that is if the Client itself do not start using an algorithm to select messages to go to the secondary address would it encounter a Protocol Error "DIAMETER_TOO_BUSY".

## 5.3	Overload of the Online Charging System (OCS)

### 5.3.1	Existing overload control mechanism on Diameter charging interface

#### 5.3.1.1	Failure handling

The current failure handling uses the Charging Characteristics as defined in TS 32.251 [11] Annex A, allows to specify both primary and secondary OCS addresses as well as the possibility to produce CDR as a backup for the Diameter online charging.

#### 5.3.1.2	Limitation

The current solution will only be used when there is a failure and cannot be used for preventing an overload situation, i.e. it will manage the event that a failure occurs but it cannot decrease the load if the systems are getting close to their limits.

### 5.3.2	Solutions for Diameter overload indication handling

#### 5.3.2.1	Alternative solution #2.1: OLR with buffer mechanism

The default loss algorithm together with buffer mechanism is proposed to be alternative solution 1. Since it is not possible to buffer a request for quota to grant service access, the mechanism will buffer the current online mechanism and use it as a sort of offline mechanism. This means that they will only be provided after service has been granted. The reacting node determines if overload control treatment should be applied to the request based on the OLR. One approach that could be taken for each request is to select a random number between 1 and 100. If the random number is less than or equal to the indicated reduction percentage received by the reacting node then the request is buffered, otherwise the request is given normal routing treatment. This algorithm enables request messages of the reduction percentage to be buffered and sent offline. And when the overload ends, according to FIFO (first-in first-out) principle, the buffered request messages will be sent out to OCS in order, with an indication that they have been buffered. The shortage of the solution is that it means that the system falls back to an offline behaviour even if it uses the online mechanism when an overload occurs and can no longer support online charging, this may result in loss of revenue depending on the account status when the buffered online requests are sent to the OCS.

OLR_DEFAULT_ALGO (0x0000000000000001) of OC-Feature-Vector AVP specified in IETF RFC 7683 [403] is used.

The above described abatement mechanism is illustrated by the diagram in Figure 5.3.2.1.1, which assumes configuration with a single OCS; configuration with redundant standby OCS is not considered in the present document.

Figure 5.3.2.1.1: Basic buffer mechanism solution

1. 	Charging session is ongoing.

2.	CTF sends its supported overload control features stating support for the default loss algorithm.

3.	OCS selects OC-Reduction-Percentage based on load situation (i.e. CPU load) and default loss algorithm as expected overload control algorithm and fill value of OLR_DEFAULT_ALGO in OC-Feature-Vector AVP

4.	OCS responds with the selected values.

5.	CTF is set to enforce the buffer mechanism based on the result of the default loss algorithm until next CCR.

6a.	An overload occurs

6b. Any CCA response during overload condition may contain the validity duration and reduction percentage, resulting in buffering of CCRs.

7. When the overload ends, this can either be after the OLR has timed out or a new OLR is sent without restriction, the buffered CCRs are sent to OCS in the order they were buffered, with an indication that they were buffered, for further processing.

#### 5.3.2.2	Alternative solution #2.2: OCS without DOIC mechanism

In this alternative solution 2 the OCS will not use the DOIC but instead selects which messages to reply with a Protocol Error "DIAMETER_TOO_BUSY" and for these the client will follow the Charging Characteristics on the treatment, e.g. try the secondary address or producing CDR. This means that the OCS would have to decide which request that the CTF should treat in accordance with the Charging Characteristics, this can in this case be based on type of service. This means that the OCS will still receive all Diameter messages and will reject them one by one. There is a possibility for the CTF to apply the treatment according to the Charging Characteristics not only to the CCR where the Protocol Error "DIAMETER_TOO_BUSY" was received, but also to other CCRs.

#### 5.3.2.3	Alternative solution #2.3: OLR with service specific mechanism

A new specific loss algorithm together with buffer mechanism is proposed to be alternative solution 3. Since it is not possible to buffer a request for quota to grant service access, the mechanism will buffer the current online mechanism and use it as a sort of offline mechanism. This means that they will only be provided after service has been granted. The reacting node determines if overload control treatment should be applied to the request based on the OLR. The specific loss algorithm has a more detailed information (compared to alternative solution 1) for the reduction mechanism, based on Rating-Group or interrogation type (e.g. First, Intermediate and Final). For each of these there may be a separate reduction percentage.

When the overload ends, according to FIFO (first-in first-out) principle, the buffered request messages will be sent out to OCS in order, with an indication that they have been buffered. The shortage of the solution is that it means that the system falls back to an offline behaviour even if it uses the online mechanism when an overload occurs and can no longer support online charging, this may result in loss of revenue depending on the account status when the buffered online requests are sent to the OCS.

A new value of OC-Feature-Vector AVP is needed.

The above described abatement mechanism is illustrated by the diagram in Figure 5.3.2.3.1, which assumes configuration with a single OCS; configuration with redundant standby OCS is not considered in the present document.

Figure 5.3.2.3.1: Basic buffer mechanism solution

1)	Charging session is ongoing.

2)	CTF sends its supported overload control features stating support for the new specific loss algorithm, it may be more detailed stating for which types are supported e.g. for rating group, service identifier or type of interrogation.

3)	OCS selects the new specific loss algorithm as expected overload control algorithm. The OC-Reduction-Percentage is selected based on load situation (e.g. CPU load) and specified for each type it wants separate treatment of.

4)	OCS responds with the types and their respective selected OC-Reduction-Percentage value.

5)	CTF is set to enforce the mechanism based on the result of the service loss algorithm until next CCR.

6a)	An overload occurs.

6b) Any CCA response during overload condition may contain the reduction percentage for specified Rating Group and interrogation type as well as validity duration and default percentage for others, resulting in buffering of CCRs matching.

7)	When the overload ends, this can either be after the OLR has timed out or a new OLR is sent without restriction, the buffered CCRs are sent to OCS in the order they were buffered, with an indication that they were buffered, for further processing.

# 6	Evaluation and recommendation

All the solutions #1.1, #1.2, #2.1, #2.2 and #2.3 are valid and could be used to solve the issue and giving better control in the case of an overload scenario. To require support of at least one of the #1.1 and #1.2 as well as one of the #2.1, #2.2 and #2.3 is necessary to be able to handle overload situations.

This will mean that a new clause in the TS 32.299[50] is needed to introduce and define Overload Control.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-12 | SA#78 | SP-170 |  |  |  | Incorporating MCC and EditHelp editorial fixes .Presented for information and approval. | 1.0.0 |
| 2018-01 |  |  |  |  |  | Change control version | 15.0.0 |