3GPP TR 32.871 V15.0.0 (2018-06)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Study on policy management for mobile networks based on Network Function Virtualization (NFV) scenarios

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

management,NFV, policy

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2018, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	7

4	Concepts and Background	7

4.1 	Concepts	7

4.2 	NFV policy conflict detection and resolution	7

5	Use cases	7

5.1 	Use cases for network deployment	7

5.1.1 	Deploy the network function in specific location	7

5.1.1.1 	Introduction and assumption	7

5.1.1.2 	Actor and roles	8

5.1.1.3 	Pre-condition	8

5.1.1.4 	Description	8

5.1.1.5 	Post-condition	8

5.1.2 	Deploy the corresponding network functions in different location	9

5.1.2.1 	Introduction and assumption	9

5.1.2.2 	Actor and roles	9

5.1.2.3 	Pre-condition	9

5.1.2.4 	Description	9

5.1.2.5 	Post-condition	9

5.1.3 	Deploy the network function based on certain location constraints	10

5.1.3.1 	Introduction and assumption	10

5.1.3.2 	Actor and roles	11

5.1.3.3 	Pre-condition	11

5.1.3.4 	Description	11

5.1.3.5 	Post-condition	11

6	Potential requirements	12

6.1 	Requirements for the policy operation	12

6.2 	Requirements for network deployment	12

7	Policy management architecture	12

7.1 	Overview	12

7.2 	Architecture alternative	12

7.3 	Mapping of Policy Related Function to MANO functional blocks	14

7.3.1 	Overview	14

7.3.2 	Mapping the PF to the NFVO and the PAF to the OSS/BSS	14

7.3.3 	Mapping the PF to the VNFM and the PAF to the NFVO and/or the EM	14

7.3.4 	Mapping the PF to the VIM and the PAF to the NFVO or VNFM	14

7.3.5 	Summary of PAF mapping	15

7.3.6 	PVF Mapping	15

7.3.7 	PSE Mapping	15

8	Potential solutions	15

8.1 	General procedure of policy operation	15

8.2 	General message flow for policy create	15

8.3 	General message flow for policy query	16

8.4 	General message flow for policy active/deactive	16

8.5	 General message flow for policy update	16

8.6 	General message flow for policy conflict report	17

9	Conclusions	17

Annex <A>:	Change history	18

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

There are many benefits to introduce NFV in mobile network, such as resource sharing and automatic network deployment, efficient network scaling capability and flexible network optimization and healing. The use of NFV-related policies can help realize the benefits.

Policy management from the end-to-end mobile service and network function management perspective will enrich and enhance the automation capability during the different lifecycle phases (e.g. network deployment, network updating, network scaling or migration, network termination etc.).

The present document investigates concepts of network policy management of mobile network based on NFV. It investigates the policy classification, functional roles and the scenarios for policy application, analyses processes of the policy management and proposes corresponding solutions. Whether the existing management solution could be reused or enhanced will also be investigated.

# 1	Scope

From mobile service and network function management perspective, the present document studies the end-to-end network policy management for mobile network based on NFV scenarios:

-	Study the concepts and classification of the policy management for mobile networks based on the NFV scenarios, analyse and classify the potential policy management scenarios.

-	Identify the use cases and requirements of policy management for mobile networks based on the NFV scenario.

-	Study the potential logical roles of the policy management for mobile networks based on the NFV scenarios.

-	Identify the impacts of logical architecture and the potential requirements to 3GPP management system. Analyse the existing 3GPP Management reference model, interfaces, protocols and procedures to determine what can be re-used, adapted or extended. Propose enhancements or extensions to the 3GPP solutions, if an impact is foreseen.

-	Identify the impacts to NFV-MANO framework from the 3GPP service perspective.

-	Provide recommendations to the standardization of policy management for mobile networks based on the NFV scenario.

The study will focus on the policy solutions which are related to NFV scenarios and will take into account of the outcome of SA5 existing specifications and the outcome produced by ETSI. The study will investigate whether the existing specification can be reused or not.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ETSI GR NFV-IFA 023 (V3.1.1): "Network Function Virtualisation (NFV); Management and Orchestration; Report on Policy Management in MANO; Release 3".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

# 4	Concepts and Background

## 4.1 	Concepts

Policy Administration Function (PAF) defines the network policy.

Policy Storage Entity (PSE) stores the defined network policy.

Policy Function (PF) makes the policy execution decisions.

Policy Verification Function (PVF) evaluates the effectiveness of the network policy execution.

## 4.2 	NFV policy conflict detection and resolution

NFV policy management system may have policy conflict problem, which is a newly created policy may be conflict with an existing policy.

There are two potential policy conflict detection approaches, one is policy conflict detection in the PF, and the other one is policy conflict detection in the PAF (As described in clauses 5.8.8 and 5.8.9 of GR NFV-IFA 023 [2]). And GR NFV-IFA 023 [2] also refers to ECA (Event-Condition-Action) model to explain the UCs and the resolution mechanism.

There are multiple scenarios for policy conflict, for example:

1)	If the events are the same and the same condition are triggered (e.g. scaling policy on a VNF), but two policies have different actions, the conflict should be detected and resolved.

2)	If the event and condition of two policies are not be the same, but the different actions (e.g. scaling policy vs. termination policy on a VNF) are towards the same entity. The conflict should be detected and resolved.

3)	If scenario 1) or 2) is enforced by different PFs. The conflict should be detected and resolved.

It is not easier for one PF to resolve all of scenarios of the policy conflict. Hence, it is reasonable to enable PAF to resolve policy conflict, while the conflict can be detected by PAF or PF based on different specific scenarios.

# 5	Use cases

## 5.1 	Use cases for network deployment

### 5.1.1 	Deploy the network function in specific location

#### 5.1.1.1 	Introduction and assumption

According to the network design and deployment location requirement, operator usually requires some specific type virtualized network function to be deployed in specific DC (data centre) or central office. Moreover, for some dedicated network function, which need specific virtualized or hardware resource capability, operator need select appropriate resource in the data centre to deploy it.

The assumptions are:

1) 	The operator has two types' DCs, core DCs are centralized in some individual region and edge DCs are distributed in most cities.

2) 	The operator determines core DC supports the control plane network function which handles signalling and edge DC supports the media plane or forwarding plane network function which handles session service and data service.

3)	 The operator need deploy IMS network and determine CSCF should be deployed in core DC and SBC should be in edge DC, where the hardware resource should support the capability of transcoding and encryption and decryption.

#### 5.1.1.2 	Actor and roles

1. PAF creates the IMS deployment policy.

2. PF executes the IMS deployment policy and can store the policy.

3. PSE stores the IMS deployment policy. It exists standalone or is embedded in PF.

4. PVF evaluates the policy execution.

#### 5.1.1.3 	Pre-condition

1. The operator designs the IMS network deployment requirement and need deploy the network based on the requirement.

2. Management systems (i.e. OSS, EMS and NFV-MANO system) are running normally.

Note: 	The relation between PAF/PF/PSE and management system is out of scope of the present document.

#### 5.1.1.4 	Description

1. The operator sets up the IMS network deployment policy through PAF.

-	If exists standalone policy storage entity (PSE), the operator can create the policy into PSE. The policy is:

-	For the IMS network, CSCF is allowed to be deployed in core DC and SBC is only allowed to be deployed in edge DC, where the hardware resource should support the capability of transcoding and encryption and decryption.

Or,

-	If the PSE is embedded in PF, the PAF can create the policy into PF.

2. The PAF activates the policy and notifies related PF.

3. The PF follows the policy to design the IMS network then the IMS network is instantiated by the management system.

Note: 	Whether and how NSD or VNFD supports the policy is out of scope of the present document.

4. The PF informs the PAF the policy has been executed.

5. The PAF requests PVF to evaluate the effectiveness of the policy execution if needed.

6. The PVF evaluates the effectiveness of the policy according to the result of the policy execution. In this case the effectiveness is CSCF(s) and SBC(s) in the IMS network are deployed in appropriate DCs and on appropriate hardware resource.

#### 5.1.1.5 	Post-condition

The IMS network is correctly configured and normally running.

### 5.1.2 	Deploy the corresponding network functions in different location

#### 5.1.2.1 	Introduction and assumption

According to the network design, load balancing, redundancy and restoration requirement, operator usually requires some virtualized network functions with same functionality cannot be deployed in the same resource pool, which is controlled by one VIM, even they cannot be deployed in the same DC.

The assumptions are:

1) 	The operator determines to deploy one IMS network with 3 load sharing CSCFs.

2) 	The operator determines it is not allowed to deploy more than 2 CSCFs in the same core DC.

#### 5.1.2.2 	Actor and roles

1. PAF creates the IMS deployment policy.

2. PF executes the IMS deployment policy and can store the policy.

3. PSE stores the IMS deployment policy. It exists standalone or is embedded in PF.

4. PVF evaluates the policy execution.

#### 5.1.2.3 	Pre-condition

1. The operator designs the IMS network deployment requirement and need deploy the network based on the requirement.

2. Management systems (i.e. OSS, EMS and NFV-MANO system) are running normally.

Note: 	The relation between PAF/PF/PSE and management system is out of scope of the present document.

#### 5.1.2.4 	Description

1. The operator sets up the IMS network deployment policy through PAF.

-	If exists standalone policy storage entity (PSE), the operator can create the policy into PSE. The policy is:

-	For the IMS network, each CSCF should be deployed in a standalone core DC. And it is not allowed to deploy more than 2 CSCFs in the same core DC.

Or,

-	If the PSE is embedded in PF, the PAF can create the policy into PF.

2. The PAF activates the policy and notifies related PF.

3. The PF follows the deployment policy to design the IMS network then the IMS network is instantiated by the management system.

Note : 	Whether and how NSD or VNFD supports the policy is out of scope of the present document.

4. The PF informs the PAF the policy has been executed.

5. The PAF requests PVF to evaluate the effectiveness of the policy execution if needed.

6. The PVF evaluates the effectiveness of the policy according to the result of the policy execution. In this case the effectiveness is all CSCFs in the IMS network are deployed in appropriate DCs.

#### 5.1.2.5 	Post-condition

The IMS network is correctly configured and normally running.

### 5.1.3 	Deploy the network function based on certain location constraints

#### 5.1.3.1 	Introduction and assumption

According to the IMS and EPC network design and the requirement of avoiding media detour which may cause the transport network congestion, operator usually requires one kind of the network functions should be adjacent to another kind of the network functions.

The adjacent relationship means the two kinds of network functions, which interact with each other, should be deployed in the same DC or in the nearby same type DCs.

For instance:

Instance1: For IMS network, the IMS SBC (Session Border Controller) should be adjacent to EPC SAE GW (Serving-GW/PDN-GW).

When the SAE GW is in the core DC (if it is virtualized) or centralized central office (if it is non-virtualized), the virtualized SBC should not be allowed to be deployed in the edge DC (as shown Figure 5.1.3-1) due to the voice flow detour, when the SAE GW is in the edge DC or remote central office, the virtualized SBC is better to be deployed in the edge DC (as shown in Figure 5.1.3-2) to avoid voice flow detour.

Figure 5.1.3-1: Voice flow detour if SBC is not adjacent to SAE GW

Figure 5.1.3-2: No voice flow detour if SBC is adjacent to SAE GW

Instance2: For EPC network, there is the similar requirement that the SAE GW should be adjacent to some content resource (e.g. CDN cache point).

When the CDN cache is in the edge DC, the virtualized SAE GW should not be allowed to be deployed in the core DC, when the CDN cache is in the core DC, the virtualized SAE GW is better to be deployed in the core DC.

The assumptions are (for Instance1):

1) 	The operator has two types' DCs, core DCs are centralized in some individual region and edge DCs are distributed in most cities.

2) 	The operator need deploy IMS network in one region and the EPC network has already been running.

#### 5.1.3.2 	Actor and roles

1. PAF creates the IMS deployment policy.

2. PF executes the IMS deployment policy and can store the policy.

3. PSE stores the IMS deployment policy. It is standalone or is embedded in PF.

4. PVF evaluates the policy execution.

#### 5.1.3.3 	Pre-condition

1. The operator designs the IMS network deployment requirement and need deploy the network based on the requirement.

2. Management systems (i.e. NM, EM and NFV-MANO system) are running normally.

3. The PF has the adjacent location information of the data centre(s) and central office(s).

Editor's note: How the PF gets the adjacent location information is out of scope of the present document.

#### 5.1.3.4 	Description

1. The operator sets up the IMS network deployment policy through PAF.

-	If exists standalone policy storage entity (PSE), the operator can create the policy into PSE. The policy is:

-	For the IMS network, virtualized SBC should be deployed adjacently to the SAE GW(S-GW/P-GW) which it interacts with. That is when the SAE GW is in the core DC (if it is virtualized) or centralized central office (if it is non-virtualized), the virtualized SBC should not be allowed to be deployed in the edge DC, when the SAE GW is in the edge DC or remote central office, the virtualized SBC is better to be deployed in the edge DC.

Or,

-	If the PSE is embedded in PF, the PAF can create the policy into PF.

2. The PAF delivers the policy to PF and activates the policy if needed.

3. The PF follows the policy to design the IMS network and determine the virtualized SBC location after getting the SAE GW location.

4. The management systems follow the location constraint to design template (e.g. NSD or VNFD) and instantiate the IMS network.

5. The PF informs the PAF the policy has been executed.

6. The PAF requests PVF to evaluate the effectiveness of the policy execution if needed.

7. The PVF evaluates the effectiveness of the policy according to the result of the policy execution. In this case the effectiveness is the virtualized SBC is deployed adjacently to the running SAE GW.

#### 5.1.3.5 	Post-condition

The IMS network is correctly configured and normally running.

# 6	Potential requirements

## 6.1 	Requirements for the policy operation

REQ-POM_PO-CON-1 The PAF should be able to support the capability to create/query/update/delete/activate/de-activate the network policy.

REQ-POM_PO-CON-2 The PF should be able to support the capability to execute the activated network policy.

REQ-POM_PO-CON-3 The PAF should be able to support the capability to verify whether the policy is executed correctly.

## 6.2 	Requirements for network deployment

REQ-POM_ND-CON-1 The PF should be able to design the mobile network which includes the virtualized network function(s) according to the defined network policy.

REQ-POM_ND-CON-2 The PF should be able to determine deployment location and the appropriate virtualized resource of the virtualized network function(s) according to the defined network policy.

REQ-POM_ND-CON-3 The PF should be able to determine deployment location of the virtualized network function based on certain location constraint with other virtualized or non-virtualized network function(s) according to the defined network policy.

# 7	Policy management architecture

## 7.1 	Overview

The following clauses elaborate the potential NFV policy management architecture of mobile networks which include virtualized network functions. All of the functions, entities and the interfaces are logical designed. The concepts and definitions basically refer to the MANO policy in [2]. The study will elaborate the differences if there is a gap between this study and MANO policy [2].

## 7.2 	Architecture alternative

According to the concepts of NFV policy, there are four important logical functions.

1) 	Policy Administration Function (PAF) provides the services as follows:

- 	Allows the operator to define and administrate the network policy

2) 	Policy Storage Entity (PSE) provides the services as follows:

- 	Services for storage/update/query/deletion of the defined network policy (consumed by PAF)

- 	Services for retrieval of the defined network policy (consumed by PF)

3) 	Policy Function (PF) makes the decision for policy execution and provides the services as follows:

- 	Activating/deactivating the network policy

- 	Event subscription/notification of the network policy execution

- 	Providing relevant data and parameters during the process of policy execution (e.g. execution time consuming)

4) 	Policy Verification Function (PVF) provides the services as follows:

- 	Evaluation of the effectiveness of the network policy execution based on necessary analysis data (e.g. the data derived from the PF and/or the network).

And the network is composed of non-virtualized NE(s), virtualized network function(s) and the NFVI resources which the virtualized network function(s) relies on.

Hence, one option for NFV policy management reference architecture is as follows. Service-based interfaces are used through Npaf, Npse, Npvf and Npf interfaces.

Note: 	N1 interface is used for PVF to get analysis data (e.g. resource utilization). Whether PVF can get the analysis data from other logical entity is out of scope of the present document.

Figure 7.2-1: NFV policy management architecture option

Figure 7.2-2: NFV policy management architecture option in reference point presentation

## 7.3 	Mapping of Policy Related Function to MANO functional blocks

### 7.3.1 	Overview

The PAF, PF, PVF and the PSE are the abstracted roles applied in high-level use case description of policy management. In the context of MANO architecture, the PAF, PF, PVF and the PSE are mapped to MANO functional blocks in different ways. The subsequent clauses elaborate the cases of PF mapping in the MANO architecture and summarize the PAF, PVF and PSE mapping.

### 7.3.2 	Mapping the PF to the NFVO and the PAF to the OSS/BSS

When the policy is applied for network service LCM operations and orchestration of resources managed across VIM, the PF may be mapped to the NFVO and the PAF may be mapped to the OSS/BSS. Those Network Service LCM operations include but not be limited to: NS instantiation, NS scaling, NS healing and NS termination, and examples of resource orchestration are (but not limited to): virtualised resource reservation, and virtualised resource quota management. The mapping relationship is shown in figure 7.3.2-1. This mapping is exemplified by the use case in clause 5.

Figure 7.3.2-1: Mapping the PF to the NFVO and the PAF to the OSS/BSS

### 7.3.3 	Mapping the PF to the VNFM and the PAF to the NFVO and/or the EM

When the policy is applied for VNF LCM operations, the PF may be mapped to the VNFM and the PAF may be mapped to the NFVO or the EM. Those VNF LCM operations include but not are limited to: VNF instantiation, VNF scaling, VNF healing and VNF termination. The mapping relationship is shown in figure 7.3.3-1.

Figure 7.3.3-1: Mapping the PF to the VNFM and the PAF to the NFVO and/or the EM

### 7.3.4 	Mapping the PF to the VIM and the PAF to the NFVO or VNFM

When the policy is applied for virtualized resource management operations, the PF may be mapped to the VIM and the PAF may be mapped to the NFVO. The mapping relationship is shown in figure 7.3.4-1.

The policies applicable for this mapping include: virtualised resource allocation policy, and virtualised resource reservation policy (only applicable if the NFVO is the PAF), virtualised resource quota policy (only applicable if the NFVO is the PAF), and virtualised resource capacity policy (only applicable if the NFVO is the PAF).

Figure 7.3.4-1: Mapping the PF to the VIM and the PAF to the NFVO

When the policy is applied for virtualized resource management operations, the PF may be mapped to the VIM and the PAF may also be mapped to the VNFM. The mapping relationship is shown in figure 7.3.4-2. This case is only applicable to the virtual resource(s) used by the VNF which the VNFM manages.

Figure 7.3.4-2: Mapping the PF to the VIM and the PAF to the VNFM

### 7.3.5 	Summary of PAF mapping

Within MANO, the PAF is mapped to the NFVO. The PAF mapped to the VNFM is only applicable to the virtual resource used by the VNF which the VNFM manages. The PAF can also be mapped to OSS/BSS or the EM.

### 7.3.6 	PVF Mapping

The PVF may be mapped to NFVO or OSS/BSS.

### 7.3.7 	PSE Mapping

The PSE may be mapped to NFVO and VNFM/EM.

# 8	Potential solutions

## 8.1 	General procedure of policy operation

1.	The PAF defines and creates the network policy into the PSE for storage.

2.	The PAF delivers the policy to the PF and activates the policy if needed.

3.	The PF executes the policy towards the network when the given condition is triggered.

4.	The PF notifies the information of policy completion to the PAF or optionally to the PVF when the policy is executed.

## 8.2 	General message flow for policy create

After the PAF decides to create a policy, it will send the create request to the PF combined with PSE. For the create request, it will include a callback url for the PF to report the policy conflict information to the PAF.

Figure 8.2-1: policy create message flow

## 8.3 	General message flow for policy query

After the PAF decides to query a policy, it will send the query request to the PF combined with PSE. The PF send back the response with the policy to the PAF.

Figure 8.3-1: policy query message flow

## 8.4 	General message flow for policy active/deactive

After the PAF decides to active, deactive a policy, it will send the active/deactive request to the PF. For the active request, it will include a callback url for the PF to report the policy conflict information to the PAF.

Figure 8.4-1: policy active/deactive message flow

## 8.5	 General message flow for policy update

After the PAF decides to update a policy, it will send the update request to the PF. For the update request, it will include a callback url for the PF to report the policy conflict information to the PAF.

Figure 8.5-1: policy update message flow

## 8.6 	General message flow for policy conflict report

When the PF combined with PSE receives a new or updated policy from PAF, it will check whether current received policy is conflicted with the previous storage policy. If the conflict is detected, the PF will repost this information to the PAF.

Figure 8.6-1: policy conflict report message flow

# 9	Conclusions

The present document makes the investigation on the use cases, potential requirements, management architecture and potential solutions on policy management for mobile networks based on NFV scenarios. Based on the output of the study, normative work for policy management is needed in the following aspects:

-	Management architecture aspect: It is suggested to start the normative work in R16 and the services-based interface architecture is suggested to be chosen as the only one architecture.

-	Management functions aspect: four policy management functions including PF, PAF, PSE and PVF are introduced. The PFs can be mapped to the NFVO. The PAF can be mapped to the OSS/BSS, NFVO respectively. The PSE can be mapped to NFVO and EM. The PVF can be mapped to the NFVO or OSS/BSS.

-	Interface aspect: normative work for policy management on Policy create, deletion, update, query, activation and deactivation and Policy conflict notification between PAF and PF which are mapped to the OSS/BSS and EM in 3GPP domain are suggested to be standardized.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-05 | SA5#113 | S5-173253 |  |  |  | Skeleton of the TR “network policy management based on NFV scenarios” | 0.1.0 |
| 2017-05 | SA5#113 | S5-173538 |  |  |  | Adding the introduction of the TR “network policy management based on NFV scenarios” | 0.1.0 |
| 2017-05 | SA5#113 | S5-173539 |  |  |  | Adding the scope of the TR “network policy management based on NFV scenarios”. | 0.1.0 |
| 2017-05 | SA5#113 | S5-173540 |  |  |  | Adding the deployment use case of the TR “network policy management based on NFV scenarios”. | 0.1.0 |
| 2017-05 | SA5#113 | S5-173541 |  |  |  | Adding the use case of the TR “network policy management based on NFV scenarios”. | 0.1.0 |
| 2017-05 | SA5#113 | S5-173542 |  |  |  | Adding the concepts of the TR “network policy management based on NFV scenarios”. | 0.1.0 |
| 2017-08 | SA5#114 | S5-174451 |  |  |  | Adding the deployment use case. | 0.2.0 |
| 2017-08 | SA5#114 | S5-174452 |  |  |  | Adding the deployment requirements. | 0.2.0 |
| 2017-08 | SA5#114 | S5-174453 |  |  |  | Adding the policy operation requirements. | 0.2.0 |
| 2017-08 | SA5#114 | S5-174454 |  |  |  | Adding the general process. | 0.2.0 |
| 2017-08 | SA5#114 | S5-174261 |  |  |  | Modification on the concepts. | 0.2.0 |
| 2017-10 | SA5#115 | S5-175457 |  |  |  | The alignment of NFV policy terms and concept definition with ETSI MANO policy study | 0.3.0 |
| 2017-10 | SA5#115 | S5-175140 |  |  |  | The terms of NFV policy update | 0.3.0 |
| 2017-12 | SA5#116 | S5-176587 |  |  |  | Adding the PVF concept | 0.4.0 |
| 2017-12 | SA5#116 | S5-176588 |  |  |  | Adding the NFV policy management architecture | 0.4.0 |
| 2017-12 | SA5#116 | S5-176589 |  |  |  | Updating the UCs after the concept modification | 0.4.0 |
| 2018-01 | SA5#116 | S5-176589 |  |  |  | Adding NFV policy conflict detection and resolution | 0.5.0 |
| 2018-04 | SA5#118 | S5-182567 |  |  |  | Adding General message flow for policy related operation and adding the mapping of Policy related function to MANO functional blocks | 0.6.0 |
| 2018-05 | SA5#119 | S5-183084 |  |  |  | Modification on the general procedure of policy operation | 0.7.0 |
| 2018-05 | SA5#119 | S5-183086 |  |  |  | typo correction | 0.7.0 |
| 2018-05 | SA5#119 | S5-183591 |  |  |  | Conclusions and Recommendations | 0.7.0 |
| 2018-06 | SA#80 | SP-180416 |  |  |  | Implementing EditHelp's comments. Presented for information and approval | 1.0.0 |
| 2018-06 | SA#80 |  |  |  |  | Upgrade to change control version | 15.0.0 |