3GPP TS 34.131 V15.0.0(2018-07)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

Test Specification for C-language binding to

(Universal) Subscriber Interface Module ((U)SIM)

Application Programming Interface (API)

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

GSM, UMTS, LTE, SIM, API

3GPP

Postal address

3GPP support office address

650 Route des Lucioles – Sophia Antipolis

Valbonne – France

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

©  2018, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	10

1	Scope	11

2	References	11

3	Definitions and Acronyms	12

3.1	Definitions	12

3.2	Acronyms	12

4	Test Environment	13

4.1	Applicability	13

4.2	Test Environment Description	13

4.3	Test Format	14

4.3.1	Test Area Reference	14

4.3.1.1	Conformance Requirements	14

4.3.1.2	Test Area Files	15

4.3.1.3	Test Procedure	15

4.3.1.4	Test Coverage	15

4.4	Initial Conditions	15

4.5	Test Equipment	15

4.5.1	APDU Tool	15

4.6	Testing Methodology	16

4.6.1	Test Interfaces and Facilities	16

5	Test Plan	16

6	API Test Plan	16

6.1	UICC File Store Access	16

6.1.1	CatSelect	16

6.1.1.1	Conformance Requirements	16

6.1.1.2	Test Procedure	17

6.1.1.3	Test Coverage	18

6.1.2	CatStatus	18

6.1.2.1	Conformance Requirements	18

6.1.2.2	Test Procedure	18

6.1.2.3	Test Coverage	19

6.1.3	CatGetCHVStatus	19

6.1.3.1	Conformance Requirements	19

6.1.3.2	Test Procedure	19

6.1.3.3	Test Coverage	19

6.1.4	CatReadBinary	19

6.1.4.1	Conformance Requirements	20

6.1.4.2	Test Procedure	20

6.1.4.3	Test Coverage	21

6.1.5	CatUpdateBinary	21

6.1.5.1	Conformance Requirements	21

6.1.5.2	Test Procedure	22

6.1.5.3	Test Coverage	22

6.1.6	CatReadRecord	23

6.1.6.1	Conformance Requirements	23

6.1.6.2	Test Procedure	24

6.1.6.3	Test Coverage	26

6.1.7	CatUpdateRecord	26

6.1.7.1	Conformance Requirements	26

6.1.7.2	Test Procedure	27

6.1.7.3	Test Coverage	29

6.1.8	CatSearch	29

6.1.8.1	Conformance Requirements	29

6.1.8.2	Test Procedure	30

6.1.8.3	Test Coverage	31

6.1.9	CatIncrease	31

6.1.9.1	Conformance Requirements	31

6.1.9.2	Test Procedure	32

6.1.9.3	Test Coverage	33

6.1.10	CatInvalidate	33

6.1.10.1	Conformance Requirements	33

6.1.10.2	Test Procedure	33

6.1.10.3	Test Coverage	34

6.1.11	CatRehabilitate	34

6.1.11.1	Conformance Requirements	34

6.1.11.2	Test Procedure	35

6.1.11.3	Test Coverage	35

6.2	Registry	35

6.2.1	CatSetMenuString	35

6.2.1.1	Conformance Requirements	35

6.2.1.2	Test Procedure	35

6.2.1.3	Test Coverage	36

6.2.2	CatNotifyOnFrameworkEvent	36

6.2.2.1	Conformance Requirements	36

6.2.2.2	Test Suite Files	36

6.2.2.3	Test Procedure	36

6.2.2.4	Test Coverage	37

6.2.3	CatNotifyOnEnvelope	37

6.2.3.1	Conformance Requirements	37

6.2.3.2	Test Procedure	37

6.2.3.3	Test Coverage	37

6.2.4	CatNotifyOnEvent	37

6.2.4.1	Conformance Requirements	37

6.2.4.2	Test Procedure	38

6.2.4.3	Test Coverage	38

6.3	Man-Machine Interface	38

6.3.1	CatAddItem	38

6.3.1.1	Conformance Requirements	38

6.3.1.2	Test Procedure	38

6.3.1.3	Test Coverage	39

6.3.2	CatSelectItem	39

6.3.2.1	Conformance Requirements	39

6.3.2.2	Test Procedure	39

6.3.2.3	Test Coverage	39

6.3.3	CatEndSelectItem	39

6.3.3.1	Conformance Requirements	39

6.3.3.2	Test Procedure	40

6.3.3.3	Test Coverage	40

6.3.4	CatDisplayText	40

6.3.4.1	Conformance Requirements	40

6.3.4.2	Test Procedure	41

6.3.4.3	Test Coverage	41

6.3.5	CatGetInKey	41

6.3.5.1	Conformance Requirements	41

6.3.5.2	Test Procedure	42

6.3.5.3	Test Coverage	42

6.3.6	CatGetInput	42

6.3.6.1	Conformance Requirements	42

6.3.6.2	Test Procedure	43

6.3.6.3	Test Coverage	43

6.3.7	CatSetupIdleModeText	43

6.3.7.1	Conformance Requirements	43

6.3.7.2	Test Procedure	44

6.3.7.3	Test Coverage	44

6.3.8	CatPlayTone	44

6.3.8.1	Conformance Requirements	44

6.3.8.2	Test Procedure	44

6.3.8.3	Test Coverage	45

6.4	Timers	45

6.4.1	CatGetTimer	45

6.4.1.1	Conformance Requirements	45

6.4.1.2	Test Procedure	45

6.4.1.3	Test Coverage	45

6.4.2	CatFreeTimer	45

6.4.2.1	Conformance Requirements	45

6.4.2.2	Test Procedure	46

6.4.2.3	Test Coverage	46

6.4.3	CatStartTimer	46

6.4.3.1	Conformance Requirements	46

6.4.3.2	Test Procedure	46

6.4.3.3	Test Coverage	47

6.4.4	CatGetTimerValue	47

6.4.4.1	Conformance Requirements	47

6.4.4.2	Test Procedure	47

6.4.4.3	Test Coverage	47

6.5	Supplementary Card Reader Management	47

6.5.1	CatPowerOnCard	47

6.5.1.1	Conformance Requirements	48

6.5.1.2	Test Procedure	48

6.5.1.3	Test Coverage	48

6.5.2	CatPowerOffCard	48

6.5.2.1	Conformance Requirements	48

6.5.2.2	Test Procedure	49

6.5.2.3	Test Coverage	49

6.5.3	CatPerformCardAPDU	49

6.5.3.1	Conformance Requirements	49

6.5.3.2	Test Procedure	49

6.5.3.3	Test Coverage	50

6.5.4	CatGetReaderStatus	50

6.5.4.1	Conformance Requirements	50

6.5.4.2	Test Procedure	50

6.5.4.3	Test Coverage	50

6.6	Network Services	50

6.6.1	CatGetLocationInformation	50

6.6.1.1	Conformance Requirements	51

6.6.1.2	Test Procedure	51

6.6.1.3	Test Coverage	51

6.6.2	CatGetTimingAdvance	51

6.6.2.1	Conformance Requirements	51

6.6.2.2	Test Procedure	52

6.6.2.3	Test Coverage	52

6.6.3	CatGetIMEI	52

6.6.3.1	Conformance Requirements	52

6.6.3.2	Test Procedure	52

6.6.3.3	Test Coverage	53

6.6.4	CatGetNetworkMeasurementResults	53

6.6.4.1	Conformance Requirements	53

6.6.4.2	Test Procedure	53

6.6.4.3	Test Coverage	53

6.6.5	CatGetDateTimeAndTimeZone	53

6.6.5.1	Conformance Requirements	53

6.6.5.2	Test Procedure	54

6.6.5.3	Test Coverage	54

6.6.6	CatGetLanguage	54

6.6.6.1	Conformance Requirements	54

6.6.6.2	Test Procedure	55

6.6.6.3	Test Coverage	55

6.6.7	CatSetupCall	55

6.6.7.1	Conformance Requirements	55

6.6.7.2	Test Procedure	55

6.6.7.3	Test Coverage	56

6.6.8	CatSendShortMessage	56

6.6.8.1	Conformance Requirements	56

6.6.8.2	Test Procedure	56

6.6.8.3	Test Coverage	56

6.6.9	CatSendSS	57

6.6.9.1	Conformance Requirements	57

6.6.9.2	Test Procedure	57

6.6.9.3	Test Coverage	57

6.6.10	CatSendUSSD	57

6.6.10.1	Conformance Requirements	57

6.6.10.2	Test Procedure	58

6.6.10.3	Test Coverage	58

6.6.11	CatOpenCSChannel	58

6.6.11.1	Conformance Requirements	58

6.6.11.2	Test Procedure	59

6.6.11.3	Test Coverage	59

6.6.12	CatOpenGPRSChannel	59

6.6.12.1	Conformance Requirements	59

6.6.12.2	Test Procedure	59

6.6.12.3	Test Coverage	60

6.6.13	CatCloseChannel	60

6.6.13.1	Conformance Requirements	60

6.6.13.2	Test Procedure	60

6.6.13.3	Test Coverage	60

6.6.14	CatReceiveData	60

6.6.14.1	Conformance Requirements	60

6.6.14.2	Test Procedure	61

6.6.14.3	Test Coverage	61

6.6.15	CatSendData	61

6.6.15.1	Conformance Requirements	61

6.6.15.2	Test Procedure	62

6.6.15.3	Test Coverage	62

6.6.16	CatGetChannelStatus	62

6.6.16.1	Conformance Requirements	62

6.6.16.2	Test Procedure	62

6.6.16.3	Test Coverage	63

6.6.17	CatServiceSearch	63

6.6.17.1	Conformance Requirements	63

6.6.17.2	Test Procedure	63

6.6.17.3	Test Coverage	64

6.6.18	CatGetServiceInformation	64

6.6.18.1	Conformance Requirements	64

6.6.18.2	Test Procedure	64

6.6.18.3	Test Coverage	65

6.6.19	CatDeclareService	65

6.6.19.1	Conformance Requirements	65

6.6.19.2	Test Procedure	66

6.6.19.3	Test Coverage	66

6.6.20	CatRunATCommand	66

6.6.20.1	Conformance Requirements	66

6.6.20.2	Test Procedure	67

6.6.20.3	Test Coverage	67

6.6.21	CatSendDTMFCommad	67

6.6.21.1	Conformance Requirements	67

6.6.21.2	Test Procedure	68

6.6.21.3	Test Coverage	68

6.7	Toolkit Application	68

6.7.1	 main	68

6.7.1.1	Conformance Requirements	68

6.7.1.2	Test Procedure	69

6.7.1.3	Test Coverage	69

6.7.2	 CatGetFrameworkEvent	69

6.7.2.1	Conformance Requirements	69

6.7.2.2	Test Procedure	69

6.7.2.3	Test Coverage	69

6.7.3	CatExit	70

6.7.3.1	Conformance Requirements	70

6.7.3.2	Test Procedure	70

6.7.3.3	Test Coverage	70

6.8	Miscellaneous	70

6.8.1	CatGetTerminalProfile	70

6.8.1.1	Conformance Requirements	70

6.8.1.2	Test Procedure	71

6.8.1.3	Test Coverage	71

6.8.2	CatMoreTime	71

6.8.2.1	Conformance Requirements	71

6.8.2.2	Test Procedure	71

6.8.2.3	Test Coverage	72

6.8.3	CatPollingOff	72

6.8.3.1	Conformance Requirements	72

6.8.3.2	Test Procedure	72

6.8.3.3	Test Coverage	72

6.8.4	CatPollInterval	72

6.8.4.1	Conformance Requirements	72

6.8.4.2	Test Procedure	73

6.8.4.3	Test Coverage	73

6.8.5	CatRefresh	73

6.8.5.1	Conformance Requirements	73

6.8.5.2	Test Procedure	74

6.8.5.3	Test Coverage	74

6.8.6	CatLanguageNotification	74

6.8.6.1	Conformance Requirements	74

6.8.6.2	Test Procedure	74

6.8.6.3	Test Coverage	75

6.8.7	CatLaunchBrowser	75

6.8.7.1	Conformance Requirements	75

6.8.7.2	Test Procedure	75

6.8.7.3	Test Coverage	76

6.9	Low-Level Interface	76

6.9.1	CatResetBuffer	76

6.9.1.1	Conformance Requirements	76

6.9.1.2	Test Procedure	76

6.9.1.3	Test Coverage	76

6.9.2	CatStartProactiveCommand	76

6.9.2.1	Conformance Requirements	76

6.9.2.2	Test Procedure	77

6.9.2.3	Test Coverage	77

6.9.3	CatSendProactiveCommand	77

6.9.3.1	Conformance Requirements	77

6.9.3.2	Test Procedure	78

6.9.3.3	Test Coverage	78

6.9.4	CatOpenEnvelope	78

6.9.4.1	Conformance Requirements	78

6.9.4.2	Test Procedure	78

6.9.4.3	Test Coverage	79

6.9.5	CatSendEnvelopeResponse	79

6.9.5.1	Conformance Requirements	79

6.9.5.2	Test Procedure	79

6.9.5.3	Test Coverage	80

6.9.6	CatSendEnvelopeErrorResponse	80

6.9.6.1	Conformance Requirements	80

6.9.6.2	Test Procedure	80

6.9.6.3	Test Coverage	80

6.9.7	CatPutData	80

6.9.7.1	Conformance Requirements	81

6.9.7.2	Test Procedure	81

6.9.7.3	Test Coverage	81

6.9.8	CatPutByte	81

6.9.8.1	Conformance Requirements	82

6.9.8.2	Test Procedure	82

6.9.8.3	Test Coverage	82

6.9.9	CatPutTLV	82

6.9.9.1	Conformance Requirements	82

6.9.9.2	Test Procedure	83

6.9.9.3	Test Coverage	83

6.9.10	CatPutBytePrefixedTLV	83

6.9.10.1	Conformance Requirements	83

6.9.10.2	Test Procedure	83

6.9.10.3	Test Coverage	84

6.9.11	CatPutOneByteTLV	84

6.9.11.1	Conformance Requirements	84

6.9.11.2	Test Procedure	84

6.9.11.3	Test Coverage	85

6.9.12	CatPutTwoByteTLV	85

6.9.12.1	Conformance Requirements	85

6.9.12.2	Test Procedure	85

6.9.12.3	Test Coverage	85

6.9.13	CatGetByte	85

6.9.13.1	Conformance Requirements	85

6.9.13.2	Test Procedure	86

6.9.13.3	Test Coverage	86

6.9.14	CatGetData	86

6.9.14.1	Conformance Requirements	86

6.9.14.2	Test Procedure	86

6.9.14.3	Test Coverage	87

6.9.15	CatFindNthTLV	87

6.9.15.1	Conformance Requirements	87

6.9.15.2	Test Procedure	87

6.9.15.3	Test Coverage	88

6.9.16	CatFindNthTLVInUserBuffer	88

6.9.16.1	Conformance Requirements	88

6.9.16.2	Test Procedure	89

6.9.16.3	Test Coverage	89

Annex A (normative):	Script file syntax and format description	90

A.1	Syntax description	90

A.2	Semantics	91

A.3	Example	91

A.4	Style and formatting	92

Annex B (normative):	Default Prepersonalisation	93

B.1	General Default Prepersonalisation	93

B.2	File System Access Default Prepersonalisation	94

B.2.1	DFSIMTEST (SIM Test)	94

B.2.2	EFTNR (Transparent Never Read)	94

B.2.3	EFTNU (Transparent Never Update)	94

B.2.4	EFTARU (Transparent Always Read and Update)	94

B.2.5	EFCNR (Cyclic Never Read)	95

B.2.6	EFCNU (Cyclic Never Update)	95

B.2.7	EFCNIC (Cyclic Never Increase)	95

B.2.8	EFCNIV (Cyclic Never Invalidate)	96

B.2.9	EFCNRH (Cyclic Never Rehabilitate)	96

B.2.10	EFCARU (Cyclic Always Read and Update)	96

B.2.11	EFLNR (Linear Fixed Never Read)	97

B.2.12	EFLNU (Linear Fixed Never Update)	97

B.2.13	EFLARU (Linear Fixed Always Read and Update)	97

B.2.14	EFCINA (Cyclic Increase Not Allowed)	98

B.2.15	EFTRAC (Transparent Read Access Condition CHV2)	98

B.2.16	EFTIAC (Transparent Invalidate Access Condition CHV1)	98

B.2.17	EFCIAC (Cyclic Increase Access Condition CHV2)	99

B.2.18	EFCIAA (Cyclic Increase Access Condition ADM)	99

B.2.19	EFCNRI (Cyclic Never Rehabilitate Invalidated)	99

Annex C (informative):	Change history	100

# Foreword

This Technical Specification has been produced by the 3GPP.

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of this TS, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3 or greater	Indicates TSG approved document under change control.

Y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document describes the technical characteristics and methods of test for testing the SIM API for the C programming language 3GPP TS 31.131 [11] implemented in  the subscriber identity modules for GSM and 3G networks. It specifies the following parts:

-	test applicability

-	test environment description

-	tests format

-	test area reference

-	conformance requirements

-	test suite files

-	test procedure

-	test coverage and,

-	a description of the associated testing tools that may be used.

# 2	References

The following documents contain provisions, which, through reference in this text, constitute provisions of the present document.

- 	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- 	For a specific reference, subsequent revisions do not apply.

- 	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 31.111: "3rd Generation Partnership Project; Technical Specification Group; USIM Application Toolkit (USAT)".

[3]	3GPP TS 23.048: "3rd Generation Partnership Project; Technical Specification Group Terminals; Security Mechanisms for the SIM application toolkit".

[4]	3GPP TS 42.019: "3rd Generation Partnership Project; Technical Specification Group Terminals; Subscriber Identity Module Application Programming Interface (SIM API); Stage 1".

[5]	ISO 639 (1988): "Code for the representation of names of languages".

[6]	3GPP TS 23.038: "Alphabets and language-specific information".

[7]	ISO/IEC 9899 Second Edition 1999-12-01: "Programming Languages – C".

[8]	3GPP TS 11.14: "Specification of the SIM Application Toolkit for the Subscriber Identity Module – Mobile Equipment (SIM – ME) interface" version 4.0.0 Release 4.

[9]	Tool Interface Standard (TIS) Executable and Linking Format Specification Version 1.2

[10]	SYSTEM V Application Binary Interface, Edition 4.1

[11]	3GPP TS 31.131 V1.1.2: "'C'-language binding to (U)SIM API".

[12]	GSM 11.10-1: "Digital cellular telecommunication system (Phase 2+); Mobile Station (MS) conformance specification; Part 1: Conformance specification".

[13]	3GPP TS 51.011: "Specification of the Subscriber Identity Module – Mobile Equipment (SIM-ME) interface".

[14]	ETSI TS 102.221: "UICC-Terminal interface; Physical and logical characteristics".

[15]	ETSI TS 102.226: "Remote APDU Structure for UICC based Applications."

# 3	Definitions and Acronyms

## 3.1	Definitions

The definitions specified in GSM 11.10-1 [12] shall apply, unless otherwise specified in the present clause.

Application: A computer program that defines and implements a useful domain-specific functionality.  The term may apply to the functionality itself, to the representation of the functionality in a programming language, or to the realization of the functionality as executable code.

Application Executable: The representation of an application as collection of executable codes.

Application Program: The representation of an application in a programming language such as assembly language, C, Java, WML or XHTML.

Application Programming Interface: A collection of entry points and data structures that an application program can access when translated into an application executable.

Byte Code: A processor-independent representation of a basic computer operation such as "increment by one" that is executed by computer program called a byte code interpreter.

Data Structure: A memory address that can be accessed by an application executable in order to read or write data.

Entry Point: A memory address that can be branched to by an application executable in order to access functionality defined by an application-programming interface. Depending on the software technology, an entry point is also called a subroutine, a function or a method.

Executable Code: The generic term for either byte code or native code.

Framework : A framework defines a set of Application Programming Interface (API) functions for developing applications and for providing system services to those applications.

Native Code: A processor-dependent representation of a basic computer operation such as "increment by one" that is executed by the hardware circuitry of a computer's central processing unit.

Null Operation: A computer operation that accomplishes nothing.  Abbreviated as NOP and pronounced "No Op".

Toolkit Application: An application that uses the commands described in [2].

## 3.2	Acronyms

For the purpose of the present document, the following abbreviations apply:

AC	Application Code

AID	Application Identifier

APDU	Application Protocol Data Unit

API	Application Programming Interface

CAD	Card Acceptance Device

CPDU	Command Protocol Data Unit

CAT	Card Application Toolkit

DF	Dedicated File

DTMF	Dual Tone Multiple Frequency

EF	Elementary File

FID	File Identifier

GSM	Global System for Mobile communications

IFD	Interface Device

NOP	Null Operation

ME	Mobile Equipment

NVM	Non-Volatile Memory

ROM	Read-Only Memory

SE	Sending Entity

SIM	Subscriber Identity Module

SMS	Short Message Service

STK	SIM ToolKit

TLV	Tag, Length, Value

TPDU	Transport Protocol Data Unit

UICC	(not an acronym)

URL	Uniform Resource Locator

USIM	Universal Subscriber Interface Module

# 4	Test Environment

This clause specifies requirements that shall be met and the testing rules that shall be followed during the test procedure.

## 4.1	Applicability

The tests defined in this specification shall be performed taking into account the services supported by the card as specified in the EFSST file.

The tests can be performed with either C source code files (.c) or with ELF loadfiles (.elf).

## 4.2	Test Environment Description

The C SIM API test specification covers applications that are linked into the smart card mask and stored in read-only memory as well as applications that are loaded into read/write memory after it has been manufactured. The C SIM API test specification also covers applications whose executable form is either as byte codes or as native codes.

The general architecture for the test environment is given in the following diagram:

## 4.3	Test Format

### 4.3.1	Test Area Reference

Each test area is referenced as follows:

API: API Testing:  'API_[entry point name]' where

entry point name:

API entry point defined in 3GPP TS 31.131 [11]

#### 4.3.1.1	Conformance Requirements

The conformance requirements are expressed in the following way:

-	Entry point prototype as listed in 3GPP TS 31.131 [11] specification.

-	Normal execution:

-	Contains normal execution and correct parameters limit values, each referenced as a Conformance Requirement Reference Normal (CRRN)

-	Parameters error:

-	Contains parameter errors and incorrect parameter limit values, each referenced as a Conformance Requirement Reference Parameter Error (CRRP)

-	Context error:

-	Contains errors due to the context the entry point is used in, each referenced as a Conformance Requirement Reference Context Error (CRRC)

#### 4.3.1.2	Test Area Files

The test area contains the source code of a program that translates the test procedure found in each section below into C test application program (.c) and an APDU Tool test script (.s).  A test is conducted by compiling the C program, installing it on the UICC, and running the associated test script.

The test area also contains the C header files that are referenced by the output of the translator. The test scripts must be processed through a C pre-processor [7] before they are provided to the APDU Tool.

#### 4.3.1.3	Test Procedure

Each test procedure contains pseudo-code that describes the test together with the expected responses from the API and the expected command and response APDU traffic.  The test procedure may be translated into a C test application program and APDU Tool test script using the program in the Test Area

#### 4.3.1.4	Test Coverage

The table at the end of each test procedure indicates the correspondence between the Conformance Requirements Reference (CRR) and the different test cases.

## 4.4	Initial Conditions

The Initial Conditions are a set of general prerequisites for the (U)SIM prior to the execution of testing. For each test procedure described in this document, the following rules apply to the Initial Conditions:

-	unless otherwise stated, the file system and the files' content shall 15ulfil the requirements described in the "Default Prepersonalisation" paragraph at the time of running the test with test number 1;

-	unless otherwise stated, tests with test number 1 through i have been successfully executed before the test with test number i+1 is executed.

When both statements apply, a test procedure is said to be in the "Default Initial Conditions" state.

## 4.5	Test Equipment

These subclauses recommend a minimum specification for each of the items of test equipment referenced in the tests.

### 4.5.1	APDU Tool

This test tool shall meet the following requirements:

-	be able to send commands to the card TPDU;

-	be able to check none, only a part, or all of the data returned;

-	be able to check none, only part, or all of the status returned;

-	be able to accept all valid status codes returned;

-	be able to support CAD commands;

-	be able to generate a log file for each test execution.

-	if more data is returned than defined in the test specification the event shall be noted in the log and the tool shall continue;

-	if less data is returned than defined in the test specification the event shall be noted in the log and the tool shall continue;

-	if there is an error in data or status returned the event shall be noted in the log and the tool shall continue.

The log file produced by the test tool shall include the following information:

-	all commands issued;

-	all data returned;

-	all status returned;

-	all errors codes;

-	expected data and status in case of error;

-	comments from the scripts;

a log message to report success or failure of the test;

transmission error events including anomalies in returned data;

## 4.6	Testing Methodology

### 4.6.1	Test Interfaces and Facilities

The SIM-ME interface provides the main transport interface for the purpose of performing conformance tests.

The SIM API interface provides the main test interface for the purpose of performing conformance tests.

# 5	Test Plan

The test plan is divided according to the entry point sections of 3GPP TS 31.131 [11] although the sections of the test plan do not appear in the same order as the corresponding sections in TS 31.131 [11].

# 6	API Test Plan

## 6.1	UICC File Store Access

### 6.1.1	CatSelect

Test Area Reference: API_CatSelect

#### 6.1.1.1	Conformance Requirements

The entry point with the following signature shall be compliant to its definition in the API.

WORD CatSelect(CatFID FileIdentifier, CatFileStatus *Status)

6.1.1.1.1	Normal execution

CRRN1: If the desired file is selected, the proper status information has been returned in *Status.

CRRN2: After selecting a DF/MF no EF is selected.

CRRN3: After selecting a linear fixed EF no record is selected.

CRRN4: After selecting a cyclic EF the first record which is the last updated record is selected.

CRRN5: The file with a file identifier that matches fid shall be found according to the following selection rules:

1)	An immediate child EF or DF of the current MF/DF can be selected,

2)	A sibling DF of the current DF can be selected,

3)	The current MF/DF it self can be selected,

4)	The parent MF/DF of the current DF can be selected,

The MF can always be selected.

CRRN6: If status is NULL the invocation is a NOP.

6.1.1.1.2	Parameter errors

6.1.1.1.3	Context errors

CRRC1: If the file with a file identitifier which matches FileIdentitifer could not be found according to the selection rules listed in CRRN5 then the FILE_NOT_FOUND status word shall be returned.

CRRC2: If the entry point call causes a memory problem (e.g. memory access error), the MEMORY_PROBLEM status word shall be returned.

CRRC3: If the entry point call causes an error to occur that is not expected and thus not handled, the INTERNAL_ERROR status word shall be returned.

#### 6.1.1.2	Test Procedure

SET Test TO CatSelect.

/* Test Case 1: Select MF in MF (Transparent EF) */

IF   {CatSelect(FID_MF, &FCP) == SW_OK}

THEN {FCP.fileType == FT_MF}.

/* Test Case 2: Select EF-ICCID in MF (Transparent EF) */

IF   {CatSelect(FID_EF_ICCID, &FCP) == SW_OK}

THEN {FCP.fileType == FT_EF}.

/* Test Case 3: Select DF-GSM in MF*/

IF   {CatSelect(FID_DF_GSM, &FCP) == SW_OK}

THEN {FCP.fileType == FT_DF}.

/* Test Case 4: Select EF-ACM in DF-GSM (Cyclic EF) */

IF   {CatSelect(FID_EF_ACM, &FCP) == SW_OK}

THEN {FCP.fileType == FT_EF}.

/* Test Case 5: Select MF*/

IF   {CatSelect(FID_MF, &FCP) == SW_OK}

THEN {FCP.fileType == FT_MF}.

/* Test Case 6: Select DF-TELECOM in MF */

IF   {CatSelect(FID_DF_TELECOM, &FCP) == SW_OK}

THEN {FCP.fileType == FT_DF}.

/* Test Case 7: Select EF-FDN in DF-TELECOM (Linear Fixed EF) */

IF   {CatSelect(FID_EF_FDN, &FCP) == SW_OK}

THEN {FCP.fileType == FT_EF}.

/* Test Case 8: Status is null*/

IF   {CatSelect(FID_EF_FDN, NULL) == SW_OK}

THEN {FCP.fileType == FT_EF}.

/* Test Case 9: EF not selected after MF/DF selection */

IF   {CatSelect(FID_MF, &FCP) == SW_OK}

AND  {CatSelect(FID_EF_ICCID, &FCP) == SW_OK}

AND  {CatSelect(FID_MF, &FCP) == SW_OK}

THEN {CatReadBinary(0, &bytes, buffer) == SW_NO_CURRENT_FILE}

AND  {FCP.fileType == FT_UNKNOWN}.

/* Test Case 10: Selected file not available */

IF   {CatSelect(FID_MF, &FCP) == SW_OK}

THEN {CatSelect(FID_EF_ACM, &FCP) == SW_FILE_NOT_FOUND}

AND  {FCP.fileType == FT_UNKNOWN}.

/* Test Case 11: No record is selected after selecting linear fixed EF */

IF   {CatSelect(FID_MF, &FCP) == SW_OK}

AND  {CatSelect(FID_DF_SIMTEST, &FCP) == SW_OK}

AND  {CatSelect(FID_EF_LARU, &FCP) == SW_OK}

THEN {CatReadRecord(0, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, &bytes, buffer) == SW_RECORD_NOT_FOUND}

AND  {FCP.fileType == FT_EF}.

#### 6.1.1.3	Test Coverage

| CRR Number | Test Case Number |
| --- | --- |
| N1 | 1,2, 3 |
| N2 | 4 |
| N3 | 5 |
| N4 | 6 |
| N5 | 7 |
| N6 | 8 |
| C1 | 9, 10, 11 |
| C2, C3 | Not Tested |

### 6.1.2	CatStatus

Test Area Reference: API_CatStatus

#### 6.1.2.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

WORD CatStatus(CatFileStatus *Status)

6.1.2.1.1	Normal execution

CRRN1: The status information of the current EF is returned.

CRRN2: If Status is NULL the invocation is a NOP.

6.1.2.1.2	Parameter errors

6.1.2.1.3	Context errors

#### 6.1.2.2	Test Procedure

SET Test TO CatStatus.

/* Test Case 1: Status of MF */

IF   {CatSelect(FID_MF, &FCP) == SW_OK}

AND  {CatStatus(&Status) == SW_OK}

THEN {memcmp(&FCP, &Status, sizeof(CatDFStatus)) == 0}

AND  {Status.fileType == FT_MF}.

/* Test Case 2: Status of EF-ICCID */

IF   {CatSelect(FID_EF_ICCID, &FCP) == SW_OK}

AND  {CatStatus(&Status) == SW_OK}

THEN {memcmp(&FCP, &Status, sizeof(CatEFStatus)) == 0}.

/* Test Case 3: Status of DF-TELECOM /

IF   {CatSelect(FID_DF_TELECOM, &FCP) == SW_OK}

AND  {CatStatus(&Status) == SW_OK}

THEN {memcmp(&FCP, &Status, sizeof(CatDFStatus)) == 0}.

/* Test Case 4: Status with NULL argument */

IF   {CatStatus(NULL) == SW_OK}

THEN {NOP}.

#### 6.1.2.3	Test Coverage

| CRR Number | Test Case Number |
| --- | --- |
| N1 | 1, 2, 3 |
| N2 | 4 |

### 6.1.3	CatGetCHVStatus

Test Area Reference: API_CatGetCHVStatus

#### 6.1.3.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

WORD CatGetCHVStatus(BYTE *CHV)

6.1.3.1.1	Normal execution

CRRN1: The current CHV status information is returned.

6.1.3.1.2	Parameter errors

6.1.3.1.3	Context errors

#### 6.1.3.2	Test Procedure

SET Test TO CatGetCHVStatus.

/* Test Case 1: Get the ATR CHV Status */

IF   {(CatGetCHVStatus(CHVStatus),SW_OK) == SW_OK}

THEN {CHVStatus[0] == 0}

AND  {CHVStatus[1] == 0}

AND  {CHVStatus[2] == 0}

AND  {CHVStatus[3] == 0}.

#### 6.1.3.3	Test Coverage

| CRR Number | Test Case Number |
| --- | --- |
| N1 | 1 |

### 6.1.4	CatReadBinary

Test Area Reference: API_CatReadBinary

#### 6.1.4.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

WORD CatReadBinary(DWORD Offset, DWORD *Bytes, void *Buffer);

6.1.4.1.1	Normal execution

CRRN1: If data can be accessed at the specified offset in the currently selected transparent file, an attempt is made to read the number of bytes in Bytes.  The bytes read are returned in Buffer and the actual number of bytes read returned in Bytes.

CRRN2: If Bytes is non-null and Buffer is null, then the number of bytes that could be read is returned in Bytes.

CRRN3: If Bytes is null or *Bytes is zero the invocation is a NOP

6.1.4.1.2	Parameter errors

CRRP1: If Offset exceeds the length of the file, the status word OUT_OF_FILE_BOUNDARIES shall be returned.

6.1.4.1.3	Context errors

CRRC1: If there is no currently selected EF,  the status word NO_EF_SELECTED shall be returned.

CRRC2: If the currently selected EF is not transparent, the status word FILE_INCONSISTENT shall be returned.

CRRC3: If the calling application does not fulfil the access condition, READ, to perform this function, the status word SECURITY_CONDITION_NOT_SATISFIED shall be returned.

CRRC4: If the currently selected EF is invalidated and the file status of the EF does not allow for the reading of an invalidated file, the status word INVALIDATION_STATUS_CONTRADICTION shall be returned.

CRRC5: If the entry point call causes a memory problem (e.g. memory access error), an instance the status word MEMORY_PROBLEM shall be returned.

CRRC6: If the entry point call causes an error to occur that is not expected and thus not handled, an instance of the status word INTERNAL_ERROR shall be returned.

#### 6.1.4.2	Test Procedure

SET Test TO CatReadBinary

AND Data TO {0x0F, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF}.

/* Test Case 1: Read from EF-ICCID in MF */

IF   {CatSelect(FID_EF_ICCID, NULL) == SW_OK}

AND  {memset(buffer, 0, sizeof(buffer))}

AND  {bytes = 5}

THEN {CatReadBinary(0, &bytes, buffer) == SW_OK}

AND  {bytes == 5}

AND  {memcmp(buffer, Data, bytes) == 0}.

/* Test Case 2: Offset < EOF < Offset+Bytes */

IF   {CatSelect(FID_EF_ICCID, NULL) == SW_OK}

AND  {memset(buffer, 0, sizeof(buffer))}

AND  {bytes = 5}

THEN {CatReadBinary(6, &bytes, buffer) == SW_OK}

AND  {bytes == 4}

AND  {memcmp(buffer, &Data[1], bytes) == 0}.

/* Test Case 3: EOF < Offset */

IF   {CatSelect(FID_EF_ICCID, NULL) == SW_OK}

AND  {memset(buffer, 0, sizeof(buffer))}

AND  {bytes = 5}

THEN {CatReadBinary(100, &bytes, buffer) == SW_OK}

AND  {bytes == 0}.

/* Test Case 4: Buffer is null */

IF   {CatSelect(FID_EF_ICCID, NULL) == SW_OK}

AND  {bytes = 5}

THEN {CatReadBinary(6, &bytes, NULL) == SW_OK}

AND  {bytes == 4}.

/* Test Case 5: EF is not Transparent */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_LARU, NULL) == SW_OK}

THEN {CatReadBinary(0, &bytes, NULL) == SW_INCOMPATIBLE_FILE_STRUCTURE}.

/* Test Case 6: Access condition not fulfilled */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_TNR, NULL) == SW_OK}

THEN {CatReadBinary(0, &bytes, NULL) == SW_SECURITY_STATUS_NOT_SATISFIED}.

/* Test Case 7: EF is invalidated */

IF   {CatSelect(FID_EF_TNU, NULL) == SW_OK}

AND  {(CatInvalidate(), SW_OK) == SW_OK}

THEN {CatReadBinary(0, &bytes, NULL) == SW_REFERENCED_DATA_INVALIDATED}.

/* Test Case 8: No EF selected */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

THEN {CatReadBinary(0, &bytes, NULL) == SW_NO_CURRENT_FILE}.

/* Test Case 9: NULL arguments */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_TARU, NULL) == SW_OK}

THEN {CatReadBinary(0, NULL, NULL) == SW_OK}.

#### 6.1.4.3	Test Coverage

| CRR Number | Test Case Number |
| --- | --- |
| N1 | 1,2 |
| N2 | 4 |
| N3 | 9 |
| P1 | 3 |
| C1 | 8 |
| C2 | 5 |
| C3 | 6 |
| C4 | 7 |
| C5, C6 | Not tested |

### 6.1.5	CatUpdateBinary

Test Area Reference: API_CatUpdateBinary

#### 6.1.5.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

UINT16 CatUpdateBinary(DWORD Offset, DWORD Bytes, void *Buffer);

6.1.5.1.1	Normal execution

CRRN1: The currently selected transparent file is updated starting at Offset with the sequence of Bytes bytes in Buffer.

6.1.5.1.2	Parameter errors

CRRP1: If Bytes is positive and Buffer is null, then INCORRECT_PARAMETERS shall be returned.

6.1.5.1.3	Context errors

CRRC1: If there is no currently selected EF,  the status word NO_EF_SELECTED shall be returned.

CRRC2: If the currently selected EF is not transparent, the status word FILE_INCONSISTENT shall be returned.

CRRC3: If the calling application does not fulfil the access condition, READ, to perform this function, the status word SECURITY_CONDITION_NOT_SATISFIED shall be returned.

CRRC4: If the currently selected EF is invalidated and the file status of the EF does not allow for the reading of an invalidated file, the status word INVALIDATION_STATUS_CONTRADICTION shall be returned.

CRRC5: If the entry point call causes a memory problem (e.g. memory access error), an instance the status word MEMORY_PROBLEM shall be returned.

CRRC6: If the entry point call causes an error to occur that is not expected and thus not handled, an instance of the status word INTERNAL_ERROR shall be returned.

#### 6.1.5.2	Test Procedure

SET Test TO CatUpdateBinary

AND Data TO {1,2,3,4,5}.

/* Test Case 1: Update Transparent EF */

IF   {CatSelect(FID_EF_ICCID, NULL) == SW_OK}

AND  {bytes = 5}

THEN {CatUpdateBinary(0, bytes, Data) == SW_OK}

AND  {CatReadBinary(0, &bytes, buffer) == SW_OK}

AND  {bytes == 5}

AND  {memcmp(Data, buffer, bytes) == 0}.

/* Test Case 2: Offset < EOF and Offset + Bytes > EOF*/

IF   {CatSelect(FID_EF_ICCID, NULL) == SW_OK}

AND  {bytes = 5}

THEN {CatUpdateBinary(7, bytes, Data) == SW_OK}

AND  {CatReadBinary(7, &bytes, buffer) == SW_OK}

AND  {bytes == 5}

AND  {memcmp(Data, buffer, bytes) == 0}.

/* Test Case 3: Offset > EOF */

IF   {CatSelect(FID_EF_ICCID, NULL) == SW_OK}

THEN {CatUpdateBinary(11, 5, Data) == SW_OK}

AND  {CatReadBinary(11, &bytes, buffer) == SW_OK}

AND  {bytes == 5}

AND  {memcmp(Data, buffer, bytes) == 0}.

/* Test Case 4: Buffer is NULL */

IF   {CatSelect(FID_EF_ICCID, NULL) == SW_OK}

THEN {CatUpdateBinary(0, 5, NULL) == SW_OK}.

/* Test Case 5: No EF selected */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

THEN {CatUpdateBinary(0, 5, Data) == SW_NO_CURRENT_FILE}.

/* Test Case 6: EF is not transparent */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_LARU, NULL) == SW_OK}

THEN {CatUpdateBinary(0, 5, Data) == SW_INCOMPATIBLE_FILE_STRUCTURE}.

/* Test Case 7: Access condition not fulfilled */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_TNU, NULL) == SW_OK}

THEN {CatUpdateBinary(0, 5, Data) == SW_SECURITY_STATUS_NOT_SATISFIED}.

/* Test Case 8: EF is invalidated*/

IF   {CatSelect(FID_EF_TNU, NULL) == SW_OK}

AND  {(CatInvalidate(), SW_OK) == SW_OK}

THEN {CatUpdateBinary(0, 5, Data) == SW_REFERENCED_DATA_INVALIDATED}.

#### 6.1.5.3	Test Coverage

| CRR Number | Test Case Number |
| --- | --- |
| N1 | 1,2,3 |
| P1 | 4 |
| C1 | 5 |
| C2 | 6 |
| C3 | 7 |
| C4 | 8 |
| C5, C6 | Not Tested |

### 6.1.6	CatReadRecord

Test Area Reference: API_CatReadRecord

#### 6.1.6.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

UINT16 CatReadRecord(DWORD RecordNumber, CatRecordAccessModes Mode, 
                         DWORD Offset, DWORD *NumBytes, void *Buffer);

6.1.6.1.1	Normal execution

CRRN1: NumBytes bytes starting at Offset from the record specified by Mode and RecordNumber of the currently selected linear fixed or cyclic EF are read into Buffer.

CRRN2: If the access mode is ABSOLUTE or CURRENT:

-	if RecordNumber is not 0, the record addressed by RecordNumber will be read;

-	if RecordNumber is 0 the current selected record will be read; and

-	the current record pointer shall not change.

CRRN3: If the access mode is NEXT:

-	the next record relative to the current selected record will be selected and read;

-	if no current record is selected, the first record will be selected and read;

-	if the current record pointer is set to the last record for a cyclic EF the record pointer is set to the first record and the record is read;

-	the current record pointer of any other Application shall not be changed.

CRRN4: If the access mode is PREVIOUS:

-	the previous record relative to the current selected record will be selected and read;

-	if no current record is selected, the last record will be selected and read;

-	if the current record pointer is set to the first record, for a linear fixed EF the entry point responses with an error exception and for a cyclic EF the record pointer is set to the last record and the record is read;

-	the current record pointer of any other Application shall not be changed.

6.1.6.1.2	Parameter errors

CRRP1: If the currently selected EF is linear fixed and the access mode is ABSOLUTE and RecordNumber is greater than records available,  the status word RECORD_NUMBER_NOT_AVAILABLE shall be returned

CRRP2: If the currently selected EF is linear fixed and the access mode is CURRENT, RecordNumber is 0 and there is no current record selected,  the status word RECORD_NUMBER_NOT_AVAILABLE shall be returned.

CRRP3: If the currently selected EF is linear fixed and the access mode is NEXT and the current record pointer is set to the last record, the status word RECORD_NUMBER_NOT_AVAILABLE shall be returned.

CRRP4: If the currently selected EF is linear fixed and the access mode is PREVIOUS and the current record pointer is set to the first record,  the status word RECORD_NUMBER_NOT_AVAILABLE shall be returned.

6.1.6.1.3	Context errors

CRRC1: If the calling Application has currently no EF selected,  the status word NO_EF_SELECTED shall be returned.

CRRC2: If the currently selected EF is neither linear fixed nor cyclic, the status word FILE_INCONSISTENT shall be returned.

CRRC3: If the calling Application does not fulfil the access condition, READ, to perform this function,  the status word AC_NOT_FULFILLED shall be returned.

CRRC4: If the currently selected EF is invalidated and the file status of the EF does not allow for reading an invalidated file, the status word INVALIDATION_STATUS_CONTRADICTION shall be returned.

CRRC5: If the entry point call causes a memory problem (e.g. memory access error), the status word MEMORY_PROBLEM shall be returned.

CRRC6: If the entry point call causes an error to occur that is not expected and thus not handled, the status word INTERNAL_ERROR shall be returned.

#### 6.1.6.2	Test Procedure

SET Test TO CatReadRecord

AND Data55 TO {0x55,0x55,0x55,0x55}

AND DataAA TO {0xAA,0xAA,0xAA,0xAA}.

/* Test Case 1: Read Absolute Linear Fixed EF */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {memset(buffer, 0, sizeof(buffer))}

AND  {bytes = 4}

THEN {CatReadRecord(1, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, &bytes, buffer) == SW_OK}

AND  {bytes == 4}

AND  {memcmp(buffer, Data55, bytes) == 0}.

/* Test Case 2: Read Current Fixed EF */

IF   {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {memset(buffer, 0, sizeof(buffer))}

AND  {bytes = 4}

THEN {CatReadRecord(0, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, &bytes, buffer) == SW_OK}

AND  {bytes == 4}

AND  {memcmp(buffer, Data55, bytes) == 0}.

/* Test Case 3: Read Next from Linear Fixed EF */

IF   {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {memset(buffer, 0, sizeof(buffer))}

AND  {bytes = 4}

THEN {CatReadRecord(0, REC_ACC_MODE_NEXT, 0, &bytes, buffer) == SW_OK}

AND  {bytes == 4}

AND  {memcmp(buffer, DataAA, bytes) == 0}.

/* Test Case 4: Read  Next from Linear Fixed EF */

IF   {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {memset(buffer, 0, sizeof(buffer))}

AND  {bytes = 0}

THEN {CatReadRecord(0, REC_ACC_MODE_NEXT, 0, &bytes, buffer) == SW_RECORD_NOT_FOUND}.

/* Test Case 5: Read Previous from Linear Fixed EF */

IF   {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {memset(buffer, 0, sizeof(buffer))}

AND  {bytes = 4}

THEN {CatReadRecord(0, REC_ACC_MODE_PREVIOUS, 0, &bytes, buffer) == SW_OK}

AND  {bytes == 4}

AND  {memcmp(buffer, Data55, bytes) == 0}.

/* Test Case 6: Read Previous from Linear Fixed EF */

IF   {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {memset(buffer, 0, sizeof(buffer))}

AND  {bytes = 0}

THEN {CatReadRecord(0, REC_ACC_MODE_PREVIOUS, 0, &bytes, buffer) == SW_RECORD_NOT_FOUND}.

/* Test Case 7: Read Absolute and Current from Cyclic EF */

IF   {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {memset(buffer, 0, sizeof(buffer))}

AND  {bytes = 4}

THEN {CatReadRecord(0, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, &bytes, buffer) == SW_OK}

AND  {bytes == 4}

AND  {memcmp(buffer, Data55, bytes) == 0}.

/* Test Case 8: Read Next from Cyclic EF */

IF   {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {memset(buffer, 0, sizeof(buffer))}

AND  {bytes = 4}

THEN {CatReadRecord(0, REC_ACC_MODE_NEXT, 0, &bytes, buffer) == SW_OK}

AND  {bytes == 4}

AND  {memcmp(buffer, DataAA, bytes) == 0}.

/* Test Case 9: Read Next from Cyclic EF */

IF   {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {memset(buffer, 0, sizeof(buffer))}

THEN {CatReadRecord(0, REC_ACC_MODE_NEXT, 0, &bytes, buffer) == SW_OK}

AND  {bytes == 4}

AND  {memcmp(buffer, Data55, bytes) == 0}.

/* Test Case 10: Read Previous from Cyclic EF */

IF   {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {memset(buffer, 0, sizeof(buffer))}

AND  {bytes = 4}

THEN {CatReadRecord(0, REC_ACC_MODE_PREVIOUS, 0, &bytes, buffer) == SW_OK}

AND  {bytes == 4}

AND  {memcmp(buffer, DataAA, bytes) == 0}.

/* Test Case 11: Read Previous from Cyclic EF */

IF   {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {memset(buffer, 0, sizeof(buffer))}

AND  {bytes = 4}

THEN {CatReadRecord(0, REC_ACC_MODE_PREVIOUS, 0, &bytes, buffer) == SW_OK}

AND  {bytes == 4}

AND  {memcmp(buffer, Data55, bytes) == 0}.

/* Test Case 12: NumBytes > Record Length */

IF   {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {bytes = 5}

THEN {CatReadRecord(0, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, &bytes, buffer) == SW_INCORRECT_P1_P2}.

/* Test Case 13: No current record in linear fixed EF */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {bytes = 4}

THEN {CatReadRecord(0, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, &bytes, buffer) == SW_RECORD_NOT_FOUND}.

/* Test Case 14: No EF selected */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_TNR, NULL) == SW_OK}

THEN {CatReadRecord(0, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, &bytes, buffer) == SW_INCOMPATIBLE_FILE_STRUCTURE}.

/* Test Case 15: EF is neither Cyclic nor Linear Fixed */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

THEN {CatReadRecord(0, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, &bytes, buffer) == SW_NO_CURRENT_FILE}.

/* Test Case 16: Access condition not fulfilled */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_LNR, NULL) == SW_OK}

THEN {CatReadRecord(0, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, &bytes, buffer) == SW_SECURITY_STATUS_NOT_SATISFIED}.

/* Test Case 17: EF is invalidated */

IF   {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {(CatInvalidate(), SW_OK) == SW_OK}

THEN {CatReadRecord(0, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, &bytes, buffer) == SW_REFERENCED_DATA_INVALIDATED}.

#### 6.1.6.3	Test Coverage

| CRR Number | Test Case Number |
| --- | --- |
| N1 | 1,2,3,5,7,8,9,10,11 |
| N2 | 1,7 |
| N3 | 3,4,8,9 |
| N4 | 5,6,10,11 |
| P1 | Not Tested |
| P2 | Not Tested |
| P3 | 4 |
| P4 | 6 |
| C1 | 14 |
| C2 | 15 |
| C3 | 16 |
| C4 | 17 |
| C5, C6 | Not Tested |

### 6.1.7	CatUpdateRecord

Test Area Reference: API_CatUpdateRecord

#### 6.1.7.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

UINT16 CatUpdateRecord(DWORD RecordNumber, CatRecordAccessModes Mode, 
                           DWORD Offset, DWORD NumBytes, void *Buffer);

6.1.7.1.1	Normal execution

CRRN1: NumBytes of data starting at Offset in the record specified by Mode and RecordNumber of the current selected linear fixed or cyclic EF are overwritten by the data in Buffer.

CRRN2: If the access mode is ABSOLUTE or CURRENT and the file is a linear fixed EF:

-	the record addressed by RecordNumber will be updated;

-	if RecordNumber is 0 the current selected record will be updated; and

-	the current record pointer shall not change.

CRRN3: If the access mode is NEXT and the file is a linear fixed EF:

-	the next record relative to the current selected record will be selected and updated;

-	if no current record is selected, the first record will be selected and updated;

-	the current record pointer of any other application shall not be changed.

CRRN4: If the access mode is PREVIOUS:

-	the previous record relative to the current selected record will be selected and updated;

-	if no current record is selected, the last record will be selected and updated;

-	if a cyclic EF is updated, the oldest record will be updated independent of the current record pointer and this record becomes record number 1 and the current record;

-	the current record pointer of any other application shall not be changed in case of a linear fixed EF.

6.1.7.1.2	Parameter errors

CRRP1: If the currently selected EF is linear fixed and the access mode is ABSOLUTE and RecordNumber is greater than records available,  the status word RECORD_NUMBER_NOT_AVAILABLE shall be returned

CRRP2: If the currently selected EF is linear fixed and the access mode is CURRENT, RecordNumber is 0 and there is no current record selected,  the status word RECORD_NUMBER_NOT_AVAILABLE shall be returned.

CRRP3: If the currently selected EF is linear fixed and the access mode is NEXT and the current record pointer is set to the last record, the status word RECORD_NUMBER_NOT_AVAILABLE shall be returned.

CRRP4: If the currently selected EF is linear fixed and the access mode is PREVIOUS and the current record pointer is set to the first record,  the status word RECORD_NUMBER_NOT_AVAILABLE shall be returned.

CRRP5: If the currently selected EF is linear fixed and Offset plus NumBytes is greater than the record length, then the status word OUT_OF_FILE_BOUNDARIES shall be returned.

6.1.7.1.3	Context errors

CRRC1: If the calling Application has currently no EF selected,  the status word NO_EF_SELECTED shall be returned.

CRRC2: If the currently selected EF is neither linear fixed nor cyclic, the status word FILE_INCONSISTENT shall be returned.

CRRC3: If the calling Application does not fulfil the access condition, UPDATE, to perform this function,  the status word AC_NOT_FULFILLED shall be returned.

CRRC4: If the currently selected EF is invalidated and the file status of the EF does not allow for reading an invalidated file, the status word INVALIDATION_STATUS_CONTRADICTION shall be returned.

CRRC5: If the entry point call causes a memory problem (e.g. memory access error), the status word MEMORY_PROBLEM shall be returned.

CRRC6: If the entry point call causes an error to occur that is not expected and thus not handled, the status word INTERNAL_ERROR shall be returned.

#### 6.1.7.2	Test Procedure

SET Test TO CatUpdateRecord

AND Data TO {1,2,3,4}.

/* Test Case 1: Update Absolute Record in Linear Fixed EF */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {memset(buffer, 0, sizeof(buffer))}

AND  {bytes = 4}

THEN {CatUpdateRecord(1, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, bytes, Data) == SW_OK}

AND  {bytes == 4}

AND  {CatReadRecord(0, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, &bytes, buffer) == SW_OK}

AND  {memcmp(buffer, Data, bytes) == 0}.

/* Test Case 2: Update Current  Record in Linear Fixed EF */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {memset(buffer, 0, sizeof(buffer))}

AND  {bytes = 4}

THEN {CatUpdateRecord(0, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, bytes, Data) == SW_OK}

AND  {bytes == 4}

AND  {CatReadRecord(0, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, &bytes, buffer) == SW_OK}

AND  {memcmp(buffer, Data, bytes) == 0}.

/* Test Case 3: Update Next from Linear Fixed EF, no record pointer set */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {memset(buffer, 0, sizeof(buffer))}

AND  {bytes = 4}

THEN {CatUpdateRecord(0, REC_ACC_MODE_NEXT, 0, bytes, Data) == SW_OK}

AND  {bytes == 4}

AND  {CatReadRecord(0, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, &bytes, buffer) == SW_OK}

AND  {memcmp(buffer, Data, bytes) == 0}.

/* Test Case 4: Update Next from Linear Fixed EF, record pointer set */

IF   {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {memset(buffer, 0, sizeof(buffer))}

AND  {bytes = 4}

AND  {CatReadRecord(1, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, &bytes, buffer) == SW_OK}

THEN {CatUpdateRecord(0, REC_ACC_MODE_NEXT, 0, bytes, Data) == SW_OK}

AND  {bytes == 4}

AND  {CatReadRecord(0, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, &bytes, buffer) == SW_OK}

AND  {memcmp(buffer, Data, bytes) == 0}.

/* Test Case 5: Update Next from Linear Fixed EF, no more records */

IF   {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {bytes = 4}

THEN {CatUpdateRecord(0, REC_ACC_MODE_NEXT, 0, bytes, Data) == SW_RECORD_NOT_FOUND}.

/* Test Case 6: Update Previous from Linear Fixed EF, no record pointer set */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {memset(buffer, 0, sizeof(buffer))}

AND  {bytes = 4}

THEN {CatUpdateRecord(0, REC_ACC_MODE_PREVIOUS, 0, bytes, Data) == SW_OK}

AND  {bytes == 4}

AND  {CatReadRecord(0, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, &bytes, buffer) == SW_OK}

AND  {memcmp(buffer, Data, bytes) == 0}.

/* Test Case 7: Update Previous from Linear Fixed EF, record pointer set */

IF   {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {memset(buffer, 0, sizeof(buffer))}

AND  {bytes = 4}

AND  {CatReadRecord(2, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, &bytes, buffer) == SW_OK}

THEN {CatUpdateRecord(0, REC_ACC_MODE_PREVIOUS, 0, bytes, Data) == SW_OK}

AND  {bytes == 4}

AND  {CatReadRecord(0, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, &bytes, buffer) == SW_OK}

AND  {memcmp(buffer, Data, bytes) == 0}.

/* Test Case 8: Update Previous from Linear Fixed EF, no more records */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {memset(buffer, 0, sizeof(buffer))}

AND  {bytes = 4}

AND  {CatReadRecord(1, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, &bytes, buffer) == SW_OK}

THEN {CatUpdateRecord(0, REC_ACC_MODE_PREVIOUS, 0, bytes, Data) == SW_RECORD_NOT_FOUND}.

/* Test Case 9: Update Previous from Cyclic EF */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_CARU, NULL) == SW_OK}

AND  {memset(buffer, 0, sizeof(buffer))}

AND  {bytes = 4}

THEN {CatUpdateRecord(0, REC_ACC_MODE_PREVIOUS, 0, bytes, Data) == SW_OK}

AND  {bytes == 4}

AND  {CatReadRecord(0, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, &bytes, buffer) == SW_OK}

AND  {memcmp(buffer, Data, bytes) == 0}.

/* Test Case 10: No EF selected */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

THEN {CatUpdateRecord(0, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, bytes, buffer) == SW_NO_CURRENT_FILE}.

/* Test Case 11: Update Absolute from Linear Fixed EF beyond Records */

IF   {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {bytes = 5}

THEN {CatUpdateRecord(3, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, bytes, buffer) == SW_INCORRECT_P1_P2}.

/* Test Case 12: No current record in linear fixed EF, update current */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {bytes = 4}

THEN {CatUpdateRecord(0, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, bytes, buffer) == SW_RECORD_NOT_FOUND}.

/* Test Case 13: Record Length < Offset + NumBytes  */

IF   {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {bytes = 5}

THEN {CatUpdateRecord(0, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, bytes, buffer) == SW_INCORRECT_P1_P2}.

/* Test Case 14: EF is neither Cyclic nor Linear Fixed */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_TNR, NULL) == SW_OK}

THEN {CatUpdateRecord(0, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, bytes, buffer) == SW_INCOMPATIBLE_FILE_STRUCTURE}.

/* Test Case 15: Access condition not fulfilled */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_LNU, NULL) == SW_OK}

THEN {CatUpdateRecord(0, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, bytes, buffer) == SW_SECURITY_STATUS_NOT_SATISFIED}.

/* Test Case 16: EF is invalidated */

IF   {CatSelect(FID_EF_LARU, NULL) == SW_OK}

AND  {(CatInvalidate(), SW_OK) == SW_OK}

THEN {CatUpdateRecord(0, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, bytes, buffer) == SW_REFERENCED_DATA_INVALIDATED}.

#### 6.1.7.3	Test Coverage

| CRR Number | Test Case Number |
| --- | --- |
| N1 | 1,2,3,4,6,7,9 |
| N2 | 2 |
| N3 | 3,4 |
| N4 | 6,7,9 |
| P1 | 11 |
| P2 | 12 |
| P3 | 5 |
| P4 | 8 |
| P5 | 13 |
| C1 | 10 |
| C2 | 14 |
| C3 | 15 |
| C4 | 16 |
| C5, C6 | Not Tested |

### 6.1.8	CatSearch

Test Area Reference: API_CatSearch

#### 6.1.8.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

UINT16 CatSearch(CatSeekModes Mode, DWORD Offset, DWORD PatternLength, const void *Pattern);

6.1.8.1.1	Normal execution

CRRN1: If the pattern in Pattern with the length PatternLength is found in the record being specified by mode starting at Offset, the current record pointer is set to that record and the record number is returned.

CRRN2: If mode is BEGINNING_FORWARD, the search starts with the first record forward towards the end of the file.

CRRN3: If mode is END_BACKWARD, the search starts with the last record backward towards the beginning of the file.

CRRN4: If mode is NEXT_FORWARD, the search starts from the next record after the current record pointer forward towards the end of file. If no current record pointer is selected, the search starts with the first record.

CRRN5: If mode is PREVIOUS_BACKWARD, the search starts from the previous record before the current record pointer backward towards the beginning of the file. If no current record pointer is selected the search starts with the last record.

CRRN6: If pattern in patt is not found, the status word PATTERN_NOT_FOUND shall be returned.

CRRN7: If mode is NEXT_FORWARD and the record pointer is at the last record, the status word PATTERN_NOT_FOUND shall be returned.

CRRN8: If mode is PREVIOUS_BACKWARD and the record pointer is at the first record, the status word  PATTERN_NOT_FOUND shall be returned.

6.1.8.1.2	Parameter errors

CRRP1: If PatternLength is greater than the size of the record of the currently selected EF, the status word OUT_OF_RECORD_BOUNDARIES shall be returned.

CRRP2: If Offset plus PatternLength is greater than the length of the pattern array patt.length, the status word OUT_OF_RECORD_BOUNDARIES shall be returned.

6.1.8.1.3	Context errors

CRRC1: If the calling Application has currently no EF selected,  the status word NO_EF_SELECTED shall be returned.

CRRC2: If the currently selected EF is neither linear fixed nor cyclic, the status word FILE_INCONSISTENT shall be returned.

CRRC3: If the calling Application does not fulfil the access condition, READ, to perform this function,  the status word AC_NOT_FULFILLED shall be returned.

CRRC4: If the currently selected EF is invalidated and the file status of the EF does not allow for reading an invalidated file, the status word INVALIDATION_STATUS_CONTRADICTION shall be returned.

CRRC5: If the entry point call causes a memory problem (e.g. memory access error), the status word MEMORY_PROBLEM shall be returned.

CRRC6: If the entry point call causes an error to occur that is not expected and thus not handled, the status word INTERNAL_ERROR shall be returned.

#### 6.1.8.2	Test Procedure

SET Test TO CatSearch

AND PatDA TO {0xDA, 0xDA, 0xDA}

AND Pat55 TO {0x55, 0x55, 0x55}

AND PatAA TO {0xAA, 0xAA, 0xAA}

AND Pat12 TO {1,2,3,4,5,6,7,8,9,0}.

/* Test Case 1: Pattern not found */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_LARU, NULL) == SW_OK}

THEN {CatSearch(BEGINNING_FORWARD, 0, sizeof(PatDA), PatDA)== SW_REFERENCED_DATA_NOT_FOUND}.

/* Test Case 2: Search from beginning forward */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_LARU, NULL) == SW_OK}

THEN {CatSearch(BEGINNING_FORWARD, 0, sizeof(Pat55), Pat55)== SW_OK}.

/* Test Case 3: Search from end backward */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_LARU, NULL) == SW_OK}

THEN {CatSearch(END_BACKWARD, 0, sizeof(Pat55), Pat55)== SW_OK}.

/* Test Case 4: Search from next forward */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_LARU, NULL) == SW_OK}

THEN {CatSearch(NEXT_FORWARD, 0, sizeof(PatAA), PatAA)== SW_OK}.

/* Test Case 5: Last record, search next forward */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_LARU, NULL) == SW_OK}

THEN {CatSearch(NEXT_FORWARD, 0, sizeof(Pat55), Pat55)== SW_REFERENCED_DATA_NOT_FOUND}.

/* Test Case 6: Search from next forward */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_LARU, NULL) == SW_OK}

THEN {CatSearch(PREVIOUS_BACKWARD, 0, sizeof(Pat55), Pat55)== SW_OK}.

/* Test Case 7: First record, search previous backward */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_LARU, NULL) == SW_OK}

THEN {CatSearch(NEXT_FORWARD, 0, sizeof(Pat55), Pat55)== SW_REFERENCED_DATA_NOT_FOUND}.

/* Test Case 8: Pattern not found, start beyond end of record */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_LARU, NULL) == SW_OK}

THEN {CatSearch(BEGINNING_FORWARD, 10, sizeof(Pat55), Pat55)== SW_REFERENCED_DATA_NOT_FOUND}.

/* Test Case 9: Pattern not found, pattern longer than record */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_LARU, NULL) == SW_OK}

THEN {CatSearch(BEGINNING_FORWARD, 0, sizeof(Pat12), Pat12)== SW_REFERENCED_DATA_NOT_FOUND}.

/* Test Case 10: Pattern not found, offset < record length < offest+pattern length*/

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_LARU, NULL) == SW_OK}

THEN {CatSearch(BEGINNING_FORWARD, 2, sizeof(Pat55), Pat55)== SW_REFERENCED_DATA_NOT_FOUND}.

/* Test Case 11: Pattern not found, EF not linear */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_TARU, NULL) == SW_OK}

THEN {CatSearch(BEGINNING_FORWARD, 0, sizeof(Pat55), Pat55)== SW_INCOMPATIBLE_FILE_STRUCTURE}.

/* Test Case 12: No EF selected */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

THEN {CatSearch(BEGINNING_FORWARD, 0, sizeof(Pat55), Pat55) == SW_NO_CURRENT_FILE}.

/* Test 13: Security condition not satisfied */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_TNR, NULL) == SW_OK}

THEN {CatSearch(BEGINNING_FORWARD, 0, sizeof(Pat55), Pat55) == SW_SECURITY_STATUS_NOT_SATISFIED}.

/* Test Case 14: EF is invalidated */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_TNU, NULL) == SW_OK}

AND  {(CatInvalidate(), SW_OK) == SW_OK}

THEN {CatSearch(BEGINNING_FORWARD, 0, sizeof(Pat55), Pat55) == SW_REFERENCED_DATA_INVALIDATED}.

#### 6.1.8.3	Test Coverage

| CRR Number | Test Case Number |
| --- | --- |
| N1 | 2 |
| N2 | 3 |
| N3 | 4 |
| N4 | 6 |
| N5 | 8 |
| N6 | 1 |
| N7 | 5 |
| N8 | 7 |
| P1 | 9 |
| P2 | 10 |
| C1 | 12 |
| C2 | 11 |
| C3 | 13 |
| C4 | 14 |
| C5, C6 | Not tested |

### 6.1.9	CatIncrease

Test Area Reference: API_CatIncrease

#### 6.1.9.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

UINT16 CatIncrease(DWORD Increment, DWORD *Value);

6.1.9.1.1	Normal execution

CRRN1: The value Increment is added to the value of the last increased  / updated record in the currently selected cyclic EF. The result is stored in the oldest record and returned in Value if Value is non-null. The updated record becomes record number 1 and is selected as current record.

6.1.9.1.2	Parameter errors

CRRP1: If the result of the addition is greater than the maximum value a DWORD, the status word MAX_VALUE_REACHED shall be returned.

6.1.9.1.3	Context errors

CRRC1: If the calling Application has currently no EF selected,  the status word NO_EF_SELECTED shall be returned.

CRRC2: If the currently selected EF is neither linear fixed nor cyclic, the status word FILE_INCONSISTENT shall be returned.

CRRC3: If the calling Application does not fulfil the access condition, READ, to perform this function,  the status word AC_NOT_FULFILLED shall be returned.

CRRC4: If the currently selected EF is invalidated and the file status of the EF does not allow for reading an invalidated file, the status word INVALIDATION_STATUS_CONTRADICTION shall be returned.

CRRC5: If the entry point call causes a memory problem (e.g. memory access error), the status word MEMORY_PROBLEM shall be returned.

CRRC6: If the entry point call causes an error to occur that is not expected and thus not handled, the status word INTERNAL_ERROR shall be returned.

CRRC 7: If Increase not allowed due to an entry in the FCP them the status word FILE_INCONSISTENT shall be returned.

#### 6.1.9.2	Test Procedure

SET Test TO CatIncrease.

/* Test Case 1: Increase, verify response */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_CARU, NULL) == SW_OK}

THEN {CatIncrease((DWORD)0x00000100, &bytes) == SW_OK}

AND  {bytes == (DWORD)0x00000100}.

/* Test Case 2: Increase, verify file */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_CARU, NULL) == SW_OK}

THEN {CatIncrease((DWORD)0x00000100, &bytes) == SW_OK}

AND  {CatReadRecord(0, REC_ACC_MODE_ABSOLUTE_CURRENT, 0, &bytes, buffer) == SW_OK}

AND  {*(DWORD *)buffer == (DWORD)0x00000100}.

/* Test Case 3: Increase, exceed maximum */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_CARU, NULL) == SW_OK}

THEN {CatIncrease((DWORD)0xFFFFFFFF, &bytes) == SW_INCREASE_AT_MAX_VALUE}.

/* Test Case 4: No EF selected */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

THEN {CatIncrease((DWORD)0x00000100, &bytes) == SW_NO_CURRENT_FILE}.

/* Test Case 5: EF not cyclic */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_TARU, NULL) == SW_OK}

THEN {CatIncrease((DWORD)0x00000100, &bytes) == SW_INCOMPATIBLE_FILE_STRUCTURE}.

/* Test Case 6: Security condition not satisfied */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_CNU, NULL) == SW_OK}

THEN {CatIncrease((DWORD)0x00000100, &bytes) == SW_SECURITY_STATUS_NOT_SATISFIED}.

/* Test Case 7: EF is invalidated */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_CARU, NULL) == SW_OK}

AND  {(CatInvalidate(), SW_OK) == SW_OK}

THEN {CatIncrease((DWORD)0x00000100, &bytes) == SW_REFERENCED_DATA_INVALIDATED}.

#### 6.1.9.3	Test Coverage

| CRR Number | Test Case Number |
| --- | --- |
| N1 | 1,2 |
| P1 | 3 |
| C1 | 4 |
| C2 | 5 |
| C3 | 6 |
| C4 | 7 |
| C5 | Not tested |
| C6, C7 | Not tested |

### 6.1.10	CatInvalidate

Test Area Reference: API_CatInvalidate

#### 6.1.10.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

UINT16 CatInvalidate(void)

6.1.10.1.1	Normal execution

CRRN1: The currently selected EF of the calling application shall be invalidated, i.e. the flag in the EF file status shall be changed accordingly.

6.1.10.1.2	Parameter errors

6.1.10.1.3	Context errors

CRRC1: If the calling application has currently no EF selected, the status word NO_EF_SELECTED shall be returned.

CRRC2: If the calling application does not 33ulfil the access condition, INVALIDATE, the status word AC_NOT_FULFILLED shall be returned.

CRRC3: If the currently selected EF is already invalidated, the status word INVALIDATION_STATUS_CONTRADICTION shall be returned.

CRRC4: If the entry point call causes a memory problem (e.g. memory access error), an instance of the status word MEMORY_PROBLEM shall be returned.

CRRC5: If the entry point call causes an error to occur that is not expected and thus not handled, the status word INTERNAL_ERROR shall be returned.

#### 6.1.10.2	Test Procedure

SET Test TO CatInvalidate.

/* Test Case 1: Invalidate */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_TNR, NULL) == SW_OK}

THEN {CatInvalidate() == SW_OK}.

/* Test Case 2: No EF selected */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

THEN {CatInvalidate() == SW_NO_CURRENT_FILE}.

/* Test Case 3: Security condition not satisfied */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_CNIV, NULL) == SW_OK}

THEN {CatInvalidate() == SW_SECURITY_STATUS_NOT_SATISFIED}.

/* Test Case 4: EF is already invalidated */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_CARU, NULL) == SW_OK}

AND  {(CatInvalidate(), SW_OK) == SW_OK}

THEN {CatInvalidate() == SW_REFERENCED_DATA_INVALIDATED}.

#### 6.1.10.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| C1 | 2 |
| C2 | 3 |
| C3 | 4 |
| C4, C5 | Not Tested |

### 6.1.11	CatRehabilitate

Test Area Reference: API_CatRehabilitate

#### 6.1.11.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

UINT16 CatRehabilitate(void)

6.1.11.1.1	Normal execution

CRRN1: The currently selected EF of the calling application shall be rehabilitated, i.e. the flag in the EF file status shall be changed accordingly.

6.1.11.1.2	Parameter errors

6.1.11.1.3	Context errors

CRRC1: If the calling application has currently no EF selected, the status word NO_EF_SELECTED shall be returned.

CRRC2: If the calling application does not 34ulfil the access condition, INVALIDATE, the status word AC_NOT_FULFILLED shall be returned.

CRRC3: If the currently selected EF is already invalidated, the status word INVALIDATION_STATUS_CONTRADICTION shall be returned.

CRRC4: If the entry point call causes a memory problem (e.g. memory access error), an instance of the status word MEMORY_PROBLEM shall be returned.

CRRC5: If the entry point call causes an error to occur that is not expected and thus not handled, the status word INTERNAL_ERROR shall be returned.

#### 6.1.11.2	Test Procedure

SET Test TO CatRehabilitate.

/* Test Case 1: Rehabilitate */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_TNR, NULL) == SW_OK}

AND  {(CatInvalidate(), SW_OK) == SW_OK}

THEN {CatRehabilitate() == SW_OK}.

/* Test Case 2: No EF selected */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

THEN {CatRehabilitate() == SW_NO_CURRENT_FILE}.

/* Test Case 3: Security condition not satisfied */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_TNR, NULL) == SW_OK}

THEN {CatRehabilitate() == SW_SECURITY_STATUS_NOT_SATISFIED}.

/* Test Case 4: EF is may not be rehabilitated */

IF   {CatSelect(FID_DF_SIMTEST, NULL) == SW_OK}

AND  {CatSelect(FID_EF_CNRI, NULL) == SW_OK}

THEN {CatRehabilitate() == SW_REFERENCED_DATA_INVALIDATED}.

#### 6.1.11.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| C1 | 2 |
| C2 | 3 |
| C3 | 4 |
| C4, C5 | Not Tested |

## 6.2	Registry

### 6.2.1	CatSetMenuString

Test Area Reference: API_CatSetMenuString

#### 6.2.1.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Void CatSetMenuString (BYTE MenuID, BYTE MenuStringLength, const void *MenuString,
                           const CatIconIdentifier *IconIdentifier,
                           BYTE HelpAvailable, BYTE NextAction);

6.2.1.1.1	Normal execution

CRRN1:  Menu string with optional modifiers is added to the main menu.

CRNN2: If MenuString is null, the invocation is a NOP.

6.2.1.1.2	Parameter errors

6.2.1.1.3	Context errors

#### 6.2.1.2	Test Procedure

SET Test TO CatSetMenuString.

/* Test Case 1: Set non-null menu string */

IF   {(CatSetMenuString(1, 12, "Hello, world", NULL, 0, 0), SW_OK) == SW_OK}

THEN {NOP}.

/* Test Case 2: Set null menu string */

IF   {(CatSetMenuString(1, 0, NULL, NULL, 0, 0), SW_OK) == SW_OK}

THEN {NOP}.

#### 6.2.1.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

### 6.2.2	CatNotifyOnFrameworkEvent

Test Area Reference: API_CatNotifyOnFrameworkEvent

#### 6.2.2.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Void CatNotifyOnFrameworkEvent(CatFrameworkEventType Event, BYTE Enabled);

6.2.2.1.1	Normal execution

CRRN1: Enable or disable monitoring of the framework event indicated by Event depending on the setting of Enabled.

6.2.2.1.2	Parameter errors

6.2.2.1.3	Context errors

#### 6.2.2.2	Test Suite Files

Additional requirements for the UICC personalisation: None

Test Script: 				API_CatNotifyOnFrameworkEvent.tst

Test Application: 		API_ CatNotifyOnFrameworkEvent.c

#### 6.2.2.3	Test Procedure

| Id | Description | API Expectation | APDU Expectation |
| --- | --- | --- | --- |
| 0 | UICC Initialisation | Responses ignored. |  |
| 1 | Enable Monitoring of an Event 1- CatNotifyOnFrameworkEvent(EVENT_UNRECOGNIZED_ENVELOPE, 1); | 1 – No return |  |
| 2 | Disable Monitoring of an Event 1 – CatNotifyOnFrameworkEvent(EVENT_UNRECOGNIZED_ENVELOPE, 0); | 1 – No return |  |

#### 6.2.2.4	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1,2 |

### 6.2.3	CatNotifyOnEnvelope

Test Area Reference: API_CatNotifyOnEnvelope

#### 6.2.3.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Void CatNotifyOnEnvelope(CatEnvelopeTagType Tag, BYTE Enabled);

6.2.3.1.1	Normal execution

CRRN1: Enable or disable monitoring of the envelope event indicated by Tag depending on the setting of Enabled.

6.2.3.1.2	Parameter errors

6.2.3.1.3	Context errors

#### 6.2.3.2	Test Procedure

SET Test TO CatNotifyOnFrameworkEvent.

/* Test Case 1: Enable monitoring of event */

IF   {(CatNotifyOnFrameworkEvent(SMS_PP_DOWNLOAD_TAG, 1), SW_OK) == SW_OK}

THEN {NOP}.

/* Test Case 2: Disable monitoring of event */

IF   {(CatNotifyOnFrameworkEvent(SMS_PP_DOWNLOAD_TAG, 0), SW_OK) == SW_OK}

THEN {NOP}.

#### 6.2.3.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1, 2 |

### 6.2.4	CatNotifyOnEvent

Test Area Reference: API_CatNotifyOnEvent

#### 6.2.4.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Void CatNotifyOnEvent(CatEventType EventType, BYTE Enabled);

6.2.4.1.1	Normal execution

CRRN1: Enable or disable monitoring of the terminal event indicated by Event depending on the setting of Enabled.

6.2.4.1.2	Parameter errors

6.2.4.1.3	Context errors

#### 6.2.4.2	Test Procedure

SET Test TO CatNotifyOnEvent.

/* Test Case 1: Enable monitoring of event */

IF   {(CatNotifyOnEvent(MT_CALL_EVENT, 1), SW_OK) == SW_OK}

THEN {NOP}.

/* Test Case 2: Disable monitoring of event */

IF   {(CatNotifyOnEvent(MT_CALL_EVENT, 0), SW_OK) == SW_OK}

THEN {NOP}.

#### 6.2.4.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1, 2 |

## 6.3	Man-Machine Interface

### 6.3.1	CatAddItem

Test Area Reference: API_CatAddItem

#### 6.3.1.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Void CatAddItem(BYTE ItemTextLength, const void *ItemText, BYTE ItemIdentifier);

6.3.1.1.1	Normal execution

CRRN1:  Add the provided item with the provided item identifier to the list of items in the current select list.

CRRN2:  If ItemTextLength is zero or ItemText is null, the invocation is a NOP.

6.3.1.1.2	Parameter errors

6.3.1.1.3	Context errors

#### 6.3.1.2	Test Procedure

SET Test TO CatAddItem.

/* Test Case 1: Add non-null item */

IF   {(CatAddItem(12, "Hello, world", 0), SW_OK) == SW_OK}

THEN {NOP}.

/* Test Case 2: Add null item */

IF   {(CatAddItem(0, NULL, 0), SW_OK) == SW_OK}

THEN {NOP}.

#### 6.3.1.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

### 6.3.2	CatSelectItem

Test Area Reference: API_CatSelectItem

#### 6.3.2.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Void CatSelectItem(BYTE TitleLength, const void *Title, CatSelectItemOptions Options);

6.3.2.1.1	Normal execution

CRRN1: Create a new select list with the provided title and options.

CRRN2: If TitleLength is zero or Title is null, the select list has no title.

6.3.2.1.2	Parameter errors

6.3.2.1.3	Context errors

#### 6.3.2.2	Test Procedure

SET Test TO CatSelectItem.

/* Test Case 1: Initialize with title */

IF   {(CatSelectItem(5, "Title", 0), SW_OK) == SW_OK}

THEN {NOP}.

/* Test Case 2: Initialize without title */

IF   {(CatSelectItem(0, NULL, 0), SW_OK) == SW_OK}

THEN {NOP}.

#### 6.3.2.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

### 6.3.3	CatEndSelectItem

Test Area Reference: API_CatEndSelectItem

#### 6.3.3.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatEndSelectItem(BYTE *SelectedItem, const CatIconIdentifier *IconIdentifier);

6.3.3.1.1	Normal execution

CRRN1: Send a SELECT ITEM proactive command to the terminal using the current select list and the indicated icon; return the item picked in SelectedItem.

CRNN2: If SelectedItem is null, the invocation is for the return value only.

CRNN3: If there is no current select list or if no items have been added to the current select list, the invocation does not send a proactive command to the terminal but rather returns CAT_SUCCESS and sets *SelelectedItem to zero if SelelectedItem is non-null.

6.3.3.1.2	Parameter errors

6.3.3.1.3	Context errors

#### 6.3.3.2	Test Procedure

SET Test TO CatEndSelectItem.

/* Test Case 1: Non-null select list and non-null select item */

IF   {(CatSelectItem(4, "Pick", 0), SW_OK) == SW_OK}

AND  {(CatAddItem(1, "1", 0), SW_OK) == SW_OK}

AND  {(CatAddItem(1, "2", 0), SW_OK) == SW_OK}

THEN {CatEndSelectItem(buffer, NULL) == CAT_COMMAND_SUCCESSFUL}

AND  {memcmp(proactiveCommandBuffer, endSelectItem, sizeof(endSelectItem)) == 0}.

/* Test Case 2: Null select item */

IF   {(CatSelectItem(4, "Pick", 0), SW_OK) == SW_OK}

AND  {(CatAddItem(1, "1", 0), SW_OK) == SW_OK}

AND  {(CatAddItem(1, "2", 0), SW_OK) == SW_OK}

THEN {CatEndSelectItem(buffer, NULL) == CAT_COMMAND_SUCCESSFUL}

AND  {memcmp(proactiveCommandBuffer, endSelectItem, sizeof(endSelectItem)) == 0}.

/* Test Case 3: Null select list */

IF   {CatEndSelectItem(NULL, NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {NOP}.

#### 6.3.3.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |
| N3 | 3 |

### 6.3.4	CatDisplayText

Test Area Reference: API_CatDisplayText

#### 6.3.4.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatDisplayText(CatDCSValue TextDCS, BYTE TextLength, const void *Text,
                CatDisplayTextOptions Options, const CatIconIdentifier *IconIdentifier,
                BYTE ImmediateResponse);

6.3.4.1.1	Normal execution

CRRN1: Send a DISPLAY TEXT proactive command to the terminal with the provided text with the provided options and icon.

CRRN2: If TextLength is zero or Text is null, the invocation is a NOP that returns CAT_SUCCESS.

6.3.4.1.2	Parameter errors

6.3.4.1.3	Context errors

#### 6.3.4.2	Test Procedure

SET Test TO CatDisplayText.

/* Test Case 1: Non-null text and non-null text length */

IF   {CatDisplayText(DCS_UCS2, 12, "Hello, world",

NORMAL_PRIORITY_AUTO_CLEAR, NULL, 0) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, displayText, sizeof(displayText)) == 0}.

/* Test Case 2: Zero length text */

IF   {CatDisplayText(DCS_UCS2, 0, "Hello, world",

NORMAL_PRIORITY_AUTO_CLEAR, NULL, 0) == CAT_COMMAND_SUCCESSFUL}

THEN {NOP}.

/* Test Case 3: Null text */

IF   {CatDisplayText(DCS_UCS2, 0, NULL,

NORMAL_PRIORITY_AUTO_CLEAR, NULL, 0) == CAT_COMMAND_SUCCESSFUL}

THEN {NOP}.

#### 6.3.4.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2, 3 |

### 6.3.5	CatGetInKey

Test Area Reference: API_CatGetInKey

#### 6.3.5.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatGetInKey(CatDCSValue TitleDCS, BYTE TitleLength, const void *Title,
             CatGetInKeyOptions Options, const CatIconIdentifier *IconIdentifier,
             CatDCSValue *DCSOut, void *KeyOut);

6.3.5.1.1	Normal execution

CRRN1: Send a GET IN KEY proactive command to the terminal with the text in Title and according to the provided Options; return a key value in *KeyOut and the alphabet indicator of the key value in *DCSOut.

CRRN2: If TitleLength is zero or Title is null, then no title is included in the proactive command.

CRRN3: If KeyOut is null, then no key value is returned.

CRRN4: If DCSOut is null, then no key value alphabet indicator is returned.

6.3.5.1.2	Parameter errors

6.3.5.1.3	Context errors

#### 6.3.5.2	Test Procedure

SET Test TO CatGetInKey.

/* Test Case 1: All arguments non-null */

IF   {CatGetInKey(DCS_UCS2, 9, "Hit a Key", 0, NULL, &bytes, buffer) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, getInKey1, sizeof(getInKey1)) == 0}.

/* Test Case 2: Text null */

IF   {CatGetInKey(DCS_UCS2, 0, NULL, 0, NULL, &bytes, buffer) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, getInKey2, sizeof(getInKey2)) == 0}.

/* Test Case 3: Key out null */

IF   {CatGetInKey(DCS_UCS2, 9, "Hit a Key", 0, NULL, &bytes, NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, getInKey1, sizeof(getInKey1)) == 0}.

/* Test Case 4: DCS out null */

IF   {CatGetInKey(DCS_UCS2, 9, "Hit a Key", 0, NULL, NULL, buffer) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, getInKey1, sizeof(getInKey1)) == 0}.

#### 6.3.5.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |
| N3 | 3 |
| N4 | 4 |

### 6.3.6	CatGetInput

Test Area Reference: API_CatGetInput

#### 6.3.6.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatGetInput(CatDCSValue TitleDCS, BYTE TitleLength, const void *Title, 
            CatGetInputOptions Options, CatDCSValue DefaultReplyDCS, 
            BYTE DefaultReplyLength, const void *DefaultReply, 
            BYTE MinimumResponseLength, BYTE MaximumResponseLength, 
            const CatIconIdentifier *IconIdentifier,
            CatDCSValue *MsgOutDCS, BYTE *MsgOutLength, void *MsgOut);

6.3.6.1.1	Normal execution

CRRN1:  Send a GET INPUT proactive command to the terminal with the provided Title and acquire a sequence of key hits according to Options, MinimumResponseLength and MaximumResponseLength; return the MsgOutLength length sequence in MsgOut and the alphabet indicator of the returned values in MsgOutDCS.

CRRN2: If TitleLength is zero or Title is null then no title is displayed.

CRRN3: If MsgOut is null, then the sequence is not returned.

CRRN4: If MsgOutDCS is null, then no alphabet indicator is returned.

CRRN5: If MinimumResponseLength is greater than MaximumResponseLength then it is taken to be equal to MaximumResponseLength.

6.3.6.1.2	Parameter errors

6.3.6.1.3	Context errors

#### 6.3.6.2	Test Procedure

SET Test TO CatGetInput.

/* Test Case 1: All arguments non-null */

IF   {CatGetInput(DCS_UCS2, 4, "Pick", 0, DCS_UCS2, 5, buffer,

1, 10, NULL, &dcs, &bytes, buffer)== CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, getInput1, sizeof(getInput1)) == 0}.

/* Test Case 2: Text null */

IF   {CatGetInput(DCS_UCS2, 0, NULL, 0, DCS_UCS2, 5, buffer,

1, 10, NULL, &dcs, &bytes, buffer)== CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, getInput2, sizeof(getInput2)) == 0}.

/* Test Case 3: Message out null */

IF   {CatGetInput(DCS_UCS2, 4, "Pick", 0, DCS_UCS2, 5, buffer,

1, 10, NULL, &dcs, &bytes, NULL)== CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, getInput1, sizeof(getInput1)) == 0}.

/* Test Case 4: DCS out null */

IF   {CatGetInput(DCS_UCS2, 4, "Pick", 0, DCS_UCS2, 5, buffer,

1, 10, NULL, NULL, &bytes, buffer)== CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, getInput1, sizeof(getInput1)) == 0}.

/* Test Case 5: Maximum response length < Minimum response length */

IF   {CatGetInput(DCS_UCS2, 4, "Pick", 0, DCS_UCS2, 5, buffer,

10, 1, NULL, &dcs, &bytes, buffer)== CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, getInput3, sizeof(getInput3)) == 0}.

#### 6.3.6.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |
| N3 | 3 |
| N4 | 4 |
| N5 | 5 |

### 6.3.7	CatSetupIdleModeText

Test Area Reference: API_CatSetupIdleModeText

#### 6.3.7.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatSetupIdleModeText(CatDCSValue TextDCS, BYTE TextLength, const void *Text,
                                          const CatIconIdentifier *IconIdentifier);

6.3.7.1.1	Normal execution

CRRN1:  Send a SETUP IDLE MODE TEXT proactive command to the terminal with Text in the indicated alphabet and with an optional icon.

CRRN2: If TextLength is zero or Text is null, then the invocation is a NOP.

6.3.7.1.2	Parameter errors

6.3.7.1.3	Context errors

#### 6.3.7.2	Test Procedure

SET Test TO CatSetupIdleModeText.

/* Test Case 1: All arguments non-null */

IF   {CatSetupIdleModeText(DCS_UCS2, 3, "MMS", NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, setupIdleModeText1, sizeof(setupIdleModeText1)) == 0}.

/* Test Case 2: Text null */

IF   {CatSetupIdleModeText(DCS_UCS2, 3, NULL, NULL)== CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, setupIdleModeText2, sizeof(setupIdleModeText2)) == 0}.

/* Test Case 3: Length null */

IF   {CatSetupIdleModeText(DCS_UCS2, 0, "MMS", NULL)== CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, setupIdleModeText3, sizeof(setupIdleModeText3)) == 0}.

#### 6.3.7.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2, 3 |

### 6.3.8	CatPlayTone

Test Area Reference: API_CatPlayTone

#### 6.3.8.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatPlayTone(BYTE TextLength, const void *Text,
                                 CatTone Tone, CatTimeUnit Units, BYTE Duration,
                                 const CatIconIdentifier *IconIdentifier);

6.3.8.1.1	Normal execution

CRRN1:  Send a PLAY TONE proactive command to the terminal with the provided data.

CRRN2:  If Duration is zero, the invocation is a NOP that returns CAT_SUCCESS.

6.3.8.1.2	Parameter errors

6.3.8.1.3	Context errors

#### 6.3.8.2	Test Procedure

SET Test TO CatPlayTone.

/* Test Case 1: Duration positive, text non-null */

IF   {CatPlayTone(4, "Beep", GENERAL_BEEP, GSM_SECONDS, 5, NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, playTone1, sizeof(playTone1)) == 0}.

/* Test Case 2: Duration positive, text null */

IF   {CatPlayTone(0, NULL, GENERAL_BEEP, GSM_SECONDS, 5, NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, playTone2, sizeof(playTone2)) == 0}.

/* Test Case 3: Duration zero */

IF   {CatPlayTone(4, "Beep", GENERAL_BEEP, GSM_SECONDS, 0, NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, playTone3, sizeof(playTone3)) == 0}.

#### 6.3.8.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1, 2 |
| N2 | 3 |

## 6.4	Timers

### 6.4.1	CatGetTimer

Test Area Reference: API_CatGetTimer

#### 6.4.1.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

BYTE CatGetTimer(void);

6.4.1.1.1	Normal execution

CRRN1:  Send a GET TIMER proactive command to the terminal; return a timer index.

6.4.1.1.2	Parameter errors

6.4.1.1.3	Context errors

#### 6.4.1.2	Test Procedure

SET Test TO CatGetTimer.

/* Test Case 1: Acquire a timer */

IF   {(CatGetTimer(),CAT_COMMAND_SUCCESSFUL)  == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, getTimer, sizeof(getTimer)) == 0}.

#### 6.4.1.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |

### 6.4.2	CatFreeTimer

Test Area Reference: API_CatFreeTimer

#### 6.4.2.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Void CatFreeTimer(BYTE TimerID);

6.4.2.1.1	Normal execution

CRRN1:  Send a FREE TIMER proactive command to the terminal with the provided timer index.

6.4.2.1.2	Parameter errors

6.4.2.1.3	Context errors

#### 6.4.2.2	Test Procedure

SET Test TO CatFreeTimer.

/* Test Case 1: Free a timer */

IF   {(bytes = CatGetTimer(),CAT_COMMAND_SUCCESSFUL)  == CAT_COMMAND_SUCCESSFUL}

AND  {(CatFreeTimer((BYTE)bytes),CAT_COMMAND_SUCCESSFUL)  == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, freeTimer, sizeof(freeTimer)) == 0}.

#### 6.4.2.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |

### 6.4.3	CatStartTimer

Test Area Reference: API_CatStartTimer

#### 6.4.3.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Void CatStartTimer(BYTE TimerID, CatTimerValue *TimerValue);

6.4.3.1.1	Normal execution

CRRN1:  Send a START TIMER proactive command to the terminal with the provided data.

CRRN2:  If TimerValue is null, the invocation is a NOP.

6.4.3.1.2	Parameter errors

6.4.3.1.3	Context errors

#### 6.4.3.2	Test Procedure

SET Test TO CatStartTimer.

/* Test Case 1: Start a timer */

IF   {(bytes = CatGetTimer(),CAT_COMMAND_SUCCESSFUL)  == CAT_COMMAND_SUCCESSFUL}

AND  {(CatStartTimer((BYTE)bytes, &duration),CAT_COMMAND_SUCCESSFUL)  == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, startTimer, sizeof(startTimer)) == 0}.

/* Test Case 2: Null duration */

IF   {(bytes = CatGetTimer(),CAT_COMMAND_SUCCESSFUL)  == CAT_COMMAND_SUCCESSFUL}

AND  {(CatStartTimer((BYTE)bytes, NULL),CAT_COMMAND_SUCCESSFUL)  == CAT_COMMAND_SUCCESSFUL}

THEN {NOP}.

#### 6.4.3.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

### 6.4.4	CatGetTimerValue

Test Area Reference: API_CatGetTimerValue

#### 6.4.4.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Void CatGetTimerValue(BYTE TimerID, CatTimerValue *TimerValue);

6.4.4.1.1	Normal execution

CRRN1:  Send a GET TIMER VALUE to the terminal; return a timer value in *TimerValue if TimerValue is non-null.

CRRN2:  If TimerValue is null, the invocation is a NOP.

6.4.4.1.2	Parameter errors

6.4.4.1.3	Context errors

#### 6.4.4.2	Test Procedure

SET Test TO CatGetTimerValue.

/* Test Case 1: Get a timer value */

IF   {(bytes = CatGetTimer(),CAT_COMMAND_SUCCESSFUL)  == CAT_COMMAND_SUCCESSFUL}

AND  {(CatGetTimerValue((BYTE)bytes, &duration),CAT_COMMAND_SUCCESSFUL)  == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, getTimerValue, sizeof(getTimerValue)) == 0}.

/* Test Case 2: Null duration */

IF   {(bytes = CatGetTimer(),CAT_COMMAND_SUCCESSFUL)  == CAT_COMMAND_SUCCESSFUL}

AND  {(CatGetTimerValue((BYTE)bytes, NULL),CAT_COMMAND_SUCCESSFUL)  == CAT_COMMAND_SUCCESSFUL}

THEN {NOP}.

#### 6.4.4.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

## 6.5	Supplementary Card Reader Management

### 6.5.1	CatPowerOnCard

Test Area Reference: API_CatPowerOnCard

#### 6.5.1.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatPowerOnCard(CatDevice DeviceID, BYTE *ATRLength, void *ATR);

6.5.1.1.1	Normal execution

CRRN1: Send a POWER ON CARD proactive command to the terminal with the provided data; return the ATR of the card in the indicated card reader in ATR.

CRRN2: If either ATRLength or ATR is null, then the proactive command is issued but the ATR is not returned.

6.5.1.1.2	Parameter errors

6.5.1.1.3	Context errors

#### 6.5.1.2	Test Procedure

SET Test TO CatPowerOnCard.

/* Test Case 1: Power on a card */

IF   {CatPowerOnCard(DEVICE_CARD_READER_7, (BYTE *)&bytes, buffer) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, powerOnCard, sizeof(powerOnCard)) == 0}.

/* Test Case 2: ATR null */

IF   {CatPowerOnCard(DEVICE_CARD_READER_7, (BYTE *)&bytes, NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, powerOnCard, sizeof(powerOnCard)) == 0}.

/* Test Case 3: ATR length null */

IF   {CatPowerOnCard(DEVICE_CARD_READER_7, NULL, buffer) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, powerOnCard, sizeof(powerOnCard)) == 0}.

#### 6.5.1.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2, 3 |

### 6.5.2	CatPowerOffCard

Test Area Reference: API_CatPowerOffCard

#### 6.5.2.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatPowerOffCard(CatDevice DeviceID);

6.5.2.1.1	Normal execution

CRRN1: Send a POWER OFF CARD proactive command to the terminal.

6.5.2.1.2	Parameter errors

6.5.2.1.3	Context errors

#### 6.5.2.2	Test Procedure

SET Test TO CatPowerOffCard.

/* Test Case 1: Power off a card */

IF   {CatPowerOffCard(DEVICE_CARD_READER_7) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, powerOffCard, sizeof(powerOffCard)) == 0}.

#### 6.5.2.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |

### 6.5.3	CatPerformCardAPDU

Test Area Reference: API_CatPerformCardAPDU

#### 6.5.3.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatPerformCardAPDU(CatDevice DeviceID,
 									BYTE CAPDULength, const void *CAPDU,
                                    BYTE *RAPDULength, void *RAPDU);

6.5.3.1.1	Normal execution

CRRN1: Send a PERFORM CARD APDU command to the terminal with the provided data; return the response of the card at RAPDU.

CRRN2: If RAPDULength or RAPDU is null, the response of the card in the indicated card reader is not returned.

6.5.3.1.2	Parameter errors

6.5.3.1.3	Context errors

#### 6.5.3.2	Test Procedure

SET Test TO CatPerformCardAPDU

AND CAPDU TO {0x00, 0xA4, 0x00, 0x00, 0x02, 0x3F, 0x00}.

/* Test Case 1: Perform card APDU */

IF   {CatPerformCardAPDU(DEVICE_CARD_READER_7,

sizeof(CAPDU), CAPDU, (BYTE *)(&bytes), buffer) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, performCardAPDU, sizeof(performCardAPDU)) == 0}.

/* Test Case 2: Null RAPDU length */

IF   {CatPerformCardAPDU(DEVICE_CARD_READER_7,

sizeof(CAPDU), CAPDU, NULL, buffer) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, performCardAPDU, sizeof(performCardAPDU)) == 0}.

/* Test Case 3: NULL RAPDU */

IF   {CatPerformCardAPDU(DEVICE_CARD_READER_7,

sizeof(CAPDU), CAPDU, (BYTE *)(&bytes), NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, performCardAPDU, sizeof(performCardAPDU)) == 0}.

#### 6.5.3.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2, 3 |

### 6.5.4	CatGetReaderStatus

Test Area Reference: API_CatGetReaderStatus

#### 6.5.4.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatGetReaderStatus(CatDevice DeviceID, CatReaderStatusOptions Options,

BYTE *Status);

6.5.4.1.1	Normal execution

CRRN1:  Send a GET READER STATUS proactive command with the provided data to the terminal; return the status of the indicated device at Status.

CRRN2:  If Status is null then the invocation is a NOP that returns CAT_SUCCESS.

6.5.4.1.2	Parameter errors

6.5.4.1.3	Context errors

#### 6.5.4.2	Test Procedure

SET Test TO CatGetReaderStatus.

/* Test Case 1: Perform card APDU */

IF   {CatGetReaderStatus(DEVICE_CARD_READER_7, 0, buffer) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, getReaderStatus, sizeof(getReaderStatus)) == 0}.

/* Test Case 2: Null status */

IF   {CatGetReaderStatus(DEVICE_CARD_READER_7, 0, NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, getReaderStatus, sizeof(getReaderStatus)) == 0}.

#### 6.5.4.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

## 6.6	Network Services

### 6.6.1	CatGetLocationInformation

Test Area Reference: API_CatGetLocationInformation

#### 6.6.1.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatGetLocationInformation(CatLocationInformation *LocationInformation);

6.6.1.1.1	Normal execution

CRRN1: Send a PROVIDE LOCAL INFORMATION proactive command to the terminal; return the retrieved local information in LocationInformation.

CRRN2: If LocationInformation is null, the invocation is a NOP that returns CAT_SUCCESS.

6.6.1.1.2	Parameter errors

6.6.1.1.3	Context errors

#### 6.6.1.2	Test Procedure

SET Test TO CatGetLocationInformation.

/* Test Case 1: Get location information */

IF   {CatGetLocationInformation( &locationInformation) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, getLocationInformation, sizeof(getLocationInformation)) == 0}.

/* Test Case 2: Null location information */

IF   {CatGetLocationInformation(NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {NOP}.

#### 6.6.1.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

### 6.6.2	CatGetTimingAdvance

Test Area Reference: API_CatGetTimingAdvance

#### 6.6.2.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatGetTimingAdvance(CatTimingAdvance *TimingAdvance);

6.6.2.1.1	Normal execution

CRRN1: Send a PROVIDE LOCAL INFORMATION proactive command to the terminal to retrieve the timing advance information; return the retrieved timing advance information at TimingAdvance.

CRRN2: If TimingAdvance is null, the invocation is a NOP that returns CAT_SUCCESS.

6.6.2.1.2	Parameter errors

6.6.2.1.3	Context errors

#### 6.6.2.2	Test Procedure

SET Test TO CatGetTimingAdvance.

/* Test Case 1: Get timing advance */

IF   {CatGetTimingAdvance(&timingAdvance) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, getTimingAdvance, sizeof(getTimingAdvance)) == 0}.

/* Test Case 2: Null timing advance */

IF   {CatGetTimingAdvance(NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {NOP}.

#### 6.6.2.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

### 6.6.3	CatGetIMEI

Test Area Reference: API_CatGetIMEI

#### 6.6.3.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatGetIMEI(BYTE IMEI[8]);

6.6.3.1.1	Normal execution

CRRN1: Send a PROVIDE LOCAL INFORMATION proactive command to the termina to retrieve the IMEI; return the retrieved IMEI in IMEI.

CRRN2: If IMEI is null, the invocation is a NOP that returns CAT_SUCCESS.

6.6.3.1.2	Parameter errors

6.6.3.1.3	Context errors

#### 6.6.3.2	Test Procedure

SET Test TO CatGetIMEI.

/* Test Case 1: Get IMEI */

IF   {CatGetIMEI(buffer) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, getIMEI, sizeof(getIMEI)) == 0}.

/* Test Case 2: Null IMEI */

IF   {CatGetIMEI(NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {NOP}.

#### 6.6.3.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

### 6.6.4	CatGetNetworkMeasurementResults

Test Area Reference: API_CatGetNetworkMeasurementResults

#### 6.6.4.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatGetNetworkMeasurementResults(BYTE MeasurementResults[10]);

6.6.4.1.1	Normal execution

CRRN1: Send a PROVIDE LOCAL INFORMATION proactive command to the terminal to retrieve the measurement results; return the retrieved measurement results in MeasurementResults.

CRRN2: If MeasurementResults is null, the invocation is a NOP that returns CAT_SUCCESS.

6.6.4.1.2	Parameter errors

6.6.4.1.3	Context errors

#### 6.6.4.2	Test Procedure

SET Test TO CatGetNetworkMeasurementResults.

/* Test Case 1: Get network measurement results */

IF   {CatGetNetworkMeasurementResults(buffer) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, getNetworkMeasurementResults, sizeof(getNetworkMeasurementResults)) == 0}.

/* Test Case 2: Null network measurement results */

IF   {CatGetNetworkMeasurementResults(NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {NOP}.

#### 6.6.4.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

### 6.6.5	CatGetDateTimeAndTimeZone

Test Area Reference: API_CatGetDateTimeAndTimeZone

#### 6.6.5.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatGetDateTimeAndTimeZone(BYTE DateTimeAndTimeZone[7]);

6.6.5.1.1	Normal execution

CRRN1: Send a PROVIDE LOCAL INFORMATION proactive command to the terminal to retrieve the date, time and time zone; return the retrieved date, time and time zone in DateTimeAndTimeZone.

CRRN2: If DateTimeAndTimeZone is null, the invocation is a NOP that returns CAT_SUCCESS.

6.6.5.1.2	Parameter errors

6.6.5.1.3	Context errors

#### 6.6.5.2	Test Procedure

SET Test TO CatGetDateTimeAndTimeZone.

/* Test Case 1: Get date, time and time zone */

IF   {CatGetDateTimeAndTimeZone(buffer) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, getDateTimeAndTimeZone, sizeof(getDateTimeAndTimeZone)) == 0}.

/* Test Case 2: Null date, time and time zone */

IF   {CatGetDateTimeAndTimeZone(NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {NOP}.

#### 6.6.5.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

### 6.6.6	CatGetLanguage

Test Area Reference: API_CatGetLanguage

#### 6.6.6.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatGetLanguage(BYTE Language[2]);

6.6.6.1.1	Normal execution

CRRN1: Send a PROVIDE LOCAL INFORMATION proactive command to the terminal to retrieve the current language selection; return the retrieved language selection in Language.

CRRN2: If Language is null, the invocation is a NOP that returns CAT_SUCCESS.

6.6.6.1.2	Parameter errors

6.6.6.1.3	Context errors

#### 6.6.6.2	Test Procedure

SET Test TO CatGetLanguage.

/* Test Case 1: Get language */

IF   {CatGetLanguage(buffer) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, getLanguage, sizeof(getLanguage)) == 0}.

/* Test Case 2: Null language */

IF   {CatGetLanguage(NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {NOP}

#### 6.6.6.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

### 6.6.7	CatSetupCall

Test Area Reference: API_CatSetupCall

#### 6.6.7.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatSetupCall(BYTE UserConfirmationMessageLength,

const void *UserConfirmationMessage, 
              CatTypeOfNumberAndNumberingPlanIdentifier TONandNPI, 
              BYTE DiallingNumberLength, const void *DiallingNumber, 
              CatSetupCallOptions Options,
              const CatIconIdentifier *UserConfirmationIconIdentifier,
              BYTE CallSetupMessageLength, const void *CallSetupMessage,
              const CatIconIdentifier *CallSeupIconIdentifier);

6.6.7.1.1	Normal execution

CRRN1: Send a SETUP CALL proactive command to the terminal with the provided data.

6.6.7.1.2	Parameter errors

- CRRP1: If DiallingNumberLength is zero or Dialling Number is null, the status word CAT_REQUIRED_VALUES_MISSING is returned.

6.6.7.1.3	Context errors

#### 6.6.7.2	Test Procedure

SET Test TO CatSetUpCall.

/* Test Case 1: Set up call */

IF   {CatSetupCall(3, "AOK", TON_UNKNOWN_AND_NPI_UNKNOWN, 12, "+155512345678",

0, NULL, 10, "Calling…", NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, setUpCall, sizeof(setUpCall)) == 0}.

/* Test Case 2: Null dialing number */

IF   {CatSetupCall(3, "AOK", TON_UNKNOWN_AND_NPI_UNKNOWN, 0, NULL,

0, NULL, 10, "Calling…", NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {NOP}.

#### 6.6.7.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| P1 | 2 |

### 6.6.8	CatSendShortMessage

Test Area Reference: API_CatSendShortMessage

#### 6.6.8.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatSendShortMessage(BYTE TitleLength, const void *Title, 
                     CatTypeOfNumberAndNumberingPlanIdentifier TONandNPI, 
                     BYTE AddressLength, const void *Address, 
                     BYTE SmsTPDULength, const void *SmsTPDU,
                     CatSendShortMessageOptions Options,
                     const CatIconIdentifier *IconIdentifier);

6.6.8.1.1	Normal execution

CRRN1: Send a SEND SHORT MESSAGE proactive command to the terminal with the provided data.

6.6.8.1.2	Parameter errors

- CRRP1: If AddressLength is zero or Address is null, the value CAT_REQUIRED_VALUES_MISSING is returned.

- CRRP2: If SmsTPDULength is zero or SmsTPDU is null, the value CAT_REQUIRED_VALUES_MISSING is returned.

6.6.8.1.3	Context errors

#### 6.6.8.2	Test Procedure

SET Test TO CatSendShortMessage

AND SmsTPDU TO {0x00, 0x01, 0x02, 0x03}.

/* Test Case 1: Send short message */

IF   {CatSendShortMessage(3, "SMS", TON_UNKNOWN_AND_NPI_UNKNOWN, 12, "+155512345678",

(BYTE)sizeof(SmsTPDU), SmsTPDU, 0, NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, sendShortMessage, sizeof(sendShortMessage)) == 0}.

/* Test Case 2: Null dialing number */

IF   {CatSendShortMessage(3, "SMS", TON_UNKNOWN_AND_NPI_UNKNOWN, 0, NULL,

(BYTE)sizeof(SmsTPDU), SmsTPDU, 0, NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {NOP}.

/* Test Case 3: Null SMS */

IF   {CatSendShortMessage(3, "SMS", TON_UNKNOWN_AND_NPI_UNKNOWN, 12, "+155512345678",

0, NULL, 0, NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, sendShortMessage, sizeof(sendShortMessage)) == 0}.

#### 6.6.8.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| P1 | 2 |
| P2 | 3 |

### 6.6.9	CatSendSS

Test Area Reference: API_CatSendSS

#### 6.6.9.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatSendSS(BYTE TitleLength, const void *Title, 
           CatTypeOfNumberAndNumberingPlanIdentifier TONandNPI, 
           BYTE SSStringLength, const void *SSString,
           const CatIconIdentifier *IconIdentifier);

6.6.9.1.1	Normal execution

CRRN1: Send a SEND SS proactive command to the terminal with the provided data.

CRRN2: If SSStringLength is zero or SSString is null, the invocation is a NOP that returns CAT_SUCCESS.

6.6.9.1.2	Parameter errors

6.6.9.1.3	Context errors

#### 6.6.9.2	Test Procedure

SET Test TO CatSendSS

AND SSString TO {0x00, 0x01, 0x02, 0x03}.

/* Test Case 1: Send SS */

IF   {CatSendSS(2, "SS", TON_UNKNOWN_AND_NPI_UNKNOWN,

(BYTE)sizeof(SSString), SSString, NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, sendSS, sizeof(sendSS)) == 0}.

/* Test Case 2: Null SS */

IF   {CatSendSS(2, "SS", TON_UNKNOWN_AND_NPI_UNKNOWN,

0, NULL, NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {NOP}.

#### 6.6.9.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

### 6.6.10	CatSendUSSD

Test Area Reference: API_CatSendUSSD

#### 6.6.10.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatSendUSSD(BYTE TitleLength, const void *Title, 
             CatDCSValue MessageDCS, BYTE MessageLength, const void *Message,
             CatDCSValue *MsgOutDCS, BYTE *MsgOutLength, void *MsgOut,
             const CatIconIdentifier *IconIdentifier);

6.6.10.1.1	Normal execution

CRRN1: Send a SEND USSD proactive command to the terminal with the provided data.

CRRN2: If MessageLength is zero or Message is null, the invocation is a NOP that returns CAT_SUCCESS.

CRRN3: If MsgOutDCS, MsgOutLength or MsgOut is null, only the CatGeneralResult return value of execution the  function is returned.

6.6.10.1.2	Parameter errors

6.6.10.1.3	Context errors

#### 6.6.10.2	Test Procedure

SET Test TO CatSendUSSD

AND SSString TO {0x00, 0x01, 0x02, 0x03}.

/* Test Case 1: Send SS */

IF   {CatSendUSSD(4, "USSD", DCS_UCS2, 4, "Time", &dcs, (BYTE*)&bytes, buffer, NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, sendUSSD1, sizeof(sendUSSD1)) == 0}.

/* Test Case 2: Null USSD */

IF   {CatSendUSSD(0, NULL, DCS_UCS2, 4, "Time", &dcs, (BYTE*)&bytes, buffer, NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {NOP}.

/* Test Case 3: Null response */

IF   {CatSendUSSD(4, "USSD", DCS_UCS2, 4, "Time", NULL, NULL, NULL, NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, sendUSSD2, sizeof(sendUSSD2)) == 0}.

#### 6.6.10.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |
| N3 | 3 |

### 6.6.11	CatOpenCSChannel

Test Area Reference: API_CatOpenCSChannel

#### 6.6.11.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatOpenCSChannel(CatOpenChannelOptions Options,
                 BYTE UserConfirmationLength, const void *UserConfirmation,
                 const CatIconIdentifier *UserConfimationIconIdentifier,
                 CatTypeOfNumberAndNumberingPlanIdentifier TONandNPI, 
                 BYTE DiallingNumberLength, const void *DiallingNumber,
                 BYTE BearerDescription[3], WORD *BufferSize, CatDevice *ChannelIdentifier);

6.6.11.1.1	Normal execution

CRRN1: Send an OPEN CHANNEL proactive command to the terminal that opens a circuit switched channel.

6.6.11.1.2	Parameter errors

- CRRP1: If DiallingNumberLength is zero or Dialling Number is null, the status word CAT_REQUIRED_VALUES_MISSING is returned.

6.6.11.1.3	Context errors

#### 6.6.11.2	Test Procedure

SET Test TO CatOpenCSChannel

AND bearer TO {0x00, 0x01, 0x02}.

/* Test Case 1: Open circuit channel */

IF   {CatOpenCSChannel(OPEN_CHANNEL_ON_DEMAND, 2, "OK", NULL, TON_UNKNOWN_AND_NPI_UNKNOWN,

12, "+155512345678", bearer, (WORD *)&bytes, &device) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, openCSChannel, sizeof(openCSChannel)) == 0}.

/* Test Case 2: Null dialing number */

IF   {CatOpenCSChannel(OPEN_CHANNEL_ON_DEMAND, 2, "OK", NULL, TON_UNKNOWN_AND_NPI_UNKNOWN,

12, "+155512345678", bearer, (WORD *)&bytes, &device) == CAT_COMMAND_SUCCESSFUL}

THEN {NOP}.

#### 6.6.11.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| P1 | 2 |

### 6.6.12	CatOpenGPRSChannel

Test Area Reference: API_CatOpenGPRSChannel

#### 6.6.12.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatOpenGPRSChannel(CatOpenChannelOptions Options,
                   BYTE UserConfirmationLength, const void *UserConfirmation,
                   const CatIconIdentifier *UserConfirmationIconIdentifier, 
                   BYTE BearerDescription[8], WORD *BufferSize, CatDevice *ChannelIdentifier);

6.6.12.1.1	Normal execution

CRRN1: Send an OPEN CHANNEL proactive command with the provided data to the terminal that opens a GPRS channel.

6.6.12.1.2	Parameter errors

6.6.12.1.3	Context errors

#### 6.6.12.2	Test Procedure

SET Test TO CatOpenGPRSChannel

AND bearer TO {0x00, 0x01, 0x02, 0x03, 0x04, 0x05, 0x06, 0x07}.

/* Test Case 1: Open GPRS channel */

IF   {CatOpenGPRSChannel(OPEN_CHANNEL_ON_DEMAND, 2, "OK", NULL,

bearer, (WORD *)&bytes, &device) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, openGPRSChannel, sizeof(openGPRSChannel)) == 0}.

#### 6.6.12.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |

### 6.6.13	CatCloseChannel

Test Area Reference: API_CatCloseChannel

#### 6.6.13.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatCloseChannel(CatDevice ChannelIdentifier,
                 BYTE TitleLength, const void *Title,
                 const CatIconIdentifier *IconIdentifier);

6.6.13.1.1	Normal execution

CRRN1: Send a CLOSE CHANNEL proactive command with the  provided data to the terminal.

6.6.13.1.2	Parameter errors

6.6.13.1.3	Context errors

#### 6.6.13.2	Test Procedure

SET Test TO CatCloseChannel.

/* Test Case 1: Close channel */

IF   {CatCloseChannel(DEVICE_CHANNEL_1, 3, "Bye", NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, closeChannel, sizeof(closeChannel)) == 0}.

#### 6.6.13.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |

### 6.6.14	CatReceiveData

Test Area Reference: API_CatReceiveData

#### 6.6.14.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatReceiveData(CatDevice ChannelIdentifier,
                BYTE TitleLength, const void *Title, BYTE RequestedChannelDataLength,
                const CatIconIdentifier *IconIdentifier,
                BYTE *ChannelData, BYTE *NumChannelBytesRead, BYTE *NumChannelBytesLeft);

6.6.14.1.1	Normal execution

CRRN1: Send a RECEIVE DATA proactive command to the terminal with the provided data.

CRRN2: If ChannelData or NumChannelBytesRead is null, the number of bytes available for reception on the channel is returned at NumChannelBytesLeft.

CRRN3: If NumChannelBytesLeft is null, the invocation is a NOP that returns CAT_SUCCESS.

6.6.14.1.2	Parameter errors

6.6.14.1.3	Context errors

#### 6.6.14.2	Test Procedure

SET Test TO CatReceiveData.

/* Test Case 1: Receive data */

IF   {CatReceiveData(DEVICE_CHANNEL_1, 8, "Incoming", 10, NULL,

buffer, (BYTE *)(&bytes), (BYTE *)(&bytes)) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, receiveData1, sizeof(receiveData1)) == 0}.

/* Test Case 2: Null data buffer */

IF   {CatReceiveData(DEVICE_CHANNEL_1, 8, "Incoming", 10, NULL,

NULL, (BYTE *)(&bytes), (BYTE *)(&bytes)) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, receiveData2, sizeof(receiveData2)) == 0}.

/* Test Case 3: Null bytes left */

IF   {CatReceiveData(DEVICE_CHANNEL_1, 8, "Incoming", 10, NULL,

buffer, (BYTE *)(&bytes), NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, receiveData3, sizeof(receiveData3)) == 0}.

#### 6.6.14.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |
| N3 | 3 |

### 6.6.15	CatSendData

Test Area Reference: API_CatSendData

#### 6.6.15.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatSendData(CatDevice ChannelIdentifier,
             CatSendDataOptions Options, BYTE TitleLength, const void *Title,
             BYTE ChannelDataLength, const void *ChannelData,
             const CatIconIdentifier *IconIdentifier, BYTE *ActualBytesSent);

6.6.15.1.1	Normal execution

CRRN1: Send a SEND DATA proactive command to the terminal with the provided data; if ActualBytesSent is non-null, the number of bytes actually sent is returnedin ActualBytesSent.

CRRN2: If ChannelDataLength is zero or ChannelData is null, the invocation is a NOP that returns CAT_SUCCESS.

6.6.15.1.2	Parameter errors

6.6.15.1.3	Context errors

#### 6.6.15.2	Test Procedure

SET Test TO CatSendData.

/* Test Case 1: Receive data */

IF   {CatSendData(DEVICE_CHANNEL_1, 0, 8, "Outgoing",

10, buffer, NULL, (BYTE *)&bytes) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, sendData1, sizeof(sendData1)) == 0}.

/* Test Case 2: Null data buffer */

IF   {CatSendData(DEVICE_CHANNEL_1, 0, 8, "Outgoing",

10, NULL, NULL, (BYTE *)&bytes) == CAT_COMMAND_SUCCESSFUL}

THEN {NOP}.

/* Test Case 3: Null bytes sent */

IF   {CatSendData(DEVICE_CHANNEL_1, 0, 8, "Outgoing",

10, buffer, NULL, NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, sendData2, sizeof(sendData2)) == 0}.

#### 6.6.15.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1,2 |
| N2 | 3 |

### 6.6.16	CatGetChannelStatus

Test Area Reference: API_CatGetChannelStatus

#### 6.6.16.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatGetChannelStatus(CatDevice ChannelIdentifier, void *ChannelStatus);

6.6.16.1.1	Normal execution

CRRN1: Send a GET CHANNEL STATUS proactive command to the terminal; if ChannelStatus is non-null return the channel status at ChannelStatus.

CRRN2: If ChannelStatus is null, invocation is a NOP that returns CAT_SUCCESS.

6.6.16.1.2	Parameter errors

6.6.16.1.3	Context errors

#### 6.6.16.2	Test Procedure

SET Test TO CatGetChannelStatus.

/* Test Case 1: Get channel status */

IF   {CatGetChannelStatus(DEVICE_CHANNEL_1, buffer) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, sendData1, sizeof(sendData1)) == 0}.

/* Test Case 2: Null status buffer */

IF   {CatGetChannelStatus(DEVICE_CHANNEL_1, NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {NOP}.

#### 6.6.16.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

### 6.6.17	CatServiceSearch

Test Area Reference: API_CatServiceSearch

#### 6.6.17.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatServiceSearch(CatBearer BearerId,
                BYTE AttributeLength, void *Attributes, void *ServiceAvailability);

6.6.17.1.1	Normal execution

CRRN1: Send a SERVICE SEARCH proactive command to the terminal with the provided data.

CRRN2: If AttributeLength is zero, Attributes is null or ServiceAvailability is null, invocation is a NOP that returns CAT_SUCCESS.

6.6.17.1.2	Parameter errors

6.6.17.1.3	Context errors

#### 6.6.17.2	Test Procedure

SET Test TO CatServiceSearch.

/* Test Case 1: Service search */

IF   {CatServiceSearch(BEARER_GPRS, 20, buffer, &bytes) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, serviceSearch, sizeof(serviceSearch)) == 0}.

/* Test Case 2: Attributes null */

IF   {CatServiceSearch(BEARER_GPRS, 20, buffer, &bytes) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, serviceSearch, sizeof(serviceSearch)) == 0}.

/* Test Case 3: Availability null */

IF   {CatServiceSearch(BEARER_GPRS, 20, buffer, &bytes) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, serviceSearch, sizeof(serviceSearch)) == 0}.

#### 6.6.17.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2, 3 |

### 6.6.18	CatGetServiceInformation

Test Area Reference: API_CatGetServiceInformation

#### 6.6.18.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatGetServiceInformation(BYTE TitleLength, const BYTE *Title,
                const CatIconIdentifier *IconIdentifier,
                BYTE BearerId, BYTE *AttributeLength, void *Attributes,
                void *ServiceInformation);

6.6.18.1.1	Normal execution

CRRN1:  Send a GET SERVICE INFORMATION proactive command to the terminal with the provided data.

CRNN2: If AttributesLength, Attributes or ServiceInformation is null, the invocation is a NOP that returns CAT_SUCCESS.

6.6.18.1.2	Parameter errors

6.6.18.1.3	Context errors

#### 6.6.18.2	Test Procedure

SET Test TO CatGetServiceInformation.

/* Test Case 1: Get service information */

IF   {CatGetServiceInformation(7, "Whither", NULL, BEARER_GPRS,

(BYTE *)&bytes, buffer, buffer) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, getServiceInformation, sizeof(getServiceInformation)) == 0}.

/* Test Case 2: Attributes null */

IF   {CatGetServiceInformation(7, "Whither", NULL, BEARER_GPRS,

(BYTE *)&bytes, NULL, buffer) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, getServiceInformation, sizeof(getServiceInformation)) == 0}.

/* Test Case 3: Service information null */

IF   {CatGetServiceInformation(7, "Whither", NULL, BEARER_GPRS,

(BYTE *)&bytes, buffer, NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, getServiceInformation, sizeof(getServiceInformation)) == 0}.

| Id | Description | API Expectation | APDU Expectation |
| --- | --- | --- | --- |
| 0 | UICC Initialisation | Responses ignored. |  |
| 1 | All arguments non-null CatGeneralResult rv; Char *title="Title"; BYTE attr[] = {1,2,3,4}, service[16]; 1 – rv = CatGetServiceInformation(strlen(title), title,NULL, BEARER_SMS, sizeof(attr), attr, service); | 1 – rv = CAT_SUCCESS | 1 – SERVICE INFORMATION proactive command |
| 2 | ServiceInformation nul CatGeneralResult rv; Char *title="Title"; BYTE attr[] = {1,2,3,4}, service[16]; 1 – rv = CatGetServiceInformation(strlen(title), title,NULL, BEARER_SMS, sizeof(attr), attr, NULL); | 1 –rv = CAT_SUCCESS |  |

#### 6.6.18.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

### 6.6.19	CatDeclareService

Test Area Reference: API_CatDeclareService

#### 6.6.19.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatDeclareService(BYTE BearerId, BYTE ServiceId,
                CatTransportProtocol TransportProtocol,
                WORD *PortNumber, BYTE ServiceRecordLength, void *ServiceRecord);

6.6.19.1.1	Normal execution

CRRN1:  Send a DECLARE SERVICE proactive command to the terminal with the provided data.

CRRN2: If ServiceRecordLength is zero or PortNumber or ServiceRecord is null, the invocation is a NOP that returns CAT_SUCCESS.

6.6.19.1.2	Parameter errors

6.6.19.1.3	Context errors

#### 6.6.19.2	Test Procedure

SET Test TO CatDeclareService.

/* Test Case 1: Declare service */

IF   {CatDeclareService(BEARER_GPRS, 1, TRANSPORT_PROTOCOL_TCP,

(WORD *)(&bytes), 10, buffer) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, declareService, sizeof(declareService)) == 0}.

/* Test Case 2: Service record null */

IF   {CatDeclareService(BEARER_GPRS, 1, TRANSPORT_PROTOCOL_TCP,

(WORD *)(&bytes), 10, buffer) == CAT_COMMAND_SUCCESSFUL}

THEN {NOP}.

#### 6.6.19.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

### 6.6.20	CatRunATCommand

Test Area Reference: API_CatRunATCommand

#### 6.6.20.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatRunATCommand(BYTE TitleLength, const void *Title, 
                 BYTE CommandLength, const void *Command,
                 const CatIconIdentifier *IconIdentifier,
                 void *Response, BYTE *ResponseLength);

6.6.20.1.1	Normal execution

CRRN1: Send a RUN AT COMMAND proactive command to the terminal with the provided data; *ResponseLength bytes of the result of the command are returned in Response if both are non-null.

CRNN2: If ResponseLength or Response is null, no results are returned.

CRNN3: If CommandLength is zero or Command is null, invocation is a NOP that returns CAT_SUCCESS.

6.6.20.1.2	Parameter errors

6.6.20.1.3	Context errors

#### 6.6.20.2	Test Procedure

SET Test TO CatRunATCommand.

/* Test Case 1: Run AT command */

IF   {CatRunATCommand(2, "AT", 10, buffer, NULL, buffer, (BYTE *)(&bytes)) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, runATCommand, sizeof(runATCommand)) == 0}.

/* Test Case 2: Command null */

IF   {CatRunATCommand(2, "AT", 0, NULL, NULL, buffer, (BYTE *)(&bytes)) == CAT_COMMAND_SUCCESSFUL}

THEN {NOP}.

/* Test Case 3: Response null */

IF   {CatRunATCommand(2, "AT", 10, buffer, NULL, NULL, NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, runATCommand, sizeof(runATCommand)) == 0}.

#### 6.6.20.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |
| N3 | 3 |

### 6.6.21	CatSendDTMFCommad

Test Area Reference: API_CatSendDTMFCommand

#### 6.6.21.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatSendDTMFCommand(BYTE TitleLength, const void *Title,
                    BYTE DTMFCodeLength, const void *DTMFCode,
                    const CatIconIdentifier *IconIdentifier);

6.6.21.1.1	Normal execution

CRRN1: Send a SEND DTMF COMMAND proactive command to the terminal with the provided data.

CRRN2: If DTMFCodeLength is zero or DTMFCode is null, the invocation is a NOP that returns CAT_SUCCESS.

6.6.21.1.2	Parameter errors

6.6.21.1.3	Context errors

#### 6.6.21.2	Test Procedure

SET Test TO CatSendDTMFCommand.

/* Test Case 1: Send DTMF command */

IF   {CatSendDTMFCommand(4, "DTMF", 10, buffer, NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, sendDTMFCommand, sizeof(sendDTMFCommand)) == 0}.

/* Test Case 2: Command null */

IF   {CatSendDTMFCommand(4, "DTMF", 0, NULL, NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {NOP}.

#### 6.6.21.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

## 6.7	Toolkit Application

### 6.7.1		main

Test Area Reference: API_main

#### 6.7.1.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Void main(void)

6.7.1.1.1	Normal execution

CRRN1: Load and execute the null application: main(){}.

6.7.1.1.2	Parameter errors

6.7.1.1.3	Context errors

#### 6.7.1.2	Test Procedure

SET Test TO main.

/* Test Case 1: main */

IF   {(CatExit(), SW_OK) == SW_OK}

THEN {NOP}.

#### 6.7.1.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |

### 6.7.2		CatGetFrameworkEvent

Test Area Reference: API_CatGetFrameworkEvent

#### 6.7.2.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatFrameworkEventType CatGetFrameworkEvent(void);

6.7.2.1.1	Normal execution

CRRN1:  Return the current framework event.

6.7.2.1.2	Parameter errors

6.7.2.1.3	Context errors

#### 6.7.2.2	Test Procedure

SET Test TO CatGetFrameworkEvent.

/* Test Case 1: Get framework event */

IF   {CatGetFrameworkEvent()}

THEN {NOP}.

#### 6.7.2.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |

### 6.7.3	CatExit

Test Area Reference: API_CatExit

#### 6.7.3.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Void CatExit(void);

6.7.3.1.1	Normal execution

CRRN1: Load and execute the exit application: main(){catExit();}

6.7.3.1.2	Parameter errors

6.7.3.1.3	Context errors

#### 6.7.3.2	Test Procedure

SET Test TO CatExit.

/* Test Case 1: CatExit */

IF   {(CatExit(), SW_OK) == SW_OK}

THEN {NOP}.

#### 6.7.3.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |

## 6.8	Miscellaneous

### 6.8.1	CatGetTerminalProfile

Test Area Reference: API_CatGetTerminalProfile

#### 6.8.1.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Void CatGetTerminalProfile(BYTE *ProfileOutLength, BYTE *Profile);

6.8.1.1.1	Normal execution

CRRN1: Retrieve the current terminal profile.

CRRN2: If ProfileOutLength or Profile is null, the invocation is a NOP.

6.8.1.1.2	Parameter errors

6.8.1.1.3	Context errors

#### 6.8.1.2	Test Procedure

SET Test TO CatGetTerminalProfile.

/* Test Case 1: Get terminal profile */

IF   {(CatGetTerminalProfile((BYTE *)(&bytes), buffer), SW_OK) == SW_OK}

THEN {NOP}.

/* Test Case 2: Null return */

IF   {(CatGetTerminalProfile(NULL, NULL), SW_OK) == SW_OK}

THEN {NOP}.

#### 6.8.1.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

### 6.8.2	CatMoreTime

Test Area Reference: API_CatMoreTime

#### 6.8.2.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatMoreTime(void);

6.8.2.1.1	Normal execution

CRRN1: Issue a MORE TIME request to the terminal.

6.8.2.1.2	Parameter errors

6.8.2.1.3	Context errors

Test Application: 		API_ CatMoreTime.c

#### 6.8.2.2	Test Procedure

SET Test TO CatMoreTime.

/* Test Case 1: Get more time */

IF   {CatMoreTime() == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, moreTime, sizeof(moreTime)) == 0}.

#### 6.8.2.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |

### 6.8.3	CatPollingOff

Test Area Reference: API_CatPollingOff

#### 6.8.3.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatPollingOff(void);

6.8.3.1.1	Normal execution

CRRN1:  Send the POLLING OFF proactive command to the terminal.

6.8.3.1.2	Parameter errors

6.8.3.1.3	Context errors

#### 6.8.3.2	Test Procedure

SET Test TO CatPollingOff.

/* Test Case 1: Turn polling off */

IF   {CatPollingOff() == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, pollingOff, sizeof(pollingOff)) == 0}.

#### 6.8.3.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |

### 6.8.4	CatPollInterval

Test Area Reference: API_CatPollInterval

#### 6.8.4.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatPollInterval(CatTimeUnit Unit, BYTE Interval,
                 CatTimeInterval *ActualIntervalOut);

6.8.4.1.1	Normal execution

CRRN1:  Send the POLL INTERVAL proactive command to the terminal.

CRRN2: If ActualIntervalOut is null, the actual interval set by the terminal is not returned.

6.8.4.1.2	Parameter errors

6.8.4.1.3	Context errors

#### 6.8.4.2	Test Procedure

SET Test TO CatPollInterval.

/* Test Case 1: Set polling interval */

IF   {CatPollInterval(GSM_SECONDS, 5, &timeInterval) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, pollInterval, sizeof(pollInterval)) == 0}.

/* Test Case 2: Null time interval return */

IF   {CatPollInterval(GSM_SECONDS, 5, &timeInterval) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, pollInterval, sizeof(pollInterval)) == 0}.

#### 6.8.4.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

### 6.8.5	CatRefresh

Test Area Reference: API_CatRefreshWithFileList

#### 6.8.5.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatRefreshWithFileList(CatRefreshOptions Options,
                        BYTE FileListLength, const void *FileList);

6.8.5.1.1	Normal execution

CRRN1:  Send a REFRESH proactive command to the terminal with the provided data.

CRRN2: If FileListLength is zero or FileList is null, send a REFRESH proactive command that refreshes all files.

6.8.5.1.2	Parameter errors

6.8.5.1.3	Context errors

#### 6.8.5.2	Test Procedure

AND fileList TO {0x10, 0x00, 0x20, 0x00}.

/* Test Case 1: Refresh file list */

IF   {CatRefreshWithFileList(REFRESH_SIM_RESET, 2, fileList) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, refreshWithFileList1, sizeof(refreshWithFileList1)) == 0}.

/* Test Case 2: Refresh all files */

IF   {CatRefreshWithFileList(REFRESH_SIM_RESET, 0, NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, refreshWithFileList2, sizeof(refreshWithFileList2)) == 0}.

#### 6.8.5.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

### 6.8.6	CatLanguageNotification

Test Area Reference: API_CatLanguageNotification

#### 6.8.6.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Void CatLanguageNotification(CatLanguageNotificationOptions Options,
                                 const void *Language);

6.8.6.1.1	Normal execution

CRRN1:  Send a LANGUAGE NOTIFICATION proactive command to the terminal with the provided data.

CRRN2: If Langauge is null, the invocation is a NOP.

6.8.6.1.2	Parameter errors

6.8.6.1.3	Context errors

#### 6.8.6.2	Test Procedure

SET Test TO CatLanguageNotification.

/* Test Case 1: Refresh file list */

IF   {(CatLanguageNotification(LANGUAGE_SPECIFIC_NOTIFICATION, buffer), SW_OK) == SW_OK}

THEN {memcmp(proactiveCommandBuffer, languageNotification, sizeof(languageNotification)) == 0}.

/* Test Case 2: Null buffer */

IF   {(CatLanguageNotification(LANGUAGE_SPECIFIC_NOTIFICATION, NULL), SW_OK) == SW_OK}

THEN {NOP}.

#### 6.8.6.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

### 6.8.7	CatLaunchBrowser

Test Area Reference: API_CatLaunchBrowser

#### 6.8.7.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatLaunchBrowser(CatLaunchBrowserOptions Options,
                  BYTE TitleLength, const void *Title,
                  BYTE URLLength, const void *URL,
                  const CatIconIdentifier *IconIdentifier);

6.8.7.1.1	Normal execution

CRRN1: Send a LAUNCH BROWSER proactive command to the terminal with the provided data.

6.8.7.1.2	Parameter errors

6.8.7.1.3	Context errors

#### 6.8.7.2	Test Procedure

SET Test TO CatLanguageNotification.

/* Test Case 1: Launch browser */

IF   {CatLaunchBrowser(LAUNCH_USE_EXISTING_BROWSER, 6, "Google",

21, "http://www.google.com", NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, launchBrowser, sizeof(launchBrowser)) == 0}.

#### 6.8.7.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |

## 6.9	Low-Level Interface

### 6.9.1	CatResetBuffer

Test Area Reference: API_CatResetBuffer

#### 6.9.1.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Void CatResetBuffer(void);

6.9.1.1.1	Normal execution

CRRN1:  Clear the buffer that is used to construct proactive commands.

6.9.1.1.2	Parameter errors

6.9.1.1.3	Context errors

#### 6.9.1.2	Test Procedure

SET Test TO CatResetBuffer.

/* Test Case 1: Reset proactive command buffer */

IF   {(CatResetBuffer(), SW_OK) == SW_OK}

THEN {NOP}.

#### 6.9.1.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |

### 6.9.2	CatStartProactiveCommand

Test Area Reference: API_CatStartProactiveCommand

#### 6.9.2.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Void CatStartProactiveCommand(BYTE Command, BYTE Options, CatDevice To);

6.9.2.1.1	Normal execution

CRRN1: Initialize the constructed proactive command buffer to contain the proactive command header that includes the provided data.

6.9.2.1.2	Parameter errors

6.9.2.1.3	Context errors

#### 6.9.2.2	Test Procedure

/*

** Test Procedure for CatStartProactiveCommand

*/

SET Test TO CatStartProactiveCommand.

/* Test Case 1: Start construction of a proactive command */

IF   {(CatStartProactiveCommand(SEND_SS_COMMAND, 0, DEVICE_NETWORK), SW_OK) == SW_OK}

THEN {NOP}.

#### 6.9.2.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |

### 6.9.3	CatSendProactiveCommand

Test Area Reference: API_CatSendProactiveCommand

#### 6.9.3.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatGeneralResult CatSendProactiveCommand(BYTE *Length);

6.9.3.1.1	Normal execution

CRRN1: Send the proactive command in the constructed proactive command to the terminal and return the result of the terminal's execution of the command.

CRRN2: If Length is null then the invocation is for the general result only.

6.9.3.1.2	Parameter errors

6.9.3.1.3	Context errors

#### 6.9.3.2	Test Procedure

SET Test TO CatSendProactiveCommand.

/* Test Case 1: Send a proactive command */

IF   {(CatStartProactiveCommand(SEND_SS_COMMAND, 0, DEVICE_NETWORK), SW_OK) == SW_OK}

AND  {CatSendProactiveCommand((BYTE *)&bytes) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, sendProactiveCommand, sizeof(sendProactiveCommand)) == 0}.

/* Test Case 2: Null argument */

IF   {(CatStartProactiveCommand(SEND_SS_COMMAND, 0, DEVICE_NETWORK), SW_OK) == SW_OK}

AND  {CatSendProactiveCommand(NULL) == CAT_COMMAND_SUCCESSFUL}

THEN {memcmp(proactiveCommandBuffer, sendProactiveCommand, sizeof(sendProactiveCommand)) == 0}.

#### 6.9.3.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

### 6.9.4	CatOpenEnvelope

Test Area Reference: API_CatOpenEnvelope

#### 6.9.4.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

CatEnvelopeTagType CatOpenEnvelope(BYTE *Length);

6.9.4.1.1	Normal execution

CRRN1: Return the tag and the length of the envelope command that arrived for the application.

CRRN2: If Length is null, on the tag is returned.

6.9.4.1.2	Parameter errors

6.9.4.1.3	Context errors

#### 6.9.4.2	Test Procedure

SET Test TO CatOpenEnvelope.

/* Test Case 1: Open an incoming envelope */

IF   {CatOpenEnvelope((BYTE *)&bytes) == EVENT_DOWNLOAD_TAG}

THEN {NOP}.

/* Test Case 2: Null argument */

IF   {CatOpenEnvelope(NULL) == EVENT_DOWNLOAD_TAG}

THEN {NOP}.

#### 6.9.4.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

### 6.9.5	CatSendEnvelopeResponse

Test Area Reference: API_CatSendEnvelopeResponse

#### 6.9.5.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Void CatSendEnvelopeResponse(void);

6.9.5.1.1	Normal execution

CRRN1: Send the contents of the constructed proactive command buffer to the terminal as a successful response to a envelope command.

6.9.5.1.2	Parameter errors

6.9.5.1.3	Context errors

#### 6.9.5.2	Test Procedure

SET Test TO CatSendEnvelopeResponse.

/* Test Case 1: Send a response to an envelope APDU */

IF    {(CatResetBuffer(), SW_OK) == SW_OK}

AND   {(CatPutByte(0x77), SW_OK) == SW_OK}

AND   {(CatSendEnvelopeResponse(), SW_OK) == SW_OK}

THEN {memcmp(proactiveCommandBuffer, sendEnvelopeResponse, sizeof(sendEnvelopeResponse)) == 0}.

#### 6.9.5.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |

### 6.9.6	CatSendEnvelopeErrorResponse

Test Area Reference: API_CatSendEnvelopeErrorResponse

#### 6.9.6.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Void CatSendEnvelopeErrorResponse(void);

6.9.6.1.1	Normal execution

CRRN1: Send the contents of the constructed proactive command buffer to the terminal as an unsuccessful response to a envelope command.

6.9.6.1.2	Parameter errors

6.9.6.1.3	Context errors

#### 6.9.6.2	Test Procedure

SET Test TO CatSendEnvelopeErrorResponse

AND error TO {0x6D, 0x00}.

/* Test Case 1: Send an error response to an envelope APDU */

IF    {(CatResetBuffer(), SW_OK) == SW_OK}

AND   {(CatPutData(sizeof(error), error), SW_OK) == SW_OK}

AND   {(CatSendEnvelopeErrorResponse(), SW_OK) == SW_OK}

THEN {memcmp(proactiveCommandBuffer, sendEnvelopeErrorResponse, sizeof(sendEnvelopeErrorResponse)) == 0}.

#### 6.9.6.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |

### 6.9.7	CatPutData

Test Area Reference: API_CatPutData

#### 6.9.7.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Void CatPutData(BYTE Length, const void *Data);

6.9.7.1.1	Normal execution

CRRN1: Append the provided data to the data in the constructed proactive command buffer.

CRRN2: If Length is zero or Data is null, the invocation is a NOP.

6.9.7.1.2	Parameter errors

6.9.7.1.3	Context errors

#### 6.9.7.2	Test Procedure

SET Test TO CatPutData

AND data TO {0x01, 0x02, 0x03, 0x04}.

/* Test Case 1: Put some data into the proactive command buffer */

IF    {(CatResetBuffer(), SW_OK) == SW_OK}

AND   {(CatPutData(sizeof(data), data), SW_OK) == SW_OK}

THEN {memcmp(proactiveCommandBuffer, putData, sizeof(putData)) == 0}.

/* Test Case 2: Null arguments */

IF    {(CatResetBuffer(), SW_OK) == SW_OK}

AND   {(CatPutData(0, NULL), SW_OK) == SW_OK}

THEN {NOP}.

#### 6.9.7.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

### 6.9.8	CatPutByte

Test Area Reference: API_CatPutByte

#### 6.9.8.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Void CatPutByte(BYTE Data)

6.9.8.1.1	Normal execution

CRRN1: Append the provided byte to the data in the constructed proactive command buffer.

6.9.8.1.2	Parameter errors

6.9.8.1.3	Context errors

#### 6.9.8.2	Test Procedure

SET Test TO CatPutByte.

/* Test Case 1: Put a byte into the proactive command buffer */

IF    {(CatResetBuffer(), SW_OK) == SW_OK}

AND   {(CatPutByte(0x77), SW_OK) == SW_OK}

THEN {memcmp(proactiveCommandBuffer, putByte, sizeof(putByte)) == 0}.

#### 6.9.8.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |

### 6.9.9	CatPutTLV

Test Area Reference: API_CatPutTLV

#### 6.9.9.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Void CatPutTLV(BYTE Tag, BYTE Length, const void *Value);

6.9.9.1.1	Normal execution

CRRN1: Append the provided TLV to the data in the constructed proactive command buffer.

CRRN2: If Value is null, the invocation is a NOP.

6.9.9.1.2	Parameter errors

6.9.9.1.3	Context errors

#### 6.9.9.2	Test Procedure

SET Test TO CatPutTLV

AND value TO {0x01, 0x02, 0x03, 0x04}.

/* Test Case 1: Put a TLV into the proactive command buffer */

IF    {(CatResetBuffer(), SW_OK) == SW_OK}

AND   {(CatPutTLV(R_APDU_TAG, sizeof(value), value), SW_OK) == SW_OK}

THEN {memcmp(proactiveCommandBuffer, putTLV, sizeof(putTLV)) == 0}.

/* Test Case 21: Null argument */

IF    {(CatResetBuffer(), SW_OK) == SW_OK}

AND   {(CatPutTLV(0, 0, NULL), SW_OK) == SW_OK}

THEN {NOP}.

#### 6.9.9.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

### 6.9.10	CatPutBytePrefixedTLV

Test Area Reference: API_CatPutBytePrefixedTLV

#### 6.9.10.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Void CatPutBytePrefixedTLV(BYTE Tag, BYTE Prefix, BYTE Length, const void *Value);

6.9.10.1.1	Normal execution

CRRN1: Append the provided data to the data in the constructed proactive command buffer.

CRRN2: If Value is null the invocation is a NOP.

6.9.10.1.2	Parameter errors

6.9.10.1.3	Context errors

#### 6.9.10.2	Test Procedure

SET Test TO CatPutBytePrefixedTLV

AND value TO {0x01, 0x02, 0x03, 0x04}.

/* Test Case 1: Put a TLV into the proactive command buffer */

IF    {(CatResetBuffer(), SW_OK) == SW_OK}

AND   {(CatPutBytePrefixedTLV(R_APDU_TAG, 0x77, sizeof(value), value), SW_OK) == SW_OK}

THEN {memcmp(proactiveCommandBuffer, putBytePrefixedTLV, sizeof(putBytePrefixedTLV)) == 0}.

/* Test Case 21: Null argument */

IF    {(CatResetBuffer(), SW_OK) == SW_OK}

AND   {(CatPutBytePrefixedTLV(0, 0, 0, NULL), SW_OK) == SW_OK}

THEN {NOP}.

#### 6.9.10.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |

### 6.9.11	CatPutOneByteTLV

Test Area Reference: API_CatPutOneByteTLV

#### 6.9.11.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Void CatPutOneByteTLV(BYTE Tag, BYTE Value);

6.9.11.1.1	Normal execution

CRRN1: Append the provided data to the data in the constructed proactive command buffer.

6.9.11.1.2	Parameter errors

6.9.11.1.3	Context errors

#### 6.9.11.2	Test Procedure

SET Test TO CatPutOneByteTLV.

/* Test Case 1: Put a TLV into the proactive command buffer */

IF    {(CatResetBuffer(), SW_OK) == SW_OK}

AND   {(CatPutOneByteTLV(R_APDU_TAG, 0x77), SW_OK) == SW_OK}

THEN {memcmp(proactiveCommandBuffer, putOneByteTLV, sizeof(putOneByteTLV)) == 0}.

#### 6.9.11.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |

### 6.9.12	CatPutTwoByteTLV

Test Area Reference: API_CatPutTwoByteTLV

#### 6.9.12.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Void CatPutTwoByteTLV(BYTE Tag, BYTE Value1, BYTE Value2);

6.9.12.1.1	Normal execution

CRRN1: Append the provided data to the data in the constructed proactive command buffer.

6.9.12.1.2	Parameter errors

6.9.12.1.3	Context errors

#### 6.9.12.2	Test Procedure

SET Test TO CatPutTwoByteTLV.

/* Test Case 1: Put a TLV into the proactive command buffer */

IF    {(CatResetBuffer(), SW_OK) == SW_OK}

AND   {(CatPutTwoByteTLV(R_APDU_TAG, 0x77, 0x88), SW_OK) == SW_OK}

THEN {memcmp(proactiveCommandBuffer, putTwoByteTLV, sizeof(putTwoByteTLV)) == 0}.

#### 6.9.12.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |

### 6.9.13	CatGetByte

Test Area Reference: API_CatGetByte

#### 6.9.13.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

BYTE CatGetByte(void);

6.9.13.1.1	Normal execution

CRRN1: Return the byte at the current buffer pointer and advance the pointer by one byte.

6.9.13.1.2	Parameter errors

6.9.13.1.3	Context errors

#### 6.9.13.2	Test Procedure

SET Test TO CatGetByte.

/* Test Case 1: Get a byte from the proactive command buffer */

IF   {(CatResetBuffer(), SW_OK) == SW_OK}

AND  {(CatPutTwoByteTLV(R_APDU_TAG, 0x77, 0x88), SW_OK) == SW_OK}

AND  {(CatResetBuffer(), SW_OK) == SW_OK}

THEN {CatGetByte() == R_APDU_TAG}.

#### 6.9.13.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |

### 6.9.14	CatGetData

Test Area Reference: API_CatGetData

#### 6.9.14.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Const void *CatGetData(BYTE Length);

6.9.14.1.1	Normal execution

CRRN1: Return the current data pointer and then increment the pointer by Length.

6.9.14.1.2	Parameter errors

6.9.14.1.3	Context errors

#### 6.9.14.2	Test Procedure

SET Test TO CatGetData

AND data TO {R_APDU_TAG, 0x02, 0x77, 0x88}.

/* Test Case 1: Get some data from the proactive command buffer */

IF   {(CatResetBuffer(), SW_OK) == SW_OK}

AND  {(CatPutTwoByteTLV(R_APDU_TAG, 0x77, 0x88), SW_OK) == SW_OK}

AND  {(CatResetBuffer(), SW_OK) == SW_OK}

THEN {memcmp((BYTE *)CatGetByte(), data, sizeof(data)) == 0}.

#### 6.9.14.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |

### 6.9.15	CatFindNthTLV

Test Area Reference: API_CatFindNthTLV

#### 6.9.15.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Const void *CatFindNthTLV(BYTE Tag, BYTE Occurrence, BYTE *Length);

6.9.15.1.1	Normal execution

CRRN1: Return a pointer to the Occurance appearance of a TLV with the provided Tag in the data buffer and the length of this TLV at Length.

CRRN2: If there are less than Occurance appearances of TLVs with the provided Tag, return null.

CRRN3: If Length is null, do not return the Length of the TLV if one is found.

6.9.15.1.2	Parameter errors

6.9.15.1.3	Context errors

#### 6.9.15.2	Test Procedure

SET Test TO CatFindNthTLV

AND tlvs TO {0x80, 0x01, 0x00, 0x81, 0x01, 0x01, 0x81, 0x02, 0x01, 0x02}.

/* Test Case 1: Find a TLV */

IF   {(CatResetBuffer(), SW_OK) == SW_OK}

AND  {(CatPutData(sizeof(tlvs), tlvs), SW_OK) == SW_OK}

AND  {(CatResetBuffer(), SW_OK) == SW_OK}

THEN {memcmp((BYTE *)CatFindNthTLV(0x81, 2, buffer), tlvs+6, buffer[0]) == 0}.

/* Test Case 2: Too few appearances */

IF   {(CatResetBuffer(), SW_OK) == SW_OK}

AND  {(CatPutData(sizeof(tlvs), tlvs), SW_OK) == SW_OK}

AND  {(CatResetBuffer(), SW_OK) == SW_OK}

THEN {(BYTE *)CatFindNthTLV(0x81, 3, buffer) == NULL}.

/* Test Case 3: Null argument */

IF   {(CatResetBuffer(), SW_OK) == SW_OK}

AND  {(CatPutData(sizeof(tlvs), tlvs), SW_OK) == SW_OK}

AND  {(CatResetBuffer(), SW_OK) == SW_OK}

THEN {memcmp((BYTE *)CatFindNthTLV(0x81, 2, NULL), tlvs+6, 4) == 0}.

#### 6.9.15.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |
| N3 | 3 |

### 6.9.16	CatFindNthTLVInUserBuffer

Test Area Reference: API_CatFindNthTLVInUserBuffer

#### 6.9.16.1	Conformance Requirements

The entry point with the following header shall be compliant to its definition in the API.

Const void *CatFindNthTLVInUserBuffer(BYTE BufferLen, 
                           const void *Buffer, BYTE Tag, 
                           BYTE Occurrence, BYTE *Length);

6.9.16.1.1	Normal execution

CRRN1: Return a pointer to the Occurance appearance of a TLV with the provided Tag in the provided buffer and the length of this TLV at Length.

CRRN2: If there are less than Occurance appearances of TLVs with the provided Tag, return null.

CRRN3: If Length is null, do not return the Length of the TLV if one is found.

CRNN4: If BufferLen is zero or Buffer is null, the invocation is a NOP that returns null.

6.9.16.1.2	Parameter errors

6.9.16.1.3	Context errors

#### 6.9.16.2	Test Procedure

SET Test TO CatFindNthTLVInUserBuffer

AND tlvs TO {0x80, 0x01, 0x00, 0x81, 0x01, 0x01, 0x81, 0x02, 0x01, 0x02}.

/* Test Case 1: Find a TLV */

IF   {OK}

THEN {memcmp((BYTE *)CatFindNthTLVInUserBuffer(sizeof(tlvs), tlvs, 0x81, 2, buffer), tlvs+6, buffer[0]) == 0}.

/* Test Case 2: Too few appearances */

IF   {OK}

THEN {(BYTE *)CatFindNthTLVInUserBuffer(sizeof(tlvs), tlvs, 0x81, 3, buffer) == NULL}.

/* Test Case 3: Length null */

IF   {OK}

THEN {CatFindNthTLVInUserBuffer(sizeof(tlvs), tlvs, 0x81, 2, NULL) == tlvs+6}.

/* Test Case 4: Buffer null */

IF   {OK}

THEN {CatFindNthTLVInUserBuffer(0, NULL, 0x81, 2, buffer) == 0}.

#### 6.9.16.3	Test Coverage

| CRR number | Test Case Number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |
| N3 | 3 |
| N4 | 4 |

Annex A (normative):
Script file syntax and format description

# A.1	Syntax description

Following is a syntax description in BNF.

<statement list> ::= 	[ <statement> \n] +

<statement> ::= 	<simple> | <switch> | <blank line>

<simple> ::= 	<reset> | <init> | <command> | <remark>

<reset> ::= 	RST

<init> ::= 	INI <hexdata>

<command> ::=  	CMD <hexdata> [ <response> ] ( <status> )

<response> ::= 	[ <hexdata> ]

<status> ::= 	( <hexdata> )

<remark> ::= 	REM <text line>

<switch> ::= 	SWI { [<labelled list>] + }

<labelled list> ::= 	<label> : \n <statement list>

Description of syntax metalanguage :

\n	represents a linebreak

[ x ]	means x can appear optionally

[ x ] +	means 1 or more appearances of x

x | y	means x or y

[]{}:	(bold) these are characters that appear literally in the script files

<text line>	any character until the end of the line

<blank line>	a line containing no text is acceptable

<hexdata>	data written in hexadecimal, each byte separated from the following by a whitespace

Each simple statement beginning with 3 characters different than the ones defined indicates another tool command, and shall be ignored by the parser if not recognised.

' ', '\t' : Can be used as separator

A long statement can be broken into several lines by using the character '\' at the end of each line which is not the last one in the statement.

For more details refer to the examples in A.3.

# A.2	Semantics

Following is the meaning of each of the statements:

CMD : 	Sends an APDU Command to  the card, including (optionally) the expected response data and also (optionally) the expected status words SW1, SW2.

RST : 	Resets and powers on the card

INI : 	Performs the terminal profile with the following data. Afterwards, it shall perform all the fetch and terminal response commands until there is no proactive session in progress.

REM : 	Used for comments

SWI : 	Activates a switch condition. Every labelled list represents a list of statements to be executed, if the label matches the SW resulting from the previously executed command.

Evaluation of expected response and status in the case of a CMD:

<response> 	data within […] has to be checked, it needs to be present for an outgoing command. Bytes written as XX shall not be checked by the APDU tool.

<status>	status contained within (…) has to be checked; when several status are valid they shall be separated by commas. Bytes written as XX shall not be checked by the APDU tool.

# A.3	Example

REM this is an example

RST

INI FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF

REM Case 1 example

CMD A0 C2 00 00 00 (91 33 , 69 XX)

REM Case 2 example

CMD A0 B6 00 00 07 \

[XX XX XX 55 55 XX 55] \

(91 33 , 67 XX)

CMD A0 B6 00 00 07 \

(91 33 , 67 XX)

CMD A0 C0 00 00 1F \

[10 A0 00 00 00 09 00 02 FF FF FF FF 89 28 A4 05 \

02 0D CC CC CC CC CC CC CC CC CC CC CC CC CC ] \

(90 00)

REM Case 3 example

CMD A0 C2 00 00 33 \

D1 31 82 02 83 81 06 05 80 11 22 33 44 8B 24 40 \

08 00 24 23 85 18 41 04 51 10 10 00 00 00 00 13 \

02 70 00 00 0E 0D 00 00 00 00 28 A4 05 00 00 00 \

00 00 00 \

(90 00)

REM Case 4 example with switch statement

CMD 00 A4 04 00 10 \

A0 00 00 00 09 00 02 FF FF FF FF 89 41 04 44 02 \

(61 XX, 6A 82)

SWI {

61 XX:

CMD 00 C0 00 00 14 \

[10 A0 00 00 00 09 00 02 FF FF FF FF 89 41 04 44 \

02 02 CC CC] \

(90 00)

CMD A0 A4 00 00 02 \

3F 00

6A 82:

RST

}

## A.4	Style and formatting

In order to  show a common appearance all the scripts shall follow those format rules:

	start always with a 'RST' followed by an 'INI' command.

	The command, data to be checked and status to be checked shall be presented in the following order:

CMD COMMAND  [EXPECTED DATA]  (EXPECTED STATUS)

	APDU shall be presented with command (CLA INS P1 P2 P3) in one line and data (if present) in next line grouped  16 bytes per line (see example above).

	The expected data (if present) shall be presented in 16 bytes groups per line (see example above).

Annex B (normative):
Default Prepersonalisation

# B.1	General Default Prepersonalisation

This table shows the default prepersonalisation, the file system and the files' content, that the test SIM cards shall contain  unless otherwise stated.

| Name | Identifier | Default Value | Special Features |
| --- | --- | --- | --- |
| EFICCID | 2FE2 | 0F FF FF FF FF FF FF FF FF FF | This value is not compliant with GSM 11.11 |
| EFIMSI | 6F07 | FF FF FF FF FF FF FF FF FF | This value is not compliant with GSM 11.11 |
| EFLP | 6F05 | 01 FF FF FF |  |
| EFKc | 6F20 | FF FF FF FF FF FF FF FF 07 |  |
| EFPLMNsel | 6F30 | FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF |  |
| EFHPLMN | 6F31 | 05 |  |
| EFACMmax | 6F37 | 00 00 00 | Access condition UPDATE: CHV1 |
| EFSST | 6F38 | FF 3F C3 03 0C 00 FF 0F 00 33 |  |
| EFACM | 6F39 | 00 00 00 | Access condition UPDATE: CHV1 |
| EFPUCT | 6F41 | FF FF FF 00 00 | Access condition UPDATE: CHV1 |
| EFBCCH | 6F74 | FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF |  |
| EFACC | 6F78 | 00 00 |  |
| EFFPLMN | 6F7B | FF FF FF FF FF FF FF FF FF FF FF FF |  |
| EFLOCI | 6F7E | FF FF FF FF 00 F0 00 00 00 FF 01 |  |
| EFAD | 6FAD | 00 FF FF |  |
| EFPhase | 6FAE | 03 |  |
| EFFDN | 6F3B | Default value in all the records: FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF | Records: 5 |
| EFSMSP | 6F42 | FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF | Records: 1 |
| EFLND | 6F44 | FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF | Records: 1 |
| EFSMSS | 6F43 | FF FF |  |
| EFSMS | 6F3C | 1st record: 00 FF … FF(length 176) 2nd record:00 FF … FF(length 176) 3rd record: 00 FF … FF(length 176) | Records: 3 |
| EFADN | 6F3A | FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF | Records: 1 |
| EFCCP | 6F3D | FF FF FF FF FF FF FF FF FF FF FF FF FF FF |  |
| EFMSISDN | 6F40 | FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF | Records: 1 |
| EFSDN | 6F49 | FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF | Records: 1 |
| EFSUME | 6F54 | 85 0C 54 4F 4F 4C 4B 49 54 20 54 45 53 54 FF FF FF FF |  |
| EFCBMI | 6F45 | FF FF |  |
| EFIM | 4F20 | FF FF FF FF FF FF FF FF FF FF FF |  |

The default value for the CHV1 shall be "0x31 0x31 0x31 0x31 0xFF 0xFF 0xFF 0xFF" and its state shall be 'disabled' during test Applications execution.

# B.2	File System Access Default Prepersonalisation

## B.2.1	DFSIMTEST (SIM Test)

Identifier: '3113'

## B.2.2	EFTNR (Transparent Never Read)

| Identifier: '6F01' |  |  | Structure: transparent |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| File size: 3 bytes |  |  |  | Update activity: low |  |  |
| Access Conditions: READ NEVER UPDATE ALWAYS INVALIDATE ALWAYS REHABILITATE ALWAYS |  |  |  |  |  |  |
| Bytes | Description | Default Value |  |  | M/O | Length |
| 1 – 3 | Test Data | AA AA AA |  |  | M | 3 bytes |

## B.2.3	EFTNU (Transparent Never Update)

| Identifier: '6F02' |  |  | Structure: transparent |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| File size: 3 bytes |  |  |  | Update activity: low |  |  |
| Access Conditions: READ ALWAYS UPDATE NEVER INVALIDATE ALWAYS REHABILITATE ALWAYS |  |  |  |  |  |  |
| Bytes | Description | Default Value |  |  | M/O | Length |
| 1 – 3 | Test Data | 55 55 55 |  |  | M | 3 bytes |

## B.2.4	EFTARU (Transparent Always Read and Update)

| Identifier: '6F03' |  |  | Structure: transparent |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| File size: 260 bytes |  |  |  | Update activity: low |  |  |
| Access Conditions: READ ALWAYS UPDATE ALWAYS INVALIDATE ALWAYS REHABILITATE ALWAYS |  |  |  |  |  |  |
| Bytes | Description | Default Value |  |  | M/O | Length |
| 1 – 260 | Test Data | FF … FF |  |  | M | 260 bytes |

## B.2.5	EFCNR (Cyclic Never Read)

| Identifier: '6F04' |  | Structure: cyclic |  |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 3 bytes |  |  |  | Update activity: high |  |  |  |
| Access Conditions: READ NEVER UPDATE ALWAYS INCREASE ALWAYS INVALIDATE ALWAYS REHABILITATE ALWAYS |  |  |  |  |  |  |  |
| Logical Record Number | Description |  | Default Value |  | M/O |  | Length |
| 1 | Test Data |  | 00 00 00 |  | M |  | 3 bytes |
| 2 | Test Data |  | 00 00 00 |  | M |  | 3 bytes |

## B.2.6	EFCNU (Cyclic Never Update)

| Identifier: '6F05' |  | Structure: cyclic |  |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 3 bytes |  |  |  | Update activity: high |  |  |  |
| Access Conditions: READ ALWAYS UPDATE NEVER INCREASE NEVER INVALIDATE ALWAYS REHABILITATE ALWAYS |  |  |  |  |  |  |  |
| Logical Record Number | Description |  | Default Value |  | M/O |  | Length |
| 1 | Test Data |  | 00 00 00 |  | M |  | 3 bytes |
| 2 | Test Data |  | 00 00 00 |  | M |  | 3 bytes |

## B.2.7	EFCNIC (Cyclic Never Increase)

| Identifier: '6F06 |  | Structure: cyclic |  |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 3 bytes |  |  |  | Update activity: high |  |  |  |
| Access Conditions: READ ALWAYS UPDATE ALWAYS INCREASE NEVER INVALIDATE ALWAYS REHABILITATE ALWAYS |  |  |  |  |  |  |  |
| Logical Record Number | Description |  | Default Value |  | M/O |  | Length |
| 1 | Test Data |  | 00 00 00 |  | M |  | 3 bytes |
| 2 | Test Data |  | 00 00 00 |  | M |  | 3 bytes |

## B.2.8	EFCNIV (Cyclic Never Invalidate)

| Identifier: '6F07 |  | Structure: cyclic |  |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 3 bytes |  |  |  | Update activity: high |  |  |  |
| Access Conditions: READ ALWAYS UPDATE ALWAYS INCREASE ALWAYS INVALIDATE NEVER REHABILITATE ALWAYS |  |  |  |  |  |  |  |
| Logical Record Number | Description |  | Default Value |  | M/O |  | Length |
| 1 | Test Data |  | 00 00 00 |  | M |  | 3 bytes |
| 2 | Test Data |  | 00 00 00 |  | M |  | 3 bytes |

## B.2.9	EFCNRH (Cyclic Never Rehabilitate)

| Identifier: '6F08' |  | Structure: cyclic |  |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 3 bytes |  |  |  | Update activity: high |  |  |  |
| Access Conditions: READ ALWAYS UPDATE ALWAYS INCREASE ALWAYS INVALIDATE ALWAYS REHABILITATE NEVER |  |  |  |  |  |  |  |
| Logical Record Number | Description |  | Default Value |  | M/O |  | Length |
| 1 | Test Data |  | 00 00 00 |  | M |  | 3 bytes |
| 2 | Test Data |  | 00 00 00 |  | M |  | 3 bytes |

## B.2.10	EFCARU (Cyclic Always Read and Update)

| Identifier: '6F09' |  | Structure: cyclic |  |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 3 bytes |  |  |  | Update activity: high |  |  |  |
| Access Conditions: READ ALWAYS UPDATE ALWAYS INCREASE ALWAYS INVALIDATE ALWAYS REHABILITATE ALWAYS |  |  |  |  |  |  |  |
| Logical Record Number | Description |  | Default Value |  | M/O |  | Length |
| 1 | Test Data |  | 55 55 55 |  | M |  | 3 bytes |
| 2 | Test Data |  | AA AA AA |  | M |  | 3 bytes |

## B.2.11	EFLNR (Linear Fixed Never Read)

| Identifier: '6F0A' |  | Structure: linear fixed |  |  | Mandatory |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 4 bytes |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ NEVER UPDATE ALWAYS INVALIDATE ALWAYS REHABILITATE ALWAYS |  |  |  |  |  |  |  |
| Logical Record Number | Description |  | Default Value |  |  | M/O | Length |
| 1 | Test Data – Record 1 |  | FF FF FF FF |  |  | M | 4 bytes |
| 2 | Test Data – Record 2 |  | FF FF FF FF |  |  | M | 4 bytes |

## B.2.12	EFLNU (Linear Fixed Never Update)

| Identifier: '6F0B' |  | Structure: linear fixed |  |  | Mandatory |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 4 bytes |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALWAYS UPDATE NEVER INVALIDATE ALWAYS REHABILITATE ALWAYS |  |  |  |  |  |  |  |
| Logical Record Number | Description |  | Default Value |  |  | M/O | Length |
| 1 | Test Data – Record 1 |  | FF FF FF FF |  |  | M | 4 bytes |
| 2 | Test Data – Record 2 |  | FF FF FF FF |  |  | M | 4 bytes |

## B.2.13	EFLARU (Linear Fixed Always Read and Update)

| Identifier: '6F0C' |  | Structure: linear fixed |  |  | Mandatory |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 4 bytes |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALWAYS UPDATE ALWAYS INVALIDATE ALWAYS REHABILITATE ALWAYS |  |  |  |  |  |  |  |
| Logical Record Number | Description |  | Default Value |  |  | M/O | Length |
| 1 | Test Data – Record 1 |  | 55 55 55 55 |  |  | M | 4 bytes |
| 2 | Test Data – Record 2 |  | AA AA AA AA |  |  | M | 4 bytes |

## B.2.14	EFCINA (Cyclic Increase Not Allowed)

| Identifier: '6F0D' |  | Structure: cyclic |  |  | Mandatory |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 3 bytes |  |  |  | Update activity: high |  |  |  |
| Access Conditions: READ ALWAYS UPDATE ALWAYS INCREASE ALWAYS (see note 1) INVALIDATE ALWAYS REHABILITATE ALWAYS |  |  |  |  |  |  |  |
| Logical Record Number | Description |  | Default Value |  |  | M/O | Length |
| 1 | Test Data |  | 00 00 00 |  |  | M | 3 bytes |
| 2 | Test Data |  | 00 00 00 |  |  | M | 3 bytes |
| Note 1: This file will be personalised in a way such that increase is not allowed, as indicated by the FCI byte 8, bit 7 (GSM 11.11: FCI structure of an EF returned by the SELECT command) |  |  |  |  |  |  |  |

## B.2.15	EFTRAC (Transparent Read Access Condition CHV2)

| Identifier: '6F0E' |  | Structure: transparent |  |  | Mandatory |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 3 bytes |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ CHV2 UPDATE ALWAYS INCREASE ALWAYS INVALIDATE ALWAYS REHABILITATE ALWAYS |  |  |  |  |  |  |  |
| Logical Record Number | Description |  | Default Value |  |  | M/O | Length |
| 1 | Test Data |  | 00 00 00 |  |  | M | 3 bytes |

## B.2.16	EFTIAC (Transparent Invalidate Access Condition CHV1)

| Identifier: '6F0F' |  | Structure: transparent |  |  | Mandatory |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 3 bytes |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALWAYS UPDATE ALWAYS INCREASE ALWAYS INVALIDATE CHV1 REHABILITATE ALWAYS |  |  |  |  |  |  |  |
| Logical Record Number | Description |  | Default Value |  |  | M/O | Length |
| 1 | Test Data |  | 00 00 00 |  |  | M | 3 bytes |

## B.2.17	EFCIAC (Cyclic Increase Access Condition CHV2)

| Identifier: '6F10' |  | Structure: cyclic |  |  | Mandatory |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 3 bytes |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALWAYS UPDATE ALWAYS INCREASE CHV2 INVALIDATE ALWAYS REHABILITATE ALWAYS |  |  |  |  |  |  |  |
| Logical Record Number | Description |  | Default Value |  |  | M/O | Length |
| 1 | Test Data |  | 00 00 00 |  |  | M | 3 bytes |
| 2 | Test Data |  | 00 00 00 |  |  | M | 3 bytes |

## B.2.18	EFCIAA (Cyclic Increase Access Condition ADM)

| Identifier: '6F11' |  | Structure: cyclic |  |  | Mandatory |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 3 bytes |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALWAYS UPDATE ALWAYS INCREASE ADM INVALIDATE ALWAYS REHABILITATE ALWAYS |  |  |  |  |  |  |  |
| Logical Record Number | Description |  | Default Value |  |  | M/O | Length |
| 1 | Test Data |  | 00 00 00 |  |  | M | 3 bytes |
| 2 | Test Data |  | 00 00 00 |  |  | M | 3 bytes |

## B.2.19	EFCNRI (Cyclic Never Rehabilitate Invalidated)

| Identifier: '6F12' |  | Structure: cyclic |  |  | Mandatory |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 3 bytes |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALWAYS UPDATE ALWAYS INCREASE ALWAYS INVALIDATE ALWAYS REHABILITATE NEVER |  |  |  |  |  |  |  |
| Logical Record Number | Description |  | Default Value |  |  | M/O | Length |
| 1 | Test Data |  | 00 00 00 |  |  | M | 3 bytes |
| 2 | Test Data |  | 00 00 00 |  |  | M | 3 bytes |

The file status shall be invalidated as defined in [12]

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc | CR | Rev | Cat | Subject/Comment | Old | New |
| 2003-03 | TP-19 | - | - | - | - | approved | 2.0.0 | 6.0.0 |
| 2005-01 | - | - | - | - | - | History box updated to show approval; TS title expanded to show abbreviations. | 6.0.0 | 6.0.1 |
| 2007-06 | - | - | - | - | - | Update to Rel-7 version (MCC) | 6.0.1 | 7.0.0 |
| 2009-03 | CP-43 |  |  |  |  | Upgrade to Rel-8 without technical change | 7.0.0 | 8.0.0 |
| 2009-04 |  |  |  |  |  | Update change history table | 8.0.0 | 8.0.1 |
| 2009-12 | CT-46 | - | - | - | - | Upgrade of the specification to Rel-9 | 8.0.1 | 9.0.0 |
| 2011-03 | SP-51 | - | - | - | - | Upgrade of the specification to Rel-10 | 9.0.0 | 10.0.0 |
| 2012-09 | SP-57 | - | - | - | - | Upgrade of the specification to Rel-11 | 10.0.0 | 11.0.0 |
| 2014-10 | SP-65 | - | - | - | - | Upgrade of the specification to Rel-12 | 11.0.0 | 12.0.0 |
| 2015-12 | SP-70 | - | - | - | - | Upgrade of the specification to Rel-13 | 12.0.0 | 13.0.0 |
| 2017-03 | SA-75 | - | - | - | - | Update to Rel-14 version (MCC) | 13.0.0 | 14.0.0 |
| 2018-07 | SA-80 |  |  |  |  | Update to Rel-15 version (MCC) | 14.0.0 | 15.0.0 |