3GPP TR 36.748 V15.0.0 (2017-09)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Technical report for 450MHz E-UTRA FDD Band for LTE PPDR

and PMR/PAMR in Europe

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<E-UTRA, Radio>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	6

3.3	Abbreviations	6

4	Background	6

5	Band plan allocation and regulatory background	6

5.1	Regulatory background	6

5.2	Frequency arrangement for the 400MHz option of the ECC Decision (16)02	7

6	List of band specific issues	9

7	General issues	9

7.1	Operating band and channel bandwidths	9

8	Study of E-UTRA specific issues	11

8.1	UE duplex-filter study	11

8.2	UE transmitter requirements	12

8.2.1	Maximum output power:	12

8.2.2	Spurious emission band UE co-existence:	12

8.3	UE receiver requirements	17

8.3.1	REFSENS	17

8.3.2	Inband blocking	18

8.3.3	Out of band blocking:	18

8.4	BS aspect issues	18

9	Study of MSR specific issues	21

10	Channel numbering for E-UTRA, MSR	21

11	Required changes to E-UTRA and MSR specifications	21

11.1	Changes to TS 36.101	22

Annex A:	Change history	24

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is a technical report for the work item “450MHz E-UTRA FDD Band for LTE PPDR and PMR/PAMR in Europe”, which was approved at TSG RAN#74 [3].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 30.007: "Guideline on WI/SI for new Operating Bands ".

[3]	RP-162514: " New WI proposal: 450MHz E-UTRA FDD Band for LTE PPDR and PMR/PAMR in Europe ".

[4]	R4-165137: " ETSI standards / 3GPP specifications for BB-PPDR equipment; technical conditions in CEPT ".

[5]	R4-166636: "ETSI standards / 3GPP specifications for Broadband PPDR ".

[6]	ECC Decision (16)02: "Harmonised technical conditions and frequency bands for the implementation of Broadband Public Protection and Disaster Relief (BB-PPDR) systems".

[7]	ECC Report 199: "User requirements and spectrum needs for future European broadband PPDR systems (Wide Area Networks)".

[8]	ECC Report 218: "Harmonised conditions and spectrum bands for the implementation of future European Broadband Public Protection and Disaster Relief (BB-PPDR) systems".

[9]	ECC Report 240: "Compatibility studies regarding Broadband PPDR and other radio applications in 410-430 MHz and 450-470 MHz and adjacent bands".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

PAMR	Public Access Mobile Radio

PMR	Private Mobile Radio

PPDR	Public Protection and Disaster Relief

# 4	Background

During its June 2016 meeting in Sweden, ECC Plenary approved ECC DEC(16)02 entitled “Harmonised technical conditions and frequency bands for the implementation of Broadband Public Protection and Disaster Relief (BB-PPDR) systems” (http://www.erodocdb.dk/Docs/doc98/official/pdf/ECCDEC1602.PDF).

In this decision, CEPT concluded on several harmonized frequency arrangements for BB-PPDR at 450MHz:

-	2x5,5MHz allocation: 450,5 – 456 MHz (UL) /460,5 – 466 MHz (DL)

-	2x5,5MHz allocation: 452 – 457,5 MHz (UL) /462 – 467,5 MHz (DL)

For those frequency arrangements, the UE unwanted emissions levels above 470 MHz shall be -42 dBm / 8MHz.

CEPT has sent a Liaison Statement [4] to 3GPP in which CEPT kindly requests 3GPP to consider the inclusion of the technical requirements as described above in the 3GPP specifications.

Input document [5] submitted at RAN4#80 meeting (August 2016) from European administrations, PPDR organisations, manufacturers and operators indicated their intention to support the standardization of  700 MHz and 450 MHz PPDR bands for Europe, including the frequency bands above.

Many administrations in Europe plan to deploy LTE based PPDR services in this harmonized frequency arrangement and therefore support the development of a new E-UTRA band that includes this spectrum and taking into account the above OOBE requirements.

In addition to future PPDR services there are a range of existing CDMA2000 networks working in the 451-456 / 461-466 MHz band in Europe. Operators in Austria, Czech Republic, Germany and the Netherlands among others are providing broadband or machine-type-communication services today and have the plan to deploy LTE Private Mobile Radio (PMR) / Public Access Mobile Radio (PAMR) services once this new E-UTRA band is introduced in 3GPP specifications.

# 5	Band plan allocation and regulatory background

## 5.1	Regulatory background

ECC CEPT initiated the work on future BB-PPDR by addressing  the  user  requirements  and  the  spectrum  needs  for  future  European  broadband  PPDR  systems  (Wide  Area  Networks). The outcome is the ECC Report 199 [7] which was approved in May 2013.

It was complemented by ECC report 218 [8] that addresses the possible harmonisation options which support the implementation of the user requirements and spectrum needs. It describes and evaluates proposed frequency arrangement options for BB-PPDR. The evaluation was done assuming LTE and its evolution as a common technical standard to support BB-PPDR.

Several candidate BB-PPDR frequency ranges for harmonisation were investigated in ECC report 218 [8]:

-	400 MHz (410-430 MHz and 450-470 MHz)

-	700 MHz (694-791 MHz)

Conclusions on spectrum compatibility of the options included in ECC Report 218 [8] are derived from ECC Reports 239 (for the 700MHz option) and 240 [9] (for the 400MHz option).

Based on the conclusions of ECC report 218 [8], ECC has approved for publication in June 2016 ECC DEC(16)02 [6] on “Harmonised technical conditions and frequency bands for the implementation of Broadband Public Protection and Disaster Relief (BB-PPDR) systems”.

CEPT ECC concluded, in particular, on the following for harmonisation:

1.	Frequency arrangements  for BB-PPDR:

a)	450.5 - 456.0 MHz (Uplink) / 460.5 - 466.0 MHz (Downlink);

b)	452.0 - 457.5 MHz (Uplink) / 462.0 - 467.5 MHz (Downlink);

c)	698 - 703 MHz (Uplink) / 753 - 758 MHz (Downlink);

d)	703 - 733 MHz (Uplink) / 758 - 788 MHz (Downlink);

e)	733 - 736 MHz (Uplink) 788 - 791 MHz (Downlink);

2.	700 MHz range:

The UE unwanted emissions levels below 694 MHz under normal environmental conditions  shall be -42dBm/8MHz  noting that a different level can be considered for such BB-PPDR UE operating in the 698-703 MHz block in extreme environmental conditions (i.e. outside normal environmental conditions) for equipment conformance tests but shall not exceed -30dBm/8MHz.

3.	400 MHz ranges:

The maximum UE unwanted emissions above 470MHz shall be -42dBm/8MHz.

The band 410-430 MHz which was covered in ECC Report 218 [8] is not covered by the ECC Decision (16)02 [6]. The band 410-430 MHz has been excluded due to:

-	the incompleteness of the technical studies in support of that band for BB-PPDR

-	a lack of corresponding standardisation activities for BB-PPDR in this band in 3GPP, ETSI and other international organisations

-	Spectrum sharing and coexistence difficulties identified in ECC Report 240 [9]

## 5.2	Frequency arrangement for the 400MHz option of the ECC Decision (16)02

This technical report addresses the inclusion of the technical requirements for the 400MHz option that is included in the ECC Decision (16)02 [6]:

a)	450.5 - 456.0 MHz (Uplink) / 460.5 - 466.0 MHz (Downlink);

b)	452.0 - 457.5 MHz (Uplink) / 462.0 - 467.5 MHz (Downlink);

These frequency allocations and the adjacent services are depicted on the Figure 5.2-1.

Figure 5.2-1: Frequency allocation for BB-PPDR in 450-470MHz band and adjacent services

Coexistence issues with adjacent services were studied. Methodology, results and conclusions of the coexistence studies can be found in ECC Report 240 [9]:

-	PPDR impact on existing systems within the 450-470 MHz sub-bands and adjacent bands

-	Impact on TETRA system

-	Impact on ETS300-113-related systems (e.g. TETRAPOL)

-	Impact on CDMA-PAMR

-	Impact on analogue FM PMR

-	Impact on DTT reception

-	Impact on satellite services

-	Impact on Military Radars

-	Impact on Radio Astronomy at 406.1 - 410 MHz

-	Impact of existing systems within the 450-470 MHz sub-bands and adjacent bands  on BB-PPDR  systems

-	TETRA impact on LTE 450 system

-	TETRAPOL impact on LTE 450 system

-	CDMA-PAMR impact on LTE 450 system

-	25kHz analogue FM impact on LTE 450 system

-	DTT impact on LTE450 system

Conclusions on spectrum compatibility of the option 450-470MHz listed above are summarised in section 9.2.1 of the ECC Report 218 [8]. It is stated that “This band is suitable for the deployment of BB-PPDR services” under “certain circumstances” such as RF performances and deployment guidelines. They are detailed in sections 8.1.1.1 to 8.1.1.5 of ECC Report 218 [8] and shall be taken into account while deriving the RF requirements applicable to UE and eNodeB that operate into a sub-band of 450-470MHz.

# 6	List of band specific issues

General issues

-	Frequency band arrangement

-	Coexistence with systems in adjacent spectrum

-	Coexistence with PMR/PAMR services (TETRAPOL, TETRA, CDMA-PAMR, Analogue FM…)

-	Coexistence with Digital Terrestrial Television

-	Coexistence with radioastronomy

-	Coexistence with radiolocation and military radars

-	Coexistence with satellite services

E-UTRA issues

-	UE duplexer

-	UE RF requirements

-	eNodeB RF requirements

# 7	General issues

## 7.1	Operating band and channel bandwidths

The initial study phase of the WI concentrated on agreeing on band(s) arrangement focusing on the following options:

i.	2x5MHz within 450,5 – 456 MHz (UL) / 460,5 – 466 MHz (DL) and 2x5MHz within 452 – 457,5 MHz (UL) / 462 – 467,5 MHz (DL)

ii.	2x7MHz: 450,5 – 457,5 (UL) / 460,5 – 467,5 (DL)

During the WI a duplex-filter study was done, see section 8.1 and the conclusion was that from filter performace point of view 2x5 MHz band plans are feasible but 2x5.5 MHz band plans are not suitable for full duplex operation.

Based on the 2x5 MHz bandwidth limitation and current spectrum holdings in Europe, see Table 7.1-1 it was concluded that exiting band 31 and a new band of uplink = 451 - 456 MHz and downlink = 461 – 466 MHz, see Figure 7.1-1 would have very good geographical coverage in Europe.

Figure 7.1-1

Table 7.1-1: Frequency arrangements in Europe

| Country | UL (MHz) |  | DL (MHz) |  | Block Size (MHz) | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Austria | 451,300 | 455,740 | 461,300 | 465,740 | 4,440 | New band |
| Germany | 451,000 | 455,740 | 461,000 | 465,740 | 4,740 | New band |
| Slovakia | 451,310 | 455,730 | 461,310 | 465,730 | 4,420 | New band |
| Czech Republic | 451,300 | 455,740 | 461,300 | 465,740 | 4,440 | New band |
| Netherlands | 451,768 | 454,768 | 461,768 | 464,768 | 3,000 | New band |
| Turkey | 450,000 | 457,500 | 460,000 | 465,500 | 7,500 | Frequency band not decided yet |
| Spain | 452,125 | 457,125 | 462,125 | 467,125 | 5,000 | Frequency band not decided yet |
| France | 452,500 | 455,500 | 462,500 | 465,500 | 3,000 | Band 31 filter |
| Poland | 452,525 | 457,000 | 462,525 | 467,000 | 4,475 | Band 31 filter |
| Hungary | 450,000 | 457,380 | 460,000 | 467,380 | 7,380 | LTE in band 31 deployed |
| Denmark | 453,000 | 457,450 | 463,000 | 467,450 | 4,450 | LTE in band 31 deployed |
| Sweden | 452,500 | 457,500 | 562,500 | 467,500 | 5,000 | LTE in band 31 deployed |
| Norway | 453,000 | 457,500 | 463,000 | 467,500 | 4,500 | LTE in band 31 deployed |
| Finland | 452,425 | 456,925 | 462,425 | 466,925 | 4,500 | LTE in band 31 deployed |
| Russia | 453,000 | 457,400 | 463,000 | 467,400 | 4,400 | LTE in band 31 deployed |
| Estonia | 453,000 | 457,475 | 463,000 | 457,475 | 4,475 | Band 31 filter |
| Latvia | 450,000 | 457,500 | 460,000 | 467,500 | 7,500 | LTE in band 31 deployed |

In a conclusion one new band was decided to be standardized are presented in Table 7.1-2 with channel bandwidths presented in Table 7.2-3.

Table 7.1-2 Band plan for 450 MHz band

| E-UTRA Operating Band |  | Uplink (UL) operating band BS receive UE transmit |  |  |  | Downlink (DL) operating band BS transmit UE receive |  |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |  |
| 72 | 451 MHz |  | – | 456 MHz | 461 MHz |  | – | 466 MHz | FDD |  |

Table 7.1-3: E-UTRA channel bandwidths for 450 MHz band

| E-UTRA band / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 72 | Yes | Yes | Yes |  |  |  |

- 7.2	Coexistence with systems in adjacent spectrum

Compatibility studies have been performed in the ECC Report 240 [9] on compatibility and sharing studies for the BB-PPDR systems operating in the frequency bands 410-430MHz and 450-470MHz. The following scenarios have been studied:

-	Section 3 of [9]: BB-PPDR impacts on existing systems within 410-430 / 450-470MHz sub-band and adjacent bands (scenarios where eNodeB and UE are aggressors)

-	Section 4 of [9]: Impacts of existing systems within 410-430 / 450-470MHz sub-band and adjacent bands on BB-PPDR (scenarios where eNodeB and UE are victims)

The conclusions of these studies can be found in section 8.1.1 of [8].

Conclusions applicable to this work item (BB-PPDR operating in 450-470MHz frequency band and UE output power and spectrum mask compliant with 3GPP specifications) are detailed in the following sub-sections.

- 7.2.1	Coexistence with PMR/PAMR

Conclusions of coexistence studies with PMR/PAMR can be found in section 8.1.1.1 of [8].

For the OOBE of the LTE eNodeB, a level of -43dBm/100kHz is recommended to protect the PMR/PAMR BS receiver.

Since the PMR/PAMR uplink band(s) are lower in frequency than the uplink of the band 72 defined in section 7.1, the duplexer of the eNodeB provide the necessary attenuation to meet the recommended level, as stated in section 8.1.1.1 of [8]. Thus there is no need for additional requirements to be included in the TS36.104 for coexistence with PMR/PAMR.

- 7.2.2	Coexistence with satellite services

Conclusions of coexistence studies with satellite services can be found in section 8.1.1.2 of [8]: no coexistence issues were identified.

- 7.2.3	Coexistence with the radiolocation service (including defence radar)

Conclusions of coexistence studies with radiolocation services and defence radar can be found in section 8.1.1.3 of [8]: no coexistence issues were identified when BB-PPDR is operating in the 450-470MHz range, as stated in the last sentence. Thus, no issues are anticipated for Band 72 defined in section 7.1.

- 7.2.4	Coexistence with Digital Terrestrial Television

Conclusions of coexistence studies with radiolocation services and defence radar can be found in section 8.1.1.4 of [8], where it is mentioned that the risk of the eNodeB to interfere the DTT receivers is reduced when a guard band up to 3MHz is applied between the BS transmission and the first channel used for DTT. In case of Band 72 defined in 7.2.1, the guard band is at least 3.5MHz between the higher edge of the downlink band and 470MHz.

Other engineering solutions (such as external filters in vulnerable DTT areas) may also be applied in a case-by-case basis to mitigate the interference.

- 7.2.5	Coexistence with radioastronomy

Conclusions of coexistence studies with radioastronomy can be found in section 8.1.1.5 of [8]. Coexistence issues shall be considered on a case-by-case basis. They can be solved by an additional filtering of the OOBE of the eNodeB depending on the geographical separation between the radioastronomy station and the eNodeB.

# 8	Study of E-UTRA specific issues

## 8.1	UE duplex-filter study

In the WID RP-162514 two options were listed for band plan.

1.	Initial study phase:

a.	Agree on band(s) arrangement focusing on the following options:

i.	2x5MHz within 450,5 – 456 MHz (UL) / 460,5 – 466 MHz (DL) and 2x5MHz within 452 – 457,5 MHz (UL) / 462 – 467,5 MHz (DL)

ii.	2x7MHz: 450,5 – 457,5 (UL) / 460,5 – 467,5 (DL)

Note:	Other options are not precluded considering the filter performance constrained by the small duplex gap of each option and capability to achieve OOBE requirement.

Duplex-filter data was collected for 2x5.5 MHz band plan to see if it would be feasible to create bands covering entire 450,5 – 456 MHz (UL) / 460,5 – 466 MHz (DL) and 452 – 457,5 MHz (UL) / 462 – 467,5 MHz (DL) ranges instead of only 2x5 MHz band plans.

Filter data based on simulations from two vendors were received and is presented in Table 8.1-1. Data in the Table 8.1-1 is worst case data including temperature and manufacturing variations. Filter performance is quite similar when comparing the data from the two vendors which increases the confidence for the accuracy of the data. The achievable isolation for 2x5.5 MHz band plans seems not to be good enough to support full duplex operation as the own transmitter would interfere reception. Typically isolation values of 50 – 55 dB are expected from the duplex-filters.

Table 8.1-1: Filter data for 2x5.5 MHz band plan.

| Parameter | Vendor A | Vendor B |
| --- | --- | --- |
| Tx IL | 4.0 dB (max) | 4.7 dB (max) |
| Rx IL | 5.4 dB (max) | 3.6 dB (max) |
| Tx Iso | 45 dB (min) | 42 dB (min) |
| Rx Iso | 36 dB (min) | 38 dB (min) |

In addition to data presented in Table 8.1-1 band 31 duplex-filter data was received and presented in Table 8.1-2. Band 31 frequency range is 452.5 – 457,5 MHz (UL) / 462.5 – 467,5 MHz (DL) and aligns with one of the target 5 MHz band plans listed in WID objectives. It can be assumed that shifting the filter downwards to cover the other 5 MHz band plan mentioned in the WID do not impact the filter parameters shown in Table 8.1-2.

When comparing the data in Tables 8.1-1 and 8.1-2 it can be seen that isolation values of 2x5 MHz filter compared to 2x5.5 MHz filter are much higher and in a same time Tx and Rx insertion losses of 2x5 MHz filter are smaller. These differences are of course beneficial for UE operation.

Table 8.1-2: Filter data for Band 31

| Parameter | Vendor B |
| --- | --- |
| Tx IL | 4.2 dB (max) |
| Rx IL | 3.1 dB (max) |
| Tx Iso | 50 dB (min) |
| Rx Iso | 50 dB (min) |

From filter performance point of view presented in Tables 8.1-1 and 8.1-2 it can be concluded that 2x5 MHz band plans are feasible but 2x5.5 MHz band plans are not suitable for full duplex operation.

8.2	UE transmitter requirements

Due to very similar band arrangement with band 31 the transmitter requirements will be very similar for band 72 as those are for band 31.

### 8.2.1	Maximum output power:

Standard LTE Transmitter output power class will be specified as power class 3 with ± 2 dB tolerances.

UE category M1 Transmitter output power classes will be specified as power classes 3 and 5 with ± 2 dB tolerances.

UE category NB1/NB2 Transmitter output power classes will be specified as power classes 3 and 5 with ± 2 dB tolerances and power class 6 with ± 2.5 dB tolerances.

### 8.2.2	Spurious emission band UE co-existence:

Spurious emissions towards other 3GPP bands were already agreed in R4-1704068 in RAN4#82bis and are captured below. For UE category M1 and NB1/NB2 standard LTE requirements apply.

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 1 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 65, 67, 68, 69, [72] | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 15, 27 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 15, 26, 27 |
| 2 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 3 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 41, 43, 44, 45, 65, 67, 68, 69, [72] | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | E-UTRA Band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |
| 4 | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 43, 48, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 65, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 39 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 39 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 39 |
| 6 | E-UTRA Band 1, 9, 11, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 875 | -37 | 1 |  |
|  | Frequency range | 875 | - | 895 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 | 7 |
|  |  | 1884.5 | - | 1915.7 |  |  | 8 |
| 7 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 65, 66, 67, 68, [72] | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 15, 21 |
| 8 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 65, 67, 68, 69, [72] | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 23 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 15, 23 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 23 |
| 9 | E-UTRA Band 1, 11, 18, 19, 21, 26, 28, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 10 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 43, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 11 | E-UTRA Band 1, 11, 18, 19, 21, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 12 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 48, | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 13 | E-UTRA Band 2, 4, 5, 10, 12, 13, 17, 25, 26, 27, 29, 41, 48, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 24, 30 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 11, 15 |
| 14 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 23, 24, 25, 26, 27, 29, 30, 41, 48, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 12, 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 11, 12, 15 |
| 17 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 48 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 18 | E-UTRA Band 1, 11, 21, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 19 | E-UTRA Band 1, 11, 21, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 20 | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 65, 67, [72] | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 38, 42, 69 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| 21 | E-UTRA Band 1, 18, 19, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 22 | E-UTRA Band 1, 3, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 43, 65, 67, 69, [72] | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 3510 | - | 3525 | -40 | 1 | 15 |
|  | Frequency range | 3525 | - | 3590 | -50 | 1 |  |
| 23 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 23, 24, 26, 27, 29, 30, 41, 66 | FDL_low | - | FDL_high | -50 | 1 |  |
| 24 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
| 25 | E-UTRA Band 4, 5, 10,12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 26 | E-UTRA Band 1, 2, 3, 4, 5, 10, 11, 12, 13, 14, 17, 18,19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 40, 42, 43, 48, 65, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 27 | E-UTRA Band 1, 2, 3, 4, 5, 7, 10, 12, 13, 14, 17, 25, 26, 27, 29, 30, 31, 38, 40, 41, 42, 43, 65, 66 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 28 | FDL_low | - | 790 | -50 | 1 |  |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 |  |
| 28 | E-UTRA Band 1, 4, 10, 22, 42, 43, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 40, 41, 66, [72] | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |
| 30 | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 48, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
| 31 | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 38, 40, 42, 43, 65, 67, 69 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band [72] | FDL_low | - | FDL_high | TBD | 1 |  |
| … |  |  |  |  |  |  |  |
| 33 | E-UTRA Band 1, 7, 8, 20, 22, 28, 31, 32, 34, 38, 40, 42, 43, 65, 67, 69, [72] | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 34 | E-UTRA Band 1, 3, 7, 8, 11, 18, 19, 20, 21, 22, 26, 28, 31, 32, 33, 38,39, 40, 41, 42, 43, 44, 45, 65, 67, 69, [72] | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 35 |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |
| 37 |  |  | - |  |  |  |  |
| 38 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13, 14, 17, 20, 22, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 65, 66, 67, 68, [72] | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 15, 22, 26 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 15, 22 |
| 39 | E-UTRA Band 1, 8, 22, 26, 34, 40, 41, 42, 44, 45 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 33 |
|  | Frequency range | 1855 |  | 1880 | -15.5 | 5 | 15,26,33 |
| 40 | E-UTRA Band 1, 3, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 42, 43, 44, 45, 65, 67, 68, 69, [72] | FDL_low | - | FDL_high | -50 | 1 |  |
| 41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 45, 48, 65, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 30 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8, 30 |
| 42 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 11, 18, 19, 20, 21, 25, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 45, 65, 66, 67, 69, [72] | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 43 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 20, 25, 26, 27, 28, 31,32, 33, 34, 38, 40, 65, 66, 67, 69, [72] | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22 | FDL_low | - | FDL_high | [-50] | [1] | 3 |
| 44 | E-UTRA Band 1, 40, 42, 45 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 5, 8, 34, 39, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
| 45 | E-UTRA Band 1, 3, 5, 8, 34, 39, 40, 41, 42.44 | FDL_low | - | FDL_high | -50 | 1 |  |
| … |  |  |  |  |  |  |  |
| 47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, [72] | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 38 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 38 |
|  | Frequency range | 5795 | - | 5815 | [-65] | 1 | 38 |
| 48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
| 65 | E-UTRA Band 1, 7, 8, 20, 22, 28, 31, 32, 38, 40, 42, 43, 65, 69, [72] | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 5, 11, 18, 19, 21, 26, 27, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 36 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 37 |
|  | Frequency range | 1900 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |
| 66 | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 68 | E-UTRA Band 3, 7, 8, 28, 38, 40, [72] | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| … |  |  |  |  |  |  |  |
| 70 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| … |  |  |  |  |  |  |  |
| [72] | E-UTRA Band 1, 7, 20, 22, 28, 31, 32, 33, 34, 38, 42, 43, 47, 65, 68, [72] | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 8, 40 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |

8.3	UE receiver requirements

Due to very similar band arrangement with band 31 the receiver requirements will be very similar for band 72 as those are for band 31.

### 8.3.1	REFSENS

Standard LTE REFSENS will be specified to be the same as band 31 i.e.

Table 7.3.1-1: Reference sensitivity QPSK PREFSENS

| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 72 | -99.0 | -95.7 | -93.5 |  |  |  | FDD |

Table 7.3.1-2: Uplink configuration for reference sensitivity

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| 72 | 6 | 54 | 54 |  |  |  | FDD |
| NOTE 4: 4 refers to Band 31; in the case of 3 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 9 and in the case of 5 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 10. |  |  |  |  |  |  |  |

UE category M1 REFSENS will be specified to be the same as band 31 i.e.

Table 7.3.1E-3:  Reference sensitivity for FDD and TDD UE category M1 QPSK PREFSENS

| E-UTRA Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 72 | -96.5 | FDD |

Table 7.3.1E-4: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| E-UTRA Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 72 | -97.3 | HD-FDD |

Table 7.3.1E-5: FDD and TDD UE category M1 Uplink configuration for reference sensitivity

| E-UTRA Band | NRB | Duplex Mode |
| --- | --- | --- |
| 72 | 61 | FDD and HD-FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). |  |  |

UE category NB1/NB2 REFSENS will be specified to be the same as band 31 i.e.

Table 7.3.1F.1-1: Reference sensitivity for UE category NB1 and NB2

| Operating band | REFSENS [dBm] |
| --- | --- |
| 1, 2, 3, 5, 8, 11, 12, 13, 17, 18, 19, 20, 25, 26, 28, 31, 66, 70, 72 | - 108.2 |

### 8.3.2	Inband blocking

Standard LTE inband blocking: Band 72 is added into Table 7.6.1.1-2

### 8.3.3	Out of band blocking:

Standard LTE Out of band blocking: Band 72 is added into Table 7.6.2.1-2

UE category NB1/NB2: Note 3 is modified in Table 7.6.2.1F-1 to take into account that lower frequency limit of band 72 downlink is 461 MHz.

NOTE 3:	For operating bands which downlink band frequency range is between 461 MHz < f < 467.5 MHz the power level of the interferer (PInterferer) for Range 3 shall be modified to: – 18 dBm for the frequency range which is bounded by FDL_low  - 150 MHz of the lowest band that UE supports in frequency range 461 MHz < f < 467.5 0 MHz and FDL_high  + 150 MHz of the lowest band that UE supports supports in frequency range 461 MHz < f < 467.5.

## 8.4	BS aspect issues

The following BS specific 36.104 changes are expected due to introduction of the new FDD 450 MHz band [Band 72] in Europe:

6.6.3	Operating band unwanted emissions

6.6.3.1	Minimum requirements for Wide Area BS (Category A)

For E-UTRA BS operating in Bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 26, 27, 28, 29, 31, 44, 68, [72] emissions shall not exceed the maximum levels specified in Tables 6.6.3.1-1 to 6.6.3.1-3.

6.6.3.2	Minimum requirements for Wide Area BS (Category B)

6.6.3.2.1	Category B requirements (Option 1)

For E-UTRA BS operating in Bands 5, 8, 12, 13, 14, 17, 20, 26, 27, 28, 29, 31, 44, 68, 67, [72] emissions shall not exceed the maximum levels specified in Tables 6.6.3.2.1-1 to 6.6.3.2.1-3:

6.6.4.3	Additional spurious emissions requirements

6.6.4.3.1	Minimum Requirement

Table 6.6.4.3.1-1: BS Spurious emissions limits for E-UTRA BS for co-existence with systems operating in other frequency bands

| System type for E-UTRA to co-exist with | Frequency range for co-existence requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| E-UTRA Band 31 | 462.5 - 467.5 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 31 and [72]. |
|  | 452.5 - 457.5 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 31, since it is already covered by the requirement in subclause 6.6.4.2. This requirement does not apply to E-UTRA BS operating in band [72]. |
| E-UTRA Band [72] | 461 - 466 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 31 and [72]. |
|  | 451 - 456 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band [72], since it is already covered by the requirement in sub-clause 6.6.4.2. |

6.6.4.4	Co-location with other base stations

6.6.4.4.1	Minimum Requirement

Table 6.6.4.4.1-1: BS Spurious emissions limits for Wide Area BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| WA E-UTRA Band [72] | 451 - 456 MHz | -96 dBm | 100 kHz |  |

Table 6.6.4.4.1-2: BS Spurious emissions limits for Local Area BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| LA E-UTRA Band [72] | 451 - 456 MHz | -88 dBm | 100 kHz |  |

Table 6.6.4.4.1-3: BS Spurious emissions limits for Medium range BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| MR E-UTRA Band [72] | 451 - 456 MHz | -91 dBm | 100 kHz |  |

7.6.1	General blocking requirement

7.6.1.1	Minimum requirement

Table 7.6.1.1-1: Blocking performance requirement for Wide Area BS for E-UTRA

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 31, [72] | (FUL_low -20) | to | (FUL_high +5) | -43 | PREFSENS +6dB* | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB* |  | CW carrier |

Table 7.6.1.1-1a: Blocking performance requirement for Local Area BS for E-UTRA

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 31, [72] | (FUL_low -20) | to | (FUL_high +5) | -35 | PREFSENS +6dB* | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB* |  | CW carrier |

Table 7.6.1.1-1c: Blocking performance requirement for Medium Range BS for E-UTRA

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset to the lower/higher Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 31, [72] | (FUL_low -20) | to | (FUL_high +5) | -38 | PREFSENS +6dB* | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB* |  | CW carrier |

7.6.2	Co-location with other base stations

7.6.2.1	Minimum requirement

Table 7.6.2.1-1: Blocking performance requirement for E-UTRA and NB-IoT Wide Area BS when co-located with BS in other frequency bands.

| Co-located BS type | Centre Frequency of Interfering Signal (MHz) | Interfering Signal mean power (dBm) | Wanted Signal mean power (dBm) | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| WA E-UTRA Band [72] | 461 – 466 | +16** | PREFSENS + 6dB* | CW carrier |

Table 7.6.2.1-2: Blocking performance requirement for Local Area BS when co-located with BS in other frequency bands.

| Co-located BS type | Centre Frequency of Interfering Signal (MHz) | Interfering Signal mean power (dBm) | Wanted Signal mean power (dBm) | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| LA E-UTRA Band [72] | 461 – 466 | -6 | PREFSENS + 6dB* | CW carrier |

Table 7.6.2.1-3: Blocking performance requirement for E-UTRA Medium Range BS when co-located with BS in other frequency bands.

| Co-located BS type | Centre Frequency of Interfering Signal (MHz) | Interfering Signal mean power (dBm) | Wanted Signal mean power (dBm) | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| MR E-UTRA Band [72] | 461 – 466 | +8 | PREFSENS + 6dB* | CW carrier |

# 9	Study of MSR specific issues

There are no specific MSR issues for the new 450 MHz band.

# 10	Channel numbering for E-UTRA, MSR

The E-UTRA Absolute Radio Frequency Channel Numbers (EARFCNs) allocated for 450MHz FDD band in Europe are outlined in Table 10-1.

Table 10-1: E-UTRA channel numbers

| E-UTRA Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low [MHz] | NOffs-DL | Range of NDL | FUL_low [MHz] | NOffs-UL | Range of NUL |
| [72] | 461 | 68936 | 68936-68985 | 451 | 133472 | 133472-133521 |

# 11	Required changes to E-UTRA and MSR specifications

The required changes to the 3GPP specifications for the new band are summarised in a Table 11-1.

Table 11-1: Overview of 3GPP specifications with required changes

| 3GPP specification | Clause in TR 30.007 where the required changes are given | Clause in the present document identifying additional changes |
| --- | --- | --- |
| TS 36.101 | 8.2.1.1 |  |
| TS 36.104 | 8.2.1.2 |  |
| TS 36.113 | 8.2.1.4 |  |
| TS 36.124 | 8.2.1.5 |  |
| TS 36.133 | 8.2.1.6 |  |
| TS 36.141 | 8.2.1.7 |  |
| TS 25.101 | 8.2.2.1 |  |
| TS 25.104 | 8.2.2.3 |  |
| TS 25.123 | 8.2.2.7 |  |
| TS 25.133 | 8.2.2.9 |  |
| TS 25.141 | 8.2.2.10 |  |
| TS 25.461 | 8.2.2.15 |  |
| TS 25.466 | 8.2.2.16 |  |
| TS 37.104 | 8.2.3.1 |  |
| TS 37.113 | 8.2.2.2 |  |
| TS 37.141 | 8.2.2.3 |  |

## 11.1	Changes to TS 36.101

Following changes are needed into TS 36.101 for adding band 72 into 36.101 for standard E-UTRA, MTC or NB-IoT operation.

| Clause | Title | Expected changes |
| --- | --- | --- |
| 5.5 | Operating bands | Band number 72 is adde into the Table 5.5-1 with uplink frequency range of 451-456 MHz and downlink frequency range of 461 – 466 MHz |
| 5.5E | Operating bands for UE category 0, UE category M1 and UE category 1bis | Add band 72 for UE category M1 |
| 5.5F | Operating bands for category NB1 and NB2 | Add band 72 for UE category NB1 |
| 5.6.1 | Channel bandwidths per operating band | 1.4, 3 and 5 MHz channel bandwidths are listed in Table 5.6.1-1 for the band 72 |
| 5.7.1 | Channel spacing | No change |
| 5.7.2 | Channel raster |  |
| 5.7.3 | Carrier frequency and EARFCN | Add band 72 into Table 5.7.3-1: E-UTRA channel numbers |
| 5.7.4 | TX–RX frequency separation | 10 MHz is specified as Tx-Rx separation in Table 5.7.4-1 |
| 6.2.2 | UE maximum output power | Power Class 3 is assigned for the band 72 |
| 6.2.2E | UE maximum output power for Category M1 UE | Power Class 3 and 5 are assigned for the band 72 |
| 6.2.2F | UE maximum output power for category NB1 and NB2 | Power Class 3, 5 and 6 are assigned for the band 72 |
| 6.2.3 | UE Maximum Output power for modulation / channel bandwidth | No change |
| 6.2.4 | UE maximum output power with additional requirements |  |
| 6.2.5 | Configured transmitted power |  |
| 6.3.2 | UE Minimum output power |  |
| 6.3.3A | UE Transmit OFF power |  |
| 6.3.4A | ON/OFF time mask |  |
| 6.3.5A | Power control |  |
| 6.5.1A | Frequency error |  |
| 6.5.2A | Transmit modulation quality |  |
| 6.6.1A | Occupied bandwidth |  |
| 6.6.2.1 | Spectrum emission mask |  |
| 6.6.2.2 | Additional Spectrum Emission Mask |  |
| 6.6.2.3.1 | (ACLR) Minimum requirement E-UTRA |  |
| 6.6.2.3.2 | (ACLR) Minimum requirement UTRA |  |
| 6.6.3.1 | (Spurious emissions) Minimum requirements |  |
| 6.6.3.2 | Spurious emission band UE co-existence | Requirement for protection against other European bands and from other European bands is added in to Table 6.6.3.2-1. DTT band protection is added. |
| 6.6.3.3 | Additional spurious emissions | No change |
| 6.7.1 | (Transmit intermodulation) Minimum requirement |  |
| 7.3.1 | (REFSENS) Minimum requirements (QPSK) | REFSENS for band 72 is added into Table 7.3.1-1 |
| 7.3.1E | Minimum requirements (QPSK) for UE category 0, M1, and 1bis | Band 72 is added into Table 7.3.1E-3, Table 7.3.1E-4 and Table 7.3.1E-5 |
| 7.3.1F | Minimum requirements for UE category NB1 and NB2 | Band 72 is added into Table 7.3.1F.1-1 |
| 7.4.1 | (Maximum input level) Minimum requirements | No change |
| 7.5.1 | (ACS) Minimum requirements |  |
| 7.6.1.1 | (IBB) Minimum requirements | New band is added into Table 7.6.1.1-2 |
| 7.6.2.1 | (OBB) Minimum requirements | New band is added into Table 7.6.2.1-2 |
| 7.6.2.1F | (OBB) Minimum requirements for category NB1 and NB2 | Note 3 is modified in Table 7.6.2.1F-1 |
| 7.6.3.1 | (NBB) Minimum requirements | No change |
| 7.7.1 | (Spurious response) Minimum requirements for CA |  |
| 7.8.1 | (Wideband intermodulation) Minimum requirements for CA |  |
| 7.9.1 | (Spurious emissions) Minimum requirements |  |

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-02 | RAN4#82 | R4-1700433 |  |  |  | Report skeleton | 0.1.0 |
| 2017-04 | RAN4#82bis | R4-1704069 |  |  |  | R4-1703581 Text Proposal for section 5 of TR36.748, Airbus DS SLC, Nokia R4-1703360 TP to TR 36.748: 450 MHz PPDR band frequency arrangement, Nokia, Airbus DS SLC R4-1703867 TP to 36.748: Channel numbering, Nokia R4-1703361 TP to TR 36.748: 450 MHz PPDR Band filter study, Nokia R4-1703866 TP to 36.748: BS aspect issues, Nokia R4-1703868 TP to 36.748: MSR specific issues, Nokia | 0.2.0 |
| 2017-05 | RAN4#83 | R4-1705505 |  |  |  | R4-1705504 Text Proposal for section 6 of TR36.748, Airbus DS SLC R4-1705452 TP to TR 36.748: Band 72 transmitter requirements, Nokia R4-1705882 TP to TR 36.748: Expected specification changes to TS 36.101 due to introduction of Band 72, Nokia R4-1705883 TP to TR 36.748: Band 72 receiver requirements, Nokia R4-1705884 Text Proposal for Section 7.2 of TR36.748, Airbus DS SLC | 0.3.0 |
| 2017-06 | RP-76 | RP-170912 |  |  |  | Submitted to RAN#76 for information | 1.0.0 |
| 2017-09 | RP-77 | RP-171709 |  |  |  | Submitted to RAN#77 for Approval | 2.0.0 |
| 2017-09 | RP-77 | RP-171709 |  |  |  | TR Approved | 15.0.0 |