3GPP TR 36.755 V15.0.0 (2017-09)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

US 600 MHz Band for LTE;

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<LTE, Radio>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	6

3.3	Abbreviations	6

4	Background	7

5	Frequency band arrangements and regulatory background	7

5.1	Operating band and channel bandwidth	7

5.2	Adjacent 3GPP bands	8

5.3	Nearby non-3GPP Services	9

5.3.1 	Introduction	9

5.3.2 	Coexistence between 600 MHz and TV Services	9

5.3.2 	Coexistence between 600 MHz and Channel 37 WMTS and RAS Services	10

5.3.2.1 	Coexistence with WMTS	10

5.3.2.2 	Coexistence with RAS	10

5.3.3 	Coexistence between 600 MHz and Unlicensed White Space Devices & Wireless Microphones	10

5.4	FCC regulation requirement	11

6	List of band specific issues for US 600 MHz frequency band	12

7	Study of E-UTRA specific issues	12

7.1	UE aspect issues	12

7.1.1 	UE transmitter requirements	12

7.1.1.1	UE co-existence	12

7.1.1.2	UE TX maximum output power	12

7.1.1.3 	NS value and Spectrum Emission Mask	13

7.1.2	UE receiver requirements	14

7.1.2.1	UE REFSENS	14

7.1.2.2	In-band blocking	15

7.1.2.3 	Out-of-band blocking	17

7.2	BS aspect issues	18

8	Study of MSR specific issues	22

9	Channel numbering for E-UTRA, MSR	22

10	Required changes to E-UTRA and MSR specifications	22

Annex A:	Change history	24

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is a technical report for the work item of US 600 MHz LTE FDD band.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-170740, New WID US 600 MHz Band for LTE.

[3]	Expanding the Economic and Innovation Opportunities of Spectrum Through Incentive Auctions, GN Docket No. 12-268, Report and Order, FCC 14-50.

[4]	PROCEDURES FOR COMPETITIVE BIDDING IN AUCTION 1000, GN Docket No. 12-268, PUBLIC NOTICE, FCC 15-78.

[5]	"Clearing Target of 84 MHz set for Stage 4 of the Broadcast Television Spectrum Incentive Auction", FCC Public Notice, DA16-1354.

[6]	"Experimental Radio, Auxiliary, Special Broadcast and Other Program Distributional Services", 47 CFR Part 74.

[7]	"Radio Broadcast Services", 47 CFR Part 73.

[8]	Miscellaneous Wireless Communications Services, Title 47, Part 27 of eCFR https://www.law.cornell.edu/cfr/text/47/part-27.

[9]	ETSI EN 300 422-1 (V1.4.2) (2011-08): "Electromagnetic compatibility and Radio spectrum Matters (ERM); Wireless microphones in the 25 MHz to 3 GHz frequency range; Part 1: Technical characteristics and methods of measurement".

[10]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[11]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

BW	Bandwidth

f	Separation between the channel edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency

fmax 	The largest value of f used for defining the requirement

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

f_offset 	Separation between the channel edge frequency and the centre of the measuring filter

f_offsetmax 	The maximum value of f_offset used for defining the requirement

NDL 	Downlink EARFCN

NOffs-DL 	Offset used for calculating downlink EARFCN

NOffs-UL 	Offset used for calculating uplink EARFCN

NUL 	Uplink EARFCN

PREFSENS	Reference Sensitivity power level

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

BS	Base Station

CBRS	Citizens Broadband Radio Service

CIM	Counter-InterModulation

CW	Continuous Wave

DL	Downlink

DTV	Digital Television

EARFCN 	E-UTRA Absolute Radio Frequency Channel Number

eCFR	electronic Code of Federal Regulation

EIRP	Effective Isotropic Radiated Power

ERP 	Effective Radiated Power

ETSI	European Telecommunications Standards Institute

E-UTRA	Evolved UTRA

FCC	Federal Communications Commission

FDD	Frequency Division Duplex

GEHC	GE Healthcare

HAAT	Height Above Average Terrain

LA	Local Area

LO 	Local Oscillator

MOP	Maximum Output Power

MR	Medium Range

MSR	Multi-Standard Radio

NF	Noise Figure

NRAO	National Radio Astronomy Observatory

NSF	National Science Foundation

PA 	Power Amplifier

PRB	Physical Resource Block

RAS	Radio Astronomy Service

REFSENS	Reference Sensitivity

RF	Radio Frequency

RX	Receiver

SEM	Spectrum Emission Mask

TX	Transmitter

UE	User Equipment

UHF	Ultra High Frequency

UL	Uplink

UMTS	Universal Mobile Telecommunications System

UTRA	UMTS Terrestrial Radio Access

VLBA	Very Long Baseline Array

WA	Wide Area

WMTS	Wireless Medical Telemetry Service

# 4	Background

The FCC has adopted rules for the Broadcast Incentive Auction [3]. The "clock phase" of the auction recently closed at stage 4, which aims to clear 84 MHz for mobile broadband services. This 84 MHz band plan [4] features a 2x35 MHz FDD spectrum with the uplink band (mobile transmit) in the upper 35 MHz from 663 to 698 MHz and downlink band (BS transmit) in the lower 35 MHz from 617 to 652 MHz; it also features an 11 MHz duplex gap and a 3 MHz guard band above Channel 37. The band plan is shown in Figure 4-1. With the "clock phase" now concluded, the 84 MHz band plan will be the final FCC band plan.

To facilitate the US 600 MHz eco-system development in a timely manner, it is proposed to standardize a new E-UTRA operating band based on the FCC 84 MHz band plan [2].

Figure 4-1: US 600 MHz Band Plan Scenarios

# 5	Frequency band arrangements and regulatory background

## 5.1	Operating band and channel bandwidth

The band plan for Band 71 is shown below. The Tx-Rx is "reverse-duplex"; in other words, the downlink frequency band is below the duplex gap while the uplink frequency band is above the duplex gap. This arrangement is opposite to conventional notation; however, for this band, it provides the benefit of aligning the uplink band adjacent to that of 700 MHz bands thereby minimizing interference conditions at the 698 MHz boundary.

Table 5.1-1: E-UTRA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

Supported bandwidths are shown in Table 5.1-2.

Figure 5.1-1: Band 71 spectrum and possible full or split duplexer architectures

Figure 5.1-1 above provides a view of both full band duplexer and a 15 MHz overlap split duplexer approach. Given that frequency blocks A to G are allocated on a 5 MHz grid both approaches support all possible 20 MHz channels.

Band 28 already supports single and split duplexer for the same reason as seen for Band 71. This was captured in the specification by adding a note to the 20 MHz channel bandwidth support specifying the possible allocated carriers within two frequency ranges. In order to apply the same approach for Band 71 a note is introduced stating that for the 20 MHz bandwidth, the minimum requirements are specified for E-UTRA UL carrier frequencies confined to either 673-678 MHz or 683-688 MHz. The proposed changes to Table 5.6.1-1 in TS 36.101 [10] are highlighted in the extract of the table below.

Table 5.1-2: E-UTRA channel bandwidth

| E-UTRA band / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 71 |  |  | Yes | Yes | Yes | Yes1 |
| NOTE: 1For the 20MHz bandwidth, the minimum requirements are specified for E-UTRA UL carrier frequencies confined to either 673-678 MHz or 683-688 MHz. |  |  |  |  |  |  |

Tx-Rx separation is shown in Table 5.1-3.

Table 5.1-3: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 71 | -46 MHz |

## 5.2	Adjacent 3GPP bands

The nearby 3GPP bands are summarized as figure 5.2-1. The Band 12, Band 17, Band 13, Band 14, Band 28 and Band 29 are Region 2 bands, which should be considered especially in co-existence study of US 600 MHz band. However, Band 44, which is not utilized in Region 2, could be not taken into account in study of US 600 MHz band.

Figure 5.2-1: Illustration on nearby 3GPP Bands

## 5.3	Nearby non-3GPP Services

### 5.3.1 	Introduction

Existing primary operations near the 600 MHz Band include Television services and Channel 37 Wireless Medical Telemetry Service (WMTS) and Radio Astronomy Service (RAS), which are entitled to interference protection from 600 MHz broadband wireless services. While, unlicensed white space device and wireless microphone operations are allowed in the duplex gap, guard band and service band where 600 MHz licensees have not commenced operations; they are subjected to FCC rules to protect 600 MHz services from harmful interference.

Coexistence issues and possible impacts on 600 MHz band RF requirements are analysed below.

### 5.3.2 	Coexistence between 600 MHz and TV Services

84 MHz band plan consists of seven 100% unimpaired blocks in all 416 Partial Economic Areas [5], meaning that Broadcast TV services are present only below channel 37. Such services include licensee of full power TV stations and low-power TV stations that have been accorded primary status as a Class A TV licensee under the section 73.6001(a) of electronic Code of Federal Regulation (eCFR).

Low power DTV stations are allowed to operate at a maximum ERP of 15 kW in UHF channels; while the power level of emissions on frequencies outside the authorized channel of operation must be attenuated below the average transmitted power within the authorized 6 MHz channel no less than what specified in section 74.794 of eCFR [6].

Full power DTV stations are allowed to operate in UHF channels at a maximum ERP of 1000 kW if their antenna height above average terrain (HAAT) is at or below 365 meters; and with lower maximum ERP levels as specified in section 73.622 (f) of eCFR at higher HAAT levels. The power level of emissions on frequencies outside the authorized channel of operation must be attenuated no less than 47 dB in the first 500 kHz from the channel edge, no less than 110 dB at more than 6 MHz from the channel edge; and no less than −11.5(ΔF 3.6) dB (where ΔF is the frequency difference in MHz from the edge of the channel) at any frequency between 0.5 and 6 MHz from the channel edge [7]. This attenuation is based on a measurement bandwidth of 500 kHz.

Broadcast TV services and Band 71 Downlink are separated by 9 MHz guard band (including channel 37), which is 2 MHz larger than the minimum frequency separation required by FCC to protect against a TV transmitter interfering with a mobile broadband UE receiver and an BS transmitter interfering with a TV receiver [3]. Being such protection guard band evaluated considering the emission limit specified by FCC in [8], no additional emission limits to protect TV services are considered necessary to be included in RF requirements for Band 71.

On the other hand, taking into account that the highest likely television signal that a wireless device may experience is -15dBm (before antenna gain is applied) as estimated in [3], the presence of broadcast TV operations in the close channel 36 requires to consider such RX power level in the definition of UE in band blocking requirement.

### 5.3.2 	Coexistence between 600 MHz and Channel 37 WMTS and RAS Services

#### 5.3.2.1 	Coexistence with WMTS

Wireless Medical Telemetry Service (WMTS) is used for remote monitoring of patients'vital signs and other important health parameters. WMTS devices, regulated by subpart H of 47 CFR Part 95, are authorized to operate anywhere within a health care facility.

Both narrow band and broadband (i.e. spread spectrum) technologies are allowed. If spread spectrum is used, 4 non-overlapping 1.5 MHz channels are defined, and WMTS devices shall be capable to operate within one or more of such 1.5 MHz channels.

WMTS is widely deployed in Channel 37 (more than 100K WMTS devices deployed, and WMTS infrastructure installed in 1300 hospitals - according to America Society of Health care engineering estimation at the end of 2012) Transmitters are generally frequency division multiplexed with typical occupied bandwidth of 10 kHz and employ relatively small TX power to permit several days of continuous data transmission.

For channel 37 operations, WMTS devices (patient units) are generally TX only and the DASs (routing data to central monitoring stations) are RX only.

The maximum field strength authorized for WMTS stations in the 608-614 MHz band is 200 mV/m (10.8 dBm/120 kHz EIRP), as measured at a distance of 3 meters, using measuring instrumentation with a CISPR quasi-peak detector; while the out-of-band emissions limit below 960 MHz is 200 V/m (-49.2 dBm/120 kHz).The worst case WMTS emission at the FCC limit (200mV/m at 3 meter separation) would result in UE received power of -32.9 dBm for a single narrowband WMTS interferer (assuming -6 dBi UE antenna). There are no plans to standardize UE requirements for blocking protection against WMTS transmitters.

With reference to wireless broadband BSs operations that pose a harmful interference risk if placed adjacent to channel 37 in locations near WMTS sites, to meet the GEHC protection criteria limiting the field strength, as measured at the perimeter of a health care facility at the edge of channel 37, to 20mV/m/MHz to protect WMTS from overload interference, and to 10μV/m/100 kHz (in channel 37) to provide WMTS protection from out-of-band interference, FCC specified 3 MHz guard band between 600 MHz DL and channel 37, and calculated example separation distances to protect WMTS from overload and OOB interference (up to 96 meters and 245 meters respectively). No additional emission limits to protect WMTS services are considered necessary to be included in BS RF requirements for Band 71.

#### 5.3.2.2 	Coexistence with RAS

Radio Astronomy Service (RAS) is a receive-only service that uses highly sensitive receivers to examine and study radio waves of cosmic origin. There are twelve RAS telescopes that have been using channel 37 or plan to use it in the near future. Of these, ten comprise the National Radio Astronomy Observatory's (NRAO's) Very Long Baseline Array (VLBA), which are distributed in several locations in the United States and its territories. The remaining two telescopes are characterized as single dish instruments (Green Bank - WV and Arecibo - PR observatories).

To protect RAS from harmful interference, FCC requires 600 MHz wireless BSs located within 25 kilometres of VLBA observatories to coordinate with the National Science Foundation (NSF) prior to commencing operation; while those located close to dish radio astronomy installations are required to notify technical details of the proposed operations according to section 1.924 of Part 47 CFR.

No additional emission limits need to be included in BS RF requirements for the 600 MHz band.

### 5.3.3 	Coexistence between 600 MHz and Unlicensed White Space Devices & Wireless Microphones

Unlicensed wireless microphones and white space devices operations in 600 MHz band are regulated by Part 15 of Title 47 of eCFR [8], which limits power, and specifies frequency and distance separations to protect authorized services, as reported below.

White space devices, subject to the interference protection requirements in paragraph 15.711 and 15.712 and to emission limits in paragraph 15.709, are permitted to operate in the:

-	Upper 6 MHz segment of 600MHz duplex gap at 40 mW EIRP.

-	TV band and 600 MHz service band, where 600 MHz licensees have not commenced operations, at specified protection distances at up to 10 W EIRP (if fixed devices) and at up to 100 mW EIRP (if personal/portable devices). Fixed device operation in the 602-620 MHz band is limited to a maximum of 4 W (36 dBm) EIRP.

-	Channel 37 at up to 4 W EIRP (if fixed devices) and at up to 100 mW EIRP (if personal/portable devices).

Unlicensed wireless microphones are permitted to operate in the:

-	upper 6 MHz segment of duplex gap at up to 20 mW EIRP;

-	guard bands adjacent to channel 37 excluding the 1MHz segment farthest from channel 37 at up to 20 mW EIRP;

-	service band, where 600 MHz licensees have not commenced operations, at specified separation distances beyond wireless licensee's service areas at up to 50 mW EIRP;

-	channels allocated and assigned for the broadcast television service at up to 50 mW EIRP.

Emissions within the band from 1 MHz below to 1 MHz above the carrier frequency shall comply with the ETSI emission mask in section 8.3 of 9. Emissions outside this band shall comply with the limit specified at the edges of the ETSI mask.

Low power auxiliary stations may operate in the 4 MHz segment from 1 to 5 MHz above the lower edge of the 600 MHz duplex gap at up to 20 mW EIRP.

Eventual impacts on 600 MHz services of such unlicensed operations in the duplex gap and guard band are not taken into account for the specifications of 600 MHz band RF requirements.

## 5.4	FCC regulation requirement

Wireless communication services in 600 MHz band are governed by Part 27 of electronic Code of Federal Regulations (eCFR) [8] adopting for such band technical rules similar to those in the adjacent lower 700 MHz band. Summary of the FCC rules relevant to RAN4 is provided below.

1)	Power Limits

600 MHz fixed and base stations operating in non-rural areas must not exceed an effective radiated power (ERP) of 1000 watts (for emission bandwidths less than 1 MHz) or 1000 watts/MHz (for emission bandwidths greater than 1 MHz) and an antenna height of 305 meter height above average terrain (HAAT). Fixed and base stations located in a county with population density of 100 or fewer persons per square mile must not exceed 2000 watts or 2000 watts/MHz ERP and an antenna height of 305 meter HAAT. Antenna heights greater than 305 m HAAT are permitted if power levels are reduced below the maximum limits in accordance with Table 5.4-1 below.

Table 5.4-1: Power limits vs. Antenna HAAT

| Antenna HAAT (meters/feet) | ERP (watts) |  | ERP (watts/MHz) |  |
| --- | --- | --- | --- | --- |
|  | Emission BW  1MHz |  | Emission BW > 1MHz |  |
|  | Non-rural areas | Rural areas* | Non-rural areas | Rural areas* |
| > 1372 (4500) | 65 | 130 | 65 | 130 |
| 1220 (4000) to 1372 (4500) | 70 | 140 | 70 | 140 |
| 1067 (3500) to 1220 (4000) | 75 | 150 | 75 | 150 |
| 915 (3000) to 1067 (3500) | 100 | 200 | 100 | 200 |
| 763 (2500) to 915 (3000) | 140 | 280 | 140 | 280 |
| 610 (2000) to 763 (2500) | 200 | 400 | 200 | 400 |
| 458 (1500) to 610 (2000) | 350 | 700 | 350 | 700 |
| 305 (1000) to 458 (1500) | 600 | 1200 | 600 | 1200 |
| Up to 305 (1000) | 1000 | 2000 | 1000 | 2000 |
| NOTE:* with population density of 100 or fewer persons per square mile. |  |  |  |  |

Portable stations (hand-held devices) and fixed and mobile stations in the 600 MHz uplink band are limited to 3 watts (34.8 dBm) ERP.

2)	Emission Limits

The power of any emission outside a licensee´s frequency band/bands of operations shall be attenuated below the transmitter power P, measured in watts, by at least 43+10log10(P) dB, considering a resolution bandwidth of 100 kHz or greater, except within 0-100 kHz immediately outside and adjacent a licensee´s frequency block where it is required to be 30 kHz. That corresponds to a single absolute emission power spectral density limit of -13 dBm (Emission limit = PdBW-43-PdB= - 43 dBW = -13 dBm) over the specified resolution bandwidth for all transmitters (including base stations and portable devices) regardless of actual ERP or frequency offset.

3)	Basic interoperability requirement

Mobile and portable stations that operate on any portion of frequencies in the 600 MHz band must be capable of operating on all frequencies in the 600 MHz band using the same air interfaces that the equipment utilizes on any frequencies in the 600 MHz band.

# 6	List of band specific issues for US 600 MHz frequency band

Below is the list of issues that are studied for the US 600 MHz Band 71:

-	Co-existence with other 3GPP operating bands and nearby non-3GPP service in the USA.

-	BS unwanted emission.

-	UE MOP, unwanted emissions, REFSENS, in-band blocking and A-MPR.

# 7	Study of E-UTRA specific issues

## 7.1	UE aspect issues

### 7.1.1 	UE transmitter requirements

#### 7.1.1.1	UE co-existence

It is proposed to add new 600MHz Band 71 into Table 6.6.3.2-1 of TS 36.101 to address UE co-existence of the Band 71 with other Region 2 bands utilized in US, which include Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66 and 70, with legacy emission requirement of -50 dBm/MHz. No A-MPR or RB restrictions are needed. Harmonic exception should be allowed for Band71 UL to protect the DL of Band 2, 25, 41 and 70.

#### 7.1.1.2	UE TX maximum output power

Undoubtedly, power class 3 should be still maintained for US 600 MHz Band 71. However considering current frequency range and filter implementation challenges, following aspects should be considered in the discussion of the lower tolerance of maximum output power of.

-	Challenged relative Duplex Gap (11 MHz/657.5 MHz) of 1.67%, legacy agreement is lower accuracy relaxation on band edge.

-	Additional duplexer switch loss should be considered if dual-duplexer structure used. Furthermore it is not expected that delta Tc needs to be defined to account for the filter rolloff at frequencies within 4 MHz of the band edge due to such filter assumption.

-	According to survey on simulation results on filter performance, TX insertion loss is comparable to practical TX IL of Band 28 duplexer. Corresponding relaxation on accuracy of Band 28 can be adopted for Band 71.

Based on above discussion it is proposed to introduce additional 0.5 dB lower tolerance as an average one rather than normal way with delta Tc on the band edge. Hence the MOP requirement in table 7.1.1.2-1 shall apply for Band 71.

Table 7.1.1.2-1: UE Power Class

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 71 |  |  |  |  | 23 | +2/-2.5 |  |  |

#### 7.1.1.3 	NS value and Spectrum Emission Mask

In a study of the regulatory requirements, the emission limits for the 600 MHz band were found to be consistent with those for the lower 700 MHz bands. The lower 700 MHz bands have been defined in 3GPP with consideration for FCC emission requirements by network signalling value NS_06. When NS_06 is signalled by the network, the additional emission requirements shown in the table below apply. These reflect the attenuation requirement of 43 + 10log(P) outside of the licensee's band with measurement bandwidth of at least 100 kHz except for within the first 100 kHz where 30 kHz can be used. NS_06 does not allow any A-MPR backoff as defined today for Bands 12, 13, 14, and 17. However, two observations are made that deserve consideration before applying these same requirements to the 600 MHz band. The first observation is that the existing NS_06 only includes channel bandwidths up to 10 MHz since, of course, the existing 700 MHz bands do not support 15 and 20 MHz channels. The second observation, is that the FCC regulation specifies a measurement bandwidth of 100 kHz for the 43 + 10log(P) attenuation requirement. However, the 3GPP-defined NS_06 requirement specifies a measurement bandwidth of 1 MHz for the same attenuation requirement. This is to maintain consistency with the 3GPP general SEM requirement which is defined with measurement bandwidth of 1 MHz. The 3GPP requirement with measurement bandwidth of 1 MHz is more stringent than the FCC requirement. Therefore, compliance with the 3GPP requirement automatically implies compliance with the FCC requirement. The disadvantage, however, is that the 3GPP requirement may be unnecessarily tight and may require A-MPR for the 15 and 20 MHz channels.

Given the above observations, there are three options to consider when extending the NS_06 requirement to Band 71. The first option is to maintain the NS_06 requirement, maintain the 1 MHz measurement bandwidth, and simply extend by adding 15 and 20 MHz channels. As described above, this would result in a 3GPP requirement that is tighter than the regulatory requirement.

Table 7.1.1.3-1: Additional requirements

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
|  0-0.1 | -13 | -13 | -15 | -18 | -20 | -21 | 30 kHz |
|  0.1-1 | -13 | -13 | -13 | -13 | -13 | -13 | 100 kHz |
|  1-2.5 | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
|  2.5-2.8 | -25 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
|  2.8-5 |  | -13 | -13 | -13 | -13 | -13 | 1 MHz |
|  5-6 |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
|  6-10 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
|  10-15 |  |  |  | -25 | -13 | -13 | 1 MHz |
|  15-20 |  |  |  |  | -25 | -13 | 1 MHz |
|  20-25 |  |  |  |  |  | -25 | 1 MHz |

The second option is to define the measurement bandwidth to be 100 kHz in exact accordance with the FCC rules. One could argue that the existing NS_06 could be modified in a similar manner even for the 1.4, 3, 5, and 10 MHz channel bandwidths since this is how they are specified according to FCC rules, however, since these requirement have been in place since the Rel-8 specifications and since there is no A-MPR "penalty" associated with them, it is recommended that any such change to the measurement bandwidth be only applied in Band 71. For example, a new SEM table to supplement the existing NS_06 table could be added as shown below. If this option is taken, due to the change in the requirement, it may be prudent to define a new NS value for Band 71 rather than to try to reuse NS_06.

Table 7.1.1.3-2: Additional requirements for 15 MHz and 20 MHz channels

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |
| --- | --- | --- | --- |
| ΔfOOB (MHz) | 15 MHz | 20 MHz | Measurement bandwidth |
|  0-0.1 | -20 | -21 | 30 kHz |
|  0.1-15 | -13 | -13 | 100 kHz |
|  15-20 | -251 | -13 | 100 kHz |
|  20-25 |  | -25 | 1 MHz |
| Note: 1The measurement bandwidth shall be 1 MHz. |  |  |  |

A third option also exists to apply the 100 kHz measurement bandwidth to all channel bandwidths 5, 10, 15, and 20 MHz in Band 71. A new NS would be needed applicable only to Band 71 as the 700 MHz bands would retain NS_06. However, this option may create some confusion to outside observers or regulators to see that different NS values and different requirements apply between 600 and 700 MHz bands in 3GPP since the FCC applies the same rules to both bands.

Table 7.1.1.3-3: Additional requirements

| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth (unless otherwise stated) |
| --- | --- | --- | --- | --- | --- |
|  0-0.1 | -15 | -18 | -20 | -21 | 30 kHz |
|  0.1-6 | -13 | -13 | -13 | -13 | 100 kHz |
|  6-10 | -251 | -13 | -13 | -13 | 100 kHz |
|  10-15 |  | -251 | -13 | -13 | 100 kHz |
|  15-20 |  |  | -251 | -13 | 100 kHz |
|  20-25 |  |  |  | -25 | 1 MHz |
| Note: 1The measurement bandwidth shall be 1 MHz. |  |  |  |  |  |

It is proposed that the third option is adopted in the definition of Band 71 since it most accurately reflects the regulatory requirements for this band.

### 7.1.2	UE receiver requirements

#### 7.1.2.1	UE REFSENS

Evaluation results for REFSENS of Band 71 are presented in table 7.1.2.1-1. And it is agreed to use average value to define the corresponding requirements.

Table 7.1.2.1-1: Evaluation result summary on REFSENS(2RX) for Band 71

| Source# | REFSENS for 5MHz(dBm) | REFSENS for 10MHz(dBm) | REFSENS for 15MHz(dBm) | REFSENS for 20MHz(dBm) |
| --- | --- | --- | --- | --- |
| 1 | -97 | -94 | -91.6 | -82.2 |
| 2 | -97 | -94 | -92.2 | -87.9 |
| 3 | -97 | -94 | -92.2 | -90 |
| 4 | -97 | -94 | -91.2 | -85.5 |
| 5 | -97 | -94 | -92.2 | -88.1 |
| 6 | -98 | -95 | -92.9 | -89.2 |
| 7 |  |  |  | -89.6 |
| Average value | -97.2 | -94.2 | -92 | -87.5 |

Based on above assessments, the reference sensitivity requirement specified in table 7.1.2.1-2 shall be met for E-UTRA Band 71 for an uplink transmission bandwidth less than or equal to the specified in and table 7.1.2.1-3.

Table 7.1.2.1-2: Reference sensitivity QPSK PREFSENS(2RX)

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 71 | - | - | -97.2 | -94.2 | -92 | -87.5 | FDD |

Table 7.1.2.1-3: Uplink configuration for reference sensitivity

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| 71 |  |  | 25 | 25 1 | 20 1 | 20 1 | FDD |
| NOTE: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. |  |  |  |  |  |  |  |

#### 7.1.2.2	In-band blocking

Rx blocking interference from broadcast TV sources was studied previously in the context of Band 17 and MediaFLO as the interferer. An additional in-band Rx blocking requirement was imposed on Band 17 as shown below.

Table 7.1.2.2-1: In-band blocking (including old Band 17 requirements)

| E-UTRA band | Parameter | Unit | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 | -30 | -30 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 | -BW/2 – 15 & -BW/2 – 9 | -BW/2 – 10 |
| 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 13, 14, 18, 19, 20, 21, 22, 23, 25, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43 | FInterferer | MHz | (Note 2) | FDL_low – 15 to FDL_high + 15 |  |  |
| 12 | FInterferer | MHz | (Note 2) | FDL_low – 10 to FDL_high + 15 |  | FDL_low – 10 |
| 17 | FInterferer | MHz | (Note 2) | FDL_low – 9 to FDL_high + 15 | FDL_low – 15 and FDL_low – 9 |  |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band. NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1. NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer centre frequencies. NOTE 4: Case 3 and Case 4 only apply to assigned UE channel bandwidth of 5 MHz. |  |  |  |  |  |  |

The case 3 blocker was defined to be -30 dBm to account for the Tx power of the TV broadcaster as well as the propagation loss to the UE receiver. The wanted signal power level is set at 6 to 9 dB above reference sensitivity and the blocker is defined as a 5 MHz LTE signal. The Tx power of the TV broadcaster for MediaFLO is 50 kW and the separation from edge to edge is 12 MHz and 6 MHz between Band 17 downlink and 700 MHz blocks D and E, respectively. The propagation loss was predicted based on models and drive tests including data provided in [R4-100430]. It was concluded that with high probability, the Rx power level at the UE antenna should be -30 dBm or lower. Thus, the Rx blocker level was defined to be -30 dBm.

For the 600 MHz band, a similar situation exists with TV broadcast in adjacent channels. However, one significant difference is that power of the TV broadcast at 600 MHz is up to 1 MW. Assuming a similar propagation loss in 600 MHz compared to 700 MHz, the power level reaching the UE receiver is 13 dB higher for 600 MHz band compared to Band 17. However, considering that the propagation is slightly better at 600 MHz than 700 MHz, the maximum Rx power with high probability is 15 dBm higher, resulting in an Rx blocker level of -15 dBm. In FCC R&O 14-50, the television signal strength received by the UE is estimated at -23 dBm. This derivation is based on a received power level of -15 dBm coupled with a UE Rx antenna gain of -8 dBi. However, the 3GPP specifications also account for the case where the antenna gain may be as high as 0 dBi for different types of devices, for example, fixed or nomadic. In TS 36.101, it is stated that a reference antenna with a gain of 0 dBi is assumed for each antenna port. Taking this into account, the received power level is then also -15 dBm, consistent with the proposal here.

For the US 600 MHz band plan, the closest broadcast TV is located on Channel 36. The upper edge of Channel 36 is 608 MHz while the lower edge of the 600 MHz band is 617 MHz; thus, there exists a 9 MHz guard band. The separation from the lower band edge of the 600 MHz band to the centre frequency of Channel 36 is 12 MHz.

Table 7.1.2.2-2: In-band blocking

| E-UTRA band | Parameter | Unit |  | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 |  | -44 | -15 | Void | -38 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 |  | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |  |  | -BW/2 - 11 |
| 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 22, 23, 25, 26, 27, 28, 31, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 44, 45, 65, 66, 68, 70 | FInterferer | MHz |  | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |  |  |  |
| 71 |  |  |  |  | FDL_low – 12 to FDL_high + 15 | FDL-low - 12 |  |  |
| 30 | FInterferer | MHz |  | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |  |  | FDL_low – 11 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer centre frequencies |  |  |  |  |  |  |  |  |

While the above describes the necessary condition for the UE to withstand TV blocking interference, the feasibility of the UE to meet such requirement should also be considered. It is expected that the Rx filter in the duplexer will be required to provide rejection of the TV blocker. Given that the normal requirement at 12 MHz offset from the band edge is -44 dBm, there is a 29 dB difference to overcome by the filter, assuming that the normal requirement can be met without filter assistance.

Filter simulation data is shown below. Filter vendor "B" was able to provide Rx attenuation in Channel 36 of 30 dB while vendor "A" was only able to provide 14 dB. Moreover, vendor "A" was only able to provide 39 dB Tx isolation under typical conditions; therefore, this filter is not usable for this band without further improvement. Filter vendor C attenuation over Channel 36 frequencies are shown in the figure below. At the upper edge of Channel 36 at 608 MHz, it can be seen that the Rx filter attenuation is approximately 40 dB under typical conditions. Thus, it is expected that under worst case conditions including manufacturing margin and temperature drift, an attenuation of at least 29 dB can be met. Thus, the feedback from two filter vendors is that the necessary attenuation can be met in the Rx filter to support a blocking requirement of -15 dBm from Channel 36.

Table 7.1.2.2-3: Simulation results for the US 600 MHz band plan

| Vendor | pass-band | Tx IL (typ.) | Rx IL (typ.) | Rx Iso (typ.) | Tx Iso (typ.) | RX Att @CH36 |
| --- | --- | --- | --- | --- | --- | --- |
| A | 35 MHz | 3.4 | 2.3 | 65 | 39 | 14 |
| B | 35 MHz | 3 (max) | 2.6 (max) | 55 (min) | 55 (min) | 30 (min) |
| C | 35 MHz | 3.0 (max) | 3.6 (max) | 50 (min) | 55 (min) | See below |

Figure 7.1.2.2-1: Filter Attenuation

#### 7.1.2.3 	Out-of-band blocking

For out-of-band blocking, the standard requirements  apply to Band 71 as well. Table 7.6.2.1-2 of [10] including Band 71 is reported below.

Table 7.1.2.3-1: Out of band blocking

| E-UTRA band | Parameter | Units | Frequency |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 | Range 4 |
|  | PInterferer | dBm | -44 | -30 | -15 | -15 |
| 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 30, 31, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42 (NOTE 2), 43 (NOTE 2), 44, 45, 48 (NOTE 2), 65, 66, 68, 70, 71 | FInterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz | - |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz | - |
| 2, 5, 12, 17 | FInterferer | MHz | - | - | - | FUL_low - FUL_high (NOTE 5) |
| NOTE 1: For the UE which supports both Band 11 and Band 21 the out of blocking is FFS. NOTE 2: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. NOTE 3: For the UE that supports both Band 4 and Band 66, the out-of-blocking frequency range for Band 4 is defined relative to FDL_low and FDL_high of Band 66. NOTE 4: For a UE supporting CA_20A-28A the requirements for Band 20 and Band 28 apply with FDL_low given by the lower limit of the restricted operating frequency range in Band 28 and FDL_high by Band 20 (Table 5.5A-2). NOTE 5: Range 4 requirement does not apply to category M1. |  |  |  |  |  |  |

## 7.2	BS aspect issues

The following BS specific TS 36.104 [11] changes are expected due to introduction of Band 71 in the USA:

Start of the extract of TS 36.104

6.6.3	Operating band unwanted emissions

6.6.3.1 	Minimum requirements for Wide Area BS (Category A)

For E-UTRA BS operating in Bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 26, 27, 28, 29, 31, 44, 68, 71 emissions shall not exceed the maximum levels specified in Tables 6.6.3.1-1 to 6.6.3.1-3.

6.6.3.2	Minimum requirements for Wide Area BS (Category B)

6.6.3.2.1	Category B requirements (Option 1)

For E-UTRA BS operating in Bands 5, 8, 12, 13, 14, 17, 20, 26, 27, 28, 29, 31, 44, 68, 67, 71 emissions shall not exceed the maximum levels specified in Tables 6.6.3.2.1-1 to 6.6.3.2.1-3:

6.6.3.3	Additional requirements

In certain regions the following requirement may apply. For E-UTRA BS operating in Bands 12, 13, 14, 17, 29, 71 emissions shall not exceed the maximum levels specified in Table 6.6.3.3-3.

Table 6.6.3.3-3: Additional operating band unwanted emission limits for E-UTRA (bands 12, 13, 14, 17, 29 and 71)

| Channel bandwidth | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 8) |
| --- | --- | --- | --- | --- |
| All | 0 MHz  f < 100 kHz | 0.015 MHz  f_offset < 0.085 MHz | -13 dBm | 30 kHz |
| All | 100 kHz  f < fmax | 150 kHz  f_offset < f_offsetmax | -13 dBm | 100 kHz |

6.6.4.3	Additional spurious emissions requirements

6.6.4.3.1	Minimum Requirement

Table 6.6.4.3.1-1: BS Spurious emissions limits for E-UTRA BS for co-existence with systems operating in other frequency bands

| System type for E-UTRA to co-exist with | Frequency range for co-existence requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| E-UTRA Band 71 | 617 - 652 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 71 |
|  | 663 – 698 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 71, since it is already covered by the requirement in sub-clause 6.6.4.2 |

Table 6.6.4.3.1-1A: Home BS Spurious emissions limits for co-existence with Home BS operating in other frequency bands

| Type of coexistence BS | Frequency range for co-location requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| E-UTRA Band 71 | 663 – 698 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 71, since it is already covered by the requirement in sub-clause 6.6.4.2. |

6.6.4.4	Co-location with other base stations

6.6.4.4.1	Minimum Requirement

Table 6.6.4.4.1-1: BS Spurious emissions limits for Wide Area BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| WA E-UTRA Band 71 | 663 - 698 MHz | -96 dBm | 100 kHz |  |

Table 6.6.4.4.1-2: BS Spurious emissions limits for Local Area BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| LA E-UTRA Band 71 | 663 - 698 MHz | -88 dBm | 100 kHz |  |

Table 6.6.4.4.1-3: BS Spurious emissions limits for Medium range BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| MR E-UTRA Band 71 | 663 - 698 MHz | -91 dBm | 100 kHz |  |

7.6.1	General blocking requirement

7.6.1.1	Minimum requirement

Table 7.6.1.1-1: Blocking performance requirement for Wide Area BS for E-UTRA

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -43 | PREFSENS +6dB* | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15 | PREFSENS +6dB* |  | CW carrier |

Table 7.6.1.1-1a: Blocking performance requirement for Local Area BS for E-UTRA

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -35 | PREFSENS +6dB* | See table 7.6.1. 1-2 | See table 7.6.1.1-2 |
|  | 1 | to | (FUL_low -11) 12750 | -15 | PREFSENS +6dB* |  | CW carrier |
|  | (FUL_high +20) | to |  |  |  |  |  |

Table 7.6.1.1-1b: Blocking performance requirement for Home BS for E-UTRA

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the channel edge of the wanted signal [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -27 | PREFSENS +14dB* | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15 | PREFSENS +14dB* |  | CW carrier |

Table 7.6.1.1-1c: Blocking performance requirement for Medium Range BS for E-UTRA

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset to the lower/higher Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -38 | PREFSENS +6dB* | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15 | PREFSENS +6dB* |  | CW carrier |

7.6.2	Co-location with other base stations

7.6.2.1	Minimum requirement

Table 7.6.2.1-1: Blocking performance requirement for E-UTRA and NB-IoT Wide Area BS when co-located with BS in other frequency bands.

| Co-located BS type | Centre Frequency of Interfering Signal (MHz) | Interfering Signal mean power (dBm) | Wanted Signal mean power (dBm) | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| WA E-UTRA Band 71 | 617 – 652 | +16** | PREFSENS + 6dB* | CW carrier |

Table 7.6.2.1-2: Blocking performance requirement for Local Area BS when co-located with BS in other frequency bands.

| Co-located BS type | Centre Frequency of Interfering Signal (MHz) | Interfering Signal mean power (dBm) | Wanted Signal mean power (dBm) | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| LA E-UTRA Band 71 | 617 – 652 | -6 | PREFSENS + 6dB* | CW carrier |

Table 7.6.2.1-3: Blocking performance requirement for E-UTRA Medium Range BS when co-located with BS in other frequency bands.

| Co-located BS type | Centre Frequency of Interfering Signal (MHz) | Interfering Signal mean power (dBm) | Wanted Signal mean power (dBm) | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| MR E-UTRA Band 71 | 617 – 652 | +8 | PREFSENS + 6dB* | CW carrier |

End of the extract of TS 36.104

# 8	Study of MSR specific issues

There are no specific MSR issues for the new 600 MHz band.

# 9	Channel numbering for E-UTRA, MSR

The E-UTRA Absolute Radio Frequency Channel Numbers (EARFCNs) allocated for 600MHz FDD band in the USA are outlined in Table 9-1.

Table 9-1: E-UTRA channel numbers

| E-UTRA Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low [MHz] | NOffs-DL | Range of NDL | FUL_low [MHz] | NOffs-UL | Range of NUL |
| 71 | 617 | 68586 | 68586-68935 | 663 | 133122 | 133122-133471 |

# 10	Required changes to E-UTRA and MSR specifications

The required changes to the 3GPP specifications for the new 600 MHz band are summarised in Table 10-1.

Table 10-1: 3GPP specifications with required changes

| Affected existing specifications |  |  |  |
| --- | --- | --- | --- |
| Spec No. | Subject of the CR | Comments | CR (Tdoc) |
| 36.101 | E-UTRA; UE Radio transmission and reception | Core part | R4-1708808 |
| 36.104 | E-UTRA; Base Station (BS) radio transmission and reception | Core part | R4-1707612 |
| 36.113 | E-UTRA; BS and repeater EMC | Core part | R4-1707613 |
| 36.124 | E-UTRA; EMC requirements for mobile terminals and ancillary equipment | Core part | R4-1707614 |
| 36.133 | E-UTRA; Requirements for support of RRM | Core part | R4-1707789 |
| 37.104 | E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception | Core part | R4-1707790 |
| 37.113 | E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) Electromagnetic Compatibility (EMC) | Core part | R4-1707616 |
| 25.104 | Base Station (BS) radio transmission and reception (FDD) | Core part | R4-1707611 |
| 25.461 | UTRAN Iuant interface: Layer 1 | Core part | R3-173243 |
| 25.466 | UTRAN Iuant interface: Application part | Core part | R3-173244 |
| 25.101 | User Equipment (UE) radio transmission and reception (FDD) | Core part | R4-1707088 |
| 25.133 | UTRAN: Requirements for support of radio resource management (FDD) | Perf part | R4-1707787 |
| 37.141 | E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) conformance testing | Perf part | R4-1707911 |
| 36.141 | E-UTRA; Base Station (BS) conformance testing | Perf part | R4-1707615 |
| 25.141 | Base Station (BS) conformance testing (FDD) | Perf part | R4-1707788 |
| 25.123 | UTRAN: Requirements for support of radio resource management (TDD) | Perf part | R4-1707786 |

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-04 | 3GPP RAN4#82BIS | R4-1702796 |  |  |  | TR skeleton | 0.0.1 |
| 2017-05 | 3GPP RAN4#83 | R4-1705224 |  |  |  | The following agreed text proposals have been included: R4-1703732: TP to B71 TR: Coexistence between 600 MHz and TV services R4-1703733: TP to B71 TR: Coexistence between 600 MHz and Channel 37 operations R4-1703734: TP to B71 TR: Coexistence between 600 MHz and Unlicensed services R4-1703735: TP to B71 TR: Regulatory Framework R4-1703736: TP to B71 TR: Required changes to E-UTRA and MSR specifications R4-1704071: TP to 600MHz TR: Channel numbering R4-1704072: TP on Band 71 co-existence with other 3GPP bands R4-1703984: TP for TR ab.cde: NS value and spectrum emission mask R4-1704073: TP to 600MHz TR: BS aspect issues R4-1703871: TP to 600MHz TR: MSR specific issues Editorial changes on text. | 0.1.0 |
| 2017-08 | 3GPP RAN4#84 | R4-1707912 |  |  |  | The following agreed text proposals have been included: R4-1704967: TP to TR 36.755 Section 3 (Symbol and Abbreviations) R4-1705885: TP to TR 36.755: Update of section on Coexistence with WMTS services R4-1705886: TP on section 6 in TR36.755 R4-1705160: TP on Maximum output power for Band 71 R4-1705683: TP for TR 36.755: Frequency ranges R4-1705161: TP on Spurious emission on Band 71 UE co-existence R4-1705684: TP for TR 36.755: Rx blocking R4-1705863: TP on REFSENS for Band 71 Editorial changes Added 25.101 to Table 10-1 | 0.2.0 |
| 2017-09 | RP-77 | RP-171631 |  |  |  | Added CR Tdoc numbers to Table 10-1 Editorial changes | 1.0.0 |
| 2017-09 | RP-77 | RP-171631 |  |  |  | TR approved | 15.0.0 |