3GPP TR 36.787 V15.0.0 (2018-06)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Vehicle-to-Everything (V2X) new band combinations

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2018, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Background	7

4.1	Justification	7

4.2	Objective	7

5	Additional UE RF: General part	8

5.1	UE RF requirements	8

5.1.1	Candidate solution for self-interference issues	10

5.1.1.1	Solution1: Harmonics problem on Band 47	10

5.1.1.2	Solution2: IMD problem on licensed Bands	12

5.1.2	Summary of coexistence study	12

5.2	RRM requirements	13

6	Con-current V2X operation with 2CCs : Specific Band Combinations Part	13

6.1	V2X_20A-47A	13

6.1.1	Operating bands for V2X comminication	13

6.1.2	Channel bandwidths per operating band for V2X_20A-47A	13

6.1.3	Coexistence studies	14

6.1.4	∆TIB and ∆RIB values	16

6.1.5	REFSENS requirements	16

6.2	V2X_34A-47A	17

6.2.1	Operating bands	17

6.2.2	Channel bandwidths per operating band for V2X_34A-47A	17

6.2.3	Coexistence studies	17

6.2.4	∆TIB and ∆RIB values	19

6.3	V2X_5A-47A	19

6.3.1	Operating bands for V2X comminication	19

6.3.2	Channel bandwidths per operating band for V2X_5A-47A	20

6.3.3	Coexistence studies	20

6.3.4	∆TIB and ∆RIB values	22

6.3.5	REFSENS requirements	22

6.4	V2X_28A-47A	23

6.4.1	Operating bands for V2X comminication	23

6.4.2	Channel bandwidths per operating band	24

6.4.3	Coexistence studies	24

6.4.4	∆TIB and ∆RIB values	26

6.4.5	REFSENS requirements	26

6.5	V2X_71A-47A	28

6.5.1	Operating bands for V2X comminication	28

6.5.2	Channel bandwidths per operating band	28

6.5.3	Coexistence studies	28

6.5.4	∆TIB and ∆RIB values	30

6.5.5	REFSENs requirements	30

7	Con-current V2X operation with 3CCs : Specific Band Combinations Part	31

8	Other specification impacts (if applicable)	31

Annex A:	Change history	32

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is the Technical Report on TR on V2X new ban combinations for LTE.

The purpose of the present document is to study the extension of the band combinations for V2X service to grow the LTE V2X ecosystem. Specially, analyze the harmonics/IMDs problems on the own Rx frequency bands for daul transmission. (e.g. transmission of Band 47 on PC5 operation and transmission of licensed band on Uu operation).

Also RAN4 study on increasing of the number of multi-carrier for LTE-based V2X service. It can support concurrent operation of LTE CA band combinations on Uu interface and Band 47 on PC5 interface. Operator propose new band combinations up to 3CCs during the work item. In future study, the number of multiple carrier can be increased up to 5CCs.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-170752: " New WID on V2X new band combinations for LTE ".

[3]	3GPP TR 36.786 V1.0.0: " V2X Services based on LTE sidelink; User Equipment (UE) radio transmission and reception”.

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

(Void)

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

D2D	Device to Device

D2DSS	D2D Synchronization Signal

PD2DSCH	Physical D2D Synchronization Channel

PD2DSS	Primary D2D Synchronization Signal

ProSe	Proximity based Services

PS	Public Safety

PSBCH	Physical Sidelink Broadcast CHannel

PSCCH	Physical Sidelink Control CHannel

PSDCH	Physical Sidelink Discovery CHannel

PSS	Primary Synchronization Signal

PSSCH	Physical Sidelink Shared CHannel

PSSS	Primary Sidelink Synchronization Signal

SD2DSS	Secondary D2D Synchronization Signal

SSS	Secondary Synchronization Signal

SSSS	Secondary Sidelink Synchronization Signal

V2V	Vehicle to Vehicle

V2X	Vehicle to Anything

# 4	Background

## 4.1	Justification

3GPP is now completing the Release 14 work on LTE V2X including RAN4 minimum requirements and frequency bands for V2X operation. The current specification considers an initial set of frequency bands for Uu and PC5 interface that we requested as part of Release 14 work.

However, in order to continue to grow the LTE V2X ecosystem, it will be necessary to extend the frequency bands support by the V2X UE on Uu with concurrent operation of Band 47 on PC5 in Release 15 and later, and in addition to include RF requirements for:

-	LTE Carrier Aggregation on Uu concurrently with PC5 operation

## 4.2	Objective

The objective of this work item is to specify RAN4 RF requirements for the following:

-	Concurrent operation of additional LTE Uu frequency bands and PC5 operation on Band 47

-	Highest priority, and target for RAN#76 completion, shall be given to the following combination:

| V2X MCC Band | V2X Band | Channel bandwidths | Interface |
| --- | --- | --- | --- |
| V2X_20_47 | 20 | 5, 10, 15, 20 | Uu |
|  | 47 | 10, 20 | PC5 |

-	Other LTE Uu frequency bands are not precluded. For the target completion date, RAN4 should guaranteed the minimum two RAN4 WG meeting from the starting for new band combinations.

| V2X inter-band concurrent combination | V2X Band | Channel bandwidths [MHz] | Interface |
| --- | --- | --- | --- |
| V2X_34_47 | 34 | 5, 10, 15 | Uu |
|  | 47 | 10, 20 | PC5 |
| V2X_5_47 | 5 | 5, 10 | Uu |
|  | 47 | 10, 20 | PC5 |
| V2X_28_47 | 28 | 3, 5, 10, 15, 20 | Uu |
|  | 47 | 10, 20 | PC5 |
| V2X_71_47 | 71 | 5, 10, 15, 20 | Uu |
|  | 47 | 10, 20 | PC5 |

-	Concurrent operation of LTE Uu Carrier Aggregation and PC5 operation on Band 47. Proposed combinations to be proposed during the work item inline with the CA basket WID approach.

-	Based on agreed work plan (R4-1703287), the total number of carriers of LTE Uu CA and PC5 operation is restricted up to 3CCs in this WI.

-	E.g. CA_XA-YA (Uu) + 47A (PC5) or XA (Uu) + 47B (PC5)

Newly proposed combinations may be included to this work item at every RAN plenary, but shall have been proposed at the RAN4 meeting prior to the corresponding RAN plenary where they are agreed.

Once a band combination is completed, it shall be included in the 3GPP specifications immediately, and Release independence of that combination shall be supported from release 14 (same approach as for LTE frequency bands and CA combinations today).

# 5	Additional UE RF: General part

## 5.1	UE RF requirements

For LTE-based V2X service in Rel-14, RAN4 considered example RF architecture as shown in Figure 5.1-1.

Figure 5.1-1: Example RF architecture of inter-band con-current operation without harmonic problems

In rel-14, RAN4 only added V2X band combination without harmonics/IMDs problem to own V2X received frequency bands based on the agreed WF in rel-14 as below

Agreed WF: For introducing new license bands for V2X operation, RAN4 agreed the follow phased approach

-	Due to time limitation to complete at March 2017 in rel-14, the phased approach proposed to add new licensed bands as below

-	1st phase, Add new licensed bands w/o harmonics/IMD problems into Band 47.

-	So Band 3/7/8/39/41 are added for the new inter-band con-current V2X operation in V2X WI.

-	2nd phase, RAN4 study how to solve the harmonics/IMDs problems of multi-carrier V2X band combination.

Hence, RAN4 need to study how to solve the harmonics/IMD problem into the own receiving frequency bands for inter-band con-current V2X operation.

Table 5.1-1 shows the harmonics and IMDs problem by legacy LTE UL transmission or dual transmission with licensed band and unlicensed band.

Table 5.1-1: Harmonics /IMDs impacts on own Rx bands

| Licensed Band |  |  | B47 Range (MHz) | Comments for harmonics/IMD problems |
| --- | --- | --- | --- | --- |
|  |  |  | 5855 – 5925 |  |
| E-UTRA band | UL Range (MHz) | Harm. Order | Harmonic Range (MHz) |  |
| B3 | 1710 – 1785 | 3x | 5130 – 5355 | No Harmonics/IMDs |
| B7 | 2500 - 2570 | 2x | 5000 - 5140 | No Harmonics/IMDs |
| B8 | 880 -915 | 6x | 5280 – 5490 | No Harmonics/IMDs |
| B39 | 1880 - 1920 | 3x | 5640 – 5760 | No Harmonics/IMDs |
| B41 | 2496 - 2690 | 2x | 4992 – 5380 | No Harmonics/IMDs |
| B1 | 1920 - 1980 | 3x | 5760 – 5940 | 1) Harmonic into B47 2) 5th IMD into B47 |
| B5 | 824 - 849 | 7x | 5768 – 5943 | 1) Harmonic into B47, No IMD problems |
| B19 | 830 - 845 | 7x | 5810 – 5915 | 1) Harmonic into B47 No IMD problems |
| B20 | 832 - 862 | 7x | 5824 – 6034 | 1) Harmonic into B47, No IMD problems |
| B21 | 1448 - 1463 | 4x | 5792 – 5852 | 1) Side-lobe into B47 2) 4th IMD into B21 |
| B26 | 814 - 849 | 7x | 5698 – 5943 | 1) Harmonic into B47 No IMD problems |
| B28 | 703 - 748 | 8x | 5624 – 5984 | 1) Harmonic into B47 No IMD problems |
| B31 | 452.5 – 457.5 | 13x | 5882.5 – 5947.5 | 1) Harmonic into B47 but 13th harmonic impact can ignore. No IMD problems |
| B34 | 2010 - 2025 | 3x | 6030 - 6075 | No Harmonics/IMDs |
| B65 | 1920 - 2010 | 3x | 5760 – 6030 | 1) Harmonic into B47 2) 5th IMD into B47 and B65 |
| B71 | 663 - 698 | 8x | 5304 - 5584 | No Harmonics/IMDs |
| The other Licensed bands | - | Xx | - | No Harmonic impact. But need to IMD analysis into Band 47 and own licensed operating bands |

Based on the analysis Table 5.1-1, we summary the coexistence problem to own V2X Rx bands as below

-	Issue 1: Harmonics problem into Band 47

-	Issue 2: IMD problem into Band 47  Not occurred issues

-	Issue 3: IMD problem into licensed band (Band 21, Band 65)

However, the issue 2 is not occurred for inter-band con-current V2X UE including Band 47. Since Band 47 could not receive signal during Band 47 sidelink transmission as shown in Figure 5.1-2. Therefore, we can summary two major problems for inter-band con-current V2X operation to define new V2X band combinations.

Figure 5.1-2: 3 problems for inter-band con-current V2X operation

### 5.1.1	Candidate solution for self-interference issues

#### 5.1.1.1	Solution1: Harmonics problem on Band 47

The 7th harmonics of Band 20 is would interfere the reception of the Band 47. To analyse the self- desense level, we consider two option as below

-	Option1: Separate Ant. Architecture w/ HTF

-	Option2: Separate Ant. Architecture w/o HTF

The example RF architecture for option1 on V2X_20A-47A UE is shown in Figure 5.1.1.1-1.

Figure 5.1.1.1-1: Example RF architecture for V2X_20A-47A con-current operation with harmonic trap filter

The Table 5.1.1.1-1 is the required MSD level for V2X_20A-47A is derived as below.

Table 5.1.1.1-1: Estimated MSD level at Band 47 (V2X_20A-47A)

|  | Thermal | W/ HTF |  | W/O HTF |  |
| --- | --- | --- | --- | --- | --- |
|  |  | H7 level (dBm) | MSD (dB) | H7 level (dBm) | MSD (dB) |
| Main Path | -101 | -101.0 | 3.08 | -93.8 | 8.06 |
| Diversity Path | -101 | -100.6 | 3.27 | -93.7 | 8.12 |
| After MRC |  |  | 0.16 |  | 5.08 |

From the estimated MSD level at Band 47 by 7th harmonics from Band 20 is about 0dB when we consider the harmonics trap filter to mitigate the harmonics products.

Hence we propose as below to solve the harmonics problems into Band 47

-	Proposal: For V2X band combination with harmonics problems, RAN4 should consider the harmonics trap filter to mitigate the harmonic products into Band 47 and protect the safety message.

However, low-order harmonic will be more interfered the reception of the Band 47 such as V2X_1A-47A UE or V2X_47A-65A UE.

The Table 5.1.1.1-2 shows the estimated MSD level for V2X_1A-47A UE as below.

Table 5.1.1.1-2: Estimated MSD level at Band 47 (V2X_1A-47A) in current satatus

|  | Thermal | W/ HTF |  | W/O HTF |  |
| --- | --- | --- | --- | --- | --- |
|  |  | H3 level (dBm) | MSD (dB) | H3 level (dBm) | MSD (dB) |
| Main Path | -101 | -81.9 | 19.2 | -74.9 | 26.2 |
| Diversity Path | -101 | -81.9 | 19.2 | -74.9 | 26.2 |
| After MRC |  |  | 16.2 |  | 23.2 |

To derive the MSD level, the dominant factor in the RF component parameters for V2X_1A-47A UE are B1 PA attenuation level to protect Band 47 and PCB isolation level (PA to B47 LNA).

If PA vendor consider Band 47 as LTE protection band, the B1 PA attenuation level will be improved, and also the UE vendor should be improved the PCB isolation level by considering state-of-art technology. Then the required MSD level can be achieved as 0dB for V2X_1A-47A UE as below Table 5.1.1.1-3.

Table 5.1.1.1-3: Estimated MSD level at Band 47(V2X_1A-47A) in state-of-art technology

|  | Thermal | W/ HTF |  | W/O HTF |  |
| --- | --- | --- | --- | --- | --- |
|  |  | H3 level (dBm) | MSD (dB) | H3 level (dBm) | MSD (dB) |
| Main Path | -101 | -100.8 | 3.18 | -90.5 | 10.98 |
| Diversity Path | -101 | -100.4 | 3.37 | -90.5 | 11.01 |
| After MRC |  |  | 0.26 |  | 7.98 |

So RAN4 recommend when V2X band combination has low-order harmonic problem, UE vendor could consider state-of-art technology to increase the Band 1 PA attenuation level at the frequency range of Band 47. Also need to get the tight PCB isolation by implementation.  Then RAN4 define 0dB MSD in Band 47 for V2X inter-band concurrent operation UE with low-order harmonic problem.

However, this approach is quite difficult to apply all phone type UE in current state-of-art technology status. So the alternative solution is that 0dB MSD would be guaranteed when V2X UE transmit the V2X message within 1920~1950MHz frequency range since 3rd harmonic products do not fall into the own Band 47 band. However, V2X UE send a message in 1950~1975MHz, then the received signal in Band 47 would be desensitized by generated 3rd harmonic products. So RAN4 should define A-MPR requirements to protect V2X safety messages in Band 47.

Hence we propose as below to solve the harmonics problems into Band 47.

-	Proposal: If the MSD level is not guarantee 0dB for V2X band combination even if use harmonic trap filter, RAN4 shall define two minimum requirement. One is to guarantee 0dB MSD in a certain TX frequency band since the generated harmonics products will not impacted to receive the V2X safety message in Band 47. The other is defining of the A-MPR requirements to protect the safety message in the harmonic range.

#### 5.1.1.2	Solution2: IMD problem on licensed Bands

The IMD problem into the own receiving Band XX can be raised by dual transmission from the V2X_21A-47A UE and V2X_47A-65A UE in Table 5.1-1.

The problems on the own licensed Band is quite similar to inter-band CA combination. So this IMD problem can be easily solved by following dual uplink CA band combination.

In dual uplink CA, RAN4 define MSD requirements to allow sensitivity degradation in own receiving frequency band.

Hence, we propose as below to solve the IMD problems into licensed Rx Band.

-	Proposal: For V2X band combination with IMD problems by dual transmission, RAN4 define MSD level to allow sensitivity degradation in own receiving frequency band.

### 5.1.2	Summary of coexistence study

Table 5.1.2-1 summarizes the V2X band combinations with self-interference problems.

Table 5.1.2-1: Summary of Self-interference analysis for V2X band combination

| V2X con-current configuration | Dual transmission Configuration | Harmonic relation to own band | intermodulation to own band without uplink | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| V2X_5A-47A | V2X_5A-47A | 7th order harmonic into Band 47 | - | - | - 0.0 dB MSD w/ HTF. Only allow Tib,Rib by HTF |
| V2X_20A-47A | V2X_20A-47A | 7th order harmonic into Band 47 | - | - | - 0.0 dB MSD w/ HTF . Only allow Tib,Rib by HTF |
| V2X_28A-47A | V2X_20A-47A | 8th order harmonic into Band 47 | - | - | - 0.0 dB MSD w/ HTF Only allow Tib,Rib by HTF |
| V2X_34A-47A | V2X_34A-47A | - | - | - | - No harmonic/IMD issues |
| V2X_71A-47A | V2X_71A-47A | - | - | - | - No harmonic/IMD issues |

## 5.2	RRM requirements

Editor Note: It will be added in future [FFS]

# 6	Con-current V2X operation with 2CCs : Specific Band Combinations Part

## 6.1	V2X_20A-47A

### 6.1.1	Operating bands for V2X comminication

Table 6.1.1-1: Inter-band con-current V2X operating bands for V2X_20A-47A

| V2X con-current configuration | E-UTRA OperatingBand | Interface | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| V2X_20-47 | 20 | Uu | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz | – | 5925 MHz | TDD |

### 6.1.2	Channel bandwidths per operating band for V2X_20A-47A

Table 6.1.2-1: V2X inter-band con-current configurations and bandwidth combination sets for V2X_20A-47A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X inter-band Configuration | E-UTRA operating Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| V2X_20A-47A | 20 |  |  | Yes | Yes | Yes | Yes | [40] | 0 |
|  | 47 |  |  |  | Yes |  | [Yes] |  |  |

### 6.1.3	Coexistence studies

For own reception desensitization by dual transmission on V2X_20A-47A, RAN4 study up to 8th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.3-1 and 6.1.3-2.

Table 6.1.3-1: Harmonics analysis for V2X_20A-47A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 5855 | 5925 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 11710 | 11850 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 17565 | 17775 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3328 | 3448 | 23420 | 23700 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4160 | 4310 | 29275 | 29625 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 4992 | 5172 | 35130 | 35550 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 5824 | 6034 | 40985 | 41475 |
| 8th harmonics frequency limits | 8*fx_low | 8*fx_high | 8* fy_low | 8* fy_high |
| 8th harmonics frequency limits (MHz) | 6656 | 6896 | 46840 | 47400 |

Table 6.1.3-2: IMD analysis for V2X_20A-47A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 5855 | 5925 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 11710 | 11850 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 17565 | 17775 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4993 | 5093 | 6687 | 6787 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4261 | 4131 | 10848 | 11018 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7519 | 7649 | 12542 | 12712 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3429 | 3269 | 16703 | 16943 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8351 | 8511 | 18397 | 18637 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 10186 | 9986 | 13374 | 13574 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 22868 | 22558 | 2407 | 2597 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 24252 | 24562 | 9183 | 9373 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 16111 | 15841 | 9124 | 9354 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 19229 | 19499 | 14206 | 14436 |

When V2X inter-band con-current operating UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.1.3-3 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.1.3-3: Harmonic and IMDs analysis of V2X_20A-47A UE for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Based on the harmonics/IMD analysis tables, the 7th harmonics product will be impact to the own reception of V2X_20A-47A UE. This impact will be studied to slove the self-interference problems.

The impact on ISM band at 2.4GHz was already covered in 2DL/2UL CA WI in rel-12 which was solved by in-device coexistence solution such as TDM or FDM operation between dual uplink in licensed band and reception in unlicensed band.

### 6.1.4	∆TIB and ∆RIB values

For V2X inter-band con-current operation for V2X_20-47, same requirements on values for ∆TIB and ∆RIB can apply for CA UE with LAA band 47 in rel-13. RAN4 consider the following additional ILs for V2X_20A-47A UE.

Table 6.1.4-1: ΔTIB,c

| Inter-band V2X concurrent Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| V2X_20-47 | 20 | 0.2 |

Table 6.1.4-2: ΔRIB,c

| Inter-band V2X concurrent Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| V2X_20-47 | 20 | 0.2 |

### 6.1.5	REFSENS requirements

The 7th harmonics of Band 20 is would interfere the reception of the Band 47. To analyse the self- desense level, RAN4 consider two option as below

-	Option1: Separate Ant. Architecture w/ HTF

-	Option2: Separate Ant. Architecture w/o HTF

The table 6.1.5-1 is the common parameters for MSD analysis for V2X_20A-47A.

Table 6.1.5-1: V2X_20A-47A UE RF FE component isolation parameters

| Parameter | Option1 : W/ HTF |  |  |  | Option2: W/O HTF |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Primary |  | Diversity |  | Primary |  | Diversity |  |
|  | Value | H7 level | Value | H7 level | Value | H7 level | Value | H7 level |
| B20 Tx in PA output | 28 |  | 28 |  | 28 |  | 28 |  |
| B20 PA H7 attenuation | 69.8 | -41.8 | 69.8 | -41.8 | 69.8 | -41.8 | 69.8 | -41.8 |
| B20 duplexer H7 attenuation | 15 | -56.8 | 15 | -56.8 | 15 | -56.8 | 15 | -56.8 |
| Harmonic filter | 20 | -76.8 | 20 | -76.8 | 0 | -56.8 | 0 | -56.8 |
| LB switch H7 | -120 | -76.8 | -120 | -76.8 | -120 | -56.8 | -120 | -56.8 |
| Diplexer attenuation | 25 | -101.8 | 25 | -101.8 | 25 | -81.8 | 25 | -81.8 |
| Antenna isolation | 10 | -111.8 | 10 | -111.8 | 10 | -91.8 | 10 | -91.8 |
| Diplexer pathloss | 0.7 | -112.5 | 0.7 | -112.5 | 0.7 | -92.5 | 0.7 | -92.5 |
| UHB switch attenuation | 0.7 | -113.2 | 0.7 | -113.2 | 0.7 | -93.2 | 0.7 | -93.2 |
| UHB switch H7 | -130 | -113.1 | -110 | -108.3 | -130 | -93.2 | -110 | -93.1 |
| B47 Rx filter attenuation | 1.5 | -114.6 | 1.5 | -109.8 | 1.5 | -94.7 | 1.5 | -94.6 |
| B47 Rx filter H7 | -110 | -108.7 | -110 | -106.9 | -110 | -94.5 | -110 | -94.4 |
| B20 PA to B47 LNA isolation | 60 | -101.8 | 60 | -101.8 | 60 | -101.8 | 60 | -101.8 |
| Composite |  | -101.0 |  | -100.6 |  | -93.8 |  | -93.7 |

From the Table 6.1.5-1, MSD level is derived as below.

Table 6.1.5.2: Estimated MSD level at B47

|  | Thermal | W/ HTF |  | W/O HTF |  |
| --- | --- | --- | --- | --- | --- |
|  |  | H7 level (dBm) | MSD (dB) | H7 level (dBm) | MSD (dB) |
| Main Path | -101 | -101.0 | 3.08 | -93.8 | 8.06 |
| Diversity Path | -101 | -100.6 | 3.27 | -93.7 | 8.12 |
| After MRC |  |  | 0.16 |  | 5.08 |

Based on the MSD analysis, RAN4 recommend shall consider the harmonic trap filter to protect Band 47 for vehicle safety message.

Table 6.1.5-3: Proposed MSD values for V2X_20A-47A

| V2X con-current configuration | LGE | Company 2 | Company 3 | Avg. MSD [dB] |
| --- | --- | --- | --- | --- |
| V2X_20A-47A | 0.16 | TBD | TBD | TBD |
| Note 1: For the V2X con-current configuration UE with harmonic problem into Band 47, harmonic trap filter should be considered to protect the V2X safety message in Band 47 |  |  |  |  |

## 6.2	V2X_34A-47A

### 6.2.1	Operating bands

Table 6.2.1-1: Inter-band con-current V2X operating bands for V2X_34A-47A

| V2X con-current configuration | E-UTRA OperatingBand | Interface | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| V2X_34-47 | 34 | Uu | 2010MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz | TDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz | – | 5925 MHz | TDD |

### 6.2.2	Channel bandwidths per operating band for V2X_34A-47A

Table 6.2.2-1: V2X inter-band con-current configurations and bandwidth combination sets for V2X_34A-47A

| V2X multicarrier operation configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X inter-band Configuration | E-UTRA operating Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| V2X_34A-47A | 34 |  |  | Yes | Yes | Yes |  | [35] | 0 |
|  | 47 |  |  |  | Yes |  | [Yes] |  |  |

### 6.2.3	Coexistence studies

The analysis of harmonics for band 34 and band 47 are listed in table 6.2.3-1 below. As the harmonics of band 47 distribute in the frequency range higher than 5.9GHz, only the harmonics of band 34 are listed. It can be observed that no harmonics of band 34 will fall into band 47 as well as other IMT bands, which means there is no harmonic issue caused by this band combination.

Table 6.2.3-1: Harmonic analysis for B34 and B47

| Operating Band | Band 34 |  |  | Band 47 |  |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | fx_low | fx_high |  | fy_low | fy_high |
|  | 2010 | 2025 |  | 5855 | 5925 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | No effect |  |
|  | 4020 - 4050 |  |  |  |  |
| 3nd harmonics frequency limits | 3*fx_low |  | 3*fx_high | No effect |  |
|  | 6030 - 6075 |  |  |  |  |

The IMD analysis of band 34 and band 47 are listed in table 6.2.3-2 below.  According to the analysis, the 3rd order IMD products will fall into the UL band of band 2, and the 5th order IMD products will fall into the DL band of band 23. However, it should be noted that band 2 and band 23 are not intended for use in the same geographical area as Band 34. Hence, there is no IMD issue caused by the combination of band 34 and band 47.

Table 6.2.3-2: IMD analysis for B34 and B47

| Operating Band | Band 34 |  |  | Band 47 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | fx_low | fx_high |  | fy_low | fy_high |  |
|  | 2010 | 2025 |  | 5855 | 5925 |  |
| 2nd order IMD products frequency limits (MHz) | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ |  | ¦fy_high + fx_high¦ |
|  | 3830-3915 |  |  | 7865-7950 |  |  |
| Two-tone 3rd order IMD products frequency limits (MHz) | ¦2*fx_low – fy_high¦ |  | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ |  | ¦2*fy_high – fx_low¦ |
|  | 1805-1905 (band 2 UL) |  |  | 9685-9840 |  |  |
| Two-tone 3rd order IMD products frequency limits (MHz) | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ |  | ¦2*fy_high + fx_high¦ |
|  | 9875-9975 |  |  | 13720-13875 |  |  |
| Two-tone 4th order IMD products frequency limits (MHz) | ¦3*fx_low –fy_high¦ | ¦3*fx_high – fy_low¦ |  | ¦3*fy_low- fx_high¦ |  | ¦3*fy_high -fx_low¦ |
|  | 105-220 |  |  | 15540-15765 |  |  |
| Two-tone 4th order IMD products frequency limits (MHz) | ¦2*fx_low –2* fy_high¦ |  |  | ¦2*fx_high –2* fy_low¦ |  |  |
|  | 7660-7830 |  |  |  |  |  |
| Two-tone 4th order IMD products frequency limits (MHz) | ¦3*fx_low +fy_low¦ | ¦3*fx_high +fy_high¦ |  | ¦3*fy_low +fx_low¦ |  | ¦3*fy_high+fx_high¦ |
|  | 11885-12000 |  |  | 19575-19800 |  |  |
| Two-tone 4th order IMD products frequency limits (MHz) | ¦2*fx_low +2* fy_low¦ |  |  | ¦2*fx_high +2* fy_high¦ |  |  |
|  | 15730-15900 |  |  |  |  |  |
| Two-tone 5th order IMD products frequency limits (MHz) | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ |  | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
|  | 21395-21690 |  |  | 2115-2245 (band 23 DL) |  |  |
| Two-tone 5th order IMD products frequency limits (MHz) | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |
|  | 13515-13755 |  |  | 5635-5820 |  |  |
| Two-tone 5th order IMD products frequency limits (MHz) | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ |  | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
|  | 25430 - 25725 |  |  | 13895 - 14025 |  |  |
| Two-tone 5th order IMD products frequency limits (MHz) | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
|  | 21585 - 21825 |  |  | 17740 - 17925 |  |  |

Table 6.2.3-3 below shows the harmonics and IMD analysis when band 34 and band 47 operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and IMD products will have no impact on these systems.

Table 6.2.3-3:  B34+ B47 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

### 6.2.4	∆TIB and ∆RIB values

For V2X inter-band multicarrier operation V2X_34A_47A, the TIB,c and RIB,c values are shown in table 6.2.4-1 and in table 6.2.4-2:

Table 6.2.4-1: ΔTIB,c

| V2X inter-band multicarrier configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| V2X_34A-47A | 34 | 0 |
|  | 47 | 0 |

Table 6.2.4-2: ΔRIB,c

| V2X inter-band multicarrier configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| V2X_34A-47A | 34 | 0 |
|  | 47 | 0 |

## 6.3	V2X_5A-47A

### 6.3.1	Operating bands for V2X comminication

Table 6.3.1-1: Inter-band con-current V2X operating bands for V2X_5A-47A

| V2X con-current configuration | E-UTRA OperatingBand | Interface | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| V2X_5-47 | 5 | Uu | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz | – | 5925 MHz | HD |

### 6.3.2	Channel bandwidths per operating band for V2X_5A-47A

Table 6.3.2-1: V2X inter-band con-current configurations and bandwidth combination sets for V2X_5A-47A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X inter-band Configuration | E-UTRA operating Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| V2X_5A-47A | 5 |  |  | Yes | Yes |  |  | 30 | 0 |
|  | 47 |  |  |  | Yes |  | Yes |  |  |

### 6.3.3	Coexistence studies

For own reception desensitization by dual transmission on V2X_5A-47A, RAN4 study up to 8th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.3.3-1 and 6.3.3-2.

Table 6.3.3-1: Harmonics analysis for V2X_5A-47A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 5855 | 5925 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 11710 | 11850 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 17565 | 17775 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 23420 | 23700 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 29275 | 29625 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 4944 | 5094 | 35130 | 35550 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 5768 | 5943 | 40985 | 41475 |
| 8th harmonics frequency limits | 8*fx_low | 8*fx_high | 8* fy_low | 8* fy_high |
| 8th harmonics frequency limits (MHz) | 6592 | 6792 | 46840 | 47400 |

Table 6.3.3-2: IMD analysis for V2X_5A-47A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 5855 | 5925 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5006 | 5101 | 6679 | 6774 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4277 | 4157 | 10861 | 11026 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7503 | 7623 | 12534 | 12699 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3453 | 3308 | 16716 | 16951 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8327 | 8472 | 18389 | 18624 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 10202 | 10012 | 10062 | 10152 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 22876 | 22571 | 2459 | 2629 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 24244 | 24549 | 9151 | 9321 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 16127 | 15867 | 9163 | 9378 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 19213 | 19473 | 14182 | 14397 |

When V2X inter-band con-current operating UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.3.3-3 lists if up to 8th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.3.3-3: Harmonic and IMDs analysis of V2X_5A-47A UE for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5, 3rd harmonics from B5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5, 3rd harmonics from B5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 7th harmonics from B5 |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 7th harmonics from B5 |

Based on the harmonics/IMD analysis tables, the 7th harmonics product will be impact to the own reception of V2X_5A-47A UE. This impact will be studied to slove the self-interference problems.

The impact on ISM band at 2.4GHz and 5GHz were already covered in 2DL/2UL CA WI in rel-12 which was solved by in-device coexistence solution such as TDM or FDM operation between dual uplink in licensed band and reception in unlicensed band.

### 6.3.4	∆TIB and ∆RIB values

For V2X inter-band con-current operation for V2X_5-47, RAN4 agreed to use the harmonic trap filter to mitigate the harmonic interference level in Band 47. So the additional IL will be increased as below Table 6.3.4-1 and 6.3.4-2.

Table 6.3.4-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| V2X_5-47 | 5 | 0.2 |

Table 6.3.4-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| V2X_5-47 | 5 | 0.2 |

### 6.3.5	REFSENS requirements

The 7th harmonics of Band 5 is would interfere the reception of the Band 47. To analyse the self- desense level, RAN4 consider two option as below

-	Option1: Separate Ant. Architecture w/ HTF

-	Option2: Separate Ant. Architecture w/o HTF

The Table 6.3.5-1 is the common parameters for MSD analysis for V2X_5A-47A.

Table 6.3.5-1: V2X_5A-47A UE RF FE component isolation parameters

| Parameter | Option1 : W/ HTF |  |  |  | Option2: W/O HTF |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Primary |  | Diversity |  | Primary |  | Diversity |  |
|  | Value | H7 level | Value | H7 level | Value | H7 level | Value | H7 level |
| B5 Tx in PA output | 28 |  | 28 |  | 28 |  | 28 |  |
| B5 PA H7 attenuation | 69 | -41 | 68 | -41 | 69 | -41 | 69 | -41 |
| B5 duplexer H7 attenuation | 15 | -56 | 15 | -56 | 15 | -56 | 15 | -56 |
| Harmonic filter | 20 | -76 | 20 | -76 | 0 | -56 | 0 | -56 |
| LB switch H7 | -120 | -76 | -120 | -76 | -120 | -56 | -120 | -56 |
| Diplexer attenuation | 25 | -101 | 25 | -101 | 25 | -81 | 25 | -81 |
| Antenna isolation | 10 | -111 | 10 | -111 | 10 | -91 | 10 | -91 |
| Diplexer pathloss | 0.7 | -111.7 | 0.7 | -111.7 | 0.7 | -91.7 | 0.7 | -91.7 |
| UHB switch attenuation | 0.7 | -112.4 | 0.7 | -112.4 | 0.7 | -92.4 | 0.7 | -92.4 |
| UHB switch H7 | -130 | -112.3 | -110 | -108.0 | -130 | -92.4 | -110 | -92.3 |
| B47 Rx filter attenuation | 1.5 | -113.8 | 1.5 | -109.5 | 1.5 | -93.9 | 1.5 | -93.8 |
| B47 Rx filter H7 | -110 | -108.5 | -110 | -106.7 | -110 | -93.8 | -110 | -93.7 |
| B20 PA to B47 LNA isolation | 60 | -101.0 | 60 | -101.0 | 60 | -101.0 | 60 | -101.0 |
| Composite |  | -100.3 |  | -100.0 |  | -93.0 |  | -93.0 |

From the Table 6.3.5-1, MSD level is derived as below.

Table 6.3.5-2: Estimated MSD level at B47

|  | Thermal | W/ HTF |  | W/O HTF |  |
| --- | --- | --- | --- | --- | --- |
|  |  | H7 level (dBm) | MSD (dB) | H7 level (dBm) | MSD (dB) |
| Main Path | -101 | -100.3 | 3.44 | -93.0 | 8.69 |
| Diversity Path | -101 | -100.0 | 3.61 | -93.0 | 8.75 |
| After MRC |  |  | 0.51 |  | 5.71 |

Based on the MSD analysis, RAN4 recommend shall consider the harmonic trap filter to protect Band 47 for vehicle safety message.

Table 6.3.5-3: Proposed MSD values for V2X_5A-47A

| V2X con-current configuration | LGE | Avg. MSD [dB] |  |
| --- | --- | --- | --- |
| V2X_5A-47A | 0.51 |  | 0.5 |
| Note 1: For the V2X con-current configuration UE with harmonic problem into Band 47, harmonic trap filter should be considered to protect the V2X safety message in Band 47 |  |  |  |

## 6.4	V2X_28A-47A

### 6.4.1	Operating bands for V2X comminication

Table 6.4.1-1: Inter-band con-current V2X operating bands for V2X_28A-47A

| V2X con-current configuration | E-UTRA OperatingBand | Interface | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| V2X_28-47 | 28 | Uu | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz | – | 5925 MHz | HD |

### 6.4.2	Channel bandwidths per operating band

Table 6.4.2-1: V2X inter-band con-current configurations and bandwidth combination sets for V2X_28A-47A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X inter-band Configuration | E-UTRA operating Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| V2X_28A-47A | 28 |  | Yes | Yes | Yes | Yes | Yes | 40 | 0 |
|  | 47 |  |  |  | Yes |  | Yes |  |  |

### 6.4.3	Coexistence studies

For own reception desensitization by dual transmission on V2X_28A-47A, RAN4 study up to 8th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.4.3-1 and 6.4.3-2.

Table 6.4.3-1: Harmonics analysis for V2X_28A-47A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 5855 | 5925 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 11710 | 11850 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 17565 | 17775 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 23420 | 23700 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 29275 | 29625 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 4218 | 4488 | 35130 | 35550 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 4921 | 5236 | 40985 | 41475 |
| 8th harmonics frequency limits | 8*fx_low | 8*fx_high | 8* fy_low | 8* fy_high |
| 8th harmonics frequency limits (MHz) | 5624 | 5984 | 46840 | 47400 |

Table 6.1.3-2: IMD analysis for V2X_28A-47A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 5855 | 5925 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 11710 | 11850 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 17565 | 17775 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5107 | 5222 | 6558 | 6673 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4359 | 4519 | 10962 | 11147 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7261 | 7421 | 12413 | 12598 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3816 | 3611 | 16817 | 17072 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7964 | 8169 | 18268 | 18523 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 10214 | 10444 | 13116 | 13346 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 22672 | 22997 | 3113 | 2863 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 24123 | 24448 | 8667 | 8917 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 16069 | 16369 | 9466 | 9741 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 18971 | 19271 | 13819 | 14094 |

When V2X inter-band con-current operating UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.4.3-3 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.4.3-3: Harmonic and IMDs analysis of V2X_28A-47A UE for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | HD8, IMD2 |
|  | 5150 | - | 5350 | YES | Europe | IMD2 |
|  | 5470 | - | 5725 | YES |  | HD8 |
|  | 5150 | - | 5825 | YES | Asia | HD8, IMD2 |

Based on the harmonics/IMD analysis tables, the 8th harmonics product will impact to the own reception of V2X_28A-47A UE. This impact will be studied to solve the self-interference problems.

The impact on ISM band at 5GHz was already covered in 2DL/2UL CA WI in rel-12 which was solved by in-device coexistence solution such as TDM or FDM operation between dual uplink in licensed band and reception in unlicensed band.

### 6.4.4	∆TIB and ∆RIB values

For V2X inter-band con-current operation for V2X_28-47, RAN4 agreed to use the harmonic trap filter to mitigate the harmonic interference level in Band 47. So the additional IL will be increased as below Table 6.4.4-1 and 6.4.4-2.

Table 6.4.4-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| V2X_28-47 | 28 | 0.2 |

Table 6.4.4-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| V2X_28-47 | 28 | 0.2 |

### 6.4.5	REFSENS requirements

The 8th harmonics of Band 28 is would interfere the reception of the Band 47. To analyse the self- desense level, we consider two option as below

-	Option1: Separate Ant. Architecture w/ HTF

-	Option2: Separate Ant. Architecture w/o HTF

Table 6.4.5-1 is the common isolation assumption for the V2X_28A-47A UE FE components.

Table 6.4.5-1: V2X_28A-47A UE RF FE component isolation parameters

|  | Attenuation Value |
| --- | --- |
| B28 Tx in PA output | 28 |
| B28 PA H8 attenuation | 70.2 |
| B28 duplexer H8 attenuation | 15 |
| Harmonic filter | 20 or 0 (w/ or w/o) |
| LB switch H8 | 120 |
| Diplexer attenuation | 25 |
| Antenna isolation | 10 |
| Diplexer pathloss | 0.7 |
| UHB switch attenuation | 0.7 |
| UHB switch H8 | 130/110 (primary/secondary) |
| B47 Rx filter attenuation | 1.5 |
| B47 Rx filter H8 | 110/110 (primary/secondary) |
| B28 PA to B47 LNA isolation | 60 |

The Table 6.4.5-2 is the common parameters for MSD analysis for V2X_28A-47A.

Table 6.4.5-2: V2X_28A-47A UE RF FE component isolation parameters

| Parameter | Option1 : W/ HTF |  |  |  | Option2: W/O HTF |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Primary |  | Diversity |  | Primary |  | Diversity |  |
|  | Value | H8 level | Value | H8 level | Value | H8 level | Value | H8 level |
| B28 Tx in PA output | 28 |  | 28 |  | 28 |  | 28 |  |
| B28 PA H8 attenuation | 70 | -42 | 70 | -42 | 70 | -42 | 70 | -42 |
| B28 duplexer H8 attenuation | 15 | -57 | 15 | -57 | 15 | -57 | 15 | -57 |
| Harmonic filter | 20 | -77 | 20 | -77 | 0 | -57 | 0 | -57 |
| LB switch H8 | -120 | -77 | -120 | -77 | -120 | -57.0 | -120 | -57.0 |
| Diplexer attenuation | 25 | -102.0 | 25 | -102.0 | 25 | -82 | 25 | -82 |
| Antenna isolation | 10 | -112.0 | 10 | -112.0 | 10 | -92 | 10 | -92 |
| Diplexer pathloss | 0.7 | -112.7 | 0.7 | -112.7 | 0.7 | -92.7 | 0.7 | -92.7 |
| UHB switch attenuation | 0.7 | -113.4 | 0.7 | -113.4 | 0.7 | -93.4 | 0.7 | -93.4 |
| UHB switch H8 | -130 | -113.3 | -110 | -108.4 | -130 | -93.4 | -110 | -93.3 |
| B48 Rx filter attenuation | 1.5 | -114.8 | 1.5 | -109.9 | 1.5 | -94.9 | 1.5 | -94.8 |
| B48 Rx filter H8 | -110 | -108.8 | -110 | -106.9 | -110 | -94.8 | -110 | -94.7 |
| B28 PA to B48 LNA isolation | 60 | -102.0 | 60 | -101.9 | 60 | -102.0 | 60 | -101.9 |
| Composite |  | -101.2 |  | -100.8 |  | -94.0 |  | -93.9 |

From the Table 6.4.5-2, MSD level for V2X_28A-47A is derived as below.

Table 6.4.5-3: Estimated MSD level at Band 47

|  | Thermal | W/ HTF |  | W/O HTF |  |
| --- | --- | --- | --- | --- | --- |
|  |  | H8 level (dBm) | MSD (dB) | H8 level (dBm) | MSD (dB) |
| Main Path | -101 | -101.2 | 2.98 | -93.8 | 7.86 |
| Diversity Path | -101 | -100.8 | 3.17 | -93.7 | 7.93 |
| After MRC |  |  | 0.06 |  | 4.88 |

Based on the MSD analysis, RAN4 recommend shall consider the harmonic trap filter to protect Band 47 for vehicle safety message.

Table 6.4.5-4: Proposed MSD values for V2X_28A-47A

| V2X con-current configuration | LGE | Avg. MSD [dB] |  |
| --- | --- | --- | --- |
| V2X_28A-47A | 0.06 |  | 0.0 |
| Note 1: For the V2X con-current configuration UE with harmonic problem into Band 47, harmonic trap filter should be considered to protect the V2X safety message in Band 47 |  |  |  |

## 6.5	V2X_71A-47A

### 6.5.1	Operating bands for V2X comminication

Table 6.5.1-1: Inter-band con-current V2X operating bands for V2X_71A-47A

| V2X con-current configuration | E-UTRA OperatingBand | Interface | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| V2X_71-47 | 71 | Uu | 663MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz | – | 5925 MHz | TDD |

### 6.5.2	Channel bandwidths per operating band

Table 6.5.2-1: V2X inter-band con-current configurations and bandwidth combination sets for V2X_71A-47A

| V2X multicarrier operation configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X inter-band Configuration | E-UTRA operating Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| V2X_71A-47A | 71 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  | 47 |  |  |  | Yes |  | Yes |  |  |

### 6.5.3	Coexistence studies

The analysis of harmonics for band 71 and band 47 are listed in table 6.5.3-1 below. It can be observed that no harmonics of band 71 will fall into band 47, which means there is no harmonic issue caused by this band combination.

Table 6.5.3-1: Harmonic analysis for B71 and B47

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 5855 | 5925 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1396 | 11710 | 11850 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 17565 | 17775 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 23420 | 23700 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 29275 | 29625 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 3978 | 4188 | 35130 | 35550 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 4641 | 4886 | 40985 | 41475 |
| 8th harmonics frequency limits | 8*fx_low | 8*fx_high | 8* fy_low | 8* fy_high |
| 8th harmonics frequency limits (MHz) | 5304 | 5584 | 46840 | 47400 |

The IMD analysis of band 71 and band 47 are listed in table 6.5.3-2 below. According to the analysis, there is no IMD product falling into one of receiving band of band 71 and band 47.

Table 6.5.3-2: IMD analysis for B71 and B47

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 5855 | 5925 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5157 | 5262 | 6518 | 6623 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4459 | 4599 | 11012 | 11187 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7181 | 7321 | 12373 | 12548 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3761 | 3936 | 16867 | 17112 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7844 | 8019 | 18228 | 18473 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 10314 | 10524 | 13036 | 13246 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 22722 | 23037 | 3063 | 3273 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 24083 | 24398 | 8507 | 8717 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 16169 | 16449 | 9616 | 9861 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 18891 | 19171 | 13699 | 13944 |

When V2X_71A-47A operates with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.5.3-3 lists up to 8rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.5.3-3: B71+ B47 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd IMD, 8th harmonic of B71 |
|  | 5150 | - | 5350 | Yes | Europe | 2nd IMD, 8th harmonic of B71 |
|  | 5470 | - | 5725 | Yes |  | 8th harmonic of B71 |
|  | 5150 | - | 5825 | Yes | Asia | 2nd IMD, 8th harmonic of B71 |

### 6.5.4	∆TIB and ∆RIB values

For V2X inter-band con-current operation for V2X_71-47, there was no harmonic/IMD problem by own transmission. Therefore, there was no need to define the additional ILs for V2X_71A-47A UE.

Table 6.5.4-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| V2X_71-47 | 71 | 0.0 |

Table 6.5.4-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| V2X_71-47 | 71 | 0.0 |

### 6.5.5	REFSENs requirements

There is no MSD requirement.

# 7	Con-current V2X operation with 3CCs : Specific Band Combinations Part

Editor Note: It will be added in future [FFS]

# 8	Other specification impacts (if applicable)

Editor Note: If it is applicable, then it can be added in future [FFS]

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-04 | RAN4 #82-BIS | R4-1703288 |  |  |  | TR36.787 v0.0.1 | 0.0.1 |
| 2017-05 | RAN4 #83 | R4-1704867 |  |  |  | TR 36.787 v0.1.0 - Approved TP was captured in TR R4-1704060 “TP on the harmonics/IMD analysis for V2X_20_47 con-current operation” | 0.1.0 |
| 2017-08 | RAN4 #84 | R4-1708106 |  |  |  | TR 36.787 v0.2.0 - Approved TPs were captured in TR and agreed revised WID R4-1704870 “Candidate solution for Harmonics/IMD problem on own licensed band and ITS band” R4-1704879 “TP on the additional ILs level for V2X_20-47 concurrent operation UE” RP-170944 “Revised WID on V2X new band combinations for LTE” | 0.2.0 |
| 2017-10 | RAN4 #84BIS | R4-1710856 |  |  |  | TR 36.787 v0.3.0 - Approved TPs were captured in TR R4-1708144 "MSD analysis for V2X_5-47 concurrent operation," R4-1708142 "TP on the additional Ils for V2X_5-47 con-current operation R4-1708126 "TP on the harmonics/IMD analysis for V2X-5A-47A con-current operation," R4-1707499 "TP to introduce the combination of band 34 and 47 for V2X" | 0.3.0 |
| 2018-04 | RAN4 #86BIS | R4-1803997 |  |  |  | TR36.787 v0.4.0 - Approved TPs were captured in TR R4-1803346 “TP on slef-desense analysis results” R4-1803835 TP for 36.787: To introduce the combination of band 71 and 47 for V2X R4-1803836 CR_TS36 101 on new V2X band combination V2X_71A-47A in rel-15 R4-1803998 TP on self-desense analysis for V2X_28A-47A and V2X_71A-47A con-current operation | 0.4.0 |
| 2018-05 | RAN4 #87 | R4-1806683 |  |  |  | TR36.787 v0.5.0 - Approved TPs were captured in TR R4-1807208 TP for 36.787 Completion on V2X_28A-47A R4-1806684 CR on introduction of new V2X band combinations in rel-15 R4-1806685 CR on new V2X band combinations and eV2X feature in TS36.307 rel-14 R4-1806686 CR on new V2X band combinations and eV2X feature in TS36.307 rel-15 | 0.5.0 |
| 2018-06 | RAN #80 | RP-180775 |  |  |  | TR36.787 v1.0.0 presented to plenary for approval | 1.0.0 |
| 2018-06 | RAN#80 |  |  |  |  | Approved by plenary – Rel-15 spec under change control | 15.0.0 |