3GPP TS 37.213 V15.3.0 (2020-03)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Physical layer procedures for shared spectrum channel access

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Radio layer 1

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	6

4	Channel access procedure	6

4.1	Downlink channel access procedures	6

4.1.1	Channel access procedure for transmission(s) including PDSCH/PDCCH/EPDCCH	6

4.1.2	Channel access procedure for transmissions including discovery signal transmission(s) and not including PDSCH	8

4.1.3	Channel access procedure for transmissions including PDCCH and not including PDSCH	8

4.1.4	Contention window adjustment procedure	8

4.1.5	Energy detection threshold adaptation procedure	9

4.1.6	Channel access procedure for transmission(s) on multiple carriers	10

4.1.6.1	Type A multi-carrier access procedures	10

4.1.6.1.1	Type A1	10

4.1.6.1.2	Type A2	10

4.1.6.2	Type B multi-carrier access procedure	11

4.1.6.2.1	Type B1	11

4.1.6.2.2	Type B2	11

4.2	Uplink channel access procedures	11

4.2.1	Channel access procedure for uplink transmission(s)	12

4.2.1.1	Type 1 UL channel access procedure	15

4.2.1.2	Type 2 UL channel access procedure	16

4.2.2	Contention window adjustment procedure	16

4.2.3	Energy detection threshold adaptation procedure	18

4.2.3.1	Default maximum energy detection threshold computation procedure	18

Annex X (informative): Change history	20

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies and establishes the characteristics of the physical layer procedures for shared spectrum channel.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[3]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[4]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

CWp	Contention window for a given priority class

CWmax,p	Maximum contention window for a given priority class

CWmin,p	Minimum contention window for a given priority class

Tmcot,p	Maximum channel occupancy time for a given priority class

Tulmcot,p	Maximum Uplink channel occupancy time for a given priority class

XThresh	Energy detection threshold

Maximum energy detection threshold

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AUL-DFI	Autonomous UL Downlink feedback indication

COT	Channel Occupancy Time

LAA	Licensed Assisted Access

MCOT	Maximum Channel Occupancy Time

# 4	Channel access procedure

## 4.1	Downlink channel access procedures

An eNB operating LAA Scell(s) shall perform the channel access procedures described in this clause for accessing the channel(s) on which the LAA Scell(s) transmission(s) are performed.

### 4.1.1	Channel access procedure for transmission(s) including PDSCH/PDCCH/EPDCCH

The eNB may transmit a transmission including PDSCH/PDCCH/EPDCCH on a carrier on which LAA Scell(s) transmission(s) are performed , after first sensing the channel to be idle during the slot durations of a defer duration Td; and after the counter Nis zero in step 4. The counter N is adjusted by sensing the channel for additional slot duration(s) according to the steps below:

1)	set N=Ninit, where Ninit is a random number uniformly distributed between 0 and CWp, and go to step 4;

2)	if N>0 and the eNB chooses to decrement the counter, set N=N−1;

3)	sense the channel for an additional slot duration, and if the additional slot duration is idle, go to step 4; else, go to step 5;

4)	if N=0, stop; else, go to step 2.

5)	sense the channel until either a busy slot is detected within an additional defer duration Td or all the slots of the additional defer duration Td are detected to be idle;

6)	if the channel is sensed to be idle during all the slot durations of the additional defer duration Td, go to step 4; else, go to step 5;

If an eNB has not transmitted a transmission including PDSCH/PDCCH/EPDCCH on a carrier on which LAA Scell(s) transmission(s) are performed after step 4 in the procedure above, the eNB may transmit a transmission including PDSCH/PDCCH/EPDCCH on the carrier, if the channel is sensed to be idle at least in a slot duration Tsl when the eNB is ready to transmit PDSCH/PDCCH/EPDCCH and if the channel has been sensed to be idle during all the slot durations of a defer duration Td immediately before this transmission. If the channel has not been sensed to be idle in a slot duration Tsl when the eNB first senses the channel after it is ready to transmit or if the channel has been sensed to be not idle during any of the slot durations of a defer duration Td immediately before this intended transmission, the eNB proceeds to step 1 after sensing the channel to be idle during the slot durations of a defer duration Td.

The defer duration Td consists of duration Tf=16us immediately followed by mpconsecutive slot durations where each slot duration is , and includes an idle slot duration Tsl at start of ;

A slot duration Tslis considered to be idle if the eNB senses the channel during the slot duration, and the power detected by the eNB for at least 4uswithin the slot duration is less than energy detection threshold XThresh. Otherwise, the slot duration Tsl is considered to be busy.

CWmin,p≤CWp≤CWmax,pis the contention window. CWpadjustment is described in clause 4.1.4.

CWmin,pand are chosen before step 1 of the procedure above.

mp, CWmin,p, and are based on channel access priority class associated with the eNB transmission, as shown in Table 4.1.1-1.

XThresh adjustment is described in clause 4.1. 5

If the eNB transmits discovery signal transmission(s) not including PDSCH/PDCCH/EPDCCH when N>0in the procedure above, the eNB shall not decrement N during the slot duration(s) overlapping with discovery signal transmission.

The eNB shall not continuously transmit on a carrier on which the LAA Scell(s) transmission(s) are performed, for a period exceedingTmcot,p as given in Table 4.1.1-1.

For p=3and p=4 , if the absence of any other technology sharing the carrier can be guaranteed on a long term basis (e.g. by level of regulation), Tmcot,p=10ms, otherwise, Tmcot,p=8ms.

Table 4.1.1-1: Channel Access Priority Class

| Channel Access Priority Class (p) | mp | CWmin,p | CWmax,p | Tmcot,p | allowed CWpsizes |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3 | 7 | 2 ms | {3,7} |
| 2 | 1 | 7 | 15 | 3 ms | {7,15} |
| 3 | 3 | 15 | 63 | 8 or 10 ms | {15,31,63} |
| 4 | 7 | 15 | 1023 | 8 or 10 ms | {15,31,63,127,255,511,1023} |

For LAA operation in Japan, if the eNB has transmitted a transmission after N=0in step 4 of the procedure above, the eNB may transmit the next continuous transmission, for duration of maximum =4 ms, immediately after sensing the channel to be idle for at least a sensing interval of =34us and if the total sensing and transmission time is not more than 1000⋅Tmcot+Tmcot  Tj−1⋅Tjs µsec. consists of duration Tf=16usimmediately followed by two slot durations  each and includes an idle slot duration Tslat start of . The channel is considered to be idle for Tjsif it is sensed to be idle during the during the slot durations of Tjs.

### 4.1.2	Channel access procedure for transmissions including discovery signal transmission(s) and not including PDSCH

An eNB may transmit a transmission including discovery signal but not including PDSCH on a carrier on which LAA Scell(s) transmission(s) are performed immediately after sensing the channel to be idle for at least a sensing interval Tdrs=25us and if the duration of the transmission is less than 1 ms. Tdrs consists of a duration Tf=16usimmediately followed by one slot duration  and includes an idle slot duration Tslat start of . The channel is considered to be idle for Tdrsif it is sensed to be idle during the slot durations of Tdrs.

### 4.1.3	Channel access procedure for transmissions including PDCCH and not including PDSCH

If 'COT sharing indication' in AUL-UCI in subframe n indicates '1', an eNB may transmit a transmission in subframe n+X, where X is subframeOffsetCOT-Sharing, including PDCCH but not including PDSCH on the same carrier immediately after sensing the channel to be idle for at least a sensing interval Tpdcch=25us, if the duration of the PDCCH is less than or equal to two OFDM symbols length and it shall contain at least AUL-DFI or UL grant to the UE from which the PUSCH transmission indicating COT sharing was received. Tpdcch consists of a duration Tf=16usimmediately followed by one slot duration Tsl=9us and Tfincludes an idle slot duration Tslat start of Tf. The channel is considered to be idle for Tpdcch if it is sensed to be idle during the slot durations of Tpdcch.

### 4.1.4	Contention window adjustment procedure

If the eNB transmits transmissions including PDSCH that are associated with channel access priority class pon a carrier, the eNB maintains the contention window value CWp and adjusts CWp before step 1 of the procedure described in clause 4.1.1 for those transmissions using the following steps:

1)	for every priority class set CWp=CWmin,p

2)	if at least Z=80% of HARQ-ACK values corresponding to PDSCH transmission(s) in reference subframe kare determined as NACK, increase CWp for every priority class to the next higher allowed value and remain in step 2; otherwise, go to step 1.

Reference subframe k is the starting subframe of the most recent transmission on the carrier made by the eNB, for which at least some HARQ-ACK feedback is expected to be available.

The eNB shall adjust the value of CWp for every priority class based on a given reference subframe k only once.

If CWp=CWmax,p, the next higher allowed value for adjusting CWp is CWmax,p.

For determining Z,

-	if the eNB transmission(s) for which HARQ-ACK feedback is available start in the second slot of subframe k, HARQ-ACK values corresponding to PDSCH transmission(s) in subframe k+1are also used in addition to the HARQ-ACK values corresponding to PDSCH transmission(s) in subframe k.

-	if the HARQ-ACK values correspond to PDSCH transmission(s) on an LAA SCell that are assigned by (E)PDCCH transmitted on the same LAA SCell,

-	if no HARQ-ACK feedback is detected for a PDSCH transmission by the eNB, or if the eNB detects 'DTX', 'NACK/DTX' or 'any' state, it is counted as NACK.

-	if the HARQ-ACK values correspond to PDSCH transmission(s) on an LAA SCell that are assigned by (E)PDCCH transmitted on another serving cell,

-	if the HARQ-ACK feedback for a PDSCH transmission is detected by the eNB, 'NACK/DTX' or 'any' state is counted as NACK, and 'DTX' state is ignored.

-	if no HARQ-ACK feedback is detected for a PDSCH transmission by the eNB

-	if PUCCH format 1b with channel selection is expected to be used by the UE, 'NACK/DTX' state corresponding to 'no transmission' as described in clauses 10.1.2.2.1, 10.1.3.1 and 10.1.3.2.1 is counted as NACK, and 'DTX' state corresponding to 'no transmission' is ignored in [4].

-	Otherwise, the HARQ-ACK for the PDSCH transmission is ignored.

-	if a PDSCH transmission has two codewords, the HARQ-ACK value of each codeword is considered separately

-	bundled HARQ-ACK across M subframes is considered as M HARQ-ACK responses.

If the eNB transmits transmissions including PDCCH/EPDCCH with DCI format 0A/0B/4A/4B and not including PDSCH that are associated with channel access priority class pon a channel starting from time t0, the eNB maintains the contention window value CWp and adjusts CWp before step 1 of the procedure described in clause 4.1.1 for those transmissions using the following steps:

1)	for every priority class set CWp=CWmin,p

2)	if less than 10% of the UL transport blocks scheduled by the eNB using Type 2 channel access procedure (described in clause 4.2.1.2) in the time interval betweent0 and t0+TCOhave been received successfully, increase CWp for every priority class to the next higher allowed value and remain in step 2; otherwise, go to step 1.

where TCOis computed as described in clause 4.2.1.

If the CWp=CWmax,p is consecutively used K times for generation of Ninit,CWp is reset to CWmin,p only for that priority class pfor which CWp=CWmax,p is consecutively used K times for generation of Ninit. K is selected by eNB from the set of values {1, 2, …,8} for each priority class .

### 4.1.5	Energy detection threshold adaptation procedure

An eNB accessing a carrier on which LAA Scell(s) transmission(s) are performed, shall set the energy detection threshold (XThresh) to be less than or equal to the maximum energy detection threshold .

is determined as follows:

-	If the absence of any other technology sharing the carrier can be guaranteed on a long term basis (e.g. by level of regulation) then:

-

-	 is Maximum energy detection threshold defined by regulatory requirements in dBm when such requirements are defined, otherwise

-	Otherwise,

-	XThres_max=max−72+10⋅log10(BWMHz20MHz) dBm,

-	Where:

-	= 10dB for transmission(s) including PDSCH;

-	= 5dB for transmissions including discovery signal transmission(s) and not including PDSCH;

-	 = 23 dBm;

-	 is the set maximum eNB output power in dBm for the carrier;

-	eNB uses the set maximum transmission power over a single carrier irrespective of whether single carrier or multi-carrier transmission is employed

-	;

-	BWMHz is the single carrier bandwidth in MHz.

### 4.1.6	Channel access procedure for transmission(s) on multiple carriers

An eNB can access multiple carriers on which LAA Scell(s) transmission(s) are performed, according to one of the Type A or Type B procedures described in this clause.

#### 4.1.6.1	Type A multi-carrier access procedures

The eNB shall perform channel access on each carrier ci∈C, according to the procedures described in clause 4.1.1, where C is a set of carriers on which the eNB intends to transmit, and i=0,1,…q−1, and qis the number of carriers on which the eNB intends to transmit.

The counter N described in clause 4.1.1 is determined for each carrier ci and is denoted asNci. Nci is maintained according to Clause 4.1.6.1.1 or 4.1.6.1.2.

##### 4.1.6.1.1	Type A1

Counter N as described in clause 4.1.1 is independently determined for each carrier ci and is denoted asNci.

If the absence of any other technology sharing the carrier cannot be guaranteed on a long term basis (e.g. by level of regulation), when the eNB ceases transmission on any one carrier cj∈C, for each carrier ci≠cj, the eNB can resume decrementing Nci when idle slots are detected either after waiting for a duration of 4⋅Tsl, or after reinitialisingNci.

##### 4.1.6.1.2	Type A2

Counter Nis determined as described in clause 4.1.1 for carrier cj∈C, and is denoted as Ncj, where cjis the carrier that has the largest CWpvalue. For each carrier ci, Nci=Ncj.

When the eNB ceases transmission on any one carrier for which Nci is determined, the eNB shall reinitialiseNcifor all carriers.

#### 4.1.6.2	Type B multi-carrier access procedure

A carrier cj∈Cis selected by the eNB as follows

-	the eNB selects cjby uniformly randomly choosing cjfrom before each transmission on multiple carriers ci∈C, or

-	the eNB selects cjno more frequently than once every 1 second,

where C is a set of carriers on which the eNB intends to transmit, i=0,1,…q−1, and qis the number of carriers on which the eNB intends to transmit.

To transmit on carrier cj

-	the eNB shall perform channel access on carrier cjaccording to the procedures described in clause 4.1.1 with the modifications described in clause 4.1.6.2.1 or 4.1.6.2.2.

To transmit on carrier ci≠cj, ci∈C

-	for each carrier ci, the eNB shall sense the carrier cifor at least a sensing interval Tmc=25us immediately before the transmitting on carriercj, and the eNB may transmit on carrier ciimmediately after sensing the carrier ci to be idle for at least the sensing interval Tmc. The carrier ciis considered to be idle for Tmcif the channel is sensed to be idle during all the time durations in which such idle sensing is performed on the carrier cjin given interval Tmc.

The eNB shall not continuously transmit on a carrier ci≠cj, ci∈C, for a period exceedingTmcot,p as given in Table 4.1.1-1, where the value of Tmcot,p is determined using the channel access parameters used for carrier cj.

##### 4.1.6.2.1	Type B1

A single CWpvalue is maintained for the set of carriers C.

For determining CWp for channel access on carrier cj, step 2 of the procedure described in clause 4.1.4 is modified as follows

-	if at least Z=80% of HARQ-ACK values corresponding to PDSCH transmission(s) in reference subframe kof all carriers ci∈Care determined as NACK, increase CWp for each priority class to the next higher allowed value; otherwise, go to step 1.

##### 4.1.6.2.2	Type B2

A CWpvalue is maintained independently for each carrier ci∈Cusing the procedure described in clause 4.1.4.

For determining Ninit for carrier cj, CWpvalue of carrier cj1∈Cis used, where cj1is the carrier with largest CWpamong all carriers in set C.

## 4.2	Uplink channel access procedures

A UE and a eNB scheduling UL transmission(s) for the UE shall perform the procedures described in this clause for the UE to access the channel(s) on which the LAA Scell(s) transmission(s) are performed.

### 4.2.1	Channel access procedure for uplink transmission(s)

The UE can access a carrier on which LAA Scell(s) UL transmission(s) are performed according to one of Type 1 or Type 2 UL channel access procedures. Type 1 channel access procedure is described in clause 4.2.1.1. Type 2 channel access procedure is described in clause 4.2.1.2.

If an UL grant scheduling a PUSCH transmission indicates Type 1 channel access procedure, the UE shall use Type 1 channel access procedure for transmitting transmissions including the PUSCH transmission unless stated otherwise in this clause.

A UE shall use Type 1 channel access procedure for transmitting transmissions including the PUSCH transmission on autonomous UL resources unless stated otherwise in this clause.

If an UL grant scheduling a PUSCH transmission indicates Type 2 channel access procedure, the UE shall use Type 2 channel access procedure for transmitting transmissions including the PUSCH transmission unless stated otherwise in this clause.

The UE shall use Type 1 channel access procedure for transmitting SRS transmissions not including a PUSCH transmission. UL channel access priority class p=1 is used for SRS transmissions not including a PUSCH.

If the UE is scheduled to transmit PUSCH and SRS in subframe n, and if the UE cannot access the channel for PUSCH transmission in subframe n, the UE shall attempt to make SRS transmission in subframe n according to uplink channel access procedures specified for SRS transmission.

The total length of autonomous uplink transmission(s) obtained by the channel access procedure in this clause, including the following DL transmission if the UE sets 'COT sharing indication' in AUL-UCI to '1' in a subframe within the autonomous uplink transmission(s), shall not exceed , where  is given in Table 4.2.1-1.

Table 4.2.1-1: Channel Access Priority Class for UL

| Channel Access Priority Class (p) | mp | CWmin,p | CWmax,p | Tulmcot,p | allowed CWpsizes |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 7 | 2 ms | {3,7} |
| 2 | 2 | 7 | 15 | 4 ms | {7,15} |
| 3 | 3 | 15 | 1023 | 6ms or 10 ms | {15,31,63,127,255,511,1023} |
| 4 | 7 | 15 | 1023 | 6ms or 10 ms | {15,31,63,127,255,511,1023} |
| NOTE1: For p=3,4, Tulmcot,p =10ms if the higher layer parameter 'absenceOfAnyOtherTechnology-r14' indicates TRUE, otherwise, Tulmcot,p =6ms. NOTE 2: When Tulmcot,p=6ms it may be increased to 8 ms by inserting one or more gaps. The minimum duration of a gap shall be 100 µs. The maximum duration before including any such gap shall be 6 ms. |  |  |  |  |  |

If the UL duration and offset' field configures an 'UL offset' l and an 'UL duration' d for subframe n, then

the scheduled UE may use channel access Type 2 for transmissions in subframes n+l+iwhere i=0,1,…d−1, irrespective of the channel access Type signalled in the UL grant for those subframes, if the end of UE transmission occurs in or before subframe n+l+d−1.

If the 'UL duration and offset' field configures an 'UL offset' l and an 'UL duration' d for subframe n and the 'COT sharing indication for AUL' field is set to true, then a UE configured with autonomous UL may use channel access Type 2 for autonomous UL transmissions corresponding to any priority class in subframes n+l+iwhere i=0,1,…d−1, if the end of UE autonomous UL transmission occurs in or before subframe n+l+d−1 and the autonomous UL transmission between n+l and n+l+d−1 shall be contiguous.

If the 'UL duration and offset' field configures an 'UL offset' l and an 'UL duration'd for subframe n and the 'COT sharing indication for AUL' field is set to false, then a UE configured with autonomous UL shall not transmit autonomous UL in subframes n+l+i where i=0,1,…d−1.

If the UE scheduled to transmit transmissions including PUSCH in a set subframes n0,n1,⋯,nw−1 using PDCCH DCI Format 0B/4B, and if the UE cannot access the channel for a transmission in subframe nk, the UE shall attempt to make a transmission in subframe nk+1 according to the channel access type indicated in the DCI, where k∈0,1,⋯w−2, and wis the number of scheduled subframes indicated in the DCI.

If the UE is scheduled to transmit transmissions without gaps including PUSCH in a set of subframes n0,n1,⋯,nw−1 using one or more PDCCH DCI Format 0A/0B/4A/4B and the UE performs a transmission in subframe nk after accessing the carrier according to one of Type 1 or Type 2 UL channel access procedures, the UE may continue transmission in subframes after nk where k∈0,1,⋯w−1.

If the beginning of UE transmission in subframen+1 immediately follows the end of UE transmission in subframe n, the UE is not expected to be indicated with different channel access types for the transmissions in those subframes.

If the UE is scheduled to transmit transmissions including PUSCH Mode 1 using the Type 1 channel access procedure indicated in DCI, and if the UE cannot access the channel for a transmission according to the PUSCH starting position indicated in the DCI, the UE shall attempt to make a transmission at symbol 7 in the same subframe according to Type 1 channel access procedure. There is no limit on the number of attempts the UE can make using Type 1 channel access procedure.

If the UE is scheduled to transmit transmissions including PUSCH Mode 1 using the Type 2 channel access procedure indicated in DCI, and if the UE cannot access the channel for a transmission according to the PUSCH starting position indicated in the DCI, the UE may attempt to make a transmission at symbol 7 in the same subframe and according to Type 2 channel access procedure. The number of attempts the UE should make within the consecutively scheduled subframes including the transmission is limited to w+1, where w is the number of consecutively scheduled subframes using Type 2 channel access procedure.

If the UE is scheduled to transmit without gaps in subframes n0,n1,⋯,nw−1 using one or more PDCCH DCI Format 0A/0B/4A/4B, and if the UE has stopped transmitting during or before subframe nk1, k1∈0,1,⋯w−2, and if the channel is sensed by the UE to be continuously idle after the UE has stopped transmitting, the UE may transmit in a later subframe nk2, k2∈1,⋯w−1 using Type 2 channel access procedure. If the channel sensed by the UE is not continuously idle after the UE has stopped transmitting, the UE may transmit in a later subframe nk2, k2∈1,⋯w−1 using Type 1 channel access procedure with the UL channel access priority class indicated in the DCI corresponding to subframe nk2.

If the UE receives an UL grant and the DCI indicates a PUSCH transmission starting in subframe n using Type 1 channel access procedure, and if the UE has an ongoing Type 1 channel access procedure before subframe n.

-	if the UL channel access priority class value  used for the ongoing Type 1 channel access procedure is same or larger than the UL channel access priority class value  indicated in the DCI, the UE may transmit the PUSCH transmission in response to the UL grant by accessing the carrier by using the ongoing Type 1 channel access procedure.

-	if the UL channel access priority class value  used for the ongoing Type 1 channel access procedure is smaller than the UL channel access priority class value indicated in the DCI, the UE shall terminate the ongoing channel access procedure.

If the UE

-	is scheduled to transmit on a set of carriers C in subframe n, and if the UL grants scheduling PUSCH transmissions on the set of carriers Cindicate Type 1 channel access procedure, and if the same 'PUSCH starting position' is indicated for all carriers in the set of carriers C , or

-	intends to perform an autonomous uplink transmission on the set of carriers C in subframe n with Type 1 channel access procedure, and if the same NStartFS3 is used for all carriers in the set of carriers C, and

if the carrier frequencies of set of carriers C is a subset of one of the sets of carrier frequencies defined in Clause 5.7.4 in [2]

-	the UE may transmit on carrier ci∈C using Type 2 channel access procedure,

-	if Type 2 channel access procedure is performed on carrier ciimmediately before the UE transmission on carrier cj∈C, i≠j, and

-	if the UE has accessed carrier cjusing Type 1 channel access procedure,

-	where carrier cjis selected by the UE uniformly randomly from the set of carriers C before performing Type 1 channel access procedure on any carrier in the set of carriers C.

If the UE is scheduled to transmit on carrier  by a UL grant received on carrier cj, i≠j, and if the UE is transmitting using autonomous UL on carrier , the UE shall terminate the ongoing PUSCH transmissions using the autonomous UL at least one subframe before the UL transmission according to the received UL grant.

If the UE is scheduled by a UL grant received on a carrier to transmit a PUSCH transmission(s) starting from subframe n on the same carrier using Type 1 channel access procedure and if at least for the first scheduled subframe occupies  resource blocks  and the indicated 'PUSCH starting position is OFDM symbol zero, and if the UE starts autonomous UL transmissions before subframe n using Type 1 channel access procedure on the same carrier, the UE may transmit UL transmission(s) according to the received UL grant from subframe n without a gap, if the priority class value of the performed channel access procedure is larger than or equal to priority class value indicated in the UL grant, and the autonomous UL transmission in the subframe preceding subframe n shall end at the last OFDM symbol of the subframe regardless of the higher layer parameter endingSymbolAUL. The sum of the lengths of the autonomous UL transmission(s) and the scheduled UL transmission(s) shall not exceed the maximum channel occupancy time corresponding to the priority class value used to perform the autonomous uplink channel access procedure. Otherwise, the UE shall terminate the ongoing autonomous UL transmission at least one subframe before the start of the UL transmission according to the received UL grant on the same carrier.

If subframe n occurs within the time interval starting at t0and ending at t0+TCO, where TCO=Tmcot,p+Tg, where

-	t0 is the time instant when the eNB has started transmission on the carrier according to the channel access procedure described in clause 4.1.1,

-	Tmcot,pvalue is determined by the eNB as described in clause 4.1.1,

-	Tg is the total duration of all gaps of duration greater than 25us that occur between the DL transmission of the eNB and UL transmissions scheduled by the eNB, and between any two UL transmissions scheduled by the eNB starting from t0,

then,

-	the eNB may indicate Type 2 channel access procedure in the DCI of an UL grant scheduling transmission(s) including PUSCH on a carrier in subframe n when the eNB has transmitted on the carrier according to the channel access procedure described in clause 4.1.1,

-	or the eNB may indicate using the 'UL duration and offset' field that the UE may perform a Type 2 channel access procedure for transmissions(s) including PUSCH on a carrier in subframe nwhen the eNB has transmitted on the carrier according to the channel access procedure described in clause 4.1.1,

-	or the eNB may indicate using the 'UL duration and offset' field and 'COT sharing indication for AUL' field that a UE configured with autonomous UL may perform a Type 2 channel access procedure for autonomous UL transmissions(s) including PUSCH on a carrier in subframe n when the eNB has transmitted on the carrier according to the channel access procedure described in clause 4.1.1 and acquired the channel using the largest priority class value and the eNB transmission includes PDSCH,

-	or the eNB may schedule transmissions including PUSCH on a carrier in subframe n, that follows a transmission by the eNB on that carrier after Tshort_ul=25us.

The eNB shall schedule UL transmissions between t0and t0+TCOin contiguous subframes if they can be scheduled contiguously.

For an UL transmission on a carrier that follows a transmission by the eNB on that carrier after Tshort_ul=25us, the UE may use Type 2 channel access procedure for the UL transmission.

If the eNB indicates Type 2 channel access procedure for the UE in the DCI, the eNB indicates the channel access priority class used to obtain access to the channel in the DCI.

#### 4.2.1.1	Type 1 UL channel access procedure

The UE may transmit the transmission using Type 1 channel access procedure after first sensing the channel to be idle during the slot durations of a defer duration Td; and after the counter Nis zero in step 4. The counter N is adjusted by sensing the channel for additional slot duration(s) according to the steps described below.

1)	set N=Ninit, where Ninit is a random number uniformly distributed between 0 and CWp, and go to step 4;

2)	if N>0 and the UE chooses to decrement the counter, set N=N−1;

3)	sense the channel for an additional slot duration, and if the additional slot duration is idle, go to step 4; else, go to step 5;

4)	if N=0, stop; else, go to step 2.

5)	sense the channel until either a busy slot is detected within an additional defer duration Td or all the slots of the additional defer duration Td are detected to be idle;

6)	if the channel is sensed to be idle during all the slot durations of the additional defer duration Td, go to step 4; else, go to step 5;

If the UE has not transmitted a transmission including PUSCH or SRS on a carrier on which LAA Scell(s) transmission(s) are performed after step 4 in the procedure above, the UE may transmit a transmission including PUSCH or SRS on the carrier, if the channel is sensed to be idle at least in a slot duration Tsl when the UE is ready to transmit the transmission including PUSCH or SRS, and if the channel has been sensed to be idle during all the slot durations of a defer duration Td immediately before the transmission including PUSCH or SRS. If the channel has not been sensed to be idle in a slot duration Tsl when the UE first senses the channel after it is ready to transmit, or if the channel has not been sensed to be idle during any of the slot durations of a defer duration Td immediately before the intended transmission including PUSCH or SRS, the UE proceeds to step 1 after sensing the channel to be idle during the slot durations of a defer duration Td.

The defer duration Td consists of duration immediately followed by mpconsecutive slot durations where each slot duration is Tsl=9us, and includes an idle slot duration Tslat start of ;

A slot duration Tslis considered to be idle if the UE senses the channel during the slot duration, and the power detected by the UE for at least 4uswithin the slot duration is less than energy detection threshold XThresh. Otherwise, the slot duration Tsl is considered to be busy.

CWmin,p≤CWp≤CWmax,p is the contention window. CWpadjustment is described in clause 4.2.2.

CWmin,p and are chosen before step 1 of the procedure above.

mp, CWmin,p, and  are based on channel access priority class signalled to the UE, as shown in Table 4.2.1-1.

XThresh adjustment is described in clause 4.2.3.

#### 4.2.1.2	Type 2 UL channel access procedure

If the UL UE uses Type 2 channel access procedure for a transmission including PUSCH, the UE may transmit the transmission including PUSCH immediately after sensing the channel to be idle for at least a sensing interval Tshort_ul=25us. Tshort_ul consists of a duration Tf=16usimmediately followed by one slot duration  and includes an idle slot duration Tslat start of . The channel is considered to be idle for Tshort_ulif it is sensed to be idle during the slot durations of Tshort_ul.

### 4.2.2	Contention window adjustment procedure

If the UE transmits transmissions using Type 1 channel access procedure that are associated with channel access priority class pon a carrier, the UE maintains the contention window value CWp and adjusts CWp for those transmissions before step 1 of the procedure described in clause 4.2.1.1, using the following procedure

-	If the UE receives an UL grant or an AUL-DFI, the contention window size for all the priority classes is adjusted as following:

-	If the NDI value for at least one HARQ process associated with HARQ_ID_ref is toggled, or if the HARQ-ACK value(s) for at least one of the HARQ processes associated with HARQ_ID_ref received in the earliest AUL-DFI after nref+3 indicates ACK.

-	For every priority class set CWp=CWmin,p

-	Otherwise, increase CWp for every priority class to the next higher allowed value;

-	If there exist one or more previous transmissions {T0, … , Tn} using Type 1 channel access procedure, from the start subframe(s) of the previous transmission(s) of which, N or more subframes have elapsed and neither UL grant nor AUL-DFI was received, where N = max (contentionWindowSizeTimer, Ti burst length+1) if contentionWindowSizeTimer > 0 and N = 0 otherwise, for each transmission Ti, CWp is adjusted as following:

-	increase CWp for every priority class  to the next higher allowed value;

-	The CWp is adjusted once

-	Else if the UE transmits transmissions using Type 1 channel access procedure before N subframes have elapsed from the start of previous UL transmission burst using Type 1 channel access procedure and neither UL grant nor AUL-DFI is received,

-	the CWp is unchanged.

-	If the UE receives an UL grant or an AUL-DFI indicates feedback for one or more previous transmissions {T0, … , Tn} using Type 1 channel access procedure, from the start subframe(s) of the previous transmission(s) of which, N or more subframes have elapsed and neither UL grant nor AUL-DFI was received, where N = max (contentionWindowSizeTimer, Ti burst length+1) if contentionWindowSizeTimer > 0 and N = 0 otherwise, the UE may recompute CWp as follows:

-	The UE reverts CWp to the value used to transmit at nT0 using Type 1 channel access procedure.

-	The UE updates CWp sequentially in the order of the transmission {T0, … , Tn}

-	If the NDI value for at least one HARQ process associated with HARQ_ID_ref' is toggled, or if the HARQ-ACK value(s) for at least one of the HARQ processes associated with HARQ_ID_ref' received in the earliest AUL-DFI after nTi+3 indicates ACK.

-	For every priority class set CWp=CWmin,p

-	Otherwise, increase CWp for every priority class to the next higher allowed value

-	If the UE transmits transmissions using Type 1 channel access procedure before N subframes have elapsed from the start of previous UL transmission burst using Type 1 channel access procedure and neither UL grant nor AUL-DFI is received,

-	CWp is unchanged.

HARQ_ID_ref is the HARQ process ID of UL-SCH in reference subframe nref. The reference subframe nref is determined as follows

-	If the UE receives an UL grant or an AUL-DFI in subframe ng, subframe nwis the most recent subframe before subframe ng−3 in which the UE has transmitted UL-SCH using Type 1 channel access procedure.

-	If the UE transmits transmissions including UL-SCH without gaps starting with subframe n0and in subframes n0,n1,⋯,nw and the UL-SCH in subframe n0 is not PUSCH Mode 1 that starts in the second slot of the subframe, reference subframe nref is subframe n0,

-	If the UE transmits transmissions including PUSCH Mode 1 without gaps starting with second slot of subframe n0 and in subframes n0,n1,⋯,nw and the, reference subframe nref is subframe n0 and n1,

-	otherwise, reference subframe nref is subframe nw,

HARQ_ID_ref' is the HARQ process ID of UL-SCH in reference subframe nTi. The reference subframe nTi is determined as the start subframe of a transmission Ti using Type 1 channel access procedure and of which, N subframes have elapsed and neither UL grant nor AUL-DFI was received.

If the AUL-DFI with DCI format 0A is indicated to a UE that is activated with AUL transmission and transmission mode 2 is configured for the UE for grant-based uplink transmissions, the spatial HARQ-ACK bundling shall be performed by logical OR operation across multiple codewords for the HARQ process not configured for autonomous UL transmission.

If CWp changes during an ongoing channel access procedure, the UE shall draw a counter Ninit and applies it to the ongoing channel access procedure.

The UE may keep the value of CWpunchanged for every priority class, if the UE scheduled to transmit transmissions without gaps including PUSCH in a set subframes n0,n1,⋯,nw−1using Type 1 channel access procedure, and if the UE is not able to transmit any transmission including PUSCH in the set of subframes.

The UE may keep the value of CWp for every priority class the same as that for the last scheduled transmission including PUSCH using Type 1 channel access procedure, if the reference subframe for the last scheduled transmission is also nref.

If CWp=CWmax,p, the next higher allowed value for adjusting CWpis CWmax,p.

If the CWp=CWmax,p is consecutively used K times for generation of Ninit,CWp is reset to CWmin,p only for that priority class pfor which CWp=CWmax,p is consecutively used K times for generation of Ninit. K is selected by UE from the set of values {1, 2, …,8} for each priority class .

### 4.2.3	Energy detection threshold adaptation procedure

A UE accessing a carrier on which LAA Scell(s) transmission(s) are performed, shall set the energy detection threshold (XThresh) to be less than or equal to the maximum energy detection threshold .

is determined as follows:

-	If the UE is configured with higher layer parameter 'maxEnergyDetectionThreshold-r14',

-	is set equal to the value signalled by the higher layer parameter.

-	otherwise

-	the UE shall determine X'Thresh_maxaccording to the procedure described in clause 4.2.3.1

-	if the UE is configured with higher layer parameter 'energyDetectionThresholdOffset-r14'

-	is set by adjusting X'Thresh_maxaccording to the offset value signalled by the higher layer parameter

-	otherwise

-	The UE shall set XThresh_max=X'Thresh_max

#### 4.2.3.1	Default maximum energy detection threshold computation procedure

If the higher layer parameter 'absenceOfAnyOtherTechnology-r14' indicates TRUE:

-	X'Thresh_max=minTmax+10dB,where

-	 is Maximum energy detection threshold defined by regulatory requirements in dBm when such requirements are defined, otherwise

otherwise

-	X'Thres_max=max−72+10⋅log10(BWMHz20MHz) dBm,

Where

-	= 10dB

-	 = 23 dBm;

-	PTXis the set to the value of PCMAX_H,c as defined in [3].

-	;

-	BWMHz is the single carrier bandwidth in MHz.

Annex X (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018-04 | RAN1#92bis | R1-1804453 |  |  |  | First version | 0.0.1 |
| 2018-04 | RAN1#92bis | R1-1805351 |  |  |  | Removal of FeLAA agreements | 0.0.2 |
| 2018-04 | RAN1#92bis | R1-1805352 |  |  |  | Addition of FeLAA agreements | 0.0.3 |
| 2018-04 | RAN1#92bis | R1-1805416 |  |  |  | First endorsed version | 0.1.0 |
| 2018-05 | RAN1#92bis | R1-1805788 |  |  |  | Correction to FeLAA agreements and alignment with other specifications | 0.1.1 |
| 2018-05 | RAN1#92bis | R1-1805790 |  |  |  | Second endorsed version | 0.2.0 |
| 2018-05 | RAN1#93 | R1-1807911 |  |  |  | Update based on agreements at RAN1 #93 | 0.2.1 |
| 2018-06 | RAN1#93 | R1-1807932 |  |  |  | Endorsed version | 1.0.0 |
| 2018-06 | RAN#80 |  |  |  |  | Spec under change control further to RAN approval decision | 15.0.0 |
| 2018-09 | RAN#81 | RP-181795 | 0001 | 1 | F | Correction on RRC parameters for FeLAA in 37.213 | 15.1.0 |
| 2018-09 | RAN#81 | RP-181795 | 0002 | - | F | Correction on starting position of Partial PUSCH Mode 1 for FeLAA in 37.213 | 15.1.0 |
| 2018-09 | RAN#81 | RP-181795 | 0003 | - | F | Correction on COT length for AUL transmission | 15.1.0 |
| 2019-03 | RAN#83 | RP-190444 | 0004 | - | F | Corrections on channel access procedures in 37.213 | 15.2.0 |
| 2020-03 | RAN#87-e | RP-200200 | 0006 | - | F | Correction on conditions for indicating Type 2 channel access procedure | 15.3.0 |