APPENDIX I

(to Recommendation Q.764)

Contents of the interface elements between
the ISDN user part and the SCCP

These interface elements are defined in SCCP Recommendations (Q.711–Q.716) and are included here for information.

## I.1	Contents of the REQUEST type 1

The REQUEST type 1 interface element may contain the following parameters:

–	connection identification (for further study);

–	receipt confirmation selection;

–	expedited data selection;

–	quality of service parameter set.

## I.2	Contents of the REQUEST type 2

The REQUEST type 2 interface element may contain the following parameters:

–	network indicator (for further study);

–	protocol class;

–	credit;

–	connection identification (for further study);

–	source local reference;

–	originating signalling point code;

–	reply request;

–	refusal indicator.

## I.3	Contents of the REPLY

The REPLY interface element may contain the following parameters:

–	source local reference;

–	protocol class;

–	credit;

–	connection identification (for further study).