MTP LEVEL 2

TEST NUMBER:	5.5

PAGE:	1 OF 1

REFERENCE:	Q.703 § 2	STD:	Fig. 11

TITLE:	SU delimitation, alignment, error detection and correction

SUB TITLE:	Reception of single and multiple flags between MSUs

PURPOSE:	To check that single and multiple flags between MSUs can be received

PRE–TEST CONDITIONS:	Link in service

CONFIGURATION:	1

TYPE OF TEST:	VAT

EXPECTED SIGNAL UNIT SEQUENCE:

SP		B

SP		A

Link

Link

1 – 0	FISU

---------------------------->

case 1

MSU F MSU

case 2

MSU F F MSU

n( ³ 2)

F:	Flag

n=number of flags

1 – 0	FISU

---------------------------->

TEST DESCRIPTION

1.

Check that single and n flags, case 1 and case 2 respectively, can be received.

MTP LEVEL 2

TEST NUMBER:	6.1

PAGE:	1 OF 1

REFERENCE:	Q.703 § 10.2	STD:	Fig. 11, Fig. 18, Fig. 8

TITLE:	SUERM check

SUB TITLE:	Error rate of 1 in 256 – Link remains in service

PURPOSE:	To check the SUERM at a link error rate of 1 in 256 units

PRE–TEST CONDITIONS:	Link in service

CONFIGURATION:	1

TYPE OF TEST:	VAT

EXPECTED SIGNAL UNIT SEQUENCE:

SP		B

SP		A

Link

Link

<--------------------------

1 – 0	FISU

1 – 0	FISU

---------------------------->

½
Ct	½  :  corrupt 1
	½  : in 256
	½

TEST DESCRIPTION

1.

Check that “In service” state is maintained. The test should run for several minutes.

2.

Ct = the count of corrupted FISUs.

Note – 1)	The number (x) of corrupt signal units before an SIOS returned is calculated according to the following formula (a = number of correct signal units):

x = for a < 256

2)In this case as a = 255, so x = infinity.

MTP LEVEL 2

TEST NUMBER:	6.2

PAGE:	1 OF 1

REFERENCE:	Q.703 § 10.2	STD:	Fig. 11, Fig. 18, Fig. 8

TITLE:	SUERM check

SUB TITLE:	Error rate of 1 in 254 – Link out of service

PURPOSE:	To check the SUERM at a link error rate of 1 in 254 units

PRE–TEST CONDITIONS:	Link in service

CONFIGURATION:	1

TYPE OF TEST:	VAT

EXPECTED SIGNAL UNIT SEQUENCE:

SP		B

SP		A

Link

Link

<---------------------------

1 – 0			FISU

1 – 0	FISU

---------------------------->

½
	½  : corrupt 1
	½  : in 254
Ct	½

<---------------------------

1 – 0			SIOS

TEST DESCRIPTION

1.

SIOS should be returned after approx. 8192 corrupt FISUs (eg. CRC error).

2.

Ct = the count of corrupted FISUs.

MTP LEVEL 2

TEST NUMBER:	6.3

PAGE:	1 OF 1

REFERENCE:	Q.703 § 10.2	STD:	Fig. 11, Fig. 18, Fig. 8

TITLE:	SUERM check

SUB TITLE:	Consecutive corrupted SUs

PURPOSE:	To test the SUERM on consecutive corrupted signal units

PRE–TEST CONDITIONS:	Link in service

CONFIGURATION:	1

TYPE OF TEST:	VAT

EXPECTED SIGNAL UNIT SEQUENCE:

SP		B

SP		A

Link

Link

<--------------------------

1 – 0			FISU

1 – 0	FISU

--------------------------->

½
	½  : corrupt 1
	½  : in 1
Ct	½

<--------------------------

1 – 0			SIOS

TEST DESCRIPTION

1.

SIOS should be returned after approx. 64 corrupt FISUs (eg. CRC error).

2.

Ct = the count of corrupted FISUs.

MTP LEVEL 2

TEST NUMBER:	6.4

PAGE:	1 OF 1

REFERENCE:	Q.703 § 10.2 	STD:	Fig. 11, Fig. 18

TITLE:	SUERM check

SUB TITLE:	Time controlled break of the link

PURPOSE:	To check response to a range of time controlled breaks of Tx or Rx

PRE–TEST CONDITIONS:	Link in service

CONFIGURATION:	1

TYPE OF TEST:	VAT

EXPECTED SIGNAL UNIT SEQUENCE:

SP		B

SP		A

Link

Link

<--------------------------

1 – 0			FISU

1 – 0	FISU

--------------------------->

:  break Tx

:  restore Tx

FISU

--------------------------->

<--------------------------

1 – 0		FISU

TEST DESCRIPTION

1.

Break the transmission link, and restore before level 2 goes out of service. (Break time is less than approx. 128ms for 64 kbit/s).

2.

Check that A enters and leaves the octet counting mode on reception of an FISU.

MTP LEVEL 2

TEST NUMBER:	7.1

PAGE:	1 OF 1

REFERENCE:	Q.703 § 10.3	STD:	Fig. 9, Fig. 11, Fig. 17

TITLE:	AERM check

SUB TITLE:	Error rate below the normal threshold

PURPOSE:	To test the AERM on error rates below the normal threshold

PRE–TEST CONDITIONS:	Link out of service

CONFIGURATION:	1

TYPE OF TEST:	VAT

EXPECTED SIGNAL UNIT SEQUENCE:

SP		B

SP		A

Link

Link

<-------------------------

1 – 0			SIOS

1 – 0	SIOS

--------------------------->

:	start

<--------------------------

1 – 0			SIO

1 – 0	SIO

--------------------------->

<--------------------------

1 – 0			SIN

1 – 0	SIN

--------------------------->

½

corrupt LSSUs

--------------------------->

½  T4

--------------------------->

½

1 – 0	SIN

<--------------------------

1 – 0			FISU

TEST DESCRIPTION

1.

Start link at A.

2.

Generate x number of corrupt LSSUs (e.g. CRC error) at B.(x < Tin).

3.

Check that the proving period continues and the link aligns successfully.

MTP LEVEL 2

TEST NUMBER:	7.2

PAGE:	1 OF 1

REFERENCE:	Q.703 § 10.3	STD:	Fig. 9, Fig. 11, Fig. 17

TITLE:	AERM check

SUB TITLE:	Error rate at the normal threshold

PURPOSE:	To test the AERM at an error rate equal to the normal threshold

PRE–TEST CONDITIONS:	Link out of service

CONFIGURATION:	1

TYPE OF TEST:	VAT

EXPECTED SIGNAL UNIT SEQUENCE:

SP		B

SP		A

Link

<--------------------------

1 – 0		SIOS

1 – 0	SIOS

---------------------------->

:	start

<--------------------------

1 – 0		SIO

1 – 0	SIO

--------------------------->

<--------------------------

1 – 0		SIN

1 – 0	SIN

--------------------------->

1 – 0	corrupt LSSUs

--------------------------->

SIN

--------------------------->

½

½  T4

½

<--------------------------

1 – 0		FISU

TEST DESCRIPTION

1.

Start link at A.

2.

Generate x number of corrupt LSSUs (e.g. CRC error) at B.(x  Tin).

3.

Check that the proving period is aborted, then restarted and link aligns successfully.

MTP LEVEL 2

TEST NUMBER:	7.3

PAGE:	1 OF 1

REFERENCE:	Q.703 § 10.3	STD:	Fig. 9, Fig. 11, Fig. 17

TITLE:	AERM check

SUB TITLE:	Error rate above the normal threshold

PURPOSE:	To test the AERM at an error rate above the threshold over five proving periods

PRE–TEST CONDITIONS:	Link out of service

CONFIGURATION:	1

TYPE OF TEST:	VAT

EXPECTED SIGNAL UNIT SEQUENCE:

SP		B

SP		A

Link

Link

<--------------------------

1 – 0			SIOS

1 – 0	SIOS

--------------------------->

:	start

<--------------------------

1 – 0			SIO

1 – 0	SIO

--------------------------->

<--------------------------

1 – 0			SIN

1 – 0	SIN

--------------------------->

1 – 0	corrupt LSSUs

--------------------------->

<--------------------------

1 – 0			SIN

1 – 0	SIN

--------------------------->

1 – 0	corrupt LSSUs

--------------------------->

<--------------------------

1 – 0			SIN

1 – 0	SIN

--------------------------->

1 – 0	corrupt LSSUs

--------------------------->

<--------------------------

1 – 0			SIN

1 – 0	SIN

--------------------------->

1 – 0	corrupt LSSUs

--------------------------->

<--------------------------

1 – 0			SIN

1 – 0	SIN

--------------------------->

1 – 0	corrupt LSSUs

--------------------------->

<--------------------------

1 – 0			SIOS

TEST DESCRIPTION

1.

Start link at A.

2.

Generate x number of corrupt LSSUs (e.g. CRC error) at B.(x  Tin).

3.

Observe that 5 proving period attempts are made before link out of service state.